3GPP TSG-SA5 Meeting #163	S5-254541

Wuhan, China, 13. - 17. October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0728 | rev | 3 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR 28.552 PM for inter-CU LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-20 mirror. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of PM measurements for inter-gNB LTM. Measurements are added under their already existing placeholder. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OAM will be unable to do basic measurements of the performance of inter-CU LTM, as well as comparing LTM performance with other mobility mechanisms. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.13, 5.1.1.6.13.1 (new), 5.1.1.6.13.2 (new), 5.1.1.6.13.3 (new), 5.1.1.6.13.4 (new), 5.1.1.6.13.5 (new), 5.1.1.6.13.6 (new), 5.1.1.6.13.7 (new), 5.1.1.6.13.8 (new), 5.1.1.6.13.9 (new), 5.1.1.6.13.10 (new), 5.1.1.6.13.11 (new), 5.1.1.6.13.12 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-253675, S5-254108 |  |  |  |  |  |  |  |  |

First change

##### 5.1.1.6.13	Inter-gNB LTM Cell Switches

###### 5.1.1.6.13.1	Number of requested inter-gNB LTM cell switch preparations

a)	This measurement provides the number of inter-gNB LTM cell switch preparations requested by the source gNB.

b)	CC.

c)	On transmission of HANDOVER REQUEST message (see TS 38.423 [13]), where the message denotes an inter-gNB cell switch, by the source NR cell CU to target NR cell CU, for requesting the preparation of resources at the target NR cell CU.

d)	A single integer value.

e)	MM.LTMPrepInterReq.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.13.2	Number of successful inter-gNB LTM cell switch preparations

a)	This measurement provides the number of successful inter-gNB LTM cell switch preparations received by the source NR cell CU.

b)	CC.

c)	On receipt of HANDOVER REQUEST ACKNOWLEDGE message (see TS 38.423 [13]), where the message corresponds to a previously sent inter-gNB LTM cell switch HANDOVER REQUEST message, by the source NR cell CU from the target NR cell CU, for informing that the resources for the handover have been prepared at the target NR cell CU.

d)	A single integer value.

e)	MM.LTMPrepInterSucc.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.13.3	Number of failed inter-gNB LTM cell switch preparations

a)	This measurement provides the number of failed inter-gNB LTM cell switch preparations received by the source NR cell CU. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On receipt of HANDOVER PREPARATION FAILURE message (see TS 38.423 [13]), where the message corresponds to a previously sent inter-gNB LTM cell switch HANDOVER REQUEST message, by the source NR cell CU from the target NR cell CU, for informing that the preparation of resources at the target NR cell CU has failed. Each received HANDOVER PREPARATION FAILURE message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.LTMPrepInterFail.Cause.

Where Cause identifies the failure cause of the handover preparations.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.13.4	Number of requested inter-gNB LTM cell switch resource allocations

a)	This measurement provides the number of inter-gNB LTM cell switch resource allocation requests received by the target NR cell CU.

b)	CC.

c)	On receipt of HANDOVER REQUEST message (see TS 38.423 [13]), where the message denotes an inter-gNB LTM cell switch, by the target NR cell CU from the source NR cell CU, for requesting the preparation of resources for handover.

d)	A single integer value.

e)	MM.LTMResAlloInterReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.13.5	Number of successful inter-gNB LTM cell switch resource allocations

a)	This measurement provides the number of successful inter-gNB LTM cell switch resource allocations at the target NR cell CU for the handover.

b)	CC.

c)	On transmission of HANDOVER REQUEST ACKNOWLEDGE message (see TS 38.423 [13]), where the message corresponds to a previously received inter-gNB LTM cell switch HANDOVER REQUEST message, by the target NR cell CU to the source NR cell CU, for informing that the resources for the handover have been prepared.

d)	A single integer value.

e)	MM.LTMResAlloInterSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.13.6	Number of failed inter-gNB LTM cell switch resource allocations

a)	This measurement provides the number of failed inter-gNB LTM cell switch resource allocations at the target NR cell CU for the handover. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On transmission of HANDOVER PREPARATION FAILURE message (see TS 38.423 [13]), where the message corresponds to a previously sent inter-gNB LTM cell switch HANDOVER REQUEST message, by the target NR cell CU to the source NR cell CU, for informing that the preparation of resources has failed. Each transmitted HANDOVER PREPARATION FAILURE message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.LTMResAlloInterFail.Cause.

Where Cause identifies the failure cause of the inter-gNB LTM cell switch resource allocations.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.13.7	Number of requested inter-gNB LTM cell switch executions

a)	This inter gNB handover measurement provides the number of outgoing inter-gNB LTM cell switch executions requested by the source gNB.

b)	CC.

c)	On transmission of RRCReconfiguration message, where the message denotes an inter-gNB LTM cell switch, to the UE triggering the inter-gNB LTM cell switch from the source NRCellCU to the target NRCellCU, indicating the attempt of an outgoing inter gNB inter-gNB LTM cell switch (see TS 38.331 [20]), the counter is stepped by 1.

d)	A single integer value.

e)	MM.LTMExeInterReq.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.13.8	Number of successful inter-gNB LTM cell switch executions

a)	This inter gNB handover measurement provides the number of successful inter-gNB LTM cell switch executions received by the source gNB.

b)	CC.

c)	On receipt at the source gNB of UE CONTEXT RELEASE TS 38.423 [13] over Xn from the target gNB following a successful handover, where the message denotes an inter-gNB LTM cell switch, the counter is stepped by 1.

d)	A single integer value.

e)	MM.LTMExeInterSucc.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.13.9	Number of failed inter-gNB LTM cell switch executions

a)	This inter gNB handover measurement provides the number of failed inter-gNB LTM cell switch executions for a source gNB.

b)	CC.

c)	This counter is incremented when handover execution failures occur. It is assumed that the UE context is available in the source gNB. The following events are counted:

1)	On reception of RrcReestablishmentRequest (see TS 38.331 [20]) after a failed LTM cell switch, where the reestablishmentCause is handoverFailure, from the UE in the source gNB, where the reestablishment occurred in the source gNB;

2)	On expiry of a Handover Execution supervision timer in the source gNB;

3)	On reception of XnAP RETRIEVE UE CONTEXT REQUEST (see TS 38.423 [13]) in the source gNB, when the reestablishment occurred in another gNB.

As one handover failure might cause more than one of the above events, duplicates need to be filtered out.

d)	Each subcounter is an integer value.

e)	MM.LTMExeInterFail.cause,
 where cause is one of RrcReestabReq, HoExeSupTimer or RetrUeCtxtReq.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.13.10	Number of UEs for which inter-gNB LTM cell switch preparations are requested

a)	This measurement provides the number of UEs for which inter-gNB LTM cell switch preparations were requested by the source gNB.

b)	CC.

c)	On transmission of HANDOVER REQUEST message (see TS 38.423 [13] clause 8.2.1) where the message denotes an inter-gNB LTM cell switch preparation, from the source NR cell CU to the target NR cell CU, for requesting the preparation of resources at the target NR cell CU. The counter is incremented by 1 for each UE, even if HANDOVER REQUEST messages were sent to several cells.

d)	A single integer value.

e)	MM.ChoPrepInterReqUes.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.13.11	Number of UEs for which inter-gNB LTM cell switch preparations were successful

a)	This measurement provides the number of UEs for which successful inter-gNB LTM cell switch preparations were received by the source NR cell CU.

b)	CC.

c)	On receipt of HANDOVER REQUEST ACKNOWLEDGE message (see TS 38.423 [13] clause 8.2.1) where the message corresponds to a previously sent inter-gNB LTM cell switch HANDOVER REQUEST message by the source NR cell CU, from the target NR cell CU, for informing that the resources for the inter-gNB LTM cell switch have been prepared at the target NR cell CU. The counter is incremented by 1 for each UE, even if HANDOVER REQUEST ACKNOWLEDGE messages were received from several cells.

d)	A single integer value.

e)	MM.ChoPrepInterSuccUes.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.13.12	Number of UEs for which inter-gNB LTM cell switch preparations failed

a)	This measurement provides the number of UEs for which inter-gNB LTM cell switch preparations failed, as received by the source NR cell CU. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On receipt of HANDOVER PREPARATION FAILURE message (see TS 38.423 [13] clause 8.2.1.3) where the message corresponds to a previously sent inter-gNB LTM cell switch HANDOVER REQUEST message by the source NR cell CU, from the target NR cell CU, for informing that the preparation of resources at the target NR cell CU has failed. Each received HANDOVER PREPARATION FAILURE message increments the relevant subcounter per failure cause by 1. The counter is incremented by 1 for each UE, even if HANDOVER PREPARATION FAILURE messages were received from several cells.

d)	Each subcounter is an integer value.

e)	MM.ChoPrepInterFailUes.cause.

where cause identifies the failure cause of the inter-gNB LTM cell switch preparations.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurements is for performance assurance

End of changes