3GPP TSG-SA5 Meeting #163	S5-254592

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0745 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS28.552 Fix MOI for VR usage of NF related PMs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed expla下nations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | GNBCUFunction is invalid MOI in bullet f) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify GNBCUFunction to NRCellCU in VR usage of NF related PMs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid MOI in bullet f) causes confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.1.1, 5.7.1.2.1, 5.7.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

### 5.7.1	VR usage of NF

#### 5.7.1.1	Virtual CPU usage

##### 5.7.1.1.1	Mean virtual CPU usage

a)	This measurement provides the mean usage of the underlying virtualized CPUs for a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the VcpuUsageMeanVnf.vComputeId measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of each received measurement from VNFC instance to the MOI(s) of NF(s). The measurement is generated by taking the weighted average of the values of the VcpuUsageMeanVnf.vComputeId measurement(s) whose measured object(s) are mapped to the MOI of the measured NF. The algorithm of the weighted average is vendor specific.

d)	A single integer value (Unit: %).

e)	VR.VCpuUsageMean

f)	GNBCUCPFunction (for 3 split scenario)
GNBCUUPFunction (for 3 split scenario)
NRCellCU (for 2 split scenario)
AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

EASFunction

EESFunction

ECSFunction

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.7.1.2	Virtual memory usage

##### 5.7.1.2.1	Mean virtual memory usage

a)	This measurement provides the mean usage of the underlying virtualized memories for a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the VmemoryUsageMeanVnf.vComputeId measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of each received measurement from VNFC instance to the MOI(s) of NF(s). The measurement is generated by taking the weighted average of the values of the VmemoryUsageMeanVnf.vComputeId measurement(s) whose measured object(s) are mapped to the MOI of the measured NF. The algorithm of the weighted average is vendor specific.

d)	A single integer value (Unit: %).

e)	VR.VMemoryUsageMean

f)	GNBCUCPFunction (for 3 split scenario)
GNBCUUPFunction (for 3 split scenario)
NRCellCU (for 2 split scenario)
AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

EASFunction

EESFunction

ECSFunction

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.7.1.3	Virtual disk usage

##### 5.7.1.3.1	Mean virtual disk usage

a)	This measurement provides the mean usage of the underlying virtualized disks for a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the VdiskUsageMeanVnf.vComputeId measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of each received measurement from VNFC instance to the MOI(s) of NF(s). The measurement is generated by taking the weighted average of the values of the VdiskUsageMeanVnf.vComputeId measurement(s) whose measured object(s) are mapped to the MOI of the measured NF. The algorithm of the weighted average is vendor specific.

d)	A single integer value (Unit: %).

e)	VR.VDiskUsageMean

f)	GNBCUCPFunction (for 3 split scenario)
GNBCUUPFunction (for 3 split scenario)
NRCellCU(for 2 split scenario)
AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

EASFunction

EESFunction

ECSFunction

g)	Valid for packet switched traffic.

h)	5GS.

*** END OF CHANGE 1 ***