3GPP TSG-SA WG6 Meeting #69	S6-254069

Wuhan, China 13th – 17th October 2025

Source:	InterDigital

Title:	New solution on KI#3

Spec:	3GPP TR 23.700-26

Agenda item:	9.8

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes a new solution for KI#3 on enabling provisioning and presence monitoring for AIoT devices.

2. Reason for Change

This contribution proposes a new solution for KI#3 which is related to aspects of provisioning information and enabling presence monitoring of AIoT devices by the application enablement layer.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

## 7.x	Solution #x: Support of information provisioning for AIoT device presence monitoring

### 7.x.1	Solution description

#### 7.x.1.1 Introduction

This solution addresses Key Issue #3 on provisioning information in the application enablement layer an enabling presence monitoring of AIoT devices.

This solution proposes that an AIoT Enablement Server (AIOTES) offers a service to authorized VAL Servers for provisioning the information required to enable AIoT device presence monitoring.

This solution proposes to:

- define a new procedure and information flows to enable provisioning of AIoT presence information in the AIOTES for presence reporting;

- define a new procedure and information flows to enable managing a presence reporting session;

- define a new procedure and information flow for detecting events and reporting presence of AIoT devices.

#### 7.x.1.2 AIoT presence information provisioning

Figure 7.x.1.2-1 describes a procedure for AIoT presence information provisioning.

Figure 7.x.1.2-1. AIoT Presence Information Provisioning

1.	The requestor (e.g., VAL server) sends an AIoT presence information provisioning request to the AIOTES. The request includes the requestor identifier, information about AIoT device(s) [device identifier, area information] and information about the presence reporting configuration [notification endpoint, frequency, schedule, location indication, change only indication].

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES allocates a presence reporting session identifier, and stores the associated AIoT device information and presence reporting configuration.

If the request indicates immediate reporting, the AIOTES initiates the presence monitoring by sending the inventory command to the core network as described in 3GPP TS 23.369 [3] for the provisioned AIoT devices, and configures the reporting based on the presence reporting configuration. If the AIOTES sends the inventory command and the command is successful, the AIOTES stores the transaction identifier returned by the inventory command. The AIOTES sends inventory command to the core network as needed based on the requested reporting configuration.

3.	The AIOTES sends an AIoT Information Provisioning response to the VAL server. If the request was authorized, the response includes an indication of success for the request, the presence reporting session identifier and an indication that the presence monitoring has started. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.x.1.3 AIoT presence reporting

Figure 7.x.1.3-1 describes a procedure for AIoT presence reporting.

Figure 7.x.1.3-1. AIoT Presence Reporting

1.	The requestor (e.g., VAL server) sends an AIoT presence reporting request to the AIOTES. The request includes the requestor identifier, the presence reporting session identifier that was returned in the AIoT presence information provisioning procedure in clause 7.1.x.2, and an indication whether to start or stop or notify for the presence reporting session identifier.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized and the reporting session identifier is valid, the AIOTES retrieves the AIoT device information and presence reporting configuration associated with the reporting session identifier.

If the request includes an indication to start the reporting session and the reporting session is not already started, the AIOTES initiates the presence monitoring by sending the inventory command to the core network as described in 3GPP TS 23.369 [3] for the provisioned AIoT devices, and configures the reporting based on the received configuration. If the AIOTES sends the inventory command and the command is successful, the AIOTES stores the transaction identifier. The AIOTES sends inventory command to the core network as needed based on the requested reporting configuration.

If the request includes an indication to stop the reporting session, the AIOTES stops reporting presence information to the endpoint configured in the reporting configuration.

If the request includes an indication to notify, the AIOTES immediately notifies the endpoint configured in the reporting configuration with the known AIOTES presence state as described in clause 7.x.1.4.

3.	The AIOTES sends an AIoT presence reporting  response to the VAL server. If the request was authorized and the reporting session identifier valid, the response includes an indication of success for the request. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.x.1.4 AIoT presence reporting notification

Figure 7.x.1.4-1 describes a procedure for AIoT presence reporting notification.

Figure 7.x.1.4-1. AIoT Presence Reporting Notification

1.	The AIOTES detects a presence reporting event, which may be one of the core network sending an inventory command  notification to the AIOTES or the VAL server sending an AIoT presence reporting request including a notify indication.

2.	The AIOTES sends an AIoT presence reporting notification to the endpoint configured in the presence reporting configuration.

If the detected event is a notification from the core network to an inventory command as described in 3GPP TS 23.369 [3], the AIOTES uses the received transaction identifier to retrieve the associated reporting session identifier, reporting session configuration and AIoT device information. The AIOTES updates the latest AIoT device presence information (e.g., presence, location) for the AIoT devices associated with the reporting session and includes the updated presence information in the notification. If the received core network notification indicates the last report indication, and presence reporting is still needed, the AIOTES sends a new inventory command to the core network and associates the new transaction identifier with the reporting session identifier.

If the detected event is an AIoT presence reporting request from the VAL server including a notify indication, the AIOTES uses the reporting session identifier included in the request to retrieve the reporting session configuration and includes the latest AIoT device presence information available locally in the notification.

### 7.x.2	Architecture Impacts

This solution proposes that a new functional entity, as an AIoT Enablement Server (AIOTES) which is a SEAL Server, is supported in the application enablement layer.

### 7.x.3	Corresponding APIs

#### 7.x.3.1	AIoT Presence Information Provisioning request

Table 7.x.3.1-1 describes the AIoT Presence Information Provisioning request.

Table 7.x.3.1-1: AIoT Presence Information Provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Immediate reporting indication | O | The indication that presence monitoring is initiated immediately. |
| AIoT device information | M | The information about AIoT devices. |
| > AIoT device identifiers | M | The list of device identifiers for which the presence needs to be monitored |
| > Area information | O | The area information where AIoT devices are expected to be located. |
| Reporting configuration | M | The configuration for presence reporting. |
| > Notification endpoint | M | The endpoint information (e.g., URL, URI, IP address) where presence reports should be sent. |
| > Frequency | O | The frequency at which the presence is reported to the requestor. |
| > Schedule | O | The time schedule (e.g., duration, time of day, period) at which presence reporting is required. |
| > Location information indication | O | The indication that location information is requested in the presence report. |
| > Change only indication | O | The indication that only device for which the presence status has changed is included in a report. If the IE is absent, the report includes all device. |

#### 7.x.3.2	AIoT Presence Information Provisioning response

Table 7.x.3.2-1 describes the AIoT Presence Information Provisioning response.

Table 7.x.3.2-1: AIoT Presence Information Provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > Reporting session identifier | M | The identifier for the presence reporting session. |
| > Session initiated indication | O | The indication that presence monitoring has been initiated. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.x.3.3	AIoT Presence Reporting request

Table 7.x.3.1-1 describes the AIoT Presence Reporting request.

Table 7.x.3.3-1: AIoT Presence Reporting request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Reporting session identifier | M | The identifier for the presence reporting session. |
| Reporting session operation | M | The operation to be performed on the presence reporting session, which may be one of start or stop or notify. |

#### 7.x.3.4	AIoT Presence Information Provisioning response

Table 7.x.3.4-1 describes the AIoT Presence Reporting response.

Table 7.x.3.4-1: AIoT Presence Reporting response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.x.3.5	AIoT Presence Reporting notification

Table 7.x.3.5-1 describes the AIoT Presence Reporting notification.

Table 7.x.3.5-1: AIoT Presence Reporting response

| Information element | Status | Description |
| --- | --- | --- |
| Reporting session identifier | M | The identifier for the presence reporting session. |
| Presence report | M | The presence report information. |
| > AIoT device identifiers | M | The list of device identifiers for which presence is reported |
| > Presence status | M | The status of presence which is one of present or absent or unknown. |
| > Location information | O | The location information of the AIoT device. |
| > Presence status duration | O | The duration since the last presence status change. |

* * * End of Changes * * * *