3GPP TSG-SA WG6 Meeting #69	S6-254082

Wuhan, China, 13th – 17th October 2025

Source:	China Telecom

Title:	New solution for KI#1 on AIML model storage and deployment on satellite

Spec:	3GPP TR 23.700-002 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	Zhou Zhe <zhouz11@chinatelecom.cn>

1. Introduction

Application enabled layer could enhance AIML service by deploying models on satellites, thereby reducing data transmission burden and latency. It is necessary to study how to deploy AIMLE on satellite and how to storage AIML models on satellite for AIMLE service. Thus, TR 23.700-02 v0.1.0 identifies the following  key issue to be addressed:

Key issue #1: AIML model storage and deployed on satellite

This paper proposes a new solution on AIMLE and AIML models deployment on satellite, performing data processing and interference based on collected remote data, to improve AIMLE information and analysis data transmission. AIMLE on board obtains remote data in a certain area via satellite as required by data consumer. With AIMLE entities and AIML models deployed on satellite, AIMLE server provides simple analysis service based on the remote data directly from satellite. The analysis data and result could be transmitted to the data consumer on the ground via feeder links.

2. Reason for Change

To support AIML model storage and deployment on satellite.

3. Conclusions

This paper proposes solution on AIML model storage and deployment on satellite..

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-002 v0.1.0.

* * * First Change * * * *

5.X	Solution#X: AIML model storage and deployment on satellite

5.X.1	Solution Description

This solution proposes to address the following open issues in KI#1

-	Investigate different deployment options for AIMLAPP when AIML models are deployed on board satellite. Investigate different deployment options for AIMLAPP when AIML models are deployed on satellite and study whether/how to deploy AIMLE on satelllite (e.g. ML repository/ADAES on satellite to perform data processing/interference based on collected remote data).

-	Whether/how could AIML model discovery and storage be optimized.Whether/how to storage AIML models for AIMLE (e.g. Light weight models storaged on remote sensing satellite to process collected remote data, Model transmission and sharing between satellites and terrestrial network).

5.X.1.1 General

Figure 5.X.1.1-1: satellite AIMLE deployment

In this deployment option, ​​the​​ UE is on the ground or in the air (e.g., ​​a drone​​). The space AIMLE components (e.g., ​​the​​ AIMLE client) are deployed on ​​a​​ satellite. The RAN (e.g., gNB) can be deployed on ​​the​​ ground/sea (e.g., on a ship). The 5GS control plane functions (e.g., AMF, SMF) are deployed on the ground, ​​though they are not shown in the figure for simplicity​​. ​​The​​ satellite AIMLE client are deployed on ​​the​​ satellite to handle data and information collected by ​​the​​ satellite.

The UE may request the AIMLE server on ground to discover AIMLE client for satellite access information analysis model traning. To satisfy such requirement, an AIMLE client is deployed on board to train the model by utilizing local data on the satellite.

When AIMLE server discovers an appropriate satellite AIMLE client, AIMLE server requests the satellite AIMLE client to attend model training and offers ML model information to the satellite AIMLE client.

5.X.1.2	Procedure

Figure 5.Y.1.2-1: Satellite AIMLE client discovery

Figure 5.Y.1.2-1 illustrates the procedure on satellite AIMLE client discovery. The corresponding procedure in detail is as follow:

- In step 1, AIMLE service consumer requests a AIMLE server to perform AIMLE client discovery, as in TS 23.482 clause 8.8. In order to train a model analyzing satellite access information (e.g. satelllite access RAT and satellite access related QoS/QoE) and satisfy such a request from AIMLE service consumer, the AIMLE server needs to find an AIMLE client that may locally obtain satellite access related information. So the needed AIMLE client should support NTN technology and collect satellite access related RAT and QoS/QoE information, and also handle the above information locally to train a model. To discover such type of AIMLE client, the request from AIMLE service consumer should include the elements to discribe AIMLE clients for satellite access analysis capability. The information of satellite access anaysis capability is specified in Table 5.X.1.3.1-1.

- The AIMLE server performs authentication and authorization as in TS 23.482 clause 8.8. if the requestor is authorized, the AIMLE server performs the AIMLE clients discovery as in TS 23.482 clause 8.8

- The AIMLE server sends an AIMLE client discovery response as in TS 23.482 clause 8.8.

5.X.1.3	Information Flows

5.X.1.3.1	satellite access anaysis capability

Table 5.X.1.3.1-1 describes information elements of satellite access anaysis capability for the AIMLE client discovery request from the AIMLE client to the AIMLE server.

Table 5.X.1.3.1-1: Satellite access anaysis capability

| Information element | Status | Description |
| --- | --- | --- |
| Satellite Access Analysis Capability | M | Indicates AIMLE client capability in satellite access information analysis. |
| > ​​supported NTN types | M | Indicates the ​​Non-Terrestrial Network types​​ supported by an AIMLE client (e.g. NR-NTN, LTE-NTN). |
| > ​​supported Analysis Metrics​​ | M | Indicates the ​​satellite access metrics​​ that an AIMLE client can collect and analyze (e.g. NTN RSRP, NTN RSRQ, Doppler Shift, BER, NTN Latency, NTN Throughput). |
| > ​​positioning Capability​​ | O | Indicates whether the client has ​​high-precision positioning capability​​ (e.g., GNSS). |

5.Y.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

5.Y.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

5.Y.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

* * * End of Changes * * * *