3GPP TSG-SA6 Meeting #69	S6-254160

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0166 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SEALDD signalling transmission connection establishment procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI 20, SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The VAL server could allocate SEALDD-S connection information for receiving traffic from the SEALDD server by using the SEALDD policy configuration. Accordingly, the relevant description should be incorporated into the SEALDD signalling transmission connection establishment procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the mechanism for intitiating SEALDD service by using the SEALDD policy configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mechanism for intitiating SEALDD service by using the SEALDD policy configuration is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.2.2.1	SEALDD enabled signalling transmission connection establishment procedure

Figure 9.2.2.1-1 illustrate the procedure for signalling transmission connection establishment.

Pre-condition:

-	The VAL server has discovered and selected the SEALDD server by CAPIF functions.

Figure 9.2.2.1-1: SEALDD signalling transmission connection establishment procedure

1a.	The VAL server decides to use SEALDD service for application signalling transfer and allocates as SEALDD-S connection information for receiving the application signalling traffic from SEALDD server. The VAL server sends Sdd_RegularTransmission request to the SEALDD server. The service request includes the VAL server ID, VAL service ID to identify the VAL application signalling traffic, the SEALDD-S connection information of the VAL server side.

1b.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server allocates a specific address used for SEALDD signalling traffic transfer with the incoming SEALDD client(s) for the VAL server and responds with a SEALDD service response.

NOTE 1:	If the SEALDD server does not allocate SEALDD-S connection information in this step, then VAL server uses wildcard endpoint as SEALDD-S connection information for SEALDD-S reference point to receive all the application signalling traffic.

2.	Alternatively, the VAL server may initiate the SEALDD service by using the SEALDD policy configuration procedure as described in clause 9.10.

3.	The VAL client sends a SEALDD service request to SEALDD client. The service request also indicates to establish application signalling transmission connection. The VAL client receives a SEALDD service response to the SEALDD client. The response indicates that whether the SEALDD service request is successful or not.

4.	The VAL/SEALDD client discovers and selects the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, and the SEALDD client obtains the SEALDD server's address.

5.	The SEALDD client allocates SEALDD-UU flow ID(s) mapping to application signalling traffic for application signalling transmission. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD-UU flow ID(s), VAL server ID, VAL service ID and the SEALDD traffic descriptor of the SEALDD client side (the SEALDD-S connection information of the SEALDD client for receiving the downlink SEALDD-UU signalling traffic). The request message also contains the selected VAL server endpoint information. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434 [4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the signalling connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the signalling connection establishment, otherwise SEALDD server returns failed result e.g. performs connection reject.

NOTE 2:	The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application signalling traffic, and it is mapped to the VAL service ID.

6.	The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. SEALDD-S connection information, transport layer protocol) mapping to the application traffic.

7.	The SEALDD server stores the SEALDD client ID, SEALDD-UU flow ID(s) to identify the SEALDD-UU signalling traffic and establishes SEALDD-S connection with VAL server for the VAL client to transmit application signalling traffic mapping to the SEALDD traffic. SEALDD server may use different SEALDD connection information to establish the SEALDD-S signalling transmission connection for application signalling transfer towards the VAL server for different SEALDD client and SEALDD-UU flow. Then each VAL client will have different SEALDD-S signalling transmission connection at the SEALDD server side.

8.	The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD signalling connection establishment.

NOTE 3:	If the UE's address for SEALDD traffic transfer is different from the address used in the control plane interaction (step 5 and 6), another SEALDD interaction procedure may be triggered to notify the SEALDD server about the SEALDD-S connection information used by the SEALDD client for SEALDD traffic transfer. Or the SEALDD server reuses the SEALDD client's address used in step 5 for SEALDD traffic transfer.

After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the SEALDD client's address and SEALDD server's address for the SEALDD traffic transmission). The SEALDD client gets the mapping information (i.e. SEALDD-UU flow ID for the application signalling transfer). The SEALDD server gets the mapping information between the SEALDD-UU flow ID, the signalling transmission Session ID and the SEALDD-S connection. The SEALDD client and SEALDD server store the mapping between the application traffic and SEALDD traffic.

Upon receiving application signalling traffic from VAL client, the SEALDD client maps it into SEALDD traffic with SEALDD traffic descriptor as negotiated with SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID, SEALDD-UU flow ID. The SEALDD server sends the received application traffic to VAL server via the connection established in step 7 according to the mapping relationship between the SEALDD-S connection and the SEALDD traffic.

For the downlink application signalling traffic in response to the uplink application signalling, the VAL server responds to the source address (SEALDD-S address of the SEALDD server side) of the uplink signalling traffic. Upon receiving the downlink application signalling traffic from the SEALDD-S connection, the SEALDD server maps the downlink application signalling traffic to the related SEALDD client ID and SEALDD-UU flow ID and send the mapped SEALDD traffic to the SEALDD client. The rest of the downlink application traffic transfer is processed similarly with the uplink traffic.

After the connection establishment, the VAL server communicates with VAL client for application layer signalling traffic transfer via the established SEALDD connection.

NOTE 4: The VAL server communicating with the VAL client may not be the VAL server initiating the SEALDD service in step 1 or step 2.

* * * End of Change * * * *