3GPP TSG-SA WG6 Meeting #69	S6-254164

Wuhan, China, 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	China Mobile

Title:	Overall evaluation and conclusion for KI#1

Spec:	3GPP TR 23.700-51

Agenda item:	9.5

Document for:	Approval

Contact:	Tangqing Liu, liutangqing@chinamobile.com

1. Introduction

This paper proposes to add overall evaluation and conclusion for KI#1.

2. Reason for Change

Add overall evaluation and conclusion for KI#1.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51.

* * * First Change * * * *

7	Overall evaluation

7.1	General

Various solutions proposed in the present document aim to enhance the application enabling layer to support for XR service, including the QoS differentiation and policy control for non-3GPP tethered devices, transmission enhancement etc.

7.2	Architecture evaluation

As discussed in clause 7.1, solutions have dependencies on other application enablement capabilities.

-	6 solutions are based on SEALDD capability, no architecture impact.

-	1 solution is dependent on PINAPP, no architecture impact.

7.3	Solution evaluation

7.3.1	Key issue #1 solutions evaluation

The open issues studied in the Key Issue #1 are as follows:

-	How could application enabling layer be enhanced to support for policy (Multi-modal flows alignment policy, Synchronization policy, UE-to-UE policy, expedited transfer indication, policy enforcement notification, etc.).

-	How could application enabling layer be enhanced to support for XR related policy (e.g. multi-modal XR policy, multi-modal SEALDD policy) configuration, management, synchronization of multiple VAL clients.

Solution #1, Solution #5 and Solution #6 addresses the key issue 1.

Solution #1 resolves open issue 1) and 2) and enable VAL server subscribes to SEALDD server for policy enforcement notification and be notified when policy is enforced.

Solution #5 resolves open issue 2) and enables SEALDD server to provide suitble QoS requirements based ont the type of XR traffic.

Solution #6 resolves open issue 1) and enhancement to support for multi model flow delay differences measurement.

* * * Next Change * * * *

10.1	General conclusions

XR related transmission services are supported based on existing SEALDD architecture and PINAPP architecture. Specific service enhancements are concluded in the following clauses.

10.2	Conclusions of key issue #1

Solution #1, solution#5 and solution #6 can be modified and adopted into normative work with the following principles:

-	Solution #1 can be used to subscribe and notify VAL server of policy enforcement.

-	Solution #5 can be used for XR traffic transmission.

-	Solution #6 can be used for multi model flow delay differences measurement enhancement.

* * * End Change * * * *