# 3GPP TSG-SA WG6 Meeting #69	S6-254175

# Wuhan, China, 13th – 17th Oct 2025

Source:	ZTE Corporation

Title:	Solution for KI#1 on support for sensing results exposure

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Wei Luo <luo.wei11@zte.com.cn>

1. Introduction

This paper proposes a solution to KI#1 on support for sensing results exposure procedure.

2. Reason for Change

This paper proposes a solution to KI# 1 on support for sensing results exposure procedure. This solution addresses the key issue on how to acquire the sensing results from the appropriate sensing service supplier and expose the (enhanced) sensing results to the sensing consumer.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

# 6.x	Solution #x: Support the sensing results exposure based on the sensing subscription request

6.x.1	Solution description

This solution aims to address the issues identified in Key Issue 1 and provides a possible procedure to perform the sensing results exposure.

In this solution it is proposed to introduce a SEAL Sensing Management Server (SMS), which determines and coordinates a sensing procedure between sensing service supplier and consumer. The SEAL SMS selects the appropriate sensing service supplier(s) and determines the sensing report configuration according to the sensing target and requirements of the sensing consumer. After receiving the sensing results from one or more sensing service suppliers for the same sensing target, it may further generate enhanced sensing results based on the received results and expose them to the sensing service consumer.

In this solution, the sensing service supplier which can provide sensing results can be a 5GS NF or vertical application server/client. The  sensing service consumer can be a VAL server or client.

6.x.2	Procedures

The high-level procedure of sensing service exposure is shown in Figure 6.x.2-1.

Pre-conditions:

- The Sensing Management Server has received and stored one or more Sensing Service Supplier profile(s) information.

Figure 6.x.2-1: Sensing service exposure procedure

- The VAL Server/Client acting as sensing consumer performs sensing service subscription procedure by sending sensing service subscription request and receiving sensing service subscription response. During the sensing service subscription procedure, the SMS can acquire the sensing requirements of the VAL Server/Client such as Sensing Service Area of interest, Sensing target object type, Accuracy requirements, QoS requirements and so on.

- The SMS selects one or more Sensing Service Supplier(s) according to the stored Sensing Service Supplier profile(s) information and sensing requirement as received in step1 (e.g. by matching Sensing Service Supplier profile information to the sensing subscription parameters received from the sensing consumer), and determines the sensing report configuration (e.g., event triggered report or periodic report) for each selected Sensing Service Supplier.

- The SMS sends sensing report configuration to each selected Sensing Service Supplier. Different Sensing Service Supplier may be given different sensing reporting configuration.

- Each Sensing Service Supplier sends sensing results to the sensing management server based on the sensing report configuration respectively.

- If the SMS receives sensing results from more than one Sensing Service Suppliers for the same sensing target object or Area, it may further generate enhanced sensing results based on the multiple received results.

- The SMS determines exposing the (enhanced) sensing results based on subscription information of the VAL Server/Client.

- The SMS sends the (enhanced) sensing results to the VAL Server/Client.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *