3GPP TSG-SA WG6 Meeting #69	S6-254195

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New Solution on support of energy saving for EDGE services

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution to support the energy saving for EDGE services.

2. Reason for Change

To proposes a new solution to support the energy saving for EDGE services.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * *

6.x	Solution #x: Support of energy saving for EDGE services

6.x.1	Solution description

This solution addresses the KI#1 and KI#3.

The following are the procedures which describe how the EAS registered the energy related information to the EES (clause 6.x.1.1) and how the EAS to be discovered considering the energy related information (clause 6.x.1.2).

6.x.1.1	Procedure of EAS resgistration on energy related information

The following procedure illustrates the high-level procedure of the EAS registration to the EES about the energy related information (e.g., the energy consumption information), which is updated based on the clause of 8.4.3.2.2 of 3GPP TS 23.558 [23558].

The updates/enhancements for the existing procedure are all marked in red.

8.4.3.2.2	EAS registration

Pre-conditions:

1.	The EAS has been configured with an EASID;

2.	The EAS has been configured with the address (e.g. URI) of the EES; and

3.	Both the EAS and EES have the necessary credentials to enable communications.

Figure 8.4.3.2.2-1: EAS Registration procedure

1.	The EAS determines that registration to the EES is needed (e.g. the EAS is instantiated and started up).

2.	The EAS sends an EAS registration request to the EES. The request shall include the EAS profile and may include proposed expiration time for the registration and its energy related information (e.g., energy consumption information.)

NOTE:	The EAS may get the energy related information from the 3rd parties or from OAM as described in 3GPP TS 28.552 [5] and 3GPP TS 28.554 [6].

3.	The EES performs an authorization check to verify whether the EAS has the authorization to register on the EES.

4.	Upon successful authorization, the EES stores the EAS Profile for later use (e.g. for serving EAS discovery requests received from EECs, etc.) and replies to the EAS with an EAS registration response. If the request includes bundle ID, the EES stores the received information. The EES may provide an expiration time to indicate to the EAS when the registration will automatically expire. To maintain the registration, the EAS shall send a registration update request prior to the expiration time. If a registration update request is not received prior to the expiration time, the EES shall treat the EAS as implicitly de-registered.

6.x.1.2	Procedure of EAS discovery based on energy related information

Figure 6.x.1.2-1 illustrates the high-level procedure of EAS discovery considering the energy related information.

NOTE:	The following procedure also applies for the EAS discovery subscription/subscription update procedures.

Pre-condition:

-	The EMS (Energy Management Server) is a new enabler in the application enabled layer which collects the energy related information (e.g., energy consumption information) per different granularity (e.g., node-level, UE-level, service-level) for different services. It assumed that the procedures for the EMS collecting the related information are described in other solutions for the KI#1.

Figure 6.x.1.2-1: Procedure of EAS discovery based on energy related information

- Similar with step 1 of clause 8.5.2.2 in 3GPP TS 23.558[23558], the EEC decides to send an EAS discovery request to the EES. The enhancement for this solution is the EAS discovery request not only includes the parameters as defined in the Table 8.5.3.2-1 of 3GPP TS 23.558[23558], but includes the EAS energy information in the EAS discovery filters and the energy saving indicator which indicate the EES to consider energy saving when selecting the EAS.

- Similar with step 1 of clause 8.5.2.2 in 3GPP TS 23.558[23558], the EES checks if the EEC is authorized to discover the requested EAS(s).

If the request is authorized, the EES will consider selecting an EAS or a group of EASs that meet the service requirements (e.g., energy saving indicator, EAS discovery filters).

-	If the requested EAS has provided its energy related information (e.g., energy consumption information) to the EES as described in clause 6.x.1.1, the EES will select a proper EAS or a group of EASs that meet the service request with lowest energy consumption information.

-	If the requested EAS is not previously registered on EES, or the registration information has expired, the EES may ask the OAM or the EMS to obtain the energy related information for the requested EAS as described in step 3a and step 3b respectively.

3a.	The EES may obtain the energy related information for the requested EAS from the OAM as described in clause 6.7.3.1 of 3GPP TS 28.554 [6].

3b. The EES may obtain the energy related information for the requested EAS from the EMS as described in other solutions for the KI#1.

Editor’s note：How the EMS performing the operations for the energy related information is FFS.

- Upon receiving the energy related information for the requested EAS from different resources (e.g., target EAS, OAM or EMS), the EES will select a proper EAS or a group of EASs that meet the service request (e.g., EAS discovery filters) with lowest energy consumption.

- Similar with step 3 of clause 8.5.2.2 in 3GPP TS 23.558[23558], the EES sends an EAS discovery response to the EEC, including the information about the discovered EASs.

6.x.2	Architecture Impacts

If the EMS is not utilized via EES, this solution has no impact on the existing architecture.

If the EMS is used via EES, the architecture, functionalities and relevant procedures for the EDGE service may need some enhancements to support the new application enabler (i.e. EMS) and new functionalities for the energy saving.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[23558]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

* * * End of Change * * * *