3GPP TSG-SA WG6 Meeting #69	S6-254196

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New Solution on new architecture for sensing services in application enablement layer

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution to set up a new architecture for sensing services in application enablement layer.

2. Reason for Change

The use cases and scenarios for the sensing services are very rich (such as UAV detection, HD map, V2X), it’s necessary to set up a generic new architecture and functional model to describe how to obtain and store the sensing results and then generate the value-added sensing results for different sensing related use cases.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * *

5.x	Solution #x: New architecture for sensing services in application enablement layer

This solution addresses the KI#1.

5.x.1	Architecture and functional model

This solution proposes a new architecture and functional model for sensing services in application enablement layer.

Figure 5.x.1-1 illustrates the generic on-network functional model for sensing services.

Figure 5.x.1-1: On-network functional model for Sensing services

The sensing management client provides the support for sensing management functions to the VAL client(s) over SM-C reference point. The VAL server(s) communicate with the sensing management server over the SM-S reference point. The VAL client communicates with the VAL server over the VAL-UU reference point which is outside the scope of this document.

The sensing management server, acting as an untrusted AF, may obtain sensing results from the 3GPP core network via NEF (over N33 reference point). If the sensing management server is a trusted AF, it may interact with sensing entities (e.g., SF) in 3GPP core network directly.

NOTE :	The sensing management server obtains the sensing results from the 3GPP core network depends on SA2’s exposure progress.

5.x.2	Procedure of exposure of sensing results

This solution proposes a new enabler (i.e.,Sensing Management Server) to support the exposure of sensing results to the consumers (e.g., VAL server). The SM server may interact with other SEAL enabler to get the value-added sensing results.

Figure 5.x.2.1-1 illustrates the high-level procedure of exposure of sensing results.

Figure 5.x.2.1-1: Procedure of exposure of sensing results

- The VAL server sends a sensing service request to the SM server, including the service type (e.g., UAV detection, dynamic HD map, Vehicle tracking, etc.), service ID, sensing target object information (e.g., UAV ID, UE ID, Vehicle ID), sensing service requirements for the target object (e.g., the target area, sensing accuracy, sensing resolution, max sensing service latency, etc.), the reporting conditions (e.g., periodic reporting or event-triggered reporting), the time duration for the sensing, etc.

2.	The SM server checks whether the VAL server is authorized to invoke such request, may be based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the SM server may invoke the sensing related service request to the 3GPP CN and obtain the sensing results from the 3GPP CN.

Editor’s note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

4.  The SM server may interact with SEAL enablers (e.g., SEALDD, ADAES) to get more sensing results (e.g., sensing service latency, sensing accuracy analysis) utilizing the obtained sensing results from the 3GPP CN, in case the 3GPP core network will not provide.

-	The SM server may interact with ADAES to get the e.g., sensing accuracy analysis and the predicted analysis for the target object utilizing the obtained sensing results from the 3GPP CN;

-	The SM server may interact with SEALDD to get the e.g., sensing service latency for the target object utilizing the obtained sensing results from the 3GPP CN.

5.	The SM server may analyse and aggregate the sensing results obtained in step 3 and step 4 to generate the value-added sensing results (e.g. the statistic results in a location/time/direction/height granularity), and then expose to the VAL server if the results can fulfil the requested sensing service requirements.

6.	The SM server sends the value-added sensing results to the VAL server via sensing service response message, including all of target objects and the value-added sensing information.

5.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *