3GPP TSG-SA WG6 Meeting #69	S6-254197

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New Solution on enhancements of UAV services utilizing sensing results

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution for support enhancement of UAV services utilizing sensing results.

2. Reason for Change

To proposes a new solution to support the enhancement of UAV services utilizing sensing results.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * *

5.x	Solution #x: Enhancements of UAV services utilizing the sensing results

5.x.1	Solution description

This solution addresses the KI#2.

The following high-level solution principles apply to this solution:

-	A high-level procedure for sensing request and exposure is proposed with a Sensing Management (SM) server in the application enablement layer. Whether it is an independent network element/function, or it can be combined with the existing network elements/functions of the application enablement layer (e.g., UAE) will be determined via the solutions and final conclusion for KI#1.

-	The solution assumes the 3GPP core network will expose the sensing results related to UAVs to the AF.

5.x.1.1	Procedure of detecting UAVs utilizing sensing results

Figure 5.x.1.1-1 illustrates the high-level procedure of detecting UAVs utilizing sensing results.

Pre-condition:

-	The following SM (Sensing Management) server is a server that implements the sensing functions in the application enablement layer.  It may be independent or co-located with UAE server. For the new architecture for the SM server, pls check the solution for the KI#1.

Figure 5.x.1.1-1: Procedure of detecting UAVs utilizing sensing results

- The VAL server sends a UAV detect request to the SM server to detect the UAVs in a certain area, including the service type (i.e., UAV detection), service ID, sensing service requirements for the detected UAVs (e.g., location accuracy, velocity, latency, sensing resolution, etc.), the target area, the reporting conditions (e.g., periodic reporting or event-triggered reporting), the reporting time intervals if the request is periodic, optional tracked object information (e.g., UAV ID, flight trajectory) if the object needs to be tracked, etc.

2.	The SM server checks whether the VAL server is authorized to invoke such request based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the SM server may invoke the sensing related service request to the 3GPP CN and obtain the sensing results from the 3GPP CN via NEF if the SM server is an untrusted AF or interact with the SF entity in 5GC directly as a trusted AF.

Editor’s note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

4.  The SM server may interact with SEAL enablers (e.g., LMS, ADAES) to get more sensing results (e.g., location information, sensing accuracy analysis) utilizing the obtained sensing results from the 3GPP CN in the following ways:

-	The SM server may interact with ADAES to get the sensing accuracy analysis (i.e. location accuracy analysis) for the detected UAVs utilizing the obtained sensing results from the 3GPP CN;

-	The SM server may interact with LMS to get the location trajectory (including the velocity, direction, altitude, etc.) for the detected UAVs utilizing the obtained sensing results from the 3GPP CN.

5.	The SM server aggregates and combines the sensing results obtained in Step 4 to determine all UAVs entering the target area as well as their flight trajectories, and then compares the flight trajectory with normal flight trajectory included in service request in Step 1 to identify if the UAVs entering the target area are invalid or not.

6.	The SM server sends the analysed sensing results to the VAL server via UAV detect response message, including all UAVs entering the target area and the invalid UAVs that not allowed entering the target area.

5.x.2	Architecture Impacts

This solution proposes a new enabler (Sensing Management Server) to support the sensing service if the SM server is standalone and not combined with UAE enabler.

For the new architecture for the SM server, pls check the solution for the KI#1.

5.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *