3GPP TSG-SA WG6 Meeting #69	S6-254230

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#5 Network Slice Energy Optimization based on Energy Saving VAL Server Policy

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #5 (TR 23.700-44 clause 5.5), the study aspects on NSCE enhancement for energy efficiency include:

- Whether and how to enhance functionalities of NSCE to support for energy efficiency and energy saving.

- Whether and how to use energy consumption information to fulfill network slice selection, modification, adaptation, etc.

This pCR proposes a solution for network slice energy optimization based on energy saving VAL server policy. This solution does not add new procedure. Instead, it proposes introducing a new VAL server policy in section 9.5, 'Network slice optimization based on VAL server policy', TS 23.435. The new policies enable NSCE server performs slice modifications based on monitoring energy-related information, such as energy consumption and the renewable energy ratio, at the slice level.

2. Reason for Change

This pCR proposes new services to address the study aspects in Key Issues #5, on network slice energy optimization based on energy saving VAL server policy.

3. Conclusions

This paper proposes solution on network slice energy optimization based on energy saving VAL server policy.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[zz]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

* * * Next Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #Y1 |  |  |  |  | X |  |

* * * Next Change * * * *

## 6.Y1	Network Slice Energy Optimization based on Energy Saving VAL Server Policy

### 6.Y1.1	Solution Description

This solution does not add new procedure. Instead, it proposes introducing a new VAL server policy in clause 9.5, 'Network slice optimization based on VAL server policy', 3GPP TS 23.435 [zz]. The new policies enable NSCE server performs slice modifications based on monitoring energy-related information, such as energy consumption and the renewable energy ratio, at the slice level.

The high-level principles are as follows:

- The VAL server provisions energy saving policies to the NSCE server.

- Based on the monitored energy consumption of a network slice - categorized by energy source (renewable or non-renewable) - over a specific duration (such as one hour) or as accumulated over a longer period (for example, a calendar month), slice modification is triggered when the measured value exceeds or falls below a defined threshold. This modification may involve reducing the slice's maximum throughput, slightly increasing allowable latency, or decreasing redundancy levels (such as removing a redundant UPF instance or redundant RAN cell).

- Based on the monitored renewable energy ratio of a network slice, a slice modification with predefined parameters is triggered when the energy ratio or crosses a specified threshold. Such modifications may include increasing the slice’s maximum throughput, raising allowable latency, expanding the maximum number of supported UEs, or increasing the maximum number of PDU sessions.

- The NSCE server monitors a slice's energy usage by gathering energy-related data from the slice's OAM system.

- Based on the defined triggers, the NSCE server carries out the actions outlined in energy-saving policies, aiming to either reduce the slice’s energy consumption or encourage the use of renewable energy sources.

The following clauses specify network slice energy optimization based on energy saving VAL server policy.

#### 6.Y1.1.1	Procedure

Figure 6.Y1.1.1-1 outlines the procedure, where Steps 1-4 represent existing operations. Step 5 introduces a new function in which the NSCE server monitors energy-related information for a network slice. Step 6 involves the NSCE server taking action to adjust the slice, either to conserve energy or to encourage the use of renewable energy sources.

Figure 6.Y1.1.1-1: VAL server policy provision and enforcment for energy optimization

- The consumer (VAL, SEAL Server) sends a VAL server policy provisioning request to the NSCE server by following clause 9.5 of 3GPP TS 23.435 [zz]. New energy saving related policies (see examples in clause 6.Y1.1.2) are added:

  - Based on the monitored energy consumption of a network slice - categorized by energy source (renewable or non-renewable) - over a specific duration (such as one hour) or as accumulated over a longer period (for example, a calendar month), slice modification is triggered when the measured value exceeds or falls below a defined threshold. This modification may involve reducing the slice's maximum throughput, slightly increasing allowable latency, or decreasing redundancy levels (such as removing a redundant UPF instance or redundant RAN cell).

  - Based on the monitored renewable energy ratio of a network slice, a slice modification with predefined parameters is triggered when the energy ratio or crosses a specified threshold. Such modifications may include increasing the slice’s maximum throughput, raising allowable latency, expanding the maximum number of supported UEs, or increasing the maximum number of PDU sessions.

The action of the above policy may include charging policy update when slice SLA is modified to save energy to attract the VAL server to use the policies.

- The NSCE server performs a VAL server policy check. This is the same as step 2 in clause 9.5.2.1, 3GPP TS 23.435 [zz].

- The NSCE server performs Slice Harmonization. This step is the same as clause 9.5.2.1.4 Policy harmonization, 3GPP TS 23.435 [zz].

- The NSCE server sends back a VAL server policy provisioning response to the Consumer (VAL/ASP), confirming policy provision. This is the same as step 2 in clause 9.5.2, 3GPP TS 23.435 [zz].

- The NSCE server initiates the retrieval of energy consumption data from the slice OAM to monitor energy-related information, including but not limited to the following parameters:

  - slice accumulated energy consumption refers to summing the average energy usage recorded during each slice OAM measurement interval (for example, every 15 minutes) by the OAM system and aggregating it into the total energy consumption over an extended period, such as a calendar month

  - slice average energy consumption: e.g. Energy-Consumption/hour

  - slice renewable energy ratio: e.g. 30%

- The NSCE server carries out slice modifications in accordance with the policy action defined in Step 1, through its interaction with the Slice OAM and the 5GC.

#### 6.Y1.1.2	Information Flows

##### 6.Y1.1.2.1	General

The following information flows are specified for VAL server policy.

##### 6.Y1.1.2.2	Policy of Reducing Energy Consumption

Table 6.Y1.1.2.2-1 shows the request sent by a consumer to an NSCE server for VAL server policy provisioning procedure.

Table 6.Y1.1.2.2-1: Policy of reducing energy consumption

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Reduce Energy Consumption |
| >Area of interest | M | The service area for which the policy profile applies, which can be expressed as a geographical area (e.g. geographical coordinates), or a topological area (e.g. a list of TA). |
| >Trigger event | M | Threshold information, i.e. slices accumulate energy consumption, slices average energy consumption reached the threshold |
| >Recommended action | M | Examples: - Modification of PDU sessions / max number of UEs (step for decrease in %). - Decrease slice capacity (step for decrease in %). - Decrease packet delay, and increase packet error rate etc. - Update the energy efficiency parameter. - Decrease GBR requirements. - Apply better charging offering for the slice after adjustment. |
| Lifetime or number of events | M | Time duration or number of times the policy can take action. |
| Priority | O | Indicates the priority of the policy. |
| Scheduling period | O | Indicates the scheduling of policy in terms of time. |
| >Start time | M | Indicates the scheduled start time. |
| >End time | M | Indicates the scheduled end time. |
| Preemption | O | Indicates the pre-empt capability of the policy. |

##### 6.Y1.1.2.3	Policy of Promoting renewable energy consumption

Table 6.Y1.1.2.3-1 shows the request sent by a consumer to an NSCE server for VAL server policy provisioning procedure.

Table 6.Y1.1.2.3-1: Policy of Promoting renewable energy consumption

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Promote renewable energy usage |
| >Area of interest | M | The service area for which the policy profile applies, which can be expressed as a geographical area (e.g. geographical coordinates), or a topological area (e.g. a list of TA). |
| >Trigger event | M | Threshold information, i.e. when renewable energy reached a threshold % |
| >Recommended action | M | Examples: - Modification of PDU sessions/max number of UEs (step for increase in %). - Increase slice capacity. |
| Lifetime or number of events | M | Time duration or number of times the policy can take action. |
| Priority | O | Indicates the priority of the policy. |
| Scheduling period | O | Indicates the scheduling of policy in terms of time. |
| >Start time | M | Indicates the scheduled start time. |
| >End time | M | Indicates the scheduled end time. |
| Preemption | O | Indicates the pre-empt capability of the policy. |

### 6.Y1.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.Y1.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***