3GPP TSG-SA WG6 Meeting #69	S6-254234

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#2 on Sensing Coverage Information Exposure for Supporting UAV Services

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #2 (TR 23.700-15 clause 4.2), the study aspects on Enhance UAV service utilizing sensing results include:

- How to enhance the UAS application enabler to support enhanced trajectory tracking for UAVs utilizing the sensing results.

- How to enhance the UAS application enabler to support enhanced DAA service/functionalities for UAVs utilizing the sensing results.

The Spatial Map Server can benefit from 3GPP and non-3GPP sensing (including 3rd Party sensing services) data or results to build high-quality Spatial Map. Thus, this pCR proposes to also consider non-3GPP sensing results and sensing application enablement service (SAE, which is introduced in S6-254233) for creating spatial map.

2. Reason for Change

To address the study aspects in Key Issue #2, this pCR proposes a new solution on sensing coverage information exposure for supporting UAV services.

3. Conclusions

This paper proposes solution on sensing coverage information exposure for supporting UAV services.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X2 |  | X |  |  |

* * * Next Change * * * *

## 6.X2	Solution #X2: Sensing Coverage Information Exposure for Supporting UAV Services

### 6.X2.1	Solution Description

The Spatial Map Server can benefit from 3GPP and non-3GPP sensing (including 3rd Party sensing services) data or results to build high-quality Spatial Map.

As sensing capabilities rely on the CSP’s infrastructure, which may provide full coverage. And the rollout of the CSP’s sensing service takes time, coverage remains incomplete during the deployment phase. Given that sensing capabilities are critical for the safety of UAVs and automotive systems, comprehensive sensing coverage information is essential for these sectors to develop robust services and ensure operational safety. Currently, there is no mechanism for CSP to expose the sensing coverage information.

The following clauses specify sensing coverage information exposure for supporting UAV services.

#### 6.X2.1.1	Procedure

Figure 6.X2.1.1-1: Sensing coverage information exposure

- The Consumer (VAL Server, SEAL Server) sends check sensing coverage information request to the SAE Server.

- The SAE Server authenticates the request, checks the stored sensing coverage information, and determines the next actions (e.g., to collect sensing coverage information from sensing entities).

- The SAE Server responds to the VAL server for the check sensing coverage information request.

- The SAE Server interacts with the SAE Client (deployed on a non-3GPP sensor) for non-3GPP sensing coverage information.

4a.	The SAE Server sends sensing coverage information request to the SAE Client to collect the sensing coverage information for non-3GPP sensing data.

4b.	The SAE Client responds to the SAE Server with its sensing coverage information.

- The SAE Server interacts with the 3rd party SAE server to collect the sensing coverage information of the 3rd party sensing data.

- The SAE Server aggregates the sensing coverage information from different sensing entities received in steps 4-5.

- The SAE Server sends check sensing coverage information response to the VAL server with the aggregated sensing coverage information.

#### 6.X2.1.2	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.X2.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X2.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***