3GPP TSG-SA WG6 Meeting #69	S6-254280

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0076 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling Editor’s Notes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Handling of the Editor’s Notes EN1 : Editor's Note: How SAn sever handles the “auto delete” of the spatial anchor on expiry is FFS. EN2: Editor's Note: How the SAn client handles expiration of Spatial anchor is FFS. EN3: Editor's Note: Spatial map management procedures to request object tracking need to be provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose to delete the EN1 and EN2 by providing clarification at 8.2.2.1.1 and 8.2.2.1.2. Also related informaton is present in the profile IE description on how the expiry will be handled. Propose to delete the EN3 since there is a similar EN in 8.3.5.2.1 Subscribe |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.1.1, 8.2.2.1.2, 7.3.3.1, 8.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 8.2.2.1.1	Spatial anchors (SAn) server

The SAn server acts as a SEAL server and it is used to provide services to create/update/delete/discover spatial anchors for the VAL application (i.e. metaverse application). If the spatial anchor profile contains the time period, the SAn server shall decide to delete the spatial anchor or change the spatial anchor state to inactive based on the expiry handling indication provided in the profile. It can communicate with other SAn server in the distributed SEAL deployments.

##### 8.2.2.1.2	Spatial anchors (SAn) client

The SAn client acts as a SEAL client for spatial anchors server. It supports client side functionalities for creating/updating/deleting/discovering/retrieving spatial anchors for VAL application (i.e. metaverse application). If the spatial anchor profile contains the time period, the SAn client may decide to delete the locally stored spatial anchor or change the locally stored spatial anchor state to inactive based on the expiry handling indication provided in the profile.The SAn client receives the request from VAL client to perform specific functionality.

# * * * * Next change * * * *

#### 7.3.3.1	Spatial map information

Table 7.3.3.1-1 describes information elements for the spatial map information stored in the SM server.

Table 7.3.3.1-1: Spatial map information

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | The identifier of the spatial map |
| Spatial map coverage | M | The three-dimensional space coordinates information of the spatial map |
| Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map |
| Spatial map media format | O | The media format of the spatial map. |
| List of spatial map layers | M | The list of spatial map layer information. At least one or more spatial map layers should be included. Each element includes the information described below. |
| > Spatial map layer ID | M | The identifier of the corresponding spatial map layer. |
| > Spatial map layer information | O | A container to carry the corresponding spatial map layer specific information |
| >> area | O | The area covered by this layer. This area can be the same or a subset of the Spatial map coverage space. |
| >> associated spatial map | O | The details about a spatial map associated with this layer. For example, if the layer is for a particular floor of the building, the detailed spatial map of that particular floor can be associated with this layer. |
| > List of objects | O | List of object information. Each element contains the information as specified in Table 7.3.3.1-2. |

Table 7.3.3.1-2 describes object information details for the spatial map information specified in the Table 7.3.3.1-1.

Table 7.3.3.1-2: Spatial map object information

| Information element | Status | Description |
| --- | --- | --- |
| Object ID | M | The identifier of the corresponding object |
| Object type | M | The type of object |
| Associated UE ID | O | Identifier of the UE associated with the object. |
| Object mobility information | M | Classifies whether the object is stationary or trackable. |
| > Object position | O | The 3D position of the object in the layer. For stationary object, this IE shall be present. For trackable object, 3D position of it shall be provided while it is being tracked, otherwise no value. |
| > Direction (NOTE) | O | Direction of the trackable object while it’s being tracked, otherwise no value. |
| > Velocity (NOTE) | O | Velocity of the trackable object while it’s being tracked, otherwise no value. |
| Object specific information | O | Object specific attribute information |
| NOTE: These IEs may be present when object is moving or changes it’s location frequently. |  |  |

NOTE 1:	Capability of SM server to distinguish whether object is stationary or trackable is outside the scope of this specification.

NOTE 2:	For the current release, the object specific information is implementation specific and out of scope.

# * * * * Next change * * * *

#### 8.3.1.2	Procedure

Figure 8.3.1.2-1 illustrates the spatial anchor creation procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.1.2-1: Spatial anchor creation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor create request to the SAn server. The request includes a requestor identifier, security credentials, and list of spatial anchors to be created as described in clause 8.3.1.3.1. The request includes a position, allowed entities list defining which entities are permitted to discover and access the spatial anchor, and discoverability information (e.g., indicating whether the spatial anchor is discoverable by any user). The requestor may want to associate spatial anchor with the stationary or trackable object in the spatial map. For such cases, the request includes spatial map position with corresponding spatial map ID and object ID. If the request includes application specific spatial anchor information, that information remains opaque to the SAn server and can be stored with the spatial anchor. The request can indicate to create multiple spatial anchors. The requsted message may include one spatial anchor group ID allocated by requestor for each spatial anchor to be created, if the requestor has got an available spatial anchor group ID. If the spatial anchor group ID is not available, then the SAn server may create the spatial anchor group for the spatial anchors and associate the spatial anchor with the group.

NOTE 1:	The application specific spatial anchor information is out of scope of standardization. For example, the application specific spatial anchor information can contain specific service information like a URL, a media data, a type, a manufacturing company, a date, a ratings, and customer premise information (e.g. a residence, office or shop) etc. Such information may be stored in key-value pair and is opaque to the SAn server.

NOTE 2:	The spatial anchor group ID is available with the requestor through spatial anchor group creation or through mechanism which could be implementation specific.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the spatial anchor(s). If the request is to create spatial anchor on an object, then the SAn server can create it with spatial map position information included in the request. SAn server assigns spatial anchor identifier to the newly created spatial anchor(s) and stores the spatial anchor information.

3.	The SAn server sends a spatial anchor create response to the requestor. If the SAn server created spatial anchors, the response includes an indication of success and spatial anchor identifier(s) of newly created spatial anchor(s). Otherwise, the response includes an indication of failure and can include a reason for failure.

# * * * * Next change * * * *

#### 8.3.4.2	Procedure

Figure 8.3.4.2-1 illustrates the procedure for spatial anchor discovery following the request-response model.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.4.2-1: Spatial anchor discovery

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor discovery request message to the SAn server to obtain information about spatial anchors of interest. The request includes a requestor identifier, security credentials, and can include spatial anchor discovery filters as described in clause 8.3.4.3.1. If the discovery request was triggered by a VAL client on a UE, the spatial anchor discovery request can additionally include a VAL client identifier and location of the client.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server identifies spatial anchor(s) that satisfies the spatial anchor discovery filters.

NOTE 1:	User preferences in the spatial anchor discovery filters can be compared with spatial anchor specific information.

For each matching spatial anchor, the SAn server checks whether the requestor is authorized to obtain the spatial anchor based on the identity and location of the requestor, and based on the allowed entities list. To determine the location of interest of the requestor or location of the requestor(client), the SAn server may use location information provided in the request. If location information is not provided in the request, the SAn server may obtain location information by invoking the 3GPP Core Network Location Services as described in 3GPP TS 23.273 [6] and 3GPP TS 23.502 [5] directly or via the NEF, or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4].

When 'Area of interest' discovery filter is specified in the request, for spatial anchors on stationary objects, the SAn server finds 3D position of the object from the corresponding spatial map information. For spatial anchors associated with trackable objects, the SAn server can obtain the current position information of the object at the time of discovery from SEAL SM server through SEAL-X (AnM) reference point. Additionally, SAn server can consider estimated duration of tracking, which can be obtained from SM server, to determine spatial anchors in the area of interest.

3.	The SAn server sends a spatial anchor discovery response to the requestor. If the SAn server has identified spatial anchor(s), the response includes an indication of success and the identified spatial anchors and may includes the recommended list of spatial anchors. The SAn server shall not incude the spatial anchors for which the state is inactive in the discovery response. Otherwise, the response includes an indication of failure and may include a reason for failure.

NOTE 2:	The logic to generate a recommended list of spatial anchors is out of scope and implementation specific.

NOTE 3:	The discovery response can include the List of discovered spatial anchors which is sorted starting with the closest spatial anchor to the location of the requestor(client).

4.	Upon receiving the spatial anchor discovery response, if the response indicates success, the requestor can store the spatial anchor information in a spatial anchor cache. The requestor evaluates the validity conditions associated with the discovered spatial anchors prior to using the spatial anchor. If the requestor is the SAn client, the SAn client can provide spatial anchor information to the VAL client(s). Upon receiving valid spatial anchor information, VAL client(s) can access the service(s) associated with each spatial anchor.

# * * * * End change * * * *