3GPP TSG-SA WG6 Meeting #69	S6-254295

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0171 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements of SEALDD enabled congestion control for VAL applications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI 20, SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When serving PSA UPF or NG-RAN is changed e.g. due to inter-NG-RAN handover or PSA UPF relocation, target NG-RAN and target PSA UPF, if supported, should continue to perform ECN marking for L4S for the QoS Flow. However, if not available (i.e. ECN marking for L4S is not supported in both, target NG-RAN and target PSA UPF), AF should be notified that ECN marking for L4S can no longer be performed if ECN marking for L4S had been enabled for the QoS Flow based on AF request as described in the TS 23.501. Currently, clause 9.8.2.2 does not support the change of the congestion control method according to the change in whether the L4S is supported or not in 5GC according to the mobility of the UE. Therefore, it may be necessary to change the congestion control method in the SEALDD server and the SEALDD client according to a change in whether the L4S is supported or not in 5GC while using the L4S. |  |  |  |  |  |  |  |  |
| Summary of change: |  | According to the change in whether L4S is supported or not in 5GC while using L4S, it is necessary to change the congestion control method in the SEALDD server and SEALDD client as well. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Updating of congestion control method considering whether L4S is supported or not in 5GC is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.8.2.2	SEALDD enabled congestion control for VAL applications

Based on the congestion information exposed by 5GS (i.e., by using ECN marking for L4S), the SEALDD server performs the differentiated congestion control for multiple VAL applications, after receiving the L4S feedback from the SEALDD client. Figure 9.8.2.2-1 illustrates the procedure for SEALDD enabled congestion control for VAL applications.

Pre-conditions:

1.	The VAL server has discovered and selected the SEALDD server by CAPIF functions as specified in clause 9.4.2.

Figure 9.8.2.2-1: SEALDD enabled congestion control procedure

1-6. Same as clause 9.2.2.2 step 1 to step 7 with difference that in step 5 the SEALDD client includes also L4S feedback capability in Sdd_RegularTransmissionConnection_Establish request.

7.	If the L4S feedback capability is received in step 5, the SEALDD server requests 5GC to perform the ECN marking for L4S for the required VAL applications by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of 3GPP TS 23.502 [6].

8.	Same as step 9 of clause 9.2.2.2.

9.	For downlink traffic, if ECN marking is identified, the L4S feedback is performed in transport layer.

10.	After receiving the L4S feedback in step 9, the SEALDD server may perform the congestion mitigation for downlink traffics from multiple VAL applications. For the same UE, the SEALDD server determines differentiated congestion control/rate control with the calculated congestion level or the calculated transmission rate for multiple VAL applications based on internal policy considering the received QoS requirements of different VAL applications identified by VAL service ID in step 1.

11.	[Optional] The SEALDD server sends Sdd_connection status notification to VAL server with the calculated congestion level for the VAL service.

12. If the ECN marking for L4S information (e.g. ECN marking for L4S can no longer be performed or ECN marking for L4S can be performed again) is received from 5GC, the SEALDD server determines a change in the congestion control scheme. When receiving a notification of ECN marking for L4S can no longer be performed from 5GC while using L4S-based congestion control, the SEALDD server determines to use the TCP-based congestion control.

13. If the SEALDD server determines to change the congestion control scheme in step 12, the SEALDD server notify the SEALDD client that the congestion control method has been changed. When the congestion control is performed in the VAL server as in step 11, the SEALDD server also notify the VAL server that the congestion control method has been changed.

14. The SEALDD client perform the corresponding feedback when the ECN is identified based on the changed congestion control method notified from the SEALDD server in step 13.

NOTE 1: 	According to the congestion level provided by SEALDD server, the VAL server supporting media can perform congestion control, e.g. by adjusting the encoding scheme, and rendering scheme, etc.

NOTE 2: 	Step 11 is performed if the SEALDD server cannot perform transmission rate adaptation to mitigate the congestion by itself.

* * * End of Change * * * *