3GPP TSG-SA WG6 Meeting #69	S6-254298

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	Samsung

Title:	New Solution for support of QoS differentiation for non-3GPP devices

Spec:	3GPP TR 23.700-51 V1.0.0

Agenda item:	9.5

Document for:	Approval

Contact:	jerry.bae@samsung.com

1. Introduction

This paper introduces a solution for the KI#4 to support of QoS differentiation for non-3GPP devices

2. Reason for Change

This solution addresses on application enabler layer enhancement to support of QoS differentiation for non-3GPP devices for XR applications.

In Rel 19, the SA2 defined UIA_ARC to support QoS differentiation of traffic transmitted to non-3GPP devices.

In the existing PIN-based service, it was difficult to distinguish traffic from the server to the non-3GPP device connected to the tethering device within 5GC. In the UIA_ARC, the UE may send the Non-3GPP Device Identifier(s) and the user plane address(es) of the non-3GPP device(s) to the network to support QoS differentiation of application traffic transmitted from the application server to the non-3GPP device(s).

This paper proposes a new solution that uses UIA_ARC to support QoS differentiation handling of application traffic related to non-3gpp devices.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 V1.0.0.

* * * First Change * * * *

5	Solutions

5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 |
| --- | --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |  |
| Sol #2 |  | x |  |  |  |
| Sol #3 |  |  | x |  |  |
| Sol #4 |  |  |  | x |  |
| Sol #5 | x |  |  |  |  |
| Sol #6 | x |  |  |  |  |
| Sol #x |  |  |  | x |  |

* * * Next Change * * * *

5.x	Solution #x: Support of QoS differentiation for non-3GPP devices

5.x.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [x].

5.x.2	Solution description

This solution addresses on application enabler layer enhancement to support of QoS differentiation for non-3GPP devices for XR applications.

In Rel 19, the SA2 defined UIA_ARC to support QoS differentiation of traffic transmitted to non-3GPP devices.

In the existing PIN-based service, it was difficult to distinguish traffic from the server to the non-3GPP device connected to the tethering device within 5GC. In the UIA_ARC, the UE may send the Non-3GPP Device Identifier(s) and the user plane address(es) of the non-3GPP device(s) to the network to support QoS differentiation of application traffic transmitted from the application server to the non-3GPP device(s). In order to support the QoS differentiation of traffic transmitted to the non-3GPP device when providing the VAL service using the non-3GPP device, the VAL server may include the QoS requirements for non-3GPP device(s) in the SEALDD regular transmission request and send it to the SEALDD server. The VAL client may send a SEALDD service request including Non-3GPP device information (e.g. Non-3GPP Device Identifier and/or Non-3GPP Device information etc.) to be used when using the SEALDD service to the SEALDD client. The SEALDD client in the UE may perform a SEALDD-UUc connection establishment procedure with the SEALDD client in the Tethered device based on the Non-3GPP device information received from the VAL client. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD-UU flow ID(s), the SEALDD traffic descriptor of the SEALDD client side (SEALDD-S connection information of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID, Non-3GPP Device Identifier(s) and Non-3GPP Device information (e.g. video or audio etc.) associated with Non-3GPP Device Identifier(s). For example, the Non-3GPP Device information may include non-3GPP device type (e.g. glass, wireless earphones, etc.), data information in service (e.g. video or audio). After receiving the Non-3GPP device identifier(s) and Non-3GPP Device information (e.g. video or audio etc.) associated with Non-3GPP Device Identifier(s) from the SEALDD client, the SEALDD server may determine support of QoS differentiation for Non-3GPP device(s) in the VAL service requested by the SEALDD client. The SEALDD server may request support of QoS differentiation for non-3GPP deivces by provisioning QoS information for the non-3GPP device(s) to 5GC that require differentiated QoS treatment.

5.x.2.1	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [x] can be enhanced (highlighted in bold italics) as follows.

9.2.2.2 SEALDD enabled regular data transmission connection establishment procedure Figure 9.2.2.2-1 illustrate the procedure for establishing regular SEALDD data transmission connection. Pre-condition: - The VAL server has discovered and selected the SEALDD server by CAPIF functions. Figure 9.2.2.2-1: SEALDD enabled regular data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for application data traffic transfer and allocates address as SEALDD-S connection information for receiving the traffic packets from SEALDD server. The VAL server sends Sdd_RegularTransmission request to the SEALDD server discovered by CAPIF. The request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S connection information of the VAL server side, and optionally, the QoS information for the application traffic, e.g. QoS requirements. The request also includes the QoS requirements for non-3GPP device(s) in the SEALDD regular transmission request. 2. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates SEALDD-S connection information of the SEALDD server to receive the application data traffic from the VAL server for application data transfer as SEALDD-S connection information of the SEALDD server side. The SEALDD server allocates a specific SEALDD-S connection information used for SEALDD traffic transfer with the specific UE for the VAL server and responds with a SEALDD service response (including SEALDD-S connection information of the SEALDD server side). The VAL server and SEALDD server use SEALDD-S connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the SEALDD-S connection information allocated by SEALDD server, and the QoS information for application traffic. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. NOTE 1: The SEALDD-S connection information of the SEALDD server side is optional to respond to the VAL server, if the SEALDD server uses the downlink pull mode to obtain the data/content from the address provided by the VAL server in step 1, and uses the uplink push mode to send the data/content to the address provided by VAL server. 3. Data transmission session information is provisioned to the VAL client by the VAL server via application signalling. The data transmission session information may contain the SEALDD address information which is discovered and selected by VAL server. NOTE 2: The SEALDD address information for VAL server and SEALDD client is the same. NOTE 3: The application signalling may be transmitted via direct application layer connection or via the SEALDD layer. 4. The VAL client sends a SEALDD service request to SEALDD client. The VAL client receives a SEALDD service response to the SEALDD client. If the data transmission session information contains the SEALDD address information, then the VAL client sends the selected SEALDD address information to SEALDD client in service request message. The response indicates that whether the SEALDD service request is successful or not. The VAL client may send a SEALDD service request including non-3GPP device information (e.g. Non-3GPP Device Identifier and/or Non-3GPP Device information etc.) to support QoS differentiation of application traffic in SEALDD service using Non-3GPP devices. The SEALDD client in the UE may perform a SEALDD-UUc connection establishment procedure with the SEALDD client in the tethered device based on the non-3GPP device information received from the VAL client. 5. If the data transmission session information does not contain the SEALDD address information, then the VAL/SEALDD client discover and select the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client obtains the SEALDD server's address. 6. The SEALDD client allocates SEALDD-UU flow ID(s) mapping to the identifiers of the application traffic. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD-UU flow ID(s), the SEALDD traffic descriptor of the SEALDD client side (SEALDD-S connection information of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID, Non-3GPP Device Identifier(s) and Non-3GPP Device information (e.g. video or audio etc.) associated with Non-3GPP Device Identifier(s). The request message also contains the selected VAL server endpoint information and UE ID. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the data connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the data connection establishment, otherwise SEALDD server returns a failed result e.g. performs connection reject. After receiving the Non-3GPP device identifier(s) from the SEALDD client, the SEALDD server may determine support of QoS differentiation for non-3GPP device(s) in the VAL service requested by the SEALDD client. The SEALDD server may request support of QoS differentiation for non-3gpp deivces by provisioning QoS information for the non-3GPP device(s) to 5GC that require differentiated QoS treatment. NOTE 4: The SEALDD server can use or update the association between SEALDD-UU connection and SEALDD-S connection that associated with UE ID, VAL service ID, VAL server endpoint, which is used to correlate the SEALDD traffic and the VAL application traffic. NOTE 5: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic of the same SEALDD client. The SEALDD-UU flow ID may be same with the identifiers of the application traffic or new simplified IDs allocated by SEALDD. 7. The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. address allocated in step 2, transport layer protocol) mapping to the application data traffic. 8. If the connection between VAL server and SEALDD server is not established in step 2, the SEALDD server establishes connection with VAL server for the VAL client to transmit application data traffic mapping to the SEALDD traffic according to the SEALDD-S information negotiated in step 1-2. 9. The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD connection establishment. After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the UE's address and SEALDD server's address for the SEALDD traffic transmission). After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between application data traffic and SEALDD-UU flow ID. The SEALDD server gets the mapping information between the SEALDD-UU flow ID and the SEALDD-S connection. Upon receiving application data traffic from VAL client, the SEALDD maps it to SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 6 and step 7. The SEALDD traffic is sent to the SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID and SEALDD-UU flow ID. The SEALDD server sends the recovered application traffic to the address provided by VAL server in step 1, via the connection established in step 2 or 8 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly. The SEALDD server receives any UE location change notification using SEAL LM services defined in 3GPP TS 23.434 [4] clause 9.3.12, then the SEALDD server performs the data delivery in alignment with the geofence policy. If the UE is in the forbidden location or not allowed for the given VAL service to send/receive data as per the Geofence policy, then the SEALDD server performs action like releases the connection and informs the VAL server that UE is not reachable because in a forbidden location using connection event status procedure. If UE enters the allowed location area then the SEALDD server initiates the connection establishment using the procedure defined in clause 9.2.2.3.

5.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *