3GPP TSG-SA WG6 Meeting #70	S6-255076

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0397 | rev | - | Current version: | 19.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving Notes on server-to-server messages (MCData) information flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Deleting the notes in clauses 7.17.2.3 Ad hoc group data session request (MCData server – MCData server) and 7.17.2.6 Ad hoc group data session response (MCData server – MCData server): NOTE: Whether the server-to-server message is needed in a call request is out of scope of the present document. In fact the server-to-server message in a data session requests is already in use in the procedures 7.17.3.2.1, 7.17.3.2.3, 7.17.3.2.4, 7.17.3.2.6. Therefore these Notes can be removed. For the sake of clarity, the changes in the corresponding procedures (7.17.3.2) are discussed in a separate CR. - Adding information flows Ad hoc group data communicatoin release request/response and Ad hoc group data communicatoin leave request/response between MCData servers, to be consitent through the document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleting Notes, adding 4 new information flows between MCData servers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguities in specification. Notes remaining in specification and missing server to server IF may mislead of server to server communication process. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.2.3, 7.17.2.6, 7.17.2.8x (new), 7.17.2.9x (new), 7.17.2.13x (new), 7.17.2.14x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes in the corresponding procedures (7.17.3.2) are discussed in CR0398 (S6-255077). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 7.17.2.3	Ad hoc group data session request (MCData server – MCData server)

Table 7.17.2.3-1 describes the information flow ad hoc group data session request between the MCData servers.

Table 7.17.2.3-1 Ad hoc group data session request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ad hoc group ID | M | The MCData group ID to be associated with the ad hoc group data communication |
| MCData ID list (see NOTE 1, NOTE 3) | O | MCData IDs of the participants being invited for the ad hoc group data communication |
| SDP offer | M | Media parameters of MCData server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for a broadcast ad hoc group data communication |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for ad hoc group imminent peril data communication |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for ad hoc group emergency data communication |
| Preconfigured MCData group ID | O | Group identity whose configuration is to be applied for this ad hoc group data communication. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Requested priority | O | Application priority level requested for this group data communication |
| NOTE 1: This element is included only when the data communication initiating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

* * * Second Change * * *

#### 7.17.2.6	Ad hoc group data session response (MCData server – MCData server)

Table 7.17.2.6-1 describes the information flow ad hoc group data session response between the MCData servers.

Table 7.17.2.6-1: Ad hoc group data session response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user receiving data |
| Functional alias | O | The functional alias of the MCData user receiving data |
| MCData ad hoc group ID | M | The MCData group ID associated with the ad hoc group data communication |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group data communication request (success or failure) |

* * * Thrid Change * * *

#### 7.17.2.8x	Ad hoc group data session release request (MCData server – MCData server)

Table 7.17.2.8x-1 describes the information flow ad hoc group data session release request between MCData servers.

Table 7.17.2.8x-1 Ad hoc group data session release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the user authorized to release the ad hoc group data communication |
| Functional alias | O | The functional alias of the authorized user requesting to release the ad hoc group call |
| MCData ad hoc group ID | M | The MCData group ID of the ad hoc group data communication on which data communication is released |

* * * Forth Change * * *

#### 7.17.2.9x	Ad hoc group data session release response (MCData server – MCData server)

Table 7.17.2.9x-1 describes the information flow ad hoc group data session release response between MCData servers.

Table 7.17.2.9x-1 Ad hoc group data session release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the user authorized to release the ad hoc group data communication |
| Functional alias | O | The functional alias of the authorized user requesting to release the ad hoc group data communication |
| MCData ad hoc group ID | M | The MCData group ID of the ad hoc group data communication on which data communication is released |

* * * Fifth Change * * *

#### 7.17.2.13x	Ad hoc group data session leave request (MCData server – MCData server)

Table 7.17.2.13x-1 describes the information flow ad hoc group data session leave request between MCData servers.

Table 7.17.2.13x-1 Ad hoc group data session leave request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the ad hoc group data communication participant leaving the ad hoc group data communication` |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group data communication on which data session is ongoing |
| Reason to leave (see NOTE) | O / M | Carries the reason of why the MCData client is leaving ongoing ad hoc group data communication (e.g. Due to migration). |
| NOTE: This element is only mandatory in the relayed request from client to server (see 7.17.2.13a) |  |  |

* * * Sixth Change * * *

#### 7.17.2.14x	Ad hoc group data session leave response (MCData server – MCData server)

Table 7.17.2.14x-1 describes the information flow Ad hoc group data session leave response between MCData servers.

Table 7.17.2.14x-1 Ad hoc group data session leave response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the ad hoc group data communication participant leaving the ad hoc group data communication |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group data communication on which data session is ongoing |
| Result (See NOTE) | M | Result of the ad hoc group communication leave request – success or failure. |
| NOTE: This element is only included in the relayed response from server to a client (see 7.17.2.14a) |  |  |

* * * End of Changes * * *