3GPP TSG-SA WG6 Meeting #70	S6-255146

Dallas, USA, 17th – 21st Novemeber 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0178 | rev | - | Current version: | 19.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause 10.2.2.2 EN resolution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editor's note: How SEALDD server knows the recipient UE information is FFS. Above EN is removed by updating the procedure to make it complete. #### 10.2.2.1 SEALDD enabled data transmission for delay-tolerant satellite services procedure for downlink data transmission from VAL server to VAL client with SEALDD server considering the feeder link availability and unavailability. #### Similarly, 10.2.2.2 Client initiated data transmission for delay-tolerant satellite services procedure was defined to handle uplink data transmission from VAL client to VAL server with SEALDD server considering the feeder link availability and unavailabily. Accordingly, the procedure 10.2.2.2 is updated and the existing EN is removed |  |  |  |  |  |  |  |  |
| Summary of change: |  | EN is removed and procedure updated to make it complete and alinged. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Un resolved EN leads to incomplete requirement leading to incomplete feature implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.2.2.2	Client initiated data transmission for delay-tolerant satellite services

Figure 10.2.2.2-1 illustrates the procedure for establishing S&F data transmission connection, and the SEALDD facilitates the delay-tolerant satellite services to transmit data initiated by the VAL client to VAL server.

Pre-condition:

1.	The SEALDD client is authorized to request regular data transmission services on behalf of the VAL client when the VAL client initiates transmission service.

2.	The VAL client has discovered the VAL server.

Figure 10.2.2.2-1: Client initiated data transmission for delay-tolerant satellite services

1.	Same as steps 1 – 5 of clause 9.2.2.7.

2.	The SEALDD server subscribes and is notified about the store and forward satellite operation information from the 3GPP CN as specified in clause 5.6.3.10 of 3GPP TS 23.682 [24] e.g., whether the UE is registered in S&F Mode, the estimated S&F DL delivery time indicating the estimated/expected time required to deliver the data from the UE, feeder link availability period. The SEALDD server stores the received S&F satellite operation information for further control the uplink application data transmission towards VAL server.

3.	The SEALDD client sends the UL data to the SEALDD server via the 3GPP network

4.	The SEALDD server controls the uplink application data delivery to the VAL Server based on the S&F satellite operation information, when the UE is registered in S&F mode:

-	pending the UL data transmission towards VAL server when the Feeder Link is not available, until the complete UL data is received;

-	If estimated S&F UL delivery time is received from 3GPP CN, it may be used by the SEALDD server to estimate whether the transmitted UL application data is completely delivered to the VAL Server and its corresponding storage is available for new UL data. For each Feeder Link available period, the SEALDD server starts an estimated delivery timer based on the estimated S&F UL delivery time which is corresponding to the current feeder link. The estimated delivery timer may be greater than or equal to the estimated S&F DL delivery time. In next Feeder Link available period, the SEALDD server checks the previous estimated delivery timer is expired or not before transmitting the DL data. Also, the maximum S&F data storage quota is also considered

a.	If the timer expires, the SEAL server sends UL data. If the maximum S&F data storage quota is configured, the SEALDD server may further calculate the volume (e.g., increasing the volume by certain data size (e.g., size of data previously delivered to the SEALDD client)) of new UL data to be sent.

b.	If the timer does not expire, the SEALDD server may not send the UL data and wait until the timer expires. If the maximum S&F data storage quota is configured, the SEALDD server may further also checks the available data volume within the maximum S&F data storage quota. The SEALDD server may send UL data limited within the available data volume. If no available data volume, the SEALDD server stores the DL data transmission until the timer expires.

NOTE 1:	The SEALDD server can use the estimated S&F DL delivery time as the estimated/expected time required to deliver the data to VAL Server from the time data has been received at the SEALDD server.

NOTE 2:	The maximum S&F data storage quota can be per UE, per application or per SEALDD server or any combination of them.  .

5.	The SEALDD server sends the UL data to the VAL server.

* * * End of Change * * * *