3GPP TSG-SA WG6 Meeting #70	S6-255259

# Dallas, USA, 17th – 21st November 2025

Source:	Huawei, HiSilicon

Title:	Overall evaluation of KI#1

Spec:	3GPP TR 23.700-52 v0.4.0

Agenda item:	9.7

Document for:	Approval

Contact:	changlinhui@huawei.com

- Introduction

This pCR is proposed to provide the overall evaluation of KI#1.

2. Reason for Change

To provide the overall evaluation of KI#1.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-52 v0.4.0.

# * * * First Change * * * *

# 7	Overall evaluation

7.1	General

Various solutions proposed in the present document aim to provide potential enhancements to the SEALDD, including the data delivery using network multicast/broadcast capability from both 4G and 5G, the SEAL-DD user plane (i.e., SEALDD-UU, SEALDD-S) aspects (e.g., protocol stack(s), user plane functionality description).

7.2	Architecture enhancements

There is no architecture impact.

7.3	Solution evaluations

7.3.2	Key issue #1 solution evaluation

Four solutions - Solution #2, Solution #3, Solution #4 and solution#x, have been proposed for Key Issue #1, aimed at designing a multicast/broadcast data delivery service which is able to hide the complexities for the VAL server to use the 3GPP network broadcast/multicast capability to delivery its content to its VAL users:

.

In all solutions, the SEALDD server is acting as the ingress point for the DL data from VAL server to hide the complexity of 3GPP network. Also the NRM-UU messages are used for the session/service announcement, listening status report, UE session join notification.

Specifically, each solution has its own design:

Solution#2 resolves the multicast aspect of key issue#1, and it introduces a new and individual multicast delivery service. It triggers the network resource management by interacting with the 3GPP network directly or indirectly via the NRM server to manage the 5G multicast MBS session.

Solution#3 resolves the multicast aspect of key issue#1, and it introduces a new and individual multicast delivery service using both eMBMS and 5G MBS. it triggers the network resource management by interacting with the 3GPP network directly or indirectly via the NRM server to manage the eMBMS bearer and 5MBS broadcast session.

Solution#4 introduces a single multicast/broadcast delivery service only consider 5G MBS capabilities. It triggers the network resource management via the NRM server only.

-	Solution X converges solution #2, #3, #4 and introduces a single multicast/broadcast delivery service to cover both multicast and broadcast from 4G eMBMS and 5G MBS. It triggers the network resource management by interacting with the 3GPP network directly or indirectly via the NRM server to manage the 5G multicast MBS session.

# * * * End of Change * * * *