3GPP TSG-SA WG6 Meeting #70	S6-255276

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Samsung

Title:	Pseudo-CR on Open issues to Key Issue #2

Spec:	3GPP TR 23.700-43 v0.2.0

Agenda item:	9.11

Document for:	Approval

Contact:	Narendranath Durga Tangudu (n.tangudu@samsung.com)

1. Introduction

This pCR proposes new open issues to Key Issue #2, to handle the control of usage of API provider domain service API information considering multi tenant CAPIF scenarios.

2. Reason for Change

CAPIF supports service APIs from multiple API provider domains, registering API provider functions from various providers at the CCF. Consequently, the service API information related to multiple API provider domains is available at the CCF. This pCR proposes various issues arising due to usage of service API information from multiple API provider domains, for offering services such as analytics.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 v0.2.0.

* * * First Change * * * *

## 4.2	Key issue #2: Enhancements to API administration in CAPIF domain and API provider domain

### 4.2.1	Description

3GPP SA1 has specified the service requirements for exposure in 3GPP TS 22.261 [5] including service exposure, information exposure, etc. And 3GPP SA6 has already specified many APIs in various SEAL specifications, EDGEAPP specification etc. And 3GPP SA6 has defined CAPIF as a framework to support the management of such APIs exposure.

API provider administrator: An authorized user with special permissions for the API provider to perform administration of the service APIs.

Below updated definition for API management function in 3GPP TS 23.222 [2] shows the relationship of the API provider administrator and the API management function:

API management function: The entity which enables the API provider administrator at the API provider domain to perform administration (e.g. minimizing downtime, ensuring high availability and performance, identifying potential threats) of the service APIs.

API provider administrator (leveraging API management function) and CAPIF administrator plays a pivotal role in managing and overseeing the CAPIF operations, ensuring smooth deployment, maintenance, and updates, and monitoring performance to meet service level agreements (SLAs). As an API provider administrator and CAPIF administrator, tracking and analysing API usage is crucial for ensuring optimal performance, security, and user satisfaction. However, currently in the CAPIF 3GPP TS 23.222 [2] there is limited support for administrator role via API management function. That is, API management function can Monitor service API invocation and Query service API log for auditing purpose. Hence, going by emerging trends the current support to administrator needs enhancement e.g. via analytics data.

Some key types of API analytics that would be useful for an API provider administrator and CAPIF administrator are Usage Metrics, Error Tracking, User Behaviour Analysis, Security Monitoring, Performance Optimization, Business Insights, Compliance and Auditing.

By leveraging these analytics, API provider administrators and CAPIF administrator can ensure that their APIs are reliable, secure, and aligned with business objectives.

NOTE:	When the CAPIF provider and the service API provider are the same, then the API provider administrators and CAPIF administrator can be the same administrator.

### 4.2.2	Multiple API provider domains service API information

As per 3GPP TS 23.222, using Logging server API invocations procedure, the AEF logs the service API invocations information at the CCF. CAPIF also supports service APIs from multiple API provider domains. So, the service API information such as service API invocation logs related to multiple API provider domains is available at the CCF.

The service API information (e.g., service API invocation logs) available at CCF is related to an API provider domain and belongs to or proprietary to the API provider domain. This information is not for consumption by/sharing with application entities (e.g., API provider domain functions of other API providers, Application Functions, etc) outside of API provider domain, without proper authorization. In scenarios where the API analytics is generated using the service API information related to multiple API provider domains, it is up to the API provider to control the usage/consumption of API provider domain’s service API information for analytics services to application entities outside of API provider domain.

Currently the CCF lacks awareness of which API provider domain’s related service API information (e.g., service API invocation logs) to be used for services such as generating analytics. CCF doesn’t not perform any checks on the API provider domain information before using or sharing the service API information, with the application entities outside of API provider domain. This oversight can lead to the exposure of sensitive API provider domain-specific information to unauthorized entities, compromising the multi-tenant requirements of CAPIF deployments.

Moreover, consumers of the analytics or services, such as API provider domain functions, Application Functions, may have preferences regarding the usage of service API information from specific API provider domains. These preferences could be based on trust or other factors, and failure to respect these preferences can undermine the reliability and acceptance of the analytics or services offered.

* * * Next Change * * * *

### 4.2.3	Open issues

To support the enhancements to API administrator role, the following aspects need to be studied:

-	What are the key types of API analytics that would be useful for an API provider administrator?

-	How to provide API analytics for the CAPIF entities in the API provider domain?

-	whether and what are the key types of API analytics that would be useful for an CAPIF administrator?

-	whether and how to provide API analytics to the CAPIF administrator?

-	whether and how the API provider permits the usage of service API information.

-	whether and how the analytics consumer’s preference to the usage of service API information.

* * * End of Changes * * * *