3GPP TSG-SA WG6 Meeting #70	S6-255290

Dallas, USA 17th – 21st November 2025	(revision of S6-255xyz)

Source:	Ericsson

Title:	Pseudo-CR on solution for Open Discovery policies Service API and AEF operational status

Spec:	3GPP TR 23.700-43

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides a solution for KI#A for TR 23.700-43.

2. Reason for Change

KI#A describes one existing gap in current deployments, about how the CAPIF Core Function can become aware of the Service API information that can be disseminated when a requester (not identified in the CAPIF Core Function) invokes the Open Discovery service API.

This solution proposes enhancements to the CAPIF framework to ensure correct handling of Open Discovery API invocations by:

-	Getting awareness about API Provider preferences for the dissemination of service API information by the update of the Publish service API, to include a new IE, Open Discovery shareable information, that indicates the sharable service API information per requester type.

-	Getting awareness about the CAPIF administrator policies for the dissemination of service API information.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.2.0)

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #A |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  | X |  | X |  |  |  |
| Sol #6 |  | X |  |  |  |  |  |
| Sol #x |  |  |  |  |  |  | A |

* * * Next Change * * * *

## 6.x	Solution #x: API Provider domain and CAPIF Administrator policies for Open Discovery

### 6.x.1	General

This solution relates to KI #A.

KI#A describes one existing gap in current deployments, about how the CAPIF Core Function can become aware of the Service API information that can be disseminated when a requester (not identified in the CAPIF Core Function) invokes the Open Discovery service API.

This solution proposes enhancements to the CAPIF framework to ensure correct handling of Open Discovery API invocations by:

-	Getting awareness about API Provider preferences for the dissemination of service API information by the update of the Publish service API, to include a new IE, Open Discovery shareable information, that indicates the sharable service API information per requester type.

-	Getting awareness about the CAPIF administrator policies for the dissemination of service API information.

### 6.x.2	Architecture Impacts

This solution does not have arhictectural impacts.

### 6.x.3	Solution description

#### 6.x.3.0	General

##### 6.x.3.0.1	Introduction

This solution proposes two CAPIF enhancements:

1.	Enhancements on Service API Publish to enable the APF can indicate the API Provider domain preferences about the service API information that can be disseminated using the Open Discovery API (see clause 6.x.3.0.2 and 6.x.3.1)

2.	Enhancements on the Configuration data stored in the CAPIF core function and provided by the CAPIF Administrator (see clause 6.x.3.0.3).

The related enhancements on the Open Discovery procedure are described in clause 6.x.3.0.4.

##### 6.x.3.0.2	Service API Publish with API Provider domain preferences for Open Discovery

### 8.3.3 Procedure Figure 8.3.3-1 illustrates the procedure for publishing the service APIs. The service API publish mechanism is supported by the CAPIF core function. Pre-conditions: 1. Authorization details of the APF are available with the CAPIF core function. 2. API invokers may have subscribed with the CAPIF core function to obtain new service API information. Figure 8.3.3-1: Publish service APIs 1. The API publishing function sends a service API publish request to the CAPIF core function, with the details of the service API. If the service API is to be shared to other CAPIF core functions, the shareable information and the CAPIF provider domain information are included. The API publishing function may also include the shareable information when service API information is requested using the Open Discovery request. 2. Upon receiving the service API publish request, the CAPIF core function checks whether the API publishing function is authorized to publish service APIs. If the check is successful, the service API information provided by the API publishing function is stored at the CAPIF core function (API registry). 3. The CAPIF core function provides a service API publish response to the API publishing function indicating success or failure result and triggers notifications to subscribed API invokers as described in subclause 8.8.4.

##### 6.x.3.0.3	CCF configuration with CAPIF Administrator preferences for Open Discovery

Annex E (normative): Configuration data for CAPIF The configuration data is stored in the CAPIF core function and provided by the CAPIF administrator. The configuration data for CAPIF is specified in table E-1. Table E-1: Configuration data for CAPIF Reference; Parameter description. Subclause 4.2.2; List of published service API discovery restrictions. > Service API identification. > API invoker identity information. > Service API category. Subclause 4.3.2; List of service API authorization restrictions per API invoker. > API Invoker identity information. > Service API identification. > Per resource, operation authorization information. (see NOTE 2). > Network slice identification. Subclause 4.7.2; List of service API log storage durations. > Service API identification. > Service API log storage duration (in hours) (see NOTE 1). Subclause 4.7.4; List of API invoker interactions log storage durations. > Service API identification. API invoker interactions log storage duration (in hours) (see NOTE 1). Subclause 4.10; List of access control policy (for an API provider domain) per API invoker and optionally per network slice. > Volume limit on service API invocations (total number of invocations allowed). > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed). > Rate limit on service API invocations (allowed service API invocations per second). > Service API identification. > API invoker identity information. > Network Slice Info. Subclause 8.34.3; Group context information. > Group identifier. > List of UE IDs (group members). > UE identifier of the Group Resource Owner. > Authorization information related to the list of supported applications. >> Application identifier. >> Purpose. >> Scope. Subclause 8.38; List of service API Open Discovery restrictions. > Service API identification. > Service API category. NOTE 1: If no value is set for the duration, the duration is assumed to be unlimited. NOTE 2: The API invoker request for access authorization to specific resources or operations of a Service API is specified in 3GPP TS 33.122 [12]..

##### 6.x.3.0.4	Updated procedure for Open Discovery considering API Provider Domain and CAPIF administrator preferences

## 8.38 Open Discover service APIs ### 8.38.1 General The following procedure in this subclause corresponds to supporting open discover service APIs. Such open discovery operation allows requestor to discover service API information (e.g., API name, API category) about the available set of APIs offered by CCF before onboarding. ### 8.38.2 Information flows #### 8.38.2.1 Open Service API discover request Table 8.38.2.1-1 describes the information flow open service API discover request from the requestor to the CAPIF core function. Table 8.38.2.1-1: Open Service API discover request Information element; Status; Description. Query information; O (see NOTE); Criteria for discovering matching service APIs. It may contain Service API information described in table 8.3.2.1-1, except service API interface details.. NOTE: It should be possible to discover all the service APIs.. #### 8.38.2.2 Open Service API discover response Table 8.38.2.2-1 describes the information flow open service API discover response from the CAPIF core function to the requestor. Table 8.38.2.2-1: Open Service API discover response Information element; Status; Description. Result; M; Indicates the success or failure of the open discovery of the service API information. Service API information; O (see NOTE); Filtered service APIs information that can be shared to the requestor not a recognized user of the CAPIF. It may contain Service API information described in table 8.3.2.1-1, except service API interface details.. NOTE: The service API information shall be present if the Result information element indicates that the open service API discover operation is successful. Otherwise, it shall not be present.. ### 8.38.3 Procedure Figure 8.38.3-1 illustrates the procedure for open discover service APIs. The open service API discovery mechanism is supported by the CAPIF core function. Pre-conditions: 1. The requestor is not a recognized user of the CAPIF. 2. The CAPIF core function is aware of the Service API information that can be disseminated according to the requestor (i.e., Open Discovery policy to restrict discovery of APIs) who is not a recognized user of the CAPIF. Figure 8.38.3-1: Open Discover service APIs 1. The requestor sends an open service API discover request to the CAPIF core function. It may include the query information from the requestor. NOTE: The security aspects of this procedure are unspecified in this release of the specification. Editor's note: The security aspects of this procedure depend on the evolution of SA3 on Open Discovery security. 2. Upon receiving the open service API discover request, if query information is included, the CAPIF core function retrieves the stored service API(s) information as per the query information in the open service API discover request, otherwise all stored service API(s) information is retrieved. Further, since the requestor is not a recognized user of CAPIF, the CAPIF core function applies the Open Discovery sharable information received from the APF (clause 8.3.2.1), and if its not available, the configured Open Discovery restrictions (specified in Annex E) and performs filtering of service APIs information. 3. The CAPIF core function sends an open service API discover response to the requestor with the filtered list of service API information.

#### 6.x.3.1	Information flows

#### 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M (see NOTE 2); The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), service API operation(s) and resource(s) (optional), and Network Slice Info (optional), security methods.. Shareable information; O (see NOTE 1); Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained.. Open Discovery shareable information; O; Indicates whether the service API information or the service API category are allowed to be discovered using the Open Discovery API. It also includes a list of requester types to whom the service API information or the service API category can be disseminated.. NOTE 1: If the shareable information is not present, the service API information is not allowed to be shared. NOTE 2: The active and inactive states of the service APIs of 3GPP AEFs are based on the respective combinations of the management states (administrative, operational) specified in the management domain in TS 28.541 [17], TS 28.532 [18] and TS 28.538 [19].. The Service KPIs is defined as below: Table 8.3.2.1-2: Service KPIs Information element; Status; Description. Maximum Request rate; O; Maximum request rate from the API Invoker supported by the server.. Maximum Response time; O; The maximum response time advertised for the API Invoker's service requests.. Availability; O; Advertised percentage of time the server is available for the API Invoker's use.. Available Compute; O; The maximum compute resource available for the API Invoker.. Available Graphical Compute; O; The maximum graphical compute resource available for the API Invoker.. Available Memory; O; The maximum memory resource available for the API Invoker.. Available Storage; O; The maximum storage resource available for the API Invoker.. Connection Bandwidth; O; The connection bandwidth in Kbit/s advertised for the API Invoker's use..

### 6.x.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

* * * End of Change * * * *

<Proposed change in revision marks>