3GPP TSG-SA WG6 Meeting #70	S6-255291

Dallas, USA 17th – 21st November 2025	(revision of S6-255xyz)

Source:	Ericsson

Title:	Pseudo-CR on solution for KI#4, on Service API and AEF operational status

Spec:	3GPP TR 23.700-43

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides a solution for KI#5 for TR 23.700-43.

2. Reason for Change

KI#4 describes one existing gap in current deployments, about how to get awareness of API status (AEF/Service API) when the connection between the CAPIF core function and the AMF and/or APF, entities in charge of indicating AEF status, is disrupted due to e.g., network issues. Moreover, the AMF/APF may be unaware of API status when the AEF becomes not operational because the AEF are disconnected/unreachable or do not work properly, and consequently, they’re not able to properly interact with the the CCF.

Solution#4 for KI#4 develops a mechanism to notify about AEF availability, but does not cover how the CCF becomes aware of AEF/Service API availability.

Solution#5 for KI#4 develops a mechanism based on the collaboration of the API invoker with the CCF by notifying the AEF/Service API unavailability detected in CAPIF-2/2e interface.

However, none of these solutions tackle specific aspects of availability based on well known and deployed mechanisms.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.2.0)

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | … |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  | X |  | X |  |  |  |
| Sol #6 |  | X |  |  |  |  |  |
| Sol #X |  |  |  | X |  |  |  |

* * * Next Change * * * *

## 6.x	Solution #x: Determination of Service API and AEF unavailability

### 6.x.1	General

This solution relates to KI #4.

KI#4 describes one existing gap in current deployments, about how to get awareness of API status (AEF/Service API) when the connection between the CAPIF core function and the AMF and/or APF, entities in charge of indicating AEF/Service API status, is disrupted due to e.g., network issues. Moreover, the AMF/APF may be unaware of API status when the AEF becomes not operational because the AEF are disconnected/unreachable or do not work properly, and consequently, they’re not able to properly interact with the the CCF.

This solution discusses the currently availale procedures in CAPIF and proposes some enhancements in the description of the CAPIF framework to enable the determination AEF/Service API availability based on current functional level of 3GPP TS 23.222 [2] and industry adopted procedures.

### 6.x.2	Architecture Impacts

There are no architecture impacts.

### 6.x.3	Solution description

#### 6.x.3.0	General

##### 6.x.3.0.1	Introduction

This clause describes the optional procedure that may be supported by the CCF to detect unavailability of the AEF, APF and AMF.

In the current level of the specification of 3GPP TS 23.222 [2]:

-	The AMF manages the registration of the AEF, APF and AMF functions in the CCF using the CAPIF_API_Provider_Management_API as described in clauses 8.28, 8.29 and 8.30. The AMF monitors the status of the service APIs as specified in clause 6.3.6 and 6.4.8.

-	The APF publishes API provider the service APIs information in order to enable the discovery of service APIs by the API invoker (see clause 6.3.5)

Based on implementation specific means, and current functional level of 3GPP TS 23.222 [2], when the AMF detects that a service API becomes unavailable (3GPP TS 23.222 [2], clause 6.3.6 and 6.4.8), the AMF, based on implementation specific means, can notify the APF, which could update the CCF with the change on the service API status from Active to Inactive (3GPP TS 23.222 [2], clause 6.3.5).

In case the AMF becomes unavailable, neither the APF and hence the CCF can become aware of changes on the service API status. If during AMF unavailability the service API becomes also unavailable, whether the CCF detects AEF/Service API availability is implementation specific. Equivalently, in case the APF becomes unavailable, whether the CCF detects AEF/Service API availability is implementation specific.

The procedure is described in clause 6.x.3.0.2.

The potential impacts in 3GPP TS 23.222 [2] are collected in clause 6.x.3.0.3.

##### 6.x.3.0.2	Detection of AEF/Service API availability

This clause describes the optional procedure that may be supported by the CCF to detect AEF/Service API unavailability.

Figure 6.x.3.0.2-1 describes the detection of AEF/Service API availability.

Preconditions:

- The AMF supports the detection of AEF/Servie API availability by implementation specific means.

- The APF, based on AEF/Service API availability detected by the AMF, supports the update of the active/inactive status for a Service API.

- The CCF determines AMF/APF/AEF availability by implementation specific means.

Figure 6.x.3.0.2-1: Detection of AEF/Service API availability

1.	The API invoker subscribes with the CCF to notifications about Availability of Service API events (i.e., Service API Active/Inactive).

2.	The CCF acknowledges the subscription.

3.	The AEF becomes unavailable.

4.	The AMF detects AEF failure (e.g. based on heartbeat or any other implementation specific mechanism), the APF becomes aware of the AEF unavailability and determines the AEF/Service API change of status to Service API Inactive.

5.	The APF updates the service API(s) status in the CCF using the Publish service API.

6.	The CCF acknowledges the update.

7.	The service API status event triggers the notification to the API invoker about Service API status Inactive.

8.	The API invoker acknowledges the notification.

9.	AEF becomes available again.

10.	The AMF detects AEF recovery, the APF becomes aware of the AEF unavailability and determines the AEF/Service API change of status to Service API Active.

11.	The APF updates the service API(s) status in the CCF using the Publish service API.

12.	The CCF acknowledges the update.

13.	The service API status event triggers the notification to the API invoker about Service API status Active.

14.	The API invoker acknowledges the notification.

The API Invoker, based on the received Service API status, may determine further CAPIF-1/1e and CAPIF-2/2e interactions.

Based on implementation specific mechanisms, the CCF could monitor AEF status and based on the service API status change, may trigger the notification to the API Invoker e.g. about Service API information change.

##### 6.x.3.0.3	Potential impacts in 3GPP TS 23.222 [2]

### 6.3.5 API publishing function The API publishing function enables the API provider to publish the service APIs information in order to enable the discovery of service APIs by the API invoker. The API publishing function consists of the following capability: - Publishing the service API information of the API provider to the CAPIF core function; - Updating the Service API information related to the published API; and - Retrieving service API information from the CAPIF core function.

#### 6.x.3.1	Information flows

This solution does not impact the existing Information flows.

### 6.x.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

* * * End of Change * * * *

<Proposed change in revision marks>