3GPP TSG-SA WG6 Meeting #70	S6-255661

Dallas, USA 17th-21st November 2025	(revision of S6-255569)

Source:	Ericsson, AT&T

Title:	Pseudo-CR on new solution to KI#1 on application user consent architecture

Spec:	3GPP TR 23.700-42 v0.2.1

Agenda item:	9.10

Document for:	Approval

Contact:	Cristina Badulescu, cristina.badulescu@ericsson.com

1. Introduction

This pCR adds new solution for KI#1 on the application user consent architecture.

2. Reason for Change

New solution to the KI#1 on addressing the architecture for the application user consent management: a standalone application user consent management function is proposed, that can address the KI#1 constraints, identified open issues and the use cases from the TR.

3. Conclusions

New solution to be added to the TR.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-42.

* * * First Change * * * *

# 8	Solutions

## 8.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | … |
| --- | --- | --- | --- |
| Sol #1 | x |  |  |
| Sol #2 |  |  |  |
| Sol #... |  |  |  |

## 8.x	Solution #1: Standalone Application user consent management function

### 8.x.1	General

This solution relates to KI #1 on the architecture of application user consent management. The open issues to be studied from this KI are described in clause 7.x.2.

### 8.x.2	Architecture impacts

This solution proposes a standalone application user consent function. The following aspects, indicated in the use cases analysis in clauses 4.2.2, 4.3.2 and 4.4.2 and in the business relationships, are covered in this solution for the architecture of the application user consent management function:

- The exposure of interfaces to allow the consumers (e.g., Exposure platforms) to check the application user consent for an user, or to subscribe to be notified of application user consent data changes.

- The support for the management of the data needed for the application user consent, covering application user consent per application and additional data (subject to the applicable regional regulation, such as purpose and user data to be made available for user data scope).

  - While additional operational data is expected to be also recorded by the application user consent management, the extent and composition of operational data can vary based on regional and provider specific needs, and therefore is not studied and addressed in the present document. An example for reference is available in Annex G.4.2 of the GSMA PRD OPG.02 [2].

  - As the storage of the application user consent data and of any additional operational data are handled by the application user consent function, no specific interfaces need to be standardized for the actual storage as these are internal to the application user consent function.

- The support for operating immediately application user consent data changes, at any time the user chooses to change his/her application user consent for an application. This can include update/revocation of application user consent for the indicated application and application context (e.g., purposes), notifying the subscribed entities to the application user consent data chnages (so they can apply the change to the authorization result for ongoing and new application traffic for that user).

Figure 8.x.2-1: Standalone Application user consent management function

Editor’s Note: The architecture will be updated to address further use cases and updates to requirements.

The APCOT-1 is exposed by the Application user consent management function and supports application user consent check requests and responses, as well as subscribe/notify to application user consent data changes and queries for consent data. Consumers of APCOT-1 include Exposure platforms.

The APCOT-2 supports application user consent data management operations (application user consent create, updates/revocation, removal). The Consent capture solutions are specific to each data controllers (e.g., PLMN operator) and are not in scope of the study.

Editor’s Note: the security aspects for the APCOT-1 and APCOT-2 are handled by SA3.

### 8.x.3	Solution description

An example of interaction between the consumer of APCOT-1 interface (an Exposure platform) and the application user consent management function is described below.

Figure 8.x.3-1: Application user consent check over APCOT-1

1.	The Consumer (Exposure platform) requests the application user consent check for the specific user and application context information over APCOT-1.

2.	The Application user consent management function checks the available application user consent data for the received request to determine whether the consent was granted or not.

3.	The Application user consent management function provides the check result back to the Exposure platform, indicating if the application user consent was granted or not.

NOTE: when the Exposure platform and the application user consent management function are collocated, the steps 1 and 3 are internal and do not require use of a standard interface (an example is CAPIF Authorization Function).

* * * Next Changes * * * *

# 11	Recommended requirements

## 11.1 	General

Subject to applicable regional regulations, different type of data is required to be obtained from the user as part of user’s agreement provided to a data controller (e.g., PLMN operator) to share his/her data to applications.

## 11.2	Description

This section provides description of the proposed requirements for the subsequent normative work.

[APCOT-11.2-x] The Application user consent management shall expose an interface to support the application user consent check requests.

[APCOT-11.2-y] The Application user consent management shall expose an interface to support the subscription requests for application user consent data changes and shall send the due notifications to the subscribed entities.

[APCOT-11.2-z] The Application user consent management shall expose an interface to support the application user consent management operations (create, update/revoke, delete) on the application user consent data, at any time the user wishes to change his/her application user consent for an application.

[APCOT-11.2-w] The application user consent management function shall manage the application user consent provided to a data controller (e.g., PLMN operator). The application user consent is per application targeted for user data sharing, per purpose of the application and per user’s data that is being shared.

Editor’s Note: requirements will be updated during study conclusion phase.

* * * End of Changes * * * *