3GPP TSG-SA WG6 Meeting #69	S6-254771

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254513

Source:	Ericsson

Title:	New Solution for KI#1 on New Service on Energy Saving Assistance

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-44 clause 5.1), the study aspects on enhance application enablement layer architecture and services to support energy saving include:

- Whether enhancements to existing application enablement layer services or a new application enabler service is required to support energy savings operations in the application enablement layer.

According to the use cases and comment functions identified in S6-254224, service on energy saving assistance is needed for supporting consumer on energy saving. This pCR proposes new Energy Saving Enablement (ESE) service to assist consumer on energy saving.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1 by introducing a new Energy Saving Enablement (ESE) service to assist consumer on energy saving.

3. Conclusions

This paper proposes solution on new Energy Assistance and Management Enablement (ESE) service to assist consumer on energy saving.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #X2 | X |  |  |  |  |  |

* * * Next Change * * * *

## 6.X2	Solution #X2: New Service on Energy Saving Assistance

### 6.X2.1	Solution Description

#### 6.X2.1.0	General

The following clauses specify the procedure and information flow on energy saving assistance provisioning.

#### 6.X2.1.1	Procedure

Figure 6.X2.1.1-1: Energy saving assistance

1.	A consumer (e.g., VAL server, SEAL server/client) sends energy saving assistance request to SEAL server. The request includes information as described in Table 6.X2.1.2.2-1.

2.	The SEAL server authenticates and authorizes the request. If authorized, the SEAL server determines the next actions (e.g. collect energy-related data, monitor energy-related information/status) for assisting energy saving. The ESE server can consolidate similar requests from Consumers (e.g., data collection from same source into single southbound requests, avoid re-fetching data that it already has if the request parameters allow it such as time frame matches, etc.).

3.	The SEAL server sends energy-related data collection response to the consumer. The response includes information as described in Table 6.X2.1.2.3-1.

4.	The SEAL server collects energy consumption information, monitor energy status, and generates assistance information for energy saving based on the data/information obtained in step 4.

5.	The SEAL server sends energy saving assistance notify to the consumer with the assistance information. The notify includes information as described in Table 6.X2.1.2.4-1.

Editor’s Note:	The update of the solution is FFS.

#### 6.X2.1.2	Information Flows

##### 6.X2.1.2.1	General

The following information flows are specified for energy saving assistance.

##### 6.X2.1.2.2	Energy saving assistance request

Table 6.X2.1.2.2-1 shows the request sent by a SEAL service consumer to a SEAL server for energy saving assistance procedure.

Table 6.X2.1.2.2-1: Energy saving assistance request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| VAL service ID | M | The identity of the VAL service for which the request applies. |
| Target energy saving entity(ies) | M | The identity(ies) of the target energy saving entity(ies) (e.g. VAL UE ID(s), SEAL server ID(s)) |
| Assistance requirements | M | Identifies the requirement for the assistance on energy saving. |
| >Assistance information type | M | Identifies the assistance information type required for energy saving (e.g. information on entity which consume the most/least energy, assistance information for selection of entity(ies)). |

##### 6.X2.1.2.3	Energy saving assistance response

Table 6.X2.1.2.3-1 shows the response sent by the SEAL server to the consumer for energy saving assistance procedure.

Table 6.X2.1.2.3-1: Energy saving assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement). |
| Subscription ID | O | Identifier of the subscription. It shall be provided if the request if the Result is positive acknowledgement. |

##### 6.X2.1.2.4	Energy saving assistance notify

Table 6.X2.1.2.4-1 shows the notification sent by the SEAL server to the consumer for energy saving assistance procedure.

Table 6.X2.1.2.4-1: Energy saving assistance notify

| Information element | Status | Description |
| --- | --- | --- |
| Success status | O (NOTE) | Indicates that energy saving assistance was successful. |
| >Assistance information | M | Provides energy saving assistance information corresponding to the request. |
| Failure response | O (NOTE) | Indicates that energy saving assistance was failure. |
| >Cause | M | Reason for the failure. |
| NOTE: One of the information elements shall be provided in the output. |  |  |

### 6.X2.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X2.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***