3GPP TSG-SA WG6 Meeting #68	S6-253602

Stor-Göteborg, Sweden 25th – 29th August 2025                                       was S6-253179

Source:	Nokia

Title:	Pseudo-CR on updating Adoption Solution 1

Spec:	3GPP TR 23.700-35 v1.0.0

Agenda item:	9.3

Document for:	Approval

Contact:	Rajesh Babu Natarajan rajesh_babu.natarajan@nokia.com

- Introduction

Adoption solution #1 proposes updates to align with Service based Architecture of 3GPP system supporting SEAL services, to address the adoption gap identified in GAP# 5-1-2-1

- Reason for Change

Adoption solution #1 does not highlight the NEF (N33) used in the NBI and SEAL services conforming to CAPIF framework. Hence, figure 8.2.1-1 is updated to include the same.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v1.0.0.

* * * First Change * * * *

#### 8.2.1.1	Solution description

This solution is to solve GAP# 5-1-2-1.

Figure 8.2.1-1 Service based Architecture of 3GPP system supporting SEAL services

In the figure 8.1.1-1, SEAL services layer represents the SEAL server(s) supporting SEAL services defined in the SEAL specification 3GPP TS 23.434 [3].

SEAL services which are part of the 3GPP system can be deployed either by the MNO or third party.

The interactions between the vertical application server(s) and SEAL server(s) for discovery, authentication and authorization are supported by CAPIF as specified in 3GPP TS 23.222 [9]. The VAL server acts as CAPIF's API invoker and SEAL server acts as CAPIF's API exposing function.

SEAL server(s) can invoke service APIs exposed by 3GPP core network functions directly over the Core network SBA when deployed by MNO’s trusted domain. SEAL server(s) can invoke service APIs exposed by 3GPP core network functions via NEF as a non-trusted AF when deployed by the third party.

SEAL server(s) can invoke service APIs exposed by the OAM system over the OAM SBMA when deployed by MNO as specified in 3GPP TS28.533[14]. SEAL server(s) can invoke services APIs exposed by 3GPP core network functions via EGMF as non-trusted AF, when deployed by the third party.

SEAL clients take two different responsibilities: 1) consumer of SEAL server API(s) 2) providing required terminal side information/resource to support the SEAL server function(s)

SEAL services API can be exposed by MNOs or third party, to the Application layer(VAL layer) or to the SEAL client(s) or third party’s platform.

NOTE 1:	 It is up to application developer’s implementation how to call the SEAL server’s APIs in their application layer software.

NOTE 2:	3GPP TS 23.222 [9] supports where the API invokers can either be on the network side (Application server) or on the UE side (e.g., SEAL clients), to use SEAL server API(s).

* * * End of Change * * * *