3GPP TSG-SA WG6 Meeting #69	S6-254610

Wuhan, China, 13th – 17th October 2025	(revision of S6-254243)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-35 | CR | 0007 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Conclusion for technical gap#4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_SEAL_Ph4 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MonitoringEvent API is included in the solution, but not the procedure in clause 8.1.11.3 of solution 11 related to the power saving configurations. Therefore, this CR includes it in step 3. Furthermore, as there is no conclusion on the way forward related to technical gap#4, this CR concludes technical gap#4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addressing EN related to conclusion for technical gap#4 and adding Monitoring API in step 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN is not addressed in the study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.11.3 and 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 8.1.11.3	Procedure case 1

Figure 8.1.11.3-1 describes the procedure when the VAL server triggers power saving assist service for power saving in IoT devices.

Figure 8.1.11.3-1: Procedure for VAL server triggers power saving assist service

1.	VAL server requests the NRM Server to assist power saving of the VAL user(s) that are part of VAL Service. The request contains the list of VAL UEs and VAL Users that are targets for the power saving, VAL service ID, power saving requirements, e.g., mode (power-saving, balanced, performance) (see Table 8.1.11.2-1).

2.	NRM server determines, based on the VAL service ID, UE(s) access type and power saving mode, the power saving configuration, network parameters and communication pattern parameters for each requested VAL UE(s). Furthermore, the SEAL Server may determine the VAL UE ID(s) associated with the VAL user(s) included in step 1.

3.	NRM Server interacts with 3GPP CN (e.g., CpProvisioning, NpConfiguration and/or MonitoringEvent APIs) to provision the VAL UE(s) network parameters and communication pattern parameters determined in step 2.

4.	NRM Server interacts with NRM Client with recommended power saving configuration for the UE (e.g., the determined power saving configuration in step 2) (see Table 8.1.11.2.-2).

5.	The target UE interacts with 3GPP network as per clause 4.2.2.2.2 of 3GPP TS 23.502 [11], to indicate the desired power saving configuration, e.g., DRX or eDRX configuration. The UE receives from 3GPP RAN the accepted power saving configuration.

NOTE 2:	It is up to the 3GPP system to determine the accepted, i.e., final DRX configurations to be reconfigured in the VAL UE.

6.	NRM Client responds to the NRM Server with power saving configuration response and indicates the accepted power saving configuration (see Table 8.1.11.2-3).

7.	NRM Server responds to the VAL Server with power saving assist response (see Table 8.1.11.2-4).

# * * * * Second change * * * *

## 10.2	Conclusions of SEAL services technical gap #x

This clause provides the final conclusion for each technical gap.

Technical Gap#1:

Editor's Note: The conclusion for the technical gap#1 is FFS.

Technical Gap#2: Technical solution#1 is recommended as basis for normative work.

Technical Gap#3: Technical solution#4 is recommended as basis for normative work.

Technical Gap#4:

Solution 11, which is a generalized solution with respect to solution 9, is recommended for the normative phase.

Technical Gap#5: Technical solution#5 is recommended as basis for normative work.

Technical Gap#6: Technical solution#8 is recommended as basis for normative work.

Technical Gap#7: Technical solution#12 is recommended as basis for normative work.

Technical Gap#8:

Editor's Note: The conclusion for the technical gap#8 is FFS.

# * * * * End of changes * * * *