3GPP TSG-SA WG6 Meeting #69	S6-254702

Wuhan, China 13th-17th October 2025	(revision of S6-254381, S6-254134)

Source:	Ericsson

Title:	Pseudo-CR on terms clarifications (3GPP)

Spec:	3GPP TR 23.700-42 v0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Cristina Badulescu, cristina.badulescu@ericsson.com

1. Introduction

This pCR brings proposal for the clause on terms clarifications based on terms usage in 3GPP specifications.

2. Reason for Change

Add use case to the TR.

S6-254702 has same content as S6-254381.

3. Conclusions

Add the proposed text to the TR.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-42.

* * * First Change * * * *

### 6.2.	Industry group #3: 3GPP

No term definition for user consent, or for consent, was found in the 3GPP specifications studied.

In SA3, there are two related terms that are used for different purposes:

- User consent:

  - covers Core Network data shared to NFs or to the Edge applications.

  - Requirements and procedures are defined in Annex V of 3GPP TS 33.501 [9]. User consent data is managed and stored in UDM.

  - Requirements are defined on the NFs that are an enforcement point for the user consent in Annex V.3 of [9], as well as additional requirements on NWDAF in Annex X.7 of [9].

  - For edge applications, 3GPP TS 33.558 [10] reuses the same procedures by referencing 3GPP TS 33.501 [9] (Annex V). Either EES or the NEF act as the consent enforcing entity of the user consent for the edge applications.

- RO authorization:

  - has a definition in 3GPP TS 33.122 [14] in context of RNAA:
"Resource owner authorization: The permission provided by the resource owner to allow the API invoker to access the resource owner’s resource via the northbound API."

  - It covers the resource owner (subscriber) agreement, to expose over northbound APIs, specific user data from API Provider domain to applications.

Several SA2 specifications mention user consent, but no definition of the term is provided. In the procedures, the user consent data is in UDM/UDR and is accessed and enforced by Core Network NFs: EIF in 3GPP TS 23.501 [4], NEF in 3GPP TS 23.502 [5], LMF, NWDAF in 3GPP TS 23.288 [8], and Location functions in 3GPP TS 23.273 [6] and 3GPP TS 23.271 [7].

The CT4 specification 3GPP TS 29.503 [11] defined in clause 6.1.6.3.20 “Enumeration: UcPurpose” the user consent ennumeration values that are stored in UDM, which can be consumed by e.g., NFs and trusted AF:

- "ANALYTICS": User consent for analytics

- "MODEL_TRAINING": User consent for model training

- "NW_CAP_EXPOSURE": User consent for network capability exposure.

- "EDGEAPP_UE_LOCATION": User consent for the manipulation of UE information for the purpose of UE Location retrieval by the EDGEAPP EAS entity.

SA6 specifications have introduced user consent aspects in:

- CAPIF 3GPP TS 23.222 [13] under RNAA, which includes the user consent for applications under the term RO authorization defined in 3GPP TS 33.122 [14]. 
Also in CAPIF 3GPP TS 23.222 [13] it is stated in clause 6.2.3. in NOTE 4:
"The authorization information from the resource owner used by CCF (described in 3GPP TS 33.122 " [14] ") is independent from the user consent information used from user subscription data at UDM/UDR (described in Annex V of 3GPP TS 33.501" [9] "). In the current release of 3GPP specifications, no synergy between CCF and UDM is specified."

- EDGEAPP 3GPP TS 23.558 [12] has user consent for edge applications, referencing 3GPP TS 33.558 [10] and 3GPP TS 33.501 [9] (Annex V). A summary of the user consent for edge applications is provided in Table 6.2-2 below.

For reference, a summary of the user consent purpose and specifications indicating consent handling in the above 3GPP SA6 EDGEAPP specification is in Table 6.2-1 below.

Table 6.2-1: SA6 EDGEAPP user consent summary

| User Consent purpose (per 3GPP TS 23.558 [12]) | User Data (subjected to consent) | Enforcer | User Consent management |
| --- | --- | --- | --- |
| Generally per each AF/ app | UE Location - clause 8.14.2.3 | LMF, NEF (via enforcing the OAuth token scope) | UDM (referred in SA3 3GPP TS 33.558 [10] and 3GPP TS 33.501 [9]) |
| EDGEAPP (per app/AF) | GPSI (MSISDN) - clause 7.2.6 | EES (clauses 6.3.2; 8.5.2.2) | UDM |
| EDGEAPP (per app/AF) | UE tunnel e2e info (security) – clause 8.3.3.2.2 | EES (clauses 6.3.2; 8.5.2.2) | UDM |
| EDGEAPP (per app/AF) | AF specific UE identifier (MSISDN) – clause 8.6.5.2 | EES (clauses 6.3.2; 8.5.2.2) | UDM |

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]

[3]

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[6]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)"

[7]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)"

[8]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support network data analytics services"

[9]	3GPP TS 33.501: " Security architecture and procedures for 5G system "

[10]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications"

[11]	3GPP TS 29.503: "5G System; Unified Data Management Services"

[12]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

[13]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs"

[14] 	3GPP TS 33.122: " Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * End of Changes * * * *