3GPP TSG-SA WG6 Meeting #69	S6-254669

Wuhan, P.R.China, 13th – 17th October 2025	(revision of S6-254150)

Source:	Huawei, Hisilicon

Title:	New solution of Ambient service provisioning

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

<Introduction part >

This solution aims to address the issues identified in Key Issue 2 and Key Issue 4.

2. Reason for Change

In order to enabler the AIoT application layer to use their pre-configured application layer task, the AIoT enabler layer should provide a service provisioning service to the AIoT application layer. In this way, the AIoT enabler layer could easily promote the market adotation from one AIoT application to the others. From the AIoT application layer, they does not need to deep into the 3GPP network layer service API or 3GPP network reported AIoT data.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x |  | Y |  | Y |
| Sol #2 |  |  |  |  |

* * * Next Change * * * *

7.x	Solution #x: Solution for AIoT service provision

7.x.1	Solution description

This solution address key issue#2, and #4 to introduce a solution for AIoT service provision.

AIoT network service can provide the information of what (AIoT device IDs) and where (AF ID and the external Target Area information), as shown in the TS 23.369[7]. However, these network exposured data can’t be directly consumed by the 3rd party appication layer and needs to be correlated with application information. The core idea is to establish a mapping relationship for network layer information and the application data.

The figure 7.x-1 show the high-level procedures of AIoT service provisioning.

Figure 7.x-1: service data provisioning

1.	The AIoT application layer consumer invokes the service provisioning service of AIoT enabler server by sending the AIoT service data provisioning request. The request contains the following information:

-	a.	Mapping of AIoT devece IDs and specific application objects (such as a product, a pallet, a tool, or a person) ID. This is the premise of all associations. When attaching AIoT devices to objects of the ASP, it is necessary to create a "AIoT devece ID and application object" association in theAIoT enabler server to record this binding relationship.

b.	The application object information including the object ID, name, type, other attributes, etc.

c.	AIoT application service regions and its associated target area information, the associated AIoT service scenarios.

d.	AIoT service scenario and its cooresponding service requriements and interested AIoT service events

The AIoT service scenario is tightly correlated with the application service meanings. The cooresponding service requirements and the interested AIoT service event will be used be later inforced during the invocation of AIoT network services and the processing of network exposured AIoT data before providing enriched information to the AIoT application layer.

The AIoT service scenario categoried based on the application requirements, such as inbound/outbound, inventory, monitor and tag command, and each type may need different kinds of 3GPP AIoT network service. For different scenarios, the interested AIoT service events may be different, which should be set according to the requirements of the application layer. For example, the warehouse storage inventory scenario may need the inventory result about what objects are exist or what objects are missing or the counting of the objects.

An example of those information is listed as table 7.x.1-1.

Table 7.x.1: an example of scenario type, service requirement, interested service event

| AIoT service scenario Type | AIoT network services requements | interested AIoT service events |
| --- | --- | --- |
| Inbond/Outbound | Rapid inventory and report with time interval X for result aggregation | Inbond/Outbound event/ Alarm Event |
| Storage Inventory | non-rapid inventory and report time interval Y for result aggregation | Inventory/Alarm/ Counting Event |
| Monitor | periodic inventory | Moving/Alarm Event |
| Tag command | Read/Write/Disable service | Read/Write/Disable result event |

2.	The AIoT enabler layer performs the authorization check. If authorized, it stores the information received in the request for futher usage.

3.	The AIoT enabler layer returns a AIoT service data provisioning response indicating the success of service provisioning.

7.x.2	Architecture Impacts

This solution introduces the AIoT service provisioning capability to the AIoT enabler layer.

7.x.3	Corresponding APIs

A new AIoT service provisioning service is introduced.

7.x.4	Solution evaluation

* * * Next Change * * * *