3GPP TSG-SA WG6 Meeting #69	S6-254197

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New Solution on enhancements of UAV services utilizing sensing results

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution for support enhancement of UAV services utilizing sensing results.

2. Reason for Change

To proposes a new solution to support the enhancement of UAV services utilizing sensing results.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * *

5.x	Solution #x: Enhancements of UAV services utilizing the sensing results

5.x.1	Solution description

This solution addresses the KI#2.

The following high-level solution principles apply to this solution:

-	A high-level procedure for sensing request and exposure is proposed with a new Sensing enabler in the application enablement layer.

-	The solution assumes the 3GPP core network will expose the sensing results related to UAVs to the AF (e.g., VAL server).

5.x.1.1	Procedure of detecting UAVs utilizing sensing results

Figure 5.x.1.1-1 illustrates the high-level procedure of detecting UAVs utilizing sensing results.

Pre-condition:

-	The following Sensing enabler is a new enabler that implements the sensing functions in the application enablement layer.

Figure 5.x.1.1-1: Procedure of detecting UAVs utilizing sensing results

- The VAL server sends a UAV detect request to the UAE to detect the UAVs in a certain area, including the service type (i.e., UAV detection, invalid UAV detection ), service ID, sensing service requirements for the detected UAVs (e.g., location accuracy, velocity, latency, sensing resolution, etc.), the target area, the reporting conditions (e.g., periodic reporting or event-triggered reporting), the reporting time intervals if the request is periodic, time duration, optional tracked object information (e.g., UAV ID, flight trajectory) if the object needs to be tracked, etc.

- The UAE checks whether the VAL server is authorized to invoke such request based on e.g. the pre-configurations or the operator policies.

- If the request is authorized, the UAE sends the UAV related sensing results request to the sensing enabler to ask for the UAV related sensing results.

4.	The sensing enabler may invoke the sensing related service request to the 3GPP CN and obtain the sensing results from the 3GPP CN via NEF if the SM server is an untrusted AF or interact with the SF entity in 5GC directly as a trusted AF.

Editor’s note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

5.  Upon received the exposure from network layer, the sensing enabler may determine the detected UAVs that entering the target area and then the sensing enabler may interact with LMS to track the detected UAVs and obtain the location trajectory (including the velocity, direction, altitude, etc.) for them in the requested time duration.

Editor’s note:	How the LMS tracks the detected UAVs that without UE modules is FFS.

6.	The sensing enabler aggregates and combines the sensing results obtained in step 4 and step 5 to determine all of UAVs entering the target area as well as their flight trajectories, and then the sensing enabler compares the detected UAV flight trajectory with requested flight trajectory included in Step 1 to identify if the UAVs entering the target area are invalid or not.

7.	The sensing enabler notifies the UAE for the UAV related sensing results, including all UAVs entering the target area and the invalid UAVs that not allowed entering the target area.

8.	The UAE sends the analysed sensing results to the VAL server via UAV detect response message, including all UAVs entering the target area and the invalid UAVs that not allowed entering the target area.

5.x.2	Architecture Impacts

This solution proposes a new sensing enabler to support the sensing service.

For the new architecture for the sensing enabler, pls check the solution for the KI#1.

5.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *