3GPP TSG-SA WG6 Meeting #69	S6-254733

Wuhan, China 13th – 17th October 2025	(revision of S6-254654, S6-254118)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Overview of Frameworks CAPIF

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

This contribution provides the Overview of Frameworks CAPIF text for the Guidelines for 3GPP Application Enablement usage (AppEnable_EXT) technical report.

2. Reason for Change

This contribution proposes text for the Overview of Frameworks CAPIF of 3GPP TR 23.947.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.1.0.

* * * First Change * * * *

## 4.2	Common API Framework (CAPIF)

The Common API Framework (CAPIF) is a 3GPP framework that provides common functions for secure API exposure, discovery, onboarding, and authorization across multiple domains, 3GPP TS 23.222 [2]. Its primary role is to harmonize how service APIs are published and consumed, offering a single point of entry for applications (API invokers) towards the common API aspects (CAPF APIs) and a consistent method for API providers to expose their service APIs. By introducing a common API framework, CAPIF prevents fragmentation because application developers and third-party service providers can use one standardized approach, rather than dealing with independent interfaces for each service API. CAPIF is designed to be used in conjunction with other frameworks, where it is agnostic to the content of the APIs and instead focuses on the exposure mechanism for APIs.

CAPIF was first specified in 3GPP Release 15 and has since been adopted for exposing APIs from core network functions (e.g. NEF for network capabilities, MBMS, etc.) as well as from application-enablement layer services, in a secure and controlled manner.

The CAPIF architecture is based on a service-oriented model with three main entity groups as illustrated in Figure 4.2.1, specifically a CAPIF Core Function (CCF), the API Invoker and one or more API Provider domains:

-	CAPIF Core Function (CCF): The CCF handles the onboarding of API invokers and API registering of API provider domain functions (establishing trust and identity for each). It maintains the service API registry for discovery, and provides common services like authentication, authorization token issuance and logging.

-	API Exposing Function (AEF): A function on the API provider domain side that hosts one or more service APIs and exposes them to external consumers (API Invokers). The AEF is the entry point through which an API invoker accesses the provider’s service API; it enforces the service’s access policies and handles the runtime service API calls (e.g. rate limiting). A 3GPP network function or application server can presented themselves as an AEF to offer their service APIs in a standardized way.

-	API Publishing Function (APF): A provider-side function responsible for publishing the service API information and endpoints of the AEF to the CAPIF Core Function. When an API is made available, the APF registers the API’s meta-information (e.g. name, version, resource paths, supported operations) with the CCF, so that prospective invokers can discover it.

-	API Management Function: A provider-side management entity that allows the API provider to administer and monitor their APIs. The API Management Function interfaces with the CCF for reporting and retrieving logs of API invocations (for auditing), subscribing to events (e.g. notifications of certain API usage or threshold breaches).

-	API Invoker: The consuming entity that uses CAPIF to discover and invoke service APIs. An API Invoker could be an external third-party application or another network function/application within the operator’s environment. The invoker first onboards with the CAPIF core function to establish its identity and credentials, then queries the API registry to discover available services, and finally invokes the desired API on the corresponding AEF, obtaining authorization from the CCF as required. In a developer context, the API Invoker role is played by the developer’s application or service that seeks to utilize operator or third-party APIs in a secure manner.

Figure 4.2.1: Functional model for the CAPIF as defined in 3GPP TS 23.222 [2]

CAPIF operates through a set of well-defined procedures and interfaces. At a high level, the lifecycle of API exposure and consumption in CAPIF is as follows:

1.	Onboarding and registration: An API provider (such as an Edge Configuration Server in an edge computing scenario) will register with the CCF to become a recognized provider, typically using credentials or certificates issued by the operator’s CAPIF authority (see clause 8.28 of 3GPP TS 23.222 [2]). Similarly, an application acting as an API invoker (e.g. a vertical application server or a UE client) will onboard with the CCF to obtain a secure invoker identity (see clause 8.1 of 3GPP TS 23.222 [2]). The CAPIF core function can provision digital certificates or JSON Web Tokens (JWT) and record the entity’s public information (like its name, domain, and roles) in its database. This establishes a trust relationship: the CCF recognizes the entity as an authorized API provider or consumer in the system.

2.	API Publication (see clause 8.3 of 3GPP TS 23.222 [2]): Once registered, an API provider exposes its service API through CAPIF. The provider’s API Publishing Function sends the API description to the CAPIF core (via the CAPIF publishing interface, e.g. CAPIF-4) to register the API in the registry. This publication typically includes the API’s endpoint URI (which can point to the AEF), the resource structure and methods, required security scopes or policies, and any other metadata (version, category, etc.). For instance, in an EDGEAPP scenario, after an Edge Configuration Server (ECS) registers, it would publish its available service APIs (like edge node registration API, configuration APIs) to CAPIF so that edge clients or other components can later discover them. The CAPIF core can verify that the publishing entity is authorized to expose those APIs (e.g. checking that the provider is indeed entitled to publish an “Edge Configuration” service). Once published, the API is discoverable by the authorized invokers.

3.	Discovery (see clause 8.7 of 3GPP TS 23.222 [2]): An API invoker queries the CAPIF Core Function to discover information a service API. Discovery can be performed using various criteria (service name, category, provider, etc.). The CCF will return the details of the AEF address, interface details and information on how to invoke it. CAPIF supports a federated discovery if multiple CCFs are interconnected, but within a single domain the CCF acts as the central lookup for all APIs. For example, an Edge Enabler Client (EEC) that needs to call a service on an Edge Application Server (EAS) can query CAPIF (via the CCF) to get the API endpoint and requirements for the EAS’s service API.

4.	API Invocation with Authorization: Before invoking the target API, the API invoker obtains authorization from CAPIF. In CAPIF’s security framework, the CCF can function as an OAuth2 Authorization Server for API access tokens, see 3GPP TS 33.122 [7]. The invoker (acting as an OAuth client) requests an access token from the CCF, proving its identity (using credentials established at onboarding) and specifying the API it wants to call. The CCF issues a OAuth2 token if the invoker is allowed to use that API, with appropriate scope/permissions encoded. Armed with this token, the invoker then sends the API request to the provider’s AEF (over the CAPIF-internal interface to the AEF, e.g. CAPIF-2) and includes the token in the invocation. The AEF validates the token (by verifying its signature and checking with CCF if needed) and, if valid, allows the request to be processed by the underlying service. This ensures that only properly onboarded and authorized applications can invoke protected APIs. CAPIF also supports additional authorization models – for example, a mode where a “Resource Owner” (end-user or owner of the accessed resource) involvement is required for authorization (termed Resource Owner Aware API Access, RNAA).

5.	Logging and Monitoring (see clause 8.19 and 8.21 respectively of 3GPP TS 23.222 [2]): As APIs are invoked, CAPIF provides a logging service. Each service API invocation and response can be logged via the CAPIF core (or the provider’s API Management Function reporting to CCF), recording details such as who invoked which API, when, and with what result. These logs enable auditing and troubleshooting. CAPIF can also emit events or notifications – for instance, if an API provider wants to be notified when a new invoker enrols to its API or when certain thresholds are exceeded, the framework can support that through an events interface.

For application developers, CAPIF greatly simplifies integration with 3GPP network services and other service platforms. Instead of dealing with separate authentication and discovery mechanisms for each API (e.g. one for location services, another for messaging, etc.), a developer can rely on CAPIF to provide a one-stop hub for all API interactions. For example, a developer building a fleet management application could use CAPIF to discover the operator’s “vehicle location API” and invoke it securely, without needing to know the internal details of the network’s location function – CAPIF mediates trust and access. Likewise, if the developer’s application provides its own API (say it’s an analytics service that the operator might call), the developer can publish this API via CAPIF so that it’s uniformly accessible to other authorized parties.

* * * End of Changes * * * *