3GPP TSG-SA WG6 Meeting #70	S6-255335

Dallas, United States 17th – 21st November 2025	(revision of S6-255086)

Source:	Netherlands Police

Title:	Pseudo-CR on Key issue Recording and logging of migrated MC user

Spec:	3GPP TR 23700-39-040

Agenda item:	9.2

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

This pCR introduces a key issue related to recording and logging of the communications of an MC user that has migrated to a partner system.

2. Reason for Change

This key issue should be addressed in the study.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-39 v 0.4.0.

* * * First Change * * * *

## 5.x	Key issue x: Recording and logging of migrated MC user

An MC user that has migrated to a partner MC system receives MC service within the security domain of the partner MC system. Recording and logging of the communications of a migrated target MC user may require involvement from (an authorized MC user of) the primary MC system of the target MC user and from (an authorized MC user of) the partner MC system.

The scenario is illustrated in figure 5.x-1 below.

Figure 5.x-1: Recording and logging of migrated MC user

Issues:

-	(An authorised MC user BA of) the partner MC system B should authorize recording and logging of communications of the migrated MC user AB by primary MC system A.

-	There may be a need to restrict the provided content of communications to that which is originated by the migrated MC user AB, and not to provide content which originates from other MC users receiving service within the partner MC system B.

-	There may be a need to restrict metadata relating to the communications to hide the identities and other information relating to MC users and MC groups in partner MC system B.

-	(An authorised MC user AA of) the primary MC system A of the migrated MC user AB should authorize recording and logging of the communications of the migrated MC service user AB by MC system B.

* * * End of Changes * * * *