3GPP TSG-SA WG6 Meeting #69	S6-254726

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254224, 4511

Source:	Ericsson, InterDigital, Lenovo

Title:	New Solution for KI#1 on Functional Architecture to Support Energy Saving

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-44 clause 5.1), the study aspects on enhance application enablement layer architecture and services to support energy saving include:

- Whether enhancements to existing application enablement layer services or a new application enabler service is required to support energy savings operations in the application enablement layer.

SA1 introduced the use case (in TR 22.882 clause 5.14) on energy saving relevant to application layer edge and AIML services. In the use case, some compute tasks have flexibility in both when and where they are executed, i.e., such type of workload could be executed in any computing node and tolerate some delays if the workload gets completed within certain given deadline. For example, some of AI/ML training, simulation, and video processing tasks might not require a quick response, which would allow flexibility to delay the execution of the related workloads in a computing node until, e.g., the utilized energy is deemed satisfactory in terms of GHG emissions. In addition, to support energy saving for the AIML tasks, the services on FL member selection, AIMLE client selection, split AI/ML operation can be enhanced to take energy requirements into consideration.

In above use cases, there are common requirements from different application enabler services to support energy saving, for example energy-related data collection, processing (combination/aggregation), information/status monitoring, and energy saving assistance. Enhancements to the application enablement layer architecture are needed to introduce common functionalities to fulfil these comment requirements.

Thus, this pCR proposes to enhance the application enablement layer architecture by introducing new enabler service to support energy saving.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1, to enhance the application enablement layer architecture by introducing new enabler service to support energy saving.

3. Conclusions

This paper proposes solution on functional architecture to support energy saving.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[xx]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

* * * Next Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #X | X |  |  |  |  |  |

* * * Next Change * * * *

## 6.X	Solution #X: Functional Architecture to Support Energy Saving

### 6.X.1	Solution Description

#### 6.X.1.0	General

There are common requirements from different application enabler services to support energy saving, for example, energy-related data collection and processing (combination/aggregation), information/status monitoring, and energy saving assistance. Enhancements to the application enablement layer architecture are needed to introduce common functionalities to fulfil these common requirements.

The following clauses specify generic functional model of ESE (Energy Saving Enablement) for support application layer energy saving services.

#### 6.X.1.1	Functional Model

##### 6.X.1.1.1	General

The functional architecture enhancement for the ESE (Energy Saving Enablement) is based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [xx]. It is organized into functional entities to describe an architecture enhancement which addresses the support for energy saving aspects for vertical applications.

##### 6.X.1.1.2	On-Network Functional Model

Figure 6.X.1.1.2-1: On-network functional model

Figure 6.X.1.1.2-1 illustrates the on-network functional model with ESE. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. The SEAL functional entities with ESE function on the UE and the server are grouped into ESE client(s) and ESE server(s) respectively. The ESE consists of a common set of services (e.g. Energy-related data collection and processing (combination/aggregation), Energy-related information/status monitoring, energy saving assistance) and reference points. The ESE offers its services to the vertical application layer (VAL).

The ESE client(s) communicates with the ESE server(s) over the ESE-UU reference points. The ESE client(s) provides the service enabler layer support functions to the VAL client(s) over ESE-C reference points. The VAL server(s) communicate with the ESE server(s) over the ESE-S reference points. The ESE server(s) may communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system.

Editor’s Note:	Whether a standalone architecture for energy saving enablement or integrate energy saving functionality with the existing SEAL enablers will be decided during solution evaluation and conclusion stage of the study phase.

Editor’s Note:	Whether energy-related data collection via new energy saving enablement service/functionality or via existing enablement service (e.g. A-DCCF) is FFS.

#### 6.X.1.2	Functional Entities Description

##### 6.X.1.2.1	General

The ESE functional entities with energy saving assistance and management functions are described in the following subclauses.

##### 6.X.1.2.2	ESE client

The ESE client functional entity acts as the application client supporting energy-related data collection, and energy-related information/status monitoring, interacting with the VAL client to obtain such energy-related data/information. It also interacts with the ESE server.

##### 6.X.1.2.3	ESE server

The ESE server functional entity provides for energy saving assistance and management supported within the vertical application layer.

#### 6.X.1.3	Reference Points Description

##### 6.X.1.3.1	General

The reference points for the functional model for assistance and management of energy saving are described in the following subclauses.

##### 6.X.1.3.2	ESE-UU

The interactions related to energy saving assistance and management functions between the ESE client and ESE server are supported by ESE-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [2] and 3GPP TS 23.501 [3].

##### 6.X.1.3.3	ESE-S

The interactions related to energy saving assistance and management functions between the VAL server(s) and the ESE server are supported by ESE-S reference point.

##### 6.X.1.3.4	ESE-C

The interactions related to energy saving assistance and management functions between the VAL client(s) and the ESE client(s) within a VAL UE are supported by ESE-C reference point.

### 6.X.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***