3GPP TSG-SA WG6 Meeting #69	S6-254740

Wuhan, China 13th – 17th October 2025	(revision of S6-254657, S6-254121)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Use case Crowd Counting Video Analytics

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

This contribution provides the Realisation over SA6 Frameworks for the proposed use case on Crowd Counting Video Analytics text for the Guidelines for 3GPP Application Enablement usage (AppEnable_EXT) technical report.

2. Reason for Change

This contribution proposes text for a Realisation over SA6 Frameworks for clause 5 of 3GPP TR 23.947.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.1.0.

* * * First Change * * * *

### 5.1.3	Use case Realisation over Application Enablement Frameworks

#### 5.1.3.1	General

From the developer’s point of view, it is essential to understand which entity handles which tasks in the EDGEAPP system. The AC is deliberately kept simple: it talks only to the EEC, and all network complexity is abstracted away. When the AC is designed, only the calls to the EEC’s APIs need to be implemented to leverage the EDGEAPP capabilities. Everything else — serving EES discovery, ECS interaction, and service API resolution via CAPIF — is handled transparently by the edge system. Section 5.1.3.2, though, explain the end-to-end flows for developers to understand the full system.

For this use case, we are considering centralized CAPIF core function deployment option as described in Annex A.5.3 of 3GPP TS 23.558 [4].

#### 5.1.3.2	AC Discovering the Serving EES

The EEC running in the UE, needs to discover its EES. The EEC can learn which EES to register with through multiple mechanisms:

- Pre-provisioned in the UE/application profile.

- DNS-based discovery, using operator domain naming conventions.

- Network-provisioned, delivered at onboarding or during mobile attachment.

- Bootstrap via ECS, where the EEC queries ECS to obtain the correct serving EES.

#### 5.1.3.3	CAPIF roles of EDGEAPP entities

While AC does not interact directly with CCF, it is important to understand the roles that each EDGEAPP entity plays in relation to CAPIF to consume APIs. In this use case, the EEC behaves as a CAPIF API Invoker. The EEC needs to discover EAS APIs and obtain OAuth tokens to consume them. In particular, the EEC queries the CCF to discover the Video Analytics service API exposed by the EAS.

The EEC then exposes the resolved API base URI and parameters to the AC via EDGE-5 (Eeec_EASDiscovery). The AC uses this information to send its video stream (i.e., application data traffic) directly to the EAS and expects to receive analytics results based on that. EES acts as an API Invoker consuming APIs from ECS and is an API provider as it exposes EES APIs to ECS. EAS acts as an API Invoker consuming APIs from EES and is an API provider as it exposes EAS APIs plus Application specific APIs. ECS acts as API Provider as it exposes APIs to EES.

With these considerations, the next section provides an end-to-end view with step-by-step flows describing all EDGEAPP and CAPIF entities and how they interact based in the Developers´ use of EDGE-5 APIs between AC and EEC.

### 5.1.4	Use case Flows

Editor’s note: Appropriate “Focus for Application developer” highlighting, if any, in the figures in this clause is FFS.

#### 5.1.4.1	General

From a developer’s perspective, the use case flows describe how an Application Client (AC) interacts with its local Edge Enabler Client (EEC) to request services, discover the right Edge Application Server (EAS), and sustain service continuity when the UE moves between edge zones. The AC is abstracted from all network-level complexity; it only consumes the EDGE-5 APIs provided by the EEC. Behind the scenes, the EEC engages with the EES, ECS, and CAPIF to resolve service endpoints, discover APIs, and coordinate relocation. Each step below outlines the function, rationale, and developer implications.

#### 5.1.4.2	Step 1: AC Registration & Provisioning

The first step is for the AC to register with the EEC so that it can start consuming services. The developer must ensure the AC calls the Eeec_ACRegistration API. If the EEC already aware of its serving EES (through provisioning, DNS discovery, or cached state), it registers directly as depicted in Figure 5.1.4.2-1. Otherwise, the EEC bootstraps via the ECS to discover the serving EES as depicted in Figure 5.1.4.2-2. This step establishes the control context needed for subsequent service requests.

Figure 5.1.4.2-1: AC registration in EEC (option 1)

Alternatively, EEC can discover a suitable EES from the ECS.

Figure 5.1.4.2-2: AC registration in EEC (option 2)

#### 5.1.4.3	Step 2: EAS Discovery

Once registered, the AC needs to know which EAS can provide the video analytics service. The developer calls Eeec_EASDiscovery, and the EEC queries the EES to find suitable EAS instances in the serving zone. The response includes candidate EAS endpoints, which the EEC returns to the AC via callback. The flow is shown in Figure 5.1.4.3-1.

At this stage, the AC learns where to send its data, but not yet how to use the service API.

Figure 5.1.4.3-1: EAS Discovery

#### 5.1.4.4	Step 3: CAPIF API Discovery

The AC now requires the actual API contract to interact with the selected EAS. The developer does not handle CAPIF directly; instead, the EEC calls CAPIF_Discover_Service_API towards the CCF. CAPIF responds with the API base URI, supported operations, and authorization details. The EEC then informs the AC via a response callback. The flow is shown in Figure 5.1.4.4-1.

The developer can now use this endpoint for direct service invocation.

Figure 5.1.4.4-1: CAPIF API Discovery

EEC notifies the API endpoint information obtained from CAPIF.

##### 5.1.4.5	Step 4: Service Invocation

With the endpoint and API details, the AC begins sending its video stream directly to the EAS. From the developer’s perspective, this involves implementing the client logic that captures frames and invokes the Video Analytics service API as specified by CAPIF. The EAS processes the video in real time and returns crowd counting results, which the AC can render or act upon. The flow is shown in Figure 5.1.4.5-1.

Figure 5.1.4.5-1: Video Analytics API Consumption

#### 5.1.4.6	Step 5: Mobility with Application Context Relocation

The EEC does not detect mobility itself. Mobility/zone-change is determined in the network and conveyed to the EES (e.g., via operator control-plane and exposure functions). The EES then informs the EEC over EDGE-1 using the service continuity/mobility procedures. The EEC translates that into the local EDGE-5 notification the AC can consume. From the developer’s perspective, they only need to:

-	ensure the AC has subscribed (e.g., Eeec_Services_Subscribe) to mobility/ACR notifications, and

-	handle updateNotificationCallback(mobilityEvent: zoneChange → Zone2) to gracefully pause/buffer and wait for the new endpoint before resuming the stream.

Once the EEC has notified the AC of a mobility event (zone change), the next stage is to relocate the application context. The EES is the anchor for this process. When it detects that the UE should be served by another edge zone, it coordinates with the ECS to identify the target EES/EAS. The EEC can either trigger the ACR procedure explicitly (via Eeec_ACRTrigger) or let the EES manage it automatically.

NOTE: Each edge zone corresponds to an Edge Data Network (EDN) as defined in 3GPP TS 23.558 [4]. An EDN represents the localized network domain that hosts the edge components (EES, EAS, and supporting UPF) providing low-latency application services near the UE. From an application-enablement perspective, the edge zone is the logical area in which an Application Client (AC) is served by a specific EES and its associated EAS instances. When the UE moves to another edge zone, this corresponds to a change of serving EDN at the network level. The EES anchors the control context of each EDN and coordinates the Application Context Relocation (ACR) process to maintain service continuity across edge zones.

From the developer’s perspective, the AC does not implement relocation signalling. It only needs to:

-	handle the notification callback (from Step 4),

-	optionally request relocation via Eeec_ACRTrigger, and

-	be ready to consume the new endpoint information once relocation is complete.

The state transfer between EAS 1 and EAS 2, as well as the context push between EESs, is handled by the edge system.

Figure 5.1.4.6-1: Mobility and Application Context Relocation

#### 5.1.4.7	Step 6: API Update & Resume Streaming

After relocation, the EEC refreshes API discovery with CAPIF to retrieve the new endpoint for EAS 2. The AC receives this updated endpoint via notification callback. The developer must handle this event by re-establishing the video stream to EAS 2 using the new service API. From the AC’s perspective, this looks like a service update rather than a failure, maintaining continuity of the analytics.

Figure 5.1.4.7-1: API Update & Resume Streaming

At the end of the procedure, the Application Client (AC) has successfully transitioned its session from EAS 1 in Zone 1 to EAS 2 in Zone 2. The Application Context Relocation ensured that service state was preserved across edge zones, while the updated API discovery via CAPIF allowed the AC to obtain the new endpoint seamlessly. From the developer’s perspective, the AC continued to stream video to the analytics service with minimal disruption — first to Zone 1’s EAS and, after mobility and relocation, directly to Zone 2’s EAS. This demonstrates how the EDGEAPP framework hides the complexity of mobility and service continuity, enabling the developer to focus on application logic rather than network details.

* * * End of Changes * * * *