3GPP TSG-SA WG6 Meeting #69	S6-254717

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254220, 4397

Source:	Ericsson, China Mobile

Title:	Updates to Solution #17 on Enhancement of AIMLE to support model inference

Spec:	3GPP TR 23.700-83 v1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

The Solution #17 on Enhancement of AIMLE to support model inference, as defined in TR 23.700-83 v1.0.0 aims to provide support for ML model inference enabling UE-edge collaborative inference, where a VAL UE (with an AIMLE client) and a nearby edge AIMLE server jointly execute an ML model inference. The inference is partitioning such that the UE performs the privacy-sensitive and the ultra low-latency portion of the model and the AIMLE/ADAE server processes the inference starting on the remaining layers to generate the final output.

This solution can be enhanced by enabling the AIMLE/ADAE server to negotiate with the AIMLE/ADAE client at the UE to determine the optimal ML split configuration, in cases where the UE does not provide the inference partitioning information. Furthermore, the information flows are added describing the information elements required to perform the ML split inference task.

Therefore, this paper proposes enhancing Solution #7 of TR 23.700-83 v1.0.0 by adding the elements described in the previous paragraph.

2. Reason for Change

To enhance the solution #7 of TR.23.700-83 v1.0.0.

3. Conclusions

This paper proposes to enhance the solution #1 of TR 23.700-83 v1.0.0 on Enhancement of AIMLE to support model inference.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.0.0.

* * * First Change * * * *

## 7.17	Solution #17: Enhancement of AIMLE to support model inference

### 7.17.1	Solution description

This proposal addresses Key Issue #1, Open Issue 5. The proposed solution enables UE-edge collaborative inference, where a VAL UE (with an AIMLE client) and a nearby edge AIMLE server jointly execute an ML model inference. In this approach, the inference pipeline is partitioned: the VAL UE performs the privacy-sensitive and ultra-low-latency portion of the model locally, while the computation-intensive remainder is offloaded to an edge AIMLE server. By splitting the inference execution between the device and edge, the solution meets strict latency requirements and keeps sensitive raw data on the device, while leveraging edge computing for heavy processing.

Although the proposed UE-edge collaborative inference procedure shares the general concept of distributed inference with the pipeline solution defined in Rel-19, there are fundamental differences in terms of architecture, coordination entities, and applicable scenarios. The current proposal specifically targets collaboration between the AIMLE client (residing in the UE) and the edge AIMLE server, which hosts part of the ML model and provides inference service functions. This solution introduces a new set of interactions, including model selection, authorization, intermediate data upload, and result feedback, none of which are covered by the Rel-19 pipeline solution.

This clause describes the procedure for supporting collaborative inference between AIMLE client and edge AIMLE server.

Figure 7.17.1-1 illustrates the procedure for AIMLE server to support collaborative inference between AIMLE client and edge AIMLE server.

Pre-condition:

1	The ML model needed to be used for this ML model inference is available by the AIMLE server.

Figure 7.17.1-1: ML model collaborative inference

1.	The AIMLE client initiates the inference service by sending a request to the edge AIMLE server. This request indicates that the UE intends to perform an ML model inference collaboratively (split between UE and edge AIMLE). The AIMLE client identifies the target ML model or inference task and that an edge AIMLE is needed to complete the inference. This request consists of the ML model identifier, it may carry the inference partitioning info, an inference session ID or a request for the edge to allocate resources. The information elements presented in the request are described in Table 7.17.1.1.1-1.

2.	The edge AIMLE server performs an authorization check. It also ensures the specified model (or sub-model for the later layers) is available. If the client only provided a task type (no specific model ID), the AIMLE server selects an appropriate ML model for the task using the AIMLE model discovery/selection procedure (per 3GPP TS 23.482 clause 8.11.3).

3.	Upon selecting the ML model for the inference, the edge AIMLE server negotiates with the AIMLE client the desired ML model split configuration.  If the inference partitioning info is included in the ML model collaborative inference request message, the AIMLE server splits the ML model according to the inference partitioning info provided by the AIMLE client. Otherwise, the AIMLE server decides the ML split. In this case, if the AIMLE server contains all the necessary information to determine the appropriate ML split configuration, then the AIMLE server will make the decision. Otherwise, the AIMLE server interacts with the AIMLE client to collect the required information to make the ML split configuration decision.

4.	If the request is authorized and a model is ready, the edge AIMLE server replies with a ML Model collaborative inference response, the Inference Setup Response acknowledging readiness, otherwise a failure response indicates the reason for failure. The information elements presented in the message are described in Table 7.17.1.1.2-1

5.	Upon confirmation, the AIMLE client proceeds to execute the first part of the ML model locally. The AIMLE client runs the input data (provided by the VAL application) through the initial model layers (up to the agreed split point).

6.	Intermediate data upload: The AIMLE client sends the intermediate inference output to the edge AIMLE server. This intermediate data, such as a feature vector output from the last locally executed model layer (e.g., FeatureX), serves as input to the next stage of inference. The message includes an identifier to associate it with the earlier setup (e.g. the session ID or pipeline ID from step 2) so that the edge knows which model/task this data corresponds to. The information elements presented in the message are described in Table 7.17.1.1.3-1.

7.	The Edge AIMLE server uses the received intermediate data as input to the remaining portion of the ML model. It runs the model's subsequent layers or inference function to produce the final result.

8.	The edge AIMLE server sends a collaborative inference result response message back to the AIMLE client. This response contains the final inference result (for example, the predicted class label and confidence score, or other output data of the model). It also can include the ML model ID that was used, a status code indicates success or failure, cause value, etc. The information elements presented in the request are described in Table 7.17.1.1.4-1.

#### 7.17.1.1	Information Flows

##### 7.17.1.1.1 	ML model collaborative inference request

Table 7.17.1.1.1-1 describes the information elements included in the ML model collaborative inference request message sent by the AIMLE/ADAE client to the edge AIMLE/ADAE server.

Table 7.17.1.1.1-1: ML model collaborative inference request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identify | M | The identifier of the requestor (e.g. identifier of the AIMLE client or the VAL UE). |
| ML model ID | O (NOTE 1) | The ML model identifier |
| Inference task | O (NOTE 1) | Indicates the inference task |
| >Inference task type | O (NOTE 2) | Description of the inference task type to be performed (e.g., prediction, classification) |
| >Inference task ID | O (NOTE 2) | The ID of the inference task |
| Inference partitioning info | O | The partitioning information of the ML model specifies how many layers are to be executed on the UE and how many layers are to be executed on the server |
| Inference session ID | O (NOTE 3) | The indentifier of the inference session requested by the UE |
| Allocate resources request | O (NOTE 3) | It indicates the request for resource allocation |
| NOTE 1: Only one of the IEs shall be provided. NOTE 2: Only one of the IEs shall be provided. NOTE 3: Only one of the IEs shall be provided. |  |  |

##### 7.17.1.1.2	ML model collaborative inference response

Table 7.17.1.1.2-1 describes the information elements included in the ML model collaborative inference response message sent by the AIMLE server to the edge AIMLE client.

Table 7.17.1.1.2-1: ML model collaborative inference response

| Information element | Status | Description |
| --- | --- | --- |
| Inference Setup Response acknowledging readiness | O (NOTE) | Contains all the necessary information for the ML split inference setup |
| Failure response | O (NOTE) | The error message in case of failure |
| NOTE: Only one of the IE shall be provided. |  |  |

##### 7.17.1.1.3 	Intermediate inference output

Table 7.17.1.1.3-1 describes the information elements included in the intermediate inference output message sent by the AIMLE client to the edge AIMLE server.

Table 7.17.1.1.3-1: Intermediate inference output

| Information element | Status | Description |
| --- | --- | --- |
| Intermediate output | M | Intermediate data (e.g., feature vector output from the last locally executed model layer) |
| ML model ID | O (NOTE 1) | The ML model identifier |
| Inference task | O (NOTE 1) | Indicates the inference task |
| >Inference task type | O (NOTE 2) | Description of the inference task type to be performed (e.g., prediction, classification) |
| >Inference task ID | O (NOTE 2) | The ID of the inference task |
| Inference partitioning info | O | The partitioning information of the ML model specifies how many layers are to be executed on the UE and how many layers are to be executed on the server. |
| Inference session ID | O | The identifier of the inference session requested by the UE |
| NOTE 1: One of the IE shall be provided. NOTE 2: One of the IE shall be provided. |  |  |

##### 7.17.1.1.4 	ML Model collaborative inference result response

Table 7.17.1.1.4-1 describes the information elements included in the ML model collaborative inference result response sent by the AIMLE server to the edge AIMLE client.

Table 7.17.1.1.4-1: ML model collaborative Inference result response

| Information element | Status | Description |
| --- | --- | --- |
| Inference output | M | The final inference output (e.g., the predicted class label and confidence score, or other output data of the model) |
| Model ID | O | The identifier of the Model and contains the information necessary for the server to identify the correct layer of the Model to continue the inference execution |
| Status code | O | Indicates success or failure, cause value, etc. |

### 7.17.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.17.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.17.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *