3GPP TSG-SA WG6 Meeting #69	S6-254756

Wuhan, China 13th – 17th October 2025	(Revision of S6-254680, S6-254049)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on <Solution for Key issue#2: sensing based DAA service >

Spec:	3GPP TR 23.700-15 V0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Lei Chen(chenlei79@huawei.com)

1. Introduction

This contribution proposes a solution of sensing based DAA service for key issue#2.

2. Reason for Change

In TR 23.700-15, there are the following open issues for Key issue #2: Enhance UAV service utilizing sensing results:

-	How to enhance the UAS application enabler to support enhanced trajectory tracking for UAVs utilizing the sensing results.

-	How to enhance the UAS application enabler to support enhanced DAA service/functionalities for UAVs utilizing the sensing results.

This pCR is to handle the second open issue.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15.

* * * First Change * * * *

6.x	Solution #x: Sensing based DAA service

6.x.1	Solution description

This solution intends to solve the second open issue of key issue #2, i.e. enhance DAA service by utilizing sensing results. This solution is based on the procedures in clause 7.7.2 of 3GPP TS 23.255.

6.x.2	Procedures

#### 6.x.2.1	Configuration of sensing based DAA policies to the UAE layer

##### 6.x.2.1.1	Management of DAA support configuration

Figure 6.x.2.1.1-1 illustrates the DAA support management procedure where the UAE server receives an application request for managing the DAA application policy from the UAS application specific server.

Pre-condition:

1.	The UAV has received its UAS ID from the UAS application specific server.

2.	The UAV has performed the UAS UE registration procedure.

Figure 6.x.2.1.1-1: DAA support management procedure

1.	The UAS application specific server sends to the UAE server a DAA support management request. The request includes the UAV (UAE client) identifier and the DAA application policy. If the procedure is for LDGS assisted DAA, then the request may include a list of UAE client identifiers (LDGS IDs) and/or geographical area so UAE server may select the relevant UAE client(s). The DAA application policy includes parameters for the control of LDGS assisted DAA. DAA application policy could includes parameters for the control of sensing assisted DAA.

2.	The UAE server shall send to the UAS application specific server a DAA support management response with a positive or negative acknowledgement of the request.

3.	The UAE server shall timestamp and store the DAA application policy and execute the DAA configuration according to clause 7.7.2.1.2 and sensing based DAA support configuration procedure according to clause 6.x.2.1.2 if DAA application policy include parameter for the control of sensing assisted DAA.

4.	After execution of DAA configuration, the UAE server shall send a DAA support management complete to the UAS application specific server.

##### 6.x.2.1.2	Sensing based DAA support configuration procedure

Figure 6.x.2.1.2-1 illustrates the sensing based DAA support configuration procedure. This procedure enables the configuration of the UAE server, based on a request from UAS application specific server to configure the sensing based DAA application policy.

Pre-conditions:

1.	UAE server has performed the DAA support management procedure according to clause 6.x.2.1.1.

Figure 6.x.2.1.2-1: sensing based DAA support configuration procedure

1.	The UAE server obtains and initiates tracking the UAV location from the location management server as speicified in 3GPP TS 23.434.

2.	The UAE server determines the dynamic area for sensing based on the location of the UAV and the sensing based DAA application policy.

3.	The UAE server sends request and obtain sensing results of the area determined in Step 2.

4.	The UAE detects UAVs in approching based on the obtained sensing results and sensing based DAA application policy.

Editor’s Note: how to request and obtain sensing results depends on SA2

#### 6.x.2.2	Sensing based DAA event reporting

Figure 6.x.2.2-1 illustrates the procedure of sensing based DAA event reporting.

Pre-conditions:

1.	UAE server has detected UAVs in approaching as specified in 6.x.2.1.2.

Figure 6.x.2.2-1: Sensing based DAA event reporting

1.	The UAE server shall send a sensing based DAA event information to the UAS application specific server with information about one or more UAVs in approaching as detected and the corresponding timestamp.

2.	The UAS application specific server provides a sensing based DAA event information acknowledge to the UAE server.

Editor’s Note 1: UAE server how to consider sensing based detection result and other detection methods jointly to determine DAA event is FFS

Editor’s Note 2: The message name of sensing based DAA event information report depends on what information is used to trigger the report and it is FFS