3GPP TSG-SA WG6 Meeting #69	S6-254781

Wuhan, China 13th – 17th October 2025	(revision of S6-254684, S6-254088)

Source:	InterDigital

Title:	Solution for KI #3: Use of sensing results for spatial maps

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Quang Ly <Quang.Ly@InterDigital.com>

1. Introduction

This pCR proposes a new solution for KI#3 on use of sensing results for spatial maps.

2. Reason for Change

The contribution proposes a solution to address Key Issue #3, open issues #2 and #4. The solution proposes enhancements to both the create and update spatial map procedures in 3GPP TS 23.437 by providing a sensing service policy to the SM server. The SM server uses information in the policy to trigger sensing service.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1. 0.

* * * First Change * * * *

## Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #x |  |  | X |  |

* * * Next Change * * * *

## 6.x	Solution #x: Sensing results for spatial maps

### 6.x.1	Solution description

This solution addresses Key Issue #3, open issues #2 and #4 on how the sensing enabler layer can support SM server with creating a spatial map and to associate a sensing result with an object in a spatial map. The solution proposes enhancements to Create spatial map and Update spatial map procedures in 3GPP TS 23.437 [r23437].

The enhancments enable a VAL server or SM client to specify a sensing service policy when requesting to create or update a spatial map. The SM server uses the information in the sensing service policy to send requests to a sensing service to obtain objects for the area of interest. Within the sensing service policy, there are sensing configuration parameters the SM server can use for sending the requests to the sensing service. An Object label classification policy is provided to enable the SM server to label sensing results received from the sensing service with object IDs.

### 6.x.2	Procedures

#### 6.x.2.1	Impact to Create spatial map procedure

The Create spatial map procedure and information flows in 3GPP TS 23.437 [r23437] are enhanced (highlighted in bold italics) as follows.

#### 9.3.1.2 Procedure Figure 9.3.1.2-1 depicts the procedure for creating a spatial map. For the request from VAL server or SEAL SM client, as a spatial map consumer, SEAL SM server creates a spatial map. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. three dimensional space of area of interest, objects). Figure 9.3.1.2-1: Create spatial map procedure 1) The requestor (e.g., VAL server or SEAL SM client) sends a request message to the SEAL SM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as allowed entities list defining which entities are permitted to discover and access the spatial map. The request may include spatial map layering information parameters and may also include augmented layer information that can be requested with the spatial map. The request may include a required media format for the spatial map. A sensing service policy may be provided to configure the SM server to use a sensing service to obtain object information for the spatial map. NOTE: SEAL SM server can obtain augmented layer information and can provide the obtained augmented layer information to the requestor in the response message. 2) The SEAL SM server authorizes the requestor, and validates the request. The SEAL SM server produces a requested spatial map using layering information and processed sensor data, according to the requested spatial map media format. The SEAL SM server does not create a spatial map if the requested spatial map media format is not supported and can create a spatial map based on local configuration if no spatial map media format is included in the request. a) If the augmented layer information indicates to include VAL UE information, the SEAL SM server fetches the list of VAL UEs in the area of interest from LM server using clause 9.3.10 of 3GPP TS 23.434 [4] and/or from NEF as specified in 3GPP TS 23.502 [5]. b) If the augmented layer information indicates to include spatial anchors information within area of interest, the SEAL SM server fetches the list of spatial anchors in the area of interest as specified in clause 8.3.4. c) If a sensing service policy is provided, the SM server triggers a sensing service to obtain object information using information in the sensing service policy. NOTE: Processed sensor data is stored at the VAL layer (e.g., in database at VAL layer) or within the SM server (e.g., when sensing service policy is provided). When processed sensor data is stored at the VAL layer, the SEAL SM server gets access to such database to create a spatial map via application specific way which is out of scope. When processed sensor data is stored within the SM server, the SM server will use the sensing service policy to configure and manage sensing service(s) to obtain object information for the spatial map. ML models are used to derive object information from sensing results and the object information is saved within the SM server. Editor’s Note: The procedure of triggering of sensing service by the SM server is FFS. 3) The SEAL SM server sends response message to the requestor with a indication of success, in-progress or fail. If the requested spatial map has been created sussesfully, the response message includes assigned spatial map ID and information which includes three-dimensional space defined by the spatial map and can include a list of spatial map layers with their corresponding layer ID, objects belong to the layer, etc.. If the response indicates "in-progress", an assigned spatial map ID is included in the message. Further, if the response indicates "in-progress" and the requestor has not provided a notification target address, then the requestor subscribes for spatial map event to receive notification when the spatial map is created. Otherwise, the response includes an indication of failure and may include a reason for the failure and indication of supported spatial map media formats if the failure is related to an unsupported spatial map media format. 4) If the response indicates "in-progress", the SM server detects when the SEAL SM server has sufficient information to map all the objects related to area of interest and the spatial map is ready to provide service (e.g. employed for localization). If the requestor has provided a notification target address in the request or has subscribed for spatial map events, the SEAL SM server notifies the requestor with the spatial map ready notification message. The notification includes parameters as specifed in Table 9.3.6.3.3-1. #### 9.3.1.3 Information flows ##### 9.3.1.3.1 Create spatial map request Table 9.3.1.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SM server for create spatial map request. Table 9.3.1.3.1-1: Create spatial map request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor (e.g., VAL server or VAL User or VAL UE). Security credentials; M; The security credentials of the requestor.. Application service ID; M; ID of the requesting VAL application service. Area of interest; M; Three-dimensional area information to produce the spatial map. Notification Target Address; O; Notification target address (e.g. URL, URI, IP) where the notifications should be sent.. Spatial map media format; O; The media format requested for the spatial map.. Information to include; O; Information to be included in the spatial map. > Allowed entities list; O; Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map. > List of spatial map layers; O; List of spatial map layer information. Each element includes the information described below.. >> Spatial map layer information; O; Information to specify the corresponding spatial map layer specific information. > Augmented layer information; O; List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_USERS; - SPATIAL_ANCHORS.. Sensing service policy; O; A policy to provide configuration parameters for the SM server to trigger sensing service to obtain object information for the spatial map. The contents of the policy is described in Table 9.3.1.3.1-y.. NOTE 1: For the current release, the spatial map layer information is implementation specific and out of scope. Table 9.3.1.3.1-y: Sensing service policy Information element; Status; Description. Sensing service policy ID; M; The identifier of the sensing service policy.. Sensing service provider IDs; O; The identifier of the sensing service providers the SM server should use to trigger sensing service.. Sensing area; O; Three-dimensional area information of the sensing area.. Sensing resolution; O; A resolution to provide to the sensing service.. Sensing schedule; O; A schedule to trigger sensing service.. Sensing refresh interval; O; An interval to refresh SM object information provided by a sensing service.. Sensing output requirements; M; A listing of object characteristics required from the sensing service. Object characteristics describes the size, shape, orientation, speed, and absolute and relative location of an object obtained from sensing service.. Object label classification policy; O; A policy to assist with the labeling of sensing results obtained from the sensing service. The policy provides e.g. a list of ML models that derives object information from sensing results received by the SM server.. NOTE 2: Sensing service policy values are within the range defined in Table 9.3.1.3.1-1. Editor's Note: Whether the Object label classification policy IE is included in the sensing service policy is FFS.

#### 6.x.2.2	Impact to Update spatial map procedure

The Update spatial map procedure and information flows in 3GPP TS 23.437 [r23437] are enhanced (highlighted in bold italics) as follows.

### 9.3.4 Update spatial map #### 9.3.4.1 General The update spatial map procedure enables the consumers to request the SEAL SM Server to update a spatial map. #### 9.3.4.2 Procedure Figure 9.3.4.2-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server updates the spatial map as requested. Figure 9.3.4.2-1: Update spatial map procedure 1) The requestor (e.g., VAL server or SEAL SM client) sends a request message to the SEAL SM server to update a spatial map. The request includes requestor ID, security credentials, spatial map ID, information on what to update such as updated spatial map layering information and updated spatial map coverage area. As a part of update, the request may include augmented layer information (e.g., VAL users, spatial anchors). The request may include a list of UE-object association information for the SEAL SM server to identify associations between UEs and objects in a spatial map. A sensing service policy may be provided to configure the SM server to use a sensing service to obtain object information for the spatial map. 2) The SEAL SM server authorizes the requestor and validates the request. The SEAL SM server updates the spatial map as requested. If a sensing service policy is provided, the SM server triggers a sensing service to obtain object information with the information in the sensing service policy. Editor’s Note: The procedure of triggering of sensing service by the SM server is FFS. 3) The SEAL SM server sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID. SEAL SM server may include the requested augmented layer information in the response message. #### 9.3.4.3 Information flows ##### 9.3.4.3.1 Update spatial map request Table 9.3.4.3.1-1 descibes the information elements for update spatial map request from VAL server or SEAL SM client to SEAL SM server. Table 9.3.4.3.1-1: Update spatial map request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor (e.g., VAL server or VAL User or VAL UE). Security credentials; M; The security credentials of the requestor.. Application service ID; M; ID of the requesting VAL application service. Spatial map ID; M; ID of the spatial map to update. Spatial map update information; M; The spatial map Information to update as described in described in Table 7.3.3.1-1. Only IEs which need to be updated are included.. > Augmented layer information; O; List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_UE; - SPATIAL_ANCHORS.. List of UE-object association information; O; List of UE-object association information to identify associations between UEs and objects in a spatial map as described in Table 7.3.3.x-1.. Sensing service policy; O; A policy to provide configuration parameters for the SM server to trigger sensing service to obtain object information for the spatial map. The contents of the policy is described in Table 9.3.1.3.1-y.. NOTE: At least one of the information elements shall be provided..

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 22.137, "Service requirements for Integrated Sensing and Communication; Stage 1".

[4] 		3GPP TS 23.255, "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows;".

[5]	3GPP TS 23.286, "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows; "

[6]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication (Release 19)".

[r23437]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial map and Spatial anchors".

* * * End of Change * * * *