3GPP TSG-SA WG6 Meeting #69	S6-254507

Wuhan, P.R.China, 13th – 17th October 2025	(revision of S6-254153)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on Update key issue#4

Spec:	3GPP TR 23.700-43 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed to update the key issue#4.

2. Reason for Change

(1) Currently the CCF obtains the service API status from the API management function as described in clause 6.3.6 and 6.4.8.

(2) From the management system,the AEF is regarded as the NEF instance or ECS/EES/EAS instance in SA5 specification 3GPP TS 28.541 and  3GPP TS 28.538.

The Managed[Edge]NFService defined by SA5 describes the service provided by the NEF  or edge entity. Each Managed[Edge]NFService has the adminstrativeState and the operationalState and they different combinations controls the active and inactive of the service status.

For NEF instance, the NEF Function IOC includes the managedNFProfile, where a heartBeatTimer is included indicating the time between two consecutive heart-beat messages from an NF Instance to the NRF defined in seconds. So in order to maintain the aviliability of AEF, such NEF instance can also maintain a heartbeat with the CCF  per node level.

For the EES/EAS/ECS case, similarly, the EASFunction contains the EASProfile, where an eASAvailabilityReportingPeriod indicates to the EES how often it needs to check the EAS's availability after a successful registration (See EAS Availability Reporting Period defined in clause 8.2.4 in TS 23.558 [2]). EAS instance can also maintain a heartbeat with the CCF per node level.

(3) for the statement “Of note, registration of AEF does not include time-to-live (TTL) information, so an AEF can be registered indefinitely whether they are up or not. This can lead to a CCF providing erroneous information to API Invokers regarding AEFs that are no longer active.” It is not fully correct as in SA5, e.g., in EASFunction, there is registrationExpiry IE to control the TTL of registration. And the

（4）From the API invoker point of view, it only cares the status of service API rather than the AEF status.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 v0.0.0.

* * * First Change * * * *

4.4	Key Issue #4: Service API and AEF operational status

4.4.1	Description

Currently, the CCF obtains the service API status (e.g., active, inactive) via the API management function as described in clause 6.3.6 and 6.4.8 of 3GPP TS 23.222, or via the APF publish and publish updated procedure.

However, in some scenarios the API Provider Domain is unable to provide the status of Service APIs as the connection between the CAPIF core function and the API provider domain functions is disrupted due to e.g., network issues. In other scenarios, CCF cannot be aware whether an AEF is operational or not where AEFs can be disconnected/unreachable or do not work properly. The consequence is that the CCF can be unaware of whether an AEF and the API(s) it offers are active and able to offer service in above scenarios. Of note, registration of AEF does not include time-to-live (TTL) information, so an AEF can be registered indefinitely whether they are up or not.

The implementation and deployment of API provider domain functions may impact the service API status availability to the CCF via APF or API management function when considering the connectivity. The AEF, APF, API management functions in the API provider domain can be implemented in different entities and deployed in different locations. E.g., multiple AEF function instance is distributed deployed and the APF and the API management function instance are individually deployed in a centralized location. In this case, the network connectivity between the CCF and the AEF is different from the network connectivity between the CCF and the APF or API management function. In another example, the AEF and APF may be implemented and deployed together but the API management function is deployed in different locations. In this case, the network connectivity between the CCF and the AEF+APF is different from the network connectivity between the CCF and API management function.

In clause 8.6.2.1 of 3GPP TS 23.222 [2] specifying the contents of the service API update request sent from the APF to CCF there is a note stating that "how to monitor service API status when the APF is unable to update service API status is not specified in this release".

Examples of how this issue has been solved in other SA6 specifications include implementation of a heartbeat mechanism between servers, which is further specified by SA5 in their specification. There are some similar mechanisms:

-	For the AEF is the NEF instance, the NEF Function IOC includes the managedNFProfile, where a heartBeatTimer is included indicating the time between two consecutive heart-beat messages from an NF Instance to the NRF defined in seconds.

-	For AEF is the the EES/EAS/ECS case, similarly, the EAS Function in SA5 specification contains the EASProfile, where an eASAvailabilityReportingPeriod indicates to the EES how often it needs to check the EAS's availability after a successful registration (See EAS Availability Reporting Period defined in clause 8.2.4 in TS 23.558 [2]).

Now the CAPIF supports the dynamic routing of service API invocation and topology hiding. In those cases, the API invocation is firstly received and handled by one AEF (AEF#1) acting as service communication entry point, then AEF#1 will forward it to the destination AEF (AEF#2) for handling service API invocation. During service API invocation stage, if the AEF(AEF#2) does not work well, then the problem becomes how to route the service API invocation to an AEF providing such service API and work well. i.e., the service API status on this AEF is in active status.

4.4.2	Open issues

-	This key issue will study approaches to address scenarios in which the API Provider Domain is unable to provide the status of its AEFs with associated Service APIs and whether and how to enhance the architecture and related functions to handle the scenarios in which the connection between the CAPIF core function and the API provider domain functions is disrupted due to network issues.

-	During service API invocation stage, if the one AEF does not work well, how to route the service API invocation to another AEF providing such service API and work well. i.e., the service API status on this AEF is in active status.