| 3GPP TS 36.133 V16.20.0 (2024-03) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.6	General	104

3.6.1	Applicability of requirements in this specification version	104

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	110

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	112

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	113

3.6.2	Applicability of requirements for EN-DC operation	114

3.6.3	Applicability of requirements for NE-DC operation	115

3.6.4	Applicability of requirements for NGEN-DC operation	116

4	E-UTRAN RRC_IDLE state mobility	116

4.1	Cell Selection	116

4.2	Cell Re-selection	116

4.2.1	Introduction	116

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	118

4.2.2.2	Void	119

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	119

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	121

4.2.2.5	Measurements of inter-RAT cells	123

4.2.2.5.1	Measurements of UTRAN FDD cells	123

4.2.2.5.2	Measurements of UTRAN TDD cells	125

4.2.2.5.3	Measurements of GSM cells	126

4.2.2.5.4	Measurements of HRPD cells	127

4.2.2.5.5	Measurements of cdma2000 1X	128

4.2.2.5.6	Measurements of NR cells	129

4.2.2.5.7	Measurements of NR cells subject to CCA	131

4.2.2.6	Evaluation of cell re-selection criteria	133

4.2.2.7	Maximum interruption in paging reception	133

4.2.2.8	void	134

4.2.2.9	UE measurement capability	134

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	134

4.2.2.10	Reselection to CSG cells	135

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	135

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	136

4.2.2.11	Void	137

4.2.2.12	Void	137

4.2.2.13	Void	137

4.3	Minimization of Drive Tests (MDT)	137

4.3.1	Introduction	138

4.3.2	Measurements	138

4.3.2.1	Requirements	138

4.3.3	Relative Time Stamp Accuracy	138

4.3.3.1	Requirements	138

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	138

4.3.4.1	 Requirements	138

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	139

4.3.5.1	Requirements for timeSinceFailure	139

4.4	MBSFN Measurements	139

4.4.1	Introduction	139

4.4.2	MBSFN RSRP measurements	139

4.4.3	MBSFN RSRQ measurements	139

4.4.4	MCH BLER measurements	140

4.5	Proximity-based Services	140

4.5.1	Introduction	140

4.5.2	Requirements	140

4.5.2.1	Interruptions with ProSe Direct Discovery	140

4.5.2.2	Interruptions with ProSe Direct Communication	140

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	140

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	141

4.6	Cell Selection and Re-selection Requirements for UE category NB1	141

4.6.1	Cell Selection	141

4.6.2	Cell Re-selection	141

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	142

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	143

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	144

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	145

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	147

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	147

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	149

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	150

4.6.2.7	Maximum interruption in paging reception in normal coverage	152

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	152

4.6.2.8	UE measurement capability	152

4.6.2.9	WUS receptions for NB1	152

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	153

4.6.3.1	Introduction	153

4.6.3.2	Requirements on UE synchronization for transmission using PUR	153

4.6.3.3	Requirements on TA validation for transmission using PUR	153

4.7	Cell Selection and Re-selection Requirements for UE category M1	154

4.7.1	Cell Selection	154

4.7.2	Cell Re-selection	154

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	154

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	154

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	155

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	156

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	158

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	159

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	160

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	160

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	160

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	161

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	162

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	165

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	166

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	167

4.7.2.3	WUS receptions for UE category M1	167

4.7.3	Channel quality report for UE Category M1 in idle mode	167

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	168

4.7.4.1	Introduction	168

4.7.4.2	Requirements on UE synchronization for transmission using PUR	168

4.7.4.3	Requirements on TA validation for transmission using PUR	168

4.8	Idle State Positioning Measurement Requirements for UE category NB1	169

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	169

4.8.1.1	RSTD Measurement Reporting Delay	171

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	171

4.8.2.1	RSTD Measurement Reporting Delay	173

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	173

4.8.3.1	RSTD Measurement Reporting Delay	174

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	175

4.8.4.1	RSTD Measurement Reporting Delay	176

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	177

4.8.5.1	Measurement Reporting Delay	178

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	178

4.8.6.1	Measurement Reporting Delay	179

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	180

4.8.7.1	Measurement Reporting Delay	181

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	181

4.8.8.1	Measurement Reporting Delay	182

4.9	Idle Mode CA Measurement	183

4.9.1	Introduction	183

4.9.2	Requirement	183

4.9.2.1	Detected cell requirement during state transition and Idle mode	183

4.9.2.2	Measurements of inter-frequency CA candidate cells	184

4.9.2.3	Measurements on serving cell	184

4A	E-UTRAN RRC_INACTIVE state mobility	185

4A.1	Cell Re-selection	185

4A.1.1	Introduction	186

4A.1.2	Requirements	186

4A.1.2.1	UE measurement capability	186

4A.1.2.2	Measurement and evaluation of serving cell	186

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	186

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	186

4A.1.2.5	Evaluation of cell re-selection criteria	186

4A.1.2.6	Maximum interruption in paging reception	186

4A.1.2.7	Measurements of inter-RAT NR cells	186

4A.2	Requirements for UE Category M1	186

4A.2.1	Introduction	186

4A.2.2	Cell Selection	186

4A.2.3	Cell Reselection	186

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	186

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	186

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	187

A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	187

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	188

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	188

4A.2.4	Channel quality report for UE Category M1 in idle mode	189

5	E-UTRAN RRC_CONNECTED state mobility	189

5.1	E-UTRAN Handover	190

5.1.1	Introduction	190

5.1.2	Requirements	190

5.1.2.1	E-UTRAN FDD – FDD	190

5.1.2.1.1	Handover delay	190

5.1.2.1.2	Interruption time	190

5.1.2.2	E-UTRAN FDD – TDD	192

5.1.2.2.1	(Void)	192

5.1.2.2.2	(Void)	192

5.1.2.3	E-UTRAN TDD – FDD	192

5.1.2.3.1	(Void)	192

5.1.2.3.2	(Void)	192

5.1.2.4	E-UTRAN TDD – TDD	192

5.1.2.4.1	Handover delay	192

5.1.2.4.2	Interruption time	192

5.1.2.5	E-UTRAN HD–FDD	194

5.1.2.5.1	Handover delay	194

5.1.2.5.2	Interruption time	194

5.1.2.6	E-UTRAN FDD – FDD conditional handover	195

5.1.2.6.1	Handover delay	195

5.1.2.6.2	Measurement time	196

5.1.2.6.3	Preparation time	196

5.1.2.6.4	Interruption time	196

5.1.2.7	E-UTRAN FDD – TDD conditional handover	197

5.1.2.8	E-UTRAN TDD – FDD conditional handover	197

5.1.2.9	E-UTRAN TDD – TDD conditional handover	197

5.2	Void	197

5.3	Handover to other RATs	197

5.3.1	E-UTRAN - UTRAN FDD Handover	197

5.3.1.1	Introduction	197

5.3.1.1.1	Handover delay	197

5.3.1.1.2	Interruption time	197

5.3.2	E-UTRAN - UTRAN TDD Handover	198

5.3.2.1	Introduction	198

5.3.2.2	Requirements	198

5.3.2.2.1	Handover delay	198

5.3.2.2.2	Interruption time	198

5.3.3	E-UTRAN - GSM Handover	199

5.3.3.1	Introduction	199

5.3.3.2	Requirements	199

5.3.3.2.1	Handover delay	199

5.3.3.2.2	Interruption time	199

5.3.4	E-UTRAN - NR FR1 Handover	200

5.3.4.1	Introduction	200

5.3.4.2	Handover delay	200

5.3.4.3	Interruption time	200

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	200

5.3.4A.1	Introduction	200

5.3.4A.2	Handover delay	201

5.3.4A.3	Interruption time	201

5.3.5	E-UTRAN - NR FR2 Handover	202

5.3.5.1	Introduction	202

5.3.5.2	Handover delay	202

5.3.5.3	Interruption time	202

5.4	Handover to Non-3GPP RATs	203

5.4.1	E-UTRAN – HRPD Handover	203

5.4.1.1	Introduction	203

5.4.1.1.1	Handover delay	203

5.4.1.1.2	Interruption time	203

5.4.2	E-UTRAN – cdma2000 1X Handover	204

5.4.2.1	Introduction	204

5.4.2.1.1	Handover delay	204

5.4.2.1.2	Interruption time	204

5.5	E-UTRAN Handover for Cat-M1 UEs	204

5.5.1	Introduction	204

5.5.2	Requirements in CEModeA	204

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	204

5.5.2.1.1	Handover delay	205

5.5.2.1.2	Interruption time	205

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	205

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	205

5.5.2.3.1	Void	206

5.5.2.3.2	Void	206

5.5.3	Requirements in CEModeB	206

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	206

5.5.3.1.1	Handover delay	206

5.5.3.1.2	Interruption time	206

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	207

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	207

5.6	Void	207

5.7	E-UTRAN DAPS Handover	207

5.7.1	Introduction	207

5.7.2	Requirements	207

5.7.2.1	E-UTRAN FDD – FDD	207

5.7.2.1.1	DAPS Handover delay	207

5.7.2.1.2	Interruption time	208

5.7.2.2	E-UTRAN FDD – TDD	208

5.7.2.3	E-UTRAN TDD – FDD	209

5.7.2.4	E-UTRAN TDD – TDD	209

6	RRC Connection Mobility Control	209

6.1	RRC Re-establishment	209

6.1.1	Introduction	209

6.1.2	Requirements	209

6.1.2.1	UE Re-establishment delay requirement	209

6.2	Random Access	210

6.2.1	Introduction	210

6.2.2	Requirements	210

6.2.2.1	Contention based random access	210

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	210

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	210

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	210

6.2.2.1.4	Void	210

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	210

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	210

6.2.2.2	Non-Contention based random access	211

6.2.2.2.1	Correct behaviour when receiving Random Access Response	211

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	211

6.2.3	Requirements for Cat-M1 UEs	211

6.3	RRC Connection Release with Redirection	211

6.3.1	Introduction	211

6.3.2	Requirements	211

6.3.2.1	RRC connection release with redirection to UTRAN FDD	211

6.3.2.2	RRC connection release with redirection to GERAN	212

6.3.2.3	RRC connection release with redirection to UTRAN TDD	212

6.3.2.4	RRC connection release with redirection to NR	213

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	213

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	215

6.4.1	Introduction	215

6.4.2	Requirements	215

6.5	RRC Re-establishment for NB-IoT UEs	215

6.5.1	Introduction	215

6.5.2	Requirements	215

6.5.2.1	UE Re-establishment delay requirement in normal coverage	215

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	216

6.6	Random Access for UE category NB1	216

6.6.1	Introduction	216

6.6.2	Requirements	216

6.6.2.1	Correct behaviour when receiving Random Access Response reception	217

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	217

6.6.2.3	Correct behaviour when receiving a NACK on msg3	217

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	217

6.6.2.5	Correct behaviour when contention Resolution timer expires	217

6.6.2.6	MSG3-based channel quality report for UE Category NB1	217

6.6.3	Requirements for NPRACH configuration	218

6.7	RRC Re-establishment for Cat-M1 UEs	218

6.7.1	Introduction	218

6.7.2	Requirements	218

6.7.2.1	UE Re-establishment delay requirement for CEModeA	218

6.7.2.2	UE Re-establishment delay requirement for CEModeB	219

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	219

6.8.1	Introduction	219

6.8.2	Requirements	220

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	220

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	220

6.9.1	Introduction	220

6.9.2	Requirements	220

7	Timing and signalling characteristics	221

7.1	UE transmit timing	221

7.1.1	Introduction	221

7.1.2	Requirements	222

7.2	UE timer accuracy	223

7.2.1	Introduction	223

7.2.2	Requirements	223

7.3	Timing Advance	223

7.3.1	Introduction	223

7.3.2	Requirements	223

7.3.2.1	Timing Advance adjustment delay	223

7.3.2.2	Timing Advance adjustment accuracy	224

7.4	Cell phase synchronization accuracy (TDD)	224

7.4.1	Definition	224

7.4.2	Minimum requirements	224

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	225

7.5.1	Introduction	225

7.5.2	eNodeB Synchronization Requirements	225

7.5.2.1	Synchronized E-UTRAN	225

7.5.2.2	Non-Synchronized E-UTRAN	225

7.6	Radio Link Monitoring	225

7.6.1	Introduction	225

7.6.2	Requirements	227

7.6.2.1	Minimum requirement when no DRX is used	227

7.6.2.2	Minimum requirement when DRX is used	227

7.6.2.3	Minimum requirement at transitions	229

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	229

7.6.2.5	Minimum requirement under IDC Interference	229

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	229

7.7.1	Introduction	229

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	230

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	231

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	232

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	233

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	234

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	235

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	236

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	236

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	236

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	237

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	238

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	239

7.7.14	SCell Activation Delay Requirement for Dormant SCell	239

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	241

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	241

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	243

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	243

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	245

7.8	Interruptions with Carrier Aggregation	247

7.8.1	Introduction	247

7.8.2	Requirements	247

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	247

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	247

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	247

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	248

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	248

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	248

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	248

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	248

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	249

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	250

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	250

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	250

7.8.2.13	Interruptions at SRS carrier based switching	251

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	252

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	252

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	252

7.8.2.17	Interruptions during CQI measurement on dormant SCell	252

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	253

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	253

7.8.2.20	Interruptions at SCell hibernation	253

7.8.2.21	Interruptions at direct SCell activation and hibernation	253

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	254

7.9.1	Introduction	254

7.9.2	Minimum Requirements for Interband Carrier Aggregation	254

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	254

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	255

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	255

7.10.1	Introduction	255

7.10.2	Requirements	255

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	255

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	255

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	256

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	256

7.11	Radio Link Monitoring for UE Category 0	257

7.11.1	Introduction	257

7.11.2	Requirements for FD-FDD and TDD	258

7.11.2.1	Minimum requirement when no DRX is used	258

7.11.2.2	Minimum requirement when DRX is used	259

7.11.2.3	Minimum requirement at transitions	259

7.11.3	Requirements for HD-FDD	260

7.11.3.1	Minimum requirement when no DRX is used	260

7.11.3.2	Minimum requirement when DRX is used	260

7.11.3.3	Minimum requirement at transitions	261

7.12	Interruptions with Dual Connectivity	261

7.12.1	Introduction	261

7.12.2	Requirements	261

7.12.2.1	Interruptions at PSCell addition/release	261

7.12.2.2	Interruptions at transitions between active and non-active during DRX	261

7.12.2.3	Interruptions at transitions from non-DRX to DRX	262

7.12.2.4	Interruptions at SCell addition/release	262

7.12.2.5	Interruptions at SCell activation/deactivation	262

7.12.2.6	Interruptions during measurements on SCC	263

7.12.2.7	Interruptions at SRS carrier based switching	263

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	264

7.13.1	Definition	264

7.13.2	Minimum requirements	264

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	264

7.14.1	Introduction	264

7.14.2	PSCell Addition Delay Requirement	264

7.14.3	PSCell Release Delay Requirement	265

7.15	Maximum Receive Timing Difference in Dual Connectivity	265

7.15.1	Introduction	265

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	265

7.16	Proximity-based Services	265

7.16.1	Introduction	265

7.16.2	Requirements	265

7.16.2.1	ProSe UE transmission timing	265

7.16.2.1.1	Serving cell or PCell as timing reference	266

7.16.2.1.2	SCell or non-serving cell as timing reference	266

7.16.3	Interruptions with ProSe	266

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	266

7.16.3.2	Interruptions at ProSe Direct Communication configuration	267

7.16.3.3	Interruptions during ProSe Direct Discovery	267

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	267

7.16.3.5	Interruptions during ProSe Direct Communication	268

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	268

7.16.4.1	Measurement and evaluation of selected cell	268

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	268

7.16.5	Selection / Reselection of ProSe relay UE	269

7.16.6	ProSe operation under deactivated SCell	269

7.17	Maximum Transmission Timing Difference in Dual Connectivity	269

7.17.1	Introduction	269

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	270

7.18.1	Introduction	270

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	270

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	270

7.19	Radio Link Monitoring for UE Category M1	270

7.19.1	Introduction	270

7.19.2	Requirements for FD-FDD and TDD CE mode A	271

7.19.2.1	Minimum requirement when no DRX is used	272

7.19.2.2	Minimum requirement when DRX is used	273

7.19.2.3	Minimum requirement at transitions	274

7.19.3	Requirements for HD-FDD with CE mode A	274

7.19.3.1	Minimum requirement when no DRX is used	275

7.19.3.2	Minimum requirement when DRX is used	275

7.19.3.3	Minimum requirement at transitions	276

7.19.4	Requirements for FD-FDD and TDD with CE mode B	276

7.19.4.1	Minimum requirement when no DRX is used	278

7.19.4.2	Minimum requirement when DRX is used	279

7.19.4.3	Minimum requirement at transitions	280

7.19.5	Requirements for HD-FDD with CE mode B	280

7.19.5.1	Minimum requirement when no DRX is used	280

7.19.5.2	Minimum requirement when DRX is used	281

7.19.5.3	Minimum requirement at transitions	282

7.20	UE transmit timing for NB-IoT	282

7.20.1	Introduction	282

7.20.2	Requirements	282

7.21	 UE timer accuracy for NB-IoT	283

7.21.1	Introduction	283

7.21.2	Requirements	283

7.22	Timing Advance for NB-IoT	283

7.22.1	Introduction	283

7.22.2	Requirements	283

7.22.2.1	Timing Advance adjustment delay	283

7.22.2.2	Timing Advance adjustment accuracy	283

7.23	Radio Link Monitoring for Category NB1 UE	283

7.23.1	Introduction	283

7.23.2	Requirements for Category NB1 UE	284

7.23.2.1	Minimum requirement when no DRX is used	284

7.23.2.2	Minimum requirement when DRX is used	285

7.23.2.3	Minimum requirement at transitions	285

7.24	UE transmit timing for Category M1	285

7.24.1	Introduction	285

7.24.2	Requirements	286

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	286

7.25.1	Definition	286

7.25.2	Minimum requirements	287

7.26	UE transmit timing for Category M2	287

7.26.1	Introduction	287

7.26.2	Requirements	287

7.27	UE timer accuracy for category M1	287

7.27.1	Introduction	287

7.27.2	Requirements	287

7.28	Timing Advance for Category M1	288

7.28.1	Introduction	288

7.28.2	Requirements	288

7.29	Interruptions requirements with FeMBMS	288

7.29.1	Introduction	288

7.29.2	Requirements	288

7.30	Numerology switching delay requirements with FeMBMS	288

7.30.1	Introduction	288

7.30.2	Requirements	288

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	288

7.31.1	Introduction	288

7.31.2	NR PSCell Addition Delay Requirement	289

7.31.3	NR PSCell Release Delay Requirement	289

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	290

7.31A.1	Introduction	290

7.31A.2	NR PSCell Addition Delay Requirement	290

7.31A.3	NR PSCell Release Delay Requirement	291

7.32	Interruptions with EN-DC	291

7.32.1	Introduction	291

7.32.2	Requirements	292

7.32.2.1	Interruptions at PSCell addition/release	292

7.32.2.2	Interruptions at transitions between active and non-active during DRX	292

7.32.2.3	Interruptions at transitions from non-DRX to DRX	292

7.32.2.4	Interruptions at SCell addition/release	292

7.32.2.5	Interruptions at SCell activation/deactivation	292

7.32.2.6	Interruptions during measurements on SCC	293

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	293

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	293

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	293

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	294

7.32.2.7	Interruptions at active BWP switching	294

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	295

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	295

7.32.2.10	Interruptions at SCell hibernation	295

7.32.2.11	Interruptions at direct SCell activation and hibernation	295

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	295

7.32.2.13	Interruptions at NR SRS carrier based switching	295

7.32.2.14	Interruptions at NR SCell dormancy	296

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	296

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	297

7.32.2.15	Interruption during NR measurement with autonomous gaps	297

7.32.2.16	Interruptions at SRS carrier based switching	297

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	298

7.33.1	Introduction	298

7.33.2	Requirements	298

7.34	Void	298

7.35	Interruptions with SFTD measurements	298

7.35.1	Introduction	298

7.35.2	Requirements	298

7.36	Interruptions with NE-DC	299

7.36.1	Introduction	299

7.36.2	Requirements	299

7.36.2.1	Interruptions at transitions between active and non-active during DRX	299

7.36.2.2	Interruptions at transitions from non-DRX to DRX	299

7.36.2.3	Interruptions at SCell addition/release	299

7.36.2.4	Interruptions at SCell activation/deactivation	300

7.36.2.5	Interruptions during measurements on SCC	300

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	300

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	300

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	300

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	301

7.36.2.6	Interruptions at active BWP switching	301

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	302

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	302

7.36.2.9	Interruptions at SCell hibernation	302

7.36.2.10	Interruptions at direct SCell activation and hibernation	302

7.36.2.11	Interruptions at NR SRS carrier based switching	302

7.36.2.12	Interruptions at NR SCell dormancy	303

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	303

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	303

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	303

7.36.2.14	Interruption during NR measurement with autonomous gaps	304

7.36.2.15	Interruptions at SRS carrier based switching	304

7.37	Interruptions during NR measurement with autonomous gaps	305

7.37.1	Introduction	305

7.37.2	Requirements	305

8	UE Measurements Procedures in RRC_CONNECTED State	305

8.1	General Measurement Requirements	305

8.1.1	Introduction	305

8.1.2	Requirements	306

8.1.2.1	UE measurement capability	306

8.1.2.1.1	Monitoring of multiple layers using gaps	314

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	315

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	316

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	318

8.1.2.1.2	Network controlled small gap	319

8.1.2.2	E-UTRAN intra frequency measurements	320

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	320

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	325

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	329

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	330

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	331

8.1.2.3	E-UTRAN inter frequency measurements	332

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	332

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	338

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	344

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	344

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	345

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	346

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	347

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	348

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	349

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	356

8.1.2.4	Inter RAT measurements	356

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	356

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	361

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	361

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	365

8.1.2.4.5	E-UTRAN FDD – GSM measurements	365

8.1.2.4.6	E-UTRAN TDD – GSM measurements	370

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	370

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	372

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	372

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	372

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	373

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	373

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	373

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	373

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	375

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	375

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	375

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	375

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	376

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	376

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	378

8.1.2.4.21	E-UTRAN FDD – NR measurements	378

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	382

8.1.2.4.22	E-UTRAN TDD – NR measurements	386

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	386

8.1.2.4.23	Void	386

8.1.2.4.24	Void	386

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	386

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	388

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	388

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	389

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	389

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	389

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	391

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	393

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	395

8.1.2.6.5	Void	396

8.1.2.6.6	Void	396

8.1.2.6.7	Void	396

8.1.2.6.8	Void	396

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	396

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	397

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	398

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	400

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	402

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	404

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	405

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	407

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	409

8.1.2.7	E-UTRAN E-CID Measurements	411

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	411

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	412

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	413

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	414

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	414

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	415

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	417

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	420

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	424

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	427

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	427

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	428

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	428

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	429

8.1.2.10	Void	429

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	429

8.2.1	Introduction	429

8.2.2	Requirements	429

8.3	Measurements for E-UTRA carrier aggregation	434

8.3.1	Introduction	434

8.3.2	Measurements of the primary component carrier	434

8.3.3	Measurements of a secondary component carrier	434

8.3.3.1	Measurements of a secondary component carrier with active SCell	434

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	435

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	435

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	436

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	438

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	438

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	438

8.4.1	Introduction	438

8.4.2	Measurements on the primary component carrier	439

8.4.3	Measurements on a secondary component carrier	440

8.4.4	Measurements on both primary component carrier and a secondary component carrier	440

8.4.5	Measurements on different secondary component carriers	441

8.5	Measurements for UE category 0	442

8.5.1	Introduction	442

8.5.2 	Requirements	443

8.5.2.1 	E-UTRAN intra frequency measurements	443

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	443

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	446

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	448

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	452

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	452

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	453

8.6	Discovery signal measurements	454

8.6.1	Introduction	454

8.6.2	Requirements for CRS based discovery signal measurements	454

8.6.2.1	E-UTRAN intra frequency measurements	454

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	454

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	457

8.6.2.2	E-UTRAN inter frequency measurements	459

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	460

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	462

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	465

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	465

8.6.3	Requirements for CSI-RS based discovery signal measurements	465

8.6.3.1	E-UTRAN intra frequency measurements	465

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	466

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	468

8.6.3.2	E-UTRAN inter frequency measurements	470

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	471

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	473

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	476

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	476

8.7	Discovery signal measurements for E-UTRA carrier aggregation	476

8.7.1	Introduction	476

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	477

8.7.2.1	Measurements of the primary component carrier	477

8.7.2.2	Measurements of a secondary component carrier	477

8.7.2.3	Measurements of a secondary component carrier with active SCell	477

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	477

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	477

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	478

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	480

8.7.3.1	Measurements of the primary component carrier	480

8.7.3.2	Measurements of a secondary component carrier	480

8.7.3.3	Measurements of a secondary component carrier with active SCell	480

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	480

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	480

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	482

8.8	Measurements for E-UTRA dual connectivity	483

8.8.1	Introduction	483

8.8.2	Intra-frequency measurements requirements on PCell	483

8.8.3	Intra-frequency measurements requirements on PSCell	484

8.8.4	Inter-frequency and inter-RAT measurement requirements	484

8.8.5	Intra-frequency measurements with autonomous gaps	484

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	484

8.8.5.2	ECGI reporting delay	485

8.8.6	Inter-frequency measurements with autonomous gaps	485

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	485

8.8.6.2	ECGI reporting delay	486

8.8.7	SSTD Measurements	486

8.8.7.1	Introduction	486

8.8.7.2	SSTD Measurement requirements	486

8.8.7.3	SSTD Measurement Reporting Delay	487

8.8.8	Intra-frequency measurements requirements on SCell	487

8.9	MBSFN Measurements	487

8.9.1	Introduction	487

8.9.2	MBSFN RSRP Measurements	487

8.9.3	MBSFN RSRQ Measurements	488

8.9.4	MCH BLER Measurements	488

8.10	Proximity-based Services	488

8.10.1	Introduction	488

8.10.2	Requirements	488

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	488

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	489

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	489

8.11.1	Introduction	489

8.11.2	CRS based discovery signal measurements	490

8.11.2.1	E-UTRAN intra-frequency measurements	490

8.11.2.1.1	Requirements	490

8.11.2.1.1.1	Requirements when no DRX is used	490

8.11.2.1.1.1.1	Measurement Reporting Requirements	492

8.11.2.1.1.2	Requirements when DRX is used	493

8.11.2.1.1.2.1	Measurement Reporting Requirements	495

8.11.2.2	E-UTRAN inter-frequency measurements	496

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	496

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	500

8.11.3	CSI-RS based discovery signal measurements	500

8.11.3.1	E-UTRAN intra-frequency measurements	500

8.11.3.1.1	Requirements	500

8.11.3.1.1.1	Requirements when no DRX is used	500

8.11.3.1.1.1.1	Measurement Reporting Requirements	502

8.11.3.1.1.2	Requirements when DRX is used	502

8.11.3.1.1.2.1	Measurement Reporting Requirements	503

8.11.3.2	E-UTRAN inter-frequency measurements	504

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	504

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	508

8.11.4	RSSI measurements	508

8.11.4.1	E-UTRAN intra-frequency measurements	508

8.11.4.2	E-UTRAN inter-frequency measurements	508

8.11.5	Channel occupancy measurements	509

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	509

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	509

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	509

8.12.1	Introduction	509

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	509

8.12.2.1	Introduction	509

8.12.2.2	Measurements of a secondary component carrier	509

8.12.2.3	Measurements of a secondary component carrier with active SCell	509

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	510

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	510

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	512

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	515

8.12.3.1	Introduction	515

8.12.3.2	Measurements of a secondary component carrier	515

8.12.3.3	Measurements of a secondary component carrier with active SCell	515

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	515

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	515

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	517

8.13	Measurements for UE Category M1	519

8.13.1	Introduction	519

8.13.2	Requirements for UE category M1 with CE mode A	519

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	520

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	520

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	525

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	527

8.13.2.2	Void	531

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	531

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	531

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	534

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	536

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	537

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	537

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	539

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	541

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	542

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	542

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	542

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	543

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	543

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	543

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	544

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	544

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	545

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	546

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	546

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	547

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	551

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	553

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	557

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	557

8.13.3	Requirements for UE category M1 with CE mode B	558

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	558

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	559

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	563

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	566

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	571

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	572

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	572

8.13.3.2	Void	573

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	573

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	573

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	576

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	578

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	579

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	579

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	579

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	580

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	580

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	580

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	581

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	581

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	581

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	586

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	588

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	592

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	593

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	593

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	595

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	597

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	598

8.14	Measurements for UE category NB1	598

8.14.1	Introduction	598

8.14.2	NB-IoT intra frequency measurements under normal coverage	599

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	599

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	599

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	599

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	599

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	599

8.14.4	Connected mode channel quality report for UE Category NB1	599

8.15	Void	600

8.16	Measurements for UE Category M2	600

8.16.1	Introduction	600

8.16.2	Requirements for UE category M2 with CE mode A	600

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	600

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	601

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	601

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	602

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	602

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	602

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	603

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	603

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	605

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	608

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	608

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	608

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	611

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	613

8.16.3	Requirements for UE category M2 with CE mode B	614

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	614

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	614

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	616

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	619

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	619

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	620

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	622

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	624

8.17	Measurements for E-UTRA – NR Dual Connectivity	625

8.17.1	Introduction	625

8.17.1.1	Measurement Gap Sharing	625

8.17.1A	Intrafrequency Measurements	625

8.17.2	SFTD Measurements	626

8.17.2.1	Introduction	626

8.17.2.2	SFTD Measurement requirements	626

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	626

8.17.2.3	SFTD Measurement Reporting Delay	628

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	628

8.17.3.1	Introduction	628

8.17.3.2	E-UTRAN FDD inter frequency measurements	628

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	628

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	630

8.17.3.3	E-UTRAN TDD inter frequency measurements	632

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	632

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	633

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	635

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	635

8.17.4.1.1	NR Inter-RAT cell identification	636

8.17.4.1.2	NR Inter-RAT measurement	637

8.17.4.1.3	NR Inter-RAT measurement reporting	638

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	639

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	639

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	639

8.17.4A.1.1	NR Inter-RAT cell identification	640

8.17.4A.1.2	NR Inter-RAT measurement	641

8.17.4A.1.3	NR Inter-RAT measurement reporting	642

8.17.4A.1.4	NR inter-RAT RSSI measurements	642

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	643

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	643

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	644

8.17.5.1	Introduction	644

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	644

8.17.5.2.1	Identification of a new UTRA FDD cell	644

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	644

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	644

8.17.5.2.4	Periodic Reporting	645

8.17.5.2.5	Event Triggered Reporting	645

8.17.5.2.6	Event-triggered Periodic Reporting	645

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	646

8.17.5.3.1	Periodic Reporting	647

8.17.5.3.2	Event Triggered Reporting	647

8.17.5.3.3	Event-triggered Periodic Reporting	647

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	648

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	648

8.17.7.1	Introduction	648

8.17.7.2	Identification of a new UTRA FDD cell for SON	648

8.17.7.2.1	Requirements when no DRX is used	648

8.17.7.2.2	Requirements when DRX is used	648

8.17.7.2.3	Reporting Delay	649

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	650

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	650

8.17.9.1	Introduction	650

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	650

8.17.9.2.1	Identification of a new UTRA FDD cell	650

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	650

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	651

8.17.9.2.4	Periodic Reporting	651

8.17.9.2.5	Event Triggered Reporting	651

8.17.9.2.6	Event-triggered Periodic Reporting	652

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	652

8.17.9.3.1	Periodic Reporting	653

8.17.9.3.2	Event Triggered Reporting	653

8.17.9.3.3	Event-triggered Periodic Reporting	653

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	654

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	654

8.17.11.1	Introduction	654

8.17.11.2	Identification of a new UTRA TDD cell for SON	654

8.17.11.2.1	Requirements when no DRX is used	654

8.17.11.2.2	Requirements when DRX is used	654

8.17.11.2.3	Reporting Delay	655

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	655

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	656

8.17.13.1	Introduction	656

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	656

8.17.13.2.1	GSM carrier RSSI	656

8.17.13.2.2	BSIC verification	656

8.17.13.2.3	Enhanced BSIC verification	658

8.17.13.2.4	Periodic Reporting	658

8.17.13.2.5	Event Triggered Reporting	658

8.17.13.2.6	Event-triggered Periodic Reporting	659

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	659

8.17.13.3.1	GSM carrier RSSI	659

8.17.13.3.2	BSIC verification	659

8.17.13.3.3	Periodic Reporting	661

8.17.13.3.4	Event Triggered Reporting	661

8.17.13.3.5	Event-triggered Periodic Reporting	661

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	661

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	661

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	661

8.17.15.1.1	RSTD Measurement Reporting Delay	662

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	662

8.17.15.2.1	RSTD Measurement Reporting Delay	662

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	662

8.17.15.3.1	RSTD Measurement Reporting Delay	663

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	663

8.17.15.4.1	RSTD Measurement Reporting Delay	663

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	664

8.17.16.1	Introduction	664

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	664

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	664

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	664

8.17.17.1	Introduction	664

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	664

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	664

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	664

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	664

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	664

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	664

8.18	Measurements for non-BL/CE UE	664

8.18.1	Introduction	664

8.18.2	Requirements for non-BL/CE UE with CE Mode B	665

8.18.2.1	E-UTRAN intra frequency measurements	665

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	665

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	665

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	666

8.19	Measurements for NR – E-UTRA Dual Connectivity	667

8.19.1	Introduction	667

8.19.2	Intra-frequency Measurements	667

8.19.3	Inter-frequency Measurements	667

8.19.4	Void	667

8.19.5	Intra-frequency E-CID Measurements	667

8.19.6	Intra-frequency measurements with autonomous gaps	668

8.19.6.1	Introduction	668

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	668

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	668

8.19.7	Inter-frequency measurements with autonomous gaps	668

8.19.7.1	Introduction	668

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	668

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	668

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	668

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	668

9	Measurements performance requirements for UE	668

9.1	E-UTRAN measurements	668

9.1.1	Introduction	668

9.1.2	Intra-frequency RSRP Accuracy Requirements	669

9.1.2.1	Absolute RSRP Accuracy	669

9.1.2.2	Relative Accuracy of RSRP	669

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	670

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	671

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	672

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	673

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	674

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	675

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	676

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	676

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	676

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	677

9.1.2B.1	Introduction	677

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	677

9.1.3	Inter-frequency RSRP Accuracy Requirements	678

9.1.3.1	Absolute RSRP Accuracy	678

9.1.3.2	Relative Accuracy of RSRP	679

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	680

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	680

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	681

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	681

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	682

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	683

9.1.3B.1	Introduction	683

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	683

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	683

9.1.4	RSRP Measurement Report Mapping	684

9.1.5	Intra-frequency RSRQ Accuracy Requirements	684

9.1.5.1	Absolute RSRQ Accuracy	684

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	685

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	686

9.1.5.4	Absolute WB-RSRQ Accuracy	687

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	688

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	689

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	689

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	689

9.1.5B.1	Introduction	689

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	689

9.1.6	Inter-frequency RSRQ Accuracy Requirements	690

9.1.6.1	Absolute RSRQ Accuracy	690

9.1.6.2	Relative Accuracy of RSRQ	691

9.1.6.3	Absolute WB-RSRQ Accuracy	692

9.1.6.4	Relative WB-RSRQ Accuracy	692

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	693

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	694

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	694

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	694

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	695

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	696

9.1.6B.1	Introduction	696

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	696

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	696

9.1.7	RSRQ Measurement Report Mapping	697

9.1.8	Power Headroom	697

9.1.8.1	Period	698

9.1.8.2	Reporting Delay	698

9.1.8.3	Void	698

9.1.8.4	Report Mapping	698

9.1.9	UE Rx – Tx time difference	698

9.1.9.1	Measurement Requirement	698

9.1.9.2	Measurement Report mapping	699

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	700

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	701

9.1.10	Reference Signal Time Difference (RSTD)	702

9.1.10.1	Intra-Frequency Accuracy Requirement	702

9.1.10.2	Inter-Frequency Accuracy Requirement	703

9.1.10.3	RSTD Measurement Report Mapping	705

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	705

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	706

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	707

9.1.11	Carrier aggregation measurement accuracy	708

9.1.11.1	Primary component carrier accuracy requirement	709

9.1.11.2	Secondary component carrier accuracy requirement	709

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	709

9.1.11.4	Secondary component carrier relative accuracy requirement	709

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	709

9.1.13	Measurement accuracy for UE category 0	710

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	710

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	710

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	711

9.1.14	Accuracy requirements for Discovery Signal Measurements	712

9.1.14.1	Introduction	712

9.1.14.2	RSRP measurements in discovery signal occasions	712

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	712

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	712

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	712

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	713

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	714

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	714

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	714

9.1.14.3.3	CSI-RSRP measurement report mapping	715

9.1.14.4	RSRQ measurements in discovery signal occasions	715

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	715

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	716

9.1.15.1.1	Primary component carrier accuracy requirement	716

9.1.15.1.2	Secondary component carrier accuracy requirement	716

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	716

9.1.15.1.4	Secondary component carrier relative accuracy requirement	716

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	716

9.1.15.2.1	Primary component carrier accuracy requirement	716

9.1.15.2.2	Secondary component carrier accuracy requirement	716

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	716

9.1.15.2.4	Secondary component carrier relative accuracy requirement	716

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	717

9.1.17	RS-SINR Measurements	717

9.1.17.1	Measurement Report Mapping	717

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	718

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	718

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	718

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	718

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	719

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	720

9.1.18.1	Introduction	720

9.1.18.2	RSRP measurements	720

9.1.18.2.1	RSRP measurement report mapping	720

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	720

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	721

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	721

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	722

9.1.18.3	RSRQ measurements	722

9.1.18.3.1	RSRQ measurement report mapping	722

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	722

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	723

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	723

9.1.18.4	CSI-RSRP measurements	724

9.1.18.4.1	CSI-RSRP measurement report mapping	724

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	724

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	724

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	725

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	725

9.1.18.5	RSSI measurements	726

9.1.18.5.1	RSSI measurement report mapping	726

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	726

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	727

9.1.18.6	Channel occupancy measurements	727

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	727

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	727

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	728

9.1.19.1	Introduction	728

9.1.19.2	Accuracy requirements for measurements on SCC	728

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	728

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	728

9.1.20	SFN and Subframe Time Difference (SSTD)	729

9.1.20.1	SSTD Accuracy Requirement	729

9.1.20.2	SSTD Measurement Report Mapping	729

9.1.21	Measurement accuracy for UE category M1	730

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	730

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	732

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	733

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	735

9.1.21.5	RSRP Measurement Report Mapping	736

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	736

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	737

9.1.21.8	RSRQ Measurement Report Mapping	738

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	738

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	739

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	740

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	741

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	742

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	743

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	744

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	745

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	746

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	747

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	748

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	749

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	752

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	755

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	755

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	758

9.1.22	Measurement accuracy for UE Category NB1	759

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	759

9.1.22.2	Void	760

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	760

9.1.22.4	Void	761

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	761

9.1.22.6	Void	762

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	762

9.1.22.8	Void	763

9.1.22.9	NRSRP Measurement Report Mapping	763

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	763

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	764

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	765

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	766

9.1.22.14	NRSRQ Measurement Report Mapping	767

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	768

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	768

9.1.23	Power Headroom for UE Category NB1	769

9.1.23.1	Period	769

9.1.23.2	Reporting Delay	769

9.1.23.3	Report Mapping for UE Category NB1	769

9.1.23.3.1	Void	771

9.1.23.3.2	Void	771

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	771

9.1.24	Void	772

9.1.25	Measurement accuracy for UE category M2	772

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	772

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	773

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	774

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	775

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	777

9.1.26	Measurement Accuracy for non-BL CE UE	778

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	779

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	781

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	781

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	783

9.1.26.5	RSRP Measurement Report Mapping	784

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	784

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	784

9.1.26.8	RSRQ Measurement Report Mapping	785

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	785

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	786

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	787

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	788

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	789

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	789

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	789

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	790

9.1.27	SFN and frame Timing Difference (SFTD)	791

9.1.27.1	SFTD Accuracy Requirement	791

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	793

9.1.28.1	SFTD Accuracy Requirement under CCA	793

9.2	UTRAN FDD Measurements	794

9.2.1	UTRAN FDD CPICH RSCP	794

9.2.2	Void	795

9.2.3	UTRAN FDD CPICH Ec/No	795

9.3	UTRAN TDD Measurements	795

9.3.1	UTRAN TDD P-CCPCH RSCP	796

9.3.2	Void	796

9.3.3	Void	796

9.4	GSM Measurements	796

9.4.1	GSM carrier RSSI	796

9.5	CDMA2000 1x RTT Measurements	796

9.5.1	CDMA2000 1x RTT Pilot Strength	796

9.6	PCMAX,c	797

9.6.1	Report Mapping	797

9.6.2 	Estimation Period	797

9.6.3	Reporting Delay	797

9.7	IEEE802.11 Measurements	797

9.7.1	WLAN RSSI	797

9.7.2	WLAN RSSI Measurement Report Mapping	797

9.8	MBSFN Measurements	798

9.8.1	Introduction	798

9.8.2	MBSFN RSRP	798

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	798

9.8.2.2	MBSFN RSRP measurement report mapping	799

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	799

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	800

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	800

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	800

9.8.3	MBSFN RSRQ	801

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	801

9.8.3.2	MBSFN RSRQ measurement report mapping	801

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	802

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	802

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	802

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	803

9.8.4	MCH BLER	803

9.8.4.1	Measurement report mapping for MCH BLER	803

9.8.4.2	Measurement report mapping for MCH Block Number	804

9.9	ProSe Measurements	805

9.9.1	Introduction	805

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	805

9.9.2.1	Absolute S-RSRP Accuracy	805

9.9.2.2	Relative Accuracy of S-RSRP	806

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	807

9.9.3.1	Absolute SD-RSRP Accuracy	807

9.9.3.2	Relative Accuracy of SD-RSRP	807

9.10	V2X Measurements	808

9.10.1	Introduction	808

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	808

9.10.2.1	Absolute S-RSRP Accuracy	808

9.10.2.2	Relative Accuracy of S-RSRP	809

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	809

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	809

9.10.4	S-RSSI Measurement Accuracy Requirements	810

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	810

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	810

9.11	NR Measurements	811

9.11.1	NR SS-RSRP Measurements	811

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	811

9.11.2	NR SS-RSRQ Measurements	812

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	812

9.11.3	NR SS-SINR Measurements	812

9.11.4	NR SS-RSRP Measurements under CCA	813

9.11.5	NR SS-RSRQ Measurements under CCA	813

9.11.6	NR SS-SINR Measurements under CCA	813

9.11.7	NR RSSI Measurements under CCA	814

9.11.8	NR Channel Occupancy Measurements under CCA	814

10	Measurements Performance Requirements for E-UTRAN	814

10.1	Received Interference Power	814

10.1.1	Absolute accuracy requirement	814

10.1.2	Relative accuracy requirement	815

10.1.3	Received Interference Power measurement report mapping	815

10.2	Angle of Arrival (AOA)	815

10.2.1	Range/mapping	815

10.3	Timing Advance (TADV)	815

10.3.1	Report mapping	815

11	ProSe Requirements in Any Cell Selection state	816

11.1	Introduction	816

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	816

11.2.1	Introduction	816

11.2.2	ProSe UE transmission timing	816

11.3	Initiation/Cease of SLSS Transmissions	817

11.3.1	Introduction	817

11.3.2	Requirements	817

11.4	Measurements for ProSe in Any Cell Selection State	817

11.4.1	Introduction	817

11.4.2	Requirements	817

11.4.2.1	E-UTRA FDD	817

11.4.2.2	E-UTRA TDD	818

11.5	Selection / Reselection of ProSe Synchronization Reference	818

11.5.1	Introduction	818

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	818

11.5.2.1	Introduction	818

11.5.2.2	Requirements	818

11.6	Void	819

11.7	Selection / Reselection of ProSe relay UE	819

11.7.1	Introduction	819

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	819

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	819

12.1	Introduction	819

12.2	Transmit Timing	820

12.2.1	GNSS as timing reference	820

12.3	Interruption	820

12.4	Reliability of GNSS signal	820

13	V2X Requirements	820

13.1	Introduction	820

13.2	UE Transmit Timing	821

13.2.1	Introduction	821

13.2.2	GNSS as synchronization reference source	821

13.2.3	Serving cell/PCell as synchronization reference source	821

13.2.4	SyncRef UE as synchronization reference source	821

13.3	Initiation/Cease of SLSS Transmissions	821

13.3.1	Introduction	821

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	821

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	822

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	822

13.4	Selection / Reselection of V2X Synchronization Reference Source	823

13.5	Autonomous Resource Selection/Reselection measurements	824

13.5.1	Introduction	824

13.5.2	PSSCH-RSRP measurements	824

13.5.3	S-RSSI measurements	824

13.6	Congestion Control measurements	824

13.7	Interruption	824

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	824

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	824

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	825

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	825

13.8	Reliability of GNSS signal	825

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	826

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	826

Annex A (normative): Test Cases	827

A.1	Purpose of annex	827

A.2	Requirement classification for statistical testing	827

A.2.1	Types of requirements in TS 36.133	827

A.2.1.1	Time and delay requirements on UE higher layer actions	827

A.2.1.2	Measurements of power levels, relative powers and time	827

A.2.1.3	Implementation requirements	828

A.2.1.4	Physical layer timing requirements	828

A.3	RRM test configurations	829

A.3.1	Reference Measurement Channels	829

A.3.1.1	PDSCH	829

A.3.1.1.1	FDD	829

A.3.1.1.2	TDD	834

A.3.1.1.3	FDD for UE category 0	837

A.3.1.1.4	HD-FDD for UE category 0	838

A.3.1.1.5	TDD for UE category 0	839

A.3.1.1.6	Frame Structure 3	840

A.3.1.2	PCFICH/PDCCH/PHICH	841

A.3.1.2.1	FDD	841

A.3.1.2.2	TDD	841

A.3.1.2.3	HD-FDD for UE category 0	842

A.3.1.2.4	FS 3	842

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	842

A.3.1.3.1	FDD in CEModeA	843

A.3.1.3.2	HD-FDD in CEModeA	843

A.3.1.3.3	TDD in CEModeA	844

A.3.1.3.4	FDD in CEModeB	844

A.3.1.3.5	HD-FDD in CEModeB	845

A.3.1.3.6	TDD in CEModeB	845

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	846

A.3.1.4.1	FDD in CEModeA	846

A.3.1.4.2	HD-FDD in CEModeA	847

A.3.1.4.3	TDD in CEModeA	848

A.3.1.4.4	FDD in CEModeB	849

A.3.1.4.5	 HD-FDD in CEModeB	850

A.3.1.4.6	TDD in CEModeB	850

A.3.1.5	NPDSCH Reference Channel for UE category NB1	851

A.3.1.5.1	HD-FDD in-band operation	851

A.3.1.5.2	Void	852

A.3.1.5.3	HD-FDD standalone operation	852

A.3.1.5.4	Void	853

A.3.1.5.5	HD-FDD guard band operation	853

A.3.1.5.6	Void	854

A.3.1.5.7	TDD in-band operation	854

A.3.1.5.8	TDD standalone operation	854

A.3.1.5.9	TDD guard band operation	855

A.3.1.6	NPDCCH Reference Channel for UE category NB1	855

A.3.1.6.1	In-band operation	855

A.3.1.6.2	Void	856

A.3.1.6.3	Standalone operation	856

A.3.1.6.4	Void	856

A.3.1.6.5	Guard band operation	856

A.3.1.6.6	Void	857

A.3.2	OFDMA Channel Noise Generator (OCNG)	857

A.3.2.1	OCNG Patterns for FDD	857

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	858

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	858

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	859

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	860

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	860

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	861

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	861

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	861

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	862

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	863

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	863

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	864

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	864

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	865

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	865

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	866

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	866

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	867

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	867

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	868

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	868

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	869

A.3.2.2	OCNG Patterns for TDD	869

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	870

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	870

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	871

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	871

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	872

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	873

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	874

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	875

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	875

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	876

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	876

A.3.2.3	OCNG Patterns for Narrowband IoT	877

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	878

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	879

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	879

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	880

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	881

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	882

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	883

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	884

A.3.2.4	OCNG Patterns for V2X sidelink	885

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	885

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	886

A.3.3	Reference DRX Configurations	886

A.3.4	ABS Transmission Configurations	887

A.3.4.1	Non-MBSFN ABS Transmission Configurations	887

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	887

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	887

A.3.4.2	MBSFN ABS Transmission Configurations	888

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	888

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	889

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	889

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	889

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	890

A.3.6.1	Introduction	890

A.3.7	Test Cases with Different Channel Bandwidths	890

A.3.7.1	Introduction	890

A.3.7.2	Principle of testing	890

A.3.8	Antenna Configuration	890

A.3.8.1	Antenna connection for 4 Rx capable UEs	890

A.3.8.1.1 Introduction	890

A.3.8.1.2 Principle of testing	890

A.3.8.1.2.1 Single carrier tests	890

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	891

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	892

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	892

A.3.8.2	Antenna connection for 8 Rx capable UEs	892

A.3.8.2.1	Introduction	892

A.3.8.2.2	Principle of testing	892

A.3.8.2.2.1	Single carrier tests	892

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	893

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	893

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	893

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	893

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	893

A.3.9.1	Introduction	893

A.3.9.2	Principle of testing	893

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	894

A.3.10.1	Introduction	894

A.3.10.2	Principle of testing	894

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	894

A.3.11.1	Introduction	894

A.3.11.2	Principle of Testing	894

A.3.12	Proximity-based Services	894

A.3.12.1	Introduction	894

A.3.12.2	Reference DRX configurations for ProSe tests	894

A.3.12.3	Test Cases with Different Channel Bandwidths	895

A.3.12.3.1	Introduction	895

A.3.12.3.2	Principle of testing	895

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	895

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	902

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	905

A.3.12.6.1	FDD	905

A.3.12.7	Reference measurement channels for ProSe Direct Communication	905

A.3.12.7.1	FDD	905

A.3.12.8	ProSe Receive Traffic Generator	906

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	906

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	906

A.3.13	Time Offset between Cells	907

A.3.13.1	Introduction	907

A.3.13.2	Definition	907

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	907

A.3.14.1	Introduction	907

A.3.14.2	Principle of testing	907

A.3.15	Dual connectivity test cases with different combination of duplex mode	907

A.3.15.1	Introduction	907

A.3.15.2	Principle of testing	907

A.3.16	Reference PRACH Configurations	908

A.3.17	Listen before talk model	908

A.3.17.1	Introduction	908

A.3.17.2	Definition	908

A.3.18	Reference NPRACH Configurations	909

A.3.19	Dual connectivity test cases with different bandwidth combinations	910

A.3.19.1	Introduction	910

A.3.19.2	Principle of testing	910

A.3.20	Category M1 UE Test Cases	911

A.3.20.1	Introduction	911

A.3.20.2	Principle of Cat-M1 UE Testing	911

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	912

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	913

A.3.21.1	Introduction	913

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	913

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	914

A.3.22	Category 1bis UE Test Cases	915

A.3.22.1	Introduction	915

A.3.22.2	Principle of Category 1bis UE Testing	915

A.3.23	Category NB2 UE Test Cases	919

A.3.23.1	Introduction	919

A.3.23.2	Principle of Category NB2 UE Testing	919

A.3.24	V2X sidelink communication	922

A.3.24.1	Introduction	922

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	923

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	927

A.3.25	Category M2 UE Test Cases	928

A.3.25.1	Introduction	928

A.3.25.2	Principle of Cat-M2 UE Testing	928

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	929

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	930

A.3.26.1	Introduction	930

A.3.26.2	Principle of testing	930

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	930

A.3.27.1	Introduction	930

A.3.27.2	Principle of INACTIVE cell re-selection Testing	930

A.4	E-UTRAN RRC_IDLE state	930

A.4.2	Cell Re-Selection	930

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	930

A.4.2.1.1	Test Purpose and Environment	930

A.4.2.1.2	Test Requirements	932

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	933

A.4.2.2.1	Test Purpose and Environment	933

A.4.2.2.2	Test Requirements	934

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	935

A.4.2.3.1	Test Purpose and Environment	935

A.4.2.3.2	Test Requirements	936

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	937

A.4.2.4.1	Test Purpose and Environment	937

A.4.2.4.2	Test Requirements	938

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	939

A.4.2.5.1	Test Purpose and Environment	939

A.4.2.5.2	Test Requirements	940

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	941

A.4.2.6.1	Test Purpose and Environment	941

A.4.2.6.2	Test Requirements	942

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	943

A.4.2.7.1	Test Purpose and Environment	943

A.4.2.7.2	Test Requirements	944

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	945

A.4.2.8.1	Test Purpose and Environment	945

A.4.2.8.2	Test Requirements	946

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	947

A.4.2.9.1	Test Purpose and Environment	947

A.4.2.9.2	Test Requirements	947

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	947

A.4.2.10.1	Test Purpose and Environment	947

A.4.2.10.2	Test Requirements	951

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	951

A.4.2.11.1	Test Purpose and Environment	951

A.4.2.11.2	Test Requirements	955

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	955

A.4.2.12.1	Test Purpose and Environment	955

A.4.2.12.2	Test Requirements	957

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	958

A.4.2.13.1	Test Purpose and Environment	958

A.4.2.13.2	Test Requirements	959

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	960

A.4.2.14.1	Test Purpose and Environment	960

A.4.2.14.2	Test Requirements	961

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	962

A.4.2.15.1	Test Purpose and Environment	962

A.4.2.15.2	Test Requirements	963

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	964

A.4.2.16.1	Test Purpose and Environment	964

A.4.2.16.2	Test Requirements	965

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	966

A.4.2.17.1	Test Purpose and Environment	966

A.4.2.17.2	Test Requirements	967

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	968

A.4.2.18.1	Test Purpose and Environment	968

A.4.2.18.2	Test Requirements	970

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	971

A.4.2.19.1	Test Purpose and Environment	971

A.4.2.19.2	Test Requirements	973

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	974

A.4.2.20.1	Test Purpose and Environment	974

A.4.2.20.2	Test Requirements	975

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	976

A.4.2.21.1	Test Purpose and Environment	976

A.4.2.21.2	Test Requirements	977

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	978

A.4.2.22.1	Test Purpose and Environment	978

A.4.2.22.2	Test Requirements	979

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	980

A.4.2.23.1	Test Purpose and Environment	980

A.4.2.23.2	Test Requirements	981

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	982

A.4.2.24.1	Test Purpose and Environment	982

A.4.2.24.2	Test Requirements	985

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	986

A.4.2.25.1	Test Purpose and Environment	986

A.4.2.25.2	Test Requirements	987

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	988

A.4.2.26.1	Test Purpose and Environment	988

A.4.2.26.2	Test Requirements	989

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	990

A.4.2.27.1	Test Purpose and Environment	990

A.4.2.27.2	Test Requirements	991

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	992

A.4.2.28.1	Test Purpose and Environment	992

A.4.2.28.2	Test Requirements	993

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	994

A.4.2.29.1	Test Purpose and Environment	994

A.4.2.29.2	Test Requirements	995

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	996

A.4.2.30.1	Test Purpose and Environment	996

A.4.2.30.2	Test Requirements	997

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	998

A.4.2.31.1	Test Purpose and Environment	998

A.4.2.31.2	Test Requirements	999

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1000

A.4.2.32.1	Test Purpose and Environment	1000

A.4.2.32.2	Test Requirements	1001

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1002

A.4.2.33.1	Test Purpose and Environment	1002

A.4.2.33.2	Test Requirements	1003

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1004

A.4.2.34.1	Test Purpose and Environment	1004

A.4.2.34.2	Test Requirements	1005

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1006

A.4.2.35.1	Test Purpose and Environment	1006

A.4.2.35.2	Test Requirements	1008

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1009

A.4.2.36.1	Test Purpose and Environment	1009

A.4.2.36.2	Test Requirements	1011

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1012

A.4.2.37.1	Test Purpose and Environment	1012

A.4.2.37.2	Test Requirements	1014

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1015

A.4.2.38.1	Test Purpose and Environment	1015

A.4.2.38.2	Test Requirements	1017

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1018

A.4.2.39.1	Test Purpose and Environment	1018

A.4.2.39.2	Test Requirements	1019

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1020

A.4.2.40.1	Test Purpose and Environment	1020

A.4.2.40.2	Test Requirements	1021

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1022

A.4.2.41.1	Test Purpose and Environment	1022

A.4.2.41.2	Test Requirements	1025

A.4.2.42.2	Test Requirements	1029

A.4.2.43.2	Test Requirements	1032

A.4.2.44.2	Test Requirements	1036

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1036

A.4.2.45.1	Test Purpose and Environment	1036

A.4.2.45.2	Test Requirements	1039

A.4.2.46.2	Test Requirements	1043

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1043

A.4.2.47.1	Test Purpose and Environment	1043

A.4.2.47.2	Test Requirements	1047

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1047

A.4.2.48.1	Test Purpose and Environment	1047

A.4.2.48.2	Test Requirements	1049

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1050

A.4.2.49.1	Test Purpose and Environment	1050

A.4.2.49.2	Test Requirements	1052

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1053

A.4.2.50.1	Test Purpose and Environment	1053

A.4.2.50.2	Test Requirements	1055

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1056

A.4.2.51.1	Test Purpose and Environment	1056

A.4.2.51.2	Test Requirements	1058

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1059

A.4.2.52.1	Test Purpose and Environment	1059

A.4.2.52.2	Test Requirements	1061

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1062

A.4.2.53.1	Test Purpose and Environment	1062

A.4.2.53.2	Test Requirements	1064

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1065

A.4.2.54.1	Test Purpose and Environment	1065

A.4.2.54.2	Test Requirements	1067

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1068

A.4.2.55.1	Test Purpose and Environment	1068

A.4.2.55.2	Test Requirements	1070

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1071

A.4.2.56.1	Test Purpose and Environment	1071

A.4.2.56.2	Test Requirements	1072

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1073

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1073

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1073

A.4.3.1.1.1	Test Purpose and Environment	1073

A.4.3.1.1.2	Test Requirements	1075

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1075

A.4.3.1.2.1	Test Purpose and Environment	1075

A.4.3.1.2.2	Test Requirements	1078

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1078

A.4.3.1.3.1	Test Purpose and Environment	1078

A.4.3.1.3.2	Test Requirements	1081

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1081

A.4.3.1.4.1	Test Purpose and Environment	1081

A.4.3.1.4.2	Test Requirements	1082

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1082

A.4.3.1.5.1	Test Purpose and Environment	1082

A.4.3.1.5.2	Test Requirements	1085

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1086

A.4.3.2.1	Test Purpose and Environment	1086

A.4.3.2.1.1	Void	1086

A.4.3.2.1.2	1.28Mcps TDD option	1086

A.4.3.2.1.3	Void	1088

A.4.3.2.2	Test Requirements	1088

A.4.3.2.2.1	1.28Mcps TDD option	1088

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1088

A.4.3.2A.1	Test Purpose and Environment	1088

A.4.3.2A.2	Test Requirements	1092

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1092

A.4.3.3.1	Test Purpose and Environment	1092

A.4.3.3.2	Test Requirements	1095

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1095

A.4.3.3A.1	Test Purpose and Environment	1095

A.4.3.3A.2	Test Requirements	1100

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1101

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1101

A.4.3.4.1.1	Test Purpose and Environment	1101

A.4.3.4.1.2	Test Requirements	1103

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1103

A.4.3.4.2.1	Test Purpose and Environment	1103

A.4.3.4.2.2	Test Requirements	1105

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1105

A.4.3.4.3.1	Test Purpose and Environment	1105

A.4.3.4.3.2	Test Requirements	1108

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1108

A.4.3.4.4.1	Test Purpose and Environment	1108

A.4.3.4.4.2	Test Requirements	1112

A.4.4	E-UTRAN to GSM Cell Re-Selection	1112

A.4.4.1	E-UTRAN FDD – GSM:	1112

A.4.4.1.1	Test Purpose and Environment	1112

A.4.4.1.2	Test Requirements	1114

A.4.4.2	E-UTRAN TDD – GSM:	1114

A.4.4.2.1	Test Purpose and Environment	1114

A.4.4.2.2	Test Requirements	1116

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1117

A.4.5.1	E-UTRAN FDD – HRPD	1117

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1117

A.4.5.1.1.1	Test Purpose and Environment	1117

A.4.5.1.1.2	Test Requirements	1119

A.4.5.2	E-UTRAN TDD – HRPD	1119

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1119

A.4.5.2.1.1	Test Purpose and Environment	1119

A.4.5.2.1.2	Test Requirements	1122

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1122

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1122

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1122

A.4.6.1.1.1	Test Purpose and Environment	1122

A.4.6.1.1.2	Test Requirements	1125

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1125

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1125

A.4.6.2.1.1	Test Purpose and Environment	1125

A.4.6.2.1.2	Test Requirements	1128

A.4.7	Idle State Positioning Measurement for UE category NB1	1128

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1128

A.4.7.1.1	Test Purpose and Environment	1128

A.4.7.1.2	Test Requirements	1133

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1134

A.4.7.2.1	Test Purpose and Environment	1134

A.4.7.4.2	Test Requirements	1138

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1139

A.4.7.3.1	Test Purpose and Environment	1139

A.4.7.3.2	Test Requirements	1143

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1144

A.4.7.4.1	Test Purpose and Environment	1144

A.4.7.4.2	Test Requirements	1148

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1149

A.5.1	E-UTRAN Handover	1149

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1149

A.5.1.1.1	Test Purpose and Environment	1149

A.5.1.1.2	Test Requirements	1151

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1151

A.5.1.2.1	Test Purpose and Environment	1151

A.5.1.2.2	Test Requirements	1153

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1153

A.5.1.3.1	Test Purpose and Environment	1153

A.5.1.3.2	Test Requirements	1155

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1155

A.5.1.4.1	Test Purpose and Environment	1155

A.5.1.4.2	Test Requirements	1157

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1157

A.5.1.5.1	Test Purpose and Environment	1157

A.5.1.5.2	Test Requirements	1159

A.5.1.6	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1159

A.5.1.6.1	Test Purpose and Environment	1159

A.5.1.6.2	Test Requirements	1161

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1161

A.5.1.7.1	Test Purpose and Environment	1161

A.5.1.7.2	Test Requirements	1164

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1164

A.5.1.8.1	Test Purpose and Environment	1164

A.5.1.8.2	Test Requirements	1167

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1167

A.5.1.9.1	Test Purpose and Environment	1167

A.5.1.9.2	Test Requirements	1168

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1168

A.5.1.10.1	Test Purpose and Environment	1168

A.5.1.10.2	Test Requirements	1170

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1170

A.5.1.11.1	Test Purpose and Environment	1170

A.5.1.11.2	Test Requirements	1172

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1172

A.5.1.12.1	Test Purpose and Environment	1172

A.5.1.12.2	Test Requirements	1174

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1174

A.5.1.13.1	Test Purpose and Environment	1174

A.5.1.13.2	Test Requirements	1176

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1177

A.5.1.14.1	Test Purpose and Environment	1177

A.5.1.14.2	Test Requirements	1178

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1179

A.5.1.15.1	Test Purpose and Environment	1179

A.5.1.15.2	Test Requirements	1180

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1181

A.5.1.16.1	Test Purpose and Environment	1181

A.5.1.16.2	Test Requirements	1182

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1183

A.5.1.17.1	Test Purpose and Environment	1183

A.5.1.17.2	Test Requirements	1184

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1185

A.5.1.18.1	Test Purpose and Environment	1185

A.5.1.18.2	Test Requirements	1186

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1187

A.5.1.19.1	Test Purpose and Environment	1187

A.5.1.19.2	Test Requirements	1188

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1189

A.5.1.20.1	Test Purpose and Environment	1189

A.5.1.20.2	Test Requirements	1190

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1191

A.5.1.21.1	Test Purpose and Environment	1191

A.5.1.21.2	Test Requirements	1192

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1192

A.5.1.22.1	Test Purpose and Environment	1192

A.5.1.22.2	Test Requirements	1194

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1194

A.5.1.23.1	Test Purpose and Environment	1194

A.5.1.23.2	Test Requirements	1196

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1196

A.5.1.24.1	Test Purpose and Environment	1196

A.5.1.24.2	Test Requirements	1198

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1198

A.5.1.25.1	Test Purpose and Environment	1198

A.5.1.25.2	Test Requirements	1200

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1201

A.5.1.26.1	Test Purpose and Environment	1201

A.5.1.26.2	Test Requirements	1202

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1203

A.5.1.27.1	Test Purpose and Environment	1203

A.5.1.27.2	Test Requirements	1204

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1205

A.5.1.28.1	Test Purpose and Environment	1205

A.5.1.28.2	Test Requirements	1206

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1207

A.5.1.29.1	Test Purpose and Environment	1207

A.5.1.29.2	Test Requirements	1208

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1209

A.5.1.30.1	Test Purpose and Environment	1209

A.5.1.30.2	Test Requirements	1210

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1211

A.5.1.31.1	Test Purpose and Environment	1211

A.5.1.31.2	Test Requirements	1212

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1213

A.5.1.32.1	Test Purpose and Environment	1213

A.5.1.32.2	Test Requirements	1214

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1215

A.5.1.33.1	Test Purpose and Environment	1215

A.5.1.33.2	Test Requirements	1216

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1217

A.5.1.34.1	Test Purpose and Environment	1217

A.5.1.34.2	Test Requirements	1218

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1219

A.5.1.35.1	Test Purpose and Environment	1219

A.5.1.35.2	Test Requirements	1220

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1221

A.5.1.36.1	Test Purpose and Environment	1221

A.5.1.36.2	Test Requirements	1222

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1223

A.5.1.37.1	Test Purpose and Environment	1223

A.5.1.37.2	Test Requirements	1224

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1225

A.5.1.38.1	Test Purpose and Environment	1225

A.5.1.38.2	Test Requirements	1226

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1227

A.5.1.39.1	Test Purpose and Environment	1227

A.5.1.39.2	Test Requirements	1228

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1229

A.5.1.40.1	Test Purpose and Environment	1229

A.5.1.40.2	Test Requirements	1230

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1231

A.5.1.41.1	Test Purpose and Environment	1231

A.5.1.41.2	Test Requirements	1233

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1234

A.5.1.42.1	Test Purpose and Environment	1234

A.5.1.42.2	Test Requirements	1235

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1236

A.5.1.43.1	Test Purpose and Environment	1236

A.5.1.43.2	Test Requirements	1237

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1238

A.5.1.44.1	Test Purpose and Environment	1238

A.5.1.44.2	Test Requirements	1239

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1240

A.5.1.45.1	Test Purpose and Environment	1240

A.5.1.45.1	Test Requirements	1241

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1242

A.5.1.46.1	Test Purpose and Environment	1242

A.5.1.46.2	Test Requirements	1243

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1244

A.5.1.47.1	Test Purpose and Environment	1244

A.5.1.47.2	Test Requirements	1245

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1246

A.5.1.48.1	Test Purpose and Environment	1246

A.5.1.48.2	Test Requirements	1247

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1248

A.5.1.49.1	Test Purpose and Environment	1248

A.5.1.49.2	Test Requirements	1249

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1250

A.5.1.50.1	Test Purpose and Environment	1250

A.5.1.50.2	Test Requirements	1251

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1252

A.5.1.51.1	Test Purpose and Environment	1252

A.5.1.51.2	Test Requirements	1254

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1254

A.5.152.1	Test Purpose and Environment	1254

A.5.1.52.2	Test Requirements	1257

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1258

A.5.1.53.1	Test Purpose and Environment	1258

A.5.1.53.2	Test Requirements	1260

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1261

A.5.1.54.1	Test Purpose and Environment	1261

A.5.1.54.2	Test Requirements	1263

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1264

A.5.1.55.1	Test Purpose and Environment	1264

A.5.1.55.2	Test Requirements	1267

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1268

A.5.1.56.1	Test Purpose and Environment	1268

A.5.1.56.2	Test Requirements	1271

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1272

A.5.1.57.1	Test Purpose and Environment	1272

A.5.1.57.2	Test Requirements	1274

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1274

A.5.1.58.1	Test Purpose and Environment	1274

A.5.1.58.2	Test Requirements	1277

A.5.2	E-UTRAN Handover to other RATs	1277

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1277

A.5.2.1.1	Test Purpose and Environment	1277

A.5.2.1.2	Test Requirements	1279

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1280

A.5.2.2.1	Test Purpose and Environment	1280

A.5.2.2.2	Test Requirements	1283

A.5.2.3 E-UTRAN FDD- GSM Handover	1283

A.5.2.3.1	Test Purpose and Environment	1283

A.5.2.3.2	Test Requirements	1284

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1285

A.5.2.4.1	Test Purpose and Environment	1285

A.5.2.4.1.1	Void	1285

A.5.2.4.1.2	1.28 Mcps TDD option	1285

A.5.2.4.1.3	Void	1287

A.5.2.4.2	Test Requirements	1287

A.5.2.4.2.1	Void	1287

A.5.2.4.2.2	1.28 Mcps TDD option	1287

A.5.2.4.2.3	Void	1287

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1287

A.5.2.5.1	Test Purpose and Environment	1287

A.5.2.5.1.3	Void	1290

A.5.2.5.2	Test Requirements	1290

A.5.2.5.2.1	Void	1290

A.5.2.5.2.2	1.28 Mcps TDD option	1290

A.5.2.5.2.3	Void	1290

A.5.2.6	E-UTRAN TDD - GSM Handover	1290

A.5.2.6.1	Test Purpose and Environment	1290

A.5.2.6.2	Test Requirements	1292

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1293

A.5.2.7.1	Test Purpose and Environment	1293

A.5.2.7.2	Test Requirements	1295

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1295

A.5.2.8.1	Test Purpose and Environment	1295

A.5.2.8.2	Test Requirements	1296

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1297

A.5.2.9.1	Test Purpose and Environment	1297

A.5.2.9.2	Test Requirements	1298

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1299

A.5.2.10.1	Test Purpose and Environment	1299

A.5.2.10.2	Test Requirements	1301

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1301

A.5.2.10A.1	Test Purpose and Environment	1301

A.5.2.10A.2	Test Requirements	1304

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1304

A.5.2.10B.1	Test Purpose and Environment	1304

A.5.2.10B.2	Test Requirements	1307

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1307

A.5.2.11.1	Test Purpose and Environment	1307

A.5.2.11.2	Test Requirements	1307

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1308

A.5.3.1	E-UTRAN FDD – HRPD Handover	1308

A.5.3.1.1	Test Purpose and Environment	1308

A.5.3.1.2	Test Requirements	1310

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1310

A.5.3.2.1	Test Purpose and Environment	1310

A.5.3.2.2	Test Requirements	1313

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1313

A.5.3.3.1	Test Purpose and Environment	1313

A.5.3.3.2	Test Requirements	1316

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1316

A.5.3.4.1	Test Purpose and Environment	1316

A.5.3.4.2	Test Requirements	1317

A.5.3.5	E-UTRAN TDD – HRPD Handover	1318

A.5.3.5.1	Test Purpose and Environment	1318

A.5.3.5.2	Test Requirements	1320

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1321

A.5.3.6.1	Test Purpose and Environment	1321

A.5.3.6.2	Test Requirements	1324

A.6	RRC Connection Control	1324

A.6.1	RRC Re-establishment	1324

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1324

A.6.1.1.1	Test Purpose and Environment	1324

A.6.1.1.2	Test Requirements	1325

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1326

A.6.1.2.1	Test Purpose and Environment	1326

A.6.1.2.2	Test Requirements	1327

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1328

A.6.1.3.1	Test Purpose and Environment	1328

A.6.1.3.2	Test Requirements	1329

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1330

A.6.1.4.1	Test Purpose and Environment	1330

A.6.1.4.2	Test Requirements	1331

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1332

A.6.1.5.1	Test Purpose and Environment	1332

A.6.1.5.2	Test Requirements	1332

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1332

A.6.1.6.1	Test Purpose and Environment	1332

A.6.1.6.2	Test Requirements	1334

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1335

A.6.1.7.1	Test Purpose and Environment	1335

A.6.1.7.2	Test Requirements	1336

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1337

A.6.1.8.1	Test Purpose and Environment	1337

A.6.1.8.2	Test Requirements	1338

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1339

A.6.1.9.1	Test Purpose and Environment	1339

A.6.1.9.2	Test Requirements	1340

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1341

A.6.1.10.1	Test Purpose and Environment	1341

A.6.1.10.2	Test Requirements	1342

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1343

A.6.1.11.1	Test Purpose and Environment	1343

A.6.1.11.2	Test Requirements	1344

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1345

A.6.1.12.1	Test Purpose and Environment	1345

A.6.1.12.2	Test Requirements	1346

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1347

A.6.1.13.1	Test Purpose and Environment	1347

A.6.1.13.2	Test Requirements	1348

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1349

A.6.1.14.1	Test Purpose and Environment	1349

A.6.1.14.2	Test Requirements	1350

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1351

A.6.1.15.1	Test Purpose and Environment	1351

A.6.1.15.2	Test Requirements	1353

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1354

A.6.1.16.1	Test Purpose and Environment	1354

A.6.1.16.2	Test Requirements	1356

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1357

A.6.1.17.1	Test Purpose and Environment	1357

A.6.1.17.2	Test Requirements	1358

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1359

A.6.1.18.1	Test Purpose and Environment	1359

A.6.1.18.2	Test Requirements	1360

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1361

A.6.1.19.1	Test Purpose and Environment	1361

A.6.1.19.2	Test Requirements	1362

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1363

A.6.1.20.1	Test Purpose and Environment	1363

A.6.1.20.2	Test Requirements	1364

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1365

A.6.1.21.1	Test Purpose and Environment	1365

A.6.1.21.2	Test Requirements	1366

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1367

A.6.1.22.1	Test Purpose and Environment	1367

A.6.1.22.2	Test Requirements	1368

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1369

A.6.1.23.1	Test Purpose and Environment	1369

A.6.1.23.2	Test Requirements	1371

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1372

A.6.1.24.1	Test Purpose and Environment	1372

A.6.1.24.2	Test Requirements	1374

A.6.2	Random Access	1375

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1375

A.6.2.1.1	Test Purpose and Environment	1375

A.6.2.1.2	Test Requirements	1376

A.6.2.1.2.1	Random Access Response Reception	1376

A.6.2.1.2.2	No Random Access Response Reception	1376

A.6.2.1.2.3	Receiving a NACK on msg3	1376

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1377

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1377

A.6.2.1.2.6	Contention Resolution Timer expiry	1377

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1377

A.6.2.2.1	Test Purpose and Environment	1377

A.6.2.2.2	Test Requirements	1378

A.6.2.2.2.1	Random Access Response Reception	1379

A.6.2.2.2.2	No Random Access Response Reception	1379

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1379

A.6.2.3.1	Test Purpose and Environment	1379

A.6.2.3.2	Test Requirements	1381

A.6.2.3.2.1	Random Access Response Reception	1381

A.6.2.3.2.2	No Random Access Response reception	1381

A.6.2.3.2.3	Receiving a NACK on msg3	1381

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1382

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1382

A.6.2.3.2.6	Contention Resolution Timer expiry	1382

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1382

A.6.2.4.1	Test Purpose and Environment	1382

A.6.2.4.2	Test Requirements	1384

A.6.2.4.2.1	Random Access Response Reception	1384

A.6.2.4.2.2	No Random Access Response Reception	1384

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1384

A.6.2.5.1	Test Purpose and Environment	1384

A.6.2.5.2	Test Requirements	1385

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1385

A.6.2.6.1	Test Purpose and Environment	1385

A.6.2.6.2	Test Requirements	1385

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1386

A.6.2.7.1	Test Purpose and Environment	1386

A.6.2.7.2	Test Requirements	1388

A.6.2.7.2.1	Random Access Response Reception	1388

A.6.2.7.2.2	No Random Access Response Reception	1388

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1388

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1389

A.6.2.8.1	Test Purpose and Environment	1389

A.6.2.8.2	Test Requirements	1391

A.6.2.8.2.1	Random Access Response Reception	1391

A.6.2.8.2.2	No Random Access Response Reception	1391

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1391

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1392

A.6.2.9.1	Test Purpose and Environment	1392

A.6.2.9.2	Test Requirements	1395

A.6.2.9.2.1	Random Access Response Reception	1395

A.6.2.9.2.2	No Random Access Response Reception	1396

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1396

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1397

A.6.2.10.1	Test Purpose and Environment	1397

A.6.2.10.2	Test Requirements	1399

A.6.2.10.2.1	Random Access Response Reception	1399

A.6.2.10.2.2	No Random Access Response Reception	1400

A.6.2.10.2.3	Receiving a NACK on msg3	1400

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1400

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1400

A.6.2.10.2.6	Contention Resolution Timer expiry	1400

A.6.2.10.2.7	PRACH Resource Selection	1401

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1401

A.6.2.11.1	Test Purpose and Environment	1401

A.6.2.11.2	Test Requirements	1403

A.6.2.11.2.1	Random Access Response Reception	1403

A.6.2.11.2.2	No Random Access Response Reception	1404

A.6.2.11.2.3	Receiving a NACK on msg3	1404

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1404

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1404

A.6.2.11.2.6	Contention Resolution Timer expiry	1404

A.6.2.11.2.7	PRACH Resource Selection	1405

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1405

A.6.2.12.1	Test Purpose and Environment	1405

A.6.2.12.2	Test Requirements	1407

A.6.2.12.2.1	Random Access Response Reception	1407

A.6.2.12.2.2	No Random Access Response Reception	1408

A.6.2.12.2.3	Receiving a NACK on msg3	1408

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1408

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1408

A.6.2.12.2.6	Contention Resolution Timer expiry	1408

A.6.2.12.2.7	PRACH Resource Selection	1409

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1409

A.6.2.13.1	Test Purpose and Environment	1409

A.6.2.13.2	Test Requirements	1411

A.6.2.13.2.1	Random Access Response Reception	1411

A.6.2.13.2.2	No Random Access Response Reception	1412

A.6.2.13.2.3	Receiving a NACK on msg3	1412

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1412

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1412

A.6.2.13.2.6	Contention Resolution Timer expiry	1412

A.6.2.13.2.7	PRACH Resource Selection	1413

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1413

A.6.2.14.1	Test Purpose and Environment	1413

A.6.2.14.2	Test Requirements	1415

A.6.2.14.2.1	Random Access Response Reception	1415

A.6.2.14.2.2	No Random Access Response Reception	1416

A.6.2.14.2.3	Receiving a NACK on msg3	1416

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1416

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1416

A.6.2.14.2.6	Contention Resolution Timer expiry	1416

A.6.2.14.2.7	PRACH Resource Selection	1417

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1417

A.6.2.15.1	Test Purpose and Environment	1417

A.6.2.15.2	Test Requirements	1419

A.6.2.15.2.1	Random Access Response Reception	1419

A.6.2.15.2.2	No Random Access Response Reception	1420

A.6.2.15.2.3	Receiving a NACK on msg3	1420

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1420

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1420

A.6.2.15.2.6	Contention Resolution Timer expiry	1420

A.6.2.15.2.7	PRACH Resource Selection	1421

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1421

A.6.2.16.1	Test Purpose and Environment	1421

A.6.2.16.2	Test Requirements	1424

A.6.2.16.2.1	Random Access Response Reception	1424

A.6.2.16.2.2	No Random Access Response Reception	1425

A.6.2.16.2.3	Receiving a NACK on msg3	1425

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1425

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1425

A.6.2.16.2.6	Contention Resolution Timer expiry	1425

A.6.2.16.2.7	NPRACH Resource Selection	1425

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1426

A.6.2.17.1	Test Purpose and Environment	1426

A.6.2.17.2	Test Requirements	1429

A.6.2.17.2.1	Random Access Response Reception	1429

A.6.2.17.2.2	No Random Access Response Reception	1430

A.6.2.17.2.3	Receiving a NACK on msg3	1430

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1430

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1430

A.6.2.17.2.6	Contention Resolution Timer expiry	1430

A.6.2.17.2.7	NPRACH Resource Selection	1430

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1431

A.6.2.18.1	Test Purpose and Environment	1431

A.6.2.18.2	Test Requirements	1434

A.6.2.18.2.1	Random Access Response Reception	1434

A.6.2.18.2.2	No Random Access Response Reception	1435

A.6.2.18.2.3	Receiving a NACK on msg3	1435

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1435

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1435

A.6.2.18.2.6	Contention Resolution Timer expiry	1435

A.6.2.18.2.7	NPRACH Resource Selection	1435

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1436

A.6.2.19.1	Test Purpose and Environment	1436

A.6.2.19.2	Test Requirements	1438

A.6.2.19.2.1	Random Access Response Reception	1438

A.6.2.19.2.2	No Random Access Response Reception	1439

A.6.2.19.2.3	Receiving a NACK on msg3	1439

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1439

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1439

A.6.2.19.2.6	Contention Resolution Timer expiry	1439

A.6.2.19.2.7	NPRACH Resource Selection	1439

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1440

A.6.2.20.1	Test Purpose and Environment	1440

A.6.2.20.2	Test Requirements	1442

A.6.2.20.2.1	Random Access Response Reception	1442

A.6.2.20.2.2	No Random Access Response Reception	1443

A.6.2.20.2.3	Receiving a NACK on msg3	1443

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1443

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1443

A.6.2.20.2.6	Contention Resolution Timer expiry	1443

A.6.2.20.2.7	NPRACH Resource Selection	1443

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1444

A.6.2.21.1	Test Purpose and Environment	1444

A.6.2.21.2	Test Requirements	1446

A.6.2.21.2.1	Random Access Response Reception	1446

A.6.2.21.2.2	No Random Access Response Reception	1447

A.6.2.21.2.3	Receiving a NACK on msg3	1447

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1447

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1447

A.6.2.21.2.6	Contention Resolution Timer expiry	1447

A.6.2.21.2.7	NPRACH Resource Selection	1447

A.6.3	RRC Connection Release with Redirection	1448

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1448

A.6.3.1.1	Test Purpose and Environment	1448

A.6.3.1.2	Test Requirements	1450

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1450

A.6.3.2.1	Test Purpose and Environment	1450

A.6.3.2.2	Test Requirements	1452

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1452

A.6.3.3.1	Test Purpose and Environment	1452

A.6.3.3.2	Test Requirements	1453

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1454

A.6.3.4.1	Test Purpose and Environment	1454

A.6.3.4.2	Test Requirements	1455

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1456

A.6.3.5.1	Test Purpose and Environment	1456

A.6.3.5.2	Test Requirements	1458

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1458

A.6.3.6.1	Test Purpose and Environment	1458

A. 6.3.6.2	Test Requirements	1461

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1461

A.6.3.7.1	Test Purpose and Environment	1461

A.6.3.7.2	Test Requirements	1464

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1464

A.6.3.8.1	Test Purpose and Environment	1464

A.6.3.8.2	Test Requirements	1467

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1467

A.6.3.9.1	Test Purpose and Environment	1467

A.6.3.9.2	Test Requirements	1469

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1469

A.6.3.10.1	Test Purpose and Environment	1469

A.6.3.10.2	Test Requirements	1471

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1471

A.6.3.11.1	Test Purpose and Environment	1471

A.6.3.11.2	Test Requirements	1473

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1473

A.6.3.12.1	Test Purpose and Environment	1473

A.6.3.12.2	Test Requirements	1476

A.7	Timing and Signalling Characteristics	1478

A.7.1	UE Transmit Timing	1478

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1478

A.7.1.1.1	Test Purpose and Environment	1478

A.7.1.1.2	Test Requirements	1479

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1480

A.7.1.2.1	Test Purpose and Environment	1480

A.7.1.2.2	Test Requirements	1482

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1483

A.7.1.3.1	Test Purpose and Environment	1483

A.7.1.3.2	Test Requirements	1485

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1486

A.7.1.4.1	Test Purpose and Environment	1486

A.7.1.4.2	Test Requirements	1487

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1488

A.7.1.4A.1	Test Purpose and Environment	1488

A.7.1.4A.2	Test Requirements	1488

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1488

A.7.1.5.1	Test Purpose and Environment	1488

A.7.1.5.2	Test Requirements	1489

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1489

A.7.1.6.1	Test Purpose and Environment	1489

A.7.1.6.2	Test Requirements	1491

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1492

A.7.1.7.1	Test Purpose and Environment	1492

A.7.1.7.2	Test Requirements	1493

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1494

A.7.1.7A.1	Test Purpose and Environment	1494

A.7.1.7A.2	Test Requirements	1494

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1494

A.7.1.7B.1	Test Purpose and Environment	1494

A.7.1.7B.2	Test Requirements	1494

A.7.1.8	Void	1495

A.7.1.8.1	Void	1495

A.7.1.8.2	Void	1495

A.7.1.9	Void	1495

A.7.1.9.1	Void	1495

A.7.1.9.2	Void	1495

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1495

A.7.1.10.1	Test Purpose and Environment	1495

A.7.1.10.2	Test Requirements	1497

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1497

A.7.1.11.1	Test Purpose and Environment	1497

A.7.1.11.2	Test Requirements	1499

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1499

A.7.1.12.1	Test Purpose and Environment	1499

A.7.1.12.2	Test Requirements	1501

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1501

A.7.1.13.1	Test Purpose and Environment	1501

A.7.1.13.2	Test Requirements	1503

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1504

A.7.1.14.1	Test Purpose and Environment	1504

A.7.1.14.2	Test Requirements	1505

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1505

A.7.1.15.1	Test Purpose and Environment	1505

A.7.1.15.2	Test Requirements	1506

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1507

A.7.1.16.1	Test Purpose and Environment	1507

A.7.1.16.2	Test Requirements	1508

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1509

A.7.1.17.1	Test Purpose and Environment	1509

A.7.1.17.2	Test Requirements	1511

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1512

A.7.1.18.1	Test Purpose and Environment	1512

A.7.1.18.2	Test Requirements	1514

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1515

A.7.1.19.1	Test Purpose and Environment	1515

A.7.1.19.2	Test Requirements	1516

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1516

A.7.1.20.1	Test Purpose and Environment	1516

A.7.1.20.2	Test Requirements	1518

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1518

A.7.1.21.1	Test Purpose and Environment	1518

A.7.1.21.2	Test Requirements	1520

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1520

A.7.1.22.1	Test Purpose and Environment	1520

A.7.1.22.2	Test Requirements	1522

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1522

A.7.1.23.1	Test Purpose and Environment	1522

A.7.1.23.2	Test Requirements	1524

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1524

A.7.1.24.1	Test Purpose and Environment	1524

A.7.1.24.2	Test Requirements	1525

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1526

A.7.1.25.1	Test Purpose and Environment	1526

A.7.1.25.2	Test Requirements	1527

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1528

A.7.1.26.1	Test Purpose and Environment	1528

A.7.1.26.2	Test Requirements	1529

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1530

A.7.1.27.1	Test Purpose and Environment	1530

A.7.1.27.2	Test Requirements	1532

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1532

A.7.1.28.1	Test Purpose and Environment	1532

A.7.1.28.2	Test Requirements	1534

A.7.2	UE Timing Advance	1535

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1535

A.7.2.1.1	Test Purpose and Environment	1535

A.7.2.1.2	Test Requirements	1536

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1537

A.7.2.2.1	Test Purpose and Environment	1537

A.7.2.2.2	Test Requirements	1539

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1539

A.7.2.3.1	Test Purpose and Environment	1539

A.7.2.3.2	Test Requirements	1539

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1539

A.7.2.4.1	Test Purpose and Environment	1539

A.7.2.4.2	Test Requirements	1542

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1542

A.7.2.5.1	Test Purpose and Environment	1542

A.7.2.5.2	Test Requirements	1544

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1544

A.7.2.5A.1	Test Purpose and Environment	1544

A.7.2.5A.2	Test Requirements	1545

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1545

A.7.2.5B.1	Test Purpose and Environment	1545

A.7.2.5B.2	Test Requirements	1545

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1545

A.7.2.6.1	Test Purpose and Environment	1545

A.7.2.6.2	Test Requirements	1548

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1548

A.7.2.7.1	Test Purpose and Environment	1548

A.7.2.7.2	Test Requirements	1550

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1550

A.7.2.8.1	Test Purpose and Environment	1550

A.7.2.8.2	Test Requirements	1552

A.7.2.9.2	Test Requirements	1553

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1554

A.7.2.10.1	Test Purpose and Environment	1554

A.7.2.10.2	Test Requirements	1555

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1555

A.7.2.11.1	Test Purpose and Environment	1555

A.7.2.11.2	Test Requirements	1557

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1557

A.7.2.12.1	Test Purpose and Environment	1557

A.7.2.12.2	Test Requirements	1559

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1560

A.7.2.13.1	Test Purpose and Environment	1560

A.7.2.13.2	Test Requirements	1561

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1562

A.7.2.14.1	Test Purpose and Environment	1562

A.7.2.14.2	Test Requirements	1564

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1564

A.7.2.15.1	Test Purpose and Environment	1564

A.7.2.15.2	Test Requirements	1566

A.7.3	Radio Link Monitoring	1566

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1566

A.7.3.1.1	Test Purpose and Environment	1566

A.7.3.1.2	Test Requirements	1570

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1570

A.7.3.2.1	Test Purpose and Environment	1570

A.7.3.2.2	Test Requirements	1574

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1574

A.7.3.3.1	Test Purpose and Environment	1574

A.7.3.3.2	Test Requirements	1578

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1578

A.7.3.4.1	Test Purpose and Environment	1578

A.7.3.4.2	Test Requirements	1582

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1582

A.7.3.5.1	Test Purpose and Environment	1582

A.7.3.5.2	Test Requirements	1585

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1586

A.7.3.6.1	Test Purpose and Environment	1586

A.7.3.6.2	Test Requirements	1589

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1589

A.7.3.7.1	Test Purpose and Environment	1589

A.7.3.7.2	Test Requirements	1592

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1592

A.7.3.8.1	Test Purpose and Environment	1592

A.7.3.8.2	Test Requirements	1595

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1595

A.7.3.9.2	Test Requirements	1598

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1598

A.7.3.10.1	Test Purpose and Environment	1598

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1602

A.7.3.11.1	Test Purpose and Environment	1602

A.7.3.11.2	Test Requirements	1607

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1607

A.7.3.12.1	Test Purpose and Environment	1607

A.7.3.12.2	Test Requirements	1612

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1612

A.7.3.13.1	Test Purpose and Environment	1612

A.7.3.13.2	Test Requirements	1615

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1615

A.7.3.14.1	Test Purpose and Environment	1615

A.7.3.14.2	Test Requirements	1618

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1618

A.7.3.15.1	Test Purpose and Environment	1618

A.7.3.15.2	Test Requirements	1622

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1622

A.7.3.16.1	Test Purpose and Environment	1622

A.7.3.16.2	Test Requirements	1627

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1627

A.7.3.17.1	Test Purpose and Environment	1627

A.7.3.17.2	Test Requirements	1631

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1631

A.7.3.18.1	Test Purpose and Environment	1631

A.7.3.18.2	Test Requirements	1635

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1635

A.7.3.19.1	Test Purpose and Environment	1635

A.7.3.19.2	Test Requirements	1640

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1640

A.7.3.20.1	Test Purpose and Environment	1640

A.7.3.20.2	Test Requirements	1645

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1645

A.7.3.21.1	Test Purpose and Environment	1645

A.7.3.21.2	Test Requirements	1650

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1650

A.7.3.22.1	Test Purpose and Environment	1650

A.7.3.22.2	Test Requirements	1655

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1655

A.7.3.23.1	Test Purpose and Environment	1655

A.7.3.23.2	Test Requirements	1656

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1656

A.7.3.24.1	Test Purpose and Environment	1656

A.7.3.24.2	Test Requirements	1657

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1657

A.7.3.25.1	Test Purpose and Environment	1657

A.7.3.25.2	Test Requirements	1658

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1658

A.7.3.26.1	Test Purpose and Environment	1658

A.7.3.26.2	Test Requirements	1661

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1661

A.7.3.27.1	Test Purpose and Environment	1661

A.7.3.27.2	Test Requirements	1664

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1664

A.7.3.28.1	Test Purpose and Environment	1664

A.7.3.28.2	Test Requirements	1667

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1667

A.7.3.29.1	Test Purpose and Environment	1667

A.7.3.29.2	Test Requirements	1670

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1670

A.7.3.30.1	Test Purpose and Environment	1670

A.7.3.30.2	Test Requirements	1673

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1673

A.7.3.31.1	Test Purpose and Environment	1673

A.7.3.31.2	Test Requirements	1676

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1676

A.7.3.32.1	Test Purpose and Environment	1676

A.7.3.32.2	Test Requirements	1679

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1679

A.7.3.33.1	Test Purpose and Environment	1679

A.7.3.33.2	Test Requirements	1682

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1682

A.7.3.34.1	Test Purpose and Environment	1682

A.7.3.34.2	Test Requirements	1685

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1685

A.7.3.35.1	Test Purpose and Environment	1685

A.7.3.35.2	Test Requirements	1688

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1688

A.7.3.36.1	Test Purpose and Environment	1688

A.7.3.36.2	Test Requirements	1691

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1691

A.7.3.37.1	Test Purpose and Environment	1691

A.7.3.37.2	Test Requirements	1694

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1694

A.7.3.38.1	Test Purpose and Environment	1694

A.7.3.38.2	Test Requirements	1697

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1697

A.7.3.39.1	Test Purpose and Environment	1697

A.7.3.39.2	Test Requirements	1700

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1701

A.7.3.40.1	Test Purpose and Environment	1701

A.7.3.40.2	Test Requirements	1703

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1704

A.7.3.41.1	Test Purpose and Environment	1704

A.7.3.41.2	Test Requirements	1707

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1707

A.7.3.42.1	Test Purpose and Environment	1707

A.7.3.42.2	Test Requirements	1710

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1710

A.7.3.43.1	Test Purpose and Environment	1710

A.7.3.43.2	Test Requirements	1713

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1713

A.7.3.44.1	Test Purpose and Environment	1713

A.7.3.44.2	Test Requirements	1716

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1717

A.7.3.45.1	Test Purpose and Environment	1717

A.7.3.45.2	Test Requirements	1719

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1720

A.7.3.46.1	Test Purpose and Environment	1720

A.7.3.46.2	Test Requirements	1723

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1723

A.7.3.47.1	Test Purpose and Environment	1723

A.7.3.47.2	Test Requirements	1726

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1726

A.7.3.48.1	Test Purpose and Environment	1726

A.7.3.48.2	Test Requirements	1729

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1729

A.7.3.49.1	Test Purpose and Environment	1729

A.7.3.49.2	Test Requirements	1732

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1732

A.7.3.50.1	Test Purpose and Environment	1732

A.7.3.50.2	Test Requirements	1735

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1735

A.7.3.51.1	Test Purpose and Environment	1735

A.7.3.51.2	Test Requirements	1738

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1738

A.7.3.52.1	Test Purpose and Environment	1738

A.7.3.52.2	Test Requirements	1741

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1741

A.7.3.53.1	Test Purpose and Environment	1741

A.7.3.53.2	Test Requirements	1744

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1744

A.7.3.54.1	Test Purpose and Environment	1744

A.7.3.54.2	Test Requirements	1747

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1747

A.7.3.55.1	Test Purpose and Environment	1747

A.7.3.55.2	Test Requirements	1750

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1750

A.7.3.56.1	Test Purpose and Environment	1750

A.7.3.56.2	Test Requirements	1753

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1753

A.7.3.57.1	Test Purpose and Environment	1753

A.7.3.57.2	Test Requirements	1756

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1756

A.7.3.58.1	Test Purpose and Environment	1756

A.7.3.58.2	Test Requirements	1759

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1759

A.7.3.59.1	Test Purpose and Environment	1759

A.7.3.59.2	Test Requirements	1762

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1762

A.7.3.60.1	Test Purpose and Environment	1762

A.7.3.60.2	Test Requirements	1766

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1766

A.7.3.61.1	Test Purpose and Environment	1766

A.7.3.61.2	Test Requirements	1770

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1770

A.7.3.62.1	Test Purpose and Environment	1770

A.7.3.62.2	Test Requirements	1774

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1774

A.7.3.63.1	Test Purpose and Environment	1774

A.7.3.63.2	Test Requirements	1778

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1778

A.7.3.64.1	Test Purpose and Environment	1778

A.7.3.64.2	Test Requirements	1781

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1782

A.7.3.65.1	Test Purpose and Environment	1782

A.7.3.65.2	Test Requirements	1785

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1786

A.7.3.66.1	Test Purpose and Environment	1786

A.7.3.66.2	Test Requirements	1788

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1789

A.7.3.67.1	Test Purpose and Environment	1789

A.7.3.67.2	Test Requirements	1792

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1793

A.7.3.68.1	Test Purpose and Environment	1793

A.7.3.68.2	Test Requirements	1795

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1795

A.7.3.69.1	Test Purpose and Environment	1795

A.7.3.69.2	Test Requirements	1797

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1797

A.7.3.70.1	Test Purpose and Environment	1797

A.7.3.70.2	Test Requirements	1799

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1799

A.7.3.71.1	Test Purpose and Environment	1799

A.7.3.71.2	Test Requirements	1802

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1802

A.7.3.72.1	Test Purpose and Environment	1802

A.7.3.72.2	Test Requirements	1804

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1804

A.7.3.73.1	Test Purpose and Environment	1804

A.7.3.73.2	Test Requirements	1806

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1806

A.7.3.74.1	Test Purpose and Environment	1806

A.7.3.74.2	Test Requirements	1809

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1809

A.7.3.75.1	Test Purpose and Environment	1809

A.7.3.75.2	Test Requirements	1811

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1812

A.7.3.76.1	Test Purpose and Environment	1812

A.7.3.76.2	Test Requirements	1815

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1815

A.7.3.77.1	Test Purpose and Environment	1815

A.7.3.77.2	Test Requirements	1818

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1818

A.7.3.78.1	Test Purpose and Environment	1818

A.7.3.78.2	Test Requirements	1821

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1821

A.7.3.79.1	Test Purpose and Environment	1821

A.7.3.79.2	Test Requirements	1824

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1824

A.7.3.80.1	Test Purpose and Environment	1824

A.7.3.80.2	Test Requirements	1827

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1827

A.7.3.81.1	Test Purpose and Environment	1827

A.7.3.81.2	Test Requirements	1830

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1830

A.7.3.82.1	Test Purpose and Environment	1830

A.7.3.82.2	Test Requirements	1833

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1833

A.7.3.83.1	Test Purpose and Environment	1833

A.7.3.83.2	Test Requirements	1834

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1835

A.7.3.84.1	Test Purpose and Environment	1835

A.7.3.84.2	Test Requirements	1837

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1837

A.7.3.85.1	Test Purpose and Environment	1837

A.7.3.85.2	Test Requirements	1838

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1839

A.7.3.86.1	Test Purpose and Environment	1839

A.7.3.86.2	Test Requirements	1841

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1841

A.7.3.87.1	Test Purpose and Environment	1841

A.7.3.87.2	Test Requirements	1843

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1844

A.7.3.88.1	Test Purpose and Environment	1844

A.7.3.88.2	Test Requirements	1848

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1848

A.7.3.89.1	Test Purpose and Environment	1848

A.7.3.89.2	Test Requirements	1852

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1852

A.7.3.90.1	Test Purpose and Environment	1852

A.7.3.90.2	Test Requirements	1856

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1856

A.7.3.91.1	Test Purpose and Environment	1856

A.7.3.91.2	Test Requirements	1860

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1860

A.7.3.92.1	Test Purpose and Environment	1860

A.7.3.92.2	Test Requirements	1863

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1864

A.7.3.93.1	Test Purpose and Environment	1864

A.7.3.93.2	Test Requirements	1867

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1868

A.7.3.94.1	Test Purpose and Environment	1868

A.7.3.94.2	Test Requirements	1870

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1871

A.7.3.95.1	Test Purpose and Environment	1871

A.7.3.95.2	Test Requirements	1874

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1874

A.7.3.96.1	Test Purpose and Environment	1874

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1876

A.7.3.97.1	Test Purpose and Environment	1876

A.7.3.97.2	Test Requirements	1879

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1879

A.7.3.98.1	Test Purpose and Environment	1879

A.7.3.98.2	Test Requirements	1882

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1882

A.7.3.99.1	Test Purpose and Environment	1882

A.7.3.99.2	Test Requirements	1885

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1885

A.7.3.100.1	Test Purpose and Environment	1885

A.7.3.100.2	Test Requirements	1887

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1887

A.7.3.101.1	Test Purpose and Environment	1887

A.7.3.101.2	Test Requirements	1889

A.7.4	Interruption for Dual Connectivity	1889

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1889

A.7.4.1.1	Test Purpose and Environment	1889

A.7.4.1.2	Test Requirements	1892

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1892

A.7.4.2.1	Test Purpose and Environment	1892

A.7.4.2.2	Test Requirements	1894

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1894

A.7.4.3.1	Test Purpose and Environment	1894

A.7.4.3.2	Test Requirements	1896

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1896

A.7.4.4.1	Test Purpose and Environment	1896

A.7.4.4.2	Test Requirements	1899

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1899

A.7.4.5.1	Test Purpose and Environment	1899

A.7.4.5.2	Test Requirements	1901

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1901

A.7.4.6.1	Test Purpose and Environment	1901

A.7.4.6.2	Test Requirements	1903

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1903

A.7.4.7.1	Test Purpose and Environment	1903

A.7.4.7.2	Test Requirements	1906

A.7.5	Proximity-based Services	1906

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1906

A.7.5.1.1	Test Purpose and Environment	1906

A.7.5.1.2	Test Requirements	1907

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1908

A.7.5.2.1	Test Purpose and Environment	1908

A.7.5.1.2	Test Requirements	1908

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1909

A.7.5.3.1	Test Purpose and Environment	1909

A.7.5.3.2	Test Requirements	1910

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1911

A.7.5.4.1	Test Purpose and Environment	1911

A.7.5.4.2	Test Requirements	1912

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1913

A.7.5.5.1	Test Purpose and Environment	1913

A.7.5.5.2	Test Requirements	1915

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1916

A.7.5.6.1	Test Purpose and Environment	1916

A.7.5.6.2	Test Requirements	1917

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1918

A.7.5.7.1	Test Purpose and Environment	1918

A.7.5.7.2	Test Requirements	1920

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1920

A.7.5.8.1	Test Purpose and Environment	1920

A.7.5.8.2	Test Requirements	1922

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1923

A.7.5.9.1	Test Purpose and Environment	1923

A.7.5.9.2	Test Requirements	1925

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1926

A.7.5.10.1	Test Purpose and Environment	1926

A.7.5.10.2	Test Requirements	1928

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1929

A.7.5.11.1	Test Purpose and Environment	1929

A.7.5.11.2	Test Requirements	1931

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1931

A.7.5.12.1	Test Purpose and Environment	1931

A.7.5.12.2	Test Requirements	1935

A.7.6	Interruption for carrier aggregation	1936

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1936

A.7.6.1.1	Test Purpose and Environment	1936

A.7.6.1.2	Test Requirements	1939

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1939

A.7.6.2.1	Test Purpose and Environment	1939

A.7.6.2.2	Test Requirements	1942

A.8	UE Measurements Procedures	1942

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1942

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1942

A.8.1.1.1	Test Purpose and Environment	1942

A.8.1.1.2	Test Requirements	1944

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1944

A.8.1.2.1	Test Purpose and Environment	1944

A.8.1.2.2	Test Requirements	1946

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1946

A.8.1.3.1	Test Purpose and Environment	1946

A.8.1.3.2	Test Requirements	1949

A.8.1.4	Void	1949

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1949

A.8.1.5.1	Test Purpose and Environment	1949

A.8.1.5.2	Test Requirements	1951

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1951

A.8.1.6.1	Test Purpose and Environment	1951

A.8.1.6.2	Test Requirements	1953

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1953

A.8.1.7.1	Test Purpose and Environment	1953

A.8.1.7.2	Test Requirements	1955

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1956

A.8.1.8.1	Test Purpose and Environment	1956

A.8.1.8.2	Test Requirements	1958

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1959

A.8.1.9.1	Test Purpose and Environment	1959

A.8.1.9.2	Test Requirements	1959

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1959

A.8.1.10.1	Test Purpose and Environment	1959

A.8.1.10.2	Test Requirements	1960

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1960

A.8.1.11.1	Test Purpose and Environment	1960

A.8.1.11.2	Test Requirements	1962

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1963

A.8.1.12.1	Test Purpose and Environment	1963

A.8.1.12.2	Test Requirements	1964

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1965

A.8.1.13.1	Test Purpose and Environment	1965

A.8.1.13.2	Test Requirements	1967

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1967

A.8.1.14.1	Test Purpose and Environment	1967

A.8.1.14.2	Test Requirements	1969

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1970

A.8.1.15.1	Test Purpose and Environment	1970

A.8.1.15.2	Test Requirements	1971

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1972

A.8.1.16.1	Test Purpose and Environment	1972

A.8.1.16.2	Test Requirements	1974

A.8.1.17	Void	1974

A.8.1.18	Void	1974

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1974

A.8.1.19.1	Test Purpose and Environment	1974

A.8.1.19.2	Test Requirements	1976

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1977

A.8.1.20.1	Test Purpose and Environment	1977

A.8.1.20.2	Test Requirements	1979

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1979

A.8.1.21.1	Test Purpose and Environment	1979

A.8.1.21.2	Test Requirements	1981

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1982

A.8.1.22.1	Test Purpose and Environment	1982

A.8.1.22.2	Test Requirements	1984

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1984

A.8.1.23.1	Test Purpose and Environment	1984

A.8.1.23.2	Test Requirements	1985

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1986

A.8.1.24.1	Test Purpose and Environment	1986

A.8.1.24.2	Test Requirements	1988

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1988

A.8.1.25.1	Test Purpose and Environment	1988

A.8.1.25.2	Test Requirements	1990

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1990

A.8.1.26.1	Test Purpose and Environment	1990

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1993

A.8.1.27.1	Test Purpose and Environment	1993

A.8.1.27.2	Test Requirements	1995

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1995

A.8.1.28.1	Test Purpose and Environment	1995

A.8.1.28.2	Test Requirements	1997

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1997

A.8.1.29.1	Test Purpose and Environment	1997

A.8.1.29.2	Test Requirements	1999

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2000

A.8.1.30.1	Test Purpose and Environment	2000

A.8.1.30.2	Test Requirements	2002

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2002

A.8.1.31.1	Test Purpose and Environment	2002

A.8.1.31.2	Test Requirements	2004

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2005

A.8.1.32.1	Test Purpose and Environment	2005

A.8.1.32.2	Test Requirements	2006

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2007

A.8.1.33.1	Test Purpose and Environment	2007

A.8.1.33.2	Test Requirements	2008

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2009

A.8.1.34.1	Test Purpose and Environment	2009

A.8.1.34.2	Test Requirements	2010

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2011

A.8.1.35.1	Test Purpose and Environment	2011

A.8.1.35.2	Test Requirements	2012

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2013

A.8.1.36.1	Test Purpose and Environment	2013

A.8.1.36.2	Test Requirements	2014

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2015

A.8.1.37.1	Test Purpose and Environment	2015

A.8.1.37.2	Test Requirements	2017

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2017

A.8.1.38.1	Test Purpose and Environment	2017

A.8.1.38.2	Test Requirements	2018

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2019

A.8.1.39.1	Test Purpose and Environment	2019

A.8.1.39.2	Test Requirements	2021

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2021

A.8.1.40.1	Test Purpose and Environment	2021

A.8.1.40.2	Test Requirements	2023

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2023

A.8.1.41.1	Test Purpose and Environment	2023

A.8.1.41.2	Test Requirement	2025

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2025

A.8.1.42.1	Test Purpose and Environment	2025

A.8.1.42.2	Test Requirement	2026

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2026

A.8.1.43.1	Test Purpose and Environment	2026

A.8.1.43.2	Test Requirements	2029

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2029

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2029

A.8.2.1.1	Test Purpose and Environment	2029

A.8.2.1.2	Test Requirements	2031

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2032

A.8.2.2.1	Test Purpose and Environment	2032

A.8.2.2.2	Test Requirements	2034

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2034

A.8.2.3.1	Test Purpose and Environment	2034

A.8.2.3.2	Test Requirements	2036

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2037

A.8.2.4.1	Test Purpose and Environment	2037

A.8.2.4.2	Test Requirements	2039

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2039

A.8.2.5.1	Test Purpose and Environment	2039

A.8.2.5.2	Test Requirements	2041

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2042

A.8.2.6.1	Test Purpose and Environment	2042

A.8.2.6.2	Test Requirements	2045

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2046

A.8.2.7.1	Test Purpose and Environment	2046

A.8.2.7.2	Test Requirements	2047

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2048

A.8.2.8.1	Test Purpose and Environment	2048

A.8.2.8.2	Test Requirements	2050

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2050

A.8.2.9.1	Test Purpose and Environment	2050

A.8.2.9.2	Test Requirements	2052

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2053

A.8.2.10.1	Test Purpose and Environment	2053

A.8.2.10.2	Test Requirements	2055

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2055

A.8.2.11.1	Test Purpose and Environment	2055

A.8.2.11.2	Test Requirements	2057

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2057

A.8.2.12.1	Test Purpose and Environment	2057

A.8.2.12.2	Test Requirements	2059

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2060

A.8.2.13.1	Test Purpose and Environment	2060

A.8.2.13.2	Test Requirements	2062

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2062

A.8.2.14.1	Test Purpose and Environment	2062

A.8.2.14.2	Test Requirement	2064

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2064

A.8.2.15.1	Test Purpose and Environment	2064

A.8.2.15.2	Test Requirements	2066

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2066

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2066

A.8.3.1.1	Test Purpose and Environment	2066

A.8.3.1.2	Test Requirements	2068

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2068

A.8.3.2.1	Test Purpose and Environment	2068

A.8.3.2.2	Test Requirements	2071

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2071

A.8.3.3.1	Test Purpose and Environment	2071

A.8.3.3.2	Test Requirements	2074

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2074

A.8.3.4.1	Test Purpose and Environment	2074

A.8.3.4.2	Test Requirements	2076

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2076

A.8.3.5.2	Test Requirements	2078

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2078

A.8.3.6.1	Test Purpose and Environment	2078

A.8.3.6.2	Test Requirements	2080

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2081

A.8.3.7.1	Test Purpose and Environment	2081

A.8.3.7.2	Test Requirements	2083

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2084

A.8.3.8.1	Test Purpose and Environment	2084

A.8.3.8.2	Test Requirements	2087

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2087

A.8.3.9.1	Test Purpose and Environment	2087

A.8.3.9.2	Test Requirements	2091

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2092

A.8.3.10.1	Test Purpose and Environment	2092

A.8.3.10.2	Test Requirements	2093

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2093

A.8.3.11.1	Test Purpose and Environment	2093

A.8.3.11.2	Test Requirement	2095

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2096

A.8.3.12.1	Test Purpose and Environment	2096

A.8.3.12.2	Test Requirement	2097

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2098

A.8.3.13.1	Test Purpose and Environment	2098

A.8.3.13.2	Test Requirement	2099

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2100

A.8.3.14.1	Test Purpose and Environment	2100

A.8.3.14.2	Test Requirement	2101

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2102

A.8.3.15.1	Test Purpose and Environment	2102

A.8.3.15.2	Test Requirement	2103

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2104

A.8.3.16.1	Test Purpose and Environment	2104

A.8.3.16.2	Test Requirement	2106

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2106

A.8.3.17.1	Test Purpose and Environment	2106

A.8.3.17.2	Test Requirement	2109

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2109

A.8.3.18.1	Test Purpose and Environment	2109

A.8.3.18.2	Test Requirement	2112

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2112

A.8.3.19.1	Test Purpose and Environment	2112

A.8.3.19.2	Test Requirement	2115

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2115

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2115

A.8.4.1.1	Test Purpose and Environment	2115

A.8.4.1.2	Test Requirements	2117

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2117

A.8.4.2.1	Test Purpose and Environment	2117

A.8.4.2.2	Test Requirements	2120

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2120

A.8.4.3.1	Test Purpose and Environment	2120

A.8.4.3.2	Test Requirements	2122

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2123

A.8.4.4.1	Test Purpose and Environment	2123

A.8.4.4.2	Test Requirements	2125

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2126

A.8.4.5.1	Test Purpose and Environment	2126

A.8.4.5.2	Test Requirements	2128

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2128

A.8.4.6.1	Test Purpose and Environment	2128

A.8.4.6.2	Test Requirements	2129

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2129

A.8.4.7.1	Test Purpose and Environment	2129

A.8.4.7.2	Test Requirements	2133

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2133

A.8.4.8.1	Test Purpose and Environment	2133

A.8.4.8.2	Test Requirements	2136

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2136

A.8.4.9.1	Test Purpose and Environment	2136

A.8.4.9.2	Test Requirements	2141

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2141

A.8.4.10.1	Test Purpose and Environment	2141

A.8.4.10.2	Test Requirements	2143

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2143

A.8.4.11.1	Test Purpose and Environment	2143

A.8.4.11.2	Test Requirement	2145

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2145

A.8.4.12.1	Test Purpose and Environment	2145

A.8.4.12.2	Test Requirement	2147

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2148

A.8.4.13.1	Test Purpose and Environment	2148

A.8.4.13.2	Test Requirement	2149

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2150

A.8.4.14.1	Test Purpose and Environment	2150

A.8.4.14.2	Test Requirement	2152

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2152

A.8.4.15.1	Test Purpose and Environment	2152

A.8.4.15.2	Test Requirement	2155

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2155

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2155

A.8.5.1.1	Test Purpose and Environment	2155

A.8.5.1.2	Test Requirements	2157

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2157

A.8.5.2.1	Test Purpose and Environment	2157

A.8.5.2.2	Test Requirements	2159

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2159

A.8.5.3.1	Test Purpose and Environment	2159

A.8.5.3.2	Test Requirements	2162

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2162

A.8.5.4.1	Test Purpose and Environment	2162

A.8.5.4.2	Test Requirements	2164

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2164

A.8.5.5.1	Test Purpose and Environment	2164

A.8.5.5.2	Test Requirements	2167

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2167

A.8.5.6.1	Test Purpose and Environment	2167

A.8.5.6.2	Test Requirements	2168

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2169

A.8.5.7.1	Test Purpose and Environment	2169

A.8.5.7.2	Test Requirements	2169

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2169

A.8.5.8.1	Test Purpose and Environment	2169

A.8.5.8.2	Test Requirements	2172

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2172

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2172

A.8.6.1.1	Test Purpose and Environment	2172

A.8.6.1.2	Test Requirements	2174

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2174

A.8.6.2.1	Test Purpose and Environment	2174

A.8.6.2.2	Test Requirements	2177

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2177

A.8.6.3.1	Test Purpose and Environment	2177

A.8.6.3.2	Test Requirements	2180

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2180

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2180

A.8.7.1.1	Test Purpose and Environment	2180

A.8.7.1.1.1	Void	2180

A.8.7.1.1.2	1.28 Mcps TDD option	2180

A.8.7.1.1.3	Void	2182

A.8.7.1.2	Test Requirements	2182

A.8.7.1.2.1	Void	2182

A.8.7.1.2.2	1.28 Mcps TDD option	2182

A.8.7.1.2.3	Void	2182

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2182

A.8.7.2.1	Test Purpose and Environment	2182

A.8.7.2.2	Test Requirements	2185

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2186

A.8.7.3.1	Test Purpose and Environment	2186

A.8.7.3.2	Test Parameters	2186

A.8.7.3.3	Test Requirements	2187

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2188

A.8.7.4.1	Test Purpose and Environment	2188

A.8.7.4.2	Test Requirements	2190

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2190

A.8.7.5.1	Test Purpose and Environment	2190

A.8.7.5.2	Test Requirements	2192

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2192

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2192

A.8.7A.1.1	Test Purpose and Environment	2192

A.8.7A.1.2	Test Requirements	2195

A.8.8	E-UTRAN FDD – GSM Measurements	2195

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2195

A.8.8.1.1	Test Purpose and Environment	2195

A.8.8.1.2	Test Requirements	2197

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2197

A.8.8.2.1	Test Purpose and Environment	2197

A.8.8.2.2	Test Requirements	2199

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2200

A.8.8.3.1	Test Purpose and Environment	2200

A.8.8.3.2	Test Requirements	2201

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2202

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2202

A.8.9.1.1	Test Purpose and Environment	2202

A.8.9.1.2	Test Requirements	2203

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2204

A.8.9.2.1	Test Purpose and Environment	2204

A.8.9.2.2	Test Requirements	2206

A.8.10	E-UTRAN TDD – GSM Measurements	2206

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2206

A.8.10.1.1	Test Purpose and Environment	2206

A.8.10.1.2	Test Requirements	2207

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2208

A.8.10.2.1	Test Purpose and Environment	2208

A.8.10.2.2	Test Requirements	2210

A.8.11	Monitoring of Multiple Layers	2210

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2210

A.8.11.1.1	Test Purpose and Environment	2210

A.8.11.1.2	Test Requirements	2212

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2213

A.8.11.2.1	Test Purpose and Environment	2213

A.8.11.2.2	Test Requirements	2214

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2215

A.8.11.3.1	Test Purpose and Environment	2215

A.8.11.3.2	Test Requirements	2217

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2217

A.8.11.4.1	Test Purpose and Environment	2217

A.8.11.4.2	Test Requirements	2220

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2220

A.8.11.5.1	Test Purpose and Environment	2220

A.8.11.5.2	Test Requirements	2222

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2223

A.8.11.6.1	Test Purpose and Environment	2223

A.8.11.6.2	Test Requirements	2225

A.8.12	RSTD Intra-frequency Measurements	2226

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2226

A.8.12.1.1	Test Purpose and Environment	2226

A.8.12.1.2	Test Requirements	2230

A.8.12.1.2A	Test Requirements for UE Category 1bis	2230

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2230

A.8.12.2.1	Test Purpose and Environment	2230

A.8.12.2.2	Test Requirements	2235

A.8.12.2.2A	Test Requirements for UE Category 1bis	2235

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2235

A.8.12.3.1	Test Purpose and Environment	2235

A.8.12.3.2	Test Requirements	2241

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2241

A.8.12.4.1	Test Purpose and Environment	2241

A.8.12.4.2	Test Requirements	2246

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2246

A.8.12.5.1	Test Purpose and Environment	2246

A.8.12.5.2	Test Requirements	2251

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2251

A.8.12.6.1	Test Purpose and Environment	2251

A.8.12.6.2	Test Requirements	2256

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2256

A.8.12.7.1	Test Purpose and Environment	2256

A.8.12.7.2	Test Requirements	2261

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2261

A.8.12.8.1	Test Purpose and Environment	2261

A.8.12.8.2	Test Requirements	2266

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2266

A.8.12.9.1	Test Purpose and Environment	2266

A.8.12.9.2	Test Requirements	2271

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2271

A.8.12.10.1	Test Purpose and Environment	2271

A.8.12.10.2	Test Requirements	2276

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2276

A.8.12.11.1	Test Purpose and Environment	2276

A.8.12.11.2	Test Requirements	2281

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2281

A.8.12.12.1	Test Purpose and Environment	2281

A.8.12.12.2	Test Requirements	2286

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2286

A.8.12.13.1	Test Purpose and Environment	2286

A.8.12.13.2	Test Requirements	2291

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2291

A.8.12.14.1	Test Purpose and Environment	2291

A.8.12.14.2	Test Requirements	2296

A.8.13	RSTD Inter-frequency Measurements	2296

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2296

A.8.13.1.1	Test Purpose and Environment	2296

A.8.13.1.2	Test Requirements	2301

A.8.13.1.2A	Test Requirements for UE Category 1bis	2301

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2301

A.8.13.2.1	Test Purpose and Environment	2301

A.8.13.2.2	Test Requirements	2307

A.8.13.2.2A	Test Requirements for UE Category 1bis	2307

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2307

A.8.13.3.1	Test Purpose and Environment	2307

A.8.13.3.2	Test Requirements	2314

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2314

A.8.13.4.1	Test Purpose and Environment	2314

A.8.13.4.2	Test Requirements	2320

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2320

A.8.13.5.1	Test Purpose and Environment	2320

A.8.13.5.2	Test Requirements	2326

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2326

A.8.13.6.1	Test Purpose and Environment	2326

A.8.13.6.2	Test Requirements	2332

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2332

A.8.13.7.1	Test Purpose and Environment	2332

A.8.13.7.2	Test Requirements	2338

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2338

A.8.13.8.1	Test Purpose and Environment	2338

A.8.13.8.2	Test Requirements	2344

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2344

A.8.13.9.1	Test Purpose and Environment	2344

A.8.13.9.2	Test Requirements	2350

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2350

A.8.13.10.1	Test Purpose and Environment	2350

A.8.13.10.2	Test Requirements	2356

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2356

A.8.13.11.1	Test Purpose and Environment	2356

A.8.13.11.2	Test Requirements	2362

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2362

A.8.13.12.1	Test Purpose and Environment	2362

A.8.13.12.2	Test Requirements	2368

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2368

A.8.13.13.1	Test Purpose and Environment	2368

A.8.13.13.2	Test Requirements	2374

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2374

A.8.13.14.1	Test Purpose and Environment	2374

A.8.13.14.2	Test Requirements	2380

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2380

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2380

A.8.14.1.1	Test Purpose and Environment	2380

A.8.14.1.2	Test Requirements	2382

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2382

A.8.14.2.1	Test Purpose and Environment	2382

A.8.14.2.2	Test Requirements	2385

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2385

A.8.14.3.1	Test Purpose and Environment	2385

A.8.14.3.2	Test Requirements	2387

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2388

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2388

A.8.15.1.1	Test Purpose and Environment	2388

A.8.15.1.2	Test Requirements	2389

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2389

A.8.15.2.1	Test Purpose and Environment	2389

A.8.15.2.2	Test Requirements	2392

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2392

A.8.15.3.1	Test Purpose and Environment	2392

A.8.15.3.2	Test Requirements	2394

A.8.16	E-UTRAN Carrier Aggregation Measurements	2395

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2395

A.8.16.1.1	Test Purpose and Environment	2395

A.8.16.1.2	Test Requirements	2397

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2397

A.8.16.2.1	Test Purpose and Environment	2397

A.8.16.2.2	Test Requirements	2399

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2399

A.8.16.3.1	Test Purpose and Environment	2399

A.8.16.3.2	Test Requirements	2401

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2402

A.8.16.3A.1	Test Purpose and Environment	2402

A.8.16.3A.2	Test Requirements	2403

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2404

A.8.16.4.1	Test Purpose and Environment	2404

A.8.16.4.2	Test Requirements	2405

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2406

A.8.16.4A.1	Test Purpose and Environment	2406

A.8.16.4A.2	Test Requirements	2408

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2409

A.8.16.5.1	Test Purpose and Environment	2409

A.8.16.5.2	Test Requirements	2409

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2409

A.8.16.6.1	Test Purpose and Environment	2409

A.8.16.6.2	Test Requirements	2410

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2410

A.8.16.7.1	 Test Purpose and Environment	2410

A.8.16.7.2	 Test Requirements	2411

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2411

A.8.16.8.1	 Test Purpose and Environment	2411

A.8.16.8.2	 Test Requirements	2412

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2412

A.8.16.9.1	Test Purpose and Environment	2412

A.8.16.9.2	Test Requirements	2413

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2413

A.8.16.10.1	Test Purpose and Environment	2413

A.8.16.10.2	Test Requirements	2414

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2414

A.8.16.11.1	 Test Purpose and Environment	2414

A.8.16.11.2	 Test Requirements	2415

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2415

A.8.16.12.1	 Test Purpose and Environment	2415

A.8.16.12.2	 Test Requirements	2415

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2415

A.8.16.13.1	Test Purpose and Environment	2415

A.8.16.13.2	Test Requirements	2416

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2416

A.8.16.14.1	Test Purpose and Environment	2416

A.8.16.14.2	Test Requirements	2417

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2417

A.8.16.15.1	 Test Purpose and Environment	2417

A.8.16.7.2	 Test Requirements	2417

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2417

A.8.16.16.1	 Test Purpose and Environment	2417

A.8.16.16.2	 Test Requirements	2418

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2418

A.8.16.17.1	Test Purpose and Environment	2418

A.8.16.17.2	Test Requirements	2420

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2421

A.8.16.17A.1	Test Purpose and Environment	2421

A.8.16.17A.2	Test Requirements	2421

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2421

A.8.16.17B.1	Test Purpose and Environment	2421

A.8.16.17B.2	Test Requirements	2422

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2422

A.8.16.17C.1	Test Purpose and Environment	2422

A.8.16.17C.2	Test Requirements	2422

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2422

A.8.16.18.1	Test Purpose and Environment	2422

A.8.16.18.2	Test Requirements	2424

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2425

A.8.16.18A.1	Test Purpose and Environment	2425

A.8.16.18A.2	Test Requirements	2425

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2425

A.8.16.18B.1	Test Purpose and Environment	2425

A.8.16.18B.2	Test Requirements	2426

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2426

A.8.16.18C.1	Test Purpose and Environment	2426

A.8.16.18C.2	Test Requirements	2426

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2426

A.8.16.18D.1	Test Purpose and Environment	2426

A.8.16.18D.2	Test Requirements	2427

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2427

A.8.16.19.1	Test Purpose and Environment	2427

A.8.16.19.2	Test Requirements	2429

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2429

A.8.16.19A.1	Test Purpose and Environment	2429

A.8.16.19A.2	Test Requirements	2430

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2430

A.8.16.19B.1	Test Purpose and Environment	2430

A.8.16.19B.2	Test Requirements	2430

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2430

A.8.16.19C.1	Test Purpose and Environment	2430

A.8.16.19C.2	Test Requirements	2431

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2431

A.8.16.20.1	Test Purpose and Environment	2431

A.8.16.20.2	Test Requirements	2433

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2434

A.8.16.20A.1	Test Purpose and Environment	2434

A.8.16.20A.2	Test Requirements	2434

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2434

A.8.16.20B.1	Test Purpose and Environment	2434

A.8.16.20B.2	Test Requirements	2435

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2435

A.8.16.20C.1	Test Purpose and Environment	2435

A.8.16.20C.2	Test Requirements	2435

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2435

A.8.16.20D.1	Test Purpose and Environment	2435

A.8.16.20D.2	Test Requirements	2436

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2436

A.8.16.21.1	Test Purpose and Environment	2436

A.8.16.21.2	Test Requirements	2438

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2438

A.8.16.22.1	Test Purpose and Environment	2438

A.8.16.22.2	Test Requirements	2440

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2440

A.8.16.23.1	Test Purpose and Environment	2440

A.8.16.23.2	Test Requirements	2443

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2443

A.8.16.24.1	Test Purpose and Environment	2443

A.8.16.24.2	Test Requirements	2446

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2446

A.8.16.25.1	Test Purpose and Environment	2446

A.8.16.25.2	Test Requirements	2449

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2449

A.8.16.26.1	Test Purpose and Environment	2449

A.8.16.26.2	Test Requirements	2452

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2452

A.8.16.27.1	Test Purpose and Environment	2452

A.8.16.27.2	Test Requirements	2457

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2457

A.8.16.28.1	Test Purpose and Environment	2457

A.8.16.28.2	Test Requirements	2462

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2462

A.8.16.29.1	Test Purpose and Environment	2462

A.8.16.29.2	Test Requirements	2467

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2467

A.8.16.30.1	Test Purpose and Environment	2467

A.8.16.30.2	Test Requirements	2472

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2472

A.8.16.31.1	Test Purpose and Environment	2472

A.8.16.31.2	Test Requirements	2477

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2477

A.8.16.32.1	Test Purpose and Environment	2477

A.8.16.32.2	Test Requirements	2482

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2482

A.8.16.33.1	Test Purpose and Environment	2482

A.8.16.33.2	Test Requirements	2487

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2487

A.8.16.34.1	Test Purpose and Environment	2487

A.8.16.34.2	Test Requirements	2492

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2492

A.8.16.35.1	Test Purpose and Environment	2492

A.8.16.35.2	Test Requirements	2494

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2495

A.8.16.36.1	Test Purpose and Environment	2495

A.8.16.36.2	Test Requirements	2497

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2498

A.8.16.37.1	Test Purpose and Environment	2498

A.8.16.37.2	Test Requirements	2500

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2501

A.8.16.38.1	Test Purpose and Environment	2501

A.8.16.38.2	Test Requirements	2503

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2504

A.8.16.39.1	Test Purpose and Environment	2504

A.8.16.39.2	Test Requirements	2507

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2507

A.8.16.40.1	Test Purpose and Environment	2507

A.8.16.40.2	Test Requirements	2510

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2510

A.8.16.41.1	Test Purpose and Environment	2510

A.8.16.41.2	Test Requirements	2512

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2513

A.8.16.42.1	Test Purpose and Environment	2513

A.8.16.42.2	Test Requirements	2515

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2516

A.8.16.43.1	Test Purpose and Environment	2516

A.8.16.43.2	Test Requirements	2519

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2519

A.8.16.44.1	Test Purpose and Environment	2519

A.8.16.44.2	Test Requirements	2522

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2522

A.8.16.45.1	Test Purpose and Environment	2522

A.8.16.45.2	Test Requirements	2525

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2525

A.8.16.46.1	Test Purpose and Environment	2525

A.8.16.46.2	Test Requirements	2527

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2528

A.8.16.47.1	Test Purpose and Environment	2528

A.8.16.47.2	Test Requirements	2530

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2531

A.8.16.48.1	Test Purpose and Environment	2531

A.8.16.48.2	Test Requirements	2533

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2534

A.8.16.49.1	Test Purpose and Environment	2534

A.8.16.49.2	Test Requirements	2537

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2537

A.8.16.50.1	Test Purpose and Environment	2537

A.8.16.50.2	Test Requirements	2539

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2540

A.8.16.51.1	Test Purpose and Environment	2540

A.8.16.51.2	Test Requirements	2545

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2545

A.8.16.52.1	Test Purpose and Environment	2545

A.8.16.52.2	Test Requirements	2550

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2550

A.8.16.53.1	Test Purpose and Environment	2550

A.8.16.53.2	Test Requirements	2554

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2554

A.8.16.54.1	Test Purpose and Environment	2554

A.8.16.54.2	Test Requirements	2558

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2558

A.8.16.55.1	Test Purpose and Environment	2558

A.8.16.55.2	Test Requirements	2564

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2564

A.8.16.56.1	Test Purpose and Environment	2564

A.8.16.56.2	Test Requirements	2570

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2570

A.8.16.57.1	Test Purpose and Environment	2570

A.8.16.57.2	Test Requirements	2572

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2573

A.8.16.58.1	Test Purpose and Environment	2573

A.8.16.58.2	Test Requirements	2575

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2576

A.8.16.59.1	Test Purpose and Environment	2576

A.8.16.59.2	Test Requirements	2580

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2581

A.8.16.60.1	Test Purpose and Environment	2581

A.8.16.60.2	Test Requirements	2584

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2585

A.8.16.61.1	Test Purpose and Environment	2585

A.8.16.61.2	Test Requirements	2588

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2588

A.8.16.62.1	Test Purpose and Environment	2588

A.8.16.62.2	Test Requirements	2590

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2591

A.8.16.63.1	Test Purpose and Environment	2591

A.8.16.63.2	Test Requirements	2594

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2595

A.8.16.64.1	Test Purpose and Environment	2595

A.8.16.64.2	Test Requirements	2599

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2599

A.8.16.65.1	Test Purpose and Environment	2599

A.8.16.65.2	Test Requirements	2604

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2604

A.8.16.66.1	Test Purpose and Environment	2604

A.8.16.66.2	Test Requirements	2609

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2609

A.8.16.67.1	Test Purpose and Environment	2609

A.8.16.67.2	Test Requirements	2614

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2614

A.8.16.68.1	Test Purpose and Environment	2614

A.8.16.68.2	Test Requirements	2619

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2619

A.8.16.69.1	Test Purpose and Environment	2619

A.8.16.69.2	Test Requirements	2623

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2623

A.8.16.70.1	Test Purpose and Environment	2623

A.8.16.70.2	Test Requirements	2627

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2627

A.8.16.71.1	Test Purpose and Environment	2627

A.8.16.71.2	Test Requirements	2632

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2632

A.8.16.72.1	Test Purpose and Environment	2632

A.8.16.72.2	Test Requirements	2637

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2637

A.8.16.73.1	Test Purpose and Environment	2637

A.8.16.73.2	Test Requirements	2644

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2644

A.8.16.74.1	Test Purpose and Environment	2644

A.8.16.74.2	Test Requirements	2651

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2651

A.8.16.75.1	Test Purpose and Environment	2651

A.8.16.75.2	Test Requirements	2657

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2657

A.8.16.76.1	Test Purpose and Environment	2657

A.8.16.76.2	Test Requirements	2663

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2663

A.8.16.77.1	Test Purpose and Environment	2663

A.8.16.77.2	Test Requirements	2667

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2667

A.8.16.78.1	Test Purpose and Environment	2667

A.8.16.78.2	Test Requirements	2669

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2670

A.8.16.79.1	Test Purpose and Environment	2670

A.8.16.79.2	Test Requirements	2677

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2677

A.8.16.80.1	Test Purpose and Environment	2677

A.8.16.80.2	Test Requirements	2684

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2684

A.8.16.81.1	Test Purpose and Environment	2684

A.8.16.81.2	Test Requirements	2690

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2690

A.8.16.82.1	Test Purpose and Environment	2690

A.8.16.82.2	Test Requirements	2696

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2696

A.8.16.83.1	Test Purpose and Environment	2696

A.8.16.83.2	Test Requirements	2701

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2701

A.8.16.84.1	Test Purpose and Environment	2701

A.8.16.84.2	Test Requirements	2707

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2707

A.8.16.85.1	Test Purpose and Environment	2707

A.8.16.85.2	Test Requirements	2711

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2711

A.8.16.86.1	Test Purpose and Environment	2711

A.8.16.86.2	Test Requirements	2714

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2714

A.8.16.87.1	Test Purpose and Environment	2714

A.8.16.87.2	Test Requirements	2720

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2720

A.8.16.88.1	Test Purpose and Environment	2720

A.8.16.88.2	Test Requirements	2728

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2728

A.8.16.89.1	Test Purpose and Environment	2728

A.8.16.89.2	Test Requirements	2735

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2735

A.8.16.90.1	Test Purpose and Environment	2735

A.8.16.90.2	Test Requirements	2742

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2742

A.8.16.91.1	Test Purpose and Environment	2742

A.8.16.91.2	Test Requirements	2748

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2748

A.8.16.92.1	Test Purpose and Environment	2748

A.8.16.92.2	Test Requirements	2756

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2756

A.8.16.93.1	Test Purpose and Environment	2756

A.8.16.93.2	Test Requirements	2763

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2763

A.8.16.94.1	Test Purpose and Environment	2763

A.8.16.94.2	Test Requirements	2770

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2770

A.8.16.95.1	Test Purpose and Environment	2770

A.8.16.95.2	Test Requirements	2778

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2778

A.8.16.96.1	Test Purpose and Environment	2778

A.8.16.96.2	Test Requirements	2788

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2788

A.8.16.97.1	Test Purpose and Environment	2788

A.8.16.97.2	Test Requirements	2795

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2795

A.8.16.98.1	Test Purpose and Environment	2795

A.8.16.98.2	Test Requirements	2803

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2803

A.8.16.99.1	Test Purpose and Environment	2803

A.8.16.99.2	Test Requirements	2813

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2813

A.8.16.100.1	Test Purpose and Environment	2813

A.8.16.100.2	Test Requirements	2823

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2823

A.8.16.101.1	Test Purpose and Environment	2823

A.8.16.101.2	Test Requirements	2833

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2833

A.8.16.102.1	Test Purpose and Environment	2833

A.8.16.102.2	Test Requirements	2844

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2844

A.8.16.103.1	Test Purpose and Environment	2844

A.8.16.103.2	Test Requirements	2849

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2850

A.8.16.104.1	Test Purpose and Environment	2850

A.8.16.104.2	Test Requirements	2854

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2855

A.8.16.105.1	Test Purpose and Environment	2855

A.8.16.105.2	Test Requirements	2859

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2860

A.8.16.106.1	Test Purpose and Environment	2860

A.8.16.106.2	Test Requirements	2864

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2864

A.8.16.107.1	Test Purpose and Environment	2864

A.8.16.107.2	Test Requirements	2866

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2867

A.8.16.108.1	Test Purpose and Environment	2867

A.8.16.108.2	Test Requirements	2869

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2870

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2870

A.8.17.1.1	Test Purpose and Environment	2870

A.8.17.1.2	Test Requirements	2876

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2876

A.8.17.2.1	Test Purpose and Environment	2876

A.8.17.2.2	Test Requirements	2883

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2883

A.8.17.3.1	Test Purpose and Environment	2883

A.8.17.3.2	Test Requirements	2884

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2884

A.8.17.4.1	Test Purpose and Environment	2884

A.8.17.4.2	Test Requirements	2885

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2885

A.8.17.5.1	Test Purpose and Environment	2885

A.8.17.5.2	Test Requirements	2886

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2886

A.8.17.6.1	Test Purpose and Environment	2886

A.8.17.6.2	Test Requirements	2887

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2888

A.8.17.7.1	Test Purpose and Environment	2888

A.8.17.7.2	Test Requirements	2888

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2889

A.8.17.8.1	Test Purpose and Environment	2889

A.8.17.8.2	Test Requirements	2889

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2890

A.8.17.9.1	Test Purpose and Environment	2890

A.8.17.9.2	Test Requirements	2891

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2891

A.8.17.10.1	Test Purpose and Environment	2891

A.8.17.10.2	Test Requirements	2898

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2899

A.8.17.11.1	Test Purpose and Environment	2899

A.8.17.11.2	Test Requirements	2905

A.8.18	E-UTRAN TDD – HRPD Measurements	2906

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2906

A.8.18.1.1	Test Purpose and Environment	2906

A.8.18.1.2	Test Requirements	2908

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2908

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2908

A.8.19.1.1	Test Purpose and Environment	2908

A.8.19.1.2	Test Requirements	2909

A.8.20	Inter-frequency/RAT Measurements in CA mode	2910

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2910

A.8.20.1.1	Test Purpose and Environment	2910

A.8.20.1.2	Test Requirements	2911

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2911

A.8.20.2.1	Test Purpose and Environment	2912

A.8.20.2.2	Test Requirements	2913

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2914

A.8.20.2A.1	Test Purpose and Environment	2914

A.8.20.2A.2	Test Requirements	2914

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2914

A.8.20.2B.1	Test Purpose and Environment	2914

A.8.20.2B.2	Test Requirements	2917

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2917

A.8.20.3.1	Test Purpose and Environment	2917

A.8.20.3.2	Test Requirements	2919

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2919

A.8.20.4.1	Test Purpose and Environment	2919

A.8.20.4.1.1	1.28 Mcps TDD option	2919

A.8.20.4.2	Test Requirements	2921

A.8.20.4.2.1	1.28 Mcps TDD option	2921

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2921

A.8.20.4A.1	Test Purpose and Environment	2921

A.8.20.4A.1.1	1.28 Mcps TDD option	2921

A.8.20.4A.2	Test Requirements	2921

A.8.20.4A.2.1	1.28 Mcps TDD option	2921

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2922

A.8.20.4B.1	Test Purpose and Environment	2922

A.8.20.4B.1.1	1.28 Mcps TDD option	2922

A.8.20.4B.2	Test Requirements	2924

A.8.20.4B.2.1	1.28 Mcps TDD option	2924

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2924

A.8.21.1	Test Purpose and Environment	2924

A.8.21.2 Test Requirements	2928

A.8.22	E-UTRAN Discovery Signal Measurements	2928

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2928

A.8.22.1.1	Test Purpose and Environment	2928

A.8.22.1.2	Test Requirements	2931

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2931

A.8.22.2.1	Test Purpose and Environment	2931

A.8.22.2.2	Test Requirements	2934

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2934

A.8.22.3.1	Test Purpose and Environment	2934

A.8.22.3.2	Test Requirements	2937

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2937

A.8.22.4.1	Test Purpose and Environment	2937

A.8.22.4.2	Test Requirements	2940

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2940

A.8.22.5.1	Test Purpose and Environment	2940

A.8.22.5.2	Test Requirements	2944

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2944

A.8.22.6.1	Test Purpose and Environment	2944

A.8.22.6.2	Test Requirements	2948

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2948

A.8.22.7.1	Test Purpose and Environment	2948

A.8.22.7.2	Test Requirements	2952

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2952

A.8.22.8.1	Test Purpose and Environment	2952

A.8.22.8.2	Test Requirements	2956

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2956

A.8.22.9.1	Test Purpose and Environment	2956

A.8.22.9.2	Test Requirements	2958

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2959

A.8.22.10.1	Test Purpose and Environment	2959

A.8.22.10.2	Test Requirements	2961

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2962

A.8.22.11.1	Test Purpose and Environment	2962

A.8.22.11.2	Test Requirements	2965

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2965

A.8.22.12.1	Test Purpose and Environment	2965

A.8.22.12.2	Test Requirements	2969

A.8.23	E-UTRAN Dual Connectivity Measurements	2969

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2969

A.8.23.1.1	Test Purpose and Environment	2969

A.8.23.1.2	Test Requirements	2972

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2972

A.8.23.2.1	Test Purpose and Environment	2972

A.8.23.2.2	Test Requirements	2975

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2975

A.8.23.3.1	Test Purpose and Environment	2975

A.8.23.3.2	Test Requirements	2978

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2978

A.8.23.4.1	Test Purpose and Environment	2978

A.8.23.4.2	Test Requirements	2981

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2981

A.8.23.5.1	Test Purpose and Environment	2981

A.8.23.5.2	Test Requirements	2984

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2984

A.8.23.6.1	Test Purpose and Environment	2984

A.8.23.6.2	Test Requirements	2987

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2987

A.8.23.7.1	Test Purpose and Environment	2987

A.8.23.7.2	Test Requirements	2989

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2990

A.8.23.8.1	Test Purpose and Environment	2990

A.8.23.8.2	Test Requirements	2992

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2993

A.8.23.9.1	Test Purpose and Environment	2993

A.8.23.9.2	Test Requirements	2996

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2996

A.8.23.10.1	Test Purpose and Environment	2996

A.8.23.10.2	Test Requirements	2999

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2999

A.8.23.11.1	Test Purpose and Environment	2999

A.8.23.11.2	Test Requirements	3002

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3002

A.8.23.12.1	Test Purpose and Environment	3002

A.8.23.12.2	Test Requirements	3005

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3005

A.8.23.13.1	Test Purpose and Environment	3005

A.8.23.13.2	Test Requirements	3008

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3008

A.8.23.14.1	Test Purpose and Environment	3008

A.8.23.14.2	Test Requirements	3011

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3011

A.8.23.15.1	Test Purpose and Environment	3011

A.8.23.15.2	Test Requirements	3014

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3014

A.8.23.16.1	Test Purpose and Environment	3014

A.8.23.16.2	Test Requirements	3015

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3016

A.8.23.17.1	Test Purpose and Environment	3016

A.8.23.17.2	Test Requirements	3018

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3018

A.8.23.18.1	Test Purpose and Environment	3018

A.8.23.18.2	Test Requirements	3019

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3020

A.8.23.19.1	Test Purpose and Environment	3020

A.8.23.19.2	Test Requirements	3021

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3022

A.8.23.20.1	Test Purpose and Environment	3022

A.8.23.20.2	Test Requirements	3023

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3024

A.8.23.21.1	Test Purpose and Environment	3024

A.8.23.21.2	Test Requirements	3025

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3026

A.8.23.22.1	Test Purpose and Environment	3026

A.8.23.22.2	Test Requirements	3027

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3028

A.8.23.23.1	Test Purpose and Environment	3028

A.8.23.23.2	Test Requirements	3029

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3030

A.8.23.24.1	Test Purpose and Environment	3030

A.8.23.24.2	Test Requirements	3032

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3033

A.8.23.25.1	Test Purpose and Environment	3033

A.8.23.25.2	Test Requirements	3035

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3036

A.8.23.26.1	Test Purpose and Environment	3036

A.8.23.26.2	Test Requirements	3038

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3039

A.8.23.27.1	Test Purpose and Environment	3039

A.8.23.27.2	Test Requirements	3042

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3042

A.8.23.28.1	Test Purpose and Environment	3042

A.8.23.28.2	Test Requirements	3046

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3046

A.8.23.29.1	Test Purpose and Environment	3046

A.8.23.29.2	Test Requirements	3050

A.8.24	Proximity-based Services	3050

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3050

A.8.24.1.1	Test Purpose and Environment	3050

A.8.24.1.2	Test Requirements	3051

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3052

A.8.24.2.1	Test Purpose and Environment	3052

A.8.24.2.2	Test Requirements	3053

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3053

A.8.24.3.1	Test Purpose and Environment	3054

A.8.24.3.2	Test Requirements	3055

A.8.25	E-UTRAN-WLAN Measurements	3056

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3056

A.8.25.1.1	Test Purpose and Environment	3056

A.8.25.1.2	Test Requirements	3058

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3058

A.8.25.2.1	Test Purpose and Environment	3058

A.8.25.2.2	Test Requirements	3060

A.8.26 Frame Structure 3 (FS3)	3060

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3060

A.8.26.1.1	Test Purpose and Environment	3060

A.8.26.1.2	Test Requirements	3062

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3063

A.8.26.2.1	Test Purpose and Environment	3063

A.8.26.2.2	Test Requirements	3065

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3066

A.8.26.3.1	Test Purpose and Environment	3066

A.8.26.3.2	Test Requirements	3069

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3069

A.8.26.3A.1	Test Purpose and Environment	3069

A.8.26.3A.2	Test Requirements	3073

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3073

A.8.26.4.1	Test Purpose and Environment	3073

A.8.26.4.2	Test Requirements	3077

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3077

A.8.26.4A.1	Test Purpose and Environment	3077

A.8.26.4A.2	Test Requirements	3081

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3081

A.8.26.5.1	Test Purpose and Environment	3081

A.8.26.5.2	Test Requirements	3084

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3084

A.8.26.5A.1	Test Purpose and Environment	3084

A.8.26.5A.2	Test Requirements	3088

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3088

A.8.26.6.1	Test Purpose and Environment	3088

A.8.26.6.2	Test Requirements	3092

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3092

A.8.26.6A.1	Test Purpose and Environment	3092

A.8.26.6A.2	Test Requirements	3096

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3096

A.8.26.7.1	Test Purpose and Environment	3096

A.8.26.7.2	Test Requirements	3099

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3099

A.8.26.8.1	Test Purpose and Environment	3099

A.8.26.8.2	Test Requirements	3103

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3103

A.8.26.9.1	Test Purpose and Environment	3103

A.8.26.9.2	Test Requirements	3107

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3107

A.8.26.10.1	Test Purpose and Environment	3107

A.8.26.10.2	Test Requirements	3110

A.9	Measurement Performance Requirements	3109

A.9.1	RSRP	3109

A.9.1.1	FDD Intra frequency case	3109

A.9.1.1.1	Test Purpose and Environment	3109

A.9.1.1.2	Test parameters	3109

A.9.1.1.3	Test Requirements	3112

A.9.1.2	TDD Intra frequency case	3112

A.9.1.2.1	Test Purpose and Environment	3112

A.9.1.2.2	Test parameters	3112

A.9.1.2.3	Test Requirements	3113

A.9.1.3	FDD—FDD Inter frequency case	3114

A.9.1.3.1	Test Purpose and Environment	3114

A.9.1.3.2	Test parameters	3114

A.9.1.3.3	Test Requirements	3117

A.9.1.4	TDD—TDD Inter frequency case	3117

A.9.1.4.1	Test Purpose and Environment	3117

A.9.1.4.2	Test parameters	3117

A.9.1.4.3	Test Requirements	3119

A.9.1.5	FDD—TDD Inter frequency case	3120

A.9.1.5.1	Test Purpose and Environment	3120

A.9.1.5.2	Test parameters	3120

A.9.1.5.3	Test Requirements	3121

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3122

A.9.1.6.1	Test Purpose and Environment	3122

A.9.1.6.2	Test parameters	3122

A.9.1.6.3	Test Requirements	3125

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3125

A.9.1.7.1	Test Purpose and Environment	3125

A.9.1.7.2	Test parameters	3125

A.9.1.7.3	Test Requirements	3128

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3128

A.9.1.8.1	Test Purpose and Environment	3128

A.9.1.8.2	Test parameters	3128

A.9.1.8.3	Test Requirements	3132

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3132

A.9.1.9.1	Test Purpose and Environment	3132

A.9.1.9.2	Test parameters	3132

A.9.1.9.3	Test Requirements	3134

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3135

A.9.1.10.1	Test Purpose and Environment	3135

A.9.1.10.2	Test parameters	3135

A.9.1.10.3	Test Requirements	3138

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3138

A.9.1.11.1	Test Purpose and Environment	3138

A.9.1.11.2	Test parameters	3138

A.9.1.11.3	Test Requirements	3142

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3142

A.9.1.12.1	Test Purpose and Environment	3142

A.9.1.12.2	Test parameters	3142

A.9.1.12.3	Test Requirements	3143

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3143

A.9.1.13.1	Test Purpose and Environment	3143

A.9.1.13.2	Test parameters	3143

A.9.1.13.3	Test Requirements	3144

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3144

A.9.1.14.1	Test Purpose and Environment	3144

A.9.1.14.2	Test parameters	3144

A.9.1.14.3	Test Requirements	3148

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3148

A.9.1.15.1	Test Purpose and Environment	3148

A.9.1.15.2	Test parameters	3148

A.9.1.15.3	Test Requirements	3150

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3151

A.9.1.16.1	Test Purpose and Environment	3151

A.9.1.16.2	Test parameters	3151

A.9.1.16.3	Test Requirements	3152

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3152

A.9.1.17.1	Test Purpose and Environment	3152

A.9.1.17.2	Test parameters	3153

A.9.1.17.3	Test Requirements	3154

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3154

A.9.1.18.1	Test Purpose and Environment	3154

A.9.1.18.2	Test parameters	3154

A.9.1.18.3	Test Requirements	3155

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3156

A.9.1.19.1	Test Purpose and Environment	3156

A.9.1.19.2	Test parameters	3156

A.9.1.19.3	Test Requirements	3156

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3156

A.9.1.20.1	Test Purpose and Environment	3156

A.9.1.20.2	Test parameters	3156

A.9.1.20.3	Test Requirements	3158

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3158

A.9.1.21.1	Test Purpose and Environment	3158

A.9.1.21.2	Test parameters	3158

A.9.1.21.3	Test Requirements	3158

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3158

A.9.1.22.1	Test Purpose and Environment	3158

A.9.1.22.2	Test parameters	3159

A.9.1.22.3	Test Requirements	3163

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3163

A.9.1.23.1	Test Purpose and Environment	3163

A.9.1.23.2	Test parameters	3163

A.9.1.23.3	Test Requirements	3167

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3167

A.9.1.24.1	Test Purpose and Environment	3167

A.9.1.24.2	Test parameters	3167

A.9.1.24.3	Test Requirements	3168

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3168

A.9.1.25.1	Test Purpose and Environment	3168

A.9.1.25.2	Test parameters	3168

A.9.1.25.3	Test Requirements	3171

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3171

A.9.1.26.1	Test Purpose and Environment	3171

A.9.1.26.2	Test parameters	3171

A.9.1.26.3	Test Requirements	3174

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3174

A.9.1.27.1	Test Purpose and Environment	3174

A.9.1.27.2	Test parameters	3174

A.9.1.27.3	Test Requirements	3177

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3177

A.9.1.28.1	Test Purpose and Environment	3177

A.9.1.28.2	Test parameters	3177

A.9.1.28.3	Test Requirements	3180

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3180

A.9.1.29.1	Test Purpose and Environment	3180

A.9.1.29.2	Test parameters	3180

A.9.1.29.3	Test Requirements	3183

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3183

A.9.1.30.1	Test Purpose and Environment	3183

A.9.1.30.2	Test parameters	3184

A.9.1.30.3	Test Requirements	3187

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3187

A.9.1.31.1	Test Purpose and Environment	3187

A.9.1.31.2	Test parameters	3187

A.9.1.31.3	Test Requirements	3190

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3191

A.9.1.32.1	Test Purpose and Environment	3191

A.9.1.32.2	Test parameters	3191

A.9.1.32.3	Test Requirements	3194

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3194

A.9.1.33.1	Test Purpose and Environment	3194

A.9.1.33.2	Test parameters	3194

A.9.1.33.3	Test Requirements	3197

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3197

A.9.1.34.1	Test Purpose and Environment	3197

A.9.1.34.2	Test parameters	3197

A.9.1.34.3	Test Requirements	3200

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3200

A.9.1.35.1	Test Purpose and Environment	3200

A.9.1.35.2	Test parameters	3200

A.9.1.35.3	Test Requirements	3203

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3204

A.9.1.36.1	Test Purpose and Environment	3204

A.9.1.36.2	Test parameters	3204

A.9.1.36.3	Test Requirements	3207

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3207

A.9.1.37.1	Test Purpose and Environment	3207

A.9.1.37.2	Test parameters	3207

A.9.1.37.3	Test Requirements	3210

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3211

A.9.1.38.1	Test Purpose and Environment	3211

A.9.1.38.2	Test parameters	3211

A.9.1.38.3	Test Requirements	3214

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3215

A.9.1.39.1	Test Purpose and Environment	3215

A.9.1.39.2	Test parameters	3215

A.9.1.39.3	Test Requirements	3221

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3221

A.9.1.40.1	Test Purpose and Environment	3221

A.9.1.40.2	Test parameters	3222

A.9.1.40.3	Test Requirements	3228

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3228

A.9.1.41.1	Test Purpose and Environment	3228

A.9.1.41.2	Test parameters	3228

A.9.1.41.3	Test Requirements	3231

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3231

A.9.1.42.1	Test Purpose and Environment	3231

A.9.1.42.2	Test parameters	3231

A.9.1.42.3	Test Requirements	3234

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3234

A.9.1.43.1	Test Purpose and Environment	3234

A.9.1.43.2	Test parameters	3234

A.9.1.43.3	Test Requirements	3235

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3236

A.9.1.44.1	Test Purpose and Environment	3236

A.9.1.44.2	Test parameters	3236

A.9.1.44.3	Test Requirements	3241

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3241

A.9.1.45.1	Test Purpose and Environment	3241

A.9.1.45.2	Test parameters	3241

A.9.1.45.3	Test Requirements	3249

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3249

A.9.1.46.1	Test Purpose and Environment	3249

A.9.1.46.2	Test parameters	3249

A.9.1.46.3	Test Requirements	3258

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3259

A.9.1.47.1	Test Purpose and Environment	3259

A.9.1.47.2	Test parameters	3259

A.9.1.47.3	Test Requirements	3268

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3268

A.9.1.48.1	Test Purpose and Environment	3268

A.9.1.48.2	Test parameters	3269

A.9.1.48.3	Test Requirements	3279

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3280

A.9.1.49.1	Test Purpose and Environment	3280

A.9.1.49.2	Test parameters	3280

A.9.1.49.3	Test Requirements	3290

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3291

A.9.1.50.1	Test Purpose and Environment	3291

A.9.1.50.2	Test parameters	3291

A.9.1.50.3	Test Requirements	3298

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3299

A.9.1.51.1	Test Purpose and Environment	3299

A.9.1.51.2	Test parameters	3299

A.9.1.51.3	Test Requirements	3305

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3306

A.9.1.52.1	Test Purpose and Environment	3306

A.9.1.52.2	Test parameters	3306

A.9.1.52.3	Test Requirements	3309

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3309

A.9.1.52A.1	Test Purpose and Environment	3309

A.9.1.52A.2	Test parameters	3309

A.9.1.52A.3	Test Requirements	3310

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3310

A.9.1.53.1	Test Purpose and Environment	3310

A.9.1.53.2	Test parameters	3311

A.9.1.53.3	Test Requirements	3314

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3314

A.9.1.53A.1	Test Purpose and Environment	3314

A.9.1.53A.2	Test parameters	3314

A.9.1.53A.3	Test Requirements	3315

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3316

A.9.1.54.1	Test Purpose and Environment	3316

A.9.1.54.2	Test parameters	3316

A.9.1.54.3	Test Requirements	3319

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3319

A.9.1.55.1	Test Purpose and Environment	3319

A.9.1.55.2	Test parameters	3319

A.9.1.55.3	Test Requirements	3323

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3323

A.9.1.56.1	Test Purpose and Environment	3323

A.9.1.56.2	Test parameters	3323

A.9.1.56.3	Test Requirements	3326

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3327

A.9.1.57.1	Test Purpose and Environment	3327

A.9.1.57.2	Test parameters	3327

A.9.1.57.3	Test Requirements	3330

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3330

A.9.1.57A.1	Test Purpose and Environment	3330

A.9.1.57A.2	Test parameters	3330

A.9.1.57A.3	Test Requirements	3331

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3331

A.9.1.58.1	Test Purpose and Environment	3331

A.9.1.58.2	Test parameters	3332

A.9.1.58.3	Test Requirements	3335

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3335

A.9.1.58A.1	Test Purpose and Environment	3335

A.9.1.58A.2	Test parameters	3335

A.9.1.58A.3	Test Requirements	3336

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3336

A.9.1.59.1	Test Purpose and Environment	3336

A.9.1.59.2	Test parameters	3337

A.9.1.59.3	Test Requirements	3339

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3339

A.9.1.60.1	Test Purpose and Environment	3339

A.9.1.60.2	Test parameters	3339

A.9.1.60.3	Test Requirements	3342

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3343

A.9.1.61.1	Test Purpose and Environment	3343

A.9.1.61.2	Test parameters	3343

A.9.1.61.3	Test Requirements	3346

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3346

A.9.1.62.1	Test Purpose and Environment	3346

A.9.1.62.2	Test parameters	3346

A.9.1.62.3	Test Requirements	3349

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3349

A.9.1.53.1	Test Purpose and Environment	3349

A.9.1.53.2	Test parameters	3349

A.9.1.63.3	Test Requirements	3352

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3352

A.9.1.64.1	Test Purpose and Environment	3352

A.9.1.64.2	Test parameters	3352

A.9.1.64.3	Test Requirements	3353

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3354

A.9.1.65.1	Test Purpose and Environment	3354

A.9.1.65.2	Test parameters	3354

A.9.1.65.3	Test Requirements	3360

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3360

A.9.1.66.1	Test Purpose and Environment	3360

A.9.1.66.2	Test parameters	3360

A.9.1.66.3	Test Requirements	3366

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3366

A.9.1.67.1	Test Purpose and Environment	3366

A.9.1.67.2	Test parameters	3366

A.9.1.67.3	Test Requirements	3367

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3368

A.9.1.68.1	Test Purpose and Environment	3368

A.9.1.68.2	Test parameters	3368

A.9.1.68.3	Test Requirements	3372

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3372

A.9.1.69.1	Test Purpose and Environment	3372

A.9.1.69.2	Test parameters	3372

A.9.1.69.3	Test Requirements	3376

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3377

A.9.1.70.1	Test Purpose and Environment	3377

A.9.1.70.2	Test parameters	3377

A.9.1.70.3	Test Requirements	3389

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3390

A.9.1.71.1	Test Purpose and Environment	3390

A.9.1.71.2	Test parameters	3390

A.9.1.71.3	Test Requirements	3402

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3403

A.9.1.72.1	Test Purpose and Environment	3403

A.9.1.72.2	Test parameters	3403

A.9.1.72.3	Test Requirements	3415

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3416

A.9.1.73.1	Test Purpose and Environment	3416

A.9.1.73.2	Test parameters	3416

A.9.1.73.3	Test Requirements	3419

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3419

A.9.1.74.1	Test Purpose and Environment	3419

A.9.1.74.2	Test parameters	3419

A.9.1.74.3	Test Requirements	3420

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3420

A.9.1.75.1	Test Purpose and Environment	3420

A.9.1.75.2	Test parameters	3421

A.9.1.74.3	Test Requirements	3422

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3422

A.9.1.76.1	Test Purpose and Environment	3422

A.9.1.76.2	Test parameters	3422

A.9.1.76.3	Test Requirements	3425

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3425

A.9.1.77.1	Test Purpose and Environment	3425

A.9.1.77.2	Test parameters	3425

A.9.1.77.3	Test Requirements	3428

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3428

A.9.1.78.1	Test Purpose and Environment	3428

A.9.1.78.2	Test parameters	3428

A.9.1.78.3	Test Requirements	3431

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3431

A.9.1.79.1	Test Purpose and Environment	3431

A.9.1.79.2	Test parameters	3431

A.9.1.79.3	Test Requirements	3434

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3434

A.9.1.80.1	Test Purpose and Environment	3434

A.9.1.80.2	Test parameters	3434

A.9.1.80.3	Test Requirements	3437

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3437

A.9.1.81.1	Test Purpose and Environment	3437

A.9.1.78.2	Test parameters	3437

A.9.1.81.3	Test Requirements	3440

A.9.2	RSRQ	3440

A.9.2.1	FDD Intra frequency case	3440

A.9.2.1.1	Test Purpose and Environment	3440

A.9.2.1.2	Test parameters	3440

A.9.2.1.3	Test Requirements	3443

A.9.2.2	TDD Intra frequency case	3443

A.9.2.2.1	Test Purpose and Environment	3443

A.9.2.2.2	Test parameters	3443

A.9.2.2.3	Test Requirements	3445

A.9.2.3	FDD—FDD Inter frequency case	3445

A.9.2.3.1	Test Purpose and Environment	3445

A.9.2.3.2	Test parameters	3445

A.9.2.3.3	Test Requirements	3448

A.9.2.4	TDD—TDD Inter frequency case	3448

A.9.2.4.1	Test Purpose and Environment	3448

A.9.2.4.2	Test parameters	3448

A.9.2.4.3	Test Requirements	3450

A.9.2.4A	FDD—TDD Inter frequency case	3451

A.9.2.4A.1	Test Purpose and Environment	3451

A.9.2.4A.2	Test parameters	3451

A.9.2.4A.3	Test Requirements	3453

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3453

A.9.2.5.1	Test Purpose and Environment	3453

A.9.2.5.2	Test parameters	3454

A.9.2.5.3	Test Requirements	3457

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3457

A.9.2.6.1	Test Purpose and Environment	3457

A.9.2.6.2	Test parameters	3458

A.9.2.6.3	Test Requirements	3461

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3461

A.9.2.7.1	Test Purpose and Environment	3461

A.9.2.7.2	Test parameters	3461

A.9.2.7.3	Test Requirements	3465

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3465

A.9.2.8.1	Test Purpose and Environment	3465

A.9.2.8.2	Test parameters	3465

A.9.2.8.3	Test Requirements	3469

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3469

A.9.2.9.1	Test Purpose and Environment	3469

A.9.2.9.2	Test parameters	3469

A.9.2.9.3	Test Requirements	3473

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3473

A.9.2.10.1	Test Purpose and Environment	3473

A.9.2.10.2	Test parameters	3473

A.9.2.10.3	Test Requirements	3477

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3477

A.9.2.11.1	Test Purpose and Environment	3477

A.9.2.11.2	Test parameters	3477

A.9.2.11.3	Test Requirements	3478

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3478

A.9.2.12.1	Test Purpose and Environment	3478

A.9.2.12.2	Test parameters	3479

A.9.2.12.3	Test Requirements	3479

A.9.2.13	Void	3480

A.9.2.13.1	Void	3480

A.9.2.13.2	Void	3480

A.9.2.13.3	Void	3480

A.9.2.14	Void	3480

A.9.2.14.1	Void	3480

A.9.2.14.2	Void	3480

A.9.2.14.3	Void	3480

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3480

A.9.2.15.1	Test Purpose and Environment	3480

A.9.2.15.2	Test parameters	3480

A.9.2.15.3	Test Requirements	3484

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3484

A.9.2.16.1	Test Purpose and Environment	3484

A.9.2.16.2	Test parameters	3484

A.9.2.16.3	Test Requirements	3489

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3489

A.9.2.17.1	Test Purpose and Environment	3489

A.9.2.17.2	Test parameters	3489

A.9.2.17.3	Test Requirements	3491

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3491

A.9.2.18.1	Test Purpose and Environment	3491

A.9.2.18.2	Test parameters	3491

A.9.2.18.3	Test Requirements	3494

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3494

A.9.2.19.1	Test Purpose and Environment	3494

A.9.2.19.2	Test parameters	3494

A.9.2.19.3	Test Requirements	3496

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3496

A.9.2.20.1	Test Purpose and Environment	3496

A.9.2.20.2	Test parameters	3496

A.9.2.20.3	Test Requirements	3499

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3499

A.9.2.21.1	Test Purpose and Environment	3499

A.9.2.21.2	Test parameters	3499

A.9.2.21.3	Test Requirements	3502

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3502

A.9.2.22.1	Test Purpose and Environment	3503

A.9.2.22.2	Test parameters	3503

A.9.2.22.3	Test Requirements	3503

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3503

A.9.2.23.1	Test Purpose and Environment	3503

A.9.2.23.2	Test parameters	3503

A.9.2.23.3	Test Requirements	3506

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3506

A.9.2.24.1	Test Purpose and Environment	3506

A.9.2.24.2	Test parameters	3506

A.9.2.24.3	Test Requirements	3507

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3507

A.9.2.25.1	Test Purpose and Environment	3507

A.9.2.25.2	Test parameters	3507

A.9.2.25.3	Test Requirements	3512

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3512

A.9.2.26.1	Test Purpose and Environment	3512

A.9.2.26.2	Test parameters	3512

A.9.2.26.3	Test Requirements	3516

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3516

A.9.2.27.1	Test Purpose and Environment	3516

A.9.2.27.2	Test parameters	3516

A.9.2.27.3	Test Requirements	3517

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3517

A.9.2.28.1	Test Purpose and Environment	3517

A.9.2.28.2	Test parameters	3517

A.9.2.28.3	Test Requirements	3520

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3520

A.9.2.29.1	Test Purpose and Environment	3520

A.9.2.29.2	Test parameters	3520

A.9.2.29.3	Test Requirements	3523

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3523

A.9.2.30.1	Test Purpose and Environment	3523

A.9.2.30.2	Test parameters	3523

A.9.2.30.3	Test Requirements	3526

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3526

A.9.2.31.1	Test Purpose and Environment	3526

A.9.2.31.2	Test parameters	3526

A.9.2.31.3	Test Requirements	3529

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3529

A.9.2.32.1	Test Purpose and Environment	3529

A.9.2.32.2	Test parameters	3529

A.9.2.32.3	Test Requirements	3532

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3532

A.9.2.33.1	Test Purpose and Environment	3532

A.9.2.33.2	Test parameters	3532

A.9.2.33.3	Test Requirements	3536

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3536

A.9.2.34.1	Test Purpose and Environment	3536

A.9.2.34.2	Test parameters	3536

A.9.2.34.3	Test Requirements	3539

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3539

A.9.2.35.1	Test Purpose and Environment	3539

A.9.2.35.2	Test parameters	3539

A.9.2.35.3	Test Requirements	3542

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3542

A.9.2.36.1	Test Purpose and Environment	3542

A.9.2.36.2	Test parameters	3542

A.9.2.36.3	Test Requirements	3543

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3544

A.9.2.37.1	Test Purpose and Environment	3544

A.9.2.37.2	Test parameters	3544

A.9.2.37.3	Test Requirements	3545

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3546

A.9.2.38.1	Test Purpose and Environment	3546

A.9.2.38.2	Test parameters	3546

A.9.2.38.3	Test Requirements	3550

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3550

A.9.2.39.1	Test Purpose and Environment	3550

A.9.2.39.2	Test parameters	3550

A.9.2.39.3	Test Requirements	3554

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3554

A.9.2.40.1	Test Purpose and Environment	3554

A.9.2.40.2	Test parameters	3554

A.9.2.40.3	Test Requirements	3557

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3558

A.9.2.41.1	Test Purpose and Environment	3558

A.9.2.41.2	Test parameters	3558

A.9.2.41.3	Test Requirements	3561

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3561

A.9.2.42.1	Test Purpose and Environment	3561

A.9.2.42.2	Test parameters	3561

A.9.2.42.3	Test Requirements	3564

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3564

A.9.2.43.1	Test Purpose and Environment	3564

A.9.2.43.2	Test parameters	3564

A.9.2.43.3	Test Requirements	3567

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3567

A.9.2.44.1	Test Purpose and Environment	3567

A.9.2.44.2	Test parameters	3567

A.9.2.44.3	Test Requirements	3569

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3569

A.9.2.45.1	Test Purpose and Environment	3569

A.9.2.45.2	Test parameters	3569

A.9.2.45.3	Test Requirements	3573

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3574

A.9.2.46.1	Test Purpose and Environment	3574

A.9.2.46.2	Test parameters	3574

A.9.2.46.3	Test Requirements	3578

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3579

A.9.2.47.1	Test Purpose and Environment	3579

A.9.2.47.2	Test parameters	3579

A.9.2.47.3	Test Requirements	3583

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3583

A.9.2.48.1	Test Purpose and Environment	3583

A.9.2.48.2	Test parameters	3584

A.9.2.48.3	Test Requirements	3588

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3589

A.9.2.49.1	Test Purpose and Environment	3589

A.9.2.49.2	Test parameters	3589

A.9.2.49.3	Test Requirements	3595

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3596

A.9.2.50.1	Test Purpose and Environment	3596

A.9.2.50.2	Test parameters	3596

A.9.2.50.3	Test Requirements	3602

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3602

A.9.2.51.1	Test Purpose and Environment	3602

A.9.2.51.2	Test parameters	3603

A.9.2.51.3	Test Requirements	3607

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3607

A.9.2.52.1	Test Purpose and Environment	3607

A.9.2.52.2	Test parameters	3607

A.9.2.52.3	Test Requirements	3610

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3611

A.9.2.53.1	Test Purpose and Environment	3611

A.9.2.53.2	Test parameters	3611

A.9.2.53.3	Test Requirements	3617

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3618

A.9.2.54.1	Test Purpose and Environment	3618

A.9.2.54.2	Test parameters	3618

A.9.2.54.3	Test Requirements	3624

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3624

A.9.2.55.1	Test Purpose and Environment	3624

A.9.2.55.2	Test parameters	3625

A.9.2.55.3	Test Requirements	3630

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3630

A.9.2.56.1	Test Purpose and Environment	3630

A.9.2.56.2	Test parameters	3630

A.9.2.56.3	Test Requirements	3635

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3635

A.9.2.57.1	Test Purpose and Environment	3635

A.9.2.57.2	Test parameters	3636

A.9.2.57.3	Test Requirements	3640

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3641

A.9.2.58.1	Test Purpose and Environment	3641

A.9.2.58.2	Test parameters	3641

A.9.2.58.3	Test Requirements	3645

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3646

A.9.2.59.1	Test Purpose and Environment	3646

A.9.2.59.2	Test parameters	3646

A.9.2.59.3	Test Requirements	3651

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3652

A.9.2.60.1	Test Purpose and Environment	3652

A.9.2.60.2	Test parameters	3652

A.9.2.60.3	Test Requirements	3655

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3655

A.9.2.61.1	Test Purpose and Environment	3655

A.9.2.61.2	Test parameters	3655

A.9.2.61.3	Test Requirements	3658

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3658

A.9.2.62.1	Test Purpose and Environment	3658

A.9.2.62.2	Test parameters	3658

A.9.2.62.3	Test Requirements	3661

A.9.3	UTRAN FDD CPICH RSCP	3661

A.9.3.1	E-UTRAN FDD	3661

A.9.3.1.1	Test Purpose and Environment	3661

A.9.3.1.2	Parameters	3661

A.9.3.1.3	Test Requirements	3664

A.9.3.2 E-UTRAN TDD	3664

A.9.3.2.1	Test Purpose and Environment	3664

A.9.3.2.2	Parameters	3664

A.9.3.2.3	Test Requirements	3667

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3667

A.9.3.3.1	Test Purpose and Environment	3667

A.9.3.3.2	Parameters	3667

A.9.3.3.3	Test Requirements	3668

A.9.4	UTRAN FDD CPICH Ec/No	3669

A.9.4.1	E-UTRAN FDD	3669

A.9.4.1.1	Test Purpose and Environment	3669

A.9.4.1.2	Parameters	3669

A.9.4.1.3	Test Requirements	3671

A.9.4.2	E-UTRAN TDD	3672

A.9.4.2.1	Test Purpose and Environment	3672

A.9.4.2.2	Parameters	3672

A.9.4.2.3	Test Requirements	3674

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3675

A.9.4.3.1	Test Purpose and Environment	3675

A.9.4.3.2	Parameters	3675

A.9.4.3.3	Test Requirements	3676

A.9.5	UTRAN TDD measurement	3676

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3676

A.9.5.1.1	Test Purpose and Environment	3676

A.9.5.1.2	Test parameters	3676

A.9.5.1.3	Test Requirements	3678

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3679

A.9.5.2.1	Test Purpose and Environment	3679

A.9.5.2.2	Test parameters	3679

A.9.5.2.3	Test Requirements	3680

A.9.6	GSM Carrier RSSI	3681

A.9.6.1	E-UTRAN FDD	3681

A.9.6.1.1	Test Purpose and Environment	3681

A.9.6.1.2	Test Requirements	3682

A.9.6.2	E-UTRAN TDD	3682

A.9.6.2.1	Test Purpose and Environment	3682

A.9.6.2.2	Test Requirements	3684

A.9.7	UE Rx – Tx Time Difference	3685

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3685

A.9.7.1.1	Test Purpose and Environment	3685

A.9.7.1.2	Test parameters	3685

A.9.7.1.3	Test Requirements	3686

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3686

A.9.7.2.1	Test Purpose and Environment	3686

A.9.7.2.2	Test parameters	3686

A.9.7.2.3	Test Requirements	3688

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3688

A.9.7.3.1	Test Purpose and Environment	3688

A.9.7.3.2	Test parameters	3688

A.9.7.3.3	Test Requirements	3691

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3691

A.9.7.4.1	Test Purpose and Environment	3691

A.9.7.4.2	Test Parameters	3691

A.9.7.4.3	Test Requirements	3694

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3694

A.9.7.5.1	Test Purpose and Environment	3694

A.9.7.5.2	Test parameters	3694

A.9.7.5.3	Test Requirements	3697

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3697

A.9.7.6.1	Test Purpose and Environment	3697

A.9.7.6.2	Test Parameters	3697

A.9.7.6.3	Test Requirements	3700

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3700

A.9.7.7.1	Test Purpose and Environment	3700

A.9.7.7.2	Test parameters	3700

A.9.7.7.3	Test Requirements	3701

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3702

A.9.7.8.1	Test Purpose and Environment	3702

A.9.7.8.2	Test parameters	3702

A.9.7.8.3	Test Requirements	3703

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3703

A.9.7.9.1	Test Purpose and Environment	3703

A.9.7.9.2	Test parameters	3703

A.9.7.9.3	Test Requirements	3704

A.9.8	RSTD	3705

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3705

A.9.8.1.1	Test Purpose and Environment	3705

A.9.8.1.2	Test Requirements	3707

A.9.8.1.2A	Test Requirements for UE Category 1bis	3707

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3708

A.9.8.2.1	Test Purpose and Environment	3708

A.9.8.2.2	Test Requirements	3711

A.9.8.2.2A	Test Requirements for UE Category 1bis	3711

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3712

A.9.8.3.1	Test Purpose and Environment	3712

A.9.8.3.2	Test Requirements	3714

A.9.8.3.2A	Test Requirements for UE Category 1bis	3714

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3715

A.9.8.4.1	Test Purpose and Environment	3715

A.9.8.4.2	Test Requirements	3717

A.9.8.4.2A	Test Requirements for UE Category 1bis	3717

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3718

A.9.8.5.1	Test Purpose and Environment	3718

A.9.8.5.2	Test Requirements	3720

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3720

A.9.8.6.1	Test Purpose and Environment	3720

A.9.8.6.2	Test Requirements	3723

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3723

A.9.8.7.1	Test Purpose and Environment	3723

A.9.8.7.2	Test Requirements	3724

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3724

A.9.8.8.1	Test Purpose and Environment	3724

A.9.8.8.2	Test Requirements	3725

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3725

A.9.8.9.1	Test Purpose and Environment	3725

A.9.8.9.2	Test Requirements	3726

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3726

A.9.8.10.1	Test Purpose and Environment	3726

A.9.8.10.2	Test Requirements	3727

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3727

A.9.8.11.1	Test Purpose and Environment	3727

A.9.8.11.2	Test Requirements	3728

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3728

A.9.8.12.1	Test Purpose and Environment	3728

A.9.8.12.2	Test Requirements	3729

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3729

A.9.8.13.1	Test Purpose and Environment	3729

A.9.8.13.2	Test Requirements	3730

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3730

A.9.8.14.1	Test Purpose and Environment	3730

A.9.8.14.2	Test Requirements	3737

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3737

A.9.8.15.1	Test Purpose and Environment	3737

A.9.8.15.2	Test Requirements	3743

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3743

A.9.8.16.1	Test Purpose and Environment	3743

A.9.8.16.2	Test Requirements	3746

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3746

A.9.8.17.1	Test Purpose and Environment	3746

A.9.8.17.2	Test Requirements	3749

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3749

A.9.8.18.1	Test Purpose and Environment	3749

A.9.8.18.2	Test Requirements	3752

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3752

A.9.8.19.1	Test Purpose and Environment	3752

A.9.8.19.2	Test Requirements	3755

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3755

A.9.8.20.1	Test Purpose and Environment	3755

A.9.8.20.2	Test Requirements	3758

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3758

A.9.8.21.1	Test Purpose and Environment	3758

A.9.8.21.2	Test Requirements	3762

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3763

A.9.8.22.1	Test Purpose and Environment	3763

A.9.8.22.2	Test Requirements	3766

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3767

A.9.8.23.1	Test Purpose and Environment	3767

A.9.8.23.2	Test Requirements	3770

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3770

A.9.8.24.1	Test Purpose and Environment	3770

A.9.8.24.2	Test Requirements	3774

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3774

A.9.8.25.1	Test Purpose and Environment	3774

A.9.8.25.2	Test Requirements	3778

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3779

A.9.8.26.1	Test Purpose and Environment	3779

A.9.8.26.2	Test Requirements	3781

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3781

A.9.8.27.1	Test Purpose and Environment	3781

A.9.8.27.2	Test Requirements	3784

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3784

A.9.8.28.1	Test Purpose and Environment	3784

A.9.8.28.2	Test Requirements	3787

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3787

A.9.8.29.1	Test Purpose and Environment	3787

A.9.8.29.2	Test Requirements	3791

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3792

A.9.8.30.1	Test Purpose and Environment	3792

A.9.8.30.2	Test Requirements	3795

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3796

A.9.8.31.1	Test Purpose and Environment	3796

A.9.8.31.2	Test Requirements	3800

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3800

A.9.8.32.1	Test Purpose and Environment	3800

A.9.8.32.2	Test Requirements	3803

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3803

A.9.8.33.1	Test Purpose and Environment	3803

A.9.8.33.2	Test Requirements	3806

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3806

A.9.8.34.1	Test Purpose and Environment	3806

A.9.8.34.2	Test Requirements	3809

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3809

A.9.8.35.1	Test Purpose and Environment	3809

A.9.8.35.2	Test Requirements	3812

A.9.9	RSRP and RSRQ on the serving cell	3812

A.9.9.1	FDD Intra frequency serving cell case	3812

A.9.9.1.1	Test Purpose and Environment	3812

A.9.9.1.2	Test parameters	3812

A.9.9.1.3	Test Requirements	3815

A.9.9.2	TDD Intra frequency serving cell case	3815

A.9.9.2.1	Test Purpose and Environment	3815

A.9.9.2.2	Test parameters	3815

A.9.9.2.3	Test Requirements	3817

A.9.10	SSTD	3817

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3817

A.9.10.1.1	Test Purpose and Environment	3817

A.9.10.1.2	Test parameters	3817

A.9.10.1.3	Test Requirements	3819

A.9.10.2	Void	3819

A.9.10.3	Void	3819

A.9.10.4	Void	3819

A.9.11 RSSI	3819

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	3819

A.9.11.1.1	Test Purpose and Environment	3819

A.9.11.1.2	Test parameters	3819

A.9.11.1.3	Test Requirements	3822

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	3822

A.9.11.2.1	Test Purpose and Environment	3822

A.9.11.2.2	Test parameters	3822

A.9.12	Channel occupancy	3825

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	3825

A.9.12.1.1	Test Purpose and Environment	3825

A.9.12.1.2	Test parameters	3825

A.9.12.1.3	Test Requirements	3828

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	3828

A.9.12.2.1	Test Purpose and Environment	3828

A.9.12.2.2	Test parameters	3828

A.9.12.2.3	Test Requirements	3831

A.9.13	RS-SINR	3831

A.9.13.1	FDD Intra-Frequency Case	3831

A.9.13.1.1	Test Purpose and Environment	3831

A.9.13.1.2	Test parameters	3831

A.9.13.1.3	Test Requirements	3834

A.9.13.2	TDD Intra-Frequency Case	3834

A.9.13.2.1	Test Purpose and Environment	3834

A.9.13.2.2	Test parameters	3834

A.9.13.2.3	Test Requirements	3837

A.9.13.3	FDD—FDD Inter frequency case	3837

A.9.13.3.1	Test Purpose and Environment	3837

A.9.13.3.2	Test parameters	3837

A.9.13.3.3	Test Requirements	3840

A.9.13.4	TDD—TDD Inter frequency case	3840

A.9.13.4.1	Test Purpose and Environment	3840

A.9.13.4.2	Test parameters	3840

A.9.13.4.3	Test Requirements	3844

A.9.13.5	FDD—TDD Inter frequency case	3845

A.9.13.5.1	Test Purpose and Environment	3845

A.9.13.5.2	Test parameters	3845

A.9.13.5.3	Test Requirements	3851

A.9.13.6	TDD—FDD Inter frequency case	3851

A.9.13.6.1	Test Purpose and Environment	3851

A.9.13.6.2	Test parameters	3851

A.9.13.6.3	Test Requirements	3855

A.9.14	Channel quality reporting accuracy	3855

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3855

A.9.14.1.1	Test Purpose and Environment	3855

A.9.14.1.2	Test parameters	3856

A.9.14.1.3	Test Requirements	3856

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3856

A.9.14.2.1	Test Purpose and Environment	3856

A.9.14.2.2	Test parameters	3857

A.9.14.2.3	Test Requirements	3857

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3858

A.9.14.3.1	Test Purpose and Environment	3858

A.9.14.3.2	Test parameters	3858

A.9.14.3.3	Test Requirements	3859

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3859

A.9.14.4.1	Test Purpose and Environment	3859

A.9.14.4.2	Test parameters	3859

A.9.14.4.3	Test Requirements	3860

A.10	Proximity-based Services in Any Cell Selection State	3883

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3883

A.10.1.1	Test Purpose and Environment	3883

A.10.1.2	Test Requirements	3884

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3885

A.10.2.1	Test Purpose and Environment	3885

A.10.2.2	Test Requirements	3886

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3886

A.10.3.1	Test Purpose and Environment	3886

A.10.3.2	Test Requirements	3888

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3889

A.10.4.1	Test Purpose and Environment	3889

A.10.4.2	Test Requirements	3891

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3891

A.11.1	V2V UE Transmission Timing Accuracy Test	3891

A.11.1.1	Test Purpose and Environment	3891

A.11.1.2	Test requirements	3892

A.11.2	Interruptions due to V2V sidelink communication	3892

A.11.2.1	Test Purpose and Environment	3892

A.11.2.2	Test Requirements	3894

A.12	3894

A.12.1	V2X UE Transmission Timing Accuracy Test	3894

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3894

A.12.1.1.1	Test Purpose and Environment	3894

A.12.1.1.2	Test requirements	3895

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3896

A.12.1.2.1	Test Purpose and Environment	3896

A.12.1.2.2	Test Requirements	3896

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3897

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3897

A.12.2.1.1	Test Purpose and Environment	3897

A.12.2.1.2	Test Requirements	3898

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3899

A.12.2.2.1	Test Purpose and Environment	3899

A.12.2.2.2	Test Requirements	3900

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3900

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3900

A.12.3.1.1	Test Purpose and Environment	3900

A.12.3.1.2	Test Requirements	3903

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3904

A.12.3.2.1	Test Purpose and Environment	3904

A.12.3.1.2	Test Requirements	3906

A.12.4	Congestion Control Measurement Test for V2X UE	3906

A.12.4.1	Test Purpose and Environment	3906

A.12.4.2	Test Requirements	3908

A.12.5	Interruptions due to V2X Sidelink Communication	3908

A.12.5.1	Test Purpose and Environment	3908

A.12.5.2	Test Requirements	3910

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3910

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3910

A.12.6.1.1	Test Purpose and Environment	3910

A.12.6.1.2	Test Requirements	3912

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3912

A.12.6.2.1	Test Purpose and Environment	3912

A.12.6.1.2	Test Requirements	3914

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3914

A.12.7.1	Test Purpose and Environment	3914

A.12.7.2	Test Requirements	3917

A.12.8	Interruptions due to V2X Carrier Aggregation	3918

A.12.8.1	Interruptions on a FDD PCell	3918

A.12.8.1.1	Test Purpose and Environment	3918

A.12.8.1.2	Test Requirements	3919

A.12.8.2	Interruptions on a TDD PCell	3919

A.12.8.2.1	Test Purpose and Environment	3919

A.12.8.2.2	Test Requirements	3921

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3922

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3922

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3922

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3922

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3922

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3924

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3925

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3925

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3925

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3925

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3927

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3928

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3928

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3928

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3929

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3929

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3930

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3930

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3930

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3931

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3931

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3932

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3932

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3932

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3932

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3933

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3933

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3933

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3934

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3934

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3934

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3935

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3935

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3936

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3937

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3938

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3939

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3940

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3940

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3940

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3940

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3941

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3941

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3943

B.3	Conditions for measurements performance requirements for UE	3943

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3943

B.3.2	Void	3943

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3943

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3943

B.3.5	Conditions for UE Rx – Tx time difference	3943

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3944

B.3.7	Conditions for inter-frequency RSTD measurements	3944

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3944

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3944

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3944

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3945

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3945

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3945

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3945

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3945

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3945

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3946

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3946

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3946

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3947

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3947

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3947

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3947

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3947

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3947

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3947

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3947

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3947

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3948

B.3.21.1	Conditions for RSRP measurements	3948

B.3.21.2	Conditions for RSRQ measurements	3948

B.3.21.3	Conditions for CSI-RSRP measurements	3948

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3948

B.3.22.1	Conditions for RSRP measurements	3948

B.3.22.2	Void	3949

B.3.22.3	Conditions for CSI-RSRP measurements	3949

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3949

B.3.23.1	Conditions for RSRP measurements	3949

B.3.23.2	Conditions for RSRQ measurements	3949

B.3.23.3	Conditions for CSI-RSRP measurements	3949

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3950

B.3.24.1	Conditions for RSRP measurements	3950

B.3.24.2	Conditions for RSRQ measurements	3950

B.3.24.3	Conditions for CSI-RSRP measurements	3950

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3950

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3950

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3950

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3951

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3951

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3951

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3951

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3951

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3952

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3952

B.4	RRM Requirements Exceptions	3952

B.4.1	General	3952

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3952

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3952

B.4.3.1	Inter-band carrier aggregation	3952

B.4.3.2	Intra-band non-contiguous carrier aggregation	3952

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3953

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3953

B.5.1	Conditions for S-RSRP Accuracy Requirements	3953

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3953

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3954

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3954

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3955

B.6	Conditions for V2X	3955

B.6.1	Test parameters for GNSS signals	3955

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3955

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3955

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3956

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3956

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3957

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3957

Annex C (informative): Change history:	3958