# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

The present document is part 1 of a multi-part deliverable covering the 5G System (5GS) User Equipment (UE) conformance specification, as identified below:

-	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment" (the present document).

-	3GPP TS 38.508-2 [10]: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

# 1	Scope

The present document defines the test environment for the 5G System.

This specification covers all aspects, including NG-RAN, 5GC and interworking between 5GS and EPS used for conformance tests of User Equipment (UE).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[3]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[4]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[5]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[6]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[7]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[8]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[9]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[10]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

[11]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)".

[12]	3GPP TS 38.523-1: "5GS; User Equipment (UE) conformance specification; Part 1: Protocol".

[13]	3GPP TS 38.133: “NR; Requirements for support of radio resource management”.

[14]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone".

[15]	3GPP TS 38.521-2: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”.

[16]	3GPP TS 38.521-3: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[17]	3GPP TS 38.521-4: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”.

[18]	3GPP TS 38.533: “NR; User Equipment (UE) conformance specification; Radio resource management”.

[19]	3GPP TS 38.523-2: “5GS; User Equipment (UE) conformance specification; Part 2: Applicability of protocol test cases”.

[20]	3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”.

[21]	3GPP TS 38.214: “NR; Physical layer procedures for data”.

[22]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

[23]	3GPP TS 38.523-3: "5GS; UE conformance specification; Part 3: Protocol Test Suites".

[24]	3GPP TR 38.810: “NR; Study on test methods”

[25]	3GPP TS 23.041: “Technical realization of Cell Broadcast Service (CBS)”

[26]	3GPP TS 23.003: “Numbering, addressing and identification”

[27]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[28]	3GPP TS 24.501: “Non-Access-Stratum (NAS) protocol for 5G System (5GS);Stage 3”

[29]	3GPP TS 38.211: "NR; Physical channels and modulation".

[30]	IETF RFC 4187: " Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA) ".

[31]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[32]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[33]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[34]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[35]	3GPP TS 24.502: “Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN); Stage 3”

[36]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification ".

[37]	3GPP TS 36.523-2: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[38]	3GPP TR 38.903: “NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”

[39]	3GPP TS 37.571-1: "Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 1: Conformance test specification".

[40]	3GPP TS 37.571-2: "Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance".

[41]	3GPP TS 36.523-3: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test Suites".

[42]	3GPP TS 36.523-1: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[43]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[44]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[45]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS);Multimedia Telephony;Media handling and interaction".

[46]	IETF RFC 4566: "SDP: Session Description Protocol".

[47]	3GPP TS 34.229-5: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)".

[48]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[49]	3GPP TS 37.571-5: "User Equipment (UE) conformance specification for UE positioning; Part 5: Test scenarios and assistance data".

[50]	STANAG 4294: "NATO STANAG 4294. Navstar Global Positioning System (GPS) System Characteristics".

[51]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP);User Equipment (UE) conformance specification;Part 2: Implementation Conformance Statement (ICS) specification".

[52]	3GPP TS 34.108: "Common Test Environments for User Equipment (UE); Conformance testing".

[53]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[54]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS)".

[55]	3GPP TS 38.306: "User equipment (UE) radio access capabilities".

[56]	3GPP TS 38.101-5: "NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[57]	3GPP TS 38.521-5: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[58]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[59]	3GPP TS 24.555: "Proximity-services (ProSe) in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[60]	3GPP TS 23.304: "Proximity-services (ProSe) in the 5G System (5GS) ".

[61]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[62]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[63]	3GPP TS 33.503: "Security aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[64]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

B: a value followed by "B" is a binary value.

H: a value followed by "H" is a hexadecimal value.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GMM	5GS Mobility Management

5GS	5G System

5GSM	5GS Session Management

ATG	Air-To-Ground

EN-DC	E-UTRA-NR Dual Connectivity

MCG	Master Cell Group

MR-DC	Multi-RAT Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGC	NG Core Network. Synonym of 5GC.

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NG-RAN	NG Radio Access Network

NR	NR Radio Access

RedCap	Reduced Capability

RRC	Radio Resource Control

SCG	Secondary Cell Group

SS	System Simulator

# 4	Common test environments

## 4.1	Environmental conditions

The requirements in this clause apply to all types of UE(s).

### 4.1.1	Temperature

Regarding FR1 the UE shall fulfil all the requirements in the full temperature range of:

Table 4.1.1-1: Temperature conditions for FR1

| +15C to +35C | For normal conditions (with relative humidity up to 75 %) |
| --- | --- |
| -10C to +55C | For extreme conditions (see IEC publications 68-2-1 and 68-2-2) |

Outside this temperature range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 38.101-1 [7] clause 6.2 for extreme operation.

The normative reference for this requirement is TS 38.101-1 [7] Annex E.2.

All RF requirements for UEs operating in FR2 are defined over the air and can only be tested in an OTA chamber.

Regarding FR2 the UE shall fulfil all requirements in the temperature range defined in Table 4.1.1-2.

Table 4.1.1-2: Temperature conditions for FR2

| +15C to +35C | For normal (room temperature) conditions with relative humidity up to 75% |
| --- | --- |
| -10C to +55C | For extreme conditions |

Outside this temperature range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 38.101-2[8] clause 6.2 for extreme operation.

The normative reference for this requirement is TS 38.101-2 [8] Annex E.2.

Some tests are performed also in extreme temperature conditions. These test conditions are denoted as TL (temperature low, -10C) and TH (temperature high, +55C).

### 4.1.2	Voltage

Editor's Note: This clause is incomplete. The following items are missing or are incomplete:

- Methodology to control the voltage in a case which a power cable is not connected to DUT is FFS since it is not agreed whether we can connect the power cable to DUT at the OTA measurement situation yet.

Regarding both FR1 and FR2 the UE shall fulfil all the requirements in the full voltage range, i.e. the voltage range between the extreme voltages.

The manufacturer shall declare the lower and higher extreme voltages and the approximate shutdown voltage. For the equipment that can be operated from one or more of the power sources listed below, the lower extreme voltage shall not be higher, and the higher extreme voltage shall not be lower than that specified below.

Table 4.1.2-1: Voltage conditions

| Power source | Lower extreme voltage | Higher extreme voltage | Normal conditions voltage |
| --- | --- | --- | --- |
| AC mains | 0.9 * nominal | 1.1 * nominal | nominal |
| Regulated lead acid battery | 0.9 * nominal | 1.3 * nominal | 1.1 * nominal |
| Non regulated batteries: Leclanché Lithium Mercury/nickel & cadmium | 0.85 * nominal 0.95 * nominal 0.90 * nominal | nominal 1.1 * nominal | nominal 1.1 * nominal nominal |

Outside this voltage range the UE if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 38.101-1[7] and TS 38.101-2[8] clause 6.2 for extreme operation. In particular, the UE shall inhibit all RF transmissions when the power supply voltage is below the manufacturer declared shutdown voltage.

The normative reference for this requirement is TS 38.101-1 [7] Annex E.2 and TS 38.101-2 [8] Annex E.2.

Some tests are performed also in extreme voltage conditions. These test conditions are denoted as VL (lower extreme voltage) and VH (higher extreme voltage).

## 4.2	Common requirements of test equipment

Mobile conformance testing can be categorized into 3 distinct areas:

-	RF Conformance Testing consisting of:

-	Transmission and Reception Conformance Testing.

-	Performance Conformance Testing.

-	RRM Conformance Testing.

-	Signalling Conformance Testing.

The test equipment required for each category of testing may or not be different, depending on the supplier of the test equipment. However, there will be some generic requirements of the test equipment that are essential for all three categories of test, and these are specified in this clause.

In addition, there will be requirements to test operation in multi-system configurations other than E-UTRA and NR dual connectivity (EN-DC). However, these would not form a common test equipment requirement for the three test areas and are not considered in the present document.

### 4.2.1	General functional requirements

NOTE:	This clause has been written such that it does not constrain the implementation of different architectures and designs of test equipment.

All test equipment used to perform conformance testing for frequency range 1 on a UE shall provide the following minimum functionality:

-	Conducted test method

All test equipment used to perform conformance testing for frequency range 2 on a UE shall provide the following minimum functionality:

-	OTA test method

All test equipment used to perform conformance testing on a UE shall provide a platform suitable for testing UE's in at least one of the following connectivity values:

-	NR; or

-	EN-DC; or

-	NE-DC; or

-	NR-DC; or

-	WLAN; or

-	E-UTRA/EPC

All test equipment used to perform conformance testing on a UE shall provide a platform suitable for testing UE's that are either of following duplex mode for NR and E-UTRA (EN-DC or NE-DC connectivity values only) respectively.

a)	FDD Mode; or

b)	TDD Mode; or

c)	both FDD/TDD Modes; or

d)	HD-FDD Mode (NR RedCap only).

All test equipment shall provide the following minimum functionality.

-	The capability of emulating a single NR cell and a single E-UTRA (for EN-DC or NE-DC connectivity values only) cell with the appropriate channels to allow the UE to register on the cell.

-	The capability to allow the UE to set up an RRC connection with the system simulator, and to maintain the connection for the duration of the test.

-	The capability (for the specific test):

-	to select and support an appropriate radio bearer for the downlink;

-	to set up and support the appropriate radio bearer for the uplink;

### 4.2.2	Minimum functional requirements

#### 4.2.2.1	Supported Cell Configuration

The System Simulator shall provide the capability to simulate a minimum number of cells whose number and capabilities are governed by the test cases that need to be performed (test cases are defined in TS 38.523-1 [12] (Signalling), TS 38.521-1 [14], TS 38.521-2 [15], TS 38.521-3 [16] (TRx), TS 38.521-4 [17] (Performance), TS 38.533 [18] (RRM), TS 37.571-1 [39] and TS 37.571-2 [40] (Positioning)).

To perform test cases requiring multiple cell(s), the system simulator shall provide multiple cells offering the capabilities as required by the test case.

The type and number of channels (especially physical channels) constitute an important set of capabilities for a cell. The following clauses list possible channels that may be supported by the SS. Each channel type, however, and the minimum number of channels needed are only mandatory if specific test cases require them.

The mapping between Logical and Transport channels is as described in TS 38.321 [20]. Similarly, the mapping between Transport channels and Physical channels is as described in TS 38.211, TS 38.302 and TS 38.212. The reference measurement channels (mapping between Transport channels and Physical channels for PDSCH/PDCCH) are defined in TS 38.521-1 [14] annex A

##### 4.2.2.1.1	Supported Channels for an E-UTRA cell (NSA mode only)

Requirement for supported channels for E-UTRA cell is described in TS 36.508[2].

##### 4.2.2.1.2	Supported Channels for a NR cell

###### 4.2.2.1.2.1	Logical channels

| Logical channel | Minimum number | Comments |
| --- | --- | --- |
| BCCH | 0 for EN-DC, 1 for SA |  |
| CCCH | 0 for EN-DC, 1 for SA |  |
| DCCH | 0 for EN-DC, 2 for SA | Split SRB or SRB3 is optional in EN-DC |
| PCCH | 0 for EN-DC, 1 for SA |  |
| DTCH | n | Depending on SS's support for RB service testing |

###### 4.2.2.1.2.2	Transport channels

| Transport channel | Minimum number | Comments |
| --- | --- | --- |
| BCH | 1 |  |
| PCH | N/A for EN-DC, 1 for SA |  |
| RACH | 1 |  |
| DL-SCH | 1 |  |
| UL-SCH | 1 |  |

###### 4.2.2.1.2.3	Physical channels

| Physical channel | Minimum number | Comments |
| --- | --- | --- |
| PBCH | 1 | Physical Broadcast Channel |
| PDCCH | 1 | The physical downlink control channel carries scheduling assignments and other control information. |
| PDSCH | 1 | Physical Downlink Shared Channel |
| PUCCH | 1 | The physical uplink control channel carries uplink control information |
| PUSCH | 1 | Physical Uplink Shared Channel |
| PRACH | 1 | Physical Random Access Channel |

###### 4.2.2.1.2.4	Physical signals

| Physical signal | Minimum number | Comments |
| --- | --- | --- |
| Demodulation reference signal | NA | UL |
| Sounding Reference signal | NA | UL, if applicable |
| Phase Tracking Reference Signal | NA | UL, if applicable |
| Demodulation reference signal(PDSCH) | NA | DL |
| Demodulation reference signal(PDCCH) | NA | DL |
| Demodulation reference signal(PBCH) | NA | DL |
| Phase Tracking Reference Signal | NA | DL, if applicable |
| CSI reference signal | NA | DL |
| Primary synchronisation signal | NA | DL |
| Secondary synchronisation signal | NA | DL |

## 4.3	Reference test conditions

### 4.3.1	Test frequencies

Editor's Note: n65 (High: 50MHz), n66 (High: 45MHz), n75(High: 50MHz), and n80 (High: 40MHz) are currently not yet included in the test channel bandwidth tables due to test frequency have not been defined yet. Further analysis is needed for the test coverage of these test channel bandwidths and bands.

#### 4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8], TS 38.101-3 [9] and TS 38.101-5 [56].

#### 4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1, Table 4.3.1.0A-2 and Table 4.3.1.0A-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap UE Mid Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 25 | 15 |
| n5 | 15 | 15 |
| n7 | 25 | 15 |
| n8 | 15 | 15 |
| n12 | 10 | 10 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 2511 | 15 |
| n26 | 15 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 10 | 10 |
| n38 | 20 | 15 |
| n39 | 20 | 15 |
| n40 | 50 | 15 |
| n41 | 50 | 15 |
| n46 | 604, 405 | N/A |
| n48 | 2011 | 15 |
| n50 | 404, 205,11 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 15 | 15 |
| n66 | 20 | 15 |
| n70 | 152, 103 | 152, 103 |
| n71 | 153, 202 | 15 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 50 | 20 |
| n80 | 203 | [15] |
| n81 | 153 | [15] |
| n82 | 153 | [15] |
| n83 | 153 | [15] |
| n84 | 253 | [15] |
| n85 | 10 | 10 |
| n86 | 203 | [15] |
| n91 | 52, 103 | [52,103] |
| n92 | 15 | [15] |
| n93 | 52, 103 | [52,103] |
| n94 | 15 | [15] |
| n95 | 103 | [10] |
| n96 | 60 | N/A |
| n97 | 503 | [15] |
| n99 | 103 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n106 | 3 | N/A |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1: - If the bandwidth listed above is equal or higher than maximum bandwidth supported in the UE release according to 38.101-1 [7] Table 5.3.5-1, the closest channel BW to the arithmetic average among the values supported in the UE release specification shall be tested. - Otherwise, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 (most updated version of latest release available) [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: This Mid test channel bandwidth is chosen since it is more commonly used. Note 11: This is an exception to Note 1 in this table to avoid not allowed asymmetric channel bandwidths. |  |  |

Table 4.3.1.0A-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 200 | 100 |
| n258 | 200 | 100 |
| n259 | 200 | N/A |
| n260 | 200 | N/A |
| n261 | 200 | 100 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-2 [15] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. |  |  |

Table 4.3.1.0A-3: Mid Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 15 |
| n255 | 15 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-5 [57] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in TS 38.306 clause 4.2.7.2 [55] for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

#### 4.3.1.0B	Low test channel bandwidth

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1, Table 4.3.1.0B-2 and Table 4.3.1.0B-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR Band | UE Low Test Channel bandwidth [MHz]1, 1a, 1b | RedCap UE Low Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 5 | 5 |
| n2 | 5 | 5 |
| n3 | 5 | 5 |
| n5 | 5 | 5 |
| n7 | 5 | 5 |
| n8 | 5 | 5 |
| n12 | 5 | 5 |
| n13 | 5 | 5 |
| n14 | 5 | 5 |
| n20 | 5 | 5 |
| n24 | 5 | 5 |
| n25 | 5 | 5 |
| n26 | 36,5 | 5 |
| n28 | 36,5 | 5 |
| n29 | 52 | N/A |
| n30 | 5 | 5 |
| n34 | 5 | 5 |
| n38 | 5 | 5 |
| n39 | 5 | 5 |
| n40 | 54,105 | 10 |
| n41 | 10 | 10 |
| n46 | 104,205 | N/A |
| n48 | 54, 105 | 10 |
| n50 | 54,105 | 10 |
| n51 | 5 | 5 |
| n53 | 5 | 5 |
| n65 | 5 | 5 |
| n66 | 5 | 5 |
| n70 | 5 | 5 |
| n71 | 5 | 5 |
| n74 | 5 | 5 |
| n75 | 52 | N/A |
| n76 | 52 | N/A |
| n77 | 10 | 10 |
| n78 | 10 | 10 |
| n79 | 10 | 10 |
| n80 | 53 | [5] |
| n81 | 53 | [5] |
| n82 | 53 | [5] |
| n83 | 53 | [5] |
| n84 | 53 | [5] |
| n85 | 36,5 | 5 |
| n86 | 53 | [5] |
| n91 | 5 | [5] |
| n92 | 5 | [5] |
| n93 | 5 | [5] |
| n94 | 5 | [5] |
| n95 | 53 | [5] |
| n96 | 20 | N/A |
| n97 | 53 | [5] |
| n99 | 53 | [5] |
| n100 | 5 | 5 |
| n101 | 36,5 | 5 |
| n106 | 3 | N/A |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable for use as SCell in CA or SCell in DC configuration. Note 5: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 6: This UE channel bandwidth is optional in Release 18. |  |  |

Table 4.3.1.0B-2: Low Test Channel bandwidths for each NR band, FR2

| NR Band | UE Low Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 50 | 50 |
| n258 | 50 | 50 |
| n259 | 50 | N/A |
| n260 | 50 | N/A |
| n261 | 50 | 50 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. |  |  |

Table 4.3.1.0B-3: Low Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 5 |
| n255 | 5 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

#### 4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1, Table 4.3.1.0C-2 and Table 4.3.1.0C-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz]10, 11, 12 | RedCap UE High Test Channel bandwidth [MHz] 10, 11, 12 |
| --- | --- | --- |
| n1 | 50 | 20 |
| n2 | 40 | 20 |
| n3 | 50 | 20 |
| n5 | 201,14, 252 | 20 |
| n7 | 50 | 20 |
| n8 | 201,14, 352 | 20 |
| n12 | 15 | 15 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 20 | 20 |
| n24 | 10 | 10 |
| n25 | 401, 452 | 20 |
| n26 | 20 | 20 |
| n28 | 30 | 20 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 15 | 15 |
| n38 | 25, 4013 | 15 |
| n39 | 40 | 20 |
| n40 | 100 | 20 |
| n41 | 100 | 20 |
| n46 | 80 | N/A |
| n48 | 403, 1004 | 20 |
| n50 | 601,14, 802 | 20 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 20 | 20 |
| n66 | 40 | 20 |
| n70 | 151,14, 252 | 151, 202 |
| n71 | 201, 352 | 20 |
| n74 | 20 | 20 |
| n75 | 202 | N/A |
| n76 | 52 | N/A |
| n77 | 100 | 20 |
| n78 | 100 | 20 |
| n79 | 100 | 20 |
| n80 | 301 | [20] |
| n81 | 201 | [20] |
| n82 | 201 | [20] |
| n83 | 301 | [20] |
| n84 | 501 | [20] |
| n85 | 15 | 15 |
| n86 | 401 | [20] |
| n91 | 101, 52,14 | [10] |
| n92 | 20 | [20] |
| n93 | 101, 52,14 | [10] |
| n94 | 20 | [20] |
| n95 | 151 | [15] |
| n96 | 80 | N/A |
| n97 | 1001 | [20] |
| n99 | 101 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n106 | 3 | N/A |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-1 [7] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 11: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-1 [7]is mandatory without capability parameter (i.e., purely mandatory) as defined in [55] TS 38.306 clauses 4.2.7.6/4.2.7.8 for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). Note 12: For CA, DC, SDL and SUL, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA, DC, SDL or SUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth is tested instead values described in Note 12 in Table 4.3.1.0C-1. Note 13: These UE channel bandwidths are applicable to sidelink operation. Note 14: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so the value signalled by the UE in supportedBandwidthDL/supportedBandwidthUL is used in single carrier operation instead values described in Table 4.3.1.0C-1.

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR Band | UE High Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 400 | 100 |
| n258 | 400 | 100 |
| n259 | 400 | N/A |
| n260 | 400 | N/A |
| n261 | 400 | 100 |
| Note 1: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-2 [8] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-2 [8] is mandatory without capability parameter (i.e., purely mandatory) as defined in [55] TS 38.306 clauses 4.2.7.6/4.2.7.8 for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). Note 3: For CA and DC, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA and DC, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth is tested instead values described in Note 3 in Table 4.3.1.0C-2. |  |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so the value signalled by the UE is used in single carrier operation instead values described in Table 4.3.1.0C-2.

Table 4.3.1.0C-3: High Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 20 |
| n255 | 20 |
| Note 1: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-5 [56] is mandatory without capability parameter (i.e., purely mandatory) as defined in TS 38.306 clauses 4.2.7.6/4.2.7.8 [55] for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). |  |

#### 4.3.1.0D	Bandwidth part

The value of locationAndBandwidth in BWP for FR1 is given in Table 4.3.1.0D-1. The value of locationAndBandwidth in BWP for FR2 is given in Table 4.3.1.0D-2.

Table 4.3.1.0D-1: locationAndBandwidth in BWP for FR1

| BW (MHz) | SCS (kHz) | LRBs (Note 2) | locationAndBandwidth (Note 1) |
| --- | --- | --- | --- |
| 3 | 15 | 15 | 3850 |
| 5 | 15 | 25 | 6600 |
| 5 | 30 | 11 | 2750 |
| 5 | 60 | N/A | N/A |
| 10 | 15 | 52 | 14025 |
| 10 | 30 | 24 | 6325 |
| 10 | 60 | 11 | 2750 |
| 15 | 15 | 79 | 21450 |
| 15 | 30 | 38 | 10175 |
| 15 | 60 | 18 | 4675 |
| 20 | 15 | 106 | 28875 |
| 20 | 30 | 51 | 13750 |
| 20 | 60 | 24 | 6325 |
| 25 | 15 | 133 | 36300 |
| 25 | 30 | 65 | 17600 |
| 25 | 60 | 31 | 8250 |
| 30 | 15 | 160 | 32174 |
| 30 | 30 | 78 | 21175 |
| 30 | 60 | 38 | 10175 |
| 35 | 15 | 188 | 24474 |
| 35 | 30 | 92 | 25025 |
| 35 | 60 | 44 | 11825 |
| 40 | 15 | 216 | 16774 |
| 40 | 30 | 106 | 28875 |
| 40 | 60 | 51 | 13750 |
| 45 | 15 | 242 | 9624 |
| 45 | 30 | 119 | 32450 |
| 45 | 60 | 58 | 15675 |
| 50 | 15 | 270 | 1924 |
| 50 | 30 | 133 | 36300 |
| 50 | 60 | 65 | 17600 |
| 60 | 15 | N/A | N/A |
| 60 | 30 | 162 | 31624 |
| 60 | 60 | 79 | 21450 |
| 70 | 15 | N/A | N/A |
| 70 | 30 | 189 | 24199 |
| 70 | 60 | 93 | 25300 |
| 80 | 15 | N/A | N/A |
| 80 | 30 | 217 | 16499 |
| 80 | 60 | 107 | 29150 |
| 90 | 15 | N/A | N/A |
| 90 | 30 | 245 | 8799 |
| 90 | 60 | 121 | 33000 |
| 100 | 15 | N/A | N/A |
| 100 | 30 | 273 | 1099 |
| 100 | 60 | 135 | 36850 |
| Note 1: The value for locationAndBandwidth parameter is calculated as the RIV value in accordance to TS 38.214 [21] with = 275, =0 and for each bandwidth and subcarrier spacing. Note 2: = Max NRB is the default configuration, unless explicitly specified. |  |  |  |

Table 4.3.1.0D-2: locationAndBandwidth in BWP for FR2

| BW (MHz) | SCS (kHz) | LRBs (Note 2) | locationAndBandwidth (Note 1) |
| --- | --- | --- | --- |
| 50 | 60 | 66 | 17875 |
| 50 | 120 | 32 | 8525 |
| 100 | 60 | 132 | 36025 |
| 100 | 120 | 24(Note 3) | 6325 |
| 100 | 120 | 48(Note 3) | 12925 |
| 100 | 120 | 66 | 17875 |
| 200 | 60 | 264 | 3574 |
| 200 | 120 | 132 | 36025 |
| 400 | 60 | N/A | N/A |
| 400 | 120 | 264 | 3574 |
| Note 1: The value for locationAndBandwidth parameter is calculated as the RIV value in accordance to TS 38.214 [21] with = 275 and =0 for each bandwidth and subcarrier spacing. Note 2: = Max NRB is the default configuration, unless explicitly specified. Note 3: For configurations where the is explicitly specified to be < Max NRB. |  |  |  |

#### 4.3.1.0E	Void

#### 4.3.1.1	Test frequencies for NR operating bands in FR1

##### 4.3.1.1.1	NR operating bands in FR1

###### 4.3.1.1.1.1	Reference test frequencies for NR operating band n1

Table 4.3.1.1.1.1-1: Test frequencies for NR operating band n1 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2119.39 | 423878 | 102 |  | 5350 | 427970 | 8 | 1 | 0 (0) | 103 |
|  |  |  | High | 2167.5 | 433500 | 2074.53 | 414906 | 504 |  | 5418 | 433470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1857.03 | 371406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1977.5 | 395500 | 1974.17 | 394834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2116.96 | 423392 | 102 |  | 5344 | 427490 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 2165 | 433000 | 2069.6 | 413920 | 504 |  | 5405 | 432490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1854.6 | 370920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1975 | 395000 | 1969.24 | 393848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2114.53 | 422906 | 102 |  | 5338 | 427010 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 2162.5 | 432500 | 2064.67 | 412934 | 504 |  | 5395 | 431570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1852.17 | 370434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1972.5 | 394500 | 1964.31 | 392862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2112.1 | 422420 | 102 |  | 5332 | 426530 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 2160 | 432000 | 2059.74 | 411948 | 504 |  | 5382 | 430590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1849.74 | 369948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1970 | 394000 | 1959.38 | 391876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2109.67 | 421934 | 102 |  | 5326 | 426050 | 4 | 0 | 1 (2) | 104 |
|  |  |  | High | 2157.5 | 431500 | 2054.81 | 410962 | 504 |  | 5369 | 429610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1920.53 | 384106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1847.31 | 369462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1967.5 | 393500 | 1954.45 | 390890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2107.24 | 421448 | 102 |  | 5320 | 425570 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 2155 | 431000 | 2049.88 | 409976 | 504 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1935 | 387000 | 1920.6 | 384120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1844.88 | 368976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1965 | 393000 | 1949.52 | 389904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2102.2 | 420440 | 102 |  | 5308 | 424610 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 2150 | 430000 | 2039.84 | 407968 | 504 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1940 | 388000 | 1920.56 | 384112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1839.84 | 367968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1960 | 392000 | 1939.48 | 387896 | 6 |  | - | - | - | - | - | - |
| 45 | 242 | Downlink | Low | 2132.5 | 426500 | 2110.72 | 422144 | 0 | 15 | 5284 | 422690 | 2 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2099.86 | 419972 | 102 |  | 5302 | 424130 | 6 | 1 | 1(2) | 105 |
|  |  |  | High | 2147.5 | 429500 | 2035 | 407000 | 504 |  | 5320 | 425570 | 10 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1920.72 | 384144 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1837.5 | 367500 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1957.5 | 391500 | 1934.64 | 386928 | 6 |  | - | - | - | - | - | - |
| 50 | 270 | Downlink | Low | 2135 | 427000 | 2110.7 | 422140 | 0 | 15 | 5282 | 422650 | 2 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 2140 | 428000 | 2097.34 | 419468 | 102 |  | 5296 | 423650 | 2 | 0 | 2 (4) | 106 |
|  |  |  | High | 2145 | 429000 | 2029.98 | 405996 | 504 |  | 5307 | 424590 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1945 | 389000 | 1920.7 | 384140 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1834.98 | 366996 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1955 | 391000 | 1929.62 | 385924 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.1-2: Test frequencies for NR operating band n1 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2098.96 | 419792 | 102 |  | 5350 | 427970 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 2165 | 433000 | 1979.24 | 395848 | 504 |  | 5411 | 432970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1764.24 | 352848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1975 | 395000 | 1968.52 | 393704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2140 | 428000 | 2096.44 | 419288 | 102 |  | 5344 | 427490 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2162.5 | 432500 | 1974.22 | 394844 | 504 |  | 5401 | 432050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1761.72 | 352344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1972.5 | 394500 | 1963.5 | 392700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2094.1 | 418820 | 102 |  | 5338 | 427010 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 2160 | 432000 | 1969.38 | 393876 | 504 |  | 5388 | 431070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1759.38 | 351876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1970 | 394000 | 1958.66 | 391732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2091.58 | 418316 | 102 |  | 5332 | 426530 | 2 | 0 | 2 (7) | 218 |
|  |  |  | High | 2157.5 | 431500 | 1964.36 | 392872 | 504 |  | 5375 | 430090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1920.8 | 384160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1756.86 | 351372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1967.5 | 393500 | 1953.64 | 390728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2089.24 | 417848 | 102 |  | 5326 | 426050 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2155 | 431000 | 1959.52 | 391904 | 504 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1935 | 387000 | 1920.96 | 384192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1754.52 | 350904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1965 | 393000 | 1948.8 | 389760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2084.2 | 416840 | 102 |  | 5314 | 425090 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 2150 | 430000 | 1949.48 | 389896 | 504 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1940 | 388000 | 1920.92 | 384184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1749.48 | 349896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1960 | 392000 | 1938.76 | 387752 | 6 |  | - | - | - | - | - | - |
| 45 | 119 | Downlink | Low | 2132.5 | 426500 | 2111.08 | 422216 | 0 | 15 | 5290 | 423170 | 6 | 0 | 3(8) | 16 |
|  |  |  | Mid | 2140 | 428000 | 2081.86 | 416372 | 102 |  | 5308 | 424610 | 10 | 0 | 2(7) | 218 |
|  |  |  | High | 2147.5 | 429500 | 1944.64 | 388928 | 504 |  | 5326 | 426050 | 14 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1921.08 | 384216 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1747.14 | 349428 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1957.5 | 391500 | 1933.92 | 386784 | 6 |  | - | - | - | - | - | - |
| 50 | 133 | Downlink | Low | 2135 | 427000 | 2111.06 | 422212 | 0 | 15 | 5288 | 423130 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2140 | 428000 | 2079.34 | 415868 | 102 |  | 5302 | 424130 | 18 | 0 | 2 (7) | 218 |
|  |  |  | High | 2145 | 429000 | 1939.62 | 387924 | 504 |  | 5313 | 425070 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1945 | 389000 | 1921.06 | 384212 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1744.62 | 348924 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1955 | 391000 | 1928.9 | 385780 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.1-3: Test frequencies for NR operating band n1 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  | Mid | 2140 | 428000 | 2062.6 | 412520 | 102 |  | - | 427568 |
|  |  |  | High | 2165 | 433000 | 1798.16 | 359632 | 504 |  | - | 432568 |
|  |  | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1583.16 | 316632 | 504 |  | - | - |
|  |  |  | High | 1975 | 395000 | 1966.72 | 393344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  | Mid | 2140 | 428000 | 2060.08 | 412016 | 102 |  | - | 427064 |
|  |  |  | High | 2162.5 | 432500 | 1793.14 | 358628 | 504 |  | - | 431564 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1580.64 | 316128 | 504 |  | - | - |
|  |  |  | High | 1972.5 | 394500 | 1961.7 | 392340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  | Mid | 2140 | 428000 | 2057.92 | 411584 | 102 |  | - | 426632 |
|  |  |  | High | 2160 | 432000 | 1788.48 | 357696 | 504 |  | - | 430632 |
|  |  | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1578.48 | 315696 | 504 |  | - | - |
|  |  |  | High | 1970 | 394000 | 1957.04 | 391408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  | Mid | 2140 | 428000 | 2055.4 | 411080 | 102 |  | - | 426128 |
|  |  |  | High | 2157.5 | 431500 | 1783.46 | 356692 | 504 |  | - | 429628 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1921.34 | 384268 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1575.96 | 315192 | 504 |  | - | - |
|  |  |  | High | 1967.5 | 393500 | 1952.02 | 390404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  | Mid | 2140 | 428000 | 2052.88 | 410576 | 102 |  | - | 425624 |
|  |  |  | High | 2155 | 431000 | 1778.44 | 355688 | 504 |  | - | 428624 |
|  |  | Uplink | Low | 1935 | 387000 | 1921.32 | 384264 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1573.44 | 314688 | 504 |  | - | - |
|  |  |  | High | 1965 | 393000 | 1947 | 389400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  | Mid | 2140 | 428000 | 2048.2 | 409640 | 102 |  | - | 424688 |
|  |  |  | High | 2150 | 430000 | 1768.76 | 353752 | 504 |  | - | 426688 |
|  |  | Uplink | Low | 1940 | 388000 | 1921.64 | 384328 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1568.76 | 313752 | 504 |  | - | - |
|  |  |  | High | 1960 | 392000 | 1937.32 | 387464 | 6 |  | - | - |
| 45 | 58 | Downlink | Low | 2132.5 | 426500 | 2111.62 | 422324 | 0 | 15 | - | 422684 |
|  |  |  | Mid | 2140 | 428000 | 2045.68 | 409136 | 102 |  | - | 424184 |
|  |  |  | High | 2147.5 | 429500 | 1763.74 | 352748 | 504 |  | - | 425684 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1921.62 | 384324 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1566.24 | 313248 | 504 |  | - | - |
|  |  |  | High | 1957.5 | 391500 | 1932.3 | 386460 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 2135 | 427000 | 2111.6 | 422320 | 0 | 15 | - | 422680 |
|  |  |  | Mid | 2140 | 428000 | 2043.16 | 408632 | 102 |  | - | 423680 |
|  |  |  | High | 2145 | 429000 | 1758.72 | 351744 | 504 |  | - | 424680 |
|  |  | Uplink | Low | 1945 | 389000 | 1921.6 | 384320 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1563.72 | 312744 | 504 |  | - | - |
|  |  |  | High | 1955 | 391000 | 1927.28 | 385456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.2	 Reference test frequencies for NR operating band n2

Table 4.3.1.1.1.2-1: Test frequencies for NR operating band n2 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1932.5 | 386500 | 1930.25 | 386050 | 0 | 15 | 4829 | 386410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1939.39 | 387878 | 102 |  | 4900 | 391970 | 8 | 1 | 0 (0) | 103 |
|  |  |  | High | 1987.5 | 397500 | 1894.53 | 378906 | 504 |  | 4968 | 397470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1852.5 | 370500 | 1850.25 | 370050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1787.03 | 357406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1904.17 | 380834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1935 | 387000 | 1930.32 | 386064 | 0 | 15 | 4830 | 386430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1936.96 | 387392 | 102 |  | 4894 | 391490 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1985 | 397000 | 1889.6 | 377920 | 504 |  | 4955 | 396490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.32 | 370064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1784.6 | 356920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1899.24 | 379848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1937.5 | 387500 | 1930.39 | 386078 | 0 | 15 | 4831 | 386450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1934.53 | 386906 | 102 |  | 4888 | 391010 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 1982.5 | 396500 | 1884.67 | 376934 | 504 |  | 4945 | 395570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.39 | 370078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1782.17 | 356434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1894.31 | 378862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1940 | 388000 | 1930.46 | 386092 | 0 | 15 | 4832 | 386650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1932.1 | 386420 | 102 |  | 4882 | 390530 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1980 | 396000 | 1879.74 | 375948 | 504 |  | 4932 | 394590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.46 | 370092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1779.74 | 355948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1889.38 | 377876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 1942.5 | 388500 | 1930.53 | 386106 | 0 | 15 | 4833 | 386670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1929.67 | 385934 | 102 |  | 4876 | 390050 | 4 | 0 | 1 (2) | 104 |
|  |  |  | High | 1977.5 | 395500 | 1874.81 | 374962 | 504 |  | 4919 | 393610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.53 | 370106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1777.31 | 355462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1884.45 | 376890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 1945 | 389000 | 1930.6 | 386120 | 0 | 15 | 4834 | 386690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1927.24 | 385448 | 102 |  | 4870 | 389570 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 1975 | 395000 | 1869.88 | 373976 | 504 |  | 4909 | 392690 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.6 | 370120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1774.88 | 354976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1879.52 | 375904 | 6 |  | - | - | - | - | - | - |
| 35 | 188 | Downlink | Low | 1947.5 | 389500 | 1930.58 | 386116 | 0 | 15 | 4832 | 386650 | 10 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 1960 | 392000 | 1924.72 | 384944 | 102 |  | 4864 | 389090 | 2 | 1 | 1 (2) | 105 |
|  |  |  | High | 1972.5 | 394500 | 1864.86 | 372972 | 504 |  | 4896 | 391710 | 6 | 2 | 2 (4) | 510 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1850.58 | 370116 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1772.36 | 354472 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892.5 | 378500 | 1874.5 | 374900 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 1950 | 390000 | 1930.56 | 386112 | 0 | 15 | 4833 | 386670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1922.2 | 384440 | 102 |  | 4858 | 388610 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 1970 | 394000 | 1859.84 | 371968 | 504 |  | 4880 | 390490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.56 | 370112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1769.84 | 353968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1890 | 378000 | 1869.48 | 373896 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.2-2: Test frequencies for NR operating band n2 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1935 | 387000 | 1930.68 | 386136 | 0 | 15 | 4836 | 386910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1918.96 | 383792 | 102 |  | 4900 | 391970 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 1985 | 397000 | 1799.24 | 359848 | 504 |  | 4961 | 396970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.68 | 370136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1694.24 | 338848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1898.52 | 379704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1937.5 | 387500 | 1930.66 | 386132 | 0 | 15 | 4837 | 386930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1960 | 392000 | 1916.44 | 383288 | 102 |  | 4894 | 391490 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1982.5 | 396500 | 1794.22 | 358844 | 504 |  | 4951 | 396050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.66 | 370132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1691.72 | 338344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1893.5 | 378700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1940 | 388000 | 1930.82 | 386164 | 0 | 15 | 4835 | 386890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1914.1 | 382820 | 102 |  | 4888 | 391010 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1980 | 396000 | 1789.38 | 357876 | 504 |  | 4938 | 395070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.82 | 370164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1689.38 | 337876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1888.66 | 377732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 1942.5 | 388500 | 1930.8 | 386160 | 0 | 15 | 4836 | 386910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1911.58 | 382316 | 102 |  | 4882 | 390530 | 2 | 0 | 2 (7) | 218 |
|  |  |  | High | 1977.5 | 395500 | 1784.36 | 356872 | 504 |  | 4925 | 394090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.8 | 370160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1686.86 | 337372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1883.64 | 376728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 1945 | 389000 | 1930.96 | 386192 | 0 | 15 | 4837 | 386930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1909.24 | 381848 | 102 |  | 4876 | 390050 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1975 | 395000 | 1779.52 | 355904 | 504 |  | 4912 | 392930 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.96 | 370192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1684.52 | 336904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1878.8 | 375760 | 6 |  | - | - | - | - | - | - |
| 35 | 92 | Downlink | Low | 1947.5 | 389500 | 1930.94 | 386188 | 0 | 15 | 4838 | 387130 | 2 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 1960 | 392000 | 1906.72 | 381344 | 102 |  | 4870 | 389570 | 6 | 0 | 2 (7) | 218 |
|  |  |  | High | 1972.5 | 394500 | 1774.5 | 354900 | 504 |  | 4899 | 391950 | 14 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1850.94 | 370188 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1682 | 336400 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892.5 | 378500 | 1873.78 | 374756 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 1950 | 390000 | 1930.92 | 386184 | 0 | 15 | 4836 | 386910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1904.2 | 380840 | 102 |  | 4864 | 389090 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 1970 | 394000 | 1769.48 | 353896 | 504 |  | 4886 | 390970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.92 | 370184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1679.48 | 335896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1890 | 378000 | 1868.76 | 373752 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.2-3: Test frequencies for NR operating band n2 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1935 | 387000 | 1931.04 | 386208 | 0 | 15 | - | 386568 |
|  |  |  | Mid | 1960 | 392000 | 1882.6 | 376520 | 102 |  | - | 391568 |
|  |  |  | High | 1985 | 397000 | 1618.16 | 323632 | 504 |  | - | 396568 |
|  |  | Uplink | Low | 1855 | 371000 | 1851.04 | 370208 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1513.16 | 302632 | 504 |  | - | - |
|  |  |  | High | 1905 | 381000 | 1896.72 | 379344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1937.5 | 387500 | 1931.02 | 386204 | 0 | 15 | - | 386564 |
|  |  |  | Mid | 1960 | 392000 | 1880.08 | 376016 | 102 |  | - | 391064 |
|  |  |  | High | 1982.5 | 396500 | 1613.14 | 322628 | 504 |  | - | 395564 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1851.02 | 370204 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1510.64 | 302128 | 504 |  | - | - |
|  |  |  | High | 1902.5 | 380500 | 1891.7 | 378340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1940 | 388000 | 1931.36 | 386272 | 0 | 15 | - | 386632 |
|  |  |  | Mid | 1960 | 392000 | 1877.92 | 375584 | 102 |  | - | 390632 |
|  |  |  | High | 1980 | 396000 | 1608.48 | 321696 | 504 |  | - | 394632 |
|  |  | Uplink | Low | 1860 | 372000 | 1851.36 | 370272 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1508.48 | 301696 | 504 |  | - | - |
|  |  |  | High | 1900 | 380000 | 1887.04 | 377408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 1942.5 | 388500 | 1931.34 | 386268 | 0 | 15 | - | 386628 |
|  |  |  | Mid | 1960 | 392000 | 1875.4 | 375080 | 102 |  | - | 390128 |
|  |  |  | High | 1977.5 | 395500 | 1603.46 | 320692 | 504 |  | - | 393628 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1851.34 | 370268 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1505.96 | 301192 | 504 |  | - | - |
|  |  |  | High | 1897.5 | 379500 | 1882.02 | 376404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 1945 | 389000 | 1931.32 | 386264 | 0 | 15 | - | 386624 |
|  |  |  | Mid | 1960 | 392000 | 1872.88 | 374576 | 102 |  | - | 389624 |
|  |  |  | High | 1975 | 395000 | 1598.44 | 319688 | 504 |  | - | 392624 |
|  |  | Uplink | Low | 1865 | 373000 | 1851.32 | 370264 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1503.44 | 300688 | 504 |  | - | - |
|  |  |  | High | 1895 | 379000 | 1877 | 375400 | 6 |  | - | - |
| 35 | 44 | Downlink | Low | 1947.5 | 389500 | 1931.66 | 386332 | 0 | 15 | - | 386692 |
|  |  |  | Mid | 1960 | 392000 | 1870.72 | 374144 | 102 |  | - | 389192 |
|  |  |  | High | 1972.5 | 394500 | 1593.78 | 318756 | 504 |  | - | 391692 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1851.66 | 370332 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1501.28 | 300256 | 504 |  | - | - |
|  |  |  | High | 1892.5 | 378500 | 1872.34 | 374468 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 1950 | 390000 | 1931.64 | 386328 | 0 | 15 | - | 386688 |
|  |  |  | Mid | 1960 | 392000 | 1868.2 | 373640 | 102 |  | - | 388688 |
|  |  |  | High | 1970 | 394000 | 1588.76 | 317752 | 504 |  | - | 390688 |
|  |  | Uplink | Low | 1870 | 374000 | 1851.64 | 370328 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1498.76 | 299752 | 504 |  | - | - |
|  |  |  | High | 1890 | 378000 | 1867.32 | 373464 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.3	Reference test frequencies for NR operating band n3

Table 4.3.1.1.1.3-1: Test frequencies for NR operating band n3 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1807.5 | 361500 | 1805.25 | 361050 | 0 | 15 | 4518 | 361470 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1842.5 | 368500 | 1821.89 | 364378 | 102 |  | 4604 | 368410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1877.5 | 375500 | 1784.53 | 356906 | 504 |  | 4693 | 375410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1654.53 | 330906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1782.5 | 356500 | 1779.17 | 355834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1810 | 362000 | 1805.32 | 361064 | 0 | 15 | 4519 | 361490 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1842.5 | 368500 | 1819.46 | 363892 | 102 |  | 4598 | 367930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 1875 | 375000 | 1779.6 | 355920 | 504 |  | 4680 | 374430 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1652.1 | 330420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1780 | 356000 | 1774.24 | 354848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1812.5 | 362500 | 1805.39 | 361078 | 0 | 15 | 4517 | 361450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1817.03 | 363406 | 102 |  | 4592 | 367450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 1872.5 | 374500 | 1774.67 | 354934 | 504 |  | 4667 | 373450 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1649.67 | 329934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1777.5 | 355500 | 1769.31 | 353862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1815 | 363000 | 1805.46 | 361092 | 0 | 15 | 4518 | 361470 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1814.6 | 362920 | 102 |  | 4586 | 366970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 1870 | 374000 | 1769.74 | 353948 | 504 |  | 4657 | 372530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1647.24 | 329448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1775 | 355000 | 1764.38 | 352876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 1817.5 | 363500 | 1805.53 | 361106 | 0 | 15 | 4519 | 361490 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1812.17 | 362434 | 102 |  | 4580 | 366490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 1867.5 | 373500 | 1764.81 | 352962 | 504 |  | 4644 | 371550 | 4 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1644.81 | 328962 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1772.5 | 354500 | 1759.45 | 351890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 1820 | 364000 | 1805.6 | 361120 | 0 | 15 | 4520 | 361690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1842.5 | 368500 | 1809.74 | 361948 | 102 |  | 4574 | 366010 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 1865 | 373000 | 1759.88 | 351976 | 504 |  | 4631 | 370570 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1642.38 | 328476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1770 | 354000 | 1754.52 | 350904 | 6 |  | - | - | - | - | - | - |
| 35 | 188 | Downlink | Low | 1822.5 | 364500 | 1805.58 | 361116 | 0 | 15 | 4521 | 361710 | 6 | 2 | 2 (4) | 6 |
|  |  |  | Mid | 1842.5 | 368500 | 1807.22 | 361444 | 102 |  | 4568 | 365530 | 6 | 1 | 0 (0) | 103 |
|  |  |  | High | 1862.5 | 372500 | 1754.86 | 350972 | 504 |  | 4621 | 369650 | 10 | 0 | 2(4) | 508 |
|  |  | Uplink | Low | 1727.5 | 345500 | 1710.58 | 342116 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1639.86 | 327972 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1767.5 | 353500 | 1749.5 | 349900 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 1825 | 365000 | 1805.56 | 361112 | 0 | 15 | 4519 | 361490 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1804.7 | 360940 | 102 |  | 4562 | 365050 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1860 | 372000 | 1749.84 | 349968 | 504 |  | 4608 | 368670 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1637.34 | 327468 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1765 | 353000 | 1744.48 | 348896 | 6 |  | - |  | - | - | - | - |
| 45 | 242 | Downlink | Low | 1827.5 | 365500 | 1805.72 | 361144 | 0 | 15 | 4520 | 361690 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1842.5 | 368500 | 1802.36 | 360472 | 102 |  | 4556 | 364570 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1857.5 | 371500 | 1745 | 349000 | 504 |  | 4595 | 367690 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1732.5 | 346500 | 1710.72 | 342144 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1635 | 327000 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1762.5 | 352500 | 1739.64 | 347928 | 6 |  | - |  | - | - | - | - |
| 50 | 270 | Downlink | Low | 1830 | 366000 | 1805.7 | 361140 | 0 | 15 | 4521 | 361710 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1842.5 | 368500 | 1799.84 | 359968 | 102 |  | 4550 | 364090 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 1855 | 371000 | 1739.98 | 347996 | 504 |  | 4582 | 366530 | 10 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1735 | 347000 | 1710.7 | 342140 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1632.48 | 326496 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1760 | 352000 | 1734.62 | 346924 | 6 |  | - |  | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.3-2: Test frequencies for NR operating band n3 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1810 | 362000 | 1805.68 | 361136 | 0 | 15 | 4525 | 361970 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1801.46 | 360292 | 102 |  | 4604 | 368410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 1875 | 375000 | 1689.24 | 337848 | 504 |  | 4686 | 374910 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1561.74 | 312348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1780 | 356000 | 1773.52 | 354704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1812.5 | 362500 | 1805.66 | 361132 | 0 | 15 | 4523 | 361930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1798.94 | 359788 | 102 |  | 4598 | 367930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1872.5 | 374500 | 1684.22 | 336844 | 504 |  | 4673 | 373930 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1559.22 | 311844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1777.5 | 355500 | 1768.5 | 353700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1815 | 363000 | 1805.82 | 361164 | 0 | 15 | 4524 | 361950 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1796.6 | 359320 | 102 |  | 4592 | 367450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 1870 | 374000 | 1679.38 | 335876 | 504 |  | 4663 | 373010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1556.88 | 311376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1775 | 355000 | 1763.66 | 352732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 1817.5 | 363500 | 1805.8 | 361160 | 0 | 15 | 4525 | 361970 | 6 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1794.08 | 358816 | 102 |  | 4586 | 366970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 1867.5 | 373500 | 1674.36 | 334872 | 504 |  | 4650 | 372030 | 2 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1554.36 | 310872 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1772.5 | 354500 | 1758.64 | 351728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 1820 | 364000 | 1805.96 | 361192 | 0 | 15 | 4523 | 361930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1791.74 | 358348 | 102 |  | 4580 | 366490 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1865 | 373000 | 1669.52 | 333904 | 504 |  | 4637 | 371050 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1552.02 | 310404 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1770 | 354000 | 1753.8 | 350760 | 6 |  | - | - | - | - | - | - |
| 35 | 92 | Downlink | Low | 1822.5 | 364500 | 1805.94 | 361188 | 0 | 15 | 4524 | 361950 | 14 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1789.22 | 357844 | 102 |  | 4574 | 366010 | 10 | 0 | 1 (6) | 216 |
|  |  |  | High | 1862.5 | 372500 | 1664.5 | 332900 | 504 |  | 4627 | 370130 | 2 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1727.5 | 345500 | 1710.94 | 342188 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1549.5 | 309900 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1767.5 | 353500 | 1748.78 | 349756 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 1825 | 365000 | 1805.92 | 361184 | 0 | 15 | 4525 | 361970 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1786.7 | 357340 | 102 |  | 4568 | 365530 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1860 | 372000 | 1659.48 | 331896 | 504 |  | 4611 | 368910 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1546.98 | 309396 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1765 | 353000 | 1743.76 | 348752 | 6 |  | - |  | - | - | - | - |
| 45 | 119 | Downlink | Low | 1827.5 | 365500 | 1806.08 | 361216 | 0 | 15 | 4526 | 362170 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 1842.5 | 368500 | 1784.36 | 356872 | 102 |  | 4562 | 365050 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 1857.5 | 371500 | 1654.64 | 330928 | 504 |  | 4601 | 368170 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1732.5 | 346500 | 1711.08 | 342216 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1544.64 | 308928 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1762.5 | 352500 | 1738.92 | 347784 | 6 |  | - |  | - | - | - | - |
| 50 | 133 | Downlink | Low | 1830 | 366000 | 1806.06 | 361212 | 0 | 15 | 4524 | 361950 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1781.84 | 356368 | 102 |  | 4556 | 364570 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1855 | 371000 | 1649.62 | 329924 | 504 |  | 4588 | 367010 | 2 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1735 | 347000 | 1711.06 | 342212 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1542.12 | 308424 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1760 | 352000 | 1733.9 | 346780 | 6 |  | - |  | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.3-3: Test frequencies for NR operating band n3 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1810 | 362000 | 1806.04 | 361208 | 0 | 15 | - | 361568 |
|  |  |  | Mid | 1842.5 | 368500 | 1765.1 | 353020 | 102 |  | - | 368068 |
|  |  |  | High | 1875 | 375000 | 1508.16 | 301632 | 504 |  | - | 374568 |
|  |  | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1380.66 | 276132 | 504 |  | - | - |
|  |  |  | High | 1780 | 356000 | 1771.72 | 354344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1812.5 | 362500 | 1806.02 | 361204 | 0 | 15 | - | 361564 |
|  |  |  | Mid | 1842.5 | 368500 | 1762.58 | 352516 | 102 |  | - | 367564 |
|  |  |  | High | 1872.5 | 374500 | 1503.14 | 300628 | 504 |  | - | 373564 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1378.14 | 275628 | 504 |  | - | - |
|  |  |  | High | 1777.5 | 355500 | 1766.7 | 353340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1815 | 363000 | 1806.36 | 361272 | 0 | 15 | - | 361632 |
|  |  |  | Mid | 1842.5 | 368500 | 1760.42 | 352084 | 102 |  | - | 367132 |
|  |  |  | High | 1870 | 374000 | 1498.48 | 299696 | 504 |  | - | 372632 |
|  |  | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1375.98 | 275196 | 504 |  | - | - |
|  |  |  | High | 1775 | 355000 | 1762.04 | 352408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 1817.5 | 363500 | 1806.34 | 361268 | 0 | 15 | - | 361628 |
|  |  |  | Mid | 1842.5 | 368500 | 1757.9 | 351580 | 102 |  | - | 366628 |
|  |  |  | High | 1867.5 | 373500 | 1493.46 | 298692 | 504 |  | - | 371628 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1373.46 | 274692 | 504 |  | - | - |
|  |  |  | High | 1772.5 | 354500 | 1757.02 | 351404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 1820 | 364000 | 1806.32 | 361264 | 0 | 15 | - | 361624 |
|  |  |  | Mid | 1842.5 | 368500 | 1755.38 | 351076 | 102 |  | - | 366124 |
|  |  |  | High | 1865 | 373000 | 1488.44 | 297688 | 504 |  | - | 370624 |
|  |  | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1370.94 | 274188 | 504 |  | - | - |
|  |  |  | High | 1770 | 354000 | 1752 | 350400 | 6 |  | - | - |
| 35 | 44 | Downlink | Low | 1822.5 | 364500 | 1806.66 | 361332 | 0 | 15 | - | 361692 |
|  |  |  | Mid | 1842.5 | 368500 | 1753.22 | 350644 | 102 |  | - | 365692 |
|  |  |  | High | 1862.5 | 372500 | 1483.78 | 296756 | 504 |  | - | 369692 |
|  |  | Uplink | Low | 1727.5 | 345500 | 1711.66 | 342332 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1368.78 | 273756 | 504 |  | - | - |
|  |  |  | High | 1767.5 | 353500 | 1747.34 | 349468 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 1825 | 365000 | 1806.64 | 361328 | 0 | 15 | - | 361688 |
|  |  |  | Mid | 1842.5 | 368500 | 1750.7 | 350140 | 102 |  | - | 365188 |
|  |  |  | High | 1860 | 372000 | 1478.76 | 295752 | 504 |  | - | 368688 |
|  |  | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1366.26 | 273252 | 504 |  | - | - |
|  |  |  | High | 1765 | 353000 | 1742.32 | 348464 | 6 |  | - | - |
| 45 | 58 | Downlink | Low | 1827.5 | 365500 | 1806.62 | 361324 | 0 | 15 | - | 361684 |
|  |  |  | Mid | 1842.5 | 368500 | 1748.18 | 349636 | 102 |  | - | 364684 |
|  |  |  | High | 1857.5 | 371500 | 1473.74 | 294748 | 504 |  | - | 367684 |
|  |  | Uplink | Low | 1732.5 | 346500 | 1711.62 | 342324 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1363.74 | 272748 | 504 |  | - | - |
|  |  |  | High | 1762.5 | 352500 | 1737.3 | 347460 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 1830 | 366000 | 1806.6 | 361320 | 0 | 15 | - | 361680 |
|  |  |  | Mid | 1842.5 | 368500 | 1745.66 | 349132 | 102 |  | - | 364180 |
|  |  |  | High | 1855 | 371000 | 1468.72 | 293744 | 504 |  | - | 366680 |
|  |  | Uplink | Low | 1735 | 347000 | 1711.6 | 342320 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1361.22 | 272244 | 504 |  | - | - |
|  |  |  | High | 1760 | 352000 | 1732.28 | 346456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.4	FFS

###### 4.3.1.1.1.5	Reference test frequencies for NR operating band n5

Table 4.3.1.1.1.5-1: Test frequencies for NR operating band n5, SCS 15 kHz and ΔFRaster 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 871.5 | 174300 | 869.25 | 173850 | 0 | 15 | 2178 | 174270 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 881.5 | 176300 | 860.89 | 172178 | 102 |  | 2203 | 176210 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 826.5 | 165300 | 824.25 | 164850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 743.53 | 148706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 846.5 | 169300 | 843.17 | 168634 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 874 | 174800 | 869.32 | 173864 | 0 | 15 | 2179 | 174290 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 881.5 | 176300 | 858.46 | 171692 | 102 |  | 2197 | 175730 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 829 | 165800 | 824.32 | 164864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 741.1 | 148220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 838.24 | 167648 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 876.5 | 175300 | 869.39 | 173878 | 0 | 15 | 2177 | 174250 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 856.03 | 171206 | 102 |  | 2191 | 175250 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 831.5 | 166300 | 824.39 | 164878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 738.67 | 147734 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 833.31 | 166662 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 879 | 175800 | 869.46 | 173892 | 0 | 15 | 2178 | 174270 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 853.6 | 170720 | 102 |  | 2185 | 174770 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 884 | 176800 | 783.74 | 156748 | 504 |  | 2192 | 175450 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 834 | 166800 | 824.46 | 164892 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 736.24 | 147248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 828.38 | 165676 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 881.5 | 176300 | 869.53 | 173906 | 0 | 15 | 2179 | 174290 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 836.5 | 167300 | 826.96 | 165392 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. Note 4: 20/25 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.5-2: Test frequencies for NR operating band n5, SCS 30 kHz and ΔFRaster 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 874 | 174800 | 869.68 | 173936 | 0 | 30 | 2185 | 174770 | 14 | 0 | 1 (1) | 2 |
|  |  |  | Mid | 881.5 | 176300 | 840.46 | 168092 | 102 |  | 2203 | 176210 | 18 | 0 | 0 (0) | 204 |
|  |  |  | High | 889 | 177800 | 703.24 | 140648 | 504 |  | 2224 | 177890 | 6 | 0 | 3 (3) | 1014 |
|  |  | Uplink | Low | 829 | 165800 | 824.68 | 164936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 650.74 | 130148 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 837.52 | 167504 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 876.5 | 175300 | 869.66 | 173932 | 0 | 30 | 2183 | 174730 | 2 | 0 | 1 (1) | 2 |
|  |  |  | Mid | 881.5 | 176300 | 837.94 | 167588 | 102 |  | 2197 | 175730 | 2 | 0 | 1 (1) | 206 |
|  |  |  | High | 886.5 | 177300 | 698.22 | 139644 | 504 |  | 2208 | 176670 | 6 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 831.5 | 166300 | 824.66 | 164932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 648.22 | 129644 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 832.5 | 166500 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 879 | 175800 | 869.82 | 173964 | 0 | 30 | 2184 | 174750 | 22 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 835.6 | 167120 | 102 |  | 2191 | 175250 | 22 | 0 | 0 (0) | 204 |
|  |  |  | High | 884 | 176800 | 693.38 | 138676 | 504 |  | 2195 | 175690 | 2 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 834 | 166800 | 824.82 | 164964 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 645.88 | 129176 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 827.66 | 165532 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 881.5 | 176300 | 869.8 | 173960 | 0 | 30 | 2185 | 174770 | 6 | 0 | 1 (1) | 2 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 836.54 | 167300 | 827.32 | 165464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. Note 4: 20/25 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.6	FFS

###### 4.3.1.1.1.7	Reference test frequencies for NR operating band n7

Table 4.3.1.1.1.7-1: Test frequencies for NR operating band n7 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2622.5 | 524500 | 2620.25 | 524050 | 0 | 15 | 6554 | 524410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2634.39 | 526878 | 102 |  | 6636 | 530910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 2687.5 | 537500 | 2594.53 | 518906 | 504 |  | 6718 | 537410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2502.5 | 500500 | 2500.25 | 500050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2442.03 | 488406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2567.5 | 513500 | 2564.17 | 512834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2625 | 525000 | 2620.32 | 524064 | 0 | 15 | 6555 | 524430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2631.96 | 526392 | 102 |  | 6630 | 530430 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 2685 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2505 | 501000 | 2500.32 | 500064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2439.6 | 487920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2565 | 513000 | 2559.24 | 511848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2627.5 | 525500 | 2620.39 | 524078 | 0 | 15 | 6556 | 524450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2629.53 | 525906 | 102 |  | 6624 | 529950 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 2682.5 | 536500 | 2584.67 | 516934 | 504 |  | 6692 | 535450 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2500.39 | 500078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2437.17 | 487434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2562.5 | 512500 | 2554.31 | 510862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2630 | 526000 | 2620.46 | 524092 | 0 | 15 | 6557 | 524650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2627.1 | 525420 | 102 |  | 6618 | 529470 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 2680 | 536000 | 2579.74 | 515948 | 504 |  | 6682 | 534530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2510 | 502000 | 2500.46 | 500092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2434.74 | 486948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2560 | 512000 | 2549.38 | 509876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 2632.5 | 526500 | 2620.53 | 524106 | 0 | 15 | 6558 | 524670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2624.67 | 524934 | 102 |  | 6612 | 528990 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 2677.5 | 535500 | 2574.81 | 514962 | 504 |  | 6669 | 533550 | 4 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2500.53 | 500106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2432.31 | 486462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2557.5 | 511500 | 2544.45 | 508890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 2635 | 527000 | 2620.6 | 524120 | 0 | 15 | 6559 | 524690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2622.24 | 524448 | 102 |  | 6606 | 528510 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 2675 | 535000 | 2569.88 | 513976 | 504 |  | 6656 | 532570 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2515 | 503000 | 2500.6 | 500120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2429.88 | 485976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2555 | 511000 | 2539.52 | 507904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 2640 | 528000 | 2620.56 | 524112 | 0 | 15 | 6558 | 524670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2617.2 | 523440 | 102 |  | 6594 | 527550 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 2670 | 534000 | 2559.84 | 511968 | 504 |  | 6633 | 530670 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 2520 | 504000 | 2500.56 | 500112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2424.84 | 484968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2550 | 510000 | 2529.48 | 505896 | 6 |  | - | - | - | - | - | - |
| 50 | 270 | Downlink | Low | 2645 | 529000 | 2620.7 | 524140 | 0 | 15 | 6557 | 524650 | 2 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 2655 | 531000 | 2612.34 | 522468 | 102 |  | 6582 | 526590 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 2665 | 533000 | 2549.98 | 509996 | 504 |  | 6607 | 528530 | 10 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2525 | 505000 | 2500.7 | 500140 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2419.98 | 483996 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2545 | 509000 | 2519.62 | 503924 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.7-2: Test frequencies for NR operating band n7 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2625 | 525000 | 2620.68 | 524136 | 0 | 15 | 6561 | 524910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2613.96 | 522792 | 102 |  | 6636 | 530910 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 2685 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 2505 | 501000 | 2500.68 | 500136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2349.24 | 469848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2565 | 513000 | 2558.52 | 511704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2627.5 | 525500 | 2620.66 | 524132 | 0 | 15 | 6562 | 524930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2655 | 531000 | 2611.44 | 522288 | 102 |  | 6630 | 530430 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2682.5 | 536500 | 2494.22 | 498844 | 504 |  | 6698 | 535930 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2500.66 | 500132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2346.72 | 469344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2562.5 | 512500 | 2553.5 | 510700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2630 | 526000 | 2620.82 | 524164 | 0 | 15 | 6560 | 524890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2609.1 | 521820 | 102 |  | 6624 | 529950 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 2680 | 536000 | 2489.38 | 497876 | 504 |  | 6688 | 535010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2510 | 502000 | 2500.82 | 500164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2344.38 | 468876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2560 | 512000 | 2548.66 | 509732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 2632.5 | 526500 | 2620.8 | 524160 | 0 | 15 | 6561 | 524910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2606.58 | 521316 | 102 |  | 6618 | 529470 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 2677.5 | 535500 | 2484.36 | 496872 | 504 |  | 6675 | 534030 | 2 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2500.8 | 500160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2341.86 | 468372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2557.5 | 511500 | 2543.64 | 508728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 2635 | 527000 | 2620.96 | 524192 | 0 | 15 | 6562 | 524930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2604.24 | 520848 | 102 |  | 6612 | 528990 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2675 | 535000 | 2479.52 | 495904 | 504 |  | 6662 | 533050 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2515 | 503000 | 2500.96 | 500192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2339.52 | 467904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2555 | 511000 | 2538.8 | 507760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 2640 | 528000 | 2620.92 | 524184 | 0 | 15 | 6561 | 524910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2599.2 | 519840 | 102 |  | 6600 | 528030 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  | 6636 | 530910 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 2520 | 504000 | 2500.92 | 500184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2334.48 | 466896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2550 | 510000 | 2528.76 | 505752 | 6 |  | - | - | - | - | - | - |
| 50 | 133 | Downlink | Low | 2645 | 529000 | 2621.06 | 524212 | 0 | 15 | 6563 | 525130 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2655 | 531000 | 2594.34 | 518868 | 102 |  | 6588 | 527070 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2665 | 533000 | 2459.62 | 491924 | 504 |  | 6613 | 529010 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2525 | 505000 | 2501.06 | 500212 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2329.62 | 465924 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2545 | 509000 | 2518.9 | 503780 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.7-3: Test frequencies for NR operating band n7 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2625 | 525000 | 2621.04 | 524208 | 0 | 15 | - | 524568 |
|  |  |  | Mid | 2655 | 531000 | 2577.6 | 515520 | 102 |  | - | 530568 |
|  |  |  | High | 2685 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
|  |  | Uplink | Low | 2505 | 501000 | 2501.04 | 500208 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2168.16 | 433632 | 504 |  | - | - |
|  |  |  | High | 2565 | 513000 | 2556.72 | 511344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2627.5 | 525500 | 2621.02 | 524204 | 0 | 15 | - | 524564 |
|  |  |  | Mid | 2655 | 531000 | 2575.08 | 515016 | 102 |  | - | 530064 |
|  |  |  | High | 2682.5 | 536500 | 2313.14 | 462628 | 504 |  | - | 535564 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2501.02 | 500204 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2165.64 | 433128 | 504 |  | - | - |
|  |  |  | High | 2562.5 | 512500 | 2551.7 | 510340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2630 | 526000 | 2621.36 | 524272 | 0 | 15 | - | 524632 |
|  |  |  | Mid | 2655 | 531000 | 2572.92 | 514584 | 102 |  | - | 529632 |
|  |  |  | High | 2680 | 536000 | 2308.48 | 461696 | 504 |  | - | 534632 |
|  |  | Uplink | Low | 2510 | 502000 | 2501.36 | 500272 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2163.48 | 432696 | 504 |  | - | - |
|  |  |  | High | 2560 | 512000 | 2547.04 | 509408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 2632.5 | 526500 | 2621.34 | 524268 | 0 | 15 | - | 524628 |
|  |  |  | Mid | 2655 | 531000 | 2570.4 | 514080 | 102 |  | - | 529128 |
|  |  |  | High | 2677.5 | 535500 | 2303.46 | 460692 | 504 |  | - | 533628 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2501.34 | 500268 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2160.96 | 432192 | 504 |  | - | - |
|  |  |  | High | 2557.5 | 511500 | 2542.02 | 508404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 2635 | 527000 | 2621.32 | 524264 | 0 | 15 | - | 524624 |
|  |  |  | Mid | 2655 | 531000 | 2567.88 | 513576 | 102 |  | - | 528624 |
|  |  |  | High | 2675 | 535000 | 2298.44 | 459688 | 504 |  | - | 532624 |
|  |  | Uplink | Low | 2515 | 503000 | 2501.32 | 500264 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2158.44 | 431688 | 504 |  | - | - |
|  |  |  | High | 2555 | 511000 | 2537 | 507400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 2640 | 528000 | 2621.64 | 524328 | 0 | 15 | - | 524688 |
|  |  |  | Mid | 2655 | 531000 | 2563.2 | 512640 | 102 |  | - | 527688 |
|  |  |  | High | 2670 | 534000 | 2288.76 | 457752 | 504 |  | - | 530688 |
|  |  | Uplink | Low | 2520 | 504000 | 2501.64 | 500328 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2153.76 | 430752 | 504 |  | - | - |
|  |  |  | High | 2550 | 510000 | 2527.32 | 505464 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 2645 | 529000 | 2621.6 | 524320 | 0 | 15 | - | 524680 |
|  |  |  | Mid | 2655 | 531000 | 2558.16 | 511632 | 102 |  | - | 526680 |
|  |  |  | High | 2665 | 533000 | 2278.72 | 455744 | 504 |  | - | 528680 |
|  |  | Uplink | Low | 2525 | 505000 | 2501.6 | 500320 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2148.72 | 429744 | 504 |  | - | - |
|  |  |  | High | 2545 | 509000 | 2517.28 | 503456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.8	Reference test frequencies for NR operating band n8

Table 4.3.1.1.1.8-1: Test frequencies for NR operating band n8 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 927.5 | 185500 | 925.25 | 185050 | 0 | 15 | 2318 | 185530 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 921.89 | 184378 | 102 |  | 2354 | 188410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 957.5 | 191500 | 864.53 | 172906 | 504 |  | 2393 | 191530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 930 | 186000 | 925.32 | 185064 | 0 | 15 | 2319 | 185550 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 919.46 | 183892 | 102 |  | 2348 | 187930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 955 | 191000 | 859.6 | 171920 | 504 |  | 2383 | 190610 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2344 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2344 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 52 | Uplink | Low | 892.5 | 178500 | 887.82 | 177564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 896.74 | 179348 | 6 |  | - | - | - | - | - | - |
| 10/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 897.5 | 179500 | 892.82 | 178564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 79 | Downlink | Low | 932.5 | 186500 | 925.39 | 185078 | 0 | 15 | 2320 | 185570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 917.03 | 183406 | 102 |  | 2342 | 187450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 952.5 | 190500 | 854.67 | 170934 | 504 |  | 2370 | 189630 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 15/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 79 | Uplink | Low | 892.5 | 178500 | 885.39 | 177078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 894.31 | 178862 | 6 |  | - | - | - | - | - | - |
| 15/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 | Uplink | Low | 897.58 | 179500 | 890.398 | 178078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 106 | Downlink | Low | 935 | 187000 | 925.46 | 185092 | 0 | 15 | 2318 | 185530 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 914.6 | 182920 | 102 |  | 2336 | 186970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 950 | 190000 | 849.74 | 169948 | 504 |  | 2357 | 188650 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 106 | Uplink | Low | 892.5 | 178500 | 882.96 | 176592 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 891.88 | 178376 | 6 |  | - | - | - | - | - | - |
| 20/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 897.5 | 179500 | 887.96 | 177592 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.8-2: Test frequencies for NR operating band n8 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 930 | 186000 | 925.68 | 185136 | 0 | 15 | 2325 | 186030 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 901.46 | 180292 | 102 |  | 2354 | 188410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 955 | 191000 | 769.24 | 153848 | 504 |  | 2389 | 191090 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 24 | Uplink | Low | 892.5 | 178500 | 888.18 | 177636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 896.02 | 179204 | 6 |  | - | - | - | - | - | - |
| 10/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 | Uplink | Low | 897.56 | 179500 | 893.18 | 178636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 38 | Downlink | Low | 932.5 | 186500 | 925.66 | 185132 | 0 | 15 | 2326 | 186050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 898.94 | 179788 | 102 |  | 2348 | 187930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 952.5 | 190500 | 764.22 | 152844 | 504 |  | 2373 | 189870 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 15/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 38 | Uplink | Low | 892.5 | 178500 | 885.66 | 177132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 893.5 | 178700 | 6 |  | - | - | - | - | - | - |
| 15/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38 | Uplink | Low | 897.5 | 179500 | 890.66 | 178132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 51 | Downlink | Low | 935 | 187000 | 925.82 | 185164 | 0 | 15 | 2324 | 186010 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 896.6 | 179320 | 102 |  | 2342 | 187450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 950 | 190000 | 759.38 | 151876 | 504 |  | 2360 | 188890 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 51 | Uplink | Low | 892.5 | 178500 | 883.32 | 176664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 891.16 | 178232 | 6 |  | - | - | - | - | - | - |
| 20/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 897.5 | 179500 | 888.32 | 177664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.9 to 4.3.1.1.1.11	FFS

###### 4.3.1.1.1.12	Reference test frequencies for NR operating band n12

Table 4.3.1.1.1.12-1: Test frequencies for NR operating band n12 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 731.5 | 146300 | 729.25 | 145850 | 0 | 15 | 1828 | 146210 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 737.5 | 147500 | 716.89 | 143378 | 102 |  | 1843 | 147410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 743.5 | 148700 | 650.53 | 130106 | 504 |  | 1858 | 148610 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 701.5 | 140300 | 699.25 | 139850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 614.53 | 122906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 713.5 | 142700 | 710.17 | 142034 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 734 | 146800 | 729.32 | 145864 | 0 | 15 | 1829 | 146410 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 737.5 | 147500 | 714.46 | 142892 | 102 |  | 1837 | 146930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 741 | 148200 | 645.6 | 129120 | 504 |  | 1845 | 147630 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 704 | 140800 | 699.32 | 139864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 612.1 | 122420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 705.24 | 141048 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 736.5 | 147300 | 729.39 | 145878 | 0 | 15 | 1830 | 146430 | 4 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 737.5 | 147500 | 712.03 | 142406 | 102 |  | 1831 | 146450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 738.5 | 147700 | 640.67 | 128134 | 504 |  | 1832 | 146650 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 706.5 | 141300 | 699.39 | 139878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 609.67 | 121934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 700.31 | 140062 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.12-2: Test frequencies for NR operating band n12 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 734 | 146800 | 729.68 | 145936 | 0 | 15 | 1835 | 146890 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 737.5 | 147500 | 696.46 | 139292 | 102 |  | 1843 | 147410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 741 | 148200 | 555.24 | 111048 | 504 |  | 1851 | 148110 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 704 | 140800 | 699.68 | 139936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 521.74 | 104348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 704.52 | 140904 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 736.5 | 147300 | 729.66 | 145932 | 0 | 15 | 1833 | 146670 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 737.5 | 147500 | 693.94 | 138788 | 102 |  | 1837 | 146930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 738.5 | 147700 | 550.22 | 110044 | 504 |  | 1838 | 147130 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 706.5 | 141300 | 699.66 | 139932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 519.22 | 103844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 699.5 | 139900 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.13	Reference test frequencies for NR operating band n13

Table 4.3.1.1.1.13-1: Test frequencies for NR operating band n13 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 748.5 | 149700 | 748.75 | 149750 | 0 | 15 | 1872 | 149790 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 751 | 150200 | 732.64 | 146528 | 102 |  | 1878 | 150270 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 753.5 | 150700 | 663.03 | 132606 | 504 |  | 1883 | 150730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 779.5 | 155900 | 777.25 | 155450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 782 | 156400 | 689.03 | 156234 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 784.5 | 156900 | 781.17 | 156234 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 751 | 150200 | 751.32 | 150264 | 0 | 15 | 1873 | 149810 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 782 | 156400 | 782.32 | 156464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcchConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.13-2: Test frequencies for NR operating band n13 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 751 | 150200 | 747.36 | 149472 | 0 | 15 | 1878 | 150270 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 782 | 156400 | 778.36 | 155672 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcchConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.14	Reference test frequencies for NR operating band n14

Table 4.3.1.1.1.14-1: Test frequencies for NR operating band n14 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 763 | 152600 | 742.39 | 148478 | 102 |  | 1909 | 152690 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 765.5 | 153100 | 672.53 | 134506 | 504 |  | 1913 | 153130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 790.5 | 158100 | 788.25 | 157650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 793 | 158600 | 700.03 | 140006 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 795.5 | 159100 | 792.17 | 158434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 793 | 158600 | 788.32 | 157664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcchConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.14-2: Test frequencies for NR operating band n14 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 763 | 152600 | 758.68 | 151736 | 0 | 15 | 1909 | 152690 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 793 | 158600 | 788.68 | 157736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcchConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.15 to 4.3.1.1.1.17	FFS

###### 4.3.1.1.1.18	Reference test frequencies for NR operating band n18

Table 4.3.1.1.1.18-1: Test frequencies for NR operating band n18 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 862.5 | 172500 | 860.25 | 172050 | 0 | 15 | 2157 | 172590 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 867.5 | 173500 | 846.89 | 169378 | 102 |  | 2168 | 173530 | 4 | 1 | 1 (2) | 105 |
|  |  |  | High | 872.5 | 174500 | 779.53 | 155906 | 504 |  | 2182 | 174530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 817.5 | 163500 | 815.25 | 163050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 729.53 | 145906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 827.5 | 165500 | 824.17 | 164834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 865 | 173000 | 860.32 | 172064 | 0 | 15 | 2158 | 172610 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 867.5 | 173500 | 844.46 | 168892 | 102 |  | 2162 | 173050 | 6 | 1 | 1 (2) | 105 |
|  |  |  | High | 870 | 174000 | 774.6 | 154920 | 504 |  | 2169 | 173550 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 820 | 164000 | 815.32 | 163064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 727.1 | 145420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 825 | 165000 | 819.24 | 163848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 867.5 | 173500 | 860.39 | 172078 | 0 | 15 | 2156 | 172570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 822.5 | 164500 | 815.39 | 163078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  | High |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.18-2: Test frequencies for NR operating band n18 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 865 | 173000 | 860.68 | 172136 | 0 | 15 | 2164 | 173090 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 867.5 | 173500 | 826.46 | 165292 | 102 |  | 2168 | 173530 | 10 | 0 | 2 (7) | 218 |
|  |  |  | High | 870 | 174000 | 684.24 | 136848 | 504 |  | 2175 | 174030 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 820 | 164000 | 815.68 | 163136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 636.74 | 127348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 825 | 165000 | 818.52 | 163704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 867.5 | 173500 | 860.66 | 172132 | 0 | 15 | 2162 | 173050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 822.5 | 164500 | 815.66 | 163132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  | High |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.19	FFS

###### 4.3.1.1.1.20	Reference test frequencies for NR operating band n20

Table 4.3.1.1.1.20-1: Test frequencies for NR operating band n20 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 793.5 | 158700 | 791.25 | 158250 | 0 | 15 | 1983 | 158670 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 806 | 161200 | 785.39 | 157078 | 102 |  | 2015 | 161290 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 818.5 | 163700 | 725.53 | 145106 | 504 |  | 2047 | 163730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 796 | 159200 | 791.32 | 158264 | 0 | 15 | 1984 | 158690 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 806 | 161200 | 782.96 | 156592 | 102 |  | 2009 | 160810 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 816 | 163200 | 720.6 | 144120 | 504 |  | 2034 | 162750 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 798.5 | 159700 | 791.39 | 158278 | 0 | 15 | 1982 | 158650 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 806 | 161200 | 780.53 | 156106 | 102 |  | 2003 | 160330 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 813.5 | 162700 | 715.67 | 143134 | 504 |  | 2021 | 161770 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.39 | 166478 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 749.17 | 149834 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 854.5 | 170900 | 846.31 | 169262 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 801 | 160200 | 791.46 | 158292 | 0 | 15 | 1983 | 158670 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 806 | 161200 | 778.1 | 155620 | 102 |  | 1997 | 159850 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 811 | 162200 | 710.74 | 142148 | 504 |  | 2011 | 160850 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 842 | 168400 | 832.46 | 166492 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 746.74 | 149348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 852 | 170400 | 841.38 | 168276 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.20-2: Test frequencies for NR operating band n20 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 796 | 159200 | 791.68 | 158336 | 0 | 15 | 1990 | 159170 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 806 | 161200 | 764.96 | 152992 | 102 |  | 2015 | 161290 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 816 | 163200 | 630.24 | 126048 | 504 |  | 2040 | 163230 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 798.5 | 159700 | 791.66 | 158332 | 0 | 15 | 1988 | 159130 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 806 | 161200 | 762.44 | 152488 | 102 |  | 2006 | 160570 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 813.5 | 162700 | 625.22 | 125044 | 504 |  | 2027 | 162250 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.66 | 166532 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 658.72 | 131744 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 854.5 | 170900 | 845.5 | 169100 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 801 | 160200 | 791.82 | 158364 | 0 | 15 | 1989 | 159150 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 806 | 161200 | 760.1 | 152020 | 102 |  | 2000 | 160090 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 811 | 162200 | 620.38 | 124076 | 504 |  | 2014 | 161090 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 842 | 168400 | 832.82 | 166564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 656.38 | 131276 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 852 | 170400 | 840.66 | 168132 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.21 to 4.3.1.1.1.23	FFS

###### 4.3.1.1.1.24	Reference test frequencies for NR operating band n24

Table 4.3.1.1.1.24-1a: Test frequencies for NR operating band n24,SCS 15 kHz and ΔFRaster 15 kHz

| UL/DL CBW [MHz] |  | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 3 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 |  | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1531 | 306200 | 1510.39 | 302078 | 102 |  | 3829 | 306290 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  |  |  | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1632.5 | 326500 | 1539.53 | 307906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |  | - | - | - | - | - | - |
| 10/5) |  | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1531 | 306200 | 1510.39 | 302078 | 102 |  | 3829 | 306290 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  |  | 52 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low Mid | 1531.0 | 306200 | 1526.32 | 305264 | 0 | 15 | 3823 | 305810 | 2 | 1 | 2 (4) | 5 |  |
|  |  |  | High | 1531.0 | 306200 | 1435.6 | 287120 | 504 |  | 3823 | 305810 | 2 | 1 | 2 (4) | 509 |  |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 | - | - | - | - | - | - | - |  |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |  | - | - | - | - | - | - |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. Note 4: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-2a: Test frequencies for NR operating band n24, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 3 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low Mid | 1531.0 | 306200 | 1526.68 | 305336 | 0 | 30 | 3829 | 306290 | 6 | 0 | 3 (3) | 6 |
|  |  |  | High | 1531.0 | 306200 | 1345.24 | 269048 | 504 |  | 3829 | 306290 | 6 | 0 | 3 (3) | 1014 |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1628.18 | 325636 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1645.02 | 329004 | 6 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-3a: Test frequencies for NR operating band n24, SCS 60 kHz and ΔFRaster 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low Mid | 1531.0 | 306200 | 1527.04 | 305408 | 0 | 15 | - | 305768 |
|  |  |  | High | 1531.0 | 306200 | 1164.16 | 232832 | 504 |  | - | 305768 |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1628.54 | 325708 | 0 | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1643.22 | 328644 | 6 |  | - | - |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: Void. |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.25	Reference test frequencies for NR operating band n25

Table 4.3.1.1.1.25-1: Test frequencies for NR operating band n25 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1932.5 | 386500 | 1930.25 | 386050 | 0 | 15 | 4829 | 386410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1941.89 | 388378 | 102 |  | 4904 | 392410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1992.5 | 398500 | 1899.53 | 379906 | 504 |  | 4979 | 398410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1852.5 | 370500 | 1850.25 | 370050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1789.53 | 357906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1912.5 | 382500 | 1909.17 | 381834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 1935 | 387000 | 1930.32 | 386064 | 0 | 15 | 4830 | 386430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1939.46 | 387892 | 102 |  | 4898 | 391930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 1990 | 398000 | 1894.6 | 378920 | 504 |  | 4969 | 397490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.32 | 370064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1787.1 | 357420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1910 | 382000 | 1904.24 | 380848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 1937.5 | 387500 | 1930.39 | 386078 | 0 | 15 | 4831 | 386450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1937.03 | 387406 | 102 |  | 4892 | 391450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 1987.5 | 397500 | 1889.67 | 377934 | 504 |  | 4956 | 396510 | 0 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.39 | 370078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1784.67 | 356934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1899.31 | 379862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 1940 | 388000 | 1930.46 | 386092 | 0 | 15 | 4832 | 386650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1934.6 | 386920 | 102 |  | 4886 | 390970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 1985 | 397000 | 1884.74 | 376948 | 504 |  | 4943 | 395530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.46 | 370092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1782.24 | 356448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1894.38 | 378876 | 6 |  | - | - | - | - | - | - |
| 25/25 | 133 | Downlink | Low | 1942.5 | 388500 | 1930.53 | 386106 | 0 | 15 | 4833 | 386670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1932.17 | 386434 | 102 |  | 4880 | 390490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 1982.5 | 396500 | 1879.81 | 375962 | 504 |  | 4933 | 394610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.53 | 370106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1779.81 | 355962 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1889.45 | 377890 | 6 |  | - | - | - | - | - | - |
| 30/30 | 160 | Downlink | Low | 1945 | 389000 | 1930.6 | 386120 | 0 | 15 | 4834 | 386690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1929.74 | 385948 | 102 |  | 4874 | 390010 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 1980 | 396000 | 1874.88 | 374976 | 504 |  | 4920 | 393630 | 2 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.6 | 370120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1777.38 | 355476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1884.52 | 376904 | 6 |  | - | - | - | - | - | - |
| 35/35 | 188 | Downlink | Low | 1947.5 | 389500 | 1930.58 | 386116 | 0 | 15 | 4832 | 386650 | 10 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 1962.5 | 392500 | 1927.22 | 385444 | 102 |  | 4868 | 389530 | 6 | 1 | 0 (0) | 103 |
|  |  |  | High | 1977.5 | 395500 | 1869.86 | 373972 | 504 |  | 4907 | 392650 | 10 | 0 | 0 (0) | 508 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1850.58 | 370116 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1774.86 | 354972 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1879.5 | 375900 | 6 |  | - | - | - | - | - | - |
| 40/40 | 216 | Downlink | Low | 1950 | 390000 | 1930.56 | 386112 | 0 | 15 | 4833 | 386670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1924.7 | 384940 | 102 |  | 4862 | 389050 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1975 | 395000 | 1864.84 | 372968 | 504 |  | 4894 | 391490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.56 | 370112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1772.34 | 354468 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1874.48 | 374896 | 6 |  | - | - | - | - | - | - |
| 40/45 | 242 | Downlink | Low | 1952.5 | 390500 | 1930.72 | 386144 | 0 | 15 | 4834 | 386690 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1922.36 | 384472 | 102 |  | 4856 | 388570 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1972.5 | 394500 | 1860 | 372000 | 504 |  | 4881 | 390510 | 2 | 0 | 0 (0) | 504 |
|  | 216 | Uplink | Low | 1872.5 | 374500 | 1853.06 | 370612 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1772.34 | 354468 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892,5 | 378500 | 1871.98 | 374396 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.25-2: Test frequencies for NR operating band n25 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 1935 | 387000 | 1930.68 | 386136 | 0 | 15 | 4836 | 386910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1921.46 | 384292 | 102 |  | 4904 | 392410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 1990 | 398000 | 1804.24 | 360848 | 504 |  | 4975 | 397970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.68 | 370136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1696.74 | 339348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1910 | 382000 | 1903.52 | 380704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 1937.5 | 387500 | 1930.66 | 386132 | 0 | 15 | 4837 | 386930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1962.5 | 392500 | 1918.94 | 383788 | 102 |  | 4898 | 391930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1987.5 | 397500 | 1799.22 | 359844 | 504 |  | 4962 | 396990 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.66 | 370132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1694.22 | 338844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1898.5 | 379700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 1940 | 388000 | 1930.82 | 386164 | 0 | 15 | 4835 | 386890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1916.6 | 383320 | 102 |  | 4892 | 391450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 1985 | 397000 | 1794.38 | 358876 | 504 |  | 4949 | 396010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.82 | 370164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1691.88 | 338376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1893.66 | 378732 | 6 |  | - | - | - | - | - | - |
| 25/25 | 65 | Downlink | Low | 1942.5 | 388500 | 1930.8 | 386160 | 0 | 15 | 4836 | 386910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1914.08 | 382816 | 102 |  | 4886 | 390970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 1982.5 | 396500 | 1789.36 | 357872 | 504 |  | 4939 | 395090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.8 | 370160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1689.36 | 337872 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1888.64 | 377728 | 6 |  | - | - | - | - | - | - |
| 30/30 | 78 | Downlink | Low | 1945 | 389000 | 1930.96 | 386192 | 0 | 15 | 4837 | 386930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1911.74 | 382348 | 102 |  | 4880 | 390490 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1980 | 396000 | 1784.52 | 356904 | 504 |  | 4926 | 394110 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.96 | 370192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1687.02 | 337404 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1883.8 | 376760 | 6 |  | - | - | - | - | - | - |
| 35/35 | 92 | Downlink | Low | 1947.5 | 389500 | 1930.94 | 386188 | 0 | 15 | 4838 | 387130 | 2 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 1962.5 | 392500 | 1909.22 | 381844 | 102 |  | 4874 | 390010 | 10 | 0 | 1 (6) | 216 |
|  |  |  | High | 1977.5 | 395500 | 1779.5 | 355900 | 504 |  | 4913 | 393130 | 2 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1850.94 | 370188 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1684.5 | 336900 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1878.78 | 375756 | 6 |  | - | - | - | - | - | - |
| 40/40 | 106 | Downlink | Low | 1950 | 390000 | 1930.92 | 386184 | 0 | 15 | 4836 | 386910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1906.7 | 381340 | 102 |  | 4868 | 389530 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1975 | 395000 | 1774.48 | 354896 | 504 |  | 4900 | 391970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.92 | 370184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1681.98 | 336396 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1873.76 | 374752 | 6 |  | - | - | - | - | - | - |
| 40/45 | 119 | Downlink | Low | 1952.5 | 390500 | 1931.08 | 386216 | 0 | 15 | 4840 | 387170 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 1962.5 | 392500 | 1904.36 | 380872 | 102 |  | 4862 | 389050 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 1972.5 | 394500 | 1769.64 | 353928 | 504 | - | 4887 | 390990 | 18 | 0 | 0 (5) | 1018 |
|  | 106 | Uplink | Low | 1872.5 | 374500 | 1853.42 | 370684 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1681.98 | 336396 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892.5 | 378500 | 1871.26 | 374252 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.25-3: Test frequencies for NR operating band n25 and SCS 60 kHz without CORESET#0

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 11 | Downlink | Low | 1935 | 387000 | 1931.04 | 386208 | 0 | 15 | - | 386568 |
|  |  |  | Mid | 1962.5 | 392500 | 1885.1 | 377020 | 102 |  | - | 392068 |
|  |  |  | High | 1990 | 398000 | 1623.16 | 324632 | 504 |  | - | 397568 |
|  |  | Uplink | Low | 1855 | 371000 | 1851.04 | 370208 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1515.66 | 303132 | 504 |  | - | - |
|  |  |  | High | 1910 | 382000 | 1901.72 | 380344 | 6 |  | - | - |
| 15/15 | 18 | Downlink | Low | 1937.5 | 387500 | 1931.02 | 386204 | 0 | 15 | - | 386564 |
|  |  |  | Mid | 1962.5 | 392500 | 1882.58 | 376516 | 102 |  | - | 391564 |
|  |  |  | High | 1987.5 | 397500 | 1618.14 | 323628 | 504 |  | - | 396564 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1851.02 | 370204 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1513.14 | 302628 | 504 |  | - | - |
|  |  |  | High | 1907.5 | 381500 | 1896.7 | 379340 | 6 |  | - | - |
| 20/20 | 24 | Downlink | Low | 1940 | 388000 | 1931.36 | 386272 | 0 | 15 | - | 386632 |
|  |  |  | Mid | 1962.5 | 392500 | 1880.42 | 376084 | 102 |  | - | 391132 |
|  |  |  | High | 1985 | 397000 | 1613.48 | 322696 | 504 |  | - | 395632 |
|  |  | Uplink | Low | 1860 | 372000 | 1851.36 | 370272 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1510.98 | 302196 | 504 |  | - | - |
|  |  |  | High | 1905 | 381000 | 1892.04 | 378408 | 6 |  | - | - |
| 25/25 | 31 | Downlink | Low | 1942.5 | 388500 | 1931.34 | 386268 | 0 | 15 | - | 386628 |
|  |  |  | Mid | 1962.5 | 392500 | 1877.9 | 375580 | 102 |  | - | 390628 |
|  |  |  | High | 1982.5 | 396500 | 1608.46 | 321692 | 504 |  | - | 394628 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1851.34 | 370268 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1508.46 | 301692 | 504 |  | - | - |
|  |  |  | High | 1902.5 | 380500 | 1887.02 | 377404 | 6 |  | - | - |
| 30/30 | 38 | Downlink | Low | 1945 | 389000 | 1931.32 | 386264 | 0 | 15 | - | 386624 |
|  |  |  | Mid | 1962.5 | 392500 | 1875.38 | 375076 | 102 |  | - | 390124 |
|  |  |  | High | 1980 | 396000 | 1603.44 | 320688 | 504 |  | - | 393624 |
|  |  | Uplink | Low | 1865 | 373000 | 1851.32 | 370264 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1505.94 | 301188 | 504 |  | - | - |
|  |  |  | High | 1900 | 380000 | 1882 | 376400 | 6 |  | - | - |
| 35/35 | 44 | Downlink | Low | 1947.5 | 389500 | 1931.66 | 386332 | 0 | 15 | - | 386692 |
|  |  |  | Mid | 1962.5 | 392500 | 1873.22 | 374644 | 102 |  | - | 389692 |
|  |  |  | High | 1977.5 | 395500 | 1598.78 | 319756 | 504 |  | - | 392692 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1851.66 | 370332 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1503.78 | 300756 | 504 |  | - | - |
|  |  |  | High | 1897.5 | 379500 | 1877.34 | 375468 | 6 |  | - | - |
| 40/40 | 51 | Downlink | Low | 1950 | 390000 | 1931.64 | 386328 | 0 | 15 | - | 386688 |
|  |  |  | Mid | 1962.5 | 392500 | 1870.7 | 374140 | 102 |  | - | 389188 |
|  |  |  | High | 1975 | 395000 | 1593.76 | 318752 | 504 |  | - | 391688 |
|  |  | Uplink | Low | 1870 | 374000 | 1851.64 | 370328 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1501.26 | 300252 | 504 |  | - | - |
|  |  |  | High | 1895 | 379000 | 1872.32 | 374464 | 6 |  | - | - |
| 40/45 | 58 | Downlink | Low | 1952.5 | 390500 | 1931.62 | 386324 | 0 | 15 | - | 386684 |
|  |  |  | Mid | 1962.5 | 392500 | 1868.18 | 373636 | 102 |  | - | 388684 |
|  |  |  | High | 1972.5 | 394500 | 1588.74 | 317748 | 504 |  | - | 390684 |
|  | 51 | Uplink | Low | 1872.5 | 374500 | 1854.14 | 370828 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1501.26 | 300252 | 504 |  | - | - |
|  |  |  | High | 1892.5 | 378500 | 1869.82 | 373964 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.26	Reference test frequencies for NR operating band n26

Table 4.3.1.1.1.26-1: Test frequencies for NR operating band n26 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 860.5 | 172100 | 859.15 | 171830 | 0 | 15 | 30938 | 172070 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 876.5 | 175300 | 856.79 | 171358 | 102 |  | 31017 | 175270 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 892.5 | 178500 | 800.43 | 160086 | 504 |  | 31099 | 178530 | 4 | 0 | 3 (2) | 506 |
|  |  | Uplink | Low | 815.5 | 163100 | 814.15 | 162830 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 739.43 | 147886 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 847.5 | 169500 | 845.07 | 169014 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 861.5 | 172300 | 859.25 | 171850 | 0 | 15 | 2153 | 172330 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 855.89 | 171178 | 102 |  | 2189 | 175210 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 816.5 | 163300 | 814.25 | 162850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 738.53 | 147706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 846.5 | 169300 | 843.17 | 168634 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 864 | 172800 | 859.32 | 171864 | 0 | 15 | 2154 | 172350 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 853.46 | 170692 | 102 |  | 2183 | 174730 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 819 | 163800 | 814.32 | 162864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 736.1 | 147220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 838.24 | 167648 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 866.5 | 173300 | 859.39 | 171878 | 0 | 15 | 2155 | 172370 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 851.03 | 170206 | 102 |  | 2177 | 174250 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 821.5 | 164300 | 814.39 | 162878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 733.67 | 146734 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 833.31 | 166662 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 869 | 173800 | 859.46 | 171892 | 0 | 15 | 2153 | 172330 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 876.5 | 175300 | 848.6 | 169720 | 102 |  | 2171 | 173770 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 884 | 176800 | 783.74 | 156748 | 504 |  | 2192 | 175450 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 824 | 164800 | 814.46 | 162892 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 731.24 | 146248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 828.38 | 165676 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.26-2: Test frequencies for NR operating band n26 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 864 | 172800 | 859.68 | 171936 | 0 | 15 | 2160 | 172830 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 876.5 | 175300 | 835.46 | 167092 | 102 |  | 2189 | 175210 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 889 | 177800 | 703.24 | 140648 | 504 |  | 2224 | 177890 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 819 | 163800 | 814.68 | 162936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 645.74 | 129148 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 837.52 | 167504 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 866.5 | 173300 | 859.66 | 171932 | 0 | 15 | 2161 | 172850 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 876.5 | 175300 | 832.94 | 166588 | 102 |  | 2183 | 174730 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 886.5 | 177300 | 698.22 | 139644 | 504 |  | 2208 | 176670 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 821.5 | 164300 | 814.66 | 162932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 643.22 | 128644 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 832.5 | 166500 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 869 | 173800 | 859.82 | 171964 | 0 | 15 | 2159 | 172810 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 876.5 | 175300 | 830.6 | 166120 | 102 |  | 2177 | 174250 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 884 | 176800 | 693.38 | 138676 | 504 |  | 2195 | 175690 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 824 | 164800 | 814.82 | 162964 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 640.88 | 128176 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 827.66 | 165532 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.27	FFS

###### 4.3.1.1.1.28	Reference test frequencies for NR operating band n28

Table 4.3.1.1.1.28-1: Test frequencies for NR operating band n28 and SCS 15 kHz when not used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 759.5 | 151900 | 758.15 | 151630 | 0 | 15 | 30432 | 151870 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 780.5 | 156100 | 760.79 | 152158 | 102 |  | 30537 | 156070 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 801.5 | 160300 | 709.43 | 141886 | 504 |  | 30642 | 160270 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 704.5 | 140900 | 703.15 | 140630 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 633.43 | 126686 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 746.5 | 149300 | 744.07 | 148814 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 759.89 | 151978 | 102 |  | 1949 | 156010 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 800.5 | 160100 | 707.53 | 141506 | 504 |  | 2002 | 160130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 757.46 | 151492 | 102 |  | 1943 | 155530 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 798 | 159600 | 702.6 | 140520 | 504 |  | 1989 | 159150 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 765.5 | 153100 | 758.39 | 151678 | 0 | 15 | 1901 | 152170 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 780.5 | 156100 | 755.03 | 151006 | 102 |  | 1937 | 155050 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 795.5 | 159100 | 697.67 | 139534 | 504 |  | 1976 | 158170 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 768 | 153600 | 758.46 | 151692 | 0 | 15 | 1902 | 152190 | 10 | 1 | 1 (2) | 3 |
| (Note 2) |  |  | Mid | 783 | 156600 | 755.1 | 151020 | 102 |  | 1938 | 155070 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 793 | 158600 | 692.74 | 138548 | 504 |  | 1966 | 157250 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 627.74 | 125548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 727.38 | 145476 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low Mid | 773 | 154600 | 758.6 | 151720 | 0 | 15 | 1901 | 152170 | 6 | 0 | 1 (2) | 2 |
| (Note 4) |  |  | High | 788 | 157600 | 682.88 | 136576 | 504 |  | 1940 | 155290 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low Mid | 718 | 143600 | 703.6 | 140720 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.28-1A: Test frequencies for NR operating band n28 and SCS 15 kHz when used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] (Note 5) | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low Mid | 785.5 | 157100 | 783.25 | 156650 | 0 | 15 | 1963 | 157010 | 0 | 0 | 0 (0) | 0 |
|  |  |  | High | 790.5 | 158100 | 697.53 | 139506 | 504 |  | 1977 | 158190 | 0 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low Mid | 730.5 | 146100 | 728.25 | 145650 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 735.5 | 147100 | 732.17 | 146434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low Mid High | 788 | 157600 | 764.96 | 152992 | 102 | 15 | 1964 | 157210 | 2 | 1 | 2 (4) | 107 |
|  |  | Uplink | Low Mid High | 733 | 146600 | 637.6 | 127520 | 504 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. Note 5: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. (Reference: TS 38.101-3 [9], Table 5.5B.4.1-1 for DC_21A_n28 and Note 17) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.28-2: Test frequencies for NR operating band n28 and SCS 30 kHz when not used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 763 | 152600 | 758.68 | 151736 | 0 | 15 | 1909 | 152690 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 780.5 | 156100 | 739.46 | 147892 | 102 |  | 1949 | 156010 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 798 | 159600 | 612.24 | 122448 | 504 |  | 1995 | 159630 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 539.74 | 107948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 736.52 | 147304 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 765.5 | 153100 | 758.66 | 151732 | 0 | 15 | 1907 | 152650 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 780.5 | 156100 | 736.94 | 147388 | 102 |  | 1943 | 155530 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 795.5 | 159100 | 607.22 | 121444 | 504 |  | 1982 | 158650 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 537.22 | 107444 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 731.5 | 146300 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 768 | 153600 | 758.82 | 151764 | 0 | 15 | 1908 | 152670 | 14 | 0 | 2 (7) | 14 |
| (Note 2) |  |  | Mid | 783 | 156600 | 737.1 | 147420 | 102 |  | 1944 | 155550 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 793 | 158600 | 602.38 | 120476 | 504 |  | 1969 | 157490 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 537.38 | 107476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 726.66 | 145332 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low Mid | 773 | 154600 | 758.96 | 151792 | 0 | 15 | 1907 | 152650 | 22 | 0 | 1 (6) | 12 |
| (Note 4) |  |  | High | 788 | 157600 | 592.52 | 118504 | 504 |  | 1943 | 155530 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low Mid | 718 | 143600 | 703.96 | 140792 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 716.8 | 143360 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.28-2A: Test frequencies for NR operating band n28 and SCS 30 kHz when used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 5 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low Mid High | 788 | 157600 | 746.96 | 149392 | 102 | 15 | 1970 | 157690 | 6 | 0 | 3 (8) | 220 |
|  |  | Uplink | Low Mid High | 733 | 146600 | 547.24 | 109448 | 504 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. Note 5: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. (Reference: TS 38.101-3 [9], Table 5.5B.4.1-1 for DC_21A_n28 and Note 17) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.29	Reference test frequencies for NR operating band n29 (SDL)

Table 4.3.1.1.1.29-1: Test frequencies for NR operating band n29 and SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 719.5 | 143900 | 717.25 | 143450 | 0 | 15 | - | 143810 |
|  |  |  | Mid | 722.5 | 144500 | 701.89 | 140378 | 102 |  | - | 144410 |
|  |  |  | High | 725.5 | 145100 | 632.53 | 126506 | 504 |  | - | 145010 |
| 10 | 52 | Downlink | Low | 722 | 144400 | 717.32 | 143464 | 0 | 15 | - | 143824 |
|  |  |  | Mid | 722.5 | 144500 | 699.46 | 139892 | 102 |  | - | 143924 |
|  |  |  | High | 723 | 144600 | 627.6 | 125520 | 504 |  | - | 144024 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.29-2: Test frequencies for NR operating band n29 and SCS 30 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 722 | 144400 | 717.68 | 143536 | 0 | 15 | - | 143896 |
|  |  |  | Mid | 722.5 | 144500 | 681.46 | 136292 | 102 |  | - | 143996 |
|  |  |  | High | 723 | 144600 | 537.24 | 107448 | 504 |  | - | 144096 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.30	Reference test frequencies for NR operating band n30

Table 4.3.1.1.1.30-1: Test frequencies for NR operating band n30 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2352.5 | 470500 | 2350.25 | 470050 | 0 | 15 | 5879 | 470410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2355 | 471000 | 2334.39 | 466878 | 102 |  | 5886 | 470910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 2357.5 | 471500 | 2264.53 | 452906 | 504 |  | 5893 | 471410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2307.5 | 461500 | 2305.25 | 461050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2310 | 462000 | 2217.03 | 443406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2312.5 | 462500 | 2309.17 | 461834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2355 | 471000 | 2350.32 | 470064 | 0 | 15 | 5880 | 470430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 2310 | 462000 | 2305.32 | 461064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.30-2: Test frequencies for NR operating band n30 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2355 | 471000 | 2350.68 | 470136 | 0 | 15 | 5886 | 470910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 2310 | 462000 | 2305.68 | 461136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.31 to 4.3.1.1.1.33	FFS

###### 4.3.1.1.1.34	Reference test frequencies for NR operating band n34

Table 4.3.1.1.1.34-1: Test frequencies for NR operating band n34 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2012.5 | 402500 | 2010.25 | 402050 | 0 | 15 | 5032 | 402530 | 4 | 1 | 1 (2) | 3 |
|  |  | & | Mid | 2017.5 | 403500 | 1996.89 | 399378 | 102 |  | 5043 | 403470 | 8 | 1 | 0 (0) | 103 |
|  |  | Uplink | High | 2022.5 | 404500 | 1929.53 | 385906 | 504 |  | 5054 | 404410 | 0 | 0 | 0 (0) | 504 |
| 10 | 52 | Downlink | Low | 2015 | 403000 | 2010.32 | 402064 | 0 | 30 | 5036 | 402970 | 2 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2017.5 | 403500 | 1994.46 | 398892 | 102 |  | 5043 | 403470 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2020 | 404000 | 1924.6 | 384920 | 504 |  | 5050 | 403970 | 2 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 2017.5 | 403500 | 2010.39 | 402078 | 0 | 30 | 5037 | 402990 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.34-2: Test frequencies for NR operating band n34 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2015 | 403000 | 2010.68 | 402136 | 0 | 30 | 5036 | 402970 | 14 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2017.5 | 403500 | 1976.46 | 395292 | 102 |  | 5043 | 403470 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2020 | 404000 | 1834.24 | 366848 | 504 |  | 5050 | 403970 | 14 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 2017.5 | 403500 | 2010.66 | 402132 | 0 | 30 | 5037 | 402990 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.34-3: Test frequencies for NR operating band n34 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2015 | 403000 | 2011.04 | 402208 | 0 | 15 | - | 402568 |
|  |  | & | Mid | 2017.5 | 403500 | 1940.1 | 388020 | 102 |  | - | 403068 |
|  |  | Uplink | High | 2020 | 404000 | 1653.16 | 330632 | 504 |  | - | 403568 |
| 15 | 18 | Downlink | Low | 2017.5 | 403500 | 2011.02 | 402204 | 0 | 15 | - | 402564 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.35 to 4.3.1.1.1.37	FFS

###### 4.3.1.1.1.38	Reference test frequencies for NR operating band n38

Table 4.3.1.1.1.38-1: Test frequencies for NR operating band n38 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2572.5 | 514500 | 2570.25 | 514050 | 0 | 15 | 6432 | 514590 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2592.5 | 518500 | 2571.89 | 514378 | 102 |  | 6479 | 518410 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2617.5 | 523500 | 2524.53 | 504906 | 504 |  | 6543 | 523470 | 8 | 1 | 0 (0) | 505 |
| 10 | 52 | Downlink | Low | 2575 | 515000 | 2570.32 | 514064 | 0 | 30 | 6439 | 515090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 2595 | 519000 | 2571.96 | 514392 | 102 |  | 6486 | 518910 | 6 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2615 | 523000 | 2519.6 | 503920 | 504 |  | 6536 | 522970 | 2 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 2577.5 | 515500 | 2570.39 | 514078 | 0 | 30 | 6437 | 515050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2569.53 | 513906 | 102 |  | 6480 | 518430 | 8 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2612.5 | 522500 | 2514.67 | 502934 | 504 |  | 6526 | 522050 | 0 | 1 | 1 (6) | 511 |
| 20 | 106 | Downlink | Low | 2580 | 516000 | 2570.46 | 514092 | 0 | 30 | 6438 | 515070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2567.1 | 513420 | 102 |  | 6474 | 517950 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2610 | 522000 | 2509.74 | 501948 | 504 |  | 6513 | 521070 | 2 | 1 | 1 (6) | 511 |
| 25 | 133 | Downlink | Low | 2582.5 | 516500 | 2570.53 | 514106 | 0 | 30 | 6439 | 515090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2564.67 | 512934 | 102 |  | 6468 | 517470 | 0 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 2607.5 | 521500 | 2504.81 | 500962 | 504 |  | 6500 | 520090 | 4 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 2585 | 517000 | 2570.6 | 514120 | 0 | 30 | 6437 | 515050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2595 | 519000 | 2562.24 | 512448 | 102 |  | 6462 | 516990 | 2 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 2605 | 521000 | 2499.88 | 499976 | 504 |  | 6487 | 518930 | 6 | 0 | 0 (2) | 506 |
| 40 | 216 | Downlink | Low | 2590 | 518000 | 2570.56 | 514112 | 0 | 30 | 6439 | 515090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2557.2 | 511440 | 102 |  | 6450 | 516030 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2600 | 520000 | 2489.84 | 497968 | 504 |  | 6461 | 516970 | 10 | 1 | 0 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 for SSB_SCS = 15kHz and Table 13-3 for SSB_SCS=30kHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.38-2: Test frequencies for NR operating band n38 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2575 | 515000 | 2570.68 | 514136 | 0 | 30 | 6439 | 515090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2595 | 519000 | 2553.96 | 510792 | 102 |  | 6486 | 518910 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2615 | 523000 | 2429.24 | 485848 | 504 |  | 6536 | 522970 | 14 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 2577.5 | 515500 | 2570.66 | 514132 | 0 | 30 | 6437 | 515050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2551.44 | 510288 | 102 |  | 6480 | 518430 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2612.5 | 522500 | 2424.22 | 484844 | 504 |  | 6526 | 522050 | 18 | 0 | 2 (2) | 1012 |
| 20 | 51 | Downlink | Low | 2580 | 516000 | 2570.82 | 514164 | 0 | 30 | 6438 | 515070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2549.1 | 509820 | 102 |  | 6474 | 517950 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2610 | 522000 | 2419.38 | 483876 | 504 |  | 6513 | 521070 | 14 | 0 | 2 (2) | 1012 |
| 25 | 65 | Downlink | Low | 2582.5 | 516500 | 2570.8 | 514160 | 0 | 30 | 6439 | 515090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2546.58 | 509316 | 102 |  | 6468 | 517470 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2607.5 | 521500 | 2414.36 | 482872 | 504 |  | 6500 | 520090 | 22 | 0 | 2 (2) | 1012 |
| 30 | 78 | Downlink | Low | 2585 | 517000 | 2570.96 | 514192 | 0 | 30 | 6437 | 515050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2595 | 519000 | 2544.24 | 508848 | 102 |  | 6462 | 516990 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2605 | 521000 | 2409.52 | 481904 | 504 |  | 6487 | 518930 | 6 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 2590 | 518000 | 2570.92 | 514184 | 0 | 30 | 6439 | 515090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2539.2 | 507840 | 102 |  | 6450 | 516030 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2600 | 520000 | 2399.48 | 479896 | 504 |  | 6461 | 516970 | 22 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.38-3: Test frequencies for NR operating band n38 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2575 | 515000 | 2571.04 | 514208 | 0 | 30 | - | 514928 |
|  |  | & | Mid | 2595 | 519000 | 2517.6 | 503520 | 102 |  | - | 518928 |
|  |  | Uplink | High | 2615 | 523000 | 2248.16 | 449632 | 504 |  | - | 522928 |
| 15 | 18 | Downlink | Low | 2577.5 | 515500 | 2571.02 | 514204 | 0 | 30 | - | 514924 |
|  |  | & | Mid | 2595 | 519000 | 2515.08 | 503016 | 102 |  | - | 518424 |
|  |  | Uplink | High | 2612.5 | 522500 | 2243.14 | 448628 | 504 |  | - | 521924 |
| 20 | 24 | Downlink | Low | 2580 | 516000 | 2571.36 | 514272 | 0 | 30 | - | 514992 |
|  |  | & | Mid | 2595 | 519000 | 2512.92 | 502584 | 102 |  | - | 517992 |
|  |  | Uplink | High | 2610 | 522000 | 2238.48 | 447696 | 504 |  | - | 520992 |
| 25 | 31 | Downlink | Low | 2582.5 | 516500 | 2571.34 | 514268 | 0 | 30 | - | 514988 |
|  |  | & | Mid | 2595 | 519000 | 2510.4 | 502080 | 102 |  | - | 517488 |
|  |  | Uplink | High | 2607.5 | 521500 | 2233.46 | 446692 | 504 |  | - | 519988 |
| 30 | 38 | Downlink | Low | 2585 | 517000 | 2571.32 | 514264 | 0 | 30 | - | 514984 |
|  |  | & | Mid | 2595 | 519000 | 2507.88 | 501576 | 102 |  | - | 516984 |
|  |  | Uplink | High | 2605 | 521000 | 2228.44 | 445688 | 504 |  | - | 518984 |
| 40 | 51 | Downlink | Low | 2590 | 518000 | 2571.64 | 514328 | 0 | 15 | - | 514688 |
|  |  | & | Mid | 2595 | 519000 | 2503.2 | 500640 | 102 |  | - | 515688 |
|  |  | Uplink | High | 2600 | 520000 | 2218.76 | 443752 | 504 |  | - | 516688 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.39	Reference test frequencies for NR operating band n39

Table 4.3.1.1.1.39-1: Test frequencies for NR operating band n39 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1882.5 | 376500 | 1880.25 | 376050 | 0 | 15 | 4707 | 376590 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 1899.1 | 379820 | 1878.49 | 375698 | 102 |  | 4747 | 379730 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 1917.5 | 383500 | 1824.53 | 364906 | 504 |  | 4793 | 383530 | 4 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink | Low | 1885 | 377000 | 1880.32 | 376064 | 0 | 30 | 4714 | 377090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 1900 | 380000 | 1876.96 | 375392 | 102 |  | 4750 | 379970 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1915 | 383000 | 1819.6 | 363920 | 504 |  | 4789 | 383090 | 6 | 2 | 1 (6) | 512 |
| 15 | 79 | Downlink | Low | 1887.5 | 377500 | 1880.39 | 376078 | 0 | 30 | 4712 | 377050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1874.53 | 374906 | 102 |  | 4744 | 379490 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1912.5 | 382500 | 1814.67 | 362934 | 504 |  | 4773 | 381870 | 0 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 1890 | 378000 | 1880.46 | 376092 | 0 | 30 | 4713 | 377070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1872.1 | 374420 | 102 |  | 4738 | 379010 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1910 | 382000 | 1809.74 | 361948 | 504 |  | 4760 | 380890 | 2 | 0 | 0 (2) | 506 |
| 25 | 133 | Downlink | Low | 1892.5 | 378500 | 1880.53 | 376106 | 0 | 30 | 4714 | 377090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1869.67 | 373934 | 102 |  | 4732 | 378530 | 8 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1907.5 | 381500 | 1804.81 | 360962 | 504 |  | 4750 | 379970 | 0 | 2 | 0 (2) | 508 |
| 30 | 160 | Downlink | Low | 1895 | 379000 | 1880.6 | 376120 | 0 | 30 | 4712 | 377050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 1900 | 380000 | 1867.24 | 373448 | 102 |  | 4726 | 378050 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1905 | 381000 | 1799.88 | 359976 | 504 |  | 4737 | 378990 | 2 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 1900 | 380000 | 1880.56 | 376112 | 0 | 30 | 4714 | 377090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 for SSB_SCS = 15kHz and Table 13-3 for SSB_SCS=30kHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.39-2: Test frequencies for NR operating band n39 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1885 | 377000 | 1880.68 | 376136 | 0 | 30 | 4714 | 377090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 1900 | 380000 | 1858.96 | 371792 | 102 |  | 4750 | 379970 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1915 | 383000 | 1729.24 | 345848 | 504 |  | 4789 | 383090 | 6 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 1887.5 | 377500 | 1880.66 | 376132 | 0 | 30 | 4712 | 377050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1856.44 | 371288 | 102 |  | 4744 | 379490 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1912.5 | 382500 | 1724.22 | 344844 | 504 |  | 4773 | 381870 | 6 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 1890 | 378000 | 1880.82 | 376164 | 0 | 30 | 4713 | 377070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1854.1 | 370820 | 102 |  | 4738 | 379010 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1910 | 382000 | 1719.38 | 343876 | 504 |  | 4760 | 380890 | 2 | 0 | 0 (0) | 1008 |
| 25 | 65 | Downlink | Low | 1892.5 | 378500 | 1880.8 | 376160 | 0 | 30 | 4714 | 377090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1851.58 | 370316 | 102 |  | 4732 | 378530 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1907.5 | 381500 | 1714.36 | 342872 | 504 |  | 4750 | 379970 | 6 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 1895 | 379000 | 1880.96 | 376192 | 0 | 30 | 4712 | 377050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 1900 | 380000 | 1849.24 | 369848 | 102 |  | 4726 | 378050 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1905 | 381000 | 1709.52 | 341904 | 504 |  | 4737 | 378990 | 2 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 1900 | 380000 | 1880.92 | 376184 | 0 | 30 | 4714 | 377090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.39-3: Test frequencies for NR operating band n39 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1885 | 377000 | 1881.04 | 376208 | 0 | 30 | - | 376928 |  |
|  |  | & | Mid | 1900 | 380000 | 1822.6 | 364520 | 102 |  | - | 379928 |  |
|  |  | Uplink | High | 1915 | 383000 | 1548.16 | 309632 | 504 |  | - | 382928 |  |
| 15 | 18 | Downlink | Low | 1887.5 | 377500 | 1881.02 | 376204 | 0 | 30 | - | 376924 |  |
|  |  | & | Mid | 1900 | 380000 | 1820.08 | 364016 | 102 |  | - | 379424 |  |
|  |  | Uplink | High | 1912.5 | 382500 | 1543.14 | 308628 | 504 |  | - | 381924 |  |
| 20 | 24 | Downlink | Low | 1890 | 378000 | 1881.36 | 376272 | 0 | 30 | - | 376992 |  |
|  |  | & | Mid | 1900 | 380000 | 1817.92 | 363584 | 102 |  | - | 378992 |  |
|  |  | Uplink | High | 1910 | 382000 | 1538.48 | 307696 | 504 |  | - | 380992 |  |
| 25 | 31 | Downlink | Low | 1892.5 | 378500 | 1881.34 | 376268 | 0 | 30 | - | 376988 |  |
|  |  | & | Mid | 1900 | 380000 | 1815.4 | 363080 | 102 |  | - | 378488 |  |
|  |  | Uplink | High | 1907.5 | 381500 | 1533.46 | 306692 | 504 |  | - | 379988 |  |
| 30 | 38 | Downlink | Low | 1895 | 379000 | 1881.32 | 376264 | 0 | 30 | - | 376984 |  |
|  |  | & | Mid | 1900 | 380000 | 1812.88 | 362576 | 102 |  | - | 377984 |  |
|  |  | Uplink | High | 1905 | 381000 | 1528.44 | 305688 | 504 |  | - | 378984 |  |
| 40 | 51 | Downlink | Low | 1900 | 380000 | 1881.64 | 376328 | 0 | 30 | - | 377048 |  |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.40	Reference test frequencies for NR operating band n40

Table 4.3.1.1.1.40-1: Test frequencies for NR operating band n40 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2302.5 | 460500 | 2300.25 | 460050 | 0 | (Note 3) | - | 460770 | - | - | - | - |
|  |  | & | Mid | 2350 | 470000 | 2329.39 | 465878 | 102 |  | - | 470270 | - | - | - | - |
|  |  | Uplink | High | 2397.5 | 479500 | 2304.53 | 460906 | 504 |  | - | 479770 | - | - | - | - |
| 10 | 52 | Downlink | Low | 2305 | 461000 | 2300.32 | 460064 | 0 | 30 | 5764 | 461090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 2350 | 470000 | 2326.96 | 465392 | 102 |  | 5875 | 469970 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2395 | 479000 | 2299.6 | 459920 | 504 |  | 5989 | 479090 | 6 | 2 | 1 (6) | 512 |
| 15 | 79 | Downlink | Low | 2307.5 | 461500 | 2300.39 | 460078 | 0 | 30 | 5762 | 461050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2324.53 | 464906 | 102 |  | 5869 | 469490 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2392.5 | 478500 | 2294.67 | 458934 | 504 |  | 5973 | 477870 | 0 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 2310 | 462000 | 2300.46 | 460092 | 0 | 30 | 5763 | 461070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2322.1 | 464420 | 102 |  | 5863 | 469010 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2390 | 478000 | 2289.74 | 457948 | 504 |  | 5960 | 476890 | 2 | 0 | 0 (2) | 506 |
| 25 | 133 | Downlink | Low | 2312.5 | 462500 | 2300.53 | 460106 | 0 | 30 | 5764 | 461090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2319.67 | 463934 | 102 |  | 5857 | 468530 | 8 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2387.5 | 477500 | 2284.81 | 456962 | 504 |  | 5950 | 475970 | 0 | 2 | 0 (2) | 508 |
| 30 | 160 | Downlink | Low | 2315 | 463000 | 2300.6 | 460120 | 0 | 30 | 5762 | 461050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2350 | 470000 | 2317.24 | 463448 | 102 |  | 5851 | 468050 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2385 | 477000 | 2279.88 | 455976 | 504 |  | 5937 | 474990 | 2 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 2320 | 464000 | 2300.56 | 460112 | 0 | 30 | 5764 | 461090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2312.2 | 462440 | 102 |  | 5839 | 467090 | 2 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 2380 | 476000 | 2269.84 | 453968 | 504 |  | 5914 | 473090 | 2 | 1 | 1 (6) | 511 |
| 50 | 270 | Downlink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 | 30 | 5763 | 461070 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2350 | 470000 | 2307.34 | 461468 | 102 |  | 5827 | 466130 | 6 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 2375 | 475000 | 2259.98 | 451996 | 504 |  | 5888 | 471130 | 6 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.40-2: Test frequencies for NR operating band n40 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2305 | 461000 | 2300.68 | 460136 | 0 | 30 | 5764 | 461090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2350 | 470000 | 2308.96 | 461792 | 102 |  | 5875 | 469970 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2395 | 479000 | 2209.24 | 441848 | 504 |  | 5989 | 479090 | 6 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 2307.5 | 461500 | 2300.66 | 460132 | 0 | 30 | 5762 | 461050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2306.44 | 461288 | 102 |  | 5869 | 469490 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2392.5 | 478500 | 2204.22 | 440844 | 504 |  | 5973 | 477870 | 6 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 2310 | 462000 | 2300.82 | 460164 | 0 | 30 | 5763 | 461070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2304.1 | 460820 | 102 |  | 5863 | 469010 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2390 | 478000 | 2199.38 | 439876 | 504 |  | 5960 | 476890 | 2 | 0 | 0 (0) | 1008 |
| 25 | 65 | Downlink | Low | 2312.5 | 462500 | 2300.8 | 460160 | 0 | 30 | 5764 | 461090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2301.58 | 460316 | 102 |  | 5857 | 468530 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2387.5 | 477500 | 2194.36 | 438872 | 504 |  | 5950 | 475970 | 6 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 2315 | 463000 | 2300.96 | 460192 | 0 | 30 | 5762 | 461050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2350 | 470000 | 2299.24 | 459848 | 102 |  | 5851 | 468050 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2385 | 477000 | 2189.52 | 437904 | 504 |  | 5937 | 474990 | 2 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 2320 | 464000 | 2300.92 | 460184 | 0 | 30 | 5764 | 461090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2294.2 | 458840 | 102 |  | 5839 | 467090 | 14 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2380 | 476000 | 2179.48 | 435896 | 504 |  | 5914 | 473090 | 14 | 0 | 2 (2) | 1012 |
| 50 | 133 | Downlink | Low | 2325 | 465000 | 2301.06 | 460212 | 0 | 30 | 5763 | 461070 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2350 | 470000 | 2289.34 | 457868 | 102 |  | 5827 | 466130 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2375 | 475000 | 2169.62 | 433924 | 504 |  | 5888 | 471130 | 18 | 0 | 2 (2) | 1012 |
| 60 | 162 | Downlink | Low | 2330 | 466000 | 2300.84 | 460168 | 0 | 30 | 5762 | 461050 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2284.12 | 456824 | 102 |  | 5812 | 464930 | 14 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2370 | 474000 | 2159.4 | 431880 | 504 |  | 5862 | 468990 | 10 | 0 | 1 (1) | 1010 |
| 70 | 189 | Downlink | Low | 2335 | 467000 | 2300.98 | 460196 | 0 | 30 | 5764 | 461090 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2279.26 | 455852 | 102 |  | 5800 | 463970 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2365 | 473000 | 2149.54 | 429908 | 504 |  | 5839 | 467090 | 10 | 0 | 2 (2) | 1012 |
| 80 | 217 | Downlink | Low | 2340 | 468000 | 2300.94 | 460188 | 0 | 30 | 5763 | 461070 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2274.22 | 454844 | 102 |  | 5788 | 463010 | 10 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2360 | 472000 | 2139.5 | 427900 | 504 |  | 5813 | 465130 | 2 | 0 | 3 (3) | 1014 |
| 90 | 245 | Downlink | Low | 2345 | 469000 | 2300.9 | 460180 | 0 | 30 | 5762 | 461050 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2269.18 | 453836 | 102 |  | 5776 | 462050 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2355 | 471000 | 2129.46 | 425892 | 504 |  | 5787 | 462990 | 6 | 0 | 1 (1) | 1010 |
| 100 | 273 | Downlink | Low | 2350 | 470000 | 2300.86 | 460172 | 0 | 30 | 5764 | 461090 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2264.14 | 452828 | 102 |  | 5764 | 461090 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2350 | 470000 | 2119.42 | 423884 | 504 |  | 5764 | 461090 | 18 | 0 | 2 (2) | 1012 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.40-3: Test frequencies for NR operating band n40 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2305 | 461000 | 2301.04 | 460208 | 0 | 30 | - | 460928 |
|  |  | & | Mid | 2350 | 470000 | 2272.6 | 454520 | 102 |  | - | 469928 |
|  |  | Uplink | High | 2395 | 479000 | 2028.16 | 405632 | 504 |  | - | 478928 |
| 15 | 18 | Downlink | Low | 2307.5 | 461500 | 2301.02 | 460204 | 0 | 30 | - | 460924 |
|  |  | & | Mid | 2350 | 470000 | 2270.08 | 454016 | 102 |  | - | 469424 |
|  |  | Uplink | High | 2392.5 | 478500 | 2023.14 | 404628 | 504 |  | - | 477924 |
| 20 | 24 | Downlink | Low | 2310 | 462000 | 2301.36 | 460272 | 0 | 30 | - | 460992 |
|  |  | & | Mid | 2350 | 470000 | 2267.92 | 453584 | 102 |  | - | 468992 |
|  |  | Uplink | High | 2390 | 478000 | 2018.48 | 403696 | 504 |  | - | 476992 |
| 25 | 31 | Downlink | Low | 2312.5 | 462500 | 2301.34 | 460268 | 0 | 30 | - | 460988 |
|  |  | & | Mid | 2350 | 470000 | 2265.4 | 453080 | 102 |  | - | 468488 |
|  |  | Uplink | High | 2387.5 | 477500 | 2013.46 | 402692 | 504 |  | - | 475988 |
| 30 | 38 | Downlink | Low | 2315 | 463000 | 2301.32 | 460264 | 0 | 30 | - | 460984 |
|  |  | & | Mid | 2350 | 470000 | 2262.88 | 452576 | 102 |  | - | 467984 |
|  |  | Uplink | High | 2385 | 477000 | 2008.44 | 401688 | 504 |  | - | 474984 |
| 40 | 51 | Downlink | Low | 2320 | 464000 | 2301.64 | 460328 | 0 | 30 | - | 461048 |
|  |  | & | Mid | 2350 | 470000 | 2258.2 | 451640 | 102 |  | - | 467048 |
|  |  | Uplink | High | 2380 | 476000 | 1998.76 | 399752 | 504 |  | - | 473048 |
| 50 | 65 | Downlink | Low | 2325 | 465000 | 2301.6 | 460320 | 0 | 30 | - | 461040 |
|  |  | & | Mid | 2350 | 470000 | 2253.16 | 450632 | 102 |  | - | 466040 |
|  |  | Uplink | High | 2375 | 475000 | 1988.72 | 397744 | 504 |  | - | 471040 |
| 60 | 79 | Downlink | Low | 2330 | 466000 | 2301.56 | 460312 | 0 | 30 | - | 461032 |
|  |  | & | Mid | 2350 | 470000 | 2248.12 | 449624 | 102 |  | - | 465032 |
|  |  | Uplink | High | 2370 | 474000 | 1978.68 | 395736 | 504 |  | - | 469032 |
| 70 | 93 | Downlink | Low | 2335 | 467000 | 2301.52 | 460304 | 0 | 30 | - | 461024 |
|  |  | & | Mid | 2350 | 470000 | 2243.08 | 448616 | 102 |  | - | 464024 |
|  |  | Uplink | High | 2365 | 473000 | 1968.64 | 393728 | 504 |  | - | 467024 |
| 80 | 107 | Downlink | Low | 2340 | 468000 | 2301.48 | 460296 | 0 | 30 | - | 461016 |
|  |  | & | Mid | 2350 | 470000 | 2238.04 | 447608 | 102 |  | - | 463016 |
|  |  | Uplink | High | 2360 | 472000 | 1958.6 | 391720 | 504 |  | - | 465016 |
| 90 | 121 | Downlink | Low | 2345 | 469000 | 2301.44 | 460288 | 0 | 30 | - | 461008 |
|  |  | & | Mid | 2350 | 470000 | 2233 | 446600 | 102 |  | - | 462008 |
|  |  | Uplink | High | 2355 | 471000 | 1948.56 | 389712 | 504 |  | - | 463008 |
| 100 | 135 | Downlink | Low | 2350 | 470000 | 2301.4 | 460280 | 0 | 30 | - | 461000 |
|  |  | & | Mid | 2350 | 470000 | 2227.96 | 445592 | 102 |  | - | 461000 |
|  |  | Uplink | High | 2350 | 470000 | 1938.52 | 387704 | 504 |  | - | 461000 |
| Note: FR1 carrier without CORSESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.41	Reference test frequencies for NR operating band n41

Table 4.3.1.1.1.41-1: Test frequencies for NR operating band n41, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 2501.01 | 500202 | 2496.33 | 499266 | 0 | 15 | 6246 | 499710 | 4 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2569.965 | 513993 | 102 |  | 6477 | 518190 | 7 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2685 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
| 15 | 79 | Downlink | Low | 2503.5 | 500700 | 2496.39 | 499278 | 0 | 15 | 6246 | 499710 | 0 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2567.535 | 513507 | 102 |  | 6471 | 517710 | 9 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2682.495 | 536499 | 2584.665 | 516933 | 504 |  | 6693 | 535470 | 11 | 0 | 0 (0) | 504 |
| 20 | 106 | Downlink | Low | 2506.005 | 501201 | 2496.465 | 499293 | 0 | 15 | 6246 | 499710 | 7 | 1 | 0 (0) | 1 |
|  |  | & | Mid | 2593.005 | 518601 | 2565.105 | 513021 | 102 |  | 6465 | 517230 | 11 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2679.99 | 535998 | 2579.73 | 515946 | 504 |  | 6681 | 534510 | 8 | 1 | 0 (0) | 505 |
| 30 | 160 | Downlink | Low | 2511 | 502200 | 2496.6 | 499320 | 0 | 15 | 6246 | 499710 | 10 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2593.005 | 518601 | 2560.245 | 512049 | 102 |  | 6453 | 516270 | 3 | 1 | 2 (4) | 107 |
|  |  | Uplink | High | 2674.995 | 534999 | 2569.875 | 513975 | 504 |  | 6657 | 532590 | 1 | 1 | 1 (2) | 507 |
| 40 | 216 | Downlink | Low | 2516.01 | 503202 | 2496.57 | 499314 | 0 | 15 | 6246 | 499710 | 0 | 1 | 0 (0) | 1 |
|  |  | & | Mid | 2593.005 | 518601 | 2555.205 | 511041 | 102 |  | 6441 | 515310 | 7 | 2 | 2 (4) | 108 |
|  |  | Uplink | High | 2670 | 534000 | 2559.84 | 511968 | 504 |  | 6633 | 530670 | 6 | 1 | 2 (4) | 509 |
| 50 | 270 | Downlink | Low | 2521.005 | 504201 | 2496.705 | 499341 | 0 | 15 | 6246 | 499710 | 3 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2593.005 | 518601 | 2550.345 | 510069 | 102 |  | 6426 | 514110 | 3 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2664.99 | 532998 | 2549.97 | 509994 | 504 |  | 6606 | 528510 | 4 | 0 | 0 (0) | 504 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.41-2: Test frequencies for NR operating band n41, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2501.01 | 500202 | 2496.69 | 499338 | 0 | 30 | 6252 | 500190 | 20 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2551.95 | 510390 | 102 |  | 6483 | 518670 | 0 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 2685 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (0) | 1008 |
| 15 | 38 | Downlink | Low | 2503.5 | 500700 | 2496.66 | 499332 | 0 | 30 | 6252 | 500190 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2549.43 | 509886 | 102 |  | 6474 | 517950 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2682.48 | 536496 | 2494.2 | 498840 | 504 |  | 6699 | 535950 | 10 | 0 | 1 (1) | 1010 |
| 20 | 51 | Downlink | Low | 2506.02 | 501204 | 2496.84 | 499368 | 0 | 30 | 6252 | 500190 | 10 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2547.09 | 509418 | 102 |  | 6471 | 517710 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 2679.99 | 535998 | 2489.37 | 497874 | 504 |  | 6687 | 534990 | 12 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 2511 | 502200 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2542.23 | 508446 | 102 |  | 6456 | 516510 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2674.98 | 534996 | 2479.5 | 495900 | 504 |  | 6663 | 533070 | 6 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 | 6252 | 500190 | 4 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2537.19 | 507438 | 102 |  | 6444 | 515550 | 16 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  | 6636 | 530910 | 2 | 0 | 0 (0) | 1008 |
| 50 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 | 6252 | 500190 | 18 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2592.99 | 518598 | 2532.33 | 506466 | 102 |  | 6432 | 514590 | 20 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  | 6612 | 528990 | 20 | 0 | 0 (0) | 1008 |
| 60 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 | 6252 | 500190 | 10 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2527.11 | 505422 | 102 |  | 6420 | 513630 | 0 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  | 6588 | 527070 | 14 | 0 | 2 (2) | 1012 |
| 70 | 189 | Downlink | Low | 2531.01 | 506202 | 2496.99 | 499398 | 0 | 30 | 6252 | 500190 | 0 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2522.25 | 504450 | 102 |  | 6408 | 512670 | 4 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2655 | 531000 | 2439.54 | 487908 | 504 |  | 6564 | 525150 | 6 | 0 | 3 (3) | 1014 |
| 80 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2517.21 | 503442 | 102 |  | 6396 | 511710 | 20 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  | 6537 | 522990 | 4 | 0 | 1 (1) | 1010 |
| 90 | 245 | Downlink | Low | 2541 | 508200 | 2496.9 | 499380 | 0 | 30 | 6252 | 500190 | 6 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2512.17 | 502434 | 102 |  | 6381 | 510510 | 4 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2644.98 | 528996 | 2419.44 | 483888 | 504 |  | 6513 | 521070 | 10 | 0 | 2 (2) | 1012 |
| 100 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 | 6252 | 500190 | 8 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2507.13 | 501426 | 102 |  | 6369 | 509550 | 20 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  | 6486 | 518910 | 6 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.41-3: Test frequencies for NR operating band n41, SCS 60 kHz and ΔFRaster 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2501.01 | 500202 | 2497.05 | 499410 | 0 | 15 | - | 499770 |
|  |  | & | Mid | 2593.005 | 518601 | 2515.605 | 503121 | 102 |  | - | 518169 |
|  |  | Uplink | High | 2685 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
| 15 | 18 | Downlink | Low | 2503.5 | 500700 | 2497.02 | 499404 | 0 | 15 | - | 499764 |
|  |  | & | Mid | 2593.005 | 518601 | 2513.085 | 502617 | 102 |  | - | 517665 |
|  |  | Uplink | High | 2682.495 | 536499 | 2313.135 | 462627 | 504 |  | - | 535563 |
| 20 | 24 | Downlink | Low | 2506.005 | 501201 | 2497.365 | 499473 | 0 | 15 | - | 499833 |
|  |  | & | Mid | 2593.005 | 518601 | 2510.925 | 502185 | 102 |  | - | 517233 |
|  |  | Uplink | High | 2679.99 | 535998 | 2308.47 | 461694 | 504 |  | - | 534630 |
| 30 | 38 | Downlink | Low | 2511 | 502200 | 2497.32 | 499464 | 0 | 15 | - | 499824 |
|  |  | & | Mid | 2593.005 | 518601 | 2505.885 | 501177 | 102 |  | - | 516225 |
|  |  | Uplink | High | 2674.995 | 534999 | 2298.435 | 459687 | 504 |  | - | 532623 |
| 40 | 51 | Downlink | Low | 2516.01 | 503202 | 2497.65 | 499530 | 0 | 15 | - | 499890 |
|  |  | & | Mid | 2593.005 | 518601 | 2501.205 | 500241 | 102 |  | - | 515289 |
|  |  | Uplink | High | 2670 | 534000 | 2288.76 | 457752 | 504 |  | - | 530688 |
| 50 | 65 | Downlink | Low | 2521.005 | 504201 | 2497.605 | 499521 | 0 | 15 | - | 499881 |
|  |  | & | Mid | 2593.005 | 518601 | 2496.165 | 499233 | 102 |  | - | 514281 |
|  |  | Uplink | High | 2664.99 | 532998 | 2278.71 | 455742 | 504 |  | - | 528678 |
| 60 | 79 | Downlink | Low | 2526 | 505200 | 2497.56 | 499512 | 0 | 15 | - | 499872 |
|  |  | & | Mid | 2593.005 | 518601 | 2491.125 | 498225 | 102 |  | - | 513273 |
|  |  | Uplink | High | 2659.995 | 531999 | 2268.675 | 453735 | 504 |  | - | 526671 |
| 70 | 93 | Downlink | Low | 2531.01 | 506202 | 2497.53 | 499506 | 0 | 15 | - | 499866 |
|  |  | & | Mid | 2593.005 | 518601 | 2486.085 | 497217 | 102 |  | - | 512265 |
|  |  | Uplink | High | 2655 | 531000 | 2258.64 | 451728 | 504 |  | - | 524664 |
| 80 | 107 | Downlink | Low | 2536.005 | 507201 | 2497.485 | 499497 | 0 | 15 | - | 499857 |
|  |  | & | Mid | 2593.005 | 518601 | 2481.045 | 496209 | 102 |  | - | 511257 |
|  |  | Uplink | High | 2649.99 | 529998 | 2248.59 | 449718 | 504 |  | - | 522654 |
| 90 | 121 | Downlink | Low | 2541 | 508200 | 2497.44 | 499488 | 0 | 15 | - | 499848 |
|  |  | & | Mid | 2593.005 | 518601 | 2476.005 | 495201 | 102 |  | - | 510249 |
|  |  | Uplink | High | 2644.995 | 528999 | 2238.555 | 447711 | 504 |  | - | 520647 |
| 100 | 135 | Downlink | Low | 2546.01 | 509202 | 2497.41 | 499482 | 0 | 15 | - | 499842 |
|  |  | & | Mid | 2593.005 | 518601 | 2470.965 | 494193 | 102 |  | - | 509241 |
|  |  | Uplink | High | 2640 | 528000 | 2228.52 | 445704 | 504 |  | - | 518640 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.42 to 4.3.1.1.1.45	FFS

###### 4.3.1.1.1.46	Reference test frequencies for NR operating band n46

Table 4.3.1.1.1.46-1: Test frequencies for NR operating band n46 and SCS 15 kHz (Note 3)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 4 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 5730 | 782000 | 5725.32 | 781688 | 0 | 30 | - | 781928 | 31 | - | - | - |
|  |  | & | Mid |  |  |  |  |  |  | - |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5734.62 | 782308 | 504 |  | - | 788596 | 31 | - | - | - |
| 20 | 106 | Downlink | Low | 5160 | 744000 | 5150.46 | 743364 | 0 | 30 | - | 743604 | 31 | - | - | - |
|  |  | & | Mid | 5479.98 | 765332 | 5452.08 | 763472 | 102 |  | - | 764936 | 31 | - | - | - |
|  |  | Uplink | High | 5905.02 | 793668 | 5804.76 | 786984 | 504 |  | - | 793272 | 31 | - | - | - |
| 40 | 216 | Downlink | Low | 5170.02 | 744668 | 5150.58 | 743372 | 0 | 30 | - | 743612 | 31 | - | - | - |
|  |  | & | Mid | 5509.98 | 767332 | 5472.18 | 764812 | 102 |  | - | 766276 | 31 | - | - | - |
|  |  | Uplink | High | 5895 | 793000 | 5784.84 | 785656 | 504 |  | - | 791944 | 31 | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: NR band n46 with SCS =15 kHz is restricted for operation when carrier is configured as an SCell part of a DC or CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 4: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.46-2: Test frequencies for NR operating band n46 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 4 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 5730 | 782000 | 5725.68 | 781712 | 0 | 30 | - | 781952 | 31 | - | - | - |
| Note 3 |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5644.26 | 776284 | 504 |  | - | 788620 | 31 | - | - | - |
| 20 | 51 | Downlink | Low | 5160 | 744000 | 5150.82 | 743388 | 0 | 30 | 8996 | 743712 | 12 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5479.98 | 765332 | 5434.08 | 762272 | 102 |  | 9218 | 765024 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5905.02 | 793668 | 5714.4 | 780960 | 504 |  | 9513 | 793344 | 0 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 5170.02 | 744668 | 5150.94 | 743396 | 0 | 30 | 8996 | 743712 | 4 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5509.98 | 767332 | 5454.18 | 763612 | 102 |  | 9232 | 766368 | 20 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5895 | 793000 | 5694.48 | 779632 | 504 |  | 9499 | 792000 | 8 | 0 | 1 (1) | 1010 |
| 60 | 162 | Downlink | Low | 5200.02 | 746668 Note 5 | 5170.86 | 744724 | 0 | 30 | 9010 | 745056 | 20 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5500.02 | 766668 | 5434.14 | 762276 | 102 |  | 9218 | 765024 | 12 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5884.98 | 792332 | 5674.38 | 778292 | 504 |  | 9485 | 790656 | 4 | 0 | 1 (1) | 1010 |
| 80 | 217 | Downlink | Low | 5190 | 746000 | 5150.94 | 743396 | 0 | 30 | 8996 | 743712 | 4 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5530,02 | 768668 | 5454,24 | 763616 | 102 |  | 9232 | 766368 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5875.02 | 791668 | 5654.52 | 776968 | 504 |  | 9472 | 789408 | 8 | 1 | 3 (3) | 1016 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of a DC or CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 4: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. Note 5: The next lowest allowed ARFCN values has been selected as the lowest ARFCN value in TS 38.101 [7], Table 5.4.2.3-2 for CBW=60 MHz will cause the required guardband to exceed the bands lower edge. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.46-3: Test frequencies for NR operating band n46 and SCS 60 kHz without CORESET#0 (Note 1)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 2 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 5730 | 782000 | 5726.04 | 781736 | 0 | 30 | - | 781976 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5463.18 | 764212 | 504 |  | - | 788644 |
| 20 | 24 | Downlink | Low | 5160 | 744000 | 5151.36 | 743424 | 0 | 30 | - | 743664 |
|  |  | & | Mid | 5479.98 | 765332 | 5397.9 | 759860 | 102 |  | - | 764996 |
|  |  | Uplink | High | 5905.02 | 793668 | 5533.5 | 768900 | 504 |  | - | 793332 |
| 40 | 51 | Downlink | Low | 5170.02 | 744668 | 5151.66 | 743444 | 0 | 30 | - | 743684 |
|  |  | & | Mid | 5509.98 | 767332 | 5418.18 | 761212 | 102 |  | - | 766348 |
|  |  | Uplink | High | 5895 | 793000 | 5513.76 | 767584 | 504 |  | - | 792016 |
| 60 | 79 | Downlink | Low | 5179.98 | 745332 | 5151.54 | 743436 | 0 | 30 | - | 743676 |
|  |  | & | Mid | 5500.02 | 766668 | 5398.14 | 759876 | 102 |  | - | 765012 |
|  |  | Uplink | High | 5884.98 | 792332 | 5493.66 | 766244 | 504 |  | - | 790676 |
| 80 | 107 | Downlink | Low | 5190 | 746000 | 5151.48 | 743432 | 0 | 30 | - | 743672 |
|  |  | & | Mid | 5530,02 | 768668 | 5418,06 | 761204 | 102 |  | - | 766340 |
|  |  | Uplink | High | 5875.02 | 791668 | 5473.62 | 764908 | 504 |  | - | 789340 |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.47	FFS

###### 4.3.1.1.1.48	Reference test frequencies for NR operating band n48

Table 4.3.1.1.1.48-1: Test frequencies for NR operating band n48 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | (Note 4) | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 3552.51 | 636834 | 3550.26 | 636684 | 0 | (Note 3) | - | - | - | - | - | - |
|  |  | & | Mid | 3625.005 | 641667 | 3604.395 | 640293 | 102 |  | - | - | - | - | - | - |
|  |  | Uplink | High | 3697.5 | 646500 | 3604.53 | 640302 | 504 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 3624.99 | 641666 | 3601.95 | 640130 | 102 |  | 7933 | 641664 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 3624.99 | 641666 | 3599.52 | 639968 | 102 |  | 7931 | 641472 | 4 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 20 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3597.09 | 639806 | 102 |  | 7930 | 641376 | 10 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 3565.02 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3592.23 | 639482 | 102 |  | 7926 | 640992 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 40 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3587.19 | 639146 | 102 |  | 7923 | 640704 | 10 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 50 | 270 | Downlink | Low | 3575.01 | 638334 | 3550.71 | 636714 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 4) | Mid | 3624.99 | 641666 | 3582.33 | 638822 | 102 |  |  | 640320 | 31 |  |  |  |
|  |  | Uplink | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  |  | 643680 | 31 |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell. Note 4: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.48-2: Test frequencies for NR operating band n48 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 3624.99 | 641666 | 3583.95 | 638930 | 102 |  | 7933 | 641664 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 3624.99 | 641666 | 3581.43 | 638762 | 102 |  | 7931 | 641472 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 20 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3579.09 | 638606 | 102 |  | 7930 | 641376 | 10 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3574.23 | 638282 | 102 |  | 7926 | 640992 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3569.19 | 637946 | 102 |  | 7923 | 640704 | 22 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 50 | 133 | Downlink | Low | 3575.01 | 638334 | 3551.07 | 636738 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3564.33 | 637622 | 102 |  |  | 640320 | 31 |  |  |  |
|  |  | (Note 3) | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  |  | 643680 | 31 |  |  |  |
| 60 | 162 | Downlink | Low | 3580.02 | 638668 | 3550.86 | 636724 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3559.11 | 637274 | 102 |  |  | 640032 | 31 |  |  |  |
|  |  | (Note 3) | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  |  | 643008 | 31 |  |  |  |
| 70 | 189 | Downlink (Note 3) | Low | 3585 | 639000 | 3550.98 | 636732 | 0 | 30 | - | 637056 | 31 | - | - | - |
|  |  |  | Mid | 3624.99 | 641666 | 3554.25 | 636950 | 102 |  | - | 639648 | 31 | - | - | - |
|  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | - | 642336 | 31 | - | - | - |
| 80 | 217 | Downlink | Low | 3590.01 | 639334 | 3550.95 | 636730 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3549.21 | 636614 | 102 |  |  | 639360 | 31 |  |  |  |
|  |  | (Note 3) | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  |  | 641664 | 31 |  |  |  |
| 90 | 245 | Downlink | Low | 3595.02 | 639668 | 3550.92 | 636728 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3544.17 | 636278 | 102 |  |  | 638976 | 31 |  |  |  |
|  |  | (Note 3) | High | 3654.99 | 643666 | 3429.45 | 628630 | 504 |  |  | 640992 | 31 |  |  |  |
| 100 | 273 | Downlink | Low | 3600 | 640000 | 3550.86 | 636724 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3539.13 | 635942 | 102 |  |  | 638688 | 31 |  |  |  |
|  |  | (Note 3) | High | 3649.98 | 643332 | 3419.4 | 627960 | 504 |  |  | 640320 | 31 |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.48-3: Test frequencies for NR operating band n48 and SCS 60 kHz without CORESET#0 (Note 1)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3555 | 637000 | 3551.04 | 636736 | 0 | 30 | - | 636976 |
|  |  | & | Mid | 3624.99 | 641666 | 3547.59 | 636506 | 102 |  | - | 641642 |
|  |  | Uplink | High | 3694.98 | 646332 | 3328.14 | 621876 | 504 |  | - | 646308 |
| 15 | 18 | Downlink | Low | 3557.52 | 637168 | 3551.04 | 636736 | 0 | 30 | - | 636976 |
|  |  | & | Mid | 3624.99 | 641666 | 3545.07 | 636338 | 102 |  | - | 641474 |
|  |  | Uplink | High | 3692.49 | 646166 | 3323.13 | 621542 | 504 |  | - | 645974 |
| 20 | 24 | Downlink | Low | 3560.01 | 637334 | 3551.37 | 636758 | 0 | 30 | - | 636998 |
|  |  | & | Mid | 3624.99 | 641666 | 3542.91 | 636194 | 102 |  | - | 641330 |
|  |  | Uplink | High | 3690 | 646000 | 3318.48 | 621232 | 504 |  | - | 645664 |
| 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 |
|  |  | & | Mid | 3624.99 | 641666 | 3574.23 | 638282 | 102 |  | 7926 | 640992 |
|  |  | Uplink | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 |
| 40 | 51 | Downlink | Low | 3570 | 638000 | 3551.64 | 636776 | 0 | 30 | - | 637016 |
|  |  | & | Mid | 3624.99 | 641666 | 3533.19 | 635546 | 102 |  | - | 640682 |
|  |  | Uplink | High | 3679.98 | 645332 | 3298.74 | 619916 | 504 |  | - | 644348 |
| 50 | 65 | Downlink(Note 2) | Low | 3575.01 | 638334 | 3551.61 | 636774 | 0 | 30 | - | 637014 |
|  |  |  | Mid | 3624.99 | 641666 | 3528.15 | 635210 | 102 |  | - | 640346 |
|  |  |  | High | 3675 | 645000 | 3288.72 | 619248 | 504 |  | - | 643680 |
| 60 | 79 | Downlink(Note 2) | Low | 3580.02 | 638668 | 3551.58 | 636772 | 0 | 30 | - | 637012 |
|  |  |  | Mid | 3624.99 | 641666 | 3523.11 | 634874 | 102 |  | - | 640010 |
|  |  |  | High | 3669.99 | 644666 | 3278.67 | 618578 | 504 |  | - | 643010 |
| 70 | 93 | Downlink (Note 2) | Low | 3585 | 639000 | 3551.52 | 636768 | 0 | 30 | - | 637008 |
|  |  |  | Mid | 3624.99 | 641666 | 3518.07 | 634538 | 102 |  | - | 639674 |
|  |  |  | High | 3664.98 | 644332 | 3268.62 | 617908 | 504 |  | - | 642340 |
| 80 | 107 | Downlink(Note 2) | Low | 3590.01 | 639334 | 3551.49 | 636766 | 0 | 30 | - | 637006 |
|  |  |  | Mid | 3624.99 | 641666 | 3513.03 | 634202 | 102 |  | - | 639338 |
|  |  |  | High | 3660 | 644000 | 3258.6 | 617240 | 504 |  | - | 641672 |
| 90 | 121 | Downlink(Note 2) | Low | 3595.02 | 639668 | 3551.46 | 636764 | 0 | 30 | - | 637004 |
|  |  |  | Mid | 3624.99 | 641666 | 3507.99 | 633866 | 102 |  | - | 639002 |
|  |  |  | High | 3654.99 | 643666 | 3248.55 | 616570 | 504 |  | - | 641002 |
| 100 | 135 | Downlink(Note 2) | Low | 3600 | 640000 | 3551.4 | 636760 | 0 | 30 | - | 637000 |
|  |  |  | Mid | 3624.99 | 641666 | 3502.95 | 633530 | 102 |  | - | 638666 |
|  |  |  | High | 3649.98 | 643332 | 3238.5 | 615900 | 504 |  | - | 640332 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.49	FFS

###### 4.3.1.1.1.50	Reference test frequencies for NR operating band n50

Table 4.3.1.1.1.50-1: Test frequencies for NR operating band n50 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset[RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | (Note 3) | - | 287170 | - | - | - | - |
|  |  | & | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | - | 295170 | - | - | - | - |
|  |  | Uplink | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | - | 303170 | - | - | - | - |
| 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 30 | 3591 | 287310 | 6 | 1 | 0 (2) | 3 |
|  |  | & | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3687 | 294990 | 6 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3780 | 302430 | 10 | 0 | 1 (6) | 510 |
| 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 30 | 3592 | 287330 | 8 | 1 | 0 (2) | 3 |
|  |  | & | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3678 | 294270 | 0 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3767 | 301450 | 0 | 1 | 1 (6) | 511 |
| 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 30 | 3590 | 287290 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (2) | 506 |
| 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 30 | 3591 | 287310 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3648 | 291870 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3705 | 296430 | 6 | 3 | 0 (2) | 509 |
| 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 30 | 3593 | 287530 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3636 | 290910 | 2 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3682 | 294530 | 6 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.50-2: Test frequencies for NR operating band n50 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 30 | 3591 | 287310 | 18 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (2) | 1012 |
| 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 30 | 3592 | 287330 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (2) | 1012 |
| 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 30 | 3590 | 287290 | 2 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 1452 | 290400 | 1432.92 | 286584 | 0 | 30 | 3591 | 287310 | 2 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1418.7 | 283740 | 102 |  | 3648 | 291870 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1497 | 299400 | 1296.48 | 259296 | 504 |  | 3705 | 296430 | 18 | 0 | 1 (1) | 1010 |
| 50 | 133 | Downlink | Low | 1457 | 291400 | 1433.06 | 286612 | 0 | 30 | 3593 | 287530 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1474.5 | 294900 | 1413.84 | 282768 | 102 |  | 3636 | 290910 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1492 | 298400 | 1286.62 | 257324 | 504 |  | 3682 | 294530 | 18 | 0 | 2 (2) | 1012 |
| 60 | 162 | Downlink | Low | 1462 | 292400 | 1432.84 | 286568 | 0 | 30 | 3592 | 287330 | 14 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1408.62 | 281724 | 102 |  | 3624 | 289950 | 6 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1487 | 297400 | 1276.4 | 255280 | 504 |  | 3653 | 292330 | 14 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 1472 | 294400 | 1432.94 | 286588 | 0 | 30 | 3593 | 287530 | 2 | 0 | 3 (3) | 6 |
| (Note 3) |  |  | Mid | 1474.5 | 294900 | 1398.72 | 279744 | 102 |  | 3600 | 288030 | 2 | 0 | 3 (3) | 210 |
|  |  |  | High | 1477 | 295400 | 1256.5 | 251300 | 504 |  | 3607 | 288530 | 2 | 0 | 3 (3) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This UE channel bandwidth is applicable only to downlink (TS 38.101-1 table 5.3.5-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.50-3: Test frequencies for NR operating band n50 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1437 | 287400 | 1433.04 | 286608 | 0 | 30 | - | 287328 |
|  |  | & | Mid | 1474.5 | 294900 | 1397.1 | 279420 | 102 |  | - | 294828 |
|  |  | Uplink | High | 1512 | 302400 | 1145.16 | 229032 | 504 |  | - | 302328 |
| 15 | 18 | Downlink | Low | 1439.5 | 287900 | 1433.02 | 286604 | 0 | 30 | - | 287324 |
|  |  | & | Mid | 1474.5 | 294900 | 1394.58 | 278916 | 102 |  | - | 294324 |
|  |  | Uplink | High | 1509.5 | 301900 | 1140.14 | 228028 | 504 |  | - | 301324 |
| 20 | 24 | Downlink | Low | 1442 | 288400 | 1433.36 | 286672 | 0 | 30 | - | 287392 |
|  |  | & | Mid | 1474.5 | 294900 | 1392.42 | 278484 | 102 |  | - | 293892 |
|  |  | Uplink | High | 1507 | 301400 | 1135.48 | 227096 | 504 |  | - | 300392 |
| 40 | 51 | Downlink | Low | 1452 | 290400 | 1433.64 | 286728 | 0 | 30 | - | 287448 |
|  |  | & | Mid | 1474.5 | 294900 | 1382.7 | 276540 | 102 |  | - | 291948 |
|  |  | Uplink | High | 1497 | 299400 | 1115.76 | 223152 | 504 |  | - | 296448 |
| 50 | 65 | Downlink | Low | 1457 | 291400 | 1433.6 | 286720 | 0 | 30 | - | 287440 |
|  |  | & | Mid | 1474.5 | 294900 | 1377.66 | 275532 | 102 |  | - | 290940 |
|  |  | Uplink | High | 1492 | 298400 | 1105.72 | 221144 | 504 |  | - | 294440 |
| 60 | 79 | Downlink | Low | 1462 | 292400 | 1433.56 | 286712 | 0 | 30 | - | 287432 |
|  |  | & | Mid | 1474.5 | 294900 | 1372.62 | 274524 | 102 |  | - | 289932 |
|  |  | Uplink | High | 1487 | 297400 | 1095.68 | 219136 | 504 |  | - | 292432 |
| 80 | 107 | Downlink | Low | 1472 | 294400 | 1433.48 | 286696 | 0 | 30 | - | 287416 |
| (Note 2) |  |  | Mid | 1474.5 | 294900 | 1362.54 | 272508 | 102 |  | - | 287916 |
|  |  |  | High | 1477 | 295400 | 1075.6 | 215120 | 504 |  | - | 288416 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: This UE channel bandwidth is applicable only to downlink (TS 38.101-1 table 5.3.5-1). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.51	Reference test frequencies for NR operating band n51

Table 4.3.1.1.1.51-1: Test frequencies for NR operating band n51 and SCS 15 kHz

| [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.52	FFS

###### 4.3.1.1.1.53	Reference test frequencies for NR operating band n53

Table 4.3.1.1.1.53-1: Test frequencies for NR operating band n53 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2486 | 497200 | 2483.75 | 496750 | 0 | 15 | 6215 | 497290 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2489.3 | 497860 | 2468.69 | 493738 | 102 |  | 6221 | 497770 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2492.5 | 498500 | 2399.53 | 479906 | 504 |  | 6232 | 498530 | 4 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink | Low | 2488.5 | 497700 | 2483.82 | 496764 | 0 | 15 | 6216 | 497310 | 2 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2489.3 | 497860 | 2466.6 | 493252 | 102 |  | 6215 | 497290 | 2 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2490 | 498000 | 2394.6 | 478920 | 504 |  | 6219 | 497550 | 6 | 1 | 1 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.53-2: Test frequencies for NR operating band n53 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2488.5 | 497700 | 2484.18 | 496836 | 0 | 30 | 6222 | 497790 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2489.3 | 497860 | 2448.26 | 489652 | 102 |  | 6221 | 497770 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2490 | 498000 | 2304.24 | 460848 | 504 |  | 6225 | 498030 | 10 | 0 | 2 (2) | 1012 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.53-3: Test frequencies for NR operating band n53 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2488.5 | 497700 | 2484.54 | 496908 | 0 | 15 | - | 497268 |
|  |  | & | Mid | 2489.3 | 497860 | 2411.9 | 482380 | 102 |  | - | 497428 |
|  |  | Uplink | High | 2490 | 498000 | 2123.16 | 424632 | 504 |  | - | 497568 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.54 to 4.3.1.1.1.64	FFS

###### 4.3.1.1.1.65	Reference test frequencies for NR operating band n65

Table 4.3.1.1.1.65-1: Test frequencies for NR operating band n65 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2134.39 | 426878 | 102 |  | 5389 | 431090 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 2197.5 | 439500 | 2104.53 | 420906 | 504 |  | 5493 | 439470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1872.03 | 374406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2007.5 | 401500 | 2004.17 | 400834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2131.96 | 426392 | 102 |  | 5383 | 430610 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 2195 | 439000 | 2099.6 | 419920 | 504 |  | 5480 | 438490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1869.6 | 373920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1999.24 | 399848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2129.53 | 425906 | 102 |  | 5377 | 430130 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 2192.5 | 438500 | 2094.67 | 418934 | 504 |  | 5470 | 437570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1867.17 | 373434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1994.31 | 398862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2155 | 431000 | 2127.1 | 425420 | 102 |  | 5371 | 429650 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 2190 | 438000 | 2089.74 | 417948 | 504 |  | 5457 | 436590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1864.74 | 372948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1989.38 | 397876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.65-2: Test frequencies for NR operating band n65 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2155 | 431000 | 2113.96 | 422792 | 102 |  | 5389 | 431090 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 2195 | 439000 | 2009.24 | 401848 | 504 |  | 5486 | 438970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1779.24 | 355848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1998.52 | 399704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2155 | 431000 | 2111.44 | 422288 | 102 |  | 5380 | 430370 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 2192.5 | 438500 | 2004.22 | 400844 | 504 |  | 5476 | 438050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1776.72 | 355344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1993.5 | 398700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2155 | 431000 | 2109.1 | 421820 | 102 |  | 5374 | 429890 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 2190 | 438000 | 1999.38 | 399876 | 504 |  | 5463 | 437070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1774.38 | 354876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1988.66 | 397732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.65-3: Test frequencies for NR operating band n65 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  | Mid | 2155 | 431000 | 2077.6 | 415520 | 102 |  | - | 430568 |
|  |  |  | High | 2195 | 439000 | 1828.16 | 365632 | 504 |  | - | 438568 |
|  |  | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1598.16 | 319632 | 504 |  | - | - |
|  |  |  | High | 2005 | 401000 | 1996.72 | 399344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  | Mid | 2155 | 431000 | 2075.08 | 415016 | 102 |  | - | 430064 |
|  |  |  | High | 2192.5 | 438500 | 1823.14 | 364628 | 504 |  | - | 437564 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1595.64 | 319128 | 504 |  | - | - |
|  |  |  | High | 2002.5 | 400500 | 1991.7 | 398340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  | Mid | 2155 | 431000 | 2072.92 | 414584 | 102 |  | - | 429632 |
|  |  |  | High | 2190 | 438000 | 1818.48 | 363696 | 504 |  | - | 436632 |
|  |  | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1593.48 | 318696 | 504 |  | - | - |
|  |  |  | High | 2000 | 400000 | 1987.04 | 397408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.66	Reference test frequencies for NR operating band n66

Table 4.3.1.1.1.66-1: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 15 kHz

| UL/DLCBW Combination (Asymmetric BCS) Note 3 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2124.39 | 424878 | 102 |  | 5361 | 428910 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  | 5443 | 435410 | 0 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2152.5 | 430500 | 2124.6 | 424920 | 102 |  | 5364 | 429150 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2084.74 | 416948 | 504 |  | 5446 | 435650 | 6 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2124.67 | 424934 | 102 |  | 5365 | 429170 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2187.5 | 437500 | 2084.81 | 416962 | 504 |  | 5444 | 435610 | 0 | 0 | 2 (4) | 508 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1742.5 | 348500 | 1649.53 | 329906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1769.17 | 353834 | 6 |  | - | - | - | - | - | - |
| 5/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1737.5 | 347500 | 1644.53 | 328906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1762.5 | 352500 | 1759.17 | 351834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2150 | 430000 | 2122.1 | 424420 | 102 |  | 5357 | 428650 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2180 | 436000 | 2079.74 | 415948 | 504 |  | 5432 | 434650 | 6 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2122.17 | 424434 | 102 |  | 5358 | 428670 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2182.5 | 436500 | 2079.81 | 415962 | 504 |  | 5433 | 434670 | 8 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1644.6 | 328920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1759.24 | 351848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  | 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2114.67 | 422934 | 102 |  | 5337 | 426990 | 8 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2167.5 | 433500 | 2064.81 | 412962 | 504 |  | 5394 | 431550 | 4 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2114.7 | 422940 | 102 |  | 5337 | 426990 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2064.84 | 412968 | 504 |  | 5394 | 431550 | 2 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2112.24 | 422448 | 102 |  | 5331 | 426510 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2165 | 433000 | 2059.88 | 411976 | 504 |  | 5381 | 430570 | 6 | 0 | 1 (2) | 506 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2112.2 | 422440 | 102 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2059.84 | 411968 | 504 |  | 5383 | 430610 | 10 | 1 | 1 (2) | 507 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  | 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
| N.A./40 (Note4) | 40 | 216 | Downlink | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
| 45/45 | 45 | 242 | Downlink | Low | 2132.5 | 426500 | 2110.72 | 422144 | 0 | 15 | 5284 | 422690 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2104.86 | 420972 | 102 |  | 5313 | 425070 | 10 | 1 | 0 (0) | 103 |
|  |  |  |  | High | 2157.5 | 431500 | 2045 | 409000 | 504 |  | 5345 | 427690 | 2 | 1 | 2 (4) | 509 |
|  | 45 | 242 | Uplink | Low | 1732.5 | 346500 | 1710.72 | 342144 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1632.5 | 326500 | 504 | - | - | - | - | - | - | - |
|  |  |  |  | High | 1757.5 | 351500 | 1734.64 | 346928 | 6 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. Note 4: Used only for non-contiguous CA x+40 combination and CA x+x+40 combination with no UL configuration on the SB in Table 4.3.1.1.5.66.1-1 and Table 4.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-1A: Void

Table 4.3.1.1.1.66-2: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 30 kHz

| UL/DLCBW Combination (Asymmetric BCS) Note 3 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  | 5361 | 428910 | 18 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  | 5436 | 434910 | 18 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2150 | 430000 | 2104.1 | 420820 | 102 |  | 5360 | 428890 | 2 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1989.38 | 397876 | 504 |  | 5435 | 434890 | 2 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2104.08 | 420816 | 102 |  | 5361 | 428910 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2182.5 | 436500 | 1989.36 | 397872 | 504 |  | 5436 | 434910 | 10 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2104.24 | 420848 | 102 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2185 | 437000 | 1989.52 | 397904 | 504 |  | 5437 | 434930 | 6 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1554.24 | 310848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1758.52 | 351704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  | 5355 | 428430 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  | 5423 | 433930 | 2 | 0 | 1 (6) | 1020 |
|  | 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  | 5349 | 427950 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  | 5413 | 433010 | 18 | 0 | 1 (6) | 1020 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2096.58 | 419316 | 102 |  | 5343 | 427470 | 6 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2167.5 | 433500 | 1974.36 | 394872 | 504 |  | 5400 | 432030 | 2 | 0 | 2 (7) | 1022 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2096.7 | 419340 | 102 |  | 5343 | 427470 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1974.48 | 394896 | 504 |  | 5400 | 432030 | 18 | 0 | 1 (6) | 1020 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2094.24 | 418848 | 102 |  | 5337 | 426990 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2165 | 433000 | 1969.52 | 393904 | 504 |  | 5387 | 431050 | 22 | 0 | 1 (6) | 1020 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2094.2 | 418840 | 102 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1969.48 | 393896 | 504 |  | 5389 | 431090 | 14 | 0 | 2 (7) | 1022 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2089.2 | 417840 | 102 |  | 5325 | 426030 | 18 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2160 | 432000 | 1959.48 | 391896 | 504 |  | 5361 | 428910 | 2 | 0 | 0 (5) | 1018 |
|  | 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |  | - | - | - | - | - | - |
| N.A./40 (Note4) | 40 | 106 | Downlink | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
| 45/45 | 45 | 119 | Downlink | Low | 2132.5 | 426500 | 2111.08 | 422216 | 0 | 15 | 5290 | 423170 | 6 | 0 | 3 (8) | 16 |
|  |  |  |  | Mid | 2145 | 429000 | 2086.86 | 417372 | 102 |  | 5319 | 425550 | 14 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2157.5 | 431500 | 1954.64 | 390928 | 504 |  | 5351 | 428170 | 6 | 0 | 3 (8) | 1024 |
|  | 45 | 119 | Uplink | Low | 1732.5 | 346500 | 1711.08 | 342216 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1542.14 | 308428 | 504 | - | - | - | - | - | - | - |
|  |  |  |  | High | 1757.5 | 351500 | 1733.92 | 346784 | 6 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. Note 4: Used only for non-contiguous CA x+40 combination and CA x+x+40 combination with no UL configuration on the SB in Table 4.3.1.1.5.66.1-2 and Table 4.3.1.1.5.66.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-2A: Void

Table 4.3.1.1.1.66-3: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 60 kHz without CORESET#0

| UL/DLCBW (Asymmetric BCS) Note 2 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  |  | Mid | 2145 | 429000 | 2067.6 | 413520 | 102 |  | - | 428568 |
|  |  |  |  | High | 2175 | 435000 | 1808.16 | 361632 | 504 |  | - | 434568 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/20 | 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  |  | Mid | 2150 | 430000 | 2067.92 | 413584 | 102 |  | - | 428632 |
|  |  |  |  | High | 2180 | 436000 | 1808.48 | 361696 | 504 |  | - | 434632 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/25 | 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2067.9 | 413580 | 102 |  | - | 428628 |
|  |  |  |  | High | 2182.5 | 436500 | 1808.46 | 361692 | 504 |  | - | 434628 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/30 | 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2067.88 | 413576 | 102 |  | - | 428624 |
|  |  |  |  | High | 2185 | 437000 | 1808.44 | 361688 | 504 |  | - | 434624 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2155 | 431000 | 2063.2 | 412640 | 102 |  | - | 427688 |
|  |  |  |  | High | 2180 | 436000 | 1798.76 | 359752 | 504 |  | - | 432688 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1373.16 | 274632 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1756.72 | 351344 | 6 |  | - | - |
| 15/15 | 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  |  | Mid | 2145 | 429000 | 2065.08 | 413016 | 102 |  | - | 428064 |
|  |  |  |  | High | 2172.5 | 434500 | 1803.14 | 360628 | 504 |  | - | 433564 |
|  | 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1375.64 | 275128 | 504 |  | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1761.7 | 352340 | 6 |  | - | - |
| 20/20 | 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  |  | Mid | 2145 | 429000 | 2062.92 | 412584 | 102 |  | - | 427632 |
|  |  |  |  | High | 2170 | 434000 | 1798.48 | 359696 | 504 |  | - | 432632 |
|  | 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |  | - | - |
|  |  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |  | - | - |
| 20/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2155 | 431000 | 2063.2 | 412640 | 102 |  | - | 427688 |
|  |  |  |  | High | 2180 | 436000 | 1798.76 | 359752 | 504 |  | - | 432688 |
|  | 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |  | - | - |
|  |  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |  | - | - |
| 25/25 | 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  |  | Mid | 2145 | 429000 | 2060.4 | 412080 | 102 |  | - | 427128 |
|  |  |  |  | High | 2167.5 | 433500 | 1793.46 | 358692 | 504 |  | - | 431628 |
|  | 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1370.96 | 274192 | 504 |  | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1752.02 | 350404 | 6 |  | - | - |
| 25/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2060.7 | 412140 | 102 |  | - | 427188 |
|  |  |  |  | High | 2175 | 435000 | 1793.76 | 358752 | 504 |  | - | 431688 |
|  | 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1370.96 | 274192 | 504 |  | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1752.02 | 350404 | 6 |  | - | - |
| 30/30 | 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  |  | Mid | 2145 | 429000 | 2057.88 | 411576 | 102 |  | - | 426624 |
|  |  |  |  | High | 2165 | 433000 | 1788.44 | 357688 | 504 |  | - | 430624 |
|  | 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1368.44 | 273688 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1747 | 349400 | 6 |  | - | - |
| 30/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2058.2 | 411640 | 102 |  | - | 426688 |
|  |  |  |  | High | 2170 | 434000 | 1788.76 | 357752 | 504 |  | - | 430688 |
|  | 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1368.44 | 273688 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1747 | 349400 | 6 |  | - | - |
| 40/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2145 | 429000 | 2053.2 | 410640 | 102 |  | - | 425688 |
|  |  |  |  | High | 2160 | 432000 | 1778.76 | 355752 | 504 |  | - | 428688 |
|  | 40 | 51 | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1363.76 | 272752 | 504 |  | - | - |
|  |  |  |  | High | 1760 | 352000 | 1737.32 | 347464 | 6 |  | - | - |
| 45/45 | 45 | 58 | Downlink | Low | 2132.5 | 426500 | 2111.62 | 422324 | 0 | 15 | - | 422684 |
|  |  |  |  | Mid | 2145 | 429000 | 2050.68 | 410136 | 102 | 15 | - | 425184 |
|  |  |  |  | High | 2157.5 | 431500 | 1773.74 | 354748 | 504 | 15 | - | 427684 |
|  | 45 | 58 | Uplink | Low | 1732.5 | 346500 | 1711.62 | 342324 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1361.24 | 272248 | 504 | - | - | - |
|  |  |  |  | High | 1757.5 | 351500 | 1732.3 | 346460 | 6 | - | - | - |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2 Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-3A: Void

###### 4.3.1.1.1.67 – 4.3.1.1.1.69	FFS

###### 4.3.1.1.1.70	Reference test frequencies for NR operating band n70

Table 4.3.1.1.1.70-1: Test frequencies for NR operating band n70 and SCS 15 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 1997.5 | 399500 | 1995.25 | 399050 | 0 | 15 | 4993 | 399410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1981.89 | 396378 | 102 |  | 5007 | 400590 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2007.5 | 401500 | 1914.53 | 382906 | 504 |  | 5018 | 401530 | 4 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1609.53 | 321906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1707.5 | 341500 | 1704.17 | 340834 | 6 |  | - | - | - | - | - | - |
| 5/15 | 15 | 79 | Downlink | Low | 2002.5 | 400500 | 1995.39 | 399078 | 0 | 15 | 4995 | 399630 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1982.03 | 396406 | 102 |  | 5006 | 400570 | 8 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2012.5 | 402500 | 1914.67 | 382934 | 504 |  | 5020 | 401570 | 8 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1609.53 | 321906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1707.5 | 341500 | 1704.17 | 340834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1979.6 | 395920 | 102 |  | 5000 | 400090 | 10 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2010 | 402000 | 1909.74 | 381948 | 504 |  | 5007 | 400590 | 10 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1700 | 340000 | 1607.03 | 321406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1702.5 | 340500 | 1699.17 | 339834 | 6 |  | - | - | - | - | - | - |
| 5/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 10/10 | 10 | 52 | Downlink | Low | 2000 | 400000 | 1995.32 | 399064 | 0 | 15 | 4994 | 399610 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1979.46 | 395892 | 102 |  | 5001 | 400110 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2005 | 401000 | 1909.6 | 381920 | 504 |  | 5008 | 400610 | 2 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1607.1 | 321420 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1699.24 | 339848 | 6 |  | - | - | - | - | - | - |
| 10/15 | 15 | 79 | Downlink | Low | 2002.5 | 400500 | 1995.39 | 399078 | 0 | 15 | 4995 | 399630 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2005 | 401000 | 1979.53 | 395906 | 102 |  | 5002 | 400130 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2007.5 | 401500 | 1909.67 | 381934 | 504 |  | 5006 | 400570 | 8 | 1 | 1 (2) | 507 |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1607.1 | 321420 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1699.24 | 339848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1979.6 | 395920 | 102 |  | 5000 | 400090 | 10 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2010 | 402000 | 1909.74 | 381948 | 504 |  | 5007 | 400590 | 10 | 1 | 1 (2) | 507 |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1607.1 | 321420 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1699.24 | 339848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 79 | Downlink | Low | 2002.5 | 400500 | 1995.39 | 399078 | 0 | 15 | 4995 | 399630 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.70-2: Test frequencies for NR operating band n70 and SCS 30 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 2000 | 400000 | 1995.68 | 399136 | 0 | 15 | 5000 | 400090 | 6 | 0 | 3 (8) | 16 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1961.46 | 392292 | 102 |  | 5007 | 400590 | 6 | 0 | 3 (8) | 220 |
|  |  |  |  | High | 2005 | 401000 | 1819.24 | 363848 | 504 |  | 5014 | 401090 | 6 | 0 | 3 (8) | 1024 |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1516.74 | 303348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1698.52 | 339704 | 6 |  | - | - | - | - | - | - |
| 10/15 | 15 | 38 | Downlink | Low | 2002.5 | 400500 | 1995.66 | 399132 | 0 | 15 | 4998 | 399870 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2005 | 401000 | 1961.44 | 392288 | 102 |  | 5005 | 400370 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2007.5 | 401500 | 1819.22 | 363844 | 504 |  | 5012 | 401050 | 18 | 0 | 2 (7) | 1022 |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1516.74 | 303348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1698.52 | 339704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 2005 | 401000 | 1995.82 | 399164 | 0 | 15 | 4999 | 399890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1961.6 | 392320 | 102 |  | 5006 | 400570 | 14 | 0 | 2 (7) | 218 |
|  |  |  |  | High | 2010 | 402000 | 1819.38 | 363876 | 504 |  | 5013 | 401070 | 14 | 0 | 2 (7) | 1022 |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1516.74 | 303348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1698.52 | 339704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 65 | Downlink | Low | 2007.5 | 401500 | 1995.8 | 399160 | 0 | 15 | 5000 | 400090 | 22 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 38 | Downlink | Low | 2002.5 | 400500 | 1995.66 | 399132 | 0 | 15 | 4998 | 399870 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 51 | Downlink | Low | 2005 | 401000 | 1995.82 | 399164 | 0 | 15 | 4999 | 399890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 65 | Downlink | Low | 2007.5 | 401500 | 1995.8 | 399160 | 0 | 15 | 5000 | 400090 | 22 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.70-3: Test frequencies for NR operating band n70 and SCS 60 kHz without CORESET#0

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 11 | Downlink | Low | 2000 | 400000 | 1996.04 | 399208 | 0 | 15 | - | 399568 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1925.1 | 385020 | 102 |  | - | 400068 |
|  |  |  |  | High | 2005 | 401000 | 1638.16 | 327632 | 504 |  | - | 400568 |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1335.66 | 267132 | 504 |  | - | - |
|  |  |  |  | High | 1705 | 341000 | 1696.72 | 339344 | 6 |  | - | - |
| 10/15 | 15 | 18 | Downlink | Low | 2002.5 | 400500 | 1996.02 | 399204 | 0 | 15 | - | 399564 |
|  |  |  |  | Mid | 2005 | 401000 | 1925.08 | 385016 | 102 |  | - | 400064 |
|  |  |  |  | High | 2007.5 | 401500 | 1638.14 | 327628 | 504 |  | - | 400564 |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1335.66 | 267132 | 504 |  | - | - |
|  |  |  |  | High | 1705 | 341000 | 1696.72 | 339344 | 6 |  | - | - |
| 10/20 | 20 | 24 | Downlink | Low | 2005 | 401000 | 1996.36 | 399272 | 0 | 15 | - | 399632 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1925.42 | 385084 | 102 |  | - | 400132 |
|  |  |  |  | High | 2010 | 402000 | 1638.48 | 327696 | 504 |  | - | 400632 |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1335.66 | 267132 | 504 |  | - | - |
|  |  |  |  | High | 1705 | 341000 | 1696.72 | 339344 | 6 |  | - | - |
| 10/25 | 25 | 31 | Downlink | Low | 2007.5 | 401500 | 1996.34 | 399268 | 0 | 15 | - | 399628 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 18 | Downlink | Low | 2002.5 | 400500 | 1996.02 | 399204 | 0 | 15 | - | 399564 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 24 | Downlink | Low | 2005 | 401000 | 1996.36 | 399272 | 0 | 15 | - | 399632 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 31 | Downlink | Low | 2007.5 | 401500 | 1996.34 | 399268 | 0 | 15 | - | 399628 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.71	Reference test frequencies for NR operating band n71

Table 4.3.1.1.1.71-1: Test frequencies for NR operating band n71 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 619.5 | 123900 | 617.25 | 123450 | 0 | 15 | 1548 | 123870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 613.89 | 122778 | 102 |  | 1587 | 126990 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 649.5 | 129900 | 556.53 | 111306 | 504 |  | 1623 | 129870 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 665.5 | 133100 | 663.25 | 132650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 587.53 | 117506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 695.5 | 139100 | 692.17 | 138434 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 622 | 124400 | 617.32 | 123464 | 0 | 15 | 1549 | 123890 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 611.46 | 122292 | 102 |  | 1581 | 126510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 647 | 129400 | 551.6 | 110320 | 504 |  | 1610 | 128890 | 10 | 1 | 0 (0) | 505 |
|  | 25 | Uplink | Low | 665.5 | 133100 | 663.25 | 132650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 585.03 | 117006 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 687.17 | 137434 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 622 | 124400 | 617.32 | 123464 | 0 | 15 | 1549 | 123890 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 611.46 | 122292 | 102 |  | 1581 | 126510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 647 | 129400 | 551.6 | 110320 | 504 |  | 1610 | 128890 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 668 | 133600 | 663.32 | 132664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 585.1 | 117020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 687.24 | 137448 | 6 |  | - | - | - | - | - | - |
| 10/15 | 79 | Downlink | Low | 624.5 | 124900 | 617.39 | 123478 | 0 | 15 | 1547 | 123850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 609.03 | 121806 | 102 |  | 1575 | 126030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 644.5 | 128900 | 546.67 | 109334 | 504 |  | 1600 | 127970 | 8 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low | 668 | 133600 | 663.32 | 132664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 582.6 | 116520 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 682.24 | 136448 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 624.5 | 124900 | 617.39 | 123478 | 0 | 15 | 1547 | 123850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 609.03 | 121806 | 102 |  | 1575 | 126030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 644.5 | 128900 | 546.67 | 109334 | 504 |  | 1600 | 127970 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.39 | 132678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 582.67 | 116534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 682.31 | 136462 | 6 |  | - | - | - | - | - | - |
| 15/20 | 106 | Downlink | Low | 627 | 125400 | 617.46 | 123492 | 0 | 15 | 1548 | 123870 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 606.6 | 121320 | 102 |  | 1569 | 125550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 642 | 128400 | 541.74 | 108348 | 504 |  | 1587 | 126990 | 10 | 1 | 1 (2) | 507 |
|  | 79 | Uplink | Low | 670.5 | 134100 | 663.39 | 132678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 580.17 | 116034 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 677.31 | 135462 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 627 | 125400 | 617.46 | 123492 | 0 | 15 | 1548 | 123870 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 606.6 | 121320 | 102 |  | 1569 | 125550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 642 | 128400 | 541.74 | 108348 | 504 |  | 1587 | 126990 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 673 | 134600 | 663.46 | 132692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 677.38 | 135476 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 629.5 | 125900 | 617.53 | 123506 | 0 | 15 | 1549 | 123890 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 604.17 | 120834 | 102 |  | 1563 | 125070 | 8 | 1 | 2 (4) | 107 |
|  |  |  | High | 639.5 | 127900 | 536.81 | 107362 | 504 |  | 1574 | 126010 | 0 | 0 | 2 (4) | 508 |
|  | 106 | Uplink | Low | 675.5 | 135100 | 665.96 | 133192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 674.88 | 134976 | 6 |  | - | - | - | - | - | - |
| 20/30 | 160 | Downlink | Low | 632 | 126400 | 617.6 | 123520 | 0 | 15 | 1550 | 124090 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 634.5 | 126900 | 601.74 | 120348 | 102 |  | 1557 | 124590 | 10 | 1 | 2 (4) | 107 |
|  |  |  | High | 637 | 127400 | 531.88 | 106376 | 504 |  | 1564 | 125090 | 10 | 1 | 2 (4) | 509 |
|  | 106 | Uplink | Low | 678 | 135600 | 668.46 | 133692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 683 | 136600 | 672.38 | 134476 | 6 |  | - | - | - | - | - | - |
| 20/35 | 188 | Downlink | Low | 634.5 | 126900 | 617.58 | 123516 | 0 | 15 | 1551 | 124110 | 6 | 2 | 2 (4) | 6 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 680.5 | 136100 | 670.96 | 134192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.71-2: Test frequencies for NR operating band n71 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 622 | 124400 | 617.68 | 123536 | 0 | 15 | 1555 | 124370 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 593.46 | 118692 | 102 |  | 1587 | 126990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 647 | 129400 | 461.24 | 92248 | 504 |  | 1616 | 129370 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 668 | 133600 | 663.68 | 132736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 494.74 | 98948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 686.52 | 137304 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 624.5 | 124900 | 617.66 | 123532 | 0 | 15 | 1553 | 124330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 590.94 | 118188 | 102 |  | 1578 | 126270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 644.5 | 128900 | 456.22 | 91244 | 504 |  | 1606 | 128450 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 668 | 133600 | 663.68 | 132736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 492.24 | 98448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 681.52 | 136304 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 624.5 | 124900 | 617.66 | 123532 | 0 | 15 | 1553 | 124330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 590.94 | 118188 | 102 |  | 1578 | 126270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 644.5 | 128900 | 456.22 | 91244 | 504 |  | 1606 | 128450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.66 | 132732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 492.22 | 98444 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 681.5 | 136300 | 6 |  | - | - | - | - | - | - |
| 15/20 | 51 | Downlink | Low | 627 | 125400 | 617.82 | 123564 | 0 | 15 | 1554 | 124350 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 588.6 | 117720 | 102 |  | 1572 | 125790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 642 | 128400 | 451.38 | 90276 | 504 |  | 1593 | 127470 | 14 | 0 | 2 (7) | 1022 |
|  | 38 | Uplink | Low | 670.5 | 134100 | 663.66 | 132732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 489.72 | 97944 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 676.5 | 135300 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 627 | 125400 | 617.82 | 123564 | 0 | 15 | 1554 | 124350 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 588.6 | 117720 | 102 |  | 1572 | 125790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 642 | 128400 | 451.38 | 90276 | 504 |  | 1593 | 127470 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 673 | 134600 | 663.82 | 132764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 676.66 | 135332 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 629.5 | 125900 | 617.8 | 123560 | 0 | 15 | 1555 | 124370 | 6 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 586.08 | 117216 | 102 |  | 1566 | 125310 | 10 | 0 | 0 (5) | 214 |
|  |  |  | High | 639.5 | 127900 | 446.36 | 89272 | 504 |  | 1580 | 126490 | 22 | 0 | 2 (7) | 1022 |
|  | 51 | Uplink | Low | 675.5 | 135100 | 666.32 | 133264 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 674.16 | 134832 | 6 |  | - | - | - | - | - | - |
| 20/30 | 78 | Downlink | Low | 632 | 126400 | 617.96 | 123592 | 0 | 15 | 1553 | 124330 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 583.74 | 116748 | 102 |  | 1560 | 124830 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 637 | 127400 | 441.52 | 88304 | 504 |  | 1567 | 125330 | 6 | 0 | 0 (5) | 1018 |
|  | 51 | Uplink | Low | 678 | 135600 | 668.82 | 133764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 683 | 136600 | 671.66 | 134332 | 6 |  | - | - | - | - | - | - |
| 20/35 | 92 | Downlink | Low | 634.5 | 126900 | 617.94 | 123588 | 0 | 15 | 1554 | 124350 | 14 | 0 | 0 (5) | 10 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 680.5 | 136100 | 671.32 | 134264 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.72 – 4.3.1.1.1.73

###### 4.3.1.1.1.74	Reference test frequencies for NR operating band n74

Table 4.3.1.1.1.74-1: Test frequencies for NR operating band n74 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1477.5 | 295500 | 1475.25 | 295050 | 0 | 15 | 3693 | 295470 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1496.5 | 299300 | 1475.89 | 295178 | 102 |  | 3742 | 299330 | 4 | 1 | 1 (2) | 105 |
|  |  |  | High | 1515.5 | 303100 | 1422.53 | 284506 | 504 |  | 3788 | 303130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1355.53 | 271106 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1467.5 | 293500 | 1464.17 | 292834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1480 | 296000 | 1475.32 | 295064 | 0 | 15 | 3694 | 295490 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1496.5 | 299300 | 1473.46 | 294692 | 102 |  | 3736 | 298850 | 6 | 1 | 1 (2) | 105 |
|  |  |  | High | 1513 | 302600 | 1417.6 | 283520 | 504 |  | 3778 | 302210 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1432 | 286400 | 1427.32 | 285464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1353.1 | 270620 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1465 | 293000 | 1459.24 | 291848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1482.5 | 296500 | 1475.39 | 295078 | 0 | 15 | 3692 | 295450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1496.5 | 299300 | 1471.03 | 294206 | 102 |  | 3730 | 298370 | 8 | 1 | 1 (2) | 105 |
|  |  |  | High | 1510.5 | 302100 | 1412.67 | 282534 | 504 |  | 3765 | 301230 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1427.39 | 285478 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1350.67 | 270134 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1462.5 | 292500 | 1454.31 | 290862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1485 | 297000 | 1475.46 | 295092 | 0 | 15 | 3693 | 295470 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1496.5 | 299300 | 1468.6 | 293720 | 102 |  | 3724 | 297890 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 1508 | 301600 | 1407.74 | 281548 | 504 |  | 3752 | 300250 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1437 | 287400 | 1427.46 | 285492 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1348.24 | 269648 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1460 | 292000 | 1449.38 | 289876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.74-2: Test frequencies for NR operating band n74 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1480 | 296000 | 1475.68 | 295136 | 0 | 15 | 3700 | 295970 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1496.5 | 299300 | 1455.46 | 291092 | 102 |  | 3742 | 299330 | 10 | 0 | 2 (7) | 218 |
|  |  |  | High | 1513 | 302600 | 1327.24 | 265448 | 504 |  | 3784 | 302690 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1432 | 286400 | 1427.68 | 285536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1262.74 | 252548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1465 | 293000 | 1458.52 | 291704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1482.5 | 296500 | 1475.66 | 295132 | 0 | 15 | 3698 | 295930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1496.5 | 299300 | 1452.94 | 290588 | 102 |  | 3736 | 298850 | 18 | 0 | 2 (7) | 218 |
|  |  |  | High | 1510.5 | 302100 | 1322.22 | 264444 | 504 |  | 3768 | 301470 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1427.66 | 285532 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1260.22 | 252044 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1462.5 | 292500 | 1453.5 | 290700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1485 | 297000 | 1475.82 | 295164 | 0 | 15 | 3699 | 295950 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1496.5 | 299300 | 1450.6 | 290120 | 102 |  | 3730 | 298370 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 1508 | 301600 | 1317.38 | 263476 | 504 |  | 3755 | 300490 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1437 | 287400 | 1427.82 | 285564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1257.88 | 251576 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1460 | 292000 | 1448.66 | 289732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.74-3: Test frequencies for NR operating band n74 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1480 | 296000 | 1476.04 | 295208 | 0 | 15 | - | 295568 |
|  |  |  | Mid | 1496.5 | 299300 | 1419.1 | 283820 | 102 |  | - | 298868 |
|  |  |  | High | 1513 | 302600 | 1146.16 | 229232 | 504 |  | - | 302168 |
|  |  | Uplink | Low | 1432 | 286400 | 1428.04 | 285608 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1081.66 | 216332 | 504 |  | - | - |
|  |  |  | High | 1465 | 293000 | 1456.72 | 291344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1482.5 | 296500 | 1476.02 | 295204 | 0 | 15 | - | 295564 |
|  |  |  | Mid | 1496.5 | 299300 | 1416.58 | 283316 | 102 |  | - | 298364 |
|  |  |  | High | 1510.5 | 302100 | 1141.14 | 228228 | 504 |  | - | 301164 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1428.02 | 285604 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1079.14 | 215828 | 504 |  | - | - |
|  |  |  | High | 1462.5 | 292500 | 1451.7 | 290340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1485 | 297000 | 1476.36 | 295272 | 0 | 15 | - | 295632 |
|  |  |  | Mid | 1496.5 | 299300 | 1414.42 | 282884 | 102 |  | - | 297932 |
|  |  |  | High | 1508 | 301600 | 1136.48 | 227296 | 504 |  | - | 300232 |
|  |  | Uplink | Low | 1437 | 287400 | 1428.36 | 285672 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1076.98 | 215396 | 504 |  | - | - |
|  |  |  | High | 1460 | 292000 | 1447.04 | 289408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.75	Reference test frequencies for NR operating band n75 (SDL)

Table 4.3.1.1.1.75-1: Test frequencies for NR operating band n75 and SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | - | 286810 |
|  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | - | 294810 |
|  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | - | 302810 |
| 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | - | 286824 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | - | 294324 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | - | 301824 |
| 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | - | 286838 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | - | 293838 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | - | 300838 |
| 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | - | 286852 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | - | 293352 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | - | 299852 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.75-2: Test frequencies for NR operating band n75 and SCS 30 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | - | 286896 |
|  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | - | 294396 |
|  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | - | 301896 |
| 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | - | 286892 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | - | 293892 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | - | 300892 |
| 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | - | 286924 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | - | 293424 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | - | 299924 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.75-3: Test frequencies for NR operating band n75 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1437 | 287400 | 1433.04 | 286608 | 0 | 15 | - | 286968 |
|  |  |  | Mid | 1474.5 | 294900 | 1397.1 | 279420 | 102 |  | - | 294468 |
|  |  |  | High | 1512 | 302400 | 1145.16 | 229032 | 504 |  | - | 301968 |
| 15 | 18 | Downlink | Low | 1439.5 | 287900 | 1433.02 | 286604 | 0 | 15 | - | 286964 |
|  |  |  | Mid | 1474.5 | 294900 | 1394.58 | 278916 | 102 |  | - | 293964 |
|  |  |  | High | 1509.5 | 301900 | 1140.14 | 228028 | 504 |  | - | 300964 |
| 20 | 24 | Downlink | Low | 1442 | 288400 | 1433.36 | 286672 | 0 | 15 | - | 287032 |
|  |  |  | Mid | 1474.5 | 294900 | 1392.42 | 278484 | 102 |  | - | 293532 |
|  |  |  | High | 1507 | 301400 | 1135.48 | 227096 | 504 |  | - | 300032 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.76	Reference test frequencies for NR operating band n76 (SDL)

Table 4.3.1.1.1.76-1: Test frequencies for NR operating band n76 and SCS 15 kHz without CORESET#0

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low, Mid, High | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | - | 285810 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.77	Reference test frequencies for NR operating band n77

Table 4.3.1.1.1.77-1: Test frequencies for NR operating band n77, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 3305.01 | 620334 | 3300.33 | 620022 | 0 | 30 | 7711 | 620352 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3750 | 650000 | 3726.96 | 648464 | 102 |  | 8020 | 650016 | 4 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 4194.99 | 679666 | 4099.59 | 673306 | 504 |  | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 15 | 79 | Downlink | Low | 3307.5 | 620500 | 3300.39 | 620026 | 0 | 30 | 7711 | 620352 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3750 | 650000 | 3724.53 | 648302 | 102 |  | 8018 | 649824 | 10 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 4192.5 | 679500 | 4094.67 | 672978 | 504 |  | 8325 | 679296 | 6 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 3310.02 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3722.1 | 648140 | 102 |  | 8016 | 649632 | 4 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 4189.98 | 679332 | 4089.72 | 672648 | 504 |  | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 25 | 133 | Downlink | Low | 3312.51 | 620834 | 3300.54 | 620036 | 0 | 30 | 7711 | 620352 | 4 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3719.67 | 647978 | 102 |  | 8015 | 649536 | 10 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 4187.49 | 679166 | 4084.8 | 672320 | 504 |  | 8319 | 678720 | 4 | 3 | 1 (6) | 513 |
| 30 | 160 | Downlink | Low | 3315 | 621000 | 3300.6 | 620040 | 0 | 30 | 7711 | 620352 | 0 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3717.24 | 647816 | 102 |  | 8013 | 649344 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 4185 | 679000 | 4079.88 | 671992 | 504 |  | 8315 | 678336 | 8 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3712.2 | 647480 | 102 |  | 8010 | 649056 | 4 | 3 | 1 (6) | 111 |
|  |  | Uplink | High | 4179.99 | 678666 | 4069.83 | 671322 | 504 |  | 8308 | 677664 | 6 | 2 | 0 (2) | 508 |
| 50 | 270 | Downlink | Low | 3325.02 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 3750 | 650000 | 3707.34 | 647156 | 102 |  | 8006 | 648672 | 4 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 4174.98 | 678332 | 4059.96 | 670664 | 504 |  | 8301 | 676992 | 4 | 1 | 0 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.77-2: Test frequencies for NR operating band n77, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 | 7711 | 620352 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3708.96 | 647264 | 102 |  | 8020 | 650016 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 4194.99 | 679666 | 4009.23 | 667282 | 504 |  | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 15 | 38 | Downlink | Low | 3307.5 | 620500 | 3300.66 | 620044 | 0 | 30 | 7711 | 620352 | 20 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3706.44 | 647096 | 102 |  | 8018 | 649824 | 16 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4192.5 | 679500 | 4004.22 | 666948 | 504 |  | 8325 | 679296 | 12 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3704.1 | 646940 | 102 |  | 8016 | 649632 | 4 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 4189.98 | 679332 | 3999.36 | 666624 | 504 |  | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 25 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 | 7711 | 620352 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3701.58 | 646772 | 102 |  | 8015 | 649536 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4187.49 | 679166 | 3994.35 | 666290 | 504 |  | 8319 | 678720 | 22 | 0 | 3 (3) | 1014 |
| 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3699.24 | 646616 | 102 |  | 8013 | 649344 | 16 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4185 | 679000 | 3989.52 | 665968 | 504 |  | 8315 | 678336 | 8 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3694.2 | 646280 | 102 |  | 8010 | 649056 | 16 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4179.99 | 678666 | 3979.47 | 665298 | 504 |  | 8308 | 677664 | 6 | 0 | 1 (1) | 1010 |
| 50 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 | 7711 | 620352 | 16 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3750 | 650000 | 3689.34 | 645956 | 102 |  | 8006 | 648672 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4174.98 | 678332 | 3969.6 | 664640 | 504 |  | 8301 | 676992 | 16 | 0 | 0 (0) | 1008 |
| 60 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3684.12 | 645608 | 102 |  | 8003 | 648384 | 16 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4170 | 678000 | 3959.4 | 663960 | 504 |  | 8294 | 676320 | 0 | 0 | 1 (1) | 1010 |
| 70 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 | 7711 | 620352 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3750 | 650000 | 3679.26 | 645284 | 102 |  | 7999 | 648000 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4164.99 | 677666 | 3949.53 | 663302 | 504 |  | 8287 | 675648 | 10 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3674.22 | 644948 | 102 |  | 7996 | 647712 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4159.98 | 677332 | 3939.48 | 662632 | 504 |  | 8280 | 674976 | 8 | 0 | 0 (0) | 1008 |
| 90 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 | 7711 | 620352 | 4 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3669.18 | 644612 | 102 |  | 7992 | 647328 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4155 | 677000 | 3929.46 | 661964 | 504 |  | 8273 | 674304 | 4 | 0 | 0 (0) | 1008 |
| 100 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3664.14 | 644276 | 102 |  | 7989 | 647040 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4149.99 | 676666 | 3919.41 | 661294 | 504 |  | 8266 | 673632 | 2 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.77-3: Test frequencies for NR operating band n77, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3305.01 | 620334 | 3301.05 | 620070 | 0 | 30 | - | 620310 |
|  |  | & | Mid | 3750 | 650000 | 3672.6 | 644840 | 102 |  | - | 649976 |
|  |  | Uplink | High | 4194.99 | 679666 | 3828.15 | 655210 | 504 |  | - | 679642 |
| 15 | 18 | Downlink | Low | 3307.5 | 620500 | 3301.02 | 620068 | 0 | 30 | - | 620308 |
|  |  | & | Mid | 3750 | 650000 | 3670.08 | 644672 | 102 |  | - | 649808 |
|  |  | Uplink | High | 4192.5 | 679500 | 3823.14 | 654876 | 504 |  | - | 679308 |
| 20 | 24 | Downlink | Low | 3310.02 | 620668 | 3301.38 | 620092 | 0 | 30 | - | 620332 |
|  |  | & | Mid | 3750 | 650000 | 3667.92 | 644528 | 102 |  | - | 649664 |
|  |  | Uplink | High | 4189.98 | 679332 | 3818.46 | 654564 | 504 |  | - | 678996 |
| 25 | 31 | Downlink | Low | 3312.51 | 620834 | 3301.35 | 620090 | 0 | 30 | - | 620330 |
|  |  | & | Mid | 3750 | 650000 | 3665.4 | 644360 | 102 |  | - | 649496 |
|  |  | Uplink | High | 4187.49 | 679166 | 3813.45 | 654230 | 504 |  | - | 678662 |
| 30 | 38 | Downlink | Low | 3315 | 621000 | 3301.32 | 620088 | 0 | 30 | - | 620328 |
|  |  | & | Mid | 3750 | 650000 | 3662.88 | 644192 | 102 |  | - | 649328 |
|  |  | Uplink | High | 4185 | 679000 | 3808.44 | 653896 | 504 |  | - | 678328 |
| 40 | 51 | Downlink | Low | 3320.01 | 621334 | 3301.65 | 620110 | 0 | 30 | - | 620350 |
|  |  | & | Mid | 3750 | 650000 | 3658.2 | 643880 | 102 |  | - | 649016 |
|  |  | Uplink | High | 4179.99 | 678666 | 3798.75 | 653250 | 504 |  | - | 677682 |
| 50 | 65 | Downlink | Low | 3325.02 | 621668 | 3301.62 | 620108 | 0 | 30 | - | 620348 |
|  |  | & | Mid | 3750 | 650000 | 3653.16 | 643544 | 102 |  | - | 648680 |
|  |  | Uplink | High | 4174.98 | 678332 | 3788.7 | 652580 | 504 |  | - | 677012 |
| 60 | 79 | Downlink | Low | 3330 | 622000 | 3301.56 | 620104 | 0 | 30 | - | 620344 |
|  |  | & | Mid | 3750 | 650000 | 3648.12 | 643208 | 102 |  | - | 648344 |
|  |  | Uplink | High | 4170 | 678000 | 3778.68 | 651912 | 504 |  | - | 676344 |
| 70 | 93 | Downlink | Low | 3335.01 | 622334 | 3301.53 | 620102 | 0 | 30 | - | 620342 |
|  |  | & | Mid | 3750 | 650000 | 3643.08 | 642872 | 102 |  | - | 648008 |
|  |  | Uplink | High | 4164.99 | 677666 | 3768.63 | 651242 | 504 |  | - | 675674 |
| 80 | 107 | Downlink | Low | 3340.02 | 622668 | 3301.5 | 620100 | 0 | 30 | - | 620340 |
|  |  | & | Mid | 3750 | 650000 | 3638.04 | 642536 | 102 |  | - | 647672 |
|  |  | Uplink | High | 4159.98 | 677332 | 3758.58 | 650572 | 504 |  | - | 675004 |
| 90 | 121 | Downlink | Low | 3345 | 623000 | 3301.44 | 620096 | 0 | 30 | - | 620336 |
|  |  | & | Mid | 3750 | 650000 | 3633 | 642200 | 102 |  | - | 647336 |
|  |  | Uplink | High | 4155 | 677000 | 3748.56 | 649904 | 504 |  | - | 674336 |
| 100 | 135 | Downlink | Low | 3350.01 | 623334 | 3301.41 | 620094 | 0 | 30 | - | 620334 |
|  |  | & | Mid | 3750 | 650000 | 3627.96 | 641864 | 102 |  | - | 647000 |
|  |  | Uplink | High | 4149.99 | 676666 | 3738.51 | 649234 | 504 |  | - | 673666 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.78	Reference test frequencies for NR operating band n78

Table 4.3.1.1.1.78-1: Test frequencies for NR operating band n78, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 3305.01 | 620334 | 3300.33 | 620022 | 0 | 30 | 7711 | 620352 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3549.99 | 636666 | 3526.95 | 635130 | 102 |  | 7881 | 636672 | 6 | 0 | 1 (6) | 108 |
|  |  | Uplink | High | 3795 | 653000 | 3699.6 | 646640 | 504 |  | 8051 | 652992 | 4 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 3307.5 | 620500 | 3300.39 | 620026 | 0 | 30 | 7711 | 620352 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3549.99 | 636666 | 3524.52 | 634968 | 102 |  | 7879 | 636480 | 0 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 3792.48 | 652832 | 3694.65 | 646310 | 504 |  | 8048 | 652704 | 10 | 2 | 1 (6) | 512 |
| 20 | 106 | Downlink | Low | 3310.02 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3522.09 | 634806 | 102 |  | 7878 | 636384 | 6 | 3 | 1 (6) | 111 |
|  |  | Uplink | High | 3789.99 | 652666 | 3689.73 | 645982 | 504 |  | 8044 | 652320 | 2 | 2 | 0 (2) | 508 |
| 25 | 133 | Downlink | Low | 3312.51 | 620834 | 3300.54 | 620036 | 0 | 30 | 7711 | 620352 | 4 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3519.66 | 634644 | 102 |  | 7876 | 636192 | 0 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 3787.5 | 652500 | 3684.81 | 645654 | 504 |  | 8041 | 652032 | 6 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 3315 | 621000 | 3300.6 | 620040 | 0 | 30 | 7711 | 620352 | 0 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3517.23 | 634482 | 102 |  | 7874 | 636000 | 6 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 3784.98 | 652332 | 3679.86 | 645324 | 504 |  | 8037 | 651648 | 0 | 1 | 0 (2) | 507 |
| 40 | 216 | Downlink | Low | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3512.19 | 634146 | 102 |  | 7871 | 635712 | 6 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 3780 | 652000 | 3669.84 | 644656 | 504 |  | 8030 | 650976 | 8 | 0 | 0 (2) | 506 |
| 50 | 270 | Downlink | Low | 3325.02 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 3549.99 | 636666 | 3507.33 | 633822 | 102 |  | 7867 | 635328 | 6 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3774.99 | 651666 | 3659.97 | 643998 | 504 |  | 8024 | 650400 | 6 | 3 | 1 (6) | 513 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.78-2: Test frequencies for NR operating band n78, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 | 7711 | 620352 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3508.95 | 633930 | 102 |  | 7881 | 636672 | 6 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3795 | 653000 | 3609.24 | 640616 | 504 |  | 8051 | 652992 | 16 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 3307.5 | 620500 | 3300.66 | 620044 | 0 | 30 | 7711 | 620352 | 20 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3506.43 | 633762 | 102 |  | 7879 | 636480 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3792.48 | 652832 | 3604.2 | 640280 | 504 |  | 8048 | 652704 | 16 | 0 | 3 (3) | 1014 |
| 20 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3504.09 | 633606 | 102 |  | 7878 | 636384 | 18 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3789.99 | 652666 | 3599.37 | 639958 | 504 |  | 8044 | 652320 | 2 | 0 | 1 (1) | 1010 |
| 25 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 | 7711 | 620352 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3501.57 | 633438 | 102 |  | 7876 | 636192 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3787.5 | 652500 | 3594.36 | 639624 | 504 |  | 8041 | 652032 | 0 | 0 | 3 (3) | 1014 |
| 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3499.23 | 633282 | 102 |  | 7874 | 636000 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3784.98 | 652332 | 3589.5 | 639300 | 504 |  | 8037 | 651648 | 12 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3494.19 | 632946 | 102 |  | 7871 | 635712 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3780 | 652000 | 3579.48 | 638632 | 504 |  | 8030 | 650976 | 8 | 0 | 0 (0) | 1008 |
| 50 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 | 7711 | 620352 | 16 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3549.99 | 636666 | 3489.33 | 632622 | 102 |  | 7867 | 635328 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  | 8024 | 650400 | 18 | 0 | 3 (3) | 1014 |
| 60 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3484.11 | 632274 | 102 |  | 7864 | 635040 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  | 8016 | 649632 | 4 | 0 | 0 (0) | 1008 |
| 70 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 | 7711 | 620352 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3549.99 | 636666 | 3479.25 | 631950 | 102 |  | 7860 | 634656 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 |  | 8010 | 649056 | 12 | 0 | 3 (3) | 1014 |
| 80 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3474.21 | 631614 | 102 |  | 7857 | 634368 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  | 8003 | 648384 | 10 | 0 | 3 (3) | 1014 |
| 90 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 | 7711 | 620352 | 4 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3469.17 | 631278 | 102 |  | 7853 | 633984 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 |  | 7996 | 647712 | 8 | 0 | 3 (3) | 1014 |
| 100 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3464.13 | 630942 | 102 |  | 7850 | 633696 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  | 7989 | 647040 | 4 | 0 | 3 (3) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.78-3: Test frequencies for NR operating band n78, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0.

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3305.01 | 620334 | 3301.05 | 620070 | 0 | 30 | - | 620310 |
|  |  | & | Mid | 3549.99 | 636666 | 3472.59 | 631506 | 102 |  | - | 636642 |
|  |  | Uplink | High | 3795 | 653000 | 3428.16 | 628544 | 504 |  | - | 652976 |
| 15 | 18 | Downlink | Low | 3307.5 | 620500 | 3301.02 | 620068 | 0 | 30 | - | 620308 |
|  |  | & | Mid | 3549.99 | 636666 | 3470.07 | 631338 | 102 |  | - | 636474 |
|  |  | Uplink | High | 3792.48 | 652832 | 3423.12 | 628208 | 504 |  | - | 652640 |
| 20 | 24 | Downlink | Low | 3310.02 | 620668 | 3301.38 | 620092 | 0 | 30 | - | 620332 |
|  |  | & | Mid | 3549.99 | 636666 | 3467.91 | 631194 | 102 |  | - | 636330 |
|  |  | Uplink | High | 3789.99 | 652666 | 3418.47 | 627898 | 504 |  | - | 652330 |
| 25 | 31 | Downlink | Low | 3312.51 | 620834 | 3301.35 | 620090 | 0 | 30 | - | 620330 |
|  |  | & | Mid | 3549.99 | 636666 | 3465.39 | 631026 | 102 |  | - | 636162 |
|  |  | Uplink | High | 3787.5 | 652500 | 3413.46 | 627564 | 504 |  | - | 651996 |
| 30 | 38 | Downlink | Low | 3315 | 621000 | 3301.32 | 620088 | 0 | 30 | - | 620328 |
|  |  | & | Mid | 3549.99 | 636666 | 3462.87 | 630858 | 102 |  | - | 635994 |
|  |  | Uplink | High | 3784.98 | 652332 | 3408.42 | 627228 | 504 |  | - | 651660 |
| 40 | 51 | Downlink | Low | 3320.01 | 621334 | 3301.65 | 620110 | 0 | 30 | - | 620350 |
|  |  | & | Mid | 3549.99 | 636666 | 3458.19 | 630546 | 102 |  | - | 635682 |
|  |  | Uplink | High | 3780 | 652000 | 3398.76 | 626584 | 504 |  | - | 651016 |
| 50 | 65 | Downlink | Low | 3325.02 | 621668 | 3301.62 | 620108 | 0 | 30 | - | 620348 |
|  |  | & | Mid | 3549.99 | 636666 | 3453.15 | 630210 | 102 |  | - | 635346 |
|  |  | Uplink | High | 3774.99 | 651666 | 3388.71 | 625914 | 504 |  | - | 650346 |
| 60 | 79 | Downlink | Low | 3330 | 622000 | 3301.56 | 620104 | 0 | 30 | - | 620344 |
|  |  | & | Mid | 3549.99 | 636666 | 3448.11 | 629874 | 102 |  | - | 635010 |
|  |  | Uplink | High | 3769.98 | 651332 | 3378.66 | 625244 | 504 |  | - | 649676 |
| 70 | 93 | Downlink& Uplink | Low | 3335.01 | 622334 | 3301.53 | 620102 | 0 | 30 |  | 620342 |
|  |  |  | Mid | 3549.99 | 636666 | 3443.07 | 629538 | 102 |  |  | 634674 |
|  |  |  | High | 3765 | 651000 | 3368.64 | 624576 | 504 |  |  | 649008 |
| 80 | 107 | Downlink | Low | 3340.02 | 622668 | 3301.5 | 620100 | 0 | 30 | - | 620340 |
|  |  | & | Mid | 3549.99 | 636666 | 3438.03 | 629202 | 102 |  | - | 634338 |
|  |  | Uplink | High | 3759.99 | 650666 | 3358.59 | 623906 | 504 |  | - | 648338 |
| 90 | 121 | Downlink | Low | 3345 | 623000 | 3301.44 | 620096 | 0 | 30 | - | 620336 |
|  |  | & | Mid | 3549.99 | 636666 | 3432.99 | 628866 | 102 |  | - | 634002 |
|  |  | Uplink | High | 3754.98 | 650332 | 3348.54 | 623236 | 504 |  | - | 647668 |
| 100 | 135 | Downlink | Low | 3350.01 | 623334 | 3301.41 | 620094 | 0 | 30 | - | 620334 |
|  |  | & | Mid | 3549.99 | 636666 | 3427.95 | 628530 | 102 |  | - | 633666 |
|  |  | Uplink | High | 3750 | 650000 | 3338.52 | 622568 | 504 |  | - | 647000 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.79	Reference test frequencies for NR operating band n79

Table 4.3.1.1.1.79-1: Test frequencies for NR operating band n79, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 103 | 52 | Downlink | Low | 4405.02 | 693668 | 4400.34 | 693356 | 0 | 30 | 8475 | 693696 | 4 | 4 | 0 (4) | 8 |
|  |  | & | Mid (Note 4) | 4700.13 | 713342 | 4677.09 | 711806 | 102 |  | 8680 | 713376 | 10 | 4 | 0 (4) | 110 |
|  |  | Uplink | High (Note 4) | 4994.76 | 732984 | 4899.36 | 726624 | 504 |  | 8884 | 732960 | 0 | 0 | 0 (4) | 508 |
| 20 | 106 | Downlink | Low | 4410 | 694000 | 4400.46 | 693364 | 0 | 30 | 8475 | 693696 | 8 | 3 | 0 (4) | 7 |
|  |  | & | Mid | 4699.98 | 713332 | 4672.08 | 711472 | 102 |  | 8676 | 712992 | 8 | 0 | 0 (4) | 106 |
|  |  | Uplink | High | 4989.99 | 732666 | 4889.73 | 725982 | 504 |  | 8878 | 732384 | 6 | 5 | 0 (4) | 513 |
| 40 | 216 | Downlink | Low | 4420.02 | 694668 | 4400.58 | 693372 | 0 | 30 | 8480 | 694176 | 0 | 43 | 0 (4) | 47 |
|  |  | & | Mid | 4699.98 | 713332 | 4662.18 | 710812 | 102 |  | 8672 | 712608 | 8 | 23 | 0 (4) | 129 |
|  |  | Uplink | High | 4980 | 732000 | 4869.84 | 724656 | 504 |  | 8864 | 731040 | 0 | 4 | 0 (4) | 512 |
| 50 | 270 | Downlink | Low | 4425 | 695000 | 4400.7 | 693380 | 0 | 30 | 8480 | 694176 | 4 | 42 | 0 (4) | 46 |
|  |  | & | Mid | 4699.98 | 713332 | 4657.32 | 710488 | 102 |  | 8672 | 712608 | 8 | 50 | 0 (4) | 156 |
|  |  | Uplink | High | 4974.99 | 731666 | 4859.97 | 723998 | 504 |  | 8864 | 731040 | 10 | 58 | 0 (4) | 566 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The High range frequency can’t cover the two RBs at the right edge of band due to the limit of GSCN range in TS 38.101-1 table 5.4.3.3-1. Note 4: This frequency is obtained applying certain exceptions over general rules defined in annex C.3.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-2: Test frequencies for NR operating band n79, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low (Note 3) | 4405.83 | 693722 | 4401.51 | 693434 | 0 | 30 | 8475 | 693696 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 4699.92 | 713328 | 4658.88 | 710592 | 102 |  | 8679 | 713280 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 4995 | 733000 | 4809.24 | 720616 | 504 |  | 8884 | 732960 | 8 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 4410 | 694000 | 4400.82 | 693388 | 0 | 30 | 8475 | 693696 | 20 | 2 | 0 (0) | 4 |
|  |  | & | Mid | 4700.01 | 713334 | 4654.11 | 710274 | 102 |  | 8676 | 712992 | 6 | 1 | 0 (0) | 206 |
|  |  | Uplink | High | 4989.99 | 732666 | 4799.37 | 719958 | 504 |  | 8878 | 732384 | 18 | 3 | 0 (0) | 1014 |
| 40 | 106 | Downlink | Low | 4420.02 | 694668 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4644.21 | 709614 | 102 |  | 8672 | 712608 | 18 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 4980 | 732000 | 4779.48 | 718632 | 504 |  | 8864 | 731040 | 0 | 3 | 0 (0) | 1014 |
| 50 | 133 | Downlink | Low | 4425 | 695000 | 4401.06 | 693404 | 0 | 30 | 8480 | 694176 | 4 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4639.35 | 709290 | 102 |  | 8672 | 712608 | 6 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 4974.99 | 731666 | 4769.61 | 717974 | 504 |  | 8864 | 731040 | 10 | 26 | 1 (4) | 1068 |
| 60 | 162 | Downlink | Low | 4430.01 | 695334 | 4400.85 | 693390 | 0 | 30 | 8480 | 694176 | 18 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4634.13 | 708942 | 102 |  | 8672 | 712608 | 18 | 36 | 1 (4) | 284 |
|  |  | Uplink | High | 4969.98 | 731332 | 4759.38 | 717292 | 504 |  | 8864 | 731040 | 20 | 54 | 1 (4) | 1124 |
| 80 | 217 | Downlink | Low | 4440 | 696000 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4624.23 | 708282 | 102 |  | 8656 | 711072 | 6 | 0 | 1 (4) | 212 |
|  |  | Uplink | High | 4959.99 | 730666 | 4739.49 | 715966 | 504 |  | 8848 | 729504 | 2 | 46 | 1 (4) | 1108 |
| 100 | 273 | Downlink | Low | 4450.02 | 696668 | 4400.88 | 693392 | 0 | 30 | 8480 | 694176 | 16 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4614.15 | 707610 | 102 |  | 8656 | 711072 | 6 | 28 | 1 (4) | 268 |
|  |  | Uplink | High | 4950 | 730000 | 4719.42 | 714628 | 504 |  | 8832 | 727968 | 20 | 37 | 1 (4) | 1090 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-6 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This frequency is obtained applying certain exceptions over general rules defined in annex C.3.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-3: Test frequencies for NR operating band n79, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0.

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 4405.02 | 693668 | 4401.06 | 693404 | 0 | 30 | - | 693644 |
|  |  | & | Mid | 4700.01 | 713334 | 4622.61 | 708174 | 102 |  | - | 713310 |
|  |  | Uplink | High | 4995 | 733000 | 4628.16 | 708544 | 504 |  | - | 732976 |
| 20 | 24 | Downlink | Low | 4410 | 694000 | 4401.36 | 693424 | 0 | 30 | - | 693664 |
|  |  | & | Mid | 4700.01 | 713334 | 4617.93 | 707862 | 102 |  | - | 712998 |
|  |  | Uplink | High | 4989.99 | 732666 | 4618.47 | 707898 | 504 |  | - | 732330 |
| 40 | 51 | Downlink | Low | 4420.02 | 694668 | 4401.66 | 693444 | 0 | 30 | - | 693684 |
|  |  | & | Mid | 4700.01 | 713334 | 4608.21 | 707214 | 102 |  | - | 712350 |
|  |  | Uplink | High | 4980 | 732000 | 4598.76 | 706584 | 504 |  | - | 731016 |
| 50 | 65 | Downlink | Low | 4425 | 695000 | 4401.6 | 693440 | 0 | 30 | - | 693680 |
|  |  | & | Mid | 4700.01 | 713334 | 4603.17 | 706878 | 102 |  | - | 712014 |
|  |  | Uplink | High | 4974.99 | 731666 | 4588.71 | 705914 | 504 |  | - | 730346 |
| 60 | 79 | Downlink | Low | 4430.01 | 695334 | 4401.57 | 693438 | 0 | 30 | - | 693678 |
|  |  | & | Mid | 4700.01 | 713334 | 4598.13 | 706542 | 102 |  | - | 711678 |
|  |  | Uplink | High | 4969.98 | 731332 | 4578.66 | 705244 | 504 |  | - | 729676 |
| 80 | 107 | Downlink | Low | 4440 | 696000 | 4401.48 | 693432 | 0 | 30 | - | 693672 |
|  |  | & | Mid | 4700.01 | 713334 | 4588.05 | 705870 | 102 |  | - | 711006 |
|  |  | Uplink | High | 4959.99 | 730666 | 4558.59 | 703906 | 504 |  | - | 728338 |
| 100 | 135 | Downlink | Low | 4450.02 | 696668 | 4401.42 | 693428 | 0 | 30 | - | 693668 |
|  |  | & | Mid | 4700.01 | 713334 | 4577.97 | 705198 | 102 |  | - | 710334 |
|  |  | Uplink | High | 4950 | 730000 | 4538.52 | 702568 | 504 |  | - | 727000 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.80	Reference test frequencies for NR operating band n80 (SUL)

Table 4.3.1.1.1.80-1: Test frequencies for NR operating band n80 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1654.53 | 330906 | 504 |
|  |  |  | High | 1782.5 | 356500 | 1779.17 | 355834 | 6 |
| 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1652.1 | 330420 | 504 |
|  |  |  | High | 1780 | 356000 | 1774.24 | 354848 | 6 |
| 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1649.67 | 329934 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1769.31 | 353862 | 6 |
| 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1647.24 | 329448 | 504 |
|  |  |  | High | 1775 | 355000 | 1764.38 | 352876 | 6 |
| 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1644.81 | 328962 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1759.45 | 351890 | 6 |
| 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1642.38 | 328476 | 504 |
|  |  |  | High | 1770 | 354000 | 1754.52 | 350904 | 6 |

Table 4.3.1.1.1.80-2: Test frequencies for NR operating band n80 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1561.74 | 312348 | 504 |
|  |  |  | High | 1780 | 356000 | 1773.52 | 354704 | 6 |
| 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1559.22 | 311844 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1768.5 | 353700 | 6 |
| 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1556.88 | 311376 | 504 |
|  |  |  | High | 1775 | 355000 | 1763.66 | 352732 | 6 |
| 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1554.36 | 310872 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1758.64 | 351728 | 6 |
| 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1552.02 | 310404 | 504 |
|  |  |  | High | 1770 | 354000 | 1753.8 | 350760 | 6 |

Table 4.3.1.1.1.80-3: Test frequencies for NR operating band n80 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1380.66 | 276132 | 504 |
|  |  |  | High | 1780 | 356000 | 1771.72 | 354344 | 6 |
| 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1378.14 | 275628 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1766.7 | 353340 | 6 |
| 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1375.98 | 275196 | 504 |
|  |  |  | High | 1775 | 355000 | 1762.04 | 352408 | 6 |
| 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1373.46 | 274692 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1757.02 | 351404 | 6 |
| 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1370.94 | 274188 | 504 |
|  |  |  | High | 1770 | 354000 | 1752 | 350400 | 6 |

###### 4.3.1.1.1.81	Reference test frequencies for NR operating band n81 (SUL)

Table 4.3.1.1.1.81-1: Test frequencies for NR operating band n81 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |
| 10 | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |
| 15 | 79 | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |
| 20 | 106 | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |

Table 4.3.1.1.1.81-2: Test frequencies for NR operating band n81 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |
| 15 | 38 | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |
| 20 | 51 | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |

###### 4.3.1.1.1.82	Reference test frequencies for NR operating band n82 (SUL)

Table 4.3.1.1.1.82-1: Test frequencies for NR operating band n82 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |
| 10 | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |
| 15 | 79 | Uplink | Low | 839.5 | 167900 | 832.39 | 166478 | 0 |
|  |  |  | Mid | 847 | 169400 | 749.17 | 149834 | 504 |
|  |  |  | High | 854.5 | 170900 | 846.31 | 169262 | 6 |
| 20 | 106 | Uplink | Low | 842 | 168400 | 832.46 | 166492 | 0 |
|  |  |  | Mid | 847 | 169400 | 746.74 | 149348 | 504 |
|  |  |  | High | 852 | 170400 | 841.38 | 168276 | 6 |

Table 4.3.1.1.1.82-2: Test frequencies for NR operating band n82 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |
| 15 | 38 | Uplink | Low | 839.5 | 167900 | 832.66 | 166532 | 0 |
|  |  |  | Mid | 847 | 169400 | 658.72 | 131744 | 504 |
|  |  |  | High | 854.5 | 170900 | 845.5 | 169100 | 6 |
| 20 | 51 | Uplink | Low | 842 | 168400 | 832.82 | 166564 | 0 |
|  |  |  | Mid | 847 | 169400 | 656.38 | 131276 | 504 |
|  |  |  | High | 852 | 170400 | 840.66 | 168132 | 6 |

###### 4.3.1.1.1.83	Reference test frequencies for NR operating band n83 (SUL)

Table 4.3.1.1.1.83-1: Test frequencies for NR operating band n83 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |
| 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |
| 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |
| 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 |
| (Note 1) |  |  | Mid | 728 | 145600 | 627.74 | 125548 | 504 |
|  |  |  | High | 738 | 147600 | 727.38 | 145476 | 6 |
| 30 | 160 | Uplink | Low Mid | 718 | 143600 | 703.6 | 140720 | 0 |
| (Note 2) |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |
| Note 1: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1 Note 2: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.83-2: Test frequencies for NR operating band n83 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 539.7 | 107948 | 504 |
|  |  |  | High | 743 | 148600 | 736.52 | 147304 | 6 |
| 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 537.22 | 107444 | 504 |
|  |  |  | High | 740.5 | 148100 | 731.5 | 146300 | 6 |
| 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 |
| (Note 1) |  |  | Mid | 728 | 145600 | 537.38 | 107476 | 504 |
|  |  |  | High | 738 | 147600 | 726.66 | 145332 | 6 |
| 30 | 78 | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 |
| (Note 2) |  |  | High | 733 | 146600 | 716.8 | 143360 | 6 |
| Note 1: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1 Note 2: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.84	Reference test frequencies for NR operating band n84 (SUL)

Table 4.3.1.1.1.84-1: Test frequencies for NR operating band n84 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1857.03 | 371406 | 504 |
|  |  |  | High | 1977.5 | 395500 | 1974.17 | 394834 | 6 |
| 10 | 52 | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1854.6 | 370920 | 504 |
|  |  |  | High | 1975 | 395000 | 1969.24 | 393848 | 6 |
| 15 | 79 | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1852.17 | 370434 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1964.31 | 392862 | 6 |
| 20 | 106 | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1849.74 | 369948 | 504 |
|  |  |  | High | 1970 | 394000 | 1959.38 | 391876 | 6 |
| 25 | 133 | Uplink | Low | 1932.5 | 386500 | 1920.53 | 384106 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1847.31 | 369462 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1954.45 | 390890 | 6 |
| 30 | 160 | Uplink | Low | 1935 | 387000 | 1920.6 | 384120 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1844.88 | 368976 | 504 |
|  |  |  | High | 1965 | 393000 | 1949.52 | 389904 | 6 |
| 40 | 216 | Uplink | Low | 1940 | 388000 | 1920.56 | 384112 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1839.84 | 367968 | 504 |
|  |  |  | High | 1960 | 392000 | 1939.48 | 387896 | 6 |
| 50 | 270 | Uplink | Low | 1945 | 389000 | 1920.7 | 384140 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1834.98 | 366996 | 504 |
|  |  |  | High | 1955 | 391000 | 1929.62 | 385924 | 6 |

Table 4.3.1.1.1.84-2: Test frequencies for NR operating band n84 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1764.24 | 352848 | 504 |
|  |  |  | High | 1975 | 395000 | 1968.52 | 393704 | 6 |
| 15 | 38 | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1761.72 | 352344 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1963.5 | 392700 | 6 |
| 20 | 51 | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1759.38 | 351876 | 504 |
|  |  |  | High | 1970 | 394000 | 1958.66 | 391732 | 6 |
| 25 | 65 | Uplink | Low | 1932.5 | 386500 | 1920.8 | 384160 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1756.86 | 351372 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1953.64 | 390728 | 6 |
| 30 | 78 | Uplink | Low | 1935 | 387000 | 1920.96 | 384192 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1754.52 | 350904 | 504 |
|  |  |  | High | 1965 | 393000 | 1948.8 | 389760 | 6 |
| 40 | 106 | Uplink | Low | 1940 | 388000 | 1920.92 | 384184 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1749.48 | 349896 | 504 |
|  |  |  | High | 1960 | 392000 | 1938.76 | 387752 | 6 |
| 50 | 133 | Uplink | Low | 1945 | 389000 | 1921.06 | 384212 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1744.62 | 348924 | 504 |
|  |  |  | High | 1955 | 391000 | 1928.9 | 385780 | 6 |

Table 4.3.1.1.1.84-3: Test frequencies for NR operating band n84 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1583.16 | 316632 | 504 |
|  |  |  | High | 1975 | 395000 | 1966.72 | 393344 | 6 |
| 15 | 18 | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1580.64 | 316128 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1961.7 | 392340 | 6 |
| 20 | 24 | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1578.48 | 315696 | 504 |
|  |  |  | High | 1970 | 394000 | 1957.04 | 391408 | 6 |
| 25 | 31 | Uplink | Low | 1932.5 | 386500 | 1921.34 | 384268 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1575.96 | 315192 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1952.02 | 390404 | 6 |
| 30 | 38 | Uplink | Low | 1935 | 387000 | 1921.32 | 384264 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1573.44 | 314688 | 504 |
|  |  |  | High | 1965 | 393000 | 1947 | 389400 | 6 |
| 40 | 51 | Uplink | Low | 1940 | 388000 | 1921.64 | 384328 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1568.76 | 313752 | 504 |
|  |  |  | High | 1960 | 392000 | 1937.32 | 387464 | 6 |
| 50 | 65 | Uplink | Low | 1945 | 389000 | 1921.6 | 384320 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1563.72 | 312744 | 504 |
|  |  |  | High | 1955 | 391000 | 1927.28 | 385456 | 6 |

###### 4.3.1.1.1.85	Reference test frequencies for NR operating band n85

Table 4.3.1.1.1.85-1: Test frequencies for NR operating band n85 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 729.5 | 145900 | 728.15 | 145630 | 0 | 15 | 30282 | 145870 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 737 | 147400 | 717.29 | 143458 | 102 |  | 30321 | 147430 | 4 | 0 | 3 (2) | 104 |
|  |  |  | High | 744.5 | 148900 | 652.43 | 130486 | 504 |  | 30357 | 148870 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 699.5 | 139900 | 698.15 | 139630 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 614.93 | 122986 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 714.5 | 142900 | 712.07 | 142414 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 730.5 | 146100 | 728.25 | 145650 | 0 | 15 | 1826 | 146170 | 0 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 737 | 147400 | 716.39 | 143278 | 102 |  | 1843 | 147410 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 743.5 | 148700 | 650.53 | 130106 | 504 |  | 1858 | 148610 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 700.5 | 140100 | 698.25 | 139650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 614.03 | 122806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 713.5 | 142700 | 710.17 | 142034 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 733 | 146600 | 728.32 | 145664 | 0 | 15 | 1826 | 146170 | 1 | 2 | 2 (4) | 6 |
|  |  |  | Mid | 737 | 147400 | 713.96 | 142792 | 102 |  | 1835 | 146890 | 2 | 2 | 1 (2) | 106 |
|  |  |  | High | 741 | 148200 | 645.6 | 129120 | 504 |  | 1846 | 147650 | 2 | 2 | 0 (0) | 506 |
|  |  | Uplink | Low | 703 | 140600 | 698.32 | 139664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 611.6 | 122320 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 705.24 | 141048 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 735.5 | 147100 | 728.39 | 145678 | 0 | 15 | 1827 | 146190 | 2 | 4 | 2 (4) | 8 |
|  |  |  | Mid | 737 | 147400 | 711.53 | 142306 | 102 |  | 1829 | 146410 | 2 | 4 | 1 (2) | 108 |
|  |  |  | High | 738.5 | 147700 | 640.67 | 128134 | 504 |  | 1832 | 146650 | 2 | 4 | 0 (0) | 508 |
|  |  | Uplink | Low | 705.5 | 141100 | 698.39 | 139678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 609.17 | 121834 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 700.31 | 140062 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.85-2: Test frequencies for NR operating band n85 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 733 | 146600 | 728.68 | 145736 | 0 | 15 | 1832 | 146650 | 12 | 0 | 1 (2) | 4 |
|  |  |  | Mid | 737 | 147400 | 695.96 | 139192 | 102 |  | 1847 | 147850 | 16 | 0 | 3 (8) | 220 |
|  |  |  | High | 741 | 148200 | 555.24 | 111048 | 504 |  | 1852 | 148130 | 16 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 703 | 140600 | 698.68 | 139736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 521.24 | 104248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 704.52 | 140904 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 735.5 | 147100 | 728.66 | 145732 | 0 | 15 | 1837 | 146930 | 22 | 1 | 2 (4) | 8 |
|  |  |  | Mid | 737 | 147400 | 693.44 | 138688 | 102 |  | 1838 | 147130 | 23 | 1 | 0 (0) | 206 |
|  |  |  | High | 738.5 | 147700 | 550.22 | 110044 | 504 |  | 1843 | 147410 | 23 | 1 | 0 (0) | 1010 |
|  |  | Uplink | Low | 705.5 | 141100 | 698.66 | 139732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 518.72 | 103744 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 699.5 | 139900 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.86	Reference test frequencies for NR operating band n86 (SUL)

Table 4.3.1.1.1.86-1: Test frequencies for NR operating band n86 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |
| 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |
|  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |
| 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |
| 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |
|  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |
| 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |
|  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |

Table 4.3.1.1.1.86-2: Test frequencies for NR operating band n86 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |
|  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |
| 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |
| 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |
|  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |
| 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |
|  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |

Table 4.3.1.1.1.86-3: Test frequencies for NR operating band n86 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |
|  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |
| 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1375.64 | 275128 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1761.7 | 352340 | 6 |
| 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |
|  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |
| 40 | 51 | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1363.76 | 272752 | 504 |
|  |  |  | High | 1760 | 352000 | 1737.32 | 347464 | 0 |

###### 4.3.1.1.1.87 to 4.3.1.1.1.90	FFS

###### 4.3.1.1.1.91	Reference test frequencies for NR operating band n91

Table 4.3.1.1.1.91-1: Test frequencies for NR operating band n91 and SCS 15 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - | - |
| 10/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.92	Reference test frequencies for NR operating band n92

Table 4.3.1.1.1.92-1: Test frequencies for NR operating band n92 and SCS 15 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | 3584 | 286810 | 0 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | 3687 | 294990 | 0 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | 3787 | 302930 | 4 | 1 | 1 (2) |  | 507 |
|  |  | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - |  | - |
| 5/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) |  | 507 |
|  | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - |  | - |
| 5/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) |  | 507 |
|  | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - |  | - |
| 5/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) |  | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) |  | 509 |
|  | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - |  | - |
| 10/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) |  | 507 |
|  |  | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - |  | - |
| 10/15 | 70 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) |  | 507 |
|  | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - |  | - |
| 10/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) |  | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) |  | 509 |
|  | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - |  | - |
| 15/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) |  | 507 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.39 | 166478 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 749.17 | 149834 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 854.5 | 170900 | 846.31 | 169262 | 6 |  | - | - | - | - | - |  | - |
| 20/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) |  | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) |  | 509 |
|  |  | Uplink | Low | 842 | 168400 | 832.46 | 166492 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 746.74 | 149348 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 852 | 170400 | 841.38 | 168276 | 6 |  | - | - | - | - | - |  | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.92-2: Test frequencies for NR operating band n92 and SCS 30 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | 3591 | 287310 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 10/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  | 24 | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.66 | 166532 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 658.72 | 131744 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 854.5 | 170900 | 845.5 | 169100 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 842 | 168400 | 832.82 | 166564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 656.38 | 131276 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 852 | 170400 | 840.66 | 168132 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.93	Reference test frequencies for NR operating band n93

Table 4.3.1.1.1.93-1: Test frequencies for NR operating band n93 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.94	Reference test frequencies for NR operating band n94

Table 4.3.1.1.1.94-1: Test frequencies for NR operating band n94 and SCS 15 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | 3584 | 286810 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | 3687 | 294990 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | 3787 | 302930 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) | 507 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/15 | 70 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.94-2: Test frequencies for NR operating band n94 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | 3591 | 287310 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.95	Reference test frequencies for NR operating band n95 (SUL)

Table 4.3.1.1.1.95-1: Test frequencies for NR operating band n95 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 2012.5 | 402500 | 2010.25 | 402050 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1924.53 | 384906 | 504 |
|  |  |  | High | 2022.5 | 404500 | 2019.17 | 403834 | 6 |
| 10 | 52 | Uplink | Low | 2015 | 403000 | 2010.32 | 402064 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1922.1 | 384420 | 504 |
|  |  |  | High | 2020 | 404000 | 2014.24 | 402848 | 6 |
| 15 | 79 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2010.39 | 402078 | 0 |

Table 4.3.1.1.1.95-2: Test frequencies for NR operating band n95 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 2015 | 403000 | 2010.68 | 402136 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1831.74 | 366348 | 504 |
|  |  |  | High | 2020 | 404000 | 2013.52 | 402704 | 6 |
| 15 | 38 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2010.66 | 402132 | 0 |

Table 4.3.1.1.1.95-3: Test frequencies for NR operating band n95 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 2015 | 403000 | 2011.04 | 402208 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1650.66 | 330132 | 504 |
|  |  |  | High | 2020 | 404000 | 2011.72 | 402344 | 6 |
| 15 | 18 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2011.02 | 402204 | 0 |

###### 4.3.1.1.1.96	 Reference test frequencies for NR operating band n96

Table 4.3.1.1.1.96-1: Test frequencies for NR operating band n96 and SCS 15 kHz (Note 3)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 4 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 106 | Downlink | Low | 5955 | 797000 | 5945.46 | 796364 | 0 | 30 | - | 796604 | 31 | - | - | - |
|  |  | & | Mid | 6535.02 | 835668 | 6507.12 | 833808 | 102 |  | - | 835272 | 31 | - | - | - |
|  |  | Uplink | High | 7114.98 | 874332 | 7014.72 | 867648 | 504 |  | - | 873936 | 31 | - | - | - |
| 40 | 216 | Downlink | Low | 5965.02 | 797668 | 5945.58 | 796372 | 0 | 30 | - | 796612 | 31 | - | - | - |
|  |  | & | Mid | 6525 | 835000 | 6487.2 | 832480 | 102 |  | - | 833944 | 31 | - | - | - |
|  |  | Uplink | High | 7084.98 | 872332 | 6974.82 | 864988 | 504 |  | - | 871276 | 31 | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: NR band n46 with SCS =15 kHz is restricted for operation when carrier is configured as an SCell part of a DC or CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 4: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.96-2: Test frequencies for NR operating band n96 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 3 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 51 | Downlink | Low | 5955 | 797000 | 5945.82 | 796388 | 0 | 30 | 9548 | 796704 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 6535.02 | 835668 | 6489.12 | 832608 | 102 |  | 9951 | 835392 | 22 | 0 | 1 (0) | 204 |
|  |  | Uplink | High | 7114.98 | 874332 | 6924.36 | 861624 | 504 |  | 10353 | 873984 | 22 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 5965.02 | 797668 | 5945.94 | 796396 | 0 | 30 | 9548 | 796704 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 6525 | 835000 | 6469.2 | 831280 | 102 |  | 9937 | 834048 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 7084.98 | 872332 | 6884.46 | 858964 | 504 |  | 10339 | 872640 | 22 | 40 | 3 (3) | 1112 |
| 60 | 162 | Downlink | Low | 5974.98 | 798332 | 5945.82 | 796388 | 0 | 30 | 9548 | 796704 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 6985.02 | 865668 | 6919.14 | 861276 | 102 |  | 10256 | 864672 | 22 | 16 | 3 (3) | 260 |
|  |  | Uplink | High | 7095 | 873000 | 6884.4 | 858960 | 504 |  | 10325 | 871296 | 0 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 5985 | 799000 | 5945.94 | 796396 | 0 | 30 | 9548 | 796704 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 6544.98 | 836332 | 6469.2 | 831280 | 102 |  | 9937 | 834048 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 7024.98 | 868332 | 6804.48 | 853632 | 504 |  | 10270 | 866016 | 22 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.96-3: Test frequencies for NR operating band n96 and SCS 60 kHz without CORESET#0 (Note 1)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 2 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 24 | Downlink | Low | 5955 | 797000 | 5946.36 | 796424 | 0 | 30 | - | 796664 |
|  |  | & | Mid | 6535.02 | 835668 | 6452.94 | 830196 | 102 |  | - | 835332 |
|  |  | Uplink | High | 7114.98 | 874332 | 6743.46 | 849564 | 504 |  | - | 873996 |
| 40 | 51 | Downlink | Low | 5965.02 | 797668 | 5946.66 | 796444 | 0 | 30 | - | 796684 |
|  |  | & | Mid | 6525 | 835000 | 6433.2 | 828880 | 102 |  | - | 834016 |
|  |  | Uplink | High | 7084.98 | 872332 | 6703.74 | 846916 | 504 |  | - | 871348 |
| 60 | 79 | Downlink | Low | 5974.98 | 798332 | 5946.54 | 796436 | 0 | 30 | - | 796676 |
|  |  | & | Mid | 6985.02 | 865668 | 6883.14 | 858876 | 102 |  | - | 864012 |
|  |  | Uplink | High | 7095 | 873000 | 6703.68 | 846912 | 504 |  | - | 871344 |
| 80 | 107 | Downlink | Low | 5985 | 799000 | 5946.48 | 796432 | 0 | 30 | - | 796672 |
|  |  | & | Mid | 6544.98 | 836332 | 6433.02 | 828868 | 102 |  | - | 834004 |
|  |  | Uplink | High | 7024.98 | 868332 | 6623.58 | 841572 | 504 |  | - | 866004 |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-3. |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.97	Reference test frequencies for NR operating band n97 (SUL)

Table 4.3.1.1.1.97-1: Test frequencies for NR operating band n97 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 2302.5 | 460500 | 2300.25 | 460050 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2329.39 | 465878 | 102 |
|  |  |  | High | 2397.5 | 479500 | 2304.53 | 460906 | 504 |
| 10 | 52 | Uplink | Low | 2305 | 461000 | 2300.32 | 460064 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2326.96 | 465392 | 102 |
|  |  |  | High | 2395 | 479000 | 2299.6 | 459920 | 504 |
| 15 | 79 | Uplink | Low | 2307.5 | 461500 | 2300.39 | 460078 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2324.53 | 464906 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2294.67 | 458934 | 504 |
| 20 | 106 | Uplink | Low | 2310 | 462000 | 2300.46 | 460092 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2322.1 | 464420 | 102 |
|  |  |  | High | 2390 | 478000 | 2289.74 | 457948 | 504 |
| 25 | 133 | Uplink | Low | 2312.5 | 462500 | 2300.53 | 460106 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2319.67 | 463934 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2284.81 | 456962 | 504 |
| 30 | 160 | Uplink | Low | 2315 | 463000 | 2300.6 | 460120 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2317.24 | 463448 | 102 |
|  |  |  | High | 2385 | 477000 | 2279.88 | 455976 | 504 |
| 40 | 216 | Uplink | Low | 2320 | 464000 | 2300.56 | 460112 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2312.2 | 462440 | 102 |
|  |  |  | High | 2380 | 476000 | 2269.84 | 453968 | 504 |
| 50 | 270 | Uplink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2307.34 | 461468 | 102 |
|  |  |  | High | 2375 | 475000 | 2259.98 | 451996 | 504 |

Table 4.3.1.1.1.97-2: Test frequencies for NR operating band n97 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 2305 | 461000 | 2300.68 | 460136 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2308.96 | 461792 | 102 |
|  |  |  | High | 2395 | 479000 | 2209.24 | 441848 | 504 |
| 15 | 38 | Uplink | Low | 2307.5 | 461500 | 2300.66 | 460132 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2306.44 | 461288 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2204.22 | 440844 | 504 |
| 20 | 51 | Uplink | Low | 2310 | 462000 | 2300.82 | 460164 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2304.1 | 460820 | 102 |
|  |  |  | High | 2390 | 478000 | 2199.38 | 439876 | 504 |
| 25 | 65 | Uplink | Low | 2312.5 | 462500 | 2300.8 | 460160 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2301.58 | 460316 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2194.36 | 438872 | 504 |
| 30 | 78 | Uplink | Low | 2315 | 463000 | 2300.96 | 460192 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2299.24 | 459848 | 102 |
|  |  |  | High | 2385 | 477000 | 2189.52 | 437904 | 504 |
| 40 | 106 | Uplink | Low | 2320 | 464000 | 2300.92 | 460184 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2294.2 | 458840 | 102 |
|  |  |  | High | 2380 | 476000 | 2179.48 | 435896 | 504 |
| 50 | 133 | Uplink | Low | 2325 | 465000 | 2301.06 | 460212 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2289.34 | 457868 | 102 |
|  |  |  | High | 2375 | 475000 | 2169.62 | 433924 | 504 |
| 60 | 162 | Uplink | Low | 2330 | 466000 | 2300.84 | 460168 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2284.12 | 456824 | 102 |
|  |  |  | High | 2370 | 474000 | 2159.4 | 431880 | 504 |
|  |  |  | Low | 2335 | 467000 | 2300.98 | 460196 | 0 |
| 70 | 189 | Uplink | Mid | 2350 | 470000 | 2279.26 | 455852 | 102 |
|  |  |  | High | 2365 | 473000 | 2149.54 | 429908 | 504 |
| 80 | 217 | Uplink | Low | 2340 | 468000 | 2300.94 | 460188 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2274.22 | 454844 | 102 |
|  |  |  | High | 2360 | 472000 | 2139.5 | 427900 | 504 |
|  |  |  | Low | 2345 | 469000 | 2300.9 | 460180 | 0 |
| 90 | 245 | Uplink | Mid | 2350 | 470000 | 2269.18 | 453836 | 102 |
|  |  |  | High | 2355 | 471000 | 2129.46 | 425892 | 504 |
|  |  |  | Low | 2350 | 470000 | 2300.86 | 460172 | 0 |
| 100 | 273 | Uplink | Mid | 2350 | 470000 | 2264.14 | 452828 | 102 |
|  |  |  | High | 2350 | 470000 | 2119.42 | 423884 | 504 |

Table 4.3.1.1.1.97-3: Test frequencies for NR operating band n97 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 2305 | 461000 | 2301.04 | 460208 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2272.6 | 454520 | 102 |
|  |  |  | High | 2395 | 479000 | 2028.16 | 405632 | 504 |
| 15 | 18 | Uplink | Low | 2307.5 | 461500 | 2301.02 | 460204 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2270.08 | 454016 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2023.14 | 404628 | 504 |
| 20 | 24 | Uplink | Low | 2310 | 462000 | 2301.36 | 460272 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2267.92 | 453584 | 102 |
|  |  |  | High | 2390 | 478000 | 2018.48 | 403696 | 504 |
| 25 | 31 | Uplink | Low | 2312.5 | 462500 | 2301.34 | 460268 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2265.4 | 453080 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2013.46 | 402692 | 504 |
| 30 | 38 | Uplink | Low | 2315 | 463000 | 2301.32 | 460264 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2262.88 | 452576 | 102 |
|  |  |  | High | 2385 | 477000 | 2008.44 | 401688 | 504 |
| 40 | 51 | Uplink | Low | 2320 | 464000 | 2301.64 | 460328 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2258.2 | 451640 | 102 |
|  |  |  | High | 2380 | 476000 | 1998.76 | 399752 | 504 |
| 50 | 65 | Uplink | Low | 2325 | 465000 | 2301.6 | 460320 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2253.16 | 450632 | 102 |
|  |  |  | High | 2375 | 475000 | 1988.72 | 397744 | 504 |
| 60 | 79 | Uplink | Low | 2330 | 466000 | 2301.56 | 460312 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2248.12 | 449624 | 102 |
|  |  |  | High | 2370 | 474000 | 1978.68 | 395736 | 504 |
|  |  |  | Low | 2335 | 467000 | 2301.52 | 460304 | 0 |
| 70 | 93 | Uplink | Mid | 2350 | 470000 | 2243.08 | 448616 | 102 |
|  |  |  | High | 2365 | 473000 | 1968.64 | 393728 | 504 |
| 80 | 107 | Uplink | Low | 2340 | 468000 | 2301.48 | 460296 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2238.04 | 447608 | 102 |
|  |  |  | High | 2360 | 472000 | 1958.6 | 391720 | 504 |
|  |  |  | Low | 2345 | 469000 | 2301.44 | 460288 | 0 |
| 90 | 121 | Uplink | Mid | 2350 | 470000 | 2233 | 446600 | 102 |
|  |  |  | High | 2355 | 471000 | 1948.56 | 389712 | 504 |
|  |  |  | Low | 2350 | 470000 | 2301.4 | 460280 | 0 |
| 100 | 135 | Uplink | Mid | 2350 | 470000 | 2227.96 | 445592 | 102 |
|  |  |  | High | 2350 | 470000 | 1938.52 | 387704 | 504 |

###### 4.3.1.1.1.98	FFS

###### 4.3.1.1.1.99	Reference test frequencies for NR operating band n99 (SUL)

Table 4.3.1.1.1.99-1: Test frequencies for NR operating band n99 and SCS 15 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 |
|  |  |  | Mid | 1635.0 | 327000 | 1542.03 | 308406 | 504 |
|  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |
| 10 | 52 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |

Table 4.3.1.1.1.99-2: Test frequencies for NR operating band n99 and SCS 30 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1645.02 | 329004 | 6 |

Table 4.3.1.1.1.99-3: Test frequencies for NR operating band n99 and SCS 60 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low Mid | 1632.5 | 326500 | 1628.54 | 325708 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1643.22 | 328644 | 6 |

###### 4.3.1.1.1.100	Reference test frequencies for NR operating band n100

Table 4.3.1.1.1.100-1: Test frequencies for NR operating band n100 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 920.9 | 184180 | 919.55 | 183910 | 0 | 15 | 31239 | 184150 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 922.2 | 184440 | 902.49 | 180498 | 102 |  | 31246 | 184410 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 923.5 | 184700 | 831.43 | 166286 | 504 |  | 31253 | 184670 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 875.9 | 175180 | 874.55 | 174910 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 877.2 | 175440 | 785.13 | 157026 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 878.5 | 175700 | 876.07 | 175214 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 921.9 | 184380 | 919.65 | 183930 | 0 | 15 | 2304 | 184350 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 922.2 | 184440 | 901.59 | 180318 | 102 |  | 2304 | 184350 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 922.5 | 184500 | 829.53 | 165906 | 504 |  | 2307 | 184590 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 876.9 | 175380 | 874.65 | 174930 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 877.2 | 175440 | 784.23 | 156846 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 877.5 | 175500 | 874.17 | 174834 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.101	Reference test frequencies for NR operating band n101

Table 4.3.1.1.1.101-1: Test frequencies for NR operating band n101 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink & Uplink | Low | 1902.5 | 380500 | 1900.25 | 380050 | 0 | 15 | 4754 | 380410 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1905 | 381000 | 1884.39 | 376878 | 102 |  | 4761 | 380910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1907.5 | 381500 | 1814.53 | 362906 | 504 |  | 4768 | 381410 | 0 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink & Uplink | Low | 1905 | 381000 | 1900.32 | 380064 | 0 | 15 | 4755 | 380430 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.101-2: Test frequencies for NR operating band n101 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink & Uplink | Low | 1905 | 381000 | 1900,68 | 380136 | 0 | 15 | 4762 | 380930 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.102 to 4.3.1.1.1.105	FFS

4.3.1.1.1.106	Reference test frequencies for NR operating band n106

Table 4.3.1.1.1.106-1: Test frequencies for NR operating band n106 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 938.0 | 187600 | 918.29 | 183658 | 102 |  | 31326 | 187630 | 4 | 0 | 3 (2) | 104 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 899.0 | 179800 | 806.93 | 161386 | 504 |  | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.2	NR inter-band CA configurations in FR1

Editor’s Note:

- Test frequencies for uplink CA_n77(2A) are FFS.

###### 4.3.1.1.2.1	NR inter-band CA configurations in FR1 (two bands)

Table 4.3.1.1.2.1-1: Inter-band NR CA configurations (FR1, two bands)

| NR CA configuration | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note 3) | NR CA downlink configuration band 2 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1A | n3A | n1A | n3A | Yes |
| CA_n1A-n8A | CA_n1A-n8A | n1A | n8A | n1A | n8A | Yes |
| CA_n1(2A)-n3A | - | CA_n1(2A) | n3A | n1A | n3A | No |
| CA_n1(2A)-n5A | - | CA_n1(2A) | n5A | - | - | No |
| CA_n1(2A)-n8A | - | CA_n1(2A) | n8A | - | - | No |
| CA_n1A-n28A | CA_n1A-n28A | n1A | n28A | n1A | n28A | Yes |
| CA_n1A-n41A | CA_n1A-n41A | n1A | n41A | n1A | n41A | Yes |
| CA_n1A-n77A | - | n1A | n77A | - | - | Yes |
| CA_n1A-n78A | CA_n1A-n78A | n1A | n78A | n1A | n78A | Yes |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1A | CA_n78(2A) | n1A | n78A | No |
| CA_n1A-n78C | CA_n1A-n78A | n1A | CA_n78C | n1A | n78A | No |
| CA_n1(2A)-n78A | - | CA_n1(2A) | n78A | - | - | No |
| CA_n1A-n79A | CA_n1A-n79A | n1A | n79A | n1A | n79A | Yes |
| CA_n2A-n5A | CA_n2A-n5A | n2A | n5A | n2A | n5A | Yes |
| CA_n2A-n14A | CA_n2A-n14A | n2A | n14A | n2A | n14A | Yes |
| CA_n2A-n30A | CA_n2A-n30A | n2A | n30A | n2A | n30A | Yes |
| CA_n2A-n48A | CA_n2A-n48A | n2A | n48A | n2A | n48A | Yes |
| CA_n2A-n48(2A) | CA_n2A-n48A | n2A | CA_n48(2A) | n2A | n48A | No |
| CA_n2A-n48B | CA_n2A-n48A | n2A | CA_n48B | n2A | n48A | No |
| CA_n2A-n66A | CA_n2A-n66A | n2A | n66A | n2A | n66A | Yes |
| CA_n2(2A)-n66A | - | CA_n2(2A) | n66A | - | - | No |
| CA_n2A-n77A | CA_n2A-n77A | n2A | n77A | n2A | n77A | Yes |
| CA_n2A-n77C | CA_n2A-n77A | n2A | CA_n77C | n2A | n77A | No |
| CA_n3A-n5A | CA_n3A-n5A | n3A | n5A | n3A | n5A | Yes |
| CA_n3(2A)-n5A | - | CA_n3(2A) | n5A | - | - | No |
| CA_n3A-n8A | CA_n3A-n8A | n3A | n8A | n3A | n8A | Yes |
| CA_n3(2A)-n8A | - | CA_n3(2A) | n8A | - | - | No |
| CA_n3A-n28A | CA_n3A-n28A | n3A | n28A | n3A | n28A | Yes |
| CA_n3A-n41A | CA_n3A-n41A | n3A | n41A | n3A | n41A | Yes |
| CA_n3A-n77A | CA_n3A-n77A | n3A | n77A | n3A | n77A | Yes |
| CA_n3A-n77(2A) | CA_n3A-n77A | n3A | CA_n77(2A) | n3A | n77A | No |
| CA_n3A-n77(2A) | CA_n77(2A) | n3A | CA_n77(2A) | - | CA_n77(2A) | No |
| CA_n3A-n78A | CA_n3A-n78A | n3A | n78A | n3A | n78A | Yes |
| CA_n3A-n78(2A) | CA_n3A-n78A | n3A | CA_n78(2A) | n3A | n78A | No |
|  | CA_n78(2A) | n3A | CA_n78(2A) | - | CA_n78(2A) | No |
| CA_n3(2A)-n78A | - | CA_n3(2A) | n78A | - | - | No |
| CA_n5A-n7A | - | n5A | n7A | - | - | Yes |
| CA_n5A-n30A | CA_n5A-n30A | n5A | n30A | n5A | n30A | Yes |
| CA_n5A-n48A | CA_n5A-n48A | n5A | n48A | n5A | n48A | Yes |
| CA_n5A-n48(2A) | CA_n5A-n48A | N5A | CA_n48(2A) | n5A | n48A | No |
| CA_n5A-n48B | CA_n5A-n48A | n5A | CA_n48B | n5A | n48A | No |
| CA_n5A-n66A | CA_n5A-n66A | n5A | n66A | n5A | n66A | Yes |
| CA_n5A-n77A | CA_n5A-n77A | n5A | n77A | n5A | n77A | Yes |
| CA_n5A-n77C | CA_n5A-n77A | n5A | CA_n77C | n5A | n77A | No |
| CA_n5A-n78A | - | n5A | n78A | - | - | Yes |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5A | CA_n78(2A) | n5A | n78A | No |
| CA_n5B-n77A | CA_n5A-n77A | CA_n5B | n77A | n5A | n77A | No |
| CA_n5B-n77C | CA_n5A-n77A | CA_n5B | CA_n77C | n5A | n77A | No |
| CA_n7A-n78A | - | n7A | n78A | - | - | Yes |
| CA_n8A-n78A | CA_n8A-n78A | n8A | n78A | n8A | n78A | Yes |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8A | CA_n78(2A) | n8A | CA_n78(2A) | No |
| CA_n14A-n30A | CA_n14A-n30A | n14A | n30A | n14A | n30A | Yes |
| CA_n14A-n66A | CA_n14A-n66A | n14A | n66A | n14A | n66A | Yes |
| CA_n14A-n77A | CA_n14A-n77A | n14A | n77A | n14A | n77A | Yes |
| CA_n20A-n78A | CA_n20A-n78A | n20A | n78A | n20A | n78A | Yes |
| CA_n24A-n41A | CA_n24A-n41A | n24A | n41A | n24A | n41A | Yes |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24A | CA_n41(2A) | n24A | n41A | No |
| CA_n24A-n48A | CA_n24A-n48A | n24A | n48A | n24A | n48A | Yes |
| CA_n24A-n48B | CA_n24A-n48A | n24A | CA_n48B | n24A | n48A | No |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24A | CA_n48(2A) | n24A | n48A | No |
| CA_n24A-n77A | CA_n24A-n77A | n24A | n77A | n24A | n77A | Yes |
| CA_n24A-n77C | CA_n24A-n77A | n24A | CA_n77C | n24A | n77A | No |
| CA_n25A-n66A | CA_n25A-n66A | n25A | n66A | n25A | n66A | Yes |
| CA_n25A-n77A | CA_n25A-n77A | n25A | n77A | n25A | n77A | Yes |
| CA_n25A-n77A(2A) | CA_n25A-n77A | n25A | CA_n77(2A) | n25A | n77A | No |
| CA_n25A-n78A | CA_n25A-n78A | n25A | n78A | n25A | n78A | Yes |
| CA_n25A-n78A(2A) | CA_n25A-n78A | n25A | CA_n78(2A) | n25A | n78A | No |
| CA_n26A-n66A | CA_n26A-n66A | n26A | n66A | n26A | n66A | Yes |
| CA_n26A-n66(2A) | CA_n26A-n66A | n26A | CA_n66(2A) | n26A | n66A | No |
| CA_n26A-n70A | CA_n26A-n70A | n26A | n70A | n26A | n70A | Yes |
| CA_n28A-n40A | - | n28A | n40A | - | - | Yes |
| CA_n28A-n41A | CA_n28A-n41A | n28A | n41A | n28A | n41A | Yes |
| CA_n28A-n77A | CA_n28A-n77A | n28A | n77A | n28A | n77A | Yes |
| CA_n28A-n77(2A) | CA_n28A-n77A | n28A | CA_n77(2A) | n28A | n77A | No |
| CA_n28A-n77(2A) | CA_n77(2A) | n28A | CA_n77(2A) | - | CA_n77(2A) | No |
| CA_n28A-n78A | CA_n28A-n78A | n28A | n78A | n28A | n78A | Yes |
| CA_n28A-n79A | CA_n28A-n79A | n28A | n79A | n28A | n79A | Yes |
| CA_n29A-n66A | - | n29A | n66A (Note 1) | - | - | Yes |
| CA_n29B-n66B | - | n29A | CA_n66B (Note 1) | - | - | No |
| CA_n29A-n66(2A) | - | n29A | CA_n66(2A) (Note 1) | - | - | No |
| CA_n29A-n70A | - | n29A | n70A (Note 1) | - | - | Yes |
| CA_n29A-n71A | - | n29A | n71A (Note 1) | - | - | Yes |
| CA_n30A-n66A | CA_n30A-n66A | n30A | n66A | n30A | n66A | Yes |
| CA_n30A-n77A | CA_n30A-n77A | n30A | n77A | n30A | n77A | Yes |
| CA_n39A-n41A | CA_n39A-n41A | n39A | n41A | n39A | n41A | Yes |
| CA_n41A-n66A | - | n41A | n66A | - | - | Yes |
| CA_n41A-n71A | - | n41A | n71A | - | - | Yes |
| CA_n41A-n77A | CA_n41A-n77A | n41A | n77A | n41A | n77A | Yes |
| CA_n41A-n79A | CA_n41A-n79A | n41A | n79A | n41A | n79A | Yes |
| CA_n41C-n79A | CA_n41A-n79A | CA_n41C | n79A | n41A | n79A | No |
|  | CA_n41C | CA_n41C | n79A | CA_n41C | - | No |
| CA_n48A-n66A | CA_n48A-n66A | n48A | n66A | n48A | n66A | Yes |
| CA_n48A-n66(2A) | CA_n48A-n66A | n48A | CA_n66(2A) | n48A | n66A | No |
| CA_n48A-n70A | CA_n48A-n70A | n48A | n70A | n48A | n70A | Yes |
| CA_n48A-n71A | CA_n48A-n71A | n48A | n71A | n48A | n71A | Yes |
| CA_n48A-n71(2A) | CA_n48A-n71A | n48A | CA_n71(2A) | n48A | n71A | No |
| CA_n48A-n77A | - | n48A | n77A | - | - | Yes |
| CA_n48A-n77C | - | CA_n77C | n48A | - | - | No |
|  | CA_n77C | CA_n77C | n48A | CA_n77C | - | No |
| CA_n48B-n66A | CA_n48A-n66A | CA_n48B | n66A | n48A | n66A | No |
| CA_n48B-n70A | CA_n48A-n70A | CA_n48B | n70A | n48A | n70A | No |
| CA_n48B-n71A | CA_n48A-n71A | CA_n48B | n71A | n48A | n71A | No |
| CA_n48B-n77A | - | n77A | CA_n48B | - | - | No |
|  | CA_n48B | CA_n48B | n77A | CA_n48B | - | No |
| CA_n48(2A)-n66A | CA_n48A-n66A | CA_n48(2A) | n66A | n48A | n66A | No |
| CA_n48(2A)-n66(2A) | CA_n48A-n66A | CA_n48(2A) | CA_n66(2A) | n48A | n66A | No |
| CA_n48(2A)-n70A | CA_n48A-n70A | CA_n48(2A) | n70A | n48A | n70A | No |
| CA_n48(2A)-n71A | CA_n48A-n71A | CA_n48(2A) | n71A | n48A | n71A | No |
| CA_n48(2A)-n71(2A) | CA_n48A-n71A | CA_n48(2A) | CA_n71(2A) | n48A | n71A | No |
| CA_n48(2A)-n77A | - | n77A | CA_n48(2A) | - | - | No |
| CA_n66A-n70A | - | n66A | n70A | - | - | Yes |
| CA_n66B-n70A | - | CA_n66B | n70A | - | - | No |
| CA_n66(2A)-n70A | - | CA_n66(2A) | n70A | - | - | No |
| CA_n66A-n71A | CA_n66A-n71A | n66A | n71A | n66A | n71A | Yes |
| CA_n66A-n71(2A) | CA_n66A-n71A | n66A | CA_n71(2A) | n66A | n71A | No |
| CA_n66B-n71A | CA_n66A-n71A | CA_n66B | n71A | n66A | n71A | No |
| CA_n66(2A)-n71A | CA_n66A-n71A | CA_n66(2A) | n71A | n66A | n71A | No |
| CA_n66(2A)-n71(2A) | CA_n66A-n71A | CA_n66(2A) | CA_n71(2A) | n66A | n71A | No |
| CA_n66A-n77A | CA_n66A-n77A | n66A | n77A | n66A | n77A | Yes |
| CA_n66A-n77A(2A) | CA_n66A-n77A | n66A | CA_n77(2A) | n66A | n77A | No |
| CA_n66A-n77C | CA_n66A-n77A | n66A | CA_n77C | n66A | n77A | No |
| CA_n66A-n78A | CA_n66A-n78A | n66A | n78A | n66A | n78A | Yes |
| CA_n66A-n78A(2A) | CA_n66A-n78A | n66A | CA_n78(2A) | n66A | n78A | No |
| CA_n70A-n71A | CA_n70A-n71A | n70A | n71A | n70A | n71A | Yes |
| CA_n70A-n71(2A) | CA_n70A-n71A | n70A | CA_n71(2A) | n70A | n71A | No |
| CA_n71A-n77A | CA_n71A-n77A | n71A | n77A | n71A | n77A | Yes |
| CA_n71A-n77A(2A) | CA_n71A-n77A | n71A | CA_n77(2A) | n71A | n77A | No |
| CA_n71A-n78A | CA_n71A-n78A | n71A | n78A | n71A | n78A | Yes |
| CA_n71A-n78A(2A) | CA_n71A-n78A | n71A | CA_n78(2A) | n71A | n78A | No |
| CA_n77A-n79A | - | n77A | n79A | - | - | Yes |
| CA_n78A-n79A | - | n78A | n79A | - | - | Yes |
| Note 1: This band is used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 2CC. Note 3: PCell is configured on this band unless otherwise stated. |  |  |  |  |  |  |

###### 4.3.1.1.2.2	NR inter-band CA configurations in FR1 (three bands)

Table 4.3.1.1.2.2-1: Inter-band NR CA configurations within FR1 (three bands)

| NR CA configuration (Note 3) | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note 4) | NR CA downlink configuration band 2 | NR CA downlink configuration band 3 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | NR CA uplink configuration band 3 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n77A | CA_n1A-n3A | n1A | n3A | n77A | n1A | n3A | - | No |
|  | CA_n3A-n77A | n3A | n77A | n1A | n3A | n77A | - | No |
| CA_n1A-n3A-n78A | CA_n1A-n3A | n1A | n3A | n78A | n1A | n3A | - | No |
|  | CA_n1A-n78A | n1A | n78A | n3A | n1A | n78A | - | No |
|  | CA_n3A-n78A | n3A | n78A | n1A | n3A | n78A | - | No |
| CA_n1A-n8A-n78A | CA_n1A-n8A | n1A | n8A | n78A | n1A | n8A | - | No |
|  | CA_n1A-n78A | n1A | n78A | n8A | n1A | n78A | - | No |
|  | CA_n8A-n78A | n8A | n78A | n1A | n8A | n78A | - | No |
| CA_n1A-n28A-n77A | CA_n28A-n77A | n28A | n77A | n1A | n28A | n77A | - | No |
| CA_n1A-n28A-n78A | CA_n1A-n28A | n1A | n28A | n78A | n1A | n28A | - | No |
|  | CA_n1A-n78A | n1A | n78A | n28A | n1A | n78A | - | No |
|  | CA_n28A-n78A | n28A | n78A | n1A | n28A | n78A | - | No |
| CA_n1A-n41A-n77A | CA_n1A-n41A | n1A | n41A | n77A | n1A | n41A | - | No |
|  | CA_n41A-n77A | n41A | n77A | n1A | n41A | n77A | - | No |
| CA_n2A-n5A-n48A | CA_n2A-n5A | n2A | n5A | n48A | n2A | n5A | - | No |
|  | CA_n2A-n48A | n2A | n48A | n5A | n2A | n48A | - | No |
|  | CA_n5A-n48A | n5A | n48A | n2A | n5A | n48A | - | No |
| CA_n2A-n5A-n48(2A) | CA_n2A-n5A | n2A | n5A | CA_n48(2A) | n2A | n5A | - | No |
|  | CA_n2A-n48A | n2A | CA_n48(2A) | n5A | n2A | n48A | - | No |
|  | CA_n5A-n48A | n5A | CA_n48(2A) | n2A | n5A | n48A | - | No |
| CA_n2A-n5A-n48B | CA_n2A-n5A | n2A | n5A | CA_n48B | n2A | n5A | - | No |
|  | CA_n2A-n48A | n2A | CA_n48B | n5A | n2A | n48A | - | No |
|  | CA_n5A-n48A | n5A | CA_n48B | n2A | n5A | n48A | - | No |
| CA_n2A-n5A-n77A | CA_n2A-n5A | n2A | n5A | n77A | n2A | n5A | - | No |
|  | CA_n2A-n77A | n2A | n77A | n5A | n2A | n77A | - | No |
|  | CA_n5A-n77A | n5A | n77A | n2A | n5A | n77A | - | No |
| CA_n2A-n5A-n77C | - | n2A | n5A | CA_n77C | - | - | - | No |
|  | CA_n2A-n5A | n2A | n5A | CA_n77C | n2A | n5A | - | No |
|  | CA_n2A-n77A | n2A | CA_n77C | n5A | n2A | n77A | - | No |
|  | CA_n5A-n77A | n5A | CA_n77C | n2A | n5A | n77A | - | No |
|  | CA_n77C | CA_n77C | n2A | n5A | CA_n77C | - | - | No |
| CA_n2A-n48A-n66A | CA_n2A-n48A | n2A | n48A | n66A | n2A | n48A | - | No |
|  | CA_n2A-n66A | n2A | n66A | n48A | n2A | n66A | - | No |
|  | CA_n48A-n66A | n66A | n48A | n2A | n66A | n48A | - | No |
| CA_n2A-n48(2A)-n66A | CA_n2A-n48A | n2A | CA_n48(2A) | n66A | n2A | n48A | - | No |
|  | CA_n2A-n66A | n2A | n66A | CA_n48(2A) | n2A | n66A | - | No |
|  | CA_n48A-n66A | n66A | CA_n48(2A) | n2A | n66A | n48A | - | No |
| CA_n2A-n48B-n66A | CA_n2A-n48A | n2A | CA_n48B | n66A | n2A | n48A | - | No |
|  | CA_n2A-n66A | n2A | n66A | CA_n48B | n2A | n66A | - | No |
|  | CA_n48A-n66A | n66A | CA_n48B | n2A | n66A | n48A | - | No |
| CA_n2A-n48A-n77A | CA_n2A-n48A | n2A | n48A | n77A | n2A | n48A | - | No |
|  | CA_n2A-n77A | n2A | n77A | n48A | n2A | n77A | - | No |
| CA_n2A-n48A-n77C | - | n2A | n48A | CA_n77C | - | - | - | No |
|  | CA_n2A-n48A | n2A | n48A | CA_n77C | n2A | n48A | - | No |
|  | CA_n2A-n77A | n2A | CA_n77C | n48A | n2A | n77A | - | No |
|  | CA_n77C | CA_n77C | n2A | n48A | CA_n77C | - | - | No |
| CA_n2A-n48(2A)-n77A | CA_n2A-n48A | n2A | CA_n48(2A) | n77A | n2A | n48A | - | No |
|  | CA_n2A-n77A | n2A | n77A | CA_n48(2A) | n2A | n77A | - | No |
| CA_n2A-n48B-n77A | CA_n2A-n48A | n2A | CA_n48B | n77A | n2A | n48A | - | No |
|  | CA_n2A-n77A | n2A | n77A | CA_n48B | n2A | n77A | - | No |
| CA_n2A-n66A-n77A | CA_n2A-n66A | n2A | n66A | n77A | n2A | n66A | - | No |
|  | CA_n2A-n77A | n2A | n77A | n66A | n2A | n77A | - | No |
|  | CA_n66A-n77A | n66A | n77A | n2A | n66A | n77A | - | No |
| CA_n2A-n66A-n77C | - | n2A | n66A | CA_n77C | - | - | - | No |
|  | CA_n2A-n66A | n2A | n66A | CA_n77C | n2A | n66A | - | No |
|  | CA_n2A-n77A | n2A | CA_n77C | n66A | n2A | n77A | - | No |
|  | CA_n66A-n77A | n66A | CA_n77C | n2A | n66A | n77A | - | No |
| CA_n3A-n8A-n78A | CA_n3A-n78A | n3A | n78A | n8A | n3A | n78A | - | No |
|  | CA_n3A-n8A | n3A | n8A | n78A | n3A | n8A | - | No |
|  | CA_n8A-n78A | n8A | n78A | n3A | n8A | n78A | - | No |
| CA_n3A-n28A-n41A | CA_n3A-n28A | n3A | n28A | n41A | n3A | n28A | - | No |
|  | CA_n3A-n41A | n3A | n41A | n28A | n3A | n41A | - | No |
|  | CA_n28A-n41A | n28A | n41A | n3A | n28A | n41A | - | No |
| CA_n3A-n28A-n77A | CA_n3A-n28A | n3A | n28A | n77A | n3A | n28A | - | No |
|  | CA_n3A-n77A | n3A | n77A | n28A | n3A | n77A | - | No |
|  | CA_n28A-n77A | n28A | n77A | n3A | n28A | n77A | - | No |
| CA_n3A-n28A-n78A | CA_n3A-n28A | n3A | n28A | n78A | n3A | n28A | - | No |
|  | CA_n3A-n78A | n3A | n78A | n28A | n3A | n78A | - | No |
|  | CA_n28A-n78A | n28A | n78A | n3A | n28A | n78A | - | No |
| CA_n3A-n41A-n77A | CA_n3A-n41A | n3A | n41A | n77A | n3A | n41A | - | No |
|  | CA_n3A-n77A | n3A | n77A | n41A | n3A | n77A | - | No |
|  | CA_n41A-n77A | n41A | n77A | n3A | n41A | n77A | - | No |
| CA_n5A-n48A-n66A | CA_n5A-n48A | n5A | n48A | n66A | n5A | n48A | - | No |
|  | CA_n5A-n66A | n5A | n66A | n48A | n5A | n66A | - | No |
|  | CA_n48A-n66A | n66A | n48A | n5A | n66A | n48A | - | No |
| CA_n5A-n48(2A)-n66A | CA_n5A-n48A | 5A | CA_n48(2A) | n66A | n5A | n48A | - | No |
|  | CA_n5A-n66A | n5A | n66A | CA_n48(2A) | n5A | n66A | - | No |
|  | CA_n48A-n66A | n66A | CA_n48(2A) | n5A | n66A | n48A | - | No |
| CA_n5A-n48B-n66A | CA_n5A-n48A | n5A | CA_n48B | n66A | n5A | n48A | - | No |
|  | CA_n5A-n66A | n5A | n66A | CA_n48B | n5A | n66A | - | No |
|  | CA_n48A-n66A | n66A | CA_n48B | n5A | n66A | n48A | - | No |
| CA_n5A-n48A-n77A | CA_n5A-n48A | n5A | n48A | n77A | n5A | n48A | - | No |
|  | CA_n5A-n77A | n5A | n77A | n48A | n5A | n77A | - | No |
| CA_n5A-n48A-n77C | - | n5A | n48A | CA_n77C | - | - | - | No |
|  | CA_n5A-n48A | n5A | n48A | CA_n77C | n5A | n48A | - | No |
|  | CA_n5A-n77A | n5A | CA_n77C | n48A | n5A | n77A | - | No |
|  | CA_n77C | CA_n77C | n5A | n48A | CA_n77C | - | - | No |
| CA_n5A-n48(2A)-n77A | CA_n5A-n48A | n5A | CA_n48(2A) | n77A | n5A | n48A | - | No |
|  | CA_n5A-n77A | n5A | n77A | CA_n48(2A) | n5A | n77A | - | No |
| CA_n5A-n48B-n77A | CA_n5A-n48A | n5A | CA_n48B | n77A | n5A | n48A | - | No |
|  | CA_n5A-n77A | n5A | n77A | CA_n48B | n5A | n77A | - | No |
| CA_n5A-n66A-n77A | CA_n5A-n66A | n5A | n66A | n77A | n5A | n66A | - | No |
|  | CA_n5A-n77A | n5A | n77A | n66A | n5A | n77A | - | No |
|  | CA_n66A-n77A | n66A | n77A | n5A | n66A | n77A | - | No |
| CA_n5A-n66A-n77C | - | n5A | n66A | CA_n77C | - | - | - | No |
|  | CA_n5A-n66A | n5A | n66A | CA_n77C | n5A | n66A | - | No |
|  | CA_n5A-n77A | n5A | CA_n77C | n66A | n5A | n77A | - | No |
|  | CA_n66A-n77A | n66A | CA_n77C | n5A | n66A | n77A | - | No |
|  | CA_n77C | CA_n77C | n5A | n66A | CA_n77C | - | - | No |
| CA_n25A-n66A-n77A | CA_n25A-n66A | n25A | n66A | n77A | n25A | n66A | - | No |
|  | CA_n25A-n77A | n25A | n77A | n66A | n25A | n77A | - | No |
|  | CA_n66A-n77A | n66A | n77A | n25A | n66A | n77A | - | No |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A | n25A | n66A | CA_n77(2A) | n25A | n66A | - | No |
|  | CA_n25A-n77A | n25A | n66A | CA_n77(2A) | n25A | n77A | - | No |
|  | CA_n66A-n77A | n25A | n66A | CA_n77(2A) | n66A | n77A | - | No |
| CA_n25A-n66A-n78 | CA_n25A-n66A | n25A | n66A | n78A | n25A | n66A | - | No |
|  | CA_n25A-n78A | n25A | n78A | n66A | n25A | n78A | - | No |
|  | CA_n66A-n78A | n66A | n78A | n25A | n66A | n78A | - | No |
| CA_n25A-n66A-n78(2A) | CA_n25A-n66A | n25A | n66A | CA_n78(2A) | n25A | n66A | - | No |
|  | CA_n25A-n78A | n25A | n66A | CA_n78(2A) | n25A | n78A | - | No |
|  | CA_n66A-n78A | n25A | n66A | CA_n78(2A) | n66A | n78A | - | No |
| CA_n26A-n66A-n70A | CA_n26A-n66A | n26A | n66A | n70A | n26A | n66A | - | No |
|  | CA_n26A-n70A | n26A | n66A | n70A | n26A | - | n70A | No |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A | n26A | CA_n66(2A) | n70A | n26A | n66A | - | No |
|  | CA_n26A-n70A | n26A | CA_n66(2A) | n70A | n26A | - | n70A | No |
| CA_n28A-n41A-n79A | CA_n28A-n41A | n28A | n41A | n79A | n28A | n41A |  | No |
| CA_n28A-n41A-n79A | CA_n28A-n79A | n28A | n41A | n79A | n28A |  | n79A | No |
| CA_n28A-n41A-n79A | CA_n41A-n79A | n28A | n41A | n79A |  | n41A | n79A | No |
| CA_n29A-n66A-n70A | - | n29A (Note1) | n66A | n70A | - | - | - | No |
| CA_n29A-n66B-n70A | - | n29A (Note1) | CA_n66B | n70A | - | - | - | No |
| CA_n29A-n66(2A)-n70A | - | n29A (Note1) | CA_n66(2A) | n70A | - | - | - | No |
| CA_n41A-n66A-n71A | - | n41A | n66A | n71A | - | - | - | No |
| CA_n48A-n66A-n70A | CA_n48A-n66A | n48A | n66A | n70A | n48A | n66A | - | No |
|  | CA_n48A-n70A | n48A | n66A | n70A | n48A | - | n70A | No |
| CA_n48A-n66A-n71A | CA_n48A-n66A | n48A | n66A | n71A | n48A | n66A | - | No |
|  | CA_n48A-n71A | n48A | n66A | n71A | n48A | - | n71A | No |
|  | CA_n66A-n71A | n48A | n66A | n71A | - | n66A | n71A | No |
| CA_n48A-n66A-n71(2A) | CA_n48A-n66A | n48A | n66A | CA_n71(2A) | n48A | n66A | - | No |
|  | CA_n48A-n71A | n48A | n66A | CA_n71(2A) | n48A | - | n71A | No |
|  | CA_n66A-n71A | n48A | n66A | CA_n71(2A) | - | n66A | n71A | No |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A | n48A | CA_n66(2A) | n70A | n48A | n66A | - | No |
|  | CA_n48A-n70A | n48A | CA_n66(2A) | n70A | n48A | - | n70A | No |
| CA_n48A-n66(2A)-n71A | CA_n48A-n66A | n48A | CA_n66(2A) | n71A | n48A | n66A | - | No |
|  | CA_n48A-n71A | n48A | CA_n66(2A) | n71A | n48A | - | n71A | No |
|  | CA_n66A-n71A | n48A | CA_n66(2A) | n71A | - | n66A | n71A | No |
| CA_n48A-n66A-n77A | CA_n48A-n66A | n66A | n48A | n77A | n66A | n48A | - | No |
|  | CA_n66A-n77A | n66A | n77A | n48A | n66A | n77A | - | No |
| CA_n48A-n66A-n77C | - | n66A | n48A | CA_n77C | - | - | - | No |
|  | CA_n48A-n66A | n66A | n48A | CA_n77C | n66A | n48A | - | No |
|  | CA_n66A-n77A | n66A | CA_n77C | n48A | n66A | n77A |  | No |
|  | CA_n77C | CA_n77C | n66A | n48A | CA_n77C | - | - | No |
| CA_n48A-n70A-n71A | CA_n48A-n70A | n48A | n70A | n71A | n48A | n70A | - | No |
|  | CA_n48A-n71A | n48A | n70A | n71A | n48A | - | n71A | No |
|  | CA_n70A-n71A | n48A | n70A | n71A | - | n70A | n71A | No |
| CA_n48A-n70A-n71(2A) | CA_n48A-n70A | n48A | n70A | CA_n71(2A) | n48A | n70A | - | No |
|  | CA_n48A-n71A | n48A | n70A | CA_n71(2A) | n48A | - | n71A | No |
|  | CA_n70A-n71A | n48A | n70A | CA_n71(2A) | - | n70A | n71A | No |
| CA_n48B-n66A-n70A | CA_n48A-n66A | CA_n48B | n66A | n70A | n48A | n66A | - | No |
|  | CA_n48A-n70A | CA_n48B | n66A | n70A | n48A | - | n70A | No |
| CA_n48B-n66A-n71A | CA_n48A-n66A | CA_n48B | n66A | n71A | n48A | n66A | - | No |
|  | CA_n48A-n71A | CA_n48B | n66A | n71A | n48A | - | n71A | No |
|  | CA_n66A-n71A | CA_n48B | n66A | n71A | - | n66A | n71A | No |
| CA_n48B-n66A-n77A | CA_n48A-n66A | n66A | CA_n48B | n77A | n66A | n48A | - | No |
|  | CA_n66A-n77A | n66A | n77A | CA_n48B | n66A | n77A | - | No |
| CA_n48B-n70A-n71A | CA_n48A-n70A | CA_n48B | n70A | n71A | n48A | n70A | - | No |
|  | CA_n48A-n71A | CA_n48B | n70A | n71A | n48A | - | n71A | No |
|  | CA_n70A-n71A | CA_n48B | n70A | n71A | - | n70A | n71A | No |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A | CA_n48(2A) | n66A | n70A | n48A | n66A | - | No |
|  | CA_n48A-n70A | CA_n48(2A) | n66A | n70A | n48A | - | n70A | No |
| CA_n48(2A)-n66A-n71A | CA_n48A-n66A | CA_n48(2A) | n66A | n71A | n48A | n66A | - | No |
|  | CA_n48A-n71A | CA_n48(2A) | n66A | n71A | n48A | - | n71A | No |
|  | CA_n66A-n71A | CA_n48(2A) | n66A | n71A | - | n66A | n71A | No |
| CA_n48(2A)-n66A-n77A | CA_n48A-n66A | n66A | CA_n48(2A) | n77A | n66A | n48A | - | No |
|  | CA_n66A-n77A | n66A | n77A | CA_n48(2A) | n66A | n77A | - | No |
| CA_n48(2A)-n70A-n71A | CA_n48A-n70A | CA_n48(2A) | n70A | n71A | n48A | n70A | - | No |
|  | CA_n48A-n71A | CA_n48(2A) | n70A | n71A | n48A | - | n71A | No |
|  | CA_n70A-n71A | CA_n48(2A) | n70A | n71A | - | n70A | n71A | No |
| CA_n66A-n70A-n71A | CA_n66A-n71A | n66A | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | n66A | n70A | n71A | - | n70A | n71A | No |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A | n66A | n70A | CA_n71(2A) | n66A | - | n71A | No |
|  | CA_n70A-n71A | n66A | n70A | CA_n71(2A) | - | n70A | n71A | No |
| CA_n66B-n70A-n71A | CA_n66A-n71A | CA_n66B | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | CA_n66B | n70A | n71A | - | n70A | n71A | No |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A | CA_n66(2A) | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | CA_n66(2A) | n70A | n71A | - | n70A | n71A | No |
| CA_n66A-n71A-n77A | CA_n66A-n71A | n66A | n71A | n77A | n66A | n71A | - | No |
|  | CA_n66A-n77A | n66A | n77A | n71A | n66A | n77A | - | No |
|  | CA_n71A-n77A | n71A | n77A | n66A | n71A | n77A | - | No |
| CA_n66A-n71A-n77A(2A) | CA_n66A-n71A | n66A | n71A | CA_n77(2A) | n66A | n71A | - | No |
|  | CA_n66A-n77A | n66A | n71A | CA_n77(2A) | n66A | n77A | - | No |
|  | CA_n71A-n77A | n66A | n71A | CA_n77(2A) | n71A | n77A | - | No |
| CA_n66A-n71A-n78A | CA_n66A-n71A | n66A | n71A | n78A | n66A | n71A | - | No |
|  | CA_n66A-n78A | n66A | n78A | n71A | n66A | n78A | - | No |
|  | CA_n71A-n78A | n71A | n78A | n66A | n71A | n78A | - | No |
| CA_n66A-n71A-n78A(2A) | CA_n66A-n71A | n66A | n71A | CA_n78(2A) | n66A | n71A | - | No |
|  | CA_n66A-n78A | n66A | n71A | CA_n78(2A) | n66A | n78A | - | No |
|  | CA_n71A-n78A | n66A | n71A | CA_n78(2A) | n71A | n78A | - | No |
| Note 1: This band cannot be used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 3CC. Note 3: The band with the lowest UL frequency is used as PCell if nothing else is specified for in the table or in the test case for the specific configuration. Note 4: PCell is configured on this band unless otherwise stated. |  |  |  |  |  |  |  |  |

###### 4.3.1.1.2.3	NR inter-band CA configurations in FR1 (four bands)

Table 4.3.1.1.2.3-1: Inter-band NR CA configurations within FR1 (four bands)

| NR CA configuration (Note 3) | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note 1) | NR CA downlink configuration band 2 | NR CA downlink configuration band 3 | NR CA downlink configuration band 4 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | NR CA uplink configuration band 3 | NR CA uplink configuration band 4 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A-n78A | - | n1A | n3A | n28A | n78A | - | - | - | - | No |
| CA_n2A-n5A-n48A-n66A | - | n2A | n5A | n48A | n66A | - | - | - | - | No |
|  | CA_n2A-n5A | n2A | n5A | n48A | n66A | n2A | n5A | - | - | No |
|  | CA_n2A-n48A | n2A | n48A | n5A | n66A | n2A | n48A | - | - | No |
|  | CA_n2A-n66A | n2A | n66A | n5A | n48A | n2A | n66A | - | - | No |
|  | CA_n5A-n48A | n5A | n48A | n2A | n66A | n5A | n48A | - | - | No |
|  | CA_n5A-n66A | n5A | n66A | n2A | n48A | n5A | n66A | - | - | No |
|  | CA_n48A-n66A | n48A | n66A | n2A | n5A | n48A | n66A | - | - | No |
| CA_n2A-n5A-n48A-n77A | - | n2A | n5A | n48A | n77A | - | - | - | - | No |
|  | CA_n2A-n5A | n2A | n5A | n48A | n77A | n2A | n5A | - | - | No |
|  | CA_n2A-n48A | n2A | n48A | n5A | n77A | n2A | n48A | - | - | No |
|  | CA_n2A-n77A | n2A | n77A | n5A | n48A | n2A | n77A | - | - | No |
|  | CA_n5A-n48A | n5A | n48A | n2A | n77A | n5A | n48A | - | - | No |
|  | CA_n5A-n77A | n5A | n77A | n2A | n48A | n5A | n77A | - | - | No |
| CA_n2A-n5A-n48A-n77C | - | n2A | n5A | n48A | CA_n77C | - | - | - | - | No |
|  | CA_n2A-n5A | n2A | n5A | n48A | CA_n77C | n2A | n5A | - | - | No |
|  | CA_n2A-n48A | n2A | n48A | n5A | CA_n77C | n2A | n48A | - | - | No |
|  | CA_n2A-n77A | n2A | CA_n77C | n5A | n48A | n2A | n77A | - | - | No |
|  | CA_n5A-n48A | n5A | n48A | n2A | CA_n77C | n5A | n48A | - | - | No |
|  | CA_n5A-n77A | n5A | CA_n77C | n2A | n48A | n5A | n77A | - | - | No |
| CA_n2A-n5A-n66A-n77A | - | n2A | n5A | n66A | n77A | - | - | - | - | No |
|  | CA_n2A-n5A | n2A | n5A | n66A | n77A | n2A | n5A | - | - | No |
|  | CA_n2A-n66A | n2A | n66A | n5A | n77A | n2A | n66A | - | - | No |
|  | CA_n2A-n77A | n2A | n77A | n5A | n66A | n2A | n77A | - | - | No |
|  | CA_n5A-n66A | n5A | n66A | n2A | n77A | n5A | n66A | - | - | No |
|  | CA_n5A-n77A | n5A | n77A | n2A | n66A | n5A | n77A | - | - | No |
|  | CA_n66A-n77A | n66A | n77A | n2A | n5A | n66A | n77A | - | - | No |
| CA_n2A-n48A-n66A-n77A | - | n2A | n48A | n66A | n77A | - | - | - | - | No |
|  | CA_n2A-n48A | n2A | n48A | n66A | n77A | n2A | n48A | - | - | No |
|  | CA_n2A-n66A | n2A | n66A | n48A | n77A | n2A | n66A | - | - | No |
|  | CA_n2A-n77A | n2A | n77A | n48A | n66A | n2A | n77A | - | - | No |
|  | CA_n48A-n66A | n48A | n66A | n2A | n77A | n48A | n66A | - | - | No |
|  | CA_n66A-n77A | n66A | n77A | n2A | n48A | n66A | n77A | - | - | No |
| CA_n2A-n48A-n66A-n77C | - | n2A | n48A | n66A | CA_n77C | - | - | - | - | No |
|  | CA_n2A-n48A | n2A | n48A | n66A | CA_n77C | n2A | n48A | - | - | No |
|  | CA_n2A-n66A | n2A | n66A | n48A | CA_n77C | n2A | n66A | - | - | No |
|  | CA_n2A-n77A | n2A | CA_n77C | n48A | n66A | n2A | n77A | - | - | No |
|  | CA_n48A-n66A | n48A | n66A | n2A | CA_n77C | n48A | n66A | - | - | No |
|  | CA_n66A-n77A | n66A | CA_n77C | n2A | n48A | n66A | n77A | - | - | No |
| CA_n3A-n28A-n41A-n77A | - | n3A | n28A | n41A | n77A | - | - | - | - | No |
| CA_n5A-n48A-n66A-n77A | - | n5A | n48A | n66A | n77A | - | - | - | - | No |
|  | CA_n5A-n48A | n5A | n48A | n66A | n77A | n5A | n48A | - | - | No |
|  | CA_n5A-n66A | n5A | n66A | n48A | n77A | n5A | n66A | - | - | No |
|  | CA_n5A-n77A | n5A | n77A | n48A | n66A | n5A | n77A | - | - | No |
|  | CA_n48A-n66A | n48A | n66A | n5A | n77A | n48A | n66A | - | - | No |
|  | CA_n66A-n77A | n66A | n77A | n5A | n48A | n66A | n77A | - | - | No |
| CA_n5A-n48A-n66A-n77C | - | n5A | n48A | n66A | CA_n77C | - | - | - | - | No |
|  | CA_n5A-n48A | n5A | n48A | n66A | CA_n77C | n5A | n48A | - | - | No |
|  | CA_n5A-n66A | n5A | n66A | n48A | CA_n77C | n5A | n66A | - | - | No |
|  | CA_n5A-n77A | n5A | CA_n77C | n48A | n66A | n5A | n77A | - | - | No |
|  | CA_n48A-n66A | n48A | n66A | n5A | CA_n77C | n48A | n66A | - | - | No |
|  | CA_n66A-n77A | n66A | CA_n77C | n5A | n48A | n66A | n77A | - | - | No |
| Note 1: PCell is configured on this band unless otherwise stated. Note 2: Protocol testing is limited to NR CA configurations with 2CC. Note 3: The band with the lowest UL frequency is used as PCell if nothing else is specified for in the table or in the test case for the specific configuration. |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.3	NR intra-band contiguous CA in FR1

###### 4.3.1.1.3.1 – 4.3.1.1.3.4	FFS

4.3.1.1.3.5	NR Intra-band contiguous configurations CA_n5B

4.3.1.1.3.5.1	CA_n5B

Editor's note: Test frequencies for CA_n66B with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz; and SCS CC1=30kHz and SCS CC2=15 kHz is FFS.

Table 4.3.1.1.3.5.1-1: NR Intra-Band contiguous CA configuration CA_n5B (PCC=CC1 and SCC=CC2), SCS 15 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5+10 | CC1 | 5 | 15 | 25 | Downlink | Low | 871.5 | 174300 | 869.25 | 173850 | 0 | 15 | 2178 | 174270 | 8 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 876.5 | 175300 | 855.89 | 171178 | 102 |  | 2189 | 175210 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 881.8 | 176360 | 788.83 | 157766 | 504 |  | 2206 | 176450 | 12 | 0 | 2 (4) | 508 |
|  |  |  |  |  | Uplink | Low | 826.5 | 165300 | 824.25 | 164850 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 831.5 | 166300 | 738.53 | 147706 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 836.8 | 167360 | 833.47 | 166694 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=7.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 878.7 | 175740 | 874.02 | 174804 | 0 | 15 | 2190 | 175230 | 10 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 883.7 | 176740 | 860.66 | 172132 | 102 |  | 2201 | 176170 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
| 10+5 | CC1 | 10 | 15 | 52 | Downlink | Low | 874 | 174800 | 869.32 | 173864 | 0 | 15 | 2179 | 174290 | 10 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 879 | 175800 | 855.96 | 171192 | 102 |  | 2190 | 175230 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 884.3 | 176860 | 788.9 | 157780 | 504 |  | 2204 | 176410 | 6 | 1 | 1 (2) | 507 |
|  |  |  |  |  | Uplink | Low | 829 | 165800 | 824.32 | 164864 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 834 | 166800 | 738.6 | 147720 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 839.3 | 167860 | 833.54 | 166708 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=7.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 881.2 | 176240 | 878.95 | 175790 | 0 | 15 | 2203 | 176210 | 8 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 886.2 | 177240 | 865.59 | 173118 | 102 |  | 2214 | 177150 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
| 5+15 | CC1 | 5 | 15 | 25 | Downlink | Low | 871.5 | 174300 | 869.25 | 173850 | 0 | 15 | 2178 | 174270 | 8 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 874 | 174800 | 853.39 | 170678 | 102 |  | 2185 | 174770 | 8 | 1 | 0 (0) | 103 |
|  |  |  |  |  |  | High | 876.9 | 175380 | 783.93 | 156786 | 504 |  | 2193 | 175470 | 12 | 0 | 2 (4) | 508 |
|  |  |  |  |  | Uplink | Low | 826.5 | 165300 | 824.25 | 164850 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 829 | 165800 | 736.03 | 147206 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 831.9 | 166380 | 828.57 | 165714 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=9.6 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 881.1 | 176220 | 873.99 | 174798 | 0 | 15 | 2190 | 175230 | 0 | 2 | 0 (0) | 2 |
|  |  |  |  |  |  | Mid | 883.6 | 176720 | 858.13 | 171626 | 102 |  | 2197 | 175730 | 0 | 2 | 0 (0) | 104 |
|  |  |  |  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
| 15+5 | CC1 | 15 | 15 | 79 | Downlink | Low | 876.5 | 175300 | 869.39 | 173878 | 0 | 15 | 2177 | 174250 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 879 | 175800 | 853.53 | 170706 | 102 |  | 2184 | 174750 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 881.9 | 176380 | 784.07 | 156814 | 504 |  | 2192 | 175450 | 8 | 1 | 1 (2) | 507 |
|  |  |  |  |  | Uplink | Low | 831.5 | 166300 | 824.39 | 164878 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 834 | 166800 | 736.17 | 147234 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 836.9 | 167380 | 828.71 | 165742 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=9.6 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 886.1 | 177220 | 883.85 | 176770 | 0 | 15 | 2213 | 177130 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 888.6 | 177720 | 867.99 | 173598 | 102 |  | 2220 | 177630 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
| 5+5 | CC1 | 5 | 15 | 25 | Downlink | Low | 871.5 | 174300 | 869.25 | 173850 | 0 | 15 | 2178 | 174270 | 8 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 879 | 175800 | 858.39 | 171678 | 102 |  | 2196 | 175710 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 886.7 | 177340 | 793.73 | 158746 | 504 |  | 2216 | 177370 | 4 | 1 | 2 (4) | 507 |
|  |  |  |  |  | Uplink | Low | 826.5 | 165300 | 824.25 | 164850 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 834 | 166800 | 741.03 | 148206 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 841.7 | 168340 | 838.37 | 167674 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=4.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 876.3 | 175260 | 874.05 | 174810 | 0 | 15 | 2190 | 175230 | 8 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 883.8 | 176760 | 863.19 | 172638 | 102 |  | 2214 | 177150 | 4 | 1 | 6 (12) | 115 |
|  |  |  |  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2234 | 178810 | 8 | 0 | 7 (16) | 520 |
| 10+10 | CC1 | 10 | 15 | 52 | Downlink | Low | 874 | 174800 | 869.32 | 173864 | 0 | 15 | 2179 | 174290 | 10 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 876.5 | 175300 | 853.46 | 170692 | 102 |  | 2183 | 174730 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 879.1 | 175820 | 783.7 | 156740 | 504 |  | 2191 | 175250 | 2 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 829 | 165800 | 824.32 | 164864 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 831.5 | 166300 | 736.1 | 147220 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 834.1 | 166820 | 828.34 | 165668 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=9.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 883.9 | 176780 | 879.22 | 175844 | 0 | 15 | 2203 | 176210 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 886.4 | 177280 | 863.36 | 172672 | 102 |  | 2210 | 176890 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.5.1-2: NR Intra-Band contiguous CA configuration CA_n5B (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+10 | CC1 | 10 | 30 | 24 | Downlink | Low | 874 | 174800 | 869.68 | 173936 | 0 | 15 |  | 2182 | 174530 |  | 6 |  | 1 | 0 (5) | 6 |
|  |  |  |  |  |  | Mid | 876.5 | 175300 | 835.46 | 167092 | 102 |  |  | 2192 | 175450 |  | 2 |  | 0 | 8 (18) | 222 |
|  |  |  |  |  |  | High | 879.1 | 175820 | 693.34 | 138668 | 504 |  |  | 2200 | 175970 |  | 2 |  | 0 | 8 (18) | 1026 |
|  |  |  |  |  | Uplink | Low | 829 | 165800 | 824.68 | 164936 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 831.5 | 166300 | 645.74 | 129148 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 834.1 | 166820 | 827.62 | 165524 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=9.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 883.9 | 176780 | 879.58 | 175916 | 0 | 15 |  | 2212 | 176930 |  | 2 |  | 0 | 8 (18) | 18 |
|  |  |  |  |  |  | Mid | 886.4 | 177280 | 845.36 | 169072 | 102 |  |  | 2213 | 177130 |  | 10 |  | 1 | 3 (8) | 213 |
|  |  |  |  |  |  | High | 889 | 177800 | 703.24 | 140648 | 504 |  |  | 2221 | 177650 |  | 10 |  | 1 | 3 (8) | 1017 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.6 – 4.3.1.1.3.39	FFS

###### 4.3.1.1.3.40	NR Intra-band contiguous configurations CA_n40

###### 4.3.1.1.3.40.1	CA_n40B

Editor's note:	CA_n40B Test frequencies are FFS.

Table 4.3.1.1.3.40.1-1: NR Intra-Band contiguous CA configuration CA_n40B (PCC=CC1 and SCC=CC2), CC1 SCS = 15kHz, CC2 SCS = 15kHz

| CBW combination [MHz] | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.1.3.40.1-2: NR Intra-Band contiguous CA configuration CA_n40B (PCC=CC1 and SCC=CC2), CC1 SCS = 30kHz, CC2 SCS = 30kHz

| CBW combination [MHz] | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

###### 4.3.1.1.3.41	NR Intra-band contiguous configurations CA_n41

###### 4.3.1.1.3.41.1	CA_n41C

Editor's note:	Test frequencies for CA_n41C with mixed numerology with SCS CC1=15 kHz and SCS CC2=30 kHz or 60 kHz; and SCS CC1=30 kHz and SCS CC2= 30 kHz or 60kHz is FFS.

Table 4.3.1.1.3.41.1-1: NR Intra-Band contiguous CA configuration CA_n41C (PCC=CC1 and SCC=CC2), CC1 SCS = 30kHz, CC2 SCS = 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40+80 | CC1 | 40 | 30 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 |  | 6252 | 500190 |  | 4 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2497.2 | 499440 | 102 |  |  | 6345 | 507630 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2590.17 | 518034 | 2389.65 | 477930 | 504 |  |  | 6438 | 515070 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=59.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2575.83 | 515166 | 2536.77 | 507354 | 0 | 30 |  | 6351 | 508110 |  | 12 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2612.82 | 522564 | 2537.04 | 507408 | 102 |  |  | 6444 | 515550 |  | 2 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 40+100 | CC1 | 40 | 30 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 |  | 6252 | 500190 |  | 4 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2487.21 | 497442 | 102 |  |  | 6321 | 505710 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2570.28 | 514056 | 2369.76 | 473952 | 504 |  |  | 6387 | 510990 |  | 10 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2585.73 | 517146 | 2536.59 | 507318 | 0 | 30 |  | 6351 | 508110 |  | 0 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2612.73 | 522546 | 2526.87 | 505374 | 102 |  |  | 6420 | 513630 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2502.33 | 500466 | 102 |  |  | 6357 | 508590 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2605.02 | 521004 | 2399.64 | 479928 | 504 |  |  | 6462 | 516990 |  | 18 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2575.98 | 515196 | 2546.82 | 509364 | 0 | 30 |  | 6378 | 510270 |  | 14 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2617.95 | 523590 | 2552.07 | 510414 | 102 |  |  | 6483 | 518670 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2492.34 | 498468 | 102 |  |  | 6333 | 506670 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2585.13 | 517026 | 2379.75 | 475950 | 504 |  |  | 6414 | 513150 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2585.88 | 517176 | 2546.82 | 509364 | 0 | 30 |  | 6378 | 510270 |  | 14 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2617.86 | 523572 | 2542.08 | 508416 | 102 |  |  | 6456 | 516510 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2482.35 | 496470 | 102 |  |  | 6309 | 504750 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2565.24 | 513048 | 2359.86 | 471972 | 504 |  |  | 6363 | 509070 |  | 6 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2595.78 | 519156 | 2546.64 | 509328 | 0 | 30 |  | 6378 | 510270 |  | 2 |  | 0 | 3(3) | 6 |
|  |  |  |  |  | & | Mid | 2617.77 | 523554 | 2531.91 | 506382 | 102 |  |  | 6432 | 514590 |  | 0 |  | 0 | 2(2) | 208 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0(0) | 1008 |
| 60+50 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2502.12 | 500424 | 102 |  |  | 6357 | 508590 |  | 10 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2610.03 | 522006 | 2399.43 | 479886 | 504 |  |  | 6462 | 516990 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2580.96 | 516192 | 2557.02 | 511404 | 0 | 30 |  | 6402 | 512190 |  | 22 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2622.96 | 524592 | 2562.3 | 512460 | 102 |  |  | 6507 | 520590 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2497.11 | 499422 | 102 |  |  | 6345 | 507630 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2599.98 | 519996 | 2389.38 | 477876 | 504 |  |  | 6438 | 515070 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2586 | 517200 | 2556.84 | 511368 | 0 | 30 |  | 6402 | 512190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.99 | 524598 | 2557.11 | 511422 | 102 |  |  | 6495 | 519630 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2487.12 | 497424 | 102 |  |  | 6321 | 505710 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2580.09 | 516018 | 2369.49 | 473898 | 504 |  |  | 6387 | 510990 |  | 4 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2595.9 | 519180 | 2556.84 | 511368 | 0 | 30 |  | 6402 | 512190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.9 | 524580 | 2547.12 | 509424 | 102 |  |  | 6471 | 517710 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2477.13 | 495426 | 102 |  |  | 6294 | 503550 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2560.2 | 512040 | 2349.6 | 469920 | 504 |  |  | 6339 | 507150 |  | 2 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2605.8 | 521160 | 2556.66 | 511332 | 0 | 30 |  | 6402 | 512190 |  | 22 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.81 | 524562 | 2536.95 | 507390 | 102 |  |  | 6444 | 515550 |  | 8 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 80+40 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2573.01 | 514602 | 2497.23 | 499446 | 102 |  |  | 6345 | 507630 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2610.18 | 522036 | 2389.68 | 477936 | 504 |  |  | 6438 | 515070 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=59.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2595.84 | 519168 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.83 | 526566 | 2577.03 | 515406 | 102 |  |  | 6543 | 523470 |  | 0 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  |  | 6636 | 530910 |  | 2 |  | 0 | 0 (0) | 1008 |
| 80+50 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2492.22 | 498444 | 102 |  |  | 6333 | 506670 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2600.13 | 520026 | 2379.63 | 475926 | 504 |  |  | 6414 | 513150 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2600.88 | 520176 | 2576.94 | 515388 | 0 | 30 |  | 6453 | 516270 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.86 | 526572 | 2572.2 | 514440 | 102 |  |  | 6531 | 522510 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2487.21 | 497442 | 102 |  |  | 6321 | 505710 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2590.08 | 518016 | 2369.58 | 473916 | 504 |  |  | 6387 | 510990 |  | 22 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2605.92 | 521184 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.89 | 526578 | 2567.01 | 513402 | 102 |  |  | 6519 | 521550 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2477.22 | 495444 | 102 |  |  | 6294 | 503550 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2570.01 | 514002 | 2349.51 | 469902 | 504 |  |  | 6336 | 506910 |  | 0 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=79.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2616 | 523200 | 2576.94 | 515388 | 0 | 30 |  | 6453 | 516270 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.98 | 526596 | 2557.2 | 511440 | 102 |  |  | 6495 | 519630 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2467.23 | 493446 | 102 |  |  | 6270 | 501630 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2550.12 | 510024 | 2329.62 | 465924 | 504 |  |  | 6288 | 503070 |  | 22 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2625.9 | 525180 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.89 | 526578 | 2547.03 | 509406 | 102 |  |  | 6468 | 517470 |  | 0 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 100+40 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2573.01 | 514602 | 2487.15 | 497430 | 102 |  |  | 6321 | 505710 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2600.28 | 520056 | 2369.7 | 473940 | 504 |  |  | 6387 | 510990 |  | 14 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2615.73 | 523146 | 2596.65 | 519330 | 0 | 30 |  | 6501 | 520110 |  | 20 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.73 | 528546 | 2586.93 | 517386 | 102 |  |  | 6570 | 525630 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  |  | 6636 | 530910 |  | 2 |  | 0 | 0 (0) | 1008 |
| 100+50 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2482.14 | 496428 | 102 |  |  | 6306 | 504510 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2590.23 | 518046 | 2359.65 | 471930 | 504 |  |  | 6363 | 509070 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2620.77 | 524154 | 2596.83 | 519366 | 0 | 30 |  | 6501 | 520110 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.76 | 528552 | 2582.1 | 516420 | 102 |  |  | 6558 | 524670 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2477.13 | 495426 | 102 |  |  | 6294 | 503550 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2580.18 | 516036 | 2349.6 | 469920 | 504 |  |  | 6339 | 507150 |  | 2 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2625.81 | 525162 | 2596.65 | 519330 | 0 | 30 |  | 6501 | 520110 |  | 20 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.79 | 528558 | 2576.91 | 515382 | 102 |  |  | 6543 | 523470 |  | 8 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2467.14 | 493428 | 102 |  |  | 6270 | 501630 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2560.11 | 512022 | 2329.53 | 465906 | 504 |  |  | 6288 | 503070 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2635.89 | 527178 | 2596.83 | 519366 | 0 | 30 |  | 6501 | 520110 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.88 | 528576 | 2567.1 | 513420 | 102 |  |  | 6519 | 521550 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.3.42 – 4.3.1.1.3.47	FFS

###### 4.3.1.1.3.48	NR Intra-band contiguous configurations CA_n48

###### 4.3.1.1.3.48.1	CA_n48B

Editor's note:	Test frequencies for CA_n48B with mixed numerology with SCS CC1=15 kHz and SCS CC2=30 kHz or 60 kHz; and SCS CC1=30 kHz and SCS CC2= 15 kHz or 60kHz is FFS.

Table 4.3.1.1.3.48.1-1: NR Intra-Band contiguous CA configuration CA_n48B (PCC=CC1 and SCC=CC2), CC1 SCS = 15kHz, CC2 SCS = 15kHz

| CBW combination [MHz] (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 + 15 | CC1 | 5 | 15 | 25 | Downlink | Low | 3552.525 | 636835 | 3550.275 | 636685 | 0 | (Note 5) | - | - | 31 | - | - | - |
| (0) |  | Note 5 |  |  | & | Mid | 3617.505 | 641167 | 3596.895 | 639793 | 102 |  | - | - | 31 | - | - | - |
|  |  |  |  |  | Uplink | High | 3682.635 | 645509 | 3589.665 | 639311 | 504 |  | - | - | 31 | - | - | - |
|  | Channel spacing CC1-CC2=9.855 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3562.38 Note 8 | 637492 | 3555.27 | 637018 | 0 | 30 | 7888 | 637344 | 2 | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3627.36 | 641824 | 3601.89 | 640126 | 102 |  | 7933 | 641664 | 2 | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 5+20 | CC1 | 5 | 15 | 25 | Downlink | Low | 3552.525 | 636835 | 3550.275 | 636685 | 0 | (Note 5) | - | - | 31 | - | - | - |
| (0) |  | Note 5 |  |  |  | Mid | 3614.985 | 640999 | 3612.735 | 640849 | 0 |  | - | - | 31 | - | - | - |
|  |  |  |  |  |  | High | 3677.715 | 645181 | 3584.745 | 638983 | 504 |  | - | - | 31 | - | - | - |
|  | Channel spacing CC1-CC2=12.285 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3564.81 Note 8 | 637654 | 3555.27 | 637018 | 0 | 30 | 7888 | 637344 | 2 | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3627.27 Note 9 | 641818 | 3599.37 | 639958 | 102 |  | 7931 | 641472 | 2 | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 10+10 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3619.98 Note 9 | 641332 | 3596.94 | 639796 | 102 |  | 7930 | 641376 | 8 | 3 | 1 (6) | 111 |
| Note 6 |  |  |  |  | Uplink | High | 3685.02 Note 9 | 645668 | 3589.62 | 639308 | 504 |  | 7975 | 645696 | 4 | 2 | 1 (6) | 512 |
|  | Channel spacing CC1-CC2=9.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3564.96 | 637664 | 3560.28 | 637352 | 0 | 30 | 7891 | 637632 | 4 | 1 | 0 (2) | 3 |
|  |  |  |  |  | & | Mid | 3629.94 | 641996 | 3606.9 | 640460 | 102 |  | 7936 | 641952 | 4 | 0 | 0 (2) | 104 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 10+15 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 Note 9 | 641166 | 3594.45 | 639630 | 102 |  | 7928 | 641184 | 6 | 1 | 1 (6) | 109 |
| Note 6 |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3584.61 | 638974 | 504 |  | 7971 | 645312 | 2 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3567.48 | 637832 | 3560.37 | 637358 | 0 | 30 | 7891 | 637632 | 10 | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3629.97 | 641998 | 3604.5 | 640300 | 102 |  | 7935 | 641856 | 8 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 10+20 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3591.96 | 639464 | 102 |  | 7926 | 640992 | 4 | 3 | 0 (2) | 107 |
| Note 6 |  |  |  |  | Uplink | High | 3675.36 | 645024 | 3579.96 | 638664 | 504 |  | 7968 | 645024 | 0 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3569.64 | 637976 | 3560.1 | 637340 | 0 | 30 | 7891 | 637632 | 4 | 2 | 0 (2) | 4 |
|  |  |  |  |  | & | Mid | 3629.64 | 641976 | 3601.74 | 640116 | 102 |  | 7933 | 641664 | 0 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 10+30 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3609.99 Note 9 | 640666 | 3586.95 | 639130 | 102 |  | 7923 | 640704 | 2 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3665.31 | 644354 | 3569.91 | 637994 | 504 |  | 7961 | 644352 | 10 | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3574.68 | 638312 | 3560.28 | 637352 | 0 | 30 | 7891 | 637632 | 4 | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3629.67 | 641978 | 3596.91 | 639794 | 102 |  | 7930 | 641376 | 10 | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 10+40 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3604.98 Note 9 | 640332 | 3581.94 | 638796 | 102 |  | 7919 | 640320 | 0 | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3655.62 Note 9 | 643708 | 3560.22 | 637348 | 504 |  | 7954 | 643680 | 8 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3579.36 | 638624 | 3559.92 | 637328 | 0 | 30 | 7891 | 637632 | 4 | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3629.34 | 641956 | 3591.54 | 639436 | 102 |  | 7926 | 640992 | 8 | 1 | 1 (6) | 109 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 10+50 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3576.96 | 638464 | 102 |  | 7916 | 640032 | 8 | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3645.6 | 643040 | 3550.2 | 636680 | 504 |  | 7947 | 643008 | 4 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=29.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 Note 7 | 15 | 270 | Downlink | Low | 3584.4 | 638960 | 3560.1 | 637340 | 0 | 30 | - | 637632 | 31 | - | - | - |
|  |  |  |  |  |  | Mid | 3629.4 | 641960 | 3586.74 | 639116 | 102 |  | - | 640608 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| 15+10 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 Note 8 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
| (0) |  |  |  |  | & | Mid | 3619.98 Note 9 | 641332 | 3594.51 | 639634 | 102 |  | 7928 | 641184 | 2 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3682.5 Note 9 | 645500 | 3584.67 | 638978 | 504 |  | 7971 | 645312 | 10 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3570 | 638000 | 3565.32 | 637688 | 0 | 30 | 7895 | 638016 | 4 | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3632.46 | 642164 | 3609.42 | 640628 | 102 |  | 7938 | 642144 | 4 | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 15+15 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 Note 8 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 Note 9 | 641166 | 3592.02 | 639468 | 102 |  | 7926 | 640992 | 0 | 3 | 0 (2) | 107 |
| Note 6 |  |  |  |  | Uplink | High | 3677.49 | 645166 | 3579.66 | 638644 | 504 |  | 7968 | 645024 | 8 | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3572.52 | 638168 | 3565.41 | 637694 | 0 | 30 | 7895 | 638016 | 10 | 0 | 1 (6) | 6 |
|  |  |  |  |  | & | Mid | 3632.49 | 642166 | 3607.02 | 640468 | 102 |  | 7937 | 642048 | 8 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 0 | 30 | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 15+20 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3589.53 | 639302 | 102 |  | 7924 | 640800 | 10 | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3672.84 | 644856 | 3575.01 | 638334 | 504 |  | 7965 | 644736 | 6 | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3574.68 | 638312 | 3565.14 | 637676 | 0 | 30 | 7895 | 638016 | 4 | 2 | 1 (6) | 8 |
|  |  |  |  |  | & | Mid | 3632.16 | 642144 | 3604.26 | 640284 | 102 |  | 7935 | 641856 | 0 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 15+30 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 Note 8 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3609.99 Note 9 | 640666 | 3584.52 | 638968 | 102 |  | 7921 | 640512 | 8 | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3662.79 | 644186 | 3564.96 | 637664 | 504 |  | 7958 | 644064 | 4 | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3579.72 | 638648 | 3565.32 | 637688 | 0 | 30 | 7895 | 638016 | 4 | 1 | 1 (6) | 7 |
|  |  |  |  |  |  | Mid | 3632.19 | 642146 | 3599.43 | 639962 | 102 |  | 7931 | 641472 | 10 | 1 | 0 (2) | 105 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 15+40 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 Note 8 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
| (0,1,2) |  |  |  |  | & | Mid | 3604.98 Note 9 | 640332 | 3579.51 | 638634 | 102 |  | 7917 | 640128 | 6 | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3653.1 Note 9 | 643540 | 3555.27 | 637018 | 504 |  | 7951 | 643392 | 2 | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3584.4 | 638960 | 3564.96 | 637664 | 0 | 30 | 7895 | 638016 | 4 | 3 | 1 (6) | 9 |
|  |  |  |  |  |  | Mid | 3631.86 | 642124 | 3594.06 | 639604 | 102 |  | 7928 | 641184 | 8 | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 15+50 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 Note 8 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3574.53 | 638302 | 102 |  | 7914 | 639840 | 2 | 0 | 1 (6) | 108 |
| Note 6 |  |  |  |  | Uplink | High | 3643.08 | 642872 | 3545.25 | 636350 | 504 |  | 7944 | 642720 | 10 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=31.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3589.425 | 639295 | 3565.125 | 637675 | 0 | 30 | - | 638016 | 31 | - | - | - |
|  |  | Note 7 |  |  |  | Mid | 3631.92 | 642128 | 3589.26 | 639284 | 102 |  | - | 640800 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| 20+10 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3619.98 Note 9 | 641332 | 3592.08 | 639472 | 102 |  | 7926 | 640992 | 8 | 2 | 0 (2) | 106 |
| Note 6 |  |  |  |  | Uplink | High | 3680.34 Note 9 | 645356 | 3580.08 | 638672 | 504 |  | 7968 | 645024 | 4 | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3574.65 | 638310 | 3569.97 | 637998 | 0 | 30 | 7898 | 638304 | 6 | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3634.62 | 642308 | 3611.58 | 640772 | 102 |  | 7940 | 642336 | 4 | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 20+15 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 Note 9 | 641166 | 3589.59 | 639306 | 102 |  | 7924 | 640800 | 6 | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3675.33 | 645022 | 3575.07 | 638338 | 504 |  | 7965 | 644736 | 2 | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3577.17 | 638478 | 3570.06 | 638004 | 0 | 30 | 7898 | 638304 | 0 | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3634.65 | 642310 | 3609.18 | 640612 | 102 |  | 7938 | 642144 | 8 | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 20+20 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3587.1 | 639140 | 102 |  | 7923 | 640704 | 4 | 2 | 1 (6) | 110 |
| Note 6 |  |  |  |  | Uplink | High | 3670.02 | 644668 | 3569.76 | 637984 | 504 |  | 7961 | 644352 | 8 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=19.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3579.99 | 638666 | 3570.45 | 638030 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3634.98 | 642332 | 3607.08 | 640472 | 102 |  | 7937 | 642048 | 4 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 20+30 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3609.99 Note 9 | 640666 | 3582.09 | 638806 | 102 |  | 7919 | 640320 | 2 | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3660.03 | 644002 | 3559.77 | 637318 | 504 |  | 7954 | 643680 | 2 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3584.97 | 638998 | 3570.57 | 638038 | 0 | 30 | 7898 | 638304 | 2 | 0 | 0 (2) | 2 |
|  |  |  |  |  |  | Mid | 3634.95 | 642330 | 3602.19 | 640146 | 102 |  | 7933 | 641664 | 6 | 2 | 0 (2) | 106 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 20+40 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3604.98 Note 9 | 640332 | 3577.08 | 638472 | 102 |  | 7916 | 640032 | 0 | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3650.28 Note 9 | 643352 | 3550.02 | 636668 | 504 |  | 7947 | 643008 | 4 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3589.71 | 639314 | 3570.27 | 638018 | 0 | 30 | 7898 | 638304 | 10 | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3634.68 | 642312 | 3596.88 | 639792 | 102 |  | 7929 | 641280 | 0 | 0 | 0 (2) | 104 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 20+50 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3572.1 | 638140 | 102 |  | 7912 | 639648 | 8 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3640.26 | 642684 | 3540 | 636000 | 504 |  | 7940 | 642336 | 0 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3594.75 | 639650 | 3570.45 | 638030 | 0 | 30 | - | 638304 | 31 | - | - | - |
|  |  | Note 7 |  |  |  | Mid | 3634.74 | 642316 | 3592.08 | 639472 | 102 |  | - | 640992 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| 30+10 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.02 Note 8 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3619.98 Note 9 | 641332 | 3587.22 | 639148 | 102 |  | 7923 | 640704 | 8 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3675.3 Note 9 | 645020 | 3570.18 | 638012 | 504 |  | 7961 | 644352 | 4 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3584.7 | 638980 | 3580.02 | 638668 | 0 | 30 | 7905 | 638976 | 8 | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3639.66 | 642644 | 3616.62 | 641108 | 102 |  | 7943 | 642624 | 4 | 2 | 0 (2) | 106 |
|  |  |  |  |  |  | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 30+15 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.02 Note 8 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3617.49 Note 9 | 641166 | 3584.73 | 638982 | 102 |  | 7921 | 640512 | 6 | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3670.29 | 644686 | 3565.17 | 637678 | 504 |  | 7958 | 644064 | 2 | 2 | 1 (6) | 512 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3587.22 | 639148 | 3580.11 | 638674 | 0 | 30 | 7905 | 638976 | 2 | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3639.69 | 642646 | 3614.22 | 640948 | 102 |  | 7942 | 642528 | 8 | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 30+20 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.02 Note 8 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3615 | 641000 | 3582.24 | 638816 | 102 |  | 7919 | 640320 | 4 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3665.04 | 644336 | 3559.92 | 637328 | 504 |  | 7954 | 643680 | 4 | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3589.98 | 639332 | 3580.44 | 638696 | 0 | 30 | 7905 | 638976 | 4 | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3639.96 | 642664 | 3612.06 | 640804 | 102 |  | 7940 | 642336 | 8 | 3 | 0 (2) | 107 |
|  |  |  |  |  |  | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 30+30 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.02 Note 8 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3609.99 Note 9 | 640666 | 3577.23 | 638482 | 102 |  | 7916 | 640032 | 2 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3654.99 | 643666 | 3549.87 | 636658 | 504 |  | 7947 | 643008 | 2 | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=30 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3595.02 | 639668 | 3580.62 | 638708 | 0 | 30 | 7905 | 638976 | 4 | 0 | 0 (2) | 2 |
|  |  |  |  |  |  | Mid | 3639.99 | 642666 | 3607.23 | 640482 | 102 |  | 7937 | 642048 | 6 | 2 | 1 (6) | 110 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 30+40 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.02 Note 8 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3604.98 Note 9 | 640332 | 3572.22 | 638148 | 102 |  | 7912 | 639648 | 0 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3645.3 Note 9 | 643020 | 3540.18 | 636012 | 504 |  | 7940 | 642336 | 0 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3599.7 | 639980 | 3580.26 | 638684 | 0 | 30 | 7905 | 638976 | 4 | 2 | 0 (2) | 4 |
|  |  |  |  |  |  | Mid | 3639.66 | 642644 | 3601.86 | 640124 | 102 |  | 7933 | 641664 | 4 | 0 | 1 (6) | 108 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 30+50 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.02 Note 8 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3600 | 640000 | 3567.24 | 637816 | 102 |  | 7909 | 639360 | 8 | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3635.28 | 642352 | 3530.16 | 635344 | 504 |  | 7933 | 641664 | 8 | 0 | 0 (2) | 506 |
|  | Channel spacing CC1-CC2=39.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3604.725 | 640315 | 3580.425 | 638695 | 0 | 30 | - | 638976 | 31 | - | - | - |
|  |  | Note 7 |  |  |  | Mid | 3639.72 | 642648 | 3597.06 | 639804 | 102 |  | - | 641376 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| 40+10 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3619.98 Note 9 | 641332 | 3582.18 | 638812 | 102 |  | 7919 | 640320 | 8 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3670.62 Note 9 | 644708 | 3560.46 | 637364 | 504 |  | 7954 | 643680 | 4 | 0 | 0 (2) | 506 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3594.36 | 639624 | 3589.68 | 639312 | 0 | 30 | 7912 | 639648 | 0 | 2 | 1 (6) | 8 |
|  |  |  |  |  |  | Mid | 3644.34 | 642956 | 3621.3 | 641420 | 102 |  | 7946 | 642912 | 4 | 0 | 0 (2) | 104 |
|  |  |  |  |  |  | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 40+15 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3617.49 Note 9 | 641166 | 3579.69 | 638646 | 102 |  | 7918 | 640224 | 6 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3665.61 | 644374 | 3555.45 | 637030 | 504 |  | 7951 | 643392 | 2 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3596.88 | 639792 | 3589.77 | 639318 | 0 | 30 | 7912 | 639648 | 6 | 1 | 1 (6) | 7 |
|  |  |  |  |  |  | Mid | 3644.37 | 642958 | 3618.9 | 641260 | 102 |  | 7945 | 642816 | 8 | 1 | 1 (6) | 109 |
|  |  |  |  |  |  | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 40+20 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3615 | 641000 | 3577.2 | 638480 | 102 |  | 7916 | 640032 | 4 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3660.3 | 644020 | 3550.14 | 636676 | 504 |  | 7947 | 643008 | 8 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3599.7 | 639980 | 3590.16 | 639344 | 0 | 30 | 7912 | 639648 | 4 | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3644.7 | 642980 | 3616.8 | 641120 | 102 |  | 7943 | 642624 | 4 | 1 | 0 (2) | 105 |
|  |  |  |  |  |  | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 40+30 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3609.99 Note 9 | 640666 | 3572.19 | 638146 | 102 |  | 7912 | 639648 | 2 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3650.31 | 643354 | 3540.15 | 636010 | 504 |  | 7940 | 642336 | 2 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3604.68 | 640312 | 3590.28 | 639352 | 0 | 30 | 7912 | 639648 | 8 | 2 | 0 (2) | 4 |
|  |  |  |  |  |  | Mid | 3644.67 | 642978 | 3611.91 | 640794 | 102 |  | 7940 | 642336 | 6 | 0 | 1 (6) | 108 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 40+40 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3604.98 Note 9 | 640332 | 3567.18 | 637812 | 102 |  | 7909 | 639360 | 0 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3640.02 Note 9 | 642668 | 3529.86 | 635324 | 504 |  | 7933 | 641664 | 4 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=39.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3609.96 | 640664 | 3590.52 | 639368 | 0 | 30 | 7912 | 639648 | 4 | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3644.94 | 642996 | 3607.14 | 640476 | 102 |  | 7937 | 642048 | 0 | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 40+50 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3562.2 | 637480 | 102 |  | 7905 | 638976 | 8 | 0 | 0 (2) | 104 |
|  |  |  |  |  | Uplink | High | 3630.06 | 642004 | 3519.9 | 634660 | 504 |  | 7926 | 640992 | 8 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=44.94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 Note 7 | 15 | 270 | Downlink | Low | 3614.94 | 640996 | 3590.64 | 639376 | 0 | 30 | - | 639648 | 31 | - | - | - |
|  |  |  |  |  |  | Mid | 3644.94 | 642996 | 3602.28 | 640152 | 102 |  | - | 641664 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell for CBW>5 MHz if nothing else is specified in the test case. For the case CC1 CBW=5 MHz then CC2 is used as Pcell- Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 5: This UE channel bandwidth is restricted for operation when carrier is configured as a downlink SCell part of a DC or CA configuration (TS 38.101-1, Table 5.3.5-1 Note 6). No SS/PBCH block fits within the channel bandwidth. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 6: UL CA is only supported for BCS0. I.e.the test frequencies for uplink CC2 are only supported for BCS0. Note 7: This UE channel bandwidth is restricted for operation when carrier is configured as a downlink SCell part of a DC or CA configuration (TS 38.101-1, Table 5.3.5-1 Note 7). As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 8: Centre frequency shifted 15 kHz to align with the synchronisation raster. Note 9: Centre frequency shifted -15 kHz to align with the synchronisation raster. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.48.1-2: NR Intra-Band contiguous CA configuration CA_n48B (PCC=CC1 and SCC=CC2), CC1 SCS = 30kHz, CC2 SCS = 30kHz

| CBW combination [MHz] (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+10 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3578.97 | 638598 | 102 |  | 7930 | 641376 | 18 | 0 | 3 (3) | 210 |
| Note 5 |  |  |  |  | Uplink | High | 3685.02 | 645668 | 3499.26 | 633284 | 504 |  | 7975 | 645696 | 4 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=9.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3564.96 | 637664 | 3560.64 | 637376 | 0 | 30 | 7891 | 637632 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3629.97 | 641998 | 3588.93 | 639262 | 102 |  | 7936 | 641952 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 10+15 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3576.45 | 638430 | 102 |  | 7928 | 641184 | 18 | 0 | 2 (2) | 208 |
| Note 5 |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3494.25 | 632950 | 504 |  | 7971 | 645312 | 2 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3567.48 | 637832 | 3560.64 | 637376 | 0 | 30 | 7891 | 637632 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3629.97 | 641998 | 3586.41 | 639094 | 102 |  | 7935 | 641856 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 10+20 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3573.96 | 638264 | 102 |  | 7926 | 640992 | 16 | 0 | 1 (1) | 206 |
| Note 5 |  |  |  |  | Uplink | High | 3675.36 | 645024 | 3489.6 | 632640 | 504 |  | 7968 | 645024 | 0 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3569.64 | 637976 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3629.64 | 641976 | 3583.74 | 638916 | 102 |  | 7933 | 641664 | 12 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 10+30 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3568.95 | 637930 | 102 |  | 7923 | 640704 | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3665.31 | 644354 | 3479.55 | 631970 | 504 |  | 7961 | 644352 | 22 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3574.68 | 638312 | 3560.64 | 637376 | 0 | 30 | 7891 | 637632 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3629.67 | 641978 | 3578.91 | 638594 | 102 |  | 7930 | 641376 | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 10+40 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3563.97 | 637598 | 102 |  | 7919 | 640320 | 10 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3655.62 | 643708 | 3469.86 | 631324 | 504 |  | 7954 | 643680 | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3579.36 | 638624 | 3560.28 | 637352 | 0 | 30 | 7891 | 637632 | 16 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.37 | 641958 | 3573.57 | 638238 | 102 |  | 7926 | 640992 | 18 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 10+70 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3590.01 | 639334 | 3548.97 | 636598 | 102 |  | 7909 | 639360 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3625.5 | 641700 | 3439.74 | 629316 | 504 |  | 7933 | 641664 | 12 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=39.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3594.48 | 639632 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.49 | 641966 | 3558.75 | 637250 | 102 |  | 7916 | 640032 | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | 7940 | 642336 | 8 | 0 | 1 (1) | 1010 |
| 10+50 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3558.96 | 637264 | 102 |  | 7916 | 640032 | 8 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3645.6 | 643040 | 3459.84 | 630656 | 504 |  | 7947 | 643008 | 16 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=29.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3584.4 | 638960 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.4 | 641960 | 3568.74 | 637916 | 102 |  | 7922 | 640608 | 4 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 10+60 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3553.95 | 636930 | 102 |  | 7912 | 639648 | 6 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3635.55 | 642370 | 3449.79 | 629986 | 504 |  | 7940 | 642336 | 14 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=34.44 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3589.44 | 639296 | 3560.28 | 637352 | 0 | 30 | 7891 | 637632 | 16 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.43 | 641962 | 3563.55 | 637570 | 102 |  | 7919 | 640320 | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| 10+80 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3585 | 639000 | 3543.96 | 636264 | 102 |  | 7905 | 638976 | 0 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3615.48 | 641032 | 3429.72 | 628648 | 504 |  | 7926 | 640992 | 8 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=44.52 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3599.52 | 639968 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.52 | 641968 | 3553.74 | 636916 | 102 |  | 7912 | 639648 | 20 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  | 7933 | 641664 | 4 | 0 | 1 (1) | 1010 |
| 10+90 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3579.99 | 638666 | 3538.95 | 635930 | 102 |  | 7902 | 638688 | 22 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3605.43 | 640362 | 3419.67 | 627978 | 504 |  | 7919 | 640320 | 6 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=49.56 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3604.56 | 640304 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.55 | 641970 | 3548.73 | 636582 | 102 |  | 7909 | 639360 | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3654.99 | 643666 | 3429.45 | 628630 | 504 |  | 7926 | 640992 | 2 | 0 | 1 (1) | 1010 |
| 15+10 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3576.45 | 638430 | 102 |  | 7928 | 641184 | 18 | 0 | 2 (2) | 208 |
| Note 5 |  |  |  |  | Uplink | High | 3682.5 | 645500 | 3494.22 | 632948 | 504 |  | 7971 | 645312 | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3570 | 638000 | 3565.68 | 637712 | 0 | 30 | 7895 | 638016 | 16 | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3632.49 | 642166 | 3591.45 | 639430 | 102 |  | 7938 | 642144 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 15+15 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3573.93 | 638262 | 102 |  | 7926 | 640992 | 18 | 0 | 1 (1) | 206 |
| Note 5 |  |  |  |  | Uplink | High | 3677.49 | 645166 | 3489.21 | 632614 | 504 |  | 7968 | 645024 | 2 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3572.52 | 638168 | 3565.68 | 637712 | 0 | 30 | 7895 | 638016 | 16 | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3632.49 | 642166 | 3588.93 | 639262 | 102 |  | 7936 | 641952 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 15+20 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3571.44 | 638096 | 102 |  | 7924 | 640800 | 16 | 0 | 0 (0) | 204 |
| Note 5 |  |  |  |  | Uplink | High | 3672.84 | 644856 | 3484.56 | 632304 | 504 |  | 7964 | 644640 | 0 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3574.68 | 638312 | 3565.5 | 637700 | 0 | 30 | 7895 | 638016 | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3632.16 | 642144 | 3586.26 | 639084 | 102 |  | 7935 | 641856 | 12 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 15+30 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3566.43 | 637762 | 102 |  | 7921 | 640512 | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3662.79 | 644186 | 3474.51 | 631634 | 504 |  | 7958 | 644064 | 22 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3579.72 | 638648 | 3565.68 | 637712 | 0 | 30 | 7895 | 638016 | 16 | 0 | 2 (2) | 4 |
|  |  |  |  |  |  | Mid | 3632.19 | 642146 | 3581.43 | 638762 | 102 |  | 7931 | 641472 | 22 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 15+70 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3590.01 | 639334 | 3546.45 | 636430 | 102 |  | 7907 | 639168 | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3622.98 | 641532 | 3434.7 | 628980 | 504 |  | 7930 | 641376 | 12 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=42 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3599.52 | 639968 | 3565.5 | 637700 | 0 | 30 | 7895 | 638016 | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3632.01 | 642134 | 3561.27 | 637418 | 102 |  | 7917 | 640128 | 22 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | 7940 | 642336 | 8 | 0 | 1 (1) | 1010 |
| 15+40 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3561.45 | 637430 | 102 |  | 7917 | 640128 | 10 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3653.1 | 643540 | 3464.82 | 630988 | 504 |  | 7951 | 643392 | 20 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3584.4 | 638960 | 3565.32 | 637688 | 0 | 30 | 7895 | 638016 | 16 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3631.89 | 642126 | 3576.09 | 638406 | 102 |  | 7928 | 641184 | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 15+50 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3556.44 | 637096 | 102 |  | 7914 | 639840 | 8 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3643.08 | 642872 | 3454.8 | 630320 | 504 |  | 7944 | 642720 | 16 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=31.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3589.44 | 639296 | 3565.5 | 637700 | 0 | 30 | 7895 | 638016 | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3631.92 | 642128 | 3571.26 | 638084 | 102 |  | 7924 | 640800 | 4 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 15+60 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3551.43 | 636762 | 102 |  | 7910 | 639456 | 6 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3633.03 | 642202 | 3444.75 | 629650 | 504 |  | 7937 | 642048 | 14 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=36.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3594.48 | 639632 | 3565.32 | 637688 | 0 | 30 | 7895 | 638016 | 16 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3631.95 | 642130 | 3566.07 | 637738 | 102 |  | 7921 | 640512 | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| 15+80 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3585 | 639000 | 3541.44 | 636096 | 102 |  | 7903 | 638784 | 0 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3612.96 | 640864 | 3424.68 | 628312 | 504 |  | 7923 | 640704 | 8 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=47.04 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3604.56 | 640304 | 3565.5 | 637700 | 0 | 30 | 7895 | 638016 | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3632.04 | 642136 | 3556.26 | 637084 | 102 |  | 7914 | 639840 | 20 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  | 7933 | 641664 | 4 | 0 | 1 (1) | 1010 |
| 20+10 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (0,2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3574.11 | 638274 | 102 |  | 7926 | 640992 | 6 | 0 | 1 (1) | 206 |
| Note 5 |  |  |  |  | Uplink | High | 3680.34 | 645356 | 3489.72 | 632648 | 504 |  | 7968 | 645024 | 16 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3574.65 | 638310 | 3570.33 | 638022 | 0 | 30 | 7898 | 638304 | 18 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3634.65 | 642310 | 3593.61 | 639574 | 102 |  | 7940 | 642336 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 20+15 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3571.59 | 638106 | 102 |  | 7924 | 640800 | 6 | 0 | 0 (0) | 204 |
| Note 5 |  |  |  |  | Uplink | High | 3675.33 | 645022 | 3484.71 | 632314 | 504 |  | 7965 | 644736 | 14 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3577.17 | 638478 | 3570.33 | 638022 | 0 | 30 | 7898 | 638304 | 18 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3634.65 | 642310 | 3591.09 | 639406 | 102 |  | 7938 | 642144 | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 20+20 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3569.1 | 637940 | 102 |  | 7923 | 640704 | 4 | 0 | 3 (3) | 210 |
| Note 5 |  |  |  |  | Uplink | High | 3670.02 | 644668 | 3479.4 | 631960 | 504 |  | 7961 | 644352 | 8 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=19.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3579.99 | 638666 | 3570.81 | 638054 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3634.98 | 642332 | 3589.08 | 639272 | 102 |  | 7937 | 642048 | 16 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 20+30 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3564.09 | 637606 | 102 |  | 7919 | 640320 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3660.03 | 644002 | 3469.41 | 631294 | 504 |  | 7954 | 643680 | 2 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3584.97 | 638998 | 3570.93 | 638062 | 0 | 30 | 7898 | 638304 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.95 | 642330 | 3584.19 | 638946 | 102 |  | 7933 | 641664 | 6 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 20+70 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3590.01 | 639334 | 3544.11 | 636274 | 102 |  | 7905 | 638976 | 14 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3620.16 | 641344 | 3429.54 | 628636 | 504 |  | 7926 | 640992 | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=44.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3604.83 | 640322 | 3570.81 | 638054 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.83 | 642322 | 3564.09 | 637606 | 102 |  | 7919 | 640320 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | 7940 | 642336 | 8 | 0 | 1 (1) | 1010 |
| 30+10 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3569.25 | 637950 | 102 |  | 7923 | 640704 | 18 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3675.3 | 645020 | 3479.82 | 631988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3584.7 | 638980 | 3580.38 | 638692 | 0 | 30 | 7905 | 638976 | 20 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3639.69 | 642646 | 3598.65 | 639910 | 102 |  | 7943 | 642624 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 30+15 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3566.73 | 637782 | 102 |  | 7921 | 640512 | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3670.29 | 644686 | 3474.81 | 631654 | 504 |  | 7958 | 644064 | 2 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3587.22 | 639148 | 3580.38 | 638692 | 0 | 30 | 7905 | 638976 | 20 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3639.69 | 642646 | 3596.13 | 639742 | 102 |  | 7941 | 642432 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 30+20 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3615 | 641000 | 3564.24 | 637616 | 102 |  | 7919 | 640320 | 16 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3665.04 | 644336 | 3469.56 | 631304 | 504 |  | 7954 | 643680 | 16 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3589.98 | 639332 | 3580.8 | 638720 | 0 | 30 | 7905 | 638976 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3639.96 | 642664 | 3594.06 | 639604 | 102 |  | 7940 | 642336 | 20 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 30+30 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3559.23 | 637282 | 102 |  | 7916 | 640032 | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3654.99 | 643666 | 3459.51 | 630634 | 504 |  | 7947 | 643008 | 14 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=30 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3595.02 | 639668 | 3580.98 | 638732 | 0 | 30 | 7905 | 638976 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3639.99 | 642666 | 3589.23 | 639282 | 102 |  | 7937 | 642048 | 6 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 30+40 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3554.25 | 636950 | 102 |  | 7912 | 639648 | 10 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3645.3 | 643020 | 3449.82 | 629988 | 504 |  | 7940 | 642336 | 12 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3599.7 | 639980 | 3580.62 | 638708 | 0 | 30 | 7905 | 638976 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3639.69 | 642646 | 3583.89 | 638926 | 102 |  | 7933 | 641664 | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 30+50 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3600 | 640000 | 3549.24 | 636616 | 102 |  | 7909 | 639360 | 8 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3635.28 | 642352 | 3439.8 | 629320 | 504 |  | 7933 | 641664 | 8 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=39.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3604.74 | 640316 | 3580.8 | 638720 | 0 | 30 | 7905 | 638976 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3639.72 | 642648 | 3579.06 | 638604 | 102 |  | 7930 | 641376 | 12 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 30+60 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3544.23 | 636282 | 102 |  | 7905 | 638976 | 6 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3625.23 | 641682 | 3429.75 | 628650 | 504 |  | 7926 | 640992 | 6 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=44.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3609.78 | 640652 | 3580.62 | 638708 | 0 | 30 | 7905 | 638976 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3639.75 | 642650 | 3573.87 | 638258 | 102 |  | 7926 | 640992 | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| 30+70 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3590.01 | 639334 | 3539.25 | 635950 | 102 |  | 7902 | 638688 | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3615.18 | 641012 | 3419.7 | 627980 | 504 |  | 7919 | 640320 | 4 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=49.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3614.82 | 640988 | 3580.8 | 638720 | 0 | 30 | 7905 | 638976 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3639.81 | 642654 | 3569.07 | 637938 | 102 |  | 7923 | 640704 | 6 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | 7940 | 642336 | 8 | 0 | 1 (1) | 1010 |
| 40+10 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3564.21 | 637614 | 102 |  | 7919 | 640320 | 18 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3670.62 | 644708 | 3470.1 | 631340 | 504 |  | 7954 | 643680 | 4 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3594.36 | 639624 | 3590.04 | 639336 | 0 | 30 | 7912 | 639648 | 0 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3644.37 | 642958 | 3603.33 | 640222 | 102 |  | 7946 | 642912 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 40+15 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3561.69 | 637446 | 102 |  | 7918 | 640224 | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3665.61 | 644374 | 3465.09 | 631006 | 504 |  | 7951 | 643392 | 2 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3596.88 | 639792 | 3590.04 | 639336 | 0 | 30 | 7912 | 639648 | 0 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3644.37 | 642958 | 3600.81 | 640054 | 102 |  | 7945 | 642816 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 40+20 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3615 | 641000 | 3559.2 | 637280 | 102 |  | 7916 | 640032 | 16 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3660.3 | 644020 | 3459.78 | 630652 | 504 |  | 7947 | 643008 | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3599.7 | 639980 | 3590.52 | 639368 | 0 | 30 | 7912 | 639648 | 16 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3644.7 | 642980 | 3598.8 | 639920 | 102 |  | 7943 | 642624 | 16 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 40+30 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3554.19 | 636946 | 102 |  | 7912 | 639648 | 14 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3650.31 | 643354 | 3449.79 | 629986 | 504 |  | 7940 | 642336 | 14 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3604.68 | 640312 | 3590.64 | 639376 | 0 | 30 | 7912 | 639648 | 8 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3644.67 | 642978 | 3593.91 | 639594 | 102 |  | 7940 | 642336 | 6 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 20+40 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3559.11 | 637274 | 102 |  | 7916 | 640032 | 22 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3650.28 | 643352 | 3459.66 | 630644 | 504 |  | 7947 | 643008 | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3589.71 | 639314 | 3570.63 | 638042 | 0 | 30 | 7898 | 638304 | 22 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.71 | 642314 | 3578.91 | 638594 | 102 |  | 7930 | 641376 | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 20+50 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3554.1 | 636940 | 102 |  | 7912 | 639648 | 20 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3640.26 | 642684 | 3449.64 | 629976 | 504 |  | 7940 | 642336 | 0 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3594.75 | 639650 | 3570.81 | 638054 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.74 | 642316 | 3574.08 | 638272 | 102 |  | 7926 | 640992 | 8 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 20+60 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3549.09 | 636606 | 102 |  | 7909 | 639360 | 18 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3630.21 | 642014 | 3439.59 | 629306 | 504 |  | 7933 | 641664 | 22 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=39.78 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3599.79 | 639986 | 3570.63 | 638042 | 0 | 30 | 7898 | 638304 | 22 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.77 | 642318 | 3568.89 | 637926 | 102 |  | 7923 | 640704 | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| 20+80 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3585 | 639000 | 3539.1 | 635940 | 102 |  | 7902 | 638688 | 12 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3610.14 | 640676 | 3419.52 | 627968 | 504 |  | 7919 | 640320 | 16 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=49.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3609.87 | 640658 | 3570.81 | 638054 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.86 | 642324 | 3559.08 | 637272 | 102 |  | 7916 | 640032 | 0 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  | 7933 | 641664 | 4 | 0 | 1 (1) | 1010 |
| 40+40 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (0,1,2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3549.21 | 636614 | 102 |  | 7909 | 639360 | 10 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3640.02 | 642668 | 3439.5 | 629300 | 504 |  | 7933 | 641664 | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=39.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3609.96 | 640664 | 3590.88 | 639392 | 0 | 30 | 7912 | 639648 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3644.97 | 642998 | 3589.17 | 639278 | 102 |  | 7937 | 642048 | 10 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 40+50 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3544.2 | 636280 | 102 |  | 7905 | 638976 | 8 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3630.06 | 642004 | 3429.54 | 628636 | 504 |  | 7926 | 640992 | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=44.94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3614.94 | 640996 | 3591 | 639400 | 0 | 30 | 7912 | 639648 | 8 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3644.94 | 642996 | 3584.28 | 638952 | 102 |  | 7933 | 641664 | 0 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 40+60 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3539.19 | 635946 | 102 |  | 7902 | 638688 | 6 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3620.07 | 641338 | 3419.55 | 627970 | 504 |  | 7919 | 640320 | 14 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3619.92 | 641328 | 3590.76 | 639384 | 0 | 30 | 7912 | 639648 | 0 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3644.91 | 642994 | 3579.03 | 638602 | 102 |  | 7930 | 641376 | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 5: UL CA is only supported for BCS0. I.e. the test frequencies for uplink CC2 are only supported for BCS0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.3.49 – 4.3.1.1.3.65	FFS

###### 4.3.1.1.3.66	NR Intra-band contiguous configurations CA_n66

###### 4.3.1.1.3.66.1	CA_n66B

Editor's note: Test frequencies for CA_n66B with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.66.1-1: NR Intra-Band contiguous CA configuration CA_n66B (PCC=CC1 and SCC=CC2), SCS 15 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5+20 | CC1 | 5 | 15 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2124.39 | 424878 | 102 |  | 5361 | 428910 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  | 5443 | 435410 | 0 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=12 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 2124.5 | 424900 | 2114.96 | 422992 | 0 | 15 | 5291 | 423370 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2157 | 431400 | 2129.1 | 425820 | 102 |  | 5373 | 429870 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2189.5 | 437900 | 2089.24 | 417848 | 504 |  | 5455 | 436370 | 6 | 0 | 0 (0) | 504 |
| 5+40 | CC1 | 5 | 15 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2135.3 | 427060 | 2114.69 | 422938 | 102 |  | 5336 | 426970 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2158.1 | 431620 | 2065.13 | 413026 | 504 |  | 5393 | 431530 | 0 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1735.3 | 347060 | 1642.33 | 328466 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1758.1 | 351620 | 1754.77 | 350954 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=21.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 2134.4 | 426880 | 2114.96 | 422992 | 0 | 15 | 5291 | 423370 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2157.2 | 431440 | 2119.4 | 423880 | 102 |  | 5348 | 427930 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
| 10+15 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 2127.3 | 425460 | 2120.19 | 424038 | 0 | 15 | 5307 | 424590 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2157.3 | 431460 | 2131.83 | 426366 | 102 |  | 5382 | 430590 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2187.3 | 437460 | 2089.47 | 417894 | 504 |  | 5457 | 436590 | 4 | 1 | 2 (4) | 509 |
| 10+20 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 2129.4 | 425880 | 2119.86 | 423972 | 0 | 15 | 5304 | 424350 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2159.4 | 431880 | 2131.5 | 426300 | 102 |  | 5379 | 430350 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2189.4 | 437880 | 2089.14 | 417828 | 504 |  | 5454 | 436350 | 6 | 0 | 0 (0) | 504 |
| 10+40 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2135.4 | 427080 | 2112.36 | 422472 | 102 |  | 5331 | 426510 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2155.7 | 431140 | 2060.3 | 412060 | 504 |  | 5381 | 430570 | 2 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1735.4 | 347080 | 1640 | 328000 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1755.7 | 351140 | 1749.94 | 349988 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 2139.3 | 427860 | 2119.86 | 423972 | 0 | 15 | 5304 | 424350 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2159.7 | 431940 | 2121.9 | 424380 | 102 |  | 5355 | 428430 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
| 15+10 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 2129.8 | 425960 | 2125.12 | 425024 | 0 | 15 | 5320 | 425570 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2157.3 | 431460 | 2134.26 | 426852 | 102 |  | 5388 | 431070 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2184.8 | 436960 | 2089.4 | 417880 | 504 |  | 5456 | 436570 | 2 | 1 | 2 (4) | 509 |
| 15+15 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 2132.5 | 426500 | 2125.39 | 425078 | 0 | 15 | 5320 | 425570 | 8 | 1 | 1 (2) | 3 |
|  |  |  |  |  |  | Mid | 2160 | 432000 | 2134.53 | 426906 | 102 |  | 5388 | 431070 | 8 | 1 | 1 (2) | 105 |
|  |  |  |  |  |  | High | 2187.5 | 437500 | 2089.67 | 417934 | 504 |  | 5456 | 436570 | 8 | 1 | 1 (2) | 507 |
| 15+20 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 2134.6 | 426920 | 2125.06 | 425012 | 0 | 15 | 5320 | 425570 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2162.1 | 432420 | 2134.2 | 426840 | 102 |  | 5388 | 431070 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2189.6 | 437920 | 2089.34 | 417868 | 504 |  | 5456 | 436570 | 6 | 1 | 2 (4) | 509 |
| 20+5 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=12 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 2132 | 426400 | 2129.75 | 425950 | 0 | 15 | 5330 | 426490 | 0 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2157 | 431400 | 2136.39 | 427278 | 102 |  | 5391 | 431310 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2182 | 436400 | 2089.03 | 417806 | 504 |  | 5455 | 436370 | 8 | 1 | 0 (0) | 505 |
| 20+10 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 2134.4 | 426880 | 2129.72 | 425944 | 0 | 15 | 5330 | 426490 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2159.4 | 431880 | 2136.36 | 427272 | 102 |  | 5391 | 431310 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2184.4 | 436880 | 2089 | 417800 | 504 |  | 5455 | 436370 | 10 | 1 | 0 (0) | 505 |
| 20+15 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 2137.1 | 427420 | 2129.99 | 425998 | 0 | 15 | 5330 | 426490 | 8 | 1 | 1 (2) | 3 |
|  |  |  |  |  |  | Mid | 2162.1 | 432420 | 2136.63 | 427326 | 102 |  | 5394 | 431550 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2187.1 | 437420 | 2089.27 | 417854 | 504 |  | 5455 | 436370 | 4 | 0 | 0 (0) | 504 |
| 40+5 | CC1 | 40 | 15 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=21.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 2151.9 | 430380 | 2149.65 | 429930 | 0 | 15 | 5379 | 430350 | 8 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 2166.9 | 433380 | 2146.29 | 429258 | 102 |  | 5418 | 433470 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2181.9 | 436380 | 2088.93 | 417786 | 504 |  | 5454 | 436350 | 8 | 1 | 0 (0) | 505 |
| 40+10 | CC1 | 40 | 15 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 2154.3 | 430860 | 2149.62 | 429924 | 0 | 15 | 5379 | 430350 | 10 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 2169.3 | 433860 | 2146.26 | 429252 | 102 |  | 5418 | 433470 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2184.3 | 436860 | 2088.9 | 417780 | 504 |  | 5454 | 436350 | 10 | 1 | 0 (0) | 505 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.66.1-2: NR Intra-Band contiguous CA configuration CA_n66B (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+15 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  |  | 5361 | 428910 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  |  | 5436 | 434910 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2127.3 | 425460 | 2120.46 | 424092 | 0 | 15 |  | 5310 | 424830 |  | 6 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2157.3 | 431460 | 2113.74 | 422748 | 102 |  |  | 5385 | 430830 |  | 6 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2187.3 | 437460 | 1999.02 | 399804 | 504 |  |  | 5460 | 436830 |  | 6 |  | 0 | 0 (5) | 1018 |
| 10+20 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  |  | 5361 | 428910 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  |  | 5436 | 434910 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 2129.4 | 425880 | 2120.22 | 424044 | 0 | 15 |  | 5310 | 424830 |  | 22 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2159.4 | 431880 | 2113.5 | 422700 | 102 |  |  | 5385 | 430830 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2189.4 | 437880 | 1998.78 | 399756 | 504 |  |  | 5460 | 436830 |  | 22 |  | 0 | 0 (5) | 1018 |
| 10+40 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2135.4 | 427080 | 2094.36 | 418872 | 102 |  |  | 5337 | 426990 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2155.7 | 431140 | 1969.94 | 393988 | 504 |  |  | 5387 | 431050 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1735.4 | 347080 | 1549.64 | 309928 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1755.7 | 351140 | 1749.22 | 349844 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2139.3 | 427860 | 2120.22 | 424044 | 0 | 15 |  | 5310 | 424830 |  | 22 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2159.7 | 431940 | 2103.9 | 420780 | 102 |  |  | 5361 | 428910 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  |  | 5411 | 432970 |  | 22 |  | 0 | 0 (5) | 1018 |
| 15+10 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  |  | 5355 | 428430 |  | 2 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2129.8 | 425960 | 2125.48 | 425096 | 0 | 15 |  | 5326 | 426050 |  | 6 |  | 0 | 3 (8) | 16 |
|  |  |  |  |  |  | Mid | 2157.3 | 431460 | 2116.26 | 423252 | 102 |  |  | 5394 | 431550 |  | 6 |  | 0 | 3 (8) | 220 |
|  |  |  |  |  |  | High | 2184.8 | 436960 | 1999.04 | 399808 | 504 |  |  | 5462 | 437050 |  | 6 |  | 0 | 3 (8) | 1024 |
| 15+15 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  |  | 5355 | 428430 |  | 2 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2132.5 | 426500 | 2125.66 | 425132 | 0 | 15 |  | 5326 | 426050 |  | 18 |  | 0 | 2 (7) | 14 |
|  |  |  |  |  |  | Mid | 2160 | 432000 | 2116.44 | 423288 | 102 |  |  | 5394 | 431550 |  | 18 |  | 0 | 2 (7) | 218 |
|  |  |  |  |  |  | High | 2187.5 | 437500 | 1999.22 | 399844 | 504 |  |  | 5462 | 437050 |  | 18 |  | 0 | 2 (7) | 1022 |
| 15+20 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  |  | 5355 | 428430 |  | 2 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 2134.6 | 426920 | 2125.42 | 425084 | 0 | 15 |  | 5323 | 425810 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2162.1 | 432420 | 2116.2 | 423240 | 102 |  |  | 5391 | 431310 |  | 2 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2189.6 | 437920 | 1998.98 | 399796 | 504 |  |  | 5459 | 436810 |  | 2 |  | 0 | 0 (5) | 1018 |
| 20+10 | CC1 | 20 | 30 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 |  | 5285 | 422890 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  |  | 5349 | 427950 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  |  | 5413 | 433010 |  | 18 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2134.4 | 426880 | 2130.08 | 426016 | 0 | 15 |  | 5336 | 426970 |  | 6 |  | 0 | 3 (8) | 16 |
|  |  |  |  |  |  | Mid | 2159.4 | 431880 | 2118.36 | 423672 | 102 |  |  | 5397 | 431790 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2184.4 | 436880 | 1998.64 | 399728 | 504 |  |  | 5461 | 436850 |  | 14 |  | 0 | 1 (6) | 1020 |
| 20+15 | CC1 | 20 | 30 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 |  | 5285 | 422890 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  |  | 5349 | 427950 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  |  | 5413 | 433010 |  | 18 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2137.1 | 427420 | 2130.26 | 426052 | 0 | 15 |  | 5336 | 426970 |  | 18 |  | 0 | 2 (7) | 14 |
|  |  |  |  |  |  | Mid | 2162.1 | 432420 | 2118.54 | 423708 | 102 |  |  | 5397 | 431790 |  | 6 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2187.1 | 437420 | 1998.82 | 399764 | 504 |  |  | 5461 | 436850 |  | 2 |  | 0 | 1 (6) | 1020 |
| 40+10 | CC1 | 40 | 30 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 |  | 5286 | 422910 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2089.2 | 417840 | 102 |  |  | 5325 | 426030 |  | 18 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2160 | 432000 | 1959.48 | 391896 | 504 |  |  | 5361 | 428910 |  | 2 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2154.3 | 430860 | 2149.98 | 429996 | 0 | 15 |  | 5385 | 430830 |  | 14 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2169.3 | 433860 | 2128.26 | 425652 | 102 |  |  | 5424 | 433950 |  | 6 |  | 0 | 3 (8) | 220 |
|  |  |  |  |  |  | High | 2184.3 | 436860 | 1998.54 | 399708 | 504 |  |  | 5460 | 436830 |  | 14 |  | 0 | 1 (6) | 1020 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.3.67 – 4.3.1.1.3.76	FFS

###### 4.3.1.1.3.77	NR Intra-band contiguous configurations CA_n77

###### 4.3.1.1.3.77.1	CA_n77C

Editor's note: Test frequencies for CA_n78C with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.77.1-1: NR Intra-Band contiguous CA configuration CA_n77C (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+100 | CC1 | 10 | 30 | 24 | Downlink | Low | 3305,01 | 620334 | 3300,69 | 620046 | 0 | 30 |  | 7711 | 620352 |  | 18 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3658,95 | 643930 | 102 |  |  | 7985 | 646656 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4095,39 | 673026 | 3909,63 | 660642 | 504 |  |  | 8260 | 673056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,6 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3359,61 | 623974 | 3310,47 | 620698 | 0 | 30 |  | 7718 | 621024 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3754,59 | 650306 | 3668,73 | 644582 | 102 |  |  | 7992 | 647328 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 15+90 | CC1 | 15 | 30 | 38 | Downlink | Low | 3307,5 | 620500 | 3300,66 | 620044 | 0 | 30 |  | 7711 | 620352 |  | 20 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3661,44 | 644096 | 102 |  |  | 7987 | 646848 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4102,92 | 673528 | 3914,64 | 660976 | 504 |  |  | 8263 | 673344 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=52,08 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3359,58 | 623972 | 3315,48 | 621032 | 0 | 30 |  | 7721 | 621312 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3757,08 | 650472 | 3676,26 | 645084 | 102 |  |  | 7997 | 647808 |  | 12 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 15+100 | CC1 | 15 | 30 | 38 | Downlink | Low | 3307,5 | 620500 | 3300,66 | 620044 | 0 | 30 |  | 7711 | 620352 |  | 20 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3656,43 | 643762 | 102 |  |  | 7983 | 646464 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4092,87 | 672858 | 3904,59 | 660306 | 504 |  |  | 8256 | 672672 |  | 6 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=57,12 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3364,62 | 624308 | 3315,48 | 621032 | 0 | 30 |  | 7721 | 621312 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3757,11 | 650474 | 3671,25 | 644750 | 102 |  |  | 7994 | 647520 |  | 10 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 20+90 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310,02 | 620668 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3659,1 | 643940 | 102 |  |  | 7985 | 646656 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4100,1 | 673340 | 3909,48 | 660632 | 504 |  |  | 8260 | 673056 |  | 16 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3364,92 | 624328 | 3320,82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3759,9 | 650660 | 3679,08 | 645272 | 102 |  |  | 7999 | 648000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 20+100 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310,02 | 620668 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3654,09 | 643606 | 102 |  |  | 7982 | 646368 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4090,05 | 672670 | 3899,43 | 659962 | 504 |  |  | 8253 | 672384 |  | 14 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3369,96 | 624664 | 3320,82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3759,93 | 650662 | 3674,07 | 644938 | 102 |  |  | 7996 | 647712 |  | 14 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 25+80 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3661,59 | 644106 | 102 |  |  | 7987 | 646848 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4107,66 | 673844 | 3914,52 | 660968 | 504 |  |  | 8263 | 673344 |  | 16 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=52,32 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3364,83 | 624322 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,33 | 650822 | 3686,55 | 645770 | 102 |  |  | 8004 | 648480 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 25+90 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3656,58 | 643772 | 102 |  |  | 7983 | 646464 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4097,64 | 673176 | 3904,5 | 660300 | 504 |  |  | 8256 | 672672 |  | 12 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=57,36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3369,87 | 624658 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,36 | 650824 | 3681,54 | 645436 | 102 |  |  | 8001 | 648192 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 25+100 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3651,57 | 643438 | 102 |  |  | 7980 | 646176 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4087,59 | 672506 | 3894,45 | 659630 | 504 |  |  | 8249 | 672000 |  | 10 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=62,4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3374,91 | 624994 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,39 | 650826 | 3676,53 | 645102 | 102 |  |  | 7997 | 647808 |  | 18 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 30+80 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3659,25 | 643950 | 102 |  |  | 7985 | 646656 |  | 18 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4105,14 | 673676 | 3909,66 | 660644 | 504 |  |  | 8260 | 673056 |  | 4 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3369,84 | 624656 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,85 | 650990 | 3689,07 | 645938 | 102 |  |  | 8006 | 648672 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 30+90 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3654,24 | 643616 | 102 |  |  | 7982 | 646368 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3899,64 | 659976 | 504 |  |  | 8253 | 672384 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3374,88 | 624992 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,88 | 650992 | 3684,06 | 645604 | 102 |  |  | 8003 | 648384 |  | 20 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 30+100 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3649,23 | 643282 | 102 |  |  | 7978 | 645984 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4085,07 | 672338 | 3889,59 | 659306 | 504 |  |  | 8246 | 671712 |  | 22 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3379,92 | 625328 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,91 | 650994 | 3679,05 | 645270 | 102 |  |  | 7999 | 648000 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 40+70 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3659,19 | 643946 | 102 |  |  | 7985 | 646656 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4110,15 | 674010 | 3909,63 | 660642 | 504 |  |  | 8260 | 673056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3374,85 | 624990 | 3340,83 | 622722 | 0 | 30 |  | 7739 | 623040 |  | 6 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,83 | 651322 | 3699,09 | 646606 | 102 |  |  | 8013 | 649344 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 40+80 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3654,21 | 643614 | 102 |  |  | 7982 | 646368 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4100,16 | 673344 | 3899,64 | 659976 | 504 |  |  | 8253 | 672384 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3379,83 | 625322 | 3340,77 | 622718 | 0 | 30 |  | 7739 | 623040 |  | 10 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,83 | 651322 | 3694,05 | 646270 | 102 |  |  | 8009 | 648960 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 40+90 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3649,2 | 643280 | 102 |  |  | 7978 | 645984 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4090,2 | 672680 | 3889,68 | 659312 | 504 |  |  | 8246 | 671712 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3384,81 | 625654 | 3340,71 | 622714 | 0 | 30 |  | 7739 | 623040 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,8 | 651320 | 3688,98 | 645932 | 102 |  |  | 8006 | 648672 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 40+100 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3644,19 | 642946 | 102 |  |  | 7975 | 645696 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4080,27 | 672018 | 3879,75 | 658650 | 504 |  |  | 8239 | 671040 |  | 6 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3389,73 | 625982 | 3340,59 | 622706 | 0 | 30 |  | 7739 | 623040 |  | 22 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,71 | 651314 | 3683,85 | 645590 | 102 |  |  | 8002 | 648288 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3720 | 648000 | 3659,34 | 643956 | 102 |  |  | 7985 | 646656 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4115,04 | 674336 | 3909,66 | 660644 | 504 |  |  | 8260 | 673056 |  | 4 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3379,98 | 625332 | 3350,82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,96 | 651664 | 3709,08 | 647272 | 102 |  |  | 8020 | 650016 |  | 8 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 50+70 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3654,33 | 643622 | 102 |  |  | 7982 | 646368 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4105,11 | 673674 | 3899,73 | 659982 | 504 |  |  | 8253 | 672384 |  | 18 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=59,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3384,9 | 625660 | 3350,88 | 623392 | 0 | 30 |  | 7746 | 623712 |  | 8 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,87 | 651658 | 3704,13 | 646942 | 102 |  |  | 8016 | 649632 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3649,35 | 643290 | 102 |  |  | 7978 | 645984 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3889,74 | 659316 | 504 |  |  | 8246 | 671712 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3389,88 | 625992 | 3350,82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,87 | 651658 | 3699,09 | 646606 | 102 |  |  | 8013 | 649344 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 50+90 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3644,34 | 642956 | 102 |  |  | 7975 | 645696 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4085,16 | 672344 | 3879,78 | 658652 | 504 |  |  | 8239 | 671040 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3394,86 | 626324 | 3350,76 | 623384 | 0 | 30 |  | 7746 | 623712 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,84 | 651656 | 3694,02 | 646268 | 102 |  |  | 8009 | 648960 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3639,33 | 642622 | 102 |  |  | 7971 | 645312 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4075,23 | 671682 | 3869,85 | 657990 | 504 |  |  | 8232 | 670368 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74,76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3399,78 | 626652 | 3350,64 | 623376 | 0 | 30 |  | 7745 | 623616 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3774,75 | 651650 | 3688,89 | 645926 | 102 |  |  | 8006 | 648672 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3720 | 648000 | 3654,12 | 643608 | 102 |  |  | 7982 | 646368 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4110 | 674000 | 3899,4 | 659960 | 504 |  |  | 8253 | 672384 |  | 16 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3390 | 626000 | 3360,84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3780 | 652000 | 3714,12 | 647608 | 102 |  |  | 8023 | 650304 |  | 8 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 60+70 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3649,11 | 643274 | 102 |  |  | 7978 | 645984 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4100,07 | 673338 | 3889,47 | 659298 | 504 |  |  | 8246 | 671712 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3394,92 | 626328 | 3360,9 | 624060 | 0 | 30 |  | 7753 | 624384 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,91 | 651994 | 3709,17 | 647278 | 102 |  |  | 8020 | 650016 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3644,13 | 642942 | 102 |  |  | 7975 | 645696 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4090,08 | 672672 | 3879,48 | 658632 | 504 |  |  | 8239 | 671040 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=69,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3399,9 | 626660 | 3360,84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,91 | 651994 | 3704,13 | 646942 | 102 |  |  | 8016 | 649632 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 60+90 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3639,12 | 642608 | 102 |  |  | 7971 | 645312 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4080,12 | 672008 | 3869,52 | 657968 | 504 |  |  | 8232 | 670368 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3404,88 | 626992 | 3360,78 | 624052 | 0 | 30 |  | 7753 | 624384 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,88 | 651992 | 3699,06 | 646604 | 102 |  |  | 8013 | 649344 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3634,11 | 642274 | 102 |  |  | 7968 | 645024 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4070,19 | 671346 | 3859,59 | 657306 | 504 |  |  | 8225 | 669696 |  | 6 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3409,8 | 627320 | 3360,66 | 624044 | 0 | 30 |  | 7752 | 624288 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3779,79 | 651986 | 3693,93 | 646262 | 102 |  |  | 8009 | 648960 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 70+60 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3649,26 | 643284 | 102 |  |  | 7978 | 645984 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4105,08 | 673672 | 3889,62 | 659308 | 504 |  |  | 8246 | 671712 |  | 20 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3399,93 | 626662 | 3370,77 | 624718 | 0 | 30 |  | 7759 | 624960 |  | 2 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3784,92 | 652328 | 3719,04 | 647936 | 102 |  |  | 8027 | 650688 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 70+70 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3644,25 | 642950 | 102 |  |  | 7975 | 645696 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4095,03 | 673002 | 3879,57 | 658638 | 504 |  |  | 8239 | 671040 |  | 18 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3404,97 | 626998 | 3370,95 | 624730 | 0 | 30 |  | 7760 | 625056 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,95 | 652330 | 3714,21 | 647614 | 102 |  |  | 8023 | 650304 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 70+80 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3639,27 | 642618 | 102 |  |  | 7971 | 645312 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4085,04 | 672336 | 3869,58 | 657972 | 504 |  |  | 8232 | 670368 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3409,95 | 627330 | 3370,89 | 624726 | 0 | 30 |  | 7760 | 625056 |  | 18 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,95 | 652330 | 3709,17 | 647278 | 102 |  |  | 8020 | 650016 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 70+90 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3634,26 | 642284 | 102 |  |  | 7968 | 645024 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4075,08 | 671672 | 3859,62 | 657308 | 504 |  |  | 8225 | 669696 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3414,93 | 627662 | 3370,83 | 624722 | 0 | 30 |  | 7760 | 625056 |  | 22 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,92 | 652328 | 3704,1 | 646940 | 102 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 70+100 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3629,25 | 641950 | 102 |  |  | 7964 | 644640 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4065,15 | 671010 | 3849,69 | 656646 | 504 |  |  | 8218 | 669024 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=84,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3419,85 | 627990 | 3370,71 | 624714 | 0 | 30 |  | 7759 | 624960 |  | 6 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3784,83 | 652322 | 3698,97 | 646598 | 102 |  |  | 8013 | 649344 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3644,22 | 642948 | 102 |  |  | 7975 | 645696 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4100,1 | 673340 | 3879,6 | 658640 | 504 |  |  | 8239 | 671040 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3409,92 | 627328 | 3380,76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,9 | 652660 | 3724,02 | 648268 | 102 |  |  | 8030 | 650976 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 80+70 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3639,21 | 642614 | 102 |  |  | 7971 | 645312 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4090,05 | 672670 | 3869,55 | 657970 | 504 |  |  | 8232 | 670368 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3414,96 | 627664 | 3380,94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3789,93 | 652662 | 3719,19 | 647946 | 102 |  |  | 8027 | 650688 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3634,23 | 642282 | 102 |  |  | 7968 | 645024 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4080 | 672000 | 3859,5 | 657300 | 504 |  |  | 8225 | 669696 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3420 | 628000 | 3380,94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3789,99 | 652666 | 3714,21 | 647614 | 102 |  |  | 8023 | 650304 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 80+90 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3629,22 | 641948 | 102 |  |  | 7964 | 644640 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4070,04 | 671336 | 3849,54 | 656636 | 504 |  |  | 8218 | 669024 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3424,98 | 628332 | 3380,88 | 625392 | 0 | 30 |  | 7766 | 625632 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,96 | 652664 | 3709,14 | 647276 | 102 |  |  | 8020 | 650016 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3624,21 | 641614 | 102 |  |  | 7961 | 644352 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4060,11 | 670674 | 3839,61 | 655974 | 504 |  |  | 8211 | 668352 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3429,9 | 628660 | 3380,76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,87 | 652658 | 3704,01 | 646934 | 102 |  |  | 8016 | 649632 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 90+60 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3639,18 | 642612 | 102 |  |  | 7971 | 645312 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3869,58 | 657972 | 504 |  |  | 8232 | 670368 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3419,88 | 627992 | 3390,72 | 626048 | 0 | 30 |  | 7773 | 626304 |  | 16 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,88 | 652992 | 3729 | 648600 | 102 |  |  | 8034 | 651360 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 90+70 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3634,17 | 642278 | 102 |  |  | 7968 | 645024 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4085,07 | 672338 | 3859,53 | 657302 | 504 |  |  | 8225 | 669696 |  | 10 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3424,92 | 628328 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,91 | 652994 | 3724,17 | 648278 | 102 |  |  | 8030 | 650976 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 90+80 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3629,19 | 641946 | 102 |  |  | 7964 | 644640 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4075,02 | 671668 | 3849,48 | 656632 | 504 |  |  | 8218 | 669024 |  | 8 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3429,96 | 628664 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,97 | 652998 | 3719,19 | 647946 | 102 |  |  | 8027 | 650688 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 90+90 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3624,18 | 641612 | 102 |  |  | 7961 | 644352 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4065 | 671000 | 3839,46 | 655964 | 504 |  |  | 8211 | 668352 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=90 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3435 | 629000 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3795 | 653000 | 3714,18 | 647612 | 102 |  |  | 8023 | 650304 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 90+100 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3619,17 | 641278 | 102 |  |  | 7957 | 643968 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4055,07 | 670338 | 3829,53 | 655302 | 504 |  |  | 8204 | 667680 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=94,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3439,92 | 629328 | 3390,78 | 626052 | 0 | 30 |  | 7773 | 626304 |  | 12 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,91 | 652994 | 3709,05 | 647270 | 102 |  |  | 8020 | 650016 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3634,14 | 642276 | 102 |  |  | 7968 | 645024 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4090,2 | 672680 | 3859,62 | 657308 | 504 |  |  | 8225 | 669696 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3429,81 | 628654 | 3400,65 | 626710 | 0 | 30 |  | 7780 | 626976 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3799,8 | 653320 | 3733,92 | 648928 | 102 |  |  | 8037 | 651648 |  | 8 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 100+70 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3629,13 | 641942 | 102 |  |  | 7964 | 644640 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4080,15 | 672010 | 3849,57 | 656638 | 504 |  |  | 8218 | 669024 |  | 2 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3434,85 | 628990 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,83 | 653322 | 3729,09 | 648606 | 102 |  |  | 8034 | 651360 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3624,15 | 641610 | 102 |  |  | 7961 | 644352 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4070,1 | 671340 | 3839,52 | 655968 | 504 |  |  | 8211 | 668352 |  | 0 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=89,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3439,89 | 629326 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,89 | 653326 | 3724,11 | 648274 | 102 |  |  | 8030 | 650976 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 100+90 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3619,14 | 641276 | 102 |  |  | 7957 | 643968 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4060,08 | 670672 | 3829,5 | 655300 | 504 |  |  | 8204 | 667680 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=94,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3444,93 | 629662 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,92 | 653328 | 3719,1 | 647940 | 102 |  |  | 8027 | 650688 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 100 +100 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3614,13 | 640942 | 102 |  |  | 7954 | 643680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4050,03 | 670002 | 3819,45 | 654630 | 504 |  |  | 8197 | 667008 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=99,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3449,97 | 629998 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,95 | 653330 | 3714,09 | 647606 | 102 |  |  | 8023 | 650304 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.3.78	NR Intra-band contiguous configurations CA_n78

###### 4.3.1.1.3.78.1	CA_n78C

Editor's note: Test frequencies for CA_n78C with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.78.1-1: NR Intra-Band contiguous CA configuration CA_n78C (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3459.33 | 630622 | 102 |  |  | 7846 | 633312 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3715.02 | 647668 | 3509.64 | 633976 | 504 |  |  | 7982 | 646368 |  | 8 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3379.98 | 625332 | 3350.82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.95 | 638330 | 3509.07 | 633938 | 102 |  |  | 7881 | 636672 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3449.34 | 629956 | 102 |  |  | 7840 | 632736 |  | 20 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3695.13 | 646342 | 3489.75 | 632650 | 504 |  |  | 7968 | 645024 |  | 14 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3389.88 | 625992 | 3350.82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.86 | 638324 | 3499.08 | 633272 | 102 |  |  | 7874 | 636000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3439.35 | 629290 | 102 |  |  | 7833 | 632064 |  | 14 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3675.24 | 645016 | 3469.86 | 631324 | 504 |  |  | 7954 | 643680 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3399.78 | 626652 | 3350.64 | 623376 | 0 | 30 |  | 7745 | 623616 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3574.77 | 638318 | 3488.91 | 632594 | 102 |  |  | 7867 | 635328 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 60+50 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3459.12 | 630608 | 102 |  |  | 7846 | 633312 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3720.03 | 648002 | 3509.43 | 633962 | 504 |  |  | 7982 | 646368 |  | 22 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3384.96 | 625664 | 3361.02 | 624068 | 0 | 30 |  | 7753 | 624384 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.96 | 638664 | 3519.3 | 634620 | 102 |  |  | 7888 | 637344 |  | 12 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3454.11 | 630274 | 102 |  |  | 7843 | 633024 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3709.98 | 647332 | 3499.38 | 633292 | 504 |  |  | 7975 | 645696 |  | 20 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3390 | 626000 | 3360.84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.99 | 638666 | 3514.11 | 634274 | 102 |  |  | 7885 | 637056 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3444.12 | 629608 | 102 |  |  | 7836 | 632352 |  | 8 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.09 | 646006 | 3479.49 | 631966 | 504 |  |  | 7961 | 644352 |  | 2 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3399.9 | 626660 | 3360.84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.9 | 638660 | 3504.12 | 633608 | 102 |  |  | 7878 | 636384 |  | 16 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3434.13 | 628942 | 102 |  |  | 7829 | 631680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3670.2 | 644680 | 3459.6 | 630640 | 504 |  |  | 7947 | 643008 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3409.8 | 627320 | 3360.66 | 624044 | 0 | 30 |  | 7752 | 624288 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3579.81 | 638654 | 3493.95 | 632930 | 102 |  |  | 7871 | 635712 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 80+50 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3449.22 | 629948 | 102 |  |  | 7839 | 632640 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3710.13 | 647342 | 3489.63 | 632642 | 504 |  |  | 7968 | 645024 |  | 22 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3404.88 | 626992 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.86 | 639324 | 3529.2 | 635280 | 102 |  |  | 7895 | 638016 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3444.21 | 629614 | 102 |  |  | 7836 | 632352 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3700.08 | 646672 | 3479.58 | 631972 | 504 |  |  | 7961 | 644352 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3409.92 | 627328 | 3380.76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.89 | 639326 | 3524.01 | 634934 | 102 |  |  | 7891 | 637632 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3434.22 | 628948 | 102 |  |  | 7829 | 631680 |  | 20 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3459.51 | 630634 | 504 |  |  | 7947 | 643008 |  | 14 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3420 | 628000 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.98 | 639332 | 3514.2 | 634280 | 102 |  |  | 7885 | 637056 |  | 16 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3424.23 | 628282 | 102 |  |  | 7822 | 631008 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3660.12 | 644008 | 3439.62 | 629308 | 504 |  |  | 7933 | 641664 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3429.9 | 628660 | 3380.76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.89 | 639326 | 3504.03 | 633602 | 102 |  |  | 7878 | 636384 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 100+50 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3439.14 | 629276 | 102 |  |  | 7832 | 631968 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3700.23 | 646682 | 3469.65 | 631310 | 504 |  |  | 7954 | 643680 |  | 10 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3424.77 | 628318 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.76 | 639984 | 3539.1 | 635940 | 102 |  |  | 7902 | 638688 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3434.13 | 628942 | 102 |  |  | 7829 | 631680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.18 | 646012 | 3459.6 | 630640 | 504 |  |  | 7947 | 643008 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3429.81 | 628654 | 3400.65 | 626710 | 0 | 30 |  | 7780 | 626976 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3599.79 | 639986 | 3533.91 | 635594 | 102 |  |  | 7898 | 638304 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3424.14 | 628276 | 102 |  |  | 7822 | 631008 |  | 20 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3670.11 | 644674 | 3439.53 | 629302 | 504 |  |  | 7933 | 641664 |  | 2 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3439.89 | 629326 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.88 | 639992 | 3524.1 | 634940 | 102 |  |  | 7891 | 637632 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 100 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| +100 |  |  |  |  | & | Mid | 3500.01 | 633334 | 3414.15 | 627610 | 102 |  |  | 7815 | 630336 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3650.04 | 643336 | 3419.46 | 627964 | 504 |  |  | 7919 | 640320 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3449.97 | 629998 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.97 | 639998 | 3514.11 | 634274 | 102 |  |  | 7885 | 637056 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.3.78.2	CA_n78B

Editor's note: Test frequencies for CA_n78B with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.78.2-1: NR Intra-Band contiguous CA configuration CA_n78B (PCC=CC1 and SCC=CC2), SCS 15 kHz and ΔFRaster 15 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20+50 | CC1 | 20 | 15 | 106 | Downlink | Low | 3310.005 | 620667 | 3300.465 | 620031 | 0 | 30 |  | 7711 | 620352 |  | 9 |  | 0 | 1 (6) | 6 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3497.1 | 633140 | 102 |  |  | 7860 | 634656 |  | 4 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3740.25 | 649350 | 3639.99 | 642666 | 504 |  |  | 8010 | 649056 |  | 6 |  | 2 | 1 (6) | 512 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3344.745 | 622983 | 3320.445 | 621363 | 0 | 30 |  | 7725 | 621696 |  | 9 |  | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3559.74 | 637316 | 3517.08 | 634472 | 102 |  |  | 7874 | 636000 |  | 4 |  | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3659.97 | 643998 | 504 |  |  | 8024 | 650400 |  | 6 |  | 3 | 1 (6) | 513 |
| 50+20 | CC1 | 50 | 15 | 270 | Downlink | Low | 3325.005 | 621667 | 3300.705 | 620047 | 0 | 30 |  | 7711 | 620352 |  | 5 |  | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3540 | 636000 | 3497.34 | 633156 | 102 |  |  | 7860 | 634656 |  | 0 |  | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3755.25 | 650350 | 3640.23 | 642682 | 504 |  |  | 8010 | 649056 |  | 2 |  | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3359.745 | 623983 | 3350.205 | 623347 | 0 | 30 |  | 7745 | 623616 |  | 5 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3574.74 | 638316 | 3546.84 | 636456 | 102 |  |  | 7895 | 638016 |  | 0 |  | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3789.99 | 652666 | 3689.73 | 645982 | 504 |  |  | 8044 | 652320 |  | 2 |  | 2 | 0 (2) | 508 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.78.2-2: NR Intra-Band contiguous CA configuration CA_n78B (PCC=CC1 and SCC=CC2), SCS 30 kHz and ΔFRaster 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20+50 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3479.1 | 631940 | 102 |  |  | 7860 | 634656 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3740.25 | 649350 | 3549.63 | 636642 | 504 |  |  | 8010 | 649056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3344.76 | 622984 | 3320.82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3559.74 | 637316 | 3499.08 | 633272 | 102 |  |  | 7874 | 636000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 50+20 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3540 | 636000 | 3479.34 | 631956 | 102 |  |  | 7860 | 634656 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3755.25 | 650350 | 3549.87 | 636658 | 504 |  |  | 8010 | 649056 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3359.76 | 623984 | 3350.58 | 623372 | 0 | 30 |  | 7745 | 623616 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3574.74 | 638316 | 3528.84 | 635256 | 102 |  |  | 7895 | 638016 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3789.99 | 652666 | 3599.37 | 639958 | 504 |  |  | 8044 | 652320 |  | 2 |  | 0 | 1 (1) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.4	Void

##### 4.3.1.1.5	NR intra-band non-contiguous CA configurations in FR1

###### 4.3.1.1.5.1	FFS

###### 4.3.1.1.5.2	CA_n2(xA)

4.3.1.1.5.2.1	CA_n2(2A)

Editor’s note:	Test frequencies for CA_n2(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.2.1-1: NR Intra-Band non-contiguous CA configuration CA_n2(2A) without UL CA, SCS=15 kHz, Max Wgap

| Sb CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n2(2A): n2A + n2A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.2-1: Low range for CBW = 5, 10, 15, 20 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.2-1: High range for CBW = 5, 10, 15, 20 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.2.1-2: NR Intra-Band non-contiguous CA configuration CA_n2(2A) without UL CA, SCS=30 kHz, Max Wgap

| Sb CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n2(2A): n2A + n2A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.2-2: Low range for CBW = 10, 15, 20 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.2-2: High range for CBW = 10, 15, 20 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

###### 4.3.1.1.5.3 – 4.3.1.1.5.47	FFS

4.3.1.1.5.48	CA_n48(xA)

4.3.1.1.5.48.1	CA_n48(2A)

Editor’s note:	Test frequencies for CA_n48(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.48.1-1: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=15 kHz, Max Wgap

| Sb CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n48(2A): n48A + n48A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.48-1: Low range for CBW = 10, 15, 20, 40, 50 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.48-1: High range for CBW = 10, 15, 20, 40, 50 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. Note 4: For the 50 MHz bandwidth or more, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |  |

Table 4.3.1.1.5.48.1-2: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n48(2A): n48A + n48A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.48-2: Low range for CBW = 10, 15, 20, 40, 50, 60, 80, 90, 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.48-2: High range for CBW = 10, 15, 20, 40, 50, 60, 80, 90, 100 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. Note 4: For the 50 MHz bandwidth or more, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |  |

4.3.1.1.5.49 – 4.3.1.1.5.65	FFS

###### 4.3.1.1.5.66	CA_n66(xA)

4.3.1.1.5.66.1	CA_n66(2A)

Editor’s note:	Test frequencies for CA_n66(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.66.1-1: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=15 kHz, Max Wgap

| Sb CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n66(2A): n66A + n66A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-1: Low range for CBW = 5, 10, 15, 20 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-1: High range for CBW = 5, 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| CBW1+CBW2 (1) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-1: Low range for CBW = 5, 10, 15, 20, 25, 30, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-1: High range for CBW = 5, 10, 15, 20, 25, 30, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| CBW1+CBW2 (2) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-1: Low range for CBW = 5, 10, 15, 20, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-1: High range for CBW = 5, 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.66.1-2: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n66(2A): n66A + n66A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-2: Low range for CBW = 10, 15, 20 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-2: High range for CBW = 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| CBW1+CBW2 (1) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-2: Low range for CBW = 10, 15, 20, 25, 30, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-2: High range for CBW = 10, 15, 20, 25, 30, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| CBW1+CBW2 (2) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-2: Low range for CBW = 10, 15, 20, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-2: High range for CBW = 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

4.3.1.1.5.66.2	CA_n66(3A)

Editor’s note:	Test frequencies for CA_n66(3A) with mixed numerology is FFS.

Table 4.3.1.1.5.66.2-1: NR Intra-Band non-contiguous CA configuration CA_n66(3A) without UL CA, SCS 15 kHz, frequency raster 100 kHz, and Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n66(3A): n66A + n66A + n66A |  |  |  |  |  |
| CBW1+CBW2 +CBW3 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-1: Low range for CBW = 5, 10, 15, 20, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-1: Mid range for CBW = 5, 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB3 | CBW3 | Downlink & Uplink |  | For CBW3, use Table 4.3.1.1.1.66-1: High range for CBW = 5, 10, 15, 20, 40 MHz, and if SB3 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for SB1, SB2 and SB3 have been specified for both downlink and uplink to enable that the CC of either SB1, SB2 or SB3 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.66.2-2: NR Intra-Band non-contiguous CA configuration CA_n66(3A) without UL CA, SCS 30 kHz, frequency raster 100 kHz, and Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n66(3A): n66A + n66A + n66A |  |  |  |  |  |
| CBW1+CBW2 +CBW3 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.66-2: Low range for CBW = 10, 15, 20, 40 MHz, and if SB1 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.66-2: Mid range for CBW = 10, 15, 20, 40 MHz, and if SB2 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
|  | SB3 | CBW3 | Downlink & Uplink |  | For CBW3, use Table 4.3.1.1.1.66-2: High range for CBW = 10, 15, 20, 40 MHz, and if SB3 is used as PCC the applicable UL/DL Bandwidth combination depending on required UL bandwidth |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwith of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for SB1, SB2 and SB3 have been specified for both downlink and uplink to enable that the CC of either SB1, SB2 or SB3 can be used as PCC. |  |  |  |  |  |

###### 4.3.1.1.5.67 – 4.3.1.1.5.70	FFS

###### 4.3.1.1.5.71	CA_n71(xA)

4.3.1.1.5.71.1	CA_n71(2A)

Editor’s note:	Test frequencies for CA_n71(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz; and SCS CC1=30kHz and SCS CC2=15 kHz is FFS.

Table 4.3.1.1.5.71.1-1: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (5MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 5+5 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 5+10 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 5+15 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 5+20 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (10MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 10+5 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 10+10 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (5-15MHz) |  |  |  |  |  |  |
| 15+5 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 15+10 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (5-10MHz) |  |  |  |  |  |  |
| 20+5 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 20+10 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.71.1-2: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (10MHz) + n71A (10-20MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (10-15MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (10MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=20 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

###### 4.3.1.1.5.72 – 4.3.1.1.5.76	FFS

###### 4.3.1.1.5.77	CA_n77(xA)

4.3.1.1.5.77.1	CA_n77(2A)

Editor’s note:	Test frequencies for CA_n77(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.77.1-1: NR Intra-Band non-contiguous CA configuration CA_n77(2A) without UL CA, SCS=15 kHz, Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A): n77A + n77A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-1: Low range for CBW = 20, 40 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-1: High range for CBW = 20, 40 MHz |
| CBW1+CBW2 (1) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-1: Low range for CBW = 10, 15, 20, 25, 30, 40, 50 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-1: High range for CBW = 10, 15, 20, 25, 30, 40, 50 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwidth of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.77.1-2: NR Intra-Band non-contiguous CA configuration CA_n77(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination (BCS) | SB (Note 2) | Bandwidth [MHz] | Range (Note 1,3) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A): n77A + n77A |  |  |  |  |  |
| CBW1+CBW2 (0) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-2: Low range for CBW = 20, 40, 80, 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-2: High range for CBW = 20, 40, 80, 100 MHz |
| CBW1+CBW2 (1) | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-2: Low range for CBW = 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-2: High range for CBW = 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz |
| Note 1: Only one range has been specified as the maximum maxAggregatedFsBW for FR1 NR intra-band non-contiguous CA without UL CA equals the full bandwidth of the NR band (Table C.2.4.3.1A-1). Note 2: PCC is configured on SB1 unless otherwise stated. Note 3: The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.77.1-3: NR Intra-Band non-contiguous CA configuration CA_n77(2A) with UL CA (PCC=CC1 and SCC=CC2), SCS 15kHz

| Aggregated BW [MHz] (BCS) (CBW combination) | Range |  | CC Note 1 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] Note 1 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 3 | offsetToPointA (SIB1) [PRBs] Note 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | UL | LOW | CC1 | 10 | 15 | 52 | 3305.01 | 620334 | 3300.33 | 620022 | 0 | 30 | 7711 | 620352 | 6 | 1 | 1 (6) | 7 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (10+10) | DL |  | CC2 | 10 | 15 | 52 | 3394.98 Note 5 | 626332 | 3390.30 | 626020 | 0 | 30 | 7773 | 626304 | 8 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 10 | 15 | 52 | 4105.02 Note 4 | 673668 | 4009.62 | 667308 | 504 | 30 | 8266 | 673632 | -0 | 1 | 0 (2) | 507 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 30 | UL | LOW | CC1 | 20 | 15 | 106 | 3310.02 Note 4 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (20+10) | DL |  | CC2 | 10 | 15 | 52 | 3395.01 | 626334 | 3390.33 | 626022 | 0 | 30 | 7773 | 626304 | 6 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 20 | 15 | 106 | 4110.00 | 674000 | 4009.74 | 667316 | 504 | 30 | 8266 | 673632 | 4 | 0 | 0 (2) | 506 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 40 | UL | LOW | CC1 | 20 | 15 | 106 | 3310.02 Note 4 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (20+20) | DL |  | CC2 | 20 | 15 | 106 | 3390.00 Note 5 | 626000 | 3380.46 | 625364 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (2) | 2 |
|  |  | HIGH | CC1 | 20 | 15 | 106 | 4110.00 Note 4 | 674000 | 4009.74 | 667316 | 504 | 30 | 8266 | 673632 | 4 | 0 | 0 (2) | 506 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 15 | 106 | 4189.98 Note 5 | 679332 | 4089.72 | 672648 | 504 | 30 | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 40 | UL | LOW | CC1 | 30 | 15 | 160 | 3315.00 | 621000 | 3300.60 | 620040 | 0 | 30 | 7711 | 620352 | 0 | 0 | 1 (6) | 6 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (30+10) | DL |  | CC2 | 10 | 15 | 52 | 3394.98 Note 5 | 626332 | 3390.30 | 626020 | 0 | 30 | 7773 | 626304 | 8 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 30 | 15 | 160 | 4115.01 Note 4 | 674334 | 4009.89 | 667326 | 504 | 30 | 8267 | 673728 | 6 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 50 | UL | LOW | CC1 | 40 | 15 | 216 | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+10) | DL |  | CC2 | 10 | 15 | 52 | 3394.98 Note 5 | 626332 | 3390.30 | 626020 | 0 | 30 | 7773 | 626304 | 8 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 40 | 15 | 216 | 4120.02 Note 4 | 674668 | 4009.86 | 667324 | 504 | 30 | 8267 | 673728 | 8 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 60 | UL | LOW | CC1 | 40 | 15 | 216 | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+20) | DL |  | CC2 | 20 | 15 | 106 | 3390.00 | 626000 | 3380.46 | 625364 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (2) | 2 |
|  |  | HIGH | CC1 | 40 | 15 | 216 | 4119.99 | 674666 | 4009.83 | 667322 | 504 | 30 | 8267 | 673728 | 10 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 15 | 106 | 4189.98 Note 5 | 679332 | 4089.72 | 672648 | 504 | 30 | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 60 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+10) | DL |  | CC2 | 10 | 15 | 52 | 3395.01 | 626334 | 3390.33 | 626022 | 0 | 30 | 7773 | 626304 | 6 | 1 | 0 (2) | 3 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4125.00 | 675000 | 4009.98 | 667332 | 504 | 30 | 8267 | 673728 | 0 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 15 | 52 | 4194.99 | 679666 | 4099.59 | 673306 | 504 | 30 | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 70 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+20) | DL |  | CC2 | 20 | 15 | 106 | 3390.00 Note 5 | 626000 | 3380.46 | 625364 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (2) | 2 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4125.00 Note 4 | 675000 | 4009.98 | 667332 | 504 | 30 | 8267 | 673728 | 0 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 15 | 106 | 4189.98 Note 5 | 679332 | 4089.72 | 672648 | 504 | 30 | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 80 | UL | LOW | CC1 | 40 | 15 | 216 | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+40) | DL |  | CC2 | 40 | 15 | 216 | 3379.98 Note 5 | 625332 | 3360.54 | 624036 | 0 | 30 | 7753 | 624384 | 0 | 3 | 1 (6) | 9 |
|  |  | HIGH | CC1 | 40 | 15 | 216 | 4120.02 Note 4 | 674668 | 4009.86 | 667324 | 504 | 30 | 8267 | 673728 | 8 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 40 | 15 | 216 | 4179.99 | 678666 | 4069.83 | 671322 | 504 | 30 | 8308 | 677664 | 6 | 2 | 0 (2) | 508 |
| 80 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+30) | DL |  | CC2 | 30 | 15 | 160 | 3384.99 Note 5 | 625666 | 3370.59 | 624706 | 0 | 30 | 7760 | 625056 | 2 | 3 | 1 (6) | 9 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4125.03 Note 4 | 675002 | 4010.01 | 667334 | 504 | 30 | 8267 | 673728 | 10 | 2 | 1 (6) | 512 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 30 | 15 | 160 | 4185.00 | 679000 | 4079.88 | 671992 | 504 | 30 | 8315 | 678336 | 8 | 2 | 0 (2) | 508 |
| 90 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+40) | DL |  | CC2 | 40 | 15 | 216 | 3380.01 | 625334 | 3360.57 | 624038 | 0 | 30 | 7753 | 624384 | 10 | 2 | 1 (6) | 8 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4125.00 | 675000 | 4009.98 | 667332 | 504 | 30 | 8267 | 673728 | 0 | 3 | 1 (6) | 513 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 40 | 15 | 216 | 4179.99 | 678666 | 4069.83 | 671322 | 504 | 30 | 8308 | 677664 | 6 | 2 | 0 (2) | 508 |
| 100 | UL | LOW | CC1 | 50 | 15 | 270 | 3325.02 Note 4 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+50) | DL |  | CC2 | 50 | 15 | 270 | 3474.99 Note 5 | 631666 | 3450.69 | 630046 | 0 | 30 | 7815 | 630336 | 2 | 2 | 0 (2) | 4 |
|  |  | HIGH | CC1 | 50 | 15 | 270 | 4025.01 Note 4 | 668334 | 3909.99 | 660666 | 504 | 30 | 8197 | 667008 | 6 | 2 | 0 (2) | 508 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 50 | 15 | 270 | 4174.98 Note 5 | 678332 | 4059.96 | 670664 | 504 | 30 | 8301 | 676992 | 4 | 1 | 0 (2) | 507 |
| Note 1: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 4: Centre frequency shifted 15 kHz to align with the synchronisation raster (According to R1-1809879). Note 5: Centre frequency shifted -15 kHz to align with the synchronisation raster (According to R1-1809879). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.5.77.1-4: NR Intra-Band non-contiguous CA configuration CA_n77(2A) with UL CA (PCC=CC1 and SCC=CC2), SCS 30kHz

| Aggregated BW [MHz] (BCS) (CBW combination) | Range |  | CC Note 1 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] Note 1 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 3 | offsetToPointA (SIB1) [PRBs] Note 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | UL | LOW | CC1 | 10 | 30 | 24 | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 | 7711 | 620352 | 18 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (10+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 10 | 30 | 24 | 4105.02 Note 4 | 673668 | 3919.26 | 661284 | 504 | 30 | 8266 | 673632 | 12 | 0 | 0 (0) | 1008 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 30 | UL | LOW | CC1 | 20 | 30 | 51 | 3310.02 Note 4 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (20+10) | DL |  | CC2 | 10 | 30 | 24 | 3395.01 | 626334 | 3390.69 | 626046 | 0 | 30 | 7773 | 626304 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 20 | 30 | 51 | 4110.00 | 674000 | 3919.38 | 661292 | 504 | 30 | 8266 | 673632 | 4 | 0 | 0 (0) | 1008 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 40 | UL | LOW | CC1 | 20 | 30 | 51 | 3310.02 Note 4 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (20+20) | DL |  | CC2 | 20 | 30 | 51 | 3390.00 Note 5 | 626000 | 3380.82 | 625388 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 20 | 30 | 51 | 4110.00 Note 4 | 674000 | 3919.38 | 661292 | 504 | 30 | 8266 | 673632 | 4 | 0 | 0 (0) | 1008 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 30 | 51 | 4189.98 Note 5 | 679332 | 3999.36 | 666624 | 504 | 30 | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 40 | UL | LOW | CC1 | 30 | 30 | 78 | 3315.00 | 621000 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (30+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 30 | 30 | 78 | 4115.01 Note 4 | 674334 | 3919.53 | 661302 | 504 | 30 | 8267 | 673728 | 18 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 50 | UL | LOW | CC1 | 40 | 30 | 106 | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 40 | 30 | 106 | 4120.02 Note 4 | 674668 | 3919.50 | 661300 | 504 | 30 | 8267 | 673728 | 20 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 60 | UL | LOW | CC1 | 40 | 30 | 106 | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+20) | DL |  | CC2 | 20 | 30 | 51 | 3390.00 | 626000 | 3380.82 | 625388 | 0 | 30 | 7766 | 625632 | 4 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 40 | 30 | 106 | 4119.99 | 674666 | 3919.47 | 661298 | 504 | 30 | 8267 | 673728 | 22 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 30 | 51 | 4189.98 Note 5 | 679332 | 3999.36 | 666624 | 504 | 30 | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 60 | UL | LOW | CC1 | 50 | 30 | 133 | 3325.02 Note 4 | 621668 | 3301.08 | 620072 | 0 | 30 | 7711 | 620352 | 16 | 0 | 1 (1) | 2 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (50+10) | DL |  | CC2 | 10 | 30 | 24 | 3395.01 | 626334 | 3390.69 | 626046 | 0 | 30 | 7773 | 626304 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 50 | 30 | 133 | 4125.00 | 675000 | 3919.62 | 661308 | 504 | 30 | 8267 | 673728 | 12 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 70 | UL | LOW | CC1 | 60 | 30 | 162 | 3330.00 | 622000 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (60+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 60 | 30 | 162 | 4130.01 Note 4 | 675334 | 3919.41 | 661294 | 504 | 30 | 8266 | 673632 | 2 | 0 | 0 (0) | 1008 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 80 | UL | LOW | CC1 | 40 | 30 | 106 | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (40+40) | DL |  | CC2 | 40 | 30 | 106 | 3379.98 Note 5 | 625332 | 3360.90 | 624060 | 0 | 30 | 7753 | 624384 | 12 | 0 | 3 (3) | 6 |
|  |  | HIGH | CC1 | 40 | 30 | 106 | 4120.02 Note 4 | 674668 | 3919.50 | 661300 | 504 | 30 | 8267 | 673728 | 20 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 40 | 30 | 106 | 4179.99 | 678666 | 3979.47 | 665298 | 504 | 30 | 8308 | 677664 | 6 | 0 | 1 (1) | 1010 |
| 80 | UL | LOW | CC1 | 70 | 30 | 189 | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 | 7711 | 620352 | 22 | 0 | 1 (1) | 2 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (70+10) | DL |  | CC2 | 10 | 30 | 24 | 3394.98 Note 5 | 626332 | 3390.66 | 626044 | 0 | 30 | 7773 | 626304 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 70 | 30 | 189 | 4135.02 Note 4 | 675668 | 3919.56 | 661304 | 504 | 30 | 8267 | 673728 | 16 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 90 | UL | LOW | CC1 | 80 | 30 | 217 | 3340.02 Note 4 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 100 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (80+10) | DL |  | CC2 | 10 | 30 | 24 | 3395.01 | 626334 | 3390.69 | 626046 | 0 | 30 | 7773 | 626304 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 80 | 30 | 217 | 4140.00 | 676000 | 3919.50 | 661300 | 504 | 30 | 8267 | 673728 | 20 | 0 | 3 (3) | 1014 |
|  |  |  | MaxFsBW = 100 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 100 | UL | LOW | CC1 | 80 | 30 | 217 | 3340.02 Note 4 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (80+20) | DL |  | CC2 | 20 | 30 | 51 | 3489.99 Note 5 | 632666 | 3480.81 | 632054 | 0 | 30 | 7836 | 632352 | 10 | 0 | 2 (2) | 4 |
|  |  | HIGH | CC1 | 80 | 30 | 217 | 4040.01 Note 4 | 669334 | 3819.51 | 654634 | 504 | 30 | 8197 | 667008 | 14 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 30 | 51 | 4189.98 Note 5 | 679332 | 3999.36 | 666624 | 504 | 30 | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 100 | UL | LOW | CC1 | 90 | 30 | 245 | 3345.00 | 623000 | 3300.90 | 620060 | 0 | 30 | 7711 | 620352 | 4 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (90+10) | DL |  | CC2 | 10 | 30 | 24 | 3494.97 Note 5 | 632998 | 3490.65 | 632710 | 0 | 30 | 7843 | 633024 | 2 | 0 | 3 (3) | 6 |
|  |  | HIGH | CC1 | 90 | 30 | 245 | 4045.02 Note 4 | 669668 | 3819.48 | 654632 | 504 | 30 | 8197 | 667008 | 16 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 110 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 90 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+10) | DL |  | CC2 | 10 | 30 | 24 | 3495.00 | 633000 | 3490.68 | 632712 | 0 | 30 | 7843 | 633024 | 0 | 0 | 3 (3) | 6 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 90 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 10 | 30 | 24 | 4194.99 | 679666 | 4009.23 | 667282 | 504 | 30 | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 120 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (0,1) | & |  | MaxFsBW = 200 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+20) | DL |  | CC2 | 20 | 30 | 51 | 3489.99 Note 5 | 632666 | 3480.81 | 632054 | 0 | 30 | 7836 | 632352 | 10 | 0 | 2 (2) | 4 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 Note 4 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 80 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 20 | 30 | 51 | 4189.98 Note 5 | 679332 | 3999.36 | 666624 | 504 | 30 | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 130 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+30) | DL |  | CC2 | 30 | 30 | 78 | 3484.98 Note 5 | 632332 | 3470.94 | 631396 | 0 | 30 | 7829 | 631680 | 20 | 0 | 1 (1) | 2 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.03 Note 4 | 670002 | 3819.45 | 654630 | 504 | 30 | 8197 | 667008 | 18 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 70 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 30 | 30 | 78 | 4185.00 | 679000 | 3989.52 | 665968 | 504 | 30 | 8315 | 678336 | 8 | 0 | 1 (1) | 1010 |
| 140 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (0,1) | & |  | MaxFsBW = 200 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+40) | DL |  | CC2 | 40 | 30 | 106 | 3480.00 | 632000 | 3460.92 | 630728 | 0 | 30 | 7822 | 631008 | 16 | 0 | 1 (1) | 2 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 60 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 40 | 30 | 106 | 4179.99 | 678666 | 3979.47 | 665298 | 504 | 30 | 8308 | 677664 | 6 | 0 | 1 (1) | 1010 |
| 150 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+50) | DL |  | CC2 | 50 | 30 | 133 | 3474.99 Note 5 | 631666 | 3451.05 | 630070 | 0 | 30 | 7815 | 630336 | 2 | 0 | 1 (1) | 2 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 Note 4 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 50 | 30 | 133 | 4174.98 Note 5 | 678332 | 3969.60 | 664640 | 504 | 30 | 8301 | 676992 | 16 | 0 | 0 (0) | 1008 |
| 160 | UL | LOW | CC1 | 80 | 30 | 217 | 3340.02 Note 4 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
| (0) | & |  | MaxFsBW = 200 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (80+80) | DL |  | CC2 | 80 | 30 | 217 | 3459.99 Note 5 | 630666 | 3420.93 | 628062 | 0 | 30 | 7794 | 628320 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 80 | 30 | 217 | 4040.01 Note 4 | 669334 | 3819.51 | 654634 | 504 | 30 | 8197 | 667008 | 14 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 80 | 30 | 217 | 4159.98 Note 5 | 677332 | 3939.48 | 662632 | 504 | 30 | 8280 | 674976 | 8 | 0 | 0 (0) | 1008 |
| 160 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+60) | DL |  | CC2 | 60 | 30 | 162 | 3469.98 Note 5 | 631332 | 3440.82 | 629388 | 0 | 30 | 7808 | 629664 | 12 | 0 | 1 (1) | 2 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.03 Note 4 | 670002 | 3819.45 | 654630 | 504 | 30 | 8197 | 667008 | 18 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 40 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 60 | 30 | 162 | 4170.00 | 678000 | 3959.40 | 663960 | 504 | 30 | 8294 | 676320 | 0 | 0 | 1 (1) | 1010 |
| 170 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+70) | DL |  | CC2 | 70 | 30 | 189 | 3465.00 | 631000 | 3430.98 | 628732 | 0 | 30 | 7801 | 628992 | 20 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 30 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 70 | 30 | 189 | 4164.99 | 677666 | 3949.53 | 663302 | 504 | 30 | 8287 | 675648 | 10 | 0 | 0 (0) | 1008 |
| 180 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (0,1) | & |  | MaxFsBW = 200 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+80) | DL |  | CC2 | 80 | 30 | 217 | 3459.99 Note 5 | 630666 | 3420.93 | 628062 | 0 | 30 | 7794 | 628320 | 18 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 Note 4 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 20 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 80 | 30 | 217 | 4159.98 Note 5 | 677332 | 3939.48 | 662632 | 504 | 30 | 8280 | 674976 | 8 | 0 | 0 (0) | 1008 |
| 190 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (1) | & |  | MaxFsBW = 200 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+90) | DL |  | CC2 | 90 | 30 | 245 | 3454.98 Note 5 | 630332 | 3410.88 | 627392 | 0 | 30 | 7787 | 627648 | 16 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.03 Note 4 | 670002 | 3819.45 | 654630 | 504 | 30 | 8197 | 667008 | 18 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 10 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 90 | 30 | 245 | 4155.00 | 677000 | 3929.46 | 661964 | 504 | 30 | 8273 | 674304 | 4 | 0 | 0 (0) | 1008 |
| 200 | UL | LOW | CC1 | 100 | 30 | 273 | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
| (0,1) | & |  | MaxFsBW = 200 MHz, maxWgap = 0 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (100+100) | DL |  | CC2 | 100 | 30 | 273 | 3450.00 | 630000 | 3400.86 | 626724 | 0 | 30 | 7780 | 626976 | 12 | 0 | 0 (0) | 0 |
|  |  | HIGH | CC1 | 100 | 30 | 273 | 4050.00 | 670000 | 3819.42 | 654628 | 504 | 30 | 8197 | 667008 | 20 | 0 | 1 (1) | 1010 |
|  |  |  | MaxFsBW = 200 MHz, maxWgap = 0 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CC2 | 100 | 30 | 273 | 4149.99 | 676666 | 3919.41 | 661294 | 504 | 30 | 8266 | 673632 | 2 | 0 | 0 (0) | 1008 |
| Note 1: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 4: Centre frequency shifted 15 kHz to align with the synchronisation raster (According to R1-1809879). Note 5: Centre frequency shifted -15 kHz to align with the synchronisation raster (According to R1-1809879). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.5.78	CA_n78(xA)

4.3.1.1.5.78.1	CA_n78(2A)

Editor’s note:	Test frequencies for CA_n78(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.78.1-1: NR Intra-Band non-contiguous CA configuration CA_n78(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A); n78A(10-50MHz) + n78A(10-50MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.78-1: Low range for CBW = 10, 20, 25, 30, 40 OR 50 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.78-1: High range for CBW = 10, 20, 25, 30, 40 OR 50 MHz |
| Note 1: CA_n78(2A) is specified in [7] 38.101-1 with and without uplink CA. PCC is configured on SB1 unless otherwise stated. This table is applicable for the case of without UL CA but test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.78.1-1A: NR Intra-Band non-contiguous CA configuration CA_n78(2A) with UL CA, SCS=15 kHz, Max Wgap

FFS

Table 4.3.1.1.5.78.1-2: NR Intra-Band non-contiguous CA configuration CA_n78(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A); n78A(10-100MHz) + n78A(10-100MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.78-2: Low range for CBW = 10, 20, 25, 30, 40, 50, 60, 70, 80, 90 OR 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.78-2: High range for CBW = 10, 20, 25, 30, 40, 50, 60, 70, 80, 90 OR 100 MHz |
| Note 1: CA_n78(2A) is specified in [7] 38.101-1 with and without uplink CA. PCC is configured on SB1 unless otherwise stated. This table is applicable for the case of without UL CA but test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.78.1-2A: NR Intra-Band non-contiguous CA configuration CA_n78(2A) with UL CA, SCS=30 kHz, Max Wgap

FFS

##### 4.3.1.1.6	NR Operating SUL band combinations in FR1

For SUL band combinations specified in TS 38.101-1 [7] Table 5.5C-1, the test frequencies and signalling parameters in clause 4.3.1.1.1 apply for each operating bands.

For SUL band combinations with intra-band non-contiguous CA specified in TS 38.101-1 [7] Table 5.5C-2, the test frequencies and signalling parameters in clause 4.3.1.1.1 apply for SUL band. The test frequencies and signalling parameters in clause 4.3.1.1.5 apply for each DL CA component carrier.

For SUL band combinations with intra-band contiguous CA specified in TS 38.101-1 [7] Table 5.5C-3, the test frequencies and signalling parameters in clause 4.3.1.1.1 apply for SUL band. The test frequencies and signalling parameters in clause 4.3.1.1.3 apply for each DL CA component carrier.

For SUL band combinations with inter-band CA specified in TS 38.101-1 [7] Table 5.5C-4, the test frequencies and signalling parameters in clause 4.3.1.1.1 apply for each operating bands.

##### 4.3.1.1.7	NR inter-band NR-DC configurations in FR1

###### 4.3.1.1.7.1	NR inter-band NR-DC configurations in FR1 (two bands)

Table 4.3.1.1.7.1-1: Inter-band NR-DC configurations (FR1, two bands)

| NR-DC configuration | Uplink NR-DC configuration | NR-DC downlink configuration band 1 | NR-DC downlink configuration band 2 | NR-DC uplink configuration band 1 | NR-DC uplink configuration band 2 | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_n48A-n70A | DC_n48A-n70A | n48A | n70A | n48A | n70A | Yes |
| Note 1: Protocol testing is limited to NR-DC configurations with 2CC. |  |  |  |  |  |  |