| 3GPP TS 38.533 V18.3.0 (2024-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	56

1	Scope	57

2	References	57

3	Definitions, symbols and abbreviations	58

3.1	Definitions	58

3.2	Symbols	59

3.3	Abbreviations	60

3A	Requirements for the support of RRM	62

3A.1	General	62

3A.1.0	Overview of RRM requirements	62

3A.1.1	Test coverage across 5G NR connectivity options	63

3A.2	Requirements Classification for Statistical Testing	63

3A.3	Antenna Configuration	64

3A.4	NR band groups	64

3A.4.0	General	64

3A.4.1	NR operating bands in FR1	64

3A.4.1A	NR operating bands for satellite access in FR1	66

3A.4.2	NR operating bands in FR2	66

3A.5	NR operating band configuration	67

3A.6	UE with Multiband Capability	68

4	EN-DC with all NR cells in FR1	68

4.0	General	68

4.1	Void	69

4.2	Void	69

4.3	RRC_CONNECTED state mobility	69

4.3.1	Void	69

4.3.2	RRC connection mobility control	69

4.3.2.1	Void	69

4.3.2.2	Random access	69

4.3.2.2.1	EN-DC FR1 contention based random access	69

4.3.2.2.2	EN-DC FR1 non-contention based random access	75

4.3.2.2.3	EN-DC FR1 2-step contention based random access	82

4.3.2.2.4	EN-DC FR1 2-step non-contention based random access	87

4.3.2.3	Void	93

4.4	Timing	93

4.4.1	UE transmit timing	93

4.4.1.0	Minimum conformance requirements	93

4.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	93

4.4.1.1	EN-DC FR1 UE transmit timing accuracy	94

4.4.2	UE timer accuracy	100

4.4.3	Timing advance	100

4.4.3.0	Minimum conformance requirements	100

4.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	100

4.4.3.0.2	Minimum conformance requirements for timing advance adjustment delay	101

4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	101

4.5	Signaling characteristics	106

4.5.1	Radio link monitoring	106

4.5.1.00	General	106

4.5.1.0	Minimum conformance requirements	106

4.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	106

4.5.1.0.2	Void	107

4.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	107

4.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	108

4.5.1.0.5	Requirements for UE configured with Relaxed Measurement Criteria	110

4.5.1.1	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	111

4.5.1.2	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	116

4.5.1.3	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	121

4.5.1.4	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	126

4.5.1.5	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	131

4.5.1.6	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	136

4.5.1.7	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	141

4.5.1.9	EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	152

4.5.2	Interruption	157

4.5.2.0	Minimum conformance requirements	157

4.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX.	157

4.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	158

4.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	159

4.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	159

4.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	161

4.5.2.0.6	Minimum conformance requirements for interruptions due to RRM and RLM/BFD measurements on deactivated NR SCG	162

4.5.2.1	EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	162

4.5.2.2	EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	166

4.5.2.3	EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	170

4.5.2.4	EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	176

4.5.2.5	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	182

4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	188

4.5.2.7	192

4.5.2.8	EN-DC FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	193

4.5.2.9	EN-DC FR1 interruptions at E-UTRA SRS carrier based switching	198

4.5.2.10	EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	204

4.5.3	SCell activation and deactivation delay	208

4.5.3.0	Minimum conformance requirements	208

4.5.3.0.1	SCell activation and deactivation delay	208

4.5.3.0.2	Fast SCell Activation Delay Requirement for Deactivated SCell	212

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	214

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	225

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX	227

4.5.3.5	EN-DC FR1 direct SCell activation at SCell addition of known SCell	229

4.5.3.6	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	238

4.5.3.7	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	247

4.5.4	UE UL carrier RRC reconfiguration delay	250

4.5.4.1	EN-DC FR1 UE UL carrier RRC reconfiguration delay	250

4.5.5	Link recovery procedures	258

4.5.5.0	Minimum conformance requirements	258

4.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	260

4.5.5.0.4	Requirements for Beam Failure Recovery in SCell	260

4.5.5.0.5	TRP specific Minimum conformance requirements	260

4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX	263

4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX	270

4.5.5.3	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	276

4.5.5.4	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX	283

4.5.5.5	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	290

4.5.5.6	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	295

4.5.5.7	EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX	302

4.5.5.8	EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	310

4.5.6	Active BWP switch delay	318

4.5.6.1	DCI-based and time-based active BWP switch	318

4.5.6.1.0	Minimum conformance requirements	318

4.5.6.1.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	321

4.5.6.1.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	328

4.5.6.2	RRC-based active BWP switch	338

4.5.6.2.0	Minimum conformance requirements	338

4.5.6.2.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	338

4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	344

4.5.6.3.0	Minimum conformance requirements	344

4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	346

4.5.6.4	Simultaneous RRC-based active BWP switch on multiple CCs	355

4.5.6.4.0	Minimum conformance requirements	355

4.5.6.4.0.1	Minimum conformance requirements for simultaneous RRC-based BWP switch delay on multiple CCs	355

4.5.6.4.0.2	Minimum conformance requirements for DCI and timer-based BWP switch delay on a single CC	356

4.5.6.4.0.3	Minimum conformance requirements for interruption for NR victim cell	357

4.5.6.4.0.3.1	Interruptions due to SCell dormancy switch	358

4.5.6.4.0.3.2	Interruptions due to CQI measurements during SCell dormancy	358

4.5.6.4.0.3.3	Interruptions due to RRM measurements during SCell dormancy	358

4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	359

4.5.6.5.0	Minimum conformance requirements	359

4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	360

4.5.7	PSCell addition and release delay	368

4.5.7.0	Minimum conformance requirements	368

4.5.7.0.1	NR PSCell Addition Delay Requirement	368

4.5.7.0.2	NR PSCell Release Delay Requirement	369

4.5.7.1	EN-DC FR1 addition and release delay of known PSCell	369

4.5.8	UL switching	374

4.5.8.0	Minimum conformance requirements	374

4.5.8.1	EN-DC FR1 interruptions at switching between two uplink carriers	375

4.5.9	UE specific CBW change	384

4.5.9.0	Minimum conformance requirements	384

4.5.9.0.1	Minimum conformance requirements for UE specific CBW change delay	384

4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN-DC	385

4.5.10	PSCell activation and deactivation delay	389

4.5.10.0	Minimum conformance requirements	389

4.5.10.1	EN-DC FR1 PSCell activation and deactivation delay	390

4.5.11	PSCell addition and release delay	395

4.5.11.0	Minimum conformance requirements	395

4.5.11.0.1	Measurement time	396

4.5.11.1	EN-DC FR1 Conditional PSCell Addition Delay	396

4.6	Measurement procedures	402

4.6.1	Intra-frequency measurements	402

4.6.1.0	Minimum conformance requirements	402

4.6.1.0.1	Minimum conformance requirements for event-triggered reporting without gap	403

4.6.1.0.2	Minimum conformance requirements for event-triggered measurements with gap	407

4.6.1.1	EN-DC FR1 event-triggered reporting without gap in non-DRX	409

4.6.1.2	EN-DC FR1 event-triggered reporting without gap in DRX	414

4.6.1.3	EN-DC FR1 event-triggered reporting with gap in non-DRX	417

4.6.1.4	EN-DC FR1 event-triggered reporting with gap in DRX	423

4.6.1.5	EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection	428

4.6.1.6	EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection	431

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	436

4.6.1.8	EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17	440

4.6.2	Inter-frequency measurements	446

4.6.2.0	Minimum conformance requirements for Inter-frequency measurements	446

4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX	449

4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX	454

4.6.2.3	Void	459

4.6.2.4	Void	459

4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	459

4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection	464

4.6.2.7	Void	469

4.6.2.8	Void	469

4.6.2.9	EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	469

4.6.3	Void	474

4.6.4	L1-RSRP measurement for beam reporting	474

4.6.4.0	Minimum conformance requirements	474

4.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	474

4.6.4.1	EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX	478

4.6.4.1.5	Test requirement	481

4.6.4.2	EN-DC FR1 SSB-based L1-RSRP measurement in DRX	482

4.6.4.2.3	Minimum conformance requirements	482

4.6.4.3	EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX	485

4.6.4.4	EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX	489

4.6.4.5	EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	493

4.6.5	CLI measurements	497

4.6.5.0	Minimum conformance requirements	497

4.6.5.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	498

4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX	499

4.6.6	Measurements with autonomous gaps	507

4.6.6.0	Minimum conformance requirements	507

4.6.6.0.1	Minimum conformance requirements for measurements with autonomous gaps	507

4.6.6.0.1.1	Minimum conformance requirements for E-UTRA FDD - NR measurements with autonomous gaps	507

4.6.6.0.1.2	Minimum conformance requirements for E-UTRA TDD - NR measurements with autonomous gaps	508

4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	508

4.6.7	L1-SINR measurement for beam reporting	513

4.6.7.0	Minimum conformance requirements	513

4.6.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	518

4.6.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	524

4.6.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	529

4.6.8	CSI-RS based intra-frequency measurement	533

4.6.8.0	Minimum conformance requirements	533

4.6.8.0.1	Minimum conformance requirements for EN-DC event triggered reporting tests without gap under DRX	533

4.6.9	CSI-RS based inter-frequency measurement	535

4.6.9.0	Minimum conformance requirements	535

4.6.9.0.1	Minimum conformance requirements for EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	535

4.7	Measurement performance requirements	537

4.7.1	SS-RSRP	537

4.7.1.0	Minimum conformance requirements	537

4.7.1.1	Intra-frequency measurements	542

4.7.1.2	Inter-frequency measurements	550

4.7.2	SS-RSRQ	557

4.7.2.0	Minimum conformance requirements	557

4.7.2.1	EN-DC FR1 SS-RSRQ measurement accuracy	559

4.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	564

4.7.2.2.1	EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy	564

4.7.2.2.2	EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy	568

4.7.3	SS-SINR	570

4.7.3.0	Minimum conformance requirements	570

4.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	570

4.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	571

4.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	572

4.7.3.1	EN-DC FR1 SS-SINR measurement accuracy	572

4.7.3.2	Inter-Frequency SS-SINR measurement accuracy	576

4.7.3.2.1	EN-DC FR1-FR1 SS-SINR absolute measurement accuracy	576

4.7.3.2.2	EN-DC FR1-FR1 SS-SINR relative measurement accuracy	580

4.7.4	L1-RSRP	582

4.7.4.0	Minimum conformance requirements	582

4.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	582

4.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	585

4.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	586

4.7.4.1	SSB based L1-RSRP measurements	587

4.7.4.1.1	EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy	587

4.7.4.1.2	EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy	592

4.7.4.2	CSI-RS based L1-RSRP measurements	594

4.7.4.2.1	EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy	594

4.7.4.2.2	EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy	598

4.7.5	SFTD	600

4.7.5.0	Minimum conformance requirements	600

4.7.5.0.1	SFTD Accuracy Requirement	600

4.7.5.1	EN-DC FR1 SFTD measurement accuracy	601

4.7.6	CLI measurements	605

4.7.6.0	Minimum conformance requirements	605

4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy	605

4.7.6.0.1.1	SRS-RSRP report mapping	606

4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	607

4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	607

4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	616

4.7.7	L1-SINR measurement for beam reporting	619

4.7.7.0	Minimum conformance requirements	619

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	619

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	622

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	624

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	628

4.7.7.1.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	628

4.7.7.1.2	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	633

4.7.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	635

4.7.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement	640

4.7.7.3.1	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	640

4.7.7.3.2	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	646

4.7.8	CSI-RSRP	648

4.7.8.0	Minimum conformance requirements	648

4.7.8.0.1	Intra-frequency absolute CSI-RSRP measurement accuracy	648

4.7.8.0.2	Intra-frequency relative CSI-RSRP measurement accuracy requirements	651

4.7.8.0.3	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	652

4.7.8.0.4	Inter-frequency relative CSI-RSRP measurement accuracy requirements	653

4.7.8.1	Intra-frequency CSI-RSRP measurement accuracy	654

4.7.8.1.1	EN-DC FR1 CSI-RSRP absolute measurement accuracy	654

4.7.8.1.2	EN-DC FR1 CSI-RSRP relative measurement accuracy	661

4.7.8.2	Inter-frequency measurement accuracy	663

4.7.8.2.1	EN-DC FR1 CSI-RSRP absolute measurement accuracy	663

4.7.8.2.2	EN-DC FR1 CSI-RSRP relative measurement accuracy	669

4.7.9	CSI-RSRQ	671

4.7.9.0	Minimum conformance requirements	671

4.7.9.0.1	Intra-frequency CSI-RSRQ accuracy requirements	671

4.7.9.0.2	Inter-frequency CSI-RSRQ accuracy requirements	672

4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	675

4.7.9.1.1	Test purpose	675

4.7.9.1.2	Test applicability	675

4.7.9.1.3	Minimum conformance requirements	675

4.7.9.1.4	Test description	675

4.7.9.1.5	Test Requirements	677

4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	681

4.7.9.2.1	Test purpose	681

4.7.9.2.2	  Test applicability	681

4.7.9.2.3	Minimum conformance requirements	681

4.7.9.2.4	Test description	681

4.7.9.2.5	Test Requirements	683

4.7.10	CSI-SINR	687

4.7.10.0	Minimum conformance requirements	687

4.7.10.0.1	Intra-frequency CSI-SINR accuracy requirements in FR1	687

4.7.10.0.1.1	Absolute CSI-SINR accuracy in FR1	687

4.7.10.0.2	Inter-frequency CSI-SINR accuracy requirements in FR1	688

4.7.10.0.2.1	Absolute accuracy of CSI-SINR in FR1	688

4.7.10.0.2.2	Relative accuracy of CSI-SINR in FR1	690

4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	691

4.7.10.1.1	Test purpose	691

4.7.10.1.2	  Test applicability	691

4.7.10.1.3	Minimum conformance requirements	691

4.7.10.1.4	Test description	691

4.7.10.1.5	Test Requirements	693

4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	696

4.7.10.2.1	Test purpose	696

4.7.10.2.2	Test applicability	696

4.7.10.2.3	Minimum conformance requirements	696

4.7.10.2.4	Test description	697

4.7.10.2.5	Test Requirements	698

4A	NE-DC with all NR cells in FR1	701

4A.0	General	701

4A.1	Signalling characteristics	701

4A.1.1	E-UTRAN PSCell addition	701

4A.1.1.0	Minimum conformance requirements	701

4A.1.1.0.1	E-UTRA PSCell Addition Delay Requirement	701

4A.1.1.0.2	E-UTRA PSCell Release Delay Requirement	702

4A.1.1.1	NE-DC FR1 addition and release delay of known PSCell	702

4A.1.2	Active BWP switch delay	708

4A.1.2.0	Minimum conformance requirements	708

4A.1.2.1	NE-DC FR1 DCI-based and timer-based DL active BWP switch in non-DRX in synchronous NE-DC	708

4A.2	Measurement performance requirements	712

4A.2.1	SFTD accuracy	712

4A.2.1.0	Minimum conformance requirements	712

4A.1.1.0.1	NE-DC SFTD accuracy Requirement	712

4A.2.1.1	NE-DC FR1 SFTD accuracy	714

5	EN-DC with at least one NR cell in FR2	720

5.0	General	720

5.1	Void	720

5.2	Void	720

5.3	RRC_CONNECTED state mobility	720

5.3.1	Void	720

5.3.2	RRC connection mobility control	720

5.3.2.1	Void	720

5.3.2.2	Random access	720

5.3.2.2.1	EN-DC FR2 contention based random access	720

5.3.2.2.2	EN-DC FR2 non-contention based random access	728

5.3.2.2.3	EN-DC FR2 2-step contention based random access	736

5.3.2.2.4	EN-DC FR2 2-step non-contention based random access	740

5.3.2.3	Void	744

5.4	Timing	744

5.4.1	UE transmit timing	744

5.4.1.0	Minimum Conformance Requirements	744

5.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	744

5.4.1.1	EN-DC FR2 UE transmit timing accuracy	746

5.4.2	UE timer accuracy	752

5.4.3	Timing advance	752

5.4.3.0	Minimum conformance requirements	752

5.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	752

5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	752

5.5	Signaling characteristics	757

5.5.1	Radio link monitoring	757

5.5.1.0	Minimum conformance requirements	757

5.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	757

5.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	758

5.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	759

5.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	762

5.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	765

5.5.1.0.6	Requirements for UE configured with Relaxed Measurement Criteria	766

5.5.1.1	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	766

5.5.1.2	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	773

5.5.1.3	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	779

5.5.1.4	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	785

5.5.1.5	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	790

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	795

5.5.1.7	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	800

5.5.1.9	EN-DC FR2 radio link monitoring UE scheduling restrictions	813

5.5.1.10	EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	818

5.5.2	Interruption	823

5.5.2.0	Minimum conformance requirements	823

5.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX	823

5.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	824

5.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	825

5.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	825

5.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	827

5.5.2.1	EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	828

5.5.2.2	EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	832

5.5.2.3	EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	836

5.5.2.4	EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	842

5.5.2.5	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	847

5.5.2.6	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	851

5.5.2.7	EN-DC FR2 interruptions at E-UTRA SRS carrier based switching	855

5.5.2.8	EN-DC FR2 interruptions at NR SRS carrier based switching	861

5.5.3	SCell activation and deactivation delay	867

5.5.3.1	EN-DC FR2 SCell activation and deactivation intra-band in non-DRX	879

5.5.3.2 to 5.5.3.6	886

5.5.3.7	EN-DC FR2 direct SCell activation at SCell addition of known SCell	886

5.5.3.8	EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band	890

5.5.4	UE UL carrier RRC reconfiguration delay	899

5.5.5	Link recovery procedures	899

5.5.5.0	Minimum conformance requirements	899

5.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	899

5.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	903

5.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	908

5.5.5.0.4	Requirements for Beam Failure Recovery in SCell	909

5.5.5.0.5	Requirements for SSB based beam failure detection for UE fulfilling relaxed measurement criteria	909

5.5.5.0.6	TRP specific Minimum conformance requirements	910

5.5.5.0.6.1	Requirements for TRP specific CSI-RS based beam failure detection	910

5.5.5.0.6.2	Requirements for CSI-RS based candidate beam detection	911

5.5.5.0.6.3	Requirements for TRP specific Beam Failure Recovery	912

5.5.5.0.6.4	Scheduling availability of UE during TRP specific beam failure detection	912

5.5.5.0.6.5	Scheduling availability of UE during TRP specific candidate beam detection	912

5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX	913

5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX	920

5.5.5.3	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	927

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX	933

5.5.5.5	EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX	940

5.5.5.6	EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX	947

5.5.5.7	EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX	954

5.5.5.8	EN-DC FR2 CSI-RS-based PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode	960

5.5.5.9	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion	967

5.5.6	Active BWP switch delay	974

5.5.6.1	DCI-based and time-based active BWP switch	974

5.5.6.1.0	Minimum conformance requirements	974

5.5.6.1.1	EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	974

5.5.6.1.2	EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	975

5.5.6.2	RRC-based active BWP switch	975

5.5.6.2.0	Minimum conformance requirements	975

5.5.6.2.1	EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	976

5.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	983

5.5.6.3.0	Minimum conformance requirements	983

5.5.6.3.0.1	Minimum conformance requirements for simultaneous DCI-based and timer-based active BWP switch on multiple CCs	983

5.5.6.3.0.1.1	Minimum conformance requirements for simultaneous DCI-based active BWP switch on multiple CCs	983

5.5.6.3.0.1.2	Minimum conformance requirements for simultaneous timer-based active BWP switch on multiple CCs	985

5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	985

5.5.6.4	SCell dormancy switch	989

5.5.6.4.0	Minimum conformance requirements	989

5.5.6.4.0.1	Minimum conformance requirements for interruptions due to SCell dormancy switch	989

5.5.6.4.0.2	Minimum conformance requirements for interruptions due to CQI measurements during SCell dormancy	990

5.5.6.4.0.3	Minimum conformance requirements for interruptions due to RRM measurements during SCell dormancy	990

5.5.6.4.0.4	Minimum conformance requirements for DCI and timer-based BWP switch delay on a single CC	990

5.5.6.4.0.5	Minimum conformance requirements for simultaneous DCI based BWP switch delay on multiple CCs	991

5.5.6.4.0.6	Minimum conformance requirements for interruptions due to NR SCell dormancy switch	993

5.5.6.4.0.7	Minimum conformance requirements for interruptions due to CSI and RRM measurements during SCell dormancy	993

5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 Scell inside active time	993

5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	997

5.5.6.5.0	Minimum conformance requirements	997

5.5.6.5.0.1	Minimum conformance requirements for E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	997

5.5.7	Void	998

5.5.8	Active TCI state switch delay	998

5.5.8.0	Minimum conformance requirements	998

5.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	998

5.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	999

5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch	1000

5.5.8.2	EN-DC FR2 RRC based active TCI state switch	1004

5.5.9	Uplink spatial relation switch delay	1008

5.5.9.0	Minimum conformance requirements	1008

5.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	1008

5.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	1008

5.5.9.1	MAC-CE based uplink spatial relation switch	1009

5.5.9.1.1	EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation	1009

5.5.9.2	RRC based uplink spatial relation switch	1013

5.5.9.2.1	EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS	1013

5.5.10	UE specific CBW change	1017

5.5.10.0	Minimum conformance requirements	1017

5.5.10.0.1	Minimum conformance requirements for UE specific CBW change	1017

5.5.10.1	UE specific CBW change on FR2 NR PSCell	1018

5.5.11	Unified TCI state switch delay	1022

5.5.11.0	Minimum conformance requirements	1022

5.5.11.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	1022

5.5.11.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	1024

5.5.11.1	EN-DC FR2 MAC-CE based active joint TCI state switch	1026

5.5.11.2	EN-DC FR2 MAC-CE based active uplink TCI state switch	1033

5.5.11.3	EN-DC FR2 MAC-CE based active downlink TCI state switch	1039

5.5.12	PSCell activation and deactivation delay	1045

5.5.12.0	Minimum conformance requirements	1045

5.5.12.1	EN-DC FR2 PSCell activation and deactivation delay	1046

5.5.13	Conditional PSCell addition and release delay	1050

5.5.13.1	EN-DC FR2 Addition and Release Delay of NR PSCell	1050

5.6	Measurement procedures	1056

5.6.1	Intra-frequency measurements	1056

5.6.1.0	Minimum conformance requirements	1056

5.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1056

5.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1059

5.6.1.1	EN-DC FR2 event-triggered reporting without gap in non-DRX	1060

5.6.1.2	EN-DC FR2 event-triggered reporting without gap in DRX	1066

5.6.1.3	EN-DC FR2 event-triggered reporting with gap in non-DRX	1070

5.6.1.4	EN-DC FR2 event-triggered reporting with gap in DRX	1076

5.6.2	Inter-frequency measurements	1081

5.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1081

5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX	1083

5.6.2.2	EN-DC FR2-FR2 event-triggered reporting in DRX	1087

5.6.2.3	EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	1092

5.6.2.4	EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection	1096

5.6.2.5	EN-DC FR1-FR2 event-triggered reporting in non-DRX	1101

5.6.2.6	EN-DC FR1-FR2 event-triggered reporting in DRX	1106

5.6.2.7	EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	1112

5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection	1117

5.6.3	L1-RSRP measurement for beam reporting	1122

5.6.3.0	Minimum conformance requirements	1123

5.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1123

5.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1124

5.6.3.1	EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX	1127

5.6.3.2	EN-DC FR2 SSB-based L1-RSRP measurement in DRX	1131

5.6.3.3	EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX	1135

5.6.3.4	EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX	1139

5.6.4	CLI measurements	1142

5.6.4.0	Minimum conformance requirements	1142

5.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	1143

5.6.4.1	EN-DC FR2 SRS-RSRP measurement in non-DRX	1144

5.6.4.2	EN-DC FR2 CLI-RSSI measurement in non-DRX	1147

5.6.5	Measurements with autonomous gaps	1151

5.6.5.0	Minimum conformance requirements	1151

5.6.5.0.1	Minimum conformance requirements for EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1151

5.6.5.0.1.1	Introduction	1151

5.6.5.0.1.2	CGI identification of an NR cell with autonomous gaps	1151

5.6.5.0.1.3	CGI reporting delay	1152

5.6.5.1	EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1152

5.6.6	L1-SINR measurement for beam reporting	1157

5.6.6.0	Minimum conformance requirements	1157

5.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1157

5.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1160

5.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1162

5.6.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1165

5.6.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1169

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1175

5.6.7	CSI-RS based Intra-frequency Measurements	1179

5.6.7.0	Minimum conformance requirements	1179

5.6.7.0.1	Minimum conformance requirements for event-triggered measurement without gap	1179

5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1182

5.6.8	CSI-RS based Inter-frequency Measurements	1187

5.6.8.0	Minimum conformance requirements for Inter-frequency measurements	1187

5.6.8.1	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1189

5.7	Measurement performance requirements	1195

5.7.1	SS-RSRP	1195

5.7.1.0	Minimum conformance requirements	1195

5.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	1195

5.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	1196

5.7.1.1	EN-DC FR2 SS-RSRP measurement accuracy	1198

5.7.1.2	EN-DC FR2-FR2 SS-RSRP measurement accuracy	1205

5.7.1.3	EN-DC FR1-FR2 SS-RSRP measurement accuracy	1212

5.7.2	SS-RSRQ	1218

5.7.2.0	Minimum conformance requirements	1218

5.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1218

5.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	1218

5.7.2.1	EN-DC FR2 SS-RSRQ measurement accuracy	1220

5.7.2.2	EN-DC FR2-FR2 SS-RSRQ measurement accuracy	1224

5.7.3	SS-SINR	1229

5.7.3.0	Minimum conformance requirements	1229

5.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1229

5.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	1229

5.7.3.1	EN-DC FR2 SS-SINR measurement accuracy	1231

5.7.3.2	EN-DC FR2-FR2 SS-SINR measurement accuracy	1235

5.7.4	L1-RSRP	1240

5.7.4.0	Minimum conformance requirements	1240

5.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	1241

5.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	1241

5.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	1242

5.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	1243

5.7.4.1	EN-DC FR2 SSB based L1-RSRP measurement accuracy	1244

5.7.4.2	EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy	1250

5.7.5	SRS-RSRP	1256

5.7.5.0	Minimum conformance requirements	1256

5.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	1256

5.7.5.1	EN-DC FR2 SRS-RSRP measurement accuracy	1258

5.7.5.2	EN-DC FR2 CLI-RSSI measurement accuracy	1263

5.7.5.2.1	Test purpose	1264

5.7.5.2.2	Test applicability	1264

5.7.5.2.3	Minimum conformance requirements	1264

5.7.5.2.4	Test description	1264

5.7.5.2.5	Test requirement	1265

5.7.6	L1-SINR measurement for beam reporting	1268

5.7.6.0	Minimum conformance requirements	1268

5.7.6.0.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1268

5.7.6.0.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1269

5.7.6.0.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1272

5.7.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	1274

5.7.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1279

5.7.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1284

5.7.7	CSI-RSRP	1289

5.7.7.0	Minimum conformance requirements	1289

5.7.7.0.1	Intra-frequency CSI-RSRP measurement accuracy requirements	1289

5.7.7.0.2	Inter-frequency CSI-RSRP measurement accuracy requirements	1291

5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1293

5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1300

5.7.8	CSI-RSRQ	1307

5.7.8.0	Minimum conformance requirements	1307

5.7.8.0.1	Intra-frequency CSI-RSRQ accuracy requirements	1307

5.7.8.0.1.1	Absolute CSI-RSRQ accuracy	1307

5.7.8.0.2	Inter-frequency CSI-RSRQ accuracy requirements	1308

5.7.8.0.2.1	Absolute CSI-RSRQ accuracy	1309

5.7.8.0.2.2	Relative CSI-RSRQ accuracy	1310

5.7.8.1	EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1311

5.7.8.1.1	Test purpose	1311

5.7.8.1.2	Test applicability	1311

5.7.8.1.3	Minimum conformance requirements	1311

5.7.8.1.4	Test description	1311

5.7.8.1.5	Test Requirements	1313

5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1315

5.7.8.2.1	Test purpose	1315

5.7.8.2.2	Test applicability	1315

5.7.8.2.3	Minimum conformance requirements	1315

5.7.8.2.4	Test description	1315

5.7.8.2.5	Test Requirements	1317

5.7.9	CSI-SINR	1319

5.7.9.0	Minimum conformance requirements	1319

5.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements	1319

5.7.9.0.1.1	Absolute CSI-SINR accuracy	1319

5.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements	1320

5.7.9.0.2.1	Absolute CSI-SINR accuracy	1320

5.7.9.0.2.2	Relative CSI-SINR accuracy	1322

5.7.9.1	EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1323

5.7.9.1.1	Test purpose	1323

5.7.9.1.2	Test applicability	1323

5.7.9.1.3	Minimum conformance requirements	1323

5.7.9.1.4	Test description	1323

5.7.9.1.5	Test Requirements	1325

5.7.9.2	EN-DC inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1327

5.7.9.2.1	Test purpose	1327

5.7.9.2.2	  Test applicability	1327

5.7.9.2.3	Minimum conformance requirements	1327

5.7.9.2.4	Test description	1328

5.7.9.2.5	Test Requirements	1329

6	NR standalone in FR1	1332

6.0	General	1332

6.1	RRC_IDLE state mobility	1332

6.1.1	NR cell re-selection	1332

6.1.1.0	Minimum conformance requirements	1332

6.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	1332

6.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	1332

6.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with highSpeedMeasFlag-r16	1335

6.1.1.0.4	Minimum conformance requirements for intra-frequency cell re-selection when UE configured with relaxed measurement criterion	1335

6.1.1.0.5	Minimum conformance requirements for inter-frequency cell re-selection when UE configured with relaxed measurement criterion	1337

6.1.1.1	NR SA FR1 cell re-selection	1339

6.1.1.1.5	Test requirement	1343

6.1.1.2	NR SA FR1-FR1 cell re-selection	1345

6.1.1.3	NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1349

6.1.1.4	NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1356

6.1.1.5	NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1361

6.1.1.6	NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1368

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16	1374

6.1.1.8	NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17	1379

6.1.2	NR – E-UTRA cell re-selection	1385

6.1.2.0	Minimum conformance requirements	1385

6.1.2.0.1	Minimum conformance requirements for NR – E-UTRA cell re-selection	1385

6.1.2.0.2	Minimum conformance requirement for inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	1387

6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA	1388

6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA	1394

6.1.2.3	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1400

6.1.2.4	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1406

6.1.2.5	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16	1411

6.2	RRC_INACTIVE state mobility	1417

6.2.0	General	1417

6.2.0.1	Minimum conformance requirements for TA validation using CG-SDT	1417

6.2.1	NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)	1418

6.2.1.1	Test Purpose	1418

6.2.1.2	Test Applicability	1418

6.2.1.3	Minimum Conformance Requirement	1418

6.2.1.4	Test Description	1418

6.2.1.4.1	Initial conditions	1419

6.2.1.4.2	Test Procedure	1421

6.2.1.5	Test Requirement	1423

6.3	RRC_CONNECTED state mobility	1424

6.3.1	Handover	1424

6.3.1.0	Minimum conformance requirements	1424

6.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	1424

6.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	1424

6.3.1.0.3	Minimum conformance requirements for NR – UTRAN handover	1425

6.3.1.0.4	Minimum conformance requirements for NR FR1- NR FR1 DAPS handover	1426

6.3.1.1	NR SA FR1 handover with known target cell	1428

6.3.1.2	NR SA FR1 handover with unknown target cell	1432

6.3.1.3	NR SA FR1-FR1 handover with unknown target cell	1435

6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell	1438

6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell	1446

6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell	1451

6.3.1.7	NR SA FR1 synchronous DAPS handover	1457

6.3.1.8	NR SA FR1 asynchronous DAPS handover	1465

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover	1470

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover	1479

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover	1485

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover	1494

6.3.2	RRC connection mobility control	1502

6.3.2.1	RRC re-establishment	1502

6.3.2.1.0	Minimum conformance requirements	1502

6.3.2.1.1	NR SA FR1 RRC re-establishment	1504

6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment	1508

6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing	1512

6.3.2.2	Random access	1516

6.3.2.2.0	Minimum conformance requirements	1516

6.3.2.2.1	NR SA FR1 contention based random access	1518

6.3.2.2.2	NR SA FR1 non-contention based random access	1524

6.3.2.2.3	NR SA FR1 2-step contention based random access	1530

6.3.2.2.4	NR SA FR1 2-step non-contention based random access	1534

6.3.2.3	RRC connection release with redirection	1539

6.3.2.3.0	Minimum conformance requirements	1539

6.3.2.3.1	NR SA FR1 RRC connection release with redirection	1540

6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection	1544

6.3.3	Conditional handover	1548

6.3.3.0	Minimum conformance requirements	1548

6.3.3.0.1	Minimum conformance requirements for NR FR1 intra-frequency conditional handover	1548

6.3.3.0.2	Minimum conformance requirements for NR FR1 inter-frequency conditional handover	1552

6.3.3.1	NR SA FR1 conditional handover	1555

6.3.3.2	NR SA FR1-FR1 conditional handover	1561

6.4	Timing	1569

6.4.1	UE transmit timing	1569

6.4.1.0	Minimum conformance requirements	1569

6.4.1.1	NR SA FR1 UE transmit timing accuracy	1570

6.4.2	UE timer accuracy	1576

6.4.3	Timing advance	1576

6.4.3.0	Minimum conformance requirement	1576

6.4.3.0.1	Minimum conformance requirement for timing advance adjustment	1576

6.4.3.1	NR SA FR1 timing advance adjustment accuracy	1576

6.5	Signaling characteristics	1581

6.5.1	Radio link monitoring	1581

6.5.1.00	General	1581

6.5.1.0	Minimum conformance requirements	1583

6.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	1583

6.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	1584

6.5.1.0.3	Minimum conformance requirements for out-of-sync and in-sync CSI-RS based RLM	1586

6.5.1.0.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	1587

6.5.1.0.5	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	1587

6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1588

6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1594

6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	1600

6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	1605

6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1612

6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1619

6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1625

6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1632

6.5.1.9	SA FR1 radio link monitoring out-of-sync Test for PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	1638

6.5.1.9.1	Test purpose	1638

6.5.1.9.2	Test applicability	1639

6.5.1.9.3	Minimum conformance requirements	1639

6.5.1.9.4	Test description	1639

6.5.1.9.5	Test requirement	1644

6.5.2	Interruption	1646

6.5.2.0	Minimum conformance requirements	1646

6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	1646

6.5.2.0.2	 Interruptions at NR SRS carrier based switching	1647

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC	1649

6.5.2.2	SA FR1 interruptions at NR SRS carrier based switching	1654

6.5.3	SCell activation and deactivation delay	1659

6.5.3.0	Minimum conformance requirements	1659

6.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	1659

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	1660

6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	1673

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX	1674

6.5.3.4	NR SA FR1 direct SCell activation at SCell addition of known SCell	1676

6.5.3.5	NR SA FR1 direct SCell activation at handover with known SCell	1685

6.5.3.6 to 6.5.3.9	1694

6.5.3.10	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	1694

6.5.3.11	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	1702

6.5.4	UE UL carrier RRC reconfiguration delay	1705

6.5.4.0	Minimum conformance requirements	1705

6.5.4.0.1	Minimum conformance requirements for UL carrier RRC reconfiguration delay	1705

6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay	1705

6.5.5	Link recovery procedures	1712

6.5.5.0	Minimum conformance requirements	1712

6.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	1712

6.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	1712

6.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	1714

6.5.5.0.4	Requirements for Beam Failure Recovery in SCell	1714

6.5.5.0.5	TRP specific Minimum conformance requirements	1714

6.5.5.0.5.1	Requirements for CSI-RS based beam failure detection	1714

6.5.5.0.5.2	Requirements for CSI-RS based candidate beam detection	1715

6.5.5.0.5.3	Requirements for TRP specific Beam Failure Recovery	1716

6.5.5.0.5.4	Scheduling availability of UE during TRP specific beam failure detection	1716

6.5.5.0.5.5	Scheduling availability of FR1 UE during TRP specific candidate beam detection	1716

6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX	1716

6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX	1724

6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	1732

6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX	1739

6.5.5.5	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	1746

6.5.5.6	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	1754

6.5.5.7	NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	1761

6.5.6	Active BWP switch delay	1769

6.5.6.1	DCI-based and time-based active BWP switch	1769

6.5.6.1.0	Minimum conformance requirements	1769

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX	1770

6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX	1781

6.5.6.2	RRC-based active BWP switch	1790

6.5.6.2.0	Minimum conformance requirements	1790

6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX	1790

6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1798

6.5.6.3.0	Minimum conformance requirements	1798

6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1798

6.5.6.4	1808

6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1808

6.5.6.5.0	Minimum conformance requirements	1808

6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch in non-DRX on multiple CCs	1809

6.5.7	DL interruptions at switching between two uplink carriers	1816

6.5.7.0	Minimum conformance requirements	1816

6.5.7.1	NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA	1816

6.5.7.2	NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA	1828

6.5.7A	NR SA FR1 DL interruptions at switching between two uplink carriers with two transmit antenna connectors	1836

6.5.7A.0	Minimum conformance requirements	1836

6.5.7A.1	NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA	1837

6.5.7A.2	NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA	1845

6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	1853

6.5.7B.0	Minimum conformance requirements	1853

6.5.7B.1	NR SA FR1 DL Interruptions at switching between two uplink bands in FDD-TDD CA	1854

6.5.7B.2	NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA	1864

6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	1874

6.5.7C.0	Minimum conformance requirements	1874

6.5.7C.1	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	1875

6.5.7C.1.1	Test purpose	1875

6.5.7C.1.2	Test applicability	1875

6.5.7C.1.3	Minimum conformance requirements	1875

6.5.7C.1.4	Test description	1875

6.5.7C.1.4.1	Initial conditions	1875

6.5.7C.1.5	Test requirements	1883

6.5.7C.2	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	1886

6.5.7C.2.1	Test purpose	1886

6.5.7C.2.2	Test applicability	1886

6.5.7C.2.3	Minimum conformance requirements	1886

6.5.7C.2.4	Test description	1887

6.5.7C.2.4.1	Initial conditions	1887

6.5.7D	 DL interruptions at UE switching across three or four uplink bands	1898

6.5.7D.0	Minimum conformance requirements	1898

6.5.7D.1	NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG	1898

6.5.7D.2	NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG	1903

6.5.7D.3	NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs	1910

6.5.8	UE specific CBW change	1915

6.5.8.0	Minimum conformance requirements	1915

6.5.8.0.1	Minimum conformance requirements for UE specific CBW change	1915

6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1916

6.6	Measurement procedures	1923

6.6.1	Intra-frequency measurements	1923

6.6.1.0	Minimum conformance requirements	1923

6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1923

6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1923

6.6.1.0.3	Void	1924

6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading	1924

6.6.1.0.5	 Void	1926

6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX	1926

6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX	1930

6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX	1935

6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX	1941

6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading	1946

6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading	1950

6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	1954

6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17	1959

6.6.2	Inter-frequency measurements	1964

6.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1964

6.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps	1964

6.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps	1964

6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX	1967

6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX	1973

6.6.2.3	Void	1979

6.6.2.4	Void	1979

6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	1979

6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection	1985

6.6.2.7	Void	1991

6.6.2.8	Void	1991

6.6.2.9	NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern	1991

6.6.2.10	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used	1996

6.6.2.11	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used	2002

6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	2008

6.6.3	Inter-RAT Measurements	2014

6.6.3.0	Minimum conformance requirements	2014

6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	2014

6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	2016

6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX	2019

6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX	2026

6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16	2032

6.6.4	L1-RSRP measurement for beam reporting	2040

6.6.4.0	Minimum conformance requirements	2040

6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	2040

6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	2040

6.6.4.0.3	Inter-cell SSB based L1-RSRP Reporting	2040

6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX	2041

6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX	2045

6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX	2048

6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX	2053

6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	2056

6.6.4.6	NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX	2062

6.6.5	UTRAN inter-RAT measurement	2067

6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX	2067

6.6.6	CLI Measurements	2074

6.6.6.0	Minimum conformance requirements	2074

6.6.6.0.1	Minimum conformance requirements for SRS-RSRP measurement	2074

6.6.6.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	2074

6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX	2074

6.6.7	NR measurements with autonomous gaps	2081

6.6.7.0	Minimum conformance requirements	2081

6.6.7.0.1	Minimum conformance requirements for SA intra-frequency CGI identification of NR neighbour cell in FR1	2081

6.6.7.0.1.1	Introduction	2081

6.6.7.0.1.2	CGI identification of an NR cell with autonomous gaps	2081

6.6.7.0.1.3	CGI reporting delay	2082

6.6.7.0.2	Minimum conformance requirements for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2082

6.6.7.0.2.1	CGI identification of an E-UTRA cell with autonomous gaps	2082

6.6.7.0.2.2	CGI reporting delay	2083

6.6.7.1	SA intra-frequency CGI identification of NR neighbour cell in FR1	2083

6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2087

6.6.8	L1-SINR measurement for beam reporting	2091

6.6.8.0	Minimum conformance requirements	2091

6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	2091

6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	2091

6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	2091

6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	2091

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	2097

6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	2103

6.6.9	Idle Mode CA/DC Measurements	2109

6.6.9.0	Minimum conformance requirements	2109

6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1	2111

6.6.10 to 6.6.14	2121

6.6.15	Idle Mode inter-RAT CA/DC Measurements	2121

6.6.15.0	Minimum conformance requirements	2121

6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting	2122

6.6.16	2135

6.6.17	NR SA FR1 event triggered reporting tests with Pre-MG	2135

6.6.17.1	NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG	2137

6.6.17.2	NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2143

6.6.18	SA event triggered reporting tests with concurrent gaps	2151

6.6.18.0	Minimum conformance requirements	2151

6.6.18.0.1	Minimum conformance requirements for Intra-frequency measurement	2151

6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement	2152

6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement	2153

6.6.18.0.4	Minimum conformance requirements for PRS measurement	2154

6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers	2154

6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers	2162

6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2169

6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2181

6.6.19	SA event triggered reporting tests with NCSG	2190

6.6.19.0	Minimum conformance requirements	2190

6.6.19.0.1	Minimum conformance requirements for intra-frequency measurement	2190

6.6.19.0.2	Minimum conformance requirements for inter-frequency measurement	2192

6.6.19.0.3	Minimum conformance requirements for inter-RAT measurement	2193

6.6.19.0.4	Minimum conformance requirements for interruptions	2194

6.6.19.1	NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1	2195

6.6.19.2	NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2201

6.6.19.3	NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2207

6.6.19.4	NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2215

6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2221

6.6.20.0	Minimum conformance requirements	2221

6.6.20.1	NR SA FR1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC	2224

6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS	2228

6.6.21.0	Minimum conformance requirements	2228

6.6.21.1	NR SA FR1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC	2231

6.7	Measurement performance requirements	2235

6.7.1	SS-RSRP	2235

6.7.1.0	Minimum conformance requirements	2235

6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	2235

6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	2235

6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	2235

6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	2236

6.7.1.1	Intra-frequency measurements	2236

6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy	2236

6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy	2243

6.7.1.2	Inter-frequency measurements	2245

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy	2245

6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy	2252

6.7.2	SS-RSRQ	2254

6.7.2.0	Minimum conformance requirements	2254

6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2254

6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	2254

6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	2254

6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy	2254

6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	2258

6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy	2258

6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy	2263

6.7.3	SS-SINR	2265

6.7.3.0	Minimum conformance requirements	2265

6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2265

6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	2265

6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	2265

6.7.3.1	NR SA FR1 SS-SINR measurement accuracy	2265

6.7.3.2	Inter-Frequency SS-SINR measurement accuracy	2269

6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy	2269

6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy	2273

6.7.4	L1-RSRP	2275

6.7.4.0	Minimum conformance requirements	2275

6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	2275

6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	2275

6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	2275

6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	2275

6.7.4.1	SSB based L1-RSRP measurements	2275

6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy	2275

6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy	2280

6.7.4.2	CSI-RS based L1-RSRP measurements	2282

6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy	2282

6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy	2286

6.7.5	E-UTRAN RSRP	2288

6.7.5.0	Minimum conformance requirements	2288

6.7.5.0.1	E-UTRAN RSRP absolute accuracy	2288

6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy	2289

6.7.6	E-UTRAN RSRQ	2296

6.7.6.0	Minimum conformance requirements	2296

6.7.6.0.1	E-UTRAN RSRQ absolute accuracy	2296

6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy	2297

6.7.7	E-UTRAN RS-SINR	2304

6.7.7.0	Minimum conformance requirements	2304

6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy	2304

6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy	2305

6.7.8	CLI Measurements	2312

6.7.8.0	Minimum conformance requirements	2312

6.7.8.0.1	Minimum conformance requirements for SRS-RSRP accuracy	2312

6.7.8.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	2312

6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy	2312

6.7.8.1.1	Test purpose	2312

6.7.8.1.2	Test applicability	2312

6.7.8.1.3	Minimum conformance requirements	2313

6.7.8.1.4	Test description	2313

6.7.8.1.5	Test requirement	2315

6.7.9	L1-SINR measurement for beam reporting	2322

6.7.9.0	Minimum conformance requirements	2322

6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2322

6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	2325

6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	2327

6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	2331

6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	2331

6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	2334

6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2336

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2339

6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2340

6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	2343

6.7.11	CSI-RSRQ	2345

6.7.11.0	Minimum conformance requirements	2345

6.7.11.0.1	Intra-frequency CSI-RSRQ accuracy requirements	2345

6.7.11.0.1.1	Absolute CSI-RSRQ accuracy	2345

6.7.11.0.2	Inter-frequency CSI-RSRQ accuracy requirements	2346

6.7.11.0.2.1	Absolute CSI-RSRQ accuracy	2347

6.7.11.0.2.2	Relative CSI-RSRQ accuracy	2348

6.7.11.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2349

6.7.11.1.1	Test purpose	2349

6.7.11.1.2	  Test applicability	2349

6.7.11.1.3	Minimum conformance requirements	2349

6.7.11.1.4	Test description	2349

6.7.11.1.5	Test Requirements	2351

6.7.11.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2356

6.7.11.2.1	Test purpose	2356

6.7.11.2.2	Test applicability	2356

6.7.11.2.3	Minimum conformance requirements	2356

6.7.11.2.4	Test description	2356

6.7.11.2.5	Test Requirements	2358

6.7.12	CSI-SINR	2363

6.7.12.0	Minimum conformance requirements	2363

6.7.12.0.1	Intra-frequency CSI-SINR accuracy requirements	2363

6.7.12.0.1.1	Absolute CSI-SINR accuracy	2363

6.7.12.0.2	Inter-frequency CSI-SINR accuracy requirements	2365

[TS 38.133, Clause 10.1.14.2 and Clause 10.1.16]	2365

6.7.12.0.2.1	Absolute CSI-SINR accuracy	2365

6.7.12.0.2.2	Relative CSI-SINR accuracy	2366

6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2367

6.7.12.1.1	Test purpose	2367

6.7.12.1.2	  Test applicability	2367

6.7.12.1.3	Minimum conformance requirements	2367

6.7.12.1.4	Test description	2367

6.7.12.1.5	Test Requirements	2369

6.7.12.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2373

6.7.12.2.1	Test purpose	2373

6.7.12.2.2	  Test applicability	2374

6.7.12.2.3	Minimum conformance requirements	2374

6.7.12.2.4	Test description	2374

6.7.12.2.5	Test Requirements	2375

7	NR standalone with at least one NR cell in FR2	2381

7.0	General	2381

7.1	RRC_IDLE state mobility	2381

7.1.1	NR cell re-selection	2381

7.1.1.0	Minimum conformance requirements	2381

7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	2381

7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	2384

7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion	2385

7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion	2386

7.1.1.1	NR SA FR2 cell re-selection	2388

7.1.1.2	NR SA FR2-FR2 cell re-selection	2393

7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2400

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2405

7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2410

7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2417

7.2	RRC_INACTIVE state mobility	2429

7.2.1	Small Data Transmission	2429

7.2.1.0	Minimum conformance requirements	2429

7.2.1.0.1	Minimum conformance requirements for CG-SDT TA validation	2429

7.2.1.1	TA Validation for CG-SDT in FR2	2430

7.3	RRC_CONNECTED state mobility	2439

7.3.1	Handover	2439

7.3.1.0	Minimum conformance requirements	2439

7.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	2439

7.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR2 handover	2440

7.3.1.0.3	Minimum conformance requirements for NR FR1 – NR FR2 DAPS handover	2441

7.3.1.1	NR SA FR1-FR2 Inter-frequency handover; unknown target cell	2442

7.3.1.2	NR SA FR2 Intra-frequency handover; unknown target cell	2447

7.3.1.3	NR SA FR2-FR2 Inter-frequency handover; unknown target cell	2451

7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover	2455

7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover	2465

7.3.2	RRC connection mobility control	2472

7.3.2.1	RRC re-establishment	2472

7.3.2.1.0	Minimum conformance requirements	2472

7.3.2.1.1	NR SA FR2 RRC re-establishment	2473

7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment	2477

7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	2480

7.3.2.2	Random access	2485

7.3.2.2.0	Minimum conformance requirements	2485

7.3.2.2.1	NR SA FR2 contention based random access	2486

7.3.2.2.2	NR SA FR2 non-contention based random access	2494

7.3.2.2.3	NR SA FR2 2-step contention based random access	2501

7.3.2.2.4	NR SA FR2 2-step non-contention based random access	2508

7.3.2.3	RRC connection release with redirection	2515

7.3.2.3.0	Minimum conformance requirements	2515

7.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection	2516

7.3.3	Conditional handover	2520

7.3.3.0	Minimum conformance requirements	2520

7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover	2520

7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover	2524

7.3.3.1	NR SA FR2 conditional handover	2528

7.3.3.2	NR SA FR2-FR2 conditional handover	2535

7.4	Timing	2542

7.4.1	UE transmit timing	2542

7.4.1.0	Minimum Conformance Requirements	2542

7.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	2542

7.4.1.1	NR SA FR2 UE transmit timing accuracy	2543

7.4.2	UE timer accuracy	2549

7.4.3	Timing advance	2549

7.4.3.0	Minimum conformance requirements	2549

7.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	2549

7.4.3.1	NR SA FR2 timing advance adjustment accuracy	2550

7.5	Signalling characteristics	2556

7.5.1	Radio link monitoring	2556

7.5.1.0	Minimum conformance requirements	2556

7.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	2556

7.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	2556

7.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	2556

7.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	2556

7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	2556

7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2556

7.5.1.1.1	Test purpose	2557

7.5.1.1.2	Test applicability	2557

7.5.1.1.3	Minimum conformance requirement	2557

7.5.1.1.4	Test description	2557

7.5.1.1.5	Test Requirement	2561

7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2562

7.5.1.2.1	Test purpose	2562

7.5.1.2.2	Test applicability	2562

7.5.1.2.3	Minimum conformance requirement	2562

7.5.1.2.4	Test description	2562

7.5.1.2.5	Test Requirement	2568

7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2569

7.5.1.3.1	Test purpose	2569

7.5.1.3.2	Test applicability	2569

7.5.1.3.3	Minimum conformance requirement	2569

7.5.1.3.4	Test description	2569

7.5.1.3.5	Test Requirement	2573

7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2574

7.5.1.4.1	Test purpose	2574

7.5.1.4.2	Test applicability	2574

7.5.1.4.3	Minimum conformance requirement	2574

7.5.1.4.4	Test description	2574

7.5.1.4.5	Test Requirement	2580

7.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	2581

7.5.1.5.1	Test purpose	2581

7.5.1.5.2	Test applicability	2581

7.5.1.5.3	Minimum conformance requirement	2581

7.5.1.5.4	Test description	2581

7.5.1.5.5	Test Requirement	2585

7.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2586

7.5.1.6.1	Test purpose	2587

7.5.1.6.2	Test applicability	2587

7.5.1.6.3	Minimum conformance requirement	2587

7.5.1.6.4	Test description	2587

7.5.1.6.5	Test Requirement	2592

7.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM RS in DRX mode	2593

7.5.1.7.1	Test purpose	2593

7.5.1.7.2	Test applicability	2593

7.5.1.7.3	Minimum conformance requirement	2593

7.5.1.7.4	Test description	2593

7.5.1.7.5	Test Requirement	2597

7.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2598

7.5.1.8.1	Test purpose	2598

7.5.1.8.2	Test applicability	2598

7.5.1.8.3	Minimum conformance requirement	2598

7.5.1.8.4	Test description	2598

7.5.1.8.5	Test Requirement	2603

7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	2604

7.5.1.9.1	Test purpose	2605

7.5.1.9.2	Test applicability	2605

7.5.1.9.3	Minimum conformance requirement	2605

7.5.1.9.4	Test description	2605

7.5.1.9.5	Test Requirement	2607

7.5.2	Interruption	2609

7.5.2.0	Minimum conformance requirements	2609

7.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	2609

7.5.2.0.2	Minimum conformance requirements for SA interruptions at NR SRS carrier-based switching	2610

7.5.2.1	NR SA FR2 interruptions during measurements on deactivated NR SCC	2612

7.5.2.1.1	Test purpose	2612

7.5.2.1.2	Test applicability	2612

7.5.2.1.3	Minimum conformance requirements	2612

7.5.2.1.4	Test description	2612

7.5.2.1.5	Test requirement	2614

7.5.2.2	SA interruptions at NR SRS carrier-based switching	2617

7.5.2.2.1	Test purpose	2617

7.5.2.2.2	Test applicability	2617

7.5.2.2.3	Minimum conformance requirements	2617

7.5.2.2.4	Test description	2617

7.5.2.2.5	Test requirement	2618

7.5.3	SCell activation and deactivation delay	2620

7.5.3.0	Minimum conformance requirements	2620

7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell	2620

7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell	2623

7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay	2623

7.5.3.1.1	Test purpose	2624

7.5.3.1.2	Test applicability	2624

7.5.3.1.3	Minimum conformance requirements	2624

7.5.3.1.4	Test description	2624

7.5.3.1.5	Test requirement	2625

7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay	2625

7.5.3.2.1	Test purpose	2626

7.5.3.2.2	Test applicability	2626

7.5.3.2.3	Minimum conformance requirements	2626

7.5.3.2.4	Test description	2626

7.5.3.2.5	Test requirement	2627

7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	2628

7.5.3.3.1	Test purpose	2628

7.5.3.3.2	Test applicability	2628

7.5.3.3.3	Minimum conformance requirements	2628

7.5.3.3.4	Test description	2628

7.5.3.3.5	Test requirement	2633

7.5.3.4	NR SA FR2 direct SCell activation at SCell addition of known SCell	2636

7.5.3.4.1	Test Purpose	2636

7.5.3.4.2	Test applicability	2636

7.5.3.4.3	Minimum conformance requirements	2636

7.5.3.4.4	Test description	2636

7.5.3.4.5	Test requirement	2641

7.5.3.5	NR SA FR2 direct SCell activation at handover with known SCell	2643

7.5.3.5.1	Test Purpose	2643

7.5.3.5.2	Test applicability	2643

7.5.3.5.3	Minimum conformance requirements	2643

7.5.3.5.4	Test description	2643

7.5.3.5.5	Test requirement	2648

7.5.3.6 to 7.5.3.12	2651

7.5.3.13	NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX	2651

7.5.3.13.1	Test Purpose	2651

7.5.3.13.2	Test applicability	2651

7.5.3.13.3	Minimum conformance requirements	2651

7.5.3.13.4	Test description	2651

7.5.3.13.5	Test requirement	2656

7.5.3.14	NR SA FR2 SCell Activation for known SCell in FR2 inter-band	2658

-	This test case is incomplete for UE power class other than PC3.7.5.3.14.1 Test purpose	2659

7.5.3.14.2	Test applicability	2659

7.5.3.14.3	Minimum conformance requirements	2659

7.5.3.14.4	Test description	2659

7.5.3.14.5	Test requirement	2664

7.5.4	Void	2665

7.5.5	Link recovery procedures	2665

7.5.5.0	Minimum conformance requirements	2665

7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	2665

7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	2665

7.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	2666

7.5.5.0.4	Requirements for Beam Failure Recovery in SCell	2666

7.5.5.0.5	TRP specific Minimum conformance requirements	2666

7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX	2667

7.5.5.1.1	Test purpose	2668

7.5.5.1.2	Test applicability	2668

7.5.5.1.3	Minimum conformance requirements	2668

7.5.5.1.4	Test description	2668

7.5.5.1.5	Test requirement	2671

7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX	2672

7.5.5.2.1	Test purpose	2673

7.5.5.2.2	Test applicability	2673

7.5.5.2.3	Minimum conformance requirements	2673

7.5.5.2.4	Test description	2673

7.5.5.2.5	Test requirement	2677

7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	2679

7.5.5.3.1	Test purpose	2679

7.5.5.3.2	Test applicability	2679

7.5.5.3.3	Minimum conformance requirements	2679

7.5.5.3.4	Test description	2679

7.5.5.3.5	Test requirement	2683

7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX	2684

7.5.5.4.1	Test purpose	2685

7.5.5.4.2	Test applicability	2685

7.5.5.4.3	Minimum conformance requirements	2685

7.5.5.4.4	Test description	2685

7.5.5.4.5	Test requirement	2689

7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX	2691

7.5.5.5.1	Test purpose	2691

7.5.5.5.2	Test applicability	2691

7.5.5.5.3	Minimum conformance requirements	2691

7.5.5.5.4	Test description	2691

7.5.5.5.5	Test requirement	2695

7.5.5.6	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX	2696

7.5.5.6.1	Test purpose	2696

7.5.5.6.2	Test applicability	2697

7.5.5.6.3	Minimum conformance requirements	2697

7.5.5.6.4	Test description	2697

7.5.5.6.5	Test requirement	2701

7.5.5.7	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX	2703

7.5.5.7.1	Test purpose	2703

7.5.5.7.2	Test applicability	2703

7.5.5.7.3	Minimum conformance requirements	2703

7.5.5.7.4	Test description	2703

7.5.5.7.5	Test requirement	2707

7.5.5.8	FFS	2709

7.5.5.9	NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	2709

7.5.5.9.1	Test purpose	2709

7.5.5.9.2	Test applicability	2709

7.5.5.9.3	Minimum conformance requirements	2709

7.5.5.9.4	Test description	2709

7.5.5.9.5	Test requirements	2714

7.5.5.10	NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX	2715

7.5.5.10.1	Test purpose	2715

7.5.5.10.2	Test applicability	2715

7.5.5.10.3	Minimum conformance requirements	2715

7.5.5.10.4	Test description	2715

7.5.5.10.5	Test requirement	2720

7.5.6	Active BWP switch delay	2722

7.5.6.1	DCI-based and time-based active BWP switch	2722

7.5.6.1.0	Minimum conformance requirements	2722

7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2722

7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX	2723

7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2724

7.5.6.2	RRC-based active BWP switch	2725

7.5.6.2.0	Minimum conformance requirements	2725

7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX	2726

7.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	2728

7.5.6.3.0	Minimum conformance requirements	2728

7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	2731

7.5.7	PSCell addition and release delay	2735

7.5.7.0	Minimum conformance requirements	2735

7.5.7.0.1	Minimum conformance requirements for PSCell addition delay	2735

7.5.7.0.2	Minimum conformance requirements for PSCell release delay	2736

7.5.7.1	NR SA FR2 addition and release delay of known PSCell	2736

7.5.7.1.1	Test purpose	2737

7.5.7.1.2	Test applicability	2737

7.5.7.1.3	Minimum conformance requirements	2737

7.5.7.1.4	Test description	2737

7.5.7.1.5	Test requirement	2738

7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell	2738

7.5.7.2.1	Test purpose	2739

7.5.7.2.2	Test applicability	2739

7.5.7.2.3	Minimum conformance requirements	2739

7.5.7.2.4	Test description	2739

7.5.7.2.5	Test requirement	2740

7.5.8	Active TCI state switch delay	2740

7.5.8.0	Minimum conformance requirements	2740

7.5.8.0.0	Known conditions for TCI state	2740

7.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	2741

7.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	2741

7.5.8.0.3 	Minimum conformance requirements for MAC-CE based active TCI state switch in HST FR2	2742

7.5.8.1	NR SA FR2 MAC-CE based active TCI state switch	2743

7.5.8.1.1	NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state	2743

7.5.8.2	NR SA FR2 RRC based active TCI state switch	2749

7.5.8.2.1	NR SA Pcell FR2 RRC based active TCI state switch for a known TCI state	2749

7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	2756

7.5.8.3.1	NR SA FR2 HST active TCI state switch for a known TCI state	2756

7.5.9	Uplink spatial relation switch delay	2762

7.5.9.0	Minimum conformance requirements	2762

7.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	2762

7.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	2762

7.5.9.1	MAC-CE based uplink spatial relation switch	2763

7.5.9.1.1	NR PCell FR2 uplink spatial relation switch associated with known DL-RS	2763

The rate of correct events observed during repeated tests shall be at least [90]%.7.5.9.2	RRC based uplink spatial relation switch	2766

7.5.9.2.1	NR PCell FR2 uplink spatial relation switch associated with known DL-RS	2766

7.5.10	UE specific CBW change	2770

7.5.10.0	Minimum conformance requirements	2770

7.5.10.0.1	Minimum conformance requirements for NR FR2 UE specific CBW change of PCell with non-DRX in SA	2770

7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	2771

7.5.10.1.1	Test purpose	2771

7.5.10.1.2	Test applicability	2771

7.5.10.1.3	Minimum conformance requirements	2771

7.5.10.1.4	Test description	2771

7.5.10.1.5	Test requirement	2772

7.5.11	UE UL carrier RRC reconfiguration delay	2774

7.5.11.1	UE UL carrier RRC reconfiguration delay	2774

7.5.11.1.1	Test purpose	2774

7.5.11.1.2	Test applicability	2775

7.5.11.1.3	Minimum conformance requirements	2775

7.5.11.1.4	Test description	2775

7.5.11.1.5	Test requirement	2777

7.5.12	Conditional PSCell addition and release delay (FR2 SA)	2780

7.5.12.1	NR SA FR2 Addition and Release Delay of PSCell	2780

7.5.13	Unified TCI state switch delay	2785

7.5.13.0	Minimum conformance requirements	2785

7.5.13.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	2785

7.5.13.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	2786

7.5.13.1	NR SA FR2 MAC-CE based active joint TCI state switch	2786

7.5.13.1.1	Test purpose	2786

7.5.13.1.2	Test applicability	2786

7.5.13.1.3	Minimum conformance requirements	2786

7.5.13.1.4	Test description	2786

7.5.13.1.5	Test requirement	2790

7.5.13.2	NR SA FR2 MAC-CE based active uplink TCI state switch	2792

7.5.13.2.1	Test purpose	2792

7.5.13.2.2	Test applicability	2792

7.5.13.2.3	Minimum conformance requirements	2792

7.5.13.2.4	Test description	2793

7.5.13.2.5	Test requirement	2796

7.5.13.3	NR SA FR2 MAC-CE based active downlink TCI state switch	2798

7.5.13.3.1	Test purpose	2798

7.5.13.3.2	Test applicability	2798

7.5.13.3.3	Minimum conformance requirements	2798

7.5.13.3.4	Test description	2799

7.5.13.3.5	Test requirement	2802

7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	2804

7.5.14.1	Test purpose	2804

7.5.14.2	Test applicability	2804

7.5.14.3	Minimum conformance requirements	2804

7.5.14.3.1	SCG Activation Delay Requirement	2804

7.5.14.3.2	SCG Deactivation Delay Requirement	2806

7.5.14.4	Test description	2806

7.5.14.4.1	Initial conditions	2806

7.5.14.4.3	Message contents	2808

7.5.14.5	Test requirement	2808

7.6	Measurement procedures	2811

7.6.1	Intra-frequency measurements	2811

7.6.1.0	Minimum conformance requirements	2811

7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	2811

7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	2812

7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX	2814

7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX	2819

7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX	2825

7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	2831

7.6.1.5	NR SA FR2 event triggered reporting test without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17	2837

7.6.2	Inter-frequency measurements	2841

7.6.2.0	Minimum conformance requirements for Inter-frequency measurements	2841

7.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps	2841

7.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps	2842

7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX	2844

7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX	2849

7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	2855

7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection	2860

7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX	2866

7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX	2872

7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	2878

7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection	2884

7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	2890

7.6.2.10	SA FR2-FR2 event triggered reporting test without gap under non-DRX	2895

7.6.2.11	SA FR2-FR2 event triggered reporting test without gap under DRX	2899

7.6.3	L1-RSRP measurement for beam reporting	2903

7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	2903

7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	2903

7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	2903

7.6.3.0.3	Minimum conformance requirements for Inter-cell SSB based L1-RSRP Reporting	2903

7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX	2905

7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX	2910

7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX	2914

7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX	2918

7.6.3.5	NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	2921

7.6.3.6	NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	2927

7.6.4	CLI measurements	2934

7.6.4.0	Minimum conformance requirements	2934

7.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	2934

7.6.4.1	NR SA FR2 SRS-RSRP measurement in non-DRX	2935

7.6.4.2	NR SA FR2 CLI-RSSI measurement in non-DRX	2939

7.6.4.2.1	Test purpose	2939

7.6.4.2.2	Test applicability	2939

7.6.4.2.3	Minimum conformance requirements	2939

7.6.4.2.4	Test description	2939

7.6.4.2.5	Test requirement	2941

7.6.5	NR measurements with autonomous gaps	2942

7.6.5.0	Minimum conformance requirements	2942

7.6.5.0.1	Minimum conformance requirements for SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	2942

7.6.5.0.1.1	Minimum conformance requirements for interruptions when identifying CGI of an NR cell with autonomous gaps	2942

7.6.5.0.1.2	Minimum conformance requirements for NR measurements with autonomous gaps	2943

7.6.5.0.1.2.1	Introduction	2943

7.6.5.0.1.2.2	CGI identification of an NR cell with autonomous gaps	2943

7.6.5.0.1.2.3	CGI reporting delay	2944

7.6.5.1	SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	2944

7.6.6	L1-SINR measurement for beam reporting	2949

7.6.6.0	Minimum conformance requirements	2949

7.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	2949

7.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	2949

7.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	2949

7.6.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	2949

7.6.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	2953

7.6.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	2958

7.6.7 to 7.6.12	2964

7.6.13	UE Rx-Tx time difference measurements for PDC	2964

7.6.13.0	Minimum conformance requirements	2964

7.6.13.0.1	PRS based UE Rx-Tx time difference for propagation delay compensation	2964

7.6.13.0.2	TRS based UE Rx-Tx time difference for propagation delay compensation	2966

7.6.13.1	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS	2968

7.6.13.2	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS	2972

7.6.14	SA event triggered reporting tests with Pre-MG	2976

7.6.14.0	Minimum conformance requirements	2976

7.6.14.1	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation	2979

7.6.14.2	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2985

7.6.15	SA event triggered reporting tests with concurrent gaps	2991

7.6.15.0	Minimum conformance requirements	2991

7.6.15.0.1	Minimum conformance requirements for Intra-frequency measurement	2991

7.6.15.0.2	Minimum conformance requirements for Inter-frequency measurement	2991

7.6.15.0.3	Minimum conformance requirements for PRS measurement	2992

7.6.15.1	NR SA FR2 event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	2992

7.6.15.2	NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	2998

7.6.15.3	NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3007

7.6.16	SA event triggered reporting tests with NCSG	3014

7.6.16.0	Minimum conformance requirements	3014

7.6.16.0.1	Minimum conformance requirements for intra-frequency measurement	3014

7.6.16.0.2	Minimum conformance requirements for inter-frequency measurement	3016

7.6.16.0.3	Minimum conformance requirements for interruptions	3017

7.6.16.1	NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX	3018

7.6.16.2	NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used (PCell in FR2)	3024

7.6.16.3	NR SA FR2 event triggered reporting tests on deactivated SCell measurement via NCSG in non-DRX	3029

7.7	Measurement performance requirements	3035

7.7.1	SS-RSRP	3035

7.7.1.0	Minimum conformance requirements	3035

7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	3035

7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	3035

7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy	3035

7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy	3043

7.7.1.3	Inter-frequency measurements between FR1 and FR2	3051

7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy	3051

7.7.1.3.2	Void	3058

7.7.2	SS-RSRQ	3058

7.7.2.0	Minimum conformance requirements	3058

7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	3058

7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	3058

7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy	3058

7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy	3062

7.7.3	SS-SINR	3068

7.7.3.0	Minimum conformance requirements	3068

7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	3068

7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	3068

7.7.3.1	NR SA FR2 SS-SINR measurement accuracy	3068

7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy	3073

7.7.4	L1-RSRP	3080

7.7.4.0	Minimum conformance requirements	3080

7.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	3080

7.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	3080

7.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	3080

7.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	3080

7.7.4.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	3080

7.7.4.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy	3085

7.7.5	SRS-RSRP	3091

7.7.5.0	Minimum conformance requirements	3091

7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	3091

7.7.5.1	NR SA FR2 SRS-RSRP measurement accuracy	3093

7.7.5.2	NR SA FR2 CLI-RSSI measurement accuracy	3098

7.7.5.2.1	Test purpose	3099

7.7.5.2.2	Test applicability	3099

7.7.5.2.3	Minimum conformance requirements	3099

7.7.5.2.4	Test description	3099

7.7.5.2.4	Test description	3099

7.7.5.2.5	Test requirement	3100

7.7.6	L1-SINR	3103

7.7.6.0	Minimum conformance requirements	3103

7.7.6.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	3103

7.7.6.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR L1-SINR measurement accuracy	3105

7.7.6.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	3107

7.7.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	3110

7.7.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy	3115

7.7.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	3120

7.7.8	CSI-RSRQ	3125

7.7.8.0	Minimum conformance requirements	3125

7.7.8.0.1	Intra-frequency CSI-RSRQ accuracy requirements	3125

7.7.8.0.1.1	Absolute CSI-RSRQ accuracy	3125

7.7.8.0.2	Inter-frequency CSI-RSRQ accuracy requirements	3126

7.7.8.0.2.1	Absolute CSI-RSRQ accuracy	3126

7.7.8.0.2.2	Relative CSI-RSRQ accuracy	3128

7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3128

7.7.8.1.1	Test purpose	3128

7.7.8.1.2	Test applicability	3129

7.7.8.1.3	Minimum conformance requirements	3129

7.7.8.1.4	Test description	3129

7.7.8.1.5	Test Requirements	3130

7.7.8.2	SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3132

7.7.8.2.1	Test purpose	3132

7.7.8.2.2	Test applicability	3132

7.7.8.2.3	Minimum conformance requirements	3132

7.7.8.2.4	Test description	3132

7.7.8.2.5	Test Requirements	3134

7.7.9	CSI-SINR	3136

7.7.9.0	Minimum conformance requirements	3136

7.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements	3136

7.7.9.0.1.1	Absolute CSI-SINR accuracy	3136

7.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements	3137

7.7.9.0.2.1	Absolute CSI-SINR accuracy	3137

7.7.9.0.2.2	Relative CSI-SINR accuracy	3138

7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3139

7.7.9.1.1	Test purpose	3139

7.7.9.1.2	  Test applicability	3139

7.7.9.1.3	Minimum conformance requirements	3139

7.7.9.1.4	Test description	3139

7.7.9.1.5	Test Requirements	3141

7.7.9.2	SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3143

7.7.9.2.1	Test purpose	3143

7.7.9.2.2	Test applicability	3143

7.7.9.2.3	Minimum conformance requirements	3143

7.7.9.2.4	Test description	3144

7.7.9.2.5	Test Requirements	3145

8.0	General	3148

8.1	Void	3148

8.2.1.0	Minimum conformance requirements	3148

8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection	3148

8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell	3150

8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3157

8.2.2.0	Minimum conformance requirements	3164

8.2.2.1	E-UTRA – NR FR1 Early Measurement Reporting	3166

8.2.2.2	E-UTRA – NR FR2 Early Measurement Reporting	3174

8.3.1.0	Minimum conformance requirements	3179

8.3.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 handover	3179

8.3.1.1	E-UTRA - NR FR1 handover with known target cell	3180

8.4.1.0	Minimum conformance requirements	3185

8.4.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 SFTD measurement delay	3185

8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX	3186

8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX	3192

8.4.2.0	Minimum conformance requirements	3197

8.4.2.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	3197

8.4.2.0.2	Void	3201

8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX	3201

8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX	3207

8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX	3213

8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX	3219

8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX	3225

8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX	3230

8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX	3235

8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX	3240

8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3245

8.5.1	SFTD measurement accuracy	3251

8.5.1.0	Minimum conformance requirements	3251

8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	3251

8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy	3252

8.5.2	Inter-RAT measurement accuracy	3258

8.5.2.1	SS-RSRP	3258

8.5.2.1.0	Minimum conformance requirements	3258

8.5.2.1.1	SS-RSRP with NR FR1 target cell	3259

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell	3264

8.5.2.2	SS-RSRQ	3269

8.5.2.2.0	Minimum conformance requirements	3269

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell	3269

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell	3272

8.5.2.3	SS-SINR	3277

8.5.2.3.0	Minimum conformance requirements	3277

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell	3277

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell	3280

9	NR sidelink	3285

9.1	NR sidelink in FR1	3285

9.1.1	UE transmit timing	3285

9.1.1.0	Minimum conformance requirements	3285

9.1.1.0.1	Minimum conformance requirements for GNSS as synchronization reference source	3285

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	3285

9.1.1.0.3	Minimum conformance requirements for FR1 NR Cell as synchronization reference source	3285

9.1.1.1	NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source	3286

9.1.1.2	NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source	3288

9.1.1.3	NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source	3291

9.1.2	Initiation/Cease of S-SSB transmission	3296

9.1.2.0	Minimum conformance requirements	3296

9.1.2.0.1	Minimum conformance requirements for FR1 NR cell as synchronization reference source	3296

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	3297

9.1.2.1	NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source	3298

9.1.2.2	NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source	3303

9.1.3	Synchronization reference selection/reselection	3307

9.1.3.0	Minimum conformance requirements	3307

9.1.3.0.1	Minimum conformance requirements for GNSS configured as the highest priority synchronization reference source	3307

9.1.3.0.2	Minimum conformance requirements for eNB/gNB configured as the highest priority synchronization reference source	3308

9.1.3.1	NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source	3310

9.1.3.2	NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source	3316

9.1.4	L1 SL-RSRP measurements	3321

9.1.4.0	Minimum conformance requirements	3321

9.1.4.0.1	Minimum conformance requirements for resource selection/reselection, re-evaluation and pre-emption	3321

9.1.4.1	NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection	3322

9.1.4.2	NR SA FR1 L1 SL-RSRP measurement for resource pre-emption	3328

9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation	3334

9.1.5	Congestion control measurement	3341

9.1.5.0	Minimum conformance requirements	3341

9.1.5.0.1	Minimum conformance requirements for congestion control measurements	3341

9.1.5.1	NR SA FR1 congestion control measurement for concurrent operation	3342

9.1.5.2	NR SA FR1 congestion control measurement for PC5-only operation	3350

9.1.6	Congestion control measurement	3356

9.1.6.0	Minimum conformance requirements	3356

9.1.6.0.1	Minimum conformance requirements for interruption to WAN due to NR sidelink communication	3356

9.1.6.1	NR SA FR1 interruption to WAN due to NR sidelink communication	3356

10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	3360

10.0	General	3360

10.0.1	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	3360

10.1	RRC_CONNECTED state mobility	3361

10.1.1	Random Access	3361

10.1.1.0	Minimum conformance requirements	3361

10.1.1.0.1	Minimum conformance requirements for 4-step RA type	3361

10.1.1.0.1.1	Contention based random access	3361

10.1.1.0.1.1.1	Correct behaviour when transmitting Random Access Preamble	3361

10.1.1.0.1.1.2	Correct behaviour when receiving Random Access Response	3362

10.1.1.0.1.1.3	Correct behaviour when not receiving Random Access Response	3362

10.1.1.0.1.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	3362

10.1.1.0.1.1.6	Correct behaviour when contention Resolution timer expires	3362

10.1.1.0.1.2	Non-Contention based random access	3362

10.1.1.0.1.2.1	Correct behaviour when transmitting Random Access Preamble	3362

10.1.1.0.1.2.2	Correct behaviour when receiving Random Access Response	3363

10.1.1.0.1.2.3	Correct behaviour when not receiving Random Access Response	3363

10.1.1.0.2	Minimum conformance requirements for 2-step RA type	3363

10.1.1.0.2.1	Contention based random access	3364

10.1.1.0.2.1.1	Correct behaviour when transmitting MsgA	3364

10.1.1.0.2.1.2	Correct behaviour when receiving MsgB	3364

10.1.1.0.2.1.3	Correct behaviour when not receiving MsgB	3365

10.1.1.0.2.2	Non-Contention based random access	3365

10.1.1.0.2.2.1	Correct behaviour when transmitting MsgA	3365

10.1.1.0.2.2.2	Correct behaviour when receiving MsgB	3365

10.1.1.0.2.2.3	Correct behaviour when not receiving MsgB	3365

10.1.1.1.1	EN-DC FR1 4-step RA type contention based random access test for NR PSCell with CCA	3366

10.1.1.1.2	EN-DC FR1 4-step RA type non-contention based random access for NR PSCell with CCA	3370

10.1.1.1.3	EN-DC FR1 2-step RA type contention-based random access for NR PSCell with CCA	3374

10.1.1.1.4	EN-DC FR1 2-step RA type non-contention based random access for NR PSCell with CCA	3378

10.1.2	NR FR1 Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	3382

10.2	Timing	3389

10.2.1	UE transmit timing	3389

10.2.1.0	Minimum conformance requirements	3389

10.2.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	3389

10.2.1.1	EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA	3389

10.2.2	UE timing advance	3396

10.2.2.0	Minimum conformance requirements	3396

10.2.2.0.1	Minimum conformance requirements for timing advance adjustment delay	3396

10.2.2.0.2	Minimum conformance requirements for timing advance adjustment accuracy	3396

10.2.2.1	EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	3396

10.3	Signalling characteristics	3402

10.3.1	Radio link monitoring	3402

10.3.1.0	General	3402

10.3.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring under CCA	3403

10.3.1.1	Requirements for determining UE in-sync or out-of-sync status	3406

10.3.1.2	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	3406

10.3.1.3	EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	3414

10.3.1.4	Void	3419

10.3.1.5	Void	3419

10.3.2	Void	3420

10.3.3	SCell activation and deactivation delay	3420

10.3.3.0	Minimum Conformance Requirements	3420

10.3.3.0.1	SCell Activation Delay Requirement for Deactivated SCell under CCA	3420

10.3.3.0.2	SCell Deactivation Delay Requirement for Activated SCell under CCA	3424

10.3.3.1	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	3424

10.3.3.2	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	3433

10.3.3.3	EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	3437

10.3.4	Beam failure detection and link recovery procedures	3442

10.3.4.0	General	3442

10.3.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	3443

10.3.4.0.3	Scheduling availability of UE during beam failure detection under CCA	3445

10.3.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	3445

10.3.4.1	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode	3445

10.3.4.2	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode	3453

10.3.5	Active BWP switching	3461

10.3.5.1	EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA	3461

10.3.5.2	DCI-based and Timer-based Active BWP Switch	3468

10.3.5.2.0	Minimum conformance requirements	3468

10.3.5.2.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	3471

10.3.5.2.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA	3479

10.3.5.3	RRC-based Active BWP Switch	3487

10.3.5.3.0	Minimum conformance requirements	3487

10.3.5.3.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	3488

10.3.6	PSCell addition and release delay	3494

10.3.6.0	Minimum conformance requirements	3494

10.3.6.0.1	NR PSCell Addition Delay Requirement	3494

10.3.6.0.2	NR PSCell Release Delay Requirement	3495

10.3.6.1	EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA	3495

10.4	Measurement procedures	3502

10.4.1	Intra-frequency measurements	3502

10.4.1.0	Minimum conformance requirements	3502

10.4.1.1	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA	3516

10.4.1.2	Void	3521

10.4.1.3	Void	3521

10.4.1.4	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA	3521

10.4.1.5	3527

10.4.1.6	3527

10.4.1.7	3527

10.4.1.8	3527

10.4.2	Inter-frequency measurements	3527

10.4.2.0	Minimum conformance requirements	3527

10.4.2.1	EN-DC FR1-FR1 RSSI measurement reporting under CCA	3533

10.4.2.2	EN-DC FR1-FR1 Channel occupancy measurement reporting under CCA	3534

10.4.2.3	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used	3536

10.4.2.4	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used	3542

10.4.2.5	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used	3549

10.4.2.6	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used	3555

10.4.2.7	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used	3562

10.4.2.8	EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used	3569

10.4.2.9	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used	3576

10.4.2.10	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used	3583

10.4.3	L1-RSRP measurement for beam reporting under CCA	3591

10.4.3.0	Minimum conformance requirements	3591

10.4.3.1	EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used	3593

10.4.3.2	EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used	3599

10.4.3.3	EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used	3603

10.4.3.4	EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used	3608

10.4.4	E-UTRAN-NR inter-RAT measurements on NR carrier frequency under CCA	3612

10.4.4.0	Minimum conformance requirements	3612

10.4.4.1	EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3615

10.4.4.2	EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3621

10.4.4.3	EN-DC FR1 E-UTRA-NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3629

10.4.4.4	EN-DC FR1 E-UTRA-NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3635

10.5	Measurement Performance	3643

10.5.1	SS-RSRP	3643

10.5.1.0	Minimum Conformance Requirements	3643

10.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3643

10.5.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	3643

10.5.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3644

10.5.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	3644

10.5.1.1	EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell	3645

10.5.1.2	EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell	3650

10.5.2	SS-RSRQ	3656

10.5.2.0	Minimum Conformance Requirements	3656

10.5.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3656

10.5.2.0.2	Void	3657

10.5.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3657

10.5.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3658

10.5.2.1	EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	3658

10.5.2.2	EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	3664

10.5.3	SS-SINR	3669

10.5.3.0	Minimum Conformance Requirements	3669

10.5.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	3669

10.5.3.0.2	Void	3670

10.5.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	3670

10.5.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	3671

10.5.3.1	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA	3671

10.5.3.2	EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA	3677

10.5.3.3	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA	3683

10.5.4	L1-RSRP measurement accuracy for beam reporting with CCA serving cell	3689

10.5.4.0	Minimum conformance requirements	3689

10.5.4.1	EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA	3691

10.5.5	RSSI	3695

10.5.5.0	Minimum Conformance Requirements	3695

10.5.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	3696

10.5.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	3696

10.5.5.1	EN-DC FR1 RSSI measurement accuracy on PSCC with CCA	3696

10.5.5.2	EN-DC FR1 RSSI measurement accuracy on SCC with CCA	3700

10.5.5.3	 EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA	3705

10.5.6	Channel occupancy	3710

10.5.6.0	Minimum Conformance Requirements	3710

10.5.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	3710

10.5.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	3710

10.5.6.1	EN-DC FR1 Channel occupancy measurement accuracy on PSCC with CCA	3711

10.5.6.2	EN-DC FR1 Channel occupancy measurement accuracy on SCC with CCA	3715

10.5.6.3	EN-DC FR1-FR1 Channel occupancy measurement accuracy on a carrier with CCA	3720

11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	3725

11.0	General	3725

11.0.1	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	3725

11.1	RRC_IDLE state mobility	3726

11.2	RRC_CONNECTED state mobility	3726

11.2.1	3726

11.2.2	RRC connection mobility control	3726

11.2.2.1	RRC re-establishment	3726

11.2.2.2	Random Access	3726

11.2.2.3	RRC connection release with redirection	3726

11.2.2.3.0	Minimum conformance requirements	3726

11.2.2.3.1	NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	3727

11.2.2.3.2	NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA	3733

11.3	Timing	3738

11.3.1	UE transmit timing	3738

11.3.1.0	Minimum conformance requirements	3738

11.3.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	3738

11.3.1.1	NR SA FR1 UE Transmit Timing Test with PCell under DL CCA	3739

11.3.2	UE timing advance	3745

11.3.2.0	Minimum conformance requirements	3745

11.3.2.0.1	Minimum conformance requirements for timing advance adjustment delay	3745

11.3.2.0.2	Minimum conformance requirements for timing advance adjustment accuracy	3745

11.3.2.1	NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA	3745

11.4	Signalling characteristics	3750

11.4.1	Radio link monitoring	3750

11.4.1.0	General	3750

11.4.1.0.1	Minimum conformance requirements for SSB based radio link monitoring under CCA	3750

11.4.1.1	Requirements for determining UE in-sync or out-of-sync status	3751

11.4.1.2	NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	3751

11.4.1.3	NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	3758

11.4.2	Void	3765

11.4.3	SCell activation and deactivation delay	3765

11.4.3.0	Minimum conformance requirements	3765

11.4.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	3765

11.4.3.1	NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	3765

11.4.3.2	NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	3774

11.4.3.3	NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	3778

11.4.4	Beam failure detection and link recovery procedures	3783

11.4.4.0	General	3783

11.4.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	3783

11.4.4.0.2	Void	3783

11.4.4.0.3	Scheduling availability of UE during beam failure detection under CCA	3784

11.4.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	3784

11.4.4.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA	3784

11.4.4.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA	3792

11.4.5	Active BWP switching	3800

11.4.5.1	NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	3800

11.4.5.2	DCI-based and Timer-based Active BWP Switch	3807

11.4.5.2.0	Minimum conformance requirements	3807

11.4.5.2.1	NR SA FR1 DCI-based DL active BWP switch of PCell with non-DRX under CCA	3809

11.4.5.2.2	NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA	3817

11.4.5.3	RRC-based Active BWP Switch	3825

11.4.5.3.0	Minimum conformance requirements	3825

11.4.5.3.1	NR SA FR1 RRC-based DL active BWP switch of Cell with non-DRX under CCA	3825

11.5	Measurement procedure	3831

11.5.1	Intra-frequency measurements	3831

11.5.1.0	Minimum Conformance Requirements	3831

11.5.1.1	SA FR1 Event-triggered reporting tests on PCC without gaps under non-DRX	3832

11.5.1.2	SA FR1 Event-triggered reporting tests on PCC without gaps under DRX	3836

11.5.1.3	SA FR1 Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	3841

11.5.1.4	SA FR1 Event-triggered reporting tests on PCC with per-UE gaps under DRX	3843

11.5.1.5 to 11.5.1.8	3844

11.5.1.9	SA FR1 RSSI measurement reporting on PCC	3845

11.5.2	Inter-frequency measurements	3846

11.5.2.0	Minimum Conformance Requirements	3846

11.5.2.1	3847

11.5.2.2	3847

11.5.2.3	NR SA FR1-FR1 Event triggered reporting tests with CCA without SSB time index detection when DRX is not used	3847

11.5.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	3852

11.5.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	3858

11.5.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	3863

11.5.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3868

11.5.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3873

11.5.2.9	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3878

11.5.2.10	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3883

11.5.3	Inter-RAT E-UTRAN measurements	3890

11.5.3.0	Minimum conformance requirements	3890

11.5.3.1	NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX	3890

11.5.3.2	NR SA FR1 - E-UTRAN event-triggered reporting in DRX	3895

11.5.4	L1-RSRP measurements for beam reporting	3900

11.5.4.0	Minimum conformance requirements	3900

11.5.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used	3900

11.5.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is used	3904

11.5.4.3	NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is not used	3908

11.5.4.4	NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is used	3912

11.6	Measurement Performance	3916

11.6.1	SS-RSRP	3916

11.6.1.0	Minimum Conformance Requirements	3916

11.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3916

11.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	3916

11.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3916

11.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	3916

11.6.1.1	NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA	3916

11.6.1.2	NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	3922

11.6.2	SS-RSRQ	3927

11.6.2.0	Minimum Conformance Requirements	3927

11.6.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3927

11.6.2.0.2	Intra-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3928

11.6.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3928

11.6.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3928

11.6.2.1	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	3928

11.6.2.2	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	3933

11.6.2.3	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA	3939

11.6.2.4	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA	3946

11.6.3	SS-SINR	3954

11.6.3.0	Minimum Conformance Requirements	3954

11.6.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	3954

11.6.3.0.2	Void	3954

11.6.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	3954

11.6.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	3954

11.6.3.1	NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA	3954

11.6.3.2	NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA	3960

11.6.3.3	NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA	3965

11.6.3.4	NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA from carrier without CCA	3971

11.6.4	L1-RSRP measurement accuracy for beam reporting with CCA serving cell	3981

11.6.4.0	Minimum conformance requirements	3981

11.6.4.0.1	Minimum conformance requitements for SSB-based L1-RSRP measurement  accuracy for beam reporting under CCA	3981

11.6.4.1	NR SA FR1 SSB based L1-RSRP measurement accuracy with CCA	3981

11.6.5	RSSI	3985

11.6.5.0	Minimum Conformance Requirements	3985

11.6.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	3985

11.6.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	3985

11.6.5.1	NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA	3986

11.6.5.2	NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA	3990

11.6.5.3	NR SA FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA	3994

11.6.6	Channel occupancy	3999

11.6.6.0	Minimum Conformance Requirements	3999

11.6.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	3999

11.6.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	3999

11.6.6.1	NR SA FR1 Intra-frequency channel occupancy measurement accuracy on PCC with CCA	3999

11.6.6.2	NR SA FR1 Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4003

11.6.6.3	NR SA FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4008

12 to 13	FFS	4013

14	NR standalone tests for Satellite access	4013

14.0	General	4013

14.0.1	Principle of testing GSO and NGSO scenarios	4013

14.0.2	Principle of testing different RRM requirements	4013

14.0.3	Principle of testing different ephemeris formats	4014

14.0.4	General setup for SIB19	4016

14.0.5	Initial test environment conditions for RRM NTN tests	4016

14.1	RRC_IDLE state mobility	4017

14.1.0	Minimum conformance requirements	4017

14.1.0.1	Measurements of intra-frequency NR cells	4017

14.1.0.2	Measurements of inter-frequency NR cells	4019

14.1.1	NR SA FR1 Cell Reselection for Satellite Access	4022

14.1.2	NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access	4022

14.1.3	NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access	4027

14.1.4	NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access	4031

14.1.5	NR SA FR1-FR1 Cell Reselection for Satellite Access	4035

14.1.6	NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access	4035

14.1.7	NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access	4035

14.1.8	NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access	4039

14.2	RRC_CONNECTED state mobility	4043

14.2.1	Handover for SAN	4043

14.2.1.0	Minimum conformance requirements	4043

14.2.1.0.1	Minimum conformance requirements for handover	4043

14.2.1.0.2	Minimum conformance requirements for conditional handover	4044

14.2.1.1	NR SA FR1 Handover for Satellite Access	4046

14.2.1.2	NR SA FR1-FR1 Handover for Satellite Access	4046

14.2.1.3	NR SA FR1 Time-based Conditional Handover for Satellite Access	4046

14.2.1.4	NR SA FR1-FR1 Time-based Conditional Handover for NR Satellite Access	4050

14.2.1.5	NR SA FR1 Distance-based Conditional Handover for Satellite Access	4053

14.2.1.6	NR SA FR1-FR1 Distance-based Conditional Handover for Satellite Access	4058

14.3	Timing for Satellite Access	4063

14.3.1	UE transmit timing for Satellite Access	4063

14.3.1.0	Minimum conformance requirements	4063

14.3.1.1	NR SA FR1 UE transmit timing accuracy for Satellite Access	4064

14.3.2	Timing Advance for Satellite Access	4071

14.3.2.0	Minimum conformance requirements	4071

14.3.2.1	NR SA FR1 Timing Advance adjustment accuracy for Satellite Access	4071

14.4	Signalling characteristics	4076

14.5	Measurement procedure	4076

14.5.1	Intra-frequency Measurements for SAN	4076

14.5.1.0	Minimum conformance requirements	4076

14.5.1.0.1	Minimum conformance requitements for intra-frequency measurements without measurement gaps	4076

14.5.1.0.2	Minimum conformance requitements for intra-frequency measurements with measurement gaps	4080

14.5.1.1	NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access	4081

14.5.1.2	NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access	4085

14.5.1.3	NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access	4089

14.5.1.4	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	4093

14.5.1.5	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access	4097

14.5.1.6	NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	4102

14.5.2	Inter-frequency Measurements for SAN	4106

14.5.2.0	Minimum conformance requirements	4106

14.5.2.1	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	4109

14.5.2.2	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access	4115

14.5.2.3	NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access	4121

14.5.2.4	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access	4127

14.5.2.5	Void	4133

14.5.2.6	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	4133

14.6	Measurement Performance Requirements	4139

14.6.1	4139

14.6.2	4139

14.6.3	SS-SINR Requirements for SAN	4139

14.6.3.0	Minimum conformance requirements	4139

14.6.3.0.1	Intra-Frequency SS-SINR accuracy requirements in FR1	4139

14.6.3.0.2	Inter-Frequency SS-SINR accuracy requirements in FR1	4140

14.6.3.1	NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access	4141

14.6.3.2	NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access	4146

14.6.4	L1-RSRP Beam Reporting Measurement Requirements for SAN	4151

14.6.4.0	Minimum conformance requirements	4151

14.6.4.0.1	Minimum conformance requirements for SSB based L1-RSRP Reporting	4151

14.6.4.0.2	Minimum conformance requirements for CSI-RS based L1-RSRP Reporting	4153

14.6.4.1	NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access	4154

14.6.4.2	NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access	4160

15	FFS	4165

16	NR standalone tests with all NR cells in FR1 for RedCap	4166

16.0	General	4166

16.1	RRC_IDLE state mobility	4166

16.1.1	NR cell re-selection	4166

16.1.1.0	Minimum conformance requirements	4166

16.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	4166

16.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	4168

16.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	4172

16.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	4175

16.1.1.1	NR SA FR1 Cell reselection for 1 Rx UE	4178

16.1.1.2	NR SA FR1 Cell reselection for 2 Rx UE	4186

16.1.1.3	NR SA FR1-FR1 Cell reselection for 1 Rx UE	4190

16.1.1.4	NR SA FR1-FR1 Cell reselection for 2 Rx UE	4195

16.1.1.5	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4199

16.1.1.6	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4207

16.1.1.7	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4212

16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4220

16.1.2	NR – E-UTRA cell re-selection	4228

16.1.2.0	Minimum conformance requirements	4228

16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx	4228

16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx	4230

16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection	4230

16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX	4232

16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX	4238

16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX	4242

16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX	4248

16.1.2.5	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	4252

16.1.2.6	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4258

16.2	RRC_INACTIVE state mobility for RedCap	4262

16.2.1	Configured Grant based Small Data Transmissions for RedCap	4262

16.2.1.0	Minimum conformance requirements	4262

16.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	4262

16.2.1.1	NR SA FR1 CG-SDT for 1Rx UE	4264

16.2.1.2	NR SA FR1 CG-SDT for 2Rx UE	4268

16.3	RRC_CONNECTED state mobility for RedCap	4271

16.3.1	Handover	4271

16.3.1.0	Minimum conformance requirements	4271

16.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	4271

16.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	4271

16.3.1.1	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE	4272

16.3.1.2	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE	4275

16.3.1.3	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	4279

16.3.1.4	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	4285

16.3.1.5	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	4290

16.3.1.6	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	4296

16.3.1.7	NR SA FR1 - E-UTRA handover for 1Rx UE	4303

16.3.1.8	NR SA FR1 - E-UTRA handover for 2Rx UE	4308

16.3.1.9	NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE	4313

16.3.1.10	NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE	4317

16.3.2	RRC connection mobility control for RedCap	4321

16.3.2.1	RRC re-establishment for RedCap	4321

16.3.2.1.0	Minimum conformance requirements	4321

16.3.2.1.1	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	4321

16.3.2.1.2	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	4324

16.3.2.1.3	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	4327

16.3.2.1.4	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	4330

16.3.2.1.5	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	4333

16.3.2.1.6	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	4336

16.3.2.2	Random access for RedCap	4339

16.3.2.2.0	Minimum conformance requirements	4339

16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	4340

16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	4343

16.3.2.2.3	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	4346

16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	4349

16.3.2.2.5	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	4352

16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	4355

16.3.2.2.7	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	4358

16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	4361

16.3.2.3	RRC connection release with redirection for RedCap	4364

16.3.2.3.0	Minimum conformance requirements	4364

16.3.2.3.1	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	4364

16.3.2.3.2	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	4368

16.3.2.3.3	NR SA FR1 - E-UTRA Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	4371

16.3.2.3.4	NR SA FR1 - E-UTRA Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	4376

16.4	Timing for RedCap	4381

16.4.1	UE transmit timing for RedCap	4381

16.4.1.0	Minimum conformance requirements	4381

16.4.1.1	NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	4382

16.4.1.2	NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	4386

16.4.2	4390

16.4.3	Timing advance for RedCap	4390

16.4.3.0	Minimum conformance requirement	4390

16.4.3.1	NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE	4390

16.4.3.2	NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE	4393

16.5	Signalling characteristics for RedCap	4396

16.5.1	Radio link monitoring for RedCap	4396

16.5.1.0	Minimum requirements	4396

16.5.1.0.1	General	4396

16.5.1.0.2	Minimum conformance requirements for SSB-based RLM	4400

16.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	4401

16.5.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	4402

16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	4406

16.5.1.3	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	4410

16.5.1.4	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	4415

16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	4419

16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	4424

16.5.1.7	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	4429

16.5.1.8	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	4434

16.5.1.9	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	4438

16.5.1.10	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	4441

16.5.1.11	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	4444

16.5.1.12	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	4447

16.5.1.13	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	4451

16.5.1.14	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	4454

16.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	4457

16.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	4460

16.5.2	Beam Failure Detection and Link recovery procedures for RedCap	4463

16.5.2.0	Minimum requirements	4463

16.5.2.0.1	General	4463

16.5.2.0.2	Minimum conformance requirements for SSB-based BFD and CBD	4465

16.5.2.0.3	Minimum conformance requirements for CSI-RS based RLM	4467

16.5.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	4469

16.5.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	4474

16.5.2.3	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	4479

16.5.2.4	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	4485

16.5.2.5	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	4490

16.5.2.6	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	4495

16.5.2.7	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	4498

16.5.2.8	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	4501

16.5.3	Active BWP switch delay for RedCap	4506

16.5.3.1	DCI-based and time-based active BWP switch for RedCap	4506

16.5.3.1.0	Minimum conformance requirements	4506

16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE	4507

16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE	4511

16.5.3.2	RRC-based active BWP switch for RedCap	4516

16.5.3.2.0	Minimum conformance requirements	4516

16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE	4517

16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE	4521

16.5.4	UE specific CBW change for RedCap	4526

16.5.4.0	Minimum conformance requirements	4526

16.5.4.0.1	Minimum conformance requirements for UE specific CBW change	4526

16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE	4527

16.5.4.2	NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE	4529

16.6	Measurement procedures for RedCap	4532

16.6.1	Intra-frequency measurements for RedCap	4533

16.6.1.0	Minimum conformance requirements	4533

16.6.1.0.1	General	4533

16.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	4534

16.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	4536

16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE	4537

16.6.1.2	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE	4543

16.6.1.3	NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE	4549

16.6.1.4	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE	4551

16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	4554

16.6.1.6	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	4557

16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	4560

16.6.1.8	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	4563

16.6.1.9	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	4566

16.6.1.10	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	4572

16.6.1.11	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	4577

16.6.1.12	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2Rx UE	4580

16.6.2	Inter-frequency measurements for RedCap	4583

16.6.2.0	Minimum conformance requirements	4583

16.6.2.0.1	Minimum conformance requirements for inter-frequency event-triggered measurement with measurement gaps	4583

16.6.2.0.2	Minimum conformance requirements for inter-frequency event-triggered measurements without measurement gaps	4585

16.6.2.1	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	4588

16.6.2.2	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	4593

16.6.2.3	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	4599

16.6.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	4604

16.6.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	4608

16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	4612

16.6.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	4616

16.6.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	4620

16.6.2.9	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	4625

16.6.2.10	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	4631

16.6.2.11	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	4636

16.6.2.12	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	4641

16.6.3	Inter-RAT measurements for RedCap	4646

16.6.3.0	Minimum conformance requirements	4646

16.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	4646

16.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	4649

16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE	4652

16.6.3.2	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE	4658

16.6.3.3	NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE	4663

16.6.3.4	NR - E-UTRA event-triggered reporting in DRX in FR1 for 2 Rx UE	4668

16.6.4	L1-RSRP measurement for beam reporting for RedCap	4673

16.6.4.0	Minimum conformance requirements	4673

16.6.4.0.1	General	4673

16.6.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	4678

16.6.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	4681

16.6.4.3	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	4684

16.6.4.4	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	4688

16.6.4.5	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	4691

16.6.4.6	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	4694

16.6.4.7	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	4696

16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	4699

16.6.5	NR measurements with autonomous gaps	4701

16.6.5.0	Minimum conformance requirements	4701

16.6.5.0.1	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	4701

16.6.5.0.2	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	4703

16.6.5.0.3	Minimum conformance requirements for CGI identification of an E-UTRA cell with autonomous gaps	4703

16.6.5.1	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	4704

16.6.5.2	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	4707

16.6.5.3	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	4710

16.6.5.4	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	4718

16.7	Measurement performance requirements for RedCap	4723

16.7.1	SS-RSRP for RedCap	4723

16.7.1.0	Minimum conformance requirements	4723

16.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	4723

16.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	4724

16.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	4725

16.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	4726

16.7.1.1	Intra-frequency measurements for RedCap	4727

16.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	4727

16.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	4733

16.7.1.2.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	4735

16.7.1.2.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	4742

16.7.1.2	4744

16.7.1.3	Inter-frequency measurements for RedCap for 1 Rx UE	4744

16.7.1.3.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	4744

16.7.1.3.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	4751

16.7.1.4	Inter-frequency measurements for RedCap for 2 Rx UE	4753

16.7.1.4.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	4753

16.7.1.4.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	4759

16.7.2	SS-RSRQ	4761

16.7.2.0	Minimum conformance requirements	4761

16.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	4761

16.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	4762

16.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	4762

16.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy for 1 Rx UE	4763

16.7.2.2	NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE	4767

16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4771

16.7.2.3.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4771

16.7.2.3.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4776

16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4778

16.7.2.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4778

16.7.2.4.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4782

16.7.3	SS-SINR	4784

16.7.3.0	Minimum conformance requirements	4784

16.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	4784

16.7.3.0.2	Inter-frequency SS-SINR absolute measurement accuracy requirements	4785

16.7.3.0.3	Inter-frequency SS-SINR relative measurement accuracy requirements	4785

16.7.3.1	NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4786

16.7.3.2	NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4790

16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4794

16.7.3.3.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4794

16.7.3.3.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4798

16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4800

16.7.3.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4800

16.7.3.4.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4804

16.7.4	L1-RSRP measurement for beam reporting	4806

16.7.4.0	Minimum conformance requirements	4806

16.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	4806

16.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	4806

16.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	4807

16.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	4807

16.7.4.1	SSB based L1-RSRP measurements for 1 Rx UE	4807

16.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 1 Rx UE	4807

16.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement for 1 Rx UE	4812

16.7.4.2	SSB based L1-RSRP measurements for 2 Rx UE	4814

16.7.4.2.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 2 Rx UE	4814

16.7.4.2.2	NR SA FR1 SSB based L1-RSRP relative measurement for 2 Rx UE	4818

16.7.4.3	CSI-RS based L1-RSRP measurements for 1 Rx UE	4820

16.7.4.3.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 1 Rx UE	4820

16.7.4.3.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 1 Rx UE	4824

16.7.4.4	CSI-RS based L1-RSRP measurements for 2 Rx UE	4826

16.7.4.4.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 2 Rx UE	4826

16.7.4.4.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE	4830

16.7.5	E-UTRAN RSRP	4832

16.7.5.0	Minimum conformance requirements	4832

16.7.5.0.1	E-UTRAN RSRP absolute accuracy	4832

16.7.5.1	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE	4833

16.7.5.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE	4837

16.7.6	E-UTRAN RSRQ	4841

16.7.6.0	Minimum conformance requirements	4841

16.7.6.0.1	E-UTRAN RSRQ absolute accuracy	4841

16.7.6.1	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE	4842

16.7.6.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE	4847

17	NR standalone tests with at least one NR cell in FR2 for RedCap	4852

17.1	RRC_IDLE state mobility	4852

17.1.1	NR cell re-selection	4852

17.1.1.0	Minimum conformance requirements	4852

17.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	4852

17.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	4854

17.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	4856

17.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	4858

17.1.1.1	NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx	4860

17.1.1.2	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx	4866

17.1.1.3	NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4872

17.1.1.4	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	4877

17.2	RRC_INACTIVE state mobility for RedCap	4884

17.2.1	Configured Grant based Small Data Transmissions for RedCap	4884

17.2.1.0	Minimum conformance requirements	4884

17.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	4884

17.2.1.1	NR SA FR2 TA validation for CG-SDT for 2 Rx	4885

17.3	RRC_CONNECTED state mobility for RedCap	4888

17.3.1	Handover for RedCap	4888

17.3.1.0	Minimum conformance requirements	4888

17.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	4888

17.3.1.1	NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	4890

17.3.1.2	NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	4892

17.3.2	RRC connection mobility control for RedCap	4899

17.3.2.1	RRC re-establishment for RedCap	4899

17.3.2.1.0	Minimum conformance requirements	4899

17.3.2.1.1	NR SA FR2 Intra-frequency RRC Re-establishment in FR2	4899

17.3.2.1.2	NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2	4901

17.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	4903

17.3.2.2	Random access for RedCap	4906

17.3.2.2.0	Minimum conformance requirements	4906

17.3.2.2.0.1	Minimum conformance requirements for Contention based random access	4906

17.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access	4906

17.3.2.2.1	NR SA FR2 4-step RA type contention based random access test in FR2 for NR Standalone	4907

17.3.2.2.2	NR SA FR2 4-step RA type non-contention based random access test in FR2 for NR Standalone	4910

17.3.2.2.3	NR SA FR2 2-step RA type contention based random access test in FR2 for NR Standalone	4913

17.3.2.2.4	NR SA FR2 2-step RA type non-contention based random access test in FR2 for NR Standalone	4916

17.3.2.3	RRC Connection Release with Redirection for RedCap	4919

17.3.2.3.0	Minimum conformance requirements	4919

17.3.2.3.0.1	Minimum conformance requirements for FR2-FR2 RRC connection release with redirection	4919

17.3.2.3.1	NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2	4919

17.4	Timing for RedCap	4921

17.4.1	UE transmit timing for RedCap	4921

17.4.1.0	Minimum Conformance Requirements	4922

17.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	4922

17.4.1.1	NR SA FR2 NR UE Transmit Timing Test for FR2	4923

17.4.2	UE timer accuracy for RedCap	4926

17.4.3	Timing advance for RedCap	4926

17.4.3.0	Minimum conformance requirements	4926

17.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	4926

17.5	Signalling characteristics for RedCap	4931

17.5.1	Radio link monitoring for RedCap	4931

17.5.1.0	Minimum requirements	4931

17.5.1.0.1	General	4931

17.5.1.0.2	Minimum conformance requirements for SSB-based RLM	4934

17.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	4936

17.5.1.1	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	4938

17.5.1.2	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	4941

17.5.1.3	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	4946

17.5.1.4	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	4949

17.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	4952

17.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	4955

17.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	4963

17.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	4967

17.5.2	Beam Failure Detection and Link recovery procedures for RedCap	4970

17.5.2.0	Minimum conformance requirements	4970

17.5.2.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	4970

17.5.2.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	4970

17.5.2.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	4970

17.5.2.1	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	4970

17.5.2.2	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	4975

17.5.2.3	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	4980

17.5.2.4	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	4984

17.5.2.5	NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	4988

17.5.3	Active BWP switch delay for RedCap	4992

17.5.3.1	DCI-based and time-based active BWP switch for RedCap	4992

17.5.3.1.0	Minimum conformance requirements for DCI-based and time-based active BWP switch	4992

17.5.3.1.1	NR SA FR2 DCI-based and Timer-based DL active BWP switch with non-DRX	4993

17.5.3.2	RRC-based active BWP switch for RedCap	5000

17.5.3.2.0	Minimum conformance requirements for RRC-based active BWP switch	5000

17.5.3.2.1	NR SA FR2 RRC-based DL active BWP switch with non-DRX	5000

17.5.4	Active TCI state switch delay for RedCap	5006

17.5.4.0	Minimum conformance requirements	5006

17.5.4.0.1	Known conditions for TCI state	5006

17.5.4.0.2	Minimum conformance requirements for MAC-CE based active TCI state switch	5006

17.5.4.0.3	Minimum conformance requirements for RRC based active TCI state switch	5006

17.5.4.1	MAC-CE based active TCI state switch for RedCap	5006

17.5.4.2	RRC based active TCI state switch for RedCap	5009

17.5.4.2.1	NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state	5009

17.5.5	Uplink spatial relation switch delay for RedCap	5012

17.5.5.1	MAC-CE based Spatial Relation switch for RedCap	5012

17.5.5.1.0	Minimum conformance requirements MAC-CE based uplink spatial relation switch delay	5012

17.5.5.1.1	NR SA FR2 PCell MAC-CE based spatial relation switch associated with known DL-RS	5013

17.5.5.2	RRC based spatial relation switch for RedCap	5016

17.5.5.2.0	Minimum conformance requirements RRC based uplink spatial relation switch delay	5016

17.5.5.2.1	NR SA FR2 PCell RRC-based spatial relation switch associated with a known DL-RS	5017

17.5.6	UE specific CBW change for RedCap	5020

17.5.6.0	Minimum conformance requirements	5020

17.5.6.0.1	Minimum conformance requirements for UE specific CBW change	5020

17.5.6.1	NR SA FR2 UE specific CBW change of PCell with non-DRX	5020

17.6	Measurement procedures for RedCap	5026

17.6.1	Intra-frequency measurements for RedCap	5026

17.6.1.0	Minimum conformance requirements	5026

17.6.1.0.1	General	5026

17.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	5027

17.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	5029

17.6.1.1	NR SA FR2 Event triggered reporting test without gap under non-DRX	5030

17.6.1.2	NR SA FR2 Event triggered reporting test without gap under DRX	5034

17.6.1.3	NR SA FR2 Event triggered reporting test with per-UE gaps under non-DRX	5035

17.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	5042

17.6.2	Inter-frequency measurements for RedCap	5045

17.6.2.0	Minimum conformance requirements	5045

17.6.2.1	NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used	5047

17.6.2.2	NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used	5051

17.6.2.3	NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used	5056

17.6.2.4	NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used	5061

17.6.3	L1-RSRP measurement for beam reporting for RedCap	5066

17.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	5066

17.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	5066

17.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	5066

17.6.3.1	NR SA FR2 SSB based L1-RSRP measurement when DRX is not used	5066

17.6.3.2	NR SA FR2 SSB based L1-RSRP measurement when DRX is used	5067

17.6.3.3	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is not used	5070

17.6.3.4	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used	5072

17.6.4	NR Measurements with autonomous gaps for RedCap	5075

17.6.4.0	Minimum conformance requirements	5075

17.6.4.0.1	Minimum conformance requirements for CGI identification of an NR cell with autonomous gaps	5075

17.6.4.1	NR SA FR2 Interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	5076

17.7	Measurement performance requirements for RedCap	5082

17.7.1	SS-RSRP for RedCap	5082

17.7.1.0	Minimum conformance requirements	5082

17.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	5082

17.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	5082

17.7.1.1	Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5082

17.7.1.2	Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5087

17.7.2	SS-RSRQ for RedCap	5094

17.7.2.0	Minimum conformance requirements	5094

17.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	5094

17.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	5094

17.7.2.1	Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5094

17.7.2.2	Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5097

17.7.3	L1-RSRP measurement for beam reporting for RedCap	5100

17.7.3.0	Minimum conformance requirements	5100

17.7.3.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	5100

17.7.3.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	5100

17.7.3.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	5100

17.7.3.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	5101

17.7.3.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	5101

17.7.3.2	NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off	5105

17.7.4	SS-SINR for RedCap	5109

17.7.4.0	Minimum conformance requirements	5109

17.7.4.0.1	Intra-frequency SS-SINR measurement accuracy requirements	5109

17.7.4.0.2	Inter-frequency SS-SINR measurement accuracy requirements	5109

17.7.4.1	Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5109

17.7.4.2	NR SA FR2-FR2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5112

18	E-UTRA - NR inter-RAT with E-UTRA serving cell for RedCap	5116

18.1	RRC_IDLE state mobility for RedCap	5116

18.1.1	Inter-rat NR cell re-selection for RedCap	5116

18.1.1.0	Minimum conformance requirements	5116

18.1.1.1	E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1	5116

18.2	RRC_CONNECTED state mobility for RedCap	5120

18.2.1	Handover for RedCap	5120

18.2.1.0	Minimum conformance requirements	5120

18.2.1.1	E-UTRA - NR FR1 handover  for 2Rx UE	5121

18.2.2.0	Minimum conformance requirements	5124

18.2.2.0.1	Redirection from E-UTRA to FR1 RedCap UE	5124

18.2.2.1	Redirection from E-UTRA to NR SA FR1 for redcap UE	5125

18.3	Measurement procedure for RedCap	5132

18.3.1	E-UTRA - NR Measurements for RedCap	5132

18.3.1.0	Minimum conformance requirements	5132

18.3.1.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	5132

18.3.1.1	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5135

18.3.1.2	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5139

18.3.1.3	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5143

18.3.1.4	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5147

18.3.1.5	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5151

18.3.1.6	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5153

18.3.1.7	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5156

18.3.1.8	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5158

Annex A (normative): RRM test configurations	5161

A.1	Reference measurement channels	5161

A.1.0	General	5161

A.1.1	PDSCH	5161

A.1.1.1	FDD	5161

A.1.1.2	TDD	5162

A.1.2	CORESET for RMSI scheduling	5165

A.1.2.1	FDD	5165

A.1.2.2	TDD	5166

A.1.3	CORESET for RMC scheduling	5169

A.1.3.1	FDD	5169

A.1.3.2	TDD	5170

A.1.4	CSI-RS	5172

A.1.4.1	FDD	5172

A.1.4.2	TDD	5175

A.1.4A	CSI-RS for tracking	5180

A.1.4A.1	FR1	5180

A.1.4A.1.1	FDD	5180

A.1.4A.1.2	TDD	5181

A.1.4A.2	FR2	5182

A.1.4A.2.1	TDD	5182

A.1.4B	CSI-IM configurations	5183

A.1.4B.1	FDD	5183

A.1.4B.2	TDD	5183

A.1.5	TDD UL/DL configuration	5185

A.1.6	PUSCH	5186

A.1A	Reference measurement channels under CCA	5187

A.1A.0	General	5187

A.1A.1	PDSCH	5187

A.1A.1.1	FDD	5187

A.1A.2	CORESET for RMSI scheduling	5188

A.1A.2.1	TDD	5188

A.1A.3	CORESET for RMC scheduling	5189

A.1A.3.1	TDD	5189

A.1A.4	TDD UL/DL configuration	5190

A.1A.4.1	TDD	5190

A.1A.5	RMC burst transmission model	5190

A.2	Reference OCGN configuration	5190

A.2.1	Generic OFDMA channel noise generator (OCNG)	5190

A.3	Reference SSB configuration	5193

A.3.1	SSB configuration for FR1	5193

A.3.2	SSB configuration for FR2	5193

A.3A	Reference SSB configuration under CCA	5194

A.3A.1	SSB configuration under CCA for FR1	5194

A.3A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	5194

A.3A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	5194

A.3A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	5195

A.3A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	5195

A.3B	Reference SSB configuration for RedCap	5196

A.3B.1	SSB configuration for FR1	5196

A.3B.2	SSB configuration for FR2	5197

A.4	Reference SMTC configuration	5197

A.4A	Reference SMTC configuration for RedCap	5197

A.5	Reference DRX configurations	5198

A.6	EN-DC test setup	5198

A.6.1	E-UTRA serving cell parameters	5198

A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1	5198

A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2	5200

A.6A	NR FR1-FR2 test setup	5202

A.6B	EN-DC test setup with unlicensed bands	5202

A.6B.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	5202

A.7	Reference PRACH configurations	5203

A.7.1	PRACH configurations for FR1	5203

A.7.1A	PRACH configurations for FR1 under CCA	5204

A.7.1A.1	FR1 PRACH configuration 1 under CCA	5204

A.7.1A.2	FR1 PRACH configuration 2 under CCA	5205

A.7.2	PRACH configurations for FR2	5206

A.7A	Reference MsgA configurations	5207

A.7A.1	MsgA configurations for FR1	5207

A.7A.1A	MsgA configurations for FR1 under CCA	5209

A.7A.1A.1	FR1 MsgA configuration 1 under CCA	5209

A.7A.1A.2	FR1 MsgA configuration 2 under CCA	5210

A.7A.2	MsgA configurations for FR2	5212

A.8	Reference BWP configurations	5214

A.8.1	Downlink BWP configurations	5214

A.8.2	Uplink BWP configurations	5214

A.8A	Reference BWP configurations for RedCap	5215

A.8A.1	Downlink BWP configurations	5215

A.8A.2	Uplink BWP configurations	5215

A.9	Angle of Arrival (AoA) for FR2 RRM test cases	5215

A.9.1	Setup 1: Single AoA in Rx beam peak direction	5216

A.9.2	Setup 2: Single AoA in non Rx beam peak direction	5216

A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	5216

A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	5216

A.9.3	Setup 3: 2 AoAs	5216

A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	5217

A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	5217

A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	5217

A.10	TCI State Configuration	5217

A.10.1	Introduction	5217

A.10.2	TCI states	5217

A.10A	Unified TCI State Configuration	5218

A.10A.1	Introduction	5218

A.10A.2	DLorJoint TCI states	5218

A.10A.3	UL TCI states	5218

A.11	NR sidelink communication	5219

A.11.1	Introduction	5219

A.11.2	Reference resource pool configurations for NR Sidelink Communication	5219

A.11.3	Reference measurement channels for NR Sidelink Communication	5222

A.12	Discovery Burst Transmission Window configuration under CCA	5223

A.12.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	5223

A.13	Channel bandwidth (CBW) configurations	5223

A.13.1	DL UE specific CBW	5223

A.13.2	UL UE specific CBW	5223

A.14	Satellite access configuration	5224

A.14.2	Satellite specific configuration for neighbour cell	5224

A.15	SRS Configurations	5225

A.15.1	SRS Configuration for SCS=15kHz	5225

A.15.2	SRS Configuration for SCS=30kHz	5226

A.15.3	SRS Configuration for SCS=30kHz	5227

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5227

B.1	Conditions for NR RRC_IDLE state mobility	5227

B.1.1	Introduction	5227

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5228

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5229

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5231

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5232

B.2	Conditions for NR RRC_CONNECTED state	5232

B.2.1	Introduction	5232

B.2.2	Conditions for NR intra-frequency measurements	5232

B.2.3	Conditions for NR inter-frequency measurements	5233

B.2.4	Conditions for NR L1-RSRP reporting	5234

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5234

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5235

B.2.5	Conditions for RRC connection release with redirection to NR	5236

B.2.6	Conditions for UE transmit timing	5237

B.2.6.1	Conditions for SSB based UE transmit timing	5237

B.2.7	Conditions for NR intra-frequency measurements for RedCap	5238

B.2.8	Conditions for NR inter-frequency measurements for RedCap	5240

B.2.9	Conditions for NR intra-frequency measurements under CCA	5242

B.2.10	Conditions for NR inter-frequency measurements under CCA	5243

B.2.11	Conditions for NR L1-RSRP reporting under CCA	5243

B.2.11.1	Conditions for SSB based L1-RSRP reporting	5243

B.2.12 to B.2.16	5243

B.2.17	Conditions for NR intra-frequency measurements for satellite access	5243

B.2.18	Conditions for NR inter-frequency measurements for satellite access	5244

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	5244

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	5244

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	5244

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	5244

B.3	RRM requirement exceptions	5245

B.3.1	Introduction	5245

B.3.2	Receiver sensitivity relaxation for CA	5245

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	5245

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	5245

B.3.2.2.1	Inter-band carrier aggregation	5245

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	5245

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	5245

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	5246

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	5246

B.3.2.4.1	Intra-band contiguous carrier aggregation	5246

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	5246

B.3.3	Receiver sensitivity relaxation for DC	5246

B.3.4	Receiver sensitivity relaxation for SUL	5246

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	5246

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	5246

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	5246

B.4	Conditions for NR sidelink	5247

B.4.1	Test parameters for GNSS signals	5247

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	5247

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	5247

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	5248

B.5	High level test procedure for SAN RRM tests	5248

Annex C (normative): Downlink physical channels and propagation conditions	5249

C.1	Downlink physical channels	5249

C.1.1	General	5249

C.1.2	Default downlink signal levels	5249

C.1.3	Default connection setup	5250

C.2	Propagation conditions	5250

C.2.0	General	5250

C.2.1	No interference	5251

C.2.2	Static propagation conditions	5251

C.2.2.0	General	5251

C.2.2.1	UE receiver with 2Rx antenna connectors	5251

C.2.2.2	UE receiver with 4Rx antenna connectors	5251

C.2.3	Multi-path fading propagation conditions	5252

Annex D (normative): Deviations from standard test configuration	5253

D.1	Test cases with different numerologies	5253

D.2	EN-DC test cases with different EN-DC configurations	5253

D.2.0	General	5253

D.2.1	Principle of testing	5253

D.3	Carrier aggregation test cases with different CA configurations	5253

D.3.0	General	5253

D.3.1	Principle of testing	5253

D.4	Antenna connection for 4Rx capable UEs	5253

D.4.0	General	5253

D.4.1	Principle of testing	5254

D.4.1.1	Single carrier tests	5254

D.4.1.2	Carrier aggregation tests	5255

D.4.1.3	EN-DC tests	5255

D.4.2	Antenna connection	5255

D.4.2.1	Antenna connection for NR bands where 2Rx is supported	5255

D.4.2.2	Antenna connection for NR bands where only 4Rx is supported	5255

D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported	5255

D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported	5256

D.4A	Antenna configurations with unlicensed bands for 4Rx capable UEs	5256

D.4A.1	Introduction	5256

D.4A.2	Principle of Testing	5256

D.4A.2.1	Single Carrier Tests	5256

D.4A.2.2	Carrier Aggregation Tests	5257

D.4A.2.3	EN-DC Tests	5257

D.4A.2.4	Antenna connection for bands where 2RX is supported	5257

D.4A.2.5	Antenna connection for bands where 4RX is supported	5257

D.4A.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported	5257

D.4A.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported	5257

D.5	Test Cases with Different Channel Bandwidths	5258

D.5.1	Test Cases with Different E-UTRA Channel Bandwidths	5258

D.5.1.1	Introduction	5258

D.5.1.2	Principle of testing	5258

D.6	Test Cases for Synchronous and Asynchronous DC Operations	5258

D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	5258

D.6.1.1	Introduction	5258

D.6.1.2	Principle of Testing	5258

D.7	CCA model	5258

D.7.1	Introduction	5258

D.7.2	CCA model for operation on a carrier frequency with CCA in FR1	5258

D.7.2.1	DL CCA model	5258

D.7.2.2	UL CCA model	5260

D.7.3	Test Cases with at Least One Cell on a Carrier Frequency with CCA	5260

D.8	Test Cases in SA and EN-DC Operations	5260

D.8.0	General	5260

D.8.1	Principle of testing	5261

Annex E (normative): Cell configuration mapping	5262

E.0	General	5262

E.1	Test frequency selection	5262

E.1.0	General	5262

E.1.1	E-UTRA PCell for EN-DC test cases	5262

E.1.2	Test cases with one NR cell	5262

E.1.3	Test cases with more than one NR cell	5262

E.1.3.1	Intra-frequency test cases	5262

E.1.3.2	Inter-frequency test cases	5262

E.1.4	Carrier aggregation test cases	5263

E.1.4.1	Inter-band carrier aggregation	5263

E.1.4.2	Intra-band contiguous carrier aggregation	5263

E.1.4.3	Intra-band non-contiguous carrier aggregation	5263

E.1.5	E-UTRA – NR inter RAT test cases	5263

E.1.6	Intra-band EN-DC test cases	5263

E.1.6.1	Intra-band non-contiguous EN-DC	5263

E.1.6.1.1	Inter frequency neighbour cell	5263

E.1.6.2	Intra-band contiguous EN-DC	5263

E.1.6.1.1	E-UTRA PCell	5263

E.1.7	NR sidelink test cases	5264

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4	5264

E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5	5267

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6	5271

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7	5276

E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8	5280

E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9	5282

E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10	5282

E.9	Cell configuration mapping for SA FR1 Shared Spectrum test cases in Chapter 11	5285

E.10 to E.13	5286

E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16	5286

E.15	Cell configuration mapping for SA FR2 test cases for RedCap in Chapter 17	5293

E.16	Cell configuration mapping for E-UTRAN – SA test cases for RedCap in Chapter 18	5295

Annex F (normative): Measurement uncertainties and test tolerances	5297

F.1	Measurement uncertainties and test tolerances for FR1 and FR2	5297

F.1.1	Acceptable uncertainty of test system (normative)	5297

F.1.1.1	Measurement of test environments	5297

F.1.1.2	Measurement of RRM requirements	5297

F.1.2	Interpretation of measurement results (normative)	5366

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)	5366

F.1.3.1	Measurement of test environments	5366

F.1.3.2	Measurement of RRM requirements	5366

Annex G (normative): Statistical testing	5516

G.1	General	5516

G.2	Statistical testing of delay and UE measurement performance in RRM tests	5516

G.2.1	General	5516

G.2.2	Design of the test	5516

G.2.3	Numerical definition of the pass fail limits	5516

G.2.4	Pass fail decision rules	5517

G.2.5	Void	5518

G.2.6	Test conditions for delay tests and UE measurement performance	5518

G.2.7	Additional considerations for tests in bands subject to CCA failure	5518

G.3	Statistical testing of NR sidelink CBR measurement tests	5519

G.3.1	General	5519

G.3.2	Design of the test	5519

G.3.3	Numerical definition of the pass fail limits	5520

G.3.4	Pass fail decision rules	5520

G.4	Theory to derive the numbers in Table G.2.3-1 (informative)	5521

G.5	Theory to derive the numbers in Table G.3.3-1 (informative)	5521

G.5.1	Algorithm to calculate pass and fail probabilities	5521

G.5.2	Method of designing pass/fail limit to approach given risk probability	5524

Annex H (normative): Default message contents for RRM	5529

H.1	Void	5529

H.2	System information blocks message content exceptions	5529

H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection	5529

H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection	5531

H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection	5532

H.3	RRC message content exceptions	5533

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration	5533

H.3.2	RRC messages and information elements contents exceptions for handover	5556

H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover	5557

H.3.4	E-UTRA RRC messages and information elements contents exceptions	5558

H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting	5569

H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting	5572

H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used	5578

H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay	5581

H.3.9	RRC messages and information elements contents exceptions for UL timing	5582

H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access	5582

Annex I (normative): RRM OTA procedures	5584

I.0	Test applicability per permitted test method	5584

I.1	Direct far field (DFF)	5584

I.1.1	RX beam peak direction search	5584

I.1.2	Search for directions corresponding to the EIS spherical coverage percentile	5584

I.2	Direct far field (DFF) simplification	5584

I.2.1	RX beam peak direction search	5584

I.2.2	Search for directions corresponding to the EIS spherical coverage percentile	5585

I.3	Indirect far field (IFF)	5585

I.3.1	RX beam peak direction search	5585

I.3.2	Search for directions corresponding to the EIS spherical coverage percentile	5585

I.3A	Enhanced indirect far field (Enhanced IFF)	5585

I.3A.1	RX beam peak direction search	5585

I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile	5585

I.4	Procedures to search test directions for RRM FR2	5585

I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search	5585

I.5	Re-Positioning Approach for RRM FR2	5586

I.5.1	1 AoA Test Cases	5587

I.5.2	2 AoA Test Cases	5588

Annex J (informative): Change history	5590