3GPP TS 24.543 V19.3.0 (2025-09)

Technical Specification

3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Data Delivery Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; (Release 19)

| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |
| --- | --- |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	12

1	Scope	14

2	References	14

3	Definitions of terms, symbols and abbreviations	15

3.1	Terms	15

3.2	Abbreviations	16

4	General description	16

5	Edge applications over 3GPP services	16

6	Functional entities	16

6.1	SEAL data delivery management client (SDDM-C)	16

6.2	SEAL data delivery management server (SDDM-S)	17

7	Data delivery management procedures	18

7.1	General	18

7.2	On-network procedures	18

7.2.1	General	18

7.2.1.1	Authenticated identity in HTTP request	18

7.2.1.2	Authenticated identity in CoAP request	18

7.2.2	SEALDD enabled signalling transmission connection establishment procedure	18

7.2.2.1	SDDM client HTTP procedure	18

7.2.2.2	SDDM server HTTP procedure	21

7.2.2.3	SDDM client CoAP procedure	22

7.2.2.4	SDDM server CoAP procedure	24

7.2.3	SEALDD enabled signalling transmission connection release procedure	25

7.2.3.1	SDDM client HTTP procedure	25

7.2.3.2	SDDM server HTTP procedure	26

7.2.3.3	SDDM client CoAP procedure	27

7.2.3.4	SDDM server CoAP procedure	28

7.2.4	SEALDD enabled E2E redundant transmission path establishment procedure	28

7.2.4.1	SDDM client HTTP procedure	28

7.2.4.2	SDDM server HTTP procedure	29

7.2.4.3	SDDM client CoAP procedure	30

7.2.4.4	SDDM server CoAP procedure	31

7.2.5	SEALDD enabled E2E redundant transmission path release procedure	32

7.2.5.1	SDDM client HTTP procedure	32

7.2.5.2	SDDM server HTTP procedure	32

7.2.5.3	SDDM client CoAP procedure	33

7.2.5.4	SDDM server CoAP procedure	33

7.2.6	SEALDD enabled E2E redundant transmission path connection update procedure	33

7.2.6.1	SDDM client HTTP procedure	33

7.2.6.2	SDDM server HTTP procedure	34

7.2.6.3	SDDM client CoAP procedure	35

7.2.6.4	SDDM server CoAP procedure	35

7.2.7	SEALDD server discovery and selection procedure	36

7.2.8	SEALDD enabled data storage creation procedure	36

7.2.8.1	SDDM client HTTP procedure	36

7.2.8.2	SDDM server HTTP procedure	36

7.2.8.3	SDDM client CoAP procedure	37

7.2.8.4	SDDM server CoAP procedure	37

7.2.9	SEALDD enabled data storage reservation procedure	38

7.2.9.1	SDDM client HTTP procedure	38

7.2.9.2	SDDM server HTTP procedure	38

7.2.9.3	SDDM client CoAP procedure	39

7.2.9.4	SDDM server CoAP procedure	39

7.2.10	SEALDD enabled data storage notification procedure	40

7.2.10.1	SDDM client HTTP procedure	40

7.2.10.2	SDDM server HTTP procedure	40

7.2.10.3	SDDM client CoAP procedure	40

7.2.10.4	SDDM server CoAP procedure	41

7.2.11	SEALDD enabled data storage query procedure	42

7.2.11.1	SDDM client HTTP procedure	42

7.2.11.2	SDDM server HTTP procedure	42

7.2.11.3	SDDM client CoAP procedure	43

7.2.11.4	SDDM server CoAP procedure	43

7.2.12	SEALDD enabled data storage management procedure	43

7.2.12.1	SDDM client HTTP procedure	43

7.2.12.2	SDDM server HTTP procedure	44

7.2.12.3	SDDM client CoAP procedure	44

7.2.12.4	SDDM server CoAP procedure	45

7.2.13	SEALDD server relocation procedure	46

7.2.14	SEALDD enabled data transmission quality measurement subscription procedure	46

7.2.14.1	SDDM client HTTP procedure	46

7.2.14.2	SDDM server HTTP procedure	47

7.2.14.3	SDDM client CoAP procedure	48

7.2.14.4	SDDM server CoAP procedure	48

7.2.15	SEALDD enabled data transmission quality measurement notification procedure	49

7.2.15.1	SDDM client HTTP procedure	49

7.2.15.2	SDDM server HTTP procedure	50

7.2.15.3	SDDM client CoAP procedure	50

7.2.15.4	SDDM server CoAP procedure	51

7.2.16	SEALDD enabled data transmission quality guarantee procedure	52

7.2.16.1	SDDM client HTTP procedure	52

7.2.16.2	SDDM server HTTP procedure	52

7.2.16.3	SDDM client CoAP procedure	53

7.2.16.4	SDDM server CoAP procedure	53

7.2.17	SEALDD enabled URLLC transmission connection deletion based on policy procedure	54

7.2.17.1	SDDM client HTTP procedure	54

7.2.17.2	SDDM server HTTP procedure	54

7.2.17.3	SDDM client CoAP procedure	55

7.2.17.4	SDDM server CoAP procedure	55

7.2.18	SEALDD enabled URLLC transmission connection establishment based on policy procedure	56

7.2.18.1	SDDM client HTTP procedure	56

7.2.18.2	SDDM server HTTP procedure	57

7.2.18.3	SDDM client CoAP procedure	57

7.2.18.4	SDDM server CoAP procedure	58

7.2.19	SEALDD enabled connection status reporting configuration subscription procedure	59

7.2.19.1	SDDM client HTTP procedure	59

7.2.19.2	SDDM server HTTP procedure	59

7.2.19.3	SDDM client CoAP procedure	60

7.2.19.4	SDDM server CoAP procedure	60

7.2.20	Void	61

7.2.21	SEALDD enabled connection status reporting configuration notification procedure	61

7.2.21.1	SDDM client HTTP procedure	61

7.2.21.2	SDDM server HTTP procedure	61

7.2.21.3	SDDM client CoAP procedure	62

7.2.21.4	SDDM server CoAP procedure	62

7.2.22	SEALDD enabled XR data transmission trigger procedure	63

7.2.22.1	SDDM client HTTP procedure	63

7.2.22.2	SDDM server HTTP procedure	63

7.2.22.3	SDDM client CoAP procedure	64

7.2.22.4	SDDM server CoAP procedure	64

7.2.23	SEALDD enabled XR data transmission inform procedure	65

7.2.23.1	SDDM client HTTP procedure	65

7.2.23.2	SDDM server HTTP procedure	65

7.2.23.3	SDDM client CoAP procedure	66

7.2.23.4	SDDM server CoAP procedure	66

7.2.24	SEALDD enabled multi-modal data transmission establishment procedure	67

7.2.24.1	SDDM client HTTP procedure	67

7.2.24.2	SDDM server HTTP procedure	67

7.2.24.3	SDDM client CoAP procedure	68

7.2.24.4	SDDM server CoAP procedure	69

7.3	Off-network procedures	70

8	Coding	71

8.1	General	71

8.2	Application unique ID	71

8.3	Structure	71

8.4	XML schema	79

8.4.1	General	79

8.4.2	XML schema	79

8.5	Data semantics	93

8.6	MIME type	105

8.7	IANA registration template	105

Annex A (normative):	CoAP resource representation and encoding	106

A.1	General	106

A.2	Data types applicable to multiple resource representations	107

A.2.1	General	107

A.2.2	Referenced structured data types	107

A.2.3	Referenced simple data types	107

A.2.4	Common structured data types	108

A.2.4.1	Type: EstablishmentResponse	108

A.2.4.2	Type: EstablishmentRequest	109

A.2.4.3	Type: URLLCEstablishmentRequest	110

A.2.4.4	Type: URLLCEstablishmentResponse	111

A.2.4.5	Type: URLLCReleaseRequest	111

A.2.4.6	Type: TransmissionAssistInfo	112

A.2.4.7	Type: PeriodicityRange	112

A.2.4.8	Type: TimeWindow	112

A.2.4.9	Type: XrAppDevCapability	113

A.2.4.10	Type: Non3gppAccessMeasurement	113

A.2.4.11	Type: MeasuredNon3gppAccess	113

A.2.4.12	Type: L4sFeedbackCapability	113

A.2.4.13	Type: EcnMarking	114

A.2.4.14	Type: L4sFeedback	114

A.2.5	Common simple data types	114

A.2.6	Common enumerations	114

A.2.6.1	Enumeration: RequestorId	114

A.2.6.2	Enumeration: ResultOp	115

A.2.6.3	Enumeration: Cause	115

A.2.6.4	Enumeration: Non3gppAccessPolicy	115

A.3	Resource representation and APIs provided by SDDM-S	115

A.3.1	Sdd_RegularTransmissionConnection API	115

A.3.1.1	API URI	115

A.3.1.2	Resources	116

A.3.1.2.1	Overview	116

A.3.1.2.2	Resource: SDD Regular Transmission Connection	116

A.3.1.2.2.1	Description	116

A.3.1.2.2.2	Resource Definition	116

A.3.1.2.2.3	Resource Standard Methods	117

A.3.1.3	Data Model	118

A.3.1.3.1	General	118

A.3.1.3.2	Structured data types	119

A.3.1.3.2.1	Void	119

A.3.1.3.2.2	Type: ReleaseRequest	119

A.3.1.3.2.3	Void	119

A.3.1.3.2.4	Void	119

A.3.1.3.2.5	Void	119

A.3.1.3.3	Simple data types and enumerations	119

A.3.1.4	Error Handling	119

A.3.1.5	CDDL Specification	119

A.3.1.5.1	Introduction	119

A.3.1.5.2	CDDL document	119

A.3.1.6	Media Types	121

A.3.1.7	Void	121

A.3.1.8	Void	121

A.3.1.9	Void	121

A.3.1.10	Void	121

A.3.1.11	Void	121

A.3.2	Sdd_TransmissionQualityMeasurement API	121

A.3.2.1	API URI	121

A.3.2.2	Resources	122

A.3.2.2.1	Overview	122

A.3.2.2.2	Resource: SDD Transmission Quality Measurement	122

A.3.2.2.2.1	Description	122

A.3.2.2.2.2	Resource Definition	123

A.3.2.2.2.3	Resource Standard Methods	123

A.3.2.2.2.3.1	POST	123

A.3.2.2.2.3.2	FETCH	123

A.3.2.2.2.3.3	DELETE	124

A.3.2.3	Data Model	124

A.3.2.3.1	General	124

A.3.2.3.2	Structured data types	126

A.3.2.3.2.1	Type: MeasurementsSubscriptionRequest	126

A.3.2.3.2.2	Type: MeasurementsSubscriptionResponse	126

A.3.2.3.2.3	Type: MeasurementNotification	127

A.3.2.3.2.4	Type: ReportingCriteria	128

A.3.2.3.2.5	Type: LatencyValue	128

A.3.2.3.2.6	Type: BitrateValue	128

A.3.2.3.2.7	Type: MeasurementConditions	129

A.3.2.3.2.8	Type: MeasurementPeriod	129

A.3.2.3.2.9	Type: SpatialConditions	129

A.3.2.3.3	Simple data types and enumerations	129

A.3.2.4	Error Handling	129

A.3.2.5	CDDL Specification	129

A.3.2.5.1	Introduction	129

A.3.2.5.2	CDDL document	129

A.3.2.6	Media Types	133

A.3.2.7	Void	133

A.3.2.8	Void	133

A.3.2.9	Void	133

A.3.3	Sdd_TransmissionQualityManagement API	133

A.3.3.1	API URI	133

A.3.3.2	Resources	134

A.3.3.2.1	Overview	134

A.3.3.2.2	Resource: SDD Transmission Quality Management	134

A.3.3.2.2.1	Description	134

A.3.3.2.2.2	Resource Definition	134

A.3.3.2.2.3	Resource Standard Methods	135

A.3.3.2.2.3.1	POST	135

A.3.3.2.2.3.2	DELETE	135

A.3.3.3	Data Model	136

A.3.3.3.1	General	136

A.3.3.3.2	Structured data types	137

A.3.3.3.2.1	Type: TxQualityManagementRequest	137

A.3.3.3.2.2	Type: TxQualityManagementResponse	137

A.3.3.3.3	Simple data types and enumerations	137

A.3.3.4	Error Handling	137

A.3.3.5	CDDL Specification	137

A.3.3.5.1	Introduction	137

A.3.3.5.2	CDDL document	138

A.3.3.6	Media Types	138

A.3.3.7	Void	138

A.3.3.8	Void	138

A.3.4	Sdd_ConnectionStatusEvent API	138

A.3.4.1	API URI	138

A.3.4.2	Resources	139

A.3.4.2.1	Overview	139

A.3.4.2.2	Resource: SDD Connection Status Event	139

A.3.4.2.2.1	Description	139

A.3.4.2.2.2	Resource Definition	139

A.3.4.2.2.3	Resource Standard Methods	140

A.3.4.2.2.3.1	POST	140

A.3.4.2.2.3.2	DELETE	140

A.3.4.2.2.3.3	FETCH	140

A.3.4.3	Data Model	141

A.3.4.3.1	General	141

A.3.4.3.2	Structured data types	142

A.3.4.3.2.1	Type: ConnectionStatusConfigurationRequest	142

A.3.4.3.2.2	Type: ConnectionStatusConfigurationResponse	142

A.3.4.3.2.3	Type: ConnectionStatusConfigurationSubscription	142

A.3.4.3.2.4	Type: ConnectionStatusNotification	143

A.3.4.3.3	Simple data types and enumerations	143

A.3.4.3.3.1	Simple data types	143

A.3.4.3.3.2	Enumeration: ConnectionStatus	143

A.3.4.3.3.3	Enumeration: ReportingMode	143

A.3.4.3.3.4	Enumeration: AccessUsage	143

A.3.4.3.3.5	Enumeration: ReportingPriority	144

A.3.4.4	Error Handling	144

A.3.4.5	CDDL Specification	144

A.3.4.5.1	Introduction	144

A.3.4.5.2	CDDL document	144

A.3.4.6	Media Types	145

A.3.4.7	Void	145

A.3.4.8	Void	145

A.3.5	Sdd_URLLCTransmissionConnection API	145

A.3.5.1	API URI	145

A.3.5.2	Resources	146

A.3.5.2.1	Overview	146

A.3.5.2.2	Resource: URLLC Transmission Connection	146

A.3.5.2.2.1	Description	146

A.3.5.2.2.2	Resource Definition	146

A.3.5.2.2.3	Resource Standard Methods	147

A.3.5.3	Data Model	147

A.3.5.3.1	General	147

A.3.5.3.2	Structured data types	148

A.3.5.3.3	Simple data types and enumerations	148

A.3.5.4	Error Handling	148

A.3.5.5	CDDL Specification	149

A.3.5.5.1	Introduction	149

A.3.5.5.2	CDDL document	149

A.3.5.6	Media Types	150

A.3.6	Sdd_XRTransmissionConnnection API	150

A.3.6.1	API URI	150

A.3.6.2	Resources	151

A.3.6.2.1	Overview	151

A.3.6.2.2	Resource: SDD XR transmission connection	151

A.3.6.2.2.1	Description	151

A.3.6.2.2.2	Resource Definition	151

A.3.6.2.2.3	Resource Standard Methods	152

A.3.6.2.2.3.1	POST	152

A.3.6.3	Data Model	152

A.3.6.3.1	General	152

A.3.6.3.2	Structured data types	153

A.3.6.3.2.1	Type: XRTriggerRequest	153

A.3.6.3.2.2	Type: XRTriggerResponse	153

A.3.6.3.3	Simple data types and enumerations	153

A.3.6.3.3.1	Simple data types	153

A.3.6.3.3.2	Enumeration: Operation	154

A.3.6.4	Error Handling	154

A.3.6.5	CDDL Specification	154

A.3.6.5.1	Introduction	154

A.3.6.5.2	CDDL document	154

A.3.6.6	Media Types	155

A.4	Resource representation and APIs provided by SDDM-C	155

A.4.1	Sdd_RegularTransmissionConnection API	155

A.4.1.1	API URI	155

A.4.1.2	Resources	156

A.4.1.2.1	Overview	156

A.4.1.2.2	Resource: SDD Regular Transmission Connection	156

A.4.1.2.2.1	Description	156

A.4.1.2.2.2	Resource Definition	157

A.4.1.2.2.3	Resource Standard Methods	157

A.4.1.3	Data Model	158

A.4.1.3.1	General	158

A.4.1.3.2	Structured data types	159

A.4.1.3.2.1	Void	159

A.4.1.3.2.2	Type: ReleaseRequest	159

A.4.1.3.3	Simple data types and enumerations	159

A.4.1.4	Error Handling	159

A.4.1.5	CDDL Specification	159

A.4.1.5.1	Introduction	159

A.4.1.5.2	CDDL document	159

A.4.1.6	Media Types	160

A.4.2	Sdd_URLLCTransmissionConnection API	160

A.4.2.1	API URI	160

A.4.2.2	Resources	161

A.4.2.2.1	Overview	161

A.4.2.2.2	Resource: URLLC Transmission Connection	162

A.4.2.2.2.1	Description	162

A.4.2.2.2.2	Resource Definition	162

A.4.2.2.2.3	Resource Standard Methods	162

A.4.2.3	Data Model	163

A.4.2.3.1	General	163

A.4.2.3.2	Structured data types	165

A.4.2.3.2.1	Void	165

A.4.2.3.2.2	Void	165

A.4.2.3.2.3	Type: URLLCUpdateRequest	165

A.4.2.3.2.4	Void	165

A.4.2.3.2.5	Type: URLLCUpdateResponse	165

A.4.2.3.3	Simple data types and enumerations	165

A.4.2.4	Error Handling	165

A.4.2.5	CDDL Specification	165

A.4.2.5.1	Introduction	165

A.4.2.5.2	CDDL document	166

A.4.2.6	Media Types	167

A.4.2.7	Void	167

A.4.2.8	Void	167

A.4.2.9	Void	167

A.4.2.10	Void	167

A.4.2.11	Void	167

A.4.3	Sdd_DataStorage API	167

A.4.3.1	API URI	167

A.4.3.2	Resources	168

A.4.3.2.1	Overview	168

A.4.3.2.2	Resource: SDD Data Storage	168

A.4.3.2.2.1	Description	168

A.4.3.2.2.2	Resource Definition	169

A.4.3.2.2.3	Resource Standard Methods	169

A.4.3.2.2.3.1	POST	169

A.4.3.2.2.3.2	PUT	169

A.4.3.2.2.3.3	DELETE	170

A.4.3.2.2.3.4	GET	170

A.4.3.2.2.3.5	FETCH	170

A.4.3.3	Data Model	171

A.4.3.3.1	General	171

A.4.3.3.2	Structured data types	173

A.4.3.3.2.1	Type: DataStorageCreationRequest	173

A.4.3.3.2.2	Type: DataStorageCreationResponse	173

A.4.3.3.2.3	Type: DataStorageReservationRequest	173

A.4.3.3.2.4	Type: DataStorageReservationResponse	173

A.4.3.3.2.5	Type: DataStorageStatusNotification	174

A.4.3.3.2.6	Type: DataStorageQueryResponse	174

A.4.3.3.2.7	Type: DataStorageMgtRequest	174

A.4.3.3.2.8	Type: StatusInformationReq	174

A.4.3.3.2.9	Type: StatusInformationRes	175

A.4.3.3.3	Simple data types and enumerations	175

A.4.3.4	Error Handling	175

A.4.3.5	CDDL Specification	175

A.4.3.5.1	Introduction	175

A.4.3.5.2	CDDL document	175

A.4.3.6	Media Types	176

A.4.3.7	Void	177

A.4.3.8	Void	177

A.4.3.9	Void	177

A.4.3.10	Void	177

A.4.3.11	Void	177

A.4.3.12	Void	177

A.4.3.13	Void	177

A.4.4	Sdd_XRTransmissionConnnection API	177

A.4.4.1	API URI	177

A.4.4.2	Resources	178

A.4.4.2.1	Overview	178

A.4.4.2.2	Resource: SDD XR transmission connection inform	178

A.4.4.2.2.1	Description	178

A.4.4.2.2.2	Resource Definition	178

A.4.4.2.2.3	Resource Standard Methods	179

A.4.4.2.2.3.1	POST	179

A.4.4.2.3	Resource: SDD XR transmission connection establish	179

A.4.4.2.3.1	Description	179

A.4.4.2.3.2	Resource Definition	179

A.4.4.2.3.3	Resource Standard Methods	179

A.4.4.2.3.3.1	POST	179

A.4.4.3	Data Model	180

A.4.4.3.1	General	180

A.4.4.3.2	Structured data types	181

A.4.4.3.2.1	Type: XRInformRequest	181

A.4.4.3.2.2	Type: XRInformResponse	181

A.4.4.3.2.3	Type: XREstablishmentRequest	181

A.4.4.3.2.4	Type: XREstablishmentResponse	182

A.4.4.3.2.5	Type: SealddFlowsInfo	182

A.4.4.3.2.6	Type: SealddMultiModalFlowInfo	182

A.4.4.3.2.7	Type: ProtocolDescriptorInfo	183

A.4.4.3.3	Simple data types and enumerations	183

A.4.4.3.3.1	Simple data types	183

A.4.4.3.3.2	Enumeration: Status	183

A.4.4.4	Error Handling	183

A.4.4.5	CDDL Specification	183

A.4.4.5.1	Introduction	183

A.4.4.5.2	CDDL document	184

A.4.4.6	Media Types	185

A.5	Media types	185

A.5.1	General	185

A.5.2	Media type structure and definition	185

A.5.3	Media type registration template for application/vnd.3gpp.seal-data-delivery-info+cbor	187

Annex B (informative):	Change history	188

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol aspects for the data delivery management capability of SEAL for the application content/data for vertical applications (e.g. V2X) over the 3GPP system as part of SEAL services specified in 3GPP TS 23.434 [3] and 3GPP TS 23.433 [2].

The present document is applicable to the user equipment (UE) supporting the data delivery management client functionality as described in 3GPP TS 23.433 [2], to the application server supporting the data delivery management server functionality as described in 3GPP TS 23.433 [2] and to the application server supporting the vertical application server (VAL server) functionality as defined in the specific vertical application service (VAL service) specifications.

NOTE:	The specification of the VAL server for a specific VAL service is out of scope of present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[4]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[5]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[6]	3GPP TS 24.546: "Configuration management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[7]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[8]	3GPP TS 24.558: "Enabling Edge Applications; Protocol specification".

[9]	3GPP TS 29.548: "Service Enabler Architecture Layer for Verticals (SEAL); SEAL Data Delivery (SEALDD) Server Services; Stage 3".

[10]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[11]	IETF RFC 3339: "Date and Time on the Internet: Timestamps".

[12]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[13]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[13A]	IETF RFC 6838: "Media Type Specifications and Registration Procedures".

[14]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[15]	IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

[16]	IETF RFC 7959: "Block-Wise Transfers in the Constrained Application Protocol (CoAP) ".

[17]	IETF RFC 8132: "PATCH and FETCH Methods for the Constrained Application Protocol (CoAP)".

[18]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[19]	IETF RFC 8610: "Concise Data Definition Language (CDDL): A Notational Convention to Express Concise Binary Object Representation (CBOR) and JSON Data Structures".

[20]	IETF RFC 8949: "Concise Binary Object Representation (CBOR)".

[21]	IETF RFC 9110: "HTTP Semantics".

[22]	IETF RFC 9177: "Constrained Application Protocol (CoAP) Block-Wise Transfer Options Supporting Robust Transmission".

[23]	OMA OMA-TS-XDM_Core-V2_1-20120403-A: "XML Document Management (XDM) Specification".

[24]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[25]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Data delivery management SEAL client: An entity that provides the client-side functionalities corresponding to the data delivery management SEAL service.

Data delivery management SEAL server: An entity that provides the server-side functionalities corresponding to the data delivery management SEAL service.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.434 [3] apply:

SEAL client

SEAL server

SEAL service

VAL user

VAL server

VAL service

Vertical

Vertical application

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACR	Application Context Relocation

API	Application Programming Interface

BAT	Burst Arrival Time

CDDL	Concise Data Definition Language

CoAP	Constrained Application Protocol

DNS	Domain Name System

EAS	Edge Application Server

ECN	Explicit Congestion Notification

ECS	Edge Configuration Server

EDN	Edge Data Network

EES	Edge Enabler Server

L4S	Low Latency, Low Loss and Scalable Throughput

MIME	Multipurpose Internet Mail Extensions

NAS	Non Access Stratum

SEAL	Service Enabler Architecture Layer for verticals

SEALDD	SEAL Data Delivery

SDDM	SEAL Data Delivery Management

SDDM-C	SEAL Data Delivery Management Client

SDDM-S	SEAL Data Delivery Management Server

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

URLLC	Ultra-Reliable Low Latency Communication

V2X	Vehicle-to-Everything

VAL	Vertical Application Layer

XCAP	XML Configuration Access Protocol

XR	eXtended Reality

# 4	General description

Data delivery management is a SEAL service that provides the data delivery management related capabilities (data delivery and storage capabilities) to one or more vertical applications. The present document enables a SEAL data delivery management client (SDDM-C) and a VAL server to communicate with a SEAL data delivery management server (SDDM-S).

# 5	Edge applications over 3GPP services

The SDDM-C and the SDDM-S can utilize edge applications over 3GPP services to support SDDM. The edge applications over 3GPP services are specified in 3GPP TS 24.558 [8] and 3GPP TS 29.558 [10]. Interactions between the SDDM-C, the SDDM-S and the edge applications over 3GPP services are described in detail in clause 7.

# 6	Functional entities

## 6.1	SEAL data delivery management client (SDDM-C)

The SDDM-C functional entity acts as the application client for data delivery management related transactions. To be compliant with the procedures in the present document the SDDM-C:

a)	shall support the role of XCAP client as specified in IETF RFC 4825 [12];

b)	shall support the role of XDMC as specified in OMA OMA-TS-XDM_Core-V2_1 [23]; and

c)	shall support the data delivery management procedures in subclause 7.2.

To be compliant with the CoAP procedures in the present document the SDDM-C:

a)	shall support the role of CoAP client as specified in IETF RFC 7252 [14];

b)	shall support the capability to observe resources as specified in IETF RFC 7641 [15];

c)	shall support the block-wise transfer as specified in IETF RFC 7959 [16];

d)	may support the robust block transfer as specified in IETF RFC 9177 [22];

e)	shall support FETCH method of CoAP as specified in IETF RFC 8132 [17];

f)	should support CoAP over TCP and Websocket as specified in IETF RFC 8323 [18];

g)	shall support CBOR encoding as specified in IETF RFC 8949 [20]; and

h)	shall support the procedures defined in clause 7.2.

NOTE 1:	The security mechanism to be supported for the CoAP procedures is described in 3GPP TS 24.547 [7].

NOTE 2:	Support for TCP for the CoAP procedures is required if the client connects over the network which blocks or impedes the use of UDP, e.g. when NATs are present in the communication path.

NOTE 3:	The CoAP protocol supports mechanism for reliable message exchange over UDP. Use of TCP can also be beneficial if reliable transport is required for other reasons, e.g. better observability of resources. Usage of CoAP over TCP is an implementation choice.

NOTE 4:	Support for the robust block transfer mechanism for the CoAP procedures is beneficial in environments where packet loss is highly asymmetrical and where performance optimization of block transfers is required.

## 6.2	SEAL data delivery management server (SDDM-S)

The SDDM-S is a functional entity used to provide data delivery management supported within the vertical application layer. To be compliant with the procedures in the present document the SDDM-S:

a)	shall support the role of XCAP server as specified in IETF RFC 4825 [12];

b)	shall support the role of XDMS as specified in OMA OMA-TS-XDM_Core-V2_1 [23]; and

c)	shall support the data delivery management procedures in subclause 7.2.

To be compliant with the CoAP procedures in the present document the SDDM-S:

a)	shall support the role of CoAP server as specified in IETF RFC 7252 [14];

b)	shall support the capability to observe resources as specified in IETF RFC 7641 [15];

c)	shall support the block-wise transfer as specified in IETF RFC 7959 [16];

d)	shall support the robust block transfer as specified in IETF RFC 9177 [22];

e)	shall support FETCH method of CoAP as specified in IETF RFC 8132 [17];

f)	shall support CoAP over TCP and Websocket as specified in IETF RFC 8323 [18];

g)	shall support CBOR encoding as specified in IETF RFC 8949 [20]; and

h)	shall support the procedures defined in clause 7.2.

NOTE:	The security mechanism to be supported for the CoAP procedures is described in 3GPP TS 24.547 [7].

# 7	Data delivery management procedures

## 7.1	General

This clause provides the procedures for data delivery management between the SDDM-C and the SDDM-S as well as interactions between the SDDM-S and the VAL server. Interactions between the SDDM-C, the SDDM-S and the edge applications over 3GPP services are also described.

NOTE:	3GPP TS 29.548 [9] specifies stage-3 protocol definitions, message flows and APIs for services offered by the SDDM-S to VAL servers over the SEALDD-S reference point and to other SEALDD servers over the SEALDD-E reference point (see 3GPP TS 23.433 [2]).

## 7.2	On-network procedures

### 7.2.1	General

This clause provides the on-network procedures for data delivery management between the SLM-C and the SLM-S as well as interactions between the SDDM-S and the VAL server.

#### 7.2.1.1	Authenticated identity in HTTP request

Upon receiving an HTTP request, the SDDM-S shall verify that the identity of the sender of the HTTP request (see IETF RFC 9110 [21]) is authorized as specified in 3GPP TS 24.547 [7], and if authentication is successful, the SDDM-S shall use the identity of the sender of the HTTP request as an authenticated identity.

#### 7.2.1.2	Authenticated identity in CoAP request

Upon receiving a CoAP request, the SDDM-S shall verify that the identity of the sender of the CoAP request as specified in 3GPP TS 24.547 [7], and if authentication is successful, the SDDM-S shall use the identity of the sender of the CoAP request as an authenticated identity.

### 7.2.2	SEALDD enabled signalling transmission connection establishment procedure

#### 7.2.2.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD regular transmission connection establishment request when it needs to request an SDDM connection establishment, the SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S.

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <establishment-req> element in the <data-delivery-info> root element which:

1)	shall include a <requestor-id> element set to "sealddclient";

2)	shall include a <sealdd-flow-id> element set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	shall include a <server-id> element set to the information of the VAL server;

4)	shall include a <endpoint-id> element set to the information of the endpoint of the selected VAL server to which the SEALDD regular transmission connection establishment request has to be sent;

5)	may include a <VAL-service-id> element set to the VAL service identity of the vertical application;

6)	may include a <traffic-descriptor-info> element specifying the information of the traffic. In the <traffic-descriptor-info> element, the SDDM-C may include:

i)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

ii)	a <port-number> child element specifying the identity of the port number of the traffic;

iii)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic;

iv)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic;

7)	may include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request;

8)	may include an <anyExt> element containing either a <bat-period-adapt-cap> element to indicate the BAT and periodicity adaptation capability of the SDDM-C or a <transmission-assist-info> element specifying a transmission assistance information.

In the <transmission-assist-info> element, the SDDM-C:

i)	shall include at least one of the following child elements:

A)	<bat> child element specifying the arrival time of the first packet of the data burst; and

B)	a <periodicity> child element specifying the time period between the start of two bursts; and

ii)	if the <bat> element is included, may include a <bat-window> child element. In the <bat-window> element the SDDM-C shall include:

A)	<start-time> child element indicating the acceptable earliest arrival time of the first packet of the data burst; and

B)	the <stop-time> child element indicating the acceptable latest arrival time of the first packet of the data burst; and

iii) 	if the <bat>, <bat-window> and <periodicity>elements are included, may include a <periodicity-range> child element specifying the periodicity range. In the <periodicity-range> element the SDDM-C shall include:

A)	a <lower-bound> child element set to the lower bound of the periodicity and an <upper-bound> child element set to the upper bound of the periodicity of the start two bursts; or

B)	a <periodicity-value-list> child element with one or more <periodicity-value> child elements set to the acceptable periodicity value;

9)	may include an <anyExt> element containing an <L4S-feedback-capability> element set to the L4S feedback capability (i.e. ECN identification, L4S feedback); and

NOTE 1:	The L4S feedback capability is used for the SEALDD enabled congestion control for VAL applications (see 3GPP TS 23.433 [2] clause 9.8.2.2).

10)	may include an <anyExt> element containing an <xr-app-device-capability> element to indicate an XR application device capability information. In the <xr-app-device-capability> element, the SDDM-C shall include:

i)	an <xr-media-capability> child element containing media capabilities (e.g. media resolution, media codec, media frame rate, media projection, media decoding time, mono vs. stereo 360 video); and

ii)	an <xr-display-capability> child element containing display capabilities (e.g. display resolution, max display refresh rate, horizontal and vertical field of view, eye to screen distance, lens separation distance).

NOTE 2:	The XR application device capability is used for the SEALDD enabled bandwidth control for different VAL users (see 3GPP TS 23.433 [2] clause 9.8.2.1).

Editor's note [WID: SEALDD_Ph2, CR#: 0052]:	Specification of <xr-media-capability> and <xr-display-capability> elements is FFS.

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <establishment-req> element included in the <data-delivery-info> root element;

the SDDM-C:

a)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <establishment-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD regular transmission connection establishment request operation. If the result is "failure", in the <result> element, the SDDM-S may include a <cause> child element specifying the cause of the failure of the operation, e.g. VAL client error;

ii)	may include a <traffic-descriptor-info> element specifying the information of the traffic. In the <traffic-descriptor-info> element, the SDDM-C may include:

A)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

B)	a <port-number >child element specifying the identity of the port number of the traffic;

C)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic; and

D)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

iii)	may include an <anyExt> element containing either a <bat-period-adapt-cap> element to indicate the BAT and periodicity adaptation capability of the SDDM-C or a <transmission-assist-info> element specifying a transmission assistance information.

In the <transmission-assist-info> element, the SDDM-C:

A)	shall include at least one of the following child elements:

I)	<bat> child element specifying the arrival time of the first packet of the data burst; and

II)	a <periodicity> child element specifying the time period between the start of two bursts;

B)	if the <bat> element is included, may include a <bat-window> child element. In the <bat-window> element the SDDM-C shall include:

I)	<start-time> child element indicating the acceptable earliest arrival time of the first packet of the data burst; and

II)	the <stop-time> child element indicating the acceptable latest arrival time of the first packet of the data burst; and

C)	if the <bat>, <bat-window> and <periodicity>elements are included, may include a <periodicity-range> child element specifying the periodicity range. In the <periodicity-range> element the SDDM-C shall include:

I)	a <lower-bound> child element set to the lower bound of the periodicity and an <upper-bound> child element set to the upper bound of the periodicity of the start two bursts; or

II)	a <periodicity-value-list> child element with one or more <periodicity-value> child elements set to the acceptable periodicity value; and

b)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.2.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <establishment-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <establishment-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD regular transmission connection establishment request operation. If the result is "failure", in the <result> element, the SDDM-S may include a <cause> child element specifying the cause of the failure of the operation, e.g. VAL client error;

ii)	may include a <traffic-descriptor-info> element specifying the information of the traffic. In the <traffic-descriptor-info> element, the SDDM-S may include:

A)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

B)	a <port-number>child element specifying the identity of the port number of the traffic;

C)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic;

D)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

iii)	may include a <expiry-time> element set to a time that triggers the re-connection from SDDM-C when bandwidth limit check is failed; and

iv)	may include a <traffic-transmission-bandwidth> element indicating suggested traffic transmission bandwidth to be used by SDDM-C; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

The SDDM-S sends an SEALDD regular transmission connection establishment request when it needs to request a regular SEALDD connection establishment towards an SDDM-C, the SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C.

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <establishment-req> element in the <data-delivery-info> root element which:

1)	shall include a <requestor-id> element set to "sealddserver";

2)	shall include a <sealdd-flow-id> element set to the identity of the SEALDD flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	shall include a <endpoint-id> element set to the information of the endpoint of the selected VAL server from which the SEALDD regular transmission connection establishment request has to be sent;

4)	may include a <sealdd-communication-lifetime> element set to the information of the data delivery communication lifetime;

5)	may include a <traffic-descriptor-info> element specifying the information of the traffic. In the <traffic-descriptor-info> element, the SDDM-S may include:

i)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

ii)	a <port-number >child element specifying the identity of the port number of the traffic;

iii)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic;

iv)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

6)	may include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user or the identity of the SDDM-S acting as the VAL UE; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.2.3	SDDM client CoAP procedure

In order to request an SEALDD regular transmission connection establishment to the SDDM-S, the SDDM-C shall send a CoAP POST request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.1.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req";

c)	shall include an "EstablishmentRequest" object:

1)	shall include a "requestorId" attribute set to "sealddclient";

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	shall include a "serverId" attribute set to the information of the VAL server;

4)	shall include an "endpointId" attribute set to the information of the endpoint of the selected VAL server to which the SDMM regular transmission connection establishment request has to be sent;

5)	may include a "valServiceId" attribute set to the VAL service identity of the vertical application;

6)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

7)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

8)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic;

9)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic;

10)	may include a "valUserId" attribute set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request; and

11)	may include a "batPeriodAdaptCap" attribute to indicate a BAT and periodicity adaptation capability or a "transmisAssistInfo" attribute specifying a transmission assistance information. In the "transmisAssistInfo" attribute the SDDM-C:

i)	shall include at least one of the following attributes:

A)	a "bat" attribute specifying the arrival time of the first packet of the data burst; and

B)	a "periodicity" attribute specifying the time period between the start of two bursts;

ii)	if the "bat" attribute is included, may include a "batWindow" attribute containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

iii)	if the "bat", "batWindow" and "periodicity" attributes are included, may include a "periodRange" attribute specifying the periodicity range. In the "periodRange" attribute the SDDM-C shall include:

A)	a "lowerBound" attribute set to the lower bound of the periodicity and an "upperBound" attribute set to the upper bound of the periodicity of the start two bursts; or

B)	a "periodicityValues" attribute set to the acceptable periodicity values;

12)	may include an "l4sFeedbackCapability" element set to the L4S feedback capability (i.e. ECN identification, L4S feedback); and

NOTE 1:	The L4S feedback capability is used for the SEALDD enabled congestion control for VAL applications (see 3GPP TS 23.433 [2] clause 9.8.2.2).

13)	may include an "xrAppDevCap" attribute to indicate an XR application device capability information. In the "xrAppDevCap" attribute, the SDDM-C shall include:

i)	an "xrMediaCap" attribute containing media capabilities (e.g. media resolution, media codec, media frame rate, media projection, media decoding time, mono vs. stereo 360 video); and

ii)	an "xrDisplayCap" attribute containing display capabilities (e.g. display resolution, max display refresh rate, horizontal and vertical field of view, eye to screen distance, lens separation distance).

NOTE 2:	The XR application device capability is used for the SEALDD enabled bandwidth control for different VAL users (see 3GPP TS 23.433 [2] clause 9.8.2.1).

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.3.1.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req", and

b)	an "EstablishmentRequest" object;

the SDDM-C shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-res";

b)	shall attempt to create the SDDM regular transmission connection resource pointed at by the CoAP URI with the content of "EstablishmentRequest" object received in the request and:

1)	if successfully created, shall include an "EstablishmentResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success";

ii)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

iii)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

iv)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic;

v)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; and

vi)	may include a "batPeriodAdaptCap" attribute to indicate a BAT and periodicity adaptation capability or a "transmisAssistInfo" attribute specifying a transmission assistance information. In the "transmisAssistInfo" attribute the SDDM-C:

A)	shall include at least one of the following attributes:

I)	a "bat" attribute specifying the arrival time of the first packet of the data burst; and

II)	a "periodicity" attribute specifying the time period between the start of two bursts;

B)	if the "bat" attribute is included, may include a "batWindow" attribute containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

C)	if the "bat", "batWindow" and "periodicity" attributes are included, may include a "periodRange" attribute specifying the periodicity range. In the "periodRange" attribute the SDDM-C shall include:

I)	a "lowerBound" attribute set to the lower bound of the periodicity and an "upperBound" attribute set to the upper bound of the periodicity of the start two bursts; or

II)	a "periodicityValues" attribute set to the acceptable periodicity values;

2)	otherwise, shall include an "EstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-S.

#### 7.2.2.4	SDDM server CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.3.1.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req", and

b)	an "EstablishmentRequest" object;

the SDDM-S shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-res";

b)	shall attempt to create the SDDM regular transmission connection resource pointed at by the CoAP URI with the content of "EstablishmentRequest" object received in the request and:

1)	if successfully created, shall include an "EstablishmentResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success";

ii)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

iii)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

iv)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

v)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; or

2)	otherwise, shall include an "EstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-C.

In order to request an SDDM regular transmission connection establishment to the SDDM-C, the SDDM-S shall send a CoAP POST request message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-S:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.4.1.1 with:

1)	the "apiRoot" set to the SDDM-C URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req";

c)	shall include an "EstablishmentRequest" object:

1)	shall include a "requestorId" attribute set to "sealddserver";

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	shall include an "endpointId" attribute set to the information of the endpoint of the selected VAL server to which the SDMM regular transmission connection establishment request has to be sent;

4)	shall include a "sealddCommunicationLifetime" attribute set to the information of the data delivery communication lifetime;

5)	may include a "valServiceId" attribute set to the VAL service identity of the vertical application;

6)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

7)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

8)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic;

9)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; and

10)	may include a "valUserId" attribute set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

### 7.2.3	SEALDD enabled signalling transmission connection release procedure

#### 7.2.3.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD data transmission connection release request when it needs to release an established SEALDD connection towards an SDDM-S, the SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <release-req> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-client-identity> element set to the identity of the SDDM-C; and

2)	shall include a <sealdd-flow-id> element set to the identity of the SEALDD flow used by the SDDM-S and SDDM-C to identify the application traffic; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <release-req> element included in the <data-delivery-info> root element;

the SDDM-C:

a)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <release-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD data transmission connection release request operation; and

b)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.3.2	SDDM server HTTP procedure

The SDDM-S sends an SEALDD data transmission connection release request when it needs to release an established SEALDD connection towards an SDDM-C, the SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <release-req> element in the <data-delivery-info> root element which:

1)	shall include a <server-id> element set to the information of the SDDM-S; and

2)	shall include a <sealdd-flow-id> element set to the identity of the SEALDD flow used by the SDDM-S and SDDM-C to identify the application traffic; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <release-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1 and:

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection release, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; or

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <release-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD regular transmission connection release request operation. If the result is "failure", in the <result> element, the SDDM-S may include a <cause> child element specifying the cause of the failure of the operation, e.g. SEALDD policy mismatch; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.3.3	SDDM client CoAP procedure

In order to request the release of an SEALDD regular data transmission connection to the SDDM-S, the SDDM-C shall send a CoAP DELETE request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP DELETE request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.1.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=release-req";

c)	shall include a "ReleaseRequest" object:

1)	shall include a "sealClientId" attribute set to the identity of the SDDM-C; and

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

Upon receiving a CoAP DELETE request where the CoAP URI of the CoAP DELETE request identifies the release resource as specified in clause A.3.1.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=release-req", and

b)	a "ReleaseRequest" object;

the SDDM-C shall generate a CoAP DELETE response according to IETF RFC 7252 [14]. In the CoAP DELETE response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor";

b)	shall attempt to release the SDDM regular transmission connection resource pointed at by the CoAP URI with the content of "ReleaseRequest" object received in the request and:

1)	if successfully release, shall use the CoAP DELETE 2.02 (Deleted) response message; or

2)	otherwise, shall include an error response in the CoAP DELETE response as specified in clause A.3.1.2.2.3.2; and

c)	shall send the CoAP DELETE response towards the SDDM-S.

#### 7.2.3.4	SDDM server CoAP procedure

Upon receiving a CoAP DELETE request where the CoAP URI of the CoAP DELETE request identifies the release resource as specified in clause A.4.1.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=release-req", and

b)	a "ReleasetRequest" object;

the SDDM-S shall generate a CoAP DELETE response according to IETF RFC 7252 [14]. In the CoAP DELETE response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor";

b)	shall attempt to release the SDDM regular transmission connection resource pointed at by the CoAP URI with the content of "ReleaseRequest" object received in the request and:

1)	if successfully created, shall use the CoAP DELETE 2.02 (Deleted) response message; or

2)	otherwise, shall include an error response in the CoAP DELETE response as specified in clause A.4.1.2.2.3.2; and

c)	shall send the CoAP DELETE response towards the SDDM-C.

In order to request the release of an SDDM regular data transmission connection to the SDDM-C, the SDDM-S shall send a CoAP DELETE request message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP DELETE request, the SDDM-S:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.1.1 with:

1)	the "apiRoot" set to the SDDM-C URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=release-req";

c)	shall include an "ReleaseRequest" object:

1)	shall include a "serverId" attribute set to the identity of the SDDM-S; and

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

### 7.2.4	SEALDD enabled E2E redundant transmission path establishment procedure

#### 7.2.4.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD URLLC transmission connection establishment request when it needs to request an SEALDD URLLC transmission connection establishment, the SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <URLLC-establishment-req> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-client-identity> element set to the identity of the SDDM-C;

2)	shall include a <sealdd-flow-id> element set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	may include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request;

4)	may include a <server-id> element set to the information of the endpoint of the selected VAL server to which the SDDM URLLC transmission connection establishment request has to be sent;

5)	may include a <VAL-service-id> element set to the VAL service identity of the vertical application; and

6)	may include a <traffic-descriptor-info> element specifying the information of the traffic of the redundant SEALDD transmission connection. In the <traffic-descriptor-info> element, the SDDM-C may include:

i)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

ii)	a <port-number> child element specifying the identity of the port number of the traffic;

iii)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic;

iv)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

7)	may include an <anyExt> element containing either a <bat-period-adapt-cap> element to indicate the BAT and periodicity adaptation capability of the SDDM-C or a <transmission-assist-info> element specifying a transmission assistance information.

In the <transmission-assist-info> element, the SDDM-C:

i)	shall include at least one of the following child elements:

A)	<bat> child element specifying the arrival time of the first packet of the data burst; and

B)	a <periodicity> child element specifying the time period between the start of two bursts;

ii)	if the <bat> element is included, may include a <bat-window> child element. In the <bat-window> element the SDDM-C shall include:

A)	<start-time> child element indicating the acceptable earliest arrival time of the first packet of the data burst; and

B)	the <stop-time> child element indicating the acceptable latest arrival time of the first packet of the data burst; and

iii)	if the <bat>, <bat-window> and <periodicity>elements are included, may include a <periodicity-range> child element specifying the periodicity range. In the <periodicity-range> element the SDDM-C shall include:

A)	a <lower-bound> child element set to the lower bound of the periodicity and an <upper-bound> child element set to the upper bound of the periodicity of the start two bursts; or

B)	a <periodicity-value-list> child element with one or more <periodicity-value> child elements set to the acceptable periodicity value; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.4.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-establishment-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1 and:

1)	if the identity of the sender of the received HTTP POST request is not authorized to request URLLC transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; or

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-establishment-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD URLLC transmission connection establishment request operation. If the result is "failure", in the <result> element, the SDDM-S may include a <cause> child element specifying the cause of the failure of the operation, e.g. VAL client error; and

ii)	may include a <traffic-descriptor-info> element specifying the information of the traffic of the redundant SEALDD transmission connection. In the <traffic-descriptor-info> element, the SDDM-S may include:

A)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

B)	a <port-number>child element specifying the identity of the port number of the traffic;

C)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic; and

D)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.4.3	SDDM client CoAP procedure

In order to request an SEADD URLLC transmission connection establishment to the SDDM-S, the SDDM-C shall send a CoAP POST request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.2.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-establishment-req";

c)	shall include a "URLLCEstablishmentRequest" object:

1)	shall include a "sealClientId" attribute set of the identity of the SDDM-C;

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	shall include a "valTgtUe" attribute set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request;

4)	may include a "serverId" attribute set to the information of the VAL server;

5)	may include a "valServiceId" attribute set to the identity of the VAL service of the vertical application;

6)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

7)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

8)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

9)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; and

10)	may include a "batPeriodAdaptCap" attribute to indicate a BAT and periodicity adaptation capability or a "transmisAssistInfo" attribute specifying a transmission assistance information. In the "transmisAssistInfo" attribute the SDDM-C:

i)	shall include at least one of the following attributes:

A)	a "bat" attribute specifying the arrival time of the first packet of the data burst; and

B)	a "periodicity" attribute specifying the time period between the start of two bursts;

ii)	if the "bat" attribute is included, may include a "batWindow" attribute containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

iii)	if the "bat", "batWindow" and "periodicity" attributes are included, may include a "periodRange" attribute specifying the periodicity range. In the "periodRange" attribute the SDDM-C shall include:

A)	a "lowerBound" attribute set to the lower bound of the periodicity and an "upperBound" attribute set to the upper bound of the periodicity of the start two bursts; or

B)	a "periodicityValues" attribute set to the acceptable periodicity values;

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

#### 7.2.4.4	SDDM server CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.4.2.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-establishment-req", and

b)	a "URLLCEstablishmentRequest" object;

the SDDM-S shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-establishment-res";

b)	shall attempt to create the URLLC transmission connection resource pointed at by the CoAP URI with the content of "URLLCEstablishmentRequest" object received in the request and:

1)	if successfully created, shall include a "URLLCEstablishmentResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success";

ii)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

iii)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

iv)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

v)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; or

2)	otherwise, shall include a "URLLCEstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response as specified in clause A.4.2.2.2.3.1; and

c)	shall send the CoAP POST response towards the SDDM-C.

### 7.2.5	SEALDD enabled E2E redundant transmission path release procedure

#### 7.2.5.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD URLLC transmission connection release request when it needs to release an established SEALDD URLLC transmission connection towards an SDDM-S, the SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-release-req> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-client-identity> element set to the identity of the SDDM-C; and

2)	shall include a <sealdd-flow-id> element set to the identity of the SEALDD flow used by the SDDM-S and SDDM-C to identify the application traffic; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.5.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-release-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection release, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-release-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD URLLC transmission connection release request operation. If the result is "failure", in the <result> element, the SDDM-S may include a <cause> child element specifying the cause of the failure of the operation, e.g. SEALDD policy mismatch; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.5.3	SDDM client CoAP procedure

In order to request the release of an SEALDD URLLC transmission connection to the SDDM-S, the SDDM-C shall send a CoAP DELETE request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP DELETE request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.2.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-release-req";

c)	shall include a "URLLCReleaseRequest" object:

1)	shall include a "sealClientId" attribute set to the identity of the SDDM-C; and

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

#### 7.2.5.4	SDDM server CoAP procedure

Upon receiving a CoAP DELETE request where the CoAP URI of the CoAP DELETE request identifies the release resource as specified in clause A.4.2.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-release-req", and

b)	a "URLLCReleaseRequest" object;

the SDDM-S shall generate a CoAP DELETE response according to IETF RFC 7252 [14]. In the CoAP DELETE response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-release-req";

b)	shall attempt to release the SDDM URLLC transmission connection resource pointed at by the CoAP URI with the content of "URLLCReleaseRequest" object received in the request and:

1)	if successfully created, shall use the CoAP DELETE 2.02 (Deleted) response message; or

2)	otherwise, shall include an error response in the CoAP DELETE response as specified in clause A.4.2.2.2.3.3; and

c)	shall send the CoAP DELETE response towards the SDDM-C.

### 7.2.6	SEALDD enabled E2E redundant transmission path connection update procedure

#### 7.2.6.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD URLLC transmission connection update request when it needs to request an SEALDD URLLC transmission connection update, the SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S.

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <URLLC-update-req> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-client-identity> element set to the identity of the SDDM-C;

2)	shall include a <sealdd-flow-id> element set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

2)	may include a <server-id> element set to the information of the endpoint of the selected VAL server to which the SDDM URLLC transmission connection establishment request has to be sent;

3)	may include a <VAL-service-id> element set to the VAL service identity of the vertical application;

4)	may include a <traffic-descriptor-info> element specifying the information of the traffic of the redundant SEALDD transmission connection. In the <traffic-descriptor-info> element, the SDDM-C may include:

i)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

ii)	a <port-number> child element specifying the identity of the port number of the traffic;

iii)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic;

iv)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.6.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-deliverydata-delivery-info+xml MIME body with a <URLLC-update-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-deliverydata-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-deliverydata-delivery-info+xml MIME body with a <URLLC-update-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD URLLC transmission connection update request operation. If the result is "failure", in the <result> element, the SDDM-S may include a <cause> child element specifying the cause of the failure of the operation, e.g. VAL client error; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.6.3	SDDM client CoAP procedure

In order to request an SEALDD URLLC transmission connection update to the SDDM-S, the SDDM-C shall send a CoAP PUT request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP PUT request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.2.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-update-req";

c)	shall include a "URLLCUpdateRequest" object:

1)	shall include a "sealClientId" attribute set of the identity of the SDDM-C;

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	may include a "serverId" attribute set to the information of the VAL server;

4)	may include a "valServiceId" attribute set to the identity of the VAL service of the vertical application;

5)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

6)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

7)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

8)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

#### 7.2.6.4	SDDM server CoAP procedure

Upon receiving a CoAP PUT request where the CoAP URI of the CoAP PUT request identifies the resource to be updated as specified in clause A.4.2.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-update-req"; and

b)	a "URLLCUpdateRequest" object;

the SDDM-S shall generate a CoAP PUT response according to IETF RFC 7252 [14]. In the CoAP PUT response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-update-res";

b)	shall attempt to update the URLLC transmission connection resource pointed at by the CoAP URI with the content of "URLLCUpdateRequest" object received in the request and:

1)	if successfully updated, shall include a "URLLCUpdateResponse" object in the CoAP PUT 2.04 (Changed) response message;

i)	shall include a "result" attribute set to "success"; or

2)	otherwise, shall include a "URLLCUpdateResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP PUT response as specified in clause A.4.2.2.2.3.2; and

c)	shall send the CoAP PUT response towards the SDDM-C.

### 7.2.7	SEALDD server discovery and selection procedure

When the VAL client sends an SEALDD request to the SDDM-C, the SDDM-C may need to discover and select the appropriate SDDM-S for the associated vertical application. After that, the VAL server is discovered and selected along with the associated SDDM-S so that the SDDM-C gets the address of the SDDM-S.

In an edge data network (EDN), the SDDM-C can use edge applications over 3GPP services (see clause 5) to obtain the EES, which supports the SDDM-S and the VAL server, and after that to obtain the SDDM-S address and the VAL server address.

NOTE 1:	The VAL server acts as an EAS in an EDN and registers to the EES with the associated SDDM-S address as EAS endpoint in the EAS profile (see 3GPP TS 23.433 [2] clause 9.4.3.2). The Eecs_ServiceProvisioning service to obtain the EES, which supports the SDDM-S and the VAL sever, and the Eees_EASDiscovery service to obtain the SDDM-S address and the VAL server address are specified in clause 7.2 and clause 5.3 of 3GPP TS 24.558 [8] respectively.

In a non EDN, the SDDM-C can obtain the SDDM-S address and the VAL server address from the VAL client or from the NAS.

NOTE 2:	ECS address provisioning over NAS to get ECS configuration information and EAS discovery to get EAS information are specified in 3GPP TS 24.501 [5].

NOTE 3:	The VAL client can use DNS query mechanism or vertical application (e.g. V2X) layer signalling mechanism to obtain the SDDM-S address and the VAL server address. The VAL client can provide the address information to the SDDM-C.

NOTE 4:	DNS query mechanism and vertical application layer signalling mechanism are out of scope of the present document.

### 7.2.8	SEALDD enabled data storage creation procedure

#### 7.2.8.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD data storage creation request when it needs to request the creation of data storage to the SDDM-S, the SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <data-storage-creation-req> element in the <data-delivery-info> root element which:

1)	shall include a <application-data> element set to the application data needed to be stored;

2)	may include a <access-control-policy> element set to the control policy for the requested data access from other consumers (e.g. SDDM-C, VAL server, other SDDM-S);

3)	may include a <expiry-time> element set to the expiration time of the data to be stored; and

4)	may include a <status-information-req> element set to the information of the stored data to be tracked or monitored by the SDDM-S (e.g. statistics of the stored data; indications of how often the stored data is accessed or managed) for corresponding notifications; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.8.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-creation-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-creation-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD data storage creation request operation; and

ii)	may include a <data-identifier> element set to the identity of the stored data; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.8.3	SDDM client CoAP procedure

In order to request an SEALDD data storage creation to the SDDM-S, the SDDM-C shall send a CoAP POST request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.3.1 with the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-req";

c)	shall include a "DataStorageCreationRequest" object:

1)	shall include an "applicationData" attribute set to the application data needed to be stored;

2)	may include an "accessControlPolicy" attribute set to the control policy for the requested data access from other consumers (e.g. SDDM-C, VAL server, other SDDM-S);

3)	may include an "expiryTime" attribute set to the expiration time of the data to be stored;

4)	may include a "statusInformationReq" attribute set to the information of the stored data to be tracked or monitored by the SDDM-S (e.g. statistics of the stored data; indications of how often the stored data is accessed or managed) for corresponding notifications; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

#### 7.2.8.4	SDDM server CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the creation of a data storage resource as specified in clause A.4.3.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-req", and

b)	a "DataStorageCreationRequest" object;

the SDDM-S shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-res";

b)	shall attempt to create the SDDM data creation storage resource pointed at by the CoAP URI with the content of "DataStorageCreationRequest" object received in the request and:

1)	if successfully created, shall include a "DataStorageCreationResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success"; and

ii)	shall include a "dataIdentifier" attribute specifying the identity of the stored data; or

2)	otherwise, shall include a "DataStorageCreationResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-C.

### 7.2.9	SEALDD enabled data storage reservation procedure

#### 7.2.9.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD data storage reservation request when it needs to request the reservation of data storage to the SDDM-S, the SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <data-storage-reservation-req> element in the <data-delivery-info> root element which:

1)	shall include a <VAL-service-id> element set to the VAL service identity of the vertical application;

2)	may include a <data-length> element set to the data length to be stored; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.9.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-reservation-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-reservation-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD data storage reservation request operation; and

ii)	may include a <address> element set to the reserved address for data storage; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.9.3	SDDM client CoAP procedure

In order to request an SEALDD data storage reservation to the SDDM-S, the SDDM-C shall send a CoAP POST request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.3.1 with the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-reservation-req";

c)	shall include a "DataStorageReservationRequest" object:

1)	shall include a "valServiceId" attribute set to the identity of the VAL service of the vertical application;

2)	may include a "dataLength" attribute set to the data length to be stored; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

#### 7.2.9.4	SDDM server CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the creation of a data storage resource as specified in clause A.4.3.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-reservation-req", and

b)	a "DataStorageReservationRequest" object;

the SDDM-S shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-reservation-res";

b)	shall attempt to create the SDDM data creation storage resource pointed at by the CoAP URI with the content of "DataStorageReservationRequest" object received in the request and:

1)	if successfully created, shall include a "DataStorageReservationResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success"; and

ii)	shall include an "address" attribute specifying the reserved address for data storage; or

2)	otherwise, shall include a "DataStorageReservationResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-C.

### 7.2.10	SEALDD enabled data storage notification procedure

#### 7.2.10.1	SDDM client HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-status-notification> element included in the <data-delivery-info> root element;

the SDDM-C:

a)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [21];

b)	may communicate the received data storage notification information to the VAL client; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.10.2	SDDM server HTTP procedure

The SDDM-S sends an SEALDD data storage notification when it needs to provide the SDDM-C with the collected management or storage status information of the stored data. The SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C:

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <data-storage-status-notification > element in the <data-delivery-info> root element which:

1)	shall include a <data-identifier> element set to the identity of the stored data; and

2)	shall include a <status-information-rsp> element set to the information of the stored data to be tracked or monitored by the SDDM-S (e.g. statistics of the stored data, indications of how often the stored data is accessed or managed), of the stored data requested by using the SEALDD data storage creation request as described in clause 7.2.7.1; and

NOTE:	Push notification service can be used to send HTTP POST request to the client. Details about the push notification service is out of scope this specification.

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.10.3	SDDM client CoAP procedure

In order for an SDDM-C to get notifications about information of an SDDM data storage resource, the SDDM-C shall first send a CoAP FETCH request message used to observe an SDDM data storage resource as specified in clause A.4.3.2.2.3.5, and containing:

a)	a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.3.1 with the "apiRoot" set to the SDDM-S URI;

b)	an "observe" option set to the value "0" (register);

c)	an Accept option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-status-notification";

d)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-mgt-req", and

e)	a "DataStorageMgtRequest" object;

1)	shall include a "dataIdentifier" data type set to the identity of the observed stored data.

Upon receiving a CoAP FETCH 2.05 (Content) response (as specified in IETF RFC 8132 [17]) to a CoAP FETCH request message used to observe an SDDM data storage resource as specified in clause A.4.3.2.2.3.5, and containing:

a)	an "observe" option;

b)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-status-notification", and

c)	a "DataStorageStatusNotification" object;

NOTE:	The SDDM-C can communicate the received data storage notification information to the VAL client.

In order for an SDDM-C to stop getting notifications about information of an SDDM data storage resource, the SDDM-C shall send a CoAP FETCH request message as specified in clause A.4.3.2.2.3.5, and containing:

a)	a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.3.1 with the "apiRoot" set to the SDDM-S URI;

b)	an "observe" option set to the value "1" (deregister);

c)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-mgt-req", and

d)	a "DataStorageMgtRequest" object;

1)	shall include a "dataIdentifier" data type set to the identity of the observed stored data.

#### 7.2.10.4	SDDM server CoAP procedure

Upon reception of a CoAP FETCH request message containing:

a)	an "observe" option set to the value "0" (register);

b)	an Accept option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-status-notification";

c)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-mgt-req"; and

d)	a "DataStorageMgtRequest" object,

the SDDM-S shall provide an SEALDD data storage notification in order to notify an SDDM-C about information of an SDDM data storage resource. The SDDM-S shall send a CoAP FETCH response (as specified in IETF RFC 8132 [17]) message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP FETCH response, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-status-notification";

b)	shall include a "DataStorageStatusNotification" object in the CoAP FETCH 2.05 (Content) response message; and

c)	shall send the CoAP FETCH response towards the SDDM-C.

### 7.2.11	SEALDD enabled data storage query procedure

#### 7.2.11.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD data storage query request when it needs to query stored data in the SDDM-S, the SDDM-C shall send an HTTP GET request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP GET request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <data-storage-query-req> element in the <data-delivery-info> root element which:

1)	shall include a <data-identifier> element set to the identity of the stored data which is queried; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.11.2	SDDM server HTTP procedure

Upon receiving an HTTP GET request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-query-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP GET request as specified in clause 7.2.1.1; and

1)	if the identity of the sender of the received HTTP GET request is not authorized to request signalling transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP GET request and shall skip rest of the steps;

2)	shall support handling an HTTP GET request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "GET Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-query-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD data storage query request operation;

ii)	shall include a <data-identifier> element set to the identity of the stored data which is queried; and

iii)	may include a <application-data> element set to the application data queried; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.11.3	SDDM client CoAP procedure

In order to query an SDDM data storage resource, the SDDM-C shall send a CoAP GET request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP GET request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.3.1 with:

1)	the "apiRoot" set to the SDDM-S URI; and

2)	the "data-identifier" query option is set to the identity of the stored data which is queried; and

b)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

#### 7.2.11.4	SDDM server CoAP procedure

Upon receiving a CoAP GET request where the CoAP URI of the CoAP GET request identifies the creation of a data storage resource as specified in clause A.4.3.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor", and

b)	a "data-identifier" query option;

the SDDM-S shall generate a CoAP GET response according to IETF RFC 7252 [14]. In the CoAP GET response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-data-storage-query-res";

b)	shall attempt to create the SDDM data creation storage resource pointed at by the CoAP URI with the content of "DataStorageQueryRequest" object received in the request and:

1)	if successfully created, shall include a "DataStorageQueryResponse" object in the CoAP GET 2.05 (Content) response message:

i)	shall include a "result" attribute set to "success";

ii)	shall include a "dataIdentifier" attribute specifying the identity of the stored data; and

iii)	may include an "applicationData" attribute set to the application data queried; or

2)	otherwise, shall include a "DataStorageQueryResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP GET response; and

c)	shall send the CoAP GET response towards the SDDM-C.

### 7.2.12	SEALDD enabled data storage management procedure

#### 7.2.12.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD data storage management request when it needs to request management of the stored data in the SDDM-S such as to update, refresh, or delete the stored data, the SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <data-storage-mgt-req> element in the <data-delivery-info> root element which:

1)	shall include a <data-identifier> element set to the identity of the stored data which is requested to be managed;

2)	shall include a <operation> element set to the operation to be performed such as to update, refresh, or delete the stored data; and

3)	may include an <application-data> element set to the application data needed to be updated if the operation to be performed is to update the stored data; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.12.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-mgt-req> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <data-storage-mgt-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD data storage query request operation;

ii)	shall include a <data-identifier> element set to the identity of the stored data which is queried; and

iii)	may include a <application-data> element set to the application data managed according to the operation requested on the stored data; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.12.3	SDDM client CoAP procedure

In order to request an SEALDD data storage management request to the SDDM-S, the SDDM-C shall send:

a)	a CoAP PUT request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14] when it needs to request update of the stored data; or

b)	a CoAP DELETE request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14] when it needs to request delete of the stored data.

In the CoAP PUT request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.3.1 with the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-mgt-req";

c)	shall include a "DataStorageMgtRequest" object:

1)	shall include a "dataIdentifier" attribute set to the identity of the stored data which is requested to be managed;

2)	shall include an "applicationData" attribute set to the application data needed to be stored; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

In the CoAP DELETE request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.3.1 with the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-mgt-req";

c)	shall include a "DataStorageMgtRequest" object:

1)	shall include a "dataIdentifier" attribute set to the identity of the stored data which is requested to be managed; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

#### 7.2.12.4	SDDM server CoAP procedure

Upon receiving a CoAP PUT request or a CoAP DELETE request where the CoAP URI of the CoAP PUT request or the CoAP DELETE request identifies the resource to be updated as specified in clause A.4.3.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-mgt-req", and

b)	a "DataStorageMgtRequest" object;

the SDDM-S shall generate either a CoAP PUT response or a CoAP DELETE response according to IETF RFC 7252 [14]. In either the CoAP PUT response message or the CoAP DELETE message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-mgt-req";

b)	if the received message is a CoAP PUT request:

1)	shall attempt to update the SDDM data storage resource pointed at by the CoAP URI with the content of "DataStorageMgtRequest" object received in the CoAP PUT request and:

i)	if successfully updated, shall use the CoAP PUT 2.04 (Changed) response message; or

ii)	otherwise, shall include an error response in the CoAP PUT response as specified in clause A.4.3.2.2.3.2; and

iii)	shall send the CoAP PUT response towards the SDDM-C; or

b)	if the received message is a CoAP DELETE request:

1)	shall attempt to release the SDDM data storage resource pointed at by the CoAP URI with the content of "DataStorageMgtRequest" object received in the CoAP DELETE request and:

i)	if successfully created, shall use the CoAP DELETE 2.02 (Deleted) response message; or

ii)	otherwise, shall include an error response in the CoAP DELETE response as specified in clause A.3.2.2.2.3.2; and

iii)	shall send the CoAP DELETE response towards the SDDM-C.

### 7.2.13	SEALDD server relocation procedure

The SDDM-S can be relocated because of either SDDM-C mobility or SDDM-S load re-balancing (see 3GPP TS 23.433 [2] clause 9.6).

NOTE 1:	SEALDD server relocation is specified in 3GPP TS 29.548 [9] as SDD_DDContext service.

In an edge data network (EDN), the SDDM-C can use edge applications over 3GPP services (see clause 5) to discover the new SDDM-S. The SDDM-C can obtain the new SDDM-S address by using the SEALDD server discovery and selection procedure described in clause 7.2.7.

NOTE 2:	The VAL server acts as an EAS in an EDN and it registers to the EES with the associated SDDM-S address as EAS endpoint in the EAS profile (see 3GPP TS 23.433 [2] clause 9.6.2.2). The SDDM-C mobility triggers the execution of an application context relocation (ACR) procedure or the VAL server triggers ACR due to load re-balancing. The Eees_ACREvents service and the Eees_AppContextRelocation service are specified in 3GPP TS 24.558 [8] and 3GPP TS 29.558 [10].

In a non EDN, the SDDM-C can obtain the new SDDM-S address from the VAL client or from the NAS. After that the SDDM-C can establish a new SEALDD communication channel including the old communication channel information. The SDDM-S can discover an equivalent SDDM-S (e.g. using DNS query mechanism).

NOTE 3:	The VAL client can use DNS query mechanism or vertical application (e.g. V2X) layer signalling mechanism to obtain the new SDDM-S address. The VAL client can provide the address information to the SDDM-C.

NOTE 4:	DNS query mechanism and vertical application layer signalling mechanism are out of scope of the present document.

NOTE 5:	The SDDM-C and vertical applications can receive one or more EAS rediscovery indication(s) from the NAS as specified in 3GPP TS 24.501 [5] and 3GPP TS 24.008 [4].

At SEALDD server relocation procedure the old SDDM-S may stop the downlink data transmission towards the SDDM-C before pushing the SEALDD context to the new SDDM-S in order to allow for service continuity on a packet-level granularity. In this case, content breakpoint information is provided by the old SDDM-S to the new SDDM-C during the data delivery (DD) context push procedure (see 3GPP TS 29.548 [9]). When the SDDM-C connects to the new SDDM-S, the new SDDM-S sends downlink traffic to the SDDM-C using the sequence number of the last transmitted data packet before the downlink data transmission stopped.

NOTE 6:	The SDD_DDContext API supports the provisioning of the content breakpoint information, which includes sequence number (e.g., TCP packet sequence number) of the last transmitted data packet, during the DD context push procedure is as specified in 3GPP TS 29.548 [9].

### 7.2.14	SEALDD enabled data transmission quality measurement subscription procedure

#### 7.2.14.1	SDDM client HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <measurements-subscription-req> element included in the <data-delivery-info> root element;

the SDDM-C:

a)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <measurements-subscription-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD data transmission quality measurement subscription request operation; and

ii)	may include a <expiry-time> element specifying the expiration time of the subscription. This element shall be included when the SEALDD data transmission quality measurement subscription request operation is "success", otherwise this element shall be ignored by the SDDM-S; and

b)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.14.2	SDDM server HTTP procedure

The SDDM-S sends an SEALDD data transmission quality measurement subscription request when it needs to request to carry out data transmission quality measurement towards an SDDM-C, the SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C.

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <measurements-subscription-req> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-flow-id> element set to the identity of the SDDM flow used by the SDDM-C and SDDM-S;

2)	shall include a <measurement-requirement-list> element specifying measurement requirement information. In the <measurement-requirement-list> element, the SDDM-S:

i)	shall include a <measurement-id> child element set to measurement identifiers, e.g. latency, bitrate, jitter;

ii)	may include a <reporting-frequency> child element set to reporting frequency of measurement results (e.g. "periodic", "now"). If not present, it implies periodic reporting;

iii)	may include a <reporting-periodicity> child element set to the reporting periodicity if the reporting frequency is periodic. This child element shall be included when the reporting frequency of a measurement identifier is "periodic";

iv)	may include a <measurement-window> child element set to the measurement period window for transmission quality measurements;

v)	may include a <expiry-time> child element set to the expiration time of the measurement;

vi)	may include a <sealdd-policy> child element specifying quality guarantee policies associated with the SEALDD connection. In the <sealdd-policy> element, the SDDM-S:

A)	shall include a <quality-guarantee-policy> child element set to the measurement threshold to be measured for the quality guarantee; and

B)	may include an <anyExt> element containing a <non-3gpp-access-policy> child element set to the non-3GPP access measurement policy, i.e., "WLAN SSID", "WLAN BSSID" or "LOCATION_BASED" measurement; and

vii)	may include a <reporting-criteria> child element set to the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criterion of more than one criterion is specified.

3)	may include a <measurement-conditions> element set to the temporal conditions, spatial conditions or both; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.14.3	SDDM client CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.3.2.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-subscription-req", and

b)	a "MeasurementsSubscriptionRequest" object;

the SDDM-C shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-subscription-res";

b)	shall attempt to create the SDDM data transmission quality measurement resource pointed at by the CoAP URI with the content of "MeasurementsSubscriptionRequest" object received in the request and:

1)	if successfully created, shall include a "MeasurementsSubscriptionResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success"; and

ii)	may include an "expiryTime" attribute specifying the expiration time of the subscription; or

2)	otherwise, shall include a "MeasurementsSubscriptionResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-S.

#### 7.2.14.4	SDDM server CoAP procedure

In order to request an SEALDD data transmission quality measurement subscription establishment to the SDDM-C, the SDDM-S shall send a CoAP POST request message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-S:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.2.1 with the "apiRoot" set to the SDDM-C URI:

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-subscription-req";

c)	shall include a "MeasurementsSubscriptionRequest" object:

1)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

2)	shall include a "measurementId" attribute set to the measurement identifiers, e.g. latency, bitrate, jitter;

3)	may include a "reportingFrequency" attribute set to reporting frequency of measurement results (e.g. "periodic", "now"). If not present, it implies periodic reporting;

4)	may include a "reportingPeriodicity" attribute set to the reporting periodicity if the reporting frequency is periodic. This attribute shall be included when the reporting frequency of a measurement identifier is "periodic";

5)	may include a "measurementWindow" attribute set to the measurement period window for transmission quality measurements;

6)	may include an "expiryTime" attribute set to the expiration time of the measurement;

7)	may include a "sealddPolicy" object specifying quality guarantee policies associated with the SEALDD connection;

8)	may include a "reportingCriteria" attribute set to the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criterion of more than one criteria is specified;

9)	may include a "measurementConditions" object specifying the temporal conditions, spatial conditions or both; and

10)	may include a "non3gppAccessPolicy" attribute specifying the non-3GPP access measurement policy, i.e., "WLAN SSID", "WLAN BSSID" or "LOCATION_BASED" measurement; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

### 7.2.15	SEALDD enabled data transmission quality measurement notification procedure

#### 7.2.15.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD data transmission quality measurement notification when it needs to provide the SDDM-S with transmission quality measurements. The SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <measurements-notification> element in the <data-delivery-info> root element which:

1)	shall include a <measurement-requirement-notify-list> element specifying measurement requirement information. In the <measurement-requirement-notify-list> element, the SDDM-S:

i)	shall include a <measurement-id> child element set to measurement identifiers, e.g. latency, bitrate, jitter;

ii)	may include an <identity-measurements> child element which shall include one of the following elements:

A)	a <VAL-ue-id-list> element with one or more <VAL-ue-id> child elements set to the identities of the VAL UEs for whom SEALDD measurement applies. For multiple VAL UEs reporting granularity set to individual UE, the associated measurement values are for individual VAL UE; or

B)	a <VAL-group-id> element set to the identity of the VAL group for whom SEALDD measurement applies for which the associated measurement values are aggregation for all VAL UEs or the VAL UE group;

iii)	may include a <average-measurement-value> child element set to the average measurement value of measurement results;

iv)	may include a <minimum-measurement-value> child element set to the minimum measurement value of measurement results;

v)	may include a <maximum-measurement-value> child element set to the maximum measurement value of measurement results;

vi)	may include a <standard-deviation-measurement-value> child element set to standard deviation measurement value of measurement results;

vii)	may include a <kpercentile-measurement-value> child element set to the kpercentile measurement value of measurement results;

viii)	may include a <measurement-period> child element set to the measurement period;

ix)	may include a <timestamp> child element set to the timestamp of measurement results; and

x)	may include an <anyExt> element containing a <non-3gpp-access-measurement-list> child element set to the measurements of the non-3GPP access. In the <non-3gpp-access-measurement-list> element the SDDM-C shall include one or more <non-3gpp-access-measurement> child elements which:

A)	may contain a <measured-non-3gpp-access> element representing an identity of the measured non-3GPP access and shall include an <ssid> or <bssid> element; and

B)	shall contain a <signal-strength-values> element containing one or more <measured-value> element.

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.15.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <measurements-notification> element included in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1 and:

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. The SDDM-S shall communicate the received data transmission quality measurement results (e.g. latency, jitter, bitrate) to the VAL server by using the SDD_TransmissionQualityMeasurement service as specified in 3GPP TS 29.548 [9]; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.15.3	SDDM client CoAP procedure

Upon reception of a CoAP FETCH request message containing:

a)	an "observe" option set to the value "0" (register);

b)	an Accept option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-notification";

c)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-subscription-req"; and

d)	a "MeasurementsSubscriptionRequest" object,

the SDDM-C shall provide an SALDD data transmission quality measurement to the SDDM-S, the SDDM-C shall send a CoAP FETCH response message to the SDDM-S according to procedures specified in IETF RFC 8132 [17] in response to a CoAP FETCH request message used to observe an an SDDM data storage resource as specified in clause A.3.2.2.2.3.2. In the CoAP FETCH response, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.3.2.1 with the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-notification";

c)	shall include a "MeasurementNotification" object:

1)	shall include a "measurementId" attribute set to the measurement identifiers, e.g. latency, bitrate, jitter;

2)	may include a "valUeIdList" attribute set to the identities of the VAL UEs for whom SEALDD measurement applies. For a single VAL UE, this attribute can be omitted and the associated measurement values are for the single VAL UE. For multiple VAL UEs with reporting granularity set to individual UE, the associated measurement values are for individual VAL UE. For multiple VAL UEs with reporting granularity set to VAL UE group or all VAL UEs, the associated measurement values are aggregation for all VAL UEs or the VAL UE group;

3)	may include an "averageMeasurementValue" attribute set to the average measurement value of measurement results;

4)	may include a "maximumMeasurementValue" attribute set to the maximum measurement value of measurement results;

5)	may include a "minimumMeasurementValue" attribute set to the minimum measurement value of measurement results;

6)	may include a "standardDeviationMeasurementValue" attribute set to the standard deviation measurement value of measurement results;

7)	may include a "kPercentileMeasurementValue" attribute set to the kpercentile measurement value of measurement results;

8)	may include a "measurementPeriod" attribute set to the measurement period;

9)	may include a "timeStamp" attribute set to the timestamp of measurement results; and

10)	may include a "non3gppAccessMeasurements" attribute to indicate measurement results for the requested non-3GPP access measurement policies; and

d)	shall send the CoAP FETCH response towards the SDDM-S.

#### 7.2.15.4	SDDM server CoAP procedure

In order for an SDDM-S to get data transmission quality measurement of an SDDM regular data transmission connection, the SDDM-S shall first send a CoAP FETCH request message (as specified in IETF RFC 8132 [17]) used to observe an SDDM data storage resource as specified in clause A.3.2.2.2.3.2, and containing:

a)	a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.2.1;

b)	an "observe" option set to the value "0" (register);

c)	an Accept option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-notification";

d)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-subscription-req", and

e)	a "MeasurementsSubscriptionRequest" object;

1)	shall include a "sealddFlowiId" data type set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic; and

2)	shall include a "measurementId" attribute set to the measurement identifiers, e.g. latency, bitrate, jitter.

Upon receiving a CoAP FETCH 2.05 (Content) response containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-notification", and

b)	a "MeasurementNotification" object;

the SDDM-S shall communicate the received data transmission quality measurement results (e.g. latency, jitter, bitrate) to the VAL server by using the SDD_TransmissionQualityMeasurement service as specified in 3GPP TS 29.548 [9].

In order for an SDDM-S to stop data transmission quality measurement of an SDDM regular data transmission connection, the SDDM-S shall send a CoAP FETCH request message as specified in clause A.3.2.2.2.3.2, and containing:

a)	an "observe" option set to the value "1" (deregister);

b)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-subscription-req", and

c)	a "MeasurementsSubscriptionRequest" object;

1)	shall include a "sealddFlowiId" data type set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic; and

2)	shall include a "measurementId" attribute set to the measurement identifiers, e.g. latency, bitrate, jitter.

### 7.2.16	SEALDD enabled data transmission quality guarantee procedure

#### 7.2.16.1	SDDM client HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <tx-quality-management-req> element included in the <data-delivery-info> root element;

the SDDM-C:

a)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <tx-quality-management-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD data transmission quality management request operation; and

b)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.16.2	SDDM server HTTP procedure

The SDDM-S sends an SEALDD data transmission quality management request when it needs to request to data transmission quality management towards an SDDM-C, the SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <tx-quality-management-req> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-flow-id> element set to the identity of the SDDM flow used by the SDDM-C and SDDM-S;

2)	shall include a <tx-quality-management-action> element set to the data transmission quality guarantee action (e.g. redundant transmission path, re-establish transmission path, switch to backup transmission path) or optimization action (back to single transmission path, transmission parameter adjustment) that was triggered by an event (e.g. measurement threshold); and

3)	if the <tx-quality-management-action> element indicates a transmission parameter adjustment, shall include an <anyExt> element that:

i)	shall contain a <bat-offset-ul> element specifying the BAT offset for the uplink data; and

ii)	if the <bat-offset-ul> element is included, may contain a <periodicity-ul> element specifying the adjusted periodicity for the uplink data; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.16.3	SDDM client CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.3.3.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=tx-quality-mgt-req"; and

b)	a "TxQualityManagementRequest" object;

the SDDM-C shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=tx-quality-mgt-res";

b)	shall attempt to create the SDDM data transmission quality guarantee resource pointed at by the CoAP URI with the content of "TxQualityManagementResquest" object received in the request and:

1)	if successfully created, shall include a "TxQualityManagementResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success"; or

2)	otherwise, shall include a "TxQualityManagementResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-S.

#### 7.2.16.4	SDDM server CoAP procedure

In order to request an SEALDD data transmission quality guarantee establishment to the SDDM-C, the SDDM-S shall send a CoAP POST request message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-S:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.3.1 with the "apiRoot" set to the SDDM-C URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=tx-quality-mgt-req";

c)	shall include a "TxQualityManagementRequest" object:

1)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic; and

2)	shall include a "txQualityManagementAction" attribute set to the data transmission quality guarantee action (e.g. redundant transmission path, re-establish transmission path, switch to backup transmission path) or optimization action (back to single transmission path, transmission parameter adjustment) that was triggered by an event (e.g. measurement threshold);

d)	if the "txQualityManagementAction" attribute indicates a transmission parameter adjustment:

1)	shall include a "batOffsetUl" attribute specifying the BAT offset for Uplink data; and

2)	may include a "periodicityUl" attribute specifying the adjusted periodicity for Uplink data; and

e)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

### 7.2.17	SEALDD enabled URLLC transmission connection deletion based on policy procedure

#### 7.2.17.1	SDDM client HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-release-req> element included in the <data-delivery-info> root element;

the SDDM-C:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to request signalling transmission connection release, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

2)	shall support handling an HTTP POST request from an SDDM-S according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-release-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD URLLC transmission connection release request operation. If the result is "failure", in the <result> element, the SDDM-C may include a <cause> child element specifying the cause of the failure of the operation, e.g. VAL client error; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.17.2	SDDM server HTTP procedure

The SDDM-S sends an SEALDD URLLC transmission connection release request when it needs to release an established SEALDD URLLC transmission connection towards an SDDM-C, the SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-release-req> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-client-identity> element set to the identity of the SDDM-C; and

2)	shall include a <sealdd-flow-id> element set to the identity of the SEALDD flow used by the SDDM-C and SDDM-S to identify the application traffic; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.17.3	SDDM client CoAP procedure

Upon receiving a CoAP DELETE request where the CoAP URI of the CoAP DELETE request identifies the release resource as specified in clause A.3.5.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-release-req", and

b)	a "URLLCReleasetRequest" object;

the SDDM-C shall generate a CoAP DELETE response according to IETF RFC 7252 [14]. In the CoAP DELETE response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-release-req";

b)	shall attempt to release the SDDM URLLC transmission connection resource pointed at by the CoAP URI with the content of "URLLCReleaseRequest" object received in the request and:

1)	if successfully created, shall use the CoAP DELETE 2.02 (Deleted) response message; or

2)	otherwise, shall include an error response in the CoAP DELETE response as specified in clause A.4.2.2.2.3.3; and

c)	shall send the CoAP DELETE response towards the SDDM-S.

#### 7.2.17.4	SDDM server CoAP procedure

In order to request the release of an SEALDD URLLC transmission connection to the SDDM-C, the SDDM-S shall send a CoAP DELETE request message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP DELETE request, the SDDM-S:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.5.1 with;

1)	the "apiRoot" set to the SDDM-C URI; and

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-release-req";

c)	shall include a "URLLCReleaseRequest" object:

1)	shall include a "sealClientId" attribute set to the identity of the SDDM-C;

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-S and SDDM-C to identify the application traffic; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

### 7.2.18	SEALDD enabled URLLC transmission connection establishment based on policy procedure

#### 7.2.18.1	SDDM client HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-establishment-req> element included in the <data-delivery-info> root element;

the SDDM-C:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1 and:

1)	if the identity of the sender of the received HTTP POST request is not authorized to request URLCC transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

2)	shall support handling an HTTP POST request from an SDDM-S according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <URLLC-establishment-rsp> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD URLLC transmission connection establishment request operation. If the result is "failure", in the <result> element, the SDDM-C may include a <cause> child element specifying the cause of the failure of the operation, e.g. VAL client error;

ii)	may include a <traffic-descriptor-info> element specifying the information of the traffic of the redundant SEALDD transmission connection. In the <traffic-descriptor-info> element, the SDDM-C may include:

A)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

B)	a <port-number>child element specifying the identity of the port number of the traffic;

C)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic; and

D)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

iii)	may include an <anyExt> element containing a <traffic-transmission-bandwidth> element indicating suggested traffic transmission bandwidth to be used by SDDM-C; and

iv)	may include an <anyExt> element containing a <bat-period-adapt-cap> element to indicate a BAT and periodicity adaptation capability or a <transmission-assist-info> element specifying a transmission assistance information. In the <transmission-assist-info> element, the SDDM-C:

A)	a <bat> child element specifying the arrival time of the first packet of the data burst;

B)	a <periodicity> child element specifying the time period between the start of two bursts;

C)	if the <bat> element is included, may include a <bat-window> child element containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

D)	if the <bat>, <bat-window> and <periodicity>elements are included, may include a <periodicity-range> child element specifying the periodicity range. In the <periodicity-range> element the SDDM-C shall include either a <lower-bound> child element set to the lower bound of the periodicity and an <upper-bound> child element set to the upper bound of the periodicity of the start two bursts or a <periodicity-value-list> child element with one or more <periodicity-value> child elements set to the acceptable periodicity value; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.18.2	SDDM server HTTP procedure

The SDDM-S sends an SEALDD URLLC transmission connection establishment request when it needs to request an SEALDD URLLC transmission connection establishment, the SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <URLLC-establishment-req> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-client-identity> element set to the identity of the SDDM-C;

2)	shall include a <sealdd-flow-id> element set to the identity of the SDDM flow used by the SDDM-S and SDDM-C to identify the application traffic;

3)	may include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and receiving the request;

4)	may include a <server-id> element set to the information of the endpoint of the selected VAL server to which the SDDM URLLC transmission connection establishment request has to be sent;

5)	may include a <VAL-service-id> element set to the VAL service identity of the vertical application; and

6)	may include a <traffic-descriptor-info> element specifying the information of the traffic of the redundant SEALDD transmission connection. In the <traffic-descriptor-info> element, the SDDM-S may include:

i)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

ii)	a <port-number> child element specifying the identity of the port number of the traffic;

iii)	a <URL> child element specifying the address of a given unique resource on the Web for the traffic;

iv)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.18.3	SDDM client CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.3.5.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-establishment-req", and

b)	a "URLLCEstablishmentRequest" object;

the SDDM-C shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-establishment-res";

b)	shall attempt to create the URLLC transmission connection resource pointed at by the CoAP URI with the content of "EstablishmentRequest" object received in the request and:

1)	if successfully created, shall include a "URLLCEstablishmentResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success";

ii)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

iii)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

iv)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

v)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; and

vi)	may include a "batPeriodAdaptCap" attribute to indicate a BAT and periodicity adaptation capability or a "transmisAssistInfo" attribute specifying a transmission assistance information. In the "transmisAssistInfo" attribute the SDDM-C:

A)	shall include at least one of the following attributes:

I)	a "bat" attribute specifying the arrival time of the first packet of the data burst; and

II)	a "periodicity" attribute specifying the time period between the start of two bursts;

B)	if the "bat" attribute is included, may include a "batWindow" attribute containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

C)	if the "bat", "batWindow" and "periodicity" attributes are included, may include a "periodRange" attribute specifying the periodicity range. In the "periodRange" attribute the SDDM-C shall include:

I)	a "lowerBound" attribute set to the lower bound of the periodicity and an "upperBound" attribute set to the upper bound of the periodicity of the start two bursts; or

II)	a "periodicityValues" attribute set to the acceptable periodicity values; or

2)	otherwise, shall include a "URLLCEstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response as specified in clause A.3.2.2.2.3.1; and

c)	shall send the CoAP POST response towards the SDDM-C.

#### 7.2.18.4	SDDM server CoAP procedure

In order to request an SEADD URLLC transmission connection establishment to the SDDM-C, the SDDM-S shall send a CoAP POST request message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-S:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.5.1 with;

1)	the "apiRoot" set to the SDDM-C URI; and

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-establishment-req";

c)	shall include a "URLLCEstablishmentRequest" object:

1)	shall include a "sealClientId" attribute set of the identity of the SDDM-C;

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-S and SDDM-S to identify the application traffic;

3)	shall include a "valTgtUe" attribute set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and receiving the request;

4)	may include a "serverId" attribute set to the information of the VAL server;

5)	may include a "valServiceId" attribute set to the identity of the VAL service of the vertical application;

6)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

7)	may include a"portNumber" attribute specifying the identity of the port number of the traffic;

8)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic;

9)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; and

c)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

### 7.2.19	SEALDD enabled connection status reporting configuration subscription procedure

#### 7.2.19.1	SDDM client HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <connection-status-configuration-req> element included within <anyExt> element in the <data-delivery-info> root element;

the SDDM-C:

a)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [18]. In the HTTP 200 (OK) response message, the SDDM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <connection-status-configuration-rsp> element within <anyExt> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD connection status reporting configuration request operation; and

b)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.19.2	SDDM server HTTP procedure

The SDDM-S sends an SEALDD connection status reporting configuration request when it needs to request connection status reporting configuration information from the SDDM-C. The SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [18]. In the HTTP POST request message, the SDDM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [12];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <connection-status-configuration-req> element within <anyExt> element in the <data-delivery-info> root element which:

1)	shall contain a <sealdd-flow-id> element set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

2)	may contain a <reporting-mode> element set to the mode of the reporting, i.e., periodic or event triggered. If the reporting mode is set to "periodic", the <reporting-mode> element:

i)	may contain a <reporting-interval> child element set to the reporting interval of the measurement results;

3)	may contain a <reporting-priority> element set to the priority of SEALDD client connection status for the requested SEALDD flow ID; and

4)	may contain a <non-3gpp-access-policy> element set to the non-3GPP access measurement policy, i.e., "WLAN SSID", "WLAN BSSID" or "LOCATION_BASED" measurement.

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.19.3	SDDM client CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the resource as specified in clause A.3.4.1, and the CoAP POST request contains:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-configuration-req"; and

b)	a "ConnectionStatusConfigurationRequest" object,

the SDDM-C shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-configuration-res";

b)	shall attempt to create the SDDM connection status reporting configuration resource pointed at by the CoAP URI with the content of "ConnectionStatusConfigurationRequest" object received in the request and:

1)	if successfully created, shall include a "ConnectionStatusConfigurationResponse" object in the CoAP POST 2.01 (Created) response message and:

i)	shall include a "result" attribute set to "success"; or

2)	otherwise, shall include a "ConnectionStatusConfigurationResponse" object with a "result" attribute set to "failure"; and

c)	shall send the CoAP POST response towards the SDDM-S.

#### 7.2.19.4	SDDM server CoAP procedure

In order to request an SEALDD connection status reporting configuration from the SDDM-C, the SDDM-S shall send a CoAP POST request message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-S:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.4.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-configuration-req";

c)	shall include a "ConnectionStatusConfigurationRequest" object:

1)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

2)	may include a "reportingMode" attribute set to the mode of the reporting;

3)	if the reporting mode is included and indicates a periodic reporting mode, may include a "reportingInterval" attribute set to the reporting interval of the measurement results;

4)	may include a "reportingPriority" attribute set to the priority of SEALDD client connection status for the requested SEALDD flow ID; and

5)	may include a "non3gppAccessPolicy" attribute set to the non-3GPP access measurement policy, i.e., "WLAN SSID", "WLAN BSSID" or "LOCATION_BASED" measurement; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

### 7.2.20	Void

### 7.2.21	SEALDD enabled connection status reporting configuration notification procedure

#### 7.2.21.1	SDDM client HTTP procedure

The SDDM-C sends a SEALDD connection status reporting notification when it needs to provide to the SDDM-S connection status reporting configuration. The SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <connection-status-notification> element within <anyExt> element in the <data-delivery-info> root element which:

1)	shall include a <client-connection-status> element specifying the status of the VAL UE, i.e. "reachable", "unreachable", or "sleeping";

2)	shall include a <access-usage> element indicating which access (3GPP or non-3GPP) is used for the SEALDD-UU data transmission; and

3)	may include a <non-3gpp-access-measurement-list> element set to the measurements of the non-3GPP access. In the <non-3gpp-access-measurement-list> element the SDDM-C shall include one or more <non-3gpp-access-measurement> child elements which:

i)	may contain a <measured-non-3gpp-access> element representing an identity of the measured non-3GPP access and shall include an <ssid> or <bssid> element; and

ii)	shall contain a <signal-strength-value-list> element containing one or more <measured-value> elements.

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.21.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <connection-status-notification> element included within <anyExt> element in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1 and:

1)	if the identity of the sender of the received HTTP POST request is not authorized user, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. The SDDM-S shall communicate to the VAL server the received connection status results by using the SDD_Transmission service as specified in 3GPP TS 29.548 [9]; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.21.3	SDDM client CoAP procedure

Upon reception of a CoAP FETCH request message containing:

a)	an "observe" option set to the value "0" (register);

b)	an Accept option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-notification";

c)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-config-subsc"; and

d)	a "ConnectionStatusConfigurationSubscription" object,

the SDDM-C shall provide an SEALDD connection status reporting notification to the SDDM-S within a CoAP FETCH 2.05 (Content) response (as specified in IETF RFC 8132 [17]). In the CoAP FETCH 2.05 (Content) response the SDDM-C:

a)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-notification";

b)	shall include a "ConnectionStatusNotification" object containing:

1)	a "clientConnectionStatus" attribute set to the status of the VAL UE, i.e. reachable, unreachable, or sleeping;

2)	an "accessUsage" attribute indicating which access (3GPP or non-3GPP) is used for the SEALDD-UU data transmission; and

3)	a "non3gppAccessMeasurements" attribute to indicate measurement results for the requested non-3GPP access measurement policies; and

c)	shall send CoAP FETCH 2.05 (Content) response towards the SDDM-S.

#### 7.2.21.4	SDDM server CoAP procedure

To get an SEALDD connection status reporting notification from the SDDM-C, the SDDM-S shall generate a CoAP FETCH request (as specified in IETF RFC 8132 [17]) message containing:

a)	a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.4.1 with the "apiRoot" set to the SDDM-C URI;

b)	an "observe" option set to the value "0" (register);

c)	an Accept option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-notification";

d)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-config-subsc"; and

e)	a "ConnectionStatusConfigurationSubscription" object.

Upon receiving a CoAP FETCH 2.05 (Content) response containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-notification"; and

b)	a "ConnectionStatusNotification" object,

the SDDM-S shall communicate the received connection status results to the VAL server by using the SDD_Transmission service as specified in 3GPP TS 29.548 [9].

To stop getting an SEALDD connection status reporting notifications from the SDDM-C, the SDDM-S shall generate a CoAP FETCH request (as specified in IETF RFC 8132 [17]) message containing:

a)	a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.4.1 with the "apiRoot" set to the SDDM-C URI;

b)	an "observe" option set to the value "1" (deregister);

c)	a Content-Format option set to application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-config-subsc", and

d)	a "ConnectionStatusConfigurationSubscription" object.

### 7.2.22	SEALDD enabled XR data transmission trigger procedure

#### 7.2.22.1	SDDM client HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <xr-trigger-req> element included within <anyExt> element in the <data-delivery-info> root element;

the SDDM-C:

a)	shall generate an HTTP 200 (OK) response message to the SDDM-S according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-C:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <xr-trigger-rsp> element within <anyExt> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD XR transmission connection trigger request operation; and

b)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

#### 7.2.22.2	SDDM server HTTP procedure

The SDDM-S sends an SEALDD XR transmission connection trigger request when it needs to trigger XR transmission connection operation towards an SDDM-C. The SDDM-S shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-C.

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with an <xr-trigger-req> element within <anyExt> element in the <data-delivery-info> root element which:

1)	shall include a <requestor-id> element set to "sealddserver";

2)	shall include a <VAL-ue-id-list> element with one or more <VAL-ue-id> child elements set to the identities of the VAL UEs for whom SEALDD XR transmission connection inform applies;

3)	shall include a <operation> element set to the action for UE-to-UE direct communication (i.e. establishment, release); and

4)	may include a <VAL-service-id> element set to the VAL service identity of the vertical application; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.22.3	SDDM client CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request is set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.6.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-trigger-req", and

b)	a "XRTriggerRequest" object;

the SDDM-C shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-trigger-res";

b)	shall attempt to create the SDD multi-modal transmission connection resource pointed at by the CoAP URI with the content of "XRTriggerRequest" object received in the request and:

1)	if successfully created, shall include a "XRTriggerResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success"; or

2)	otherwise, shall include a "XRTriggerResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-S.

#### 7.2.22.4	SDDM server CoAP procedure

In order to trigger an SEALDD XR transmission connection operation to the SDDM-C, the SDDM-S shall send a CoAP POST request message to the SDDM-C according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-S:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-C as specified in clause A.3.6.1 with the "apiRoot" set to the SDDM-C URI:

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-trigger-req";

c)	shall include a "XRTriggerRequest" object which:

1)	shall include a "requestorId" attribute set to "SEALDDSERVER";

2)	shall include a "valUeIds" attribute set to the identities of the VAL UEs for whom SEALDD XR transmission connection inform applies;

3)	shall include an "operation" attribute set to the action for UE-to-UE direct communication (i.e. establishment, release); and

4)	may include a "valServiceId" attribute set to the VAL service identity of the vertical; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

### 7.2.23	SEALDD enabled XR data transmission inform procedure

#### 7.2.23.1	SDDM client HTTP procedure

The SDDM-C sends a SEALDD XR transmission connection inform request when it needs to inform XR transmission connection status between two UEs to the SDDM-S. The SDDM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request message, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <xr-inform-req> element within <anyExt> element in the <data-delivery-info> root element which:

1)	shall include a <requestor-id> element set to "sealddclient";

2)	shall include a <VAL-ue-id-list> element with one or more <VAL-ue-id> child elements set to the identities of the VAL UEs for whom SEALDD XR transmission connection inform applies;

3)	shall include a <status> element set to the UE-to-UE direct communication status (i.e. established, released); and

4)	may include a <VAL-service-id> element set to the VAL service identity of the vertical application; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [16].

#### 7.2.23.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <xr-inform-req> element included within <anyExt> element in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1 and:

1)	if the identity of the sender of the received HTTP POST request is not authorized user, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; and

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21] In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <xr-inform-rsp> element within <anyExt> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD XR transmission connection inform request operation; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [16].

#### 7.2.23.3	SDDM client CoAP procedure

In order to inform XR transmission connection status between two UEs to the SDDM-S, the SDDM-C shall send a CoAP POST request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.4.1 with the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-inform-req";

c)	shall include a "XRInformRequest" object which:

1)	shall include a "requestorId" attribute set to "SEALDDCLIENT";

2)	shall include a "valUeIds" attribute set to the identities of the VAL UEs for whom SEALDD XR transmission connection inform applies;

3)	shall include a "status" attribute set to the UE-to-UE direct communication status (i.e. established, released); and

4)	may include a "valServiceId" attribute set to the VAL service identity of the vertical application; and

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

#### 7.2.23.4	SDDM server CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request is set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.4.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-inform-req", and

b)	a "XRInformRequest" object;

the SDDM-S shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-inform-rsp";

b)	shall attempt to create the SDD multi-modal transmission connection resource pointed at by the CoAP URI with the content of "XRInformRequest" object received in the request and:

1)	if successfully created, shall include a "XRInformResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success"; or

2)	otherwise, shall include a "XRInformResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-C.

### 7.2.24	SEALDD enabled multi-modal data transmission establishment procedure

#### 7.2.24.1	SDDM client HTTP procedure

The SDDM-C sends an SEALDD multi-modal transmission connection establishment request when it needs to request an SEALDD multi-modal transmission connection establishment, the SDDM-C generate an HTTP POST request according to procedures specified in IETF RFC 9110 [21]. In the HTTP POST request, the SDDM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SDDM-S;

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <XR-establishment-req> element within <anyExt> element in the <data-delivery-info> root element which:

1)	shall include a <sealdd-client-identity> element set to the identity of the SDDM-C;

2)	shall include a <sealdd-flows-info> element set to the information of the SEALDD flows used by the SDDM-S and SDDM-C to identify the application traffic. The <sealdd-flow-info>:

i)	shall include one or more <sealdd-flow-id> child elements set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the multi-modal traffic; and

ii)	may include a <traffic-descriptor-info> child element specifying the information of traffic descriptors for the multi-modal traffic. In the <traffic-descriptor-info>, the SDDM-C may include:

A)	a <user-plane-address> child element specifying the identity of the IP address of the traffic;

B)	a <port-number> child element specifying the identity of the port number of the traffic;

C)		a <URL> child element specifying the address of a given unique resource on the Web for the traffic; and

D)	a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic;

3)	may include an <identity> element with a <VAL-user-id> child element set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request;

4)	may include a <server-id> element set to the information of the endpoint of the selected VAL server to which the SEALDD multi-modal transmission connection establishment request has to be sent; and

5)	may include a <VAL-service-id> element set to the VAL service identity of the vertical application; and

d)	shall send the HTTP POST request as specified in IETF RFC 9110 [21].

#### 7.2.24.2	SDDM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-data-delivery-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

c)	an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <XR-establishment-req> element included within <anyExt> element in the <data-delivery-info> root element;

the SDDM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 7.2.1.1 and:

1)	if the identity of the sender of the received HTTP POST request is not authorized to request SEALDD multi-modal transmission connection establishment, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps; or

2)	shall support handling an HTTP POST request from an SDDM-C according to procedures specified in IETF RFC 4825 [12] "POST Handling";

b)	shall generate an HTTP 200 (OK) response message to the SDDM-C according to IETF RFC 9110 [21]. In the HTTP 200 (OK) response message, the SDDM-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-data-delivery-info+xml"; and

2)	shall include an application/vnd.3gpp.seal-data-delivery-info+xml MIME body with a <xr-establishment-rsp> element within <anyExt> element in the <data-delivery-info> root element which:

i)	shall include a <result> element set to "success" or "failure" indicating success or failure of the SEALDD multi-modal transmission connection establishment request operation. If the result is "failure", in the <result> element, the SDDM-S may include a <cause> child element specifying the cause of the failure of the operation, e.g. VAL client error;

ii)	if <result> element is set to "success", the <multi-modal-sealdd-flow-info> element shall be included and set to the information of the multi modal SEALDD flow used by the SDDM-S and SDDM-C. The <multi-modal-sealdd-flow-info> shall include:

A)	a <sealdd-flows-list> child element set to list of SEALDD flows used by the SDDM-S and SDDM-C:

I)	shall include one or more <sealdd-flow-id> child elements set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the multi-modal traffic; and

II)	may include a <traffic-descriptor-info> child element specifying the information of traffic descriptors for the multi-modal traffic. The <traffic-descriptor-info> may include a <user-plane-address> child element specifying the identity of the IP address of the traffic; a <port-number> child element specifying the identity of the port number of the traffic; a <URL> child element specifying the address of a given unique resource on the Web for the traffic; and a <transport-layer-protocol> child element specifying the transport layer protocol for the traffic; and

iii)	if <result> element is set to "success", the <protocol-descriptor-info> element may be included and set to the information of the protocol of the VAL traffic. In the <protocol-descriptor-info> element, the SDDM-S:

A)	may include a <header-ext-info> child element specifying the information of the header extension, e.g. RTP extension with PDU set. This child element is only included if the payload type is RTP. The<header-ext-info> element:

I)	may include a <header-ext-type> child element set to the type of the header extension protocol (e.g, RTP, SRTP); and

II)	may include a <header-ext-id> child element set to the identity of the header extension protocol with value from 1 to 255;

B)	may include a <packetization-indication> child element set to packetization indication;

C)	may include a <payload-type> child element set to the type of the payload, e.g. RTP, SRTP; and

D)	shall include a <payload-format> child element set to the format of the payload, e.g. H.264, H.265; and

c)	shall send the HTTP 200 (OK) response message as specified in IETF RFC 9110 [21].

Editor's note [WID: XRM_Ph2_App, CR#: 0094]:	The definition of multimodalSealddFlowId is FFS.

#### 7.2.24.3	SDDM client CoAP procedure

In order to request a SEALDD multi-modal transmission connection establishment to the SDDM-S, the SDDM-C shall generate a CoAP POST request to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.4.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-req"; and

c)	shall include a "XREstablishmentRequest" object:

1)	shall include a "sealClientId" attribute set of the identity of the SDDM-C;

2)	shall include a "sealddFlowsInfos" attribute set to the list of the SDDM flow used by the SDDM-C and SDDM-S;

i)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flows used by the SDDM-C and SDDM-S to identify the application traffic;

ii)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

iii)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

iv)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

v)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic;

3)	may include a "valTgtUe" attribute set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request;

4)	may include a "serverId" attribute set to the information of the VAL server; and

5)	may include a "valServiceId" attribute set to the identity of the VAL service of the vertical application.

The SDDM-C shall send the CoAP POST request to the SDDM-S protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

Editor's note [WID: XRM_Ph2_App, CR#: 0095]:	To align with stage 2 on changing XR to multi-modal is FFS.

#### 7.2.24.4	SDDM server CoAP procedure

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.4.4.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-req"; and

b)	a "XREstablishmentRequest" object;

the SDDM-S shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response, the SDDM-S:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-res"; and

b)	shall attempt to create the SDD multi-modal transmission connection resource pointed at by the CoAP URI with the content of "EstablishmentRequest" object received in the request and:

1)	if successfully created, shall include a "XREstablishmentResponse" object in the CoAP POST 2.01 (Created) response message:

i)	shall include a "result" attribute set to "success";

ii)	shall include a "mmSealddFlow" attribute set to the information of the SDDM flow used by the SDDM-C and SDDM-S to identify the multi-modal service traffic;

A)	 shall include a "sealddFlowsInfos" attribute set to the list of SDDM flows used by the SDDM-C and SDDM-S;

shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

may include a "portNumber" attribute specifying the identity of the port number of the traffic;

may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic; and

may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic;

iii)	may include a "headerExtType" attribute specifying the header extension information;

iv)	may include a "headerExtId" attribute specifying the header extension identity;

v)	may include a "packetizationIndication" attribute specifying the packet indication information;

vi)	may include a "payloadType" attribute specifying the payload type information; and

vii)	may include a "payloadFormat" attribute specifying the payload format information; and

2)	otherwise, shall include a "XR EstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response as specified in clause A.4.4.2.2.3.1.

The SDDM-S shall send the CoAP POST response towards the SDDM-C.

Editor's note [WID: XRM_Ph2_App, CR#: 0095]:	To align with stage 2 on changing XR to multi-modal is FFS.

Editor's note [WID: XRM_Ph2_App, CR#: 0095]:	The definition of multimodalSealddFlowId is FFS.

## 7.3	Off-network procedures

The off-network procedures are out of scope of the present document in this release of the specification.

# 8	Coding

## 8.1	General

This clause specifies the coding to enable an SDDM-C and an SDDM-S to communicate.

## 8.2	Application unique ID

The AUID shall be set to the VAL service ID as specified in specific VAL service specification.

## 8.3	Structure

Editor's note [WID: SEALDD_Ph2, CR#: 0085]:	The data type L4sFeedbackCapability to support L4S mechanism will be checked to be aligned with the latest stage-2 requirements.

The data delivery management document shall conform to the XML schema described in clause 8.4.

The <data-delivery-info> element shall be the root element of the SEALDataDeliveryManagement document.

The <data-delivery-info> element shall include:

1)	<establishment-req> element;

2)	<establishment-rsp> element;

3)	<release-req> element;

4)	<release-rsp> element;

5)	<URLLC-establishment-req> element;

6)	<URLLC-establishment-rsq> element;

7)	<URLLC-release-req> element;

8)	<URLLC-release-rsp> element;

9)	<URLLC-update-req> element;

10)	<URLLC-update-rsp> element;

11)	<data-storage-creation-req> element;

12)	<data-storage-creation-rsp> element;

13)	<data-storage-reservation-req> element;

14)	<data-storage-reservation-rsp> element;

15)	<data-storage-status-notification> element;

16)	<measurements-subscription-req> element;

17)	<measurements-subscription-rsp> element;

18)	<data-storage-query-req> element;

19)	<data-storage-query-rsp> element;

20)	<data-storage-mgt-req> element;

21)	<data-storage-mgt-rsp> element;

22)	<measurements-notification> element;

23)	<identity-measurements> element;

24)	<tx-quality-management-req> element;

25)	<tx-quality-management-rsp> element; and

26)	<anyExt> element.

The <anyExt> element of the <data-delivery-info> element shall include:

1)	<connection-status-configuration-req> element;

2)	<connection-status-configuration-rsp> element;

3)	<connection-status-notification> element;

4)	<xr-trigger-req> element;

5)	<xr-trigger-rsp> element;

6)	<xr-inform-req> element;

7)	<xr-inform-rsp> element;

8)	<xr-establishment-req> element; and

9)	<xr-establishment-rsp> element.

The <establishment-req> element:

a)	shall include a <requestor-id> element;

b)	shall include a <sealdd-flow-id> element;

c)	shall include a <endpoint-id> element;

d)	may include a <server-id> element;

e)	may include a <VAL-service-id> element;

f)	may include a <sealdd-communication-lifetime> element;

g)	may include a <traffic-descriptor-info> element which shall include at least one of the following sub-elements:

1)	a <user-plane-address> element;

2)	a <port-number> element;

3)	a <URL> element; or

4)	a <transport-layer-protocol> element;

h)	may include an <identity> element; and

i)	may include an <anyExt> element that may contain:

1)	an <L4S-feedback-capability> element;

i)	shall include a <ecn-marking> element that contains the following sub-elements:

A)	shall include a <supports-ect0> element;

B)	may include a <ect0-value> element;

C)	shall include <supports-ect1> element; and

D)	may include <ect1-value> element;

ii)	shall include a <L4s-feedback> element that contains the following sub-elements:

A)	shall include a <per-packet-ce-reporting> element;

B)	 may include a <feedback-interval> element;

C)	may include a <max-feedback-frequency> element; and

D)	may include a <protocol> element; and

2)	a <bat-period-adapt-cap> element; and

3)	a <transmission-assist-info> element.

The <identity> element shall include one of the following:

a)	a <VAL-user-id> element may include a <VAL-client-id> element; or

b)	a <VAL-ue-id> element.

The <transmission-assist-info> element:

a)	shall include at least one of the following sub-elements:

1)	a <bat> element; and

2	a <periodicity> element;

b)	may include a <bat-window> element; and

c)	may include a <periodicity-range> element which shall include the following sub-elements:

1)	a <lower-bound> element and an <upper-bound> element; or

2)	a <periodicity-value-list> element which shall include one or more <periodicity-value> elements.

The <establishment-rsp> element:

a)	shall include a <result> element which may include a <cause> sub-element; and

b)	may include a <traffic-descriptor-info> element which shall include at least one of the following sub-elements:

1)	a <user-plane-address> element;

2)	a <port-number> element;

3)	a <URL> element; or

4)	a <transport-layer-protocol> element;

c)	a <expiry-time> element;

d)	a <traffic-transmission-bandwidth> element; and

e)	may include an <anyExt> element that may contain:

1)	a <bat-period-adapt-cap> element; and

2)	a <transmission-assist-info> element.

The <transmission-assist-info> element:

a)	shall include at least one of the following sub-elements:

1)	a <bat> element; and

2	a <periodicity> element;

b)	may include a <bat-window> element which shall include the following sub-elements:

1)	a <start-time> element indicating the acceptable earliest time of the first packet of the data burst; or

2)	a <stop-time> element indicating the acceptable latest arrival time of the first packet of the data burst; and

c)	may include a <periodicity-range> element which shall include the following sub-elements:

1)	a <lower-bound> element and an <upper-bound> element; or

2)	a <periodicity-value-list> element which shall include one or more <periodicity-value> elements.

The <release-req> element:

a)	shall include either a <server-id> element or a <sealdd-client-identity> element; and

b)	shall include a <sealdd-flow-id> element.

The <release-rsp> element:

a)	shall include a <result> element which may include a <cause> sub-element.

The <URLLC-establishment-req> element:

a)	shall include a <sealdd-client-identity> element;

b)	shall include a <sealdd-flow-id> element;

c)	may include a <identity> element;

d)	may include a <server-id> element;

e)	may include a <VAL-service-id> element; and

f)	may include a <traffic-descriptor-info> element which shall include at least one of the following sub-elements:

1)	a <user-plane-address> element;

2)	a <port-number> element;

3)	a <URL> element; or

4)	a <transport-layer-protocol> element; and

e)	may include an <anyExt> element that may contain:

1)	a <bat-period-adapt-cap> element; and

2)	a <transmission-assist-info> element.

The <identity> element shall include one of the following:

a)	a <VAL-user-id> element may include a <VAL-client-id> element; or

b)	a <VAL-ue-id> element.

The <URLLC-establishment-rsp> element:

a)	shall include a <result> element which may include a <cause> sub-element; and

b)	may include a <traffic-descriptor-info> element which shall include at least one of the following sub-elements:

1)	a <user-plane-address> element;

2)	a <port-number> element;

3)	a <URL> element; or

4)	a <transport-layer-protocol> element; and

c)	may include an <anyExt> element containing:

1)	a <traffic-transmission-bandwidth> element; and

2)	a <bat-period-adapt-cap> element or a <transmission-assist-info> element.

The <URLLC-release-req> element:

a)	shall include a <sealdd-client-identity> element; and

b)	shall include a <sealdd-flow-id> element.

The <URLLC-release-rsp> element:

a)	shall include a <result> element which may include a <cause> sub-element.

The <URLLC-update-req> element:

a)	shall include a <sealdd-client-identity> element;

b)	shall include a <sealdd-flow-id> element;

c)	may include a <server-id> element

d)	may include a <VAL-service-id> element; and

e)	may include a <traffic-descriptor-info> element which shall include at least one of the following sub-elements:

1)	a <user-plane-address> element;

2)	a <port-number> element;

3)	a <URL> element; or

4)	a <transport-layer-protocol> element.

The <URLLC-update-rsp> element:

a)	shall include a <result> element which may include a <cause> sub-element.

The <data-storage-creation-req> element:

a)	shall include a <application-data> element;

b)	may include a <access-control-policy> element;

c)	may include a <expiry-time> element; and

d)	may include a <status-information-req> element which shall include at least one of the following sub-elements:

1)	a <no-times-data-accessed> element; and

2)	a <no-times-data-managed> element.

The <data-storage-creation-rsp> element:

a)	shall include a <result> element; and

b)	may include a <data-identifier> element.

The <data-storage-reservation-req> element:

a)	shall include a <VAL-service-id> element;

b)	may include a <data-length> element.

The <data-storage-reservation-rsp> element:

a)	shall include a <result> element; and

b)	may include a <address> element.

The <data-storage-status-notification> element:

a)	shall include a <data-identifier> element; and

b)	shall include a <status-information-rsp> element which shall include at least one of the following sub-elements:

1)	a <no-times-data-accessed-value> element; and

2)	a <no-times-data-managed-value> element.

The <data-storage-query-req> element:

a)	shall include a <data-identifier> element.

The <data-storage-query-rsp> element:

a)	shall include a <result> element;

b)	shall include a <data-identifier> element; and

c)	may include a <application-data> element.

The <data-storage-mgt-req> element:

a)	shall include a <data-identifier> element;

b)	shall include a <operation> element; and

c)	may include a <application-data> element.

The <data-storage-mgt-rsp> element:

a)	shall include a <result> element;

b)	shall include a <data-identifier> element; and

c)	may include a <application-data> element.

The <measurements-subscription-req> element:

a)	shall include a <sealdd-flow-id> element;

b)	shall include a <measurement-requirement-list> element which shall include at least one of the following sub-elements:

1)	a <measurement-id> element;

2)	a <reporting-frequency> element;

3)	a <reporting-periodicity> element;

4)	a <measurement-window> element;

5)	a <expiry time > element;

6)	a <sealdd-policy> element which shall include the following sub-elements:

i)	a <quality-guarantee-policy> element; and

ii)	an <anyExt> element containing a <non-3gpp-access-policy> element; and

7)	a <reporting-criteria> element; and

c)	may include a <measurement-conditions> element.

The <measurements-subscription-rsp> element:

a)	shall include a <result> element; and

b)	may include a <expiry-time> element.

The <measurements-notification> element:

a)	shall include a <measurement-requirement-notify-list> element which shall include at least one of the following sub-elements:

1)	a <measurement-id> element;

2)	an <identity-measurements> element;

3)	a <average-measurement-value> element;

4)	a <minimum-measurement-value> element;

5)	a <maximum-measurement-value> element;

6)	a <standard-deviation-measurement-value> element;

7)	a <kpercentile-measurement-value> element;

8)	a <measurement-period> element;

9)	a <timestamp> element; and

10)	an <anyExt> element containing a <non-3gpp-access-measurement-list> element which shall include one or more <non-3gpp-access-measurement> elements containing the following sub-elements:

i)	a <measured-non-3gpp-access> element which shall include an <ssid> or <bssid> element; and

ii)	a <signal-strength-values> element containing one or more <measured-value> elements.

The <identity-measurements> element shall include one of the following:

a)	a <VAL-ue-id-list> element which shall include:

1)	one or more <VAL-ue-id> elements: or

b)	a <VAL-group-id> element.

The <tx-quality-management-req> element:

a)	shall include a <sealdd-flow-id> element;

b)	shall include a <tx-quality-management-action> element; and

c)	may include an <anyExt> element that may include:

1)	a <bat-offset-ul> element; and

2)	a <periodicity-ul> element.

The <tx-quality-management-rsp> element:

a)	shall include a <result> element.

The <connection-status-configuration-req> element:

a)	shall include a <sealdd-flow-id> element;

b)	may include a <reporting-mode> element which may include a <reporting-interval> sub-element;

c)	may include a <reporting-priority> element; and

d)	may include a <non-3gpp-access-policy> element.

The <connection-status-configuration-rsp> element:

a)	shall include a <result> element.

The <connection-status-notification> element:

a)	shall include a <client-connection-status> element;

b)	shall include a <access-usage> element; and

c)	may include a <non-3gpp-access-measurement-list> element which shall include one or more <non-3gpp-access-measurement> elements containing the following sub-elements:

1)	a <measured-non-3gpp-access> element which shall include an <ssid> or <bssid> element; and

2)	a <signal-strength-values> element containing one or more <measured-value> elements.

The <xr-inform-req> element:

a)	shall include a <requestor-id> element;

b)	shall include a <VAL-ue-id-list> element which shall include:

1)	one or more <VAL-ue-id> elements;

c)	shall include a <status> element; and

d)	may include a <VAL-service-id> element.

The <xr-inform-rsp> element:

a)	shall include a <result> element.

The <xr-trigger-req> element:

a)	shall include a <requestor-id> element;

b)	shall include a <VAL-ue-id-list> element which shall include:

1)	one or more <VAL-ue-id> elements;

c)	shall include a <operation> element; and

d)	may include a <VAL-service-id> element.

The <xr-trigger-rsp> element:

a)	shall include a <result> element.

The <xr-establishment-req> element:

a)	shall include a <sealdd-client-identity> element;

b)	shall include a <sealdd-flow-info> element:

1)	shall include one or more <sealdd-flow-id> elements which of them;

2)	may include a <traffic-descriptor-info> element which shall include at least one of the following sub-elements:

A)	a <user-plane-address> element;

B)	a <port-number> element;

C)	a <URL> element; or

D)	a <transport-layer-protocol> element;

c)	may include an <identity> element;

d)	may include a <server-id> element; and

e)	may include a <service-id> element;

The <xr-establishment-rsp> element:

a)	shall include a <result> element which may include a <cause> sub-element;

b)	shall include a <sealdd-multi-modal-flow-info> element which shall include the following sub-elements:

1)	a <sealdd-multi-modal-flow-id> element; and

2)	a <sealdd-flows-list> element:

i)	shall include one or more <sealdd-flow-id> elements which each of them:

A)	may include a <traffic-descriptor-info> element which shall include at least one of the following sub-elements:

I)	<user-plane-address> element;

II)	a <port-number> element;

III)	a <URL> element; or

IV)	a <transport-layer-protocol> element; and

c)	may include a <protocol-descriptor-info> element which shall include at least one of the following sub-elements:

1)	a <header-ext-info> element;

i)	a <header-ext-type> element; and

ii)	a <header-ext-id> element;

2)	a <packetization-indication> element;

3)	a <payload-type> element; and

3)	a <payload-format> element.

## 8.4	XML schema

### 8.4.1	General

This clause defines the XML schema for data delivery information.

### 8.4.2	XML schema

Editor's note [WID: XRM_Ph2_App, CR#: 0094]:	The update of XML schema to support multimodalSealddFlowId is FFS.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:sealDataDeliveryInfo:1.0"

xmlns:sealdatadelivery="urn:3gpp:ns:sealDataDeliveryInfo:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns:xenc="http://www.w3.org/2001/04/xmlenc#">

<xs:annotation>

<xs:documentation>

3GPP - SDDM messages syntax based on 3GPP TS 24.543.

</xs:documentation>

</xs:annotation>

<xs:import namespace="http://www.w3.org/XML/1998/namespace"

schemaLocation="http://www.w3.org/2001/xml.xsd"/>

<!-- the root element which contains the SEALDD protocol messages -->

<xs:element name="data-delivery-info" id="DataDelivery">

<xs:complexType>

<xs:choice>

<xs:element name="establishment-req" type="sealdatadelivery:tEstablishmentReqType"/>

<xs:element name="establishment-rsp" type="sealdatadelivery:tEstablishmentRspType"/>

<xs:element name="release-req" type="sealdatadelivery:tReleaseReqType"/>

<xs:element name="release-rsp" type="sealdatadelivery:tReleaseRspType"/>

<xs:element name="URLLC-establishment-req" type="sealdatadelivery:tURLLCEstablishmentReqType"/>

<xs:element name="URLLC-establishment-rsp" type="sealdatadelivery:tURLLCEstablishmentRspType"/>

<xs:element name="URLLC-release-req" type="sealdatadelivery:tURLLCReleaseReqType"/>

<xs:element name="URLLC-release-rsp" type="sealdatadelivery:tURLLCReleaseRspType"/>

<xs:element name="URLLC-update-req" type="sealdatadelivery:tURLLCUpdateReqType"/>

<xs:element name="URLLC-update-rsp" type="sealdatadelivery:tURLLCUpdateRspType"/>

<xs:element name="data-storage-creation-req" type="sealdatadelivery:tDataStorageCreationReqType"/>

<xs:element name="data-storage-creation-rsp" type="sealdatadelivery:tDataStorageCreationRspType"/>

<xs:element name="data-storage-reservation-req" type="sealdatadelivery:tDataStorageReservationReqType"/>

<xs:element name="data-storage-reservation-rsp" type="sealdatadelivery:tDataStorageReservationRspType"/>

<xs:element name="data-storage-status-notification" type="sealdatadelivery:tDataStorageStatusNotificationType"/>

<xs:element name="data-storage-query-req" type="sealdatadelivery:tDataStorageQueryReqType"/>

<xs:element name="data-storage-query-rsp" type="sealdatadelivery:tDataStorageQueryRspType"/>

<xs:element name="data-storage-mgt-req" type="sealdatadelivery:tDataStorageMgtReqType"/>

<xs:element name="data-storage-mgt-rsp" type="sealdatadelivery:tDataStorageMgtRspType"/>

<xs:element name="measurements-subscription-req" type="sealdatadelivery:tMeasurementsSubscriptionReqType"/>

<xs:element name="measurements-subscription-rsp" type="sealdatadelivery:tMeasurementsSubscriptionRspType"/>

<xs:element name="measurements-notification" type="sealdatadelivery:tMeasurementsNotificationType"/>

<xs:element name="tx-quality-management-req" type="sealdatadelivery:tTxQualityManagementReqType"/>

<xs:element name="tx-quality-management-rsp" type="sealdatadelivery:tTxQualityManagementRspType"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs=  "unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:element>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="connection-status-configuration-req" type="sealdatadelivery:tConnectionStatusConfReqType"/>

<xs:element name="connection-status-configuration-rsp" type="sealdatadelivery:tConnectionStatusConfRspType"/>

<xs:element name="XR-establishment-req" type="sealdatadelivery:tXREstablishmentReqType"/>

<xs:element name="XR-establishment-rsp" type="sealdatadelivery:tXREstablishmentRspType"/>

<xs:element name="connection-status-notification" type="sealdatadelivery:tConnectionStatusNotificationType"/>

<xs:element name="xr-trigger-req" type="sealdatadelivery:tXrTriggerReqType"/>

<xs:element name="xr-trigger-rsp" type="sealdatadelivery:tXrTriggerRspType"/>

<xs:element name="xr-inform-req" type="sealdatadelivery:tXrInformReqType"/>

<xs:element name="xr-inform-rsp" type="sealdatadelivery:tXrInformRspType"/>

<xs:complexType name="tEstablishmentReqType">

<xs:sequence>

<xs:element name="requestor-id" type="sealdatadelivery:tRequestorIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="server-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="endpoint-id" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="VAL-service-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="sealdd-communication-lifetime" type="xs:positiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="traffic-descriptor-info" type="sealdatadelivery:tTrafficDescriptorInfoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="Identity" type="sealdatadelivery:tIdentityType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element above -->

<xs:element name="L4S-feedback-capability" type="sealdatadelivery:tL4SFeedbackCapabilityType"/>

<xs:simpleType name="tRequestorIdType">

<xs:restriction base="xs:string">

<xs:enumeration value="sealddclient"/>

<xs:enumeration value="sealddserver"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tSealddFlowIdType">

<xs:restriction base="xs:positiveInteger">

<xs:minInclusive value="1"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tIdentityType">

<xs:choice>

<xs:element name="VAL-user-id" type="sealdatadelivery:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VAL-ue-id" type="xs:string" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tTrafficDescriptorInfoType">

<xs:sequence>

<xs:element name="user-plane-address" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="port-number" type="sealdatadelivery:tPortNumberType" minOccurs="0" maxOccurs="1"/>

<xs:element name="URL" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="transport-layer-protocol" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tPortNumberType">

<xs:restriction base="xs:positiveInteger">

<xs:minInclusive value="1"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tL4SFeedbackCapabilityType">

<xs:sequence>

<xs:element name="ecn-marking" type="sealdatadelivery:tEcnMarkingType"/>

<xs:element name="l4s-feedback" type="sealdatadelivery:tL4sFeedbackType"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- ECN marking capability details -->

<xs:complexType name="EcnMarkingType">

<xs:sequence>

<xs:element name="supports-ect0" type="xs:boolean"/>

<xs:element name="ect0-value" type="xs:string" minOccurs="0"/> <!-- non-standard value, e.g. "0x01" -->

<xs:element name="supports-ect1" type="xs:boolean"/>

<xs:element name="ect1-value" type="xs:string" minOccurs="0"/> <!-- non-standard value, e.g. "0x03" -->

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- L4S per-packet CE feedback details -->

<xs:complexType name="L4sFeedbackType">

<xs:sequence>

<xs:element name="per-packet-ce-reporting" type="xs:boolean"/>

<xs:element name="feedback-interval" type="xs:positiveInteger" minOccurs="0"/> <!-- e.g. how often feedback is sent -->

<xs:element name="max-feedback-frequency" type="xs:positiveInteger" minOccurs="0"/>

<xs:element name="protocol" type="xs:string" minOccurs="0"/> <!-- e.g. "RTCP", "in-band" -->

</xs:sequence>

</xs:complexType>

<xs:complexType name="tEstablishmentRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="traffic-descriptor-info" type="sealdatadelivery:tTrafficDescriptorInfoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="expiry-time" type="xs:nonPositiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="traffic-transmission-bandwidth" type="xs:positiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tResultType">

<xs:sequence>

<xs:element name="operation-result" type="sealdatadelivery:tOperationResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="cause" type="sealdatadelivery:tCauseType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tOperationResultType">

<xs:restriction base="xs:string">

<xs:enumeration value="success"/>

<xs:enumeration value="failure"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tCauseType">

<xs:restriction base="xs:string">

<xs:enumeration value="SEALDD policy mismatch"/>

<xs:enumeration value="VAL client error"/>

<xs:enumeration value="Other"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tReleaseReqType">

<xs:sequence>

<xs:element name="server-id" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="sealdd-client-identity" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tReleaseRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tURLLCEstablishmentReqType">

<xs:sequence>

<xs:element name="sealdd-client-identity" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="identity" type="sealdatadelivery:tIdentityType" minOccurs="0" maxOccurs="1"/>

<xs:element name="server-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="VAL-service-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="traffic-descriptor-info" type="sealdatadelivery:tTrafficDescriptorInfoType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="contentType">

<xs:choice>

<xs:element name="sealURI" type="xs:anyURI"/>

<xs:element name="sealString" type="xs:string"/>

<xs:element name="sealBoolean" type="xs:boolean"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tURLLCEstablishmentRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="traffic-descriptor-info" type="sealdatadelivery:tTrafficDescriptorInfoType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tURLLCReleaseReqType">

<xs:sequence>

<xs:element name="sealdd-client-identity" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tURLLCReleaseRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tURLLCUpdateReqType">

<xs:sequence>

<xs:element name="sealdd-client-identity" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="Identity" type="sealdatadelivery:tIdentityType" minOccurs="0" maxOccurs="1"/>

<xs:element name="server-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="VAL-service-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="traffic-descriptor-info" type="sealdatadelivery:tTrafficDescriptorInfoType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tURLLCUpdateRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDataStorageCreationReqType">

<xs:sequence>

<xs:element name="application-data" type="xs:hexBinary" minOccurs="1" maxOccurs="1"/>

<xs:element name="access-control-policy" type="sealdatadelivery:tAccessControlPolicyType" minOccurs="0" maxOccurs="1"/>

<xs:element name="expiry-time" type="xs:nonPositiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="status-information-req" type="sealdatadelivery:tStatusInformationReqType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tStatusInformationReqType">

<xs:sequence>

<xs:element name="no-times-data-accessed" type="xs:boolean" minOccurs="0" maxOccurs="1"/>

<xs:element name="no-times-data-managed" type="xs:boolean" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tAccessControlPolicyType">

<xs:restriction base="xs:string">

<xs:enumeration value="SDDM-C"/>

<xs:enumeration value="VAL-server"/>

<xs:enumeration value="SDDM-S"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tDataStorageCreationRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tOperationResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="data-identifier" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDataStorageReservationReqType">

<xs:sequence>

<xs:element name="VAL-service-id" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="data-length" type="xs:positiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDataStorageReservationRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tOperationResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="address" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDataStorageStatusNotificationType">

<xs:sequence>

<xs:element name="data-identifier" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="status-information-rsp" type="sealdatadelivery:tStatusInformationRspType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tStatusInformationRspType">

<xs:sequence>

<xs:element name="no-times-data-accessed-value" type="xs:unsignedInt" minOccurs="0" maxOccurs="1"/>

<xs:element name="no-times-data-managed-value" type="xs:unsignedInt" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDataStorageQueryReqType">

<xs:sequence>

<xs:element name="data-identifier" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDataStorageQueryRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tOperationResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="data-identifier" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="application-data" type="xs:hexBinary" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDataStorageMgtReqType">

<xs:sequence>

<xs:element name="data-identifier" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="operation" type="sealdatadelivery:tOperationType" minOccurs="1" maxOccurs="1"/>

<xs:element name="application-data" type="xs:hexBinary" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tOperationType">

<xs:restriction base="xs:string">

<xs:enumeration value="update"/>

<xs:enumeration value="refresh"/>

<xs:enumeration value="delete"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tDataStorageMgtRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tOperationResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="data-identifier" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="application-data" type="xs:hexBinary" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMeasurementsSubscriptionReqType">

<xs:sequence>

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="measurement-requirement-list" type="sealdatadelivery:tMeasurementRequirementListType" minOccurs="1" maxOccurs="1"/>

<xs:element name="measurement-conditions" type="sealdatadelivery:tMeasurementConditionsType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMeasurementConditionsType">

<xs:sequence>

<xs:element name="temporal-conditions" type="sealdatadelivery:tTemporalConditionsType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="spacial-conditions" type="sealdatadelivery:tSpatialConditionsType" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tTemporalConditionsType">

<xs:sequence>

<xs:element name="time-range" type="sealdatadelivery:rangeType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="rangeType">

<xs:sequence>

<xs:element name="start-time" type="xs:dateTime" minOccurs="0"/>

<xs:element name="end-time" type="xs:dateTime" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tSpatialConditionsType">

<xs:sequence>

<xs:element name="PolygonArea" type="sealdatadelivery:tPolygonAreaType" minOccurs="0"/>

<xs:element name="EllipsoidArcArea" type="sealdatadelivery:tEllipsoidArcType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="protectionType">

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="Encrypted"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tPolygonAreaType">

<xs:sequence>

<xs:element name="Corner" type="sealdatadelivery:tPointCoordinateType" minOccurs="3" maxOccurs="15"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tEllipsoidArcType">

<xs:sequence>

<xs:element name="Center" type="sealdatadelivery:tPointCoordinateType"/>

<xs:element name="Radius" type="xs:nonNegativeInteger"/>

<xs:element name="OffsetAngle" type="xs:unsignedByte"/>

<xs:element name="IncludedAngle" type="xs:unsignedByte"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPointCoordinateType">

<xs:sequence>

<xs:element name="longitude" type="sealdatadelivery:tCoordinateType"/>

<xs:element name="latitude" type="sealdatadelivery:tCoordinateType"/>

<xs:element name="altitude" type="sealdatadelivery:tCoordinateType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tCoordinateType">

<xs:choice minOccurs="1" maxOccurs="1">

<xs:element name="threebytes" type="sealdatadelivery:tThreeByteType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:attribute name="type" type="sealdatadelivery:protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tThreeByteType">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="16777215"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tMeasurementRequirementListType">

<xs:sequence maxOccurs="unbounded">

<xs:element name="measurement-requirement" type="sealdatadelivery:tMeasurementRequirementType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="tMeasurementRequirementType">

<xs:sequence>

<xs:element name="measurement-id" type="sealdatadelivery:tMeasurementIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="reporting-frequency" type="sealdatadelivery:tReportingFrequencyType" minOccurs="0" maxOccurs="1"/>

<xs:element name="reporting-periodicity" type="xs:unsignedInt" minOccurs="0" maxOccurs="1"/>

<xs:element name="measurement-window" type="xs:unsignedInt" minOccurs="0" maxOccurs="1"/>

<xs:element name="expiry-time" type="xs:nonPositiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="sealdd-policy" type="sealdatadelivery:tSealddPolicyType" minOccurs="0" maxOccurs="1"/>

<xs:element name="reporting-criteria" type="sealdatadelivery:tReportingCriteriaType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSealddPolicyType">

<xs:sequence>

<xs:element name="quality-guarantee-event" type="sealdatadelivery:tQualityGuaranteeEventType" minOccurs="1" maxOccurs="1"/>

<xs:element name="quality-guarantee-action" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tReportingFrequencyType">

<xs:restriction base="xs:string">

<xs:enumeration value="periodic"/>

<xs:enumeration value="now"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tMeasurementIdType">

<xs:restriction base="xs:string">

<xs:enumeration value="latency"/>

<xs:enumeration value="bitrate"/>

<xs:enumeration value="jitter"/>

<xs:enumeration value="packetloss"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tQualityGuaranteeEventType">

<xs:simpleContent>

<xs:extension base="xs:integer">

<xs:attribute name="TriggerEvent" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tReportingCriteriaType">

<xs:sequence>

<xs:element name="latency-threshold-value" type="xs:positiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="above-or-below-latency-threshold-value" type="xs:boolean" minOccurs="0" maxOccurs="1"/>

<xs:element name="bitrate-threshold-value" type="xs:positiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="above-or-below-bitrate-threshold-value" type="xs:boolean" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMeasurementsSubscriptionRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tOperationResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="expiry-time" type="xs:nonPositiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMeasurementsNotificationType">

<xs:sequence>

<xs:element name="measurement-requirement-notify-list" type="sealdatadelivery:tMeasurementRequirementNotifyListType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMeasurementRequirementNotifyListType">

<xs:sequence>

<xs:element name="measurement-id" type="sealdatadelivery:tMeasurementIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="identity-measurements" type="sealdatadelivery:tIdentityMeasurementsType" minOccurs="1" maxOccurs="1"/>

<xs:element name="average-measurement-value" type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="minimum-measurement-value" type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="maximum-measurement-value" type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="standard-deviation-measurement-value" type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="kpercentile-measurement-value" type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="measurement-period" type="xs:positiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="timestamp" type="xs:dateTime" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tIdentityMeasurementsType">

<xs:choice>

<xs:element name="VAL-ue-id-list" type="sealdatadelivery:tValUeIdListType" minOccurs="0"/>

<xs:element name="VAL-group-id" type="xs:string" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tValUeIdListType">

<xs:choice>

<xs:element name="VAL-ue-id" type="xs:string" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tTxQualityManagementReqType">

<xs:sequence>

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="tx-quality-management-action" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tTxQualityManagementRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tOperationResultType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tConnectionStatusConfReqType">

<xs:sequence>

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="reporting-mode" type="sealdatadelivery:tReportingModeType" minOccurs="1" maxOccurs="1"/>

<xs:element name="reporting-priority" type="sealdatadelivery:tReportingPriorityType" minOccurs="0" maxOccurs="1"/>

<xs:element name="non-3gpp-access-policy" type="sealdatadelivery:tNon3GPPAccessType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tReportingModeType">

<xs:sequence>

<xs:element name="reporting-mode-mode" type="sealdatadelivery:tReportingModeModeType" minOccurs="1" maxOccurs="1"/>

<xs:element name="reporting-interval" type="xs:unsignedInt" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tReportingModeModeType">

<xs:restriction base="xs:string">

<xs:enumeration value="PERIODIC"/>

<xs:enumeration value="EVENT_TRIGGERED"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tReportingPriorityType">

<xs:restriction base="xs:string">

<xs:enumeration value="LOW"/>

<xs:enumeration value="HIGH"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tNon3GPPAccessType">

<xs:restriction base="xs:string">

<xs:enumeration value="WLAN SSID"/>

<xs:enumeration value="WLAN BSSID"/>

<xs:enumeration value="LOCATION_BASED"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tConnectionStatusConfRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tXREstablishmentReqType">

<xs:sequence>

<xs:element name="sealdd-client-identity" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="sealdd-flows-info" type="sealdatadelivery:tSealddFlowsInfoType" minOccurs="1" maxOccurs="1"/>

<xs:element name="Identity" type="sealdatadelivery:tIdentityType" minOccurs="0" maxOccurs="1"/>

<xs:element name="server-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="VAL-service-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSealddFlowsInfoType">

<xs:sequence>

<xs:element name="sealdd-flow-info" type="sealdatadelivery:tSealddFlowInfoType" minOccurs="1" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSealddFlowInfoType">

<xs:sequence >

<xs:element name="sealdd-flow-id" type="sealdatadelivery:tSealddFlowIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="traffic-descriptor-info" type="sealdatadelivery:tTrafficDescriptorInfoType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence >

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tXREstablishmentRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:element name="multi-modal-sealdd-flow-info" type="sealdatadelivery:tMultiModalSealddFlowInfoType" minOccurs="1" maxOccurs="1"/>

<xs:element name="protocol-descriptor-info" type="sealdatadelivery:tProtocolDescriptorInfoType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMultiModalSealddFlowInfoType">

<xs:sequence >

<xs:element name="sealdd-flows-info" type="sealdatadelivery:tSealddFlowsInfoType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence >

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tProtocolDescriptorInfoType">

<xs:sequence >

<xs:element name="header-ext-info" type="sealdatadelivery:tHeaderExtInfoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="packetization-indication" type="xs:boolean" minOccurs="0" maxOccurs="1"/>

<xs:element name="payload-type" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="payload-format" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence >

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tHeaderExtInfoType">

<xs:sequence >

<xs:element name="header-ext-type" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="header-ext-id" type="sealdatadelivery:tHeaderExtIdType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence >

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tHeaderExtIdType">

<xs:restriction base="xs:positiveInteger">

<xs:minInclusive value="1"/>

<xs:maxInclusive value="255"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tConnectionStatusNotificationType">

<xs:sequence>

<xs:element name="client-connection-status" type="sealdatadelivery:tClientConnectionStatusType"/>

<xs:element name="access-usage" type="sealdatadelivery:tAccessUsageType"/>

<xs:element name="non-3gpp-access-measurement-list" type="sealdatadelivery:tnon3gppAccessMeasurementListType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tClientConnectionStatusType">

<xs:restriction base="xs:string">

<xs:enumeration value="REACHABLE"/>

<xs:enumeration value="UNREACHABLE"/>

<xs:enumeration value="SLEEPING"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tAccessUsageType">

<xs:restriction base="xs:string">

<xs:enumeration value="3GPP_ACCESS"/>

<xs:enumeration value="NON_3GPP_ACCESS"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tnon3gppAccessMeasurementListType">

<xs:sequence>

<xs:element name="non-3gpp-access-measurement" type="sealdatadelivery:tnon3gppAccessMeasurementType" minOccurs="1" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tnon3gppAccessMeasurementType">

<xs:sequence>

<xs:element name="measured-non-3gpp-access" type="sealdatadelivery:tMeasuredNon3gppAccessType" minOccurs="0" maxOccurs="1"/>

<xs:element name="signal-strength-values" type="sealdatadelivery:tSignalStrengthValuesType" minOccurs="1" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>    </xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMeasuredNon3gppAccessType">

<xs:choice>

<xs:element name="ssid" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="bssid" type="sealdatadelivery:tBssidType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tBssidType">

<xs:restriction base="xs:string">

<!—Example: 00:1A:2B:3C:4D:5E or 00-1A-2B-3C-4D-5E as per IEEE std -->

<xs:pattern value="([0-9A-Fa-f]{2}([-:])){5}[0-9A-Fa-f]{2}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tSignalStrengthValuesType">

<xs:sequence>

<xs:element name="measured-value" type="xs:NonNegativeInteger"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>    </xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element of sealdatadelivery:tEstablishmentReqType, sealdatadelivery:tEstablishmentRspType, sealdatadelivery:tURLLCEstablishmentReqType, sealdatadelivery:tURLLCEstablishmentRspType, start-->

<xs:element name="bat-period-adapt-cap" type="xs:boolean"/>

<xs:element name="transmission-assist-info" type="sealdatadelivery:tTransmissionAssistInfoType"/>

<!-- The following elements are added for extensibility and to be placed in the anyExt element of sealdatadelivery:tEstablishmentReqType, sealdatadelivery:tEstablishmentRspType, sealdatadelivery:tURLLCEstablishmentReqType, sealdatadelivery:tURLLCEstablishmentRspType, end-->

<xs:complexType name="tTransmissionAssistInfoType">

<xs:sequence>

<xs:element name="bat" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="periodicity" type="sealdatadelivery:tUnsigned64bitInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="bat-window" type="sealdatadelivery:tBatWindowType" minOccurs="0" maxOccurs="1"/>

<xs:element name="periodicity-range" type="sealdatadelivery:tPeriodicityRangeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tUnsigned64bitInteger">

<xs:restriction base="xs:nonNegativeInteger">

<xs:maxInclusive value="18446744073709551615"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tBatWindowType">

<xs:sequence>

<xs:element name="start-time" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="stop-time" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPeriodicityRangeType">

<xs:sequence>

<xs:element name="lower-bound" type="sealdatadelivery:tUnsigned64bitInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="upper-bound" type="sealdatadelivery:tUnsigned64bitInteger" minOccurs="0" maxOccurs="1"/>

<xs:element name="periodicity-value-list" type="sealdatadelivery:tPeriodicityValueListType" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPeriodicityValueListType">

<xs:sequence maxOccurs="unbounded">

<xs:element name="periodicity-value" type="sealdatadelivery:tUnsigned64bitInteger" minOccurs="1" maxOccurs="1"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element of sealdatadelivery:tTxQualityManagementReqType, start-->

<xs:element name="bat-offset-ul" type="xs:positiveInteger"/>

<xs:element name="periodicity-ul" type="xs:positiveInteger"/>

<!-- The following elements are added for extensibility and to be placed in the anyExt element of sealdatadelivery:tTxQualityManagementReqType, end-->

<xs:complexType name="tXrInformReqType">

<xs:sequence>

<xs:element name="requestor-id" type="sealdatadelivery:tRequestorIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="VAL-ue-id-list" type="sealdatadelivery:tValUeIdListType" minOccurs="1"/>

<xs:element name="status" type="sealdatadelivery:tXrStatusType" minOccurs="1" maxOccurs="1"/>

<xs:element name="VAL-service-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tXrInformRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tXrTriggerReqType">

<xs:sequence>

<xs:element name="requestor-id" type="sealdatadelivery:tRequestorIdType" minOccurs="1" maxOccurs="1"/>

<xs:element name="VAL-ue-id-list" type="sealdatadelivery:tValUeIdListType" minOccurs="1"/>

<xs:element name="operation" type="sealdatadelivery:tXrOperationType" minOccurs="1" maxOccurs="1"/>

<xs:element name="VAL-service-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tXrTriggerRspType">

<xs:sequence>

<xs:element name="result" type="sealdatadelivery:tResultType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="sealdatadelivery:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tXrStatusType">

<xs:restriction base="xs:string">

<xs:enumeration value="ESTABLISH"/>

<xs:enumeration value="RELEASE"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="tXrOperationType">

<xs:restriction base="xs:string">

<xs:enumeration value="ESTABLISH"/>

<xs:enumeration value="RELEASE"/>

</xs:restriction>

</xs:simpleType>

<!-- XML attribute for any future extension -->

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

## 8.5	Data semantics

The <data-delivery-info> element is the root element of the XML document. The <data-delivery-info> element contains the <establishment-req>, <establishment-rsp>, <release-req>, <release-rsp>, <URLLC-establishment-req>, <URLLC-establishment-rsq>, <URLLC-release-req>, <URLLC-release-rsp>, <URLLC-update-req>, <URLLC-update-rsp>, <data-storage-creation-req>, <data-storage-creation-rsp>, <data-storage-reservation-req>, <data-storage-reservation-rsp>, <data-storage-status-notification>, <measurements-subscription-req> , <measurements-subscription-rsp>, <data-storage-query-req>, <data-storage-query-rsp>, <data-storage-mgt-req>, <data-storage-mgt-rsp>, <measurements-notification>, <identity-measurements>, <tx-quality-management-req>, <tx-quality-management-rsp>, and <anyExt> element.

The <anyExt> element contains elements defined by future versions of the present document.

The <anyExt> element of the <data-delivery-info> element contains <connection-status-configuration-req>, <connection-status-configuration-rsp>, <connection-status-notification>, <xr-trigger-req>, <xr-trigger-rsp> , <xr-inform-req>, <xr-inform-rsp>, <xr-establishment-req>, and <xr-establishment-rsp> sub-elements.

<establishment-req> element contains the following sub-elements:

a)	<requestor-id>, a mandatory element. This element contains a string set to either "sealddclient" or "sealddserver" used to specify the identity of the requestor being either an SDDM-C or an SDDM-S.

b)	<sealdd-flow-id>, a mandatory element specifying the identity of the seal flow.

c)	<endpoint-id>, a mandatory element specifying the endpoint of a selected VAL server.

d)	<server-id>, an optional element specifying the VAL server.

e)	<sealdd-communication-lifetime>, an optional element specifying the data delivery communication lifetime in milliseconds.

f)	<VAL-service-id>, an optional element specifying the VAL service identity of the vertical application.

g)	<traffic-descriptor-info>, an optional element specifying the information of the traffic that contains one or more of the following sub-elements:

1)	a <user-plane-address> element set to user plane IP address used for the traffic;

2)	a <port-number> element set to the port number for the traffic;

3)	a <URL> element set to a text format that specifies how to access the resource on the Internet for the traffic; or

4)	a <transport-layer-protocol> element set to the transport protocol used for the traffic (e.g. TCP, UDP).

h)	<identity>, an optional element set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request or the SDDM-S that performs the request.

i)	<anyExt>, an optional element that contains:

1)	<L4S-feedback-capability>, an optional element set to the L4S feedback capability (i.e. ECN identification, L4S feedback) that contains the following sub-elements:

i)	<ecn-marking>, a mandatory element specifying the ECN marking capability details that contains the following sub-elements:

A)	<supports-ect0>, a mandatory element indicating the ECT0 support. The value "1" indicates support of ECT0 (see IETF RFC 3168 [r3168]);

B)	<ect0-value>, an optional element that shall be a string set to value of ECT0 when non-standard values are used (see IETF RFC 3168 [r3168]);

C)	<supports-ect1>, a mandatory element indicating the ECT1 support. The value "1" indicates support of ECT0 (see IETF RFC 3168 [r3168]); and

D)	<ect1-value>, an optional element that shall be a string set to value of ECT1 when non-standard values are used (see IETF RFC 3168 [r3168]); and

ii)	<L4s-feedback>, a mandatory element specifying L4S feedback details that contains the following sub-elements:

A)	<per-packet-ce-reporting>, a mandatory element indicating that the entity can report congestion-experienced (CE) indications on a per-packet basis. The value "1" indicates support of reporting congention experience (CE) on a per-packet basis;

B)	 <feedback-interval>, an optional element set to the time between feedback transmissions (how often they can report without overwhelming control channels);

C)	<max-feedback-frequency>, an optional element set to the fastest rate at which feedback may be sent; and

D)	<protocol>, an optional element that shall be a string set to the transport protocol (e.g. "RTCP", "in-band");

2)	<bat-period-adapt-cap>, an optional element indicating the BAT and periodicity adaptation capability for uplink SEALDD traffic. The value "1" indicates support of the BAT and periodicity adaptation. If absent, it indicates no support of the BAT and periodicity adaptation; or

3)	<transmission-assist-info>, an optional element specifying a transmission assistance information for uplink SEALDD traffic that contains the following sub-elements:

i)	<bat>, an optional element specifying the arrival time of the first packet of the data burst that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11];

ii)	<periodicity>, an optional element specifying the time period between the start of two bursts in units of microseconds and shall be defined as an unsigned 64-bit integer;

iii)	<bat-window>, an optional element that contains the following sub-elements:

A)	a <start-time> element set to the acceptable earliest arrival time of the first packet of the data burst (i.e. start time of the time window) that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11]; or

B)	a <stop-time> element set to the acceptable latest arrival time of the first packet of the data burst (i.e. stop time of the time window) that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11]; and

iv)	<periodicity-range>, an optional element specifying the periodicity range which may be included only if the <bat>, <bat-window> and <periodicity>elements are included and contains the following sub-elements:

A)	a <lower-bound> element set to the lower bound of the periodicity and an <upper-bound> element set to the upper bound of the periodicity of the start two bursts in units of microseconds and shall be defined as an unsigned 64-bit integer; or

B)	a <periodicity-value-list> element with one or more <periodicity-value> child elements set to the acceptable periodicity value in units of microseconds and shall be defined as an unsigned 64-bit integer.

<establishment-rsp> element contains the following sub-elements:

a)	<result>, which includes a sub-element <operation-result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation. If the result is "failure", the <result> element may contain a <cause> sub-element set to the cause of the failure of the operation (e.g. VAL client error, SEALDD policy mismatch).

b)	<traffic-descriptor-info>, an optional element specifying the information of the traffic that contains one or more of the following sub-elements:

1)	a <user-plane-address> element set to user plane IP address used for the traffic;

2)	a <port-number> element set to the port number for the traffic;

3)	a <URL> element set to a text format that specifies how to access the resource on the Internet for the traffic; or

4)	a <transport-layer-protocol> element set to the transport protocol used for the traffic (e.g. TCP, UDP);

c)	a <expiry-time> element set to a time in milliseconds that triggers the re-connection from either the SDDM-C or the SDDM-S when bandwidth limit check has failed;

d)	 a <traffic-transmission-bandwidth> element set to the suggested traffic transmission bandwidth to be used by either the SDDM-C or the SDDM-S; and

i)	<anyExt>, an optional element that contains:

1)	<bat-period-adapt-cap>, an optional element indicating the BAT and periodicity adaptation capability for uplink SEALDD traffic. The value "1" indicates support of the BAT and periodicity adaptation. If absent, it indicates no support of the BAT and periodicity adaptation; or

2)	<transmission-assist-info>, an optional element specifying a transmission assistance information for uplink SEALDD traffic that contains the following sub-elements:

i)	<bat>, an optional element specifying the arrival time of the first packet of the data burst that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11];

ii)	<periodicity>, an optional element specifying the time period between the start of two bursts in units of microseconds and shall be defined as an unsigned 64-bit integer;

iii)	<bat-window>, an optional element that contains the following sub-elements:

A)	a <start-time> element set to the acceptable earliest arrival time of the first packet of the data burst (i.e. start time of the time window) that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11]; or

B)	a <stop-time> element set to the acceptable latest arrival time of the first packet of the data burst (i.e. stop time of the time window) that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11]; and

iv)	<periodicity-range>, an optional element specifying the periodicity range which may be included only if the <bat>, <bat-window> and <periodicity>elements are included and contains the following sub-elements:

A)	a <lower-bound> element set to the lower bound of the periodicity and an <upper-bound> element set to the upper bound of the periodicity of the start two bursts in units of microseconds and shall be defined as an unsigned 64-bit integer; or

B)	a <periodicity-value-list> element with one or more <periodicity-value> child elements set to the acceptable periodicity value in units of microseconds and shall be defined as an unsigned 64-bit integer.

<identity> element contains one of following sub-elements:

a)	<VAL-user-id> element specifying the identity of the VAL user; or

b)	<VAL-UE-id> element specifying the identity of the VAL UE.

<release-req> element contains the following sub-elements:

a)	<server-id>, an optional element specifying the endpoint of a selected VAL server;

b)	<sealdd-client-identity>, an optional element specifying the identity of the SDDM-C; and

c)	<sealdd-flow-id>, a mandatory element specifying the identity of the seal flow.

<release-rsp> element contains the following sub-elements:

a)	<result>, which includes a sub-element <operation-result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation. If the result is "failure", the <result> element may contain a <cause> sub-element set to the cause of the failure of the operation.

<URLLC-establishment-req> element contains the following sub-elements:

a)	<sealdd-client-identity>, a mandatory element specifying the identity of the SDDM-C;

b)	<sealdd-flow-id>, a mandatory element specifying the identity of the seal flow;

c)	<server-id>, an optional element specifying the endpoint of a selected VAL server;

d)	<identity>, an optional set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and either performing the request or receiving the request;

e)	<VAL-service-id>, an optional element specifying the VAL service identity of the vertical application; and

f)	<traffic-descriptor-info>, an optional element specifying the information of the traffic that contains one or more of the following sub-elements:

1)	a <user-plane-address> element set to user plane IP address used for the traffic;

2)	a <port-number> element set to the port number for the traffic;

3)	a <URL> element set to a text format that specifies how to access the resource on the Internet for the traffic; or

4)	a <transport-layer-protocol> element set to the transport protocol used for the traffic (e.g. TCP, UDP); and

e)	<anyExt>, an optional element that contains:

1)	<bat-period-adapt-cap>, an optional element indicating the BAT and periodicity adaptation capability for uplink SEALDD traffic. The value "1" indicates support of the BAT and periodicity adaptation. If absent, it indicates no support of the BAT and periodicity adaptation; or

2)	<transmission-assist-info>, an optional element specifying a transmission assistance information for uplink SEALDD traffic that contains the following sub-elements:

i)	<bat>, an optional element specifying the arrival time of the first packet of the data burst that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11];

ii)	<periodicity>, an optional element specifying the time period between the start of two bursts in units of microseconds and shall be defined as an unsigned 64-bit integer;

iii)	<bat-window>, an optional element that contains the following sub-elements:

A)	a <start-time> element set to the acceptable earliest arrival time of the first packet of the data burst (i.e. start time of the time window) that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11]; or

B)	a <stop-time> element set to the acceptable latest arrival time of the first packet of the data burst (i.e. stop time of the time window) that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11]; and

iv)	<periodicity-range>, an optional element specifying the periodicity range which may be included only if the <bat>, <bat-window> and <periodicity>elements are included and contains the following sub-elements:

A)	a <lower-bound> element set to the lower bound of the periodicity and an <upper-bound> element set to the upper bound of the periodicity of the start two bursts in units of microseconds and shall be defined as an unsigned 64-bit integer; or

B)	a <periodicity-value-list> element with one or more <periodicity-value> child elements set to the acceptable periodicity value in units of microseconds and shall be defined as an unsigned 64-bit integer.

<URLLC-establishment-rsp> element contains the following sub-elements:

a)	<result>, which includes a sub-element <operation-result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation. If the result is "failure", the <result> element may contain a <cause> sub-element set to the cause of the failure of the operation (e.g. SEALDD policy mismatch);

b)	<traffic-descriptor-info>, an optional element specifying the information of the traffic that contains one or more of the following sub-elements:

1)	a <user-plane-address> element set to user plane IP address used for the traffic;

2)	a <port-number> element set to the port number for the traffic;

3)	a <URL> element set to a text format (URL) that specifies how to access the resource on the Internet for the traffic; or

4)	a <transport-layer-protocol> element set to the transport protocol used for the traffic (e.g. TCP, UDP); and

c)	<anyExt>, an optional element that contains:

1)	<traffic-transmission-bandwidth>, an optional element set to the suggested traffic transmission bandwidth to be used by either the SDDM-C or the SDDM-S; and

2)	<bat-period-adapt-cap>, an optional element indicating a BAT and periodicity adaptation capability or <transmission-assist-info>, an optional element specifying a transmission assistance information for uplink SEALDD traffic. The <transmission-assist-info> element contains the following sub-elements:

i)	a <bat> element specifying the arrival time of the first packet of the data burst that shall be a string with a full-day and full-time format as defined in clause 5.6 of IETF RFC 3339 [11];

ii)	a <periodicity> element specifying the time period between the start of two bursts in units of microseconds and shall be defined as an unsigned 64-bit integer;

iii)	a <bat-window>, element containing the acceptable earliest and latest arrival time of the first packet of the data burst which may be included only if the <bat> element is included; and

iv)	a <periodicity-range>, element specifying the periodicity range which may be included only if the <bat>, <bat-window> and <periodicity>elements are included and contains the following sub-elements:

-	a <lower-bound> element set to the lower bound of the periodicity and an <upper-bound> element set to the upper bound of the periodicity of the start two bursts in units of microseconds and shall be defined as an unsigned 64-bit integer; or

-	a <periodicity-value-list> element with one or more <periodicity-value> child elements set to the acceptable periodicity value in units of microseconds and shall be defined as an unsigned 64-bit integer.

<URLLC-release-req> element contains the following sub-elements:

a)	<sealdd-client-identity>, a mandatory element specifying the identity of the SDDM-C; and

c)	<sealdd-flow-id>, a mandatory element specifying the identity of the seal flow.

<URLLC-release-rsp> element contains the following sub-elements:

a)	<result>, which includes a sub-element <operation-result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation. If the result is "failure", the <result> element may contain a <cause> sub-element set to the cause of the failure of the operation.

<URLLC-update-req> element contains the following sub-elements:

a)	<sealdd-client-identity>, a mandatory element specifying the identity of the SDDM-C;

b)	<sealdd-flow-id>, a mandatory element specifying the identity of the seal flow;

c)	<server-id>, an optional element specifying the endpoint of a selected VAL server;

d)	<VAL-service-id>, an optional element specifying the VAL service identity of the vertical application; and

e)	<traffic-descriptor-info>, an optional element specifying the information of the traffic that contains one or more of the following sub-elements:

1)	a <user-plane-address> element set to user plane IP address used for the traffic;

2)	a <port-number> element set to the port number for the traffic;

3)	a <URL> element set to a text format that specifies how to access the resource on the Internet for the traffic; or

4)	a <transport-layer-protocol> element set to the transport protocol used for the traffic (e.g. TCP, UDP).

<URLLC-update-rsp> element contains the following sub-element:

a)	<result>, which includes a sub-element <operation-result>,a mandatory element set to either "success" or "failure" indicating success or failure of the operation. If the result is "failure", the <result> element may contain a <cause> sub-element set to the cause of the failure of the operation (e.g. SEALDD policy mismatch).

<data-storage-creation-req> element contains the following sub-elements:

a)	<application-data>, a mandatory element that provides the application data in hexadecimal to be;

b)	<access-control-policy>, an optional element set to the control policy for the requested data access from other consumers (i.e. SDDM-C, VAL-server, other SDDM-S);

c)	<expiry-time>, an optional element set to the expiration time in minutes of the data to be stored; and

d)	<status-information-req>, an optional element that contains one or more of the following sub-elements:

1)	a <no-times-data-accessed> element that indicates whether information of how many times the stored data is accessed is requested for corresponding notifications; and

2)	a <no-times-data-managed> element that indicates whether information of how many times the stored data is managed is requested for corresponding notifications.

<data-storage-creation-rsp> element contains the following sub-elements:

a)	<result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation; and

b)	<data-identifier>, an optional element set to the identity of the stored data.

<data-storage-reservation-req> element contains the following sub-elements:

a)	<VAL-service-id>, a mandatory element set to the VAL service identity of the vertical application; and

b)	<data-length>, an optional element set to the data length in bytes to be stored;

<data-storage-reservation-rsp> contains the following sub-elements:

a)	<result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation; and

b)	<address>, an optional element set to the reserved address for data storage.

<data-storage-status-notification> element contains the following sub-elements:

a)	<data-identifier>, a mandatory element set to the identity of the stored data being notified; and

b)	<status-information-rsp>, a mandatory element that contains one or more of the following sub-elements:

1)	<no-times-data-accessed-value>, an optional element set to the value of how many times the stored data is accessed; and

2)	<no-times-data-managed-value> an optional element set to the value of how many times the stored data is managed.

<data-storage-query-req> element contains the following sub-element:

a)	 <data-identifier>, a mandatory element set to the identity of the stored data which is requested to be queried.

<data-storage-query-rsp> contains the following sub-elements:

a)	<result>, a mandatory element set to either set to "success" or "failure" indicating success or failure of the operation;

b)	<dta-identifier>, a mandatory element set to the identity of the stored data which is queried; and

c)	<application-data>, a mandatory element that provides the application data which is queried.

<data-storage-mgt-req> element contains the following sub-elements:

a)	<data-identifier>, a mandatory element set to the identity of the stored data which is requested to be managed;

b)	<operation>, a mandatory element set to the operation to be performed such as to "update", "refresh" or "delete" the stored data; and

c)	<application-data>, an optional element that provides the application data in hexadecimal to be updated if the operation to be performed is to update the stored data.

<data-storage-mgt-rsp> element contains the following sub-elements:

a)	<result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation;

b)	<data-identifier>, a mandatory element set to the identity of the stored data which is managed; and

c)	<application-data>, an optional element that provides the application data which is managed.

<measurements-subscription-req> element contains the following sub-elements:

a)	<sealdd-flow-id>, a mandatory element specifying the identity of the seal flow;

b)	<measurement-conditions>, an optional element specifying the temporal conditions, spatial conditions or both for the measurements; and

c)	<measurement-requirement-list>, a mandatory element that contains one or more of the <measurement-requirement> element which each contain the following sub-elements:

1)	<measurement-id>, a mandatory element set to measurement identifiers "latency", "bitrate", "jitter" or "packet loss";

2)	<reporting-frequency>, an optional element set to reporting frequency of measurement results "periodic", "now". If not present, it implies periodic reporting;

3)	<reporting-periodicity>, an optional element set to the reporting periodicity in seconds if the reporting frequency is periodic. This child element shall be included when the <reporting-frequency> element is set to "periodic" or not present;

4)	<measurement-window>, an optional element set to the measurement period window in milliseconds for transmission quality measurements;

5)	<expiry time >, an optional element set to the expiration time in milliseconds of the measurement identifier;

6)	<sealdd-policy>, an optional element that contains the following sub-elements:

i)	<quality-guarantee-policy>, a mandatory element set to the action to be performed "Redundant transmission path", "Re-establish transmission path", "Switch to backup transmission path" when the measurement event occurs, in order to meet the quality guarantee; and

ii)	<anyExt>, an optional element containing a <non-3gpp-access-policy> element set to the non-3GPP access measurement policy, i.e., "WLAN SSID", "WLAN BSSID" or "LOCATION_BASED" measurement; and

7)	<reporting-criteria>, an optional element set to the criteria for reporting measurement results that contains the following sub-elements:

i)	a <latency-threshold-value>, an optional element set to the latency threshold value for reporting measurements results in milliseconds;

ii)	a <above-or-below-latency-threshold-value>, an optional element specifying whether the criterion for reporting measurements results is based on reaching above the latency value indicated by the <latency-threshold-value> element or not. Value "1" indicates that the criterion for reporting measurements results is based on reaching above the latency value indicated by the <latency-threshold-value> element. Value "0" indicates that the criterion for reporting measurements results is based on reaching below the latency value indicated by the <latency-threshold-value> element;

iii)	a <bitrate-threshold-value> element set to the bitrate threshold value for reporting measurements results in Mbps; and

iv)	a <above-or-below-bitrate-threshold-value>, an optional element specifying whether the criterion for reporting measurements results is based on reaching above the bitrate value indicated by the <bitrate-threshold-value> element or not. Value "1" indicates that the criterion for reporting measurements results is based on reaching above the bitrate value indicated by the <bitrate-threshold-value> element. Value "0" indicates that the criterion for reporting measurements results is based on reaching below the bitrate value indicated by the <bitrate-threshold-value> element.

<measurements-subscription-rsp> element contains the following sub-elements:

a)	<result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation; and

b)	<expiry-time>, an optional element set to the expiration time in milliseconds of the measurement requested.

<measurements-notification> element contains the following sub-elements:

a)	<measurement-requirement-notify-list>, a mandatory element that contains one or more of the following sub-elements:

1)	a <measurement-id> element set to measurement identifiers "latency", "bitrate", "jitter" or "packet loss";

2)	an <identity-measurements> element set to the identity of the VAL UE(s) or VAL user(s) under SEALDD measurement;

3)	an <average-measurement-value> element set to the average measurement value of measurement results ("latency" and "jitter" are in milliseconds, "bitrate" is in Mbps, "packet loss" is in percentage of the number of packets that fail to reach their destination);

4)	a <minimum-measurement-value> element set to the minimum measurement value of measurement results ("latency" and "jitter" are in milliseconds, "bitrate", is in Mbps, "packet loss" is in percentage of the number of packets that fail to reach their destination);

5)	a <maximum-measurement-value> element set to the maximum measurement value of measurement results ("latency" and "jitter" are in milliseconds, "bitrate", is in Mbps, "packet loss" is in percentage of the number of packets that fail to reach their destination);

6)	a <standard-deviation-measurement-value> element set to standard deviation measurement value of measurement results;

7)	a <kpercentile-measurement-value> element set to the kpercentile measurement value of measurement results;

8)	a <measurement-period> element set to the measurement period in seconds;

9)	a <timestamp> element set to the timestamp in date and time of the measurement results with an offset from the UTC time; and

10)	an <anyExt> element containing a <non-3gpp-access-measurement-list>, an optional element set to the measurements of the non-3GPP access. The <non-3gpp-access-measurement-list> element includes one or more <non-3gpp-access-measurement> elements containing:

i)	a <measured-non-3gpp-access>, an optional element representing identity of the measured non-3GPP access. The <measured-non-3gpp-access> element shall include:

-	an <ssid> element set to the SSID of the access point to which the UE is attached, as specified in IEEE Std 802.11-2012 [24]; or

-	a <bssid> element set to the BSSID of the access point to which the UE is attached, as specified in IEEE Std 802.11-2012 [24]; and

ii)	a <signal-strength-values>, a mandatory element containing one or more <measured-value> elements set to signal strength value (e.g., RSSI) for the measured non-3GPP access.

<identity-measurements> element contains one of following sub-elements:

a)	<VAL-ue-id-list>, an optional element that contains one or more <VAL-ue-id> elements. Each <VAL-ue-id> element contains the identity of the VAL UE for whom SEALDD measurement applies. For multiple VAL UEs reporting granularity set to individual UE, the associated measurement values are for individual VAL UE; or

b)	<VAL-group-id>, an optional element specifying the identity of the VAL group for whom SEALDD measurement applies for which the associated measurement values are aggregation for all VAL UEs or the VAL UE group.

<tx-quality-management-req> element contains the following sub-elements:

a)	<sealdd-flow-id>, a mandatory element specifying the identity of the seal flow;

b)	<tx-quality-management-action>, a mandatory element set to the data transmission quality "Redundant transmission path", "Re-establish transmission path", "Switch to backup transmission path" or optimization action "Back to single transmission path", "Transmission parameter adjustment" that was triggered by an event (e.g. measurement threshold); and

NOTE:	The strings allowed in <tx-quality-management-action> are case sensitive.

c)	<anyExt>, an optional element that contains:

1)	<bat-offset-ul>, an optional element specifying the BAT offset of the arrival time of the data burst in units of milliseconds for the uplink data and shall be defined as a positive integer; and

2)	<periodicity-ul>, an optional element specifying the adjusted periodicity of the data bursts in units of milliseconds for the uplink data and shall be defined as a positive integer.

<tx-quality-management-rsp> element contains the following sub-element:

a)	<result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation.

<connection-status-notification> element contains the following sub-element:

a)	<client-connection-status>, a mandatory element indicating the status of VAL UEs, set to "reachable", "unreachable", or "sleeping";

b)	<access-usage>, a mandatory element indicating which access i.e., 3GPP or non-3GPP, is used for the SEALDD-UU data transmission; and

c)	<non-3gpp-access-measurement-list>, an optional element set to the measurements of the non-3GPP access which includes one or more <non-3gpp-access-measurement> elements containing:

1)	<measured-non-3gpp-access>, an optional element set to the identity of the measured non-3GPP access The <measured-non-3gpp-access> element shall include:

i)	an <ssid> element set to the SSID of the access point to which the UE is attached, as specified in IEEE Std 802.11-2012 [24]; or

ii)	a <bssid> element set to the BSSID of the access point to which the UE is attached, as specified in IEEE Std 802.11-2012 [24]; and

2)	<signal-strength-values>, a mandatory element containing one or more:

i)	<measured-value>, a mandatory element set to the signal strength value (e.g., RSSI value) defined as an NonNegativeInteger with a recommended range of 0 to 127 for the measured non-3GPP access.

<connection-status-configuration-req> element contains the following sub-elements:

a)	<sealdd-flow-id>, a mandatory element specifying the identity of the seal flow;

b)	<reporting-mode>, an optional element set to either "PERIODIC" or "EVENT_TRIGGERED" indicating the mode of the reporting. If the mode is "PERIODIC", the <reporting-mode> element may contain a <reporting-interval> sub-element set to the reporting interval, specified in seconds, to the notification;

c)	<reporting-priority>, an optional element indicating the priority of SEALDD client connection status for the requested SEALDD flow ID (e.g., "LOW" or "HIGH" priority); and

d)	<non-3gpp-access-policy>, an optional element set to the non-3GPP access measurement policy, i.e. "WLAN SSID", "WLAN BSSID" or "LOCATION_BASED" measurement.

<connection-status-configuration-rsp> element contains the following sub-element:

a)	<result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation.

<xr-inform-req> element contains the following sub-elements:

a)	<requestor-id>, a mandatory element. This element contains a string set to either "sealddclient" used to specify the identity of the requestor being an SDDM-C;

b)	<VAL-ue-id-list>, a mandatory element that contains one or more <VAL-ue-id> elements. Each <VAL-ue-id> element contains the identity of the VAL UE for whom SEALDD multi-modal transmission connection inform applies;

c)	<status>, a mandatory element indicating the UE-to-UE direct communication status (i.e. established, released); and

d)	<VAL-service-id>, an optional element set to the VAL service identity of the vertical application.

<xr-inform-rsp> element contains the following sub-element:

a)	<result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation.

<xr-trigger-req> element contains the following sub-elements:

a)	<requestor-id>, a mandatory element. This element contains a string set to "sealddserver" used to specify the identity of the requestor being an SDDM-S;

b)	<VAL-ue-id-list>, a mandatory element that contains one or more <VAL-ue-id> elements. Each <VAL-ue-id> element contains the identity of the VAL UE for whom SEALDD multi-modal transmission connection trigger applies;

c)	<operation>, a mandatory element indicating the action for UE-to-UE direct communication (i.e. establishment, release); and

d)	<VAL-service-id>, an optional element set to the VAL service identity of the vertical application.

<xr-trigger-rsp> element contains the following sub-element:

a)	<result>, a mandatory element set to either "success" or "failure" indicating success or failure of the operation.

<XR-establishment-req> element contains the following sub-elements:

a)	<sealdd-client-identity>, a mandatory element. This element contains a string specifying the identity of SDDM-C;

b)	<sealdd-flows-info>, a mandatory element. This element contains a string specifying one or more <sealdd-flow-info> elements that contains the following sub-elements:

1)	a <sealdd-flow-id>, a mandatory element. This element contains a positive integer in range 1 to 65535 specifying the identity of the sealdd flow; and

2)	a <traffic-descriptor-info>, an optional element specifying the information of the traffic that contains the following sub-elements:

i)	a <user-plane-address> an optional element. This element contains a string set to user plane IP address used for the traffic;

ii)	a <port-number> an optional element. This element contains a positive integer in range 1 to 65535 set to the port number for the traffic;

iii)	a <URL> an optional element. This element contains a string set to a text format that specifies how to access the resource on the Internet for the traffic; or

iv)	a <transport-layer-protocol> an optional element. This element contains a string set to the transport protocol used for the traffic (e.g. TCP, UDP);

c)	<identity>, an optional element which includes <VAL-user-id> an optional element set to the identity of the VAL user or <VAL-ue-id> a string set to the identity of the SDDM-C acting as the VAL UE and performing the request or the SDDM-S that performs the request. <VAL-ue-id> contains either <sealURI> which is an anyURI, <sealString> a string, or <sealBoolean> a boolean sub-elements.

d)	<server-id>, an optional element. This element contains a string specifying the endpoint of a selected VAL server; and

e)	<VAL-service-id>, an optional element. This element contains a string specifying the VAL service identity of the vertical application.

<XR-establishment-rsp> element contains the following sub-elements:

a)	<result>, which includes a sub-element <operation-result>, a mandatory element contains a string set to either "success" or "failure" indicating success or failure of the operation. If the result is "failure", the <result> element may contain a <cause> sub-element which is a string set to the cause of the failure of the operation (e.g. VAL client error, SEALDD policy mismatch). If the result is "success", <XR-establishment-rsp> shall contain the <sealdd-multi-modal-flow-info> and may contain the <protocol-descriptor-info>;

b)	<sealdd-multi-modal-flow-info>, a mandatory element specifying the information of the multi-modal sealdd flow, that contains the following sub-elements:

1)	a <sealdd-flows-info> a mandatory element specifying one or more <sealdd-flow-info> elements, which contains the following sub-elements:

i)	<sealdd-flow-id>, a mandatory element. This element contains a positive integer in range 1 to 65535 specifying the identity of the sealdd flow; and

ii)	<traffic-descriptor-info>, an optional element specifying the information of the traffic that contains one or more of the following sub-elements:

-	a <user-plane-address> an optional element. This element contains a string set to user plane IP address used for the traffic;

-	a <port-number> an optional element. This element contains a positive integer in range 1 to 65535 set to the port number for the traffic;

-	a <URL> an optional element. This element contains a string set to a text format that specifies how to access the resource on the Internet for the traffic; or

-	a <transport-layer-protocol> an optional element. This element contains a string set to the transport protocol used for the traffic (e.g. TCP, UDP);

c)	<protocol-descriptor-info>, an optional element specifying the information of the protocol that contains one or more of the following sub-elements:

1)	a <header-ext-info> an optional element set to the header extension information used for the traffic;

i)	<header-ext-type>, an optional element. This element contains a string specifying the type of the header extension protocol (e.g, RTP, SRTP); and

ii)	<header-ext-id>, an optional element. This element contains a positive integer in range 1 to 255 specifying the identity of the header extension protocol;

2)	a <packetization-indication> an optional element. This element contains a boolean set to the packetization indication for the traffic;

3)	a <payload-type> an opional element. This element contains a string set to the type of the payload (e.g, RTP, SRTP); and

4)	a <payload-format> a mandatory element. This element contains a string set to the format of the payload (e.g, H.264, H.265).

Editor's note [WID: XRM_Ph2_App, CR#: 0094]:	The definition of multimodalSealddFlowId is FFS.

## 8.6	MIME type

The MIME type for the DataDeliveryInfo document shall be "application/vnd.3gpp.seal-data-delivery-info+xml".

## 8.7	IANA registration template

Your Name:

<TS rapporteur name>

Your Email Address:

<TS rapporteur email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.seal-data-delivery-info+xml

Required parameters:

# None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in HTTP. Hence, the security considerations from IETF RFC 2616 apply while exchanging information in HTTP.

The information transported in this media type does not include active or executable content.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.543 "Data Delivery Management - Service Enabler Architecture Layer for Verticals (SEAL)", available via https://www.3gpp.org/ftp/Specs/archive/24_series/24.543.

Applications which use this media type:

Applications supporting the SEAL Data delivery management as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

Annex A (normative):
CoAP resource representation and encoding

# A.1	General

The information in this annex provides a normative description of CoAP resource representation and encoding.

The general rules for resource URI structure, cache usage, error handling, and common data types are described in clause C.1 of 3GPP TS 24.546 [6].

# A.2	Data types applicable to multiple resource representations

## A.2.1	General

This clause defines structured data types, simple data types, and enumerations that are applicable to several APIs defined for CoAP resource representations in the present specification.

## A.2.2	Referenced structured data types

Table A.2.2.1 lists structured data types referenced by multiple CoAP resource representations and defined in other specifications.

Table A.2.2.1: Referenced structured data types

| Data type | Reference | Description |
| --- | --- | --- |
| ValTargetUe | 3GPP TS 24.546 [6] | Information identifying a VAL user ID or VAL UE ID. |
| GeographicalCoordinates | 3GPP TS 24.546 [6] | Information identifying geographical coordinates. |
| GeographicArea | 3GPP TS 24.546 [6] | Information identifying a geographical area. |

## A.2.3	Referenced simple data types

Table A.2.3.1 lists simple datatypes referenced by multiple CoAP resource representations and defined in other specifications.

Table A.2.3.1: Referenced simple data types

| Type name | Reference | Description |
| --- | --- | --- |
| Uinteger | 3GPP TS 24.546 [6] | Unsigned integer, i.e. only value 0 and values above 0 are permissible. |
| TimeOfDay | 3GPP TS 24.546 [6] | String with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339 [11]. |
| DateTime | 3GPP TS 24.546 [6] | String in the standard format described by the "date-time" production in IETF RFC3339 [11]. |

## A.2.4	Common structured data types

### A.2.4.1	Type: EstablishmentResponse

Table A.2.4.1.1: Definition of type EstablishmentResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the establishment request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the establishment request (NOTE 1). |  |
| userPlaneAddress | string | O | 0..1 | Identity of the IP address of the traffic (NOTE 2). |  |
| portNumber | Uinteger | O | 0..1 | Identity of the port number of the traffic (NOTE 2). |  |
| url | string | O | 0..1 | Identity of the address of a given unique resource on the Web for the traffic (NOTE 2). |  |
| batPeriodAdaptCap | boolean | O | 0..1 | Indicates a BAT and periodicity adaptation capability of the SEALDD client side. (NOTE 3, NOTE 4) |  |
| transmisAssistInfo | TransmissionAssistInfo | O | 0..1 | Indicates a transmission assistance information for uplink SEALDD traffic. It includes a burst arrival time (BAT), BAT time window, periodicity, and periodicity range. (NOTE 3, NOTE 4) |  |
| transportLayer | string | O | 0..1 | Identity of the transport layer protocol for the traffic (NOTE 2). |  |
| NOTE 1: This attribute shall be included if result is set to "FAILURE". NOTE 2: This attribute may be included if result is set to "SUCCESS". NOTE 3: This attribute may only be included in the response to the server side initiated request. NOTE 4: The "batPeriodAdaptCap" attribute and "transmisAssistInfo" attribute are mutually exclusive. |  |  |  |  |  |

### A.2.4.2	Type: EstablishmentRequest

Table A.2.4.2.1: Definition of type EstablishmentRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | RequestorId | M | 1 | Identity of the requestor of the establishment request (NOTE 1). |  |
| sealddflowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |
| endpointId | string | M | 1 | Identity of the endpoint of the selected VAL server to which the establishment request has to be sent. |  |
| sealddCommunicationLifetime | Uinteger | O | 0..1 | Information of the data delivery communication lifetime in milliseconds (NOTE 2). |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service enabled by the SDD regular transmission connection. |  |
| userPlaneAddress | string | O | 0..1 | Identity of the IP address of the traffic. |  |
| portNumber | Uinteger | O | 0..1 | Identity of the port number of the traffic. |  |
| url | string | O | 0..1 | Identity of the address of a given unique resource on the Web for the traffic. |  |
| transportLayer | string | O | 0..1 | Identity of the transport layer protocol for the traffic. |  |
| valTgtUe | ValTargetUe | O | 0..1 | VAL user to whom the establishment request is applied. |  |
| l4sFeedbackCapability | L4sFeedbackCapability | O | 0..1 | Information of the L4S feedback capability (i.e. ECN identification, L4S feedback). |  |
| batPeriodAdaptCap | boolean | O | 0..1 | Indicates a BAT and periodicity adaptation capability of the SEALDD client side. (NOTE 3, NOTE 4) |  |
| transmisAssistInfo | TransmissionAssistInfo | O | 0..1 | Indicates a transmission assistance information for uplink SEALDD traffic. It includes a burst arrival time (BAT), BAT time window, periodicity, and periodicity range. (NOTE 3, NOTE 4) |  |
| xrAppDevCap | XrAppDevCapability | O | 0..1 | Indicates an XR application device capability information i.e. media capabilities and display capabilities. |  |
| NOTE 1: Thie requestorId attribute shall be set to either "SEALDDSERVER" if the requesting entity is the SDDM-S or."SEALDDCLIENT" if the requesting entity is the SDDM-C. NOTE 2: The sealddCommunicationLifetime attribute shall be included when the requesting entity is the SDDM-S. This attribute shall be included when the requesting entity is the SDDM-C. NOTE 3: This attribute may only be included in the SDDM-C initiated request. NOTE 4: The "batPeriodAdaptCap" attribute and "transmisAssistInfo" attribute are mutually exclusive. |  |  |  |  |  |

### A.2.4.3	Type: URLLCEstablishmentRequest

Table A.2.4.3.1: Definition of type URLLCEstablishmentRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealClientId | string | M | 1 | Identity of the SDDM-C of the URLLC establishment request. |  |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and the SDDM-S to identify the application traffic. |  |
| valTgtUe | ValTargetUe | M | 1 | VAL user to whom the establishment request is applied. |  |
| serverId | ServerId | M | 1 | Identity of the VAL server. |  |
| valServiceId | string | M | 0..1 | Identity of the VAL service enabled by the URLLC transmission connection. |  |
| userPlaneAddress | string | O | 0..1 | Identity of the IP address of the traffic. |  |
| portNumber | Uinteger | O | 0..1 | Identity of the port number of the traffic. |  |
| url | string | O | 0..1 | Identity of the address of a given unique resource on the Web for the traffic. |  |
| transportLayer | string | O | 0..1 | Identity of the transport layer protocol for the traffic. |  |
| batPeriodAdaptCap | boolean | O | 0..1 | Indicates a BAT and periodicity adaptation capability of the SEALDD client side. (NOTE 1, NOTE 2) |  |
| transmisAssistInfo | TransmissionAssistInfo | O | 0..1 | Indicates a transmission assistance information for uplink SEALDD traffic. It includes a burst arrival time (BAT), BAT time window, periodicity, and periodicity range. (NOTE 1, NOTE 2) |  |
| NOTE 1: This attribute may only be included in the SDDM-C initiated request. NOTE 2: The "batPeriodAdaptCap" attribute and "transmisAssistInfo" attribute are mutually exclusive. |  |  |  |  |  |

### A.2.4.4	Type: URLLCEstablishmentResponse

Table A.2.4.4.1: Definition of type URLLCEstablishmentResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the establishment request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the establishment request (NOTE 1). |  |
| userPlaneAddress | string | O | 0..1 | Identity of the IP address of the traffic (NOTE 2). |  |
| portNumber | Uinteger | O | 0..1 | Identity of the port number of the traffic (NOTE 2). |  |
| url | string | O | 0..1 | Identity of the address of a given unique resource on the Web for the traffic (NOTE 2). |  |
| transportLayer | string | O | 0..1 | Identity of the transport layer protocol for the traffic (NOTE 2). |  |
| batPeriodAdaptCap | boolean | O | 0..1 | Indicates a BAT and periodicity adaptation capability of the SEALDD client side. (NOTE 2, NOTE 3, NOTE 4) |  |
| transmisAssistInfo | TransmissionAssistInfo | O | 0..1 | Indicates a transmission assistance information for uplink SEALDD traffic. It includes a burst arrival time (BAT), BAT time window, periodicity, and periodicity range. (NOTE 2, NOTE 3, NOTE 4) |  |
| NOTE 1: This attribute shall be included if result is set to "failure". NOTE 2: This attribute may be included if result is set to "success". NOTE 3: This attribute may only be included in the response to the server side initiated request. NOTE 4: The "batPeriodAdaptCap" attribute and "transmisAssistInfo" attribute are mutually exclusive. |  |  |  |  |  |

### A.2.4.5	Type: URLLCReleaseRequest

Table A.2.4.5.1: Definition of type URLLCReleaseRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealClientId | string | M | 1 | Identity of the requestor of the establishment request. |  |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and the SDDM-S to identify the application traffic. |  |

### A.2.4.6	Type: TransmissionAssistInfo

Table A.2.4.6.1: Definition of type TransmissionAssistInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| bat | DateTime | O | 0..1 | Indicates the arrival time of the first packet of the data burst. |  |
| periodicity | Uinteger | O | 0..1 | Indicates the time period between the start of two bursts in units of microseconds. Unsigned 64-bit integer identifies a period of time in units of microseconds, i.e. 0 to (2^64)-1. Minimum = 0. Maximum = 18446744073709551615. |  |
| batWindow | TimeWindow | O | 0..1 | Contains the acceptable earliest and latest arrival time of the first packet of the data burst. The start time contains the earliest arrival time, and the stop time contains the latest arrival time. (NOTE 1) |  |
| periodRange | PeriodicityRange | O | 0..1 | Contains the acceptable time period range between the start of two bursts or the acceptable periodicity value(s). (NOTE 2) |  |
| NOTE 1: This attribute may only be included if the "bat" attribute is included. NOTE 2: This attribute may only be included if the "bat", "periodicity" and "batWindow" attributes are included. |  |  |  |  |  |

### A.2.4.7	Type: PeriodicityRange

Table A.2.4.7.1: Definition of type PeriodicityRange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lowerBound | Uinteger | C | 0..1 | Indicates the lower bound of the periodicity of the start two bursts in units of microseconds. Unsigned 64-bit integer identifies a period of time in units of microseconds, i.e. 0 to (2^64)-1. Minimum = 0. Maximum = 18446744073709551615. (NOTE) |  |
| upperBound | Uinteger | C | 0..1 | Indicates the upper bound of the periodicity of the start two bursts in units of microseconds. Unsigned 64-bit integer identifies a period of time in units of microseconds, i.e. 0 to (2^64)-1. Minimum = 0. Maximum = 18446744073709551615. (NOTE) |  |
| periodicityValues | array(Uinteger) | C | 1..N | Contains the acceptable periodicity values in units of microseconds. (NOTE) |  |
| NOTE: Either the "periodicityValues" attribute or both the "lowerBound" and "upperBound" attributes shall be included. |  |  |  |  |  |

### A.2.4.8	Type: TimeWindow

Table A.2.4.8.1: Definition of type TimeWindow

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| startTime | DateTime | M | 1 | Indicates the start time of the time window. |  |
| stopTime | DateTime | M | 1 | Indicates the stop time of the time window. |  |

### A.2.4.9	Type: XrAppDevCapability

Table A.2.4.9.1: Definition of type XrAppDevCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| xrMediaCap | XrMediaCap | M | 1 | Contains media capabilities (e.g. media resolution, media codec, media frame rate, media projection, media decoding time, mono vs. stereo 360 video) |  |
| xrDisplayCap | XrDisplayCap | M | 1 | Contains display capabilities (e.g. display resolution, max display refresh rate, horizontal and vertical field of view, eye to screen distance, lens separation distance). |  |

Editor's note [WID: SEALDD_Ph2, CR#: 0053]:	Specification of XrMediaCap and XrDisplayCap data types is FFS.

### A.2.4.10	Type: Non3gppAccessMeasurement

Table A.2.4.10.1: Definition of type Non3gppAccessMeasurement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measuredAccess | MeasuredNon3gppAccess | O | 0..1 | Identity of the measured non-3GPP access i.e. WLAN SSID or WLAN BSSID. (NOTE) |  |
| signalStrengthValues | array(Uinteger) | M | 1..N | List of signal strength values (e.g., RSSI) for the measured non-3GPP access. |  |
| NOTE: This attribute is not present when the non-3GPP access measurement policy indicates "LOCATION_BASED" measurement. |  |  |  |  |  |

### A.2.4.11	Type: MeasuredNon3gppAccess

Table A.2.4.11.2: Definition of type MeasuredNon3gppAccess

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ssId | string | O | 0..1 | Contains the SSID of the access point to which the UE is attached, as specified in IEEE Std 802.11-2012 [24]. |  |
| bssId | string | O | 0..1 | Contains the BSSID of the access point to which the UE is attached, as specified in IEEE Std 802.11-2012 [24]. |  |

### A.2.4.12	Type: L4sFeedbackCapability

Editor's note [WID: SEALDD_Ph2, CR#: 0085]:	The data type L4sFeedbackCapability to support L4S mechanism will be checked to be aligned with the latest stage-2 requirements.

Table A.2.4.12.1: Definition of type L4sFeedbackCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecnMarking | EcnMarking | M | 1 | Contais the ECN marking. |  |
| l4sFeedback | L4sFeedback | M | 1 | Contains the L4S feedback. |  |

### A.2.4.13	Type: EcnMarking

Table A.2.4.13.1: Definition of type EcnMarking

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supportsEct0 | boolean | M | 1 | Indicates the support of ECT0. |  |
| ect0Value | string | O | 0..1 | Indicates the value of ECT0 when non-standard values are used (NOTE). |  |
| supportsEct1 | boolean | M | 1 | Indicates the support of ECT1. |  |
| ect1Value | string | O | 0..1 | Indicates the value of ECT1 when non-standard values are used (NOTE). |  |
| NOTE: This attribute may only be included if the corresponding support flag indicates the value "1" (true). |  |  |  |  |  |

### A.2.4.14	Type: L4sFeedback

Table A.2.4.14.1: Definition of type L4sFeedback

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| perPacketCeReporting | boolean | M | 1 | Indicates that the entity can report congestion-experienced (CE) indications on a per-packet basis. |  |
| feedbackInterval | Uinteger | O | 0..1 | Indicates the time between feedback transmissions (how often they can report without overwhelming control channels) (NOTE). |  |
| maxFeedbackFrequency | Uinteger | O | 0..1 | Indicates the fastest rate at which feedback may be sent (NOTE). |  |
| protocol | string | O | 0..1 | Indicates the transport protocol (e.g. "RTCP", "in-band") (NOTE). |  |
| NOTE: This attribute may only be included if the "perPacketCeReporting" attribute indicates the value "1" (true). |  |  |  |  |  |

## A.2.5	Common simple data types

Table A.2.5.1: Simple data types applicable to multiple CoAP resource representations

| Type name | Description |
| --- | --- |
| ServerId | String representing a unique identifier of a VAL server. |

## A.2.6	Common enumerations

### A.2.6.1	Enumeration: RequestorId

Table A.2.6.1.1: RequestorId

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SEALDDCLIENT | SEALDD client is the requestor. |  |
| SEALDDSERVER | SEALDD server is the requestor. |  |

### A.2.6.2	Enumeration: ResultOp

Table A.2.6.2.1: ResultOp

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESS | Success of the operation. |  |
| FAILURE | Failure of the operation. |  |

### A.2.6.3	Enumeration: Cause

Table A.2.6.3.1: Cause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| VAL CLIENT ERROR | A VAL client error occurs. |  |
| SEALDD POLICY MISMATCH | A SEALDD policy mismatch occurs. |  |
| OTHER | Any other cause occurs than the ones defined in this table. |  |

### A.2.6.4	Enumeration: Non3gppAccessPolicy

Table A.2.6.4.1: Non3gppAccessPolicy

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| WLAN SSID | The non-3GPP access measurement policy is SSID based. |  |
| WLAN BSSID | The non-3GPP access measurement policy is BSSID based. |  |
| LOCATION_BASED | Indicates the location-based non-3GPP access measurement policy. |  |

# A.3	Resource representation and APIs provided by SDDM-S

## A.3.1	Sdd_RegularTransmissionConnection API

### A.3.1.1	API URI

The CoAP URIs used in CoAP requests from SDDM-S towards the SDMM-C shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-s";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.3.1.2.

### A.3.1.2	Resources

#### A.3.1.2.1	Overview

Figure A.3.1.2.1.1: Resource URI structure of the Sdd_RegularTransmissionConnection API provided by SDDM-S

Table A.3.1.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.3.1.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD Regular Transmission Connection | val-services/{valServiceId}/sdd-regular-transmission-connection | POST | Establish an SDDM regular transmission connection. |
|  |  | DELETE | Release an SDDM regular transmission connection |

#### A.3.1.2.2	Resource: SDD Regular Transmission Connection

##### A.3.1.2.2.1	Description

The SDD regular transmission connection resource represents an SDD regular transmission connection to be created at a given SDDM-S and SDDM-C.

##### A.3.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-s/<apiVersion>/val-services/{valServiceId}/sdd-regular-transmission-connection

This resource shall support the resource URI variables defined in the table A.3.1.2.2.2.1.

Table A.3.1.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.3.1.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.3.1.2.2.3	Resource Standard Methods

A.3.1.2.2.3.1	POST

This operation allows to establish an SDDM regular transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.3.1.2.2.3.1.1 and A.3.1.2.2.3.1.2.

Table A.3.1.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EstablishmentRequest | M | 1 | The information of request of establishment of an SDDM regular transmission connection. |

Table A.3.1.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EstablishmentResponse | M | 1 | 2.01 Created | SDDM regular transmission connection created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

A.3.1.2.2.3.2	DELETE

This operation releases an SDDM regular transmission connection.

This method shall support the request data structures the data structure, request codes and response codes specified in table A.3.1.2.2.3.2.1 and A.3.1.2.2.3.2.2.

Table A.3.1.2.2.3.2.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ReleaseRequest | M | 1 | The information of request of release of an SDDM regular transmission connection. |

Table A.3.1.2.2.3.2.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | SDDM regular transmission connection released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

### A.3.1.3	Data Model

#### A.3.1.3.1	General

Table A.3.1.3.1.1 specifies the data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.1: SDD_RegularTransmissionConnection API provided by SDDM-S specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| EstablishmentResponse | A.2.4.1 | Information identifying an SDD regular transmission connection establishment response. |  |
| EstablishmentRequest | A.2.4.2.1 | Information identifying an SDD regular transmission connection establishment request. |  |
| PeriodicityRange | A.2.4.7 | Contains the acceptable periodicity range or periodicity value(s). |  |
| ReleaseRequest | A.3.1.3.2.3 | Information identifying an SDD regular transmission connection release request. |  |
| TimeWindow | A.2.4.8 | Contains start time and stop time. |  |
| TransmissionAssistInfo | A.2.4.6 | Contains transmission assistance information for uplink SEALDD traffic. |  |
| L4sFeedbackCapability | A.2.4.12 | Information identifying the L4S feedback capability (i.e. ECN identification, L4S feedback). |  |

Table A.3.1.3.1.2 specifies the simple data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.2: SDD_RegularTransmissionConnection API provided by SDDM-S specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |
| ServerId | A.2.5 | String representing a unique identifier of a VAL server. |

Table A.3.1.3.1.3 specifies the enumerations defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.3: SDD_RegularTransmissionConnection API provided by SDDM-S specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| RequestorId | A.2.6.1 | Information identifying a VAL user ID or VAL UE ID. |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |

#### A.3.1.3.2	Structured data types

##### A.3.1.3.2.1	Void

##### A.3.1.3.2.2	Type: ReleaseRequest

Table A.3.1.3.2.2.1: Definition of type ReleaseRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serverId | ServerId | M | 1 | Identity of the VAL server. |  |
| sealddFflowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |

##### A.3.1.3.2.3	Void

##### A.3.1.3.2.4	Void

##### A.3.1.3.2.5	Void

#### A.3.1.3.3	Simple data types and enumerations

None.

### A.3.1.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.3.1.5	CDDL Specification

#### A.3.1.5.1	Introduction

The data model described in clause A.3.1.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.3.1.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the SDD_RegularTransmissionConnection API provided by the SDDM-S data model.

#### A.3.1.5.2	CDDL document

;;; EstablishmentRequest

;;+ Represents a request for establishing an SDDM regular transmission connection.

EstablishmentRequest = {

requestorId: RequestorId

sealddFlowId: Uinteger

serverId: ServerId

endpointId: string

? sealddCommunicationLifetime: Uinteger

? valServiceId: string

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

? valTgtUe: ValTargetUe

? l4sFeedbackCapability: L4sFeedbackCapability

}

;;; EstablishmentResponse

;;+ Represents a response of establishing an SDDM regular transmission connection.

EstablishmentResponse = {

result: ResultOp

? cause: Cause

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

.?.batAndPeriodicityCapability: boolean

.?.transmisAssistInfo: TransmissionAssistInfo

? transportLayer: string

}

;;; ReleaseRequest

;;+ Represents a request for releasing an SDDM regular transmission connection.

ReleaseRequest = {

serverId: ServerId

sealddFlowId: Uinteger

}

;;; RequestorId

;;+ Indicates requestor of an operation.

RequestorId = "SEALDDCLIENT" / "SEALDDSERVER"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; ServerId

;;+ Represents information identifying a unique server.

serverId = text

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; TransmissionAssistInfo

;;+ Indicates a transmission assistance information for uplink SEALDD traffic.

TransmissionAssistInfo = {

? bat: DateTime

? periodicity: Uinteger

? batWindow: TimeWindow

? periodRange: PeriodicityRange

}

;;; TimeWindow

;;+ Indicates the acceptable earliest and latest arrival time of the first packet of the data burst. The start time contains the earliest acceptable arrival time, and the stop time contains the latest acceptable arrival time.

TransmissionAssistInfo = {

? startTime: DateTime

? stopTime: DateTime

}

;;; PeriodicityRange

;;+ Indicates the acceptable time period range between the start of two bursts or the acceptable periodicity value(s).

PeriodicityRange = {

? lowerBound: Uinteger

? upperBound: Uinteger

? periodicityValues: [* Uinteger]

}

;;; L4sFeedbackCapability

;;+ Indicates the L4S feedback capability (i.e. ECN identification, L4S feedback).

L4sFeedbackCapability = {

ecnMarking: EcnMarking

l4sFeeedback: L4sFeedback

}

;;; EcnMarking

;;+ Indicates the ECN marking.

EcnMarking = {

supportsEct0: boolean

? ect0Value: string

supportsEct1: boolean

? ect1Value: string

}

;;; L4sFeedback

;;+ Indicates the L4S feedback.

L4sFeedback = {

perPacketCeReporting: boolean

? feedbackInterval: Uinteger      ; How often feedback is sent.

? maxFeedbackFrequency: Uinteger  ; The fastest rate at which feedback may be sent.

? protocol: string                ; Transport protocol e.g. "RTCP", "in-band".

}

### A.3.1.6	Media Types

See clause A.5.

### A.3.1.7	Void

### A.3.1.8	Void

### A.3.1.9	Void

### A.3.1.10	Void

### A.3.1.11	Void

## A.3.2	Sdd_TransmissionQualityMeasurement API

### A.3.2.1	API URI

The CoAP URIs used in CoAP requests from SDDM-S towards the SDMM-C shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-s";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.3.2.2.

### A.3.2.2	Resources

#### A.3.2.2.1	Overview

Figure A.3.2.2.1.1: Resource URI structure of the Sdd_TransmissionQualityMeasurement API provided by SDDM-S

Table A.3.2.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.3.2.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD Transmission Quality Measurement | val-services/{valServiceId}/sdd-transmission-quality-measurement | POST | Establish an SDDM data transmission quality measurement. |
|  |  | FETCH | Observe SDDM data transmission quality measurement of the SDDM-C. |
|  |  | DELETE | Releases an SDDM data transmission quality measurement. |

#### A.3.2.2.2	Resource: SDD Transmission Quality Measurement

##### A.3.2.2.2.1	Description

The SDD transmission quality measurement resource allows an SDDM-C to manage an SDDM data transmission quality measurement of an SDDM-S.

##### A.3.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-s/<apiVersion>/val-services/{valServiceId}/sdd--transmission-quality-measurement

This resource shall support the resource URI variables defined in the table A.3.2.2.2.2.1.

Table A.3.2.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.3.2.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.3.2.2.2.3	Resource Standard Methods

###### A.3.2.2.2.3.1	POST

This operation allows to establish an SDDM data transmission quality measurement.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.3.2.2.2.3.1.1 and A.3.2.2.2.3.1.2.

Table A.3.2.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MeasurementsSubscriptionRequest | M | 1 | The information of request of establishment of an SDDM data transmission quality measurement. |

Table A.3.2.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MeasurementsSubscriptionResponse | M | 1 | 2.01 Created | SDDM data transmission quality measurement created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

###### A.3.2.2.2.3.2	FETCH

This operation updates an SDDM data transmission quality measurement.

This method shall support the data structures, request codes and response codes specified in table A.3.2.2.2.3.2.0, A.3.2.2.2.3.2.1 and A.3.2.2.2.3.2.2.

Table A.3.2.2.2.3.2.0: Options supported by the FETCH Request on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| observe | Uinteger | O | 0..1 | When set to 0 (register) it extends the FETCH request to subscribe to the changes of this resource. When set to 1 (deregister) it cancels the subscription. |
| NOTE: Other request options also apply in accordance with normal CoAP procedures. |  |  |  |  |

Table A.3.2.2.2.3.2.1: Data structures supported by the FETCH Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MeasurementsSubscriptionReques | M | 1 | The identifier of an SDDM regular transmission connection to which SDDM datatransmission quality measurement are going to be performed. |

Table A.3.2.2.2.3.2.2: Data structures supported by the FETCH Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
|  | M | 1 | 2.05 Content | The information of SDDM data transmission quality measurement on the request from the SDDM-S. |
| NOTE: The mandatory CoAP error status codes for the PUT method listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

###### A.3.2.2.2.3.3	DELETE

This operation releases an SDDM data transmission quality measurement.

This method shall support the request data structures the data structure, request codes and response codes specified in table A.3.2.2.2.3.3.1 and A.3.2.2.2.3.3.2.

Table A.3.2.2.2.3.3.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The information of request of release of an SDDM data transmission quality measurement. |

Table A.3.2.2.2.3.3.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | SDDM data transmission quality measurement released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

### A.3.2.3	Data Model

#### A.3.2.3.1	General

Table A.3.2.3.1.1 specifies the data types defined specifically for the SDD_TransmissionQualityMeasurement API service provided by SDDM-S.

Table A.3.2.3.1.1: SDD_TransmissionQualityMeasurement API provided by SDDM-S specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| GeographicArea | A.2.2 | Information identifying a geographical area. |  |
| GeographicalCoordinates | A.2.2 | Information identifying geographical coordinates. |  |
| MeasurementsSubscriptionRequest | A.3.2.3.2.1 | Information identifying an SDD data transmission quality measurement subscription establishment request. |  |
| MeasurementsSubscriptionResponse | A.3.2.3.2.2 | Information identifying an SDD data transmission quality measurement subscription establishment response. |  |
| MeasurementNotification | A.3.2.3.2.3 | Information identifying an SDD data transmission quality measurement notification. |  |
| ReportingCriteria | A.3.2.3.2.4 | Information of the criteria for reporting measurement results. |  |
| MeasurementPeriod | A.3.2.3.2.5 | Information of the measurement period. |  |
| SpatialConditions | A.3.2.3.2.6 | Information of the spatial conditions. |  |
| MeasuredNon3gppAccess | A.2.4.11 | Indicates identity of the measured non-3GPP access. |  |
| Non3gppAccessMeasurement | A.2.4.10 | Indicates the non-3GPP access measurement information report. |  |
| Non3gppAccessPolicy | A.2.6.4 | Indicates the non-3GPP access measurement policy. |  |

Table A.3.2.3.1.2 specifies the simple data types defined specifically for the SDD_TransmissionQualityMeasurement API service provided by SDDM-S.

Table A.3.2.3.1.2: SDD_TransmissionQualityMeasurement API provided by SDDM-S specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |

Table A.3.2.3.1.3 specifies the enumerations defined specifically for the SDD_TransmissionQualityMeasurement API service provided by SDDM-S.

Table A.3.2.3.1.3: SDD_TransmissionQualityMeasurement API provided by SDDM-S specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |

#### A.3.2.3.2	Structured data types

##### A.3.2.3.2.1	Type: MeasurementsSubscriptionRequest

Table A.3.2.3.2.1.1: Definition of type MeasurementsSubscriptionRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |
| measurementId | string | M | 1 | Identity of the measurement to be performed which is set to "LATENCY", "BITRATE", "JITTER" or "PACKET LOSS". |  |
| reportingFrequency | string | O | 0..1 | Information of the reporting frequency of measurement results which is set to "PERIODIC" or "NOW". |  |
| reportingPeriodicity | Uinteger | O | 0..1 | Identity of the reporting periodicity of measurement results in seconds (NOTE). |  |
| measurementWindow | Uinteger | O | 0..1 | Identity of the measurement period window for transmission quality measurements in milliseconds. |  |
| expiryTimer | Uinteger | O | 0..1 | Identity of the expiration time of the measurement in milliseconds. |  |
| sealPolicy | string | O | 0..1 | Information of the quality guarantee policies associated with the SEALDD connection set to the action to be performed "REDUNDANT TRANSMISSION PATH", "RE-ESTABLISH TRANSMISSION PATH", "SWITCH TO BACKUP TRANSMISSION PATH" when the measurement event occurs. |  |
| reportingCriteria | array(ReportingCriteria) | O | 0..N | Information of the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criterion of more than one criteria is specified. |  |
| measurementConditions | MeasurementConditions | O | 0..1 | Information of the temporal conditions, spatial conditions or both. |  |
| valTgtUe | ValTargetUe | O | 0..1 | VAL user to whom the subscription request is applied. |  |
| non3gppAccessPolicy | Non3gppAccessPolicy | O | 0..1 | Indicates the non-3GPP access measurement policy, i.e., "WLAN SSID", "WLAN BSSID" or "LOCATION_BASED" measurement. |  |
| NOTE: This attribute shall be included if reportingFrequency is set to "PERIODIC". |  |  |  |  |  |

##### A.3.2.3.2.2	Type: MeasurementsSubscriptionResponse

Table A.3.2.3.2.2.1: Definition of type MeasurementsSubscriptionResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the establishment request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the establishment request (NOTE 1). |  |
| expiryTime | DateTime | O | 0..1 | Information of the expiration time of the subscription (NOTE 2). |  |
| NOTE 1: This attribute shall be included if result is set to "FAILURE". NOTE 2: This attribute may be included if result is set to "SUCCESS". |  |  |  |  |  |

##### A.3.2.3.2.3	Type: MeasurementNotification

Table A.3.2.3.2.3.1: Definition of type MeasurementNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |
| measurementId | string | M | 1 | Identity of the measurement performed which is set to "LATENCY", "BITRATE", "JITTER" or "PACKET LOSS". |  |
| valUeIdList | array(ValTargetUe) | O | 0..N | Information of the identities of the VAL UEs or VAL users for whom SEALDD measurement applies. This attribute can be omitted and the associated measurement values are for the single VAL UE (NOTE). |  |
| averageMeasurementValue | Uinteger | O | 0..1 | Information of average measurement value of measurement results. If measurementId is "LATENCY" or "JITTER", the value is in milliseconds. If measurementId is "BITRATE", the value is in Mbps. If the measurementId is "PACKET LOSS", the value is in percentage of the number of packets that fail to reach their destination. |  |
| maximumMeasurementValue | Uinteger | O | 0..1 | Information of the maximum measurement value of measurement results. If measurementId is "LATENCY" or "JITTER", the value is in milliseconds. If measurementId is "BITRATE", the value is in Mbps. If the measurementId is "PACKET LOSS", the value is in percentage of the number of packets that fail to reach their destination. |  |
| minimumMeasurementValue | Uinteger | O | 0..1 | Information of the minimum measurement value of measurement results. If measurementId is "LATENCY" or "JITTER", the value is in milliseconds. If measurementId is "BITRATE", the value is in Mbps. If the measurementId is "PACKET LOSS", the value is in percentage of the number of packets that fail to reach their destination. |  |
| standardDeviationMeasurementValue | Uinteger | O | 0..1 | Information of the standard deviation measurement value of measurement results. |  |
| kPercentileMeasurementValue | Uinteger | O | 0..1 | Information of the kpercentile measurement value of measurement results. |  |
| measurementPeriod | MeasurementPeriod | O | 0..1 | Information of the measurement period. |  |
| timeStamp | TimeOfDay | O | 0..1 | Information of the timestamp of measurement results. |  |
| non3gppAccessMeasurements | array(Non3gppAccessMeasurement) | O | 1..N | Information of the non-3GPP access measurement information report. |  |
| NOTE: This attribute can be omitted and the associated measurement values are for the single VAL UE. |  |  |  |  |  |

##### A.3.2.3.2.4	Type: ReportingCriteria

Table A.3.2.3.2.4.1: Definition of type ReportingCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| latency | LatencyValue | O | 0..1 | Identify whether the criterion for reporting measurements results is based on reaching above or below certain latency values (NOTE). |  |
| bitrate | BitrateValue | O | 0..1 | Identify whether the criterion for reporting measurements results is based on reaching above or below certain bitrate value (NOTE). |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

##### A.3.2.3.2.5	Type: LatencyValue

Table A.3.2.3.2.5.1: Definition of type LatencyValue

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| latencyThresholdValue | Uinteger | M | 1 | Information of the latency threshold value for reporting measurements results in milliseconds. |  |
| aboveOrBelow | boolean | M | 1 | Identify whether the criterion for reporting measurements results is based on reaching above certain latency value. Value "true" indicates that the criterion for reporting measurements results is based on reaching above the latency value indicated by the latencyThresholdValue attribute. Value "false" indicates that the criterion for reporting measurements results is based on reaching below the latency value indicated by the latencyThresholdValue attribute. |  |

##### A.3.2.3.2.6	Type: BitrateValue

Table A.3.2.3.2.6.1: Definition of type BitrateValue

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| bitrateThresholdValue | Uinteger | M | 1 | Information of the bitrate threshold value for reporting measurements results in Mbps. |  |
| aboveOrBelow | boolean | M | 1 | Identify whether the criterion for reporting measurements results is based on reaching above certain bitrate value. Value "true" indicates that the criterion for reporting measurements results is based on reaching above the bitrate value indicated by the bitrateThresholdValue attribute. Value "false" indicates that the criterion for reporting measurements results is based on reaching below the bitrate value indicated by the bitrateThresholdValue attribute. |  |

##### A.3.2.3.2.7	Type: MeasurementConditions

Table A.3.2.3.2.7.1: Definition of type MeasurementConditions

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| temporalConditions | MeasurementPeriod | O | 0..1 | Information of the temporal conditions set in time range (i.e. time-of-start and time-of-end) (NOTE). |  |
| spatialConditions | SpatialConditions | O | 0..1 | Information of the spatial conditions (i.e. geographical area, geographical coordinates or both) (NOTE). |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

##### A.3.2.3.2.8	Type: MeasurementPeriod

Table A.3.2.3.2.8.1: Definition of type MeasurementPeriod

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeStart | TimeOfDay | M | 1 | Information of the start of measurement period. |  |
| timeEnd | TimeOfDay | M | 1 | Information of the end of measurement period. |  |

##### A.3.2.3.2.9	Type: SpatialConditions

Table A.3.2.3.2.9.1: Definition of type SpatialConditions

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| geographicAreaList | array(GeographicArea) | O | 0..N | Information of specific geographical area list (NOTE). |  |
| geoCoordinatesList | array(GeographicalCoordinates) | O | 0..N | Information of specific geographical coordinates (NOTE). |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

#### A.3.2.3.3	Simple data types and enumerations

None.

### A.3.2.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.3.2.5	CDDL Specification

#### A.3.2.5.1	Introduction

The data model described in clause A.3.1.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.3.2.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the SDD_TransmissionQualityMeasurement API provided by the SDDM-S data model.

#### A.3.2.5.2	CDDL document

;;; MeasurementSubscriptionRequest

;;+ Represents a request for performing SDDM data transmission quality measurements.

MeasurementSubscriptionRequest = {

sealddFlowId: Uinteger

measurementId: string

? reportingFrequency: string

? reportingPeriodicity: Uinteger

? measurementWindow: Uinteger

? expiryTimer: Uinteger

? sealPolicy: string

? reportingCriteria: ReportingCriteria

? measurementConditions: MeasurementConditions

? valTgtUe: ValTargetUe

? non3gppAccessPolicy: Non3gppAccessPolicy

}

;;; MeasurementSubscriptionResponse

;;+ Represents the response of a request for performing SDDM data transmission quality measurements.

MeasurementSubscriptionResponse = {

result: ResultOp

? cause: Cause

? expiryTime: DateTime

}

;;; MeasurementNotification

;;+ Represents the information of SDDM data transmission quality measurements of the SDDM-C.

MeasurementNotification = {

sealddFlowId: Uinteger

measurementId: string

? valUeIdList: [* ValTargetUe]

? averageMeasurementValue: Uinteger

? maximumMeasurementValue: Uinteger

? minimumMeasurementValue: Uinteger

? standardDeviationMeasurementValue: Uinteger

? kPercentileMeasurementValue: Uinteger

? measurementPeriod: MeasurementPeriod

? timeStamp: TimeOfDay

? non3gppAccessMeasurements: [* Non3gppAccessMeasurement]

}

;;; ReportingCriteria

ReportingCriteria = {

? latency: LatencyValue

? bitrate: BitrateValue

}

;;; LatencyValue

LatencyValue = {

latencyThresholdValue: Uinteger

aboveOrBelow: bool

}

;;; BitrateValue

BitrateValue = {

bitrateThresholdValue: Uinteger

aboveOrBelow: bool

}

;;; MeasurementConditions

MeasurementConditions = {

? temporalConditions: MeasurementPeriod

? spatialConditions: SpatialConditions

}

MeasurementPeriod = {

timeStart: TimeOfDay

timeEnd: TimeOfDay

}

;;; SpatialConditons

SpatialConditions = {

? geographicAreaList: [* GeographicArea]

? geoCordinatesList: [*GeographicalCoordinates]

}

;;; TimeOfDay

;;+ String with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339. Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC).

TimeOfDay = text

;;; Non3gppAccessMeasurement

;;+ Information of the non-3GPP access measurement information report.

? measuredAccess: MeasuredNon3gppAccess

signalStrengthValues: [* Uinteger]

;;; MeasuredNon3gppAccess

;;+ Identity of the measured non-3GPP access i.e. WLAN SSID or WLAN BSSID.

? ssId: string

? bssId: string

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; GeographicArea

;;+ Geographic area specified by different shape.

GeographicArea = Point / PointUncertaintyCircle / PointUncertaintyEllipse / Polygon / PointAltitude / PointAltitudeUncertainty / EllipsoidArc

;;; GADShape

;;+ Common base type for GAD shapes.

GADShape = {

shape: SupportedGADShapes

}

;;; Point

;;+ Ellipsoid Point.

Point = {

~GADShape

point: GeographicalCoordinates

}

;;; PointUncertaintyCircle

;;+ Ellipsoid point with uncertainty circle.

PointUncertaintyCircle = {

~GADShape

point: GeographicalCoordinates

uncertainty: Uncertainty

}

;;; PointUncertaintyEllipse

;;+ Ellipsoid point with uncertainty ellipse.

PointUncertaintyEllipse = {

~GADShape

point: GeographicalCoordinates

uncertaintyEllipse: UncertaintyEllipse

confidence: Confidence

}

;;; Polygon

;;+ Polygon.

Polygon = {

~GADShape

pointList: PointList

}

;;; PointAltitude

;;+ Ellipsoid point with altitude.

PointAltitude = {

~GADShape

point: GeographicalCoordinates

altitude: Altitude

}

;;; PointAltitudeUncertainty

;;+ Ellipsoid point with altitude and uncertainty ellipsoid.

PointAltitudeUncertainty = {

~GADShape

point: GeographicalCoordinates

altitude: Altitude

uncertaintyEllipse: UncertaintyEllipse

uncertaintyAltitude: Uncertainty

confidence: Confidence

}

;;; EllipsoidArc

;;+ Ellipsoid Arc.

EllipsoidArc = {

~GADShape

point: GeographicalCoordinates

innerRadius: InnerRadius

uncertaintyRadius: Uncertainty

offsetAngle: Angle

includedAngle: Angle

confidence: Confidence

}

;;; GeographicalCoordinates

;;+ Geographical coordinates.

GeographicalCoordinates = {

lon: -180.0..180.0

lat: -90.0..90.0

}

;;; UncertaintyEllipse

;;+ Ellipse with uncertainty.

UncertaintyEllipse = {

semiMajor: Uncertainty

semiMinor: Uncertainty

orientationMajor: Orientation

}

;;; PointList

;;+ List of points.

PointList = [3*15 GeographicalCoordinates]

;;; Altitude

;;+ Indicates value of altitude.

Altitude = -32767.0..32767.0

;;; Angle

;;+ Indicates value of angle.

Angle = 0..360

;;; Uncertainty

;;+ Indicates value of uncertainty.

Uncertainty = float32 .ge 0

;;; Orientation

;;+ Indicates value of orientation angle.

Orientation = 0..180

;;; Confidence

;;+ Indicates value of confidence.

Confidence = 0..100

;;; InnerRadius

;;+ Indicates value of the inner radius.

InnerRadius = (0..327675) .and int32

;;; SupportedGADShapes

;;+ Indicates supported GAD shapes.

SupportedGADShapes = "POINT" / "POINT_UNCERTAINTY_CIRCLE" / "POINT_UNCERTAINTY_ELLIPSE" / "POLYGON" / "POINT_ALTITUDE" / "POINT_ALTITUDE_UNCERTAINTY" / "ELLIPSOID_ARC" / "LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE" / "LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID" / text

;;; Non3gppAccessPolicy

;;+ Represents the non-3GPP access measurement policy.

Non3gppAccessPolicy = "WLAN SSID" / "WLAN BSSID" / "LOCATION_BASED"

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

### A.3.2.6	Media Types

See clause A.5.

### A.3.2.7	Void

### A.3.2.8	Void

### A.3.2.9	Void

## A.3.3	Sdd_TransmissionQualityManagement API

### A.3.3.1	API URI

The CoAP URIs used in CoAP requests from SDDM-S towards the SDMM-C shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-s";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.3.3.2.

### A.3.3.2	Resources

#### A.3.3.2.1	Overview

Figure A.3.3.2.1.1: Resource URI structure of the Sdd_TransmissionQualityManagement API provided by SDDM-S

Table A.3.3.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.3.3.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD Transmission Quality Management | val-services/{valServiceId}/sdd-transmission-quality-management | POST | Establish an SDDM data transmission quality management. |
|  |  | DELETE | Releases an SDDM data transmission quality management. |

#### A.3.3.2.2	Resource: SDD Transmission Quality Management

##### A.3.3.2.2.1	Description

The SDD transmission quality management resource allows an SDDM-C to manage an SDDM data transmission quality management of an SDDM-S.

##### A.3.3.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-s/<apiVersion>/val-services/{valServiceId}/sdd--transmission-quality-management

This resource shall support the resource URI variables defined in the table A.3.3.2.2.2.1.

Table A.3.3.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.3.3.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.3.3.2.2.3	Resource Standard Methods

###### A.3.3.2.2.3.1	POST

This operation allows to establish an SDDM data transmission quality management.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.3.3.2.2.3.1.1 and A.3.3.2.2.3.1.2.

Table A.3.3.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TxQualityManagementRequest | M | 1 | The information of request of establishment of an SDDM data transmission quality management. |

Table A.3.3.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TxQualityManagementResponse | M | 1 | 2.01 Created | SDDM data transmission quality management created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

###### A.3.3.2.2.3.2	DELETE

This operation releases an SDDM data transmission quality management.

This method shall support the request data structures the data structure, request codes and response codes specified in table A.3.3.2.2.3.2.1 and A.3.3.2.2.3.2.2.

Table A.3.3.2.2.3.2.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The information of request of release of an SDDM data transmission quality management. |

Table A.3.3.2.2.3.2.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | SDDM data transmission quality management released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

### A.3.3.3	Data Model

#### A.3.3.3.1	General

Table A.3.3.3.1.1 specifies the data types defined specifically for the SDD_TransmissionQualityManagement API service provided by SDDM-S.

Table A.3.3.3.1.1: SDD_TransmissionQualityManagement API provided by SDDM-S specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| TxQualityManagementRequest | A.3.3.3.2.1 | Information identifying an SDD data transmission quality guarantee request. |  |
| TxQualityManagementResponse | A.3.3.3.2.2 | Information identifying an SDD data transmission quality guarantee response. |  |

Table A.3.3.3.1.2 specifies the simple data types defined specifically for the SDD_TransmissionQualityManagement API service provided by SDDM-S.

Table A.3.3.3.1.2: SDD_TransmissionQualityManagement API provided by SDDM-S specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |

Table A.3.3.3.1.3 specifies the enumerations defined specifically for the SDD_TransmissionQualityManagement API service provided by SDDM-S.

Table A.3.3.3.1.3: SDD_TransmissionQualityManagement API provided by SDDM-S specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |

#### A.3.3.3.2	Structured data types

##### A.3.3.3.2.1	Type: TxQualityManagementRequest

Table A.3.3.3.2.2.1: Definition of type TxQualityManagementRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |  |
| txQualityManagementAction | string | M | 1 | Identity of the data transmission quality guarantee action to be performed (set to "REDUNDANT TRANSMISSION PATH", "RE-ESTABLISH TRANSMISSION PATH" or "SWITCH TO BACKUP TRANSMISSION PATH") or optimization action (set to "BACK TO SINGLE TRANSMISSION PATH" or "TRANSMISSION PARAMETER ADJUSTMENT") that was triggered by an event (e.g. measurement threshold) |  |  |
| batOffsetUl | Uinteger | O | 0..1 | Indicates the BAT offset of the arrival time of the data burst in units of milliseconds for the uplink data. (NOTE 1) |  |  |
| periodicityUl | Uinteger | O | 0..1 | Indicates the adjusted periodicity of the data bursts in units of milliseconds for the uplink data. (NOTE 2) |  |  |
| NOTE 1: This attribute shall only be included if the txQualityManagementAction attribute is set to "TRANSMISSION PARAMETER ADJUSTMENT". NOTE 2: This attribute may only be included if the txQualityManagementAction attribute is set to "TRANSMISSION PARAMETER ADJUSTMENT" and "batOffsetUl" attribute is included. |  |  |  |  |  |  |

##### A.3.3.3.2.2	Type: TxQualityManagementResponse

Table A.3.3.3.2.2.1: Definition of type TxQualityManagementResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the establishment request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the establishment request (NOTE). |  |
| NOTE: This attribute shall be included if result is set to "FAILURE". |  |  |  |  |  |

#### A.3.3.3.3	Simple data types and enumerations

None.

### A.3.3.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.3.3.5	CDDL Specification

#### A.3.3.5.1	Introduction

The data model described in clause A.3.3.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.3.3.5.2 uses the concise data definition language described in IETF RFC 8610 [18] and provides corresponding representation of the SDD_TransmissionQualityManagement API provided by the SDDM-S data model.

#### A.3.3.5.2	CDDL document

;;; TxQualityManagementRequest

;;+ Represents a request for performing SDDM data transmission quality management.

TxQualityManagementRequest = {

sealddFlowId: Uinteger

txQualityManagementAction: string

? batOffsetUl: Uinteger

? periodicityUl: Uinteger

}

;;; TxQualityManagementResponse

;;+ Represents the response of a request for performing SDDM data transmission quality management.

TxQualityManagementResponse = {

result: ResultOp

? cause: Cause

}

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

### A.3.3.6	Media Types

See clause A.5.

### A.3.3.7	Void

### A.3.3.8	Void

## A.3.4	Sdd_ConnectionStatusEvent API

### A.3.4.1	API URI

The CoAP URIs used in CoAP requests from SDDM-S towards the SDMM-C shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-s";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.3.4.2.

### A.3.4.2	Resources

#### A.3.4.2.1	Overview

Figure A.3.4.2.1.1: Resource URI structure of the Sdd_ConnectionStatusEvent API provided by SDDM-S

Table A.3.4.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.3.4.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD Connection Status Event | val-services/{valServiceId}/sdd-connection-status-event | POST | Establish an SDDM connection status event. |
|  |  | FETCH | Observe SDDM connection status reporting configuration of the SDDM-C. |
|  |  | DELETE | Release an SDDM connection status event. |

#### A.3.4.2.2	Resource: SDD Connection Status Event

##### A.3.4.2.2.1	Description

The SDD connection status event resource represents an SDD connection status event to be created at a given SDDM-C and SDDM-S.

##### A.3.4.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-s/<apiVersion>/val-services/{valServiceId}/sdd-connection-status-event

This resource shall support the resource URI variables defined in the table A.3.4.2.2.2.1.

Table A.3.4.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.3.4.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.3.4.2.2.3	Resource Standard Methods

###### A.3.4.2.2.3.1	POST

This operation allows to establish an SDDM connection status event.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.3.4.2.2.3.1.1 and A.3.4.2.2.3.1.2.

Table A.3.4.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConnectionStatusConfigurationRequest | M | 1 | The information of request of establishment of an SDDM connection status event. |

Table A.3.4.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ConnectionStatusConfigurationResponse | M | 1 | 2.01 Created | SDDM connection status event created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

###### A.3.4.2.2.3.2	DELETE

This operation releases an SDDM connection status event.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.3.4.2.2.3.2.1 and A.3.4.2.2.3.2.2.

Table A.3.4.2.2.3.2.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The information of request of release of an SDDM connection status event. |

Table A.3.4.2.2.3.2.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | SDDM connection event released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

###### A.3.4.2.2.3.3	FETCH

This operation provides an SDD connection status reporting notification.

This method shall support the data structures, request codes and response codes specified in table A.3.4.2.2.3.3.1, A.3.4.2.2.3.3.2 and A.3.4.2.2.3.3.3.

Table A.3.4.2.2.3.3.1: Options supported by the FETCH Request on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| observe | Uinteger | O | 0..1 | When set to 0 (register) it extends the FETCH request to subscribe to the changes of this resource. When set to 1 (deregister) it cancels the subscription. |
| NOTE: Other request options also apply in accordance with normal CoAP procedures. |  |  |  |  |

Table A.3.4.2.2.3.3.2: Data structures supported by the FETCH Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConnectionStatusConfigurationSubscription | M | 1 | The identifier of SDDM connection status reporting configuration to which connection status reporting notification are going to be performed. |

Table A.3.4.2.2.3.3.3: Data structures supported by the FETCH Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ConnectionStatusNotification | M | 1 | 2.05 Content | The SDDM-C connection status reporting information. |
| NOTE: The mandatory CoAP error status codes for the FETCH method listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

### A.3.4.3	Data Model

#### A.3.4.3.1	General

Table A.3.4.3.1.1 specifies the data types defined specifically for the SDD_ConnectionStatusEvent API service provided by SDDM-S.

Table A.3.4.3.1.1: SDD_ConnectionStatusEvent API provided by SDDM-S specific data types

| Data type | Section defined | Description | Applicability |  |
| --- | --- | --- | --- | --- |
| ConnectionStatus | A.3.4.3.3.2 | Information identifying a connection status of VAL client. |  |  |
| ConnectionStatusConfigurationRequest | A.3.4.3.2.1 | Information identifying an SDD connection status event request. |  |  |
| ConnectionStatusConfigurationResponse | A.3.4.3.2.2 | Information identifying an SDD connection status event response. |  |  |
| ConnectionStatusConfigurationSubscription | A.3.4.3.2.3 | Information identifying an SDDM connection status reporting configuration. |  |  |
| ConnectionStatusNotification | A.3.4.3.2.4 | The SDDM-C connection status reporting information. |  |  |
| MeasuredNon3gppAccess | A.2.4.11 | Indicates identity of the measured non-3GPP access. |  |
| Non3gppAccessMeasurement | A.2.4.10 | Indicates the non-3GPP access measurement information report. |  |

Table A.3.4.3.1.2 specifies the simple data types defined specifically for the SDD_ConnectionStatusEvent API service provided by SDDM-S.

Table A.3.4.3.1.2: SDD_ConnectionStatusEvent API provided by SDDM-S specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |

Table A.3.4.3.1.3 specifies the enumerations defined specifically for the SDD_ConnectionStatusEvent API service provided by SDDM-S.

Table A.3.4.3.1.3: SDD_ConnectionStatusEvent API provided by SDDM-S specific enumeration

| Data type | Section defined | Description |  |
| --- | --- | --- | --- |
| AccessUsage | A.3.4.3.3.4 | Information identifying which access is used for the SEALDD-UU data transmission. |
| ReportingPriority | A.3.4.3.3.5 | Indicates the priority of the reporting. |
| ReportingMode | A.3.4.3.3.3 | Identifies the mode of the reporting. |  |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |  |

#### A.3.4.3.2	Structured data types

##### A.3.4.3.2.1	Type: ConnectionStatusConfigurationRequest

Table A.3.4.3.2.1.1: Definition of type ConnectionStatusConfigurationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |
| reportingMode | ReportingMode | O | 0..1 | Indicates the mode of the reporting. i.e. "PERIODIC" or "EVENT_TRIGGERED". |  |
| reportingInterval | Uinteger | O | 0..1 | Indicates the reporting interval, in seconds, to report the notification (NOTE). |  |
| reportingPriority | ReportingPriority | O | 0..1 | Indicates the priority of SEALDD client connection status for the requested SEALDD flow ID. |  |
| non3gppAccessPolicy | Non3gppAccessPolicy | O | 0..1 | Indicates the non-3GPP access measurement policy, i.e., "WLAN SSID", "WLAN BSSID" or "LOCATION_BASED" measurement. |  |
| NOTE: This attribute shall be included if the "reportingMode" attribute is set to "PERIODIC". |  |  |  |  |  |

##### A.3.4.3.2.2	Type: ConnectionStatusConfigurationResponse

Table A.3.4.3.2.2.1: Definition of type ConnectionStatusConfigurationResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the connection status request. |  |

##### A.3.4.3.2.3	Type: ConnectionStatusConfigurationSubscription

Table A.3.4.3.2.3.1: Definition of type ConnectionStatusConfigurationSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |

##### A.3.4.3.2.4	Type: ConnectionStatusNotification

Table A.3.4.3.2.4.1: Definition of type ConnectionStatusNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientConnectionStatus | ConnectionStatus | M | 1 | Indicates the connection status of the VAL client, i.e. reachable, unreachable, or sleeping. |  |
| accessUsage | AccessUsage | M | 1 | Indicates which access is used for the SEALDD-UU data transmission. |  |
| non3gppAccessMeasurements | array(Non3gppAccessMeasurement) | O | 1..N | Information of the non-3GPP access measurement information report. |  |

#### A.3.4.3.3	Simple data types and enumerations

##### A.3.4.3.3.1	Simple data types

Table A.3.4.3.3.1.1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |

##### A.3.4.3.3.2	Enumeration: ConnectionStatus

Table A.3.4.3.3.2.1: ConnectionStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REACHABLE | Indicates that a VAL client is reachable. |  |
| UNREACHABLE | Indicates that a VAL client is unreachable. |  |
| SLEEPING | Indicates that a VAL client is sleeping. |  |

##### A.3.4.3.3.3	Enumeration: ReportingMode

The enumeration ReportingMode represents the type of the report mode. It shall comply with the provisions defined in table A.3.4.3.3.3.1.

Table A.3.4.3.3.3.1: Enumeration ReportingMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PERIODIC | Indicates the reporting mode of type periodic. |  |
| EVENT_TRIGGERED | Indicates the reporting mode of type event triggered. |  |

##### A.3.4.3.3.4	Enumeration: AccessUsage

The enumeration AccessUsage represents type of used access. It shall comply with the provisions defined in table A.3.4.3.3.4.1.

Table A.3.4.3.3.4.1: Enumeration AccessUsage

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| 3GPP_ACCESS | Indicates the 3GPP access. |  |
| NON_3GPP_ACCESS | Indicates the non-3GPP access. |  |

##### A.3.4.3.3.5	Enumeration: ReportingPriority

The enumeration ReportingPriority represents the priority of the SEALDD client connection status for the requested SEALDD flow ID. It shall comply with the provisions defined in table A.3.4.3.3.5.1.

Table A.3.4.3.3.5.1: Enumeration ReportingPriority

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOW | Indicates the reporting priority is set to low. |  |
| HIGH | Indicates the reporting priority is set to high. |  |

### A.3.4.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.3.4.5	CDDL Specification

#### A.3.4.5.1	Introduction

The data model described in clause A.3.4.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.3.4.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the SDD_ConnectionStatusEvent API provided by the SDDM-S data model.

#### A.3.4.5.2	CDDL document

;;; ConnectionStatusConfigurationRequest

;;+ Represents a request for performing SDDM connection status reporting.

ConnectionStatusConfigurationRequest = {

sealddFlowId: Uinteger

? reportingMode: ReportingMode

? reportingInterval: Uinteger

? reportingPriority: ReportingPriority

? non3gppAccessPolicy: Non3gppAccessPolicy

}

;;; ConnectionStatusConfigurationResponse

;;+ Represents the response of a request for performing SDDM connection status reporting.

ConnectionStatusConfigurationResponse = {

result: ResultOp

}

;;; ConnectionStatusConfigurationSubscription

;;+ Represents a request for performing SDDM connection status reporting notification.

ConnectionStatusConfigurationSubscription = {

sealddFlowId: Uinteger

}

;;; ConnectionStatusNotification

;;+ Represents an SDDM connection status reporting notification.

ConnectionStatusNotification = {

clientConnectionStatus: ConnectionStatus

accessUsage: AccessUsage

? non3gppAccessMeasurements: [* Non3gppAccessMeasurement]

}

;;; Non3gppAccessMeasurement

;;+ Information of the non-3GPP access measurement information report.

? measuredAccess: MeasuredNon3gppAccess

signalStrengthValues: [* Uinteger]

;;; MeasuredNon3gppAccess

;;+ Identity of the measured non-3GPP access i.e. WLAN SSID or WLAN BSSID.

? ssId: string

? bssId: string

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; ConnectionStatus

;;+ Represents the VAL client connection status.

ConnectionStatus = "REACHABLE" / "UNREACHABLE" / "SLEEPING"

;;; ReportingPriority

;;+ Indicates the priority of SEALDD client connection status for the requested SEALDD flow ID.

ReportingPriority = "LOW" / "HIGH"

;;; ReportingMode

;;+ Represents the mode of the reporting.

ReportMode = "PERIODIC" / "EVENT_TRIGGERED"

;;; Non3gppAccessPolicy

;;+ Represents the non-3GPP access measurement policy.

Non3gppAccessPolicy = "WLAN SSID" / "WLAN BSSID" / "LOCATION_BASED"

;;; AccessUsage

;;+ Represents the type of the used access.

AccessUsage = "3GPP_ACCESS" / "NON_3GPP_ACCESS"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

### A.3.4.6	Media Types

See clause A.5.

### A.3.4.7	Void

### A.3.4.8	Void

## A.3.5	Sdd_URLLCTransmissionConnection API

### A.3.5.1	API URI

The CoAP URIs used in CoAP requests from SDDM-S towards the SDMM-C shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-s";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.3.5.2.

### A.3.5.2	Resources

#### A.3.5.2.1	Overview

Figure A.3.5.2.1.1: Resource URI structure of the Sdd_URLLCTransmissionConnection API provided by SDDM-S

Table A.3.5.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.3.5.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| URLLC Transmission Connection | val-services/{valServiceId}/urllc-transmission-connection | POST | Establish a URLLC transmission connection. |
|  |  | PUT | Update a URLLC transmission connection. |
|  |  | DELETE | Releases a URLLC transmission connection. |

#### A.3.5.2.2	Resource: URLLC Transmission Connection

##### A.3.5.2.2.1	Description

The URLLC transmission connection resource allows an SDDM-C to manage a URLCC transmission connection of an SDDM-S.

##### A.3.5.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-s<apiVersion>/val-services/{valServiceId}/urllc-transmission-connection

This resource shall support the resource URI variables defined in the table A.3.5.2.2.2.1.

Table A.3.5.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.3.5.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.3.5.2.2.3	Resource Standard Methods

A.3.5.2.2.3.1	POST

This operation retrieves the allowed registration.

This method shall support the request data structures, request codes and response codes specified in table A.3.5.2.2.3.1.1 and A.3.5.2.2.3.1.2.

Table A.3.5.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| URLLCEstablishmentRequest | M | 1 | The information of request of establishment of an SDDM URLLC transmission connection. |

Table A.3.5.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| URLLCEstablishmentResponse | M | 1 | 2.01 Created | URLLC transmission connection created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

A.3.5.2.2.3.2	DELETE

This operation releases a URLLC transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.3.5.2.2.3.2.1 and A.3.5.2.2.3.2.2.

Table A.3.5.2.2.3.2.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| URLLCReleaseRequest | M | 1 | The information of request of release of a URLCC transmission connection. |

Table A.3.5.2.2.3.2.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | URLLC transmission connection released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

### A.3.5.3	Data Model

#### A.3.5.3.1	General

Table A.3.5.3.1.1 specifies the data types defined specifically for the SDD_URLLCTransmissionConnection API service provided by SDDM-S.

Table A.3.5.3.1.1: SDD_URLLCTransmissionConnection API provided by SDDM-S specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| PeriodicityRange | A.2.4.7 | Contains the acceptable periodicity range or periodicity value(s). |  |
| TimeWindow | A.2.4.8 | Contains start time and stop time. |  |
| TransmissionAssistInfo | A.2.4.6 | Contains transmission assistance information for uplink SEALDD traffic. |  |
| URLLCEstablishmentRequest | A.2.4.3 | Information identifying a URLLC transmission connection establishment request. |  |
| URLLCEstablishmentResponse | A.2.4.4 | Information identifying a URLLC transmission connection establishment response. |  |
| URLLCReleaseRequest | A.2.4.5 | Information identifying a URLLC transmission connection release request. |  |

Table A.3.5.3.1.2 specifies the simple data types defined specifically for the SDD_URLLCTransmissionConnection API service provided by SDDM-S.

Table A.3.5.3.1.2: SDD_URLLCTransmissionConnection API provided by SDDM-S specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |
| ServerId | A.2.5 | String representing a unique identifier of a VAL server. |

Table A.3.5.3.1.3 specifies the enumerations defined specifically for the SDD_URLLCTransmissionConnection API service provided by SDDM-S.

Table A.3.5.3.1.3: SDD_URLLCTransmissionConnection API provided by SDDM-C specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |

#### A.3.5.3.2	Structured data types

None.

#### A.3.5.3.3	Simple data types and enumerations

None.

### A.3.5.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.3.5.5	CDDL Specification

#### A.3.5.5.1	Introduction

The data model described in clause A.3.3.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.3.3.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the Sdd_URLLCTransmissionConnection API provided by the SDDM-S data model.

#### A.3.5.5.2	CDDL document

;;; URLLCEstablishmentRequest

;;+ Represents a request for establishing a URLLC transmission connection.

URLLCEstablishmentRequest = {

sealClientId: string

sealFlowId: Uinteger

valTgtUe: ValTargetUe

serverId: ServerId

valServiceId: string

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

}

;;; URLLCEstablishmentResponse

;;+ Represents a response of establishing a URLLC transmission connection.

URLLCEstablishmentResponse = {

result: ResultOp

? cause: Cause

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

.?.batAndPeriodicityCapability: boolean

.?.transmisAssistInfo: TransmissionAssistInfo

}

;;; URLLCReleaseRequest

;;+ Represents a request for releasing a URLLC transmission connection.

ReleaseRequest = {

sealClientId: string

sealFlowId: Uinteger

}

;;; TransmissionAssistInfo

;;+ Indicates a transmission assistance information for uplink SEALDD traffic.

TransmissionAssistInfo = {

? bat: DateTime

? periodicity: Uinteger

? batWindow: TimeWindow

? periodRange: PeriodicityRange

}

;;; TimeWindow

;;+ Indicates the acceptable earliest and latest arrival time of the first packet of the data burst. The start time contains the earliest acceptable arrival time, and the stop time contains the latest acceptable arrival time.

TransmissionAssistInfo = {

? startTime: DateTime

? stopTime: DateTime

}

;;; PeriodicityRange

;;+ Indicates the acceptable time period range between the start of two bursts or the acceptable periodicity value(s).

PeriodicityRange = {

? lowerBound: Uinteger

? upperBound: Uinteger

? periodicityValues: [* Uinteger]

}

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; ServerId

;;+ Represents information identifying a unique server.

serverId = text

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

### A.3.5.6	Media Types

See clause A.5.

## A.3.6	Sdd_XRTransmissionConnnection API

### A.3.6.1	API URI

The CoAP URIs used in CoAP requests from SDDM-S towards the SDMM-C shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-s";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.3.6.2.

### A.3.6.2	Resources

#### A.3.6.2.1	Overview

Figure A.3.6.2.1.1: Resource URI structure of the Sdd_XRTransmissionConnnection API provided by SDDM-S

Table A.3.6.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.3.6.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD XR transmission connection | val-services/{valServiceId}/sdd-xr-transmission-connection | POST | Trigger an SDDM XR transmission connection operation. |

#### A.3.6.2.2	Resource: SDD XR transmission connection

##### A.3.6.2.2.1	Description

The SDD XR transmission connection resource represents an SDD XR transmission connection to be created at a given SDDM-C and SDDM-S.

##### A.3.6.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-s/<apiVersion>/val-services/{valServiceId}/sdd-xr-transmission-connection

This resource shall support the resource URI variables defined in the table A.3.6.2.2.2.1.

Table A.3.6.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.3.6.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.3.6.2.2.3	Resource Standard Methods

###### A.3.6.2.2.3.1	POST

This operation request triggering of an SDDM XR transmission connection operation.

This method shall support the data structures, request codes and response codes specified in table A.3.6.2.2.3.1.1 and A.3.6.2.2.3.1.2.

Table A.3.6.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| XRTriggerRequest | 1 | 1 | The information of request SDDM XR transmission connection trigger. |

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| XRTriggerResponse | 1 | 1 | 2.01 Created | SDDM XR transmission connection trigger created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

### A.3.6.3	Data Model

#### A.3.6.3.1	General

Table A.3.6.3.1.1 specifies the data types defined specifically for the Sdd_XRTransmissionConnnection API service provided by SDDM-S.

Table A.3.6.3.1.1: Sdd_XRTransmissionConnnection API provided by SDDM-S specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| XRTriggerRequest | A.3.6.3.2.1 | Information identifying an SDDM XR transmission connection trigger request. |  |
| XRTriggerResponse | A.3.6.3.2.2 | Information identifying an SDDM XR transmission connection trigger response. |  |

Table A.3.6.3.1.2 specifies the simple data types defined specifically for the Sdd_XRTransmissionConnnection API service provided by SDDM-S.

Table A.3.6.3.1.2: Sdd_XRTransmissionConnnection API provided by SDDM-S specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |
| ServerId | A.2.5 | String representing a unique identifier of a VAL server. |

Table A.3.6.3.1.3 specifies the enumerations defined specifically for the Sdd_XRTransmissionConnnection API service provided by SDDM-S.

Table A.3.6.3.1.3: Sdd_XRTransmissionConnnection API provided by SDDM-S specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |
| Operation | A.3.6.3.3.2 | Information identifying the action for a UE-to-UE direct communication. |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |

#### A.3.6.3.2	Structured data types

##### A.3.6.3.2.1	Type: XRTriggerRequest

Table A.3.6.3.2.1.1: Definition of type XRTriggerRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | RequestorId | M | 1 | Identity of the requestor of the XR transmission connection trigger. (NOTE) |  |
| valUeIds | array(ValTargetUe) | M | 1..N | Information of the identities of the VAL UEs or VAL users involved in the UE-to-UE direct communication. |  |
| operation | Operation | M | 1 | Identifies the UE-to-UE direct communication operation. |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service enabled by the XR transmission connection. |  |
| NOTE: This attribute shall be set to "SEALDDSERVER". |  |  |  |  |  |

##### A.3.6.3.2.2	Type: XRTriggerResponse

Table A.3.6.3.2.2.1: Definition of type XRTriggerResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the XR transmission connection trigger request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the XR transmission trigger request. (NOTE) |  |
| NOTE: This attribute shall be included if result is set to "FAILURE". |  |  |  |  |  |

#### A.3.6.3.3	Simple data types and enumerations

##### A.3.6.3.3.1	Simple data types

Table A.3.6.3.3.1.1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |

##### A.3.6.3.3.2	Enumeration: Operation

Table A.3.6.3.3.2.1: Operation

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ESTABLISH | Indicates the establish action for the UE-to-UE direct communication. |  |
| RELEASE | Indicates the establish action for the UE-to-UE direct communication. |  |

### A.3.6.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.3.6.5	CDDL Specification

#### A.3.6.5.1	Introduction

The data model described in clause A.3.6.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.3.6.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the Sdd_XRTransmissionConnnection API provided by the SDDM-S data model.

#### A.3.6.5.2	CDDL document

;;; XRTriggerRequest

;;+ Represents a request for performing SDDM XR transmission connection trigger.

XRTriggerRequest = {

requestorId: RequestorId

valUeIds: [* ValTargetUe]

operation: Operation

? valServiceId: string

}

;;; XRTriggerResponse

;;+ Represents the response of a request for performing SDDM XR transmission connection trigger.

XRTriggerResponse = {

result: ResultOp

? cause: Cause

}

;;; RequestorId

;;+ Indicates requestor of an operation.

RequestorId = "SEALDDCLIENT" / "SEALDDSERVER"

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Operation

;;+ Identifies the UE-to-UE direct communication operation.

Operation = "ESTABLISH" / "RELEASE"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

### A.3.6.6	Media Types

See clause A.5.

# A.4	Resource representation and APIs provided by SDDM-C

## A.4.1	Sdd_RegularTransmissionConnection API

### A.4.1.1	API URI

The CoAP URIs used in CoAP requests from SDDM-C towards the SDMM-S shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-c";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.4.1.2.

### A.4.1.2	Resources

#### A.4.1.2.1	Overview

Figure A.4.1.2.1.1: Resource URI structure of the Sdd_RegularTransmissionConnection API provided by SDDM-C

Table A.4.1.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.4.1.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD Regular Transmission Connection | val-services/{valServiceId}/sdd-regular-transmission-connection | POST | Establish an SDDM regular transmission connection. |
|  |  | DELETE | Release an SDDM regular transmission connection |

#### A.4.1.2.2	Resource: SDD Regular Transmission Connection

##### A.4.1.2.2.1	Description

The SDD regular transmission connection resource represents an SDD regular transmission connection to be created at a given SDDM-C and SDDM-S.

The establishment request resource allows an SDDM-C to request the SDDM-S to establish an SDDM regular transmission.

##### A.4.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-c/<apiVersion>/val-services/{valServiceId}/sdd-regular-transmission-connection

This resource shall support the resource URI variables defined in the table A.4.1.2.2.2.1.

Table A.4.1.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.4.1.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.4.1.2.2.3	Resource Standard Methods

A.4.1.2.2.3.1	POST

This operation allows to establish an SDDM regular transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.4.1.2.2.3.1.1 and A.4.1.2.2.3.1.2.

Table A.4.1.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EstablishmentRequest | M | 1 | The information of request of establishment of an SDDM regular transmission connection. |

Table A.4.1.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EstablishmentResponse | M | 1 | 2.01 Created | SDDM regular transmission connection created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

A.4.1.2.2.3.2	DELETE

This operation releases an SDDM regular transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.4.1.2.2.3.2.1 and A.4.1.2.2.3.2.2.

Table A.4.1.2.2.3.2.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ReleaseRequest | M | 1 | The information of request of release of an SDDM regular transmission connection. |

Table A.4.1.2.2.3.2.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | SDDM regular transmission connection released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

### A.4.1.3	Data Model

#### A.4.1.3.1	General

Table A.4.1.3.1.1 specifies the data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-C.

Table A.4.1.3.1.1: SDD_RegularTransmissionConnection API provided by SDDM-C specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| EstablishmentResponse | A.2.4.1 | Information identifying an SDD regular transmission connection establishment response. |  |
| EstablishmentRequest | A.2.4.2 | Information identifying an SDD regular transmission connection establishment request. |  |
| PeriodicityRange | A.2.4.6 | Contains the acceptable periodicity range or periodicity value(s). |  |
| ReleaseRequest | A.3.1.3.2.3 | Information identifying an SDD regular transmission connection release request. |  |
| TimeWindow | A.2.4.7 | Contains start time and stop time. |  |
| TransmissionAssistInfo | A.2.4.8 | Contains transmission assistance information for uplink SEALDD traffic. |  |

Table A.4.1.3.1.2 specifies the simple data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-C.

Table A.4.1.3.1.2: SDD_RegularTransmissionConnection API provided by SDDM-C specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |
| ServerId | A.2.5 | String representing a unique identifier of a VAL server. |

Table A.4.1.3.1.3 specifies the enumerations defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-C.

Table A.4.1.3.1.3: SDD_RegularTransmissionConnection API provided by SDDM-C specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| RequestorId | A.2.6.1 | Information identifying a VAL user ID or VAL UE ID. |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |

#### A.4.1.3.2	Structured data types

##### A.4.1.3.2.1	Void

##### A.4.1.3.2.2	Type: ReleaseRequest

Table A.4.1.3.2.2.1: Definition of type ReleaseRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealClientId | string | M | 1 | Identity of the SDDM-C. |  |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |

#### A.4.1.3.3	Simple data types and enumerations

None.

### A.4.1.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.4.1.5	CDDL Specification

#### A.4.1.5.1	Introduction

The data model described in clause A.4.1.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.4.1.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the SDD_RegularTransmissionConnection API provided by the SDDM-C data model.

#### A.4.1.5.2	CDDL document

;;; EstablishmentRequest

;;+ Represents a request for establishing an SDDM regular transmission connection.

EstablishmentRequest = {

requestorId: RequestorId

sealddFlowId: Uinteger

serverId: ServerId

endpointId: string

? valServiceId: string

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

? valTgtUe: ValTargetUe

.?.batAndPeriodicityCapability: boolean

.?.transmisAssistInfo: TransmissionAssistInfo

}

;;; EstablishmentResponse

;;+ Represents a response of establishing an SDDM regular transmission connection.

EstablishmentResponse = {

result: ResultOp

? cause: Cause

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

}

;;; ReleaseRequest

;;+ Represents a request for releasing an SDDM regular transmission connection.

ReleaseRequest = {

sealClientId: string

sealddFlowId: Uinteger

}

;;; TransmissionAssistInfo

;;+ Indicates a transmission assistance information for uplink SEALDD traffic.

TransmissionAssistInfo = {

? bat: DateTime

? periodicity: Uinteger

? batWindow: TimeWindow

? periodRange: PeriodicityRange

}

;;; TimeWindow

;;+ Indicates the acceptable earliest and latest arrival time of the first packet of the data burst. The start time contains the earliest acceptable arrival time, and the stop time contains the latest acceptable arrival time.

TransmissionAssistInfo = {

? startTime: DateTime

? stopTime: DateTime

}

;;; PeriodicityRange

;;+ Indicates the acceptable time period range between the start of two bursts or the acceptable periodicity value(s).

PeriodicityRange = {

? lowerBound: Uinteger

? upperBound: Uinteger

? periodicityValues: [* Uinteger]

}

;;; RequestorId

;;+ Indicates requestor of an operation.

RequestorId = "SEALDDCLIENT" / "SEALDDSERVER"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; ServerId

;;+ Represents information identifying a unique server.

serverId = text

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

### A.4.1.6	Media Types

See clause A.3.1.6.

## A.4.2	Sdd_URLLCTransmissionConnection API

### A.4.2.1	API URI

The CoAP URIs used in CoAP requests from SDDM-C towards the SDMM-S shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-c";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.4.2.2.

### A.4.2.2	Resources

#### A.4.2.2.1	Overview

Figure A.4.2.2.1.1: Resource URI structure of the Sdd_URLLCTransmissionConnection API provided by SDDM-C

Table A.4.2.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.4.2.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| URLLC Transmission Connection | val-services/{valServiceId}/urllc-transmission-connection | POST | Establish a URLLC transmission connection. |
|  |  | PUT | Update a URLLC transmission connection. |
|  |  | DELETE | Releases a URLLC transmission connection. |

#### A.4.2.2.2	Resource: URLLC Transmission Connection

##### A.4.2.2.2.1	Description

The URLLC transmission connection resource allows an SDDM-S to manage an URLLC transmission connection of an SDDM-C.

##### A.4.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-c/<apiVersion>/val-services/{valServiceId}/urllc-transmission-connection

This resource shall support the resource URI variables defined in the table A.4.2.2.2.2.1.

Table A.4.1.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.4.2.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.4.2.2.2.3	Resource Standard Methods

A.4.2.2.2.3.1	POST

This operation retrieves the allowed registration.

This method shall support the request data structures, request codes and response codes specified in table A.4.2.2.2.3.1.1 and A.4.2.2.2.3.1.2.

Table A.4.2.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| URLLCEstablishmentRequest | M | 1 | The information of request of establishment of an SDDM URLLC transmission connection. |

Table A.4.2.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| URLLCEstablishmentResponse | M | 1 | 2.01 Created | URLLC transmission connection created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

A.4.2.2.2.3.2	PUT

This operation updates a URLLC transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.4.2.2.2.3.2.1 and A.4.2.2.2.3.2.2.

Table A.4.2.2.2.3.2.1: Data structures supported by the PUT Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| URLLCUpdateRequest | M | 1 | The information of request of update a URLLC transmission connection. |

Table A.4.2.2.2.3.2.1: Data structures supported by the PUT Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| URLLCUpdateResponse | M | 1 | 2.04 Changed | URLLC transmission connection updated successfully. |
| NOTE: The mandatory CoAP error status codes for the PUT method listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

A.4.2.2.2.3.3	DELETE

This operation releases a URLLC transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.4.2.2.2.3.3.1 and A.4.2.2.2.3.3.2.

Table A.4.2.2.2.3.3.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| URLLCReleaseRequest | M | 1 | The information of request of release of an URLLC transmission connection. |

Table A.4.2.2.2.3.3.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | URLLC transmission connection released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

### A.4.2.3	Data Model

#### A.4.2.3.1	General

Table A.4.2.3.1.1 specifies the data types defined specifically for the SDD_URLLCTransmissionConnection API service provided by SDDM-C.

Table A.4.2.3.1.1: SDD_RegularTransmissionConnection API provided by SDDM-C specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| PeriodicityRange | A.2.4.7 | Contains the acceptable periodicity range or periodicity value(s). |  |
| TimeWindow | A.2.4.8 | Contains start time and stop time. |  |
| TransmissionAssistInfo | A.2.4.6 | Contains transmission assistance information for uplink SEALDD traffic. |  |
| URLLCEstablishmentRequest | A.2.4.3 | Information identifying an SDD URLLC transmission connection establishment request. |  |
| URLLCEstablishmentResponse | A.2.4.4 | Information identifying an SDD URLLC transmission connection establishment response. |  |
| URLLCUpdateRequest | A.4.2.3.2.3 | Information identifying an SDD URLLC transmission connection update request. |  |
| URLLCReleaseRequest | A.2.4.5 | Information identifying an SDD URLLC transmission connection release request. |  |

Table A.4.2.3.1.2 specifies the simple data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-C.

Table A.4.2.3.1.2: SDD_RegularTransmissionConnection API provided by SDDM-C specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |
| ServerId | A.2.5 | String representing a unique identifier of a VAL server. |

Table A.4.2.3.1.3 specifies the enumerations defined specifically for the SDD_URLLCTransmissionConnection API service provided by SDDM-C.

Table A.4.2.3.1.3: SDD_RegularTransmissionConnection API provided by SDDM-C specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |

#### A.4.2.3.2	Structured data types

##### A.4.2.3.2.1	Void

##### A.4.2.3.2.2	Void

##### A.4.2.3.2.3	Type: URLLCUpdateRequest

Table A.4.2.3.2.1.3: Definition of type URLLCUpdateRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealClientId | string | M | 1 | Identity of the requestor of the URLLC establishment request. |  |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |
| valServiceId | string | O | 1..N | Identity of the VAL services enabled by the SDD URLLC transmission connection. |  |
| userPlaneAddress | string | O | 0..1 | Identity of the IP address of the traffic. |  |
| portNumrber | Uinteger | O | 0..1 | Identity of the port number of the traffic. |  |
| url | string | O | 0..1 | Identity of the address of a given unique resource on the Web for the traffic. |  |
| transportLayer | string | O | 0..1 | Identity of the transport layer protocol for the traffic. |  |

##### A.4.2.3.2.4	Void

##### A.4.2.3.2.5	Type: URLLCUpdateResponse

Table A.4.2.3.2.5.1: Definition of type URLLCUpdateResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the establishment request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the establishment request (NOTE). |  |
| NOTE: This attribute shall be included if result is set to "failure". |  |  |  |  |  |

#### A.4.2.3.3	Simple data types and enumerations

None.

### A.4.2.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.4.2.5	CDDL Specification

#### A.4.2.5.1	Introduction

The data model described in clause A.4.2.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.4.2.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the SDD_URLLCTransmissionConnection API provided by the SDDM-C data model.

#### A.4.2.5.2	CDDL document

;;; URLLCEstablishmentRequest

;;+ Represents a request for establishing a URLLC transmission connection.

URLLCEstablishmentRequest = {

sealClientId: string

sealddFlowId: Uinteger

valTgtUe: ValTargetUe

serverId: ServerId

valServiceId: string

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

.?.batAndPeriodicityCapability: boolean

.?.transmisAssistInfo: TransmissionAssistInfo

}

;;; URLLCEstablishmentResponse

;;+ Represents a response of establishing a URLLC transmission connection.

URLLCEstablishmentResponse = {

result: ResultOp

? cause: Cause

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

}

;;; URLLCUpdateRequest

;;+ Represents a request for updating a URLLC transmission connection.

URLLCEstablishmentRequest = {

sealClientId: string

sealddFlowId: Uinteger

? valServiceId: string

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

}

;;; URLLCReleaseRequest

;;+ Represents a request for releasing a URLLC transmission connection.

ReleaseRequest = {

sealClientId: string

sealddFlowId: Uinteger

}

;;; TransmissionAssistInfo

;;+ Indicates a transmission assistance information for uplink SEALDD traffic.

TransmissionAssistInfo = {

? bat: DateTime

? periodicity: Uinteger

? batWindow: TimeWindow

? periodRange: PeriodicityRange

}

;;; TimeWindow

;;+ Indicates the acceptable earliest and latest arrival time of the first packet of the data burst. The start time contains the earliest acceptable arrival time, and the stop time contains the latest acceptable arrival time.

TransmissionAssistInfo = {

? startTime: DateTime

? stopTime: DateTime

}

;;; PeriodicityRange

;;+ Indicates the acceptable time period range between the start of two bursts or the acceptable periodicity value(s).

PeriodicityRange = {

? lowerBound: Uinteger

? upperBound: Uinteger

? periodicityValues: [* Uinteger]

}

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; ServerId

;;+ Represents information identifying a unique server.

serverId = text

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

### A.4.2.6	Media Types

See clause A.5.

### A.4.2.7	Void

### A.4.2.8	Void

### A.4.2.9	Void

### A.4.2.10	Void

### A.4.2.11	Void

## A.4.3	Sdd_DataStorage API

### A.4.3.1	API URI

The CoAP URIs used in CoAP requests from SDDM-C towards the SDMM-S shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-c";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.4.3.2.

### A.4.3.2	Resources

#### A.4.3.2.1	Overview

Figure A.4.3.2.1.1: Resource URI structure of the Sdd_DataStorage API provided by SDDM-C

Table A.4.3.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.4.3.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD data storage | val-services/{valServiceId}/sdd-data-storage | POST | Establish an SDDM data storage or reservation of an SDDM data storage. |
|  |  | PUT | Update an SDDM data storage. |
|  |  | DELETE | Releases an SDDM data storage. |
|  |  | GET | Retrieve an SDDM data storage. |
|  |  | FETCH | Observe an SDDM data storage. |

#### A.4.3.2.2	Resource: SDD Data Storage

##### A.4.3.2.2.1	Description

The SDDM data storage resource allows an SDDM-S to manage an SDDM data storage of an SDDM-C.

##### A.4.3.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-c/<apiVersion>/val-services/{valServiceId}/sdd-data-storage

This resource shall support the resource URI variables defined in the table A.4.3.2.2.2.1.

Table A.4.3.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.4.3.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.4.3.2.2.3	Resource Standard Methods

###### A.4.3.2.2.3.1	POST

This operation request establishment or reservation of an SDDM data storage.

This method shall support the data structures, request codes and response codes specified in table A.4.3.2.2.3.1.1 and A.4.3.2.2.3.1.2.

Table A.4.3.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataStorageCreationRequest | O | 0..1 | The information of request of establishment of an SDDM data storage. |
| DataStorageReservationRequest | O | 0..1 | The information of request of reservation of an SDDM data storage. |

Table A.4.3.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataStorageCreationResponse | O | 0..1 | 2.01 Created | SDDM data storage created successfully. |
| DataStorageReservationResponse | O | 0..1 | 2.01 Created | SDDM data storage reserved successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

###### A.4.3.2.2.3.2	PUT

This operation updates an SDDM data storage.

This method shall support the data structures, request codes and response codes specified in table A.4.3.2.2.3.2.1 and A.4.3.2.2.3.2.2.

Table A.4.3.2.2.3.2.1: Data structures supported by the PUT Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataStorageMgtRequest | M | 1 | The information of request of update an SDDM data storage. |

Table A.4.3.2.2.3.2.1: Data structures supported by the PUT Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.04 Changed | SDDM data storage updated successfully. |
| NOTE: The mandatory CoAP error status codes for the PUT method listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

###### A.4.3.2.2.3.3	DELETE

This operation releases an SDD data storage.

This method shall support the data structures, request codes and response codes specified in table A.4.3.2.2.3.3.1 and A.4.3.2.2.3.3.2.

Table A.4.3.2.2.3.3.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataStorageMgtRequest | M | 1 | The information of request of release of an SDD data storage. |

Table A.4.3.2.2.3.3.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | SDDM data storage released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

###### A.4.3.2.2.3.4	GET

This operation retrieves an SDDM data storage information.

This method shall support the URI query parameters, the data structures and response codes specified in table A.4.3.2.2.3.4.1 and A.4.3.2.2.3.4.2.

Table A.4.3.2.2.3.4.1: URI query options supported by the GET Request on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| data-identifier | string | M | 1 | The data identifier of an SDDM data storage. |

Table A.4.3.2.2.3.4.2: Data structures supported by the GET Response payload on this resource

| Data type |  | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- | --- |
| DataStorageQueryResponse |  | M | 1 | 2.05 Content | The SDDM data storage information based on the request from the SDDM-C. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

###### A.4.3.2.2.3.5	FETCH

This operation provides an SDDM data storage.

This method shall support the request options, the data structures, request codes and response codes, and the response options specified in table A.4.3.2.2.3.5.1, A.4.3.2.2.3.5.2, A.4.3.2.2.3.5.3 and A.4.3.2.2.3.5.4.

Table A.4.3.2.2.3.5.1: Options supported by the FETCH Request on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| observe | Uinteger | O | 0..1 | When set to 0 (Register) it extends the FETCH request to subscribe to the changes of this resource. When set to 1 (Deregister) it cancels the subscription. |
| NOTE: Other request options also apply in accordance with normal CoAP procedures. |  |  |  |  |

Table A.4.2.2.3.5.2: Data structures supported by the FETCH Request on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataStorageMgtRequest | M | 1 | The identifier of an SDDM data storage. |

Table A.4.2.2.3.5.3: Data structures supported by the FETCH Response payload on this resource

| Data type |  | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- | --- |
| DataStorageStatusNotification |  | M | 1 | 2.05 Content | The information of an SDDM data storage based on the request from the SDDM-C. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [6] shall also apply. |  |  |  |  |

Table A.4.2.2.3.5.4: Options supported by the 2.05 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| observe | Uinteger | O | 0..1 | Sequence number of the SDDM data storage notification. |
| NOTE: Other response options also apply in accordance with normal CoAP procedures. |  |  |  |  |

### A.4.3.3	Data Model

#### A.4.3.3.1	General

Table A.4.3.3.1.1 specifies the data types defined specifically for the SDD_DataStorage API service provided by SDDM-C.

Table A.4.3.3.1.1: SDD_DataStorage API provided by SDDM-C specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| DataStorageCreationRequest | A.4.3.3.2.1 | Information identifying an SDD data storage creation request. |  |
| DataStorageCreationResponse | A.4.3.3.2.2 | Information identifying an SDD data storage creation response. |  |
| DataStorageReservationRequest | A.4.3.3.2.3 | Information identifying an SDD data storage reservation request. |  |
| DataStorageReservationResponse | A.4.3.3.2.4 | Information identifying an SDD data storage reservation response. |  |
| DataStorageStatusNotification | A.4.3.3.2.5 | Information identifying an SDD data storage notification. |  |
| DataStorageQueryResponse | A.4.3.3.2.6 | Information identifying an SDD data storage query response. |  |
| DataStorageMgtRequest | A.4.3.3.2.7 | Information identifying an SDD data storage management request. |  |
| StatusInformationReq | A.4.3.3.2.8 | Information identifying the identity of stored data. |  |
| StatustInformationRes | A.4.3.3.2.9 | Information of the stored data returned by the SDDM-S which is tracked or monitored. |  |

Table A.4.3.3.1.2 specifies the simple data types defined specifically for the SDD_DataStorage API service provided by SDDM-C.

Table A.4.3.3.1.2: SDD_DataStorage API provided by SDDM-C specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |
| ServerId | A.2.5 | String representing a unique identifier of a VAL server. |

Table A.4.3.3.1.3 specifies the enumerations defined specifically for the SDD_DataStorage API service provided by SDDM-C.

Table A.4.3.3.1.3: SDD_DataStorage API provided by SDDM-C specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |

#### A.4.3.3.2	Structured data types

##### A.4.3.3.2.1	Type: DataStorageCreationRequest

Table A.4.3.3.2.1.1: Definition of type DataStorageCreationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationData | bytes | M | 1 | Information of the application data to be stored. |  |
| accessControlPolicy | string | M | 1 | Identity of the control policy for the requested data access from other consumers which is set to "SDDM-C", "VAL server" or "SDDM-S". |  |
| expiryTime | DateTime | O | 0..1 | Information of the expiration time of the data to be stored. |  |
| statusInformationReq | StatusInformationReq | O | 0..1 | Identity of the information of the stored data. |  |

##### A.4.3.3.2.2	Type: DataStorageCreationResponse

Table A.4.3.3.2.2.1: Definition of type DataStorageCreationResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the data storage creation request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the data storage creation request (NOTE 1). |  |
| dataIdentifier | string | O | 0..1 | Information of the identity of the stored data (NOTE 2). |  |
| NOTE 1: This attribute shall be included if result is set to "FAILURE". NOTE 2: This attribute shall be included if result is set to "SUCCESS". |  |  |  |  |  |

##### A.4.3.3.2.3	Type: DataStorageReservationRequest

Table A.4.3.3.2.3.1: Definition of type DataStorageReservationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServiceId | string | M | 1 | Identity of the VAL service of the vertical application. |  |
| dataLength | Uinteger | O | 0..1 | Identity of the data length to be stored. |  |

##### A.4.3.3.2.4	Type: DataStorageReservationResponse

Table A.4.3.3.2.4.1: Definition of type DataStorageReservationResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the data storage creation request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the data storage creation request (NOTE 1). |  |
| address | string | O | 0..1 | Information of the identity of the reserved address for data storage (NOTE 2). |  |
| NOTE 1: This attribute shall be included if result is set to "FAILURE". NOTE 2: This attribute shall be included if result is set to "SUCCESS". |  |  |  |  |  |

##### A.4.3.3.2.5	Type: DataStorageStatusNotification

Table A.4.3.3.2.5.1: Definition of type DataStorageStatusNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataIdentifier | string | M | 1 | Information of the identity of the stored data. |  |
| statusInformationRsp | StatusInformationRsp | M | 1 | Information of the stored data returned by the SDDM-S which is tracked or monitored. |  |

##### A.4.3.3.2.6	Type: DataStorageQueryResponse

Table A.4.3.3.2.6.1: Definition of type DataStorageQueryResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the data storage creation request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the data storage creation request (NOTE 1). |  |
| dataIdentifier | string | O | 0..1 | Information of the identity of the stored data (NOTE 2). |  |
| applicationData | bytes | O | 0..1 | Information of the application data to be stored (NOTE 3). |  |
| NOTE 1: This attribute shall be included if result is set to "FAILURE". NOTE 2: This attribute shall be included if result is set to "SUCCESS". NOTE 3: This attribute may be included if result is set to "SUCCESS". |  |  |  |  |  |

##### A.4.3.3.2.7	Type: DataStorageMgtRequest

Table A.4.3.3.2.7.1: Definition of type DataStorageMgtRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataIdentifier | string | M | 1 | Information of the identity of the stored data. |  |
| applicationData | bytes | O | 0..1 | Information of the application data to be stored. |  |

##### A.4.3.3.2.8	Type: StatusInformationReq

Table A.4.3.3.2.8.1: Definition of type StatusInformationReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| noTimesDataAccessed | boolean | O | 0..1 | Identify whether information of how many times the stored data is accessed is requested. Value "true" indicates that the information about how many times the stored data is accessed is requested. Value "false" indicates that the information about how many times the stored data is accessed is not requested. |  |
| noTimesDataManaged | boolean | O | 0..1 | Identify whether information of how many times the stored data is managed is requested. Value "true" indicates that the information about how many times the stored data is managed is requested. Value "false" indicates that the information about how many times the stored data is managed is not requested. |  |

##### A.4.3.3.2.9	Type: StatusInformationRes

Table A.4.3.3.2.9.1: Definition of type StatusInformationRes

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| noTimesDataAccessedValue | Uinteger | O | 0..1 | Information of the value of how many times the stored data is accessed. |  |
| noTimesDataManagedValue | Uinteger | O | 0..1 | Information of the value of how many times the stored data is managed. |  |

#### A.4.3.3.3	Simple data types and enumerations

None.

### A.4.3.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.4.3.5	CDDL Specification

#### A.4.3.5.1	Introduction

The data model described in clause A.4.3.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.4.3.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the SDD_DataStorage API provided by the SDDM-C data model.

#### A.4.3.5.2	CDDL document

;;; DataStorageCreationRequest

;;+ Represents a request for performing SDDM data data storage creation.

DataStorageCreationRequest = {

applicationData: bytes

accessControlPolicy: string

? expiryTime: DateTime

? statusInformationReq: StatusInformationReq

}

;;; DataStorageCreationResponse

;;+ Represents the response of a request for performing SDDM data data storage creation.

DataStorageCreationResponse = {

result: ResultOp

? cause: Cause

? dataIdentifier: string

}

;;; DataStorageReservationRequest

;;+ Represents a request for performing SDDM data data storage reservation.

DataStorageReservationRequest = {

valServiceId: string

? dataLength: Uinteger

}

;;; DataStorageReservationResponse

;;+ Represents the response of a request for performing SDDM data data storage reservation.

DataStorageReservationResponse = {

result: ResultOp

? cause: Cause

? address: string

}

;;; DataStorageStatusNotification

;;+ Represents the information of SDDM data storage status notification of the SDDM-C.

MeasurementSubscriptionNotification = {

dataIdentifier: string

statusInformationRsp: StatusInformationRsp

}

;;; DataStorageQueryResponse

;;+ Represents the response of a request for performing SDDM data data storage query.

DataStorageQueryResponse = {

result: ResultOp

? cause: Cause

? dataIdentifier: string

? applicationData: bytes

}

;;; DataStorageMgtRequest

;;+ Represents a request for performing SDDM data data storage management.

DataStorageMgtRequest = {

dataIdentifier: string

? applicationData: bytes

}

;;; StatusInformationReq

StatusInformationReq = {

? noTimesDataAccessed: bool

? noTimesDataManageed: bool

}

;;; StatusInformationRes

StatusInformationRes = {

noTimesDataAccessed: Uinteger

noTimesDataManageed: bool

}

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

### A.4.3.6	Media Types

See clause A.5.

### A.4.3.7	Void

### A.4.3.8	Void

### A.4.3.9	Void

### A.4.3.10	Void

### A.4.3.11	Void

### A.4.3.12	Void

### A.4.3.13	Void

## A.4.4	Sdd_XRTransmissionConnnection API

Editor's note [WID: XRM_Ph2_App, CR#: 0095]:	To align with stage 2 on changing XR to multi-modal is FFS.

### A.4.4.1	API URI

The CoAP URIs used in CoAP requests from SDDM-C towards the SDMM-S shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-c";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.4.4.2.

### A.4.4.2	Resources

#### A.4.4.2.1	Overview

Figure A.4.4.2.1.1: Resource URI structure of the Sdd_XRTransmissionConnnection API provided by SDDM-C

Table A.4.4.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.4.4.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD XR transmission connection | val-services/{valServiceId}/sdd-xr-transmission-connection | POST | Inform an SDDM XR transmission connection event. |
| SDD XR transmission connection establish | val-services/{valServiceId}/sdd-xr-transmission-connection-establish | POST | Establish an SDDM XR transmission connection event. |

#### A.4.4.2.2	Resource: SDD XR transmission connection inform

##### A.4.4.2.2.1	Description

The SDD XR transmission connection inform resource represents an SDD XR transmission connection inform to be created at a given SDDM-C and SDDM-S.

##### A.4.4.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-c/<apiVersion>/val-services/{valServiceId}/sdd-xr-transmission-connection inform

This resource shall support the resource URI variables defined in the table A.4.4.2.2.2.1.

Table A.4.4.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.4.4.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.4.4.2.2.3	Resource Standard Methods

###### A.4.4.2.2.3.1	POST

This operation allows to inform XR transmission connection status between two UEs.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.4.4.2.2.3.1.1 and A.4.4.2.2.3.1.2.

Table A.4.4.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| XRInformRequest | M | 1 | The information of the request informing of an SDDM XR transmission connection status. |

Table A.4.4.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| XRInformResponse | M | 1 | 2.01 Created | SDDM XR transmission connection inform created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

#### A.4.4.2.3	Resource: SDD XR transmission connection establish

##### A.4.4.2.3.1	Description

The SDD XR transmission connection establish resource represents an SDD XR transmission connection establish to be created at a given SDDM-C and SDDM-S.

##### A.4.4.2.3.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-c/<apiVersion>/val-services/{valServiceId}/sdd-xr-transmission-connection-establish

This resource shall support the resource URI variables defined in the table A.4.4.2.3.2.1.

Table A.4.4.2.3.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.4.4.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.4.4.2.3.3	Resource Standard Methods

###### A.4.4.2.3.3.1	POST

This operation allows to establish XR transmission connection between two UEs.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.4.4.2.3.3.1.1 and A.4.4.2.3.3.1.2.

Table A.4.4.2.3.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| XREstablishmentRequest | M | 1 | The information of request of establishment of an SDDM XR transmission connection. |

Table A.4.4.2.3.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| XREstablishmentResponse | M | 1 | 2.01 Created | SDDM XR transmission connection establishment created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

### A.4.4.3	Data Model

#### A.4.4.3.1	General

Table A.4.4.3.1.1 specifies the data types defined specifically for the Sdd_XRTransmissionConnnection API service provided by SDDM-C.

Table A.4.4.3.1.1: Sdd_XRTransmissionConnnection API provided by SDDM-C specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ProtocolDescriptorInfo | A.4.4.3.2.7 | Contains information of the protocol of the VAL traffic. |  |
| SealddFlowsInfo | A.4.4.3.2.5 | Contains the information of the XR SEALDD flows. |  |
| SealddMultiModalFlowInfo | A.4.4.3.2.6 | Contains the information of the XR SEALDD multi modal flow |  |
| valTgtUe | A.2.2 | Information identifying a VAL user ID or VAL UE ID. |  |
| XREstablishmentRequest | A.4.4.3.2.3 | Information identifying an SDDM XR transmission connection establishment request. |  |
| XREstablishmentResponse | A.4.4.3.2.4 | Information identifying an SDDM XR transmission connection establishment response. |  |
| XRInformRequest | A.4.4.3.2.1 | Information identifying an SDDM XR transmission connection inform request. |  |
| XRInformResponse | A.4.4.3.2.2 | Information identifying an SDDM XR transmission connection inform response. |  |

Table A.4.4.3.1.2 specifies the simple data types defined specifically for the Sdd_XRTransmissionConnnection API service provided by SDDM-C.

Table A.4.4.3.1.2: Sdd_XRTransmissionConnnection API provided by SDDM-C specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |
| Uinteger | A.2.3 | Unsigned integer. |
| ServerId | A.2.5 | String representing a unique identifier of a VAL server. |

Table A.4.4.3.1.3 specifies the enumerations defined specifically for the Sdd_XRTransmissionConnnection API service provided by SDDM-C.

Table A.4.4.3.1.3: Sdd_XRTransmissionConnnection API provided by SDDM-C specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |
| Cause | A.2.6.3 | Information identifying the reason of the cause of the failure of an operation. |
| ResultOp | A.2.6.2 | Information identifying the result of an operation. |
| Status | A.4.4.3.3.2 | Information identifying the UE-to-UE direct communication status. |

#### A.4.4.3.2	Structured data types

##### A.4.4.3.2.1	Type: XRInformRequest

Table A.4.4.3.2.1.1: Definition of type XRInformRequest

| Attribute name |  | Data type |  | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| requestorId |  | RequestorId |  | M | 1 | Identity of the requestor of the establishment request. (NOTE) |  |
| valUeIds |  | array(ValTargetUe) |  | M | 1..N | Information of the identities of the VAL UEs or VAL users involved in the UE-to-UE direct communication. |  |
| status |  | Status |  | M | 1 | Identifies the UE-to-UE direct communication status (i.e. established, released). |  |
| valServiceId |  | string |  | O | 0..1 | Identity of the VAL service enabled by the XR transmission connection. |  |
|  | NOTE: This attribute shall be set to "SEALDDCLIENT". |  |  |  |  |  |  |

##### A.4.4.3.2.2	Type: XRInformResponse

Table A.4.4.3.2.2.1: Definition of type XRInformResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the XR transmission connection request. |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the XR transmission connection inform request (NOTE). |  |
| NOTE: This attribute shall be included if result is set to "FAILURE". |  |  |  |  |  |

##### A.4.4.3.2.3	Type: XREstablishmentRequest

Table A.4.4.3.2.3.1: Definition of type XREstablishmentRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealClientId | string | M | 1 | Identity of the SDDM-C of the XR transmission connection establishment request. |  |
| sealddFlowsInfos | array(SealddFlowsInfo) | M | 1..N | Information of the SDDM flows used by the SDDM-C and SDDM-S. |  |
| valTgtUe | ValTargetUe | O | 0..1 | VAL user to whom the establishment request is applied. |  |
| serverId | ServerId | O | 0..1 | Identity of the VAL server. |  |
| valServiceId | String | O | 0..1 | Identity of the VAL service enabled by the URLLC transmission connection. |  |

##### A.4.4.3.2.4	Type: XREstablishmentResponse

Table A.4.4.3.2.4.1: Definition of type XREstablishmentResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | ResultOp | M | 1 | Result of the establishment request. |  |
| mmSealddFlow | SealddMultiModalFlowInfo | M | 1 | Information of the multi-modal SDDM flow used by the SDDM-C and SDDM-S (NOTE 2). |  |
| cause | Cause | O | 0..1 | Reason of the cause of the failure of the establishment request (NOTE 1). |  |
| protocolDescriptorInfo | ProtocolDescriptorInfo | O | 0..1 | Specifying the information of the protocol of the VAL traffic (NOTE 2). |  |
| NOTE 1: This attribute shall be included if result is set to "failure". NOTE 2: This attribute may be included if result is set to "success". |  |  |  |  |  |

##### A.4.4.3.2.5	Type: SealddFlowsInfo

Table A.4.4.3.2.5.1: Definition of type SealddFlowsInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealddFlowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and the SDDM-S to identify the application traffic. |  |
| userPlaneAddress | string | O | 0..1 | Identity of the IP address of the traffic. |  |
| portNumber | Uinteger | O | 0..1 | Identity of the port number of the traffic. |  |
| url | string | O | 0..1 | Identity of the address of a given unique resource on the Web for the traffic. |  |
| transportLayer | string | O | 0..1 | Identity of the transport layer protocol for the traffic. |  |

##### A.4.4.3.2.6	Type: SealddMultiModalFlowInfo

Table A.4.4.3.2.6.1: Definition of type SealddMultiModalFlowInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealddFlowsInfos | array(SealddFlowsInfo) | M | 1..N | Information of the SDDM flows used by the SDDM-C and the SDDM-S associated with the multi-modal flow. |  |

Editor's note [WID: XRM_Ph2_App, CR#: 0095]:	The definition of multimodalSealddFlowId is FFS.

##### A.4.4.3.2.7	Type: ProtocolDescriptorInfo

Table A.4.4.3.2.7.1: Definition of type ProtocolDescriptorInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| headerExtType | string | O | 0..1 | Specifying the header extension type (NOTE). |  |
| headerExtId | Uinteger | O | 0..1 | Specifying the header extension identity. Minimum: 1 Maximum: 255 (NOTE) |  |
| packetizationIndication | boolean | O | 0..1 | Indicating packetization for the traffic. "true": RTP header extension information included "false": RTP header extension information not included default value is "false". |  |
| payloadType | string | O | 0..1 | Specifying the type of the payload (e.g. RTP, SRTP). |  |
| payloadFormat | string | O | 0..1 | Specifying the format of the payload (e.g. H.264, H.265). |  |
| NOTE: This attribute shall be included if payloadType indicates RTP. |  |  |  |  |  |

#### A.4.4.3.3	Simple data types and enumerations

##### A.4.4.3.3.1	Simple data types

Table A.4.4.3.3.1.1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |

##### A.4.4.3.3.2	Enumeration: Status

Table A.4.4.3.3.2: Status

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ESTABLISHED | The UE-to-UE direct communication status is established. |  |
| RELEASED | The UE-to-UE direct communication status is released. |  |

### A.4.4.4	Error Handling

General error responses are defined in clause C.1.3 of 3GPP TS 24.546 [6].

### A.4.4.5	CDDL Specification

#### A.4.4.5.1	Introduction

The data model described in clause A.4.4.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [20].

Clause A.4.4.5.2 uses the concise data definition language described in IETF RFC 8610 [19] and provides corresponding representation of the Sdd_XRTransmissionConnnection API provided by the SDDM-C data model.

#### A.4.4.5.2	CDDL document

;;; XRInformRequest

;;+ Represents a request for performing SDDM XR transmission connection inform.

XRInformRequest = {

requestorId: RequestorId

valUeIds: [* ValTargetUe]

status: Status

? valServiceId: string

}

;;; XRInformResponse

;;+ Represents the response of a request for performing SDDM XR transmission connection inform.

XRInformResponse = {

result: ResultOp

? cause: Cause

}

;;; XREstablishmentRequest

;;+ Represents a request for establishing an XR transmission connection.

XREstablishmentRequest = {

sealClientId: string

sealddFlowsInfos: SealddFlowsInfo

valTgtUe: ValTargetUe

serverId: ServerId

valServiceId: string

}

;;; XREstablishmentResponse

;;+ Represents a response for establishing an XR transmission connection.

XREstablishmentResponse = {

result: ResultOp

mmSealddFlow: SealddMultiModalFlowInfo

? cause: Cause

? protocolDescriptorInfo: ProtocolDescriptorInfo

}

;;; SealddFlowsInfo

;;+ Represents the information of the SDDM flows.

SealddFlowsInfo = {

sealddFlowId: Uinteger

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

}

;;; SealddMultiModalFlowInfo

;;+ Represents the information of the multi-modal SDDM flow.

SealddMultiModalFlowInfo = {

sealddFlowsInfos: SealddFlowsInfo

}

;;; ProtocolDescriptorInfo

;;+ Represents the information of the protocol of the VAL traffic.

ProtocolDescriptorInfo = {

? headerExtType: string

? headerExtId: Uinteger

? packetizationIndication: boolean

? payloadType: string

? payloadFormat: string

}

;;; RequestorId

;;+ Indicates requestor of an operation.

RequestorId = "SEALDDCLIENT" / "SEALDDSERVER"

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; Status

;;+ Identifies the UE-to-UE direct communication status.

Status = "ESTABLISHED" / "RELEASED"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

Editor's note [WID: XRM_Ph2_App, CR#: 0095]:	The definition of multimodalSealddFlowId is FFS.

### A.4.4.6	Media Types

See clause A.5.

## A.5	Media types

### A.5.1	General

This clause defines media types and its model that are applicable to APIs defined for CoAP resource representations in the present specification.

NOTE:	Media types (formerly known as a multipurpose internet mail extensions (MIME) types) indicate the nature and format of a document, file, or assortment of bytes and are defined in IETF RFC 6838 [13A].

### A.5.2	Media type structure and definition

The media type for the APIs defined for CoAP resource representations shall be "application/vnd.3gpp.seal-data-delivery-info+cbor". This media type may be appended with a media type parameter to identify a particular data type, e.g., "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-req", "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-res", "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-status-notification".

Table A.5.2.1 lists the single media type for the APIs defined for CoAP resource representations with a required parameter to identify the defined data types.

Table A.5.2.1: Media type and parameter

| Media type and parameter | Section used | Description |
| --- | --- | --- |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req | 7.2.2.3, 7.2.2.4 | The media type and parameter for a request to establish an SDDM regular transmission connection. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-res | 7.2.3.3, 7.2.3.4 | The media type and parameter for a response of establishing an SDDM regular transmission connection. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=release-req | 7.2.3.3, 7.2.3.4 | The media type and parameter for a request to release an SDDM regular transmission connection. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-subscription-req | 7.2.14.3, 7.2.14.4, 7.2.15.3, 7.2.15.4 | The media type and parameter for a request to establish an SDDM data transmission quality measurement. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-subscription-res | 7.2.14.3, 7.2.14.4 | The media type and parameter for a response of establishing an SDDM data transmission quality measurement. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=measurement-notification | 7.2.15.3, 7.2.15.4 | The media type and parameter for notification of an SDDM data transmission quality measurement. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=tx-quality-mgt-req | 7.2.16.3, 7.2.16.4 | The media type and parameter for a request to establish an SDDM data transmission quality guarantee. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=tx-quality-mgt-res | 7.2.16.3, 7.2.16.4 | The media type and parameter for a response of establishing a SDDM data transmission quality guarantee. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-establishment-req | 7.2.4.3, 7.2.4.4, 7.2.18.3, 7.2.18.4 | The media type and parameter for a request to establish a URLLC transmission connection. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-establishment-res | 7.2.4.3, 7.2.4.4, 7.2.18.3, 7.2.18.4 | The media type and parameter for a response of establishing a URLLC transmission connection. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-update-req | 7.2.6.3, 7.2.6.4 | The media type and parameter for updating an established URLLC transmission connection shall. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-update-res | 7.2.6.3, 7.2.6.4 | The media type and parameter for updating an established URLLC transmission connection. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=urllc-release-req | 7.2.5.3, 7.2.5.4 | The media type and parameter for a request to release a URLLC transmission connection. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-req | 7.2.8.3, 7.2.8.4 | The media type and parameter for a request to create data storage to the SDDM-S. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-res | 7.2.8.3, 7.2.8.4 | The media type and parameter for a response of creating data storage. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-reservation-req | 7.2.9.3, 7.2.9.4 | The media type and parameter for a request to reserve data storage. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-reservation-res | 7.2.9.3, 7.2.9.4 | The media type and parameter for a response of reserving data storage. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-status-notification | 7.2.10.3, 7.2.10.4 | The media type and parameter for a data storage notification. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-data-storage-query-res | 7.2.11.3, 7.2.11.4 | The media type and parameter for a response of querying data storage. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-mgt-req | 7.2.12.3, 7.2.12.4, 7.2.10.3, 7.2.10.4 | The media type and parameter for a request to manage data storage |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-configuration-req | 7.2.19.3, 7.2.19.4 | The media type and parameter for a request to establish an SDDM connection status reporting configuration. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-configuration-res | 7.2.19.3, 7.2.19.4 | The media type and parameter for a response of establishing an SDDM connection status reporting configuration. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-notification | 7.2.21.3, 7.2.21.4 | The media type and parameter for notification of an SDDM connection status reporting. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-config-subsc | 7.2.21.3, 7.2.21.4 | The media type and parameter for request to obtain an SDDM connection status reporting. |
| application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-trigger-req | 7.2.22.3, 7.2.22.4 | The media type and parameter for a request to trigger an SDDM XR transmission connection operation. |
| application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-trigger-res | 7.2.22.3, 7.2.22.4 | The media type and parameter for a response of triggering an SDDM XR transmission connection operation. |
| application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-inform-req | 7.2.233, 7.2.23.4 | The media type and parameter for a request to inform an SDDM XR transmission connection status. |
| application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-inform-rsp | 7.2.23.3, 7.2.23.4 | The media type and parameter for a response of informing an SDDM XR transmission connection status. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-req | 7.2.24.3, 7.2.24.4 | The media type and parameter for request to establish XR transmission connection. |
| vnd.3gpp.seal-data-delivery-info+cbor;modeltype=xr-establishment-res | 7.2.24.3, 7.2.24.4 | The media type and parameter for a response of establishing XR transmission connection. |

### A.5.3	Media type registration template for application/vnd.3gpp.seal-data-delivery-info+cbor

Type name: application

Subtype name: vnd.3gpp.seal-data-delivery-info+cbor

Required parameters: none

Optional parameters: modeltype.

The "modetype" parameter identifies a specific data type, e.g, "vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req" where "establishment-req" indicates the "EstablishmentRequest" data type in 3GPP TS 24.543 clause A.2.4.2.

Encoding considerations: Must be encoded as using IETF RFC 8949 [20]. See data types defined in 3GPP TS 24.543 clause A.2, A.3, and A.4 for details. Clause A.5 provides the media type structure and definition.

Security considerations: See Section 10 of IETF RFC 8949 [20] and Section 11 of IETF RFC 7252 [14].

Interoperability considerations: Applications must ignore any key-value pairs that they do not understand. This allows backwards-compatible extensions to this specification.

Published specification: 3GPP TS 24.543 "Data Delivery Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification", available via http://www.3gpp.org/specs/numbering.htm.

Applications that use this media type: Applications supporting the SEAL data delivery management procedures as described in the published specification.

Fragment identifier considerations: Fragment identification is the same as specified for "application/cbor" media type in IETF RFC 8949 [20]. Note that currently that RFC does not define fragmentation identification syntax for "application/cbor".

Additional information:

Deprecated alias names for this type: N/A

Magic number(s): N/A

File extension(s): none

Macintosh file type code(s): none

Person & email address to contact for further information: <MCC name>, <MCC email address>

Intended usage: COMMON

Restrictions on usage: None

Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

Change controller: <MCC name>/<MCC email address>

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-03 | CT1#140 | C1-230388 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2023-03 | CT1#140 | C1-230389 C1-230394 C1-230395 C1-230868 C1-230869 C1-230870 C1-230871 C1-230872 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-230389, C1-230394, C1-230395, C1-230868, C1-230869, C1-230870, C1-230871, C1-230872; and editorial changes from the rapporteur. | 0.1.0 |
| 2023-10 | CT1#144 | C1-237196 C1-237283 C1-237546 C1-237607 C1-237654 C1-237692 C1-238061 C1-238062 C1-238063 C1-238064 C1-238065 C1-238066 C1-238067 C1-238068 C1-238070 C1-238071 C1-238072 C1-238073 C1-238075 C1-238198 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-237196, C1-237283, C1-237546, C1-237607, C1-237654, C1-237692, C1-238061, C1-238062, C1-238063, C1-238064, C1-238065, C1-238066, C1-238067, C1-238068, C1-238069, C1-238070, C1-238071, C1-238072, C1-238073, C1-238075, C1-238198; and editorial changes from the rapporteur. | 0.2.0 |
| 2023-11 | CT1#145 | C1-238643 C1-238644 C1-238649 C1-238650 C1-238656 C1-238657 C1-238658 C1-238659 C1-238660 C1-238661 C1-238663 C1-238664 C1-238665 C1-238666 C1-238667 C1-238674 C1-238675 C1-238676 C1-238677 C1-238678 C1-238679 C1-238680 C1-239159 C1-239400 C1-239401 C1-239402 C1-239403 C1-239494 C1-239405 C1-239408 C1-239409 C1-239410 C1-239411 C1-239412 C1-239414 C1-239571 C1-239577 C1-239578 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-238643, C1-238644, C1-238649, C1-238650, C1-238656, C1-238657, C1-238658, C1-238659, C1-238660, C1-238661, C1-238663, C1-238664, C1-238665, C1-238666, C1-238667, C1-238674, C1-238675, C1-238676, C1-238677, C1-238678, C1-238679, C1-238680, C1-239159, C1-239400, C1-239401, C1-239402, C1-239403, C1-239404, C1-239405, C1-239408, C1-239409, C1-239410, C1-239411, C1-239412, C1-239414, C1-239571, C1-239577, C1-239578; and editorial changes from the rapporteur. | 0.3.0 |
| 2023-11 | CT#102 | CP-233156 |  |  |  | Version 1.0.0 created for presentation to TSG CT#102 for information. | 1.0.0 |
| 2024-01 | CT1#146 | C1-240234 C1-240252 C1-240280 C1-240309 C1-240310 C1-240311 C1-240312 C1-240313 C1-240314 C1-240317 C1-240318 C1-240401 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-240234, C1-240252, C1-240280, C1-240309, C1-240310, C1-240311, C1-240312, C1-240313, C1-240314, C1-240317, C1-240318, C1-240401; and editorial changes from the rapporteur. | 1.1.0 |
| 2024-03 | CT1#147 | C1-240808 C1-240809 C1-240817 C1-240818 C1-240824 C1-240825 C1-240826 C1-241537 C1-241538 C1-241539 C1-241540 C1-241541 C1-241543 C1-241544 C1-241545 C1-241546 C1-241547 C1-241617 C1-241631 C1-241632 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-240808, C1-240809, C1-240817, C1-240818, C1-240824, C1-240825, C1-240826, C1-241537, C1-241538, C1-241539, C1-241540, C1-241541, C1-241543, C1-241544, C1-241545, C1-241546, C1-241547, C1-241617, C1-241631, C1-241632; and editorial changes from the rapporteur. | 1.2.0 |
| 2024-04 | CT1#148 | C1-242099 C1-242101 C1-242102 C1-242105 C1-242107 C1-242373 C1-242374 C1-242381 C1-242382 C1-242385 C1-242386 C1-242397 C1-242471 C1-242492 C1-242763 C1-242764 C1-242765 C1-242766 C1-242767 C1-242768 C1-242769 C1-242770 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-242099, C1-242101, C1-242102, C1-242105, C1-242107, C1-242373, C1-242374, C1-242381, C1-242382, C1-242385, C1-242386, C1-242397, C1-242471, C1-242492, C1-242763, C1-242764, C1-242765, C1-242766, C1-242767, C1-242768, C1-242769, C1-242770; and editorial changes from the rapporteur. | 1.3.0 |
| 2024-06 | CT1#149 | C1-243257 C1-243271 C1-243274 C1-243285 C1-243287 C1-243292 C1-243309 C1-243735 C1-243736 C1-243737 C1-243738 C1-243739 C1-243740 C1-243741 C1-243741 C1-243742 C1-243743 C1-243744 C1-243745 C1-243746 C1-243747 C1-243748 C1-243749 C1-243750 C1-243751 C1-243775 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-243257, C1-243271, C1-243274, C1-243285, C1-243287, C1-243292, C1-243309, C1-243735, C1-243736, C1-243737, C1-243738, C1-243739, C1-243740, C1-243741, C1-243742, C1-243743, C1-243744, C1-243745, C1-243746, C1-243747, C1-243748, C1-243749, C1-243750, C1-243751, C1-243776; and editorial changes from the rapporteur. | 1.4.0 |
| 2024-06 | CT#104 | CP-241145 |  |  |  | Version 2.0.0 created for presentation to TSG CT#104 for approval. | 2.0.0 |
| 2024-06 | CT#104 | CP-241145 |  |  |  | Approved in CT#104 | 18.0.0 |
| 2024-09 | CT#105 | CP-242196 | 0001 | 1 | F | Correction to numbering of clauses | 18.1.0 |
| 2024-09 | CT#105 | CP-242196 | 0002 | 1 | F | Correction to empty clauses | 18.1.0 |
| 2024-09 | CT#105 | CP-242196 | 0006 | 1 | F | Correction to the CDDL specification for the Sdd_TransmissionQualityMeasurement AP | 18.1.0 |
| 2024-09 | CT#105 | CP-242196 | 0003 | 2 | F | CDDL specification for the Sdd_RegularTransmissionConnection API provided by the SDDM-S | 18.1.0 |
| 2024-09 | CT#105 | CP-242196 | 0004 | 2 | F | CDDL specification for the Sdd_RegularTransmissionConnection API provided by the SDDM-C | 18.1.0 |
| 2024-09 | CT#105 | CP-242196 | 0005 | 2 | F | CDDL specification for the Sdd_URLCCTransmissionConnection API provided by the SDDM-C | 18.1.0 |
| 2024-12 | CT#106 | CP-243228 | 0007 | - | F | Correction to the <endpoint-id> element | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0009 | - | F | Correction to the SEALDD enabled E2E redundant transmission path connection update procedure based on CoAP | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0018 | - | F | HTTP related corrections | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0008 | 1 | F | Correction to the EstablishmentRequest type when provided by the SDDM-C | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0010 | 1 | F | Correction to the <sealdd-communication-lifetime> element | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0019 | 1 | F | COAP related corrections | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0011 | 1 | F | Correction to the SEALDD enabled signalling transmission connection establishment procedure based on HTTP | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0020 | 1 | F | XML schema: adding new messages | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0026 | - | F | Correction to misleading clause references | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0035 | 1 | F | Transmission quality measurement notification data type in COAP | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0042 | 4 | F | Correction to SEALDD data transmission quality measurement procedure for CoAP | 18.2.0 |
| 2024-12 | CT#106 | CP-243228 | 0034 | 4 | F | Correction to SEALDD enabled data storage notification procedure for CoAP | 18.2.0 |
| 2024-12 | CT#106 | CP-243229 | 0013 | - | C | Data semantics for SEALDD enabled URLLC transmission connection establishment based on policy procedure | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0015 | - | B | SEALDD enabled URLLC transmission connection deletion based on policy procedure based on HTTP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0025 | 1 | C | Update to the SEALDD server relocation procedure | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0021 | 1 | B | BAT and periodicity adaptation in transmission quality guarantee support in HTTP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0022 | 1 | B | BAT and periodicity adaptation in transmission quality guarantee support in COAP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0012 | 1 | B | SEALDD enabled URLLC transmission connection establishment based on policy procedure based on HTTP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0023 | 1 | B | SEALDD connection status reporting configuration support in HTTP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0024 | 1 | B | SEALDD connection status reporting configuration support in COAP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0017 | 2 | B | Introduction of Sdd_URLCCTransmissionConnection API provided by the SDDM-S | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0014 | 2 | B | SEALDD enabled URLLC transmission connection establishment based on policy procedure based on CoAP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0016 | 2 | B | SEALDD enabled URLLC transmission connection deletion based on policy procedure based on CoAP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0038 | - | B | SEALDD connection status reporting configuration notification - HTTP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0036 | 1 | B | BAT and periodicity adaptation support in SEALDD regular transmission connection establishment HTTP procedure | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0039 | 1 | B | SEALDD connection status reporting configuration notification - CoAP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0040 | 1 | B | Transmission quality measurement with Non-3GPP RAT - HTTP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0041 | 1 | B | Transmission quality measurement with Non-3GPP RAT - CoAP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0027 | 2 | B | Updates for SEALDD enabled congestion control for VAL application by supporting L4S mechanism for HTTP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0029 | 2 | B | SEALDD enabled regular data transmission connection establishment based on policy procedure based on HTTP | 19.0.0 |
| 2024-12 | CT#106 | CP-243229 | 0030 | 2 | B | Coding for SEALDD enabled regular data transmission connection establishment based on policy procedure | 19.0.1 |
| 2024-12 | CT#106 | CP-243229 | 0028 | 4 | B | Updates for SEALDD enabled congestion control for VAL application by supporting L4S mechanism for CoAP | 19.0.1 |
| 2024-12 | CT#106 | CP-243229 | 0032 | 4 | B | CDDL specification and Media types for the Sdd_URLCCTransmissionConnection API provided by the SDDM-S | 19.0.1 |
| 2024-12 | CT#106 | CP-243229 | 0033 | 4 | B | Updates to the Sdd_RegularTransmissionConnection API provided by the SDDM-S | 19.0.1 |
| 2024-12 | CT#106 | CP-243229 | 0037 | 2 | B | BAT and periodicity adaptation support in SEALDD regular transmission connection establishment COAP procedure | 19.0.1 |
| 2024-12 | CT#106 | CP-243229 | 0031 | 2 | B | SEALDD enabled regular data transmission connection establishment based on policy procedure for CoAP | 19.0.1 |
| 2025-03 | CT#107 | CP-250166 | 0044 | 1 | A | Update of MIME types for CBOR payload | 19.1.0 |
| 2025-03 | CT#107 | CP-250166 | 0046 | 1 | A | Correction to HTTP procedures | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0047 | 1 | B | Resolution of the editor's note related to CR#0033 | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0048 | 1 | F | Resolution to the editor's notes on CR#0012 and 0014 | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0049 | 1 | F | Resolution to the editor's notes on CR#0015 and 0016 | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0050 | 1 | B | Removal of EN on Sdd_ConnectionStatusEvent API update | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0051 |  | B | Removal of EN on batPeriodAdaptCap and transmisAssistInfo attributes | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0052 | 1 | B | Support of XR Application device capability information in HTTP | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0053 | 1 | B | Support of XR Application device capability information in CoAP | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0054 | 1 | B | SEALDD data transmission quality measurement with Non-3GPP RAT - HTTP | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0055 | 1 | B | SEALDD data transmission quality measurement with Non-3GPP RAT - CoAP | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0056 | 1 | B | SEALDD non-3GPP access for connection status reporting configuration procedure - HTTP | 19.1.0 |
| 2025-03 | CT#107 | CP-250152 | 0057 | 1 | B | SEALDD non-3GPP access for connection status reporting configuration procedure - CoAP | 19.1.0 |
| 2025-06 | CT#108 | CP-251166 | 0058 | 1 | B | Removal of EN on update of ConnectionStatusNotification | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0060 | 1 | B | Resolution of EN on reporting mode, interval and priority | 19.2.0 |
| 2025-06 | CT#108 | CP-251176 | 0061 | 1 | B | SEALDD XR transmission connection trigger procedure - HTTP | 19.2.0 |
| 2025-06 | CT#108 | CP-251176 | 0062 | 1 | B | SEALDD XR transmission connection trigger procedure - CoAP | 19.2.0 |
| 2025-06 | CT#108 | CP-251176 | 0063 | 1 | B | SEALDD XR transmission connection inform procedure - HTTP | 19.2.0 |
| 2025-06 | CT#108 | CP-251176 | 0064 | 1 | B | SEALDD XR transmission connection inform procedure - CoAP | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0069 | - | F | Resolution of editor's note under clause A.3.1.5.2 introduced by CR#0037 | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0059 | 2 | F | Implementation of CRs 0014 and 0031 | 19.2.0 |
| 2025-06 | CT#108 | CP-251176 | 0073 | 1 | B | SEALDD enabled XR data transmission establishment service for XR application-HTTP | 19.2.0 |
| 2025-06 | CT#108 | CP-251176 | 0074 | 1 | B | SEALDD enabled XR data transmission establishment for XR application-CoAP | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0071 | 2 | F | Resolution of editor's note under clause 8.4.2 introduced by CR#0023 | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0066 | 1 | F | Correction to the <transmission-assist-info> element | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0068 | 1 | F | Correction to the use of and/or term | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0070 | 1 | F | Resolution of editor's note under clause 8.4.2 introduced by CR#0030 | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0072 | 1 | B | Corrections on SEALDD enabled URLLC transmission connection establishment | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0065 | 2 | F | Correction to the <bat-period-adapt-cap> element | 19.2.0 |
| 2025-06 | CT#108 | CP-251166 | 0075 | 3 | F | Correction to the Sdd_RegularTransmissionConnection API provided by the SDDM-S | 19.2.0 |
| 2025-09 | CT#109 | CP-252162 | 0099 | - | F | Removal of EN related to XR data transmission inform and trigger | 19.3.0 |
| 2025-09 | CT#109 | CP-252133 | 0080 | 1 | A | Resolution of editor's note under clause A.5.2 | 19.3.0 |
| 2025-09 | CT#109 | CP-252133 | 0083 | 1 | A | Resolution of editor's note under clause 8.6 | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0076 | 1 | B | Identification of added messages | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0077 | 1 | B | BAT and periodicity adaptation for HTTP | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0078 | 1 | B | BAT and periodicity adaptation for CoAP | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0084 | 1 | F | Resolution of editor's note under clause A.2.4 | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0085 | 1 | F | Resolution of editor's note under clause A.3.1.3.2.3 | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0090 | 1 | F | Resolution of editor's note on support of L4S mechanism under the XML schema clause | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0091 | 1 | F | Correction to the <L4S-feedback-capability> element | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0092 | 1 | F | Correction to the <XR-establishment-req> and <XR-establishment-rsp> elements | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0093 | 1 | F | Correction because of approved CR in S6-251365 | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0098 | 1 | F | Removal of EN related to Connection status reporting configuration notification | 19.3.0 |
| 2025-09 | CT#109 | CP-252162 | 0094 | 2 | B | SEALDD enabled XR data transmission establishment service for XR application-HTTP-XML Schema and Data Semantics | 19.3.0 |
| 2025-09 | CT#109 | CP-252162 | 0095 | 2 | B | SEALDD enabled XR data transmission establishment for XR application-CoAP Coding | 19.3.0 |
| 2025-09 | CT#109 | CP-252151 | 0086 | 2 | F | Resolution of editor's note under clause A.3.1.5.2 | 19.3.0 |