| 3GPP TS 38.533 V19.1.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM) (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	60

1	Scope	61

2	References	61

3	Definitions, symbols and abbreviations	62

3.1	Definitions	62

3.2	Symbols	64

3.3	Abbreviations	64

3A	Requirements for the support of RRM	67

3A.1	General	67

3A.1.0	Overview of RRM requirements	67

3A.1.1	Test coverage across 5G NR connectivity options	67

3A.2	Requirements classification for statistical testing	68

3A.3	Antenna configuration	68

3A.4	NR band groups	68

3A.4.0	General	68

3A.4.1	NR operating bands in FR1	68

3A.4.1A	NR operating bands for satellite access in FR1	70

3A.4.2	NR operating bands in FR2	71

3A.5	NR operating band configuration	72

3A.6	UE with multiband capability	72

4	EN-DC with all NR cells in FR1	72

4.0	General	72

4.1	Void	73

4.2	Void	73

4.3	RRC_CONNECTED state mobility	73

4.3.1	Void	73

4.3.2	RRC connection mobility control	73

4.3.2.1	Void	73

4.3.2.2	Random access	73

4.3.2.2.1	EN-DC FR1 contention based random access	73

4.3.2.2.2	EN-DC FR1 non-contention based random access	79

4.3.2.2.3	EN-DC FR1 2-step contention based random access	86

4.3.2.2.4	EN-DC FR1 2-step non-contention based random access	92

4.3.2.3	Void	98

4.4	Timing	98

4.4.1	UE transmit timing	98

4.4.1.0	Minimum conformance requirements	98

4.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	98

4.4.1.1	EN-DC FR1 UE transmit timing accuracy	100

4.4.2	UE timer accuracy	106

4.4.3	Timing advance	106

4.4.3.0	Minimum conformance requirements	106

4.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	106

4.4.3.0.2	Minimum conformance requirements for timing advance adjustment delay	107

4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	107

4.5	Signalling characteristics	112

4.5.1	Radio link monitoring	112

4.5.1.00	General	112

4.5.1.0	Minimum conformance requirements	112

4.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	112

4.5.1.0.2	Void	113

4.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	113

4.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	114

4.5.1.0.5	Requirements for UE configured with relaxed measurement criteria	116

4.5.1.1	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	117

4.5.1.2	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	122

4.5.1.3	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	127

4.5.1.4	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	132

4.5.1.5	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	137

4.5.1.6	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	142

4.5.1.7	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	147

4.5.1.8	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	152

4.5.1.9	EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	158

4.5.2	Interruption	163

4.5.2.0	Minimum conformance requirements	163

4.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX.	163

4.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	164

4.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	165

4.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	165

4.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	167

4.5.2.0.6	Minimum conformance requirements for interruptions due to RRM and RLM/BFD measurements on deactivated NR SCG	168

4.5.2.1	EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	168

4.5.2.2	EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	172

4.5.2.3	EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	176

4.5.2.4	EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	182

4.5.2.5	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	188

4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	194

4.5.2.7	198

4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	199

4.5.2.9	E-UTRAN - NR FR1 interruptions at E-UTRA SRS carrier based switching	204

4.5.2.10	EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	210

4.5.3	SCell activation and deactivation delay	214

4.5.3.0	Minimum conformance requirements	214

4.5.3.0.1	SCell activation and deactivation delay	214

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	218

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	229

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX	231

4.5.3.4	SCell activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	233

4.5.3.5	EN-DC FR1 direct SCell activation at SCell addition of known SCell	240

4.5.3.6	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	250

4.5.3.7	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	258

4.5.3.8 to 4.5.3.10	261

4.5.3.11	EN-DC FR1 TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band in non-DRX	261

4.5.3.12	EN-DC FR1 Inter-band SSB-less SCell activation using A-TRS	271

4.5.4	UE UL carrier RRC reconfiguration delay	279

4.5.4.1	EN-DC FR1 UE UL carrier RRC reconfiguration delay	279

4.5.5	Link recovery procedures	287

4.5.5.0	Minimum conformance requirements	287

4.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	289

4.5.5.0.4	Requirements for beam failure recovery in SCell	289

4.5.5.0.5	TRP specific minimum conformance requirements	289

4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX	292

4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX	299

4.5.5.3	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	305

4.5.5.4	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX	312

4.5.5.5	EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	319

4.5.5.5.1	Test purpose	319

4.5.5.6	EN-DC FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	325

4.5.5.7	EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX	331

4.5.5.8	EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	339

4.5.6	Active BWP switch delay	347

4.5.6.1	DCI-based and time-based active BWP switch	347

4.5.6.1.0	Minimum conformance requirements	347

4.5.6.1.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	350

4.5.6.1.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	357

4.5.6.2	RRC-based active BWP switch	367

4.5.6.2.0	Minimum conformance requirements	367

4.5.6.2.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	367

4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	373

4.5.6.3.0	Minimum conformance requirements	373

4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	375

4.5.6.4	Simultaneous RRC-based active BWP switch on multiple CCs	384

4.5.6.4.0	Minimum conformance requirements	384

4.5.6.4.0.1	Minimum conformance requirements for simultaneous RRC-based BWP switch delay on multiple CCs	384

4.5.6.4.0.2	Minimum conformance requirements for DCI and timer-based BWP switch delay on a single CC	385

4.5.6.4.0.3	Minimum conformance requirements for interruption for NR victim cell	387

4.5.6.4.0.3.1	Interruptions due to SCell dormancy switch	387

4.5.6.4.0.3.2	Interruptions due to CQI measurements during SCell dormancy	387

4.5.6.4.0.3.3	Interruptions due to RRM measurements during SCell dormancy	387

4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	388

4.5.6.5	SCell dormancy switch	396

4.5.7	PSCell addition and release delay	396

4.5.7.0	Minimum conformance requirements	396

4.5.7.0.1	NR PSCell addition delay requirement	396

4.5.7.0.2	NR PSCell release delay requirement	397

4.5.7.1	EN-DC FR1 addition and release delay of known PSCell	397

4.5.8	UL switching	403

4.5.8.0	Minimum conformance requirements	403

4.5.8.1	EN-DC FR1 interruptions at switching between two uplink carriers	403

4.5.9	UE specific CBW change	409

4.5.9.0	Minimum conformance requirements	409

4.5.9.0.1	Minimum conformance requirements for UE specific CBW change delay	409

4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN-DC	410

4.5.10	PSCell activation and deactivation delay	414

4.5.10.0	Minimum conformance requirements	414

4.5.10.1	EN-DC FR1 PSCell activation and deactivation delay	415

4.5.11	PSCell addition and release delay	420

4.5.11.0	Minimum conformance requirements	420

4.5.11.0.1	Measurement time	421

4.5.11.1	EN-DC FR1 addition and release delay of PSCell	421

4.6	Measurement procedures	427

4.6.1	Intra-frequency measurements	427

4.6.1.0	Minimum conformance requirements	427

4.6.1.0.1	Minimum conformance requirements for event-triggered reporting without gap	428

4.6.1.0.2	Minimum conformance requirements for event-triggered measurements with gap	433

4.6.1.1	EN-DC FR1 event-triggered reporting without gap in non-DRX	435

4.6.1.2	EN-DC FR1 event-triggered reporting without gap in DRX	440

4.6.1.3	EN-DC FR1 event-triggered reporting with gap in non-DRX	443

4.6.1.4	EN-DC FR1 event-triggered reporting with gap in DRX	449

4.6.1.5	EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection	454

4.6.1.6	EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection	457

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	462

4.6.1.8	EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17	466

4.6.2	Inter-frequency measurements	472

4.6.2.0	Minimum conformance requirements for inter-frequency measurements	472

4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX	475

4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX	480

4.6.2.3	Void	485

4.6.2.4	Void	485

4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	485

4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection	490

4.6.2.7	Void	495

4.6.2.8	Void	495

4.6.2.9	EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	495

4.6.3	Void	499

4.6.4	L1-RSRP measurement for beam reporting	499

4.6.4.0	Minimum conformance requirements	499

4.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	499

4.6.4.1	EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX	503

4.6.4.1.5	Test requirement	507

4.6.4.2	EN-DC FR1 SSB-based L1-RSRP measurement in DRX	507

4.6.4.2.3	Minimum conformance requirements	508

4.6.4.3	EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX	511

4.6.4.4	EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX	515

4.6.4.5	EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	518

4.6.5	CLI measurements	523

4.6.5.0	Minimum conformance requirements	523

4.6.5.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	524

4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX	525

4.6.5.2	EN-DC FR1 CLI-RSSI measurement in non-DRX	530

4.6.6	Measurements with autonomous gaps	533

4.6.6.0	Minimum conformance requirements	533

4.6.6.0.1	Minimum conformance requirements for measurements with autonomous gaps	533

4.6.6.0.1.1	Minimum conformance requirements for E-UTRA FDD - NR measurements with autonomous gaps	533

4.6.6.0.1.2	Minimum conformance requirements for E-UTRA TDD - NR measurements with autonomous gaps	534

4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	534

4.6.7	L1-SINR measurement for beam reporting	539

4.6.7.0	Minimum conformance requirements	539

4.6.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	544

4.6.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	550

4.6.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	555

4.6.8	CSI-RS based intra-frequency measurement	559

4.6.8.0	Minimum conformance requirements	559

4.6.8.0.1	Minimum conformance requirements for EN-DC event triggered reporting tests without gap under DRX	559

4.6.9	CSI-RS based inter-frequency measurement	561

4.6.9.0	Minimum conformance requirements	561

4.6.9.0.1	Minimum conformance requirements for EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	561

4.7	Measurement performance requirements	563

4.7.1	SS-RSRP	563

4.7.1.0	Minimum conformance requirements	563

4.7.1.1	Intra-frequency measurements	568

4.7.1.1.1	EN-DC FR1 SS-RSRP absolute measurement accuracy	568

4.7.1.1.2	EN-DC FR1 SS-RSRP relative measurement accuracy	573

4.7.1.2	Inter-frequency measurements	576

4.7.1.2.1	EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy	576

4.7.1.2.2	EN-DC FR1-FR1 SS-RSRP relative measurement accuracy	581

4.7.2	SS-RSRQ	582

4.7.2.0	Minimum conformance requirements	582

4.7.2.1	EN-DC FR1 SS-RSRQ measurement accuracy	585

4.7.2.2	Inter-frequency SS-RSRQ measurement accuracy	589

4.7.2.2.1	EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy	589

4.7.2.2.2	EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy	592

4.7.3	SS-SINR	594

4.7.3.0	Minimum conformance requirements	594

4.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	594

4.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	595

4.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	596

4.7.3.1	EN-DC FR1 SS-SINR measurement accuracy	597

4.7.3.2	Inter-Frequency SS-SINR measurement accuracy	601

4.7.3.2.1	EN-DC FR1-FR1 SS-SINR absolute measurement accuracy	601

4.7.3.2.2	EN-DC FR1-FR1 SS-SINR relative measurement accuracy	604

4.7.4	L1-RSRP	606

4.7.4.0	Minimum conformance requirements	606

4.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	606

4.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	609

4.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	610

4.7.4.1	SSB based L1-RSRP measurements	611

4.7.4.1.1	EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy	611

4.7.4.1.2	EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy	616

4.7.4.2	CSI-RS based L1-RSRP measurements	618

4.7.4.2.1	EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy	618

4.7.4.2.2	EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy	622

4.7.5	SFTD	624

4.7.5.0	Minimum conformance requirements	624

4.7.5.0.1	SFTD accuracy requirement	624

4.7.5.1	EN-DC FR1 SFTD measurement accuracy	625

4.7.6	CLI measurements	629

4.7.6.0	Minimum conformance requirements	629

4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy	629

4.7.6.0.1.1	SRS-RSRP report mapping	630

4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	631

4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	631

4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	640

4.7.7	L1-SINR measurement for beam reporting	643

4.7.7.0	Minimum conformance requirements	643

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	643

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	646

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	648

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	652

4.7.7.1.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	652

4.7.7.1.2	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	657

4.7.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	659

4.7.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement	664

4.7.7.3.1	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	664

4.7.7.3.2	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	670

4.7.8	CSI-RSRP	672

4.7.8.0	Minimum conformance requirements	672

4.7.8.0.1	Intra-frequency absolute CSI-RSRP measurement accuracy	672

4.7.8.0.2	Intra-frequency relative CSI-RSRP measurement accuracy requirements	675

4.7.8.0.3	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	676

4.7.8.0.4	Inter-frequency relative CSI-RSRP measurement accuracy requirements	677

4.7.8.1	Intra-frequency CSI-RSRP measurement accuracy	678

4.7.8.1.1	EN-DC FR1 CSI-RSRP absolute measurement accuracy	678

4.7.8.1.2	EN-DC FR1 CSI-RSRP relative measurement accuracy	685

4.7.8.2	Inter-frequency measurement accuracy	687

4.7.8.2.1	EN-DC FR1 CSI-RSRP absolute measurement accuracy	687

4.7.8.2.2	EN-DC FR1 CSI-RSRP relative measurement accuracy	693

4.7.9	CSI-RSRQ	695

4.7.9.0	Minimum conformance requirements	695

4.7.9.0.1	Intra-frequency CSI-RSRQ accuracy requirements	695

4.7.9.0.2	Inter-frequency CSI-RSRQ accuracy requirements	696

4.7.9.1	EN-DC iintra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	699

4.7.9.1.1	Test purpose	699

4.7.9.1.2	Test applicability	699

4.7.9.1.3	Minimum conformance requirements	699

4.7.9.1.4	Test description	699

4.7.9.1.5	Test requirements	701

4.7.9.2	EN-DC iinter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	705

4.7.9.2.1	Test purpose	705

4.7.9.2.2	  Test applicability	705

4.7.9.2.3	Minimum conformance requirements	705

4.7.9.2.4	Test description	705

4.7.9.2.5	Test requirements	707

4.7.10	CSI-SINR	711

4.7.10.0	Minimum conformance requirements	711

4.7.10.0.1	Intra-frequency CSI-SINR accuracy requirements in FR1	711

4.7.10.0.1.1	Absolute CSI-SINR accuracy in FR1	711

4.7.10.0.2	Inter-frequency CSI-SINR accuracy requirements in FR1	712

4.7.10.0.2.1	Absolute accuracy of CSI-SINR in FR1	712

4.7.10.0.2.2	Relative accuracy of CSI-SINR in FR1	714

4.7.10.1	EN-DC iintra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	715

4.7.10.1.1	Test purpose	715

4.7.10.1.2	  Test applicability	715

4.7.10.1.3	Minimum conformance requirements	715

4.7.10.1.4	Test description	715

4.7.10.1.5	Test requirements	717

4.7.10.2	EN-DC iinter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	720

4.7.10.2.1	Test purpose	720

4.7.10.2.2	Test applicability	720

4.7.10.2.3	Minimum conformance requirements	720

4.7.10.2.4	Test description	721

4.7.10.2.5	Test requirements	722

4.7.11	TDCP amplitude measurement accuracy	725

4.7.11.0	Minimum conformance requirements	725

4.7.11.0.1	TDCP amplitude measurement accuracy requirements in FR1	725

4.7.11.1	TDCP amplitude measurement accuracy in EN-DC	726

4.7.11.1.1	Test purpose	726

4.7.11.1.2	Test applicability	726

4.7.11.1.3	Minimum conformance requirements	726

4.7.11.1.4	Test description	726

4.7.11.1.5	Test requirements	728

4A	NE-DC with all NR cells in FR1	730

4A.0	General	730

4A.1	Signalling characteristics	730

4A.1.1	E-UTRAN PSCell addition	730

4A.1.1.0	Minimum conformance requirements	730

4A.1.1.0.1	E-UTRA PSCell addition delay requirement	730

4A.1.1.0.2	E-UTRA PSCell release delay requirement	731

4A.1.1.1	NE-DC FR1 addition and release delay of known PSCell	731

4A.1.2	Active BWP switch delay	737

4A.1.2.0	Minimum conformance requirements	737

4A.1.2.1	NE-DC FR1 DCI-based and timer-based DL active BWP switch in non-DRX in synchronous NE-DC	737

4A.2	Measurement performance requirements	741

4A.2.1	SFTD accuracy	741

4A.2.1.0	Minimum conformance requirements	741

4A.1.1.0.1	NE-DC SFTD accuracy requirement	741

4A.2.1.1	NE-DC FR1 SFTD accuracy	743

5	EN-DC with at least one NR cell in FR2	752

5.0	General	752

5.1	Void	752

5.2	Void	752

5.3	RRC_CONNECTED state mobility	752

5.3.1	Void	752

5.3.2	RRC connection mobility control	752

5.3.2.1	Void	752

5.3.2.2	Random access	752

5.3.2.2.1	EN-DC FR2 contention based random access	752

5.3.2.2.2	EN-DC FR2 non-contention based random access	760

5.3.2.2.3	EN-DC FR2 2-step contention based random access	769

5.3.2.2.4	EN-DC FR2 2-step non-contention based random access	773

5.3.2.3	Void	777

5.4	Timing	777

5.4.1	UE transmit timing	777

5.4.1.0	Minimum Conformance Requirements	777

5.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	777

5.4.1.1	EN-DC FR2 UE transmit timing accuracy	779

5.4.1.2	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	784

5.4.2	UE timer accuracy	792

5.4.3	Timing advance	792

5.4.3.0	Minimum conformance requirements	792

5.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	792

5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	793

5.5	Signaling characteristics	798

5.5.1	Radio link monitoring	798

5.5.1.0	Minimum conformance requirements	798

5.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	798

5.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	799

5.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	800

5.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	803

5.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	806

5.5.1.0.6	Requirements for UE configured with Relaxed Measurement Criteria	807

5.5.1.1	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode	807

5.5.1.2	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode	814

5.5.1.3	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in DRX mode	819

5.5.1.4	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM-RS in DRX mode	825

5.5.1.5	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode	830

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in non-DRX mode	835

5.5.1.7	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode	840

5.5.1.8	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM-RS in DRX mode	846

5.5.1.9	EN-DC FR2 radio link monitoring UE scheduling restrictions	852

5.5.1.10	EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM-RS for UE fulfilling relaxed measurement criterion	857

5.5.1.11	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM-RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	862

5.5.2	Interruption	868

5.5.2.0	Minimum conformance requirements	868

5.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX	868

5.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	869

5.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	869

5.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	870

5.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	871

5.5.2.1	EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	872

5.5.2.2	EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	877

5.5.2.3	EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	881

5.5.2.4	EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	887

5.5.2.5	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	892

5.5.2.6	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	896

5.5.2.7	E-UTRAN - NR FR2 interruptions at E-UTRA SRS carrier based switching	900

5.5.2.8	E-UTRAN - NR FR2 interruptions at NR SRS carrier based switching	906

5.5.3	SCell activation and deactivation delay	912

5.5.3.1	EN-DC FR2 SCell activation and deactivation intra-band in non-DRX	924

5.5.3.2 to 5.5.3.5	931

5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	931

5.5.3.7	EN-DC FR2 direct SCell activation at SCell addition of known SCell	939

5.5.3.8	EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band	943

5.5.4	UE UL carrier RRC reconfiguration delay	952

5.5.5	Link recovery procedures	952

5.5.5.0	Minimum conformance requirements	952

5.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	952

5.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	956

5.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	961

5.5.5.0.4	Requirements for Beam Failure Recovery in SCell	962

5.5.5.0.5	Requirements for SSB based beam failure detection for UE fulfilling relaxed measurement criteria	962

5.5.5.0.6	TRP specific Minimum conformance requirements	963

5.5.5.0.6.1	Requirements for TRP specific CSI-RS based beam failure detection	963

5.5.5.0.6.2	Requirements for CSI-RS based candidate beam detection	964

5.5.5.0.6.3	Requirements for TRP specific Beam Failure Recovery	965

5.5.5.0.6.4	Scheduling availability of UE during TRP specific beam failure detection	965

5.5.5.0.6.5	Scheduling availability of UE during TRP specific candidate beam detection	965

5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX	966

5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX	973

5.5.5.3	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	980

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX	987

5.5.5.5	EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX	994

5.5.5.6	EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX	1001

5.5.5.7	EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX	1008

5.5.5.8	EN-DC FR2 CSI-RS-based PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode	1014

5.5.5.9	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion	1021

5.5.6	Active BWP switch delay	1028

5.5.6.1	DCI-based and time-based active BWP switch	1028

5.5.6.1.0	Minimum conformance requirements	1028

5.5.6.1.1	EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	1028

5.5.6.1.2	EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	1029

5.5.6.2	RRC-based active BWP switch	1029

5.5.6.2.0	Minimum conformance requirements	1029

5.5.6.2.1	EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	1030

5.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	1037

5.5.6.3.0	Minimum conformance requirements	1037

5.5.6.3.0.1	Minimum conformance requirements for simultaneous DCI-based and timer-based active BWP switch on multiple CCs	1037

5.5.6.3.0.1.1	Minimum conformance requirements for simultaneous DCI-based active BWP switch on multiple CCs	1037

5.5.6.3.0.1.2	Minimum conformance requirements for simultaneous timer-based active BWP switch on multiple CCs	1039

5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1039

5.5.6.4	SCell dormancy switch	1043

5.5.6.4.0	Minimum conformance requirements	1043

5.5.6.4.0.1	Minimum conformance requirements for interruptions due to SCell dormancy switch	1043

5.5.6.4.0.2	Minimum conformance requirements for interruptions due to CQI measurements during SCell dormancy	1044

5.5.6.4.0.3	Minimum conformance requirements for interruptions due to RRM measurements during SCell dormancy	1044

5.5.6.4.0.4	Minimum conformance requirements for DCI and timer-based BWP switch delay on a single CC	1044

5.5.6.4.0.5	Minimum conformance requirements for simultaneous DCI based BWP switch delay on multiple CCs	1045

5.5.6.4.0.6	Minimum conformance requirements for interruptions due to NR SCell dormancy switch	1047

5.5.6.4.0.7	Minimum conformance requirements for interruptions due to CSI and RRM measurements during SCell dormancy	1047

5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1047

5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1051

5.5.6.5.0	Minimum conformance requirements	1051

5.5.6.5.0.1	Minimum conformance requirements for E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1051

5.5.6.5.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1052

5.5.7	PSCell addition and release delay	1058

5.5.7.0	Minimum conformance requirements	1058

5.5.7.1	Addition and Release Delay of NR PSCell	1058

5.5.8	Active TCI state switch delay	1063

5.5.8.0	Minimum conformance requirements	1063

5.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	1063

5.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	1064

5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch	1065

5.5.8.2	EN-DC FR2 RRC based active TCI state switch	1069

5.5.9	Uplink spatial relation switch delay	1073

5.5.9.0	Minimum conformance requirements	1073

5.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	1073

5.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	1074

5.5.9.1	MAC-CE based uplink spatial relation switch	1074

5.5.9.1.1	E-UTRAN - NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1074

5.5.9.2	RRC based uplink spatial relation switch	1078

5.5.9.2.1	E-UTRAN - NR PSCell FR2 spatial relation switch associated with a known DL-RS	1078

5.5.10	UE specific CBW change	1082

5.5.10.0	Minimum conformance requirements	1082

5.5.10.0.1	Minimum conformance requirements for UE specific CBW change	1082

5.5.10.1	UE specific CBW change on FR2 NR PSCell	1083

5.5.11	Unified TCI state switch delay	1087

5.5.11.0	Minimum conformance requirements	1087

5.5.11.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	1087

5.5.11.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	1089

5.5.11.1	EN-DC FR2 MAC-CE based active joint TCI state switch	1091

5.5.11.2	EN-DC FR2 MAC-CE based active uplink TCI state switch	1098

5.5.11.3	EN-DC FR2 MAC-CE based active downlink TCI state switch	1104

5.5.12	PSCell activation and deactivation delay	1110

5.5.12.0	Minimum conformance requirements	1110

5.5.12.1	EN-DC FR2 PSCell activation and deactivation delay	1111

5.5.13	Conditional PSCell addition and release delay	1115

5.5.13.1	EN-DC FR2 Addition and Release Delay of NR PSCell	1115

5.6	Measurement procedures	1121

5.6.1	Intra-frequency measurements	1121

5.6.1.0	Minimum conformance requirements	1121

5.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1121

5.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1124

5.6.1.1	EN-DC FR2 event-triggered reporting without gap in non-DRX	1125

5.6.1.2	EN-DC FR2 event-triggered reporting without gap in DRX	1131

5.6.1.3	EN-DC FR2 event-triggered reporting with gap in non-DRX	1136

5.6.1.4	EN-DC FR2 event-triggered reporting with gap in DRX	1141

5.6.2	Inter-frequency measurements	1146

5.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1146

5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX	1148

5.6.2.2	EN-DC FR2-FR2 event-triggered reporting in DRX	1153

5.6.2.3	EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	1158

5.6.2.4	EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection	1163

5.6.2.5	EN-DC FR1-FR2 event-triggered reporting in non-DRX	1168

5.6.2.6	EN-DC FR1-FR2 event-triggered reporting in DRX	1173

5.6.2.7	EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	1179

5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection	1184

5.6.3	L1-RSRP measurement for beam reporting	1189

5.6.3.0	Minimum conformance requirements	1190

5.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1190

5.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1191

5.6.3.1	EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX	1194

5.6.3.2	EN-DC FR2 SSB-based L1-RSRP measurement in DRX	1198

5.6.3.3	EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX	1202

5.6.3.4	EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX	1206

5.6.4	CLI measurements	1209

5.6.4.0	Minimum conformance requirements	1209

5.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	1210

5.6.4.1	EN-DC FR2 SRS-RSRP measurement in non-DRX	1211

5.6.4.2	EN-DC FR2 CLI-RSSI measurement in non-DRX	1214

5.6.5	Measurements with autonomous gaps	1218

5.6.5.0	Minimum conformance requirements	1218

5.6.5.0.1	Minimum conformance requirements for EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1218

5.6.5.0.1.1	Introduction	1218

5.6.5.0.1.2	CGI identification of an NR cell with autonomous gaps	1218

5.6.5.0.1.3	CGI reporting delay	1219

5.6.5.1	EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1219

5.6.6	L1-SINR measurement for beam reporting	1224

5.6.6.0	Minimum conformance requirements	1224

5.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1224

5.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1227

5.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1229

5.6.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1232

5.6.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1236

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1242

5.6.7	CSI-RS based Intra-frequency Measurements	1246

5.6.7.0	Minimum conformance requirements	1246

5.6.7.0.1	Minimum conformance requirements for event-triggered measurement without gap	1246

5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1249

5.6.8	CSI-RS based Inter-frequency Measurements	1255

5.6.8.0	Minimum conformance requirements for Inter-frequency measurements	1255

5.6.8.1	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1257

5.7	Measurement performance requirements	1263

5.7.1	SS-RSRP	1263

5.7.1.0	Minimum conformance requirements	1263

5.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	1263

5.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	1264

5.7.1.1	EN-DC FR2 SS-RSRP measurement accuracy	1266

5.7.1.2	EN-DC FR2-FR2 SS-RSRP measurement accuracy	1274

5.7.1.3	EN-DC FR1-FR2 SS-RSRP measurement accuracy	1281

5.7.2	SS-RSRQ	1286

5.7.2.0	Minimum conformance requirements	1286

5.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1286

5.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	1287

5.7.2.1	EN-DC FR2 SS-RSRQ measurement accuracy	1288

5.7.2.2	EN-DC FR2-FR2 SS-RSRQ measurement accuracy	1292

5.7.3	SS-SINR	1296

5.7.3.0	Minimum conformance requirements	1296

5.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1296

5.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	1297

5.7.3.1	EN-DC FR2 SS-SINR measurement accuracy	1298

5.7.3.2	EN-DC FR2-FR2 SS-SINR measurement accuracy	1302

5.7.4	L1-RSRP	1308

5.7.4.0	Minimum conformance requirements	1308

5.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	1308

5.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	1309

5.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	1309

5.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	1310

5.7.4.1	EN-DC FR2 SSB based L1-RSRP measurement accuracy	1311

5.7.4.2	EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy	1317

5.7.5	SRS-RSRP	1323

5.7.5.0	Minimum conformance requirements	1323

5.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	1323

5.7.5.1	EN-DC FR2 SRS-RSRP measurement accuracy	1325

5.7.5.2	EN-DC FR2 CLI-RSSI measurement accuracy	1330

5.7.5.2.1	Test purpose	1331

5.7.5.2.2	Test applicability	1331

5.7.5.2.3	Minimum conformance requirements	1331

5.7.5.2.4	Test description	1331

5.7.5.2.5	Test requirement	1332

5.7.6	L1-SINR measurement for beam reporting	1335

5.7.6.0	Minimum conformance requirements	1335

5.7.6.0.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1335

5.7.6.0.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1336

5.7.6.0.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1339

5.7.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	1341

5.7.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1346

5.7.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1351

5.7.7	CSI-RSRP	1356

5.7.7.0	Minimum conformance requirements	1356

5.7.7.0.1	Intra-frequency CSI-RSRP measurement accuracy requirements	1356

5.7.7.0.2	Inter-frequency CSI-RSRP measurement accuracy requirements	1358

5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1360

5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1367

5.7.8	CSI-RSRQ	1375

5.7.8.0	Minimum conformance requirements	1375

5.7.8.0.1	Intra-frequency CSI-RSRQ accuracy requirements	1375

5.7.8.0.2	Inter-frequency CSI-RSRQ accuracy requirements	1377

5.7.8.1	EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1379

5.7.8.1.1	Test purpose	1379

5.7.8.1.2	Test applicability	1379

5.7.8.1.3	Minimum conformance requirements	1379

5.7.8.1.4	Test description	1380

5.7.8.1.5	Test Requirements	1381

5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1383

5.7.8.2.1	Test purpose	1384

5.7.8.2.2	Test applicability	1384

5.7.8.2.3	Minimum conformance requirements	1384

5.7.8.2.4	Test description	1384

5.7.8.2.5	Test Requirements	1385

5.7.9	CSI-SINR	1388

5.7.9.0	Minimum conformance requirements	1388

5.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements	1388

5.7.9.0.1.1	Absolute CSI-SINR accuracy	1388

5.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements	1389

5.7.9.0.2.1	Absolute CSI-SINR accuracy	1389

5.7.9.0.2.2	Relative CSI-SINR accuracy	1390

5.7.9.1	EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1391

5.7.9.1.1	Test purpose	1391

5.7.9.1.2	Test applicability	1391

5.7.9.1.3	Minimum conformance requirements	1391

5.7.9.1.4	Test description	1392

5.7.9.1.5	Test Requirements	1393

5.7.9.2	EN-DC inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1395

5.7.9.2.1	Test purpose	1396

5.7.9.2.2	  Test applicability	1396

5.7.9.2.3	Minimum conformance requirements	1396

5.7.9.2.4	Test description	1396

5.7.9.2.5	Test Requirements	1397

6	NR standalone in FR1	1401

6.0	General	1401

6.1	RRC_IDLE state mobility	1401

6.1.1	NR cell re-selection	1401

6.1.1.0	Minimum conformance requirements	1401

6.1.1.1	NR SA FR1 cell re-selection	1408

6.1.1.2	NR SA FR1-FR1 cell re-selection	1414

6.1.1.3	NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1418

6.1.1.4	NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1425

6.1.1.5	NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1430

6.1.1.6	NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1437

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16	1443

6.1.1.8	NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17	1448

6.1.1.9	NR SA FR1 cell re-selection for UE operating on a cell with less than 5MHz BW	1454

6.1.2	NR – E-UTRA cell re-selection	1456

6.1.2.0	Minimum conformance requirements	1456

6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA	1460

6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA	1465

6.1.2.3	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1472

6.1.2.4	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1478

6.1.2.5	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16	1483

6.2	RRC_INACTIVE state mobility	1489

6.2.0	General	1489

6.2.0.1	Minimum conformance requirements for TA validation using CG-SDT	1489

6.2.1	NR SA FR1 TA Validation for CG-SDT	1490

6.3	RRC_CONNECTED state mobility	1496

6.3.1	Handover	1496

6.3.1.0	Minimum conformance requirements	1496

6.3.1.1	NR SA FR1 handover with known target cell	1500

6.3.1.2	NR SA FR1 handover with unknown target cell	1504

6.3.1.3	NR SA FR1-FR1 handover with unknown target cell	1507

6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell	1510

6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell	1518

6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell	1523

6.3.1.7	NR SA FR1 synchronous DAPS handover	1529

6.3.1.8	NR SA FR1 asynchronous DAPS handover	1537

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover	1542

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover	1551

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover	1557

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover	1565

6.3.1.13 to 6.3.1.17	1573

6.3.1.18	NR SA FR1 handover with unknown target cell operating with 12 PRB SSB bandwidth	1573

6.3.2	RRC connection mobility control	1575

6.3.2.1	RRC re-establishment	1575

6.3.2.1.0	Minimum conformance requirements	1575

6.3.2.1.1	NR SA FR1 RRC re-establishment	1577

6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment	1581

6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing	1585

6.3.2.2	Random access	1589

6.3.2.2.0	Minimum conformance requirements	1589

6.3.2.2.1	NR SA FR1 contention based random access	1591

6.3.2.2.2	NR SA FR1 non-contention based random access	1597

6.3.2.2.3	NR SA FR1 2-step contention based random access	1603

6.3.2.2.4	NR SA FR1 2-step non-contention based random access	1607

6.3.2.3	RRC connection release with redirection	1612

6.3.2.3.0	Minimum conformance requirements	1612

6.3.2.3.1	NR SA FR1 RRC connection release with redirection	1613

6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection	1617

6.3.2.4	LTM PDCCH-order Random Access	1621

6.3.2.4.0	Minimum conformance requirements	1621

6.3.2.4.1	NR SA FR1 PDCCH-order RACH on neighbour cell when RACH BW is within active UL BWP	1622

6.3.2.4.2	NR SA FR1 PDCCH-ordered RACH to an inter-frequency candidate cell for LTM	1629

6.3.2.4.3	NR SA FR1 PDCCH-order RACH on neighbour cell without L1-RSRP measurement when RACH BW is within active UL BWP	1636

6.3.3	Conditional handover	1642

6.3.3.0	Minimum conformance requirements	1642

6.3.3.1	NR SA FR1 conditional handover	1652

6.3.3.2	NR SA FR1-FR1 conditional handover	1659

6.3.3.3	NR SA FR1 conditional handover including target MCG and target SCG	1666

6.3.3.4	NR SA FR1 conditional handover including target MCG and candidate SCG	1674

6.3.3.5	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC with complementary conditional handover configuration	1687

6.3.3.6	NR SA FR1 Intra-frequency NES triggering conditional handover	1693

6.3.3.7	NR SA FR1-FR1 Inter-frequency NES-based conditional handover	1701

6.3.4	LTM PCell Switch	1710

6.3.4.0	Minimum conformance requirements	1710

6.3.4.1	NR SA FR1 RACH-based Intra-frequency PCell switch from FR1 to FR1	1712

6.3.4.2	NR SA FR1 RACH-based Inter-frequency LTM PCell switch from FR1 to FR1	1719

6.3.4.3	NR SA FR1 RACH-less Intra-frequency PCell switch from FR1 to FR1	1728

6.3.4.4	NR SA FR1 RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement	1737

6.3.5	LTM PSCell Switch	1746

6.3.5.0	Minimum conformance requirements	1746

6.3.5.1	NR SA FR1 RACH-based intra-frequency LTM PSCell switch	1746

6.4	Timing	1755

6.4.1	UE transmit timing	1755

6.4.1.0	Minimum conformance requirements	1755

6.4.1.1	NR SA FR1 UE transmit timing accuracy	1756

6.4.2	UE timer accuracy	1762

6.4.3	Timing advance	1762

6.4.3.0	Minimum conformance requirement	1762

6.4.3.1	NR SA FR1 timing advance adjustment accuracy	1762

6.5	Signaling characteristics	1767

6.5.1	Radio link monitoring	1767

6.5.1.00	General	1767

6.5.1.0	Minimum conformance requirements	1769

6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1774

6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1780

6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	1786

6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	1791

6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1798

6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1805

6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1811

6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1818

6.5.1.9	SA FR1 radio link monitoring out-of-sync Test for PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	1824

6.5.1.10 to 6.5.1.12	1832

6.5.1.13	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW	1832

6.5.1.14	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW	1835

6.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode	1838

6.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode	1840

6.5.2	Interruption	1843

6.5.2.0	Minimum conformance requirements	1843

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC	1846

6.5.2.2	SA interruptions at NR SRS carrier based switching	1851

6.5.3	SCell activation and deactivation delay	1856

6.5.3.0	Minimum conformance requirements	1856

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	1858

6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	1871

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX	1872

6.5.3.4	NR SA FR1 direct SCell activation at SCell addition of known SCell	1874

6.5.3.5	NR SA FR1 direct SCell activation at handover with known SCell	1883

6.5.3.6 to 6.5.3.9	1892

6.5.3.10	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	1892

6.5.3.11	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	1899

6.5.3.12 to 6.5.3.14	1903

6.5.3.15	NR SA FR1 TRS based SCell Activation of SSB-less SCell in inter-band CA in non-DRX	1903

6.5.3.16	NR SA FR1 Inter-band SSB-less SCell Activation based on A-TRS in non-DRX	1910

6.5.3.17	NR SA FR1 SCell activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle with less than 5MHz BW	1918

6.5.4	UE UL carrier RRC reconfiguration delay	1920

6.5.4.0	Minimum conformance requirements	1920

6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay	1920

6.5.5	Link recovery procedures	1927

6.5.5.0	Minimum conformance requirements	1927

6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX	1930

6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX	1939

6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	1947

6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX	1954

6.5.5.5	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	1962

6.5.5.6	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	1970

6.5.5.7	NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	1977

6.5.5.8	NR SA FR1 SSB-based Beam Failure Detection and Link Recovery in non-DRX mode for a UE operating on a cell with 3 MHz BW	1985

6.5.6	Active BWP switch delay	1987

6.5.6.1	DCI-based and time-based active BWP switch	1987

6.5.6.1.0	Minimum conformance requirements	1987

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX	1989

6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX	1999

6.5.6.2	RRC-based active BWP switch	2008

6.5.6.2.0	Minimum conformance requirements	2008

6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX	2008

6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2016

6.5.6.3.0	Minimum conformance requirements	2016

6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2016

6.5.6.4	2026

6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2026

6.5.6.5.0	Minimum conformance requirements	2026

6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2027

6.5.7	DL interruptions at switching between two uplink carriers	2034

6.5.7.0	Minimum conformance requirements	2034

6.5.7.1	NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA	2034

6.5.7.2	NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA	2040

6.5.7A	NR SA FR1 DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2045

6.5.7A.0	Minimum conformance requirements	2045

6.5.7A.1	NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA	2046

6.5.7A.2	NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA	2052

6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2057

6.5.7B.0	Minimum conformance requirements	2057

6.5.7B.1	NR SA FR1 DL Interruptions at switching between two uplink bands in FDD-TDD CA	2058

6.5.7B.2	NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA	2064

6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2071

6.5.7C.0	Minimum conformance requirements	2071

6.5.7C.1	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2071

6.5.7C.2	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2077

6.5.7D	 DL interruptions at UE switching across three or four uplink bands	2084

6.5.7D.0	Minimum conformance requirements	2084

6.5.7D.1	NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG	2084

6.5.7D.2	NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG	2090

6.5.7D.3	NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs	2097

6.5.7D.4	NR SA FR1 DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs	2102

6.5.8	UE specific CBW change	2108

6.5.8.0	Minimum conformance requirements	2108

6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2109

6.5.9 to 6.5.11	2115

6.5.12	Subsequent conditional PSCell addition/change	2115

6.5.12.0	Minimum conformance requirements	2115

6.5.12.0.1	Minimum conformance requirements for Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2115

6.5.12.0.1.1	Measurement time	2116

6.5.12.0.2	Minimum conformance requirements for Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2116

6.5.12.1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2117

6.5.12.2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2125

6.5.13	Active TCI state switch delay	2133

6.5.13.0	Minimum conformance requirements	2133

6.5.13.1	NR SA FR1 MAC-CE based joint TCI state switch for mDCI with two TA when RTD is larger than CP	2134

6.6	Measurement procedures	2144

6.6.1	Intra-frequency measurements	2144

6.6.1.0	Minimum conformance requirements	2144

6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX	2149

6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX	2154

6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX	2159

6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX	2165

6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading	2170

6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading	2174

6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	2178

6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17	2182

6.6.1.9	NR SA FR1 event triggered reporting with MUSIM gap configured	2187

6.6.1.10 to 6.6.1.12	2192

6.6.1.12	NR SA FR1 event triggered reporting with SSB index detection without measurement gap in non-DRX mode for UE operating with 12 PRB SSB bandwidth	2192

6.6.1.13	NR SA FR1 event-triggered reporting tests without gap under Cell DTX	2195

6.6.1.14	NR SA FR1 Deactivated PSCell measurement test with 12 PRB SSB bandwidth	2200

6.6.1.15	NR SA FR1 Event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell	2205

6.6.2	Inter-frequency measurements	2209

6.6.2.0	Minimum conformance requirements for Inter-frequency measurements	2209

6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX	2212

6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX	2218

6.6.2.3	Void	2224

6.6.2.4	Void	2224

6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	2224

6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection	2230

6.6.2.7	Void	2236

6.6.2.8	Void	2236

6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2236

6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2241

6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	2247

6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	2253

6.6.2.13	NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority	2259

6.6.2.14	NR SA FR1-FR1 event triggered reporting using MUSIM gap with shorter MGRP	2266

6.6.2.15	NR SA FR1 event triggered reporting without SSB index detection in DRX mode for UE operating on a cell with less than 5MHz BW	2274

6.6.3	Inter-RAT Measurements	2277

6.6.3.0	Minimum conformance requirements	2277

6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX	2281

6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX	2288

6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16	2294

6.6.4	L1-RSRP measurement for beam reporting	2302

6.6.4.0	Minimum conformance requirements	2302

6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX	2303

6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX	2307

6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX	2310

6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX	2315

6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	2318

6.6.4.6	NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX	2323

6.6.5	Inter-RAT UTRAN measurements	2328

6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX	2328

6.6.6	CLI Measurements	2334

6.6.6.0	Minimum conformance requirements	2334

6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX	2334

6.6.7	NR measurements with autonomous gaps	2340

6.6.7.0	Minimum conformance requirements	2340

6.6.7.1	SA intra-frequency CGI identification of NR neighbour cell in FR1	2342

6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2348

6.6.8	L1-SINR measurement for beam reporting	2352

6.6.8.0	Minimum conformance requirements	2352

6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	2352

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	2358

6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	2364

6.6.9	Idle Mode CA/DC Measurements	2370

6.6.9.0	Minimum conformance requirements	2370

6.6.9.0.1	Idle Mode CA/DC Measurements	2370

6.6.9.0.2	Measurement report for fast CA/DC setup	2371

6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1	2372

6.6.9.2	NR SA FR1 Idle mode fast CA/DC eEMR measurement	2383

6.6.9.3	NR SA FR1 Idle mode fast CA/DC cell reselection measurement without valid reporting	2386

6.6.9.4	NR SA FR1 Idle mode fast CA/DC cell reselection measurement with valid reporting	2390

6.6.9.5	NR SA FR1 Idle mode CA/DC measurement of SCell with less than 5MHz BW	2393

6.6.10 to 6.6.14	2396

6.6.15	Idle Mode inter-RAT CA/DC Measurements	2396

6.6.15.0	Minimum conformance requirements	2396

6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting	2397

6.6.16	2408

6.6.17	NR SA FR1 event triggered reporting tests with Pre-MG	2408

6.6.17.0	Minimum conformance requirements	2408

6.6.17.1	NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG	2411

6.6.17.2	NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2417

6.6.18	SA event triggered reporting tests with concurrent gaps	2425

6.6.18.0	Minimum conformance requirements	2425

6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers	2428

6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers	2436

6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2443

6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2455

6.6.19	SA event triggered reporting tests with NCSG	2464

6.6.19.0	Minimum conformance requirements	2464

6.6.19.1	NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1	2469

6.6.19.2	NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2475

6.6.19.3	NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2481

6.6.19.4	NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2489

6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2495

6.6.20.0	Minimum conformance requirements	2495

6.6.20.1	NR SA FR1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC	2498

6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS	2502

6.6.21.0	Minimum conformance requirements	2502

6.6.21.1	NR SA FR1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC	2505

6.6.22	SA event triggered reporting tests with concurrent gaps including Pre-MG	2509

6.6.22.0	Minimum conformance requirements	2509

6.6.22.1	NR SA FR1 and FR1-FR1 event-triggered reporting with concurrent Type-2 measurement gap and Pre-MG	2511

6.6.22.2	NR SA FR1 event-triggered reporting with two concurrent Pre-MGs	2519

6.6.23	 SA event triggered reporting tests with concurrent gaps including NCSG	2527

6.6.23.0	Minimum conformance requirements	2527

6.6.23.1	NR SA FR1-FR1 event-triggered reporting with concurrent Type-2 measurement gap and NCSG	2528

6.6.23.2	NR SA FR1-FR1 event-triggered reporting with two concurrent NCSGs	2534

6.6.23.3	NR SA FR1 and FR1-FR1 event triggered reporting with concurrent Type-2 measurement gap and NCSG	2541

6.6.24	SA event triggered reporting tests with NeedForGap	2548

6.6.24.0	Minimum conformance requirements	2548

6.6.24.1	NR SA FR1 event-triggered reporting without gap with interruptions	2554

6.6.24.2	NR SA FR1-FR1 event-triggered reporting without gap with interruption with SSB index	2561

6.6.24.3	NR SA FR1-FR1 event-triggered reporting without gap with interruption	2567

6.6.24.4	NR SA FR1-FR1 event-triggered reporting without gap without interruption	2574

6.6.24.5	NR SA FR1 event-triggered reporting without gap without interruption	2579

6.6.25	SA NR - E-UTRAN event-triggered without measurement gaps	2586

6.6.25.0	Minimum conformance requirements	2586

6.6.25.1	NR SA FR1 - E-UTRAN event-triggered reporting without gap without interruption	2588

6.6.25.2	NR SA FR1 - E-UTRAN event-triggered reporting without interruption outside gap	2596

6.6.25.3	NR SA FR1 - E-UTRAN event-triggered reporting without gap when CRS contained within active DL BWP	2604

6.6.26	LTM Intra-frequency L1-RSRP measurement	2612

6.6.26.0	Minimum conformance requirements	2612

6.6.26.1	NR SA FR1 Intra-frequency SSB based L1-RSRP measurement	2613

6.6.27	LTM Inter-frequency L1-RSRP measurement with measurement gap	2620

6.6.27.0	Minimum conformance requirements	2620

6.6.27.1	NR SA FR1 Inter-frequency SSB based L1-RSRP measurement with measurement gap	2621

6.6.28	LTM Inter-frequency L1-RSRP measurement without measurement gap	2627

6.6.28.0	Minimum conformance requirements	2627

6.6.28.1	NR SA FR1 Inter-frequency SSB based L1-RSRP measurement without measurement gap	2629

6.7	Measurement performance requirements	2636

6.7.1	SS-RSRP	2636

6.7.1.0	Minimum conformance requirements	2636

6.7.1.1	Intra-frequency measurements	2636

6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy	2636

6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy	2643

6.7.1.2	Inter-frequency measurements	2645

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy	2645

6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy	2652

6.7.2	SS-RSRQ	2654

6.7.2.0	Minimum conformance requirements	2654

6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy	2654

6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	2658

6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy	2658

6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy	2662

6.7.3	SS-SINR	2664

6.7.3.0	Minimum conformance requirements	2664

6.7.3.1	NR SA FR1 SS-SINR measurement accuracy	2664

6.7.3.2	Inter-Frequency SS-SINR measurement accuracy	2668

6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy	2668

6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy	2672

6.7.4	L1-RSRP	2673

6.7.4.0	Minimum conformance requirements	2673

6.7.4.1	SSB based L1-RSRP measurements	2674

6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy	2674

6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy	2679

6.7.4.2	CSI-RS based L1-RSRP measurements	2681

6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy	2681

6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy	2685

6.7.5	E-UTRAN RSRP	2687

6.7.5.0	Minimum conformance requirements	2687

6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy	2688

6.7.6	E-UTRAN RSRQ	2695

6.7.6.0	Minimum conformance requirements	2695

6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy	2696

6.7.7	E-UTRAN RS-SINR	2703

6.7.7.0	Minimum conformance requirements	2703

6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy	2704

6.7.8	CLI Measurements	2711

6.7.8.0	Minimum conformance requirements	2711

6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy	2711

6.7.9	L1-SINR measurement for beam reporting	2721

6.7.9.0	Minimum conformance requirements	2721

6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	2730

6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	2730

6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	2733

6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2735

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2738

6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2739

6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	2742

6.7.10	CSI-RSRP	2744

6.7.10.0	Minimum conformance requirements	2744

6.7.10.1	Intra-frequency measurements	2744

6.7.10.1.1	NR SA FR1 CSI-RSRP absolute measurement accuracy	2744

6.7.10.1.2	NR SA FR1 CSI-RSRP relative measurement accuracy	2749

6.7.10.2	Inter-frequency measurements	2755

6.7.10.2.1	NR SA FR1-FR1 CSI-RSRP absolute measurement accuracy	2755

6.7.10.2.2	NR SA FR1-FR1 CSI-RSRP relative  measurement accuracy	2762

6.7.11	CSI-RSRQ	2768

6.7.11.0	Minimum conformance requirements	2768

6.7.11.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2772

6.7.11.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2779

6.7.12	CSI-SINR	2786

6.7.12.0	Minimum conformance requirements	2786

6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2790

6.7.12.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2796

6.7.13 to 6.7.16	2804

6.7.17	LTM L1-RSRP measurement	2804

6.7.17.0	Minimum conformance requirements	2804

6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1	2805

6.7.17.1.1	NR SA FR1 Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell	2805

6.7.17.1.2	Void	2816

6.7.18	TDCP amplitude measurement accuracy	2816

6.7.18.0	Minimum conformance requirements	2816

6.7.18.1	TDCP amplitude measurement accuracy in EN-DC	2816

7	NR standalone with at least one NR cell in FR2	2823

7.0	General	2823

7.1	RRC_IDLE state mobility	2823

7.1.1	NR cell re-selection	2823

7.1.1.0	Minimum conformance requirements	2823

7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	2823

7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	2826

7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion	2829

7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion	2830

7.1.1.1	NR SA FR2 cell re-selection	2831

7.1.1.2	NR SA FR2-FR2 cell re-selection	2839

7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2845

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2850

7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2855

7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2862

7.2	RRC_INACTIVE state mobility	2880

7.2.1	Small Data Transmission	2880

7.2.1.0	Minimum conformance requirements	2880

7.2.1.0.1	Minimum conformance requirements for CG-SDT TA validation	2880

7.2.1.1	TA Validation for CG-SDT in FR2	2880

7.3	RRC_CONNECTED state mobility	2889

7.3.1	Handover	2889

7.3.1.0	Minimum conformance requirements	2889

7.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	2889

7.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR2 handover	2890

7.3.1.0.3	Minimum conformance requirements for NR FR1 – NR FR2 DAPS handover	2891

7.3.1.1	NR SA FR1-FR2 Inter-frequency handover; unknown target cell	2892

7.3.1.2	NR SA FR2 Intra-frequency handover; unknown target cell	2897

7.3.1.3	NR SA FR2-FR2 Inter-frequency handover; unknown target cell	2901

7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover	2905

7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover	2916

7.3.2	RRC connection mobility control	2923

7.3.2.1	RRC re-establishment	2923

7.3.2.1.0	Minimum conformance requirements	2923

7.3.2.1.1	NR SA FR2 RRC re-establishment	2924

7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment	2928

7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	2931

7.3.2.2	Random access	2936

7.3.2.2.0	Minimum conformance requirements	2936

7.3.2.2.1	NR SA FR2 contention based random access	2937

7.3.2.2.2	NR SA FR2 non-contention based random access	2945

7.3.2.2.3	NR SA FR2 2-step contention based random access	2953

7.3.2.2.4	NR SA FR2 2-step non-contention based random access	2960

7.3.2.3	RRC connection release with redirection	2967

7.3.2.3.0	Minimum conformance requirements	2967

7.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection	2968

7.3.2.4	LTM PDCCH-order Random Access	2972

7.3.2.4.0	Minimum conformance requirements	2972

7.3.2.4.1	NR SA FR2 PDCCH-order RACH on neighbour cell when RACH BW is within active BWP	2972

7.3.2.4.2	NR SA FR2 PDCCH-order RACH on inter-frequency neighbour cell	2976

7.3.3	Conditional handover	2979

7.3.3.0	Minimum conformance requirements	2979

7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover	2979

7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover	2981

7.3.3.0.3	Minimum conformance requirements for NR FR2 – NR FR1 inter-frequency conditional handover	2981

7.3.3.0.4	Minimum conformance requirements for NR-DC conditional handover including target MCG and target SCG	2981

7.3.3.0.5	Minimum conformance requirements for NR-DC conditional handover including target MCG and candidate SCG	2983

7.3.3.1	NR SA FR2 conditional handover	2984

7.3.3.2	NR SA FR2-FR2 conditional handover	2991

7.3.3.3	NR SA FR2 intra-frequency NES triggering conditional handover	2998

7.3.3.4	NR SA FR2-FR1 inter-frequency NES triggering conditional handover	3002

7.3.3.5	NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3009

7.3.3.6	NR SA FR2 conditional handover including target MCG and candidate SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3017

7.3.3.6.1	Test purpose	3017

7.3.3.6.2	Test applicability	3017

7.3.3.6.3	Minimum conformance requirements	3017

7.3.3.6.4	Test description	3017

7.3.3.6.5	Test Requirement	3024

7.3.4	LTM PCell Switch	3027

7.3.4.0	Minimum conformance requirements	3027

7.3.4.0.1	Minimum conformance requirements for NR FR2 LTM PCell Switch	3027

7.3.4.1	NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2	3027

7.3.4.2	NR SA FR2 RACH-less Intra-frequency PCell switch from FR2 to FR2	3034

7.3.4.3	NR SA FR2 RACH-based Inter-frequency LTM PCell switch from FR2 to FR2	3042

7.3.5	LTM PSCell Switch	3050

7.3.5.0	Minimum conformance requirements	3050

7.3.5.1	NR SA FR2 RACH-based Intra-frequency LTM PSCell switch	3050

7.4	Timing	3057

7.4.1	UE transmit timing	3057

7.4.1.0	Minimum Conformance Requirements	3057

7.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	3057

7.4.1.1	NR SA FR2 UE transmit timing accuracy	3059

7.4.1.2	3065

7.4.1.3	NR SA FR2 UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	3065

7.4.2	UE timer accuracy	3075

7.4.3	Timing advance	3075

7.4.3.0	Minimum conformance requirements	3075

7.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	3075

7.4.3.1	NR SA FR2 timing advance adjustment accuracy	3075

7.5	Signalling characteristics	3081

7.5.1	Radio link monitoring	3081

7.5.1.0	Minimum conformance requirements	3081

7.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	3081

7.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	3081

7.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	3081

7.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	3081

7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	3081

7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3081

7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3087

7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3094

7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3098

7.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	3105

7.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3111

7.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM RS in DRX mode	3118

7.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3123

7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	3129

7.5.1.10	NR SA FR2 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	3134

7.5.2	Interruption	3139

7.5.2.0	Minimum conformance requirements	3139

7.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	3139

7.5.2.0.2	Minimum conformance requirements for SA interruptions at NR SRS carrier-based switching	3140

7.5.2.1	NR SA FR2 interruptions during measurements on deactivated NR SCC	3142

7.5.2.2	SA interruptions at NR SRS carrier-based switching	3148

7.5.3	SCell activation and deactivation delay	3151

7.5.3.0	Minimum conformance requirements	3151

7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell	3151

7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell	3154

7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay	3154

7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay	3156

7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3159

7.5.3.4	NR SA FR2 direct SCell activation at SCell addition of known SCell	3167

7.5.3.5	NR SA FR2 direct SCell activation at handover with known SCell	3174

7.5.3.6 to 7.5.3.12	3182

7.5.3.13	NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX	3182

7.5.3.14	NR SA FR2 SCell Activation for known SCell in FR2 inter-band	3189

7.5.4	Void	3196

7.5.5	Link recovery procedures	3196

7.5.5.0	Minimum conformance requirements	3196

7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	3196

7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	3196

7.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	3197

7.5.5.0.4	Requirements for Beam Failure Recovery in SCell	3197

7.5.5.0.5	TRP specific Minimum conformance requirements	3197

7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX	3199

7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX	3204

7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	3211

7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX	3216

7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX	3223

7.5.5.6	NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX	3228

7.5.5.7	NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX	3235

7.5.5.8	FFS	3241

7.5.5.9	NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	3241

7.5.5.10	NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX	3247

7.5.5.11 to 7.5.5.13	3254

7.5.5.14	NR SA FR2 PCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in multi-Rx operation and DRX mode	3254

7.5.6	Active BWP switch delay	3260

7.5.6.1	DCI-based and time-based active BWP switch	3260

7.5.6.1.0	Minimum conformance requirements	3260

7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX	3260

7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX	3261

7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX	3262

7.5.6.2	RRC-based active BWP switch	3263

7.5.6.2.0	Minimum conformance requirements	3263

7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX	3264

7.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	3266

7.5.6.3.0	Minimum conformance requirements	3266

7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	3269

7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3273

7.5.6.5.0	Minimum conformance requirements	3273

7.5.6.5.0.1	Minimum conformance requirements for active BWP switch on multiple SCells with non-DRX in SA	3273

7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3274

7.5.7	PSCell addition and release delay	3280

7.5.7.0	Minimum conformance requirements	3280

7.5.7.0.1	Minimum conformance requirements for PSCell addition delay	3280

7.5.7.0.2	Minimum conformance requirements for PSCell release delay	3281

7.5.7.1	NR SA FR2 addition and release delay of known PSCell	3281

7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell	3283

7.5.8	Active TCI state switch delay	3285

7.5.8.0	Minimum conformance requirements	3285

7.5.8.0.0	Known conditions for TCI state	3285

7.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	3286

7.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	3286

7.5.8.0.3	Minimum conformance requirements for MAC-CE based active TCI state switch in HST FR2	3287

7.5.8.0.5	DCI based dual DL TCI state switching delay for mDCI for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	3288

7.5.8.0.6	DCI based dual DL TCI state switching delay for sDCI for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	3289

7.5.8.1	NR SA FR2 MAC-CE based active TCI state switch	3289

7.5.8.1.1	NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state	3289

7.5.8.2	NR SA FR2 RRC based active TCI state switch	3296

7.5.8.2.1	NR SA PCell FR2 RRC based active TCI state switch for a known TCI state	3296

7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3303

7.5.8.3.1	NR SA FR2 HST active TCI state switch for a known TCI state	3303

7.5.8.3.2	NR SA FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 a known TCI state	3309

7.5.8.4	NR SA FR2 DCI based active TCI state switch with m-DCI for simultaneous reception	3314

7.5.8.5	NR SA FR2 Single-DCI based active TCI state switch with known target TCI states for simultaneous reception	3322

7.5.9	Uplink spatial relation switch delay	3328

7.5.9.0	Minimum conformance requirements	3328

7.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	3328

7.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	3329

7.5.9.1	MAC-CE based uplink spatial relation switch	3329

7.5.9.1.1	NR PCell FR2 spatial relation associated with known DL-RS	3329

7.5.9.2	RRC based uplink spatial relation switch	3333

7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3333

7.5.10	UE specific CBW change	3337

7.5.10.0	Minimum conformance requirements	3337

7.5.10.0.1	Minimum conformance requirements for NR FR2 UE specific CBW change of PCell with non-DRX in SA	3337

7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3338

7.5.11	UE UL carrier RRC reconfiguration delay	3341

7.5.11.1	UE UL carrier RRC reconfiguration delay	3341

7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3347

7.5.12.1	NR SA FR2 Addition and Release Delay of PSCell	3347

7.5.13	Unified TCI state switch delay	3352

7.5.13.0	Minimum conformance requirements	3352

7.5.13.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	3352

7.5.13.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	3353

7.5.13.0.3	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI for 2 known TCI states	3353

7.5.13.0.4	Minimum conformance requirements for active downlink TCI state switching delay for unified TCI for single-DCI mTRP	3354

7.5.13.1	NR SA FR2 MAC-CE based active joint TCI state switch	3356

7.5.13.2	NR SA FR2 MAC-CE based active uplink TCI state switch	3362

7.5.13.3	NR SA FR2 MAC-CE based active downlink TCI state switch	3368

7.5.13.4	sDCI MAC-CE based joint TCI state switching	3374

7.5.13.4.1	NR SA FR2 PCell dual downlink and uplink TCI state switch in sDCI for known case	3374

7.5.13.5	MAC-CE based dual downlink TCI state switching delay for unified TCI for single-DCI mTRP	3387

7.5.13.5.1	NR SA FR2 PCell dual downlink TCI state switch in sDCI for known case	3387

7.5.13.6	MAC-CE based active uplink TCI state switch for single-DCI mTRP	3391

7.5.13.6.1	NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states	3391

7.5.14	PSCell activation and deactivation delay	3399

7.5.14.0	Minimum conformance requirements	3399

7.5.14.0.1	SCG Activation Delay Requirement	3399

7.5.14.0.2	SCG Deactivation Delay Requirement	3400

7.5.14.1	NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3400

7.5.15	3406

7.5.16	NR SA FR2 UE L1-RSRP measurement reporting with Scheduling and Measurement Restrictions in multi-Rx operation	3406

7.5.17	3412

7.5.18	Subsequent conditional PSCell addition/change	3412

7.5.18.0	Minimum conformance requirements	3412

7.5.18.0.1	Minimum conformance requirements for Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3412

7.5.18.0.2	Minimum conformance requirements for Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3413

7.5.18.1	Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3413

7.5.18.2	Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3419

7.6	Measurement procedures	3423

7.6.1	Intra-frequency measurements	3423

7.6.1.0	Minimum conformance requirements	3423

7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	3423

7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	3425

7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX	3426

7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX	3432

7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX	3438

7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	3444

7.6.1.5	NR SA FR2 event triggered reporting test without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3450

7.6.2	Inter-frequency measurements	3454

7.6.2.0	Minimum conformance requirements for Inter-frequency measurements	3454

7.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps	3454

7.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps	3455

7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX	3457

7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX	3462

7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	3468

7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection	3473

7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX	3479

7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX	3485

7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	3491

7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection	3497

7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3503

7.6.2.10	SA event triggered reporting test without gap under non-DRX	3508

7.6.2.10.5	Test requirement	3510

7.6.2.11	SA event triggered reporting test without gap under DRX	3512

7.6.2.12 to 7.6.2.19	3516

7.6.2.20	NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority	3516

7.6.2.21	NR SA FR2-FR2 event triggered reporting using MUSIM gap with shorter MGRP	3523

7.6.3	L1-RSRP measurement for beam reporting	3531

7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	3531

7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	3531

7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	3531

7.6.3.0.3	Minimum conformance requirements for Inter-cell SSB based L1-RSRP Reporting	3531

7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX	3533

7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX	3539

7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX	3543

7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX	3547

7.6.3.5	NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3550

7.6.3.6	NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3556

7.6.4	CLI measurements	3563

7.6.4.0	Minimum conformance requirements	3563

7.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	3563

7.6.4.1	NR SA FR2 SRS-RSRP measurement in non-DRX	3564

7.6.4.2	NR SA FR2 CLI-RSSI measurement in non-DRX	3568

7.6.5	NR measurements with autonomous gaps	3571

7.6.5.0	Minimum conformance requirements	3571

7.6.5.0.1	Minimum conformance requirements for SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	3571

7.6.5.0.1.1	Minimum conformance requirements for interruptions when identifying CGI of an NR cell with autonomous gaps	3571

7.6.5.0.1.2	Minimum conformance requirements for NR measurements with autonomous gaps	3571

7.6.5.0.1.2.1	Introduction	3571

7.6.5.0.1.2.2	CGI identification of an NR cell with autonomous gaps	3572

7.6.5.0.1.2.3	CGI reporting delay	3573

7.6.5.1	SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	3573

7.6.6	L1-SINR measurement for beam reporting	3577

7.6.6.0	Minimum conformance requirements	3577

7.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	3577

7.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	3577

7.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	3577

7.6.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	3577

7.6.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	3581

7.6.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	3586

7.6.7 to 7.6.12	3592

7.6.13	UE Rx-Tx time difference measurements for PDC	3592

7.6.13.0	Minimum conformance requirements	3592

7.6.13.0.1	PRS based UE Rx-Tx time difference for propagation delay compensation	3592

7.6.13.0.2	TRS based UE Rx-Tx time difference for propagation delay compensation	3594

7.6.13.1	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS	3596

7.6.13.2	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS	3600

7.6.14	SA event triggered reporting tests with Pre-MG	3604

7.6.14.0	Minimum conformance requirements	3604

7.6.14.1	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation	3607

7.6.14.2	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	3613

7.6.15	SA event triggered reporting tests with concurrent gaps	3619

7.6.15.0	Minimum conformance requirements	3619

7.6.15.0.1	Minimum conformance requirements for Intra-frequency measurement	3619

7.6.15.0.2	Minimum conformance requirements for Inter-frequency measurement	3619

7.6.15.0.3	Minimum conformance requirements for PRS measurement	3620

7.6.15.1	NR SA FR2 event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3620

7.6.15.2	NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3626

7.6.15.3	NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3635

7.6.16	SA event triggered reporting tests with NCSG	3642

7.6.16.0	Minimum conformance requirements	3642

7.6.16.0.1	Minimum conformance requirements for intra-frequency measurement	3642

7.6.16.0.2	Minimum conformance requirements for inter-frequency measurement	3644

7.6.16.0.3	Minimum conformance requirements for interruptions	3645

7.6.16.1	NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX	3646

7.6.16.2	NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used (PCell in FR2)	3652

7.6.16.3	NR SA FR2 event triggered reporting tests on deactivated SCell measurement via NCSG in non-DRX	3657

7.6.17	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG in FR2	3663

7.6.17.0	Minimum conformance requirements	3663

7.6.17.0.1	Minimum conformance requirements for SA event triggered reporting with one pre-configured gap and one measurement gap	3663

7.6.17.0.2	Minimum conformance requirements SA event triggered reporting with autonomous activation/deactivation of Pre-MGs in FR2	3663

7.6.17.1	NR SA FR2 and FR2-FR2 event-triggered reporting with concurrent Type-2 measurement gap and Pre-MG	3664

7.6.17.2	NR SA FR2-FR2 event-triggered reporting with two concurrent Pre-MGs	3672

7.6.18	SA event triggered reporting tests with concurrent gaps and NCSG	3675

7.6.18.0	Minimum conformance requirements	3675

7.6.18.0.1	Minimum conformance requirements for concurrent measurement gaps with NCSG	3675

7.6.18.1	NR SA FR2 event-triggered reporting with concurrent Type-2 measurement gap and NCSG	3676

7.6.19	SA event triggered reporting tests with NeedForGap in FR2	3680

7.6.19.0	Minimum conformance requirements	3680

7.6.19.0.1	Minimum conformance requirements for Intra-frequency measurements with measurement gaps	3680

7.6.19.0.2	Minimum conformance requirements for Intra-frequency measurements without measurement gaps	3683

7.6.19.0.3	Minimum conformance requirements for Inter-frequency measurements without measurement gaps	3687

7.6.19.1	NR SA FR2 event-triggered reporting without interruption outside gap	3692

7.6.19.2	NR SA FR2  event triggered reporting test without gap without interruption	3698

7.6.19.3	NR SA FR2-FR2 event triggered reporting test without gap without interruption under non-DRX	3705

7.6.20	LTM Intra-frequency L1-RSRP measurement	3710

7.6.20.0	Minimum conformance requirements	3710

7.6.20.0.1	Minimum conformance requirements for SSB-based intra-frequency L1-RSRP measurement	3710

7.6.20.1	NR SA FR2 Intra-frequency SSB based L1-RSRP measurement	3712

7.6.21	LTM Inter-frequency L1-RSRP measurement with measurement gap	3718

7.6.21.0	Minimum conformance requirements	3718

7.6.21.0.1	Minimum conformance requirements for SSB-based inter-frequency L1-RSRP measurement with gap	3718

7.6.21.1	NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM	3719

7.6.22	LTM Inter-frequency L1-RSRP measurement without measurement gap	3725

7.6.22.0	Minimum conformance requirements	3725

7.6.22.1	NR SA FR2-FR2 SSB based L1-RSRP measurement without measurement gap	3727

7.7	Measurement performance requirements	3733

7.7.1	SS-RSRP	3733

7.7.1.0	Minimum conformance requirements	3733

7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	3733

7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	3733

7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy	3733

7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy	3741

7.7.1.3	Inter-frequency measurements between FR1 and FR2	3749

7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy	3749

7.7.1.3.2	Void	3756

7.7.2	SS-RSRQ	3756

7.7.2.0	Minimum conformance requirements	3756

7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	3756

7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	3756

7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy	3756

7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy	3760

7.7.3	SS-SINR	3766

7.7.3.0	Minimum conformance requirements	3766

7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	3766

7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	3766

7.7.3.1	NR SA FR2 SS-SINR measurement accuracy	3766

7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy	3771

7.7.4	L1-RSRP	3777

7.7.4.0	Minimum conformance requirements	3777

7.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	3777

7.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	3777

7.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	3777

7.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	3777

7.7.4.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	3777

7.7.4.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy	3782

7.7.5	SRS-RSRP	3788

7.7.5.0	Minimum conformance requirements	3788

7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	3788

7.7.5.1	NR SA FR2 SRS-RSRP measurement accuracy	3790

7.7.5.2	NR SA FR2 CLI-RSSI measurement accuracy	3795

7.7.6	L1-SINR	3800

7.7.6.0	Minimum conformance requirements	3800

7.7.6.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	3800

7.7.6.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR L1-SINR measurement accuracy	3802

7.7.6.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	3804

7.7.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	3807

7.7.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy	3812

7.7.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	3817

7.7.7	CSI-RSRP	3822

7.7.7.0	Minimum conformance requirements	3822

7.7.7.1	Intra-frequency measurements	3822

7.7.7.1.1	NR SA FR2 CSI-RSRP  measurement accuracy	3822

7.7.7.2	Inter-frequency measurements	3825

7.7.7.2.1	NR SA FR2-FR2 CSI-RSRP measurement accuracy	3825

7.7.8	CSI-RSRQ	3829

7.7.8.0	Minimum conformance requirements	3829

7.7.8.0.1	Intra-frequency CSI-RSRQ accuracy requirements	3829

7.7.8.0.1.1	Absolute CSI-RSRQ accuracy	3829

7.7.8.0.2	Inter-frequency CSI-RSRQ accuracy requirements	3830

7.7.8.0.2.1	Absolute CSI-RSRQ accuracy	3830

7.7.8.0.2.2	Relative CSI-RSRQ accuracy	3831

7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3832

7.7.8.2	SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3836

7.7.9	CSI-SINR	3841

7.7.9.0	Minimum conformance requirements	3841

7.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements	3841

7.7.9.0.1.1	Absolute CSI-SINR accuracy	3841

7.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements	3842

7.7.9.0.2.1	Absolute CSI-SINR accuracy	3842

7.7.9.0.2.2	Relative CSI-SINR accuracy	3843

7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3844

7.7.9.2	SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3848

7.7.10 to 7.7.13	3853

7.7.14	L1-RSRP measurement for group-based beam reporting	3853

7.7.14.1	NR SA FR2 SSB based L1-RSRP measurement for group-based beam reporting	3853

7.7.14.2	NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off	3857

7.7.15	LTM L1-RSRP measurement	3860

7.7.15.0	Minimum conformance requirements	3860

7.7.15.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	3860

7.7.15.0.2	SSB based relative L1-RSRP measurement accuracy requirements	3861

7.7.15.1	NR SA FR2 SSB based inter-frequency L1-RSRP measurement	3862

8.0	General	3871

8.1	Void	3871

8.2.1.0	Minimum conformance requirements	3871

8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell	3873

8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3880

8.2.2.0	Minimum conformance requirements	3887

8.2.2.1	E-UTRA – NR FR1 Early Measurement Reporting	3889

8.2.2.2	E-UTRA – NR FR2 Early Measurement Reporting	3897

8.3.1.0	Minimum conformance requirements	3902

8.3.1.1	E-UTRA - NR FR1 handover with known target cell	3903

8.4.1.0	Minimum conformance requirements	3908

8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX	3909

8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX	3914

8.4.2.0	Minimum conformance requirements	3919

8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX	3923

8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX	3929

8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX	3935

8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX	3941

8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX	3947

8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX	3952

8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX	3957

8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX	3962

8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3967

8.4.3	Inter-RAT measurements without gaps	3973

8.4.3.0	Minimum conformance requirements	3973

8.4.3.0.1	Minimum conformance requirements for E-UTRAN FDD/TDD – NR  event-triggered reporting without gap	3973

8.4.3.1	E-UTRAN - NR SA FR2 event-triggered reporting without gap with interruption	3975

8.4.3.2	E-UTRAN - NR SA FR1 event-triggered reporting without gap	3978

8.5.1	SFTD measurement accuracy	3982

8.5.1.0	Minimum conformance requirements	3982

8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy	3984

8.5.2	Inter-RAT measurement accuracy	3989

8.5.2.1	SS-RSRP	3989

8.5.2.1.0	Minimum conformance requirements	3989

8.5.2.1.1	SS-RSRP with NR FR1 target cell	3990

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell	3995

8.5.2.2	SS-RSRQ	3999

8.5.2.2.0	Minimum conformance requirements	3999

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell	3999

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell	4002

8.5.2.3	SS-SINR	4006

8.5.2.3.0	Minimum conformance requirements	4006

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell	4006

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell	4009

9	NR sidelink	4014

9.1	NR sidelink in FR1	4014

9.1.1	UE transmit timing	4014

9.1.1.0	Minimum conformance requirements	4014

9.1.1.1	NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source	4015

9.1.1.2	NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source	4017

9.1.1.3	NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source	4020

9.1.2	Initiation/Cease of S-SSB transmission	4025

9.1.2.0	Minimum conformance requirements	4025

9.1.2.1	NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source	4027

9.1.2.2	NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source	4032

9.1.3	Synchronization reference selection/reselection	4035

9.1.3.0	Minimum conformance requirements	4035

9.1.3.1	NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source	4039

9.1.3.2	NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source	4045

9.1.4	L1 SL-RSRP measurements	4050

9.1.4.0	Minimum conformance requirements	4050

9.1.4.1	NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection	4051

9.1.4.2	NR SA FR1 L1 SL-RSRP measurement for resource pre-emption	4057

9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation	4063

9.1.5	Congestion control measurement	4070

9.1.5.0	Minimum conformance requirements	4070

9.1.5.1	NR SA FR1 congestion control measurement for concurrent operation	4071

9.1.5.2	NR SA FR1 congestion control measurement for PC5-only operation	4079

9.1.6	Interruption	4085

9.1.6.0	Minimum conformance requirements	4085

9.1.6.1	NR SA FR1 interruption to WAN due to NR sidelink communication	4085

9.1.6.2	4089

9.1.6.3	NR SA FR1 Interruptions at NR sidelink discovery configuration	4089

9.1.7	Selection / Reselection of relay UE	4094

9.1.7.0	Minimum conformance requirements	4094

9.1.7.0.1	Minimum conformance requirements for selection / reselection of relay UE	4094

9.1.7.1	NR SA FR1 Selection / Reselection of relay UE for U2N case	4094

10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4103

10.0	General	4103

10.0.1	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	4103

10.1	RRC_CONNECTED state mobility	4103

10.1.1	Random Access	4103

10.1.1.0	Minimum conformance requirements	4103

10.1.1.0.1	Minimum conformance requirements for 4-step RA type	4103

10.1.1.0.1.1	Contention based random access	4104

10.1.1.0.1.1.1	Correct behaviour when transmitting Random Access Preamble	4104

10.1.1.0.1.1.2	Correct behaviour when receiving Random Access Response	4104

10.1.1.0.1.1.3	Correct behaviour when not receiving Random Access Response	4104

10.1.1.0.1.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	4105

10.1.1.0.1.1.6	Correct behaviour when contention Resolution timer expires	4105

10.1.1.0.1.2	Non-contention based random access	4105

10.1.1.0.1.2.1	Correct behaviour when transmitting Random Access Preamble	4105

10.1.1.0.1.2.2	Correct behaviour when receiving Random Access Response	4105

10.1.1.0.1.2.3	Correct behaviour when not receiving Random Access Response	4106

10.1.1.0.2	Minimum conformance requirements for 2-step RA type	4106

10.1.1.0.2.1	Contention based random access	4106

10.1.1.0.2.1.1	Correct behaviour when transmitting MsgA	4106

10.1.1.0.2.1.2	Correct behaviour when receiving MsgB	4107

10.1.1.0.2.1.3	Correct behaviour when not receiving MsgB	4107

10.1.1.0.2.2	Non-contention based random access	4107

10.1.1.0.2.2.1	Correct behaviour when transmitting MsgA	4107

10.1.1.0.2.2.2	Correct behaviour when receiving MsgB	4108

10.1.1.0.2.2.3	Correct behaviour when not receiving MsgB	4108

10.1.1.1.1	EN-DC FR1 4-step RA type contention based random access test for NR PSCell with CCA	4108

10.1.1.1.2	EN-DC FR1 4-step RA type non-contention based random access for NR PSCell with CCA	4116

10.1.1.1.3	EN-DC FR1 2-step RA type contention-based random access for NR PSCell with CCA	4124

10.1.1.1.4	EN-DC FR1 2-step RA type non-contention based random access for NR PSCell with CCA	4132

10.1.2	EN-DCFR1 Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4141

10.2	Timing	4150

10.2.1	UE transmit timing	4150

10.2.1.0	Minimum conformance requirements	4150

10.2.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	4150

10.2.1.1	EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA	4150

10.2.2	UE timing advance	4157

10.2.2.0	Minimum conformance requirements	4157

10.2.2.0.1	Minimum conformance requirements for timing advance adjustment delay	4157

10.2.2.0.2	Minimum conformance requirements for timing advance adjustment accuracy	4157

10.2.2.1	EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4157

10.3	Signalling characteristics	4163

10.3.1	Radio link monitoring	4163

10.3.1.0	General	4163

10.3.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring under CCA	4164

10.3.1.1	Requirements for determining UE in-sync or out-of-sync status	4167

10.3.1.2	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	4167

10.3.1.3	EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	4175

10.3.1.4	Void	4180

10.3.1.5	Void	4180

10.3.2	Void	4181

10.3.3	SCell activation and deactivation delay	4181

10.3.3.0	Minimum Conformance Requirements	4181

10.3.3.0.1	SCell Activation Delay Requirement for Deactivated SCell under CCA	4181

10.3.3.0.2	SCell Deactivation Delay Requirement for Activated SCell under CCA	4185

10.3.3.1	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4185

10.3.3.2	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4194

10.3.3.3	EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4198

10.3.4	Beam failure detection and link recovery procedures	4203

10.3.4.0	General	4203

10.3.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	4204

10.3.4.0.3	Scheduling availability of UE during beam failure detection under CCA	4206

10.3.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	4206

10.3.4.1	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode	4206

10.3.4.2	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode	4214

10.3.5	Active BWP switching	4222

10.3.5.1	EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA	4222

10.3.5.2	DCI-based and Timer-based Active BWP Switch	4229

10.3.5.2.0	Minimum conformance requirements	4229

10.3.5.2.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	4232

10.3.5.2.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA	4240

10.3.5.3	RRC-based Active BWP Switch	4248

10.3.5.3.0	Minimum conformance requirements	4248

10.3.5.3.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	4249

10.3.6	PSCell addition and release delay	4255

10.3.6.0	Minimum conformance requirements	4255

10.3.6.0.1	NR PSCell Addition Delay Requirement	4255

10.3.6.0.2	NR PSCell Release Delay Requirement	4256

10.3.6.1	EN-DC FR1 Addition and Release Delay of known NR PSCell on the carrier under CCA	4256

10.4	Measurement procedures	4263

10.4.1	Intra-frequency measurements	4263

10.4.1.0	Minimum conformance requirements	4263

10.4.1.1	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA	4277

10.4.1.2	Void	4282

10.4.1.3	Void	4282

10.4.1.4	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA	4282

10.4.2	Inter-frequency measurements	4288

10.4.2.0	Minimum conformance requirements	4288

10.4.2.1	Void	4294

10.4.2.2	Void	4294

10.4.2.3	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used	4294

10.4.2.4	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used	4299

10.4.2.5	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used	4306

10.4.2.6	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used	4311

10.4.2.7	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used	4318

10.4.2.8	EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used	4325

10.4.2.9	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used	4332

10.4.2.10	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used	4339

10.4.3	L1-RSRP measurement for beam reporting under CCA	4346

10.4.3.0	Minimum conformance requirements	4346

10.4.3.1	EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used	4348

10.4.3.2	EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used	4353

10.4.3.3	EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used	4357

10.4.3.4	EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used	4362

10.4.4	E-UTRAN-NR inter-RAT measurements on NR carrier frequency under CCA	4366

10.4.4.0	Minimum conformance requirements	4366

10.4.4.1	EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4369

10.4.4.2	EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4377

10.4.4.3	EN-DC FR1 E-UTRA-NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4385

10.4.4.4	EN-DC FR1 E-UTRA-NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4393

10.5	Measurement Performance	4400

10.5.1	SS-RSRP	4400

10.5.1.0	Minimum Conformance Requirements	4400

10.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4400

10.5.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	4400

10.5.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4401

10.5.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	4402

10.5.1.1	EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell	4402

10.5.1.2	EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell	4407

10.5.2	SS-RSRQ	4413

10.5.2.0	Minimum Conformance Requirements	4413

10.5.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4413

10.5.2.0.2	Void	4414

10.5.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4414

10.5.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	4415

10.5.2.1	EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	4415

10.5.2.2	EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	4420

10.5.3	SS-SINR	4424

10.5.3.0	Minimum Conformance Requirements	4424

10.5.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	4424

10.5.3.0.2	Void	4425

10.5.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	4425

10.5.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	4426

10.5.3.1	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA	4426

10.5.3.2	EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA	4431

10.5.3.3	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA	4436

10.5.4	L1-RSRP measurement accuracy for beam reporting with CCA serving cell	4442

10.5.4.0	Minimum conformance requirements	4442

10.5.4.1	EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA	4444

10.5.5	RSSI	4448

10.5.5.0	Minimum Conformance Requirements	4448

10.5.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	4449

10.5.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	4449

10.5.5.1	EN-DC FR1 RSSI measurement accuracy on PSCC with CCA	4449

10.5.5.2	EN-DC FR1 RSSI measurement accuracy on SCC with CCA	4454

10.5.5.3	 EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA	4458

10.5.6	Channel occupancy	4463

10.5.6.0	Minimum Conformance Requirements	4463

10.5.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	4463

10.5.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	4464

10.5.6.1	EN-DC FR1 Channel occupancy measurement accuracy on PSCC with CCA	4464

10.5.6.2	EN-DC FR1 Channel occupancy measurement accuracy on SCC with CCA	4468

10.5.6.3	EN-DC FR1-FR1 Channel occupancy measurement accuracy on a carrier with CCA	4473

11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4479

11.0	General	4479

11.0.1	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	4479

11.1	RRC_IDLE state mobility	4480

11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4480

11.1.1.0	Minimum conformance requirements	4480

11.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection with both source and target NR carrier frequencies under CCA	4480

11.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection with target NR carrier frequency under CCA	4481

11.1.1.1	NR SA FR1 Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4483

11.1.1.2	NR SA FR1 Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4489

11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4496

11.1.2.0	Minimum conformance requirements	4496

11.1.2.1	NR SA FR1 Cell re-selection to NR with source NR carrier frequency under CCA	4497

11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4505

11.1.3.0	Minimum conformance requirements	4505

11.1.3.1	NR SA FR1 Cell re-selection from NR carrier with target NR carrier frequency under CCA	4506

11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4514

11.1.4.0	Minimum conformance requirements	4514

11.1.4.0.1	Minimum conformance requirements for Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4514

11.1.4.1	NR SA FR1 Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4515

11.1.4.2	NR SA FR1 Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4521

11.2	RRC_CONNECTED state mobility	4527

11.2.1	Handover	4527

11.2.1.0	Minimum conformance requirements	4527

11.2.1.0.1	Minimum conformance requirements for NR FR1 - NR FR1 Handover	4527

11.2.1.1	NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4528

11.2.1.2	NR SA FR1 Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4532

11.2.1.3	NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4537

11.2.1.4	NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4542

11.2.1.5	NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4548

11.2.1.6	NR SA FR1 Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4554

11.2.1.7	NR SA FR1 carrier under CCA - E-UTRAN handover with known target cell	4559

11.2.1.8	NR SA FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4568

11.2.2	RRC connection mobility control	4574

11.2.2.1	RRC re-establishment	4574

11.2.2.1.0	Minimum conformance requirements	4574

11.2.2.1.1	NR SA FR1 Intra-frequency RRC Re-establishment with CCA in FR1	4576

11.2.2.1.2	NR SA FR1 Inter-frequency RRC Re-establishment with CCA in FR1	4580

11.2.2.1.3	NR SA FR1 Intra-frequency RRC Re-establishment with CCA without serving cell timing	4585

11.2.2.1.4	NR SA FR1 Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4590

11.2.2.2	Random Access	4596

11.2.2.2.0	Minimum conformance requirements	4596

11.2.2.2.1	NR SA FR1 4-step RA type contention-based random access for NR PCell with CCA	4601

11.2.2.2.2	NR SA FR1 4-step RA type non-contention based random access for NR PCell with CCA	4609

11.2.2.2.3	NR SA FR1 2-step RA type contention-based random access for NR PCell with CCA	4617

11.2.2.2.4	NR SA FR1 2-step RA type non-contention based random access for NR PCell with CCA	4625

11.2.2.3	RRC connection release with redirection	4634

11.2.2.3.0	Minimum conformance requirements	4634

11.2.2.3.1	NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4635

11.2.2.3.2	NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4641

11.3	Timing	4646

11.3.1	UE transmit timing	4646

11.3.1.0	Minimum conformance requirements	4646

11.3.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	4646

11.3.1.1	NR SA FR1 UE Transmit Timing Test with PCell under DL CCA	4647

11.3.2	UE timing advance	4653

11.3.2.0	Minimum conformance requirements	4653

11.3.2.0.1	Minimum conformance requirements for timing advance adjustment delay	4653

11.3.2.0.2	Minimum conformance requirements for timing advance adjustment accuracy	4653

11.3.2.1	NR SA FR1 UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4653

11.4	Signalling characteristics	4658

11.4.1	Radio link monitoring	4658

11.4.1.0	General	4658

11.4.1.0.1	Minimum conformance requirements for SSB based radio link monitoring under CCA	4658

11.4.1.1	Requirements for determining UE in-sync or out-of-sync status	4659

11.4.1.2	NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM-RS in non-DRX mode under CCA	4659

11.4.1.3	NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	4666

11.4.2	Void	4673

11.4.3	SCell activation and deactivation delay	4673

11.4.3.0	Minimum conformance requirements	4673

11.4.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	4673

11.4.3.1	NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4673

11.4.3.2	NR SA FR1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4682

11.4.3.3	NR SA FR1 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4686

11.4.4	Beam failure detection and link recovery procedures	4692

11.4.4.0	General	4692

11.4.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	4692

11.4.4.0.2	Void	4692

11.4.4.0.3	Scheduling availability of UE during beam failure detection under CCA	4693

11.4.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	4693

11.4.4.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA	4693

11.4.4.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA	4701

11.4.5	Active BWP switching	4709

11.4.5.1	NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4709

11.4.5.2	DCI-based and Timer-based Active BWP Switch	4716

11.4.5.2.0	Minimum conformance requirements	4716

11.4.5.2.1	NR SA FR1 DCI-based DL active BWP switch of PCell with non-DRX under CCA	4718

11.4.5.2.2	NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA	4726

11.4.5.3	RRC-based Active BWP Switch	4734

11.4.5.3.0	Minimum conformance requirements	4734

11.4.5.3.1	NR SA FR1 RRC-based DL active BWP switch of Cell with non-DRX under CCA	4734

11.5	Measurement procedure	4740

11.5.1	Intra-frequency measurements	4740

11.5.1.0	Minimum Conformance Requirements	4740

11.5.1.1	SA FR1 Event-triggered reporting tests on PCC without gaps under non-DRX	4741

11.5.1.2	SA FR1 Event-triggered reporting tests on PCC without gaps under DRX	4746

11.5.2	Inter-frequency measurements	4751

11.5.2.0	Minimum Conformance Requirements	4751

11.5.2.3	NR SA FR1-FR1 Event triggered reporting tests with CCA without SSB time index detection when DRX is not used	4751

11.5.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4757

11.5.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4764

11.5.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4770

11.5.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4776

11.5.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4782

11.5.2.9	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4787

11.5.2.10	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4793

11.5.3	Inter-RAT E-UTRAN measurements	4799

11.5.3.0	Minimum conformance requirements	4799

11.5.3.1	NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX	4800

11.5.3.2	NR SA FR1 - E-UTRAN event-triggered reporting in DRX	4805

11.5.4	L1-RSRP measurements for beam reporting	4809

11.5.4.0	Minimum conformance requirements	4809

11.5.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used	4809

11.5.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is used	4815

11.5.4.3	NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is not used	4818

11.5.4.4	NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is used	4822

11.6	Measurement Performance	4826

11.6.1	SS-RSRP	4826

11.6.1.0	Minimum Conformance Requirements	4826

11.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4826

11.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	4826

11.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	4826

11.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	4826

11.6.1.1	NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA	4826

11.6.1.2	NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4831

11.6.2	SS-RSRQ	4835

11.6.2.0	Minimum Conformance Requirements	4835

11.6.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4836

11.6.2.0.2	Void	4836

11.6.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	4836

11.6.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	4836

11.6.2.1	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	4836

11.6.2.2	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	4841

11.6.2.3	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA	4846

11.6.2.4	NR SA FR1 Inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA	4850

11.6.3	SS-SINR	4858

11.6.3.0	Minimum Conformance Requirements	4858

11.6.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	4858

11.6.3.0.2	Void	4858

11.6.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	4858

11.6.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	4858

11.6.3.1	NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA	4858

11.6.3.2	NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA	4863

11.6.3.3	NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA	4868

11.6.3.4	NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA from carrier without CCA	4873

11.6.4	L1-RSRP measurement accuracy for beam reporting with CCA serving cell	4881

11.6.4.0	Minimum conformance requirements	4881

11.6.4.0.1	Minimum conformance requitements for SSB-based L1-RSRP measurement  accuracy for beam reporting under CCA	4882

11.6.4.1	NR SA FR1 SSB based L1-RSRP measurement accuracy with CCA	4882

11.6.5	RSSI	4886

11.6.5.0	Minimum Conformance Requirements	4886

11.6.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	4886

11.6.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	4886

11.6.5.1	NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA	4887

11.6.5.2	NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA	4891

11.6.5.3	NR SA FR1 Inter-frequency RSSI measurement accuracy on a carrier with CCA	4895

11.6.6	Channel occupancy	4900

11.6.6.0	Minimum Conformance Requirements	4900

11.6.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	4900

11.6.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	4900

11.6.6.1	NR SA FR1 Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4900

11.6.6.2	NR SA FR1 Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4906

11.6.6.3	NR SA FR1 Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4911

12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4921

12.1	RRC_IDLE State Mobility	4921

12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4921

12.1.1.0	Minimum Conformance Requirements	4921

12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4922

12.2	RRC_CONNECTED State Mobility	4930

12.2.1	Handover	4930

12.2.1.0	Minimum Conformance Requirements	4930

12.2.1.1	E-UTRAN - NR with CCA handover with known target cell	4931

12.3	Void	4938

12.4	Measurement Procedure	4938

12.4.1	E-UTRAN-NR inter-RAT SFTD measurements	4938

12.4.1.0	Minimum Conformance Requirements	4938

12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4939

12.4.2	E-UTRAN-NR inter-RAT measurements on NR carrier frequency under CCA	4946

12.4.2.0	Minimum Conformance Requirements	4946

12.4.2.0.1	E-UTRAN – NR measurements	4946

12.4.2.1	E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is not used under CCA	4949

12.4.2.2	E-UTRAN-NR FR1 inter-RAT event triggered reporting without SSB time index detection when DRX is used under CCA	4955

12.4.2.3	E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is not used under CCA	4960

12.4.2.4	E-UTRAN-NR FR1 inter-RAT event triggered reporting with SSB time index detection when DRX is used under CCA	4965

12.5	Measurement Performance	4970

12.5.1	E-UTRAN-NR SFTD	4970

12.5.1.0	Minimum conformance requirements	4970

12.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy under CCA	4971

13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4976

13.1	Void	4976

13.2	Signalling Characteristics	4976

13.2.2	SCell activation and deactivation delay	4976

13.2.2.0	Minimum conformance requirements	4976

13.2.2.0.1	Minimum conformance requirements for SCell activation and deactivation delay	4976

13.2.2.1	NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4976

13.2.2.2	NR SA FR1 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4986

13.2.2.3	NR SA FR1 SCell Activation and Deactivation of unknown SCell under CCA	4990

13.3	Measurement Procedure	4995

13.3.1	Intra-frequency measurements	4995

13.3.1.0	Minimum Conformance Requirements	4995

13.3.1.1	NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under non-DRX	4996

13.3.1.2	NR SA FR1 Event-triggered reporting tests on SCC under CCA without gaps under DRX	5003

13.3.1.3	NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under non-DRX	5010

13.3.1.4	NR SA FR1 Event-triggered reporting tests on SCC under CCA with per-UE gaps under DRX	5018

13.3.2	Inter-frequency measurements	5026

13.3.2.0	Minimum Conformance Requirements	5026

13.3.2.1	Void	5026

13.3.2.2	Void	5026

13.3.2.3	NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is not used	5026

13.3.2.4	NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA without SSB time index detection when DRX is used	5033

13.3.2.5	NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is not used	5041

13.3.2.6	NR SA FR1-FR1 Event triggered reporting tests on a carrier with CCA with SSB time index detection when DRX is used	5048

13.3.3	L1-RSRP measurement for beam reporting for SCell under CCA	5055

13.3.3.0	Minimum Conformance Requirements	5055

13.3.3.1	NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is not used	5055

13.3.3.2	NR SA FR1 SSB based L1-RSRP measurement for SCell under CCA when DRX is used	5062

13.4	Measurement Performance	5068

13.4.1	SS-RSRP	5068

13.4.1.0	Minimum Conformance Requirements	5068

13.4.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	5068

13.4.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	5068

13.4.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	5068

13.4.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	5068

13.4.1.1	NR SA FR1 SS-RSRP measurement accuracy on a carrier with CCA	5068

13.4.2	SS-RSRQ	5072

13.4.2.0	Minimum Conformance Requirements	5072

13.4.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	5073

13.4.2.0.2	Void	5073

13.4.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	5073

13.4.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	5073

13.4.2.1	NR SA FR1 SS-RSRQ measurement accuracy on a carrier with CCA	5073

13.4.3	SS-SINR	5082

13.4.3.0	Minimum Conformance Requirements	5082

13.4.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	5082

13.4.3.0.2	Intra-frequency relative SS-SINR measurement accuracy requirements under CCA	5082

13.4.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	5082

13.4.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	5082

13.4.3.1	NR SA FR1 SS-SINR measurement accuracy on a carrier with CCA	5082

13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	5092

13.4.4.0	Minimum Conformance Requirements	5092

13.4.4.0.1	SSB-based L1-RSRP measurement absolute accuracy under CCA	5092

13.4.4.0.2	SSB-based L1-RSRP measurement relative accuracy under CCA	5092

13.4.4.1	NR SA FR1 L1-RSRP measurement accuracy for beam reporting on a carrier with CCA	5092

13.4.5	RSSI	5098

13.4.5.0	Minimum Conformance Requirements	5098

13.4.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	5099

13.4.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	5099

13.4.5.0.3	RSSI measurement report mapping	5099

13.4.5.1	NR SA FR1 RSSI measurement accuracy on a carrier with CCA	5099

13.4.5.2	NR SA FR1-FR1 RSSI measurement accuracy on a carrier with CCA	5104

13.4.6	Channel Occupancy	5109

13.4.6.0	Minimum Conformance Requirements	5109

13.4.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	5109

13.4.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	5109

13.4.6.1	NR SA FR1 channel occupancy measurement accuracy on a carrier with CCA	5109

13.4.6.2	NR SA FR1-FR1 channel occupancy measurement accuracy on a carrier with CCA	5115

14	NR standalone tests for Satellite access	5120

14.0	General	5120

14.0.1	Principle of testing GSO and NGSO scenarios	5120

14.0.2	Principle of testing different RRM requirements	5120

14.0.3	Principle of testing different ephemeris formats	5121

14.0.4	General setup for SIB19	5123

14.0.5	Initial test environment conditions for RRM NTN tests	5123

14.1	RRC_IDLE state mobility	5124

14.1.0	Minimum conformance requirements	5124

14.1.0.1	Measurements of intra-frequency NR cells	5124

14.1.0.2	Measurements of inter-frequency NR cells	5126

14.1.0.3	Measurements of inter-RAT NR – E-UTRA cells	5129

14.1.0.4	Measurements of inter-frequency NR cells with TN carrier	5131

14.1.1	NR SA FR1 Cell Reselection for Satellite Access	5133

14.1.2	NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access	5139

14.1.3	NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access	5144

14.1.4	NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access	5150

14.1.5	NR SA FR1-FR1 Cell Reselection for Satellite Access	5155

14.1.6	NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access	5160

14.1.7	NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access	5165

14.1.8	NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access	5169

14.1.9	NR SA FR1-FR1 Cell reselection for UE fulfilling low mobility relaxed measurement criterion for Satellite Access	5173

14.1.10	NR SA FR1-FR1 Cell reselection for UE fulfilling not-at-cell edge relaxed measurement criterion for Satellite Access	5179

14.1.11	NR SA FR1 inter-RAT Cell reselection to E-UTRAN TN carrier for Satellite Access	5185

14.1.12	NR SA FR1-FR1 Cell reselection to TN carrier for Satellite Access	5189

14.2	RRC_CONNECTED state mobility	5193

14.2.1	Handover for SAN	5193

14.2.1.0	Minimum conformance requirements	5193

14.2.1.0.1	Minimum conformance requirements for handover	5193

14.2.1.0.1.1	Handover delay	5193

14.2.1.0.1.2	Interruption time for RACH-based handover	5194

14.2.1.0.1.3	Interruption time for RACH-less handover	5194

14.2.1.0.2	Minimum conformance requirements for conditional handover	5195

14.2.1.1	NR SA FR1 Handover for Satellite Access	5197

14.2.1.2	NR SA FR1-FR1 Handover for Satellite Access	5201

14.2.1.3	NR SA FR1 Time-based Conditional Handover for Satellite Access	5205

14.2.1.4	NR SA FR1-FR1 Time-based Conditional Handover for NR Satellite Access	5212

14.2.1.5	NR SA FR1 Distance-based Conditional Handover for Satellite Access	5219

14.2.1.5.1	Test purpose	5219

14.2.1.6	NR SA FR1-FR1 Distance-based Conditional Handover for Satellite Access	5227

14.2.1.8	NR SA FR1 RACH-less Handover for Satellite Access	5235

14.2.2	RRC Connection Mobility Control for SAN	5239

14.2.2.1	RRC Re-establishment for SAN	5239

14.2.2.1.0	Minimum conformance requirements for RRC Re-establishment for Satellite Access	5240

14.2.2.1.1	NR SA FR1 RRC Re-establishment for Satellite Access	5241

14.2.2.1.2	NR SA FR1-FR1 RRC Re-establishment for Satellite Access	5246

14.2.2.2	Random Access for SAN	5250

14.2.2.2.0	Minimum conformance requirements for random access for Satellite Access	5250

14.2.2.2.1	NR SA FR1 4-step contention based random access for Satellite Access	5252

14.2.2.2.2	NR SA FR1 4-step non-contention based random access for Satellite Access	5259

14.2.2.3	RRC Connection Release with Redirection for SAN	5265

14.2.2.3.0	Minimum conformance requirements	5265

14.2.2.3.1	NR SA FR1-FR1 RRC connection release with redirection for Satellite Access	5266

14.2.2.4	NR SA FR1 RACH-based Hard Satellite switching with re-synchronization for Satellite Access	5270

14.2.2.5	NR SA FR1 RACH-less Soft Satellite switching with re-synchronization for Satellite Access	5275

14.2.3	NR SA FR1 Time-based conditional Handover without L3 measurement criteria for Satellite Access	5282

14.2.3.1	Test purpose	5282

14.2.3.2	Test applicability	5282

14.2.3.3	Minimum conformance requirement	5282

14.2.3.4	Test description	5283

14.2.3.4.1	Initial conditions	5283

14.2.3.4.2	Test Procedure	5284

14.2.3.4.3	Message Contents	5284

14.2.3.5	Test requirement	5287

14.2.4	NR SA FR1-FR1 Time-based conditional Handover without L3 measurement criteria for Satellite Access	5288

14.2.4.1	Test purpose	5288

14.2.4.2	Test applicability	5288

14.2.4.3	Minimum conformance requirement	5289

14.2.4.4	Test description	5289

14.2.4.4.1	Initial conditions	5289

14.2.4.4.2	Test Procedure	5290

14.2.4.4.3	Message Contents	5290

14.2.4.5	Test requirement	5294

14.3	Timing for Satellite Access	5295

14.3.1	UE transmit timing for Satellite Access	5295

14.3.1.0	Minimum conformance requirements	5295

14.3.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	5296

14.3.1.0.2	Minimum conformance requirements for gradual timing adjustment	5296

14.3.1.1	NR SA FR1 UE transmit timing accuracy for Satellite Access	5297

14.3.2	Timing Advance for Satellite Access	5303

14.3.2.0	Minimum conformance requirements	5303

14.3.2.1	NR SA FR1 Timing Advance adjustment accuracy for Satellite Access	5303

14.4	Signalling characteristics	5308

14.4.1	Radio link monitoring for Satellite Access	5308

14.4.1.0	General	5308

14.4.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring for Satellite Access	5309

14.4.1.0.2	Minimum conformance requirements for CSI-RS based Radio Link Monitoring for Satellite Access	5311

14.4.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access	5314

14.4.1.2	NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access	5319

14.4.1.3	NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access	5325

14.4.1.4	NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access	5331

14.4.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access	5337

14.4.1.6	NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access	5343

14.4.1.7	NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access	5349

14.4.1.8	NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access	5354

14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	5360

14.4.2.00	General	5360

14.4.2.0	Minimum conformance requirements	5361

14.4.2.0.1	Minimum conformance requitements for SSB based beam failure detection	5361

14.4.2.0.2	Measurement restriction for SSB based beam failure detection	5362

14.4.2.0.3	Minimum conformance requitements for CSI-RS based beam failure detection	5363

14.4.2.0.4	Measurement restriction for CSI-RS based beam failure detection	5364

14.4.2.0.5	Minimum requirement for L1 indication	5365

14.4.2.0.6	Minimum conformance requitements for SSB based candidate beam detection	5365

14.4.2.0.7	Measurement restriction for SSB based candidate beam detection	5366

14.4.2.0.8	Minimum conformance requitements for CSI-RS based candidate beam detection	5367

14.4.2.0.9	Measurement restriction for CSI-RS based candidate beam detection	5368

14.4.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for Satellite Access	5368

14.4.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in DRX mode for Satellite Access	5377

14.4.2.3	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access	5386

14.4.2.4	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access	5394

14.4.3	Active BWP Switch for SAN	5403

14.4.3.1	DCI-based and Timer-based Active BWP Switch	5403

14.4.3.1.0	Minimum conformance requirements	5403

14.4.3.1.1	NR SA FR1 DCI-based DL active BWP switch with non-DRX for Satellite Access	5404

14.4.3.2	RRC-based Active BWP Switch	5412

14.4.3.2.0	Minimum conformance requirements	5412

14.4.3.2.1	NR SA FR1 RRC-based DL active BWP switch with non-DRX for Satellite Access	5412

14.4.4	UE specific CBW change for SAN	5418

14.4.4.0	Minimum conformance requirements	5418

14.4.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for Satellite Access	5419

14.4.5.1	NR SA FR1 MAC-CE based pathloss reference signal switch delay for Satellite Access	5424

14.5	Measurement procedure	5430

14.5.1	Intra-frequency Measurements for SAN	5430

14.5.1.0	Minimum conformance requirements	5430

14.5.1.0.1	Minimum conformance requitements for intra-frequency measurements without measurement gaps	5430

14.5.1.0.2	Minimum conformance requitements for intra-frequency measurements with measurement gaps	5434

14.5.1.1	NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access	5435

14.5.1.2	NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access	5439

14.5.1.3	NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access	5443

14.5.1.4	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	5447

14.5.1.5	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access	5451

14.5.1.6	NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	5456

14.5.2	Inter-frequency Measurements for SAN	5460

14.5.2.0	Minimum conformance requirements	5460

14.5.2.1	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	5464

14.5.2.2	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access	5470

14.5.2.3	NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access	5475

14.5.2.4	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access	5481

14.5.2.5	Void	5487

14.5.2.6	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	5487

14.5.2.7	NR SA FR1-FR1 Event triggered reporting without gap under non-DRX for Satellite Access	5493

14.5.2.8	NR SA FR1-FR1 Event triggered reporting without gap under DRX for Satellite Access	5496

14.5.3	L1-RSRP measurement for beam reporting for SAN	5501

14.5.3.0	Minimum conformance requirements	5501

14.5.3.0.1	Minimum conformance requirements for SSB based L1-RSRP measurement for beam reporting	5501

14.5.3.0.2	Minimum conformance requirements for CSI-RS based L1-RSRP measurement for beam reporting	5502

14.5.3.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for Satellite Access	5504

14.5.3.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for Satellite Access	5508

14.5.3.3	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for Satellite Access	5511

14.5.3.4	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for Satellite Access	5514

14.6	Measurement Performance Requirements	5517

14.6.1	SS-RSRP for SAN	5517

14.6.1.0	Minimum conformance requirements	5517

14.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements for FR1 SAN	5517

14.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements for FR1 SAN	5518

14.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements for FR1 SAN	5519

14.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements for FR1 SAN	5519

14.6.1.1	NR SA FR1 SS-RSRP measurement accuracy for Satellite Access	5520

14.6.1.2	NR SA FR1-FR1 SS-RSRP measurement accuracy for Satellite Access	5524

14.6.2	SS-RSRQ for SAN	5528

14.6.2.0	Minimum conformance requirements	5528

14.6.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements for FR1 SAN	5528

14.6.2.0.2	Inter-frequency absolute SS-RSRQ measurement accuracy requirements for FR1 SAN	5529

14.6.2.0.3	Inter-frequency relative SS-RSRQ measurement accuracy requirements for FR1 SAN	5529

14.6.2.1	NR SA FR1 SS-RSRQ measurement accuracy for Satellite Access	5530

14.6.2.2	NR SA FR1-FR1 SS-RSRQ measurement accuracy for Satellite Access	5535

14.6.3	SS-SINR Requirements for SAN	5539

14.6.3.0	Minimum conformance requirements	5539

14.6.3.0.1	Intra-Frequency SS-SINR accuracy requirements in FR1	5539

14.6.3.0.2	Inter-Frequency SS-SINR accuracy requirements in FR1	5540

14.6.3.1	NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access	5541

14.6.3.2	NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access	5545

14.6.4	L1-RSRP Beam Reporting Measurement Requirements for SAN	5549

14.6.4.0	Minimum conformance requirements	5549

14.6.4.0.1	Minimum conformance requirements for SSB based L1-RSRP Reporting	5549

14.6.4.0.2	Minimum conformance requirements for CSI-RS based L1-RSRP Reporting	5551

14.6.4.1	NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access	5553

14.6.4.2	NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access	5557

15	FFS	5561

16	NR standalone tests in FR1 for RedCap	5562

16.0	General	5562

16.1	RRC_IDLE state mobility	5562

16.1.1	NR cell re-selection for RedCap	5562

16.1.1.0	Minimum conformance requirements	5562

16.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	5562

16.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	5564

16.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	5568

16.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	5571

16.1.1.1	NR SA FR1 Cell reselection for 1 Rx UE	5574

16.1.1.2	NR SA FR1 Cell reselection for 2 Rx UE	5582

16.1.1.3	NR SA FR1-FR1 Cell reselection for 1 Rx UE	5587

16.1.1.4	NR SA FR1-FR1 Cell reselection for 2 Rx UE	5592

16.1.1.5	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5596

16.1.1.6	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5604

16.1.1.7	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5610

16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5618

16.1.2	NR – E-UTRA cell re-selection for RedCap	5626

16.1.2.0	Minimum conformance requirements	5626

16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx	5626

16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx	5628

16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for UE fulfilling stationary relaxed measurement criterion	5628

16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1 Rx UE	5630

16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2 Rx UE	5636

16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1 Rx UE	5640

16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2 Rx UE	5646

16.1.2.5	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5650

16.1.2.6	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5656

16.2	RRC_INACTIVE state mobility for RedCap	5660

16.2.1	Configured Grant based Small Data Transmissions for RedCap	5660

16.2.1.0	Minimum conformance requirements	5660

16.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	5660

16.2.1.1	NR SA FR1 CG-SDT for 1 Rx UE	5662

16.2.1.2	NR SA FR1 CG-SDT for 2 Rx UE	5666

16.3	RRC_CONNECTED state mobility for RedCap	5669

16.3.1	Handover for RedCap	5669

16.3.1.0	Minimum conformance requirements	5669

16.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	5669

16.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	5669

16.3.1.1	NR SA FR1 handover with known target cell for 1 Rx UE	5670

16.3.1.2	NR SA FR1 handover with known target cell for 2 Rx UE	5673

16.3.1.3	NR SA FR1 handover with unknown target cell for 1 Rx UE	5677

16.3.1.4	NR SA FR1 handover with unknown target cell for 2 Rx UE	5683

16.3.1.5	NR SA FR1-FR1 handover with unknown target cell for 1 Rx UE	5689

16.3.1.6	NR SA FR1-FR1 handover with unknown target cell for 2 Rx UE	5695

16.3.1.7	NR SA FR1 - E-UTRA handover with known target cell for 1 Rx UE	5702

16.3.1.8	NR SA FR1 - E-UTRA handover with known target cell for 2 Rx UE	5708

16.3.1.9	NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE	5715

16.3.1.10	NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE	5719

16.3.2	RRC connection mobility control for RedCap	5724

16.3.2.1	RRC re-establishment for RedCap	5724

16.3.2.1.0	Minimum conformance requirements	5724

16.3.2.1.1	NR SA FR1 RRC Re-establishment for 1 Rx UE	5724

16.3.2.1.2	NR SA FR1 RRC Re-establishment for 2 Rx UE	5727

16.3.2.1.3	NR SA FR1-FR1 RRC Re-establishment for 1 Rx UE	5730

16.3.2.1.4	NR SA FR1-FR1 RRC Re-establishment for 2 Rx UE	5733

16.3.2.1.5	NR SA FR1 RRC Re-establishment for 1 Rx UE without serving cell timing	5736

16.3.2.1.6	NR SA FR1 RRC Re-establishment for 2 Rx UE without serving cell timing	5739

16.3.2.2	Random access for RedCap	5742

16.3.2.2.0	Minimum conformance requirements	5742

16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5743

16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5746

16.3.2.2.3	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5749

16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	5752

16.3.2.2.5	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5755

16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5758

16.3.2.2.7	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5761

16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5764

16.3.2.3	RRC connection release with redirection for RedCap	5767

16.3.2.3.0	Minimum conformance requirements	5767

16.3.2.3.1	NR SA FR1-FR1 RRC connection release with redirection for 1 Rx UE	5767

16.3.2.3.2	NR SA FR1-FR1 RRC connection release with redirection for 2 Rx UE	5771

16.3.2.3.3	NR SA FR1 - E-UTRA RRC connection release with redirection for 1 Rx UE	5774

16.3.2.3.4	NR SA FR1 - E-UTRA RRC connection release with redirection for 2 Rx UE	5780

16.4	Timing for RedCap	5786

16.4.1	UE transmit timing for RedCap	5786

16.4.1.0	Minimum conformance requirements	5786

16.4.1.1	NR SA FR1 NR UE Transmit Timing accuracy for 1 Rx UE	5787

16.4.1.2	NR SA FR1 NR UE Transmit Timing accuracy for 2 Rx UE	5791

16.4.2	5795

16.4.3	Timing advance for RedCap	5795

16.4.3.0	Minimum conformance requirement	5795

16.4.3.1	NR SA FR1 timing advance adjustment accuracy for 1 Rx UE	5795

16.4.3.2	NR SA FR1 timing advance adjustment accuracy for 2 Rx UE	5798

16.5	Signalling characteristics for RedCap	5802

16.5.1	Radio link monitoring for RedCap	5802

16.5.1.0	Minimum requirements	5802

16.5.1.0.1	General	5802

16.5.1.0.2	Minimum conformance requirements for SSB-based RLM	5806

16.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	5807

16.5.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5808

16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5813

16.5.1.3	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5816

16.5.1.4	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5821

16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5826

16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5831

16.5.1.7	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5836

16.5.1.8	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5841

16.5.1.9	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5846

16.5.1.10	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5849

16.5.1.11	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5852

16.5.1.12	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5855

16.5.1.13	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5859

16.5.1.14	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5862

16.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5865

16.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5868

16.5.2	Beam Failure Detection and Link recovery procedures for RedCap	5871

16.5.2.0	Minimum requirements	5871

16.5.2.0.1	General	5871

16.5.2.0.2	Minimum conformance requirements for SSB-based BFD and CBD	5873

16.5.2.0.3	Minimum conformance requirements for CSI-RS based RLM	5875

16.5.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5878

16.5.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5882

16.5.2.3	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5887

16.5.2.4	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5893

16.5.2.5	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5898

16.5.2.6	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5903

16.5.2.7	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5906

16.5.2.8	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5910

16.5.3	Active BWP switch delay for RedCap	5915

16.5.3.1	DCI-based and timer-based active BWP switch for RedCap	5915

16.5.3.1.0	Minimum conformance requirements	5915

16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE	5917

16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE	5922

16.5.3.2	RRC-based active BWP switch for RedCap	5927

16.5.3.2.0	Minimum conformance requirements	5927

16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE	5928

16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE	5933

16.5.4	UE specific CBW change for RedCap	5938

16.5.4.0	Minimum conformance requirements	5938

16.5.4.0.1	Minimum conformance requirements for UE specific CBW change	5938

16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE	5939

16.5.4.2	NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE	5941

16.6	Measurement procedures for RedCap	5945

16.6.1	Intra-frequency measurements for RedCap	5945

16.6.1.0	Minimum conformance requirements	5945

16.6.1.0.1	General	5945

16.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	5946

16.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	5948

16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE	5949

16.6.1.2	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE	5955

16.6.1.3	NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE	5961

16.6.1.4	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE	5964

16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5967

16.6.1.6	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5971

16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5974

16.6.1.8	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5977

16.6.1.9	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5979

16.6.1.10	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5985

16.6.1.11	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5990

16.6.1.12	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5993

16.6.2	Inter-frequency measurements for RedCap	5996

16.6.2.0	Minimum conformance requirements	5996

16.6.2.0.1	Minimum conformance requirements for inter-frequency event-triggered measurement with measurement gaps	5996

16.6.2.0.2	Minimum conformance requirements for inter-frequency event-triggered measurements without measurement gaps	5998

16.6.2.1	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	6001

16.6.2.2	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	6007

16.6.2.3	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	6013

16.6.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	6018

16.6.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	6022

16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	6026

16.6.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	6030

16.6.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	6034

16.6.2.9	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	6039

16.6.2.10	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	6045

16.6.2.11	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	6050

16.6.2.12	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	6055

16.6.3	Inter-RAT measurements for RedCap	6060

16.6.3.0	Minimum conformance requirements	6060

16.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	6060

16.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	6063

16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX for 1 Rx UE	6067

16.6.3.2	NR - E-UTRA event-triggered reporting in non-DRX for 2 Rx UE	6072

16.6.3.3	NR - E-UTRA event-triggered reporting in DRX for 1 Rx UE	6077

16.6.3.4	NR - E-UTRA event-triggered reporting in DRX for 2 Rx UE	6083

16.6.4	L1-RSRP measurement for beam reporting for RedCap	6089

16.6.4.0	Minimum conformance requirements	6089

16.6.4.0.1	General	6089

16.6.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	6094

16.6.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	6097

16.6.4.3	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	6100

16.6.4.4	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	6104

16.6.4.5	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	6107

16.6.4.6	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	6110

16.6.4.7	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	6113

16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	6116

16.6.5	NR measurements with autonomous gaps for RedCap	6119

16.6.5.0	Minimum conformance requirements	6119

16.6.5.0.1	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	6119

16.6.5.0.2	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	6121

16.6.5.0.3	Minimum conformance requirements for CGI identification of an E-UTRA cell with autonomous gaps	6121

16.6.5.1	NR SA FR1 CGI identification of NR neighbour cell for 1 Rx UE	6122

16.6.5.2	NR SA FR1 CGI identification of NR neighbour cell for 2 Rx UE	6125

16.6.5.3	NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 1 Rx UE	6128

16.6.5.4	NR SA FR1 - E-UTRA CGI Identification of E-UTRA cell for 2 Rx UE	6136

16.7	Measurement performance requirements for RedCap	6141

16.7.1	SS-RSRP for RedCap	6141

16.7.1.0	Minimum conformance requirements	6141

16.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	6141

16.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	6142

16.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	6143

16.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	6144

16.7.1.1	Intra-frequency measurements for RedCap for 1 Rx UE	6145

16.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	6145

16.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	6151

16.7.1.2	Intra-frequency measurements for RedCap for 2 Rx UE	6153

16.7.1.2.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	6153

16.7.1.2.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	6160

16.7.1.3	Inter-frequency measurements for RedCap for 1 Rx UE	6162

16.7.1.3.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	6162

16.7.1.3.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	6169

16.7.1.4	Inter-frequency measurements for RedCap for 2 Rx UE	6171

16.7.1.4.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	6171

16.7.1.4.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	6177

16.7.2	SS-RSRQ for RedCap	6179

16.7.2.0	Minimum conformance requirements	6179

16.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	6179

16.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	6180

16.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	6180

16.7.2.1	NR SA FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE	6181

16.7.2.2	NR SA FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE	6186

16.7.2.3	Inter-frequency measurement accuracy for 1 Rx UE	6191

16.7.2.3.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE	6191

16.7.2.3.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 1 Rx UE	6195

16.7.2.4	Inter-frequency measurement accuracy for 2 Rx UE	6197

16.7.2.4.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for 2 Rx UE	6197

16.7.2.4.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for 2 Rx UE	6201

16.7.3	SS-SINR for RedCap	6203

16.7.3.0	Minimum conformance requirements	6203

16.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	6203

16.7.3.0.2	Inter-frequency SS-SINR absolute measurement accuracy requirements	6204

16.7.3.0.3	Inter-frequency SS-SINR relative measurement accuracy requirements	6204

16.7.3.1	NR SA FR1 SS-SINR absolute measurement accuracy for 1 Rx UE	6205

16.7.3.2	NR SA FR1 SS-SINR absolute measurement accuracy for 2 Rx UE	6209

16.7.3.3	Inter-frequency measurement accuracy for 1 Rx UE	6213

16.7.3.3.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 1 Rx UE	6213

16.7.3.3.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy for 1 Rx UE	6217

16.7.3.4	Inter-frequency measurement accuracy for 2 Rx UE	6219

16.7.3.4.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy for 2 Rx UE	6219

16.7.3.4.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy for 2 Rx UE	6223

16.7.4	L1-RSRP measurement for beam reporting for RedCap	6225

16.7.4.0	Minimum conformance requirements	6225

16.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	6225

16.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	6225

16.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	6226

16.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	6227

16.7.4.1	SSB based L1-RSRP measurements for 1 Rx UE	6228

16.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 1 Rx UE	6228

16.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 1 Rx UE	6233

16.7.4.2	SSB based L1-RSRP measurements for 2 Rx UE	6235

16.7.4.2.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for 2 Rx UE	6235

16.7.4.2.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy for 2 Rx UE	6240

16.7.4.3	CSI-RS based L1-RSRP measurements for 1 Rx UE	6242

16.7.4.3.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 1 Rx UE	6242

16.7.4.3.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 1 Rx UE	6246

16.7.4.4	CSI-RS based L1-RSRP measurements for 2 Rx UE	6248

16.7.4.4.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for 2 Rx UE	6248

16.7.4.4.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for 2 Rx UE	6253

16.7.5	E-UTRAN RSRP for RedCap	6255

16.7.5.0	Minimum conformance requirements	6255

16.7.5.0.1	E-UTRAN RSRP absolute accuracy	6255

16.7.5.1	NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 1 Rx UE	6256

16.7.5.2	NR SA FR1 - E-UTRA RSRP absolute measurement accuracy for 2 Rx UE	6263

16.7.6	E-UTRAN RSRQ for RedCap	6269

16.7.6.0	Minimum conformance requirements	6269

16.7.6.0.1	E-UTRAN RSRQ absolute accuracy	6269

16.7.6.1	NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 1 Rx UE	6270

16.7.6.2	NR SA FR1 - E-UTRA RSRQ absolute measurement accuracy for 2 Rx UE	6277

17	NR standalone tests in FR2 for RedCap	6284

17.1	RRC_IDLE state mobility for RedCap	6284

17.1.1	NR cell re-selection for RedCap	6284

17.1.1.0	Minimum conformance requirements	6284

17.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	6284

17.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	6286

17.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	6289

17.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	6290

17.1.1.1	NR SA FR2 Cell reselection for 2 Rx UE	6292

17.1.1.2	NR SA FR2-FR2 Cell reselection for 2 Rx UE	6300

17.1.1.3	NR SA FR2 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	6308

17.1.1.4	NR SA FR2-FR2 Cell reselection for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	6313

17.2	RRC_INACTIVE state mobility for RedCap	6320

17.2.1	Configured Grant based Small Data Transmissions for RedCap	6320

17.2.1.0	Minimum conformance requirements	6320

17.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	6320

17.2.1.1	NR SA FR2 TA validation for CG-SDT for 2 Rx UE	6321

17.3	RRC_CONNECTED state mobility for RedCap	6324

17.3.1	Handover for RedCap	6324

17.3.1.0	Minimum conformance requirements	6324

17.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	6324

17.3.1.1	NR SA FR2 handover with unknown target cell for 2 Rx UE	6326

17.3.1.2	NR SA FR2-FR2 handover with unknown target cell for 2 Rx UE	6328

17.3.2	RRC connection mobility control for RedCap	6335

17.3.2.1	RRC re-establishment for RedCap	6335

17.3.2.1.0	Minimum conformance requirements	6335

17.3.2.1.1	NR SA FR2 Intra-frequency RRC Re-establishment in FR2	6335

17.3.2.1.2	NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2	6337

17.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	6339

17.3.2.2	Random access for RedCap	6342

17.3.2.2.0	Minimum conformance requirements	6342

17.3.2.2.0.1	Minimum conformance requirements for Contention based random access	6342

17.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access	6342

17.3.2.2.1	NR SA FR2 4-step RA type contention based random access for 2 Rx UE	6343

17.3.2.2.2	NR SA FR2 4-step RA type non-contention based random access for 2 Rx UE	6346

17.3.2.2.3	NR SA FR2 2-step RA type contention based random access for 2 Rx UE	6349

17.3.2.2.4	NR SA FR2 2-step RA type non-contention based random access for 2 Rx UE	6352

17.3.2.3	RRC Connection Release with Redirection for RedCap	6355

17.3.2.3.0	Minimum conformance requirements	6355

17.3.2.3.0.1	Minimum conformance requirements for FR2-FR2 RRC connection release with redirection	6355

17.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection for 2 Rx UE	6355

17.4	Timing for RedCap	6358

17.4.1	UE transmit timing for RedCap	6358

17.4.1.0	Minimum Conformance Requirements	6358

17.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	6358

17.4.1.1	NR SA FR2 NR UE Transmit Timing accuracy for 2 Rx UE	6359

17.4.2	UE timer accuracy for RedCap	6362

17.4.3	Timing advance for RedCap	6362

17.4.3.0	Minimum conformance requirements	6362

17.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	6362

17.4.3.1	NR SA FR2 timing advance adjustment accuracy for 2 Rx UE	6363

17.5	Signalling characteristics for RedCap	6367

17.5.1	Radio link monitoring for RedCap	6367

17.5.1.0	Minimum requirements	6367

17.5.1.0.1	General	6367

17.5.1.0.2	Minimum conformance requirements for SSB-based RLM	6370

17.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	6372

17.5.1.1	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	6374

17.5.1.2	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	6377

17.5.1.3	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	6381

17.5.1.4	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	6384

17.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	6387

17.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	6390

17.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	6394

17.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	6397

17.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions for 2 Rx UE	6401

17.5.2	Beam Failure Detection and Link recovery procedures for RedCap	6403

17.5.2.0	Minimum conformance requirements	6403

17.5.2.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	6403

17.5.2.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	6403

17.5.2.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	6404

17.5.2.1	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	6404

17.5.2.2	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	6407

17.5.2.3	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	6411

17.5.2.4	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	6415

17.5.2.5	NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	6419

17.5.3	Active BWP switch delay for RedCap	6422

17.5.3.1	DCI-based and time-based active BWP switch for RedCap	6422

17.5.3.1.0	Minimum conformance requirements for DCI-based and time-based active BWP switch	6422

17.5.3.1.1	NR SA FR2 DCI-based and Timer-based DL active BWP switch for 2 Rx UE	6423

17.5.3.2	RRC-based active BWP switch for RedCap	6431

17.5.3.2.0	Minimum conformance requirements for RRC-based active BWP switch	6431

17.5.3.2.1	NR SA FR2 RRC-based DL active BWP switch for 2 Rx UE	6432

17.5.4	Active TCI state switch delay for RedCap	6437

17.5.4.0	Minimum conformance requirements	6437

17.5.4.0.1	Known conditions for TCI state	6437

17.5.4.0.2	Minimum conformance requirements for MAC-CE based active TCI state switch	6437

17.5.4.0.3	Minimum conformance requirements for RRC based active TCI state switch	6437

17.5.4.1	MAC-CE based active TCI state switch for RedCap	6437

17.5.4.2	RRC based active TCI state switch for RedCap	6440

17.5.4.2.1	NR SA FR2 RRC-CE based active TCI state switch for a known TCI state for 2 Rx UE	6440

17.5.5	Uplink spatial relation switch delay for RedCap	6443

17.5.5.1	MAC-CE based Spatial Relation switch for RedCap	6443

17.5.5.1.0	Minimum conformance requirements MAC-CE based uplink spatial relation switch delay	6443

17.5.5.1.1	NR SA FR2 MAC-CE based spatial relation switch associated with known DL-RS for 2 Rx UE	6444

17.5.5.2	RRC based spatial relation switch for RedCap	6447

17.5.5.2.0	Minimum conformance requirements RRC based uplink spatial relation switch delay	6447

17.5.5.2.1	NR SA FR2 RRC-based spatial relation switch associated with a known DL-RS for 2 Rx UE	6448

17.5.6	UE specific CBW change for RedCap	6451

17.5.6.0	Minimum conformance requirements	6451

17.5.6.0.1	Minimum conformance requirements for UE specific CBW change	6451

17.5.6.1	NR SA FR2 UE specific CBW change on PCell for 2 Rx UE	6451

17.6	Measurement procedures for RedCap	6457

17.6.1	Intra-frequency measurements for RedCap	6457

17.6.1.0	Minimum conformance requirements	6457

17.6.1.0.1	General	6457

17.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	6458

17.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	6460

17.6.1.1	NR SA FR2 event triggered reporting without gap in non-DRX for 2 Rx UE	6461

17.6.1.2	NR SA FR2 event triggered reporting without gap in DRX for 2 Rx UE	6464

17.6.1.3	NR SA FR2 event triggered reporting with gap in non-DRX for 2 Rx UE	6466

17.6.1.4	NR SA FR2 event triggered reporting with gap in DRX for 2 Rx UE	6473

17.6.2	Inter-frequency measurements for RedCap	6476

17.6.2.0	Minimum conformance requirements	6476

17.6.2.1	NR SA FR2-FR2 event triggered reporting in non-DRX for 2 Rx UE	6478

17.6.2.2	NR SA FR2-FR2 event triggered reporting in DRX for 2 Rx UE	6482

17.6.2.3	NR SA FR2-FR2 event triggered reporting with SSB time index detection in non-DRX for 2 Rx UE	6487

17.6.2.4	NR SA FR2-FR2 event triggered reporting with SSB time index detection in DRX for 2 Rx UE	6492

17.6.3	L1-RSRP measurement for beam reporting for RedCap	6497

17.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	6497

17.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	6497

17.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	6497

17.6.3.1	NR SA FR2 SSB based L1-RSRP measurement in non-DRX for 2 Rx UE	6497

17.6.3.2	NR SA FR2 SSB based L1-RSRP measurement in DRX for 2 Rx UE	6498

17.6.3.3	NR SA FR2 CSI-RS based L1-RSRP measurement in non-DRX for 2 Rx UE	6501

17.6.3.4	NR SA FR2 CSI-RS based L1-RSRP measurement in DRX for 2 Rx UE	6503

17.6.4	NR Measurements with autonomous gaps for RedCap	6506

17.6.4.0	Minimum conformance requirements	6506

17.6.4.0.1	Minimum conformance requirements for CGI identification of an NR cell with autonomous gaps	6506

17.6.4.1	NR SA FR2-FR2 CGI identification of NR neighbour cell for 2 Rx UE	6507

17.7	Measurement performance requirements for RedCap	6513

17.7.1	SS-RSRP for RedCap	6513

17.7.1.0	Minimum conformance requirements	6513

17.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	6513

17.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	6513

17.7.1.1	NR SA FR2 SS-RSRP measurement accuracy for 2 Rx UE	6513

17.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy for 2 Rx UE	6518

17.7.2	SS-RSRQ for RedCap	6524

17.7.2.0	Minimum conformance requirements	6524

17.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	6524

17.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	6524

17.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy for 2 Rx UE	6524

17.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy for 2 Rx UE	6527

17.7.3	L1-RSRP measurement for beam reporting for RedCap	6531

17.7.3.0	Minimum conformance requirements	6531

17.7.3.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	6531

17.7.3.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	6531

17.7.3.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	6531

17.7.3.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	6531

17.7.3.1	NR SA FR2 SSB based L1-RSRP measurement accuracy for 2 Rx UE	6531

17.7.3.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy for 2 Rx UE	6535

17.7.4	SS-SINR for RedCap	6540

17.7.4.0	Minimum conformance requirements	6540

17.7.4.0.1	Intra-frequency SS-SINR measurement accuracy requirements	6540

17.7.4.0.2	Inter-frequency SS-SINR measurement accuracy requirements	6540

17.7.4.1	NR SA FR2 SS-SINR measurement accuracy for 2 Rx UE	6540

17.7.4.2	NR SA FR2-FR2 SS-SINR measurement accuracy for 2 Rx UE	6543

18	E-UTRA - NR inter-RAT with E-UTRA serving cell for RedCap	6547

18.1	RRC_IDLE state mobility for RedCap	6547

18.1.1	Inter-RAT cell re-selection for RedCap	6547

18.1.1.0	Minimum conformance requirements	6547

18.1.1.1	E-UTRA - NR SA FR1 E-UTRA cell re-selection to higher priority NR target Cell for 2 Rx UE	6550

18.2	RRC_CONNECTED state mobility for RedCap	6554

18.2.1	Handover for RedCap	6554

18.2.1.0	Minimum conformance requirements	6554

18.2.1.1	E-UTRA - NR SA FR1 handover for 2 Rx UE	6554

18.2.2.0	Minimum conformance requirements	6558

18.2.2.0.1	Redirection from E-UTRA to FR1 RedCap UE	6558

18.2.2.1	E-UTRA - NR SA FR1 RRC connection release with redirection for 2 Rx UE	6559

18.3	Measurement procedure for RedCap	6565

18.3.1	E-UTRA - NR Measurements for RedCap	6565

18.3.1.0	Minimum conformance requirements	6565

18.3.1.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	6565

18.3.1.1	E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in non-DRX	6568

18.3.1.2	E-UTRA - NR SA FR1 event triggered reporting without SSB time index detection in DRX	6572

18.3.1.3	E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in non-DRX	6576

18.3.1.4	E-UTRA - NR SA FR1 event triggered reporting with SSB time index detection in DRX	6580

18.3.1.5	E-UTRA - NR SA FR2 event triggered reporting without SSB time index detection in non-DRX	6584

18.3.1.6	E-UTRA - NR SA FR2 event triggered reporting without SSB time index detection in DRX	6586

18.3.1.7	E-UTRA - NR SA FR2 event triggered reporting with SSB time index detection in non-DRX	6589

18.3.1.8	E-UTRA - NR SA FR2 event triggered reporting with SSB time index detection in DRX	6591

19	NR standalone tests for ATG	6594

19.0	General	6594

19.0.1	Initial test environment conditions for ATG RRM tests	6594

19.1	RRC_IDLE state mobility for ATG	6595

19.1.0	Minimum conformance requirements	6595

19.1.0.1	Measurements of intra-frequency NR cells	6595

19.1.0.2	Measurements of inter-frequency NR cells	6596

19.2	RRC_CONNECTED state mobility	6598

19.2.1	Handover for ATG	6598

19.2.1.0	Minimum conformance requirements	6598

19.2.1.0.1	Minimum conformance requirements for NR FR1 – NR FR1 handover	6598

19.2.1.1	Intra-frequency handover from FR1 to FR1; known target cell	6599

19.2.1.2	Inter-frequency handover from FR1 to FR1; unknown target cell	6605

19.2.2	Conditional Handover for ATG	6610

19.2.2.0	Minimum conformance requirements	6610

19.2.2.0.1	Minimum conformance requirements for NR FR1 – NR FR1 conditional handover	6610

19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1	6612

19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1	6621

19.2.3	RRC Connection Mobility Control for ATG	6629

19.2.3.1	SA: RRC re-establishment for ATG	6629

19.2.3.1.0	Minimum conformance requirements	6629

19.2.3.1.1	Intra-frequency RRC Re-establishment in FR1 for ATG	6631

19.2.3.1.2	Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG	6636

19.2.3.2	Random Access for ATG	6641

19.2.3.2.0	Minimum conformance requirements	6641

19.2.3.2.1	NR SA FR1 4-step RA type contention based random access test for ATG	6641

19.2.3.2.2	NR SA FR1 4-step RA type non-contention based random access test for ATG	6643

19.2.3.2.3	NR SA FR1 2-step RA type contention based random access test for ATG	6644

19.2.3.2.4	NR SA FR1 2-step RA type non-contention based random access test for ATG	6646

19.2.3.3	SA: RRC Connection Release with Redirection for ATG	6647

19.2.3.3.0	Minimum conformance requirements	6647

19.2.3.3.1	NR SA FR1-FR1 RRC connection release with Redirection for ATG	6648

19.3	Timing for ATG	6654

19.3.1	UE transmit timing	6654

19.3.1.0	Minimum conformance requirements	6654

19.3.1.1	NR SA FR1 UE Transmit Timing Test for ATG	6656

19.3.2	UE timer accuracy for ATG	6659

19.3.3	Timing advance for ATG	6659

19.3.3.0	Minimum conformance requirements	6659

19.3.3.1	NR SA FR1 timing advance adjustment accuracy for ATG	6660

19.4	Signalling characteristics for ATG	6661

19.4.1	Radio link monitoring for ATG	6661

19.4.1.0	Minimum conformance requirements	6661

19.4.1.0.1	Minimum conformance requirements for SSB based RLM	6661

19.4.1.0.2	Minimum conformance requirements for CSI-RS based RLM	6662

19.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6663

19.4.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6667

19.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	6671

19.4.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in non-DRX mode	6675

19.4.2	Beam Failure Detection and Link recovery procedures for ATG	6679

19.4.2.0	Minimum conformance requirements	6679

19.4.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for ATG	6685

19.4.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for ATG	6689

19.4.3	Active BWP switch for ATG	6694

19.4.3.1	DCI-based and timer-based active BWP switch for ATG	6694

19.4.3.1.0	Minimum conformance requirements	6694

19.4.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for ATG	6695

19.4.3.2	RRC-based Active BWP Switch for ATG	6702

19.4.3.2.0	Minimum conformance requirements	6702

19.4.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for ATG	6703

19.4.4	UE specific CBW change for ATG	6709

19.4.4.0	Minimum conformance requirements	6709

19.4.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for ATG	6709

19.4.5	Pathloss reference signal switching delay for ATG	6714

19.4.5.0	Minimum conformance requirements	6714

19.4.5.1	NR SA FR1 MAC-CE based pathloss reference signal switch delay for ATG	6715

NOTE:	The UE shall be given proper uplink transmission grant during T2 and T3.19.5 Measurement procedure for ATG	6720

19.5.1	Intra-frequency measurements	6720

19.5.1.0	Minimum conformance requirements	6720

19.5.1.0.1	Minimum conformance requirements  for Intra-frequency measurements without measurement gaps	6720

19.5.1.0.2	Minimum conformance requirements  for Intra-frequency measurements with measurement gaps	6722

19.5.1.1	SA FR1 event triggered reporting tests without gap without SSB index reading under non-DRX	6724

19.5.1.2	SA FR1 event triggered reporting tests with per-UE gaps under non-DRX	6725

19.5.1.3	SA FR1 event triggered reporting tests without gap under non-DRX with SSB index reading	6727

19.5.1.4	SA FR1 event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	6729

19.5.2	Inter-frequency measurements	6730

19.5.2.0	Minimum conformance requirements	6730

19.5.2.0.1	Minimum conformance requirements  for Inter-frequency measurements with measurement gaps	6730

19.5.2.0.2	Minimum conformance requirements  for Inter-frequency measurements without gaps	6732

19.5.2.1	SA FR1-FR1 event triggered reporting tests without SSB time index detection when DRX is used	6734

19.5.2.2	SA FR1-FR1 event triggered reporting tests with SSB time index detection when DRX is not used	6736

19.5.2.3	SA FR1-FR1 event triggered reporting tests without gap when DRX is not used	6738

19.5.3	L1-RSRP measurement for beam reporting for ATG	6740

19.5.3.0	Minimum conformance requirements	6740

19.5.3.1	NR SA FR1 SSB based L1-RSRP measurement in non-DRX for ATG	6740

19.5.3.2	NR SA FR1 CSI-RS based L1-RSRP measurement in non-DRX for ATG	6741

19.5.4	L1-SINR measurement for beam reporting for ATG	6743

19.5.4.0	Minimum conformance requirements	6743

19.5.4.1	NR SA FR1 L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured in non-DRX for ATG	6743

19.5.4.2	NR SA FR1 L1-SINR measurement with SSB based CMR and dedicated IMR in non-DRX for ATG	6745

19.5.4.3	NR SA FR1 L1-SINR measurement with CSI-RS based CMR and dedicated IMR in non-DRX for ATG	6746

19.5.5	NR measurements with autonomous gaps for ATG	6748

19.5.5.0	Minimum conformance requirements	6748

19.5.5.1	NR SA FR1 CGI identification of NR neighbor cell for ATG	6748

19.6	Measurement Performance requirements for ATG	6749

19.6.0	General	6749

19.6.0.1 Principles of testing for ATG UE	6749

19.6.1	SS-RSRP for ATG	6750

19.6.1.0	Minimum conformance requirements	6750

19.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	6750

19.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	6750

19.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	6750

19.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	6750

19.6.1.1	Intra-frequency measurements for ATG	6750

19.6.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for ATG	6750

19.6.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for ATG	6751

19.6.1.2	Inter-frequency measurements for ATG	6753

19.6.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG	6753

19.6.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG	6755

19.6.2	SS-RSRQ for ATG	6756

19.6.2.0	Minimum conformance requirements	6756

19.6.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	6756

19.6.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	6756

19.6.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	6756

19.6.2.1	NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG	6756

19.6.2.2	Inter-frequency SS-RSRQ measurements for ATG	6758

19.6.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG	6758

19.6.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG	6759

19.6.3	SS-SINR for ATG	6761

19.6.3.0	Minimum conformance requirements	6761

19.6.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	6761

19.6.3.0.2	Inter-frequency SS-SINR absolute measurement accuracy requirements	6761

19.6.3.0.3	Inter-frequency SS-SINR relative measurement accuracy requirements	6761

19.6.3.1	NR SA FR1 SS-SINR absolute measurement accuracy for ATG	6761

19.6.3.2	Inter-frequency SS-SINR measurements for ATG	6763

19.6.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG	6763

19.6.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG	6764

19.6.4	L1-RSRP measurement for beam reporting for ATG	6766

19.6.4.0	Minimum conformance requirements	6766

19.6.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	6766

19.6.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	6766

19.6.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	6766

19.6.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	6766

19.6.4.1	SSB based L1-RSRP measurements for ATG	6766

19.6.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG	6766

19.6.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG	6767

19.6.4.2	CSI-RS based L1-RSRP measurements for ATG	6769

19.6.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG	6769

19.6.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG	6770

19.6.5	L1-SINR measurement for beam reporting for ATG	6772

19.6.5.0	Minimum conformance requirements	6772

19.6.5.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured	6772

19.6.5.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	6772

19.6.5.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	6772

19.6.5.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR for ATG	6772

19.6.5.1.1	NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG	6772

19.6.5.1.2	NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG	6773

19.6.5.2	NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG	6775

19.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR for ATG	6776

19.6.5.3.1	NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG	6776

19.6.5.3.2	NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG	6777

19.6.6	CSI-RSRP for ATG	6779

19.6.6.0	Minimum conformance requirements	6779

19.6.6.0.1	Intra-frequency absolute CSI-RSRP measurement accuracy for ATG	6779

19.6.6.0.2	Intra-frequency relative CSI-RSRP measurement accuracy for ATG	6779

19.6.6.0.3	Inter-frequency absolute CSI-RSRP measurement accuracy for ATG	6779

19.6.6.0.4	Inter-frequency relative CSI-RSRP measurement accuracy for ATG	6779

19.6.6.1	Intra-frequency measurements for ATG	6779

19.6.6.1.1	NR SA FR1 CSI-RSRP absolute measurement accuracy for ATG	6779

19.6.6.1.2	NR SA FR1 CSI-RSRP relative measurement accuracy for ATG	6785

19.6.6.2	Inter-frequency measurements for ATG	6787

19.6.6.2.1	NR SA FR1-FR1 CSI-RSRP absolute measurement accuracy for ATG	6787

19.6.6.2.2	NR SA FR1-FR1 CSI-RSRP relative measurement accuracy	6792

19.6.7	CSI-RSRQ for ATG	6794

19.6.7.0	Minimum conformance requirements	6794

19.6.7.0.1	Intra-frequency CSI-RSRQ absolute accuracy requirements for ATG	6794

19.6.7.0.2	Inter-frequency CSI-RSRQ absolute accuracy requirements for ATG	6794

19.6.7.0.3	Inter-frequency CSI-RSRQ relative accuracy requirements for ATG	6794

19.6.7.1	NR SA FR1 CSI-RSRQ absolute measurement accuracy for ATG	6794

19.6.7.2	Inter-frequency measurements for ATG	6798

19.6.7.2.1	NR SA FR1-FR1 CSI-RSRQ absolute measurement accuracy for ATG	6798

19.6.7.2.2	NR SA FR1-FR1 CSI-RSRQ relative measurement accuracy for ATG	6802

19.6.8	CSI-SINR for ATG	6804

19.6.8.0	Minimum conformance requirements	6804

19.6.8.0.1	Intra-frequency CSI-SINR absolute accuracy requirements for ATG	6804

19.6.8.0.2	Inter-frequency CSI-SINR absolute accuracy requirements for ATG	6804

19.6.8.0.3	Inter-frequency CSI-SINR relative accuracy requirements for ATG	6804

19.6.8.1	NR SA FR1 CSI-SINR absolute measurement accuracy for ATG	6804

19.6.8.2	Inter-frequency measurements for ATG	6809

19.6.8.2.1	NR SA FR1-FR1 CSI-SINR absolute measurement accuracy for ATG	6809

19.6.8.2.2	NR SA FR1-FR1 CSI-SINR relative measurement accuracy for ATG	6813

Annex A (normative):	RRM test configurations	6816

A.1	Reference measurement channels	6816

A.1.0	General	6816

A.1.1	PDSCH	6816

A.1.1.1	FDD	6816

A.1.1.2	TDD	6817

A.1.2	CORESET for RMSI scheduling	6820

A.1.2.1	FDD	6820

A.1.2.2	TDD	6821

A.1.3	CORESET for RMC scheduling	6824

A.1.3.1	FDD	6824

A.1.3.2	TDD	6825

A.1.4	CSI-RS	6829

A.1.4.1	FDD	6829

A.1.4.2	TDD	6832

A.1.4A	CSI-RS for tracking	6837

A.1.4A.1	FR1	6837

A.1.4A.1.1	FDD	6837

A.1.4A.1.2	TDD	6839

A.1.4A.2	FR2	6842

A.1.4A.2.1	TDD	6842

A.1.4B	CSI-IM configurations	6843

A.1.4B.1	FDD	6843

A.1.4B.2	TDD	6843

A.1.5	TDD UL/DL configuration	6845

A.1.6	PUSCH	6846

A.1A	Reference measurement channels under CCA	6847

A.1A.0	General	6847

A.1A.1	PDSCH	6847

A.1A.1.1	FDD	6847

A.1A.2	CORESET for RMSI scheduling	6848

A.1A.2.1	TDD	6848

A.1A.3	CORESET for RMC scheduling	6849

A.1A.3.1	TDD	6849

A.1A.4	TDD UL/DL configuration	6850

A.1A.4.1	TDD	6850

A.1A.5	RMC burst transmission model	6850

A.2	Reference OCNG configuration	6850

A.2.1	Generic OFDMA channel noise generator (OCNG)	6850

A.3	Reference SSB configuration	6853

A.3.1	SSB configuration for FR1	6853

A.3.2	SSB configuration for FR2	6853

A.3A	Reference SSB configuration under CCA	6854

A.3A.1	SSB configuration under CCA for FR1	6854

A.3A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	6854

A.3A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	6854

A.3A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	6855

A.3A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	6855

A.3B	Reference SSB configuration for RedCap	6856

A.3B.1	SSB configuration for FR1	6856

A.3B.2	SSB configuration for FR2	6857

A.4	Reference SMTC configuration	6857

A.4A	Reference SMTC configuration for RedCap	6857

A.5	Reference DRX configurations	6858

A.6	EN-DC test setup	6858

A.6.1	E-UTRA serving cell parameters	6858

A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1	6858

A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2	6860

A.6A	NR FR1-FR2 test setup	6862

A.6B	EN-DC test setup with unlicensed bands	6862

A.6B.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	6862

A.7	Reference PRACH configurations	6863

A.7.1	PRACH configurations for FR1	6863

A.7.1A	PRACH configurations for FR1 under CCA	6865

A.7.1A.1	FR1 PRACH configuration 1 under CCA	6865

A.7.1A.2	FR1 PRACH configuration 2 under CCA	6865

A.7.2	PRACH configurations for FR2	6866

A.7A	Reference MsgA configurations	6868

A.7A.1	MsgA configurations for FR1	6868

A.7A.1A	MsgA configurations for FR1 under CCA	6870

A.7A.1A.1	FR1 MsgA configuration 1 under CCA	6870

A.7A.1A.2	FR1 MsgA configuration 2 under CCA	6871

A.7A.2	MsgA configurations for FR2	6873

A.8	Reference BWP configurations	6875

A.8.1	Downlink BWP configurations	6875

A.8.2	Uplink BWP configurations	6875

A.8A	Reference BWP configurations for RedCap	6876

A.8A.1	Downlink BWP configurations	6876

A.8A.2	Uplink BWP configurations	6877

A.9	Angle of Arrival (AoA) for FR2 RRM test cases	6877

A.9.1	Setup 1: Single AoA in Rx beam peak direction	6877

A.9.2	Setup 2: Single AoA in non Rx beam peak direction	6877

A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	6877

A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	6877

A.9.3	Setup 3: 2 AoAs	6877

A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	6878

A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	6878

A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	6878

A.9.7	Setup 7: 3 AoAs	6879

A.10	TCI State Configuration	6879

A.10.1	Introduction	6879

A.10.2	TCI states	6879

A.10A	Unified TCI State Configuration	6879

A.10A.1	Introduction	6879

A.10A.2	DLorJoint TCI states	6880

A.10A.3	UL TCI states	6881

A.10B	LTM Candidate TCI State Configuration	6881

A.10B.1	Introduction	6881

A.10B.2	LTM candidate DLorJoint TCI states	6882

A.10B.3	LTM candidate UL TCI states	6882

A.11	NR sidelink communication	6882

A.11.1	Introduction	6882

A.11.2	Reference resource pool configurations for NR Sidelink Communication	6883

A.11.3	Reference measurement channels for NR Sidelink Communication	6886

A.12	Discovery Burst Transmission Window configuration under CCA	6887

A.12.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	6887

A.13	Channel bandwidth (CBW) configurations	6887

A.13.1	DL UE specific CBW	6887

A.13.2	UL UE specific CBW	6887

A.14	Satellite access configuration	6888

A.14.1	Satellite specific configuration for serving cell	6888

A.14.2	Satellite specific configuration for neighbour cell	6888

A.15	SRS Configurations	6889

A.15.1	SRS Configuration for SCS=15kHz	6889

A.15.2	SRS Configuration for SCS=30kHz	6890

A.15.3	SRS Configuration for SCS=30kHz	6891

A.16	CSI-RS configurations for RRM	6891

A.16.1	FDD	6891

A.16.2	TDD	6892

A.17	Reference Cell DTX configurations	6894

A.17.1	Cell DTX Configuration 1: Cell DTX cycle = 160 ms and TAT = Infinity	6894

A.18	Spatial Relation Configuration	6894

A.18.1	Introduction	6894

A.18.2	Spatial Relation	6894

A.19	Test Cases involving E-UTRA/FR1 and FR2 carriers	6894

A.19.1	Introduction	6894

A.19.2	Principle of Testing in EN-DC	6894

A.19.3	Principle of Testing in SA	6895

A.19.4	Principle of Testing in E-UTRA	6895

A.19.5	Principle of Testing in NR-DC	6896

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	6896

B.1	Conditions for NR RRC_IDLE state mobility	6896

B.1.1	Introduction	6896

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	6896

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	6898

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	6899

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	6900

B.1.8	Conditions for measurements on NR intra-frequency cells for cell re-selection under CCA	6900

B.1.9	Conditions for measurements on NR inter-frequency cells for cell re-selection under CCA	6900

B.2	Conditions for NR RRC_CONNECTED state	6900

B.2.1	Introduction	6900

B.2.2	Conditions for NR intra-frequency measurements	6901

B.2.3	Conditions for NR inter-frequency measurements	6901

B.2.4	Conditions for NR L1-RSRP reporting	6902

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6902

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6903

B.2.5	Conditions for RRC connection release with redirection to NR	6904

B.2.6	Conditions for UE transmit timing	6905

B.2.6.1	Conditions for SSB based UE transmit timing	6905

B.2.7	Conditions for NR intra-frequency measurements for RedCap	6906

B.2.8	Conditions for NR inter-frequency measurements for RedCap	6908

B.2.9	Conditions for NR intra-frequency measurements under CCA	6910

B.2.10	Conditions for NR inter-frequency measurements under CCA	6911

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6911

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6911

B.2.12 to B.2.16	6911

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6911

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6912

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6912

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6912

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6912

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6912

B.3	RRM requirement exceptions	6913

B.3.1	Introduction	6913

B.3.2	Receiver sensitivity relaxation for CA	6913

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6913

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6913

B.3.2.2.1	Inter-band carrier aggregation	6913

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6913

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6913

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6914

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6914

B.3.2.4.1	Intra-band contiguous carrier aggregation	6914

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6914

B.3.3	Receiver sensitivity relaxation for DC	6914

B.3.4	Receiver sensitivity relaxation for SUL	6914

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6914

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6914

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6914

B.4	Conditions for NR sidelink	6915

B.4.1	Test parameters for GNSS signals	6915

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6915

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6915

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6916

B.5	High level test procedure for SAN RRM tests	6916

Annex C (normative):	Downlink physical channels and propagation conditions	6917

C.1	Downlink physical channels	6917

C.1.1	General	6917

C.1.2	Default downlink signal levels	6917

C.1.3	Default connection setup	6918

C.2	Propagation conditions	6918

C.2.0	General	6918

C.2.1	No interference	6919

C.2.2	Static propagation conditions	6919

C.2.2.0	General	6919

C.2.2.1	UE receiver with 2Rx antenna connectors	6919

C.2.2.2	UE receiver with 4Rx antenna connectors	6919

C.2.3	Multi-path fading propagation conditions	6920

Annex D (normative):	Deviations from standard test configuration	6921

D.1	Test cases with different numerologies	6921

D.2	EN-DC test cases with different EN-DC configurations	6921

D.2.0	General	6921

D.2.1	Principle of testing	6921

D.3	Carrier aggregation test cases with different CA configurations	6921

D.3.0	General	6921

D.3.1	Principle of testing	6921

D.4	Antenna connection for 4Rx capable UEs	6921

D.4.0	General	6921

D.4.1	Principle of testing	6922

D.4.1.1	Single carrier tests	6922

D.4.1.2	Carrier aggregation tests	6923

D.4.1.3	EN-DC tests	6923

D.4.2	Antenna connection	6923

D.4.2.1	Antenna connection for NR bands where 2Rx is supported	6923

D.4.2.2	Antenna connection for NR bands where only 4Rx is supported	6923

D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported	6923

D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported	6924

D.4A	Antenna configurations with unlicensed bands for 4Rx capable UEs	6924

D.4A.1	Introduction	6924

D.4A.2	Principle of Testing	6924

D.4A.2.1	Single Carrier Tests	6924

D.4A.2.2	Carrier Aggregation Tests	6925

D.4A.2.3	EN-DC Tests	6925

D.4A.2.4	Antenna connection for bands where 2RX is supported	6925

D.4A.2.5	Antenna connection for bands where 4RX is supported	6925

D.4A.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported	6925

D.4A.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported	6925

D.4B	Antenna connection for 8Rx capable UEs	6926

D.4B.1	Introduction	6926

D.4B.1	Principle of testing	6926

D.4B.1.1	Single carrier tests	6926

D.4B.1.2	Carrier aggregation tests	6926

D.4B.1.3	EN-DC tests	6926

D.4B.2	Antenna connection	6927

D.4B.2.1	Antenna connection for NR bands where 2Rx is supported	6927

D.4B.2.2	Antenna connection for NR bands where 4Rx is supported	6927

D.4B.2.3	Antenna connection for bands where 8Rx is supported	6927

D.4B.2.4	Antenna connection for E-UTRA bands where 2Rx is supported	6927

D.4B.2.5	Antenna connection for E-UTRA bands where 4Rx is supported	6927

D.4B.2.6	Antenna connection for E-UTRA bands where 8Rx is supported	6927

D.5	Test Cases with Different Channel Bandwidths	6927

D.5.1	Test Cases with Different E-UTRA Channel Bandwidths	6927

D.5.1.1	Introduction	6927

D.5.1.2	Principle of testing	6927

D.6	Test Cases for Synchronous and Asynchronous DC Operations	6928

D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	6928

D.6.1.1	Introduction	6928

D.6.1.2	Principle of Testing	6928

D.7	CCA model	6928

D.7.1	Introduction	6928

D.7.2	CCA model for operation on a carrier frequency with CCA in FR1	6928

D.7.2.1	DL CCA model	6928

D.7.2.2	UL CCA model	6929

D.7.3	Void	6930

D.8	Test Cases in SA and EN-DC Operations	6930

D.8.0	General	6930

D.8.1	Principle of testing	6930

Annex E (normative):	Cell configuration mapping	6931

E.0	General	6931

E.1	Test frequency selection	6931

E.1.0	General	6931

E.1.1	E-UTRA PCell for EN-DC test cases	6931

E.1.2	Test cases with one NR cell	6931

E.1.3	Test cases with more than one NR cell	6931

E.1.3.1	Intra-frequency test cases	6931

E.1.3.2	Inter-frequency test cases	6931

E.1.4	Carrier aggregation test cases	6932

E.1.4.1	Inter-band carrier aggregation	6932

E.1.4.2	Intra-band contiguous carrier aggregation	6932

E.1.4.3	Intra-band non-contiguous carrier aggregation	6932

E.1.5	E-UTRA – NR inter RAT test cases	6932

E.1.6	Intra-band EN-DC test cases	6932

E.1.6.1	Intra-band non-contiguous EN-DC	6932

E.1.6.1.1	Inter-frequency neighbour cell	6932

E.1.6.2	Intra-band contiguous EN-DC	6932

E.1.6.1.1	E-UTRA PCell	6932

E.1.7	NR sidelink test cases	6933

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4	6933

E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5	6937

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6	6941

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7	6949

E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8	6955

E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9	6956

E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10	6957

E.9	Cell configuration mapping for SA FR1 Shared Spectrum test cases in Chapter 11	6960

E.12	Cell configuration mapping for SA Satellite Access test cases in Chapter 14	6966

E.13	6971

E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16	6971

E.15	Cell configuration mapping for SA FR2 test cases for RedCap in Chapter 17	6977

E.16	Cell configuration mapping for E-UTRAN – SA test cases for RedCap in Chapter 18	6979

E.17	Cell configuration mapping for SA FR1 test cases for ATG in Chapter 19	6980

Annex F (normative):	Measurement uncertainties and test tolerances	6984

F.1	Measurement uncertainties and test tolerances for FR1 and FR2	6984

F.1.1	Acceptable uncertainty of test system (normative)	6984

F.1.1.1	Measurement of test environments	6984

F.1.1.2	Measurement of RRM requirements	6984

F.1.2	Interpretation of measurement results (normative)	7080

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)	7080

F.1.3.1	Measurement of test environments	7080

F.1.3.2	Measurement of RRM requirements	7080

Annex G (normative):	Statistical testing	7302

G.1	General	7302

G.2	Statistical testing of delay and UE measurement performance in RRM tests	7302

G.2.1	General	7302

G.2.2	Design of the test	7302

G.2.3	Numerical definition of the pass fail limits	7302

G.2.4	Pass fail decision rules	7303

G.2.5	Void	7304

G.2.6	Test conditions for delay tests and UE measurement performance	7304

G.2.7	Additional considerations for tests in bands subject to CCA failure	7304

G.3	Statistical testing of NR sidelink CBR measurement tests	7305

G.3.1	General	7305

G.3.2	Design of the test	7305

G.3.3	Numerical definition of the pass fail limits	7306

G.3.4	Pass fail decision rules	7306

G.4	Theory to derive the numbers in Table G.2.3-1 (informative)	7307

G.5	Theory to derive the numbers in Table G.3.3-1 (informative)	7307

G.5.1	Algorithm to calculate pass and fail probabilities	7307

G.5.2	Method of designing pass/fail limit to approach given risk probability	7310

Annex H (normative):	Default message contents for RRM	7315

H.1	Void	7315

H.2	System information blocks message content exceptions	7315

H.2.1	System information blocks message contents exceptions for NR intra-frequency cell re-selection	7315

H.2.2	System information blocks message contents exceptions for NR inter-frequency cell re-selection	7317

H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection	7318

H.2.4	System information blocks message contents exceptions for Satellite Access	7319

H.3	RRC message content exceptions	7320

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration	7320

H.3.2	RRC messages and information elements contents exceptions for handover	7347

H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover	7348

H.3.4	E-UTRA RRC messages and information elements contents exceptions	7349

H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting	7361

H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting	7364

H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used	7370

H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay	7373

H.3.9	RRC messages and information elements contents exceptions for UL timing	7374

H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access	7375

H.3.11	RRC messages and information elements contents exceptions for UL Tx switching	7376

H.3.12	RRC messages and information elements contents exceptions for L1/L2-triggered mobility	7393

H.3.13	RRC messages and information elements contents exceptions for Satellite Access	7403

H.3.14	RRC messages and IE content exceptions for FR2 Multi-Rx	7404

Annex I (normative):	RRM OTA procedures	7408

I.0	Test applicability per permitted test method	7408

I.1	Direct far field (DFF)	7408

I.1.1	RX beam peak direction search	7408

I.1.2	Search for directions corresponding to the EIS spherical coverage percentile	7408

I.2	Direct far field (DFF) simplification	7408

I.2.1	RX beam peak direction search	7408

I.2.2	Search for directions corresponding to the EIS spherical coverage percentile	7409

I.3	Indirect far field (IFF)	7409

I.3.1	RX beam peak direction search	7409

I.3.2	Search for directions corresponding to the EIS spherical coverage percentile	7409

I.3A	Enhanced indirect far field (Enhanced IFF)	7409

I.3A.1	RX beam peak direction search	7409

I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile	7409

I.4	Procedures to search test directions for RRM FR2	7409

I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search	7409

I.5	Re-Positioning Approach for RRM FR2	7410

I.5.1	1 AoA Test Cases	7411

I.5.2	2 AoA Test Cases	7412

Annex J (informative):	Change history	7414