| 3GPP TS 23.xyz V0.2.0 (2025-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	7

4	Architecture model and concepts	8

4.1	General concept	8

4.2	Architecture	8

4.2.1	General	8

4.2.2	Architecture for AIoT RAN connectivity	8

4.2.2.1	General	8

4.2.2.2	Direct connectivity between AIOTF and AIoT RAN	10

4.2.2.3	Indirect connectivity between AIOTF and AIoT RAN via an AMF	10

4.3	Reference points	11

4.4	Service-based interfaces	12

4.5	Functional Entities	12

4.5.1	AIoT Device	12

4.5.2	AIoT Readers	12

4.5.3	AIOTF	12

4.5.4	NEF	13

4.5.5	AF	13

4.5.6	NRF	13

4.5.7	AMF	13

4.5.8	UDR	13

4.5.9	 ADM	13

4.6	Protocol Stacks	14

4.6.1	General	14

4.6.2	Protocol Stack between AIoT Device and AF	14

4.6.2.1	General	14

4.6.2.2	Protocol Stack between AF and AIoT Device for AIoT RAN Direct Path	15

4.6.2.3	Protocol Stack between AF and AIoT Device for AIoT RAN Indirect Path	15

5	High level functionality and features	15

5.1	General	15

5.2	AIoT Services	16

5.2.1	AIoT Inventory service	16

5.2.2	AIoT Command service	16

5.3	AIoT Reader Selection	16

5.4	Assistance information provided to AIoT RAN node	17

5.5	AIoT Device Profile Management	17

5.6	AF authorization to the AIoT Services	17

5.7	Identifiers	18

5.7.1	General	18

5.7.2	AIoT Device Permanent Identifier	18

6	AIoT Procedures	19

6.1	General	19

6.2	AIoT Service Procedures	19

6.2.1	General	19

6.2.2	Inventory Procedure	19

6.2.3	Procedure for Command	21

7	Network Functions Services	24

7.1	General	24

7.2	AIOTF services	24

7.2.1	General	24

7.2.2	Naiotf_ AIoT_Inventory service operation	24

7.2.3	Naiotf_ AIoT_Command service operation	25

7.2.4	Naiotf_ AIoT_Notify service operation	25

7.3	AMF services	26

7.3.1	General	26

7.3.2	Namf_AIoT_MessageDelivery service operation	26

7.3.3	Namf_AIoT_Notify service operation	26

7.4	NEF services	27

7.4.1	General	27

7.4.2	Nnef_AIoT_Inventory service operation	27

7.4.3	Nnef_AIoT_Command service operation	27

7.4.4	Nnef_AIoT_Notify service operation	27

7.5	ADM services	28

7.5.1	General	28

7.5.2	Nadm_DM_Query service operation	28

7.5.3	Nadm_DM_Update service operation	28

Annex A (informative): Change history	29

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specifies architectural enhancements to the 5G system to support Ambient power-enabled Internet of Things, complying to the requirements in TS 22.369 [1] applicable to the AIoT Device types, traffic types, use cases and connectivity topologies defined in TS 38.300 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[6]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[7]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

3	Definitions of terms and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIoT Device: An Ambient IoT device is an IoT device powered by energy harvesting, with limited energy storage capability.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ADM	AIoT Data Management

AIoT	Ambient IoT

AIOTF	Ambient IoT Function

EPC	Electronic Product Code

4	Architecture model and concepts

4.1	General concept

AIoT is a service that can be provided by the 5GS system to support Ambient power-enabled IoT devices that are powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other suitable power source.

The 5GS System architecture for AIoT include the following functions and procedures for:

-	AIoT Device identification;

-	AIoT Device inventory;

-	Providing to, and obtaining from, an AIoT Device application data.

-	Disabling AIoT Devices.

4.2	Architecture

4.2.1	General

The 5GS System architecture for AIoT includes core network functions, different AIoT Reader architectures and AIoT Devices. The different AIoT Reader architectures allow for different deployment options. The following AIoT Reader architectures are defined:

-	AIoT RAN (which supports AIoT Reader), which includes either a direct connectivity path between AIoT RAN and the AIOTF or an indirect connectivity path between AIoT RAN and the AIOTF via an AMF.

The architecture for Network Exposure Function, using reference point representation, defined in TS 23.501 [3] clause 4.2.3 is applicable for AIoT, with a southbound interface from the NEF to AIOTF.

4.2.2	Architecture for AIoT RAN connectivity

4.2.2.1	General

Editor’s Note: The term AIoT RAN needs to be aligned with RAN WGs throughout the TS.

Editor’s Note: Whether the connection to the AIOTF should be Nx or N2 is to be determined.

5GS system architecture for AIoT supports the following paths to access an AIoT RAN:

-	Direct Connectivity: AIOTF communicates with AIoT RAN directly.

-	Indirect Connectivity via AMF: AIoT RAN and the AIOTF communicate indirectly via an AMF.

Figure 4.2.2.1-1 depicts the complete non-roaming architecture showing the overall 5GS architecture for support of AIoT, including both the Indirect Connectivity and Direct Connectivity .

Figure 4.2.2.1-1: Non-roaming AIoT System Architecture

NOTE 1:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na and N2 as depicted using dashed lines, as all deployments might not use them.

Figure 4.2.2.1-2 depicts the complete non-roaming AIoT system architecture, using the reference point representation.

Figure 4.2.2.1-2: Non-roaming AIoT System Architecture (AIoT RAN Readers) in reference point representation

NOTE 2:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDR, NEF and NRF as necessary.

NOTE 3:	For clarity, the UDR and its connections with other NFs, e.g. ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

NOTE 4:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na, N2 and Nx as depicted using dashed lines, as all deployments might not use them.

The architectures in the following clauses showing parts of the overall AIoT architecture specific to each connectivity option:

-	Direct Connectivity: the AIOTF uses Nx to access AIoT RAN, and is described in clause 4.2.2.2.

-	Indirect Connectivity via AMF: the AIOTF uses an AMF which uses N2 to access AIoT RAN, and is described in clause 4.2.2.3.

NOTE 5:	A deployment that only uses, e.g., the Direct Connectivity only does not need to deploy the NFs, reference points and service-based interfaces associated with the Indirect Connectivity, and vice-versa.

4.2.2.2	Direct connectivity between AIOTF and AIoT RAN

In the Direct Connectivity architecture, the AIOTF uses Nx to communicate directly with AIoT RAN.

Figure 4.2.2.2-1 depicts the AIoT architecture, using the service-based interfaces, showing only the parts of the AIoT architecture for an AIOTF connecting to AIoT RAN directly. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.2-1: AIoT RAN – AIOT Direct connectivity Architecture

Figure 4.2.2.2-2 depicts the AIoT architecture, using the reference point representation, showing only the parts of the AIoT architecture for an AIOTF access AIoT RAN. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.2-2: AIoT RAN – AIOT Direct connectivity Architecture in reference point representation

4.2.2.3	Indirect connectivity between AIOTF and AIoT RAN via an AMF

Figure 4.2.2.3-1 depicts the AIoT architecture, using the service-based interfaces showing only the parts of the AIoT architecture for an AIOTF connects indirectly to AIoT RAN via an AMF. The remaining parts of the AIoT Architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.3-1: AIoT RAN – AIOT Indirect connectivity Architecture

Figure 4.2.2.3-2 depicts the AIoT architecture, using the reference point representation showing only the parts of the AIoT architecture for an AIOTF connects to AIoT RAN via an AMF. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged

Figure 4.2.2.3-2: AIoT RAN – AIOT Indirect connectivity Architecture in reference point representation

4.3	Reference points

Editor’s Note: The reference point names (e.g. Ny etc.) need to be finalised.

The AIoT Architecture contains the following reference points:

Ny:	Reference point between the AIoT Device and the AIOTF.

Nx:	Reference point between the AIoT RAN and the AIOTF.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

Na:	Reference point between the AIOTF and the AMF.

Nb:	Reference point between the AIOTF and the NEF.

Nc:	Reference point between the AIOTF and the NRF.

Nd:	Reference point between the AIOTF and the ADM.

Ne:	Reference point between the ADM and the UDR.

Nf:	Reference point between the ADM and the NEF.

In addition to the relevant reference points defined in TS 23.501 [3], in the case of AIoT, these reference points are as follows:

N2:	Reference point between the AIoT RAN and the AMF.

N33:	Reference point between NEF and AF.

4.4	Service-based interfaces

Naiotf:	Service-based interface exhibited by the AIOTF.

Nadm:	Service-based interface exhibited by the ADM.

In addition to the relevant services defined in TS 23.501 [3] the following service-based interfaces are enhanced for AIoT in this specification:

Namf:	Service-based interface exhibited by AMF.

Nnef:	Service-based interface exhibited by NEF.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

4.5	Functional Entities

4.5.1	AIoT Device

Editor’s note:	The definition of AIoT Device needs to align with the definition at RAN WGs.

4.5.2	AIoT Readers

Editor’s note:	AIoT RAN and AIoT Reader will be aligned later with RAN WGs. It is FFS whether to specify the relationship between AIoT RAN and AIoT Reader and which term to use in this TS.

4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an AIOT RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the trusted AF’s AIoT service operation request.

-	Triggering the AIoT RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the AIoT RAN .

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	AIoT RAN selection and optionally a list of AIoT RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the AIoT RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.

4.5.4	NEF

In addition to the functions defined in TS 23.501 [3], the NEF performs the following functions:

-	Providing a service exposure API to AFs of 3rd party for AIoT services.

-	Interacting with AF of 3rd party and AIOTF.

-	Selection of AIOTF for AIoT services.

-	Authorization of the untrusted AF’s AIoT operation request.

4.5.5	AF

The AF performs the following functions to support AIoT services:

-	Interacting with NEF for AIoT related service exposure for 3rd party .

-	Interacting with AIOTF for AIoT related service exposure for trusted AF.

4.5.6	NRF

In addition to the functions defined in TS 23.501 [3], the NRF performs the following functions:

-	Support of new NF types AIOTF and their corresponding NF profiles. The NF profile includes the AIOTF ID or IP address, NF type and the serving area information of the AIOTF.

-	Support of AIOTF discovery based on parameters such as target area information.

4.5.7	AMF

The AMF performs the following functions when the AIoT RAN and the AIOTF communicate indirectly via an AMF:

-	Relaying signalling for AIoT service between AIoT-RAN and AIOTF.

-	Providing transparent transport for AIoT NAS messages between AIoT Device and AIOTF.

-	Providing transport for AIOT RAN Node’s serving area information between AIOT RAN and AIOTF.

Editor’s note:	Further functions on AMF is FFS.

4.5.8	UDR

In addition to the functions defined in TS 23.501 [3], the UDR may support the following functions:

-	Storage of AIoT device profile data.

4.5.9		ADM

The ADM supports the following functions:

-	Management of AIoT device profile data.

4.6	Protocol Stacks

4.6.1	General

This clause specifies the protocol stacks between entities used for AIoT. The protocol stacks include the following:

-	Protocol stacks between AIoT Device and AFs, including the protocol stacks among AIoT Device, AIOT RAN, AMF, AIOTF, NEF and AFs.

Editor’s note:	The protocol stacks between RAN and 5GC in the following clauses need to align with RAN specifications.

4.6.2	Protocol Stack between AIoT Device and AF

4.6.2.1	General

The general protocol stack between an AIoT Device, AIoT RAN, AIOTF and AF is shown in Figure 4.6.2.1-1.

Legend:

-	AIoT NAS: The NAS protocol between AIoT Device and AIOTF.

-	AIoT AS: It is between the AIoT Device and the AIoT RAN is specified in TS 38.300 [5].

-	AIoT Data: It is the application data exchanged between the AIoT Device and AF.

Figure 4.6.1-1: Protocol Stack Between AIoT Device and AF

The AIoT RAN may communicate to AIOTF via different paths, see clause 4.2.2.1. The AIoT RAN protocol stack remains the same and is transparent to the difference paths. The protocol stacks and routing of information between an AIoT RAN and the AIOTF is defined in other clauses depending upon the architecture used:

-	Direct Path: When the AIoT RAN communicates with AIOTF directly, the protocol stack is specified in clause 4.6.2.2.

-	Indirect Path: When the AIoT RAN communicates with AIOTF indirectly via an AMF, the protocol stack is specified in clause 4.6.2.3.

The AIoT NAS protocol supports the inventory response and command related signalling.

4.6.2.2	Protocol Stack between AF and AIoT Device for AIoT RAN Direct Path

Legend:

-	AIoT Reader Control: It is between the AIOTF and AIoT RAN.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Path option

The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the AIoT RAN transparently.

4.6.2.3	Protocol Stack between AF and AIoT Device for AIoT RAN Indirect Path

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Path via AMF option

In this Protocol Stack, the AMF is additionally involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the AIoT RAN and AMF transparently.

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

5	High level functionality and features

5.1	General

5.2	AIoT Services

The following AIoT Services are supported:

- Inventory service

- Command service.

5.2.1	AIoT Inventory service

AIoT Inventory service is used to discover the AIoT devices, i.e., to obtain the AIoT device identifiers.

5.2.2	AIoT Command service

Three types of Command service operations are supported: Read, Write and Disable.

An AF uses the Command service Read operation to retrieve information from a specific AIoT device.

An AF uses the Command service Write operation to configure information into a specific AIoT device.

An AF uses the Command service Disable operation to permanently disable the capability of the AIoT device(s) to transmit RF signals

5.3	AIoT Reader Selection

The AIOTF selects AIoT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the AIoT RAN node(s) based on the internal area information.

Editor’s note:	Whether and how the AIOTF chooses the RAN reader(s) is FFS.

Editor’s note:	How AIOTF selects the AMF and the AIoT RAN in the indirect path is FFS.

The AF may provide the expected target area information in the AIoT service request. The NEF may convert the target area information received from the AF to internal area information. The NEF queries the NRF to obtain serving AIOTF(s) based on internal area information. Then, the NEF forwards the AIoT service request, including the internal area information, to the AIOTF.

Editor’s note:	How the NEF converts target area information to internal area information and the format of the internal area information is FFS.

Editor’s note:	It is FFS how to handle the case that the AF provides other information than target area information, for the purpose of reader selection. In such case how the NEF and/or AIOTF use it to select the RAN reader.

The AIOTF may have AIoT RAN information and the optional RAN reader information. For direct path, the AIOTF obtains the information from AIoT RAN by the NGAP or OAM configuration.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

NOTE:	RAN reader information is not exposed to the AF.

In case an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) to determine targeted RAN reader(s) directly.

5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation.

The following assistance information may be provided:

-	AIoT service type (e.g. Inventory, Command);

-	Optionally, approximate number of AIoT devices based on AF request;

-	Optionally, approximate D2R message size based on AF request.

Editor's note:		Other assistance information may be added later if necessary.

Editor's note:		How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

5.5	AIoT Device Profile Management

The ADM may hold operator’s subscription data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g., Device ID or credentials) is stored external to the network.

The AIoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in TS 23.502 [4] clause 5.2.3, it is stored in the ADM (AIoT Data Management) network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

Editor’s note:	Whether ADM support ID security mechanism pending on SA3 decision.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |

Editor’s note:	SA WG2 will align security related materials with SA WG3 decision later.

5.6	AF authorization to the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the NEF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g., inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to the AF for the AIoT mainly includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF may perform authorization of AIoT service requested by the AF, by retrieving the AF authorization data stored in the ADM as described in above Table 5.6-1.

Editor’s Note:	Whether to perform AF authorization only in NEF or in both NEF and AIOTF (including which parameter to use in Table 5.6-1), and whether a trusted AF requires authorization is FFS.

5.7	Identifiers

5.7.1	General

5.7.2	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT device permanent identifier shall be allocated to each AIoT device. An AIoT device permanent identifier is assigned either by an operator or by a third party. The identifier is used to identify an AIoT device and locate the entity where the AIoT device related information is stored.

NOTE 1:	How to configure AIoT device with the permanent AIoT device identifier is out of 3GPP scope.

The permanent AIoT device identifier includes the following components:

-	The ID type, including:

-	Information indicating whether the ID includes a network identifier or not.

-	Information indicating whether the ID includes a third party identifier or not.

-	The identification information type, i.e., EPC or unstructured information.

-	The domain information includes none, one or both of the following:

-	A network identifier (i.e. PLMN ID (MCC and MNC) and/or Network Identifier (NID) as specified in TS 23.003 [6]), when the ID includes the network identifier.

-	A third party identifier used to identify a third party when the ID includes the third party identifier.

-	The identification information which is used to distinguish different AIoT devices within the scope identified by domain information (if available).

-	EPC as defined in clause 14 of [7];

-	The unstructured information.

Figure 5.7.2-1:  AIoT device permanent identifier Structure

NOTE 2:	The coding for the above information and how to code the length for the identification information are left to stage 3.

For operator allocated ID, the network identifier is mandatory.

For third party allocated Part 1ID, the network identifier is not included and the third party identifier is optional.

NOTE 3:	The length of ID type, Network Identifier (if present) and the third party identifier (if present) are fixed. The length of the identification information is dynamic. The details are specified in TS 23.003[6] and TS 29.xxx [xx].

Editor’s note:	The reference in NOTE 2 and 3 needs to be updated, when the appropriate stage 3 document is identified.

6	AIoT Procedures

6.1	General

Editor’s Note: The contents of this general clause is FFS and is expected to describe there are service procedures which are reader type/routing agnostic and there are procedures which relate to the transports to the readers.

6.2	AIoT Service Procedures

6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command.

Editor’s Note:	Additional information in the steps, parameters, and their naming throughout the procedures requires additional details, alignment with other clauses and references adding as required.

Editor’s Note: 	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices])  service operation request to the NEF.

Editor’s Note:	The parameters for the inventory service operation need further definition.

2.	The NEF selects the AIOTF to handle the request.

Editor’s Note:	How the NEF selects the AIOTF needs further definition.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by AIoT RAN.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

Editor’s Note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, and assistance information to the selected AIoT RAN.

Editor’s Note:	AIoT Device Identification information needs further definition.

Editor’s Note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor’s Note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10. AIoT RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s).

Editor’s Note:	Whether only one, or more than one report is generated by the AIOTF to send to the AF needs clarification.

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

6.2.3	Procedure for Command

Editor’s Note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor’s Note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], (AIoT data)) to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The Target AIoT device information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Editor's note:	It is FFS how to define the filtering information.

2.	The NEF selects AIOTF(s) taking into account the target area information for AIoT service operation and Target AIoT device information in the step 1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

Editor’s Note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_ Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_ Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed.

Editor's note:	It is FFS whether the AIoT RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, NAS Command Request) to the AIoT RAN. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request..

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to AIoT RAN.

13.	The AIoT RAN sends the AS R2D message (NAS Command Request) to the AIoT device.

14.	The AIoT device sends the AS D2R message (NAS Command Response) to the AIoT RAN.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG’s specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

For Direct path as shown in clause 4.2.2.2, step 15a is performed.

15a.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

For Indirect path via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed

15b.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds an Naiotf_AIoT_Command Notify message (AMF ID, correlation identifier, Command Response (correlation identifier, Reader ID, NAS Command Response)) to the AIOTF.

16.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

17.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command Notify.

7	Network Functions Services

7.1	General

7.2	AIOTF services

7.2.1	General

The AIOTF supports to expose AIoT services towards the AF or the NEF as described in Table 7.2.1-1.

Table 7.2.1-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF, AF |
|  | Command | Request/Response | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

Editor’s note:  It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect path.

Editor’s note:  It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

7.2.2	Naiotf_ AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory.

Description: The NF consumer requests an inventory operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the inventory operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.3	Naiotf_ AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

4) Command Type: Read, Write or Permenant Disable.

5) Command specific parameters:

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Editor’s note:	The Command specific parameters for Read/Write/Disable command is FFS.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate message size from the AIoT Device for Read Operation.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.4	Naiotf_ AIoT_Notify service operation

Service operation name: Naiotf_AIoT_Notify

Description: The AIOTF uses this service operation to notify the results or status of the service operation towards the NF consumers. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Command service operation, the NF consumer implicitly subscribes to the results of the requested service operation.

Inputs, Required:

1)	Common report information: Transaction ID, List of AIoT Device ID or Failure Cause in case of Failure).

2)	Read command specific report information: Information obtained from each target AIoT Device.

Editor’s note:	It is FFS whether that fail to discovery any AIoT device is normal operation result or failure case.

Inputs, Optional: none.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor’s note:	It is FFS how to make AF be aware of the report is finished in case there are multiple notify for the same AIoT operation.

7.3	AMF services

7.3.1	General

AMF supports to expose AIoT services towards the AIOTF as described in Table 7.3.1-1. The Namf_AIoT AMF service is used when AIOT RAN and the AIoTF communicate indirectly via an AMF.

Table 7.3.1-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: the NF consumer requests to send AIoT data (i.e., AIoT inventory/command messages) towards AIOT RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages to deliver to AIoT RAN.

2)	AIOT RAN ID.

Outputs, Required: AIoT messages received from AIoT RAN, Result indication (Success or Failure), Failure Cause in case of Failure.

Editor’s note:	It is FFS whether the Correlation identifier or AIOTF ID is included in the Namf_AIoT_MessageDelivery.

7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: the NF consumer requests to receive AIoT data from AIOT RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from AIOT RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages received from AIOT RAN.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

Editor’s note:	AIoT messages is FFS and coordination with RAN3 is needed.

Editor’s note:	It is FFS how to make AIOT be aware of the report is finished in case there are multiple notify for the same operation.

Editor’s note:	It is FFS whether the Correlation identifier is included in the Namf_AIoT_Notify.

7.4	NEF services

7.4.1	General

In addition to those defined in TS 23.501 [3] clause 7.2.8 and TS 23.502 [4] clause 5.2.6, the table 7.4.1-1 illustrates additional NEF services to support AIoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

Editor’s note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

7.4.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_ AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation.

Input, Optional: Approximate number of AIoT Devices.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_ AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Disable).

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Read command type,

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also apply for the other command types.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_ AIoT_ Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required: AF Transaction ID, a list of AIoT Device ID(s) or Failure Cause in case of Failure, Information obtained from each target AIoT Device.

Input, Optional: None.

Output, Required: Result indication.

Output, Optional: None.

Editor’s note:	The NEF AIoT service operation definition needs to be revisited to align with AIOTF services

7.5	ADM services

7.5.1	General

The following table shows the Nadm_DM Service and Service Operations.

Table 7.5.1-1: NF services provided by ADM

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF |
|  | Update | Request/Response | AIOTF |

7.5.2	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query.

Description: NF service consumer may request the AIoT device profile data or the AF authorization data from the ADM.

Inputs, Required: AIoT Device Permanent ID, or AF ID.

Input, Optional: None.

Outputs, Required: the AIoT device profile data or the AF authorization data.

Output, Optional: None.

Editor’s Note:	Whether AF authorization data is stored in ADM is FFS.

7.5.3	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update.

Description: NF service consumer may update the AIoT device profile data in the ADM.

Inputs, Required: AIoT Device Permanent ID, updated AIoT device profile data.

Input, Optional: None.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

Output, Optional: None.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-01 | SA2#166 AH-e | S2-2501256 | - | - | - | Proposed skeleton agreed at SA2#166AH-e | 0.0.0 |
| 2025-01 | SA2#166 AH-e | - | - | - | - | Inclusion of documents approved in SA2#166AH-e: S2-2501253, S2-2501254, S2-2501255, S2-2501257, S2-2501258. | 0.1.0 |
| 2025-02 | SA2#167 | - | - | - | - | Inclusion of documents approved in SA2#167: S2-2502368, S2-2502555, S2-2502564, S2-2502578, S2-2502579, S2-2502581, S2-2502582, S2-2502585, S2-2502586, S2-2502588, S2-2502589, S2-2502590 | 0.2.0 |