| 3GPP TR 23.700-56 V0.1.0 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on system architecture enhancement for next generation real time communication phase 3; Stage 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	VOID	6

3.3	Abbreviations	6

4	Architectural Assumptions and Requirements	7

4.1	Architectural Assumptions	7

5	Key Issues	7

5.1	Key Issue #1: ADC establishment in alerting phase for early media	7

5.2	Key Issue #2: Enhancements to capability exposure framework for eMMTel services	7

5.3	Key Issue #3: Supporting dynamic configuration of MDC2 endpoints	8

6	Solutions	8

6.0	Mapping of Solutions to Key Issues	8

6.1	Solution #1: ADC establishment in alerting phase initiated by the UE	8

6.1.0	High-level solution Principles	8

6.1.1	Description	9

6.1.2	Procedure	9

6.1.3	Impacts on Services, Entities and Interfaces	12

6.2	Solution #2: Support for NW initiated data channel application in early media session with subscription	12

6.2.1	High level principles	12

6.2.2	Description	12

6.2.3	Procedures	13

6.2.4	Impacts to Services, Entities and Interfaces	15

6.3	Solution #3: IMS ADC establishment during alerting phase when initial INVITE does have BDC offer	15

6.3.0	High-level solution Principles	15

6.3.1	Description	15

6.3.2	Procedures	16

6.3.3	Impacts on Services, Entities and Interfaces	17

6.4	Solution #4: IMS ADC establishment during alerting phase when initial INVITE does not have BDC offer	18

6.4.0	High-level solution Principles	18

6.4.1	Description	18

6.4.2	Procedures	19

6.4.3	Impacts on Services, Entities and Interfaces	21

6.X	Solution #X: <Solution Title>	21

6.X.0	High-level solution Principles	21

6.X.1	Description	21

6.X.2	Procedures	21

6.X.3	Impacts on Services, Entities and Interfaces	21

7	Interim agreements	22

7.1	Agreed Principles	22

7.1.Y	Agreed Principles for KI#Y	22

7.2	Topics for further consideration	22

7.2.Z	Topics for further consideration for KI#Z	22

8	Conclusions	22

Annex A:	Change History	23

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document investigates further enhancements to the capabilities of IMS data channel (DC), specifically in the areas of ADC establishment in alerting phase, IMS capability exposure, and dynamic configuration of MDC2 endpoints.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 26.114: "Media handling and interaction".

[4]	3GPP TS 23.203: "Policy and charging control architecture".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	VOID

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2P	Application to Person

ADC	Application Data Channel

API	Application Programming Interface

DC	Data Channel

DCAR	Data Channel Application Repository

DCMF	Data Channel Media Function

DCMTSI	Data Channel Multimedia Telephony Service for IMS

DCSF	Data Channel Signalling Function

IMS	IP Multimedia Subsystem

MF	Media Function

MRF	Media Resource Function

NEF	Network Exposure Function

NF	Network Function

OMA	Open Mobile Alliance

P2A	Person to Application

P2A2P	Person to Application to Person

P2P	Person to Person

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

The following Architectural Assumptions are applicable to this Study:

-	The IMS and IMS DC architecture defined in TS 23.228 [2] is used as the basis for any enhancements.

-	An AS may be trusted or untrusted.

-	The functionality of existing standardized APIs (e.g. OMA APIs) exposing MMTEL services may be used where applicable when the MMTEL service does not depend on IMS DC media.

-	OMA APIs are not extended to support Data Channel and the principles outlined in TS 23.228 [2] Annex AD (IMS Subscribe/Notify framework for event monitoring).

# 5	Key Issues

## 5.1	Key Issue #1: ADC establishment in alerting phase for early media

This key issue will investigate how to support IMS application data channel establishment in alerting phase to provide early media for a normal IMS DC session.

This Key Issue will study:

-	Whether and how the capability of supporting ADC establishment in alerting phase is exchanged between UE and IMS network.

-	How the use of ADC during alerting phase is controlled.

-	Whether and how to download a specific list of DC applications that can be used during alerting phase.

-	How to establish and terminate an ADC during alerting phase.

## 5.2	Key Issue #2: Enhancements to capability exposure framework for eMMTel services

This Key Issue will investigate how to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities.

Solutions should describe how to enhance Rel-19 IMS event exposure framework and event definition to support IMS DC events supported by DCSF (e.g. application downloading event) and non-DC events, including subscriber specific and non-subscriber specific supported by IMS AS.

NOTE 1: 	This Key Issue will analyse relevant use cases, the gap between OMA and 3GPP APIs and specify exposure functionality missing from OMA APIs.

NOTE 2:	This Key Issue does not intend to transfer the responsibility of OMA API(s) to 3GPP.

This Key Issue will study:

-	Identification of new events and/or capabilities to be exposed, in addition to those already listed in Table AD.2.5.3-1 of TS 23.228 [2].

-	Assessment of whether existing OMA APIs support the exposure of the newly identified events and/or capabilities.

-	Possible enhancement of exposure functionality where the assessment above identifies gaps in supporting exposure of the newly identified events and/or capabilities.

## 5.3	Key Issue #3: Supporting dynamic configuration of MDC2 endpoints

This Key Issue will investigate whether and how to define interfaces between DCSF and NEF/AF to complete existing P2A/P2A2P Data Channel procedures by specifying DC3/DC4 interfaces to enable dynamic configuration of MDC2 endpoints.

This Key Issue will study:

-	the services and information elements that are exposed via DC3/DC4 for UE initiated P2A and P2A2P ADC session establishment.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |

## 6.1	Solution #1: ADC establishment in alerting phase initiated by the UE

### 6.1.0	High-level solution Principles

This solution addresses Key Issue #1: DC establishment in alerting phase for early media.

The solution reuses Rel-19 TS 23.228 [2] with the addition that

-	Based on the UE request (via a specific root URL) , the DSCF sends to the UE(s) an application list related with the alerting phase-	The calling party receive BDC from the terminating network during alerting phase.

-	The UE(s) decide whether and which application to download (as for Rel-19 specifications).

-	The UE(s) decide when to release an application that was running in alerting phase; they may decide to continue this application after the alerting phase has finished. (as for Rel-19 specifications).

The solution may e.g. be used for the case where the originating UE1 calls a help centre and is immediately provided with an App that the user will be using when interacting with the call centre.

### 6.1.1	Description

This solution provides a procedure on how to establish a P2A ADC in alerting phase.

Editor's note:	Current version of the solution supports P2A ADC establishment in alerting phase. it is FFS whether it can support other forms of ADC, i.e. P2A2P and P2P.

### 6.1.2	Procedure

The pre-condition of ADC establishment in alerting phase is the capability negotiation during the IMS registration phase on whether the UE supports the DC in alerting phase.

Based on the IMS DC capability negotiation defined in clause AC.7.0 of TS 23.228 [2], when the UE supporting DC in alerting phase and the UE is allowed to use DC in alerting phase, it includes the media feature tag of supporting DC in alerting phase in the Contact header field of the initial REGISTER request and any subsequent REGISTER request to allow the home IMS network to discover its capability of supporting DC in alerting phase.

If the IMS network supports DC in alerting phase, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting DC in alerting phase in the 200 OK response to the initial and any subsequent REGISTER request, which is used by the UE to discover the IMS data channel capability of its home IMS network.

The UE may receive a Feature-Caps header field indicating IMS's capability of supporting DC in alerting phase in the 200 OK response to a subsequent REGISTER request when the network starts supporting DC in alerting phase after successful initial registration of the UE.

When the UE supporting DC in alerting phase initiates an IMS session and if the UE is allowed to use DC in alerting phase, it includes the media feature tag in the Contact header field of the initial INVITE message, regardless of data channel media being part of the SDP or not.

The UE shall not include the media feature tag in the Contact header field and data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE request message, if the S-CSCF has not included the capability indication of supporting DC in alerting phase in the Feature-Caps header field in the 200 OK response either to the initial REGISTER or a subsequent REGISTER request message.

If the UE is not configured whether to use DC in alerting phase, it is up to UE implementation whether or not to include the media feature tag in the Contact header field.

With a successful capability negotiation about the support of DC in alerting phase, the procedure is illustrated in Figure 6.1.2-1, which is based on clause AC.7.2.2 in TS 23.228 [2].

Figure 6.1.2-1: Procedure of ADC establishment in alerting phase

The steps in the procedure are as follows:

1.	Same as step 1-14 in clause AC.7.1 of TS 23.228 [2], UE#1 initiates a SIP INVITE towards the IMS AS. After validating the user's subscription, the IMS AS notifies the DCSF about DC support. The DCSF provides the IMS AS with instructions and media information for establishing BDC. The IMS AS allocates resources with MF accordingly and updates the INVITE with the negotiated media. The updated SIP INVITE is forwarded to the terminating network/UE#2 for terminating network negotiation.

2-3.	UE#2 and terminating network returns an 18x response with the SDP answer to BDC to originating network. According to the received SDP answer, MF may update data channel media resource information for UE#2.

4.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, Session ID, Media Info List) to DCSF.

5.	The DCSF responds to the Nimsas notification request.

6.	The SIP message18x is forwarded to UE#1 which indicates the BDC has been established and that the alerting phase has started.

7.	The originating network P-CSCF executes QoS procedure for BDC as specified in TS 23.203 [4] and TS 23.503 [5].

8.	PRACK and 200 OK for PRACK procedure are performed.

9-10.	When the BDCs have been established between terminating MF and UE#1/UE#2, the DC application list for alerting phase and DC application for alerting phase is requested and downloaded to UE#1 and UE#2 from terminating DCSF.

The DCSF provides via MF the application list for alerting phase from the received specific URL , further to UE#1 and UE#2, based on their data channel capabilities and their choices.

The UE(s) use a specific URL related with the fact that they request a list of applications available in the alerting phase

Editor's note:	It is FFs how the UE is alerted to initiate the download.

11.	UE#1 initiates a P2A ADC for the selected DC application for alerting phase. UE#1 sends SIP UPDATE with an updated SDP to IMS AS via originating network P-CSCF and S-CSCF. The updated SDP contains the BDC information, as well as the requested ADC and the associated DC application binding information.

12.	Same as step 3-14 of clause AC.7.2.2-1 in TS 23.228 [2], the IMS AS notifies the DCSF about DC support. The DCSF instructs the IMS AS to establish MF anchoring via MDC2. The IMS AS provisions the necessary media resources at the MF and returns the information to the DCSF. The DCSF then initiates ADC setup toward the DC Application Server via DC3/DC4. After receiving the DC AS's response with its own MDC2 media resource, the DCSF instructs the IMS AS to update the MF resource accordingly. The IMS AS completes the MF configuration and notifies the DCSF.

13-14.	IMS AS sends the SIP UPDATE to remote network side and UE#2, via the originating S-CSCF, which does not include a request in the SDP for the ADC.

15-17.	UE#2 and terminating network returns a SIP UPDATE 200 OK response with SDP answer for audio/video/bootstrap.

18.	IMS AS notifies the DCSF about the successful result of media confirmation.

19.	DCSF replies to the notification.

20.	The IMS AS includes SDP answer for ADC to UE#1 in SIP UPDATE 200 OK response that it sends towards the UE 1 via the S-CSCF and P-CSCF.

21.	The originating network P-CSCF executes QoS procedure for ADC media based on the SDP answer information from the 200 OK response.

22.	SIP UPDATE 200 OK forwarded to UE#1 with ADC SDP answer.

23.	The ADC between UE#1 and DC AS is established via MF. MF forwards data channel traffic between UE#1 and DC AS based on MDC2 media point information.

24-25.	UE#2 answers the call, and SIP INVITE 200 OK response is sent to the IMS AS via I/S-CSCF.

26.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (Session ID) of the call state change.

27.	DCSF replies to the notification.

28-29.	SIP INVITE 200 OK forwarded to UE#1 which indicates the call is answered, and ACK procedure is performed.

30.	Both UE#1 and UE#2 retrieve a new application list via BDC since call is answered. The new list supersedes the previously downloaded list. The UEs send request to MF to request application list by providing a root URL, via the established BDC with its data channel capabilities. The DCSF provides the application list from the root URL provided by UE#1 via MF, further to UE#1 and UE#2, based on their data channel capabilities and their choices through MF.

31.	Subsequent procedures for DC application, as defined in clause AC.7.2 of TS 23.228 [2]. Depending on the application and user interaction, the UE(s) determine when to release the application initiated in alerting phase but this aspect is out of scope of specifications and does not require a new UE behaviour.

### 6.1.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	supporting retrieving DC application list and DC application for alerting phase; this includes providing a specific URL to get from DCSF the application list for alerting phase,

-	supporting ADC establishment in alerting phase.

IMS AS:

-	supporting negotiation of ADC in alerting phase.

Editor's note:	Impacts on IMS AS and DCSF are FFS.

DCSF:

-	providing DC application list based on the specific URL received from the UE and associated with the application list for alerting phase.

## 6.2	Solution #2: Support for NW initiated data channel application in early media session with subscription

### 6.2.1	High level principles

This solution has the following high level principles, in order to support IMS data channel application in early media session:

-	Data Channel Application in Alerting Phase (DCAP) service can be configured by the user and stored in the HSS as part of the subscription data.

-	IMS AS triggers the application data channel in alerting phase based on UE subscription data.

-	IMS AS performs as a B2BUA to update the SDP for data channel in early session and media flow for the main session.

-	The application data channel may coexistent with other early session services, e.g. CAT, CRS.

### 6.2.2	Description

This is a solution for Key issue #1 " DC establishment in alerting phase for early media".

The user of UE#2 is able to subscribe to the Data Channel Application in Alerting Phase (DCAP) service, including activate (or de-activate) the service, and update the settings, e.g. to change by configuration the active data channel application. The DCAP subscriber is able to refine the data channel application selection behaviour with configured rules, e.g. time, calling party's location, called party's location, the identity of the calling and called party. The DCAP service is able to select the appropriate data channel application according to the rules.

DCAP is a terminating network service, but can also have an originating network functional component. That is, DCAP can be selected on behalf of the called subscriber for presentation to the calling party, but the calling (IMS) subscriber can also subscribe to and activate the DCAP service.

Editor's note:	The consideration on originating user consent of NW initiated ADC is FFS.

### 6.2.3	Procedures

The overall procedures of data channel establishment in early media phase are illustrated in figure 6.2.3-1.

Figure 6.2.3-1: ADC establishment in early media phase

0.	The user of UE#2 subscribes Data Channel Application in Alerting Phase (DCAP) service to his/her serving IMS network. The data channel application to be initiated in alerting phase is selected by the user and configured in UE#2's subscription data.

1.	UE#1 sends a SIP INVITE targeting UE#2. The initial SDP contains offers for audio/video flow, bootstrap data channel. UE#1 indicates support for preconditions, reliable provisional responses, and indication of early-session supported.

2.	The IMS AS sends Nimsas_SessionEventControl_Notify to the selected DCSF to notify the session establishment.

3.	If the remote bootstrap data channel (BDC from UE#1 to the DCSF in the terminating network) is not requested by UE#1, the DCSF instructs the IMS AS to add the bootstrap data channel.

4.	The DCSF invokes the Nimsas_MediaControl_MediaInstruction instructing the IMS AS how to set up bootstrap data channel with MF both for originating and terminating side.

5.	The IMS AS instruct the MF to allocate required bootstrap data channel media resources.

6.	The IMS AS responds to the MediaInstruction request received in step 4.

7.	The DCSF responds to the Notify Request received in step 2.

8.	The IMS AS sends the INVITE to UE#2.

9.	UE#2 determines the subset of the media flows and responds with an Offer Response message back to the IMS AS.

10.	The IMS AS forwards the Offer Response message (e.g. 183 Session Progress) to UE#1. The IMS AS may add a remote bootstrap data channel with UE#1 if not request in the INVITE message.

11-12.	UE#1 reserves resource and confirms receipt of the Offer Response and sends the PRACK, which acknowledges the 183 (Session Progress) provisional response, towards UE#2.

13-14.	UE#2 reserves resource and sends a SIP 200 (OK) response for the SIP PRACK request to UE#1. The BDC between the DCSF and UE#1 is established and UE#1 downloads the DC application list from the DCSF.

15.	UE alerts the user and sends Ringing indication to the IMS AS.

16.	Base on UE#2's subscription of DCAP service and the availability of BDC to UE#1, the IMS AS decides to trigger the pre-configured data channel application to UE#1.

Editor's note:	When the IMS AS triggers the ADC is FFS (e.g. after receiving 183 or 180 message).

17.	The IMS AS notifies the DCSF with the request of data channel application (e.g. application ID) in alerting phase.

18-19.	The DCSF determines the media parameters for the application data channel and responses to the IMS AS.

20.	The IMS AS instructs the MF to reserve the media resource and MDC2 end points to the DC AS.

21.	The IMS AS response the DCSF the MediaInstruction with MDC2 end points information.

22.	The DCSF indicate the DC AS reserving the media resource for the application data channel and MDC2 end points to the MF.

23-24.	The DCSF response the notification in step 17 and instructs the MF to update the media resource.

25.	The IMS AS triggers the IMS session update to UE#1 containing SDP for the application data channel with an indication of early media.

26.	If the data channel application is not locally available, UE#1 downloads the application form the DCSF via the established bootstrap data channel.

27.	UE#1 confirms the update for the application data channel.

28.	UE#1 starts data channel application with the DC AS in early session.

29.	UE#2 accepts the session and sends 200 OK to the INVITE.

30-31.	The IMS AS updates UE#1 with SDP offer to initiate the media flow of the main session and terminate the application data channel in the early session.

32.	The IMS AS instructs the MF to update the media resource and terminate the application data channel.

33-34.	The IMS AS forwards 200 OK to UE#1. UE#1 sends ACK to UE#2.

35.	The media flow of the main session between UE#1 and UE#2 starts.

### 6.2.4	Impacts to Services, Entities and Interfaces

UE:

-	Indicates supporting of early media session in SIP INVITE message, as specified in existing specification, e.g. TS 24.229 [6].

-	Supports application data channel in the early session.

HSS:

-	Supports management of user subscription for Data Channel Application in Alerting Phase (DCAP) service.

-	Supports the user configuration of data channel application(s) in alerting phase.

IMS AS:

-	Triggers the application data channel in alerting phase.

-	Terminates the application data channel when the call accepted by the terminating UE.

-	Performs as B2BUA to update the media flow in the SDP exchanged with UE.

## 6.3	Solution #3: IMS ADC establishment during alerting phase when initial INVITE does have BDC offer

### 6.3.0	High-level solution Principles

The main principle of this solution for support of application data channel establishment during alerting phase of normal IMS DC session as follows:

-	The calling party will receive bootstrap data channel offer from the terminating network during alerting phase.

-	The calling party will establish the bootstrap data channel, download the application and based on the interest, trigger application data channel establishment by selecting one particular application from the terminating network.

-	The called party after getting notification about the application running status in the calling party device, will establish bootstrap data channel, download the application and once accepts the session, the call will be established.

### 6.3.1	Description

This paper proposes solution for supporting application data channel establishment during alerting phase when calling party has offered bootstrap data channel in the initial INVITE.

In this proposal, the calling party and the called party will establish bootstrap data channel(s), download application or application list and establish application data channel while call has not yet been established.

### 6.3.2	Procedures

The procedure of establishing application data channel during alerting phase.

Figure 6.3.3-1: ADC establishment in early media phase

1.	UE-A sends an initial SIP INVITE request to the originating IMS AS through P-CSCF and S-CSCF with an initial SDP offer for audio/video and for the bootstrap data channel establishment with bootstrap DC stream ID. The SDP contains bootstrap data channel offers for both the originating and the terminating side. P-CSCF may include the P-Early-Media header field to indicate the support for early media authorization in the initial SIP INVITE request.	NOTE: The UE is assumed to support P-Early-Media.

2-13.	Steps 2-13 of clause AC.7.1 in TS 23.228 [2] apply.

14.	UE-B returns a SIP 18X response with an SDP answer for bootstrap data channel to the originating network. The terminating IMS AS includes P-Early-Media header field in the SIP 18X response indicating early media is allowed based on the SDP answer not including any application data channel.

15.	The bootstrap data channel has been established between originating MF and UE-A. The UE shall request for a data channel application or data channel applications list from the MF via the established bootstrap data channel. The DCSF provides the data channel application or the data channel application list to UE-A through MF. UE-A may download the data channel application or the data channel application list through the established bootstrap data channel..

16.	The bootstrap data channel has also been established between terminating MF and UE-A.

17.	UE-A sends a SIP PRACK request towards the terminating side before or during step 15.

18.	UE-B returns a SIP 200 OK response for the PRACK. Simultaneously, UE-B may alert the terminating user that the incoming session is a normal IMS DC session.

19.	After the DC application is downloaded, UE-A initiates a SIP UPDATE request to update the IMS session for adding the application data channel and to inform UE-B about the associated DC application binding information. The IMS AS shall notify DCSF about the media change request event and request MF to allocate media resources for the application data channels based on instructions received from the DCSF if MF is anchoring application data channel. Once acknowledgement is received from the DCSF, the IMS AS shall send the update request to the terminating side and UE-B through S-CSCF. The SIP UPDATE request with the modified SDP offer includes the negotiated bootstrap data channel media description, requested application data channel media description and the associated data channel application binding information.

The IMS AS includes P-Early-Media header field in the SIP UPDATE request to indicate that early media is allowed based on the previously received SDP answer not indicating any application data channel.

20-22.	The bootstrap data channel has been established between the originating MF and UE-B. The bootstrap data channel has also been established between the terminating MF and UE-B. UE-B may alert the terminating user that the data channel application is running in UE-A and to download the data channel application if not already available in UE-B. UE-B shall request for the data channel application and the DC application list from the MF for downloading via the established bootstrap data channels if needed.

23-29.	When the DC application is available in UE-B, then the terminating network sends a 200 OK response to the originating side based on which the IMS AS notifies DCSF of the successful bootstrap DC and application DC establishment. Once the DCSF responds to the session establishment request, 200 OK response is forwarded to the UE-A. The originating network P-CSCF executes QoS procedures for bootstrap data channel. The IMS AS may include P-Early-Media header field in the 200 OK response to the SIP UPDATE request to indicate that early media is allowed. UE-B alerts the terminating user for running the DC application.

30.	UE-B returns a SIP 180 response indicating that UE-B is ringing.

31.	Once UE-B accepts to run the DC application, UE-B answers the IMS session. UE-B sends 200 OK to the initial SIP INVITE request and subsequent procedures follow.

Editor's note:	The consideration on originating user consent of NW initiated ADC during alerting phase is FFS.

### 6.3.3	Impacts on Services, Entities and Interfaces

Editor's note:	Impacts are FFS.

## 6.4	Solution #4: IMS ADC establishment during alerting phase when initial INVITE does not have BDC offer

### 6.4.0	High-level solution Principles

The main principle of this solution for support of application data channel establishment during alerting phase of normal IMS DC session as follows:

-	The calling party will receive bootstrap data channel offer from the terminating network during alerting phase.

-	The calling party will establish the bootstrap data channel, download the application and based on the interest, trigger application data channel establishment by selecting one particular application from the terminating network.

-	The called party after getting notification about the application running status in the calling party device, will establish bootstrap data channel, download the application and once accepts the session, the call will be established.

### 6.4.1	Description

This clause proposes a solution for supporting application data channel establishment during alerting phase when calling party has not offered bootstrap data channel in the initial INVITE.

In this proposal, one scenario is explained how the called party (e.g. restaurant customer service) network is providing bootstrap data channel offer to the calling party during alerting phase so that the calling party can establish bootstrap data channel, download the application or application list and then session is established once called party user accepts it.

Scenario.

-	The calling party triggers a normal call to restaurant customer service with intention of reserving seats for lunch.

-	The called party has taken an interactive ringback tone service which means whenever any incoming session is received for that restaurant, the network will offer bootstrap data channel and list of applications provided by the restaurant.

-	The calling party will establish the bootstrap data channel, download the list of applications and select "seat reserve" application.

-	The called party based on the notification of running application on the calling party side will download the application if not available and then accepts the session.

### 6.4.2	Procedures

The procedure of establishing application data channel during alerting phase.

Figure 6.4.3-1: ADC establishment in early media phase

1-2.	UE-A sends an initial SIP INVITE request to the terminating side through P-CSCF and S-CSCF with an initial SDP offer for audio/video. P-CSCF may include the P-Early-Media header field to indicate the support for early media authorization in the initial SIP INVITE request.

NOTE:	The UE is assumed to support P-Early-Media.

3.	UE-B returns a SIP 18X response with an SDP answer for audio/video session to the originating network. The terminating IMS AS includes P-Early-Media header field in the SIP 18X response indicating early media is allowed.

4-5.	UE-A sends a SIP PRACK request towards the terminating side and UE-B returns a SIP 200 OK response for the PRACK.

6.	UE-B sends a SIP UPDATE request to the terminating IMS AS through P-CSCF and S-CSCF with an SDP offer for audio/video and bootstrap data channel. The SDP contains bootstrap offer for the terminating side. Steps 2-10 of clause AC.7.1 in TS 23.228 [2] apply in which the terminating IMS AS determines to notify the terminating DCSF based on DC subscription and UE-B DC capability and the DCSF instructs the IMS AS and MF to establish the resource required for the terminating side. In this scenario, the DCSF allows for application selection from the remote UE, even before the application has been selected by the peer UE.

7.	IMS AS sends the modified SIP UPDATE request including the SDP offer for audio/video and bootstrap data channel adding media information of MF to the originating side and UE-A.

8.	Originating IMS AS determines to notify DCSF based on DC subscription and UE DC capability. If the network does not support DC service or UE-A does not have DC capability, then the originating network sends an SDP answer to setup only audio/video session and remove any bootstrap data channel media information.

9.	The bootstrap data channel has been established between terminating MF and UE-A. The UE shall request for a data channel application or data channel applications list from the MF via the established bootstrap data channel. The DCSF provides the data channel application or the data channel application list to UE-A through MF. UE-A may download the data channel application or the data channel application list through the established bootstrap data channel.

Editor's note:	The consideration on originating user consent of NW initiated ADC during alerting phase is FFS.

10.	Once the bootstrap data channel has been established, UE-A sends 200 OK response for the SIP UPDATE request to the terminating side.

11.	UE-B alerts the terminating user to indicate an incoming IMS session.

12.	After the DC application is downloaded, UE-A initiates a SIP UPDATE request to update the IMS session for adding the application data channel and to inform UE-B about the associated DC application binding information. The IMS AS shall notify DCSF about the media change request event. Once acknowledgement is received from the DCSF, the IMS AS shall send the update request to the terminating side and UE-B through S-CSCF. The SIP UPDATE request with the modified SDP offer includes the negotiated bootstrap data channel media description, requested application data channel media description and the associated data channel application binding information.

The IMS AS includes P-Early-Media header field in the SIP UPDATE request to indicate that early media is allowed based on the previously received SDP answer not indicating any application data channel.

13-14.	The bootstrap data channel has been established between the terminating MF and UE-B. UE-B may alert the terminating user that the data channel application is running in UE-A and to download the data channel application if not already available in UE-B. UE-B shall request for the data channel application and the DC application list from the MF for downloading via the established bootstrap data channels if needed.

15-21.	When the DC application is available in UE-B, then the terminating network sends a 200 OK response to the originating side based on which the IMS AS notifies DCSF of the successful bootstrap DC and application DC establishment. Once the DCSF responds to the session establishment request, 200 OK response is forwarded to the UE-A. The originating network P-CSCF executes QoS procedures for bootstrap data channel. The IMS AS may include P-Early-Media header field in the 200 OK response to the SIP UPDATE request to indicate that early media is allowed.

22.	UE-B alerts the terminating user for running the DC application.

23.	The UE-B returns a SIP 180 response to indicate the UE-B is ringing.

24.	Subsequent procedures continue. When the terminating user accepts to run the DC application for communication, the UE#2 answers the IMS session.

Editor's note:	How to handle the both the existing ring back tone and interactive ring back tone together is FFS.

Editor's note:	The BDC and ADC procedure should be executed before or after 180 ringing is FFS.

### 6.4.3	Impacts on Services, Entities and Interfaces

This solution may have the following impacts to existing entities and interfaces:

IMS AS:

-	Terminating IMS AS will provide a new offer for IMS bootstrap data channel based on the interactive ringback tone service subscription of called party user.

Editor's note:	Additional Impacts are FFS.

## 6.X	Solution #X: <Solution Title>

Editor's note:	This Rapporteur is picky - please use unique descriptive titles for all solution proposals.

### 6.X.0	High-level solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Further (sub-)clause(s) may be added to capture details.

### 6.X.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

### 6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#Y

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#Z

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future release.

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Annex A:
Change History

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA2#170 | S2-2507417 | - | - | - | Skeleton approved at SA2#170 | 0.0.0 |
| 2025-08 | SA2#170 | S2-2507417, S2-2507546, S2-2507547, S2-2507551, S2-2507610, S2-2507611, S2-2507612, S2-2507614, S2-2507615, S2-2507648, S2-2507655, S2-2507656 | - | - | - | Baseline Doc, Scope, References, Architectural Assumptions Key issue 1, 2 &3, Solution 1, 2, 3, &4 | 0.1.0 |