| 3GPP TR 23.700-83 V1.2.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application layer support for AI/ML services Phase 2; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	13

Introduction	14

1	Scope	15

2	References	15

3	Definitions of terms, symbols and abbreviations	15

3.1	Terms	15

3.2	Symbols	16

3.3	Abbreviations	16

4	Architectural Assumptions and Principles	16

4.1	Analysis of relation between AIMLE and ADAE services	16

4.2	Architectural Assumptions	16

5	Key issues	17

5.1	Key issue #1: Key issue on support for ML model inference	17

5.1.1	Description	17

5.1.2	Open Issues	17

5.2	Key Issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer	17

5.2.1	Description	17

5.2.2	Open Issues	18

5.3	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios	18

5.3.1	Description	18

5.3.2	Open Issues	19

5.4	Key issue #4: Support for Split AI/ML Operations Involving Multiple AIMLE clients, edge AIMLE servers and central AIMLE servers	19

5.4.1	Description	19

5.4.2	Open Issues	19

5.5	Key issue #5: Support for AI Inference Exposure	20

5.5.1	Description	20

5.5.2	Open Issues	20

5.6	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios	20

5.6.1	Description	20

5.6.2	Open Issues	21

5.7	Key Issue #7: VFL sample alignment across VAL servers	21

5.7.1	Description	21

5.7.2	Open Issues	21

5.8	Key Issue #8: Support for AI Inference service performance management and feedback	22

5.8.1	Description	22

5.8.2	Open Issues	22

6	Enhanced architecture for enabling AI/ML services	22

6.1	General	22

6.2	Enhanced AI/ML enablement functional model	22

6.2.2	AI/ML enablement functional model for roaming scenarios	25

6.3	Functional Elements	26

6.4	Reference Points	26

6.4.1	AIML-E	26

6.4.2	AIML-R	26

6.4.3	AIML-PC5	26

7	Solutions	26

7.0	Mapping of solutions to key issues	26

7.1	Solution #1: Enhancement of AIMLE to support model inference	27

7.1.1	Solution description	27

7.1.2	Architecture Impacts	28

7.1.3	Corresponding APIs	28

7.1.4	Solution evaluation	29

7.2	Solution #2: Enhance AIMLE Data Management Assistance for Data Drift Detection	30

7.2.1	Solution Description	30

7.2.1.1	Impact to existing AIMLE procedures	30

7.2.2	Architecture Impacts	35

7.2.3	Corresponding APIs	35

7.2.4	Solution evaluation	35

7.3	Solution #3: New AIMLE Service on ML Model Maintenance	35

7.3.1	Solution Description	35

7.3.1.1	Procedure	35

7.3.1.2	Information flows	37

7.3.1.2.1	ML model maintenance subscription request	37

7.3.1.2.2	ML model maintenance subscription response	37

7.3.1.2.3	ML model maintenance notification	37

7.3.2	Architecture Impacts	38

7.3.3	Corresponding APIs	38

7.3.4	Solution evaluation	38

7.4	Solution #4: Management of evaluation information for ML model performance and correctness evaluation.	38

7.4.1	Solution description	38

7.4.1.1	Introduction	38

7.4.1.2	ML model evaluation information storage	39

7.4.1.3	ML model evaluation information retrieval	39

7.4.1.4	ML model evaluation information removal	40

7.4.2	Architecture Impacts	41

7.4.3	Corresponding APIs	41

7.4.3.1	ML Evaluation Profile	41

7.4.3.2	ML Evaluation Information Storage	42

7.4.3.3	ML Evaluation Information Retrieval	43

7.4.3.4	ML Evaluation Information Removal	43

7.4.4	Solution evaluation	44

7.5	Solution #5: Solution on monitoring ML-enabled analytics correctness	44

7.5.1	Solution description	44

7.5.2	Architecture Impacts	46

7.5.3	Corresponding APIs	46

7.5.4	Solution evaluation	46

7.6	Solution #6: Enhancements to performance monitoring	46

7.6.1	Solution description	46

7.6.1.1	Enhancements to ML model performance monitoring	46

7.6.1.2	Enhancements to HFL training	49

7.6.2	Architecture Impacts	51

7.6.3	Corresponding APIs	51

7.6.4	Solution evaluation	51

7.7	Solution #7: Solution on discovering AIMLE clients in cross-PLMN scenarios	51

7.7.1	Solution description	51

7.7.2	Architecture Impacts	54

7.7.3	Corresponding APIs	54

7.7.4	Solution evaluation	56

7.8	Solution #8: Multi-Client Split AI/ML Operations Pipeline	56

7.8.1	Solution Description	56

7.8.1.1	General	56

7.8.1.2	Multi-client pipeline operation	57

7.8.1.3	Multi-client pipeline service continuity	58

7.8.1.4	Procedures to support Multi-client AI/ML split operation	58

7.8.1.4.1	Split operation pipeline join	58

7.8.1.4.2	Split operation notification	59

7.8.2	Architecture Impacts	60

7.8.3	Corresponding APIs	60

7.8.3.1	Split operation pipeline discovery request	60

7.8.3.2	Split operation pipeline discovery response	61

7.8.3.3	Split operation pipeline create request	61

7.8.3.4	Split operation pipeline join request	62

7.8.3.5	Split operation pipeline join response	62

7.8.4	Solution Evaluation	62

7.9	Solution #9: AIMLE server registration in a hierarchical AIMLE deployment	63

7.9.1	Solution description	63

7.9.1.1	AIMLE server registration	63

7.9.1.2	AIMLE server registration update	64

7.9.1.3	AIMLE server de-registration	65

7.9.2	Architecture impacts	65

7.9.3	Corresponding APIs	65

7.9.3.1	AIMLE server registration request	65

7.9.3.2	AIMLE server registration response	66

7.9.3.3	AIMLE server registration update request	66

7.9.3.4	AIMLE server registration update response	67

7.9.3.5	AIMLE server de-registration request	67

7.9.3.6	AIMLE server de-registration response	67

7.9.4	Solution evaluation	68

7.10	Solution #10: Support of AIMLE services for roaming UEs	68

7.10.1	Solution description	68

7.10.1.1	Introduction	68

7.10.1.2	Impact to existing AIMLE client registration	68

7.10.2	Architecture Impacts	76

7.10.3	Corresponding APIs	76

7.10.4	Solution evaluation	76

7.11	Solution #11: Solution of FL member registration in multi-operator scenarios	77

7.11.1	Solution description	77

7.11.2	Architecture Impacts	78

7.11.3	Corresponding APIs	78

7.11.4	Solution evaluation	78

7.12	Solution #12: Support for ML model split learning using relays	79

7.12.1	Solution description	79

7.12.1.1 	Intermediate node registration procedure	79

7.12.1.2 	Intermediate node discovery procedure	81

7.12.2	Architecture Impacts	83

7.12.3	Corresponding APIs	84

7.12.4	Solution evaluation	84

7.13	Solution #13: ML model training in a hierarchical AIMLE deployment	84

7.13.1	Solution description	84

7.13.2	Architecture impacts	85

7.13.3	Corresponding APIs	86

7.13.3.1	Edge ML model training subscription request	86

7.13.3.2	Edge ML model training subscription response	87

7.13.3.3	Edge ML model training notification	88

7.13.4	Solution evaluation	88

7.14	Solution #14: Enhance AIMLE Client Discovery	89

7.14.1	Solution Description	89

7.14.2	Architecture Impacts	91

7.14.3	Corresponding APIs	91

7.14.4	Solution evaluation	91

7.15	Solution #15: Sample alignment for VFL	92

7.15.1	Solution description	92

7.15.2	Architecture Impacts	96

7.15.3	Corresponding APIs	96

7.15.4	Solution evaluation	96

7.16	Solution #16: Solution on AIMLE service migration support in UE roaming scenarios	96

7.16.1	Solution description	96

7.16.2	Architecture Impacts	98

7.16.3	Corresponding APIs	98

7.16.4	Solution evaluation	99

7.17	Solution #17: Enhancement of AIMLE to support model inference	99

7.17.1	Solution description	99

7.17.1.1	Information Flows	101

7.17.1.1.1 	ML model collaborative inference request	101

7.17.1.1.2	ML model collaborative inference response	101

7.17.1.1.3 	Intermediate inference output	101

7.17.2	Architecture Impacts	102

7.17.3	Corresponding APIs	102

7.17.4	Solution evaluation	102

7.18	Solution #18: Solution on support for federated AI services	102

7.18.1	Solution description	102

7.18.2	Architecture Impacts	104

7.18.3	Corresponding APIs	104

7.18.4	Solution evaluation	104

7.19	Solution #19: Cross-PLMN/Domain AIMLE client discovery, selection, monitoring	105

7.19.1	Solution Description	105

7.19.1.1	General	105

7.19.1.2	Procedure	105

7.19.2	Architecture Impacts	109

7.19.3	Corresponding APIs	109

7.19.3.1	AIMLE client selection subscription request	109

7.19.3.2	AIMLE client subscription update request	109

7.19.4	Solution Evaluation	110

7.20	Solution #20: ML Model Performance Evaluation with information collection from ML model consumer	110

7.20.1	Solution Description	110

7.20.1.1	Procedure	111

7.20.1.2	Information Flows	118

7.20.1.2.1 	ML model evaluation subscription request	118

7.20.1.2.2 	ML model evaluation subscription response	118

7.20.1.2.3	ML model performance information collection request	119

7.20.1.2.4	Information collection request	119

7.20.1.2.5	Information collection response	119

7.20.1.2.6	ML model performance information collection response	120

7.20.1.2.7	ML model evaluation notification	120

7.20.2	Architecture Impacts	120

7.20.3	Corresponding APIs	121

7.20.4	Solution evaluation	121

7.21	Solution #21: Split operation node registration involving multiple AIMLE clients	121

7.21.1	Solution description	121

7.21.2	Architecture Impacts	126

7.21.3	Corresponding APIs	126

7.21.4	Solution evaluation	126

7.22	Solution #22: Sample Alignment Enablement for VAL Servers in Vertical Federated Learning	127

7.22.1	Solution Description	127

7.22.1.1	General	127

7.22.1.2	Procedures	127

7.22.2	Architecture Impacts	129

7.22.3 	Corresponding APIs	129

7.22.3.1	VFL member registration request	129

7.22.3.2	VFL member registration update request	129

7.22.3.3	VFL training/inference request	130

7.22.3.4	VFL training/inference initiation request	130

7.22.3.5	VFL training/inference initiation response	131

7.22.3.6	VFL training/inference response	131

7.22.4 	Solution Evaluation	131

7.23	Solution #23 enhancement of AIMLE to support AI Inference service performance management and feedback	132

7.23.1	Solution description	132

7.23.2	Architecture Impacts	134

7.23.3	Corresponding APIs	134

7.23.3.1	ML model inference service performance feedback subscription request	134

7.23.3.2	ML model inference service feedback subscription response	135

7.23.3.3	ML model inference service feedback notification	135

7.23.3.4	ML model inference service feedback notify acknowledgement	136

7.23.4	Solution evaluation	136

7.24	Solution #24: ML model inference in a hierarchical AIMLE deployment	137

7.24.1	Solution description	137

7.24.2	Architecture Impacts	139

7.24.3	Corresponding APIs	139

7.24.3.1	Edge ML model inference request	139

7.24.3.2	Edge ML model inference response	140

7.25	Solution #25: Enhancement of AIMLE to support AI Inference exposure	141

7.25.1	Solution description	141

7.25.2	Architecture Impacts	142

7.25.3	Corresponding APIs	142

7.25.3.1	ML model inference service discovery request	142

7.25.3.2	ML model inference service discovery response	143

7.25.4	Solution evaluation	144

7.26 	Solution #26: AIMLE server discovery	144

7.26.1 	Solution Description	144

7.26.1.1 	General	144

7.26.1.2 	Procedure	144

7.26.1.3	Information Flows	145

7.26.1.3.1 	AIMLE server discovery request	145

7.26.1.3.2	 AIMLE server discovery response	145

8	Deployment scenarios	147

8.1	General	147

8.2	Deployment model #4: Centralized AIMLE server in multi-operator scenarios	147

9	Business Relationships	150

10	Overall evaluation	152

10.1.1	Summary of new AIMLE services	152

10.2	Key issue #1: Key issue on support for ML model inference	153

10.3	Key issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer	154

10.4	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios	154

10.5	Key issue #4: Support for Split AI/ML Operations Involving Multiple AIMLE clients, edge AIMLE servers and central AIMLE servers	155

10.6	Key issue #5: Support for AI Inference Exposure	156

10.7	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios	156

10.8	Key Issue #7: VFL sample alignment across VAL servers	157

10.9	Key Issue #8: Support for AI Inference service performance management and feedback	157

11	Conclusions	158

11.1	General conclusions and recommendations	158

11.2	Conclusions of key issue #1	158

11.3	Conclusions of key issue #2	158

11.4	Conclusions of key issue #3	158

11.5	Conclusions of key issue #4	159

11.6	Conclusions of key issue #5	159

11.7	Conclusions of key issue #6	159

11.8	Conclusions of key issue #7	159

Annex A (informative): Change history	161

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

As part of Rel-19, SA6 normatively specified the application layer architecture to enable AI/ML services which introduced as a new SEAL service, aka AIMLE, as well as a new ML repository, in 3GPP TS 23.482 [2]. Furthermore, as part of Rel-19 work, in 3GPP TS 23.436 [3] enhancements have been specified for supporting ML-enabled analytics.

In Rel-20, the AIMLE functional architecture requires enhancements to further improve and enhance functionality in 3GPP for improved support for AI/ML services. In this direction, some already defined concepts need further study related to application layer ML model performance monitoring and correctness evaluation, extensions to AIML split operation, enhancements in VFL support considering also VFL participants at the edge/cloud; and enhancements for supporting different deployments and federation/multi-operator scenarios.

This technical report identifies the key issues and corresponding application architecture enhancements and related solutions with recommendations for the normative work.

# 1	Scope

The present document is a technical report which identifies enhancements to the application enablement layer architecture, capabilities, and services to support AI/ML services at the application layer.

The aspects of the study include the continuation of the investigation of application enablement impacts, the application enablement layer architecture enhancements and solutions needed to provide assistance in AI/ML operations at the VAL layer as well as at the application enablement layer (e.g., SEAL ADAES, EDGEAPP).

The study takes into consideration the work done for AI/ML in 3GPP TS 23.436 [3] and 3GPP TS 23.482 [2] and can consider other AI/ML related works within 3GPP.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service	".

[3]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Vertical Federated Learning (VFL): A federated learning technique in which multiple VFL participants jointly perform training and inference on a common sample space with different feature sets, without exchanging raw data, thereby preserving their local data privacy.

VFL process: A process of performing training or inference using the VFL technique.

VFL participant: An entity that takes part in a VFL process, either as a VFL server or a VFL client.

VFL server: A VFL participant that coordinates the VFL process by aggregating results from VFL clients and propagating updates to them during training.

VFL client: A VFL participant that performs local training and inference, sends results to the VFL server, and updates local parameters based on information received from the VFL server.

VFL sample alignment: A stage in a VFL process during which samples are aligned among VFL participants, ensuring that training and inference are performed consistently across participants on the same sample space.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architectural Assumptions and Principles

## 4.1	Analysis of relation between AIMLE and ADAE services

In this clause an analysis of the relation of ADAE analytics and AIMLE potential service on ML model inference is discussed. Initially a definition is provided about the possible role of AIMLE vs ADAES in inference and analytics:

1.	Analytics as exposed by ADAE, does not necessarily rely on AI/ML models. The consumer simply requests analytics and receives insights—without needing to know or specify how those insights are derived. Thus, when requesting analytics a consumer does not provide any information in relation to the ML model even if such model is used as it is abstracted away from it.

2.	Inference, on the other hand, is fundamentally tied to the use of a specific ML model. The consumer must either provide a model, model information or define criteria for selecting a model. This makes the ML model an integral part of the inference service. While inference can be used to support analytics, it does not itself expose analytics as a service.

## 4.2	Architectural Assumptions

The following high-level assumptions and principles will be considered in this study for ADAES:

1.	ADAES is the entity providing analytics which can be either ML-enabled or not.

2.	If ADAES analytics is ML-enabled, the ML training capability is provided by AIMLE (Rel-19 AIMLAPP).

3.	If ADAES analytics is ML-enabled, the ML inference capability can be provided as part of the analytics functionality inside ADAES (internal to ADAES logic). ML model inference is one of methods utilized by ADAE for Data analytics.

The following high-level assumptions and principles will be considered in this study for AIMLE:

1.	AIMLE provides ML support capabilities as of Rel-19 for both VAL and other SEAL servers (Rel-19 AIMLAPP).

2.	AIMLE provides ML model training capability for both VAL task and ADAE analytics (Rel-19 AIMLAPP).

3.	AIMLE is not expected to provide ML model inference for tasks related to ADAE analytics.

4.	AIMLE can only be seen as a candidate entity for performing ML model inference for VAL/ASP automation tasks (needs to be discussed further within Rel-20 study).

The following high-level assumptions and principles will be considered in this study for ML repository:

1.	The ML repository is the unique repository / registry per SEAL provider for the application-layer ML models and the parameters required for the AIMLE operations (Rel-19 AIMLAPP).

2.	Different deployments and cardinalities of ML repository within the SEAL provider's domain will be studied as part of Rel-20 work.

# 5	Key issues

## 5.1	Key issue #1: Key issue on support for ML model inference

### 5.1.1	Description

ML model inference, according to 3GPP TS 28.105, is a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

ML model inference may be provided by an analytics entity (e.g. ADAES) or may be provided by VAL server (in case of VAL-provided analytics) or potentially by AIMLE if the scenario involves the VAL server to request from AIMLE to provide ML model inference functionality (e.g. using network data or edge platform data).

In Rel-19 AIML_App, there was some basic support for operations related to split ML model inference; however, the use of ML model inference for supporting ADAE-layer analytics as well as the AIMLE support for enabling ML model inference (as part of the ML model lifecycle operations) was not discussed.

In this key issue, further enhancements in both AIMLE and ADAES will be explored for supporting ML model inference in ADAES and AIMLE, considering also impacts on the interfaces among AIMLE and ADAE entities in distributed AIMLE / ADAES deployments.

### 5.1.2	Open Issues

The following open issues need to be studied:

1.	Identify deployment models for supporting ML model inference in enablement layer.

2.	Whether and how ADAES needs be enhanced or leverage AIMLE for supporting ML model inference for ADAE analytics.

3.	Whether and how AIMLE needs to be enhanced to enable ML model inference to address VAL request (e.g. for VAL automation task or analytics).

4.	Whether and how to enhance AIMLE to support model inference in edge scenarios.

5.	Whether and how to enhance AIMLE architecture and functions to support collaborative inference between AIMLE client and edge AIMLE server.

NOTE:	Solutions for the above key issue should not introduce any inference services exposed by AIMLE (e.g., producing prediction result to third party).

## 5.2	Key Issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer

### 5.2.1	Description

KI #2 focuses on providing enhanced means to evaluate and determine ML model performance and correctness in the AIML Enablement Layer.

The evaluation of performance and correctness of a ML model may depend on the ML model, and the means for evaluating performance and correctness of a ML model may vary depending on the ML model usage and purpose.

ML model performance and correctness evaluation may require evaluating different aspects of the ML model. For example, the performance and correctness of a ML model may be related to its robustness (e.g., capability for handling unseen or drifting underlying data) or may be associated with its stability (e.g., small changes in input not leading to drastic prediction change).

A procedure to monitor ML model performance degradation by VAL server has been defined in clause 8.22 of 3GPP TS 23.482 [2]. However, this procedure is limited to evaluating performance degradation at the AIMLE client for ML model training and HFL use cases and does not clearly define the information needed for the AIML enablement layer to perform performance and correctness evaluation.

### 5.2.2	Open Issues

This key issue will study the following AIML enablement layer aspects:

1.	How to define ML model performance and ML model correctness in the context of the AIML enablement layer.

2.	Whether and how to evaluate ML model performance and correctness considering application specific ML models.

3.	Whether and how to monitor (e.g., trigger, detect and report) ML model performance and correctness degradation for different ML-enabled operations (e.g., inferencing, training, split AIML operation, HFL, VFL, transfer learning) and considering data managements assistance.

4.	Whether and how to trigger corrective actions on ML model performance and correctness degradation detection (e.g., model maintenance).

## 5.3	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios

### 5.3.1	Description

As part of Rel-19, AIMLE was introduced as an enabler for application-layer AI/ML services, supporting model training, distribution primarily within a single operator's network domain. However, as AI/ML services increasingly involve VAL UEs moving across network domains (e.g., cross-PLMN mobility, UE roaming, federation across operators), and AI/ML services becoming increasingly distributed and dynamic, the ability to ensure service continuity and mobility of AIMLE functions becomes a critical requirement.

This key issue aims to study how AIMLE services can support mobility, including roaming scenarios where UEs access AI/ML services in visited networks, and cross-PLMN environments where AI/ML operations involve entities from different operators. This will address both the continuity of AI/ML services and the federation across MNOs and CSPs/ECSPs.

Additionally, the roaming aspects of VAL UEs do not address how the AIMLE entities (AIMLE server and/or ML repository) are aware of the serving PLMN / network information of the UE associated with the AIMLE client. For example, when a UE that is associated with a AIMLE client, moves from PLMN A to PLMN B. Then, in such case, the AIMLE server and/or ML repository may not be aware of this movement of the UE associated with the VAL AIMLE client. Further, the AIMLE server and/or ML repository are not aware of the serving PLMN information of the UE associated with a AIMLE client in order to provide the required support in cross-PLMN roaming scenarios.

In particular, as ML model inference increasingly moves toward the network edge to meet ultra-low latency and bandwidth efficiency demands (e.g., in real-time video analytic, industrial automation), maintaining inference continuity across mobility events becomes critical. When VAL UEs move across network boundaries or service areas, edge AIMLE instances may need to be reselected, dynamically discovered, or seamlessly replaced to ensure uninterrupted AI/ML inference execution and delivery.

The current AIMLE specifications and ongoing studies focus on static deployments and assume the VAL UE remains within its home network. However, real-world deployments often require:

1.	VAL UEs to maintain access to AIMLE services when roaming into visited PLMNs or NPNs,

2.	Cross-border AI/ML service collaboration (e.g. in eV2X or multiplayer gaming scenarios),

3.	Edge inference or AI model access to continue seamlessly even as the UE moves between coverage areas.

Also, the current AIML specification and other aspects of the ongoing studies do not focus on various conditions (for e.g., availability of AIMLE servers, load conditions of serving AIMLE servers, change in ML tasks requirement from AIMLE client, etc.) that can trigger the mobility of AIMLE context. Only VAL UE's mobility is considered.

### 5.3.2	Open Issues

Therefore, the following aspects need to be studied:

1.	Identify the use cases for supporting multiple PLMN/NPN, and across MNOs and CSPs/ECSPs scenarios

2.	Whether and how AIMLE services can support VAL UEs that roam into visited PLMNs or NPNs, e.g. registration, discovery, and access of AIMLE in the visited domain, as well as go back to home domain.

3.	Whether and how to ensure service continuity and switching for inference tasks or AI service sessions during UE mobility across MNOs and CSPs/ECSPs.

4.	Whether and how UEs or VAL functions can discover the most suitable AIMLE instance in edge-specific scenarios while UEs move across network boundaries or service areas, considering latency, capability, and service policy.

5.	Whether and how the AIMLE server and/or ML repository are aware of the serving PLMN/network information of the UE associated with the AIML client.

6.	Whether and how to enhance AIMLE context transfer capability given various conditions related to the AIMLE server capability, load or availability changes.

NOTE: Open issues 2, 3, 4, 5 and 6 will be based on the identified use cases in open issue 1.

## 5.4	Key issue #4: Support for Split AI/ML Operations Involving Multiple AIMLE clients, edge AIMLE servers and central AIMLE servers

### 5.4.1	Description

Split AI/ML operations have been introduced in 3GPP TS 23.482 [2]. The current framework is for single AIMLE client-to-network interaction and does not consider multiple AIMLE clients collaborating in a split AI/ML scenario. If  multiple AIMLE clients were to collaboratively execute split AIML operations within the same pipeline involving an Edge Server, the operation continuity may be disrupted if one client moves out of the service area during execution. Therefore, it is important to study the service continuity and optimization for split AIML operations involving multiple AIML clients.

In addition to the above multi-client scenario, emerging AI/ML applications require collaborative inference across multiple computing nodes (UE(s), edge(s), and cloud(s)) rather than a single operation. For example, in an autonomous driving or V2X scenario, a VAL UE (e.g. a connected vehicle) may only execute part of a model locally and rely on a roadside edge server or even a cloud server to complete the remaining inference. This is due to the vehicle's limited computing resources and the need for low-latency results to ensure safe and efficient driving decisions.

In Release-19, basic support for split ML model inference between an AIMLE client and an AIMLE server was introduced in 3GPP TS 23.482 [2]). However, those solutions assumed a relatively static allocation of inference tasks and did not fully explore more dynamic or collaborative inference involving multiple nodes. Therefore, it is necessary to study whether and how to enhance the AIML application enablement architecture and functions to support cloud-edge-device collaborative inference considering the conditions changes on these computing nodes (e.g. due to variations in edge server load).

### 5.4.2	Open Issues

This key issue will study:

1.	Define the use cases supported in the AIML enablement layer involving multiple AIMLE clients, edge AIMLE servers and central AIMLE servers requiring concurrent usage of the same split AI/ML operation pipeline.

2.	Whether and how the AIML enablement layer can support split AI/ML operations involving multiple AIMLE clients, edge AIMLE servers and central AIMLE servers.

3.	Whether and how the AIMLE enablement layer can support service continuity during split AIML operation involving multiple AIMLE clients.

4. Whether and how to support dynamic inference partitioning and coordinating across AIMLE clients, edge AIMLE servers and central AIMLE servers in AIMLE layer.

NOTE:	This open issue may involve the ability to split an AI/ML inference pipeline into segments that can be executed on different nodes based on real-time conditions such as latency requirements, processing capabilities, network bandwidth, and current load (which may be provided by VAL UE or 5GC).

## 5.5	Key issue #5: Support for AI Inference Exposure

### 5.5.1	Description

With the increasing deployment of ML model at the edge and cloud, the need for reusable, secure, and discoverable inference capabilities is becoming critical for enabling intelligent applications in verticals such as industrial automation, automotive, etc.

Currently, AI/ML inference is typically tightly coupled to the operator's own applications or specific vertical services. There is a growing demand to open up these capabilities as a standardized inference service that can be consumed by third-party applications or other service enablers. By exposing inference functions through standardized APIs, operators can enable new use cases, for example, an enterprise application could send sensor data to a mobile edge and request an AI-based analysis (inference) result in real time based on a suitable ML model. This requires exposing the inference capabilities of ADAES with the support from AIMLE services Therefore, enhancements of ADAES is needed to support the exposure of inference services based on the suitable ML model provided by AIMLE service.

### 5.5.2	Open Issues

Therefore, in order to support ML Model inference services exposure, the following aspects need to be studied:

1.	Whether and how to enhance ADAES service capabilities to expose ML model inference result based on the suitable ML model provided by AIMLE service in both central and edge scenarios.

## 5.6	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios

### 5.6.1	Description

SA6 has specified the application layer architecture to enable AI/ML services. In Rel-20, the AIMLE functional architecture requires enhancements to further improve and enhance functionality in 3GPP for improved support for AI/ML services. It requires for more investigation on the implications when supporting AIMLE capabilities in scenarios where the AIMLE is deployed in distributed or hierarchical fashion as indicated in Annex A.3 and A.4 of 3GPP TS 23.482 [2]. In this study, it is expected to explore potential enhancement for supporting such deployments, considering also different implementations of the ML repository in edge or cloud platforms, and different assumptions on the access of ML repositories by VAL and other enablers. In this regard, considering the various AIML capabilities provided by the VAL UE, the collaboration procedures among AIMLE Clients, edge AIMLE servers, and central AIMLE servers need to be enhanced in terms of data management and AIML operation.

In Rel-19, the AIMLE capabilities assume that the VAL server and AIMLE servers are pre-configured with AIMLE server endpoint information (e.g., URI, IP address). Such pre-configured AIMLE server endpoint information poses the following issues or architectural limitations:

- Limited flexibility – VAL server may need to dynamically select AIMLE server based on location, application domain. For example, a healthcare related VAL server may need to communicate with a dedicated AIMLE server for healthcare for privacy reasons.

- Incompatible with mobile users – If the user location changes, the pre-configured AIMLE server may not be able to serve the user or AIMLE client. This may lead to service disruption. The current specification includes AIMLE context transfer but only after the client is associated with a new server.

- Reduced resilience – If AIMLE server becomes unreachable, there is no fall-back mechanism for the VAL server or AIMLE clients. AIMLE server can enable a graceful shutdown by informing VAL server and AIMLE clients about an alternative server end-point information. However, in case of abrupt shutdown, there is no way to get the information related to AIMLE server.

- Multi-tenant environments – In a multi-tenant setup, different AIMLE servers can enforce different trust zones or security policies. VAL servers must find an AIMLE server that is compliant with the tenant profile.

The need for availing services from another AIMLE server may arise at AIMLE client for various conditions like change in AIMLE client's ML task requirement, AIMLE client's mobility planning/requirements etc. There are no mechanisms specified in Rel-19 for discovery of AIMLE servers.

### 5.6.2	Open Issues

This key issue will study:

1.	Study and identify the use cases and definition of hierarchical and distributed AIMLE architecture.

2.	Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

3.	Whether and how to enhance the architecture and related functions to support for AIMLE services, AIML operation assuming diverse (e.g., hierarchical and distributed) deployments scenarios.

4.	Whether and how to enhance data management assistance among AIMLE Clients, edge AIMLE servers, and central AIMLE servers in diverse (e.g., hierarchical and distributed) AIMLE deployments scenarios.

5.	Whether and how the AIMLE services can support the discovery of AIMLE servers.

6.	Whether and how the conditions under which a change of AIMLE server is identified.

## 5.7	Key Issue #7: VFL sample alignment across VAL servers

### 5.7.1	Description

In the Vertical Federated Learning (VFL) process, the active party or coordinating server collects partial results from other participants (passive parties) to obtain a global model. VFL participants can be VAL servers, AIMLE servers, and/or AIMLE clients. VFL participants share the same sample space but have different feature sets. Therefore, to achieve VFL process, these different VFL participants require sample alignment to participate in a VFL operation. In some cases, how these samples are identified by different participants can also differ. Sample alignment is crucial for this process as it allows matching corresponding data entries across different datasets in a privacy-preserving manner.

The AIMLE server, which has the ability to know the identity of VAL UEs, can accomplish sample alignment securely and in a privacy-preserving way without revealing the VAL UE identity.

### 5.7.2	Open Issues

This key issue will study:

1.	Whether and how to enhance VFL operation considering also VAL servers, AIMLE clients and AIMLE servers as VFL participants.

2.	Whether and how to support/enhance sample alignment also considering VAL servers / AIMLE servers / AIMLE clients as VFL participants.

## 5.8	Key Issue #8: Support for AI Inference service performance management and feedback

### 5.8.1	Description

AIMLE architecture currently provides mechanisms to request ML inference but lacks support for managing and observing the runtime quality of inference services. For example, in a vehicle-road collaboration scenario, a vehicle may request a semantic perception result that must be returned within 50ms. If that is not achieved, the application should be notified. Similarly, in scientific computing, model inference may degrade in accuracy due to hardware limitations, the feedback to the control plane is needed. Therefore, AIMLE architecture should consider providing:

-	A way for VAL UEs or AIMLE clients to express desired service performance parameters when requesting inference (e.g., max latency, expected confidence level),

-	Runtime monitoring of whether these conditions are met, and notification or reporting mechanisms for inference failures, degradation, or exceptions,

Although AI models are central to inference, the service performance of inference delivery depends on more than the model itself. The same model may produce different service experiences depending on runtime conditions (e.g., server load, network delays). The QoS of a model is generally static unless the model is retrained, whereas the performance of inference services may vary depending on resource conditions. Even if the model degrades, the inference service may still meet the required service performance.AI inference service consumers care about whether the results are delivered on time, not just whether the model is accurate. Thus, service performance observability for inference is a distinct layer of functionality, necessary to make AIMLE more reliable and useful in dynamic and distributed deployments. This is particularly important in multi-edge deployments, and industrial scenario.

### 5.8.2	Open Issues

Therefore, the following aspects need to be studied:

-	Whether and how AIMLE should support expressing inference service performance parameters (e.g., expected latency, accuracy, confidence level) in service requests.

-	Whether AIMLE requires mechanisms for real-time feedback from clients (e.g., inference result too late, failed, or inaccurate).

# 6	Enhanced architecture for enabling AI/ML services

## 6.1	General

## 6.2	Enhanced AI/ML enablement functional model

In this clause the architecture enhancements relate to the support for multi-operator services. There are two cases for the multi-operator service support:

- An AIMLE server connected to AIMLE clients over different PLMNs

- Each AIMLE server connected to AIMLE clients of single PLMN, whereas the AIMLE server interact each other via AIML-E.

In the first scenario, the AIML service (VAL service, ASP service) runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN.

In Figure 6.2.1-1, the VAL UE#1 is connected to PLMN#1 and VAL UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. In this scenario, different VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server.

Figure 6.2.1-1 architecture for multi-operator support with common AIMLE server

In the second scenario, as depicted in Figure 6.2.1-2, the VAL UEs are assumed to have AIMLE clients installed and active and connect to different AIMLE servers provided by different MNOs or trusted 3rd parties of the MNOs. In Figure 6.2.1-2, each AIMLE server connects to each PLMN. The VAL UE 1 is connected to PLMN1 and AIMLE server 1 and VAL UE 2 is connected to PLMN 2 and AIMLE server 2.

Figure 6.2.1-2 architecture for multi-operator support with distributed AIMLE servers

The VAL server may have service agreements with one or more 3GPP network system operators, where scenarios for different interactions are based on the concluded solution

NOTE: In multi-operator support with distributed AIMLE servers, the ML repositories are deployed per each AIMLE server / PLMN, and any cross-operator ML repository interaction is not supported in this release.

### 6.2.2	AI/ML enablement functional model for roaming scenarios

This clause describes the AIMLAPP architecture for roaming UEs. This architecture is aligned with network roaming architecture, since it re-uses existing network roaming principles and describes the AIML service continuity while VAL UEs are roaming; hence how AIMLE spans across HPLMN and VPLMN is discussed. Such architecture applies to both roaming models (LBO and HR roaming architectures).

In Figure 6.2.2-1, the AIMLAPP architecture for enabling AI/ML services for roaming scenarios is showcased. In this architecture, AIML-UU is the interface between the AIMLE client and the AIMLE server, which can be over HPLMN or VPLMN which VAL UE is roaming. AIML-E supports the interaction among AIMLE servers deployed by different MNOs, whereas the MLR-E is the new interface supporting the interaction among repositories for aligning the information on the ML models and ML/FL members to allow the continuity of the AIML service while the VAL UE is roaming.

Figure 6.2.2-1 architecture for roaming support

## 6.3	Functional Elements

The functional elements have been defined in clause 5.2.3 of 3GPP TS 23.482. No additional functional element has been identified.

## 6.4	Reference Points

The reference points have been defined in clause 5.2.4 of 3GPP TS 23.482.

The enhanced reference points are described below. The enhancements over existing reference points are shown in bold italics.

### 6.4.1	AIML-E

The interactions related to AIML enablement functions between AIMLE servers (e.g., central and edge AIMLE servers, or AIMLE servers of different PLMNs).

### 6.4.2	AIML-R

The interactions related to AIML enablement functions between the AIMLE server, and the ML repository are supported by AIML-R reference point. These interactions can be also between the ML repository and common AIMLE server connected to different PLMNs in multi-operator scenarios.

### 6.4.3	AIML-PC5

The interactions related to AIML enablement functions between AIMLE clients in off-network deployments, and in scenarios where split AIMLE services are provided using relay VAL UEs.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |
| Sol #2 |  | X |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |  |
| Sol #4 |  | X |  |  |  |  |  |  |
| Sol #5 |  | X |  |  |  |  |  |  |
| Sol #6 |  | X |  |  |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |  |
| Sol #8 |  |  |  | X |  |  |  |  |
| Sol #9 |  |  |  |  |  | X |  |  |
| Sol #10 |  |  | X |  |  |  |  |  |
| Sol #11 |  |  | X |  |  |  |  |  |
| Sol #12 |  |  |  | X |  |  |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |
| Sol #14 |  |  |  |  |  | X |  |  |
| Sol #15 |  |  |  |  |  |  | X |  |
| Sol #16 |  |  | X |  |  |  |  |  |
| Sol #17 | X |  |  |  |  |  |  |  |
| Sol #18 |  |  | X |  |  |  |  |  |
| Sol #19 |  |  | X |  |  |  |  |  |
| Sol #20 |  | X |  |  |  |  |  |  |
| Sol #21 |  |  |  |  |  | X |  |  |
| Sol #22 |  |  |  |  |  |  | X |  |
| Sol #23 |  |  |  |  |  |  |  | X |
| Sol #24 |  |  |  |  |  | X |  |  |
| Sol #25 |  |  |  |  | X |  |  |  |
| Sol #26 |  |  |  |  |  | X |  |  |

## 7.1	Solution #1: Enhancement of AIMLE to support model inference

### 7.1.1	Solution description

This clause describes the procedure for supporting the ML model inference by the AIMLE server. The consumer (e.g. VAL server, AIMLE client) requests the AIMLE server for ML model inference by specifying the details about the ML model or requirements to select a ML model for inference.

Pre-condition:

-	The ML model inference service needed to be used for this ML model inference is available by the AIMLE server.

Figure 7.1.1-1 illustrates the procedure for AIMLE server to support ML model inference based on the request from the consumer.

Figure 7.1.1-1: ML model inference

1.	The consumer (e.g., VAL server, AIMLE client) sends an ML model inference request to AIMLE server, requesting the inference result from the AIMLE server. This request consists of ML model information or ML model inference requirement information, etc. The request includes information elements specified in table 7.1.3.1-1.If the VAL server has previously performed inference service discovery as described in clause 7.25, it shall include the Inference service identifier received in the discovery response.

2.	The AIMLE server checks whether the consumer is authorized to perform the ML model inference request. If no model information is provided but only the model inference requirement information is provided in step 1, the AIMLE server identifies and selects the appropriate ML model for inference based on the ML model inference requirement information using the procedure as specified in clause 8.11.3. The AIMLE server handles the request and execute the inference task.

3.	If the consumer is authorized, AIMLE server returns the inference response, otherwise a failure response indication the reason for failure.

### 7.1.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.1.3	Corresponding APIs

Table 7.1.3.1-1 describes the information flow from the consumer to the AIMLE server for requesting the ML model inference.

Table 7.1.3.1-1: ML model inference request

| Information element | Status | Description |
| --- | --- | --- |
| Inference service identifier (NOTE 2) | O | Identifier of the discovered inference service returned from the inference service discovery procedure (see clause 7.25). Used to correlate the inference request with prior discovery context, including model reference, access policy, or service location. |
| Requestor identity | M | Identity of the consumer (e.g. identifier of the VAL server, AIMLE client or the VAL UE) performing the request. |
| Inference data (NOTE 1) | O | Original data for inference that may include text, images, audio, numerical data, etc. This IE may be included in the request in the scenario, e.g. the data is small. |
| Inference data index (NOTE 1) | O | An index of the data for inference. This IE may be included in the request in the scenario, e.g. the data is big, or stored in different servers. |
| Inference Parameters | O | Additional inference parameters that may be needed, such as maximum output length, temperature, batch size, etc., depending on the specific requirements of the model. |
| ML model information (NOTE 2) | O | Identifies the ML model that the consumer specified to perform inference. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model inference requirement information (NOTE 2) | O | Identity the type or goal of the inference task, such as 'text classification,' 'image recognition,' 'sentiment analysis,' etc |
| NOTE 1: Only one of these IE shall be present. NOTE 2: At least one of these IE shall be present. |  |  |

Table 7.1.3.2-1 describes the information flow of ML model inference response from the AIMLE server to the consumer.

Table 7.1.3.2-2: ML model inference response

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | O | Indicates the service that executed the inference |
| Inference invocation ID | O | Identifier of this inference invocation for tracking or follow-up operations. |
| Inference response | M | Indicates that the ML model inference request was successful or not. |
| >Inference result (NOTE) | O | If the inference ML model inference request is successful, the Inference result is included in the response. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE server for inference. |
| > Cause (NOTE) | O | If the inference ML model inference request is failed, the reason for the failure is included in the response. |
| NOTE: Only one of these information elements shall be present |  |  |

### 7.1.4	Solution evaluation

This solution addresses the fundamental requirement of enabling AIMLE servers to support ML model inference services in response to requests from VAL servers. It defines a request and response model that allows inference to be triggered with either explicit ML model selection or abstract inference requirement descriptions.

From a functional perspective, the solution provides a foundational inference invocation capability in the AIML APP architecture. It allows the VAL server to invoke inference services hosted by the AIMLE server by supplying model details, data, and inference parameters. The optional integration with inference service discovery (clause 7.25) enhances the interoperability by enabling clients to discover suitable inference services before invocation.

### From an architectural perspective, the solution does not require new network functions or interfaces. It utilizes the existing AIMLE function and extends it with inference APIs that support various data input methods (direct input or data reference) and flexible model invocation strategies. The defined procedures align with the service-oriented architecture principles used across SA6 specifications.

## 7.2	Solution #2: Enhance AIMLE Data Management Assistance for Data Drift Detection

### 7.2.1	Solution Description

As introduced in 3GPP TS 23.482 [2], the AIMLE data management assistance service is the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. In the context of AI/ML, it is important that the data used by the ML model is not significantly different from the data used to train the model (e.g. the distribution of the features have changed over time). If the difference is higher enough, the model's performance tends to decrease. This discrepancy is known as data drift, that is data drift refers to changes in the statistical properties of data over time leading to a decline in model performance.

Therefore, it is essential that the data management system procedure provides the capability to detect whether data drift has occurred to assist in decision-making if the model needs to be updated due to a decrease in its performance.

Data drift detection procedure allows to assist ML model maintenance procedures. To support ML model management, e.g. assist ML model maintenance, the AIMLE data management assistance service need be enhanced by adding drift detection functionality.

ML model evaluation profile ID is provided as part of data drift requirements to allow the AIMLE server with obtaining ML model evaluation information for performing data drift detection.

The following clauses specify the impact to existing AIMLE procedures for the enhancements of AIMLE data management assistance service.

#### 7.2.1.1	Impact to existing AIMLE procedures

The AIMLE procedures and information flows in 3GPP TS 23.482 can be enhanced (highlighted in bold italics) as follows.

8.15 AIMLE data management assistance 8.15.1 General AIMLE data management assistance is the process of the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation, data analysis, and data drift detection. The AIMLE server offloads the AIMLE service consumer from interacting with AIMLE clients to manage the data operations. The VAL clients perform the actual data preparation, data analysis, and data drift detection operations and send the outputs to the AIMLE server for aggregation. The following clauses specify procedures, information flows, and APIs for AIMLE data management assistance. 8.15.2 Procedure Pre-conditions: 1. AIMLE clients have registered with AIMLE server. 2. UE application data for AI/ML operations have been collected and dataset identifier have been assigned to each dataset. EVEX mechanism can be reused for data collection as described in 3GPP TS 26.531 [10]. Figure 8.15.2-1: AIMLE data management assistance 1. An AIMLE service consumer (e.g., VAL server) makes a request for AIMLE data management assistance. The request includes information as described in Table 8.15.3.1-1. To prepare data for AIML operations, the AIMLE service consumer configures Data management operations to data preparation. Data preparation is performed to process data into a format that is required by ML models for the models to function properly, e.g.; before training or inferencing. The AIMLE service consumer includes data preparation requirements as described in Table 8.15.3.1-2. To analyse data for AIML operations (i.e., to perform exploratory data analysis or EDA), the AIMLE service consumer configures Data management operations to data analysis. Data analysis is performed to obtain the statistical properties of a dataset without having direct access to the data, which is a requirement for Federated Learning. The AIMLE service consumer includes data analysis requirements as described in Table 8.15.3.1-3. To perform data drift detection for AIML operations, the AIMLE service consumer configures Data management operations to data drift detection. Data drift detection is utilized to assist with determining changes in statistical properties of input data. The AIMLE service consumer includes data drift requirements as described in 8.15.3.1-x. NOTE: It will be determined during evaluation and conclusion whether to split the data drift detection into AI model performance detection related procedure. In addition to providing the data management requirements, the AIMLE service consumer provides either a list of AIMLE clients (if known) or AIMLE client selection criteria. When providing the AIMLE client selection criteria, the AIMLE service consumer indicates that the AIMLE server performs AIMLE client selection to select AIMLE clients for the data management operations. 2. The AIMLE server authenticates and authorizes the request. If authorized, the AIMILE server assigns an identifier for the subscription. 3. The AIMLE server sends an AIMLE data management assistance subscription response to the AIMLE service consumer. The response includes information as described in Table 8.15.3.2-1. 4. The AIMLE server sends Client data processing trigger requests to AIMLE clients. The request can be for data preparation, data analysis, or data drift detection. The request includes information as described in Table 8.15.3.4-1. If AIMLE client selection criteria were provided in step 1, the AIMLE server performs AIMLE client selection to select AIMLE clients that have Dataset ID and Dataset feature ID indicated in the data management requirements. 5. Each AIMLE client sends the requirements to trigger data management for the VAL client to perform the requested data operation. The VAL client performs the operation locally. NOTE: The data preparation, data analysis, and data drift detection functions operate in a similar manner as ML models. The AIMLE layer is able to transport the functions to the VAL clients and the VAL clients performs the associated function on the application data identified by the Dataset ID and Dataset feature ID as part of data preparation, data analysis, or data drift detection in a similar manner as ML training. 6. After the data operation completes, each AIMLE client sends a response to the AIMLE server with information as described in Table 8.15.3.5-1. 7. The AIMLE server aggregates the output of each AIMLE client. Aggregation includes combining or performing a statistical operation on the received data. Data received from the AIMLE clients can be numerical or categorical, which allows the AIMLE server to aggregate the output data from the AIMLE clients. If necessary, steps 4–7 is repeated to complete all the required data operations. 8. The AIMLE server sends an AIMLE data management assistance notification to the AIMLE service consumer with information as described in Table 8.15.3.3-1. 8.15.3 Information flows 8.15.3.1 AIMLE data management assistance subscription request Table 8.15.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management assistance subscription procedure. Table 8.15.3.1-1: AIMLE data management assistance subscription request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Data management operations; M; An indicator showing what data management type is being requested: data preparation, data analysis, data drift detection.. Data management requirements; M; Requirements for the data management request:. >Data preparation requirements; O (NOTE 1); Data preparation requirements as detailed in Table 8.15.3.1-2.. >Data analysis requirements; O (NOTE 1); Data analysis requirements as detailed in Table 8.15.3.1-3.. > Data drift detection requirements; O (NOTE 1); Data drift detection requirements in Table 8.15.3.1-x.. AIMLE clients list; O (NOTE 2); A list of AIMLE clients for which data management should be performed. The list may be specified by AIMLE client set identifier.. AIMLE client selection criteria; O (NOTE 2); Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided.. Table 8.15.3.1-2: Data preparation requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Data preparation requirements; M; Requirements for data preparation.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature to process.. > Data preparation function; M; This indicates the function which prepares the data, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request. A function may be used to process one or more dataset features.. Table 8.15.3.1-3: Data analysis requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Dataset analysis requirements; M; Requirements for data analysis.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature.. > Data analysis function; M; This indicates the function which performs the data analysis, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request. A function may be used to process one or more dataset features.. Table 8.15.3.1-x: Data drift detection requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Data drift detection requirements; M; Requirements for data drift detection.. > ML model evaluation profile ID; M; The identifier of the ML model evaluation profile.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifiers or names of dataset feature.. > Data drift detection function; O; This indicates the function which performs the drift detection, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request. A function may be used to process one or more dataset features. 8.15.3.3 AIMLE data management assistance notify Table 8.15.3.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure. Table 8.15.3.3-1: AIMLE data management assistance notify Information element; Status; Description. Status; M; The status for the data management operation. Aggregated data preparation outputs; O (NOTE 1, NOTE 2); Provides outputs for data preparation: dataset identifier, dataset features, and prepared data output.. Aggregated data analysis outputs; O (NOTE 1, NOTE 2); Provides outputs for data analysis: dataset identifier, statistical outputs for each feature, list of outlier and anomaly values, and feature correlation information.. Aggregated data drift detection outputs; O (NOTE 1, NOTE 2); Provides outputs for data drift detection: dataset identifier, data drift detection status (e.g. detected data drift of the dataset features).. Timestamp; O; Timestamp of the data management notification. NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal.. 8.15.3.4 Client data processing trigger request Table 8.15.3.4-1 shows the request sent by the AIMLE server to AIMLE clients for the AIMLE client data processing trigger procedure. Table 8.15.3.4-1: Client data processing trigger request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Data management type; M; An indicator showing what data management type is being requested: data preparation, data analysis.. Data management requirements; M; Requirements for the data management request:. >Data preparation requirements; O (NOTE); Data collection requirements as detailed in Table 8.15.3.1-2.. >Data analysis requirements; O (NOTE); Data collection requirements as detailed in Table 8.15.3.1-3.. > Data drift detection; O (NOTE); Data drift detection requirements as defined in Table 8.15.3.1-x.. Operational schedule; O; A schedule to perform the requested data management operation.. NOTE: At least one of the information elements shall be provided.. 8.15.3.5 Client data processing trigger response Table 8.15.3.5-1 shows the response sent by AIMLE clients to the AIMLE server for the Client data processing trigger procedure. Table 8.15.3.5-1: Client data processing trigger response Information element; Status; Description. Status; M; The status for the data management operation. Data preparation outputs; O (NOTE 1); The output data after performing data preparation. One output data is generated for each requirement. (NOTE 2). Data analysis outputs; O (NOTE 1); The output data generated by data analysis. One output data is generated for each requirement. (NOTE 2). Data drift detection outputs; O (NOTE 1); The output of data drift detection status (e.g. detected data drift of the dataset features). (NOTE 2). Timestamp; O; Timestamp of the data management operation. NOTE 1: One of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. The AIMLE server is able to process either type of data..

### 7.2.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 7.2.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 7.2.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 7.3	Solution #3: New AIMLE Service on ML Model Maintenance

### 7.3.1	Solution Description

It is important to continuously monitor the model's performance and check for any changes in the statistical nature of the data that is used for training the ML model or be used by the ML model for its task. In this scenario, ML model maintenance is needed to check e.g., whether the model's performance aligns with expectations or data drift appears. Then, the AIMLE server may initiate model update process.

AIMLE service provides model maintenance service to the consumer (e.g. VAL server) based on request, e.g., to detect data drift, and initiate ML model update process. The information of data drift in specific dataset provided to the AIMLE server may be the data drift analysis output from the AIMLE client, and/or the data drift detection outputs from the VAL client via the AIMLE client.

ML model evaluation profile ID is provided as part of ML model maintenance requirements to allow the AIMLE server with obtaining ML model evaluation information for performing ML model performance evaluation.

The following clauses specify procedures, information flows and APIs for Key Issue #2 to support new AIMLE service on ML model maintenance.

#### 7.3.1.1	Procedure

Pre-conditions:

1.	AIMLE clients have registered with AIMLE server.

Figure 7.3.1.1-1: AIMLE support ML model maintenance procedure

1.	An AIMLE service consumer (e.g., VAL server) sends a ML model maintenance subscription request to the AIMLE server. The request includes information as described in Table 7.3.1.2.1-1.

2.	The AIMLE server authenticates and authorizes the request. If authorized, the AIMLE server assigns an identifier for the subscription.

3.	The AIMLE server sends an ML model maintenance subscription response to the AIMLE service consumer. The response includes information as described in Table 7.3.1.2.2-1.

4.	The AIMLE server performs operations for ML model performance evaluation to check the ML model performance. The AIMLE server retrieves the ML model evaluation information from the provided ML Evaluation profile ID as described in clause 7.4.1.3 and performs the ML model performance evaluation. If the performance of the ML model is not as expected as request in step 1, the AIMLE server determines date drift detection. The ML model performance evaluation can use the ML model performance monitoring service as described in 3GPP TS 23.482 [2] clause 8.22 or use offline ML model evaluation procedure.

Editor's Note:	The detail procedure for offline ML model performance evaluation is FFS.

5.	The AIMLE server request and get client data processing trigger response as described in 3GPP TS 23.482 [2] clause 8.15 with data drift detection outputs from the AIMLE client, by using the enhanced AIMLE data management assistance service as described in Solution #2 steps 4-6 with enhanced information flows.

6.	Based on the data drift information received in step 5 and the ML model evaluation information obtained in step 4, the AIMLE server determines if actions are required (e.g. model replacement (e.g. select other ML model) or transfer learning (e.g. re-train the ML model based on the original ML model)) to keep the ML model performance within the performance requirements received in step 1.

7. The AIMLE server sends an ML model maintenance notification based on the notification requirements received in step 1 to the AIMLE service consumer with information as described in Table 7.3.1.2.3-1.

#### 7.3.1.2	Information flows

##### 7.3.1.2.1	ML model maintenance subscription request

Table 7.3.1.2.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the ML model maintenance subscription.

Table 7.3.1.2.1-1: ML model maintenance subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| ML model ID | M | The identifier of the model that will be maintained. |
| ML model retrieval endpoint | O (NOTE 1) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| ML model | O (NOTE 1) | The ML model that will be maintained. |
| ML model maintenance requirements | M | Requirements for the ML model maintenance. |
| > ML model performance requirements | M | ML model performance requirements for the maintenance. |
| > ML model evaluation profile ID | M | The identifier of the ML model evaluation profile. |
| > Dataset ID | O (NOTE 2) | An identifier for the dataset, which is used for training the ML model, or be used by the ML model for its task. |
| > AIMLE clients list | O (NOTE 2) | A list of AIMLE clients from which the datasets for the ML model maintenance applied for. |
| > Maintenance mode | M | The maintenance mode, e.g., periodically or event triggered (e.g. ML model performance degradation detected, data drift detected). |
| Notification requirements | M | Requirements for notification, e.g., periodically or event triggered (e.g. ML model updated). Notify with e.g. ML model, or ML model status (e.g. updated ML model ID, ML model version). |
| Notification endpoint | O | The notification endpoint where the notifications should be sent to. |
| Expiration time | O | The proposed expiration time of the subscription. |
| NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided. |  |  |

##### 7.3.1.2.2	ML model maintenance subscription response

Table 7.3.1.2.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the ML model maintenance subscription.

Table 7.3.1.2.2-1: ML model maintenance subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model maintenance subscription request was successful. |
| > Subscription identifier | M | An identifier for the subscription. |
| > Expiration time | O | Expiration time for the subscription |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of the information elements shall be present. |  |  |

##### 7.3.1.2.3	ML model maintenance notification

Table 7.3.1.2.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the ML model maintenance subscription.

Table 7.3.1.2.3-1: ML model maintenance notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier of the subscription. |
| Model maintenance outputs | M | Provides outputs for model maintenance based on request: e.g. ML model, or ML model status (e.g. updated ML model ID, ML model version). |
| Timestamp | O | Timestamp of the data management notification. |

### 7.3.2	Architecture Impacts

### There were no identified architectural impacts in this solution.

### 7.3.3	Corresponding APIs

This subclause provides a summary of the corresponding API for solution #3.

### -	ML model maintenance subscription API (subscribe / notify model; API provider: AIMLE Server; known consumer: AIMLE service consumer; corresponding to step 1, 3, and 7 of clause 7.3.1).

### 7.3.4	Solution evaluation

This solution supports ML model maintenance by addressing Key Issue #2: support enhancement for evaluation and determination of ML model performance and correctness within AIML Enablement Layer.

The AIMLE service consumers contact the AIMLE server to obtain ML model maintenance information, and the AIMLE server returns this information by retrieving it using the ML Evaluation Profile ID. In cases where the performance of the ML model is not as expected, the AIMLE server determines the factors to be considered in the ML model maintenance evaluation. Based on this information, the AIMLE server triggers actions such as model replacement or retraining, if necessary.

### This solution does not impact the current AIMLE architecture.

## 7.4	Solution #4: Management of evaluation information for ML model performance and correctness evaluation.

### 7.4.1	Solution description

#### 7.4.1.1	Introduction

This solution proposes enhancements to address Key Issue #2 – Open Issue #2 and provides procedures required to enable evaluation of ML model performance and correctness considering application specific ML models.

This solution proposes to:

-	define new procedures and information flows to enable management (e.g., storage, retrieval, removal) of ML model evaluation information in the ML model repository;

-	update information flows to link the ML model evaluation information with ML model(s).

The solution allows the owner of ML model(s) to store and manage the evaluation information associated with ML model(s). The evaluation information is information that can be retrieved to perform the evaluation of performance and correctness of ML model(s). This solution can be combined with existing and/or new procedures when evaluation of a ML model performance and correctness is required.

ML models that can be stored and managed in the AIML enablement layer are application specific ML models. Such ML models can be retrieved by the AIMLE client or VAL server to perform inferencing at the VAL layer.

ML model specific evaluation information is required for a consumer of a ML model to perform evaluation of a ML model performance and correctness. Thus, this solution proposes to enable storage and retrieval of ML model evaluation information in the ML model repository for enabling the performance and correctness evaluation of application specific ML models.

The ML model specific evaluation information can be used by monitoring solutions for data drift detection (solution #2) and monitoring a ML model's performance (solutions #3 and #6).

#### 7.4.1.2	ML model evaluation information storage

Figure 7.4.1.2-1 describes procedures for storing ML model evaluation information in the ML model repository.

Figure 7.4.1.2-1: ML evaluation information storage

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information storage request to the AIMLE server. The request includes the requestor identifier and ML evaluation information.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information storage request to store the ML evaluation information in the ML repository. The request includes the requestor identifier and ML evaluation information.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository stores the ML evaluation information, creates an ML evaluation profile including a ML evaluation profile identifier and sends an ML evaluation information storage response to the AIMLE server. The response includes an indication of success for the request and the ML evaluation profile identifier if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information storage response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation profile identifier if the request was successful.

#### 7.4.1.3	ML model evaluation information retrieval

Figure 7.4.1.3-1 describes procedures for retrieving ML model evaluation information from the ML model repository.

Figure 7.4.1.3-1: ML evaluation information retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information retrieval request to the AIMLE server. The request includes the requestor identifier and ML evaluation information retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information retrieval request to obtain the ML evaluation information from the ML repository. The request includes the requestor identifier and ML evaluation information retrieval filters.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository identifies ML evaluation profiles based on the ML evaluation information retrieval filters and sends a ML evaluation information retrieval response to the requestor. The response includes an indication of success for the request and the identified ML evaluation profiles if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information retrieval response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation profile(s) if the request was successful.

#### 7.4.1.4	ML model evaluation information removal

Figure 7.4.1.4-1 describes procedures for removal of ML model evaluation information from the ML model repository.

Figure 7.4.1.4-1. ML evaluation information removal

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information removal request to the AIMLE server. The request includes the requestor identifier and ML evaluation information identifier(s) to be removed.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information removal request to remove the ML evaluation information from the ML repository. The request includes the requestor identifier and ML evaluation information identifier(s).

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository removes the ML evaluation information based on the ML evaluation information identifier(s) and sends a ML evaluation information removal response to the requestor. The response includes an indication of success for the request and the ML evaluation information identifiers that were removed if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information removal response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation information identifiers that were removed if the request was successful.

### 7.4.2	Architecture Impacts

No architecture impact has been identified.

### 7.4.3	Corresponding APIs

#### 7.4.3.1	ML Evaluation Profile

Table 7.4.3.1-1 describes the ML evaluation profile and Table 7.4.3.1-2 describes the ML evaluation which is used in the ML evaluation information management procedures between the AIMLE client, VAL server, AIMLE server and ML model repository.

Table 7.4.3.1-1: ML Evaluation Profile

| Information element | Status | Description |
| --- | --- | --- |
| ML evaluation profile identifier | M | The identifier of the ML evaluation profile. |
| AIMLE server identifier | M | The identifier of the AIMLE server that stored the ML evaluation information. |
| ML repository identifier | M | The identifier of the ML repository where the ML evaluation information is stored. |
| List of ML evaluations | M | The list of ML evaluation information as defined in Table 7.4.3.1-2. |

Table 7.4.3.1-2: ML Evaluation Information

| Information element | Status | Description |
| --- | --- | --- |
| Evaluation identifier | M | The identifier of the evaluation information used for evaluating ML model(s). |
| List of ML models identifiers | M | The list of ML models identifiers to which the evaluation applies. |
| Source identifier | O | The identifier of the source (e.g., VAL server ID, VAL client ID) that stored the ML evaluation information in the ML repository. |
| Description | M | The description of the ML model evaluation. |
| Metric information | M | The metrics measured by the performing the ML model evaluation. |
| Input information | M | Represents the input information which is aligned with the associated ML model input parameters. The format of input information for application specific ML models is out of scope. |
| Output information | M | Represents the expected output information which is expected as a result of processing the input information by the ML model. The format of output information for application specific ML models is out of scope. |
| Grading information | M | The information for evaluating and grading the performance and correctness of a ML model (e.g., grading the obtained evaluation output based on the expected output). |
| Execution information | O | The execution information which is required to evaluate performance and correctness of a ML model. The information indicates whether the evaluation is executed once, repeatedly in a time window, has its output averaged, etc. If not included, the evaluation is evaluated and graded once. |

#### 7.4.3.2	ML Evaluation Information Storage

Table 7.4.3.2-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for storing ML evaluation information in the ML repository.

Table 7.4.3.2-1: ML Evaluation Information Storage Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of ML evaluation information | M | The list of ML evaluation information to be stored in the ML repository as described in Table 7.4.3.1-2. |

Table 7.4.3.2-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information storage request.

Table 7.4.3.2-2: ML Evaluation Information Storage Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > ML evaluation profile identifier | M | The ML evaluation profile identifier stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.4.3.3	ML Evaluation Information Retrieval

Table 7.4.3.3-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for retrieving ML evaluation information from the ML repository.

Table 7.4.3.3-1: ML Evaluation Information Retrieval Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Evaluation information filters | M | The evaluation information filter used to identify the retrieved ML evaluation information. |
| > Evaluation profile identifier | O | Represent the filtering criteria which is a ML evaluation profile identifier. |
| > Evaluation information filter | O | Represents the filtering criteria, which can be any of the ML evaluation information as defined in Table 7.4.3.1-2. |

Table 7.4.3.3-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information retrieval request.

Table 7.4.3.3-2: ML Evaluation Information Retrieval Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML evaluation profiles | M | The list of retrieved ML evaluation profiles stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.4.3.4	ML Evaluation Information Removal

Table 7.4.3.4-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for removing ML evaluation information from the ML repository.

Table 7.4.3.4-1: ML Evaluation Information Removal Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of ML evaluation profile identifiers | M | The list of evaluation profile identifiers to be removed from the ML model repository. |

Table 7.4.3.4-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information removal request.

Table 7.4.3.4-2: ML Evaluation Information Removal Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML evaluation profile identifiers | M | The list of ML evaluation profile identifiers removed from the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

### 7.4.4	Solution evaluation

This solution addresses Key Issue #2 – Open Issue #2 on whether and how to evaluate ML model performance and correctness for application-specific ML models. The solution introduces systematic management of ML model evaluation information, including procedures for storage, retrieval, and removal of evaluation artifacts in the ML repository.

This solution defines how evaluation artifacts are associated with ML model(s) that are subject to evaluation or monitoring, thereby enabling re-use of consistent evaluation information across multiple AIMLE operations.

Furthermore, this solution complements other solutions such as Solution #2 (data drift detection), Solution #3 (model maintenance), and Solution #6 (performance monitoring), which rely on the availability of ML model evaluation information to perform their respective functions.

The solution is feasible, well aligned with the AIMLE functional architecture, and does not introduce additional dependencies on 3GPP network systems.

Whether the management procedures described in this solution are added as new procedures in 3GPP TS 23.482 [2] or are combined with existing ML model management procedures described in 3GPP TS 23.482 [2] clause 8.11 will be decided in the normative phase.

## 7.5	Solution #5: Solution on monitoring ML-enabled analytics correctness

### 7.5.1	Solution description

This solution addresses Key Issue #2 and open issue 3.

In this solution, the ADAES is providing the analytics performance monitoring where the AIMLE server detects an expected or predicted model degradation performance based on the ML-enabled analytics performance feedback. Such ADAE analytics (as captured in 3GPP TS 23.436) can be about the edge load or VAL server or session performance analytics which utilizes ML methods (training by AIMLE) for predicting the end-to-end performance.

Pre-conditions:

1.	One or more ADAE services (at the ADAE server or clients) using the given ML model training service from AIMLE server are ongoing.

2.	The AIMLE server (or VAL server) detects a requirement to monitor the analytics performance. Such trigger may be based on VAL server request for the ML model performance monitoring capability as specified in 3GPP TS 23.482 clause 8.22.2.

Figure 7.5.1-1: Procedure for ML-enabled analytics monitoring

1.	The AIMLE server sends an ADAE analytics performance monitoring subscription request to the ADAE server (ADAES). The subscription request includes the following parameters: an ADAE analytics ID or type, a VAL server ID, an area of interest and time of validity, a monitoring reporting configuration (thresholds for triggering a monitoring event, e.g. minimum accuracy, confidence level, whether the reporting is one time or periodical or event-based), a KPI for the analytics service, VAL policies for triggering an action based on a monitoring event (e.g. if analytics degradation is detected, re-start the analytics process).

2.	The ADAE server authorizes the subscription request and sends an ADAE analytics performance monitoring subscription response to the AIMLE server. The response is a positive or negative acknowledgement of the subscription and includes a subscription ID in case of success and a cause in case of failure (e.g. failure to authorize).

3.	The ADAE server discovers based on the analytics ID the ADAECs or other entities (e.g. edge ADAES) which are undertaking an analytics task for the given analytics ID (e.g. local prediction at UE side or ADAES side).

4.	The ADAES server sends a request to the discovered entities (ADAEC or edge ADAES) indicating a monitoring requirement for the analytics task. This request includes the analytics ID, the requestor ID, the event trigger criteria for providing a monitoring report (e.g. reaching a threshold), the target KPIs to be monitored (e.g. accuracy), the area and time of interest, whether this is a predicted parameter or actual parameter to be monitored. Such request can be in form of an analytics correctness feedback request either real time or when the analytics task finishes.

5. The discovered ADAEC(s) and/or edge ADAES start monitoring the performance of the analytics task and in particular the attributes which are requested (accuracy, delay, evaluation vs ground truth data). This involves the continuous checking of the performance (accuracy, latency) of the analytics service for providing a notification / trigger event to the AIMLE server identifying a deviation in terms of ADAE analytics performance. Such identification can be a detection of a mismatch in the analytics performance with a possible cause (e.g. data producer, ML model, UE/ channel conditions, data drift).

6. The discovered ADAEC(s) and/or edge ADAES based on the trigger criteria, send to the ADAES a monitoring response, indicating an expected or predicted degradation of the performance of the analytics task. Such message may also include the possible cause as in step 6 (e.g. data producer, ML model, UE/ channel conditions, data drift).

7. The ADAES identifies or predicts an ADAE service degradation or performance (against the KPIs for that analytics ID) and sends a notification to the AIMLE server detecting a degradation / mismatch at the target performance for the analytics task.

8. The AIMLE server based on the received monitoring notification from the one or more AIMLE clients, it identifies an expected or predicted ML model degradation using the AIMLE capability for ML model performance monitoring as specified in 3GPP TS 23.482 clause 8.22.2 and can optionally use the ADAE capability for ML model degradation detection in clause 8.19.2.

Alternatively, the AIMLE server after step 2 and/or the ADAE server after step 3 send requests to A-DCCF for collecting analytics performance information. The procedure for ADAE Server to request data collection via A-DCCF are as specified in clause 8.12 of 3GPP TS 23.436 [4].

NOTE:	Possible utilization of A-DCCF for supporting the data collection will be decided during normative phase.

### 7.5.2	Architecture Impacts

No architecture impact identified.

### 7.5.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #5.

-	ADAE Analytics Perf Monitoring Subscribe API (request / response model; API provider: ADAES; known consumer: AIMLE server; corresponding to step 1 and 2 of clause 7.5.1).

-	ADAE Analytics Perf Monitoring Notify API (subscribe / notify model; API provider: ADAES; known consumer: AIMLE server; corresponding to step 7 of clause 7.5.1).

-	ADAE Analytics Perf Monitoring API (request / response model; API provider: ADAEC or edge ADAES; known consumer: ADAE server; corresponding to step 4 and 6 of clause 7.5.1).

### 7.5.4	Solution evaluation

This solution addresses Key Issue #2 and introduces the capability to support monitoring and detecting performance degradation related to an ML-enabled ADAE analytics service. This capability mainly introduces enhancements to ADAES for monitoring analytics service performance and reporting it to AIMLE server, and also enhancements in AIMLE server capability to consider an additional criterion to identify an expected or predicted ML model degradation as specified in 3GPP TS 23.482 clause 8.22.2.

The possible role of A-DCCF for supporting the monitoring data collection and coordination will be explored in the normative phase, given the conclusions related to A-DCCF usage for AI/ML data collection.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

## 7.6	Solution #6: Enhancements to performance monitoring

### 7.6.1	Solution description

#### 7.6.1.1	Enhancements to ML model performance monitoring

The ML model performance monitoring procedure as specified in clause 8.22 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

8.22.2 Procedure Pre-conditions: 1. One or more AIMLE services (at the AIMLE server or clients) using the given ML model are ongoing. Figure 8.22.2-1: ML model performance monitoring 1. The VAL server sends an ML model performance monitoring subscription request to AIMLE server, requesting to assist in monitoring the ML model performance. This request consists of ML model information and optionally the AIML service information, etc. The request also includes the ML model evaluation profile ID which is associated with a ML model evaluation profile containing the baseline validation dataset information which is the baseline reference for evaluation of HFL model after each round of parameter aggregation. 2. The AIMLE server checks whether the VAL server is authorized to perform the ML model performance monitoring request. 3. If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indication the reason for failure. 4. The AIMLE server identifies the AIMLE services which are utilizing the requested ML model. Such AIMLE services can be an ML model training service or an HFL service at the AIMLE server or clients. The identification of the AIMLE services may be performed via fetching ML model information from the ML repository using ML model management procedure as in clause 8.11.3. The ML Evaluation profile ID is used to obtain the ML model evaluation profile (e.g., baseline validation dataset) which is used to determine the ML model performance. Then, the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service. In order to get the information, the request is enhanced with following IEs: Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Aggregated AI/ML model parameters; M; Information about the AI/ML model and model parameters for use in HFL training.. Performance Metrics; M; The performance metrics that are used to evaluate the performance of model (e.g. accuracy, latency). ML model evaluation profile ID; M; The identifier of the ML model evaluation profile.. Each AIMLE client use the ML evaluation profile ID to obtain the ML model evaluation profile which is used to determine the ML model performance. Each AIMLE client evaluates the performance of the received aggregated global model on the local data based on the received Aggregated AI/ML model parameters, performance metrics and ML model evaluation profile. Each AIMLE client sends the performance report of aggregated global model on the local data as the response message to AILMLE Server. The performance report response message includes information as described in the following table. The response is enhanced with following IEs: Information element; Status; Description. AIMLE Client ID; M; The identifier of the AIMLE Client.. AIMLE service ID; M; The service identifier for the AIMLE HFL training operation.. Performance records; M; The evaluation performance records (e.g. on accuracy, on latency) of received aggregated global model from AIMLE Server on local data.. ML model evaluation profile ID; M; The identifier of the ML model evaluation profile used to perform the performance evaluation.. Upon receiving the performance report from the AIMLE client, the AIMLE server evaluates the model parameters based on baseline validation dataset. The AIMLE server further uses the baseline validation dataset to evaluate the global aggregated model also. 5. The AIMLE server detects an expected ML model degradation (e.g., model drift, data drift) based on the deviation of the performance of the AIMLE service as indicated in step 4 . 6. The AIMLE server based on the expected ML model degradation, it may also indicate and execute a trigger action to ensure meeting the AIMLE service requirement. Further, AIMLE Server sends a configuration request to increase training collaboration to only AIMLE Clients that have degraded performance records. Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model by the same or different AIMLE client; or termination of the AIMLE service and initiating a new AIMLE service with a new ML model. NOTE: If this action involves re-selecting an AIMLE client for the AIMLE service, this is based on the procedure defined in clause 8.9. 7. The AIMLE server notifies the VAL server on the expected ML model degradation and if requested the triggered adaptation of the AIMLE service. 8.22.3 Information flows 8.22.3.1 ML model performance monitoring subscription request Table 8.22.3.1-1 shows the request sent by a VAL server to an AIMLE server for the ML model performance monitoring. Table 8.22.3.1-1: ML model performance monitoring subscription request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. ML model identifier; M; The identifier of the ML model for which the monitoring applies.. Notification endpoint; M; The notification endpoint (e.g. URL/URI/IP address) where the notifications should be sent to.. AIML operation information; O; The AIMLE operation (ML model training, HFL, VFL, TL) for which the ML model is used.. > VAL service ID; O; The VAL service identifier of the AIMLE service using the ML model (if known by the requestor).. > AIMLE client ID(s); O; The identifier(s) of the AIMLE client(s) training the ML model (if known by the requestor).. > AIMLE service KPI; O; One or more KPIs for the AIMLE service performance (latency, accuracy, etc).. ML model evaluation profile ID; M; The identifier of the ML model evaluation profile.. Monitoring report configuration; M; The reporting configuration for the monitoring service (thresholds for triggering a monitoring event, e.g. minimum accuracy, delay, whether the reporting is one time or periodical or event-triggered).. Area of interest; O; The geographical or service area for which the monitoring applies.. Time validity; O; The time validity for the monitoring subscription.. Trigger Action requirement; O; This requirement identifies policies for triggering an action based on a monitoring event (e.g. if degradation is detected, to train a new model or re-selecting AIMLE clients)..

#### 7.6.1.2	Enhancements to HFL training

As per step 4 of Clause 8.22 of 3GPP TS 23.482 [2], for performance monitoring of AIMLE clients clause 8.12.2 is reused, quote:

“the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service.”

The current solution proposes to enhance clause 8.12 as follows (new text in bold italics):

8.12.2 Procedure Pre-conditions: 1. AIMLE client discovery and selection have been performed. 2. Datasets are available and prepared for AI/ML training at the AIMLE clients. The datasets are assigned identifiers for use in HFL training. Figure 8.12.2-1: HFL training 1. An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If the AIMLE server determines to use HFL training, the procedure continues to step 2. 2. The AIMLE server retrieves the indicated ML model using the ML model retrieval procedure as described in clause 8.1. 3. For the HFL training, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the HFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the HFL training. 4. Based on the AIMLE client set determined in step 3, the AIMLE server checks with the selected AIMLE clients for their capability and participation in the HFL training as described in the clause 8.10. 5. The AIMLE server configures training schedule for each of the AIMLE client and sends a HFL training subscription request with information as described in Table 8.12.3.1-1. The subscription request is further enhanced to include following: Information element; Status; Description. AI/ML model information; M; Information about the AI/ML model (e.g. Model size, Model architecture, etc.). Current HFL training iteration; O; The number of aggregations that have been done during HFL training. Performance Tracker; O; The KPI records (i.e. performance metrics, such as accuracy, etc.) over different subscriptions rounds.. Editor's Note: Whether performance tracker IE is needed or not is FFS. The Current HFL training iteration informs the client about current training iteration ongoing. 6. Each AIMLE client sends a HFL training subscription response with information as described in Table 8.12.3.2-2. If the AIMLE client is not able to grant the subscription (e.g., is not able to perform the training), the AIMLE client sends a response with a failure status and the procedure skips to step 8. 7. Each AIMLE client performs local training using the configured AI/ML model, model parameters, and the prepared local data associated with the dataset identifier for the specified number of samples according to the operational schedule. 8. Upon completion or due to errors in the training, each AIMLE client sends a HFL training notification to the AIMLE server. The notification includes information as described in Table 8.12.3.1-3. The AIMLE client provides an VAL service ID for the HFL training operation. 9. If errors were encountered, the procedure skips to step 10. If training was successful, the AIMLE server aggregates (e.g. averages) the model parameters received from the AIMLE clients. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 8 are repeated for the next training round. When the training schedule has been exhausted (e.g. there are no remaining training round) and if configured, the AIMLE server makes a ML model information storage request as described in clause 8.11.2 to store the AI/ML model in the ML repository. 10. The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2.

### 7.6.2	Architecture Impacts

This solution has not identified any architectural impacts.

### 7.6.3	Corresponding APIs

The inputs for the existing API is enhanced as specified in the solution description.

### 7.6.4	Solution evaluation

The solution relates to KI #2 and provides solution for following open issue:

3.	Whether and how to monitor (e.g., trigger, detect and report) ML model performance and correctness degradation for different ML-enabled operations (e.g., inferencing, training, split AIML operation, HFL, VFL, transfer learning) and considering data managements assistance.

The Clause 8.22 and clause 8.12 of 3GPP TS 23.482 [2] is for performance monitoring of ML model. In this solution, these clauses are enhanced for the performance monitoring and its adaptation for HFL training. The solution provides AIMLE enablement layer, the capability of proactively monitoring and reporting the performance of collaborating AIMLE clients with HFL training. The solution has not identified any architecture impacts.

## 7.7	Solution #7: Solution on discovering AIMLE clients in cross-PLMN scenarios

### 7.7.1	Solution description

This solution addresses key issue #3 open issues 1), 3), 4).

According to 3GPP TS 23.434, one of the business models is related to the SEAL provider having PLMN operator service arrangements with multiple home PLMN operators. Additionally, according to [AR-4.2.2-b], the SEAL shall support deployments in which SEAL services are deployed only outside of PLMN network.

This solution corresponds to the following use case:

-	An AI/ML service runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator AIMLE/VAL service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN. The AI/ML service can be an application enablement service (AIMLE client to server) where AIMLE-UU can be over multiple MNO networks.

-	In Figure 7.7.1-1, an example is shown where UE#1 is connected to PLMN#1 and UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. Different VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server.

-	One example of such scenario is that AI operation running for a V2X scenario, where VAL UEs of a traffic safety or optimization service (AI-enhanced) are needed and belong to different networks.

Figure 7.7.1-1: Use case for multi-operator AIMLE scenario

The solution provides a mechanism for discovering AI/ML participants (AIMLE clients, VAL clients) in an AIMLE-assisted operation (e.g. ML model training, FL, TL) in a given AIMLE service area. Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, VAL servers need to discover suitable AIMLE clients to fulfil the requirements for the AI/ML application. The VAL server can then use the discovered AIMLE clients to select a set of AIMLE clients to perform AI/ML operations.

In cross-PLMN scenario as in Figure 7.7.1-2, the following enhancements are provided (over AIMLE-S or SEAL-X interface if the AIMLE consumer is another SEAL server, e.g. ADAES).

8.8.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with AIMLE clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The request may also include AIML client task capability requirements to discover clients who can perform the AIML tasks like AIML model training/offload/split with compute requirements and task performance preference like green task performance. The AIMLE client discovery request includes information as described in Table 8.8.3.1-1.

This request also includes information on the serving PLMN for the communication of the VAL service, as well as the allowed MNO info from other MNO network from which the AIMLE consumer / VAL server can discover AIMLE clients / VAL UEs.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients for the VAL UEs connected to the serving PLMN. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

The AIMLE server also obtains candidate UEs connected to the serving PLMN from the ML repository.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e., AIMLE clients) that fulfils the discovery criteria based on supported services and AIMLE client profiles.

The AIMLE server determines whether the discovered UEs fulfil the location information in the discovery criteria by using their identifiers to determine their location and includes in the discovery response only those UEs that meet the requirements. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server may discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.8.3.2-1.

If the required number of AIMLE clients are not included in the response message from the AIMLE server (i.e., the AIMLE clients in the response message are less than the required minimum number of AIMLE clients), then the AIML service consumer may discover the remaining required AIMLE clients from other AIML enablement server indicating the required number of AIMLE clients. The response may also include information related to supported tasks with guaranteed task KPIs, available AIML tasks service duration, AIML task expected KPI like latency.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML) or the AIMLE clients are not available or capable for undertaking the operation, then the AIMLE server determines to discover and discovers the remaining required AIMLE clients connected to other PLMNs based on the allowed MNO information and the permissions as in step 1. The AIMLE server also obtains candidate UEs connected to the target/destination PLMN (other than the serving PLMN) from the ML repository.

The AIMLE server may also determine if a candidate UE is suitable based on roaming status and roaming policies.

### 7.7.2	Architecture Impacts

The architecture impact is the required enhancement of the AIMLE functional model to support multi-operator scenarios (as in clause 6.2).

### 7.7.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #7. This provides enhancements to the following information flows based on TS 23.482 clause 8.8.3.1 (bold italics is new):

- Table 8.8.3.1-1 on AIMLE client discovery request

- Table 8.8.3.1-2: AIMLE client discovery criteria

- Table 8.8.3.2-1: AIMLE client discovery response

Table 8.8.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| AIMLE client discovery criteria | M | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of required AIMLE clients | O | Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria. |
| Cross-PLMN discovery requirement | O | This parameter indicates whether the discover may span across multiple MNO networks or not. It can be seen as a multi-PLMN flag for the discovery or an indicator to further look in the AIMLE client discovery criteria for additional PLMNs |

Table 8.8.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Service requirement | M | Information about the required service |
| > VAL Service ID | M | VAL Service ID that the client is required to support. This identifies the service associated with the requester. |
| > Service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage). |
| Requested ML model types | O | Requested ML model types (decision trees, linear regression, neural networks). |
| Requested AIML operations | M | Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate). |
| Dataset requirements | O | Information about dataset. |
| > Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| > Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions. |
| AIML client task capability requirements | O | Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs) |
| AIMLE client velocity | O | Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static. |
| Location information | O | Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID) to discover the AIMLE clients. |
| AIMLE client QoS requirements | O | Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients. |
| Allowed MNO info | M | Information of the allowed operator(s) (e.g. MNO name, PLMN ID) from which its subscriber can consume the AIMLE services. |
| > capability limitations | O | Capability limitations per PLMN (which is not the service PLMN) are based on the service agreement between the vertical / ASP and the allowed MNO network and can provide limitations on the capabilities to be supported by the entities connected to that PLMN. One example is the reduced performance or energy limitations or exposure limitations when utilizing the access from the target PLMN (which is not the serving). |
| >Serving PLMN info | M | Information on the serving operator (MNO name, PLMN ID) where the subscriber is registered / connected. |

Table 8.8.3.2-1: AIMLE client discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIMLE client IDs | M | A list of AIMLE client IDs that matches the AIML discovery criteria. |
| > PLMN ID/name per AIMLE client | O | The PLMN in which the VAL UE is connected/registered. |
| AIML supported tasks | O | Indicates the discovered AIML task providers fulfilling the AIML task and AIML task requirements. It includes expected AIML task KPIs like latency, available AIML tasks service duration, list of AIML task providers like IDs or URI, and supported tasks. |

### 7.7.4	Solution evaluation

This solution addresses Key Issue #3 and introduces the capability to support discovery of AIMLE clients in cross-PLMN scenarios. This capability mainly introduces enhancements to AIMLE existing procedure as in clause 8.8 of 3GPP TS 23.482.

For checking the UE roaming status, it is assumed that Solution #10 is re-used.

ΝΟΤΕ: How to check the VAL UE roaming status will be further discussed in normative phase.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

## 7.8	Solution #8: Multi-Client Split AI/ML Operations Pipeline

### 7.8.1	Solution Description

#### 7.8.1.1	General

This solution addresses key issues related to split AI/ML operations involving multiple AIMLE clients. The proposed solution enhances split AI/ML operations by facilitating multiple AIMLE client collaboration within the same pipeline. It allows the creation of multi-client AI/ML split operation pipeline when such a pipeline is not yet created and allows joining the pipeline when such a pipeline has already been created. The solution also keeps AIMLE clients in sync with the latest status of the pipeline through notifications as more AIMLE clients join and the pipeline is adjusted according to the resources of the newly joining AIMLE clients. The solution ensures efficient resource utilization and continuous service, even with mobility and varying device constraints.

This solution allows AIMLE clients to share a common output while maintaining data privacy for each client as each AIMLE client performs local feature extraction and offloads intermediate model layers to the network, which are processed sequentially as received from the AIMLE clients. The resulting aggregated inference output is then provided to all participating clients of the pipeline.

Each participant to the split operation pipeline uses the same split operation profile and is using the same ML model (e.g., first stage ML model), this information is explicitly defined in the pipeline profile provided by the AIMLE server during the pipeline creation or joining process. This ensures that all AIMLE clients participating in the same pipeline align on the split point and can therefore share the same model in a common pipeline.

Use Cases:

-	Enhanced Smart City Surveillance with Mobility: Surveillance cameras, both stationary and mobile, capture video footage and process raw data on the cameras, sending subsequent layers to an edge server for immediate processing. AIML split operations enable real-time improved surveillance and threat detection for public safety through a shared pipeline that allows collaborative data processing. The pipeline needs to be re-adjusted based on computing resource powers for the surveillance cameras. Pipeline re-adjustment is also needed as the mobile surveillance cameras change service areas. This results in efficient resource utilization, energy saving, and larger surveillance.

-	Collaborative Autonomous Driving: A group of autonomous vehicles collaborate by sharing and processing data together. A more concreate example is vehicles utilizing shared Local Dynamic Maps (LDM) which is collaboratively updated and used by each vehicle locally. The raw data of each vehicle is processed locally and it is sent for subsequent layers in the edge server for immediate processing. The shared AIML split operation enable real-time collaboration to optimize driving conditions (e.g., efficient routes) and reduces computational burden in the vehicles. Pipeline re-adjustment is needed based on the computing resource power of the participants. Pipeline-readjustment is also needed as the vehicles change service area during mobility. This results in a larger collaboration coverage.

#### 7.8.1.2	Multi-client pipeline operation

Figure 7.8.1.2-1: Multi-client AI/ML split operation

The solution involves a multi-client split operations pipeline facilitated by AIMLE clients and servers. Initially, an AIMLE client initiates a discovery request to the AIMLE server to participate in a split operation pipeline, as illustrated in Block A of the figure. This is similar to clause 8.14.2.2 in 3GPP TS 23.482 split operation pipeline discovery. A key addition for this solution is an expansion of the split operation discovery filters in clause 8.14.3.2-2 with additional criteria for multi-client split operations within the pipeline. Following the discovery request, the split operation profile in the discovery response, as in clause 8.14.3.3 Table 8.14.3.3-2, is also expanded to include information about a pipeline with multi-client split operation pipeline support, the usage of the multi-client split operation pipeline (in usage information), and whether such a pipeline has already been created or not.

If in the discovery stage, as in Block B, the discovered instance of the split operation pipeline is a pipeline that has not yet been created, the AIMLE client will then request the creation of a new pipeline, explicitly mentioning the option for multi-client operations in the split operation profile, as shown in Block B, and subsequently perform a subscription to the split operation event. This is similar to clause 8.14.2.3 in 3GPP TS 23.482 and clause 8.14.2.5 in 3GPP TS 23.482, with additional expansion of clause 8.14.3.4-1 in 3GPP TS 23.482 to include multi-client support in the pipeline, expansion of the usage information to include further details on the multi-client pipeline, and also the computing resources of the AIMLE client to be able to efficiently adjust the pipeline later on as more AIMLE clients join the pipeline.

Subsequently, once a multi-client AI/ML split operation pipeline has been created, other AIMLE clients who seek to also engage in the multi-client AI/ML split operation pipeline begin by initiating a discovery procedure as in Block C, which is similar to Block A. If a pipeline is already created according to Block B, it identifies the existing created pipeline that supports multi-client AI/ML split operation, fulfilling the respective split operation discovery filters. The AIMLE client then proceeds with the procedures described in the Block D, the split operation pipeline join and the split operation pipeline subscription as defined in 3GPP TS 23.482. To enable adjustment of the pipeline considering the newly joining AIMLE clients, each client indicates its respective computing resources during the join. This process can be scaled up to accommodate an n-number of AIMLE clients, allowing for extensive multi-client collaboration in split AI/ML operations.

As AIMLE clients with different computing resources join the split operation pipeline, the pipeline may need to be adjusted, and the participating AIMLE clients need to be notified of changes to the pipeline so that they can make client-side adjustments. For this purpose, as shown in Block E, a split AI/ML operation notification is sent to all AIMLE clients as per clause 8.14.2.5.3 of 3GPP TS 23.482.

#### 7.8.1.3	Multi-client pipeline service continuity

A multi-client AI/ML split operation pipeline should be resilient to the mobility of the AIMLE clients. When edges are involved in the split operation pipeline, not only should the pipeline be adjusted, but AIMLE clients should also be enabled to collaborate in the pipeline even if they are in different service areas. This will enable better service continuity support. When an AIMLE client leaves a pipeline, the pipeline also needs to be adjusted to stay optimal.

This solution achieves service continuity in two ways:

1.	First, the pipeline is re-adjusted according to the computing resources of the AIMLE clients to obtain matching layers in the pipeline. For example, when AIMLE clients with low computing resources join, the pipeline is adjusted so that more parts of the pipeline are offloaded to the nodes. Conversely, when an AIMLE client with low resources leaves, the pipeline can be re-adjusted so that more parts of the pipeline can be handled by the remaining AIMLE clients.

2.	Second, when AIMLE clients change service areas, the pipeline can still be adjusted so that the common layers in the pipeline remain matched. When an AIMLE client moves to a different service area, adjustments are made based on the available AIMLE clients in the current service area. When the AIMLE client moves back to the original service area, re-adjustment is performed to keep the common layers in the pipeline matched. For example, when an AIMLE client with low computing resources moves to a different service area, part of the pipeline in the current service area can be relaxed for the other AIMLE clients so that more of the pipeline can be on the client side. The edges in the service area of the low-resource AIMLE clients can be part of the pipeline in line with the computing resources of the AIMLE clients in the service area to which it moved. The source and target AIMLE servers can continue to communicate as in step 1 of 8.24.2 TS 23.482.

#### 7.8.1.4	Procedures to support Multi-client AI/ML split operation

##### 7.8.1.4.1	Split operation pipeline join

Figure 7.8.1.4.1-1 illustrates the procedure for one or more AIMLE clients to join to a split operation pipeline with the AIMLE server.

Pre-conditions:

- The AIMLE client has performed split operation pipeline discovery with the AIMLE Server and a multi-client split operation pipeline has been found.

Figure 7.8.1.4.2-1: Split operation pipeline join

1.	The AIMLE client sends a request to join an existing split operation pipeline. To enable dynamic adjustment of the pipeline based on the capabilities of newly joining clients, each AIMLE client includes its computing resource information in the request.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request.

3.	The AIMLE server responds with either a positive acknowledgment or a negative acknowledgment with a cause of failure (e.g., failure to authorize).

##### 7.8.1.4.2	Split operation notification

Figure 7.8.1.4.2-2 illustrates the procedure for an AIMLE server to notify all AIMLE clients subscribed as per clause 8.14.2.5.3 of 3GPP TS 23.482.

Pre-conditions:

- The AIMLE clients have joined a pipeline.

- The AIMLE clients have subscribed to pipeline event as per step 1.a. of clause 8.14.2.5.2 of 3GPP TS 23.482.

Figure 7.8.1.4.2-1: Split operation notification

- When an AIMLE client joins the split operation pipeline, the AIMLE server evaluates the updated pipeline configuration and determines whether adjustments are needed to maintain optimal performance and coordination. This also triggers, as a result, a pipeline update event as per step 1 of clause 8.14.2.5.3 of 3GPP TS 23.482.

- The AIMLE server sends the split operation notification to all subscribed AIMLE clients as per step 2 of clause 8.14.2.5.3 of 3GPP TS 23.482.

### 7.8.2	Architecture Impacts

No architectural impact has been identified.

### 7.8.3	Corresponding APIs

#### 7.8.3.1	Split operation pipeline discovery request

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.

-	3GPP TS 23.482 Clause 8.14.3.2	Split operation pipeline discovery request

3GPP TS 23.482 Table 8.14.3.2-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline discovery request.

3GPP TS 23.482 Table 8.14.3.2-1: Split operation pipeline discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier) |
| Split operation discovery filters | M | Split operation discovery filters. |
| > multi-client split operation pipeline support | O | Indicates if multi-client AIMLE split operation pipeline support is required or not. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.) |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.) |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.) |
| > number of nodes | O | Minimum number of nodes required to support AIML operation splitting |

#### 7.8.3.2	Split operation pipeline discovery response

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.

-	3GPP TS 23.482 Clause 8.14.3.3	Split operation pipeline discovery response

3GPP TS 23.482 Table 8.14.3.3-1 shows the response sent by sent by the AIMLE server after processing of split operation pipeline discovery request.

3GPP TS 23.482 Table 8.14.3.3-1: Split operation pipeline discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 2) | Indicates that the request was successful. |
| > List of nodes (NOTE 1) | O | The list of discovered nodes. |
| > List of split operation profiles (NOTE 1) | O | The list of split operation profiles as specified in Table 8.14.3.3-2 of 3GPP TS 23.482. |
| Failure response | O (NOTE 2) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE 1: Only one of the IE shall be present for successful response. NOTE 2: One of the IEs shall be present. |  |  |

3GPP TS 23.482 Table 8.14.3.3-2: Split operation profile

| Information element | Status | Description |
| --- | --- | --- |
| Split operation pipeline identifier | M | The identifier of the split operation pipeline. |
| Head endpoint | M | The endpoint information of the head node for providing intermediate data. |
| Tail endpoint | M | The endpoint information of the tail node for obtaining results. |
| Multi-client split operation pipeline support | O | Indicates if multi-client split operation pipeline is supported or not. |
| Usage information | O | The usage information of the AI/ML split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Stage information | M | The list of stage(s) of the AI/ML split operation, the nested information is provided for each stage. |
| > Stage identifier | M | The identifier of the stage. |
| > number of nodes | O | Number of nodes included in the stage |
| > head node | M | Endpoint of the head node - for providing initial the inference data. |
| > tail node | M | Endpoint of the head node - for obtaining inference results. |
| > order of the nodes | M | The order of the nodes in the AI/ML split operation (including head node, tail node). |
| >> list of nodes | M | List of all discovered node in the order in which they process the data. |
| >> notification target | O | Endpoint information where the inference result of the split operation is sent by the nodes |
| > Model information | M | The ML model information used in the stage (e.g., identifiers, versions, etc.). |

#### 7.8.3.3	Split operation pipeline create request

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.

- 3GPP TS 23.482 Clause 8.14.3.4	Split operation pipeline create request

Table 8.14.3.4-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline create request.

3GPP TS 23.482 Table 8.14.3.4-1: Split operation pipeline create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Computing resource information | O | The computing resource information of the requestor. |
| Split operation requirements | M | Split operation requirements. |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.). |
| > Multi-client split operation pipeline support | O | Indicates if multi-client split operation pipeline support is required or not. |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| > notification target | O | Endpoint information where the result of the split operation is sent by the tail node. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.). |
| >> head node | M | Endpoint of the head node - for providing initial the inference data. |
| >> tail node | M | Endpoint of the head node - for obtaining inference results. |
| >> node information | M | List of Information about the nodes in order of the stage (e.g., identifier, endpoint, etc.). |

#### 7.8.3.4	Split operation pipeline join request

Table 7.8.3.4-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline join request.

Table 7.8.3.4-1: Split operation pipeline join request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID). |
| Split operation pipeline identifier | M | Identifier of the split operation pipeline for which the requestor is joining. |
| Computing resource information | O | The computing resource information of the requestor. |

#### 7.8.3.5	Split operation pipeline join response

Table 7.8.3.5-1 shows the response sent by an AIMLE server after processing of split operation pipeline join request.

Table 7.8.3.5-1: Split operation pipeline join response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

### 7.8.4	Solution Evaluation

The feasibility of sharing a split AI/ML operation pipeline among multiple AIMLE clients has been assessed in the context of the proposed solution. It is considered feasible, provided that the AIMLE server maintains a global view of the pipeline and coordinates participation of multiple clients. Each client reports its computing resources when joining the pipeline, allowing the server to adjust task distribution dynamically. Pipeline sharing relies on the existing procedures defined in 3GPP TS 23.482. During discovery, clients identify existing pipelines that support multi-client operations. Clients then perform the join procedure and subscribe to pipeline events. The server orchestrates the pipeline adjustments as new clients join or leave, and notifications are sent to all participating clients to allow appropriate client-side adaptation.

This approach ensures that the pipeline can accommodate a variable number of AIMLE clients while maintaining efficient resource utilization and consistent operation. It is assumed that participating clients follow the protocol, and that network and computing resources are within acceptable limits for split operation execution.

## 7.9	Solution #9: AIMLE server registration in a hierarchical AIMLE deployment

### 7.9.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

The solution proposes AIMLE server registration procedures considering AIMLE deployments in edge data networks as described in Annex A.4 of 3GPP TS 23.482 as shown on Figure 7.9.1-1. This solution makes no assumptions on the cardinality of ML repository(ies) deployed at the EDN or at the centralized DN.

Figure 7.9.1-1: Example hierarchical AIMLE deployment

The registration of an edge AIMLE server with a central AIMLE server enables the central AIMLE server to be aware of the edge AIMLE server capabilities and of the clients registered with the edge AIMLE server.

A central AIMLE server needs information about AIMLE clients available in different EDN service areas when performing operations requiring the selection of AIMLE clients. The AIMLE clients in an EDN service area may register to the edge AIMLE server, therefore, registration of the edge AIMLE server with the central AIMLE server is needed to support hierarchical AIMLE deployment scenarios.

#### 7.9.1.1	AIMLE server registration

Figure 7.9.1.1-1 describes a procedure for an AIMLE server to register with another AIMLE server. In such a scenario, an AIMLE server deployed as EAS, which is located at an EDN registers to a central AIMLE server.

Pre-conditions:

1.	AIMLE clients have registered with an edge deployed AIMLE server (i.e., an AIMLE server deployed as EAS which is located at an EDN) as described in clause 8.7.2.2 of 3GPP TS 23.482. The edge deployed AIMLE server receives AIMLE client profiles associated with each registered AIMLE client in the EDN service area.

2.	The edge deployed AIMLE server has been pre-configured with endpoint information of a central AIMLE server.

Figure 7.9.1.1-1: AIMLE server registration procedure

1.	The edge deployed AIMLE server sends an AIMLE server registration request to a central AIMLE server. The registration request includes the requestor identifier, endpoint information and a list of AIMLE clients registered with the AIMLE server. The request may include EDN information and AIMLE server capabilities.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server stores the registration information and creates a registration identifier. The central AIMLE server sends an AIMLE server registration response to the requestor. If the central AIMLE server has registered the requestor, the response includes an indication of success, a registration identifier and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure. If a registration request is received for a Requestor identifier that is already registered, the central AIMLE server shall either return the existing registration identifier or replace the old entry, according to local policy.

#### 7.9.1.2	AIMLE server registration update

Pre-conditions:

1.	The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.9.1.2-1: AIMLE server registration update procedure

1.	The edge deployed AIMLE server sends an AIMLE server registration update request to the central AIMLE server. The request includes the registration identifier, and may include updated AIMLE server information as described in Table 7.9.3.3-1. If the request is to renew the registration, only the registration identifier is included in the request.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server updates the information associated with the registration identifier and shall update the registration expiration time if the request only includes the registration identifier. The central AIMLE server sends a AIMLE server registration update response to the requestor. If the central AIMLE server has updated the registration, the response includes an indication of success and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure.

#### 7.9.1.3	AIMLE server de-registration

Pre-conditions:

1.	The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.9.1.3-1: AIMLE server de-registration procedure

1.	The edge deployed AIMLE server sends an AIMLE server de-registration request to the central AIMLE server. The de-registration request includes the registration identifier.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server terminates the registration indicated in the request. The central AIMLE server sends an AIMLE server de-registration response to the requestor. If the central AIMLE server has de-registered the requestor, the response includes an indication of success. If the requestor was not de-registered, the response includes an indication of failure and may include a reason for failure.

### 7.9.2	Architecture impacts

No architectural impact has been identified.

### 7.9.3	Corresponding APIs

#### 7.9.3.1	AIMLE server registration request

Table 7.9.3.1-1 shows the request sent by an edge deployed AIMLE server (i.e., an AIMLE server deployed as EAS which is located at an EDN) to a central AIMLE server for registering AIMLE server and clients in the EDN service area.

Table 7.9.3.1-1: AIMLE server registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the registering AIMLE server. |
| AIMLE server endpoint | M | The endpoint information (e.g., URL, URI, IP address, port) of the registering AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registering AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O (NOTE 1) | The identifier of the EDN service area associated with the registering AIMLE server. |
| EDN service area information | O (NOTE 1) | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registering AIMLE server. |
| EAS information | O (NOTE 2) | Indicates information of the AIMLE server deployed as EAS in the EDN, which may include EAS ID, DNAIs. |
| List of registered AIMLE clients | M | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registering AIMLE server. |
| NOTE 1: At least one of the information elements is present. NOTE 2: The information element shall be present for edge deployed AIMLE server. |  |  |

#### 7.9.3.2	AIMLE server registration response

Table 7.9.3.2-1 shows the response sent by the central AIMLE server to the edge deployed AIMLE server in response to the registration request.

Table 7.9.3.2-1: AIMLE server registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration was successful. |
| > Registration ID | M | The identifier of the registration. |
| > Expiration time | O | Indicates the expiration time of the registration. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration failed. |
| > Cause | O | Indicates the cause of the failure |
| NOTE: One of the information elements must be present. |  |  |

#### 7.9.3.3	AIMLE server registration update request

Table 7.9.3.3-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the AIMLE server registration update request.

Table 7.9.3.3-1: AIMLE server registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of an existing registration. |
| AIMLE server endpoint | O | The endpoint information (e.g., URL, URI, IP address, port) of the registered AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registered AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O | The identifier of the EDN service area associated with the registered AIMLE server. |
| EDN service area information | O | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registered AIMLE server. |
| EAS information | O | The information of the AIMLE server deployed as EAS in the EDN, which may include EAS ID, DNAIs. |
| List of registered AIMLE clients | O | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registered AIMLE server. |

#### 7.9.3.4	AIMLE server registration update response

Table 7.9.3.4-1 shows the AIMLE server registration update response sent by the central AIMLE server to the edge deployed AIMLE server.

Table 7.9.3.4-1: AIMLE server registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration update was successful. |
| > Expiration time | O | Indicates the expiration time of the updated registration. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration update failed. |
| > Cause | O | Indicates the cause the failure. |
| NOTE: One of the information elements must be present. |  |  |

#### 7.9.3.5	AIMLE server de-registration request

Table 7.9.3.5-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the AIMLE server de-registration request.

Table 7.9.3.5-1: AIMLE server de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the existing registration. |

#### 7.9.3.6	AIMLE server de-registration response

Table 7.9.3.6-1 shows the AIMLE server de-registration response sent by the central AIMLE server to the edge deployed AIMLE server.

Table 7.9.3.6-1: AIMLE server de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the de-registration was successful. |
| Failure response | O (NOTE) | Indicates that the de-registration failed. |
| > Cause | O | Indicates the cause of the failure. |
| NOTE: One of the information elements must be present. |  |  |

### 7.9.4	Solution evaluation

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployment scenarios. It provides a standardized procedure for AIMLE server registration, update, and de-registration in hierarchical AIMLE deployments. Through these procedures, the central AIMLE server can dynamically maintain awareness of edge AIMLE servers’ capabilities and the set of AIMLE clients connected to them, thereby enabling efficient hierarchical coordination and service discovery.

The solution reuses existing AIMLE messaging frameworks and introduces a set of information elements specific to hierarchical operation, such as the registration identifier, EAS information, EDN service area information, client list information, and expiration time. The procedures support registration of edge AIMLE servers with a central AIMLE server, registration update to refresh or modify information, and de-registration when an edge AIMLE server is removed or becomes inactive. Authorization checks are performed by the central AIMLE server to ensure that only valid and authenticated registration requests are accepted.

From an architectural perspective, this solution aligns with the hierarchical AIMLE deployment concept and enables the central AIMLE server to maintain latest status and configuration of edge AIMLE servers. It supports consistent management and service coordination between central and edge domains without introducing new functional entities or interface types. This solution thus provides an interoperable approach to AIMLE server management in hierarchical deployments.

## 7.10	Solution #10: Support of AIMLE services for roaming UEs

### 7.10.1	Solution description

#### 7.10.1.1	Introduction

This solution proposes enhancements to address Key Issue #3 – Open Issue #2 on support of AIMLE services for roaming UEs.

This solution proposes enhancing AIMLE client registration in roaming scenarios by enabling the H-AIMLE server to indicate to a registering AIMLE client whether it is allowed to use AIMLE services while in a visited network.

The solution assumes the following roaming models:

- Home Routed (HR) roaming architecture

- Local Breakout (LBO) roaming architecture

In the HR roaming architecture, the traffic of the AIMLE client is routed to the H-AIMLE server where the UE obtains AIMLE services from the home network.

In the LBO roaming architecture, the traffic of the AIMLE client is routed to the H-AIMLE Server or to the V-AIMLE Server dependent on the AIMLE server endpoint used by the AIMLE client. The AIMLE client is provided with V-AIMLE server information when registering with the H-AIMLE server as described in this solution.

#### 7.10.1.2	Impact to existing AIMLE client registration

The AIMLE client registration procedures and information flows in 3GPP TS 23.482 are enhanced (highlighted in bold italics) as follows.

# 2 References The following documents contain provisions which, through reference in this text, constitute provisions of the present document. - References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific. - For a specific reference, subsequent revisions do not apply. - For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document. [1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications". [2] 3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services". [3] 3GPP TS 28.104: "Management and orchestration; Management Data Analytics". [4] 3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service". [5] 3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows". [6] 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2". [7] 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access". [8] 3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management". [9] 3GPP TS 23.502: "Procedures for the 5G System (5GS)". [10] 3GPP TS 22.261: "Service requirements for the 5G system". [11] 3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture". [12] 3GPP TS 23.558: "Architecture for enabling Edge Applications". [13] 3GPP TS 23.273:"5G System (5GS) Location Services (LCS); Stage 2". [14] 3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals". [15] 3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

## 8.7 AIMLE client registration ### 8.7.1 General Prior to participation in AI/ML operations, AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When a UE is roaming and the roaming architecture is home routed (HR) (e.g., based on roaming agreement), the UE registers with an AIMLE server in the home network and AIMLE services provided by the visited network are not used. When a UE is roaming and the roaming architecture is local breakout (LBO) (e.g., based on roaming agreement), the UE first registers with an AIMLE server in the home network, then the UE may perform an AIMLE server registration with an AIMLE server in the visited network if AIMLE services usage is allowed in the visited network. NOTE: Usage of AIMLE service provided by a visited network is dependent on a roaming agreement. The following clauses specify procedures, information flows, and APIs for AIMLE client registration. ### 8.7.2 Procedures #### 8.7.2.1 General The following are supported for AIMLE client registration: - AIMLE client registration procedure; - AIMLE client registration update procedure; and - AIMLE client de-registration procedure #### 8.7.2.2 AIMLE client registration Pre-conditions: 1. The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server. 2. The AIMLE client has been pre-configured with an AIMLE client profile. Figure 8.7.2.2-1: AIMLE client registration 1. The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.2-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AIMLE client task capabilities with compute and task performance capabilities to assist with performing AIMLE client discovery and AIMLE client selection. If the UE is roaming, the registration request includes the serving PLMN identifier. 2. The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository. When the PLMN identifier is provided, the AIMLE server may verify the serving PLMN information from the AIMLE client, using the 3GPP core network services (for example, NEF monitoring event API as described in 3GPP TS 23.502 [9] and 3GPP TS 23.682 [15]), to obtain the UE roaming status and serving PLMN identifier. Further verify and confirm the serving PLMN information of the AIMLE client. If the UE is roaming, the AIMLE server determines if the registering AIMLE client is allowed to consume AIMLE services for the PLMN identifier and determines allowed AIMLE service operations. The AIMLE server saves the context of roaming (e.g., PLMN identifier, authorization for AIMLE services, allowed AIMLE service operations) in the ML repository. 3. The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request. If the AIMLE server has determined that the UE is allowed to consume AIMLE services with the PLMN identifier, the response includes the V-AIMLE server connectivity information and a list of allowed AIML service operations. If the UE is roaming and the response includes V-AIMLE server connectivity information, the AIMLE client registers with the V-AIMLE server. If the AIMLE client has successfully registered with the V-AIMLE server, the AIMLE client sends a registration update to the AIMLE server in the home network to indicate that it is consuming AIMLE services in the visited network. #### 8.7.2.3 AIMLE client registration update Pre-conditions: 1. The AIMLE client has already registered with the AIMLE Server. Figure 8.7.2.3-1: AIMLE client registration update 1. The AIMLE client sends an AIMLE client registration update request to the AIMLE server, the registration update shall include registration identifier, updated list of supported services and updated AIMLE client profile as described in Table 8.7.3.4-1. The AIML client removes services from the list of services identified by Service ID(s) when it ceases to provide a service and adds to the service when it starts to provide a service. If the UE roaming network has changed, the registration request includes the updated serving PLMN identifier. 2. The AIMLE server validates the registration update and performs an authentication and authorization check. Upon successful authorization, the AIMLE server updates the context of the AIMLE client registration in the ML repository. When the PLMN identifier is updated, the AIMLE server may verify the serving PLMN information from the AIMLE client, using the 3GPP core network services (for example, NEF monitoring event API as described in 3GPP TS 23.502 [9] and 3GPP TS 23.682 [15]), to obtain the UE roaming status and serving PLMN identifier. Further verify and confirm the serving PLMN information of the AIMLE client. If the UE roaming network has changed, the AIMLE server determines if the AIMLE client is allowed to consume AIMLE services for the updated PLMN identifier and determines allowed AIMLE service operations. The AIMLE server updates the context of roaming in the ML repository. 3. The AIMLE server returns an AIMLE client registration update response to the AIMLE client with the status of the update. If the AIMLE server has determined that the UE is allowed to consume AIMLE services with the updated PLMN identifier, the response includes the V-AIMLE server connectivity information and a list of allowed operations with the V-AIMLE server. If the UE is roaming and the response includes V-AIMLE server connectivity information, the AIMLE client registers with the V-AIMLE server. If the AIMLE client has successfully registered with the V-AIMLE server, the AIMLE client sends a registration update to the AIMLE server in the home network to indicate that it is consuming AIMLE services in the updated visited network. #### 8.7.2.4 AIMLE client de-registration Pre-conditions: 1. The AIMLE client has already registered with the AIMLE Server. Figure 8.7.2.4-1: AIMLE client de-registration 1. The AIMLE client sends an AIMLE client de-registration request to the AIMLE server. The request shall include registration identifier as described in Table 8.7.3.6-1. 2. The AIMLE server validates the deregistration request and performs an authentication and authorization check to determine whether the AIMLE client can deregister from AI/ML operations. Upon successful authorization, the AIMLE server removes the context of the AIMLE client registration and context of roaming from the ML repository. 3. The AIMLE server returns an AIMLE client de-registration response to the AIMLE client with the status of the request. If the UE is roaming and has previously registered with a V-AIMLE server, the AIMLE client de-registers with the V-AIMLE server. ### 8.7.3 Information flows #### 8.7.3.1 General The following information flows are specified for AIMLE client registration: - AIMLE client registration request and response; - AIMLE client registration update request and response; and - AIMLE client de-registration request and response #### 8.7.3.2 AIMLE client registration request Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request. Table 8.7.3.2-1: AIMLE client registration request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. List of supported profiles; M; Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided.. > AIMLE client profile; M; Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2.. > List of supported services; M; List of VAL service IDs with corresponding permissions.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. > PLMN Identifier; O; The identifier of the PLMN where the UE is currently registered.. Table 8.7.3.2-2: AIMLE client profile Information element; Status; Description. Supported AI/ML model types; O; AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks).. Supported AI/ML operations; M; AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split, continue perform intermediate AI/ML operation/task).. AIMLE client time schedule configurations; O; Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations (e.g. ML model training) in the given time slot(s) and/or day(s) of the week.. AIMLE client location configurations; O; Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID.. AIMLE client capabilities; M; AIMLE client capability information (e.g. ML application type, allowed resource usage level, supported roaming and roaming service operations).. Dataset availability; O; Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers.. Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions.. AIMLE client task capability; O See NOTE; Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs). NOTE: The Green and Energy-efficient task performance may not be applicable to a UE.. #### 8.7.3.3 AIMLE client registration response Table 8.7.3.3-1 shows the AIMLE client registration response sent by the AIMLE server to the AIMLE client. Table 8.7.3.3-1: AIMLE client registration response Information element; Status; Description. Successful response; O (NOTE); Indicates that the registration was successful.. > Registration ID; M; The identifier of the registration.. > Expiration time; O; Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires.. > Roaming configuration; O; The configuration for using AIMLE services in the visited network. >> Roaming connectivity information; O; The connectivity information to a V-AIMLE server.. >> Allowed roaming operations; O; The list of allowed operations with the V-AIMLE server. Failure response; O (NOTE); Indicates that the registration request failed.. > Cause; O; Indicates the cause of registration request failure. NOTE: One of the IEs shall be present.. #### 8.7.3.4 AIMLE client registration update request Table 8.7.3.4-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration update request. Table 8.7.3.4-1: AIMLE client registration update request Information element; Status; Description. Registration identifier; M; Identifier of the existing registration for which the update request applies.. AIMLE client profile; O; Update information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.2-2.. List of supported services; O; Update to supported service information. Services identified by their VAL Service ID are either removed from the list when the AIMLE client ceases to provide a service and added when the AIMLE client starts to provide a service.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. > PLMN Identifier; O; The identifier of the PLMN where the UE is currently registered.. > Roaming AIMLE service indication; O; The indication that the AIMLE client is registered with the AIMLE server in the visited network (e.g., the V-AIMLE server indicated by the H-AIMLE server).. #### 8.7.3.5 AIMLE client registration update response Table 8.7.3.5-1 shows the AIMLE client de-registration update response sent by the AIMLE server to the AIMLE client. Table 8.7.3.5-1: AIMLE client registration update response Information element; Status; Description. Successful response; O (NOTE); Indicates that the registration update request was successful.. > Expiration time; O; Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires.. > Roaming configuration; O; The configuration for using AIMLE services in the visited network. >> Roaming connectivity information; O; The connectivity information to a V-AIMLE server.. >> Allowed roaming operations; O; The list of allowed operations with the V-AIMLE server. Failure response; O (NOTE); Indicates that the registration update request failed.. > Cause; O; Indicates the cause of registration update request failure. NOTE: One of the IEs shall be present.. #### 8.7.3.6 AIMLE client de-registration request Table 8.7.3.6-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client de-registration request. Table 8.7.3.6-1: AIMLE client de-registration request Information element; Status; Description. Registration ID; M; Identifier of the existing registration for which the update request applies.. #### 8.7.3.7 AIMLE client de-registration response Table 8.7.3.7-1 shows the AIMLE client de-registration response sent by the AIMLE server to the AIMLE client. Table 8.7.3.7-1: AIMLE client de-registration response Information element; Status; Description. Successful response; O (NOTE); Indicates that the de-registration request was successful.. Failure response; O (NOTE); Indicates that the de-registration request failed.. > Cause; O; Indicates the cause of de-registration request failure. NOTE: One of the IEs shall be present..

### 7.10.2	Architecture Impacts

No architecture impact has been identified.

### 7.10.3	Corresponding APIs

API updates have been described in clause 7.10.1.2.

### 7.10.4	Solution evaluation

This solution addresses Key Issue #3 – Open Issue #2 on support of AIMLE services for roaming UEs. It provides an enhancement to the AIMLE client registration procedure in roaming scenarios by determining AIMLE client’s serving PLMN information and enabling the H-AIMLE server to indicate to a registering AIMLE client whether it is allowed to use AIMLE services while in a visited network. The solution supports the Home Routed (HR) roaming architecture and Local Breakout (LBO) roaming architecture.

In the HR roaming architecture, the traffic of the AIMLE client is routed to the H-AIMLE server where the UE obtains AIMLE services from the home network.

In the LBO roaming architecture, the traffic of the AIMLE client may be routed to the H-AIMLE Server or to the V-AIMLE Server dependent on the AIMLE server endpoint used by the AIMLE client. The AIMLE client is provided with V-AIMLE server information when registering or when updating registration with the H-AIMLE server.

### No architecture impact has been identified.

## 7.11	Solution #11: Solution of FL member registration in multi-operator scenarios

### 7.11.1	Solution description

This solution addresses Key Issue #3.

The solution provides a mechanism for supporting registration for candidate FL member (e.g. FL clients) which are application and/or application enablement layer entities (AIMLE server, VAL server, AIMLE clients, VAL clients), where the candidate FL members aim to operate in a service area which is covered by multiple networks operated by different operators, and the candidate FL members are connected to more than one network (either some members are connected to PLMN1 and some to PLMN2 or both).

In cross-PLMN scenario, for registering the FL members using the multi-operator information, the following enhancements are provided in 3GPP TS 23.482 clause 8.4.2 (for registration) and clause 8.4.3 (for registration update).

8.4.2	Procedure on FL member registration

Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.4.2-1: Procedure for registration on FL member registry

1.	The candidate FL member (e.g., VAL server via AIMLE server or AIMLE server) sends an FL member registration request to the ML repository for registering to the ML repository which acts as the AIML service registry.

In case of multi-operator support, this request also includes information on the serving PLMN for the communication of the FL member (e.g. towards other FL members and/or the network), as well as the allowed MNO info including all MNO networks from which the candidate FL member can cope with if selected in a potential FL operation.

NOTE: Such allowed MNO info is applicable to scenarios when the candidate FL member has limitations being grouped with other FL members being connected to other PLMNs. Such limitations are imposed based on the vendor providing the service in cross-PLMN scenarios, e.g. based on an interoperability indication. If the vendor is the same is the same across FL member connected to different PLMNs, then it may be possible that the FL members of different PLMNs collaborate e.g. in cases like VFL.

2.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request. The ML repository stores the identity of the candidate FL member together with a mapping to one or more PLMNs, and with associated role as serving or allowed PLMN.

3.	The ML repository sends the generated information in the FL member registration response message to the candidate FL member.

The procedure for AIMLE client to register to ML repository as FL member is introduced in clause 8.7.2.2.

8.4.3	Procedure on FL member registration update

Figure 8.4.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.4.3-1: Procedure for registration update

1.	The candidate FL member (VAL server via AIMLE server or AIMLE Server) sends an FL member registration update request to the ML repository acting as AIML service registry for updating the registration of the FL member.

In case of multi-operator support, this request also includes information on the serving PLMN for the communication of the FL member (e.g. towards other FL members and/or the network), as well as the allowed MNO info including all MNO networks from which the candidate FL member can cope with if selected in a potential FL operation.

2.	The ML repository validates the received update request and generates the identity and other security related information for the FL member. The ML repository stores the identity of the candidate FL member together with a mapping to one or more PLMNs, and with associated role as serving, or allowed PLMN.  In case of registration update, the mapping to the one or more PLMNs and/or the roles may be updated accordingly.

3.	The ML repository sends the generated information in the FL member registration update response message to the candidate FL member.

The procedure for AIMLE client to update its registration to ML repository as FL member is introduced in clause 8.7.2.3.

### 7.11.2	Architecture Impacts

The architecture impact is the required enhancement of the AIMLE functional model to support multi-operator scenarios (as in clause 6.2).

### 7.11.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #7. This provides enhancements to the following information flows based on 3GPP TS 23.482 clause 8.4.4.1. The additional information elements as described in the above procedure, w.r.t. multi-operator support, will enhance the existing API as in 3GPP TS 23.482 clause 9.3.1 (MLR_FLMemberRegister API).

### 7.11.4	Solution evaluation

This solution addresses Key Issue #3 and introduces the capability to support FL member registration and registration update in cross-PLMN scenarios. This capability mainly introduces enhancements to AIMLE existing procedures as in clause 8.4.2 and 8.4.3 of TS 23.482.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

## 7.12	Solution #12: Support for ML model split learning using relays

### 7.12.1	Solution description

This solution addresses Key Issue #4. The solution provides a mechanism for identifying and determining an intermediate entity (VAL UE) to serve as relay for a split learning operation, and particularly a split AI/ML operation.

Such intermediate entity can be an application layer relay UE which either doesn’t provide any processing and is not aware of the data to relay (encrypted), or 2) intermediate node which undertakes some layers of the split operations (e.g. layers 5-15) instead of the source VAL UE. For both cases, the intermediate entity is expected to be associated with the split operation pipeline as processing entity or just relaying entity.

The need for such intermediate node if it is a relay will help in scenarios when the network conditions from the VAL UE to the network are poor or the UE is expected to be out of coverage; hence the role of a relay will support the AIMLE service continuity without disrupting the application session performance in a UE-to-Network relay fashion. For the case when the intermediate node is handling part of the processing (some layers/cuts), the scenario can be related to high computational load or energy consumption at the UE side, which requires to offload part of its processing to another VAL UE in close proximity.

This solution has two procedures:

- Enhancing AIMLE client registration procedure to allow the AIMLE client to register its capabilities to ML repository to serve as candidate intermediate node.

- Introducing an intermediate node discovery procedure to allow discovering the intermediate node for the split learning operation.

#### 7.12.1.1 	Intermediate node registration procedure

The registration procedure provides enhancement to AIMLE client registration procedure as described in 3GPP TS 23.482 clause 8.7.2.2 (bold italics is new).

----------------------- text from 3GPP TS 23.482 clause 8.7.2.2 (bold italics is new)---------------

8.7.2.2	AIMLE client registration

Pre-conditions:

1.	The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server.

2.	The AIMLE client has been pre-configured with an AIMLE client profile.

Figure 8.7.2.2-1: AIMLE client registration

1.	The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.2-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AIMLE client task capabilities with compute and task performance capabilities to assist with performing AIMLE client discovery and AIMLE client selection.

For the case that the service is about an AI/ML split learning operation, the AIMLE client requests to register as relay/intermediate node for the service ID. Such indication can be part of the AIMLE client profile as in Table 8.7.3.2-2.

The AIMLE client may also indicate the policies / criteria in which the VAL UE can act as candidate relay/intermediate node. This includes the time and area of validity, a list of supported VAL services/servers or vendors which are part of the Split AIML operation, compute and energy requirements and constraints for the utilization of the VAL UE (e.g. only use as relay if battery is higher than 50% or in low load scenarios or in good channel conditions (e.g. cell centre)). This may also include criteria for the UE mobility or relative mobility, i.e., UEs that move towards the same direction with a speed that allows connectivity could be considered as candidate relay nodes.

2.	The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository.

The ML repository also stores the association of the AIMLE server with a certain candidate role for a split learning operation. The storage at the repository maps the AIMLE client ID with an ML task/service type (or list of services which support split learning), a role indication and a role type (relay, intermediate processing node).

3.	The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request

Table 8.7.3.2-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split). |
| AIMLE client time schedule configurations | O | Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations in the given time slot(s) and/or day(s) of the week. |
| AIMLE client location configurations | O | Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| AIMLE client capabilities | M | AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIMLE client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| Information on the model split option or topology | O | Information on how the ML model can be split, given that different option may depend on the model type. ML model split option can be equivalent to model split topology. This refers to how a model is divided and distributed across different devices or computational units for training or inference. These topologies can be broadly categorized into data parallelism, model parallelism, and hybrid approaches. Data parallelism involves splitting the training data, while model parallelism involves splitting the model itself. Hybrid approaches combine both data and model parallelism to optimize performance. |
| Split Operation/Learning related information supported role/capability | M | The VAL UE capability and availability information for acting as intermediate / relay node for the Split Operation and the type of relay (A&F, S&F, handle some layers/cuts). |
| > supported role(s) | M | The role of the AIMLE client as relay/intermediate node and the type of relay (A&F, S&F, handle some layers/cuts). |
| > VAL policies for split operation | O | The policies related to the selection of the VAL UE as relay/intermediate node (compute, load, energy constraints and requirements). |
| > List of DNs/VAL servers | M | The list of servers/DNs for which the VAL UE can be utilized in the split operation as relay. |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

--------------------------------------------------------------------------------------------------------------------------------------------

#### 7.12.1.2 	Intermediate node discovery procedure

Pre-conditions:

- A split AIML pipeline is created, and the UE and VAL server has an ongoing connection session for undertaking jointly the split learning operation.

- One or more VAL UEs or edge nodes (e.g. EAS) have registered their capabilities in the ML repository / AIMLE server.

Figure 7.12.2.1-1: Support for split relay operation discovery

1.	A VAL UE detects that a ML model performance is degraded or expected to degrade. The detection can have different flavours either based on UE mobility and channel conditions or due to load/energy status and degradation of the ongoing ML task. More specifically, the detections can be due to:

- VAL client notifies the AIMLE client that the task is delaying or expected to delay.

- AIMLE client utilizes ADAE analytics on VAL session performance (based on TS 23.436) and predicts the downgrade of the link between the VAL UE and the VAL server

- High load or energy consumption at the VAL UE side, due to high app load or lower layer processing

- UE mobility prediction to an area with no coverage or poor coverage or bad channel conditions.

2.	The AIMLE client determines a requirement for updating the pipeline to include a relay/intermediate node and sends a discovery request to the AIMLE server.

The discovery request includes discovery criteria for the selection of the relay, including capability of the VAL UE (apps onboard, workloads supported, processing power and capacity, energy consumption limitations/restrictions), the status of the radio capability of the UE (RATs/spectrum supported, channel conditions, interference limitations), whether the UE is static/low or high mobile UE, whether it supports dual-connectivity (or multi-connectivity), whether it can be connected to the server via non-3GPP means (Wifi, NTN,..), the mobility of the UE or relative mobility to the UE of interest (source UE), etc.

The requirement can include also requested topology/option for the split model operation for which the relay/intermediate nodes need to be discovered. Alternatively, the model type may be provided in the request if there is an association between the type and the topology at the server side (as pre-configuration).

NOTE: Split model topology refers to how a model is divided and distributed across different devices or computational units for training or inference. These topologies can be broadly categorized into data parallelism, model parallelism, and hybrid approaches.

3.	The AIMLE server requests from ML repository and receives the registered nodes (available AIMLE clients, EASs to serve as candidate relays) with their roles as requested at the discovery criteria.

4.	The AIMLE server sends a discovery response to the AIMLE client including the discovered nodes and their capabilities. Such response includes the split learning operation related information such as VAL UE capability and availability information for acting as intermediate / relay node for the Split Operation and the type of relay, the supported split model topologies, VAL policies on the split operation etc.

5.	The AIMLE client after receiving the list of discovered AIMLE clients, it checks whether the other VAL UEs in the list are active and in close proximity. Hence it sends a groupcast/broadcast or unicast message via AIMLE-PC5 to all listed AIMLE clients, to receive reporting on the availability and conditions related to the sidelink interface. The other AIMLE clients can provide response if available and the source AIMLE client will establish a connection/session with them if they fulfil the application layer KPI (e.g. latency for the handshake is less than a pre-defined threshold).

Additionally, the sidelink status check can be in form of requesting from the target UEs additional /supplementary information on the channel status, load, energy and mobility information locally. The status check can be also using ADAE analytics for VAL UE-to-UE session performance stats/predictions as specified in TS 23.436.

6.	The AIMLE client of the VAL UE of interest (source UE) selects one of the candidate intermediate/relay nodes (edge or UE based relays) given criteria which can be equivalent to the discovery criteria and the performance KPI for the split learning operation. This may be based on local or VAL UE or server policies on the selection, e.g. to select UEs to meet KPIs with higher energy /load remaining capacity or prefer static UEs or edge nodes vs UEs.

Then the AIMLE client sends the selected intermediate node (AIMLE client or EAS) to the AIMLE server, including the identifier of the selected node and the time to activate (time of validity) for the addition to the pipeline, as well as to a pipeline ID (of the pipeline to be modified).

7.	AIMLE server associates the selected intermediate node in the AIML pipeline. This can be performed in different ways based on whether the intermediate node is a UE or an edge node and whether the intermediate node is expected to undertake part of the processing:

- If the relay is a UE (AIMLE or VAL client), the AIML pipeline update adds the relay UE as another entity in the existing pipeline with the associated flag as relay/intermediate and the type of relay and the layers that it is going to process (or this can be an indication of the type of processing that the relay will undertake if this is pre-configured, e.g. “relay_split_X corresponds to relay node handling layers 5-15”)

- If the relay is an edge node (EAS/edge AIMLE) then the AIMLE pipeline update add the edge node as another entity in the pipeline or creates two further pipelines (VAL UE-edge node, edge node to VAL server) which are coupled/associated with the first pipeline for the give AIML task/ split learning operation.

8.	After step 7 (or this can be before previous steps given the scenario) the AIMLE server notifies the VAL server and the intermediate AIMLE client on the addition of the intermediate / relay node to the pipeline (or to the creation of dependent/partner pipelines to the existing pipeline).

### 7.12.2	Architecture Impacts

There is foreseen architecture impact since we introduce in AIMLE-PC5 new capability for supporting relaying or offloading part of the processing for the Split Learning operation. So, the AIMLE client can be either an intermediate processing or relay node for a split learning task related to another AIMLE client.

### 7.12.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #12. For the procedure in 7.12.1, the enhancements relate to existing APIs as in TS 23.482:

- AIMLE client registration (Aimles_AIMLEClientRegistration)

This subclause provides also a summary on the corresponding API for the discovery procedure.

-	Intermediate Node Discovery API (request / response model; API provider: AIMLE server or ML repository; known consumer: AIMLE client or server; corresponding to step 2-4 of clause 7.12.1).

-	Intermediate Node Selection Confirm API (request / response model; API provider: AIMLE client; known consumer: AIMLE server; corresponding to step 6 of clause 7.5.1).

- 	Intermediate Node Notify API (subscribe / notify model; API provider: AIMLE server; known consumer: AIMLE clients, VAL server; corresponding to step 8 of clause 7.5.1).

### 7.12.4	Solution evaluation

This solution addresses Key Issue #4 and provides mechanisms for identifying and determining an intermediate entity (UE, edge function) to serve as relay for a split learning operation, and particularly a split AI/ML operation. Such mechanisms include the registration of the relay/intermediate node (UE) and the discovery of the node for split learning operation. Such mechanisms propose enhancements to AIMLE client registration as in 3GPP TS 23.482 to support registering nodes as intermediate nodes in such split operations.

If the requirement for offloading the processing from one AIMLE client to another is not related to the network conditions (e.g. related to the load or energy status of the VAL UE), then an alternative would be to offload part of the processing to an edge node (e.g. EAS). Such option can be explored as complementary in normative phase.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

## 7.13	Solution #13: ML model training in a hierarchical AIMLE deployment

### 7.13.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

The solution proposes ML model training procedures considering AIMLE deployments in edge data networks as described in Annex A.4 of 3GPP TS 23.482 [2]. This solution makes no assumptions on the cardinality of ML repository(ies) deployed at the EDN or at the centralized DN.

ML model training procedure is described in Clause 8.3 of 3GPP TS 23.482 [2]. In a hierarchical AIMLE deployment scenario, a VAL server requests the central AIMLE server for ML model training. Since the member clients that will participate in the training process may be distributed in different EDN service areas, the central AIMLE server can distribute the training request to the corresponding edge AIMLE servers.

Figure 7.13.1-1 describes a procedure for central and edge AIMLE servers to support ML model training based on the request from a VAL server in a hierarchical AIMLE deployment.

Pre-conditions:

1.	AIMLE clients have registered with the corresponding edge deployed AIMLE servers as described in clause 8.7.2.2 of 3GPP TS 23.482 [2]. The edge deployed AIMLE servers receive AIMLE client profiles associated with each registered AIMLE client in the EDN service area.

2.	The edge deployed AIMLE servers have registered with the central AIMLE server as described in clause 7.9 (Solution #9).

Figure 7.13.1-1: ML model training in a hierarchical AIMLE deployment

1.	A VAL server sends an ML model training request to a central AIMLE server, as described in step 1 of Figure 8.3.2-1 in TS 23.482. The request includes information described in Table 8.3.3.1-1 in TS 23.482 [2].

2.	The central AIMLE server checks whether the VAL server is authorized to perform the ML model training request, as described in step 2 of Figure 8.3.2-1 in TS 23.482 [2]. The central AIMLE server uses information from the AIMLE server registration procedure described in clause 7.9 to identify edge AIMLE servers that can fulfil the ML model training requirements.

3.	Based on the ML model training request and the identified edge AIMLE servers, the central AIMLE server determines to distribute the training request to the identified edge AIMLE servers. The central AIMLE server identifies the EDN service areas based on information in the ML model training request (e.g. training type, list of member clients, member selection criteria) and the AIMLE client profiles (e.g. supported AI/ML operations, AIMLE client capabilities) received from the AIMLE server registrations. For example, the central AIMLE server determines the distribution of the request based on the number of required AIML clients in the request and the number of available clients in each EDN service area or based on the dataset identifiers in the request and the dataset availability of clients in each EDN service area. The central AIMLE server sends an Edge ML model training request to the edge AIMLE server in each of the identified EDN service areas.

4.	The edge AIMLE server sends an Edge ML model training response to the central AIMLE server.

5.	The central AIMLE server returns the success or failure response, as described in step 3 of Figure 8.3.2-1 in TS 23.482 [2]. The response includes information described in Table 8.3.3.2-1 in TS 23.482 [2].

6. 	The edge AIMLE server notifies the central AIMLE server of the progress of the ML model training, including the list of member clients selected or de-selected for the ML model training, the training output, the percentage completion, or errors encountered during the training process.

7.	The central AIMLE server aggregates the notifications received from the edge AIMLE servers and sends the aggregated notification to the VAL server. The notification includes information described in Table 8.3.3.3-1 in TS 23.482 [2].

### 7.13.2	Architecture impacts

No architectural impact has been identified.

### 7.13.3	Corresponding APIs

#### 7.13.3.1	Edge ML model training subscription request

Table 7.13.3.1-1 describes the information flow from the central AIMLE server to the edge AIMLE server for subscribing to ML model training in the EDN service area.

Table 7.13.3.1-1: Edge ML model training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the central AIMLE server of the edge ML training request. |
| Original requestor identity | O | Identity of the VAL server of the original ML training request. |
| Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| Training type | M | Identifies whether the VFL or HFL training to be performed. |
| List of member clients | O (NOTE) | List of member clients in the EDN service area to be utilized for training the ML model. |
| Member selection criteria | O (NOTE) | Identifies the criteria that needs to be continuously monitored for selecting the member clients (e.g., AIMLE clients in a particular location, member availability duration, new member clients registering with the required capabilities). |
| Number of required AIML clients | O | Indicates the requested number of AIML clients in the EDN service area to be selected based on member selection criteria. |
| ML model information | M | Identifies the ML model (specified in the original request or selected by the central AIMLE server) that has to be distributed to the selected member clients in the EDN service area for training. The information is described in Table-8.11.4.1 of TS 23.482 [2]. |
| Dataset identifiers | M | Identifier of dataset used to HFL or VFL training. For VFL training, multiple dataset identifiers is specified for different data domains. |
| Number of data samples | O | The number of data samples required for a round of HFL or VFL training. |
| VFL specific parameters | O (NOTE 2) | Parameters specific to VFL training |
| > Dataset common features | O (NOTE 2) | A list of one or more common features is specify for VFL training. Common features can be UE identifiers, AIMLE client identifiers, group identifier, VAL service identifier, area of interest, and VAL service area. |
| > Data domain feature ID lists | O (NOTE 2) | List of features for each data domain(s) of the datasets at the UE for VFL training. |
| > Feature alignment information | O | Information provided to align features from dataset of different domains for VFL training. Alignment information is also provided for data domain features with ground truth data. |
| > Data labels | O | Ground truth data provided for VFL training. |
| Training objective | O | Identifies the termination condition for the ML model training. Table 8.3.3.1-2 in 3GPP TS 23.482 [2] describes the information related to training objective. |
| Operational requirements | O | Requirements for the training operations, e.g. latency, QoS requirements, etc. |
| Operational schedule | O | A schedule for when training is to occur. |
| Reporting requirements | O | Indicates what information regarding the training operations needs to be sent to the central AIMLE server by the edge AIMLE server (e.g. update related to new member clients selected or de-selected). The reporting requirements includes a specific schedule for sending notifications. |
| NOTE 1: At least one of these IEs shall be present. NOTE 2: These IE shall be present for VFL training. |  |  |

#### 7.13.3.2	Edge ML model training subscription response

Table 7.13.3.2-1 describes the information flow of Edge ML model training subscription response from the edge AIMLE server to the central AIMLE server.

Table 7.13.3.2-1: Edge ML model training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE 1) | Indicates that the Edge ML model training request was successful. |
| > Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| > EDN service area identifier | M | The identifier of the EDN service area associated with the edge AIMLE server. |
| Failure response | O (NOTE 1) | Indicates that the Edge ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: Only one of these information elements shall be present. |  |  |

#### 7.13.3.3	Edge ML model training notification

Table 7.13.3.3-1 describes the information flow of Edge ML model training notification from the edge AIMLE server to the central AIMLE server.

Table 7.13.3.3-1: Edge ML model training notification

| Information element | Status | Description |
| --- | --- | --- |
| Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| EDN service area identifier | M | The identifier of the EDN service area associated with the registering AIMLE server. |
| EDN service area information | O | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the edge AIMLE server. |
| Percentage completion | M | Indicates a completion percentage for the training. |
| List of AIMLE clients | O (NOTE) | Indicates the list of AIMLE clients selected or de-selected for the ML model training in the EDN service area. |
| Training output | O (NOTE) | Output of training, e.g., ML model parameters for the training. |
| Errors list | O (NOTE) | A list of errors, if any, encountered during training process. |
| NOTE: At least one of these IEs shall be present. |  |  |

### 7.13.4	Solution evaluation

This solution addresses Key Issue 	#6: support for architecture and functions enhancement on diverse AIMLE deployment scenarios. It provides a standardized procedure for ML model training in hierarchical AIMLE deployments. The procedure allows an ML model to be trained by AIMLE clients distributed in different EDN service areas which are managed by different edge AIMLE servers.

The solution introduces a set of information elements specific to hierarchical operation, such as edge training identifier, EDN service area identifier, and EDN service area information. The procedures support distributing ML model training request from a central AIMLE server to edge AIMLE servers, and support aggregation of training outputs from the edge AIMLE servers.

### From an architectural perspective, this solution aligns with the hierarchical AIMLE deployment concept described in Annex A.3 and A.4 of 3GPP TS 23.482 [2] and enables a central AIMLE server to manage ML model training performed in such hierarchical deployment. It supports consistent management and service coordination between central and edge domains without introducing new functional entities or interface types. This solution thus provides an interoperable approach to AIMLE operational management in hierarchical deployments.

## 7.14	Solution #14: Enhance AIMLE Client Discovery

### 7.14.1	Solution Description

As introduced in 3GPP TS 23.482, the AIMLE AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses the information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When the AIMLE server/client are deployed hierarchically, enhancements to the AIMLE client discovery services are needed to support AIMLE service provisioning.

This clause specifies the impact to existing information flows for the enhancements of AIMLE client discovery services.

The AIMLE information flows in 3GPP TS 23.482 for AIMLE client discovery can be enhanced (highlighted in bold italics) as follows.

### 8.8.3 Information flows #### 8.8.3.1 AIMLE client discovery request Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure. Table 8.8.3.1-1: AIMLE client discovery request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. AIMLE client discovery criteria; M; Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. Number of required AIMLE clients; O; Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria.. Table 8.8.3.1-2: AIMLE client discovery criteria Information element; Status; Description. Service requirement; M; Information about the required service. > VAL Service ID; M; VAL Service ID that the client is required to support. This identifies the service associated with the requester.. > Service permission level; O; Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage).. Requested ML model types; O; Requested ML model types (decision trees, linear regression, neural networks).. Requested AIML operations; M; Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split.. Application layer AIMLE client capabilities; M; Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate).. Dataset requirements; O; Information about dataset.. > Dataset availability; O; Dataset availability, including dataset identifiers, dataset size, age, list of dataset features.. > Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions.. AIML client task capability requirements; O; Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs). AIMLE client velocity; O; Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static.. Location information; O; Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID, DNN and DNAI(s)) to discover the AIMLE clients.. AIMLE client QoS requirements; O; Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients.. Edge AIMLE server Information; O; Information about edge deployed AIMLE server(s).. > EAS information; O See NOTE; Indicates information of the AIMLE server deployed as EAS, which may include EAS ID, EAS Endpoint, DNAIs and service area of the EAS.. > AIMLE server ID; O See NOTE; The identifier of the AIMLE server.. NOTE: At least one of the IEs shall be provided when edge deployed AIMLE server.. #### 8.8.3.2 AIMLE client discovery response Table 8.8.3.2-1 shows the response sent by the AIMLE server to the VAL server for the AIMLE client discovery procedure. Table 8.8.3.2-1: AIMLE client discovery response Information element; Status; Description. Status; M; The status for the request: success or fail.. List of AIMLE client IDs; M; A list of AIMLE client IDs that matches the AIML discovery criteria.. > AIML supported tasks; O; Indicates the discovered AIML task providers fulfilling the AIML task and AIML task requirements. It includes expected AIML task KPIs like latency, available AIML tasks service duration, list of AIML task providers like IDs or URI, and supported tasks.. > EAS information; O See NOTE; Indicates information of the AIMLE server deployed as EAS (which serving the AIMLE client), which may include EAS ID, EAS Endpoint, DNAIs and service area of the EAS.. > AIMLE server ID; O See NOTE; The identifier of the AIMLE server which serving the AIMLE client.. NOTE: At least one of the IEs shall be provided when the AIMLE client is served by edge deployed AIMLE server..

### 7.14.2	Architecture Impacts

### There were no identified architectural impacts in this solution.

### 7.14.3	Corresponding APIs

### API updates have been described in clause 7.14.1.

### 7.14.4	Solution evaluation

This solution addresses Key Issue #6 study aspects 2 and 3 and provides enhancement of the AIMLE client discovery to support AIMLE service provisioning when the AIMLE server/client are deployed hierarchically. New IEs are added to the AIMLE client discovery criteria for edge AIMLE server information, and to the AIMLE client discovery response considering the case when AIMLE client is served by edge deployed AIMLE server.

### This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

## 7.15	Solution #15: Sample alignment for VFL

### 7.15.1	Solution description

The solution enhances the clause 8.18 as follows:

### 8.18.2 Procedure for supporting VFL Pre-conditions: 1. VFL members registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server. 2. The datasets of each of the UEs (where the AIMLE Clients (as VFL members) are deployed on) belong to more than one different data domains. 3. The VAL server has successfully subscribed/registered with the AIMLE server for model training notifications. Figure 8.18.2-1: Procedure for supporting VFL 0. An AIMLE client registers with AIMLE server as specified in clause 8.7 of 3GPP TS 23.482 [2]. The registration message includes the list of feature collection capabilities and also the data domains in which features can be collected and processed. The registration message also includes the list of feature ID(s) status its expiration time. NOTE 1: The AIMLE server keeps record of all AIMLE clients with similar feature collection capabilities and data domain. 1. An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. The request includes sample pool, and sample binding criteria. The member selection criteria are enhanced to select the members for VFL training if they have features bound to at least one sample withing the sample pool. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between different data domains), the procedure continues to step 2. The AIMLE server also assigns ML task identity to the determined task. The AIMLE server selects the candidate AIMLE clients for the training based on the corresponding feature collection record of AIMLE clients that are capable of collecting features within the scope of determined VFL task and sample pool. The AIMLE server sends response which includes ML task identity and candidate AIMLE client list. 2. [Optional] The AIMLE server retrieves the indicated ML model in step 1 using the ML model retrieval procedure as described in clause 8.1. If the retrieved model is already trained and meets the machine learning requirements requested by the consumer, the procedure continues to step 7. 3. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the VFL training as described step 4 of the clause 8.13.2. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the VFL training. 4. [Optional] The AIMLE Server sends a client feature selection request to some clients asking to activate or de-activate some of their feature ID(s)/ feature list for each data domain. The message includes information as described in following Table. Table 8.18.2-1: Client feature selection request Information element; Status; Description. Requestor identifier; M; The identifier of the AIMLE Server.. Feature ID(s); M; Server requests List of feature ID(s) targeted for activation or deactivation.. > Action type; M; Server specifies via indicators whether the request is for activation or deactivation.. Feature Priority Level indicator; O; Indicates the priority of the feature for VFL training. Minimum expiration Time indicator; O; Server informs the client UE(s) on the minimum expiration time to enforce the requested action (activate /deactivate features). 5. [Optional] AIMLE client sends response indicating the activation or deactivation status of their feature ID(s) or feature list for each data domain. The response message includes information as described in following Table. Based on the response message, the AIMLE server finds common feature ID(s) by comparing features descriptions / feature ID(s) of clients. The AIMLE server constructs the pairwise relation among candidates' datasets considering their shared feature ID(s). Table 8.18.2-2: Client feature selection response Information element; Status; Description. Successful response (NOTE); O; Indicates that the request was successful.. > Feature ID(s); M; List of feature ID(s).. >> Action Type status; M; Status of feature ID(s) activate/deactivate operation. > Feature priority level success rate; O; Indicates the percentage of the available features prioritized for VFL training.. > Feature Minimum expiration Time reliability rate; O; Client (UE)s informs the server about the specified feature set/feature ID(s) enforced expiration Time. In case of conflict of interest between UE and server, UE send remarks about further adjustment in expiration time. Failure response (NOTE); O; Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: any one IE shall be present.. NOTE 2: The AIMLE server gets the hashed feature values from the candidates for their data samples if the feature ID(s) are subsets of feature space union 6. AIMLE server send a sample binding request to AIMLE clients that are capable of collecting or having features corresponding to the defined VFL task. This request contains a set of samples that are defined to be the sample pool of the VFL task by the VAL server. The message includes information as described in following Table. Table 8.18.2-3: Sample binding request Information element; Status; Description. Requestor identifier; M; The identifier of the AIMLE Server.. VFL Task ID; M; The identifier of the VFL task assigned to AIMLE server by the VAL server. Sample Pool; M; A set of samples and their corresponding IDs that are defined within the scope of VFL task to be trained on them.. Sample binding criteria; M; Identifies the criteria (e.g. common feature IDs) that the available or collecting dataset (features) can be bound to samples within the sample pool.. 7. AIMLE client receives the binding request, and it matches its available dataset (i.e. features) at the UE to a corresponding sample within the sample pool of the VFL task according to the received sample binding criteria. ALMLE client sends the sample binding response to AIMLE server. If the ALMLE client was able to match its dataset (i.e. features) to at least one sample within the sample pool of the VLF task, it sends the successful binding response to ALMLE server to continue the process of VFL sample alignment. Otherwise, it sends a failure response to the sample binding request by AIMLE server. The binding response message includes information as described in following Table. Table 8.18.2-4: Sample binding response Information element; Status; Description. VFL Task ID; M; The identifier of the VFL task assigned to AIMLE server by the VAL server. Binding result; M; The binding result either “success” or “failure”. 8. The AIMLE Server gets VFL members information (the VFL members information may be included in the request in step 1 (Table 8.3.3.1-1) or be obtained through step 3 or according to the responses received in step 7. The AIMLE Server interacts with the VFL members (AIMLE Clients which are deployed on UEs) for each data domain for ML model training capability evaluation as described in clause 8.19.2. 9. The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 4 and parameters received in step 1 (Table 8.3.3.1-1). The AIMLE Server determines VFL members for each data domain. The criteria used by the AIMLE Server include: - Available data and minimum number of data samples for the same sample among the VFL members. - Feature alignment of the sample/datasets with data labels among the VFL members. - Available time of the VFL members for support the VFL training operations. - Capability and minimum number of the VFL members for the VFL training operations. - AIML model information for the VFL members and for the AIMLE Server. 10. The AIMLE Server coordinates the selected VFL members for VFL training. During VFL training process, the VFL members send intermediate results to the AIMLE Server, and the AIMLE Server responds to the VFL members with the updated information (e.g. gradients). The information from the AIMLE server can be used to update the model parameters maintained at each VFL member for the different data domains. 10a. The AIMLE server may report to the VAL server with the training status, that includes intermediate training results, The VAL server may adjust its request on the ML model training. If the VAL Server is providing data labels to complete the training, the VAL Server sends updated training parameters for the AIMLE Server to distribute to the VFL members. The updated training parameters apply for models of VFL members associated with each data domain. NOTE 4: How AIMLE Server coordinates the selected VFL members to perform VFL training is out of scope for this release. 11. The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 6 are repeated for the next training round.

### 7.15.2	Architecture Impacts

No architecture impacts are identified for this solution.

### 7.15.3	Corresponding APIs

The Client feature selection request message includes information elements as specified in Table 8.18.2-1. The Client feature selection response message includes information elements as specified in Table 8.18.2-2.

The Sample binding request message includes information elements as specified in Table 8.18.2-3. The Sample binding response message includes information elements as specified in Table 8.18.2-4.

### 7.15.4	Solution evaluation

The solution is related to KI #7: VFL sample alignment across VAL servers. The solution resolved open issue 1 “Whether and how to enhance VFL operation considering also VAL servers and AIMLE servers as VFL participants”. The solution proposes to find the same training samples (sample alignment problem) among training participants that have different features of it, in a way that the privacy of the participants and their data is preserved during the training process. This solution addresses the scenarios in which VAL server requests assistance from AI/ML Enablement Server to perform Vertical Federated Learning among AIMLE clients. The AI/ML Enablement Server clusters all AIMLE clients based on their data (feature) collection capability, performs sample alignment (i.e. selecting clients, which have different features of the same sample to be participated in vertical federated learning) for all samples within the scope of the defined VFL task among all different possible AIMLE clients. There is no architecture impact identified based on this solution. Whether binding criteria provided by VAL server to AIMLE server is same provided by AIMLE server to AIMLE client can be considered in normative phase.

## 7.16	Solution #16: Solution on AIMLE service migration support in UE roaming scenarios

### 7.16.1	Solution description

This solution addresses Key Issue #3. The solution provides a mechanism for supporting the continuity of the ML training / inference capability in scenarios where the ML model training / inference happens at the UE (e.g. AIMLE client) while the UE client is expected to move to an area served by another PLMN.

In this solution, the assumption is that the VAL UE undertakes an ML operation (such as training or inference) and is served by a source AIMLE server to which he is connected via H-PLMN, and the VAL UE is roaming while the operation is ongoing. In such scenario, to prevent AIML service disruption, this solution provides a mechanism to enable the migration of AIMLE service to the target AIMLE server who is bound to VPLMN.

Such solution requires that service / roaming agreements between AIMLE service providers in both PLMNs are in place to support service the roaming UEs.

Pre-conditions:

- The AIMLE-provided ML model training/inference support service is ongoing.

Figure 7.16.1-1 procedure for support for VAL UE roaming

1.	The AIMLE client via the VAL client or other SEAL clients (LMC, ADAEC) monitors UE mobility or analytics related to the expected or predicted mobility of the UE. The AIMLE client detects a mobility of UE to an area covered by different PLMN and sends a mobility report to AIMLE server indicating a cross-PLMN mobility event for the VAL UE.

2.	AIMLE server identifies (check with Core Network) the V-PLMN info and the time when the VAL UE is expected to reach V-PLMN.

3.	AIMLE server fetches from ML repository information on available AIMLE servers in target V-PLMN.

NOTE 1: If the ML repository is deployed by PLMN/MNO, then this step requires interaction among ML repositories of the same provider based on the service agreements or synchronization via the AIMLE servers. The mechanism for this coordination is up to implementation.

NOTE 2: It is assumed that the available AIMLE servers for the V-PLMN are pre-configured at the ML repository.

4.	AIMLE server sends a notification or request for selecting a new AIMLE server, where the message includes the list of available AIMLE servers in VPLMN and some context information (e.g. rating/ranking, vendor info, capability information, energy/load status).

5.	AIMLE client based on the received list, selects the T-AIMLE server from a list of AIMLE servers, based on the fetched information. The selection criteria can be:

- Capabilities (computational, processing) and coverage (geographical or topological area covered by T-AIMLE server)

- Load and energy status of the target AIMLE server, based on monitoring or querying possible overload or high energy consumption (can be provided by the target AIMLE service provider, e.g. edge platform provider).

- Expected delay / latency for accessing the T-AIMLE server (e.g., in case of relaying ML inference task). This can be estimating based on probes or based on relative location of platform hosting the T-AIMLE server or based on the interface / API statistics on delay or failures based on high delay.

- Vendor compatibility: this can be the case when the selection of a target AIMLE is based on which vendor provides the server. It can be either the same vendor only, or a list of selected vendors based on the service agreements among MNOs/SEAL providers.

- Cost for task migration: The computational effort and/or processing required, or time required, or energy required for offloading a task to the T-AIMLE server. Such cost may be based on the distance among servers (e.g. edge to regional cloud) or based on the task to be offloaded.

- A rate/ranking/weighting factor of target AIMLE server based on fulfilment of previous tasks and statistics. Such rate can be stored and updated at ML repository (or shared among repositories) after fulfilling each task based on consumer feedback or based on internal evaluation at the AIMLE (based on success or failure of tasks under similar conditions)

NOTE 3: Pre-defined policies from the S-AIMLE server or VAL server may be provided in previous steps to dictate the AIMLE server selection criteria (policies on minimum rating, allowed vendors, preferred AIMLE servers)

6.	The AIMLE client sends to the AIMLE server the selected T-AIMLE server and the expected time and area of roaming.

NOTE 4: The selection of the T-AIMLE server by the S-AIMLE server instead of AIMLE client is to be explored in normative phase.

7.	The AIMLE server requests or negotiates with target AIMLE server to undertake the task of serving the AIMLE client of interest.

8.	The AIMLE server requests AIMLE client to change serving AIMLE server and passes the target AIMLE server info, also possible limitations in target V-PLMN (lower priority, capacity limitations for ML operation). The AIMLE client confirms and connects to target AIMLE server via the source AIMLE server.

-----------UE of interest roams to target PLMN and AIMLE client connects to T-AIMLE server-------

9.	The T-AIMLE server sends a notification to S-AIMLE server about the roaming of the UE(s) of interest. Following the notification may also reach the VAL server (as expected consumer of the ML training/inference output).

10.	The T-AIMLE server if the task is about relaying the ML inference output over the VPLMN, it provides the output via the S-AIMLE server to the VAL server. If the T-AIMLE server is the new AIMLE server, it sends directly the output to VAL server.

### 7.16.2	Architecture Impacts

This solution requires that AIMLE functional model supports UE roaming scenarios, and that ML repository has informing on V-PLMN. Such architecture option is introduced in clause 6.2.2.

### 7.16.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #16.

-	Fetch multi-MNO AIMLE server API (request / response model; API provider: ML repository; known consumer: AIMLE server; corresponding to step 3 of clause 7.16.1).

-	Select V-PLMN AIMLE server API (request / response model; API provider: AIMLE client; known consumer: AIMLE server; corresponding to steps 4 and 6 of clause 7.16.1).

Request AIMLE server change API (request / response model; API provider: AIMLE server; known consumer: AIMLE server; corresponding to step 7 of clause 7.16.1).

-	Notify AIMLE server change API (subscribe / notify model; API provider: AIMLE server; known consumer: AIMLE server or AIMLE client; corresponding to step 8, 9 of clause 7.16.1).

### 7.16.4	Solution evaluation

This solution addresses Key Issue #3 and introduces the capability to support the continuity of the ML training / inference capability in scenarios where the ML model training / inference happens at the UE (e.g. AIMLE client) while the UE client is expected to move to an area served by another PLMN.

This solution supports the selection of the T-AIMLE server by the AIMLE client, since this is a VAL UE-centric solution where the VAL UE is configured to select the best AIMLE server for continuing the AIML capability when roaming; however variants of this solution where the S-AIMLE server is selected instead of AIMLE client can be also possible and can be explored in normative phase.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

## 7.17	Solution #17: Enhancement of AIMLE to support model inference

### 7.17.1	Solution description

This proposal addresses Key Issue #1, Open Issue 5. The proposed solution enables UE-edge collaborative inference, where a VAL UE (with an AIMLE client) and a nearby edge AIMLE server jointly execute an ML model inference. In this approach, the inference pipeline is partitioned: the VAL UE performs the privacy-sensitive and ultra-low-latency portion of the model locally, while the computation-intensive remainder is offloaded to an edge AIMLE server. By splitting the inference execution between the device and edge, the solution meets strict latency requirements and keeps sensitive raw data on the device, while leveraging edge computing for heavy processing.

Although the proposed UE-edge collaborative inference procedure shares the general concept of distributed inference with the pipeline solution defined in Rel-19, there are fundamental differences in terms of architecture, coordination entities, and applicable scenarios. The current proposal specifically targets collaboration between the AIMLE client (residing in the UE) and the edge AIMLE server, which hosts part of the ML model and provides inference service functions. This solution introduces a new set of interactions, including model selection, authorization, intermediate data upload, and result feedback, none of which are covered by the Rel-19 pipeline solution.

This clause describes the procedure for supporting collaborative inference between AIMLE client and edge AIMLE server.

Figure 7.17.1-1 illustrates the procedure for AIMLE server to support collaborative inference between AIMLE client and edge AIMLE server.

Pre-condition:

1	The ML model needed to be used for this ML model inference is available by the AIMLE server.

Figure 7.17.1-1: ML model collaborative inference

1.	The AIMLE client initiates the inference service by sending a request to the edge AIMLE server. This request indicates that the UE intends to perform an ML model inference collaboratively (split between UE and edge AIMLE). The AIMLE client identifies the target ML model or inference task and that an edge AIMLE is needed to complete the inference. This request consists of the ML model identifier , and potentially the inference partitioning info. It may carry an inference session ID or a request for the edge to allocate resources. The information elements presented in the request are described in Table 7.17.1.1.1-1.

2.	The edge AIMLE server performs an authorization check. It also ensures the specified model (or sub-model for the later layers) is available. If the client only provided a task type (no specific model ID), the AIMLE server selects an appropriate ML model for the task using the AIMLE model discovery/selection procedure (per 3GPP TS 23.482 clause 8.11.3).

3.	If the request is authorized and a model is ready, the edge AIMLE server replies with a ML Model collaborative inference response, the Inference Setup Response acknowledging readiness, otherwise a failure response indicates the reason for failure. The information elements presented in the message are described in Table 7.17.1.1.2-1.

4.	Upon confirmation, the AIMLE client proceeds to execute the first part of the ML model locally. The AIMLE client runs the input data (provided by the VAL application) through the initial model layers (up to the agreed split point).

5.	Intermediate data upload: The AIMLE client sends the intermediate inference output to the edge AIMLE server. This intermediate data, such as a feature vector output from the last locally executed model layer (e.g., FeatureX), serves as input to the next stage of inference. The message includes an identifier to associate it with the earlier setup (e.g. the session ID or pipeline ID from step 2) so that the edge knows which model/task this data corresponds to. The information elements presented in the message are described in Table 7.17.1.1.3-1.

6.	The Edge AIMLE server uses the received intermediate data as input to the remaining portion of the ML model. It runs the model's subsequent layers or inference function to produce the final result.

7.	The edge AIMLE server sends a collaborative inference result response message back to the AIMLE client. This response contains the final inference result (for example, the predicted class label and confidence score, or other output data of the model). It also can include the ML model ID that was used, a status code indicates success or failure, cause value, etc. The information elements presented in the request are described in Table 7.17.1.1.4-1.

#### 7.17.1.1	Information Flows

##### 7.17.1.1.1 	ML model collaborative inference request

Table 7.17.1.1.1-1 describes the information elements included in the ML model collaborative inference request message sent by the AIMLE/ADAE client to the edge AIMLE/ADAE server.

Table 7.17.1.1.1-1: ML model collaborative inference request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identify | M | The identifier of the requestor (e.g. identifier of the AIMLE client or the VAL UE). |
| ML model ID | O (NOTE 1) | The ML model identifier |
| Inference task | O (NOTE 1) | Indicates the inference task |
| >Inference task type | O (NOTE 2) | Description of the inference task type to be performed (e.g., prediction, classification) |
| >Inference task ID | O (NOTE 2) | The ID of the inference task |
| Inference partitioning info | M | The partitioning information of the ML model specifies how many layers are to be executed on the UE and how many layers are to be executed on the server |
| Inference session ID | O (NOTE 3) | The identifier of the inference session requested by the UE |
| Allocate resources request | O (NOTE 3) | It indicates the request for resource allocation. |
| NOTE 1: Only one of the IEs shall be provided. NOTE 2: Only one of the IEs shall be provided. NOTE 3: Only one of the IEs shall be provided. |  |  |

##### 7.17.1.1.2	ML model collaborative inference response

Table 7.17.1.1.2-1 describes the information elements included in the ML model collaborative inference response message sent by the AIMLE server to the edge AIMLE client.

Table 7.17.1.1.2-1: ML model collaborative inference response

| Information element | Status | Description |
| --- | --- | --- |
| Inference Setup Response acknowledging readiness | O (NOTE) | Contains all the necessary information for the ML split inference setup |
| Failure response | O (NOTE) | The error message in case of failure |
| NOTE: Only one of the IE shall be provided. |  |  |

##### 7.17.1.1.3 	Intermediate inference output

Table 7.17.1.1.3-1 describes the information elements included in the intermediate inference output message sent by the AIMLE client to the edge AIMLE server.

Table 7.17.1.1.3-1: Intermediate inference output

| Information element | Status | Description |
| --- | --- | --- |
| Intermediate output | M | Intermediate data (e.g., feature vector output from the last locally executed model layer) |
| ML model ID | O (NOTE 1) | The ML model identifier |
| Inference task | O (NOTE 1) | Indicates the inference task |
| >Inference task type | O (NOTE 2) | Description of the inference task type to be performed (e.g., prediction, classification) |
| >Inference task ID | O (NOTE 2) | The ID of the inference task |
| Inference partitioning info | O | The partitioning information of the ML model specifies how many layers are to be executed on the UE and how many layers are to be executed on the server. |
| Inference session ID | O | The identifier of the inference session requested by the UE |
| NOTE 1: One of the IE shall be provided. NOTE 2: One of the IE shall be provided. |  |  |

##### 7.17.1.1.4 	ML Model collaborative inference result response

Table 7.17.1.1.4-1 describes the information elements included in the ML model collaborative inference result response sent by the AIMLE server to the edge AIMLE client.

Table 7.17.1.1.4-1: ML model collaborative Inference result response

| Information element | Status | Description |
| --- | --- | --- |
| Inference output | M | The final inference output (e.g., the predicted class label and confidence score, or other output data of the model) |
| Model ID | O | The identifier of the Model and contains the information necessary for the server to identify the correct layer of the Model to continue the inference execution |
| Status code | O | Indicates success or failure, cause value, etc. |

### 7.17.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.17.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.17.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.18	Solution #18: Solution on support for federated AI services

### 7.18.1	Solution description

This solution addresses Key Issue #3.

In the context of AI/ML services, a federated AIML service can have different flavour based on the scenario.

- Scenario 1: an FL training operation in different Edge servers (EAS) or cloud servers (CAS): EAS#1 of EDN1 is an FL client, EAS#2 of DN#2 is another FL client and CAS of DN#3 is an FL server/aggregator.

- Scenario 2: an ML model training or inference operation split in different EDNs/DNs. In such scenario, the AIML operation (e.g. inference) is split among UE and network entities. If such network entities belong to different operators, then a federation agreement is in place and the AIML operation is split among different stakeholders /operators.

- Scenario 3: If an ML operation like model training is provided per ADAE analytics event, then the analytics service comprises the ML model training, model inference and the analytics operation which is provided by an analytics engine. Such ML-enabled analytics service may be distributed among different network or server entities provided by different network operators. One example can be the MNO #x is performing ADAES analytics on VAL performance in an analytics engine in the telco platform, and the ML model training is requested and provided by an external entity (e.g. AWS) using data provided by 3rd party. In certain use cases, the ML model training may be performed by a CAS in a less granular fashion, and the analytics is locally performed by one or more ECSPs at the respective edge platforms which may have overlapping or orthogonal coverage.

For the above scenarios, a federated AIML service can be defined as an AI/ML service which requires the joint operation of subsequent AI operations provided by different providers (MNO, ECSP, CSP, Device owner, App developer).

This solution addresses the key issue on how to configure and enforce federation agreements for AI/ML operations requiring joint or split processing by platforms operated by different providers. In particular, the solution provides a mechanism to translate a request for an AI/ML service to a requirement for a federated AI service, given the resource conditions and the federation agreements in place.

The procedure for the configuration of the federated AIML service based on the VAL request for an AI/ML task (ML model training or inference request) is shown below.

Pre-condition:

- AIMLE consumer such as AIMLE client or VAL server has ML model or address to ML model.

Figure 7.18.2-1: Procedure on support for federated AI services

- An AIMLE server of CSP #1 receives an ML model training request from a VAL server as described in clause 8.3.2 of TS 23.482 or an ML model inference request (as described in Solution #1).

- The AIMLE server determines to trigger a federated AIML service, which can be an AIMLE service such as federated training or inference spanning across different CSPs. The determination can be based on the capability or availability limitations of CSP #1 for providing the AI/ML service; hence support from another CSP is needed.

- The AIMLE server performs an ML model information discovery, to discover ML model support for federated AI scenarios, where the discovery request (as specified in 3GPP TS 23.482 clause 8.11.4.3) is enhanced to include federation information, i.e. list of information for different federation agreements related to the ML model, in particular the role of other edge/cloud providers in accessing or operating the ML model (or part of it). This includes information on partner CSPs, permissions, capabilities and availability of the CSP offering part of the AIML service.

- In this step, the AIMLE server obtains the identity and address of the target CSP which is planned to undertake part of the AI/ML service based on step 3.

The AIMLE server determines to select the target AIMLE server of CSP #2 to form the Federated AIML service and generates and Federated Service ID. The AIMLE server of CSP #1 provides a configuration of the federated AIML service including the service area of interest, time validity, operation expected per lead and partner OP, the dataset requirements by the partner OP.

Then, AIMLE server sends a Federated AIML service configuration request to the partner CSP (AIMLE server in target CSP) to provide the configuration parameters.

- The target AIMLE server after authorizing the request, it checks the federation agreement (with its ML repository) and confirms or negotiates the configuration parameters. Then, the target AIMLE server provides a positive or negative response to the source AIMLE server (or CSP #1).

In case of negotiation (which can be due to the load/resource/energy situation in the target edge platform's OP), the target AIMLE server may provide an alternative set of configuration parameters (e.g. limited coverage or time availability or time to accomplish the ML model training, or data availability) and further confirmation from source AIMLE server may be provided.

- The AIMLE server sends an ML model training/inference response including also the trained/inferred model and optionally a notification that the training/inference was outcome of a federation support (this may be needed to require more fees for the charging of the consumer).

- The AIMLE server performs an ML model information storage with the updated mapping of the ML model or AI service with the federation information.

### 7.18.2	Architecture Impacts

This solution requires that AIMLE functional model supports federated AIML services. Such architecture option will be introduced in clause 6.2.3.

### 7.18.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #18.

-	Federated AIML Service Config API (request / response model; API provider: AIMLE server; known consumer: AIMLE server; corresponding to step 4 and 5 of clause 7.18.1).

### 7.18.4	Solution evaluation

This solution addresses Key Issue #3 and introduces the capability to support translating a request for an AI/ML service to a requirement for a federated AI service, given the resource conditions and the federation agreements in place.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

## 7.19	Solution #19: Cross-PLMN/Domain AIMLE client discovery, selection, monitoring

### 7.19.1	Solution Description

#### 7.19.1.1	General

In 3GPP TS 23.482 [2], there is currently a capability to discover and select AIMLE clients. Key Issue #3 has identified that cross-domain and cross-PLMN scenarios are not covered in TS 23.482 [2] Release 19. This solution enhances AIMLE client discovery and selection to include cross-domain and cross-PLMN deployments. Figure 7.19.1.1-1 shows the deployment scenario where the AIMLE servers (AIMLE Server-1 and AIMLE Server-2) are in different domain or PLMN and the AIMLE servers in these different domains or PLMN have federation agreement to achieve discovery, selection and monitoring.

Figure 7.19.1.1-1: Use case for cross-domain/cross-PLMN federation

The following enhancements are proposed to enable cross-domain and cross-PLMN discovery, selection, and monitoring:

- Cross-domain and cross-PLMN deployments have AIMLE servers in different domains. When a discovery request fails to find sufficient AIMLE clients, the AIMLE server can discover AIMLE clients from another AIMLE server in another domain and select the desired clients.

- The AIMLE server, as described in clause 8.13 of 3GPP TS 23.482 [2], can monitor AIMLE clients in its own domain/PLMN. To monitor AIMLE clients in another domain/PLMN, the AIMLE server can subscribe to the AIMLE server in that other domain/PLMN to receive status updates about those AIMLE clients.

- Based on the status of AIMLE clients in the current domain/PLMN and notifications received from the other domain/PLMN, the AIMLE server can replace AIMLE clients in the other domain/PLMN with AIMLE clients in the current domain/PLMN when they become available, in order to minimize cost.

- During AIMLE client discovery and selection, the AIMLE server can include in its request to the other AIMLE server (in another domain/PLMN) specific AIMLE clients that have migrated, to enhance service continuity.

#### 7.19.1.2	Procedure

Figure 7.19.1.2-1: Cross-domain/cross-PLMN AIMLE client discovery, selection and monitoring

- A VAL server sends an AIMLE client selection subscription request to the AIMLE server as described in step 1 of clause 8.13.2.2 of 3GPP TS 23.482 [2]. The VAL server includes in its subscription an indication that federation with other AIMLE servers in another domain can be used.

- The AIMLE server sends the AIMLE client selection subscription response to the VAL server as described in step 3 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

- The AIMLE server monitors AIMLE clients determine whether they fulfil the selection criteria as per step 4 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

- The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients that do not fulfil the selection criteria as per step 5 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

- The AIMLE server determines that there are insufficient AIMLE clients in the current domain either because there were not enough AIMLE clients fulfilling the criteria or because previously qualifying AIMLE clients cease to fulfil the criteria or they are currently unavailable.

- The AIMLE server, when cross-domain/cross-PLMN federation is enabled, identifies an AIMLE server in another domain/PLMN (AIMLE server-2) to federate for AIMLE client selection. The AIMLE servers in different domains, based on federation agreement, can either be preconfigured in the AIMLE server or stored in a central ML repository where different domains/PLMNs makes information about AIMLE servers in their domain make available as shown in Figure 6.2.1-2 of clause 6.2.

- The AIMLE server determines the discovery requirements based on the request in step 1 and missing AIMLE clients in step 5, send an AIMLE clients discovery request to the AIMLE server in the other domain (AIMLE server-2), and receives the AIMLE clients that fulfil the provided AIMLE client discovery criteria from the AIMLE server in the other domain. The AIMLE client discovery request can be sent as Table 8.8.3.1-1 of clause 8.8.3.1 of 3GPP TS 23.482 [2] where the requester is now AIMLE server and the number of required AIMLE clients is in line with the missing AIMLE clients and the requirements in federation indication provided in step 1.

- The AIMLE server establishes an AIMLE client selection subscription with the AIMLE server in the other domain (AIMLE server-2). The AIMLE server sends AIMLE client selection subscription request, including in the selection criteria, the missing gap between the request in step 1 and the unavailable AIMLE clients from step 5, and it receives the subscription response from the AIMLE server in the other domain (AIMLE server-2). The AIMLE server following the change in monitoring as in step 11 updates the AIMLE client subscription update.

The AIMLE client selection subscription request is sent as Table 8.13.3.2-1 of clause 8.13.3.2 of 3GPP TS 23.482 [2] as updated in Table 7.19.3.1-1 of clause 7.19.3.1, where the requester is now AIMLE server and the number of required AIMLE clients is in line with the missing AIMLE clients and the requirements in federation indication provided in step 1.

The AIMLE client subscription update request is sent as Table 8.13.3.5-1 of clause 8.13.3.5 of 3GPP TS 23.482 [2] as updated in Table 7.19.3.2-1 of clause 7.19.3.2, where the requester is now AIMLE server and the number of required AIMLE clients is in line with what has changed in latest monitoring results in step 11 and the requirements in federation indication provided in step 1.

- The AIMLE server in the other domain (AIMLE server - 2) monitors AIMLE clients fulfilling the criteria from step 8. The AIMLE server in the other domain (AIMLE server - 2) interacts with the NEF and/or SEAL services (including SEALDD) within its own domain to establish monitoring as per step 4 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

The AIMLE server in the other domain (AIMLE server - 2) may determine the application QoS parameters (e.g. bandwidth, latency, jitter) for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API) in accordance with step 5 of clause 8.13.2.2 of 3GPP TS 23.482 [2]. When the AIMLE clients no longer meet the criteria, the QoS adjustment is reversed as described in step 5 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

- The AIMLE server receives the notification from the AIMLE server in the other domain (AIMLE server - 2) regarding selected and reselected AIMLE clients.

- Monitoring of AIMLE clients continues in the AIMLE server in the current domain and the AIMLE server in the federating domain (AIMLE server - 2). During monitoring, the AIMLE clients from the other domain can be replaced by the AIMLE clients from the current domain when suitable AIMLE clients become available locally by sending AIMLE client subscription update request as in step 8.

- The AIMLE server notifies the consolidated AIMLE client selection status to the VAL server after selecting the required AIMLE clients, including those from the other domain. The AIMLE server, in its notification, includes the AIMLE clients selected from both the current domain and other domain.

NOTE: When the VAL server unsubscribes from the AIMLE server by sending an AIMLE client selection unsubscribe request as specified in clause 8.13.3.7 of 3GPP TS 23.482 [2], the AIMLE server also cancels its own subscription by sending an AIMLE client selection unsubscribe request to the AIMLE server in the other domain (AIMLE server - 2) to which it had subscribed. The AIMLE server may also cancel its subscription when a sufficient number of AIMLE clients become available in the current domain.

### 7.19.2	Architecture Impacts

In order to enable AIMLE Servers in different domains to interact and discover each other, the architectural change, as shown in Figure 6.2.1-2 of clause 6.2, is required. AIMLE client discovery and selection are needed between AIMLE servers in different domains to discover and select AIMLE clients in another domain.

### 7.19.3	Corresponding APIs

#### 7.19.3.1	AIMLE client selection subscription request

The AIMLE client selection subscription request is same as Table 8.13.3.2-1 of Clause 8.13.3.2 of 3GPP TS 23.482 [2], with the following additions in bold.

Table 7.19.3.1-1: 3GPP TS 23.482 [2] Table 8.13.3.2-1 AILME client selection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. VAL Server, AIMLE Server. |
| AIMLE client selection criteria | M | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | M | Represents the endpoint at the requester (e.g. VAL server or AIMLE Server) for receiving the notifications on the selected AIMLE Client’s status update. |
| Federation indication | O | Indicates if federation with other AIMLE servers in another PLMN/domain may be used when the requester is not AIMLE Server. |

#### 7.19.3.2	AIMLE client subscription update request

The AIMLE client selection subscription request is same as Table 8.13.3.5-1 of Clause 8.13.3.5 of 3GPP TS 23.482 [2], with the following additions in bold.

Table 7.19.3.2-1: 3GPP TS 23.482 [2] Table 8.13.3.5-1 AIMLE client selection subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the existing subscription for which the update request applies. |
| AIMLE client selection criteria | O | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | O | Represents the endpoint at the requester (e.g. VAL server or AIMLE Server) for receiving the notifications on the selected AIMLE Client’s status update. |

### 7.19.4	Solution Evaluation

This solution addresses the cross-domain and cross-PLMN scenario, which Key Issue #3 identifies as a missing aspect in Rel-19. Specifically, it enables an AIMLE server to fulfil the need for additional AIMLE clients by subscribing to another AIMLE server in a different domain or PLMN. The solution extends the AIMLE client selection subscription and monitoring procedures to support federation with an AIMLE servers in another domain, allowing dynamic discovery and selection of AIMLE clients across domains.

The proposed solution is technically feasible and aligns with the existing architecture, extending established subscription, monitoring, and notification mechanisms in clause 8.13.2.2 of 3GPP TS 23.482 [2]. The solution enables the AIMLE servers in different domain/PLMN to monitor the AIMLE clients in their own domain and select AIMLE clients in a coordinated manner.

Service continuity is maintained by allowing dynamic replacement of AIMLE clients selected from the other domain with AIMLE clients in the current domain as they become available.

## 7.20	Solution #20: ML Model Performance Evaluation with information collection from ML model consumer

### 7.20.1	Solution Description

This clause describes the procedure and information flows for supporting ML model performance evaluation with information collection from ML model consumer.

#### 7.20.1.1	Procedure

Figure 7.20.1.1-1: ML model performance evaluation with information collection from ML model consumer

Figure 7.20.1.1-1 illustrates the procedure on ML model performance with information collection from ML model consumer. The corresponding procedure in detail is as follows:

1. The VAL server sends to the AIMLE server the ML model evaluation subscription request message containing the ML model ID, the VAL Service ID, the ML model evaluation ID, and the purpose on the ML model as specified in Table 7.20.1.2.1-1. The ML evaluation profile ID is used to obtain the ML model evaluation profile which is used to determine the ML model performance in step 10.

2. The AIMLE server sends the ML model performance evaluation subscription response message to the VAL server, containing a success message if the VAL server is authorized to subscribe. Otherwise, an error message is sent to the VAL server. The requested message includes the information specified in Table 7.20.1.2.2-1.

3. The AIMLE sends to the A-DCCF the ML model performance information collection request to get the ML performance of the model. The message contains the information as specified in Table 7.20.1.2.3-1.

4. The A-DCCF determine the list of ML model consumers based on ML model consumer list, the ML model ID, and the info on the purpose on the use the ML model, and the VAL service ID.

5.	The A-DCCF sends to the ML model consumer the information collection request message to obtain the ML model performance of the respective ML model consumer. The information collection request message contains the ML model info (e.g., ML model ID and the purpose on use the ML model) and it is specified in Table 7.20.1.2.4-1.

6.	The ML model consumer generates the ML model performance information based on usage of the ML model.

7. Upon generating the ML model performance, the ML model consumer sends to the A-DCCF the information collection response message containing information as specified in Table 7.20.1.2.5-1.

8.	Upon receiving the Information collection response, the A-DCCF aggregates the information received from each ML model Consumer.

9.	The A-DCCF contacts the AIMLE server with the ML model performance information collected from the ML model consumer. The message contains the ML model info (e.g., ML model ID), the purpose on the usage of ML model, ML model consumer info, the ML model performance metrics and it is specified in Table 7.20.1.2.6-1.

10.	Based on the ML model performance received in the previous step, the AIMLE server computes the ML model performance metrics based on the information provided by the ML consumers and the ML model evaluation profile information.

11.	The AIMLE server sends to the VAL server the ML model performance evaluation subscription notification containing the information about the performance of the requested ML model. The detail information contains in the notification is as specified in Table 7.20.1.2.7-1.

Alternatively, instead of steps 3-9, the AIMLE server collects ML model performance information from the ML model consumer directly.

The A-DCCF procedures and information flows in 3GPP TS 23.436 [3] can be enhanced (highlighted in bold italics) as follows.

### 8.12 Procedure for supporting data collection to A-DCCF ### 8.12.1 General This clause describes two procedures (covering both subscribe-notify and request-response models in 8.12.2.1 and 8.12.2.2 respectively) for supporting data collection at A-DCCF. ### 8.12.2 Procedure #### 8.12.2.1 Subscribe-notify model Figure 8.12.2.1-1 illustrates the procedure for consumer to request A-DCCF to initiate subscription for data or analytics collection. The procedure may be utilized by the consumer to update the subscription for data or analytics collection. Pre-conditions: 1. Consumer (ADAE server, AIMLE server) collects data or analytics via A-DCCF from data producer. Figure 8.12.2.1-1: Support for data collection over A-DCCF 1. The consumer (e.g. ADAE server, AIMLE server) sends a data collection subscription request to A-DCCF for collecting data/analytics. As specified in Table 8.12.3.2-1, the request message includes identifier of the consumer (ADAE server ID, AIMLE server ID), Data Collection Event ID, and Data Collection Requirements. The request message may include the identifier of Data Producer, Analytics ID, target data producer profile criteria, process requirements, Area of Interest, Time validity, storage requirements, and notification endpoints. The consumer decides to go via A-DCCF based on internal configuration. In cases where A-DCCF is used to obtain the ML model performance evaluation, the request should include the ML model ID, the ML consumer list, and the VAL service ID. 2. If the data producer is not identified by the consumer, the A-DCCF determines the data procedure that can provide data/analytics. If the consumer requested storage of data/analytics in an A-ADRF but the A-ADRF ID is not provided by the consumer, or the collected data/analytics is to be stored in an A-ADRF according to configuration on the A-DCCF, the A-DCCF selects an A-ADRF to store the collected data/analytics. 3. The A-DCCF determines whether the data/analytics requested in step 1 are already being collected. If the requested data/analytics are already being collected by a consumer, the A-DCCF adds the new consumer to the list of consumers that are subscribed for these data/analytics. 4. If the data/analytics subscribed in step 1 is not being collected by the A-DCCF, the A-DCCF subscribes to the data producer for data/analytics. If the data/analytics subscribed in step 1 partially matches the data/analytics that is already being collected by the A-DCCF, a modification of this subscription to the data producer would satisfy both the existing data/analytics subscriptions as well as the newly requested data/analytics, the A-DCCF requests an update of the previous subscription to the data producer. The A-DCCF adds the consumer to the list of consumers that are subscribed for these data/analytics. 5. Upon received the data/analytics subscription request from the A-DCCF, the data producer determines whether the required data/analytics can be provided and sends data/analytics subscription response to the A-DCCF. 6. The A-DCCF sends data/analytics subscription response to the consumer. 7. When the required data/analytics are available, the data producer notifies the data/analytics to the A-DCCF. 8. The A-DCCF notifies the data/analytics to all notification endpoints indicated in step 1. Data/analytics sent to notification endpoints may be processed by the DCCF upon to the request in step 1. The DCCF may store the data/analytics in the A-ADRF if requested by the consumer or if required by A-DCCF configuration. #### 8.12.2.2 Request-response model Figure 8.12.2.2-1 illustrates the procedure for the consumer to request A-DCCF for data or analytics collection. Figure 8.12.2.2-1: Support for Get data/analytics over A-DCCF 1. The consumer (e.g. ADAE server, AIMLE server) sends a Get data/analytics request to A-DCCF for data/analytics collection. As specified in Table 8.12.3.5-1, the request message includes identifier of the consumer (ADAE server ID, AIMLE server ID), and Data/Analytics Requirements. The request message may include the identifier of Data Producer, Analytics ID, target data producer profile criteria, process requirements, storage requirements, and notification endpoints. The consumer decides to go via A-DCCF based on internal configuration. In cases where A-DCCF is used to obtain the ML model performance evaluation, the request should include the ML model ID, the ML consumer list, and the VAL service ID. Additionally. 2. The A-DCCF determines whether the data/analytics requested in step 1 are already being collected. If the requested data/analytics are already being collected by a consumer, the A-DCCF adds the new consumer to the list of consumers that are subscribed for these data/analytics. 3. If the data/analytics requested in step 1 is not being collected by the A-DCCF, and the data producer is not identified by the consumer, the A-DCCF determines the data procedure that can provide data/analytics. If the consumer requested storage of data/analytics in an A-ADRF but the A-ADRF ID is not provided by the consumer, or the collected data/analytics is to be stored in an A-ADRF according to configuration on the A-DCCF, the A-DCCF selects an A-ADRF to store the collected data/analytics. 4. The A-DCCF sends Get data/analytics request to the data producer for data/analytics. 5. Upon received the data/analytics request from the A-DCCF, the data producer determines whether the required data/analytics can be provided and sends data/analytics response to the A-DCCF. The required data/analytics may be contained in the response if they are available at the data producer. 6. The A-DCCF sends data/analytics response to the consumer. The required data/analytics may be contained in the response if get from the data producer. ### 8.12.3 Information flows #### 8.12.3.1 General The following information flows are specified for data collection based on clause 8.12.2. #### 8.12.3.2 Data collection subscription request Table 8.12.3.2-1 describes information elements for the data collection subscription request from the consumer (ADAE server, AIMLE server) to the A-DCCF. Table 8.12.3.2-1: Data collection subscription request Information element; Status; Description. Consumer ID; M; The identifier of the consumer (ADAE server ID, AIMLE server ID).. Data Collection Event ID; M; The identifier of the data collection event.. Data Collection requirements; M; The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data.. Analytics ID; O; The identifier of the analytics event, for which the data collection is needed.. ML model ID; O (NOTE); The ML model identifier. ML consumer List; O (NOTE); The list of consumers that uses the ML model identified by the ML model ID. VAL service ID; O (NOTE); Indicates the VAL service ID. List of Data Producer IDs; O; The list of Data Producer IDs.. Target data producer profile criteria; O; Characteristics of the data producers to be used.. Area of Interest; O; The geographical or service area for which the requirement request applies. Time validity; O; The time validity of the request. Process requirements; O; Requirements on processing the collected data/analytics.. Storage requirements; O; Requirements on storage of the collected data/analytics.. > A-ADRF ID; O; The identifier of A-ADRF for store the collected data/analytics.. Notification endpoints ID or address; O; The identifier or address of the notification endpoints.. NOTE: The IEs shall be provided for ML model performance collection.. NOTE: Whether the Purpose on the usage of ML model IE be contained in the information flows will be discussed in the normative phase. #### 8.12.3.3 Data collection subscription response Table 8.12.3.3-1 describes information elements for the data collection subscription response from the A-DCCF to the consumer (ADAE server, AIMLE server). Table 8.12.3.3-1: Data collection subscription response Information element; Status; Description. Result; M; The result of the data collection subscription request (positive or negative acknowledgement).. #### 8.12.3.4 Data collection notification Table 8.12.3.4-1 describes information elements for the data collection notification from the A-DCCF to the consumer (ADAE server, AIMLE server). Table 8.12.3.4-1: Data collection notification Information element; Status; Description. Data Collection Event ID; M; The identifier of the data collection event.. Analytics ID; O; The identifier of the analytics event.. Analytics Type; O; The type of reported analytics, which can be statistics, prediction, etc.. ML model ID; O (NOTE); The ML model identifier. ML consumer list; O (NOTE); The list of consumers that uses the ML model identified by the ML model ID. Data Type; O; The type of reported data samples, which can be UE data, network data, application data, edge data, or different granularities/abstraction of data (e.g. real time, non-real time).. Data Output; M; The reported data/analytics, which can be inform of measurements or offline/historical data/analytics or ML model performance indication based on subscription.. Process information; O; Information of the processing of the collected data/analytics.. Storage information; O; Information of the storage of the collected data/analytics.. > A-ADRF ID; O; The identifier of A-ADRF for store the collected data/analytics.. NOTE: The IEs shall be provided for ML model performance collection.. #### 8.12.3.5 Get Data/Analytics request Table 8.12.3.5-1 describes information elements for the Get data/analytics request from the consumer (ADAE server, AIMLE server) to the A-DCCF Table 8.12.3.5-1: Get Data/Analytics request Information element; Status; Description. Consumer ID; M; The identifier of the consumer (ADAE server ID, AIMLE server).. Data/Analytics requirements; M; The requirements for data/analytics collection, including the format of data, level of abstraction of data, level of accuracy of data.. Analytics ID; O; The identifier of the analytics event for the required analytics.. ML model ID; O (NOTE); The ML model identifier. ML consumer List; O (NOTE); The list of consumers that uses the ML model identified by the ML model ID. VAL service ID; O (NOTE); Indicates the VAL service ID. List of Data Producer IDs; O; The list of Data Producer IDs.. Target data producer profile criteria; O; Characteristics of the data producers to be used.. Area of Interest; O; The geographical or service area for which the requirement request applies. Time validity; O; The time validity of the request. Process requirements; O; Requirements on processing the collected data/analytics.. Storage requirements; O; Requirements on storage of the collected data/analytics.. > A-ADRF ID; O; The identifier of A-ADRF for store the collected data/analytics.. Notification endpoints ID or address; O; The identifier or address of the notification endpoints.. NOTE: The IEs shall be provided for ML model performance collection.. #### 8.12.3.6 Data collection response Table 8.12.3.6-1 describes information elements for the data collection response from the A-DCCF to the consumer (ADAE server, AIMLE server). Table 8.12.3.6-1: Data collection response Information element; Status; Description. Result; M; The result of the analytics data request (positive or negative acknowledgement).. Analytics ID; O; The identifier of the analytics event.. Analytics Type; O; The type of reported analytics, which can be statistics, prediction, etc.. ML model ID; O (NOTE); The ML model identifier. ML consumer list; O (NOTE); The list of consumers that uses the ML model identified by the ML model ID. Data Type; O; The type of reported data samples, which can be UE data, network data, application data, edge data, or different granularities/abstraction of data (e.g. real time, non-real time).. Data Output; M; The reported data/analytics, which can be inform of measurements or offline/historical data/analytics or ML model performance indication based on subscription.. Process information; O; Information of the processing of the collected data/analytics.. Storage information; O; Information of the storage of the collected data/analytics.. > A-ADRF ID; O; The identifier of A-ADRF for store the collected data/analytics.. NOTE: The IEs shall be provided for ML model performance collection.. #### 8.12.3.7 Data producer profile The data producer profile IE includes information about the data generation/production capability of the data producer to support data collection for data analytics service and the availability/accessibility of the generated/produced data, as defined in Table 8.12.3.7-1. Table 8.12.3.7-1: Data producer profile Information element; Status; Description. Data Producer ID; M; ID of the data producer.. Data producer type (NOTE); M; Specifies the type of the data producer, e.g., ADAEC, A-DCCF, A-ADRF, VAL server, SEAL server, SEAL client, EES, EAS.. Data type (NOTE); M; Type of information that can be provided by the data producer, e.g., performance indicators, reproducer usage data, server load data, application performance, edge load.. Data producer role (NOTE); O; Role of the data producer, e.g., generating entity, original producer, repository.. Original producer ID (NOTE); O; If the data producer role is not “original producer” or “generating entity”, specifies the Producer ID of the original data producer for the data provided by this data producer. If the data producer type is A-DCCF, this is a list of Data Producer IDs.. Data freshness (NOTE); O; If the data producer role is not “original producer” or “generating entity”, length of time elapsed after the data is generated until is available at the data producer. Alternatively, the data collection rate supported by the producer is provided.. Data producer capability (NOTE); O; Indicates data producer capabilities for this data type, e.g. how long the data can be stored, support for anonymization, data generation rate and schedule.. NOTE: When the Data producer profile IE is used for Target data producer profile criteria (e.g. Table 8.12.3.2-1 and Table 8.12.3.5-1), this IE may be a list of values..

#### 7.20.1.2	Information Flows

##### 7.20.1.2.1 	ML model evaluation subscription request

Table 7.20.1.2.1-1 describes information elements for the ML model evaluation subscription request from the VAL server to the AIMLE server.

Table 7.20.1.2.1-1: ML model evaluation subscription request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| VAL service ID | M | The identifier of the VAL service |
| ML model evaluation profile ID | M | The identifier of the ML model evaluation profile. |

##### 7.20.1.2.2 	ML model evaluation subscription response

Table 7.20.1.2.2-1 describes information elements for the ML model evaluation subscription response from the AIMLE server to the VAL server.

Table 7.20.1.2.2-1: ML model evaluation subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | The identifier of the subscription. |
| Model ID | M | The identifier of the Model |
| VAL service ID | M | The identifier of the VAL service |
| Request successful | O (NOTE) | ML model evaluation request successful message |
| Error message | O (NOTE) | ML model evaluation request error message |
| NOTE: At least one of these IE shall be present. |  |  |

##### 7.20.1.2.3	ML model performance information collection request

Table 7.20.1.2.3-1 describes information elements for the ML model performance information request from the AIMLE server to the A-DCCF.

Table 7.20.1.2.3-1: ML model performance information collection request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| ML consumer List | M | The list of consumers that uses the ML model identified by the ML model ID |
| VAL service ID | M | Indicates the VAL service ID |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |

##### 7.20.1.2.4	Information collection request

Table 7.20.1.2.4-1 describes information elements for the information collection request from the A-DCCF to the ML model consumer.

Table 7.20.1.2.4-1: Information collection request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | O (NOTE) | The ML model identifier |
| Analytics ID | O (NOTE) | The identifier of the analytics service. |
| Data source IDs | M | The Identifier of the data sources |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |
| Interest time period of the historical data | O | Interest time period of the historical data. |
| NOTE: At least one of the IE shall be provided. |  |  |

##### 7.20.1.2.5	Information collection response

Table 7.20.1.2.5-1 describes information elements for the Information collection response from the ML model Consumer to the A-DCCF.

Table 7.20.1.2.5-1: Information Collection response

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | O (NOTE) | The ML model identifier |
| Analytics ID | O (NOTE) | The identifier of the analytics service. |
| ML model performance information | M | It contains the aggregated information regarding the performance of the ML model identified by the model ID on the data sources (e.g., accuracy, F1 score, AUC). |
| Data source IDs | O | The Identifier of the data sources used to collect the ML model performance |
| Interest time period of the historical data | O | Interest time period of the historical data. |
| NOTE: At least one of the IE shall be provided. |  |  |

##### 7.20.1.2.6	ML model performance information collection response

Table 7.20.1.2.6-1 describes information elements for the ML model performance information response from the A-DCCF to the AIMLE server.

Table 7.20.1.2.6-1: ML model performance information collection response

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| ML consumer list | M | The list of consumers that uses the ML model identified by the ML model ID |
| ML model performance indication | M | It contains the information regarding the performance of the ML model identified by the model ID. (e.g., accuracy, F1 score, ROC, AUC) |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |

##### 7.20.1.2.7	ML model evaluation notification

Table 7.20.1.2.7-1 describes information elements for the ML model evaluation notification from the AIMLE server to the VAL server.

Table 7.20.1.2.7-1: ML model evaluation notification

| Information element | Status | Description |
| --- | --- | --- |
| Model ID | M | The identifier of the model |
| VAL service ID | M | The identifier of the VAL service |
| ML model performance information | M | It contains the aggregated information regarding the performance of the ML model identified by the Model ID on the data sources. (e.g., accuracy, F1 score, AUC) |

### 7.20.2	Architecture Impacts

The architecture impact is the required enhancement of the A-DCCF functional model to support ML model performance collection and support AIMLE server as consumer of the enhanced A-DCCF data collection service.

### 7.20.3	Corresponding APIs

This subclause provides a summary of the corresponding APIs for solution #20.

-	ML model performance evaluation subscription API (subscribe / notify model; API provider: AIMLE Server; known consumer: VAL Server; corresponding to step 1, 2, and 11 of clause 7.20.1).

-	ML model performance information collection API (request / response model; API provider: A-DCCF; known consumer: AIMLE Server; corresponding to step 3 and 9 of clause 7.20.1).

### 7.20.4	Solution evaluation

The proposed solution address Key Issue #2: support enhancement for evaluation and determination of ML model performance and correctness within AIML Enablement Layer and, specifically, the third open issue about whether and how to monitor (e.g., trigger, detect, and report) ML model performance and correctness degradation for different ML-enabled operations (e.g., inferencing, training, split AIML operation, HFL, VFL, transfer learning) and considering data management.

In response to a VAL Server request, AIMLE collects ML model performance data from the respective ML model consumers using A-DCCF as an intermediary between the AIMLE server and the ML model consumers. Alternatively, the AIMLE server can directly contact the ML model consumers to perform the same task.

## 7.21	Solution #21: Split operation node registration involving multiple AIMLE clients

### 7.21.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

AIMLE client may provide its computing capability to act as a processing node, especially multiple AIMLE client collaborating scenario. AIMLE client can provide their idle computing resource to perform computing tasks of other UEs. Network may provide benefits for such sharing. For example, network can give a discount to the UE when UE request for network computing tasks. In this solution, both AIMLE client and VAL server can registrater in AIMLE server to participate in split AI/ML operations as a processing node.

The Split operation node registration procedure as specified in clause 8.14 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

Editor's Note:	 The UE registering as a processing node for split operation is subject to the user's consent, which is FFS.

#### 8.14.2.4 Split operation node registration ##### 8.14.2.4.1 General Clause 8.14.2.4.2 illustrates the split operation node registration procedure. Clause 8.14.2.4.3 illustrates the split operation node registration update procedure. Clause 8.14.2.4.4 illustrates the split operation node de-registration procedure. ##### 8.14.2.4.2 Split operation node registration Figure 8.14.2.4.2-1 illustrates the procedure for VAL server/AIMLE client to register with the AIMLE server to indicate split operation capabilities. Pre-conditions: 1. The VAL server/AIMLE client has received information (e.g. URI, IP address) related to the AIMLE Server; 2. The VAL server/AIMLE client has received security credentials authorizing it to communicate with the AIMLE server; Figure 8.14.2.4.2-1: Split operation node registration 1. The VAL server/AIMLE client sends a split operation node register request to the AIMLE server to indicate the split operation capabilities of the VAL server/AIMLE client. The request includes information defined in Table 8.14.3.6-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request, the AIMLE server stores the registration information. 3. The AIMLE server sends a split operation node register response to the requestor. If the AIMLE server has registered the requestor, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation node registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure. ##### 8.14.2.4.3 Split operation node registration update Figure 8.14.2.5-1 illustrates the procedure for VAL server/AIMLE client to update a split operation node registration with the AIMLE server. Pre-conditions: 1. The VAL server/AIMLE client has registered with the AIMLE Server. Figure 8.14.2.4.3-1: Split operation node registration update 1. The VAL server/AIMLE client sends a split operation node registration update request to the AIMLE server to update an existing split operation registration. The request includes information defined in Table 8.14.3.8-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the registration information. 3. The AIMLE server sends a split operation node registration update response to the requestor. If the AIMLE server has successfully updated the registration, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure. ##### 8.14.2.4.4 Split operation node de-registration Figure 8.14.2.4.4-1 illustrates the procedure for VAL server/AIMLE client to de-register for split operation with the AIMLE server. Pre-conditions: 1. The VAL server/AIMLE client has registered with the AIMLE Server. Figure 8.14.2.4.4-1: Split operation node de-registration 1. The VAL server/AIMLE client sends a split operation node de-register request to the AIMLE server. The request includes information defined in Table 8.14.3.10-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server terminates the registration for split operation. 3. The AIMLE server sends a split operation node de-register response to the requestor. If the AIMLE server has successfully terminated the registration for split operation node, the response includes an indication of success. If the registration for split operation was not terminated, the response includes an indication of failure and may include a reason for failure.

The Information flow of split operation node registration as specified in clause 8.14.3 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

#### 8.14.3.6 Split operation node register request Table 8.14.3.6-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation node register request. Table 8.14.3.6-1: Split operation node register request Information element; Status; Description. Requestor identifier; M; The identity of the requestor (e.g., VAL server ID, AIMLE client ID).. Security credentials; M; The security credentials of the requestor.. Node information; M; Information about the VAL server/AIMLE client node (e.g., identifier, endpoints, etc.).. Split operation capabilities; M; Split operation capabilities of the VAL server/AIMLE client.. > Model information; M; Information about ML model capabilities of the VAL server/AIMLE client for split operation (e.g., identifiers, versions, etc.).. > Usage information; O; Information about usage capabilities of the VAL server/AIMLE client for split operation (e.g., inputs frequency/size, output frequency/size, etc.).. Expiration time; O; The proposed expiration time of the registration.. #### 8.14.3.7 Split operation node register response Table 8.14.3.7-1 shows the response sent by an AIMLE server after processing of split operation node register request. Table 8.14.3.7-1: Split operation node register response Information element; Status; Description. Successful response; O; Indicates that the request was successful.. > Registration identifier; M; The identifier of the registration.. > Expiration time; O; The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires.. Failure response; O; Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. #### 8.14.3.8 Split operation node registration update request Table 8.14.3.8-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation node registration update request. Table 8.14.3.8-1: Split operation node registration update request Information element; Status; Description. Registration identifier; M; The identifier of the registration.. Security credentials; M; The security credentials of the requestor.. Node information; O; Information about the VAL server/AIMLE client node (e.g., identifier, endpoints, etc.).. Split operation capabilities; O; Split operation capabilities of the VAL server.. > Model information; O; Information about ML model capabilities of the VAL server/AIMLE client for split operation (e.g., identifiers, versions, etc.).. > Usage information; O; Information about usage capabilities of the VAL server/AIMLE client for split operation (e.g., inputs frequency/size, output frequency/size, etc.).. Expiration time; O; The proposed expiration time of the registration.. #### 8.14.3.9 Split operation node registration update response Table 8.14.3.9-1 shows the response sent by an AIMLE server after processing of split operation node registration update request. Table 8.14.3.9-1: Split operation node registration update response Information element; Status; Description. Successful response; O (NOTE); Indicates that the request was successful.. > Expiration time; O; The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires.. Failure response; O (NOTE); Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: Either the Successful response or the Failure response is sent, but not both.. #### 8.14.3.10 Split operation node de-register request Table 8.14.3.10-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation de-register request. Table 8.14.3.10-1: Split operation node de-register request Information element; Status; Description. Registration identifier; M; The identifier of the registration.. #### 8.14.3.11 Split operation node de-register response Table 8.14.3.11-1 shows the response sent by an AIMLE server after processing of split operation de-register request. Table 8.14.3.11-1: Split operation de-register response Information element; Status; Description. Successful response; O (NOTE); Indicates that the request was successful.. Failure response; O (NOTE); Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: Either the Successful response or the Failure respone is sent, but not both..

### 7.21.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.21.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.21.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.22	Solution #22: Sample Alignment Enablement for VAL Servers in Vertical Federated Learning

### 7.22.1	Solution Description

#### 7.22.1.1	General

This solution addresses key issue #7 which identifies VFL sample alignment among open issues. In a VFL process, a VAL server may act as a VFL server (active party), coordinating training and inference by aggregating results from other VAL servers acting as VFL clients (passive parties). In this context, the sample space consists of user equipment (UEs), each represented by local identifiers specific to the VAL servers.

The VFL process begins by first aligning the samples among VFL participants so that inference and training are performed properly. This is performed without sharing the UE identifiers with other VFL participants. Each UE in the sample space may be identified with different identifiers in each of the VFL participants such as different AF specific UE IDs or different public IP addresses of the UE. Therefore, the process of VFL sample alignment should be able to align the UEs that are involved in a VFL process without sharing the UE identifiers while resolving the potential differences in how the VFL participants identify the UEs.

This solution proposes to enable the AIMLE server to support sample alignment among VFL participants while preserving the privacy of the UEs and while resolving the potential differences in how each VFL participant identifies the UEs.

The following procedures are described to support VFL inference and training among VAL servers with sample alignment:

- 	VFL client registration

- 	VFL training and inference

#### 7.22.1.2	Procedures

##### 7.22.1.2.1	VFL client registration

FL member registration is specified in clause 8.4 of 3GPP TS 23.482 [2]. The FL members can be VAL servers that perform the registration via VAL servers. To support VFL registration the procedure needs to be further updated. For this purpose, the following changes are needed in clause 8.4.2 of 3GPP TS 23.482 [2] in the corresponding steps.

- For FL registration request, when VAL servers acting as VFL clients register as candidate FL members, they provide additional metadata including supported feature sets, and local UE identifiers in their sample space. The FL member endpoint information is also provided in the registration for communication with the FL member such as initiating VFL training/inference as in step 4 of clause 7.22.1.2.2.

- The AIMLE server before storing the VFL client registration, first maps each local UE identifier in the sample space to a corresponding unified UE identifier (e.g., GPSI such as MSISDN) via interaction with the Core Network functions and stores it along with its registration. This unified UE identifier, however, shall not be accessible to the VAL servers and is only used by the AIMLE server for discovery purposes.

- There is no change needed in the FL member registration response.

##### 7.22.1.2.2	VFL training and inference

Assumptions:

- VAL server acting as VFL server coordinates other VAL servers acting as a VFL client to perform VFL training/inference.

- The sample space is associated with UEs which are locally identified in the VFL client and VFL server by AF-Specific UE IDs, IP addresses, or similar identifiers.

Pre-conditions:

- The VAL servers acting as VFL clients and the VAL server acting as the VFL server have registered with the AIMLE server as described in clause 7.22.1.2.1, providing metadata including supported feature sets, and local UE identifiers.

- The AIMLE server maps each local UE identifier to a corresponding unified UE identifier (e.g., GPSI such as MSISDN) via interaction with the Core Network functions as described in clause 7.22.1.2.1.

Figure 7.22.1.2.2-1: VFL training/inference with sample alignment procedure

- The VFL server sends a VFL training/inference request including the required feature ID list, and the required sample ID list (that is local UE identifier(s)).

- The AIMLE server discovers and selects the desired VFL clients from registered VFL clients supporting the required feature ID and sample ID list. The UE identifier(s) in the sample ID list of the VFL clients and VFL server are matched by using the Core Network functions to resolve local identifiers (e.g. AF specific UE ID, IP addresses) into common UE identifiers (e.g, GPSI). The VFL clients are identified using the common UE identifiers for matching the sample space.

- The AIMLE server assigns a sample correlation ID for each UE identifier in the sample ID list for consistent referencing during training and inference while enabling VFL training/inference using their local identifiers of the UEs.

- The AIMLE server sends a VFL training/inference initiation request to the selected VAL servers acting as VFL clients, including the assigned sample correlation IDs to match the correlation IDs in the VFL training/inference task to their corresponding local identifiers of the UEs.

- The VAL servers acting as VFL clients send the initiation response to the AIMLE server confirming participation to the VFL training/inference.

- The AIMLE server sends a VFL training/inference response to the requesting VFL server, including the sample correlation IDs corresponding to the aligned UEs.

- The VFL server and the VFL clients proceed with the training/inference using the aligned sample ID list, with sample correlation IDs ensuring consistent sample alignment across participants.

### 7.22.2	Architecture Impacts

There is no architectural impact from this solution.

### 7.22.3 	Corresponding APIs

#### 7.22.3.1	VFL member registration request

The FL member registration request in Clause 8.4.4.2 of 3GPP TS 23.482 [2] can be expanded to enable registration of VFL members as shown in Table 7.22.3.1-1 with the additions indicated in bold.

Table 7.22.3.1-1: 3GPP TS 23.482 [2] Table 8.4.4.2-1: FL member registration request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration request. |
| FL member ID | M | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| FL member endpoint | O | Endpoint information (e.g. URI) used to communicate with the FL member. |
| FL member supported AI/ML role | M | The supported AI/ML role of FL member which can be used as FL server or FL client. |
| Supported FL task types | O | The supported FL task type by the FL member such as VFL and HFL. |
| Feature ID list | O | List of features supported by the FL member. |
| Sample ID list | O | The list of local UE identifiers that are in the sample space of the FL member. |
| FL member capabilities | O (NOTE) | FL member capability information (e.g. ML application type, allowed resource usage level). |
| FL member velocity | O | Indicates the FL member velocity (e.g., mobile or static). |
| FL member Location information | O | Indicates the location information of the UE (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses). |
| FL member Availability schedule | O (NOTE) | Availability schedule of the FL member for a certain FL member. |
| Supported ML model ID list | O | The list of ML model IDs for which the FL member can be used. |
| Area of Interest | O | The area of interest (i.e. location co-ordinates) for which the registration applies. |
| Time validity | O | The time validity for the registration of the FL member. |
| NOTE: At least one of these shall be present. |  |  |

#### 7.22.3.2	VFL member registration update request

The FL member registration update request in Clause 8.4.4.4 of 3GPP TS 23.482 [2] can be expanded to enable registration of VFL members as shown in Table 7.22.3.2-1 with the additions indicated in bold.

Table 7.22.3.2-1: 3GPP TS 23.482 [2] Table 8.4.4.4-1: FL member registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the existing registration for which the update request applies. |
| Security information | M | Information for ML repository to validate the registration update request |
| FL member ID | M | The identifier of the candidate FL member for which the update applies. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| Updated registration parameters | M | The updates of the registration parameters. |
| > FL member endpoint | O | The updated endpoint information (e.g. URI) used to communicate with the FL member. |
| > FL member supported AI/ML role | O (see NOTE) | The updated supported AI/ML role of FL member which can be FL server, FL client. |
| > Feature ID list | O | Updated list of features supported by the FL member. |
| > Sample ID list | O | The updated list of local UE identifiers that are in the sample space of the FL member. |
| > FL member Availability schedule | O (see NOTE) | Updated Availability schedule of the FL member. |
| > supported ML model ID list | O (see NOTE) | The updated list of ML model IDs for which the FL member can be used |
| > area of interest | O (see NOTE) | The updated area of interest for the registration update. |
| > time validity | O (see NOTE) | The update time validity for the FL member registration update. |
| NOTE: At least one of these information elements shall be present. |  |  |

#### 7.22.3.3	VFL training/inference request

Table 7.22.3.3-1 describes information elements for the VFL training/inference request sent from the VFL server (coordinating VAL server) to the AIMLE server.

Table 7.22.3.3-1: VFL training/inference initiation request

| Information element | Status | Description |
| --- | --- | --- |
| VFL server identifier | M | The identifier of the coordinating VFL server. |
| FL task type | M | The requested FL task which can be either inference or training. |
| Required feature ID lists | M | The required feature ID(s) for each data domain for the FL task. At least two feature lists are provided, one for each data domain. |
| Required sample ID lists | M | The required list of local UE identifiers that are common in the sample space for the FL task. |

#### 7.22.3.4	VFL training/inference initiation request

Table 7.22.3.4-1 describes information elements for the VFL training/inference initiation request sent from the AIMLE server to the selected VFL client(s).

Table 7.22.3.4-1: VFL training/inference initiation request

| Information element | Status | Description |
| --- | --- | --- |
| VFL server identifier | M | The identifier of the coordinating VFL server. |
| FL task type | M | The requested FL task which can be either inference or training. |
| Selected feature ID | M | The selected feature ID for the FL task for the VFL client. |
| Selected sample list | M | List of the selected samples for the VFL task. Each element includes the information described below. |
| > sample ID | M | The local UE identifier in the VFL client for the selected sample. |
| > correlation ID | M | The corresponding assigned correlation ID for the sample ID. |

#### 7.22.3.5	VFL training/inference initiation response

Table 7.22.3.5-1 describes information elements for the VFL training/inference initiation response sent from the selected VFL client(s) to the AIMLE server.

Table 7.22.3.5-1: VFL training/inference initiation response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 7.22.3.6	VFL training/inference response

Table 7.22.3.6-1 describes information elements for the VFL training/inference response sent from the AIMLE server to the VFL server.

Table 7.22.3.6-1: VFL training/inference initiation response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| > Selected VFL client(s) | M | List of selected VFL client(s). Each element includes the information described below. |
| >> Selected VFL client identifier | M | List of selected VFL client(s). Each element includes the information described below. |
| >> Selected Feature ID | M | The selected feature ID for the VFL client. |
| > Selected sample list | M | List of the selected samples for the VFL task. Each element includes the information described below. |
| >> sample ID | M | The local UE identifier in the VFL server for the selected sample. |
| >> correlation ID | M | The corresponding assigned correlation ID for the sample ID. |
| Failure response (NOTE) | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE: One of these IEs shall be present in the message. |  |  |

### 7.22.4 	Solution Evaluation

The solution addresses Key Issue #7, which identifies sample alignment among VAL servers in VFL as an open issue. It defines procedures and APIs to support sample alignment, enabling VFL servers to initiate training or inference tasks with consistent sample identification. VFL clients register with the AIMLE server, providing supported feature ID(s) and sample ID(s). The UE in the sample space may be referred using UE identifiers used by the specific VAL servers (i.e. local UE Identifier) which can be AF specific UE ID, ip-address or other related identifiers. The AIMLE server uses unified UE identifiers obtained via Core Network interaction to select VFL clients matching the required sample list. Correlation IDs are assigned to samples to enable consistent referencing across the VFL server and participating VFL clients. The APIs extend existing registration and update procedures and define new interfaces for initiating and coordinating VFL tasks.

### NOTE: The alignment of VFL client registration and update procedures as in proposed in clause 7.22.3.1 and 7.22.3.2 with the latest changes to 3GPP TS 23.482 [2] is to be done in normative.

## 7.23	Solution #23 enhancement of AIMLE to support AI Inference service performance management and feedback

### 7.23.1	Solution description

This proposal addresses Key Issue #8. This solution specifies a client-to-AIMLE feedback mechanism for inference service performance. After an inference request is processed per the base procedure in clause 7.1, the client (VAL server, AIMLE client) evaluates the received inference result against the expected service quality parameters (e.g., expected latency, accuracy, confidence), and may report a feedback message to AIMLE if late, failed, degraded, or inaccurate outcomes are detected. The AIMLE server validates and aggregates feedback together with runtime observations and can use the aggregated information to adjust subsequent discovery and selection decisions (e.g., update performance profiles or candidate ranking used in discovery responses). This solution applies regardless of where inference executes (edge or central). It creates a feedback loop after invocation by adding it to the performance requirements stated during the discovery phase (see Key Issue #5).

-	Inference services (AI/ML model inference services) have already been discovered and invoked through the AIMLE server according to procedures in clause 7.1. The client (VAL server, AIMLE client) has received inference results from AIMLE.

Figure 7.23.1-1 illustrates the inference feedback reporting procedure.

Figure 7.23.1-1: Inference service performance feedback

1.	The AIMLE server sends a inference service performance feedback subscription request to the VAL Server or AIMLE client to subscribe for receiving inference service feedback. The request specifies the feedback scope (target inference services), expected reporting types (e.g., latency, accuracy, confidence metrics or error conditions), and optional reporting frequency or threshold criteria.

2.	Upon receiving the subscription request, the client authenticates the AIMLE server and validates the subscription parameters. If authorization fails, the client rejects the subscription request with a corresponding cause value and the procedure terminates.

3.	If the request is authorized, the client sends a feedback subscription response message to the AIMLE server indicating successful establishment of the feedback subscription and any negotiated parameters (e.g., reporting scope or frequency).

4.	When the client subsequently invokes an inference service and receives an inference result as defined in Clause 7.1, the client locally evaluates the result against the expected service quality parameters (e.g. latency, accuracy, confidence level) that were expressed during discovery or inference service request.

5.	If the client detects that the received inference result is late, failed, inaccurate, or otherwise deviates from the expected QoS, it sends an inference service feedback notify message to the AIMLE server in accordance with the previously established subscription.

6.	Upon receiving the feedback notify, AIMLE validates the message and returns an inference service feedback notify acknowledgement message to the client, confirming successful receipt or rejecting the notify with a cause.

7.	Local AIMLE server action to updating service performance profiles and discovery policies: the AIMLE server aggregates received feedback reports and updates the service performance profile associated with the affected inference services. The updated profile is used to refine future service discovery and selection decisions (e.g., candidate ranking, node prioritization, load balancing).

NOTE: A service performance profile is an AIMLE-maintained record of aggregated QoS statistics for each inference service (e.g., average latency, success rate, accuracy level, availability). It is used for policy optimization and candidate selection.

### 7.23.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.23.3	Corresponding APIs

#### 7.23.3.1	ML model inference service performance feedback subscription request

Table 7.23.3.1-1 describes the information flow of inference service performance feedback subscription request from the AIMLE server to the AIMLE client.

Table 7.23.3.1-1: ML model inference service performance feedback subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the AIMLE server initiating the feedback subscription request. |
| Target inference service(s) | M | Identifies the inference service or set of inference services (e.g. by service ID or ML model ID) for which performance feedback is being requested. |
| Feedback metric types | M | Indicates the performance metrics and event types on which feedback is requested (e.g. inference latency, accuracy, confidence level, error conditions). At least one metric type shall be included. |
| Reporting criteria | O | Specifies optional criteria for feedback reporting, such as the frequency of reports or threshold values that trigger a notification (e.g. report only if latency exceeds a certain value). If absent, the default is that the client provides feedback for each inference execution that does not meet the expected service quality parameters. |

#### 7.23.3.2	ML model inference service feedback subscription response

Table 7.23.3.2-1 describes the information flow of the inference service feedback subscription response from the AIMLE service consumer (client) to the AIMLE server.

Table 7.23.3.2-1: inference service performance feedback subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Success response (NOTE) | O | Indicates that the inference service feedback subscription request was accepted by the client. This IE is present if and only if the subscription request is accepted. |
| > Feedback subscription identifier | M | Identifier of the established feedback subscription (echoed from the request). The client uses this identifier in subsequent feedback notifications to refer to this subscription. |
| > Accepted reporting criteria | O | If the client adjusts the requested reporting frequency or thresholds, this IE specifies the finally agreed criteria for feedback notifications (e.g. a modified minimum interval between reports). If omitted, the reporting criteria remain as initially requested. |
| Failure response (NOTE) | O | Indicates that the feedback subscription request was not accepted. This IE is present if and only if the subscription is rejected by the client. |
| NOTE: Only one of these information elements shall be present |  |  |

#### 7.23.3.3	ML model inference service feedback notification

Table 7.23.3.3-1 describes the information flow of the inference service feedback notification from the AIMLE service consumer (client) to the AIMLE server.

Table 7.23.3.3-1: ML model inference feedback notification

| Information element | Status | Description |
| --- | --- | --- |
| Feedback subscription identifier | M | Identifier of the feedback subscription context to which this notification belongs (as provided in the subscription request/response). |
| Inference service identifier | M | Identifies the specific inference service or ML model instance related to this feedback report if the subscription covers multiple inference services |
| Performance feedback type | M | Indicates the type of performance event being reported. For example, this IE can denote that the inference result was late, had insufficient accuracy/confidence, or otherwise failed to meet an expected performance criterion. |
| Observed performance metrics (NOTE) | O | If the feedback is related to a performance degradation (with an inference result delivered), this IE provides the relevant performance measurements observed by the client. For example, it may include the actual inference latency or accuracy/confidence level associated with the result. |
| Failure cause (NOTE) | O | If the inference execution failed or the result was unusable, this IE provides the reason for the failure (e.g. inference timeout, model error, or other execution failure details). |
| NOTE: Only one of these information elements shall be present |  |  |

#### 7.23.3.4	ML model inference service feedback notify acknowledgement

Table 7.23.3.4-1 describes the information flow of the inference service feedback notification acknowledgement from the AIMLE server to the client.

Table 7.23.3.4-1: ML model inference response

| Information element | Status | Description |
| --- | --- | --- |
| Success acknowledgement (NOTE) | O | Indicates that the AIMLE server has successfully received and processed the inference service feedback notification. This IE is present if and only if the notification is accepted. |
| > Feedback subscription identifier | M | Identifier of the feedback subscription for which the notification was received (echoed from the subscription context). |
| Failure acknowledgement (NOTE) | O | Indicates that the AIMLE server failed to process the feedback notification. This IE is present if and only if the notification was rejected or not applied. |
| > Cause | M | Reason for the failure to process the notification (e.g. unknown subscription ID, invalid message content, or other error). |
| NOTE: Only one of these information elements shall be present |  |  |

### 7.23.4	Solution evaluation

This solution addresses Key Issue #8 by introducing a structured feedback mechanism from inference clients (VAL servers, AIMLE clients) to the AIMLE server to report on inference service performance. It enables the AIMLE server to collect post-inference quality observations (e.g., delays, failures, degraded accuracy) and use this information to optimize future service selection decisions.

From a functional perspective, this solution complements the base inference invocation procedures by enabling a closed-loop feedback channel. Clients can evaluate inference results against quality expectations and notify the AIMLE server of deviations. The AIMLE server aggregates the feedback, updates performance profiles for inference services, and uses this information to refine discovery responses and candidate rankings.

From an architectural perspective, this solution does not introduce new network functions or interfaces. It uses the existing AIMLE architecture and augments it with feedback subscription, notification, and response mechanisms.

The solution enhances the adaptability of AIMLE-based inference services in dynamic environments (e.g., load fluctuations, model drift, hardware variance) and supports closed-loop performance optimization across centralized and edge deployments.

## 7.24	Solution #24: ML model inference in a hierarchical AIMLE deployment

### 7.24.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

The solution proposes ML model collaborative inference procedures considering AIMLE deployments in edge data networks as described in Annex A.4 of 3GPP TS 23.482 [2], where a central AIMLE server can distribute inference tasks to multiple edge AIMLE servers for parallel execution.

Such collaborative inference addresses compute-intensive scenarios such as real-time video analytics, large-scale multimodal processing, and long-sequence model inference.

For example, in a video-surveillance use case, a central AIMLE server may receive an inference request for analysing HD streams; the data itself resides at distributed edge sites. The central AIMLE server decomposes the request, assigns sub-tasks to the appropriate edge AIMLE servers where frequently used models are pre-deployed and locally available for inference, and later aggregates results to deliver a unified inference output.

This mechanism enables efficient use of distributed computing resources by offloading heavy or data-local computation to edge AIMLE servers. The general process is illustrated in Figure 7.24.1-1.

Pre-condition:

- Each edge AIMLE server has been registered with the central AIMLE server according to Solution #9, allowing the central server to maintain a registry of edge capabilities and locations.

- The ML model required for inference has been deployed or is accessible on both central and relevant edge AIMLE servers (e.g., through pre-synchronized ML model repositories).

- The central AIMLE server is able to determine how to divide inference tasks among edge nodes based on model type, data locality, and edge capability.

Figure 7.24.1-1: ML model inference in a hierarchical AIMLE deployment

- The VAL server sends an ML model inference request to the central AIMLE server. The request includes the model identifier or inference service type, input data or data reference (e.g., a URI of video streams), and optional QoS requirements (e.g., maximum latency, minimum accuracy or confidence level), as defined in Solution #1 (Clause 7.1)

- Upon receiving the inference request, the central AIMLE server performs authorization and validation of the VAL server and the requested service. If the central AIMLE determines that the inference task requires distributed or low-latency processing (for example, high concurrency, or data localized at the edge), it decides to execute the inference in a collaborative edge-inference mode. The central AIMLE selects one or more appropriate edge AIMLE servers based on registered capabilities, proximity to the data source, service area, and current load, according to local policy or performance criteria.

NOTE 1:	The central AIMLE determines whether distributed inference is needed based on metadata provided in the inference request (e.g., the use of Inference data index for large or remote data at the edge), as well as on configuration and registered information of edge AIMLE servers. This determination is deployment-specific and not defined in this specification.

- The central AIMLE server sends an Edge ML model inference request to each selected edge AIMLE server.
The message includes the assigned task identifier, model identifier, input data reference or partition, and any specific QoS constraints applicable to that edge node. Each request corresponds to a sub-task derived from the original VAL inference request.

NOTE 2:	The input data reference or partition information used in the edge inference request is derived from metadata in the VAL inference request and from the registered information of edge AIMLE servers, such as accessible data sources or EDN service areas. The actual data access or transfer is out of scope of this specification.

- Each edge AIMLE server authenticates the request, performs inference using the specified model, and returns an Edge ML model inference response to the central AIMLE server. The response includes either the inference result or a failure cause (for example, model unavailable or resource limitation).

- The central AIMLE server collects inference responses from all participating edge AIMLE servers. It aggregates or post-processes the partial results into a unified inference outcome, according to the type of model and task (e.g., combining multiple video frame analyses or merging sequence predictions). If any edge response indicates failure, the central AIMLE may perform fallback processing or return partial results, following local policy.

- After completing aggregation, the central AIMLE server returns an inference service response to the VAL server, using the same message format defined in Solution #1 (Clause 7.1). The response includes the final inference results, execution status, and optional performance metrics.

### 7.24.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.24.3	Corresponding APIs

#### 7.24.3.1	Edge ML model inference request

Table 7.24.3.1-1 describes the Edge ML model inference request sent from the central AIMLE server to an edge AIMLE server. This request extends the base ML model inference request defined in Clause 7.1.3.1 with additional information needed for distributed inference, such as a task identifier and a partitioned input reference.

Table 7.24.3.1-1: Edge ML model inference request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the central AIMLE server sending the inference request to the edge AIMLE server. |
| Task identifier | M | Unique identifier for the distributed inference sub-task, assigned by the central AIMLE server (e.g. a UUID). This identifier allows correlation of the edge inference request and response with the overall inference task. |
| Inference data (NOTE) | O | Partition of the input data for inference, included directly if the data segment is small enough to send in the request. This IE contains the portion of the original inference input that the edge AIMLE server should process. |
| Inference data index (NOTE 1) | O | Reference or identifier for the input data partition that the edge AIMLE server should process. For example, this could be a URI or data handle pointing to the relevant data segment stored at the edge. |
| ML model identifier | M | Identifies the ML model that the edge AIMLE server is requested to use for this sub-task. This identifier is provided by the central AIMLE server (e.g. a model ID or reference) and corresponds to the model specified or selected for the original inference request. |
| QoS constraints | O | Specifies any quality-of-service requirements for this sub-task as determined by the central AIMLE server. For example, this may include a deadline by which the edge must return results or priority indicators for the inference processing at the edge. |
| NOTE: Only one of these information elements shall be present. At least one of these information elements must be provided to supply the edge with the input needed for inference. |  |  |

#### 7.24.3.2	Edge ML model inference response

Table 7.24.3.2-1 describes the Edge ML model inference response from the edge AIMLE server to the central AIMLE server. This response aligns with the base ML model inference response defined in Clause 7.1.3.2, with the addition of a task identifier for correlation with the request.

Table 7.24.3.2-1: Edge ML model inference response

| Information element | Status | Description |
| --- | --- | --- |
| Success response (NOTE) | O | Indicates that the edge inference sub-task was completed successfully. This IE is present if and only if the edge was able to process the assigned inference task. |
| > Task identifier | M | Identifier of the inference sub-task for which this response is provided (echoed from the request). It correlates the response to the corresponding edge inference request. |
| > Inference result | M | The result of the ML model inference performed by the edge AIMLE server on the provided data partition. The format and content of this result correspond to the model’s output for that sub-task. |
| Failure response (NOTE) | O | Indicates that the edge AIMLE server could not complete the inference sub-task. This IE is present if and only if the inference at the edge failed. |
| > Cause | M | Reason for the inference failure at the edge (e.g. the required model was not available on the edge, insufficient resources, or an execution error occurred). |
| NOTE: Only one of these information elements shall be present. |  |  |

### 7.24.4	Solution evaluation

This solution addresses Key Issue #6 by supporting collaborative ML model inference in hierarchical AIMLE deployment scenarios. It introduces a mechanism where a central AIMLE server can distribute inference sub-tasks to edge AIMLE servers based on edge capability, service proximity, and data locality, then enabling scalable and latency-sensitive inference services.

The solution enhances the inference architecture by extending the existing inference service request model defined in clause 7.1 to support multi-node execution. It introduces dedicated procedures and information elements for edge task delegation and response aggregation. These include task identifiers, input partition references, and edge-specific QoS constraints.

This solution enables AIMLE to support parallel inference across distributed nodes, supporting real-time analytics such as video surveillance, and LLM-based sequence analysis. It uses the AIMLE registration and discovery mechanisms defined in prior solutions (e.g., solution #9) to maintain updated knowledge of edge inference capability and availability.

This solution is not impacting existing inference flows for centralized scenarios.

## 7.25	Solution #25: Enhancement of AIMLE to support AI Inference exposure

### 7.25.1	Solution description

This proposal addresses Key Issue #5, Open Issue 1. This solution proposes a standardized inference service API that allows VAL server to invoke AI/ML inference tasks on the AIMLE/ADAE). The approach treats inference as a service capability exposed by AIMLE, decoupled from its internal model execution logic. This enables any authorized VAL server can discover and utilize AI inference services without needing internal knowledge of the AIMLE implementation. Crucially, the proposed solution supports both edge-deployed AIMLE instances (for low-latency local processing) and centralized AIMLE instances (for more intensive or global tasks), adapting the interface to the deployment scenario.

The solution focuses on how a VAL server discovers an available inference service and invokes it via standardized API.

Pre-condition:

1.	One or more AIMLE/ADAE inference services have been registered to the AIMLE/ADAE server and are available for discovery. (service registration flow is not covered here).

Figure 7.25.1-1 illustrates the inference service discovery procedure prior to invocation.

Figure 7.25.1-1: ML model inference service discovery

- The VAL server initiates the discovery procedure by sending an inference service discovery request to the AIMLE/ADAE server. The request shall include the requestor identity, the service type indicating inference, and either model information or inference requirement information. Optionally, the request may also include performance and QoS requirements, etc.

- The AIMLE/ADAE server performs an authorization check on the discovery request. If the requester is not authorized, the AIMLE server returns a discovery failure response including the cause and the procedure terminates. If the requester is authorized, the AIMLE server identifies candidate inference services that match the provided model information or inference requirements. When discovery is requirement-driven rather than model ID-driven, the AIMLE server applies model selection logic to map requirements to suitable services. Optionally, the AIMLE server may evaluate and rank the identified candidates according to relevant policy and selection criteria, such as latency or throughput targets, and current service availability.

- The AIMLE/ADAE server returns an inference service discovery response to the requester. The response contains one or more candidate inference services, each of which may be described by a service identifier, endpoint and interface profile, model identifier or class, capabilities summary, service performance profile, availability and load indicators, etc. If no suitable candidate is available or a policy conflict is detected, the AIMLE server instead returns a failure response including the cause.

### 7.25.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.25.3	Corresponding APIs

#### 7.25.3.1	ML model inference service discovery request

Table 7.25.3.1-1 describes the information flow from the VAL server to the AIMLE/ADAE server for the ML model inference service discovery.

Table 7.25.3.1-1: ML model inference service discovery

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The entity initiating the discovery request (VAL server). |
| Service type | M | Indicates that the requested discovery is for inference services（to distinguish from training, maintenance, or other service types） |
| Model information or Inference requirement information | M (NOTE) | At least one of model information or inference requirement information shall be present. For example, a specific model ID/version, or requirements such as task type, modality, precision. |
| Performance / Qos requirements | O | Target latency, throughput, concurrency/batch requirements, confidence reporting, etc. Used for candidate filtering and ranking. |
| NOTE: At least one of these IE shall be present. |  |  |

#### 7.25.3.2	ML model inference service discovery response

Table 7.25.3.2-1 describes the information flow from the AIMLE/ADAE server to the VAL server as a response for the ML model information service discovery request.

Table 7.25.3.2-1: ML model inference service discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the discovery request succeeded. One of Successful response or Failure response shall be present. |
| > Candidates (list) | M | A list of one or more candidate inference services. Each candidate contains the following elements: |
| >> Service identifier | M | Unique identifier of the candidate inference service within the AIMLE domain. |
| >> Endpoint and interface profile | M | Service endpoint (e.g., URI/FQDN) together with the supported interface profile (API version, authentication scheme). |
| >> Model identifier | O | Either the specific model identifier/version used by the service, or the model type if requirement-driven selection was applied. |
| >> Capabilities summary | O | Functional capabilities of the service, such as modalities, maximum input/output length, supported quantizations, streaming/batch support. |
| >> Service performance profile | O | Nominal performance parameters of the service, such as latency ranges, throughput, and concurrency limits. |
| >>Availability and load indicators | O | Current availability status and load indication, optionally including maintenance window information. |
| Failure response | O (NOTE) | Indicates that the discovery request failed. One of successful response or failure response shall be present. |
| > Cause | M | The cause of the failure (e.g., unauthorized requester, no suitable service, policy conflict). |
| NOTE: Only one of these information elements shall be present |  |  |

Editor’s Note: This solution is to discover AI inference member (e.g., AIMIE server), to perform AI inference operation. It is FFS whether to change “ML model inference service” to “AI inference member”.

### 7.25.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.26 	Solution #26: AIMLE server discovery

### 7.26.1 	Solution Description

#### 7.26.1.1 	General

This solution addresses key issue # 6, open issue #5 (“Whether and how the AIMLE services can support the discovery of AIMLE servers”). For instance, when the services from a AIMLE server cannot be consumed, due to not supported in a given location, ML models, etc, the AIMLE service consumer may require discovering an alternate AIMLE server. Each AIMLE client or VAL server is pre-configured with a Default AIMLE server URI. The Default AIMLE server exposes a discovery API that returns a list of AIMLE servers matching given criteria. The need for availing services from another AIMLE server may arise at AIMLE client/VAL server for various conditions like change in AIMLE client's ML task requirement, AIMLE client's mobility planning/requirements etc. When such requirement arises, the AIMLE client invokes the AIMLE server discovery service to fetch the appropriate AIMLE server meeting the requirements.

This solution proposes to enable the AIMLE server to support the discovery of other AIMLE servers based on the discovery requirements from the AIMLE service consumers (i.e., AIMLE client, VAL server).

#### 7.26.1.2 	Procedure

When the AIMLE client wants to use certain ML model or change in its ML task requirement, then the AIMLE client can request the AIMLE server to discover another AIMLE server that can fulfil AIMLE requirements like training a certain ML model.

Assumptions:

- AIMLE server is aware of the other AIMLE servers’ information, for example, AIMLE servers may have registered their capabilities with ML repository.

- There are various deployments that include multiple AIMLE servers, for example, hierarchichal, Edge deployments etc.

Pre-conditions:

- Each AIMLE client or VAL server is pre-configured with an AIMLE server URI. The AIMLE server exposes the discovery API that returns a list of AIMLE servers matching given criteria.

Figure 7.26.1.2-1: AIMLE server discovery procedure

1.	An AIMLE service consumer (VAL server, AIMLE client) sends an AIMLE server discovery request message to discover the list of suitable AIMLE servers. The request message includes AIMLE server requirements to discover suitable servers based on discovery criteria like geographical proximity etc. The AIMLE server discovery request includes information as described in Table 7.26.1.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor can discover AIMLE servers. If the requestor is authorized, the AIMLE server performs the discovery based on the discovery criteria in the request message. The server utilizes the provided criteria to filter and identify suitable AIMLE servers. If there are multiple AIMLE servers that fulfil the criteria, then the AIMLE server selects the suitable servers, ensuring that the returned list is optimized for the requestor's needs, improving performance and reliability.

3.	The AIMLE servers send the list of the suitable AIMLE servers to the requestor. The information is included in Table 7.26.1.3.2-1. The available list may have an availability duration, and the requestor can communicate with any AIMLE server based on availability duration information. This duration informs the requestor about the time frame within which the servers are guaranteed to be available, allowing for better planning and resource allocation.

The discovery request needs to be initiated again by the AIMLE service consumer if the availability duration information is expired. This ensures that the information remains current and relevant, preventing issues related to outdated server availability.

NOTE: AIMLE server discovery is to identify another AIMLE server that can match the AIMLE service consumer requirements. CAPIF discovery procedure can be used for discovering the service APIs exposed by AIMLE server.

#### 7.26.1.3	Information Flows

##### 7.26.1.3.1 	AIMLE server discovery request

Table 7.26.1.3.1-1 describes information elements for the AIMLE server discovery request from the AIMLE client/VAL server to the AIMLE server.

Table 7.26.1.3.1-1: AIMLE Server Discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server, AIMLE client). |
| AIMLE server discovery criteria | M | Discovery criteria for discovering suitable AIMLE servers for AI/ML operations. |
| > Location | O (NOTE) | Indicates the location information (e.g., cell Identity, Tracking Area Identity, GPS coordinates, VAL service area ID), serving area information of the AIMLE server to be discovered. |
| > VAL service ID | O (NOTE) | The identity of the VAL service for which the request applies. |
| > Task Type | O (NOTE) | The type of AI/ML operations that should be supported (e.g., model training, testing, inference, model split, etc). |
| > ML Models Supported | O (NOTE) | Types of ML model supported identifiers of the ML models. |
| NOTE: At least one of the information elements shall be present. |  |  |

##### 7.26.1.3.2	 AIMLE server discovery response

Table 7.26.1.3.2-1 describes information elements for the AIMLE Server Discovery response from the AIMLE server to the AIMLE client / VAL server.

Table 7.26.1.3.2-1: AIMLE Server Discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIMLE servers | M | A list of AIMLE servers information and their end-point information to reach the AIMLE servers. |
| Availability Schedule | O | The availability duration or schedule of each AIMLE server in the list when they are available. |

### 7.26.2 	Architecture Impacts

This solution proposes new procedure for AIMLE discovery. The service consumer is AIMLE service consumer (e.g., AIMLE client, VAL server) and the service producer is AIMLE server. There are no new functional elements or reference points/interfaces introduced in this solution. Hence, there are no architecture impacts.

### 7.26.3	 Corresponding APIs

A new API is needed for the discovery of AIMLE server. The API details are shown below.

### A AIMLE server discovery API #### A.1 General Table A.1-1 illustrates the API for AIMLE server discovery. This API enables an AIMLE service consumer (e.g., AIMLE client, VAL server) to discover the AIMLE server information from an AIMLE server. Table 9.2.16.1-1: Aimles_ServerDiscover API API Name; API Operations; Operation Semantics; Consumer(s). Aimles_ServerDiscover; Request; Request/Response; AIMLE client, VAL server. #### A.2 Aimles_ServerDiscover_Request operation API operation name: Aimles_ServerDiscover_Request Description: The consumer requests discovery of AIMLE server to an AIMLE server. Inputs: See clause 7.26.1.3.1. Outputs: See clause 7.26.1.3.1. See clause 7.26.1.2 for details of usage of this operation.

### 7.26.4	 Solution Evaluation

This solution addresses the open issue #5 in Key issue #6. This solution enables a AIMLE service consumer to discover an alternate AIMLE server, when there is a need for AIMLE service consumer to avail services from another AIMLE server due to various conditions such as change in AIMLE task requirement, mobility requirements etc. Such information is captured in the discovery request and the receiving AIMLE server can further fetch the AIMLE server that can meet the requirements.

# 8	Deployment scenarios

## 8.1	General

This clause provides the deployment scenarios for AIML enablement (AIMLE) services, as provided in 3GPP TS 23.482 Annex A [2]:

-	AIMLE server can be deployed at a centralized cloud platform and collects data from multiple EDNs.

-	AIMLE server can be deployed at the edge platform.

-	Hierarchical AIMLE server deployment, where multiple AIML enablement services are deployed in edge or central clouds (e.g., in hierarchical architecture). Such deployment allows for local-global analytics for system wide optimization.

In addition, the following deployment models for the enhanced AIMLE architecture are described in the following sub-clauses.

## 8.2	Deployment model #4: Centralized AIMLE server in multi-operator scenarios

Figure 8.2-1 illustrates a deployment of the AIMLE server which connects to the 3GPP network systems in multiple PLMN operator domain. The AIMLE server may be co-located with the VAL server (e.g. AI server) in a single physical entity or deployed in different physical entities.

Figure 8.2-1: Deployment of AIMLE server with connections to 3GPP network systems in multiple PLMN operator domains

Figure 8.2.2 illustrates a deployment of the AIMLE server which provides AIMLE capabilities to multiple VAL servers over AIMLE-S reference point and connects to the 3GPP network systems in multiple PLMN operator domain.

Figure 8.2.2: Deployment of AIMLE server with connections to multiple VAL servers

Editor’s Note: The centralized deployment model is FFS.

## 8.3	Deployment model #5: Distributed AIMLE servers in multi-operator scenarios

The distributed deployment is where multiple AIMLE servers are deployed either in the AIMLE service provider domain or in the PLMN operator domain. The distributed deployment of the AIMLE servers provide geographical coverage or support multiple PLMN operator domains in a geographical location. The AIMLE servers interconnect via AIMLE-E and the AIMLE-S reference point is used for interaction between VAL server and the AIMLE server.

Figure 8.3-1 illustrates the deployment of AIMLE servers in multiple PLMN operator domain and provides AIMLE capabilities to the VAL server deployed in the CSP/ASP domain. The VAL server connects via AIMLE-S to the AIMLE servers.

Figure 8.3-1: Distributed deployment of AIMLE servers in multiple PLMN operator domain without interconnection between AIMLE servers

Figure 8.3-2 illustrates the deployment of multiple AIMLE servers deployed in multiple PLMN operator domains. The VAL server connects via AIMLE-S to the AIMLE server. The interconnection between AIMLE servers is via AIMLE-E and supports the AI/ML applications for the VAL UEs connected to the AIMLE servers in multiple PLMN operator domains.

Figure 8.3-2: Distributed deployment of AIMLE servers in multiple PLMN operator domain with interconnection between AIMLE servers

Figure 8.3-3 illustrates the deployment of multiple AIMLE servers in the CSP/ASP domain where AIMLE server 1 and AIMLE server 2 connect with 3GPP network system of PLMN operator domain 1 and PLMN operator domain 2 respectively. The VAL server connects via AIMLE-S to the VAE server 1. The AIMLE servers interconnect via AIMLE-E and support the VAL applications (e.g. AI apps) for the VAL UEs connected via both the PLMN operator domains.

## Figure 8.3-3: Distributed deployment of AIMLE servers in CSP/ASP domain

# 9	Business Relationships

The business relationship is in-line with the relationships defined in 3GPP TS 23.482 Annex B.

There can be different business relationships with respect to federation and roaming. In Figure 9-1, the enhanced business relationships for federated AIMLE services are illustrated.

Figure 9-1 Relationships involved in AIMLE service – federation and roaming

The end user is the consumer of the applications provided by the ASP. The End user:

-	can have ASP service agreement with a single or multiple application service providers.

-	has a PLMN subscription arrangement with a PLMN operator (HPLMN), and the UE used by the end user can register on the HPLMN network and network of its roaming partners; or has a SNPN subscription arrangement with a SNPN operator (subscribed SNPN), and the UE used by the end user can register on the subscribed SNPN and a serving SNPN.

-	can have authorization to access edge services of a single or multiple ECSPs.

The ASP consumes the AIMLE services provided by the AIMLE service provider. The ASP:

-	can have AIMLE service provider service agreement with a single or multiple AIMLE service providers.

The PLMN operator provides connectivity between the end user and the AIMLE services provided by the AIMLE service provider. The PLMN operator:

-	can have the PLMN operator service agreement with a single or multiple AIMLE service providers.

-	can have service agreement for roaming including agreements for AIMLE services, and/or federation with a single or multiple PLMN operators.

The AIMLE service provider provides the AIMLE services. The AIMLE server:

-	can have PLMN operator service agreement with a single or multiple PLMN operators which provide AIML support service.

-	can have federation partnership to share AIMLE services with a single or multiple AIMLE service providers.

The AIMLE service provider and the PLMN operator can be part of the same organization.

# 10	Overall evaluation

## 10.1	Summary of enablement capabilities and APIs

Various solutions proposed in the present document aim introducing enhancements related to AIMLE and ADAE services, as well as ML Repository to support AI/ML services for vertical use cases. This clause provides a summary of the capabilities as well as the corresponding potential enablement APIs.

### 10.1.1	Summary of new AIMLE services

This clause provides a summary of the new AIMLE and ML repository capabilities based on the Solutions.

Table 10.1.1-1 is listing solutions proposing new capabilities related to AIMLE:

Table 10.1.1-1: Solutions proposing new or enhanced AIMLE capabilities

| Solution | Proposed Capabilities | New or Enhancement |
| --- | --- | --- |
| #1 | ML model inference | New |
| #2 | AIMLE data management assistance | Enhancement |
| #3 | ML Model Maintenance | New |
| #4 | ML Model Evaluation | New |
| #6 | ML model Perf monitoring | Enhancement |
| #7 | AIMLE client discovery in cross-PLMN scenarios | Enhancement |
| #8 | Multi-client split operation pipeline | Enhancement / New |
| #9 | AIMLE server registration in hierarchical AIMLE deployment | New |
| #10 | Support of AIMLE services for roaming UEs | Enhancement |
| #12 | ML model split learning using relays | New and Enhancement |
| #13 | Edge ML model training support | New |
| #14 | AIMLE Client Discovery | Enhancement |
| #15 | Sample alignment for VFL | Enhancement |
| #16 | AIMLE service migration support in UE roaming scenarios | New |
| #17 | ML model collaborative inference | New |
| #18 | Support for federated AI services | New |
| #19 | Cross-PLMN/Domain AIMLE client discovery, selection, monitoring | New |
| #20 | ML Model Performance Evaluation with information collection from ML model consumer | New |
| #21 | Split operation node registration involving multiple AIMLE clients | Enhancement |
| #22 | Sample Alignment Enablement for VAL Servers in Vertical Federated Learning | New |
| #23 |  |  |
| #24 |  |  |
| #25 |  |  |
| #26 | AIMLE server discovery | New |

Table 10.1.1-2 is listing the solutions proposing new ML repository capabilities:

Table 10.1.1-2: Solutions proposing new or enhanced ML repository capabilities

| Solution | Proposed Capabilities | New or Enhanced |
| --- | --- | --- |
| #11 | FL member registration in multi-operator scenarios | Enhanced |
| #12 | ML model split learning using relays | Enhancement |
| #18 | Support for federated AI services | Enhancement |

### 10.1.2 	Summary of ADAE analytics enhancements

This clause provides a summary of the enhanced ADAE capabilities (including A-DCCF) based on the Solutions.

Table 10.1.2-1 is listing such enhancements.

Table 10.1.2-1: Solutions enhancing existing ADAE capabilities

| Solution | Proposed Enhancements |
| --- | --- |
| #5 | ML-enabled analytics monitoring (ADAE enhancements) |
| #20 | ML Model Performance Evaluation with information collection from ML model consumer (A-DCCF enhancements) |

## 10.2	Key issue #1: Key issue on support for ML model inference

The open issues studied in the Key Issue #1 are as follows:

1.	Identify deployment models for supporting ML model inference in enablement layer.

2.	Whether and how ADAES needs be enhanced or leverage AIMLE for supporting ML model inference for ADAE analytics.

3.	Whether and how AIMLE needs to be enhanced to enable ML model inference to address VAL request (e.g. for VAL automation task or analytics).

4.	Whether and how to enhance AIMLE to support model inference in edge scenarios.

5.	Whether and how to enhance AIMLE architecture and functions to support collaborative inference between AIMLE client and edge AIMLE server.

There is a total of two solutions that address key issue #1: solutions #1 and #17. Each of these solutions covers different factors of the Key Issue.

Solution #1 provides AIMLE support for ML model inference for a VAL request.

Solution #17 enables UE-edge collaborative inference, for the case where a VAL UE (with an AIMLE client) and a nearby edge AIMLE server jointly execute an ML model inference for a VAL request. Both solutions can be seen as complementary since Solution #1 is generic for supporting ML inference while Solution #17 covers the collaborative use case. At the same time, collaborative ML model inference has some commonalities with the concept of AIML operation splitting using pipelines; hence the Solution #17 provides an alternative (or complementary) solution to address the specific scenarios of UE to edge collaboration for jointly performing inference services.

Open issue 2 is not directly mapped to any of the solutions, however the architecture assumptions and analysis as in clause 4 cover that ADAE supports ML model inference for ADAE analytics; hence no solution is enhancing the assumptions.

Regarding the mapping of the above solutions to open issues, the solution doesn’t provide alternative, but complementary capabilities, in particular:

- Solution #1 addresses Open Issue 3

- Solution #17 addresses Open Issue 1, 4, 5

In each solution that addresses Key Issue #1, there is no AIMLE architectural impact.

## 10.3	Key issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer

This clause will provide an overall evaluation of the key issue #2.

## 10.4	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios

The open issues studied in the Key Issue #3 are as follows:

1.	Identify the use cases for supporting multiple PLMN/NPN, and across MNOs and CSPs/ECSPs scenarios

2.	Whether and how AIMLE services can support VAL UEs that roam into visited PLMNs or NPNs, e.g. registration, discovery, and access of AIMLE in the visited domain, as well as go back to home domain.

3.	Whether and how to ensure service continuity and switching for inference tasks or AI service sessions during UE mobility across MNOs and CSPs/ECSPs.

4.	Whether and how UEs or VAL functions can discover the most suitable AIMLE instance in edge-specific scenarios while UEs move across network boundaries or service areas, considering latency, capability, and service policy.

5.	Whether and how the AIMLE server and/or ML repository are aware of the serving PLMN/network information of the UE associated with the AIML client.

6.	Whether and how to enhance AIMLE context transfer capability given various conditions related to the AIMLE server capability, load or availability changes.

There is a total of six solutions that address key issue #3: solutions #7, #10, #11, #16, #18, and #19. Each of these solutions covers different factors of the Key Issue.

Solution #7 covers the deployment of one AIMLE server supporting VAL UEs connected to different PLMNs, and provides an enhancement of the AIMLE client discovery capability to account for AIMLE clients connected to different PLMNs. In additon, Solution #19 provides an enhancement to AIMLE client selection capability for a different deployment where each AIMLE server is deployed per PLMN (hence interactions covering two AIMLE servers of different providers are expected). These solutions are not conflicting since they target different AIMLE server deployments in cross-PLMN scenario.

Solution #10 and #16 support the AIMLE service continuity or migration for UEs while roaming. Solution #10 supports enhancements to AIMLE client registration procedure  to enable the AIMLE server to identify whether the AIMLE client is allowed to use AIMLE services while in a visited network. In addition, Solution #16 provides a mechanism for discovering and selecting a T-AIMLE server for continuing the AIMLE service at the V-PLMN. These solutions are complementary and target different aspects of UE roaming.

Solution #11 supports enhancements in FL member registration capability to allow ML repository being aware of the MNO information for potentially allowing AIMLE servers/clients or VAL servers being considered as candidate FL members for a given FL operation. Such solution is only for scenarios where there might be implication supporting FL members connected (or have service agreements) with different 3GPP network systems.

Solution #18 describes a new capability in AIMLE server to support Federated AI/ML services.

Regarding the mapping of the above solutions to open issues, the solution doesn’t provide alternative, but complementary capabilities, in particular:

- Solution #7 addresses Open Issue 1, 4, 5

- Solution #10 addresses Open Issue 2, 5

- Solution #11 addresses Open Issue 3, 4

- Solution #16 addresses Open Issue 1, 2, 4, 5

- Solution #18 addresses Open Issue 1, 4, 5

- Solution #19 addresses Open Issue 1, 4, 5

## In each solution that addresses Key Issue #3, there is AIMLE architectural impact since this requires the introduction of architecture enhancements for multi-operator, UE roaming and support for Federated AIMLE services.

## 10.5	Key issue #4: Support for Split AI/ML Operations Involving Multiple AIMLE clients, edge AIMLE servers and central AIMLE servers

The open issues studied in the Key Issue #4 are as follows:

1.	Define the use cases supported in the AIML enablement layer involving multiple AIMLE clients, edge AIMLE servers and central AIMLE servers requiring concurrent usage of the same split AI/ML operation pipeline.

2.	Whether and how the AIML enablement layer can support split AI/ML operations involving multiple AIMLE clients, edge AIMLE servers and central AIMLE servers.

3.	Whether and how the AIMLE enablement layer can support service continuity during split AIML operation involving multiple AIMLE clients.

4. Whether and how to support dynamic inference partitioning and coordinating across AIMLE clients, edge AIMLE servers and central AIMLE servers in AIMLE layer.

NOTE:	This open issue may involve the ability to split an AI/ML inference pipeline into segments that can be executed on different nodes based on real-time conditions such as latency requirements, processing capabilities, network bandwidth, and current load (which may be provided by VAL UE or 5GC).

This clause provides an overall evaluation of the key issue #4. The solutions #8 and #12 cover the different aspects for the open issues in the KI #4.

Solution #8 addresses open issues #1 and #2. This solution enables the management of multiple AIMLE clients collaboration in same pipeline. AIMLE server maintains the pipeline, dynamically does the task distribution and manages the participation of multiple clients. The AIMLE clients can discover and join the split pipeline. AIMLE server coordinates the AIMLE split task operation based on joining and leaving of AIMLE clients.

Solution #12 address open issues #1 and #2, but enabling an intermediate entity (UE , edge function) to serve as relay node for AIMLE split operation. The solution proposes the management aspects (registration, discovery) for such relay node. This impacts the existing AIMLE client registration procedure in 3GPP TS 23.482 [2].

Solution #8 and #12, both impact the aspects of enabling multi-client AIMLE split operation. During normative phase, Solution #12 can be part of Solution #8, where the AIMLE client node joins the multi-client AIMLE split pipeline with a specific role of intermediate or relay node.

## Solution #21 also propose changes to split operation node registration, and hence the solutions in #8 and #12 are impacted with Solution #21.

## 10.6	Key issue #5: Support for AI Inference Exposure

The open issue studied in the Key Issue #5 is as follows:

1.	Whether and how to enhance ADAES service capabilities to expose ML model inference result based on the suitable ML model provided by AIMLE service in both central and edge scenarios.

There is one solution that address key issue #5, which is solution #25.

Solution #25 enables VAL to discover ML inference services provided by AIMLE/ADAE server. This enables any authorized VAL server can discover and utilize AI inference services without needing internal knowledge of the AIMLE implementation.

Regarding the mapping of the above solution to open issue, the solution #25 maps to open issue 1, however the role ADAES needs to be elaborated, since according to the assumptions in clause 5, the ML model inference which is not related to ADAE analytics is not to be performed by ADAES (but AIMLE).

## In the solution that addresses Key Issue #5, there is no AIMLE architectural impact.

## 10.7	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios

Key Issue #6 studied potential enhancements of AI/ML services where AIML enablement layer is deployed in distributed or hierarchical fashion as indicated in Annex A.3 and A.4 of 3GPP TS 23.482 [2]. The open issues studied in KI #6 are as follows:

1.	Study and identify the use cases and definition of hierarchical and distributed AIMLE architecture.

2.	Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

3.	Whether and how to enhance the architecture and related functions to support for AIMLE services, AIML operation assuming diverse (e.g., hierarchical and distributed) deployments scenarios.

4.	Whether and how to enhance data management assistance among AIMLE Clients, edge AIMLE servers, and central AIMLE servers in diverse (e.g., hierarchical and distributed) AIMLE deployments scenarios.

5.	Whether and how the AIMLE services can support the discovery of AIMLE servers.

6.	Whether and how the conditions under which a change of AIMLE server is identified.

There are six solutions that address KI #6 for hierarchical AIMLE deployments. Each of the six solutions address use cases involving AIMLE services that can be provided in hierarchical AIMLE deployments. Solution #9, #14, and #26 complement each other in addressing open issues 2, 3, and 5 for AIMLE server registration and discovery as well as AIMLE client discovery. Solutions #13, #21, and #24 all address open issue 3 but for different aspects of AIMLE services, such as ML model training, ML model collaborative inferencing, and split node related registration procedures. There is no solution that addresses open issues 4 and 6.

Solution #9 addresses open issue 2 by supporting AIMLE servers operating in edge data networks to register to a central AIMLE server in hierarchical AIMLE deployments. Standardized procedures for AIMLE server registration, update, and de-registration are defined. Through these procedures, the central AIMLE server can dynamically maintain awareness of edge AIMLE servers’ capabilities and the set of AIMLE clients connected to them, thereby enabling efficient hierarchical coordination and service discovery.

Solution #13 addresses open issue 3 where a central AIMLE server can support ML model training in hierarchical AIMLE deployments. The procedure allows an ML model to be trained by AIMLE clients distributed in different EDN service areas which are managed by different edge AIMLE servers.

Solution #14 addresses open issues 2 and 3 by enhancing existing AIMLE client discovery to support AIMLE service provisioning in hierarchical AIMLE deployments. New IEs are added to AIMLE client discovery procedure to assist with the discovery of AIMLE clients served by edge deployed AIMLE servers.

Solution #21 addresses open issues 3 by updating existing split node registration, registration update, and de-registration procedures described in 3GPP TS 23.482 [2]. Through these procedures, AIMLE clients can inform an AIMLE server of their capability to participate in split AI/ML operations in hierarchical AIMLE deployments.

Solution #24 addresses open issue 3 where ML model collaborative inference is considered in edge data networks for hierarchical AIMLE deployments. A central AIMLE server distributes inference tasks to multiple edge AIMLE servers for parallel execution to enable efficient use of distributed computing resources by offloading heavy or data-local computation to the edge AIMLE servers.

Solution #26 addresses open issue 5 where AIMLE servers can be discovered by AIMLE service consumers. AIMLE clients or VAL servers are pre-configured with a Default AIMLE server URI to allow the AIMLE service consumers to discover other AIMLE servers when the default AIMLE server is not available to support AI/ML operations.

## 10.8	Key Issue #7: VFL sample alignment across VAL servers

Key Issue #7 studied VFL sample alignment performed by AIMLE servers to enable VFL operation. The open issues studied in KI #7 are as follows:

1.	Whether and how to enhance VFL operation considering also VAL servers, AIMLE clients and AIMLE servers as VFL participants.

2.	Whether and how to support/enhance sample alignment also considering VAL servers / AIMLE servers / AIMLE clients as VFL participants.

There are two solutions that address KI #7. Solutions #15 addesses VFL sample alignment involving AIMLE clients while solution #22 addresses VFL sample alignment involving VAL servers serving as VFL clients.

Solution #15 addresses open issues 1 by proposing to enhance existing VFL procedure described in clause 8.18 of 3GPP TR 23.482 [2]. The enhancements include client feature selection and sample binding requests sent from an AIMLE server to AIMLE clients as part of VFL alignment procedure. The client feature selection request is used by the AIMLE server to activate or de-activate feature ID(s)/feature list maintained by AIMLE clients for each data domain. The sample binding request is used by the AIMLE server to provide sample binding criteria and sample pool for AIMLE clients to bind features in available datasets to the sample pool as part of VFL sample alignment.

Solution #22 addresses open issues 1 and 2 by proposing procedures in support of sample alignment by VAL servers. Enhancements to FL registration procedure described in clause 8.4 of 3GPP TR 23.482 [2] for VFL client registration are proposed to provide VFL related metadata to discover and select VFL clients and to assist with VFL sample alignment. A new procedure for VFL training and inferencing is proposed for AIMLE servers to assist with VFL operations involving multiple VAL servers.

## 10.9	Key Issue #8: Support for AI Inference service performance management and feedback

This clause will provide an overall evaluation of the key issue #8.

# 11	Conclusions

## 11.1	General conclusions and recommendations

The present technical report described the enhanced AI/ML enablement capabilities for supporting vertical use cases. This technical report fulfills the objectives of the study on application architecture for enabling AIML services – phase 2. The report includes the following:

1.	Definition of terms and abbreviations used in the study (clause 3);

2.	Architecture assumptions and principles (clause 4);

3.	Key issues identified by the study (clause 5);

4.	Enhanced architecture for enabling AI/ML services (clause 6);

5.	Individual solutions addressing the key issues (clause 7);

6.	Deployment scenarios (clause 8), Business relationships (clause 9); and

7.	Overall evaluations of all the solutions (clause 10); For normative work in 3GPP Rel-20, it is recommended to define the following architecture enhancements as also described in clause 6:

-	AIMLE to support federation, cross-MNO and AIMLE service continuity in roaming scenarios.

-	ADAES to be enhanced with additional analytics service to support monitoring ML model performance for its ML-enabled analytics, and to perform ML model inference for ADAE analytics services.

-	ML repository to support storing ML model and ML/FL participant information in multi-MNO scenarios.

In addition, it is recommended to define:

1.	Terms and abbreviations, the definition of terms and abbreviations captured in clause 3 will be reused.

2.	Enhanced AIMLE architecture for enabling AI/ML services, the architecture variants as specified in clause 6 will be used as enhancements to AIMLE architecture.

3.	Deployment scenarios will be considered as captured in clause 9.

## 4. 	The definition of the corresponding enablement APIs based on the concluded solutions as summarized in clause 10.1.

## 11.2	Conclusions of key issue #1

Following solution considerations will be considered for the normative work for KI #1:

## Editor’s Note: Solution #1 and Solution #17 will be concluded upon completing the respective solution evaluations.

## 11.3	Conclusions of key issue #2

## 11.4	Conclusions of key issue #3

Following solution considerations will be considered for the normative work for KI #3:

- Solution #7 can be considered for normative work to support cross-PLMN AIMLE client discovery, whereas AIMLE server is deployed at ECSP/CSP with agreements to multiple MNOs.

ΝΟΤΕ 1: Whether and how alternative capabilities can be used to determine the VAL UE roaming status will be discussed in the normative phase.

- Solution #10 can be considered for normative work to support AIMLE client registration enhancements for UE roaming scenarios.

- Solution #11 can be considered for normative work to support FL member registration information storage enhancements with MNO information, for enabling the consideration of candidate FL members for a given FL task with restrictions/preferences on the PLMN to be associated with, in cross-PLMN AIMLE scenarios.

ΝΟΤΕ 2: This solution 	doesn’t consider any communication aspects of enabling FL operation over multiple PLMNs and is only applicable to registering information to the ML repository which is initiated from the candidate FL members themselves.

- Solution #16 can be considered for normative work to support AIMLE service continuity for UE roaming scenarios.

- Solution #18 can be considered for normative work to support federated AI/ML services.

- Solution #19 can be considered for normative work to support cross-PLMN AIMLE client discovery assuming AIMLE servers deployed per PLMN.

## 11.5	Conclusions of key issue #4

Following solution considerations will be considered for the normative work for KI #4:

-	Solutions #8 and #12 can be considered for normative work.

Editor’s Note: The conclusions to key issue #4 is FFS and depends on solution evaluation and editor note resolution in solution #21.

## 11.6	Conclusions of key issue #5

Editor’s Note: Conclusion of this Key Issue requires evaluation of Solution #25 and elaboration of the proposed enhancement of AIMLE / ADAES to support this solution

## 11.7	Conclusions of key issue #6

The following solutions will be considered for normative work for KI #6:

- Solution #9 provides support for AIMLE servers in edge data networks to register to central AIMLE servers in hierarchical AIMLE deployments.

- Solution #13 provides support for ML model training in hierarchical AIMLE deployments.

- Solution #14 provides support for AIMLE client discovery in hierarchical AIMLE deployments.

- Solution #24 provides support for ML model collaborative inference in hierarchical AIMLE deployments.

- Solution #26 provides support for AIMLE server discovery in hierarchical AIMLE deployments.

NOTE:	Whether to unify solution #26 with Clause 8.25 of 3GPP TS 23.482 [2] will be determined during normative phase.

Editor's Note: The conclusion for solution #21 is pending completion of the solution evaluation.

## 11.8	Conclusions of key issue #7

The following solutions will be considered for normative work for KI #7:

- Solution #15 provides support for VFL sample alignment involving AIMLE clients.

- Solution #22 provides support for VFL sample alignment involving VAL servers serving as VFL clients.

NOTE:	Consideration for merging solutions #15 and #22 will be made during normative phase.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA6#66 |  |  |  |  | TS skeleton | 0.0.0 |
| 2025-04 | SA6#66 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-251333, S6-251334, S6-251443, S6-251459, S6-251480, S6-251481, S6-251482, S6-251491 | 0.1.0 |
| 2025-05 | SA6#67 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-252512, S6-252427, S6-252494, S6-252509, S6-252430, S6-252495, S6-252480, S6-252536, S6-252434, S6-252537, S6-252437, S6-252520,S6-252438, S6-252439 | 0.2.0 |
| 2025-08 | SA6#68 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-253646, S6-253647, S6-253648, S6-253652, S6-253653, S6-253672, S6-253674, S6-253676, S6-253677, S6-253678, S6-253679, S6-253681, S6-253711, S6-253745, S6-253746, S6-253747, S6-253748, S6-253768, S6-253769, S6-253771. | 0.3.0 |
| 2025-09 | SA#109 | SP-251047 |  |  |  | Presentation for information at SA#109 | 1.0.0 |
| 2025-10 | SA6#69 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-254086, S6-254259, S6-254260, S6-254278, S6-254282, S6-254385, S6-254388, S6-254390, S6-254391, S6-254392, S6-254393, S6-254394, S6-254395, S6-254398, S6-254399, S6-254652, S6-254704, S6-254705, S6-254716, S6-254718, S6-254719, S6-254744, S6-254745, S6-254796. | 1.1.0 |
| 2025-11 | SA6#70 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-255200, S6-255201, S6-255218, S6-255401, S6-255402, S6-255403, S6-255406, S6-255407, S6-255409, S6-255410, S6-255412, S6-255413, S6-255417, S6-255611, S6-255612, S6-255620, S6-255621, S6-255635, S6-255652, S6-255653, S6-255677. | 1.2.0 |