| 3GPP TS 24.369 V0.3.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS); Stage 3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	7

4	General	7

4.1	Overview	7

4.2	AIoT NAS security	8

4.2.1	General	8

4.2.2	Authentication	8

4.2.3	Replay protection of AIoT NAS signalling messages	8

4.2.4	Integrity protection of AIoT NAS signalling messages	8

4.2.5	Ciphering of AIoT NAS signalling messages	8

4.2.6	Privacy of AIoT device identifiers	9

4.3	Primary authentication and key agreement procedure	9

4.3.1	General	9

5	Elementary procedures for AIoT NAS protocol	9

5.1	General	9

5.2	Inventory procedure	9

5.2.1	General	9

5.2.2	Inventory procedure initiation	10

5.2.3	Inventory procedure completion	10

5.2.4	Inventory procedure not accepted by AIoT device	11

5.2.5	Abnormal cases in the AIoT device	12

5.2.6	Abnormal cases in the AIOTF	12

5.3	Command procedures	12

5.3.1	General	12

5.3.2	Read command procedure	12

5.3.3	Write command procedure	13

5.3.4	Permanent disable command procedure	15

5.3.5	Abnormal cases in the AIOTF	15

6	Handling of unknown, unforeseen, and erroneous protocol data	15

6.1	General	15

6.2	Message too short or too long	16

6.2.1	Message too short	16

6.2.2	Message too long	16

6.3	Unknown or unforeseen message type	16

6.4	Non-semantical mandatory information element errors	16

6.5	Unknown and unforeseen IEs in the non-imperative message part	16

6.5.1	IEIs unknown in the message	16

6.5.2	Out of sequence IEs	17

6.5.3	Repeated IEs	17

6.6	Non-imperative message part errors	17

6.6.1	General	17

6.6.2	Syntactically incorrect optional IEs	17

6.7	Messages with semantically incorrect contents	17

7	Encoding of AIoT NAS protocol	17

7.1	Message functional definitions and contents	17

7.1.1	General	17

7.1.2	Inventory report	18

7.1.3	Read command	18

7.1.4	Read complete	19

7.1.5	Read command reject	19

7.1.6	Write command	20

7.1.7	Write complete	20

7.1.8	Write command reject	20

7.1.9	Permanent disable command	21

7.1.10	Permanent disable complete	21

7.2	Encoding of information elements	21

7.2.1	General	21

7.2.2	Message type	22

7.2.3	Security parameters	22

7.2.4	AIoT data	22

7.2.5	AIoT data length	23

7.2.6	AIoT device identity	23

7.2.7	Authentication response parameter	23

7.2.8	Cause	23

7.2.9	Offset	24

7.2.10	RAND	24

8	List of system parameters	24

8.1	General	24

8.2	<Subtitle>	24

Annex A (informative):	25

A.1	Causes related to command procedure	25

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the ambient internet of things (AIoT) non-access-stratum (NAS) protocol to support the transport of signalling and AIoT data between AIoT device and the AIoT function (AIOTF) in the 5G system (5GS) as specified in 3GPP TS 23.369 [2].

Editor's note:	The scope should be aligned with the naming of the protocol and is FFS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[3]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 23.003: "Numbering, addressing and identification".

[6]	3GPP TS 33.369: "Security aspects of ambient IoT services in 5G".

[7]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[8]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.369 [2] apply:

AIoT Device

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AIoT	Ambient Internet of Things

AIOTF	AIoT Function

NAS	Non-Access-Stratum

# 4	General

## 4.1	Overview

The AIoT NAS protocol described in the present document provide signalling between the AIoT device and the AIOTF, for 5GS. The AIoT NAS as part of the protocol stacks for support of AIoT services in 5GS is described in 3GPP TS 23.369 [2].

The main function of the AIoT NAS protocol is to support the AIoT procedures (see 3GPP TS 23.369[2]):

a)	inventory procedure; and

b)	command procedure.

For the support of the above functions, the procedures are supplied within procedures for AIoT NAS, in clause 5.

The AIoT NAS messages are transported between the AIoT device and the AIOTF via AIOT1 reference point, and this transport does not involve N1 reference point. The AIoT NAS protocol specified in the present document is unrelated to the 5GMM NAS and 5GSM NAS protocols specified in 3GPP TS 24.501 [4].

Editor's note:	AIoT NAS security functions, e.g. integrity protection and ciphering of AIoT NAS signalling messages, is FFS and depends on SA3 progress and available normative requirements in 3GPP TS 33.369 [6].

The AIoT NAS protocol for 5GS follows the protocol architecture model for layer 3 as described in 3GPP TS 24.007 [3].

## 4.2	AIoT NAS security

### 4.2.1	General

This clause describes the principles for the handling of AIoT security in the AIoT device and in the AIOTF and the procedures used for the security protection of AIoT NAS messages between AIoT device and AIOTF. AIoT NAS security includes authentication, AIoT NAS message security protection and AIoT device identifier privacy. AIoT NAS message security protection involves integrity protection and ciphering of the AIoT NAS messages.

When an AIoT NAS message needs to be sent both ciphered and integrity protected, the AIoT NAS message is first ciphered and then the ciphered AIoT NAS message is integrity protected by calculating the MAC.

The long-term credentials for authentication, integrity protection and ciphering are provisioned and stored in the AIoT device and in the network. The mechanism to provision and store long-term credentials is out of scope of 3GPP in this version of the present document. The relationship between the long-term credentials and security parameters is defined in 3GPP TS 33.369 [6].

The signalling procedures for the control of AIoT NAS security are part of the AIoT NAS protocol and are described in detail in clause 5.

### 4.2.2	Authentication

Authentication of the AIoT device is done in the inventory procedure. A network generated RANDAIOT_n is provided by the AIOTF and received by the AIoT device in the paging request. The AIoT device which is target of the paging request (see clause 5.2.3), uses the RANDAIOT_n and its long-term KAIOT to derive a RESAIOT and includes the RESAIOT and a RANDAIOT_d in the INVENTORY REPORT message to the AIOTF. The network verifies the RESAIOT to authenticate the AIoT device.

The requirements for the AIoT device to derive RESAIOT at receiving RANDAIOT_n from lower layers are described in 3GPP TS 33.369 [6].

The requirements for parameters and algorithms used at AIoT authentication are specified in 3GPP TS 33.369 [6].

Editor's note:	Details on authentication will be aligned with stage 2 normative requirements when available.

### 4.2.3	Replay protection of AIoT NAS signalling messages

Editor's note: Replay protection of AIoT NAS signalling messages is FFS, pending available stage 2 requirements.

### 4.2.4	Integrity protection of AIoT NAS signalling messages

Editor's note: Integrity protection of AIoT NAS signalling messages is FFS, pending available stage 2 requirements.

### 4.2.5	Ciphering of AIoT NAS signalling messages

The use of ciphering in a network is an operator option subject to AIOTF configuration. When operation of the network without ciphering is configured, the AIOTF shall indicate the use of "null ciphering algorithm" NEA0.

All AIoT NAS messages, except the INVENTORY REPORT message, shall be sent ciphered. If the "null ciphering algorithm" NEA0 has been selected as a ciphering algorithm, the AIoT NAS message is regarded as ciphered.

Editor's note: It is FFS whether other AIoT NAS messages than INVENTORY REPORT message can be sent unciphered.

The algorithm to calculate the ciphering of AIoT NAS messages is described in annex D.2 in 3GPP TS 33.501 [9]. In addition to the data that is to be ciphered, the constant BEARER, DIRECTION bit, COUNT, KEY and LENGTH are input to the ciphering algorithm. These parameters are described in 3GPP TS 33.369 [6].

Editor's note:	The part of the AIoT NAS messages subject to confidentiality protection is FFS.

### 4.2.6	Privacy of AIoT device identifiers

Editor's note: AIoT device identifier privacy is FFS, pending available stage 2 requirements.

## 4.3	Primary authentication and key agreement procedure

### 4.3.1	General

The primary authentication and key agreement procedure is to enable mutual authentication between the AIoT device and the network.

Editor's note:	The details of the authentication procedure can be updated according to SA3 progress.

# 5	Elementary procedures for AIoT NAS protocol

## 5.1	General

This clause describes the procedures used for AIoT NAS protocol performed between the AIoT device and the AIOTF. The format and coding of the messages and information elements are specified in clause 7.1 and 7.2 respectively.

The following AIoT NAS procedures can be distinguished:

a)	procedure related to network initiated inventory request; and

b)	procedure related to network initiated command request.

AIoT NAS messages are standard L3 messages according to 3GPP TS 24.007 [3] and error behaviour specified for L3 protocol according to 3GPP TS 24.007 [3] applies for AIoT unless otherwise specified in the present document.

Editor's note:	Whether AIoT NAS protocol supports conditional IEs and comprehension required IEs as specified in 3GPP TS 24.007 [3] is FFS.

Editor's note:	Whether AIoT NAS protocol supports TLV-E format is FFS.

## 5.2	Inventory procedure

### 5.2.1	General

The inventory procedure is used to discover one or more AIoT devices:

a)	if the AIoT identification information is not provided to NG-RAN, all the AIoT devices are to be discovered. In this case, an indication from lower layer is provided as specified in clause 5.2 of 3GPP TS 38.391 [8];

b)	if the AIoT identification information is provided to NG-RAN and the AIoT identification information includes the filtering information, a group of AIoT devices that matches the AIoT identification information are to be discovered; and

c)	if the AIoT identification information is provided to NG-RAN and the AIoT identification information includes an AIoT device identifier, a specific AIoT device that matches the AIoT identification information is to be discovered.

NOTE:	Details of AIOTF providing the AIoT identification information to NG-RAN is defined in 3GPP TS 23.369 [2].

Editor's note:	 Details of AIoT device identifier should be aligned with security consideration from SA3.

### 5.2.2	Inventory procedure initiation

To initiate the inventory procedure, the AIOTF sends an inventory request to the selected NG-RAN, as defined in 3GPP TS 38.413 [10]. The selected NG-RAN performs the AIoT paging by broadcast of the paging message that includes the AIoT identification information received in the inventory request (see 3GPP TS 38.300 [7] and 3GPP TS 38.391 [8]).

NOTE:	How AIOTF selects NG-RAN is defined in 3GPP TS 23.369 [2].

Editor's note:	The reference to TS 38.391 may be updated according to RAN2 progress.

Figure 5.2.2-1 gives an example of inventory procedure for a AIoT device which sends the INVENTORY REPORT message to AIOTF.

Editor's note:	Figure 5.2.2-1 may further align with RAN2/RAN3 according to RAN2/RAN3 progress.

Figure 5.2.2-1: Inventory procedure

Upon receiving a paging message, the lower layers of the AIoT device determines whether to respond to the paging message as specified in 3GPP TS 38.391 [8]. The evaluation in lower layers includes interaction with upper layers to assess AIoT identification information, if included in the paging message, as specified in clause 5.2. If lower layers determine to respond to the paging message, an indication is provided to upper layers.

### 5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on whether the AIoT identification information received from the lower layer is matched or not.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

The AIoT device shall include the AIoT device identity IE in the INVENTORY REPORT message.

Editor's note:	Details of the AIoT device identity IE is FFS and depends on SA3 progress.

The AIoT device determines whether the AIoT identification information is matched or not as follows:

Editor's note:	It is FFS how to determine the component of AIoT identification information for AIoT device, and this may need coordination with RAN2/3.

a)	if the AIoT identification information includes an AIoT device identifier:

Editor's note:	Details of AIoT device identifier should be aligned with security consideration from SA3. How to match the AIoT device identifier depends on the AIoT device identifier included.

b)	if the AIoT identification information includes filtering information (see clause 31.3 of 3GPP TS 23.003 [5]):

1)	based on ID type field included in the filtering information, the AIoT device determines the AIoT identification information is matched if:

A)	the PLMN ID field is not included in the filtering information, or the PLMN ID field is included in the filtering information and:

i)	the PLMN ID field is included in the AIoT device permanent identifier; and

ii)	the PLMN ID field in the AIoT device permanent identifier is the same as the PLMN ID field within the filtering information;

B)	the NID field is not included in the filtering information, or the NID field is included in the filtering information and:

i)	the NID field is included in the AIoT device permanent identifier; and

ii)	the NID field in the AIoT device permanent identifier is the same as the NID field within the filtering information;

C)	the third party ID field is not included in the filtering information, or the third party ID field is included in the filtering information and:

i)	the third party ID field is included in the AIoT device permanent identifier; and

ii)	the third party ID field in the AIoT device permanent identifier is the same as the third party ID field within the filtering information; and

D)	the identification information filters field is not included in the filtering information, or the identification information filters field is included in the filtering information and for each identification information filter, the component value within the identification information filter is the same as the corresponding bitstring from the AIoT device permanent identifier based on offset field and length field; or

2)	otherwise, the AIoT device determines the AIoT identification information is not matched.

NOTE 1:	The AIoT identification information corresponds to paging ID, as defined in 3GPP TS 38.391 [8]).

Editor's note:	The reference to filtering information (e.g. IE name) will be updated according to CT4 and RAN2 progress.

Based on whether the AIoT identification information is matched:

a)	if the AIoT identification information is matched, the AIoT device shall indicate to the lower layer that the AIoT identification information is matched, and determine to respond to the AIoT paging; or

b)	otherwise, the AIoT device shall ignore the AIoT identification information, indicate to the lower layer that the filtering information is not matched, and determine not to respond to the AIoT paging.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

Editor's note:	AIoT .device authentication of the network is FFS.

### 5.2.4	Inventory procedure not accepted by AIoT device

If the AIoT device has completed a disable command procedure, the AIoT device shall not respond to the AIoT paging message.

### 5.2.5	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	Failure of authentication.

If an AIoT device fails the authentication procedure as specified in 3GPP TS 33.369 [6], the AIoT device shall not respond to the AIoT paging message.

b)	Collision of AIoT paging.

If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall proceed with the ongoing inventory procedure and shall process the new AIoT paging after the completion of the inventory procedure.

### 5.2.6	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of an inventory failure received as specified by 3GPP TS 38.413 [10] from all the selected NG-RAN(s), the AIOTF shall abort the inventory procedure.

In the case the lower layer indication of no inventory response from all the selected NG-RAN(s), the AIOTF shall abort the inventory procedure.

## 5.3	Command procedures

### 5.3.1	General

This clause describes command procedures between the AIOTF and AIoT device. Command procedures are initiated by the AIOTF to perform read, write, or permanent disable operation on the AIoT device.

Editor's note:	Timer(s) for command procedures are FFS.

### 5.3.2	Read command procedure

#### 5.3.2.1	General

The purpose of the read command procedure is to retrieve AIoT data from the AIoT device (see example in figure 5.3.2.1.1).

AIoT device

AIOTF

READ COMMAND

READ COMPLETE

OR

READ COMMAND REJECT

Figure 5.3.2.1.1: Read command procedure

#### 5.3.2.2	Read command procedure initiation

The AIOTF initiates the read command procedure by sending a READ COMMAND message to the AIoT device. In the READ COMMAND message, the AIOTF:

a)	shall include the Read offset IE set to the offset from the start of the AIoT device's user memory, indicating where to read the AIoT data; and

b)	shall include the Read length IE set to the length of the AIoT data to be read.

#### 5.3.2.3	Read command procedure accepted by the AIoT device

Upon receiving the READ COMMAND message, if the AIoT device supports read command, the AIoT device:

a)	shall validate the offset in the Read offset IE and the length in the Read length IE;

NOTE:	The validation of the offset and the length is implementation-specific to the AIoT device.

b)	shall retrieve the AIoT data based on the Read offset IE and the Read length IE which are validated in a); and

c)	shall send a READ COMPLETE message to the AIOTF. In the READ COMPLETE message, the AIoT device shall include the AIoT data IE set to the retrieved AIoT data described in b).

#### 5.3.2.4	Read command procedure completion by the AIOTF

Upon receiving the READ COMPLETE message, the AIOTF shall consider the read command procedure as successfully completed.

#### 5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if read command is not supported by the AIoT device, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

Editor's note:	Whether to use the READ COMMAND REJECT message to reject the READ COMMAND message when the read command not supported is FFS.

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

#### 5.3.2.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The read command is unsuccessfully implemented (e.g. user memory operation error).

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

b)	The application data cannot be found in the requested place of the user memory.

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

Editor's note:	It is FFS the command and inventory procedure collision case.

### 5.3.3	Write command procedure

#### 5.3.3.1	General

The purpose of the write command procedure is to write the AIoT data to the AIoT device's user memory (see example in figure 5.3.3.1.1).

AIoT device

AIOTF

WRITE COMMAND

WRITE COMPLETE

OR

WRITE COMMAND REJECT

Figure 5.3.3.1.1: Write command procedure

#### 5.3.3.2	Write command procedure initiation

The AIOTF initiates the write command procedure by sending a WRITE COMMAND message to the AIoT device. In the WRITE COMMAND message, the AIOTF:

a)	shall include the Write offset IE set to the offset from the start of the AIoT device's user memory indicating where to write the AIoT data; and

b)	shall include the AIoT data IE set to the AIoT data to be written.

#### 5.3.3.3	Write command procedure accepted by AIoT device

Upon receiving the WRITE COMMAND message, if the AIoT device supports write command, the AIoT device:

a)	shall validate the offset in the Write offset IE and the length of AIoT data contents in the AIoT data IE;

NOTE:	The validation of the offset and the length is implementation-specific to the AIoT device.

b)	shall write the AIoT data contents in the received AIoT data IE to the AIoT device's user memory at the offset which is validated in a); and

c)	shall send a WRITE COMPLETE message to the AIOTF.

#### 5.3.3.4	Write command procedure completion by the AIOTF

Upon receiving the WRITE COMPLETE message, the AIOTF shall consider the write command procedure as successfully completed.

#### 5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if write command is not supported by the AIoT device, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Editor's note:	Whether to use the WRITE COMMAND REJECT message to reject the WRITE COMMAND message when the write command not supported is FFS.

Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

#### 5.3.3.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The write command is unsuccessfully implemented (e.g. user memory operation error)

The AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

### 5.3.4	Permanent disable command procedure

#### 5.3.4.1	General

The purpose of the permanent disable command procedure (see example in figure 5.3.4.1.1) is to permanently disable the communication capability of the AIoT device as specified in 3GPP TS 23.369 [2].

AIoT device

AIOTF

PERMANENT DISABLE COMMAND

PERMANENT DISABLE COMPLETE

Figure 5.3.4.1.1: Permanent disable command procedure

#### 5.3.4.2	Permanent disable command procedure initiation

The AIOTF initiates the permanent disable command procedure by sending a PERMANENT DISABLE COMMAND message to the AIoT device.

#### 5.3.4.3	Permanent disable command procedure accepted by AIoT device

Upon receiving the PERMANENT DISABLE COMMAND message, the AIoT device shall send a PERMANENT DISABLE COMPLETE message to the AIOTF and disable the communication capability of the AIoT device.

#### 5.3.4.4	Permanent disable command procedure completion by the AIOTF

Upon receiving the PERMANENT DISABLE COMPLETE message, the AIOTF shall consider the permanent disable command procedure as successfully completed.

### 5.3.5	Abnormal cases in the AIOTF

The following abnormal cases can be identified:

a)	Lower layer failure

In the case the lower layer indication of a command failure received as specified by 3GPP TS 38.413 [10], the AIOTF shall abort the command procedure.

# 6	Handling of unknown, unforeseen, and erroneous protocol data

## 6.1	General

The procedures specified in the present document apply to those messages which pass the checks described in this clause.

This clause also specifies procedures for the handling of unknown, unforeseen, and erroneous protocol data by the receiving entity. These procedures are called "error handling procedures", but in addition to providing recovery mechanisms for error situations they define a compatibility mechanism for future extensions of the protocols.

Clauses 6.2 to 6.7 shall be applied in order of precedence.

Detailed error handling procedures in the network are implementation dependent and may vary from network to network. However, when extensions of this protocol are developed, networks will be assumed to have the error handling that is indicated in this clause as mandatory ("shall") and that is indicated as strongly recommended ("should").

Also, the error handling of the network is only considered as mandatory or strongly recommended when certain thresholds for errors are not reached during a dedicated connection.

For definition of semantical and syntactical errors see 3GPP TS 24.007 [3], clause 11.4.2.

Editor's note:	Whether AIoT NAS protocol supports conditional IEs and comprehension required IEs is FFS.

Editor's note:	Whether AIoT NAS protocol supports TLV-E format is FFS.

## 6.2	Message too short or too long

### 6.2.1	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3].

### 6.2.2	Message too long

Editor's note:	Maximum AIoT NAS message length is FFS and to be defined by RAN3.

## 6.3	Unknown or unforeseen message type

If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message.

NOTE:	A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

## 6.4	Non-semantical mandatory information element errors

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error;

is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall ignore the AIoT NAS message and the network shall:

a)	try to treat the message (the exact further actions are implementation dependent); or

b)	ignore the message.

## 6.5	Unknown and unforeseen IEs in the non-imperative message part

### 6.5.1	IEIs unknown in the message

The AIoT device shall ignore all IEs with unknown IEI in a message.

The network shall take the same approach.

### 6.5.2	Out of sequence IEs

The AIoT device shall ignore all out of sequence IEs in a message.

The network should take the same approach.

### 6.5.3	Repeated IEs

If an information element with format T, TV or TLV is repeated in a message, the AIoT device shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element.

The network should follow the same procedures.

## 6.6	Non-imperative message part errors

### 6.6.1	General

This category includes syntactically incorrect optional IEs.

### 6.6.2	Syntactically incorrect optional IEs

The AIoT device shall treat all optional IEs that are syntactically incorrect in a message as not present in the message.

The network shall take the same approach.

## 6.7	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the AIoT device shall perform the foreseen reactions of the procedural part of clause 5. If, however no such reactions are specified, the UE shall ignore the message.

The network should follow the same procedure.

# 7	Encoding of AIoT NAS protocol

## 7.1	Message functional definitions and contents

### 7.1.1	General

Within the AIoT NAS protocol defined in the present document, every message is a standard L3 message as defined in 3GPP TS 24.007 [3]. This means that the message consists of the following parts:

1)	message type;

2)	security parameters;

3)	other information elements, as required.

The organization of an AIoT NAS message is illustrated in the example shown in figure 7.1.1-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message type |  |  |  |  |  |  |  | octet 1 |
| Security parameters |  |  |  |  |  |  |  | octet 2 octet a |
|  |  |  |  |  |  |  |  | octet a+1 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 7.1.1-1: General message organization example for an AIoT NAS message

Editor's note:	Additional content of AIoT NAS general message organization is FFS and can be added as normative requirements are available.

Editor's note:	The position of the Message type in the AIoT message as well as the position of any potential security parameters are FFS.

### 7.1.2	Inventory report

##### 7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to report the AIoT device identity.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | Security parameters | Security parameters 7.2.3 | M | V | n |
|  | RANDAIOT_d | RAND 7.2.10 | M | V | TBD |
|  | RESAIOT | Authentication response parameter 7.2.7 | M | V | TBD |
|  | AIoT device identity | AIoT device identity 7.2.6 | M | V | n |

### 7.1.3	Read command

##### 7.1.3.1	Message definition

The READ COMMAND message is sent by the AIOTF to the AIoT device to read the AIoT data from the memory.

See table 7.1.3.1-1.

Message type:	READ COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.3.1-1: READ COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Read command message identity | Message type 7.2.2 | M | V | 1 |
|  | Read offset | Offset 7.2.9 | M | V | 2 |
|  | Read length | AIoT data length 7.2.5 | M | V | TBD |

### 7.1.4	Read complete

#### 7.1.4.1	Message definition

The READ COMPLETE message is sent by the AIoT device to the AIOTF to provide the AIoT data that has been read from the memory.

See table 7.1.4.1-1.

Message type:	READ COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.4.1-1: READ COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Read complete message identity | Message type 7.2.2 | M | V | 1 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |

### 7.1.5	Read command reject

#### 7.1.5.1	Message definition

The READ COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of reading the AIoT data from the memory.

See table 7.1.5.1-1.

Message type:	READ COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.5.1-1: READ COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Read command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | Read cause | Cause 7.2.8 | M | V | 1 |

### 7.1.6	Write command

##### 7.1.6.1	Message definition

The WRITE COMMAND message is sent by the AIOTF to the AIoT device to write the indicated AIoT data to the memory.

See table 7.1.6.1-1.

Message type:	WRITE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.6.1-1: WRITE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Write command message identity | Message type 7.2.2 | M | V | 1 |
|  | Write offset | Offset 7.2.9 | M | V | 2 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |

### 7.1.7	Write complete

#### 7.1.7.1	Message definition

The WRITE COMPLETE message is sent by the AIoT device to the AIOTF to indicate that the AIoT data has been successfully written to the memory.

See table 7.1.7.1-1.

Message type:	WRITE COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.7.1-1: WRITE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Write complete message identity | Message type 7.2.2 | M | V | 1 |

### 7.1.8	Write command reject

#### 7.1.8.1	Message definition

The WRITE COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of writing the AIoT data from the memory.

See table 7.1.8.1-1.

Message type:	WRITE COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.8.1-1: WRITE COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Write command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | Write cause | Cause 7.2.8 | M | V | 1 |

### 7.1.9	Permanent disable command

##### 7.1.9.1	Message definition

The PERMANENT DISABLE COMMAND message is sent by the AIOTF to the AIoT device to permanently disable the communication capability of AIoT device.

See table 7.1.9.1-1.

Message type:	PERMANENT DISABLE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.9.1-1: PERMANENT DISABLE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Permanent disable command message identity | Message type 7.2.2 | M | V | 1 |

### 7.1.10	Permanent disable complete

#### 7.1.10.1	Message definition

The PERMANENT DISABLE COMPLETE message is sent by the AIoT device to the AIOTF to indicate that the AIoT device communication capability has been disabled.

See table 7.1.10.1-1.

Message type:	PERMANENT DISABLE COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.10.1-1: PERMANENT DISABLE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Permanent disable complete message identity | Message type 7.2.2 | M | V | 1 |

## 7.2	Encoding of information elements

### 7.2.1	General

This clause describes the encoding of the information elements used in the messages of the AIoT NAS protocol.

### 7.2.2	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [3]. Table 7.2.2.1 defines the value part of the Message type IE used in the AIoT NAS protocol.

Table 7.2.2-1: Message types for AIoT NAS

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Read command |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Read complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Read command reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Write command |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | Write complete |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | Write command reject |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Permanent disable command |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | Permanent disable complete |

Editor's note:	Whether the range for message type of 8 bits is needed or a shorter range is sufficient is FFS. If a shorter range is sufficient, remaining bits in the IE can be used for other mandatory parameters, e.g. part of the security parameters.

### 7.2.3	Security parameters

Editor's note:	This IE contains the mandatory security parameters in AIoT NAS messages and content is FFS awaiting available normative requirements in SA3. The IE can be renamed and split into multiple individual security IEs to better suit security requirements.

### 7.2.4	AIoT data

The purpose of the AIoT data information element is to carry the AIoT data that has been read from the memory or that is to be written to the memory of the AIoT device.

The AIoT data information element is coded as shown in figure 7.2.4-1 and table 7.2.4-1.

The AIoT data is a type 6 information element with a minimum length of 3 octets and a maximum length of TBD octets.

Editor's note:	the maximum length of the IE contents, when included in a READ COMPLETE message or WRITE COMMAND message, is FFS and subject to RAN2 agreement.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIoT data IEI |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of AIoT data contents |  |  |  |  |  |  |  |  | octet 2 octet 3 |  |
| AIoT data contents |  |  |  |  |  |  |  |  | octet 4 octet n |  |

Figure 7.2.4-1: AIoT data information element

Table 7.2.4-1: AIoT data information element

AIoT data contents (octets 4 to n):

The AIoT data contents contain the contents of the AIoT data that has been read from the memory or that is to be written in the memory of the AIoT device.

### 7.2.5	AIoT data length

The AIoT data length is a field of TBD octets length that indicates the length of the AIoT data to be read.

### 7.2.6	AIoT device identity

Editor's note:	This IE contains information provided by the AIoT device that can be translated by the network to a globally unique AIoT device permanent identifier. The information and coding of the IE is FFS awaiting available normative specification.

### 7.2.7	Authentication response parameter

The purpose of the authentication response parameter information element is to provide the network with the authentication response calculated in the AIoT device.

The Authentication response parameter information element is coded as shown in figure 7.2.7-1 and table 7.2.7-1.

The Authentication response parameter is a type 3 information element with TBD octets length. In an AIoT authentication challenge, the response calculated in the AIoT device is TBD octets in length, and is placed in the Authentication response parameter information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authentication response parameter IEI |  |  |  |  |  |  |  | octet 1 |
| RES value |  |  |  |  |  |  |  | octet 2 octet TBD |

Figure 7.2.7-1 Authentication response parameter information element

Table 7.2.7-1: Authentication response parameter information element

RES value (octet 2 to TBD)

The RES value consists of TBD bits. Bit 8 of octet 2 is the most significant bit while bit 1 of octet TBD is the least significant bit.

### 7.2.8	Cause

The purpose of the cause information element is to indicate the cause of the failure of the read or write operation.

The cause information element is coded as shown in figure 7.2.8-1 and table 7.2.8-1.

The cause is a type 3 information element with length of 2 octets.

Editor's note:	the possible cause values are FFS.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 7.2.8-1: Cause information element

Table 7.2.8-1: Cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Error, unspecified |
| All other values are spare and if received shall be interpreted as "error, unspecified". |  |  |  |  |  |  |  |  |  |

### 7.2.9	Offset

The offset is a field of 2 octets length that indicates the offset where to read the AIoT data from or where to write the AIoT data to.

### 7.2.10	RAND

The purpose of the RAND information element is to provide the AIoT device or the AIOTF with a non-predictable number to be used in security calculations.

The RAND information element is coded as shown in figure 7.2.10-1 and table 7.2.10-1.

The RAND is a type 3 information element with TBD octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAND IEI |  |  |  |  |  |  |  | octet 1 |
| RAND value |  |  |  |  |  |  |  | octet 2 octet TBD |

Figure 7.2.10-1 RAND information element

Table 7.2.10-1: RAND information element

RAND value (octet 2 to TBD)

The RAND value consists of TBD bits. Bit 8 of octet 2 is the most significant bit while bit 1 of octet TBD is the least significant bit.

# 8	List of system parameters

## 8.1	General

This clause describes the definition of the timers for AIoT.

## 8.2	<Subtitle>

To be edited, use Normal style

Annex A (informative):
Cause values

# A.1	Causes related to command procedure

Cause #x1 – TBD

This cause is used to indicate that the requested read or write command is not supported by the AIoT device.

Cause #x2 – TBD

This cause is used to indicate that the command type specific parameters (e.g. the offset to read application data from the AIoT device user memory, the length of the application data to read, the offset where to write the application data, or the length of the application data to write) is invalid.

Cause #x3 – TBD

This cause is used to indicate that the requested read or write command was not executed successfully.

Cause #x4 – TBD

This cause is used to indicate that the application data to be read cannot be found by the AIoT device in the requested place of the user memory.

Cause #x5 – TBD

This cause is used to indicate that the write command procedure cannot be implemented because the energy will be run out.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | CT1#154 |  |  |  |  | TS skeleton draft. C1-252552, C1-252553. | 0.1.0 |
| 2025-05 | CT1#155 |  |  |  |  | Inclusion of pCRs agreed during CT1#155: C1-253642, C1-253643, C1-253754, C1-253756, C1-254097, C1-254111, C1-254112, C1-254133. | 0.2.0 |
| 2025-08 | CT1#156 |  |  |  |  | Inclusion of pCRs agreed during CT1#156: C1-255166, C1-255170, C1-255186, C1-255189, C1-255286, C1-255330, C1-255623, C1-255624, C1-255625, C1-255626, C1-255643, C1-255644, C1-255646, C1-255647, C1-255660, C1-255661, C1-255675, C1-255704. | 0.3.0 |