3GPP TS 24.483 V19.1.0 (2025-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Mission Critical Services (MCS) Management Object (MO)

(Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

LTE, MCPTT, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	44

1	Scope	45

2	References	46

3	Definitions and abbreviations	47

3.1	Definitions	47

3.2	Abbreviations	47

3.3	Symbols	47

3.4	Management Object design and representation guidelines	48

4	MCPTT UE configuration MO	48

4.1	General	48

4.2	MCPTT UE configuration MO parameters	49

4.2.1	General	49

4.2.2	Node: <x>	49

4.2.3	/<x>/Name	49

4.2.4	/<x>/Ext/	50

4.2.5	/<x>/Common	50

4.2.6	/<x>/Common/PrivateCall	50

4.2.7	/<x>/Common/PrivateCall/MaxCallN10	50

4.2.8	/<x>/Common/MCPTTGroupCall	51

4.2.9	/<x>/Common/MCPTTGroupCall/MaxCallN4	51

4.2.10	/<x>/Common/MCPTTGroupCall/MaxTransmissionN5	51

4.2.11	/<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup	51

4.2.12	/<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>	51

4.2.13	/<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>/ MCPTTGroupID	52

4.2.14	/<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>/ MCPTTGroupPriorityHierarchy	52

4.2.15	/<x>/OnNetwork	52

4.2.16	/<x>/OnNetwork/RelayService	52

4.2.17	/<x>/OnNetwork/IPv6Preferred	53

4.2.18	/<x>/OnNetwork/RelayedMCPTTGroup	53

4.2.19	/<x>/OnNetwork/RelayedMCPTTGroup/<x>	53

4.2.20	/<x>/OnNetwork/RelayedMCPTTGroup/<x>/MCPTTGroupID	53

4.2.21	/<x>/OnNetwork/RelayedMCPTTGroup/<x>/RelayServiceCode	54

5	MCPTT user profile MO	54

5.1	General	54

5.2	MCPTT user profile MO parameters	58

5.2.1	General	58

5.2.2	Node: <x>	58

5.2.3	/<x>/Name	58

5.2.4	/<x>/Ext/	58

5.2.5	/<x>/<x>	59

5.2.6	/<x>/<x>/Common	59

5.2.7	/<x>/<x>/Common/MCPTTUserID	59

5.2.7A	/<x>/<x>/Common/MCPTTUserProfileIndex	59

5.2.7B	/<x>/<x>/Common/MCPTTUserProfileName	60

5.2.7C	/<x>/<x>/Common/PreSelectedIndication	60

5.2.8	/<x>/<x>/Common/UserAliases	60

5.2.8A	/<x>/<x>/Common/UserAliases/<x>	60

5.2.8B	/<x>/<x>/Common/UserAliases/<x>/UserAlias	61

5.2.9	/<x>/<x>/Common/AuthorisedAlias	61

5.2.10	/<x>/<x>/Common/ParticipantType	61

5.2.11	/<x>/<x>/Common/Organization	61

5.2.12	/<x>/<x>/Common/PrivateCall	62

5.2.13	/<x>/<x>/Common/PrivateCall/Authorised	62

5.2.14	/<x>/<x>/Common/PrivateCall/AuthorisedAny	62

5.2.15	/<x>/<x>/Common/PrivateCall/UserList	62

5.2.16	/<x>/<x>/Common/PrivateCall/UserList/<x>	62

5.2.16A	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry	63

5.2.17	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCPTTID	63

5.2.18	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/ DiscoveryGroupID	63

5.2.18A	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/ ApplicationLayerGroupID	63

5.2.19	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID	64

5.2.19A	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName	64

5.2.19B	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/ PrivateCallKMSURI	64

5.2.20	/<x>/<x>/Common/PrivateCall/ManualCommence	64

5.2.21	/<x>/<x>/Common/PrivateCall/AutoCommence	65

5.2.22	/<x>/<x>/Common/PrivateCall/AutoAnswer	65

5.2.23	/<x>/<x>/Common/PrivateCall/FailRestrict	65

5.2.24	/<x>/<x>/Common/PrivateCall/AllowedMediaProtection	65

5.2.25	/<x>/<x>/Common/PrivateCall/AllowedFloorControlProtection	66

5.2.26	/<x>/<x>/Common/PrivateCall/EmergencyCall	66

5.2.27	/<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised	66

5.2.28	/<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority	67

5.2.29	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCPTTPrivateRecipient	67

5.2.29A	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCPTTPrivateRecipient/Entry	67

5.2.29B	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCPTTPrivateRecipient/Entry/ID	67

5.2.29C	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCPTTPrivateRecipient/Entry/DiscoveryGroupID	68

5.2.29C1	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCPTTPrivateRecipient/Entry/ApplicationLayerGroupID	68

5.2.29D	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCPTTPrivateRecipient/Entry/UserInfoID	68

5.2.29E	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCPTTPrivateRecipient/Entry/DisplayName	68

5.2.29F	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCPTTPrivateRecipient/Entry/Usage	69

5.2.30	/<x>/<x>/Common/MCPTTGroupCall	69

5.2.31	/<x>/<x>/Common/MCPTTGroupCall/MaxSimultaneousCallsN6	69

5.2.32	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall	69

5.2.33	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/Enabled	70

5.2.34	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/ MCPTTGroupInitiation	70

5.2.34A	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/ MCPTTGroupInitiation/Entry	70

5.2.34B	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/ MCPTTGroupInitiation/Entry/GroupID	70

5.2.34C	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/ MCPTTGroupInitiation/Entry/DisplayName	71

5.2.34D	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/ MCPTTGroupInitiation/Entry/Usage	71

5.2.35	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/ CancelMCPTTGroup	71

5.2.36	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall	72

5.2.37	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/Authorised	72

5.2.38	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/Cancel	72

5.2.39	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/ MCPTTGroupInitiation	72

5.2.39A	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/ MCPTTGroupInitiation/Entry	73

5.2.39B	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/ MCPTTGroupInitiation/Entry/GroupID	73

5.2.39C	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/ MCPTTGroupInitiation/DisplayName	73

5.2.39D	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/ MCPTTGroupInitiation/Entry/Usage	73

5.2.40	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert	74

5.2.41	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Authorised	74

5.2.42	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Cancel	74

5.2.43	Void	74

5.2.43A	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry	74

5.2.43B	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/ID	75

5.2.43C	Void	75

5.2.43D	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/ DisplayName	75

5.2.43E	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/Usage	75

5.2.43F	/<x>/<x>/Common/MCPTTGroupCall/Priority	76

5.2.44	Void	76

5.2.45	/<x>/<x>/Common/MCPTTGroupBroadcast	76

5.2.46	/<x>/<x>/Common/MCPTTGroupBroadcast/Authorised	76

5.2.47	/<x>/<x>/Common/UserBroadcast	76

5.2.48	/<x>/<x>/Common/UserBroadcast/Authorised	77

5.2.48A	/<x>/<x>/OnNetwork	77

5.2.48B1	/<x>/<x>/OnNetwork/MCPTTGroupList	77

5.2.48B2	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>	77

5.2.48B3	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry	77

5.2.48B4	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ MCPTTGroupID	78

5.2.48B4A	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation	78

5.2.48B4A0	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>	78

5.2.48B4A1	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria	78

5.2.48B4A2	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>	79

5.2.48B4A3	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry	79

5.2.48B4A4	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea	79

5.2.48B4A5	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea	79

5.2.48B4A6	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>	80

5.2.48B4A7	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType	80

5.2.48B4A8	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	80

5.2.48B4A9	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	81

5.2.48B4A10	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea	81

5.2.48B4A11	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center	81

5.2.48B4A12	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/ PointCoordinateType	82

5.2.48B4A13	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/ PointCoordinateType/Longitude	82

5.2.48B4A14	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/ PointCoordinateType/Latitude	82

5.2.48B4A15	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Radius	83

5.2.48B4A16	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/OffsetAngle	83

5.2.48B4A17	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/IncludedAngle	83

5.2.48B4A18	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed	83

5.2.48B4A19	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MinimumSpeed	84

5.2.48B4A20	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MaximumSpeed	84

5.2.48B4A21	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading	84

5.2.48B4A22	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MinimumHeading	85

5.2.48B4A23	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MaximumHeading	85

5.2.48B4A24	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea	85

5.2.48B4A25	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea	86

5.2.48B4A26	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>	86

5.2.48B4A27	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType	86

5.2.48B4A28	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	87

5.2.48B4A29	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	87

5.2.48B4A30	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea	87

5.2.48B4A31	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center	88

5.2.48B4A32	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	88

5.2.48B4A33	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/ PointCoordinateType/Longitude	88

5.2.48B4A34	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/ PointCoordinateType/Latitude	89

5.2.48B4A35	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Radius	89

5.2.48B4A36	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/OffsetAngle	89

5.2.48B4A37	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/IncludedAngle	90

5.2.48B4A38	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed	90

5.2.48B4A39	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MinimumSpeed	90

5.2.48B4A40	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MaximumSpeed	91

5.2.48B4A41	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading	91

5.2.48B4A42	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MinimumHeading	91

5.2.48B4A43	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MaximumHeading	92

5.2.48B4A44	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases	92

5.2.48B4A45	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>	92

5.2.48B4A46	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry	92

5.2.48B4A47	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/ Entry/FunctionalAlias	93

5.2.48B4B	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation	93

5.2.48B4B0	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>	93

5.2.48B4B1	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria	93

5.2.48B4B2	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>	94

5.2.48B4B3	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry	94

5.2.48B4B4	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea	94

5.2.48B4B5	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea	94

5.2.48B4B6	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>	95

5.2.48B4B7	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType	95

5.2.48B4B8	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	95

5.2.48B4B9	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	96

5.2.48B4B10	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea	96

5.2.48B4B11	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center	96

5.2.48B4B12	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	97

5.2.48B4B13	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/ PointCoordinateType/Longitude	97

5.2.48B4B14	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/ PointCoordinateType/Latitude	97

5.2.48B4B15	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Radius	98

5.2.48B4B16	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/OffsetAngle	98

5.2.48B4B17	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/IncludedAngle	98

5.2.48B4B18	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed	98

5.2.48B4B19	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MinimumSpeed	99

5.2.48B4B20	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MaximumSpeed	99

5.2.48B4B21	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading	99

5.2.48B4B22	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MinimumHeading	100

5.2.48B4B23	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MaximumHeading	100

5.2.48B4B24	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea	100

5.2.48B4B25	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea	101

5.2.48B4B26	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>	101

5.2.48B4B27	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType	101

5.2.48B4B28	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	102

5.2.48B4B29	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	102

5.2.48B4B30	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea	102

5.2.48B4B31	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center	103

5.2.48B4B32	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	103

5.2.48B4B33	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	103

5.2.48B4B34	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	104

5.2.48B4B35	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Radius	104

5.2.48B4B36	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/OffsetAngle	104

5.2.48B4B37	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/IncludedAngle	105

5.2.48B4B38	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed	105

5.2.48B4B39	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MinimumSpeed	105

5.2.48B4B40	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MaximumSpeed	106

5.2.48B4B41	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading	106

5.2.48B4B42	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MinimumHeading	106

5.2.48B4B43	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MaximumHeading	107

5.2.48B4B44	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases	107

5.2.48B4B45	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>	107

5.2.48B4B46	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry	107

5.2.48B4B47	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/ Entry/FunctionalAlias	108

5.2.48B5	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/DisplayName	108

5.2.48B6	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet	108

5.2.48B7	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ RelativePresentationPriority	108

5.2.48B8	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/GMSServID	109

5.2.48B9	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ IDMSTokenEndPoint	109

5.2.48B10	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/ GroupKMSURI	109

5.2.48C1	/<x>/<x>/OnNetwork/ImplicitAffiliations	109

5.2.48C2	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>	109

5.2.48C3	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry	110

5.2.48C4	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCPTTGroupID	110

5.2.48C5	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName	110

5.2.48D	/<x>/<x>/OnNetwork/AllowedRegroup	110

5.2.48E	/<x>/<x>/OnNetwork/AllowedPresenceStatus	111

5.2.48F	/<x>/<x>/OnNetwork/AllowedPresence	111

5.2.48G	/<x>/<x>/OnNetwork/EnabledParticipation	111

5.2.48H	/<x>/<x>/OnNetwork/AllowedTransmission	111

5.2.48I	/<x>/<x>/OnNetwork/AllowedManualSwitch	112

5.2.48J	/<x>/<x>/OnNetwork/PrivateCall	112

5.2.48K	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert	112

5.2.48L	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry	112

5.2.48M	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID	113

5.2.48N	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ DisplayName	113

5.2.48O	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage	113

5.2.48P	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallBackRequest	113

5.2.48Q	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallBackCancelRequest	114

5.2.48R	/<x>/<x>/OnNetwork/PrivateCall/ AllowedRemoteInitiatedAmbientListening	114

5.2.48S	/<x>/<x>/OnNetwork/PrivateCall/ AllowedLocallyInitiatedAmbientListening	114

5.2.48T	/<x>/<x>/OnNetwork/PrivateCall/AllowedRequestFirstToAnswerCall	114

5.2.48T1	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallTransfer	115

5.2.48T2	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallTransferAny	115

5.2.48T3	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallForwardManualInput	115

5.2.48U1	/<x>/<x>/OnNetwork/RemoteGroupSelection	116

5.2.48U2	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>	116

5.2.48U3	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry	116

5.2.48U4	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCPTTID	116

5.2.48U5	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/ DisplayName	116

5.2.48V1	Void	117

5.2.48V2	Void	117

5.2.48V3	Void	117

5.2.48V4	Void	117

5.2.48V5	Void	117

5.2.48V6	Void	117

5.2.48V7	Void	117

5.2.48V8	Void	117

5.2.48V9	Void	117

5.2.48V10	Void	117

5.2.48V11	Void	117

5.2.48V12	Void	117

5.2.48V13	Void	117

5.2.48V14	Void	117

5.2.48V15	Void	117

5.2.48V16	Void	117

5.2.48V17	Void	117

5.2.48W1	/<x>/<x>/OnNetwork/AllowedRequestRemoteInitPrivateCall	117

5.2.48W2	/<x>/<x>/OnNetwork/AllowedRequestRemoteInitGroupCall	118

5.2.48W3	/<x>/<x>/OnNetwork/FunctionalAliasList	118

5.2.48W4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>	118

5.2.48W5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry	118

5.2.48W6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FunctionalAlias	119

5.2.48W6A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation	119

5.2.48W6A0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>	119

5.2.48W6A1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea	119

5.2.48W6A2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea	120

5.2.48W6A3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>	120

5.2.48W6A3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType	120

5.2.48W6A4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	121

5.2.48W6A5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	121

5.2.48W6A6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea	121

5.2.48W6A7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center	122

5.2.48W6A7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	122

5.2.48W6A8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	122

5.2.48W6A9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	123

5.2.48W6A10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ EnterSpecificArea/EllipsoidArcArea/Radius	123

5.2.48W6A11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ EnterSpecificArea/EllipsoidArcArea/OffsetAngle	123

5.2.48W6A12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/ Entry/LocationCriteriaForActivation/<x>/ EnterSpecificArea/EllipsoidArcArea/IncludedAngle	124

5.2.48W6A12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed	124

5.2.48W6A12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/ MinimumSpeed	124

5.2.48W6A12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/ MaximumSpeed	125

5.2.48W6A12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading	125

5.2.48W6A12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/ MinimumHeading	125

5.2.48W6A12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/ MaximumHeading	126

5.2.48W6A13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea	126

5.2.48W6A14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/PolygonArea	126

5.2.48W6A15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/PolygonArea/<x>	127

5.2.48W6A15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType	127

5.2.48W6A16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude	127

5.2.48W6A17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude	128

5.2.48W6A18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/EllipsoidArcArea	128

5.2.48W6A19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/EllipsoidArcArea/Center	128

5.2.48W6A19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	128

5.2.48W6A20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	129

5.2.48W6A21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	129

5.2.48W6A22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/EllipsoidArcArea/Radius	129

5.2.48W6A23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/EllipsoidArcArea/OffsetAngle	130

5.2.48W6A24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ ExitSpecificArea/EllipsoidArcArea/IncludedAngle	130

5.2.48W6A24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed	130

5.2.48W6A24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/ MinimumSpeed	131

5.2.48W6A24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/ MaximumSpeed	131

5.2.48W6A24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading	131

5.2.48W6A24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/ MinimumHeading	132

5.2.48W6A24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/ MaximumHeading	132

5.2.48W6B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation	132

5.2.48W6B0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>	132

5.2.48W6B1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea	133

5.2.48W6B2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea	133

5.2.48W6B3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>	133

5.2.48W6B3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType	134

5.2.48W6B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	134

5.2.48W6B5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	134

5.2.48W6B6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea	135

5.2.48W6B7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center	135

5.2.48W6B7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	135

5.2.48W6B8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	136

5.2.48W6B9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	136

5.2.48W6B10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Radius	136

5.2.48W6B11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/OffsetAngle	137

5.2.48W6B12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/IncludedAngle	137

5.2.48W6B12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed	137

5.2.48W6B12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/ MinimumSpeed	138

5.2.48W6B12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/ MaximumSpeed	138

5.2.48W6B12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading	138

5.2.48W6B12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/ MinimumHeading	139

5.2.48W6B12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/ MaximumHeading	139

5.2.48W6B13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea	139

5.2.48W6B14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea	140

5.2.48W6B15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>	140

5.2.48W6B15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType	140

5.2.48W6B16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	141

5.2.48W6B17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	141

5.2.48W6B18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea	141

5.2.48W6B19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center	142

5.2.48W6B19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	142

5.2.48W6B20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	142

5.2.48W6B21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	143

5.2.48W6B22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Radius	143

5.2.48W6B23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/OffsetAngle	143

5.2.48W6B24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/IncludedAngle	144

5.2.48W6B24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed	144

5.2.48W6B24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/ MinimumSpeed	144

5.2.48W6B24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/ MaximumSpeed	145

5.2.48W6B24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading	145

5.2.48W6B24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/ MinimumHeading	145

5.2.48W6B24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/ MaximumHeading	146

5.2.48W6C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ ManualDeactivationNotAllowedIfLocationCriteriaMet	146

5.2.48W7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ DisplayName	146

5.2.48W7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ MaxSimultaneousEmergencyGroupCalls	146

5.2.48W7B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FAsAllowedToCall	147

5.2.48W7C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FAsAllowedToCall/<x>	147

5.2.48W7D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FAsAllowedToCall/<x>/Entry	147

5.2.48W7E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FAsAllowedToCall/<x>/Entry/FunctionalAlias	147

5.2.48W8	/<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser	148

5.2.48W9	/<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser	148

5.2.48W10	/<x>/<x>/OnNetwork/AllowedLocationInfoWhenTalking	148

5.2.48W11	/<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding	148

5.2.48W12	/<x>/<x>/OnNetwork/MCPTTAdhocGroup	149

5.2.48W12A	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert	149

5.2.48W12A1	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/ Authorised	149

5.2.48W12A2	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/ Cancel	149

5.2.48W12A3	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/ AuthRecvParticipantInfo	150

5.2.48W12A4	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/ AuthSetupAdhocGroupCall	150

5.2.48W12B	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall	150

5.2.48W12B1	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/ Authorised	151

5.2.48W12B2	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/ AuthorisedParticipation	151

5.2.48W12B3	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/ AuthInitEmergencyCall	151

5.2.48W12B4	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/ AuthInitImminentPerilCall	151

5.2.48W12B5	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/ AuthRecvCallParticipantInfo	152

5.2.48W12B6	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/ AuthModifyCallParticipantInfo	152

5.2.48X	/<x>/<x>/OnNetwork/AuthorisedIncomingAny	152

5.2.48Y1	/<x>/<x>/OnNetwork/UserList	153

5.2.48Y2	/<x>/<x>/OnNetwork/UserList/<x>	153

5.2.48Y3	/<x>/<x>/OnNetwork/UserList/<x>/Entry	153

5.2.48Y4	/<x>/<x>/OnNetwork/UserList/<x>/Entry/MCPTTID	153

5.2.48Y5	/<x>/<x>/OnNetwork/UserList/<x>/Entry/PrivateCallKMSURI	153

5.2.48Z	/<x>/<x>/OnNetwork/AuthorisedReceiveNonAcknowledged	154

5.2.49	/<x>/<x>/OffNetwork	154

5.2.50	/<x>/<x>/OffNetwork/Authorised	154

5.2.51	/<x>/<x>/OffNetwork/MCPTTGroupInfo	154

5.2.52	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>	155

5.2.52A	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry	155

5.2.53	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/MCPTTGroupID	155

5.2.53A	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/DisplayName	155

5.2.53B	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/ RelativePresentationPriority	155

5.2.53C	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/ GMSServID	156

5.2.53D	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/ IDMSTokenEndPoint	156

5.2.53E	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/GroupKMSURI	156

5.2.54	/<x>/<x>/OffNetwork/AllowedListen	156

5.2.55	/<x>/<x>/OffNetwork/AllowedTransmission	157

5.2.56	/<x>/<x>/OffNetwork/EmergencyCallChange	157

5.2.57	/<x>/<x>/OffNetwork/ImminentPerilCallChange	157

5.2.58	/<x>/<x>/OffNetwork/UserInfoID	158

5.2.58A1	Void	159

5.2.58A2	Void	159

5.2.58A3	Void	159

5.2.58A4	Void	159

5.2.58A5	Void	159

5.2.58A6	Void	159

5.2.58A7	Void	159

5.2.58A8	Void	159

5.2.58A9	Void	159

5.2.58A10	Void	159

5.2.58A11	Void	159

5.2.58A12	Void	159

5.2.58A13	Void	159

5.2.58A14	Void	159

5.2.58A15	Void	159

5.2.58A16	Void	159

5.2.58A17	Void	159

5.2.59	/<x>/Status	159

6	MCS group configuration MO	160

6.1	General	160

6.2	MCS group configuration MO parameters	163

6.2.1	General	163

6.2.2	Node: <x>	163

6.2.3	/<x>/Name	164

6.2.4	/<x>/Ext	164

6.2.5	/<x>/<x>	164

6.2.6	/<x>/<x>/Common	164

6.2.7	/<x>/<x>/Common/MCPTTGroupID	165

6.2.8	/<x>/<x>/Common/MCPTTGroupAlias	165

6.2.9	/<x>/<x>/Common/MCPTTGroupMemberList	165

6.2.10	/<x>/<x>/Common/MCPTTGroupMemberList/<x>	165

6.2.11	/<x>/<x>/Common/MCPTTGroupMemberList/<x>/MCPTTID	166

6.2.12	/<x>/<x>/Common/MCPTTGroupMemberList/<x>/UserPriority	166

6.2.13	/<x>/<x>/Common/MCPTTGroupMemberList/<x>/ParticipantType	166

6.2.14	Void	166

6.2.15	/<x>/<x>/Common/MCPTTGroupOwner	166

6.2.16	/<x>/<x>/Common/PreferredVoiceCodec	167

6.2.17	/<x>/<x>/Common/MCPTTGroupLevel	167

6.2.18	/<x>/<x>/Common/UserLevel	167

6.2.19	/<x>/<x>/Common/AllowedEmergencyCall	168

6.2.20	/<x>/<x>/Common/AllowedImminentPerilCall	168

6.2.21	/<x>/<x>/Common/AllowedEmergencyAlert	168

6.2.22	/<x>/<x>/Common/MediaProtectionRequired	168

6.2.22A	/<x>/<x>/Common/PreconfiguredGroupUseOnly	169

6.2.23	/<x>/<x>/Common/FloorControlProtectionRequired	169

6.2.23A	/<x>/<x>/Common/MediaProtectionSecurityMaterial	169

6.2.23B	/<x>/<x>/Common/MCData	170

6.2.23C	/<x>/<x>/Common/MCData/MediaProtectionRequired	170

6.2.23D	/<x>/<x>/Common/MCData/TxControlProtectionRequired	170

6.2.23E	/<x>/<x>/Common/MCData/MediaProtectionSecurityMaterial	170

6.2.23F	/<x>/<x>/Common/MCData/AllowedSDS	171

6.2.23G	/<x>/<x>/Common/MCData/AllowedFD	171

6.2.23H	/<x>/<x>/Common/MCData/AllowedConvMgmt	171

6.2.23I	/<x>/<x>/Common/MCData/AllowedTxControl	171

6.2.23J	/<x>/<x>/Common/MCData/AllowedRxControl	172

6.2.23K	/<x>/<x>/Common/MCData/AllowedEnhSvc	172

6.2.23L	/<x>/<x>/Common/MCData/EnhSvcOpValues	172

6.2.23M	/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>	172

6.2.23N	/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/ EnhSvcOpValue	173

6.2.23NA	/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/ EnhSvcOpID	173

6.2.23O	/<x>/<x>/Common/MCData/AllowedTransmitDataInGroup	173

6.2.23P	/<x>/<x>/Common/MCData/MaxDataSingleRequest	173

6.2.23Q	/<x>/<x>/Common/MCData/MaxTimeSingleRequest	174

6.2.23R	/<x>/<x>/Common/MCVideo	174

6.2.23S	/<x>/<x>/Common/MCVideo/MediaProtectionRequired	174

6.2.23T	/<x>/<x>/Common/MCVideo/TxControlProtectionRequired	174

6.2.23U	/<x>/<x>/Common/MCVideo/MediaProtectionSecurityMaterial	175

6.2.23V	/<x>/<x>/Common/MCVideo/AllowedAudioCodecs	175

6.2.23W	/<x>/<x>/Common/MCVideo/AllowedAudioCodecs/<x>	175

6.2.23X	/<x>/<x>/Common/MCVideo/AllowedAudioCodecs/<x>/ AudioCodec	175

6.2.23Y	/<x>/<x>/Common/MCVideo/AllowedVideoCodecs	176

6.2.23Z	/<x>/<x>/Common/MCVideo/AllowedVideoCodecs/<x>	176

6.2.23AA	/<x>/<x>/Common/MCVideo/AllowedVideoCodecs/<x>/ VideoCodec	176

6.2.23AB	/<x>/<x>/Common/MCVideo/AllowedVideoResolutions	176

6.2.23AC	/<x>/<x>/Common/MCVideo/AllowedVideoResolutions/<x>	176

6.2.23AD	/<x>/<x>/Common/MCVideo/AllowedVideoResolutions/<x>/ VideoResolution	177

6.2.23AE	/<x>/<x>/Common/MCVideo/AllowedVideoFrameRates	177

6.2.23AF	/<x>/<x>/Common/MCVideo/AllowedVideoFrameRates/<x>	177

6.2.23AG	/<x>/<x>/Common/MCVideo/AllowedVideoFrameRates/<x>/ FrameRate	177

6.2.23AH	/<x>/<x>/Common/MCVideo/AllowedUrgentRealTimeMode	178

6.2.23AI	/<x>/<x>/Common/MCVideo/AllowedNonUrgentRealTimeMode	178

6.2.23AJ	/<x>/<x>/Common/MCVideo/AllowedNonRealTimeMode	178

6.2.23AK	/<x>/<x>/Common/MCVideo/ActiveNonRealTimeMode	178

6.2.23AL	/<x>/<x>/Common/MCVideo/MaxTransmitMembers	179

6.2.24	/<x>/<x>/OffNetwork	179

6.2.25	/<x>/<x>/OffNetwork/MCPTTGroupParameter	179

6.2.26	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>	179

6.2.27	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/ ProSeLayer2GroupID	180

6.2.28	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/ IPMulticastAddress	180

6.2.29	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/ RelayServiceCode	180

6.2.30	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/IPVersions	180

6.2.31	/<x>/<x>/OffNetwork/EmergencyCallCancel	181

6.2.32	/<x>/<x>/OffNetwork/ImminentPerilCallCancel	181

6.2.33	/<x>/<x>/OffNetwork/HangTime	181

6.2.34	/<x>/<x>/OffNetwork/MaxDuration	181

6.2.34A	/<x>/<x>/OffNetwork/QueueUsage	182

6.2.35	/<x>/<x>/OffNetwork/DefaultPPPP	182

6.2.36	/<x>/<x>/OffNetwork/DefaultPPPP/MCPTTGroupCallSignalling	182

6.2.37	/<x>/<x>/OffNetwork/DefaultPPPP/MCPTTGroupCallMedia	183

6.2.38	/<x>/<x>/OffNetwork/DefaultPPPP/ MCPTTEmergencyGroupCallSignalling	183

6.2.39	/<x>/<x>/OffNetwork/DefaultPPPP/ MCPTTEmergencyGroupCallMedia	183

6.2.40	/<x>/<x>/OffNetwork/DefaultPPPP/ MCPTTImminentPerilGroupCallSignalling	184

6.2.41	/<x>/<x>/OffNetwork/DefaultPPPP/ MCPTTImminentPerilGroupCallMedia	184

6.2.41A	/<x>/<x>/OffNetwork/DefaultPQI	184

6.2.41B	/<x>/<x>/OffNetwork/DefaultPQI/MCPTTGroupCallSignalling	184

6.2.41C	/<x>/<x>/OffNetwork/DefaultPQI/MCPTTGroupCallMedia	185

6.2.41D	/<x>/<x>/OffNetwork/DefaultPQI/ MCPTTEmergencyGroupCallSignalling	185

6.2.41E	/<x>/<x>/OffNetwork/DefaultPQI/ MCPTTEmergencyGroupCallMedia	185

6.2.41F	/<x>/<x>/OffNetwork/DefaultPQI/ MCPTTImminentPerilGroupCallSignalling	186

6.2.41G	/<x>/<x>/OffNetwork/DefaultPQI/ MCPTTImminentPerilGroupCallMedia	186

6.2.42	/<x>/<x>/OffNetwork/MCData	186

6.2.43	/<x>/<x>/OffNetwork/MCData/DefaultPPPP	187

6.2.44	/<x>/<x>/OffNetwork/MCData/DefaultPPPP/ MCDataGroupCallSignalling	187

6.2.45	/<x>/<x>/OffNetwork/MCData/DefaultPPPP/ MCDataGroupCallMedia	187

6.2.45A	/<x>/<x>/OffNetwork/MCData/DefaultPQI	187

6.2.45B	/<x>/<x>/OffNetwork/MCData/DefaultPQI/ MCDataGroupCallSignalling	188

6.2.45C	/<x>/<x>/OffNetwork/MCData/DefaultPQI/ MCDataGroupCallMedia	188

6.2.46	/<x>/<x>/OffNetwork/MCVideo	188

6.2.47	/<x>/<x>/OffNetwork/MCVideo/ArbitrationApproach	188

6.2.48	/<x>/<x>/OffNetwork/MCVideo/MaxTransmissions	189

6.2.49	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP	189

6.2.50	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/ MCVideoGroupCallSignalling	189

6.2.51	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/ MCVideoGroupCallMedia	190

6.2.52	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/ MCVideoEmergencyGroupCallSignalling	190

6.2.53	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/ MCVideoEmergencyGroupCallMedia	190

6.2.54	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/ MCVideoImminentPerilGroupCallSignalling	191

6.2.55	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/ MCVideoImminentPerilGroupCallMedia	191

6.2.55A	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI	191

6.2.55B	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/ MCVideoGroupCallSignalling	191

6.2.55C	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/ MCVideoGroupCallMedia	192

6.2.55D	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/ MCVideoEmergencyGroupCallSignalling	192

6.2.55E	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/ MCVideoEmergencyGroupCallMedia	192

6.2.55F	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/ MCVideoImminentPerilGroupCallSignalling	193

6.2.55G	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/ MCVideoImminentPerilGroupCallMedia	193

6.2.56	/<x>/<x>/OffNetwork/MCVideo/MaxDuration	193

6.2.57	/<x>/<x>/OffNetwork/MCVideo/EmergencyCallCancel	194

6.2.58	/<x>/<x>/OffNetwork/MCVideo/ImminentPerilCallCancel	194

7	MCPTT service configuration MO	194

7.1	General	194

7.2	MCPTT service configuration MO parameters	196

7.2.1	General	196

7.2.2	Node: <x>	196

7.2.3	/<x>/Name	196

7.2.4	/<x>/Ext/	196

7.2.5	/<x>/Common	197

7.2.6	/<x>/Common/BroadcastMCPTTGroupCall	197

7.2.7	/<x>/Common/BroadcastMCPTTGroupCall/ NumLevelGroupHierarchy	197

7.2.8	/<x>/Common/BroadcastMCPTTGroupCall/ NumLevelUserHierarchy	197

7.2.9	/<x>/Common/MinLengthAliasID	198

7.2.10	/<x>/OffNetwork	198

7.2.11	/<x>/OffNetwork/PrivateCall	198

7.2.12	/<x>/OffNetwork/PrivateCall/MaxDuration	198

7.2.13	/<x>/OffNetwork/PrivateCall/HangTime	199

7.2.14	/<x>/OffNetwork/PrivateCall/CancelTimeout	199

7.2.15	/<x>/OffNetwork/EmergencyCall	199

7.2.16	/<x>/OffNetwork/EmergencyCall/MCPTTGroupTimeout	199

7.2.17	/<x>/OffNetwork/NumLevelHierarchy	200

7.2.18	/<x>/OffNetwork/TransmitTimeout	200

7.2.19	/<x>/OffNetwork/TransmissionWarning	200

7.2.20	/<x>/OffNetwork/HangTimeWarning	200

7.2.21	/<x>/OffNetwork/DefaultPPPP	201

7.2.22	/<x>/OffNetwork/DefaultPPPP/MCPTTPrivateCallSignalling	201

7.2.23	/<x>/OffNetwork/DefaultPPPP/MCPTTPrivateCallMedia	201

7.2.24	/<x>/OffNetwork/DefaultPPPP/ MCPTTEmergencyPrivateCallSignalling	201

7.2.25	/<x>/OffNetwork/DefaultPPPP/MCPTTEmergencyPrivateCallMedia	202

7.2.25A	/<x>/OffNetwork/DefaultPQI	202

7.2.25B	/<x>/OffNetwork/DefaultPQI/MCPTTPrivateCallSignalling	202

7.2.25C	/<x>/OffNetwork/DefaultPQI/MCPTTPrivateCallMedia	202

7.2.25D	/<x>/OffNetwork/DefaultPQI/ MCPTTEmergencyPrivateCallSignalling	203

7.2.25E	/<x>/OffNetwork/DefaultPQI/MCPTTEmergencyPrivateCallMedia	203

7.2.26	/<x>/OffNetwork/LogMetadata	203

7.2.27	/<x>/OnNetwork	204

7.2.28	/<x>/OnNetwork/MCPTTAdhocGroup	204

7.2.29	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall	204

7.2.30	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/ AllowAdhocGroupCall	204

7.2.31	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/MaxNoParticipants	205

7.2.32	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/HangTime	205

7.2.33	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/ MaxDurationOfCall	205

7.2.34	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/ AllowAdhocMultiTalker	205

7.2.35	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/MaxNoTalkers	206

8	MCS UE initial configuration MO	206

8.1	General	206

8.2	MCS UE initial configuration MO parameters	211

8.2.1	General	211

8.2.2	Node: <x>	211

8.2.3	/<x>/Name	212

8.2.4	/<x>/Ext/	212

8.2.5	/<x>/DefaultUserProfile	212

8.2.6	/<x>/DefaultUserProfile/UserID	212

8.2.7	/<x>/DefaultUserProfile/UserProfileIndex	213

8.2.8	/<x>/OnNetwork	213

8.2.9	/<x>/OnNetwork/GMSURI	213

8.2.9A	/<x>/OnNetwork/GroupCreationXUI	213

8.2.9B	/<x>/OnNetwork/GMSXCAPRootURI	214

8.2.9C	/<x>/OnNetwork/CMSXCAPRootURI	214

8.2.10	/<x>/OnNetwork/Timers	214

8.2.11	/<x>/OnNetwork/Timers/T100	214

8.2.12	/<x>/OnNetwork/Timers/T101	214

8.2.13	/<x>/OnNetwork/Timers/T103	215

8.2.14	/<x>/OnNetwork/Timers/T104	215

8.2.15	/<x>/OnNetwork/Timers/T132	215

8.2.16	/<x>/OnNetwork/HPLMN	216

8.2.17	/<x>/OnNetwork/HPLMN/PLMN	216

8.2.18	/<x>/OnNetwork/HPLMN/Service	216

8.2.19	/<x>/OnNetwork/HPLMN/Service/MCPTTToConRef	216

8.2.20	/<x>/OnNetwork/HPLMN/Service/MCPTTToConRef/<x>	216

8.2.21	/<x>/OnNetwork/HPLMN/Service/MCPTTToConRef/<x>/ConRef	217

8.2.22	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef	217

8.2.23	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef/<x>	217

8.2.24	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef/ <x>/ConRef	217

8.2.25	/<x>/OnNetwork/HPLMN/Service/MCIDMToConRef	218

8.2.26	/<x>/OnNetwork/HPLMN/Service/MCIDMToConRef/<x>	218

8.2.27	/<x>/OnNetwork/HPLMN/Service/MCIDMToConRef/<x>/ConRef	218

8.2.27A1	/<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI	218

8.2.27A2	/<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI/<x>	218

8.2.27A3	/<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI/<x>/ RefSNSSAI	219

8.2.27A4	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI	219

8.2.27A5	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI/ <x>	219

8.2.27A6	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI/ <x>/RefSNSSAI	219

8.2.27A7	/<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI	220

8.2.27A8	/<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI/<x>	220

8.2.27A9	/<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI/<x>/ RefSNSSAI	220

8.2.27A10	/<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI	220

8.2.27A11	/<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI/<x>	221

8.2.27A12	/<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI/<x>/ RefSNSSAI	221

8.2.27A13	/<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI	221

8.2.27A14	/<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI/<x>	221

8.2.27A15	/<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI/<x>/ RefSNSSAI	221

8.2.28	/<x>/OnNetwork/HPLMN/VPLMN	222

8.2.28A	/<x>/OnNetwork/HPLMN/VPLMN/<x>	222

8.2.29	/<x>/OnNetwork/HPLMN/VPLMN/<x>/PLMN	222

8.2.30	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service	222

8.2.31	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef	223

8.2.32	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef/<x>	223

8.2.33	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCPTTToConRef/<x>/ConRef	223

8.2.34	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCCommonCoreToConRef	223

8.2.35	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCCommonCoreToConRef/<x>	224

8.2.36	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCCommonCoreToConRef/<x>/ConRef	224

8.2.37	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef	224

8.2.38	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef/<x>	224

8.2.39	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCIDMToConRef/<x>/ConRef	225

8.2.39A1	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCPTTRefSNSSAI	225

8.2.39A2	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCPTTRefSNSSAI/<x>	225

8.2.39A3	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCPTTRefSNSSAI/<x>/RefSNSSAI	225

8.2.39A4	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCCommonCoreRefSNSSAI	226

8.2.39A5	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCCommonCoreRefSNSSAI/<x>	226

8.2.39A6	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCCommonCoreRefSNSSAI/<x>/RefSNSSAI	226

8.2.39A7	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCIdMRefSNSSAI	226

8.2.39A8	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCIdMRefSNSSAI/<x>	227

8.2.39A9	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCIdMRefSNSSAI/<x>/RefSNSSAI	227

8.2.39A10	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCDataRefSNSSAI	227

8.2.39A11	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCDataRefSNSSAI/<x>	227

8.2.39A12	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCDataRefSNSSAI/<x>/RefSNSSAI	228

8.2.39A13	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCVideoRefSNSSAI	228

8.2.39A14	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCVideoRefSNSSAI/<x>	228

8.2.39A15	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/ MCVideoRefSNSSAI/<x>/RefSNSSAI	228

8.2.40	/<x>/OnNetwork/AppServerInfo	229

8.2.41	/<x>/OnNetwork/AppServerInfo/IDMSAuthEndpoint	229

8.2.41A	/<x>/OnNetwork/AppServerInfo/IDMSTokenEndpoint	229

8.2.41B	/<x>/OnNetwork/AppServerInfo/HTTPProxy	229

8.2.42	/<x>/OnNetwork/AppServerInfo/GMS	229

8.2.43	/<x>/OnNetwork/AppServerInfo/CMS	230

8.2.44	/<x>/OnNetwork/AppServerInfo/KMS	230

8.2.44A	/<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod	230

8.2.44B	/<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/Mutual	230

8.2.44C	/<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/X509	231

8.2.44D	/<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/Key	231

8.2.44D1	/<x>/OnNetwork/AppServerInfo/LMS	231

8.2.44E	/<x>/OnNetwork/IntegrityProtection	231

8.2.44F	/<x>/OnNetwork/ConfidentialityProtection	231

8.2.44G1	/<x>/OnNetwork/MCPTTServiceDetails	232

8.2.44G2	/<x>/OnNetwork/MCPTTServiceDetails/IPv6Required	232

8.2.44G3	/<x>/OnNetwork/MCPTTServiceDetails/ServerURI	232

8.2.44G3A	/<x>/OnNetwork/MCPTTServiceDetails/PDUSessionType	232

8.2.44G3A1	Void	233

8.2.44G3A2	Void	233

8.2.44G3A3	Void	233

8.2.44G3A4	Void	233

8.2.44G3A5	Void	233

8.2.44G3A6	Void	233

8.2.44G3A7	Void	233

8.2.44G3A8	Void	233

8.2.44G4	/<x>/OnNetwork/MCVideoServiceDetails	233

8.2.44G5	/<x>/OnNetwork/MCVideoServiceDetails/IPv6Required	233

8.2.44G6	/<x>/OnNetwork/MCVideoServiceDetails/ServerURI	233

8.2.44G6A	/<x>/OnNetwork/MCVideoServiceDetails/PDUSessionType	234

8.2.44G6A1	Void	234

8.2.44G6A2	Void	234

8.2.44G6A3	Void	234

8.2.44G6A4	Void	234

8.2.44G6A5	Void	234

8.2.44G6A6	Void	234

8.2.44G6A7	Void	234

8.2.44G6A8	Void	234

8.2.44G7	/<x>/OnNetwork/MCDataServiceDetails	234

8.2.44G8	/<x>/OnNetwork/MCDataServiceDetails/IPv6Required	234

8.2.44G9	/<x>/OnNetwork/MCDataServiceDetails/ServerURI	235

8.2.44G9A	/<x>/OnNetwork/MCDataServiceDetails/PDUSessionType	235

8.2.44G9A1	Void	236

8.2.44G9A2	Void	236

8.2.44G9A3	Void	236

8.2.44G9A4	Void	236

8.2.44G9A5	Void	236

8.2.44G9A6	Void	236

8.2.44G9A7	Void	236

8.2.44G9A8	Void	236

8.2.44G10	Void	236

8.2.44G11	Void	236

8.2.44G12	Void	236

8.2.44G13	Void	236

8.2.44G14	Void	236

8.2.44G15	Void	236

8.2.44G16	Void	236

8.2.44G17	Void	236

8.2.44G18	Void	236

8.2.44G19	Void	236

8.2.44G20	Void	236

8.2.44G21	Void	236

8.2.44G22	Void	236

8.2.44G23	Void	236

8.2.44G24	Void	236

8.2.44G25	Void	237

8.2.44G26	/<x>/OnNetwork/MCCommonCoreServiceDetails	237

8.2.44G27	/<x>/OnNetwork/MCCommonCoreServiceDetails/IPv6Required	237

8.2.44G28	/<x>/OnNetwork/MCCommonCoreServiceDetails/ServerURI	237

8.2.44G29	/<x>/OnNetwork/MCCommonCoreServiceDetails/ PDUSessionType	237

8.2.44G30	/<x>/OnNetwork/MCIdMServiceDetails	238

8.2.44G31	/<x>/OnNetwork/MCIdMServiceDetails/IPv6Required	238

8.2.44G32	/<x>/OnNetwork/MCIdMServiceDetails/ServerURI	238

8.2.44G33	/<x>/OnNetwork/MCIdMServiceDetails/PDUSessionType	238

8.2.44G34	/<x>/OnNetwork/MCIdMServiceDetails/IdmClientId	238

8.2.44H	/<x>/OnNetwork/DNInfo	239

8.2.44H1	/<x>/OnNetwork/DNInfo/<x>	239

8.2.44H2	/<x>/OnNetwork/DNInfo/<x>/DNN	239

8.2.44H3	/<x>/OnNetwork/DNInfo/<x>/DNAAAServer	239

8.2.44H4	/<x>/OnNetwork/DNInfo/<x>/PapParameters	240

8.2.44H5	/<x>/OnNetwork/DNInfo/<x>/PapParameters/UserName	240

8.2.44H6	/<x>/OnNetwork/DNInfo/<x>/PapParameters/PassWord	240

8.2.44H7	/<x>/OnNetwork/DNInfo/<x>/ChapParameters	240

8.2.44H8	/<x>/OnNetwork/DNInfo/<x>/ChapParameters/UserName	240

8.2.44H9	/<x>/OnNetwork/DNInfo/<x>/ChapParameters/PassWord	241

8.2.44H10	/<x>/OnNetwork/DNInfo/<x>/Credentials	241

8.2.44H11	/<x>/OnNetwork/DNInfo/<x>/Credentials/ID	241

8.2.44H12	/<x>/OnNetwork/SNSSAIInfo	241

8.2.44H13	/<x>/OnNetwork/SNSSAIInfo/<x>	241

8.2.44H14	/<x>/OnNetwork/SNSSAIInfo/<x>/NSSAAServer	242

8.2.44H15	/<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI	242

8.2.44H16	/<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI/Credentials	242

8.2.44H17	/<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI/Credentials/ID	242

8.2.45	/<x>/OffNetwork	242

8.2.46	/<x>/OffNetwork/Timers	243

8.2.47	/<x>/OffNetwork/Timers/TFG1	243

8.2.48	/<x>/OffNetwork/Timers/TFG2	243

8.2.49	/<x>/OffNetwork/Timers/TFG3	243

8.2.50	/<x>/OffNetwork/Timers/TFG4	244

8.2.51	/<x>/OffNetwork/Timers/TFG5	244

8.2.52	/<x>/OffNetwork/Timers/TFG11	244

8.2.53	/<x>/OffNetwork/Timers/TFG12	244

8.2.54	/<x>/OffNetwork/Timers/TFG13	245

8.2.54A	/<x>/OffNetwork/Timers/TFG14	245

8.2.55	/<x>/OffNetwork/Timers/TFP1	245

8.2.56	/<x>/OffNetwork/Timers/TFP2	246

8.2.57	/<x>/OffNetwork/Timers/TFP3	246

8.2.58	/<x>/OffNetwork/Timers/TFP4	246

8.2.59	/<x>/OffNetwork/Timers/TFP5	246

8.2.60	/<x>/OffNetwork/Timers/TFP6	247

8.2.61	/<x>/OffNetwork/Timers/TFP7	247

8.2.61A	/<x>/OffNetwork/Timers/TFP9	247

8.2.62	/<x>/OffNetwork/Timers/TFB1	247

8.2.63	/<x>/OffNetwork/Timers/TFB2	248

8.2.64	/<x>/OffNetwork/Timers/TFB3	248

8.2.65	/<x>/OffNetwork/Timers/T201	248

8.2.66	/<x>/OffNetwork/Timers/T203	248

8.2.67	/<x>/OffNetwork/Timers/T204	249

8.2.68	/<x>/OffNetwork/Timers/T205	249

8.2.69	Void	249

8.2.70	/<x>/OffNetwork/Timers/T233	249

8.2.71	/<x>/OffNetwork/Timers/TFE1	250

8.2.72	/<x>/OffNetwork/Timers/TFE2	250

8.2.73	/<x>/OffNetwork/Counters	250

8.2.74	/<x>/OffNetwork/Counters/CFP1	250

8.2.75	/<x>/OffNetwork/Counters/CFP3	251

8.2.76	/<x>/OffNetwork/Counters/CFP4	251

8.2.77	/<x>/OffNetwork/Counters/CFP6	251

8.2.78	/<x>/OffNetwork/Counters/CFG11	251

8.2.79	/<x>/OffNetwork/Counters/CFG12	252

8.2.80	/<x>/OffNetwork/Counters/C201	252

8.2.81	/<x>/OffNetwork/Counters/C204	252

8.2.82	/<x>/OffNetwork/Counters/C205	252

8.2.83	/<x>/Common	253

8.2.84	/<x>/Common/MCServiceUELabel	253

9	MCData UE configuration MO	253

9.1	General	253

9.2	MCData UE configuration MO parameters	255

9.2.1	General	255

9.2.2	Node: <x>	255

9.2.3	/<x>/Name	255

9.2.4	/<x>/Ext/	255

9.2.5	/<x>/Common	255

9.2.6	/<x>/Common/MCDataGroupTxns	256

9.2.7	/<x>/Common/MCDataGroupTxns/MaxSDSNc4	256

9.2.8	/<x>/Common/MCDataGroupTxns/SDSPresentationPriority	256

9.2.9	/<x>/Common/MCDataGroupTxns/SDSPresentationPriority/<x>	256

9.2.10	/<x>/Common/MCDataGroupTxns/SDSPresentationPriority/ <x>/MCDataGroupID	257

9.2.11	/<x>/Common/MCDataGroupTxns/SDSPresentationPriority/ <x>/MCDataGroupPriorityHierarchy	257

9.2.12	/<x>/Common/MCDataGroupTxns/MaxFDNc4	257

9.2.13	/<x>/Common/MCDataGroupTxns/FDPresentationPriority	257

9.2.14	/<x>/Common/MCDataGroupTxns/FDPresentationPriority/<x>	258

9.2.15	/<x>/Common/MCDataGroupTxns/FDPresentationPriority/ <x>/MCDataGroupID	258

9.2.16	/<x>/Common/MCDataGroupTxns/FDPresentationPriority/ <x>/MCDataGroupPriorityHierarchy	258

9.2.17	/<x>/Common/MCDataGroupTxns/ConversationPresentationPriority	258

9.2.18	/<x>/Common/MCDataGroupTxns/ ConversationPresentationPriority/<x>	259

9.2.19	/<x>/Common/MCDataGroupTxns/ ConversationPresentationPriority/<x>/MCDataGroupID	259

9.2.20	/<x>/Common/MCDataGroupTxns/ ConversationPresentationPriority/<x>/ MCDataGroupPriorityHierarchy	259

9.2.21	/<x>/Common/MCDataGroupTxns/MaxTCNc4	259

9.2.22	/<x>/Common/MCDataGroupTxns/MaxTCNc5	260

9.2.23	/<x>/Common/MCDataGroupTxns/DataPresentationPriority	260

9.2.24	/<x>/Common/MCDataGroupTxns/DataPresentationPriority/ <x>	260

9.2.25	/<x>/Common/MCDataGroupTxns/DataPresentationPriority/ <x>/MCDataGroupID	260

9.2.26	/<x>/Common/MCDataGroupTxns/DataPresentationPriority/ <x>/MCDataGroupPriorityHierarchy	261

9.2.27	/<x>/Common/MCDataGroupTxns/MaxRCNc4	261

9.2.28	/<x>/Common/MCDataGroupTxns/MaxRCNc5	261

9.2.28A	/<x>/Common/MCDataSDSCharset	261

9.2.29	/<x>/OnNetwork	262

9.2.30	/<x>/OnNetwork/RelayService	262

9.2.31	/<x>/OnNetwork/IPv6Preferred	262

9.2.32	/<x>/OnNetwork/RelayedMCDataGroup	262

9.2.33	/<x>/OnNetwork/RelayedMCDataGroup/<x>	263

9.2.34	/<x>/OnNetwork/RelayedMCDataGroup/<x>/MCDataGroupID	263

9.2.35	/<x>/OnNetwork/RelayedMCDataGroup/<x>/RelayServiceCode	263

10	MCData user profile MO	263

10.1	General	263

10.2	MCData user profile MO parameters	266

10.2.1	General	266

10.2.2	Node: <x>	266

10.2.3	/<x>/Name	267

10.2.4	/<x>/Ext/	267

10.2.5	/<x>/<x>	267

10.2.6	/<x>/<x>/Common	267

10.2.7	/<x>/<x>/Common/MCDataUserID	268

10.2.8	/<x>/<x>/Common/MCDataUserProfileIndex	268

10.2.9	/<x>/<x>/Common/MCDataUserProfileName	268

10.2.9A	/<x>/<x>/Common/MCDataUserIDKMSURI	268

10.2.10	/<x>/<x>/Common/PreSelectedIndication	269

10.2.11	/<x>/<x>/Common/UserAliases	269

10.2.12	/<x>/<x>/Common/UserAliases/<x>	269

10.2.13	/<x>/<x>/Common/UserAliases/<x>UserAlias	269

10.2.14	/<x>/<x>/Common/AuthorisedAlias	270

10.2.15	/<x>/<x>/Common/ParticipantType	270

10.2.16	/<x>/<x>/Common/Organization	270

10.2.16A	/<x>/<x>/Common/OnetoOne	270

10.2.16B	/<x>/<x>/Common/OnetoOne/UserList	271

10.2.16C	/<x>/<x>/Common/OnetoOne/UserList/<x>	271

10.2.16D	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry	271

10.2.16E	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/MCDataID	271

10.2.16F	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ DiscoveryGroupID	271

10.2.16F1	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ ApplicationLayerGroupID	272

10.2.16G	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/UserInfoID	272

10.2.16H	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ MCDataIDKMSURI	272

10.2.16I	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/DisplayName	272

10.2.16J	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation	273

10.2.16K	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ IPInformation/<x>	273

10.2.16L	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ IPInformation/<x>/Entry	273

10.2.16M	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ IPInformation/<x>/Entry/IPv4Information	273

10.2.16N	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ IPInformation/<x>/Entry/IPv6Information	274

10.2.16O	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ IPInformation/<x>/Entry/FQDN	274

10.2.17	/<x>/<x>/Common/FileDistribution	274

10.2.18	/<x>/<x>/Common/FileDistribution/CancelList	274

10.2.19	/<x>/<x>/Common/FileDistribution/CancelList/<x>	274

10.2.20	/<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry	275

10.2.21	/<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/MCDataID	275

10.2.21A	/<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/ MCDataIDKMSURI	275

10.2.22	/<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/ DisplayName	275

10.2.23	/<x>/<x>/Common/TxRxControl	276

10.2.24	/<x>/<x>/Common/TxRxControl/AuthorisedTransmit	276

10.2.25	/<x>/<x>/Common/TxRxControl/MaxData1To1	276

10.2.26	/<x>/<x>/Common/TxRxControl/MaxTime1To1	276

10.2.27	/<x>/<x>/Common/TxRxControl/ReleaseList	277

10.2.28	/<x>/<x>/Common/TxRxControl/ReleaseList/<x>	277

10.2.29	/<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry	277

10.2.30	/<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry/MCDataID	277

10.2.31	/<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry/ DisplayName	278

10.2.32	/<x>/<x>/Common/MCDataGroupBroadcast	278

10.2.33	/<x>/<x>/Common/MCDataGroupBroadcast/Authorised	278

10.2.34	/<x>/<x>/Common/UserBroadcast	278

10.2.35	/<x>/<x>/Common/UserBroadcast/Authorised	278

10.2.36	/<x>/<x>/Common/GroupEmergencyAlert	279

10.2.37	/<x>/<x>/Common/GroupEmergencyAlert/Entry	279

10.2.38	/<x>/<x>/Common/GroupEmergencyAlert/Entry/ID	279

10.2.39	/<x>/<x>/Common/GroupEmergencyAlert/Entry/DisplayName	279

10.2.40	/<x>/<x>/Common/GroupEmergencyAlert/Entry/Usage	280

10.2.41	/<x>/<x>/Common/AllowedActivateAlert	280

10.2.42	/<x>/<x>/Common/AllowedCancelAlert	280

10.2.42A	/<x>/<x>/Common/AllowedEmergencyPrivateCall	280

10.2.42B	/<x>/<x>/Common/AllowedCancelEmergencyPrivateCall	281

10.2.43	/<x>/<x>/OnNetwork	281

10.2.44	/<x>/<x>/OnNetwork/MCDataGroupList	281

10.2.45	/<x>/<x>/OnNetwork/MCDataGroupList/<x>	281

10.2.46	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry	282

10.2.47	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/MCDataGroupID	282

10.2.48	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/DisplayName	282

10.2.49	Void	282

10.2.50	Void	282

10.2.51	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/GMSServID	282

10.2.52	Void	283

10.2.53	Void	283

10.2.54	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ IdMSTokenEndPoint	283

10.2.54A	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ GroupKMSURI	283

10.2.55	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RelativePresentationPriority	283

10.2.55A	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation	284

10.2.55A0	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>	284

10.2.55A1	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/	284

10.2.55A2	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>	284

10.2.55A3	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry	285

10.2.55A4	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea	285

10.2.55A5	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea	285

10.2.55A6	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>	285

10.2.55A7	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType	286

10.2.55A8	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	286

10.2.55A9	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	286

10.2.55A10	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea	287

10.2.55A11	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center	287

10.2.55A12	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	287

10.2.55A13	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	288

10.2.55A14	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	288

10.2.55A15	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Radius	288

10.2.55A16	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/OffsetAngle	289

10.2.55A17	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/IncludedAngle	289

10.2.55A18	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed	289

10.2.55A19	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MinimumSpeed	289

10.2.55A20	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MaximumSpeed	290

10.2.55A21	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading	290

10.2.55A22	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MinimumHeading	290

10.2.55A23	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MaximumHeading	291

10.2.55A24	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea	291

10.2.55A25	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea	291

10.2.55A26	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>	292

10.2.55A27	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType	292

10.2.55A28	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	292

10.2.55A29	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	293

10.2.55A30	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea	293

10.2.55A31	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center	293

10.2.55A32	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	293

10.2.55A33	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	294

10.2.55A34	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	294

10.2.55A35	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Radius	294

10.2.55A36	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/OffsetAngle	295

10.2.55A37	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/IncludedAngle	295

10.2.55A38	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed	295

10.2.55A39	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MinimumSpeed	295

10.2.55A40	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MaximumSpeed	296

10.2.55A41	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading	296

10.2.55A42	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MinimumHeading	296

10.2.55A43	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MaximumHeading	297

10.2.55A44	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases	297

10.2.55A45	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>	297

10.2.55A46	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry	297

10.2.55A47	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/ Entry/FunctionalAlias	298

10.2.55B	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation	298

10.2.55B0	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>	298

10.2.55B1	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/	298

10.2.55B2	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>	299

10.2.55B3	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry	299

10.2.55B4	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea	299

10.2.55B5	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea	299

10.2.55B6	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>	300

10.2.55B7	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType	300

10.2.55B8	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	300

10.2.55B9	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	301

10.2.55B10	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea	301

10.2.55B11	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center	301

10.2.55B12	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	302

10.2.55B13	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	302

10.2.55B14	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	302

10.2.55B15	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Radius	303

10.2.55B16	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/OffsetAngle	303

10.2.55B17	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/IncludedAngle	303

10.2.55B18	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed	303

10.2.55B19	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MinimumSpeed	304

10.2.55B20	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MaximumSpeed	304

10.2.55B21	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading	304

10.2.55B22	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MinimumHeading	305

10.2.55B23	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MaximumHeading	305

10.2.55B24	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea	305

10.2.55B25	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea	306

10.2.55B26	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>	306

10.2.55B27	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType	306

10.2.55B28	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	307

10.2.55B29	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	307

10.2.55B30	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea	307

10.2.55B31	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center	308

10.2.55B32	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	308

10.2.55B33	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	308

10.2.55B34	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry /ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	309

10.2.55B35	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Radius	309

10.2.55B36	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/OffsetAngle	309

10.2.55B37	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/IncludedAngle	310

10.2.55B38	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed	310

10.2.55B39	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MinimumSpeed	310

10.2.55B40	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MaximumSpeed	311

10.2.55B41	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading	311

10.2.55B42	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MinimumHeading	311

10.2.55B43	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MaximumHeading	312

10.2.55B44	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases	312

10.2.55B45	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>	312

10.2.55B46	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry	312

10.2.55B47	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/ FunctionalAlias	313

10.2.55B48	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet	313

10.2.55C	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/ AllowStoreGroupCommInMsgstore	313

10.2.56	/<x>/<x>/OnNetwork/ImplicitAffiliations	313

10.2.57	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>	314

10.2.58	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry	314

10.2.59	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCDataGroupID	314

10.2.60	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName	314

10.2.61	/<x>/<x>/OnNetwork/PresenceStatus	314

10.2.62	/<x>/<x>/OnNetwork/PresenceStatus/<x>	315

10.2.63	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry	315

10.2.64	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/MCDataID	315

10.2.65	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/DisplayName	315

10.2.66	/<x>/<x>/OnNetwork/RemoteGroupChange	316

10.2.67	/<x>/<x>/>/OnNetwork/RemoteGroupChange/<x>	316

10.2.68	/<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry	316

10.2.69	/<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry/MCDataID	316

10.2.70	/<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry/DisplayName	316

10.2.71	/<x>/<x>/OnNetwork/MaxAffiliationsN2	317

10.2.72	/<x>/<x>/OnNetwork/ConversationManagement	317

10.2.73	/<x>/<x>/OnNetwork/ConversationManagement/GroupHangTime	317

10.2.74	/<x>/<x>/OnNetwork/ConversationManagement/ GroupHangTime/<x>	317

10.2.75	/<x>/<x>/OnNetwork/ConversationManagement/ GroupHangTime/<x>/Entry	317

10.2.76	/<x>/<x>/OnNetwork/ConversationManagement/ GroupHangTime/<x>/Entry/MCDataGroupID	318

10.2.77	/<x>/<x>/OnNetwork/ConversationManagement/ GroupHangTime/<x>/Entry/DisplayName	318

10.2.78	/<x>/<x>/OnNetwork/ConversationManagement/ GroupHangTime/<x>/Entry/HangTime	318

10.2.79	/<x>/<x>/OnNetwork/ConversationManagement/ DeliveredDisposition	319

10.2.80	/<x>/<x>/OnNetwork/ConversationManagement/ DeliveredDisposition/<x>	319

10.2.81	/<x>/<x>/OnNetwork/ConversationManagement/ DeliveredDisposition/<x>/Entry	319

10.2.82	/<x>/<x>/OnNetwork/ConversationManagement/ DeliveredDisposition/<x>/Entry/MCDataID	319

10.2.83	/<x>/<x>/OnNetwork/ConversationManagement/ DeliveredDisposition/<x>/Entry/DisplayName	320

10.2.84	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition	320

10.2.85	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/ <x>	320

10.2.86	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/ <x>/Entry	320

10.2.87	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/ <x>/Entry/MCDataID	321

10.2.88	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/ <x>/Entry/DisplayName	321

10.2.89	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert	321

10.2.90	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry	321

10.2.91	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/ID	321

10.2.92	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/ DisplayName	322

10.2.93	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/Usage	322

10.2.94	/<x>/<x>/OnNetwork/AllowedRegroup	322

10.2.95	/<x>/<x>/OnNetwork/AllowedPresenceStatus	322

10.2.96	/<x>/<x>/OnNetwork/AllowedPresence	323

10.2.97	/<x>/<x>/OnNetwork/AllowedManualSwitch	323

10.2.97A	/<x>/<x>/OnNetwork/MCDataContentServerURI	323

10.2.97B	/<x>/<x>/OnNetwork/FunctionalAliasList	324

10.2.97B1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>	324

10.2.97B2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry	324

10.2.97B3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FunctionalAlias	324

10.2.97B3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/ Entry/DisplayName	324

10.2.97B3B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation	325

10.2.97B3B0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>	325

10.2.97B3B1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea	325

10.2.97B3B2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea	325

10.2.97B3B3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>	326

10.2.97B3B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType	326

10.2.97B3B5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	326

10.2.97B3B6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	327

10.2.97B3B7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea	327

10.2.97B3B8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center	327

10.2.97B3B9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	327

10.2.97B3B10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	328

10.2.97B3B11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	328

10.2.97B3B12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Radius	328

10.2.97B3B13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/OffsetAngle	329

10.2.97B3B14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/IncludedAngle	329

10.2.97B3B15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed	329

10.2.97B3B16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/ MinimumSpeed	330

10.2.97B3B17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/ MaximumSpeed	330

10.2.97B3B18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading	330

10.2.97B3B19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/ MinimumHeading	331

10.2.97B3B20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/ MaximumHeading	331

10.2.97B3B21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea	331

10.2.97B3B22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/ Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/ PolygonArea	332

10.2.97B3B23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ PolygonArea/<x>	332

10.2.97B3B24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType	332

10.2.97B3B25	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	332

10.2.97B3B26	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	333

10.2.97B3B27	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea	333

10.2.97B3B28	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center	333

10.2.97B3B29	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	334

10.2.97B3B30	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	334

10.2.97B3B31	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	334

10.2.97B3B32	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Radius	335

10.2.97B3B33	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/OffsetAngle	335

10.2.97B3B34	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/IncludedAngle	335

10.2.97B3B35	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed	336

10.2.97B3B36	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/ MinimumSpeed	336

10.2.97B3B37	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/ MaximumSpeed	336

10.2.97B3B38	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading	336

10.2.97B3B39	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/ MinimumHeading	337

10.2.97B3B40	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/ MaximumHeading	337

10.2.97B3C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation	337

10.2.97B3C0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>	338

10.2.97B3C1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea	338

10.2.97B3C2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea	338

10.2.97B3C3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>	338

10.2.97B3C4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType	339

10.2.97B3C5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	339

10.2.97B3C6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	339

10.2.97B3C7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea	340

10.2.97B3C8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center	340

10.2.97B3C9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	340

10.2.97B3C10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	341

10.2.97B3C11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	341

10.2.97B3C12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Radius	341

10.2.97B3C13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/OffsetAngle	342

10.2.97B3C14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/IncludedAngle	342

10.2.97B3C15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed	342

10.2.97B3C16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/ MinimumSpeed	343

10.2.97B3C17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/ MaximumSpeed	343

10.2.97B3C18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading	343

10.2.97B3C19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/ MinimumHeading	344

10.2.97B3C20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/ MaximumHeading	344

10.2.97B3C21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea	344

10.2.97B3C22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea	345

10.2.97B3C23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>	345

10.2.97B3C24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType	345

10.2.97B3C25	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	346

10.2.97B3C26	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	346

10.2.97B3C27	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea	346

10.2.97B3C28	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center	347

10.2.97B3C29	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	347

10.2.97B3C30	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	347

10.2.97B3C31	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	348

10.2.97B3C32	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Radius	348

10.2.97B3C33	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/OffsetAngle	348

10.2.97B3C34	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/IncludedAngle	349

10.2.97B3C35	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed	349

10.2.97B3C36	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/ MinimumSpeed	349

10.2.97B3C37	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/ MaximumSpeed	350

10.2.97B3C38	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading	350

10.2.97B3C39	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/ MinimumHeading	350

10.2.97B3C40	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/ MaximumHeading	351

10.2.97B3D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ ManualDeactivationNotAllowedIfLocationCriteriaMet	351

10.2.97B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ MaxSimultaneousEmergencyGroupCalls	351

10.2.97C	/<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser	351

10.2.97D	/<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser	352

10.2.97E	/<x>/<x>/OnNetwork/MessageStoreHostname	352

10.2.97F	/<x>/<x>/OnNetwork/AuthorisedIncomingAny	352

10.2.97G	/<x>/<x>/OnNetwork/IncomingUserList	352

10.2.97G1	/<x>/<x>/OnNetwork/IncomingUserList/<x>	353

10.2.97G2	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry	353

10.2.97G3	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCDataID	353

10.2.97G4	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/ MCDataIDKMSURI	353

10.2.97H	/<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding	354

10.2.97I	/<x>/<x>/OnNetwork/AllowStoreCommsInMsgstore	354

10.2.97J	/<x>/<x>/OnNetwork/AllowStorePrivateCommsInMsgstore	354

10.2.97K	/<x>/<x>/OnNetwork/MCDataAdhocGroup	355

10.2.97K1	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert	355

10.2.97K2	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/ Authorised	355

10.2.97K3	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/ Cancel	355

10.2.97K4	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/ AuthRecvParticipantInfo	356

10.2.97K5	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/ AuthSetupAdhocGroupComn	356

10.2.97K6	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn	356

10.2.97K7	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/ Authorised	356

10.2.97K8	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/ AuthorisedParticipation	357

10.2.97K9	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/ AuthInitEmergencyComn	357

10.2.97K10	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/ AuthInitImminentPerilComn	357

10.2.97K11	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/ AuthRecvComnParticipantInfo	358

10.2.97K12	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/ AuthModifyComnParticipantInfo	358

10.2.98	/<x>/<x>/OffNetwork	358

10.2.99	/<x>/<x>/OffNetwork/Authorised	358

10.2.100	/<x>/<x>/OffNetwork/MCDataGroupList	359

10.2.101	/<x>/<x>/OffNetwork/MCDataGroupList/<x>	359

10.2.102	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry	359

10.2.103	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/MCDataGroupID	359

10.2.104	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/DisplayName	360

10.2.105	Void	360

10.2.106	Void	360

10.2.107	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/GMSServID	360

10.2.108	Void	360

10.2.109	Void	360

10.2.110	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/ IdMSTokenEndPoint	360

10.2.110A	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/ GroupKMSURI	360

10.2.111	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/ RelativePresentationPriority	361

10.2.112	/<x>/<x>/OffNetwork/UserInfoID	361

10.2.113	Void	362

10.2.114	Void	362

10.2.115	Void	362

10.2.116	Void	362

10.2.117	Void	362

10.2.118	Void	362

10.2.119	Void	362

10.2.120	Void	362

10.2.121	/<x>/Status	362

11	MCData service configuration MO	362

11.1	General	362

11.2	MCData service configuration MO parameters	363

11.2.1	General	363

11.2.2	Node: <x>	363

11.2.3	/<x>/Name	363

11.2.4	/<x>/Ext/	364

11.2.5	/<x>/Common	364

11.2.6	/<x>/Common/TxRxControl	364

11.2.7	/<x>/Common/TxRxControl/TimeTempDataWaiting	364

11.2.8	/<x>/Common/TxRxControl/TimePeriodicAnnouncement	365

11.2.9	/<x>/OffNetwork	365

11.2.10	/<x>/OffNetwork/DefaultPPPP	365

11.2.11	/<x>/OffNetwork/DefaultPPPP/MCDataOneToOneCallSignalling	365

11.2.12	/<x>/OffNetwork/DefaultPPPP/MCDataOneToOneCallMedia	366

11.2.12A	 /<x>/OffNetwork/DefaultPQI	366

11.2.12B	/<x>/OffNetwork/DefaultPQI/MCDataOneToOneCallSignalling	366

11.2.12C	/<x>/OffNetwork/DefaultPQI/MCDataOneToOneCallMedia	366

11.2.13	/<x>/OnNetwork	367

11.2.14	/<x>/OnNetwork/NotificationServers	367

11.2.15	/<x>/OnNetwork/NotificationServers/<x>	367

11.2.16	/<x>/OnNetwork/NotificationServers/<x>/NotificationServer	367

11.2.17	/<x>/OnNetwork/MCDataAdhocGroup	367

11.2.18	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn	368

11.2.19	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn/ AllowAdhocGroupComn	368

11.2.20	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn/ MaxNoParticipants	368

11.2.21	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn/HangTime	368

11.2.22	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn/ MaxDurationOfComn	369

12	MCVideo UE configuration MO	369

12.1	General	369

12.2	MCVideo UE configuration MO parameters	370

12.2.1	General	370

12.2.2	Node: <x>	370

12.2.3	/<x>/Name	370

12.2.4	/<x>/Ext/	371

12.2.5	/<x>/Common	371

12.2.5A	/<x>/Common/PrivateCall	371

12.2.5B	/<x>/Common/PrivateCall/MaxCallNc10	371

12.2.6	/<x>/Common/MCVideoGroupCall	372

12.2.7	/<x>/Common/MCVideoGroupCall/MaxCallNc4	372

12.2.8	/<x>/Common/MCVideoGroupCall/MaxTransmissionNc5	372

12.2.9	/<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup	372

12.2.10	/<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>	372

12.2.11	/<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>/ MCVideoGroupID	373

12.2.12	/<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>/ MCVideoGroupPriorityHierarchy	373

12.2.13	/<x>/OnNetwork	373

12.2.14	/<x>/OnNetwork/RelayService	373

12.2.15	/<x>/OnNetwork/IPv6Preferred	374

12.2.16	/<x>/OnNetwork/RelayedMCVideoGroup	374

12.2.17	/<x>/OnNetwork/RelayedMCVideoGroup/<x>	374

12.2.18	/<x>/OnNetwork/RelayedMCVideoGroup/<x>/MCVideoGroupID	374

12.2.19	/<x>/OnNetwork/RelayedMCVideoGroup/<x>/RelayServiceCode	375

13	MCVideo user profile MO	375

13.1	General	375

13.2	MCVideo user profile MO parameters	378

13.2.1	General	378

13.2.2	Node: <x>	378

13.2.3	/<x>/Name	378

13.2.4	/<x>/Ext/	379

13.2.5	/<x>/<x>	379

13.2.6	/<x>/<x>/Common	379

13.2.7	/<x>/<x>/Common/MCVideoUserID	379

13.2.8	/<x>/<x>/Common/MCVideoUserProfileIndex	380

13.2.9	/<x>/<x>/Common/MCVideoUserProfileName	380

13.2.10	/<x>/<x>/Common/PreSelectedIndication	380

13.2.11	/<x>/<x>/Common/UserAliases	380

13.2.12	/<x>/<x>/Common/UserAliases/<x>	381

13.2.13	/<x>/<x>/Common/UserAliases/<x>/UserAlias	381

13.2.14	Void	381

13.2.15	/<x>/<x>/Common/ParticipantType	381

13.2.16	/<x>/<x>/Common/Organization	381

13.2.17	/<x>/<x>/Common/MCVideoGroupBroadcast	381

13.2.18	/<x>/<x>/Common/MCVideoGroupBroadcast/Authorised	382

13.2.19	/<x>/<x>/Common/UserBroadcast	382

13.2.20	/<x>/<x>/Common/UserBroadcast/Authorised	382

13.2.21	Void	383

13.2.22	Void	383

13.2.23	Void	383

13.2.24	Void	383

13.2.25	Void	383

13.2.26	Void	383

13.2.27	Void	383

13.2.28	Void	383

13.2.29	/<x>/<x>/Common/AllowedActivateAlert	383

13.2.30	/<x>/<x>/Common/AllowedCancelAlert	383

13.2.31	Void	384

13.2.32	Void	384

13.2.33	Void	384

13.2.34	Void	384

13.2.35	Void	384

13.2.36	Void	384

13.2.37	Void	384

13.2.38	Void	384

13.2.38A	/<x>/<x>/Common/MCVideoGroupCall	384

13.2.38A1	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert	384

13.2.38A2	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/ Authorised	384

13.2.38A3	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Cancel	385

13.2.38A4	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry	385

13.2.38A5	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/ID	385

13.2.38A6	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/ Entry/DisplayName	385

13.2.38A7	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/ Entry/Usage	385

13.2.38B	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall	386

13.2.38C	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/Enabled	386

13.2.38D	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/ CancelMCVideoGroup	386

13.2.38D1	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/ MCVideoGroupInitiation	387

13.2.38D2	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/ MCVideoGroupInitiation/Entry	387

13.2.38D3	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/ MCVideoGroupInitiation/Entry/GroupID	387

13.2.38D4	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/ MCVideoGroupInitiation/Entry/DisplayName	387

13.2.38D5	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/ MCVideoGroupInitiation/Entry/Usage	388

13.2.38E	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall	388

13.2.38F	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/ Authorised	388

13.2.38G	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/Cancel	389

13.2.38G1	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/ MCVideoGroupInitiation	389

13.2.38G2	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/ MCVideoGroupInitiation/Entry	389

13.2.38G3	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/ MCVideoGroupInitiation/Entry/GroupID	389

13.2.38G4	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/ MCVideoGroupInitiation/DisplayName	390

13.2.38G5	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/ MCVideoGroupInitiation/Entry/Usage	390

13.2.38G6	/<x>/<x>/Common/MCVideoGroupCall/Priority	390

13.2.38G7	/<x>/<x>/Common/MCVideoGroupCall/MaxSimultaneousCallsN6	391

13.2.38H	/<x>/<x>/Common/PrivateCall	391

13.2.38H1	Void	391

13.2.38H2	Void	391

13.2.38H3	Void	391

13.2.38H4	Void	391

13.2.38H5	Void	391

13.2.38H6	Void	391

13.2.38I	/<x>/<x>/Common/PrivateCall/Authorised	391

13.2.38I1	/<x>/<x>/Common/PrivateCall/AuthorisedAny	391

13.2.38I2	/<x>/<x>/Common/PrivateCall/UserList	392

13.2.38I3	/<x>/<x>/Common/PrivateCall/UserList/<x>	392

13.2.38I4	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry	392

13.2.38I5	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID	392

13.2.38I6	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/ DiscoveryGroupID	393

13.2.38I6A	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/ ApplicationLayerGroupID	393

13.2.38I7	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID	393

13.2.38I8	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName	393

13.2.38I9	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/ PrivateCallKMSURI	394

13.2.38J	/<x>/<x>/Common/PrivateCall/ManualCommence	394

13.2.38K	/<x>/<x>/Common/PrivateCall/AutoCommence	394

13.2.38L	/<x>/<x>/Common/PrivateCall/FailRestrict	394

13.2.38M	Void	395

13.2.38N	/<x>/<x>/Common/PrivateCall/AllowedMediaProtection	395

13.2.38O	/<x>/<x>/Common/PrivateCall/EmergencyCall	395

13.2.38P	/<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised	395

13.2.38Q	/<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority	396

13.2.38R	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCVideoPrivateRecipient	396

13.2.38S	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCVideoPrivateRecipient/Entry	396

13.2.38T	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCVideoPrivateRecipient/Entry/ID	396

13.2.38U	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCVideoPrivateRecipient/Entry/DiscoveryGroupID	397

13.2.38U1	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCVideoPrivateRecipient/Entry/ApplicationLayerGroupID	397

13.2.38V	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCVideoPrivateRecipient/Entry/UserInfoID	397

13.2.38W	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCVideoPrivateRecipient/Entry/DisplayName	397

13.2.38X	/<x>/<x>/Common/PrivateCall/EmergencyCall/ MCVideoPrivateRecipient/Entry/Usage	398

13.2.38Y	/<x>/<x>/Common/PrivateCall/AutoAnswer	398

13.2.39	/<x>/<x>/OnNetwork	398

13.2.40	/<x>/<x>/OnNetwork/MCVideoGroupList	399

13.2.41	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>	399

13.2.42	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry	399

13.2.43	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ MCVideoGroupID	399

13.2.43A	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation	400

13.2.43A0	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>	400

13.2.43A1	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria	400

13.2.43A2	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>	400

13.2.43A3	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry	401

13.2.43A4	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea	401

13.2.43A5	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea	401

13.2.43A6	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>	401

13.2.43A7	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType	402

13.2.43A8	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	402

13.2.43A9	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	402

13.2.43A10	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea	403

13.2.43A11	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center	403

13.2.43A12	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	403

13.2.43A13	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	404

13.2.43A14	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	404

13.2.43A15	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Radius	404

13.2.43A16	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/OffsetAngle	405

13.2.43A17	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/IncludedAngle	405

13.2.43A18	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed	405

13.2.43A19	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MinimumSpeed	405

13.2.43A20	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MaximumSpeed	406

13.2.43A21	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading	406

13.2.43A22	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MinimumHeading	406

13.2.43A23	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MaximumHeading	407

13.2.43A24	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea	407

13.2.43A25	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea	407

13.2.43A26	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>	408

13.2.43A27	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType	408

13.2.43A28	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	408

13.2.43A29	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	409

13.2.43A30	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea	409

13.2.43A31	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center	409

13.2.43A32	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	409

13.2.43A33	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	410

13.2.43A34	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	410

13.2.43A35	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Radius	410

13.2.43A36	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/OffsetAngle	411

13.2.43A37	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/IncludedAngle	411

13.2.43A38	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed	411

13.2.43A39	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MinimumSpeed	411

13.2.43A40	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MaximumSpeed	412

13.2.43A41	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading	412

13.2.43A42	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MinimumHeading	412

13.2.43A43	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MaximumHeading	413

13.2.43A44	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases	413

13.2.43A45	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>	413

13.2.43A46	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry	413

13.2.43A47	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/ FunctionalAlias	414

13.2.43B	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation	414

13.2.43B0	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>	414

13.2.43B1	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria	414

13.2.43B2	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>	415

13.2.43B3	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry	415

13.2.43B4	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea	415

13.2.43B5	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea	415

13.2.43B6	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>	416

13.2.43B7	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType	416

13.2.43B8	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	416

13.2.43B9	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	417

13.2.43B10	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea	417

13.2.43B11	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center	417

13.2.43B12	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	418

13.2.43B13	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	418

13.2.43B14	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	418

13.2.43B15	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/Radius	419

13.2.43B16	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/OffsetAngle	419

13.2.43B17	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/EllipsoidArcArea/IncludedAngle	419

13.2.43B18	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed	419

13.2.43B19	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MinimumSpeed	420

13.2.43B20	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Speed/MaximumSpeed	420

13.2.43B21	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading	420

13.2.43B22	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MinimumHeading	421

13.2.43B23	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ EnterSpecificArea/Heading/MaximumHeading	421

13.2.43B24	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea	421

13.2.43B25	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea	422

13.2.43B26	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>	422

13.2.43B27	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType	422

13.2.43B28	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Longitude	423

13.2.43B29	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/ Latitude	423

13.2.43B30	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea	423

13.2.43B31	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center	424

13.2.43B32	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType	424

13.2.43B33	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Longitude	424

13.2.43B34	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/ Latitude	425

13.2.43B35	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/Radius	425

13.2.43B36	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/OffsetAngle	425

13.2.43B37	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/EllipsoidArcArea/IncludedAngle	426

13.2.43B38	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed	426

13.2.43B39	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MinimumSpeed	426

13.2.43B40	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Speed/MaximumSpeed	427

13.2.43B41	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading	427

13.2.43B42	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MinimumHeading	427

13.2.43B43	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ ExitSpecificArea/Heading/MaximumHeading	428

13.2.43B44	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases	428

13.2.43B45	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>	428

13.2.43B46	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry	428

13.2.43B47	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/ FunctionalAlias	429

13.2.43C	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet	429

13.2.44	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/DisplayName	429

13.2.45	Void	429

13.2.46	Void	429

13.2.47	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/GMSServID	429

13.2.48	Void	430

13.2.49	Void	430

13.2.50	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ IdMSTokenEndPoint	430

13.2.50A	Void	430

13.2.50B	Void	430

13.2.50C	Void	430

13.2.50D	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ GroupKMSURI	430

13.2.51	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/ RelativePresentationPriority	430

13.2.52	/<x>/<x>/OnNetwork/ImplicitAffiliations	431

13.2.53	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>	431

13.2.54	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry	431

13.2.55	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCVideoGroupID	431

13.2.56	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName	431

13.2.57	/<x>/<x>/OnNetwork/PresenceStatus	432

13.2.58	/<x>/<x>/OnNetwork/PresenceStatus/<x>	432

13.2.59	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry	432

13.2.60	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/MCVideoGroupID	432

13.2.61	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/DisplayName	432

13.2.62	Void	433

13.2.63	Void	433

13.2.64	Void	433

13.2.65	Void	433

13.2.66	Void	433

13.2.67	/<x>/<x>/OnNetwork/MaxAffiliationsN2	433

13.2.68	/<x>/<x>/OnNetwork/AllowedRegroup	433

13.2.69	/<x>/<x>/OnNetwork/AllowedPresenceStatus	433

13.2.70	/<x>/<x>/OnNetwork/AllowedPresence	434

13.2.71	/<x>/<x>/OnNetwork/AllowedManualSwitch	434

13.2.72	Void	434

13.2.73	/<x>/<x>/OnNetwork/MaxSimultaneousVideoStreams	434

13.2.74	/<x>/<x>/OnNetwork/MaxSimultaneousVideoStreams/MaxStreams	434

13.2.75	Void	435

13.2.76	Void	435

13.2.77	Void	435

13.2.78	Void	435

13.2.79	Void	435

13.2.80	Void	435

13.2.81	Void	435

13.2.82	Void	435

13.2.83	Void	435

13.2.84	Void	435

13.2.85	Void	435

13.2.86	Void	435

13.2.87	Void	435

13.2.87A	/<x>/<x>/OnNetwork/EnabledParticipation	435

13.2.87A1	/<x>/<x>/OnNetwork/AllowedRemoteInitiatedAmbientViewing	435

13.2.87A2	/<x>/<x>/OnNetwork/AllowedLocallyInitiatedAmbientViewing	436

13.2.87A3	/<x>/<x>/OnNetwork/FunctionalAliasList	436

13.2.87A4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>	436

13.2.87A5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry	436

13.2.87A6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FunctionalAlias	437

13.2.87A6A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation	437

13.2.87A6A0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>	437

13.2.87A6A1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea	437

13.2.87A6A2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea	438

13.2.87A6A3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>	438

13.2.87A6A3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType	438

13.2.87A6A4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	438

13.2.87A6A5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	439

13.2.87A6A6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea	439

13.2.87A6A7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center	439

13.2.87A6A7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	440

13.2.87A6A8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	440

13.2.87A6A9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	440

13.2.87A6A10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Radius	441

13.2.87A6A11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/OffsetAngle	441

13.2.87A6A12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/ EllipsoidArcArea/IncludedAngle	441

13.2.87A6A12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed	442

13.2.87A6A12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/ MinimumSpeed	442

13.2.87A6A12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/ MaximumSpeed	442

13.2.87A6A12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading	442

13.2.87A6A12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/ MinimumHeading	443

13.2.87A6A12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/ MaximumHeading	443

13.2.87A6A13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea	443

13.2.87A6A14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea	444

13.2.87A6A15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/ Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/ PolygonArea/<x>	444

13.2.87A6A15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/ <x>/PointCoordinateType	444

13.2.87A6A16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/ <x>/PointCoordinateType/Longitude	444

13.2.87A6A17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/ <x>/PointCoordinateType/Latitude	445

13.2.87A6A18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea	445

13.2.87A6A19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center	445

13.2.87A6A19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	446

13.2.87A6A20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	446

13.2.87A6A21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	446

13.2.87A6A22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Radius	447

13.2.87A6A23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/OffsetAngle	447

13.2.87A6A24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/ EllipsoidArcArea/IncludedAngle	447

13.2.87A6A24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed	448

13.2.87A6A24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/ MinimumSpeed	448

13.2.87A6A24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/ MaximumSpeed	448

13.2.87A6A24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading	448

13.2.87A6A24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/ MinimumHeading	449

13.2.87A6A24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/ MaximumHeading	449

13.2.87A6B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation	449

13.2.87A6B0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>	450

13.2.87A6B1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea	450

13.2.87A6B2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea	450

13.2.87A6B3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>	450

13.2.87A6B3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType	451

13.2.87A6B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	451

13.2.87A6B5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	451

13.2.87A6B6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea	452

13.2.87A6B7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center	452

13.2.87A6B7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	452

13.2.87A6B8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	453

13.2.87A6B9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	453

13.2.87A6B10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/Radius	453

13.2.87A6B11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/OffsetAngle	454

13.2.87A6B12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/ EllipsoidArcArea/IncludedAngle	454

13.2.87A6B12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed	454

13.2.87A6B12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/ MinimumSpeed	455

13.2.87A6B12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/ MaximumSpeed	455

13.2.87A6B12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading	455

13.2.87A6B12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/ MinimumHeading	456

13.2.87A6B12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/ MaximumHeading	456

13.2.87A6B13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea	456

13.2.87A6B14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea	456

13.2.87A6B15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>	457

13.2.87A6B15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType	457

13.2.87A6B16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType/Longitude	457

13.2.87A6B17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ PolygonArea/<x>/PointCoordinateType/Latitude	458

13.2.87A6B18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea	458

13.2.87A6B19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center	458

13.2.87A6B19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType	459

13.2.87A6B20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude	459

13.2.87A6B21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude	459

13.2.87A6B22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/Radius	460

13.2.87A6B23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/OffsetAngle	460

13.2.87A6B24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/ EllipsoidArcArea/IncludedAngle	460

13.2.87A6B24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed	461

13.2.87A6B24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/ MinimumSpeed	461

13.2.87A6B24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/ MaximumSpeed	461

13.2.87A6B24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading	461

13.2.87A6B24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/ MinimumHeading	462

13.2.87A6B24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/ MaximumHeading	462

13.2.87A6C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ ManualDeactivationNotAllowedIfLocationCriteriaMet	462

13.2.87A7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/DisplayName	463

13.2.87A7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ MaxSimultaneousEmergencyGroupCalls	463

13.2.87A7B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FAsAllowedToCall	463

13.2.87A7C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FAsAllowedToCall/<x>	463

13.2.87A7D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FAsAllowedToCall/<x>/Entry	464

13.2.87A7E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/ FAsAllowedToCall/<x>/Entry/FunctionalAlias	464

13.2.87A8	/<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser	464

13.2.87A9	/<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser	464

13.2.87A10	/<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding	465

13.2.87B	/<x>/<x>/OnNetwork/AuthorisedIncomingAny	465

13.2.87C	/<x>/<x>/OnNetwork/IncomingUserList	465

13.2.87C1	/<x>/<x>/OnNetwork/IncomingUserList/<x>	465

13.2.87C2	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry	466

13.2.87C3	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoID	466

13.2.87C4	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/ MCVideoIDKMSURI	466

13.2.87D	/<x>/<x>/OnNetwork/PrivateCall	466

13.2.87E	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert	467

13.2.87F	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry	467

13.2.87G	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID	467

13.2.87H	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ DisplayName	467

13.2.87I	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage	467

13.2.87J	/<x>/<x>/OnNetwork/RemoteGroupSelection	468

13.2.87K	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>	468

13.2.87L	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry	468

13.2.87M	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/ MCVideoID	468

13.2.87N	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/ DisplayName	469

13.2.87O1	/<x>/<x>/OnNetwork/MCVideoAdhocGroup	469

13.2.87O1A	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert	469

13.2.87O1A1	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/ Authorised	469

13.2.87O1A2	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/ Cancel	470

13.2.87O1A3	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/ AuthRecvParticipantInfo	470

13.2.87O1A4	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/ AuthSetupAdhocGroupCall	470

13.2.87O1B	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall	471

13.2.87O1B1	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/ Authorised	471

13.2.87O1B2	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/ AuthorisedParticipation	471

13.2.87O1B3	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/ AuthInitEmergencyCall	471

13.2.87O1B4	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/ AuthInitImminentPerilCall	472

13.2.87O1B5	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/ AuthRecvCallParticipantInfo	472

13.2.87O1B6	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/ AuthModifyCallParticipantInfo	472

13.2.88	/<x>/<x>/OffNetwork	473

13.2.89	/<x>/<x>/OffNetwork/Authorised	473

13.2.90	/<x>/<x>/OffNetwork/MCVideoGroupList	473

13.2.91	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>	473

13.2.92	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry	473

13.2.93	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/ MCVideoGroupID	474

13.2.94	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/DisplayName	474

13.2.95	Void	474

13.2.96	Void	474

13.2.97	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/GMSServID	474

13.2.98	Void	475

13.2.99	Void	475

13.2.100	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/ IdMSTokenEndPoint	475

13.2.100A	Void	475

13.2.100B	Void	475

13.2.100C	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/ GroupKMSURI	475

13.2.101	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/ RelativePresentationPriority	475

13.2.102	/<x>/<x>/OffNetwork/UserInfoID	476

13.2.102A	/<x>/<x>/OffNetwork/EmergencyCallChange	476

13.2.102B	/<x>/<x>/OffNetwork/ImminentPerilCallChange	476

13.2.103	/<x>/Status	476

14	MCVideo service configuration MO	477

14.1	General	477

14.2	MCVideo service configuration MO parameters	478

14.2.1	General	478

14.2.2	Node: <x>	478

14.2.3	/<x>/Name	478

14.2.4	/<x>/Ext/	478

14.2.5	/<x>/Common	478

14.2.6	/<x>/Common/BroadcastMCVideoGroupCall	479

14.2.7	/<x>/Common/BroadcastMCVideoGroupCall/ NumLevelGroupHierarchy	479

14.2.8	/<x>/Common/BroadcastMCVideoGroupCall/ NumLevelUserHierarchy	479

14.2.9	/<x>/Common/MinLengthAliasID	479

14.2.10	/<x>/OffNetwork	480

14.2.11	/<x>/OffNetwork/DefaultPPPP	480

14.2.12	/<x>/OffNetwork/DefaultPPPP/MCVideoPrivateCallSignalling	480

14.2.13	/<x>/OffNetwork/DefaultPPPP/MCVideoPrivateCallMedia	480

14.2.14	/<x>/OffNetwork/DefaultPPPP/ MCVideoEmergencyPrivateCallSignalling	481

14.2.15	/<x>/OffNetwork/DefaultPPPP/ MCVideoEmergencyPrivateCallMedia	481

14.2.15A	/<x>/OffNetwork/DefaultPQI	481

14.2.15B	/<x>/OffNetwork/DefaultPQI/MCVideoPrivateCallSignalling	482

14.2.15C	/<x>/OffNetwork/DefaultPQI/MCVideoPrivateCallMedia	482

14.2.15D	/<x>/OffNetwork/DefaultPQI/ MCVideoEmergencyPrivateCallSignalling	482

14.2.15E	/<x>/OffNetwork/DefaultPQI/ MCVideoEmergencyPrivateCallMedia	482

14.2.16	/<x>/OffNetwork/PrivateCall	483

14.2.17	/<x>/OffNetwork/PrivateCall/MaxDuration	483

14.2.18	/<x>/OffNetwork/NumLevelHierarchy	483

14.2.19	/<x>/OnNetwork	483

14.2.20	/<x>/OnNetwork/MCVideoAdhocGroup	484

14.2.21	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall	484

14.2.22	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/ AllowAdhocGroupCall	484

14.2.23	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/ MaxNoParticipants	484

14.2.24	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/HangTime	485

14.2.25	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/ MaxDurationOfCall	485

Annex A (informative): MCPTT UE configuration MO DDF	486

Annex B (informative): MCPTT user profile MO DDF	487

Annex C (informative): MCS group configuration MO DDF	488

Annex D (informative): MCPTT service configuration MO DDF	489

Annex E (informative): MCS UE initial configuration MO DDF	490

Annex F (informative): MCVideo UE configuration MO DDF	491

Annex G (informative): MCVideo user profile configuration MO DDF	492

Annex H (informative): MCVideo service configuration MO DDF	493

Annex I (informative): MCData UE configuration MO DDF	494

Annex J (informative): MCData user profile configuration MO DDF	495

Annex K (informative): MCData service configuration MO DDF	496

Annex L (normative): MO specific concepts for the support of mission critical services over 5GS	497

L.1	General	497

L.2	Aspects not applicable to 5GS	497

L.3	5GS specific aspects not applicable to EPS	497

L.4	Mapping of EPS-specific terms to 5GS	497

Annex M (informative): Change history	498

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document defines a number of Mission Critical Services (MCSs) Management Objects (MO) that are configured for the UE for the operation of MCSs. The management objects are compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA OMA-ERELD_DM-V1_2 [2].

MCSs are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

MCSs can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

The present document is applicable to an UE supporting on-line, off-line or both on-line and off-line configuration of the configuration management client.

The present document is applicable to an UE supporting off-line configuration of the group management client.

The parameters defined by the management objects in the present document are configured in the UE using on-line configuration over the CSC-4 reference point and using off-line configuration over the CSC-11 and CSC-12 reference points. Other specifications define how these parameters are used in the operation of MCSs, and whether they are applicable to on-network operation or off-network operation, or both:

-	3GPP TS 24.379 [7];

-	3GPP TS 24.380 [8];

-	3GPP TS 24.481 [9];

-	3GPP TS 24.482 [11];

-	3GPP TS 24.484 [12];

-	3GPP TS 24.282 [16];

-	3GPP TS 24.582 [17];

-	3GPP TS 24.281 [18]; and

-	3GPP TS 24.581 [19].

The common network operation means both on-network operation and off-network operation in the present document.

The following management objects are defined in the present document:

-	MCPTT UE configuration MO;

-	MCPTT user profile MO;

-	MCS group configuration MO;

-	MCPTT service configuration MO;

-	MCS UE initial configuration MO;

-	MCData UE configuration MO;

-	MCData user profile MO;

-	MCData service configuration MO;

-	MCVideo UE configuration MO;

-	MCVideo user profile MO; and

-	MCVideo service configuration MO.

The MOs listed above define repositories of data in the ME.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	OMA OMA-ERELD-DM-V1_2-20070209-A: "Enabler Release Definition for OMA Device Management, Version 1.2".

[3]	OMA OMA-TS-DM_Protocol-V1_2: "OMA Device Management Protocol".

[4]	OMA OMA-TS-XDM_Group-V1_1-20120403-A: "Group XDM Specification".

[5]	3GPP TS 23.003: "Numbering, addressing and identification".

[6]	3GPP TS 23.303: "Proximity-based Services (ProSe); Stage 2".

[7]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control Protocol specification".

[8]	3GPP TS 24.380: "Mission Critical Push To Talk (MCPTT) media plane control Protocol specification".

[9]	3GPP TS 24.481: "Mission Critical Services (MCS) group management Protocol specification".

[10]	3GPP TS 31.102: "Characteristics of the USIM Application".

[11]	3GPP TS 24.482: "Mission Critical Services (MCS) identity management Protocol specification".

[12]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management Protocol specification".

[13]	IETF RFC 4566 (July 2006): "Session Description Protocol".

[14]	Void.

[15]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2".

[16]	3GPP TS 24.282: "Mission Critical Data (MCData) signalling control Protocol specification".

[17]	3GPP TS 24.582: "Mission Critical Data (MCData) media plane control Protocol specification".

[18]	3GPP TS 24.281: "Mission Critical Video (MCVideo) signalling control Protocol specification".

[19]	3GPP TS 24.581: "Mission Critical Video (MCVideo) media plane control Protocol specification".

[20]	3GPP TS 33.180: "Security of the mission critical service".

[21]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[22]	OMA OMA-TS-DM_StdObj-V1_2-20070209-A: "OMA Device Management Standardized Objects", http://www.openmobilealliance.org/release/DM/V1_2-20070209-A/OMA-TS-DM_StdObj-V1_2-20070209-A.pdf.

[23]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[24]	IETF RFC 3748 (June 2004): "Extensible Authentication Protocol (EAP)".

[25]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[26]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[27]	IANA: Character Sets, https://www.iana.org/assignments/character-sets/character-sets.xhtml.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purpose of the present document, the following terms and definitions given in 3GPP TS 23.179 [15] apply:

Pre-selected MCPTT user profile

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

ACL	Access Control List

APN	Access Point Name

CMS	Configuration Management Server

DDF	Device Description Framework

DM	Device Management

DNN	Data Network Name

EPS	Evolved Packet System

GMS	Group Management Server

MCS	Mission Critical Service

MCSs	Mission Critical Services

MCPTT	Mission Critical Push To Talk

ME	Mobile Equipment

MO	Management Object

OMA	Open Mobile Alliance

PQI	PC5 5QI

ProSe	Proximity-based Services

RFC	Request For Comments

S-NSSAI	Single Network Slice Selection Assistance Information

URI	Uniform Resource Identifier

URN	Uniform Resource Name

XCAP	XML Configuration Access Protocol

XML	eXtensible Markup Language

XUI	XCAP Unique Identifier

## 3.3	Symbols

For the purposes of the present document, the following symbols apply to all the figures:

?	zero or one occurrences

*	zero or more occurrences

+	one or more occurrences

If none of these characters is used the default occurrence is exactly once.

## 3.4	Management Object design and representation guidelines

All the Management Objects (MOs) defined in the present document are compatible with OMA OMA-TS-DM_Protocol-V1_2 [3].

All figures are informative and the design principles of OMA OMA-TS-DM_StdObj-V1_2 [22] are applied with the following differences:

-	interior nodes are represented as rectangles with solid border;

-	leaf nodes are represented without any border;

-	un-named nodes are represented by less than and a greater than characters containing a lower-case or an uppercase character, e.g. <x>, <X>;

-	a stadium with solid border is used for informative text i.e. to point to a figure describing the contents of an MO; and

-	named nodes that share the same contents can refer to the same figure to represent their MO contents.

A representative example is shown in figure 3.4.1.

Figure 3.4.1: The MO graphical notation

# 4	MCPTT UE configuration MO

## 4.1	General

The MCPTT UE configuration Management Object (MO) is used to configure MCPTT Client behaviour for the on-network or off-network MCPTT Service. The MCPTT UE configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCPTT-UE-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCPTT UE configuration MO.

The following nodes and leaf objects are possible under the MCPTT UE configuration node as described in figure 4.1.1.

Figure 4.1.1: The MCPTT UE configuration MO

## 4.2	MCPTT UE configuration MO parameters

### 4.2.1	General

This clause describes the parameters for the MCPTT UE configuration Management Object (MO).

### 4.2.2	Node: <x>

Table 4.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCPTT UE configuration Management Object (MO). For the MCPTT UE configuration MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcptt-ue-configuration:1.0" |  |  |  |  |

-	Values: N/A

### 4.2.3	/<x>/Name

Table 4.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCPTT UE configuration settings. |  |  |  |  |

-	Values: <User displayable name>

### 4.2.4	/<x>/Ext/

Table 4.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCPTT UE configuration MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 4.2.5	/<x>/Common

Table 4.2.5.1: /<x>/Common

| Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 4.2.6	/<x>/Common/PrivateCall

Table 4.2.6.1: /<x>/Common/PrivateCall

| Common/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the private call configuration. |  |  |  |  |

### 4.2.7	/<x>/Common/PrivateCall/MaxCallN10

Table 4.2.7.1: /<x>/Common/PrivateCall/MaxCallN10

| Common/PrivateCall/MaxCallN10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of private calls. |  |  |  |  |

-	Values: 0-255

### 4.2.8	/<x>/Common/MCPTTGroupCall

Table 4.2.8.1: /<x>/Common/MCPTTGroupCall

| Common/MCPTTGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT group call configuration. |  |  |  |  |

### 4.2.9	/<x>/Common/MCPTTGroupCall/MaxCallN4

Table 4.2.9.1: /<x>/Common/MCPTTGroupCall/MaxCallN4

| Common/MCPTTGroupCall/MaxCallN4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous group calls. |  |  |  |  |

-	Values: 0-255

### 4.2.10	/<x>/Common/MCPTTGroupCall/MaxTransmissionN5

Table 4.2.10.1: /<x>/Common/MCPTTGroupCall/MaxTransmissionN5

| Common/MCPTTGroupCall/MaxTransmissionN5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of transmissions in a group. |  |  |  |  |

-	Values: 0-255

### 4.2.11	/<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup

Table 4.2.11.1: /<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup

| Common/MCPTTGroupCall/PrioritizedMCPTTGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the prioritized MCPTT group call configuration. |  |  |  |  |

### 4.2.12	/<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>

Table 4.2.12.1: /<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>

| Common/MCPTTGroupCall/PrioritizedMCPTTGroup /<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more prioritized MCPTT group call configuration. |  |  |  |  |

### 4.2.13	/<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>/
MCPTTGroupID

Table 4.2.13.1: /<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>/MCPTTGroupID

| Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>/MCPTTGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCPTT group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 4.2.14	/<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>/
MCPTTGroupPriorityHierarchy

Table 4.2.14.1: /<x>/Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>/
MCPTTGroupPriorityHierarchy

| Common/MCPTTGroupCall/PrioritizedMCPTTGroup/<x>/MCPTTGroupPriorityHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the requested presentation priority of group call. |  |  |  |  |

-	Values: 0-7

The group with the lowest MCPTTGroupPriorityHierarchy value shall be considered as the group having the lowest priority among the groups.

### 4.2.15	/<x>/OnNetwork

Table 4.2.15.1: /<x>/OnNetwork

| OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 4.2.16	/<x>/OnNetwork/RelayService

Table 4.2.16.1: /<x>/OnNetwork/RelayService

| OnNetwork/RelayService |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to use a relay service. |  |  |  |  |

When set to "true" the MCPTT group is allowed to use a relay service.

When set to "false" the MCPTT group is not allowed to use a relay service.

### 4.2.17	/<x>/OnNetwork/IPv6Preferred

Table 4.2.17.1: /<x>/OnNetwork/IPv6Preferred

| OnNetwork/IPv6Preferred |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether IPv6 is preferred over IPv4 for on-network operation when the MCPTT UE has both IPv4 and IPv6 host configuration. |  |  |  |  |

If the MCPTT UE has both IPv4 and IPv6 host configuration and:

-	if IPv6Preferred is set to "true" then the UE uses IPv6 for all on-network signalling and media; otherwise

-	if IPv6Preferred is set to "false" then the UE uses IPv4 for all on-network signalling and media.

### 4.2.18	/<x>/OnNetwork/RelayedMCPTTGroup

Table 4.2.18.1: /<x>/OnNetwork/RelayedMCPTTGroup

| OnNetwork/RelayedMCPTTGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the allowed relayed MCPTT groups. |  |  |  |  |

### 4.2.19	/<x>/OnNetwork/RelayedMCPTTGroup/<x>

Table 4.2.19.1: /<x>/OnNetwork/RelayedMCPTTGroup/<x>

| OnNetwork/RelayedMCPTTGroup/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more allowed relayed MCPTT groups. |  |  |  |  |

### 4.2.20	/<x>/OnNetwork/RelayedMCPTTGroup/<x>/MCPTTGroupID

Table 4.2.20.1: /<x>/OnNetwork/RelayedMCPTTGroup/<x>/MCPTTGroupID

| OnNetwork/RelayedMCPTTGroup/<x>/MCPTTGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCPTT group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 4.2.21	/<x>/OnNetwork/RelayedMCPTTGroup/<x>/RelayServiceCode

Table 4.2.21.1: /<x>/OnNetwork/RelayedMCPTTGroup/<x>/RelayServiceCode

| OnNetwork/RelayedMCPTTGroup/<x>/RelayServiceCode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

# 5	MCPTT user profile MO

## 5.1	General

The MCPTT user profile configuration Management Object (MO) is used to configure the MCPTT Client behaviour for the on-network or off-network MCPTT Service. The MCPTT user profile configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCPTT-user-profile:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCPTT user profile MO.

The following nodes and leaf objects are possible under the MCPTT user profile node as described in figure 5.1.1, figure 5.1.2, figure 5.1.3, figure 5.1.4, figure 5.1.5, figure 5.1.6, figure 5.1.7, figure 5.1.8, and figure 5.1.9.

Figure 5.1.1: The MCPTT user profile MO (1 of 3)

Figure 5.1.2: The MCPTT user profile MO (2 of 3)

Figure 5.1.3: The MCPTT user profile MO (3 of 3)

NOTE 1:	The LocationCriteriaForDeactivation MO contents are identical.

NOTE 2:	This figure is referenced by figures 5.1.3, 10.1.2 and 13.1.2.

Figure 5.1.4: LocationCriteriaForActivation MO contents

NOTE:	The ExitSpecificArea MO contents are identical.

Figure 5.1.5: EnterSpecificArea MO contents

NOTE 1:	The RulesForDeaffiliation MO contents are identical.

NOTE 2:	This figure is referenced by figures 5.1.3, 10.1.2 and 13.1.2.

Figure 5.1.6: RulesForAffiliation MO contents

Figure 5.1.7: ListOfLocationCriteria MO contents

Figure 5.1.8: ListOfActiveFunctionalAliases MO contents

Figure 5.1.9: MCPTTAdhocGroup MO contents

## 5.2	MCPTT user profile MO parameters

### 5.2.1	General

This clause describes the parameters for the MCPTT user profile Management Object (MO).

### 5.2.2	Node: <x>

Table 5.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCPTT user profile Management Object (MO). For the MCPTT user profile MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcptt-user-profile:1.0" |  |  |  |  |

-	Values: N/A

### 5.2.3	/<x>/Name

Table 5.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCPTT user profile settings. |  |  |  |  |

-	Values: <User displayable name>

### 5.2.4	/<x>/Ext/

Table 5.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCPTT user profile MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 5.2.5	/<x>/<x>

Table 5.2.5.1: /<x>/<x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation and off-network operation. |  |  |  |  |

### 5.2.6	/<x>/<x>/Common

Table 5.2.6.1: /<x>/<x>/Common

| <x>/Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 5.2.7	/<x>/<x>/Common/MCPTTUserID

Table 5.2.7.1: /<x>/<x>/Common/MCPTTUserID

| <x>/Common/MCPTTUserID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. |  |  |  |  |

### 5.2.7A	/<x>/<x>/Common/MCPTTUserProfileIndex

Table 5.2.7A.1: /<x>/<x>/Common/MCPTTUserProfileIndex

| <x>/Common/MCPTTUserProfileIndex |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates an index for the particular MCPTT user profile. |  |  |  |  |

-	Values: 0-255

### 5.2.7B	/<x>/<x>/Common/MCPTTUserProfileName

Table 5.2.7B.1: /<x>/<x>/Common/MCPTTUserProfileName

| <x>/Common/MCPTTUserProfileName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates a profile name for the particular MCPTT user profile. |  |  |  |  |

### 5.2.7C	/<x>/<x>/Common/PreSelectedIndication

Table 5.2.7C.1: /<x>/<x>/Common/PreSelectedIndication

| <x>/Common/PreSelectedIndication |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | null | Get, Replace |  |
|  | This leaf node indicates that this MCPTT user profile MO is designated to be the pre-selected MCPTT user profile as defined by 3GPP TS 23.179 [15] |  |  |  |  |

-	Values: null

When this leaf node is present, this MCPTT user profile MO is designated as the pre-selected MCPTT user profile.

When this leaf node is absent, this MCPTT user profile MO is not designated as the pre-selected MCPTT user profile.

If more than one MCPTT user profile MO is specified for the MCPTT user, then only one MCPTT user profile MO for the MCPTT user shall contain the <x>/Common/PreSelectedIndication leaf node.

If there is only one MCPTT user profile MO specified for the MCPTT user, then it is optional to include the <x>/Common/PreSelectedIndication leaf node.

### 5.2.8	/<x>/<x>/Common/UserAliases

Table 5.2.8.1: /<x>/<x>/Common/UserAliases

| <x>/Common/UserAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of aliases of an MCPTT user. |  |  |  |  |

### 5.2.8A	/<x>/<x>/Common/UserAliases/<x>

Table 5.2.8A.1: /<x>/<x>/Common/UserAliases/<x>

| <x>/Common/UserAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more aliases of an MCPTT user. |  |  |  |  |

### 5.2.8B	/<x>/<x>/Common/UserAliases/<x>/UserAlias

Table 5.2.8B.1: /<x>/<x>/Common/UserAliases/<x>/UserAlias

| <x>/Common/UserAliases/<x>/UserAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an alphanumeric alias of the MCPTT user. |  |  |  |  |

### 5.2.9	/<x>/<x>/Common/AuthorisedAlias

Table 5.2.9.1: /<x>/<x>/Common/AuthorisedAlias

| <x>/Common/AuthorisedAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates authorisation to create and delete aliases of other MCPTT users and their associated MCPTT user profiles. |  |  |  |  |

When set to "true" the MCPTT user is authorised to create and delete aliases of other MCPTT users and their associated MCPTT user profiles.

When set to "false" the MCPTT user is not authorised to create and delete aliases of other MCPTT user and their associated MCPTT user profiles. This is the default if this leaf node is not present.

### 5.2.10	/<x>/<x>/Common/ParticipantType

Table 5.2.10.1: /<x>/<x>/Common/ParticipantType

| <x>/Common/ParticipantType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the participant type of the MCPTT user. |  |  |  |  |

The ParticipantType means the functional category of the participant (e.g., first responder, second responder, dispatch, dispatch supervisor), typically defined by the MCPTT administrators.

### 5.2.11	/<x>/<x>/Common/Organization

Table 5.2.11.1: /<x>/<x>/Common/Organization

| <x>/Common/Organization |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the organization an MCPTT user belongs to. |  |  |  |  |

### 5.2.12	/<x>/<x>/Common/PrivateCall

Table 5.2.12.1: /<x>/<x>/Common/PrivateCall

| <x>/Common/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT private call configuration. |  |  |  |  |

### 5.2.13	/<x>/<x>/Common/PrivateCall/Authorised

Table 5.2.13.1: /<x>/<x>/Common/PrivateCall/Authorised

| <x>/Common/PrivateCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCPTT private call. |  |  |  |  |

When set to "true" the MCPTT user is allowed to make a MCPTT private call.

When set to "false" the MCPTT user is not allowed to make a MCPTT private call.

### 5.2.14	/<x>/<x>/Common/PrivateCall/AuthorisedAny

Table 5.2.14.1: /<x>/<x>/Common/PrivateCall/AuthorisedAny

| <x>/Common/PrivateCall/AuthorisedAny |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCPTT private call to any MCPTT user. |  |  |  |  |

When set to "true" any MCPTT user is authorised to make an MCPTT private call to any MCPTT user.

When set to "false" any MCPTT user is not authorised to make an MCPTT private call to any MCPTT user.

### 5.2.15	/<x>/<x>/Common/PrivateCall/UserList

Table 5.2.15.1: /<x>/<x>/PrivateCall/UserList

| <x>/Common/PrivateCall/UserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCPTT user(s) who can be called in a MCPTT private call. |  |  |  |  |

### 5.2.16	/<x>/<x>/Common/PrivateCall/UserList/<x>

Table 5.2.16.1: /<x>/<x>/Common/PrivateCall/UserList/<x>

| <x>/Common/PrivateCall/UserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCPTT users who can be called in a MCPTT private call. |  |  |  |  |

### 5.2.16A	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry

Table 5.2.16A.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry

| <x>/Common/PrivateCall/UserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCPTT users who can be called in a private call. |  |  |  |  |

### 5.2.17	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCPTTID

Table 5.2.17.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCPTTID

| <x>/Common/PrivateCall/UserList/<x>/Entry/MCPTTID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 5.2.18	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/
DiscoveryGroupID

Table 5.2.18.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DiscoveryGroupID

| <x>/Common/PrivateCall/UserList/<x>/Entry/DiscoveryGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a discovery group ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The value is used as the discovery group ID in the ProSe discovery procedures as specified in 3GPP TS 23.303 [6].

### 5.2.18A	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/
ApplicationLayerGroupID

Table 5.2.18A.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/ApplicationLayerGroupID

| <x>/Common/PrivateCall/UserList/<x>/Entry/ApplicationLayerGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates an application layer group ID as specified in 3GPP TS 23.304 [25]. |  |  |  |  |

The value is used as the application layer group ID in the ProSe discovery procedures as specified in 3GPP TS 23.304 [25].

### 5.2.19	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID

Table 5.2.19.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID

| <x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 5.2.19A	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName

Table 5.2.19A.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName

| <x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name. |  |  |  |  |

### 5.2.19B	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/
PrivateCallKMSURI

Table 5.2.19B.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/PrivateCallKMSURI

| <x>/Common/PrivateCall/UserList/<x>/Entry/PrivateCallKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS associated with the MCPTTID. If the value is empty, the KMS leaf node present in the MCS UE initial configuration MO is used. |  |  |  |  |

### 5.2.20	/<x>/<x>/Common/PrivateCall/ManualCommence

Table 5.2.20.1: /<x>/<x>/Common/PrivateCall/ManualCommence

| <x>/Common/PrivateCall/ManualCommence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCPTT private call with manual commencement. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make a MCPTT private call in manual commencement mode.

When set to "false" the MCPTT user is not authorised to make a MCPTT private call in manual commencement mode.

### 5.2.21	/<x>/<x>/Common/PrivateCall/AutoCommence

Table 5.2.21.1: /<x>/<x>/Common/PrivateCall/AutoCommence

| <x>/Common/PrivateCall/AutoCommence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCPTT private call with automatic commencement. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make a MCPTT private call in automatic commencement mode.

When set to "false" the MCPTT user is not authorised to make a MCPTT private call in automatic commencement mode.

### 5.2.22	/<x>/<x>/Common/PrivateCall/AutoAnswer

Table 5.2.22.1: /<x>/<x>/Common/PrivateCall/AutoAnswer

| <x>/Common/PrivateCall/AutoAnswer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation of MCPTT user to force automatic answer for a MCPTT private call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to force automatic answer for a MCPTT private call.

When set to "false" the MCPTT user is not authorised to force automatic answer for a MCPTT private call.

### 5.2.23	/<x>/<x>/Common/PrivateCall/FailRestrict

Table 5.2.23.1: /<x>/<x>/Common/PrivateCall/FailRestrict

| <x>/Common/PrivateCall/FailRestrict |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to restrict the provision of a notification of call failure reason for a MCPTT private call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to restrict notification of call failure reason for MCPTT private call.

When set to "false" the MCPTT user is not authorised to restrict notification of call failure reason for MCPTT private call.

### 5.2.24	/<x>/<x>/Common/PrivateCall/AllowedMediaProtection

Table 5.2.24.1: /<x>/<x>/Common/PrivateCall/AllowedMediaProtection

| <x>/Common/PrivateCall/AllowedMediaProtection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates authorisation to protect confidentiality and integrity of media for MCPTT private calls. |  |  |  |  |

When set to "true" the MCPTT user is authorised to protect confidentiality and integrity of media for MCPTT private calls.

When set to "false" the MCPTT user is not authorised to protect confidentiality and integrity of media for MCPTT private calls.

The default value is set to "true".

### 5.2.25	/<x>/<x>/Common/PrivateCall/AllowedFloorControlProtection

Table 5.2.25.1: /<x>/<x>/Common/PrivateCall/AllowedFloorControlProtection

| <x>/Common/PrivateCall/AllowedFloorControlProtection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates authorisation to protect confidentiality and integrity of floor control signalling for MCPTT private calls. |  |  |  |  |

When set to "true" the MCPTT user is authorised to protect confidentiality and integrity of floor control signalling for MCPTT private calls.

When set to "false" the MCPTT user is not authorised to protect confidentiality and integrity of floor control signalling for MCPTT private calls.

The default value is set to "true".

### 5.2.26	/<x>/<x>/Common/PrivateCall/EmergencyCall

Table 5.2.26.1: /<x>/<x>/Common/PrivateCall/EmergencyCall

| <x>/Common/PrivateCall/EmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT emergency call policy. |  |  |  |  |

### 5.2.27	/<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised

Table 5.2.27.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised

| <x>/Common/PrivateCall/EmergencyCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCPTT emergency private call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make an MCPTT emergency private call.

When set to "false" the MCPTT user is not authorised to make an MCPTT emergency private call.

### 5.2.28	/<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority

Table 5.2.28.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority

| <x>/Common/PrivateCall/EmergencyCall/CancelPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel emergency priority in an MCPTT emergency private call by an authorised MCPTT user. |  |  |  |  |

When set to "true" the MCPTT user is authorised to cancel an emergency priority in an MCPTT private call.

When set to "false" the MCPTT user is not authorised to cancel an emergency priority in an MPCTT private call.

### 5.2.29	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCPTTPrivateRecipient

Table 5.2.29.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient

| <x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT private recipient for an MCPTT emergency private call. |  |  |  |  |

### 5.2.29A	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCPTTPrivateRecipient/Entry

Table 5.2.29A.1: /<x>/<x>/Common/PrivateCall EmergencyCall/MCPTTPrivateRecipient/Entry

| <x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCPTT private recipient for an MCPTT emergency private call. |  |  |  |  |

### 5.2.29B	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCPTTPrivateRecipient/Entry/ID

Table 5.2.29B.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/ID

| <x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCPTT private recipient used upon certain criteria on initiation of an MCPTT emergency private call. |  |  |  |  |

### 5.2.29C	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCPTTPrivateRecipient/Entry/DiscoveryGroupID

Table 5.2.29C.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/
DiscoveryGroupID

| <x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/DiscoveryGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the discovery group ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 5.2.29C1	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCPTTPrivateRecipient/Entry/ApplicationLayerGroupID

Table 5.2.29C1.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/
ApplicationLayerGroupID

| <x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/ApplicationLayerGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates an application layer group ID as specified in 3GPP TS 23.304 [25]. |  |  |  |  |

### 5.2.29D	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCPTTPrivateRecipient/Entry/UserInfoID

Table 5.2.29D.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/
UserInfoID

| <x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The "UserInfoID" element identifies the MCPTT private recipient when making an off-network private call.

### 5.2.29E	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCPTTPrivateRecipient/Entry/DisplayName

Table 5.2.29E.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/
DisplayName

| <x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCPTT private recipient ID. |  |  |  |  |

### 5.2.29F	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCPTTPrivateRecipient/Entry/Usage

Table 5.2.29F.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/Usage

| <x>/Common/PrivateCall/EmergencyCall/MCPTTPrivateRecipient/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCPTT emergency private call uses the MCPTT private recipient ID. |  |  |  |  |

The valid values are 'LocallyDetermined' and 'UsePreConfigured'.

When set to 'LocallyDetermined' then if the MCPTT user selects an MCPTT ID then use that MCPTT ID for the MCPTT emergency private call, if the MCPTT user does not select a MCPTT ID then use the MCPTT ID identified by the MCPTT private recipient ID in clause 5.2.29B for an on-network MCPTT emergency private call.

When set to 'UsePreConfigured' then use the MCPTT ID identified by the MCPTT private recipient ID in clause 5.2.29B for an on-network MCPTT emergency private call.

When set to 'LocallyDetermined' then if the MCPTT user selects an MCPTT user then use the UserInfoID that corresponds to that MCPTT user for the MCPTT emergency private call, if the MCPTT user does not select a MCPTT user then use the User Info ID identified by the UserInfoID in clause 5.2.29D for an off-network MCPTT emergency private call.

When set to 'UsePreConfigured' then use the User Info ID identified by the UserInfoID in clause 5.2.29D for an off-network MCPTT emergency private call.

### 5.2.30	/<x>/<x>/Common/MCPTTGroupCall

Table 5.2.30.1: /<x>/<x>/Common/MCPTTGroupCall

| <x>/Common/MCPTTGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT group call configuration. |  |  |  |  |

### 5.2.31	/<x>/<x>/Common/MCPTTGroupCall/MaxSimultaneousCallsN6

Table 5.2.31.1: /<x>/<x>/Common/MCPTTGroupCall/MaxSimultaneousCallsN6

| <x>/Common/MCPTTGroupCall/MaxSimultaneousCallsN6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneously received MCPTT group calls (N6). |  |  |  |  |

### 5.2.32	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall

Table 5.2.32.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyCall

| <x>/Common/MCPTTGroupCall/EmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT emergency call policy. |  |  |  |  |

### 5.2.33	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/Enabled

Table 5.2.33.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/Enabled

| <x>/Common/MCPTTGroupCall/EmergencyCall/Enabled |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCPTT emergency group call functionality enabled for MCPTT user. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make an MCPTT emergency group call functionality enabled.

When set to "false" the MCPTT user is not authorised to make an MCPTT emergency group call functionality enabled.

### 5.2.34	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/
MCPTTGroupInitiation

Table 5.2.34.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation

| <x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group used on initiation of an MCPTT emergency group call. |  |  |  |  |

### 5.2.34A	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/
MCPTTGroupInitiation/Entry

Table 5.2.34A.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation/Entry

| <x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the group used on initiation of an MCPTT emergency group call. |  |  |  |  |

### 5.2.34B	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/
MCPTTGroupInitiation/Entry/GroupID

Table 5.2.34BA.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation/
Entry/GroupID

| <x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation/Entry/GroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the group used upon certain criteria on initiation of an MCPTT emergency group call. |  |  |  |  |

### 5.2.34C	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/
MCPTTGroupInitiation/Entry/DisplayName

Table 5.2.34C.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation/Entry/
DisplayName

| <x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the Group ID. |  |  |  |  |

### 5.2.34D	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/
MCPTTGroupInitiation/Entry/Usage

Table 5.2.34D.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation/Entry/
Usage

| <x>/Common/MCPTTGroupCall/EmergencyCall/MCPTTGroupInitiation/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCPTT emergency group call uses the GroupID. |  |  |  |  |

The valid values are 'UseCurrentlySelectedGroup' and 'DedicatedGroup'.

When set to 'UseCurrentlySelectedGroup' then if the MCPTT user has currently selected an MCPTT group then use that MCPTT group for an on-network MCPTT emergency group call, if the MCPTT user does not have a currently selected MCPTT group then use the MCPTT group identified by the GroupID in clause 5.2.34B for an MCPTT emergency group call.

When set to 'DedicatedGroup' then use the MCPTT group identified by the GroupID in clause 5.2.34B for an MCPTT emergency group call.

### 5.2.35	/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/
CancelMCPTTGroup

Table 5.2.35.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/CancelMCPTTGroup

| <x>/Common/MCPTTGroupCall/EmergencyCall/CancelMCPTTGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel an in progress MCPTT emergency group call associated with a group. |  |  |  |  |

When set to "true" the MCPTT user is authorised to cancel a MCPTT emergency group call.

When set to "false" the MCPTT user is not authorised to cancel a MCTT emergency group call.

### 5.2.36	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall

Table 5.2.36.1: /<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall

| <x>/Common/MCPTTGroupCall/ImminentPerilCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT imminent peril call policy. |  |  |  |  |

### 5.2.37	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/Authorised

Table 5.2.37.1: /<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/Authorised

| <x>/Common/MCPTTGroupCall/ImminentPerilCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an Imminent Peril group call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to create an MCPTT imminent peril group call.

When set to "false" the MCPTT user is not authorised to create an MCPTT imminent peril group call.

### 5.2.38	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/Cancel

Table 5.2.38.1: /<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/Cancel

| <x>/Common/MCPTTGroupCall/ImminentPerilCall/Cancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation for in-progress MCPTT imminent peril cancelation. |  |  |  |  |

When set to "true" the MCPTT user is authorised to cancel an MCPTT imminent peril call.

When set to "false" the MCPTT user is not authorised to cancel an MCPTT imminent peril call.

### 5.2.39	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/
MCPTTGroupInitiation

Table 5.2.39.1: /<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation

| <x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group used on initiation of an MCPTT imminent peril group call. |  |  |  |  |

### 5.2.39A	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/
MCPTTGroupInitiation/Entry

Table 5.2.39A.1: /<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall//MCPTTGroupInitiation/Entry

| <x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the group used on initiation of an imminent peril call. |  |  |  |  |

### 5.2.39B	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/
MCPTTGroupInitiation/Entry/GroupID

Table 5.2.39B.1: /<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation/Entry/
GroupID

| <x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation/Entry/GroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the group used upon certain criteria on initiation of an MCPTT imminent peril group call. |  |  |  |  |

### 5.2.39C	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/
MCPTTGroupInitiation/DisplayName

Table 5.2.39C.1: /<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation/Entry/
DisplayName

| <x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the Group ID. |  |  |  |  |

### 5.2.39D	/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/
MCPTTGroupInitiation/Entry/Usage

Table 5.2.39D.1: /<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation/Entry/
Usage

| <x>/Common/MCPTTGroupCall/ImminentPerilCall/MCPTTGroupInitiation/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCPTT imminent peril group call uses the GroupID. |  |  |  |  |

The valid values are 'UseCurrentlySelectedGroup' and 'DedicatedGroup'.

When set to 'UseCurrentlySelectedGroup' then if the MCPTT user has currently selected an MCPTT group then use that MCPTT group for an on-network MCPTT imminent peril group call, if the MCPTT user does not have a currently selected MCPTT group then use the MCPTT group identified by the GroupID in clause 5.2.39B for an MCPTT imminent peril group call.

When set to 'DedicatedGroup' then use the MCPTT group identified by the GroupID in clause 5.2.39B for an MCPTT imminent peril group call.

### 5.2.40	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert

Table 5.2.40.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert

| <x>/Common/MCPTTGroupCall/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT emergency alert policy. |  |  |  |  |

### 5.2.41	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Authorised

Table 5.2.41.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Authorised

| <x>/Common/MCPTTGroupCall/EmergencyAlert/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to activate an MCPTT emergency alert. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make an MCPTT emergency alert.

When set to "false" the MCPTT user is not authorised to make an MCPTT emergency alert.

### 5.2.42	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Cancel

Table 5.2.42.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Cancel

| <x>/Common/MCPTTGroupCall/EmergencyAlert/Cancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel an MCPTT emergency alert. |  |  |  |  |

When set to "true" the MCPTT user is authorised to cancel an MCPTT emergency alert.

When set to "false" the MCPTT user is not authorised to cancel an MCPTT emergency alert.

### 5.2.43	Void

### 5.2.43A	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry

Table 5.2.43A.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry

| <x>/Common/MCPTTGroupCall/EmergencyAlert/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCPTT group of an MCPTT emergency alert. |  |  |  |  |

### 5.2.43B	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/ID

Table 5.2.43B.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/ID

| <x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCPTT group used upon certain criteria on initiation of an MCPTT emergency alert. |  |  |  |  |

### 5.2.43C	Void

### 5.2.43D	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/
DisplayName

Table 5.2.43D.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/DisplayName

| <x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the ID. |  |  |  |  |

### 5.2.43E	/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/Usage

Table 5.2.43E.1: /<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/Usage

| <x>/Common/MCPTTGroupCall/EmergencyAlert/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCPTT emergency alert uses the ID. |  |  |  |  |

The valid values are 'LocallyDetermined', 'UseCurrentlySelectedGroup', 'UsePreConfigured' and 'DedicatedGroup'.

When set to 'LocallyDetermined' then if the MCPTT user selects an MCPTT ID then use that MCPTT ID for an on-network MCPTT emergency alert, if the MCPTT user does not select a MCPTT ID then use the MCPTT ID identified by the ID in clause 5.2.43B for an on-network MCPTT emergency alert.

When set to 'UseCurrentlySelectedGroup' then if the MCPTT user has currently selected an MCPTT group then use that MCPTT group for an on-network MCPTT emergency alert, if the MCPTT user does not have a currently selected MCPTT group then use the MCPTT group identified by the ID in clause 5.2.43B for an MCPTT emergency alert.

When set to 'UsePreConfigured' then use the ID identified by the ID in clause 5.2.43B for an on-network MCPTT emergency alert.

When set to 'DedicatedGroup' then use the MCPTT group identified by the ID in clause 5.2.43B for an MCPTT emergency alert.

### 5.2.43F	/<x>/<x>/Common/MCPTTGroupCall/Priority

Table 5.2.43F.1: /<x>/<x>/Common/MCPTTGroupCall/Priority

| <x>/Common/MCPTTGroupCall/Priority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the priority of the MCPTT group calls. |  |  |  |  |

-	Values: 0-255

The MCPTT group call with the lowest Priority value shall be considered as the MCPTT group call having the lowest level among the MCPTT group calls.

### 5.2.44	Void

### 5.2.45	/<x>/<x>/Common/MCPTTGroupBroadcast

Table 5.2.45.1: /<x>/<x>/Common/MCPTTGroupBroadcast

| <x>/Common/MCPTTGroupBroadcast |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group-broadcast group policy. |  |  |  |  |

### 5.2.46	/<x>/<x>/Common/MCPTTGroupBroadcast/Authorised

Table 5.2.46.1: /<x>/<x>/Common/MCPTTGroupBroadcast/Authorised

| <x>/Common/MCPTTGroupBroadcast/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to create a group-broadcast group. |  |  |  |  |

When set to "true" the MCPTT user is authorised to create a group-broadcast group.

When set to "false" the MCPTT user is not authorised to create a group-broadcast group.

### 5.2.47	/<x>/<x>/Common/UserBroadcast

Table 5.2.47.1: /<x>/<x>/Common/UserBroadcast

| <x>/Common/UserBroadcast |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the user-broadcast group policy. |  |  |  |  |

### 5.2.48	/<x>/<x>/Common/UserBroadcast/Authorised

Table 5.2.48.1: /<x>/<x>/Common/UserBroadcast/Authorised

| <x>/Common/UserBroadcast/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to create a user-broadcast group. |  |  |  |  |

When set to "true" the MCPTT user is authorised to create a user-broadcast group.

When set to "false" the MCPTT user is not authorised to create a user-broadcast group.

### 5.2.48A	/<x>/<x>/OnNetwork

Table 5.2.48A.1: /<x>/<x>/OnNetwork

| <x>/OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 5.2.48B1	/<x>/<x>/OnNetwork/MCPTTGroupList

Table 5.2.48B1.1: /<x>/<x>/OnNetwork/MCPTTGroupList

| <x>/OnNetwork/MCPTTGroupList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the list of on-network MCPTT groups that the MCPTT user is allowed to affiliate to. |  |  |  |  |

### 5.2.48B2	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>

Table 5.2.48B2.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>

| <x>/OnNetwork/MCPTTGroupList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of on-network MCPTT groups that the MCPTT user is allowed to affiliate to. |  |  |  |  |

### 5.2.48B3	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry

Table 5.2.48B3.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCPTT groups that the MCPTT user is allowed to affiliate to. |  |  |  |  |

### 5.2.48B4	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
MCPTTGroupID

Table 5.2.48B4.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/MCPTTGroupID

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/MCPTTGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCPTT group ID for the on-network MCPTT group that the MCPTT user is allowed to affiliate to. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 5.2.48B4A	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation

Table 5.2.48B4A.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the rules that control automatic affiliation. |  |  |  |  |

### 5.2.48B4A0	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>

Table 5.2.48B4A0.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the zero or more rules that control automatic affiliation. |  |  |  |  |

### 5.2.48B4A1	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria

Table 5.2.48B4A1.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 5.2.48B4A2	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>

Table 5.2.48B84A2.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 5.2.48B4A3	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry

Table 5.2.48B4A3.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 5.2.48B4A4	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea

Table 5.2.48B4A4.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 5.2.48B4A5	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea

Table 5.2.48B4A5.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 5.2.48B4A6	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>

Table 5.2.48B4A6.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 5.2.48B4A7	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 5.2.48B4A7.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 5.2.48B4A8	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 5.2.48B4A8.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 5.2.48B4A9	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 5.2.48B4A9.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 5.2.48B4A10	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea

Table 5.2.48B4A10.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 5.2.48B4A11	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center

Table 5.2.48B4A11.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A12	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/
PointCoordinateType

Table 5.2.48B4A12.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A13	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/
PointCoordinateType/Longitude

Table 5.2.48B4A13.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 5.2.48B4A14	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/
PointCoordinateType/Latitude

Table 5.2.48B4A14.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 5.2.48B4A15	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Radius

Table 5.2.48B4A15.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A16	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/OffsetAngle

Table 5.2.48B4A16.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A17	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/IncludedAngle

Table 5.2.48B4A17.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A18	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed

Table 5.2.48B4A18.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48B4A19	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MinimumSpeed

Table 5.2.48B4A19.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4A20	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MaximumSpeed

Table 5.2.48B4A20.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4A21	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading

Table 5.2.48B4A21.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48B4A22	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MinimumHeading

Table 5.2.48B4A22.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4A23	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MaximumHeading

Table 5.2.48B4A23.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4A24	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea

Table 5.2.48B4A24.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 5.2.48B4A25	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea

Table 5.2.48B4A25.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 5.2.48B4A26	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>

Table 5.2.48B4A26.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 5.2.48B4A27	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 5.2.48B4A27.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 5.2.48B4A28	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 5.2.48B4A28.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 5.2.48B4A29	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 5.2.48B4A29.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 5.2.48B4A30	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea

Table 5.2.48B4A30.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 5.2.48B4A31	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center

Table 5.2.48B4A31.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A32	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48B4A32.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A33	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/
PointCoordinateType/Longitude

Table 5.2.48B4A33.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 5.2.48B4A34	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/
PointCoordinateType/Latitude

Table 5.2.48B4A34.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 5.2.48B4A35	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Radius

Table 5.2.48B4A35.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A36	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/OffsetAngle

Table 5.2.48B4A36.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A37	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/IncludedAngle

Table 5.2.48B4A37.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4A38	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed

Table 5.2.48B4A38.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48B4A39	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MinimumSpeed

Table 5.2.48B4A39.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4A40	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MaximumSpeed

Table 5.2.48B4A40.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4A41	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading

Table 5.2.48B4A41.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48B4A42	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MinimumHeading

Table 5.2.48B4A42.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4A43	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MaximumHeading

Table 5.2.48B4A43.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4A44	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases

Table 5.2.48B4A44.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 5.2.48B4A45	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>

Table 5.2.48B4A45.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 5.2.48B4A46	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry

Table 5.2.48B4A46.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 5.2.48B4A47	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/
Entry/FunctionalAlias

Table 5.2.48B4A47.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node contains a functional alias. |  |  |  |  |

### 5.2.48B4B	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation

Table 5.2.48B4B.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the rules that control automatic deaffiliation. |  |  |  |  |

### 5.2.48B4B0	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>

Table 5.2.48B4B0.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the zero or more rules that control automatic deaffiliation. |  |  |  |  |

### 5.2.48B4B1	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria

Table 5.2.48B4B1.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 5.2.48B4B2	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>

Table 5.2.48B4B2.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 5.2.48B4B3	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry

Table 5.2.48B4B3.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 5.2.48B4B4	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea

Table 5.2.48B4B4.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which, when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 5.2.48B4B5	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea

Table 5.2.48B4B5.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 5.2.48B4B6	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>

Table 5.2.48B4B6.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 5.2.48B4B7	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 5.2.48B4B7.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesFordeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 5.2.48B4B8	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 5.2.48B4B8.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 5.2.48B4B9	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 5.2.48B4B9.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 5.2.48B4B10	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea

Table 5.2.48B4B10.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 5.2.48B4B11	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center

Table 5.2.48B4B11.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B12	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48B4B12.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B13	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/
PointCoordinateType/Longitude

Table 5.2.48B4B13.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 5.2.48B4B14	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/
PointCoordinateType/Latitude

Table 5.2.48B4B14.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 5.2.48B4B15	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Radius

Table 5.2.48B4B15.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B16	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/OffsetAngle

Table 5.2.48B4B16.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B17	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/IncludedAngle

Table 5.2.48B4B17.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B18	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed

Table 5.2.48B4B18.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48B4B19	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MinimumSpeed

Table 5.2.48B4B19.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4B20	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MaximumSpeed

Table 5.2.48B4B20.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4B21	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading

Table 5.2.48B4B21.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48B4B22	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MinimumHeading

Table 5.2.48B4B22.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4B23	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MaximumHeading

Table 5.2.48B4B23.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4B24	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea

Table 5.2.48B4B24.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 5.2.48B4B25	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea

Table 5.2.48B4B25.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 5.2.48B4B26	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>

Table 5.2.48B4B26.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 5.2.48B4B27	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 5.2.48B4B27.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 5.2.48B4B28	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 5.2.48B4B28.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 5.2.48B4B29	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 5.2.48B4B29.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 5.2.48B4B30	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea

Table 5.2.48B4B30.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 5.2.48B4B31	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center

Table 5.2.48B431.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B32	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48B4B32.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B33	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 5.2.48B4B33.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 5.2.48B4B34	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 5.2.48B4B34.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 5.2.48B4B35	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Radius

Table 5.2.48B4B35.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B36	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/OffsetAngle

Table 5.2.48B4B36.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B37	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/IncludedAngle

Table 5.2.48B4B37.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 5.2.48B4B38	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed

Table 5.2.48B4B38.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48B4B39	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MinimumSpeed

Table 5.2.48B4B39.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4B40	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MaximumSpeed

Table 5.2.48B4B40.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4B41	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading

Table 5.2.48B4B41.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48B4B42	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MinimumHeading

Table 5.2.48B4B42.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4B43	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MaximumHeading

Table 5.2.48B4B43.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4B44	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases

Table 5.2.48B4B44.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 5.2.48B4B45	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>

Table 5.2.48B4B45.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 5.2.48B4B46	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry

Table 5.2.48B4B46.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 5.2.48B4B47	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/
Entry/FunctionalAlias

Table 5.2.48B4B47.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node contains a functional alias. |  |  |  |  |

### 5.2.48B5	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/DisplayName

Table 5.2.48B5.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/DisplayName

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCPTT Group ID. |  |  |  |  |

### 5.2.48B6	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet

Table 5.2.48B6.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to deaffiliate if the affiliation criteria are met. |  |  |  |  |

### 5.2.48B7	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RelativePresentationPriority

Table 5.2.48B7.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
RelativePresentationPriority

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RelativePresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the presentation priority of the on-network group, identified by the MCPTTGroupID, relative to other on-network groups and on-network users. |  |  |  |  |

-	Values: 0-255

The lowest RelativePresentationPriority value shall be considered as the MCPTT group transaction having the lowest priority for presentation among other group MCPTT and one-to-one user transactions.

### 5.2.48B8	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/GMSServID

Table 5.2.48B8.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/GMSServID

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/GMSServID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the GMS owning the group identified by the MCPTTGroupID. |  |  |  |  |

### 5.2.48B9	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
IDMSTokenEndPoint

Table 5.2.48B9.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/IDMSTokenEndPoint

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/IDMSTokenEndPoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the IDMS token endpoint for the group identified by the MCPTTGroupID. If the value is empty, the IDMS identities (IDMSAuthEndpoint and IDMSTokenEndpoint) present in the MCS UE initial configuration MO are used. |  |  |  |  |

### 5.2.48B10	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/
GroupKMSURI

Table 5.2.48B10.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/GroupKMSURI

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/GroupKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS identity (URI) for the group identified by the MCPTTGroupID. If the value is empty, the KMS identity (URI) (kms) present in the MCS UE initial configuration MO is used. |  |  |  |  |

### 5.2.48C1	/<x>/<x>/OnNetwork/ImplicitAffiliations

Table 5.2.48C1.1: /<x>/<x>/OnNetwork/ImplicitAffiliations

| <x>/OnNetwork/ImplicitAffiliations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the implicit affiliation configuration. |  |  |  |  |

### 5.2.48C2	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>

Table 5.2.48C2.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>

| <x>/OnNetwork/ImplicitAffiliations/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more implicit affiliation configuration. |  |  |  |  |

### 5.2.48C3	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry

Table 5.2.48C3.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCPTT groups that the MCPTT user is implicitly affiliated to. |  |  |  |  |

### 5.2.48C4	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCPTTGroupID

Table 5.2.48C4.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCPTTGroupID

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCPTTGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a MCPTT group ID to which the MCPTT user is implicitly affiliated to. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 5.2.48C5	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName

Table 5.2.48C5.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCPTT Group ID. |  |  |  |  |

### 5.2.48D	/<x>/<x>/OnNetwork/AllowedRegroup

Table 5.2.48D.1: /<x>/<x>/OnNetwork/AllowedRegroup

| <x>/OnNetwork/AllowedRegroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to perform dynamic regrouping operations. |  |  |  |  |

When set to "true" the MCPTT user is authorised to perform dynamic regrouping operations.

When set to "false" the MCPTT user is not authorised to perform dynamic regrouping operations.

### 5.2.48E	/<x>/<x>/OnNetwork/AllowedPresenceStatus

Table 5.2.48E.1: /<x>/<x>/OnNetwork/AllowedPresenceStatus

| <x>/OnNetwork/AllowedPresenceStatus |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the presence status on the network of this MCPTT user is available. |  |  |  |  |

When set to "true" the presence status on the network of this MCPTT user is available.

When set to "false" the presence status on the network of this MCPTT user is not available. This is the default if this leaf node is not present.

### 5.2.48F	/<x>/<x>/OnNetwork/AllowedPresence

Table 5.2.48F.1: /<x>/<x>/OnNetwork/AllowedPresence

| <x>/OnNetwork/AllowedPresence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to obtain whether a particular MCPTT User is present on the network. |  |  |  |  |

When set to "true" the MCPTT user is authorised to obtain whether a particular MCPTT User is present on the network.

When set to "false" the MCPTT user is not authorised to obtain whether a particular MCPTT User is present on the network.

### 5.2.48G	/<x>/<x>/OnNetwork/EnabledParticipation

Table 5.2.48G.1: /<x>/<x>/OnNetwork/EnabledParticipation

| <x>/OnNetwork/EnabledParticipation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to participate in MCPTT private calls that they are invited to. |  |  |  |  |

When set to "true" the MCPTT user is allowed to participate in MCPTT private calls that they are invited to.

When set to "false" the MCPTT user is not allowed to participate in MCPTT private calls that they are invited to.

### 5.2.48H	/<x>/<x>/OnNetwork/AllowedTransmission

Table 5.2.48H.1: /<x>/<x>/OnNetwork/AllowedTransmission

| <x>/OnNetwork/AllowedTransmission |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to override transmission in a MCPTT private call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to override transmission in a MCPTT private call.

When set to "false" the MCPTT user is not authorised to override transmission in a MCPTT private call.

### 5.2.48I	/<x>/<x>/OnNetwork/AllowedManualSwitch

Table 5.2.48I.1: /<x>/<x>/OnNetwork/AllowedManualSwitch

| <x>/OnNetwork/AllowedManualSwitch |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to manually switch to off-network operation while in on-network operation. |  |  |  |  |

When set to "true" the MCPTT user is authorised to manually switch to off-network operation while in on-network operation.

When set to "false" the MCPTT user is not authorised to manually switch to off-network operation while in on-network operation.

### 5.2.48J	/<x>/<x>/OnNetwork/PrivateCall

Table 5.2.48J.1: /<x>/<x>/OnNetwork/PrivateCall

| <x>/OnNetwork/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT private call configuration. |  |  |  |  |

### 5.2.48K	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert

Table 5.2.48K.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert

| <x>/OnNetwork/PrivateCall/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT private emergency alert policy. |  |  |  |  |

### 5.2.48L	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry

Table 5.2.48L.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of an MCPTT private emergency alert for on-network. |  |  |  |  |

### 5.2.48M	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID

Table 5.2.48M.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCPTT user ID used upon certain criteria on initiation of an MCPTT private emergency alert for on-network. |  |  |  |  |

### 5.2.48N	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/
DisplayName

Table 5.2.48N.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/DisplayName

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the ID. |  |  |  |  |

### 5.2.48O	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage

Table 5.2.48O.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCPTT private emergency alert uses the ID. |  |  |  |  |

The valid values are 'LocallyDetermined' and 'UsePreConfigured'.

When set to 'LocallyDetermined' then if the MCPTT user selects an MCPTT ID then use that MCPTT ID for an on-network MCPTT private emergency alert, if the MCPTT user does not select a MCPTT ID then use the MCPTT ID identified by the ID in clause 5.2.48M for an on-network MCPTT private emergency alert.

When set to 'UsePreConfigured' then use the ID identified by the ID in clause 5.2.48M for an on-network MCPTT private emergency alert.

### 5.2.48P	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallBackRequest

Table 5.2.48P.1: /<x>/<x>/OnNetwork/PrivateCall/AllowedCallBackRequest

| <x>/OnNetwork/PrivateCall/AllowedCallBackRequest |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to request a private call call-back. |  |  |  |  |

When set to "true" the MCPTT user is allowed to request a private call call-back.

When set to "false" the MCPTT user is not allowed to request a private call call-back.

### 5.2.48Q	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallBackCancelRequest

Table 5.2.48Q.1: /<x>/<x>/OnNetwork/PrivateCall/AllowedCallBackCancelRequest

| <x>/OnNetwork/PrivateCall/AllowedCallBackCancelRequest |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to cancel an outstanding private call call-back request. |  |  |  |  |

When set to "true" the MCPTT user is allowed to cancel an outstanding private call call-back request.

When set to "false" the MCPTT user is not allowed to cancel an outstanding private call call-back request.

### 5.2.48R	/<x>/<x>/OnNetwork/PrivateCall/
AllowedRemoteInitiatedAmbientListening

Table 5.2.48R.1: /<x>/<x>/OnNetwork/PrivateCall/AllowedRemoteInitiatedAmbientListening

| <x>/OnNetwork/PrivateCall/AllowedRemoteInitiatedAmbientListening |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to request a remote initiated ambient listening call. |  |  |  |  |

When set to "true" the MCPTT user is allowed to request a remote initiated ambient listening call.

When set to "false" the MCPTT user is not allowed to request a remote initiated ambient listening call.

### 5.2.48S	/<x>/<x>/OnNetwork/PrivateCall/
AllowedLocallyInitiatedAmbientListening

Table 5.2.48S.1: /<x>/<x>/OnNetwork/PrivateCall/AllowedLocallyInitiatedAmbientListening

| <x>/OnNetwork/PrivateCall/AllowedLocallyInitiatedAmbientListening |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to request a locally initiated ambient listening call. |  |  |  |  |

When set to "true" the MCPTT user is allowed to request a locally initiated ambient listening call.

When set to "false" the MCPTT user is not allowed to request a locally initiated ambient listening call.

### 5.2.48T	/<x>/<x>/OnNetwork/PrivateCall/AllowedRequestFirstToAnswerCall

Table 5.2.48T.1: /<x>/<x>/OnNetwork/PrivateCall/AllowedRequestFirstToAnswerCall

| <x>/OnNetwork/PrivateCall/AllowedRequestFirstToAnswerCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to request a first to answer call. |  |  |  |  |

When set to "true" the MCPTT user is allowed to request a first to answer call.

When set to "false" the MCPTT user is not allowed to request a first to answer call.

### 5.2.48T1	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallTransfer

Table 5.2.48T1.1: /<x>/<x>/OnNetwork/PrivateCall/AllowedCallTransfer

| <x>/OnNetwork/PrivateCall/AllowedCallTransfer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to request a call transfer. |  |  |  |  |

When set to "true" the MCPTT user is allowed to request a call transfer.

When set to "false" the MCPTT user is not allowed to request a call transfer.

### 5.2.48T2	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallTransferAny

Table 5.2.48T2.1: /<x>/<x>/OnNetwork/PrivateCall/AllowedCallTransferAny

| <x>/OnNetwork/PrivateCall/AllowedCallTransferAny |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to request a call transfer to any target user. |  |  |  |  |

When set to "true" the MCPTT user is allowed to request a call transfer to any target user.

When set to "false" the MCPTT user is not allowed to request a call transfer to any user, permission of call transfer is checked based on on entries in the <AllowedTargetMCPTTIDList>/<AllowedTargetFunctionalAliasIDList> as defined in 3GPP TS 24.484 [12] using the procedures defined in 3GPP TS 24.379 [7].

### 5.2.48T3	/<x>/<x>/OnNetwork/PrivateCall/AllowedCallForwardManualInput

Table 5.2.48T3.1: /<x>/<x>/OnNetwork/PrivateCall/AllowedCallForwardManualInput

| <x>/OnNetwork/PrivateCall/AllowedCallForwardManualInput |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to request a call forwarding based on manual user input. |  |  |  |  |

When set to "true" the MCPTT user is allowed to request a call forwarding based on manual user input. The target of the forwarding is not restricted (e.g. can be any MCPTT user).

When set to "false" the MCPTT user is not allowed to request a call forwarding based on manual user input.

### 5.2.48U1	/<x>/<x>/OnNetwork/RemoteGroupSelection

Table 5.2.48U1.1: /<x>/<x>/OnNetwork/RemoteGroupSelection

| <x>/OnNetwork/RemoteGroupSelection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the remote group selection configuration. |  |  |  |  |

### 5.2.48U2	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>

Table 5.2.48U2.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>

| <x>/OnNetwork/RemoteGroupSelection/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more remote group selection configuration elements. |  |  |  |  |

### 5.2.48U3	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry

Table 5.2.48U3.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCPTT users whose selected MCPTT group is allowed to be remotely changed by the MCPTT user. |  |  |  |  |

### 5.2.48U4	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCPTTID

Table 5.2.48U4.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCPTTID

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCPTTID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a MCPTT ID of an MCPTT user whose selected MCPTT group is allowed to be remotely changed by the MCPTT user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 5.2.48U5	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/
DisplayName

Table 5.2.48U5.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/DisplayName

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCPTT ID of the MCPTT user. |  |  |  |  |

### 5.2.48V1	Void

### 5.2.48V2	Void

### 5.2.48V3	Void

### 5.2.48V4	Void

### 5.2.48V5	Void

### 5.2.48V6	Void

### 5.2.48V7	Void

### 5.2.48V8	Void

### 5.2.48V9	Void

### 5.2.48V10	Void

### 5.2.48V11	Void

### 5.2.48V12	Void

### 5.2.48V13	Void

### 5.2.48V14	Void

### 5.2.48V15	Void

### 5.2.48V16	Void

### 5.2.48V17	Void

### 5.2.48W1	/<x>/<x>/OnNetwork/AllowedRequestRemoteInitPrivateCall

Table 5.2.48W1.1: /<x>/<x>/OnNetwork/AllowedRequestRemoteInitPrivateCall

| <x>/OnNetwork/AllowedRequestRemoteInitPrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to request a remotely initiated private call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to request remotely initiated private calls.

When set to "false" the MCPTT user is not authorised to request remotely initiated private calls.

### 5.2.48W2	/<x>/<x>/OnNetwork/AllowedRequestRemoteInitGroupCall

Table 5.2.48W2.1: /<x>/<x>/OnNetwork/AllowedRequestRemoteInitGroupCall

| <x>/OnNetwork/AllowedRequestRemoteInitGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to request a remotely initiated group call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to request remotely initiated group calls.

When set to "false" the MCPTT user is not authorised to request remotely initiated group calls.

### 5.2.48W3	/<x>/<x>/OnNetwork/FunctionalAliasList

Table 5.2.48W3.1: /<x>/<x>/OnNetwork/FunctionalAliasList

| <x>/OnNetwork/FunctionalAliasList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias configuration. |  |  |  |  |

### 5.2.48W4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>

Table 5.2.48W4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>

| <x>/OnNetwork/FunctionalAliasList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more functional alias configuration elements. |  |  |  |  |

### 5.2.48W5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry

Table 5.2.48W5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the functional alias. |  |  |  |  |

### 5.2.48W6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FunctionalAlias

Table 5.2.48W6.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FunctionalAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a functional alias that can be activated by the MCPTT user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 5.2.48W6A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation

Table 5.2.48W6A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains the location criteria for activation of a functional alias. |  |  |  |  |

### 5.2.48W6A0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>

Table 5.2.48W6A0.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node contains the zero or more location criteria for activation of a functional alias. |  |  |  |  |

### 5.2.48W6A1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea

Table 5.2.48W6A1.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/
EnterSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers the functional alias activation. |  |  |  |  |

### 5.2.48W6A2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea

Table 5.2.48W6A2.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/
EnterSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 5.2.48W6A3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>

Table 5.2.48W6A3.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/
EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 5.2.48W6A3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 5.2.48W6A3A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6A4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 5.2.48W6A4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 5.2.48W6A5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea

Table 5.2.48W6A6.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/
EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 5.2.48W6A7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center

Table 5.2.48W6A7.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/
EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6A7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48W6A7A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6A8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 5.2.48W6A8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 5.2.48W6A9.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
EnterSpecificArea/EllipsoidArcArea/Radius

Table 5.2.48W6A10.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 5.2.48W6A11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
EnterSpecificArea/EllipsoidArcArea/OffsetAngle

Table 5.2.48W6A11.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 5.2.48W6A12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/<x>/
EnterSpecificArea/EllipsoidArcArea/IncludedAngle

Table 5.2.48W6A12.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 5.2.48W6A12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed

Table 5.2.48W6A12A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48W6A12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/
MinimumSpeed

Table 5.2.48W6A12B.1:	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6A12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/
MaximumSpeed

Table 5.2.48W6A12C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6A12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading

Table 5.2.48W6A12D.1:	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48W6A12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/
MinimumHeading

Table 5.2.48W6A12E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6A12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/
MaximumHeading

Table 5.2.48W6A12F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MaximmHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6A13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea

Table 5.2.48W6A13.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when exited by the MC service UE triggers the functional alias activation. |  |  |  |  |

### 5.2.48W6A14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/PolygonArea

Table 5.2.48W6A14.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 5.2.48W6A15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/PolygonArea/<x>

Table 5.2.48W6A15.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 5.2.48W6A15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 5.2.48W6A15A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 5.2.48W6A16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude

Table 5.2.48W6A16.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/
PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

Table 5.2.48W6A17.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/EllipsoidArcArea

Table 5.2.48W6A18.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 5.2.48W6A19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/EllipsoidArcArea/Center

Table 5.2.48W6A19.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6A19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48W6A19A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6A20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 5.2.48W6A20.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 5.2.48W6A21.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/EllipsoidArcArea/Radius

Table 5.2.48W6A22.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 5.2.48W6A23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/EllipsoidArcArea/OffsetAngle

Table 5.2.48W6A23.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 5.2.48W6A24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/
ExitSpecificArea/EllipsoidArcArea/IncludedAngle

Table 5.2.48W6A24.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 5.2.48W6A24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed

Table 5.2.48W6A24A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48W6A24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/
MinimumSpeed

Table 5.2.48W6A24B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6A24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/
MaximumSpeed

Table 5.2.48W6A24C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6A24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading

Table 5.2.48W6A24D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48W6A24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/
MinimumHeading

Table 5.2.48W6A24E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6A24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/
MaximumHeading

Table 5.2.48W6A24F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation

Table 5.2.48W6B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains the location criteria for de-activation of a functional alias. |  |  |  |  |

### 5.2.48W6B0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>

Table 5.2.48W6B0.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node contains the zero or more location criteria for de-activation of a functional alias. |  |  |  |  |

### 5.2.48W6B1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea

Table 5.2.48W6B1.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers the functional alias de-activation. |  |  |  |  |

### 5.2.48W6B2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea

Table 5.2.48W6B2.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 5.2.48W6B3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>

Table 5.2.48W6B3.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 5.2.48W6B3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 5.2.48W6B3A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 5.2.48W6B4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 5.2.48W6B5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea

Table 5.2.48W6B6.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 5.2.48W6B7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center

Table 5.2.48W6B7.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48W6B7A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 5.2.48W6B8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 5.2.48W6B9.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Radius

Table 5.2.48W6B10.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 5.2.48W6B11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 5.2.48W6B11.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 5.2.48W6B12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 5.2.48W6B12.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 5.2.48W6B12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed

Table 5.2.48W6B12A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48W6B12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/
MinimumSpeed

Table 5.2.48W6B12B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6B12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/
MaximumSpeed

Table 5.2.48W6B12C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6B12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading

Table 5.2.48W6B12D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48W6B12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/
MinimumHeading

Table 5.2.48W6B12E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6B12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/
MaximumHeading

Table 5.2.48W6B12F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6B13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea

Table 5.2.48W6B13.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when exited by the MC service UE triggers the functional alias de-activation. |  |  |  |  |

### 5.2.48W6B14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea

Table 5.2.48W6B14.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 5.2.48W6B15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>

Table 5.2.48W6B15.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 5.2.48W6B15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 5.2.48W6B15A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 5.2.48W6B16.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 5.2.48W6B17.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea

Table 5.2.48W6B18.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 5.2.48W6B19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center

Table 5.2.48W6B19.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48W6B19A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 5.2.48W6B20.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 5.2.48W6B21.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Radius

Table 5.2.48W6B22.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 5.2.48W6B23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 5.2.48W6B23.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 5.2.48W6B24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 5.2.48W6B24.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 5.2.48W6B24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed

Table 5.2.48W6B24A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48W6B24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/
MinimumSpeed

Table 5.2.48W6B24B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6B24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/
MaximumSpeed

Table 5.2.48W6B24C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6B24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading

Table 5.2.48W6B24D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48W6B24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/
MinimumHeading

Table 5.2.48W6B24E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6B24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/
MaximumHeading

Table 5.2.48W6B24F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
ManualDeactivationNotAllowedIfLocationCriteriaMet

Table 5.2.48W6C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
ManualDeactivationNotAllowedIfLocationCriteriaMet

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/ManualDeactivationNotAllowedIfLocationCriteriaMet |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to de-activate a functional alias if the location criteria is met. |  |  |  |  |

### 5.2.48W7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
DisplayName

Table 5.2.48W7.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/DisplayName

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to a functional alias that can be activated by the MCPTT user. |  |  |  |  |

### 5.2.48W7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
MaxSimultaneousEmergencyGroupCalls

Table 5.2.48W7A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
MaxSimultaneousEmergencyGroupCalls

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/MaxSimultaneousEmergencyGroupCalls |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous emergency group calls for a functional alias. |  |  |  |  |

### 5.2.48W7B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall

Table 5.2.48W7B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias configuration regarding the authorized functional aliases to be called in a private call by the functional alias indicated in the parent Entry node. |  |  |  |  |

### 5.2.48W7C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall/<x>

Table 5.2.48W7C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more functional alias configuration elements indicating the functional aliases that can be called in a private call by the functional alias indicated in the parent Entry node. |  |  |  |  |

### 5.2.48W7D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall/<x>/Entry

Table 5.2.48W7D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>/Entry

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the functional alias that can be called in a private call by the functional alias indicated in the parent Entry node. |  |  |  |  |

### 5.2.48W7E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall/<x>/Entry/FunctionalAlias

Table 5.2.48W7E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/
<x>/Entry/FunctionalAlias

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>/Entry/FunctionalAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a functional alias that can be called in a private call by the functional alias indicated in the parent Entry node. |  |  |  |  |

### 5.2.48W8	/<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser

Table 5.2.48W8.1: /<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser

| <x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised query the functional alias(es) activated by another MCPTT user. |  |  |  |  |

When set to "true" the MCPTT user is authorised to query the functional alias(es) activated by another MCPTT user.

When set to "false" the MCPTT user is not authorised to query the functional alias(es) activated by another MCPTT user.

### 5.2.48W9	/<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser

Table 5.2.48W9.1: /<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser

| <x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to take over the functional alias(es) previously activated by another MCPTT user. |  |  |  |  |

When set to "true" the MCPTT user is authorised to take over the functional alias(es) previously activated by another MCPTT user.

When set to "false" the MCPTT user is not authorised to take over the functional alias(es) previously activated by another MCPTT user.

### 5.2.48W10	/<x>/<x>/OnNetwork/AllowedLocationInfoWhenTalking

Table 5.2.48W10.1: /<x>/<x>/OnNetwork/AllowedLocationInfoWhenTalking

| <x>/OnNetwork/AllowedLocationInfoWhenTalking |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to send its location information when it is requesting the floor. |  |  |  |  |

When set to "true" the MCPTT user is authorised to send its location information when it is requesting the floor.

When set to "false" the MCPTT user is not authorised to send its location information when it is requesting the floor.

### 5.2.48W11	/<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding

Table 5.2.48W11.1: /<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding

| <x>/OnNetwork/AllowedFunctionalAliasGroupBinding |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is authorised to request the binding of a particular functional alias with a group or list of groups. |  |  |  |  |

When set to "true" the MCPTT user is authorised to request the binding of a particular functional alias with a group or list of groups.

When set to "false" the MCPTT user is not authorised to request the binding of a particular functional alias with a group or list of groups.

### 5.2.48W12	/<x>/<x>/OnNetwork/MCPTTAdhocGroup

Table 5.2.48W12.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup

| <x>/OnNetwork/MCPTTAdhocGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT adhoc group call and alert related configurations. |  |  |  |  |

### 5.2.48W12A	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert

Table 5.2.48W12A.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert

| <x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT adhoc group emergency alert policy. |  |  |  |  |

### 5.2.48W12A1	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/
Authorised

Table 5.2.48W12A1.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/Authorised

| <x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to activate an MCPTT adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make an MCPTT adhoc group emergency alert.

When set to "false" the MCPTT user is not authorised to make an MCPTT adhoc group emergency alert.

### 5.2.48W12A2	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/
Cancel

Table 5.2.48W12A2.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/Cancel

| <x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/Cancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel an MCPTT adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCPTT user is authorised to cancel an MCPTT adhoc group emergency alert.

When set to "false" the MCPTT user is not authorised to cancel an MCPTT adhoc group emergency alert.

### 5.2.48W12A3	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/
AuthRecvParticipantInfo

Table 5.2.48W12A3.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/AuthRecvParticipantInfo

| <x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/AuthRecvParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive adhoc group emergency alert participants information. |  |  |  |  |

When set to "true" the MCPTT user is authorised to receive adhoc group emergency alert participants information.

When set to "false" the MCPTT user is not authorised to receive adhoc group emergency alert participants information.

### 5.2.48W12A4	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/
AuthSetupAdhocGroupCall

Table 5.2.48W12A4.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/AuthSetupAdhocGroupCall

| <x>/OnNetwork/MCPTTAdhocGroup/EmergencyAlert/AuthSetupAdhocGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to set up an adhoc group call using the adhoc group used for the adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCPTT user is authorised to set up an adhoc group call using the adhoc group used for the adhoc group emergency alert.

When set to "false" the MCPTT user is not authorised to set up an adhoc group call using the adhoc group used for the adhoc group emergency alert.

### 5.2.48W12B	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall

Table 5.2.48W12B.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall

| <x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT adhoc group call related policy. |  |  |  |  |

### 5.2.48W12B1	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/
Authorised

Table 5.2.48W12B1.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/Authorised

| <x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCPTT adhoc group call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make an MCPTT adhoc group call.

When set to "false" the MCPTT user is not authorised to make an MCPTT adhoc group call.

### 5.2.48W12B2	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/
AuthorisedParticipation

Table 5.2.48W12B2.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthorisedParticipation

| <x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthorisedParticipation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to participate in MCPTT adhoc group call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to participate in MCPTT adhoc group calls that they are invited.

When set to "false" the MCPTT user is not authorised to participate in MCPTT adhoc group calls that they are invited.

### 5.2.48W12B3	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/
AuthInitEmergencyCall

Table 5.2.48W12B3.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthInitEmergencyCall

| <x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthInitEmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCPTT adhoc group emergency call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make an MCPTT adhoc group emergency call.

When set to "false" the MCPTT user is not authorised to make an MCPTT adhoc group emergency call.

### 5.2.48W12B4	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/
AuthInitImminentPerilCall

Table 5.2.48W12B4.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthInitImminentPerilCall

| <x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthInitImminentPerilCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCPTT adhoc group imminent-peril call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to make an MCPTT adhoc group imminent-peril call.

When set to "false" the MCPTT user is not authorised to make an MCPTT adhoc group imminent-peril call.

### 5.2.48W12B5	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/
AuthRecvCallParticipantInfo

Table 5.2.48W12B5.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthRecvCallParticipantInfo

| <x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthRecvCallParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive adhoc group call participants information. |  |  |  |  |

When set to "true" the MCPTT user is authorised to receive adhoc group call participants information.

When set to "false" the MCPTT user is not authorised to receive adhoc group call participants information.

### 5.2.48W12B6	/<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/
AuthModifyCallParticipantInfo

Table 5.2.48W12B6.1: /<x>/<x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthModifyCallParticipantInfo

| <x>/OnNetwork/MCPTTAdhocGroup/AdhocGroupCall/AuthModifyCallParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to modify adhoc group call participants information. |  |  |  |  |

When set to "true" the MCPTT user is authorised to modify adhoc group call participants information.

When set to "false" the MCPTT user is not authorised to modify adhoc group call participants information.

### 5.2.48X	/<x>/<x>/OnNetwork/AuthorisedIncomingAny

Table 5.2.48X.1: /<x>/<x>/OnNetwork/AuthorisedIncomingAny

| <x>/OnNetwork/AuthorisedIncomingAny |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive a MCPTT private call by any MCPTT user. |  |  |  |  |

When set to "true" the MCPTT user is authorised to receive an MCPTT private call by any MCPTT user.

When set to "false" the MCPTT user is not authorised to receive an MCPTT private call by any MCPTT user.

### 5.2.48Y1	/<x>/<x>/OnNetwork/UserList

Table 5.2.48Y1.1: /<x>/<x>/OnNetwork/UserList

| <x>/OnNetwork/UserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCPTT user(s) who can call this particular user in a MCPTT private call. |  |  |  |  |

### 5.2.48Y2	/<x>/<x>/OnNetwork/UserList/<x>

Table 5.2.48Y2.1: /<x>/<x>/OnNetwork/UserList/<x>

| <x>/OnNetwork/UserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCPTT users who can call this particular user in a MCPTT private call. |  |  |  |  |

### 5.2.48Y3	/<x>/<x>/OnNetwork/UserList/<x>/Entry

Table 5.2.48Y3.1: /<x>/<x>/OnNetwork/UserList/<x>/Entry

| <x>/OnNetwork/UserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCPTT users who can call the particular user in a private call. |  |  |  |  |

### 5.2.48Y4	/<x>/<x>/OnNetwork/UserList/<x>/Entry/MCPTTID

Table 5.2.48Y4.1: /<x>/<x>/OnNetwork/UserList/<x>/Entry/MCPTTID

| <x>/OnNetwork/UserList/<x>/Entry/MCPTTID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 5.2.48Y5	/<x>/<x>/OnNetwork/UserList/<x>/Entry/PrivateCallKMSURI

Table 5.2.48Y5.1: /<x>/<x>/OnNetwork/UserList/<x>/Entry/PrivateCallKMSURI

| <x>/OnNetwork/UserList/<x>/Entry/PrivateCallKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS associated with the MCPTTID. If the value is empty, the KMS leaf node present in the MCS UE initial configuration MO is used. |  |  |  |  |

### 5.2.48Z	/<x>/<x>/OnNetwork/AuthorisedReceiveNonAcknowledged

Table 5.2.48Z.1: /<x>/<x>/OnNetwork/AuthorisedReceiveNonAcknowledged

| <x>/OnNetwork/AuthorisedReceiveNonAcknowledged |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive the list of non-acknowledged MCPTT users to a group session invitation. |  |  |  |  |

When set to "true" the MCPTT user is authorised to receive information about the MCPTT Users that did not acknowledge a group session invitation.

When set to "false" the MCPTT user is not authorised to receive information about MCPTT Users that did not acknowledge a group session invitation.

### 5.2.49	/<x>/<x>/OffNetwork

Table 5.2.49.1: /<x>/<x>/OffNetwork

| <x>/OffNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for off-network operation. |  |  |  |  |

### 5.2.50	/<x>/<x>/OffNetwork/Authorised

Table 5.2.50.1: /<x>/<x>/OffNetwork/Authorised

| <x>/OffNetwork/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation for off-network services. |  |  |  |  |

When set to "true" the MCPTT user is authorised for off-network.

When set to "false" the MCPTT user is not authorised for off-network operation.

### 5.2.51	/<x>/<x>/OffNetwork/MCPTTGroupInfo

Table 5.2.51.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo

| <x>/OffNetwork/MCPTTGroupInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for group information. |  |  |  |  |

### 5.2.52	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>

Table 5.2.52.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>

| <x>/OffNetwork/MCPTTGroupInfo/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more group information. |  |  |  |  |

### 5.2.52A	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry

Table 5.2.52A.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry

| <x>/OffNetwork/MCPTTGroupInfo/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one off-network MCPTT group for use by an MCPTT user. |  |  |  |  |

### 5.2.53	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/MCPTTGroupID

Table 5.2.53.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/MCPTTGroupID

| <x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/MCPTTGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates an off-network MCPTT group for use by an MCPTT user. |  |  |  |  |

### 5.2.53A	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/DisplayName

Table 5.2.53A.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/DisplayName

| <x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCPTT group represented by the MCPTT group ID. |  |  |  |  |

### 5.2.53B	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/
RelativePresentationPriority

Table 5.2.53B.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/RelativePresentationPriority

| <x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/RelativePresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates indicating the presentation priority of the off-network group for the MCPTT user relative to other off-network groups and off-network users. |  |  |  |  |

-	Values: 0-255

The lowest RelativePresentationPriority value shall be considered as the MCPTT group transaction having the lowest priority for presentation among other group MCPTT and one-to-one user transactions.

### 5.2.53C	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/
GMSServID

Table 5.2.53C.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/GMSServID

| <x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/GMSServID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the GMS owning a specific group contained in the off-network MCPTTGroupList. |  |  |  |  |

### 5.2.53D	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/
IDMSTokenEndPoint

Table 5.2.53D.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/IDMSTokenEndPoint

| <x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/IDMSTokenEndPoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the IDMS token endpoint for a specific group contained in the off-network MCPTTGroupList. If the value is empty, the IDMS identities (IDMSAuthEndpoint and IDMSTokenEndpoint) present in the MCS UE initial configuration MO are used. |  |  |  |  |

### 5.2.53E	/<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/GroupKMSURI

Table 5.2.53E.1: /<x>/<x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/GroupKMSURI

| <x>/OffNetwork/MCPTTGroupInfo/<x>/Entry/GroupKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS identity (URI) for a specific group contained in the off-network MCPTTGroupList. If the value is empty, the KMS identity (URI) (kms) present in the MCS UE initial configuration MO is used. |  |  |  |  |

### 5.2.54	/<x>/<x>/OffNetwork/AllowedListen

Table 5.2.54.1: /<x>/<x>/OffNetwork/AllowedListen

| <x>/OffNetwork/AllowedListen |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to listen both overriding and overridden. |  |  |  |  |

When set to "true" the MCPTT user is allowed to listen both overriding and ooverridden.

When set to "false" the MCPTT user is not allowed to listen both overriding and overridden.

### 5.2.55	/<x>/<x>/OffNetwork/AllowedTransmission

Table 5.2.55.1: /<x>/<x>/OffNetwork/AllowedTransmission

| <x>/OffNetwork/AllowedTransmission |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCPTT user is allowed to transmit in case of override (overriding and/or overridden). |  |  |  |  |

When set to "true" the MCPTT user is allowed to transmit in case of override (overriding and/or overridden).

When set to "false" the MCPTT user is not allowed to transmit in case of override (overriding and/or overridden).

### 5.2.56	/<x>/<x>/OffNetwork/EmergencyCallChange

Table 5.2.56.1: /<x>/<x>/OffNetwork/EmergencyCallChange

| <x>/OffNetwork/EmergencyCallChange |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorization for a participant to change an off-network group call in-progress to an off-network MCPTT emergency group call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to change an MCPTT emergency group call.

When set to "false" the MCPTT user is not authorised to change an MCPTT emergency group call.

### 5.2.57	/<x>/<x>/OffNetwork/ImminentPerilCallChange

Table 5.2.57.1: /<x>/<x>/OffNetwork/ImminentPerilCallChange

| <x>/OffNetwork/ImminentPerilCallChange |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorization for a participant to change an off-network group call in-progress to an off-network MCPTT imminent peril group call. |  |  |  |  |

When set to "true" the MCPTT user is authorised to change an MCPTT imminent peril group call.

When set to "false" the MCPTT user is not authorised to change an MCPTT imminent peril group call.

### 5.2.58	/<x>/<x>/OffNetwork/UserInfoID

Table 5.2.58.1: /<x>/<x>/OffNetwork/UserInfoID

| <x>/OffNetwork/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 5.2.58A1	Void

### 5.2.58A2	Void

### 5.2.58A3	Void

### 5.2.58A4	Void

### 5.2.58A5	Void

### 5.2.58A6	Void

### 5.2.58A7	Void

### 5.2.58A8	Void

### 5.2.58A9	Void

### 5.2.58A10	Void

### 5.2.58A11	Void

### 5.2.58A12	Void

### 5.2.58A13	Void

### 5.2.58A14	Void

### 5.2.58A15	Void

### 5.2.58A16	Void

### 5.2.58A17	Void

### 5.2.59	/<x>/Status

Table 5.2.59.1: /<x>/Status

| Status |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether this MCPTT user profile is enabled or disabled. |  |  |  |  |

When set to "true" this MCPTT user profile is enabled.

When set to "false" this MCPTT user profile is disabled.

# 6	MCS group configuration MO

## 6.1	General

The MCS group configuration Management Object (MO) is used to configure the MCS Client behaviour for the on-network or off-network MC services. The MCS group configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

NOTE:	For historical reasons some of the elements in the MCS group configuration Management Object (MO) use the terminology "MCPTT", however this MO is common to all MCS with some MCPTT specific elements, some MCData specific elements and some MCVideo specific elements. Not all elements that contain the terminology "MCPTT" are just MCPTT specific. In the clauses that follow, it is made clear what aspects are specific to MCPTT and what aspects are specific to all MC services. MCData specific elements are contained under "MCData" nodes. MCVideo specific elements are contained under "MCVideo" nodes.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCPTT-group-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCS group configuration MO.

The following nodes and leaf objects are possible under the MCS group configuration node as described in figure 6.1.1.

Figure 6.1.1: The MCS group configuration MO (1 of 5)

Figure 6.1.2: The MCS group configuration MO (2 of 5)

Figure 6.1.3: The MCS group configuration MO (3 of 5)

Figure 6.1.4: The MCS group configuration MO (4 of 5)

Figure 6.1.5: The MCS group configuration MO (5 of 5)

## 6.2	MCS group configuration MO parameters

### 6.2.1	General

This clause describes the parameters for the MCS group configuration Management Object (MO).

### 6.2.2	Node: <x>

Table 6.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCS group configuration Management Object (MO). For the MCS group configuration MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcptt-group configuration:1.0" |  |  |  |  |

-	Values: N/A

### 6.2.3	/<x>/Name

Table 6.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCS group configuration settings. |  |  |  |  |

-	Values: <User displayable name>

### 6.2.4	/<x>/Ext

Table 6.2.4.1: /<x>/Ext

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCSgroup configuration MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 6.2.5	/<x>/<x>

Table 6.2.5.1: /<x>/<x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation and off-network operation. |  |  |  |  |

### 6.2.6	/<x>/<x>/Common

Table 6.2.6.1: /<x>/<x>/Common

| <x>/Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 6.2.7	/<x>/<x>/Common/MCPTTGroupID

Table 6.2.7.1: /<x>/<x>/Common/MCPTTGroupID

| <x>/Common/MCPTTGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCS group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

NOTE:	This element is common for all MC services.

### 6.2.8	/<x>/<x>/Common/MCPTTGroupAlias

Table 6.2.8.1: /<x>/<x>/Common/MCPTTGroupAlias

| <x>/Common/MCPTTGroupAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCS group alias. |  |  |  |  |

The value is a <display-name> element specified in OMA OMA-TS-XDM_Group-V1_1 [4].

NOTE:	This element is common for all MC services.

### 6.2.9	/<x>/<x>/Common/MCPTTGroupMemberList

Table 6.2.9.1: /<x>/<x>/Common/MCPTTGroupMemberList

| <x>/Common/MCPTTGroupMemberList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCS group members (group membership information). |  |  |  |  |

NOTE:	This element is common for all MC services.

### 6.2.10	/<x>/<x>/Common/MCPTTGroupMemberList/<x>

Table 6.2.10.1: /<x>/<x>/Common/MCPTTGroupMemberList/<x>

| <x>/Common/MCPTTGroupMemberList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list(s) of MCS group members. |  |  |  |  |

NOTE:	This element is common for all MC services.

### 6.2.11	/<x>/<x>/Common/MCPTTGroupMemberList/<x>/MCPTTID

Table 6.2.11.1: /<x>/<x>/Common/MCPTTGroupMemberList/<x>/MCPTTID

| <x>/Common/MCPTTGroupMemberList/<x>/MCPTTID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCS user identity (MC Service ID) which is a globally unique identifier within the MC service that represents the MCS user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

NOTE:	This element is common for all MC services.

### 6.2.12	/<x>/<x>/Common/MCPTTGroupMemberList/<x>/UserPriority

Table 6.2.12.1: /<x>/<x>/Common/MCPTTGroupMemberList/<x>/UserPriority

| <x>/Common/MCPTTGroupMemberList/<x>/UserPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the user priority for the group. |  |  |  |  |

The value is a <user-priority> element specified in 3GPP TS 24.481 [9] clause 7.2.4.2.

NOTE:	This element is common for all MC services.

### 6.2.13	/<x>/<x>/Common/MCPTTGroupMemberList/<x>/ParticipantType

Table 6.2.13.1: /<x>/<x>/Common/MCPTTGroupMemberList/<x>/ParticipantType

| <x>/Common/MCPTTGroupMemberList/<x>/ParticipantType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the participant type for the group. |  |  |  |  |

The ParticipantType values are defined and configured by the Mission Critical Organisation (e.g., first responder, second responder, dispatcher, dispatch supervisor, MCS administrator).

NOTE:	This element is common for all MC services.

### 6.2.14	Void

### 6.2.15	/<x>/<x>/Common/MCPTTGroupOwner

Table 6.2.15.1: /<x>/<x>/Common/MCPTTGroupOwner

| <x>/Common/MCPTTGroupOwner |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the group's owner (Mission Critical Organisation). |  |  |  |  |

NOTE:	This element is common for all MC services.

### 6.2.16	/<x>/<x>/Common/PreferredVoiceCodec

Table 6.2.16.1: /<x>/<x>/Common/PreferredVoiceCodec

| <x>/Common/PreferredVoiceCodec |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCPTT preferred voice codec for an MCS group. |  |  |  |  |

The value of the preferred voice codec is a RTP payload format name specified in IETF RFC 4566 [13].

NOTE:	This element is specific for MCPTT.

### 6.2.17	/<x>/<x>/Common/MCPTTGroupLevel

Table 6.2.17.1: /<x>/<x>/Common/MCPTTGroupLevel

| <x>/Common/MCPTTGroupLevel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the level within a group hierarchy (only applicable for group-broadcast group). |  |  |  |  |

-	Values: 0-255

The group-broadcast group with the lowest MCPTTGroupLevel value shall be considered as the group-broadcast group having the lowest level among the groups.

NOTE:	This element is common for all MC services.

### 6.2.18	/<x>/<x>/Common/UserLevel

Table 6.2.18.1: /<x>/<x>/Common/UserLevel

| <x>/Common/UserLevel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the level within user hierarchy (only applicable for user-broadcast group). |  |  |  |  |

The user-broadcast group with the lowest UserLevel value shall be considered as the user-broadcast group having the lowest level among the groups.

NOTE:	This element is common for all MC services.

### 6.2.19	/<x>/<x>/Common/AllowedEmergencyCall

Table 6.2.19.1: /<x>/<x>/Common/AllowedEmergencyCall

| <x>/Common/AllowedEmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether an MCPTT emergency group call is permitted on the MCS group. |  |  |  |  |

When set to "true" the MCPTT emergency group call is allowed on the group.

When set to "false" the MCPTT emergency group call is not allowed on the group.

NOTE:	This element is specific for MCPTT.

### 6.2.20	/<x>/<x>/Common/AllowedImminentPerilCall

Table 6.2.20.1: /<x>/<x>/Common/AllowedImminentPerilCall

| <x>/Common/AllowedImminentPerilCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether an MCPTT imminent peril group call is permitted on the MCS group. |  |  |  |  |

When set to "true" the MCPTT imminent peril group call is allowed on the group.

When set to "false" the MCPTT imminent peril group call is not allowed on the group.

NOTE:	This element is specific for MCPTT.

### 6.2.21	/<x>/<x>/Common/AllowedEmergencyAlert

Table 6.2.21.1: /<x>/<x>/Common/AllowedEmergencyAlert

| <x>/Common/AllowedEmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether an MCPTT emergency alert is possible on the MCS group. |  |  |  |  |

When set to "true" the MCPTT emergency alert is allowed on the group.

When set to "false" the MCPTT emergency alert is not allowed on the group.

NOTE:	This element is for specific for MCPTT.

### 6.2.22	/<x>/<x>/Common/MediaProtectionRequired

Table 6.2.22.1: /<x>/<x>/Common/MediaProtectionRequired

| <x>/Common/MediaProtectionRequired |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether confidentiality and integrity of MCPTT media is required on the MCS group. |  |  |  |  |

When set to "true" confidentiality and integrity of media is required on the group.

When set to "false" confidentiality and integrity of media is not required on the group.

The default value is set to "true".

NOTE:	This element is specific for MCPTT.

### 6.2.22A	/<x>/<x>/Common/PreconfiguredGroupUseOnly

Table 6.2.22A.1: /<x>/<x>/Common/PreconfiguredGroupUseOnly

| <x>/Common/PreconfiguredGroupUseOnly |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCS group is to be used only to provide configuration information for a preconfigured regroup. |  |  |  |  |

When set to "true" the group may only be used to provide configuration information for a preconfigured regroup.

When set to "false" the group may be used for group calls as well as to provide configuration information for a preconfigured regroup.

The default value is set to "false".

NOTE:	This element is common for all MC services.

### 6.2.23	/<x>/<x>/Common/FloorControlProtectionRequired

Table 6.2.23.1: /<x>/<x>/Common/FloorControlProtectionRequired

| <x>/Common/FloorControlProtectionRequired |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether confidentiality and integrity of MCPTT floor control signalling is required on the MCS group. |  |  |  |  |

When set to "true" confidentiality and integrity of floor control signalling is requiredon the group.

When set to "false" confidentiality and integrity of floor control signalling is not requiredon the group.

The default value is set to "true".

NOTE:	This element is specific for MCPTT.

### 6.2.23A	/<x>/<x>/Common/MediaProtectionSecurityMaterial

Table 6.2.23A.1: /<x>/<x>/Common/MediaProtectionSecurityMaterial

| <x>/Common/MediaProtectionSecurityMaterial |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates security material (as specified in 3GPP TS 33.180 [20]) for MCPTT media protection in the MCS group. |  |  |  |  |

NOTE:	This element is specific for MCPTT.

### 6.2.23B	/<x>/<x>/Common/MCData

Table 6.2.23B.1: /<x>/<x>/Common/MCData

| <x>/Common/MCData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a Common MCData group information |  |  |  |  |

### 6.2.23C	/<x>/<x>/Common/MCData/MediaProtectionRequired

Table 6.2.23C.1: /<x>/<x>/Common/MCData/MediaProtectionRequired

| <x>/Common/MCData/MediaProtectionRequired |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether confidentiality and integrity of media is required for MCData transactions on the MCS group. |  |  |  |  |

When set to "true" confidentiality and integrity of MCData media is required on the group.

When set to "false" confidentiality and integrity of MCData media is not required on the group.

The default value is set to "true".

### 6.2.23D	/<x>/<x>/Common/MCData/TxControlProtectionRequired

Table 6.2.23D.1: /<x>/<x>/Common/MCData/TxControlProtectionRequired

| <x>/Common/MCData/TxControlProtectionRequired |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether confidentiality and integrity protection is required for on-network and off-network MCData transmission control signalling on the MCS group |  |  |  |  |

When set to "true" confidentiality and integrity of MCData transmission control signalling is required on the group.

When set to "false" confidentiality and integrity of MCData transmission control signalling is not required on the group.

The default value is set to "true".

### 6.2.23E	/<x>/<x>/Common/MCData/MediaProtectionSecurityMaterial

Table 6.2.23E.1: /<x>/<x>/Common/MCData/MediaProtectionSecurityMaterial

| <x>/Common/MCData/MediaProtectionSecurityMaterial |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates security material (as specified in 3GPP TS 33.180 [20]) for MCData media protection in the MCS group. |  |  |  |  |

### 6.2.23F	/<x>/<x>/Common/MCData/AllowedSDS

Table 6.2.23F.1: /<x>/<x>/Common/MCData/AllowedSDS

| <x>/Common/MCData/AllowedSDS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether Short Data Service (SDS) is enabled for the group. |  |  |  |  |

When set to "true" the Short Data Service (SDS) is allowed on the group.

When set to "false" the Short Data Service (SDS) is not allowed on the group.

### 6.2.23G	/<x>/<x>/Common/MCData/AllowedFD

Table 6.2.23G.1: /<x>/<x>/Common/MCData/AllowedFD

| <x>/Common/MCData/AllowedFD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether File Distribution (FD) is enabled for the group. |  |  |  |  |

When set to "true" File Distribution (FD) is allowed on the group.

When set to "false" File Distribution (FD) is not allowed on the group.

### 6.2.23H	/<x>/<x>/Common/MCData/AllowedConvMgmt

Table 6.2.23H.1: /<x>/<x>/Common/MCData/AllowedConvMgmt

| <x>/Common/MCData/AllowedConvMgmt |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether conversation management is enabled for the group. |  |  |  |  |

When set to "true" conversation management is allowed on the group.

When set to "false" conversation management is not allowed on the group.

### 6.2.23I	/<x>/<x>/Common/MCData/AllowedTxControl

Table 6.2.23I.1: /<x>/<x>/Common/MCData/AllowedTxControl

| <x>/Common/MCData/AllowedTxControl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether transmission control is enabled for the group. |  |  |  |  |

When set to "true" transmission control is allowed on the group.

When set to "false" transmission control is not allowed on the group.

### 6.2.23J	/<x>/<x>/Common/MCData/AllowedRxControl

Table 6.2.23J.1: /<x>/<x>/Common/MCData/AllowedRxControl

| <x>/Common/MCData/AllowedRxControl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether reception control is enabled for the group. |  |  |  |  |

When set to "true" reception control is allowed on the group.

When set to "false" reception control not allowed on the group.

### 6.2.23K	/<x>/<x>/Common/MCData/AllowedEnhSvc

Table 6.2.23K.1: /<x>/<x>/Common/MCData/AllowedEnhSvc

| <x>/Common/MCData/AllowedEnhSvc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether Enhanced Service is enabled for the group. |  |  |  |  |

When set to "true" Enhanced Service is allowed on the group.

When set to "false" Enhanced Service is not allowed on the group.

### 6.2.23L	/<x>/<x>/Common/MCData/EnhSvcOpValues

Table 6.2.23L.1: /<x>/<x>/Common/MCData/EnhSvcOpValues

| <x>/Common/MCData/EnhSvcOpValues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of operational values for Enhanced Service. |  |  |  |  |

### 6.2.23M	/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>

Table 6.2.23M.1: /<x>/<x>/Common/MCData/EnhSvcOpValues/<x>

| <x>/Common/MCData/EnhSvcOpValues/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of operational values for Enhanced Service. |  |  |  |  |

### 6.2.23N	/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/
EnhSvcOpValue

Table 6.2.23N.1: /<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/EnhSvcOpValue

| <x>/Common/MCData/EnhSvcOpValues/<x>/EnhSvcOpValue |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an enhanced service operational value. |  |  |  |  |

### 6.2.23NA	/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/
EnhSvcOpID

Table 6.2.23NA.1: /<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/EnhSvcOpID

| <x>/Common/MCData/EnhSvcOpValues/<x>/EnhSvcOpID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a unique id associated with enhanced service operational value. |  |  |  |  |

### 6.2.23O	/<x>/<x>/Common/MCData/AllowedTransmitDataInGroup

Table 6.2.23O.1: /<x>/<x>/Common/MCData/AllowedTransmitDataInGroup

| <x>/Common/MCData/AllowedTransmitDataInGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether an MC data user is permitted to transmit data in the group. |  |  |  |  |

When set to "true" an MC data user is allowed to transmit data in the group.

When set to "false" an MC data user is not allowed to transmit data in the group.

### 6.2.23P	/<x>/<x>/Common/MCData/MaxDataSingleRequest

Table 6.2.23P.1: /<x>/<x>/Common/MCData/MaxDataSingleRequest

| <x>/Common/MCData/MaxDataSingleRequest |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum amount of data (in megabytes) that an MCData user can transmit in a single request during group communication. |  |  |  |  |

-	Values: 0-65535

The MaxDataSingleRequest size is in megabytes.

### 6.2.23Q	/<x>/<x>/Common/MCData/MaxTimeSingleRequest

Table 6.2.23Q.1: /<x>/<x>/Common/MCData/MaxTimeSingleRequest

| <x>/Common/MCData/MaxTimeSingleRequest |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum amount of time that an MCData user can transmit for in a single request during group communication. |  |  |  |  |

-	Values: 0-65535

The MaxTimeSingleRequest time is in seconds.

### 6.2.23R	/<x>/<x>/Common/MCVideo

Table 6.2.23R.1: /<x>/<x>/Common/MCVideo

| <x>/Common/MCVideo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a Common MCVideo group information |  |  |  |  |

### 6.2.23S	/<x>/<x>/Common/MCVideo/MediaProtectionRequired

Table 6.2.23S.1: /<x>/<x>/Common/MCVideo/MediaProtectionRequired

| <x>/Common/MCVideo/MediaProtectionRequired |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether confidentiality and integrity of media is required for MCVideo calls on the MCS group. |  |  |  |  |

When set to "true" confidentiality and integrity of MCvideo media is required on the group.

When set to "false" confidentiality and integrity of MCvideo media is not required on the group.

The default value is set to "true".

### 6.2.23T	/<x>/<x>/Common/MCVideo/TxControlProtectionRequired

Table 6.2.23T.1: /<x>/<x>/Common/MCVideo/TxControlProtectionRequired

| <x>/Common/MCVideo/TxControlProtectionRequired |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether confidentiality and integrity protection is required for on-network and off-network MCVideo transmission control signalling on the MCS group |  |  |  |  |

When set to "true" confidentiality and integrity of MCVideo transmission control signalling is required on the group.

When set to "false" confidentiality and integrity of MCVideo transmission control signalling is not required on the group.

The default value is set to "true".

### 6.2.23U	/<x>/<x>/Common/MCVideo/MediaProtectionSecurityMaterial

Table 6.2.23U.1: /<x>/<x>/Common/MCVideo/MediaProtectionSecurityMaterial

| <x>/Common/MCVideo/MediaProtectionSecurityMaterial |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates security material (as specified in 3GPP TS 33.180 [20]) for MCVideo media protection in the MCS group. |  |  |  |  |

### 6.2.23V	/<x>/<x>/Common/MCVideo/AllowedAudioCodecs

Table 6.2.23V.1: /<x>/<x>/Common/MCVideo/AllowedAudioCodecs

| <x>/Common/MCVideo/AllowedAudioCodecs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of allowed audio codecs for MCVideo sessions on the group |  |  |  |  |

### 6.2.23W	/<x>/<x>/Common/MCVideo/AllowedAudioCodecs/<x>

Table 6.2.23W.1: /<x>/<x>/Common/MCVideo/AllowedAudioCodecs/<x>

| <x>/Common/MCVideo/AllowedAudioCodecs/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of allowed audio codecs for MCVideo sessions on the group |  |  |  |  |

### 6.2.23X	/<x>/<x>/Common/MCVideo/AllowedAudioCodecs/<x>/
AudioCodec

Table 6.2.23X.1: /<x>/<x>/Common/MCVideo/AllowedAudioCodecs/<x>/AudioCodec

| <x>/Common/MCVideo/AllowedAudioCodecs/<x>/AudioCodec |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the preferred RTP payload format to be used for audio encoding in MCVideo group sessions for the group. |  |  |  |  |

### 6.2.23Y	/<x>/<x>/Common/MCVideo/AllowedVideoCodecs

Table 6.2.23Y.1: /<x>/<x>/Common/MCVideo/AllowedVideoCodecs

| <x>/Common/MCVideo/AllowedVideoCodecs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of allowed video codecs for MCVideo sessions on the group |  |  |  |  |

### 6.2.23Z	/<x>/<x>/Common/MCVideo/AllowedVideoCodecs/<x>

Table 6.2.23Z.1: /<x>/<x>/Common/MCVideo/AllowedVideoCodecs/<x>

| <x>/Common/MCVideo/AllowedVideoCodecs/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of allowed video codecs for MCVideo sessions on the group |  |  |  |  |

### 6.2.23AA	/<x>/<x>/Common/MCVideo/AllowedVideoCodecs/<x>/
VideoCodec

Table 6.2.23AA.1: /<x>/<x>/Common/MCVideo/AllowedVideoCodecs/<x>/VideoCodec

| <x>/Common/MCVideo/AllowedVideoCodecs/<x>/VideoCodec |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the preferred RTP payload format to be used for video encoding in MCVideo group sessions for the group. |  |  |  |  |

### 6.2.23AB	/<x>/<x>/Common/MCVideo/AllowedVideoResolutions

Table 6.2.23AB.1: /<x>/<x>/Common/MCVideo/AllowedVideoResolutions

| <x>/Common/MCVideo/AllowedVideoResolutions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of allowed video resolutions used in MCVideo group sessions for the group. |  |  |  |  |

### 6.2.23AC	/<x>/<x>/Common/MCVideo/AllowedVideoResolutions/<x>

Table 6.2.23AC.1: /<x>/<x>/Common/MCVideo/AllowedVideoResolutions/<x>

| <x>/Common/MCVideo/AllowedVideoResolutions/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of allowed video resolutions used in MCVideo group sessions for the group. |  |  |  |  |

### 6.2.23AD	/<x>/<x>/Common/MCVideo/AllowedVideoResolutions/<x>/
VideoResolution

Table 6.2.23AD.1: /<x>/<x>/Common/MCVideo/AllowedVideoResolutions/<x>/VideoResolution

| <x>/Common/MCVideo/AllowedVideoResolutions/<x>/VideoResolution |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates a video resolution (number of horizontal pixels x number of vertical pixels) to be used in MCVideo group sessions for the group. |  |  |  |  |

### 6.2.23AE	/<x>/<x>/Common/MCVideo/AllowedVideoFrameRates

Table 6.2.23AE.1: /<x>/<x>/Common/MCVideo/AllowedVideoFrameRates

| <x>/Common/MCVideo/AllowedVideoFrameRates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of allowed video frame rates to be used in MCVideo group sessions for the group. |  |  |  |  |

### 6.2.23AF	/<x>/<x>/Common/MCVideo/AllowedVideoFrameRates/<x>

Table 6.2.23AF.1: /<x>/<x>/Common/MCVideo/AllowedVideoFrameRates/<x>

| <x>/Common/MCVideo/AllowedVideoFrameRates/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of allowed video frame rates to be used in MCVideo group sessions for the group. |  |  |  |  |

### 6.2.23AG	/<x>/<x>/Common/MCVideo/AllowedVideoFrameRates/<x>/
FrameRate

Table 6.2.23AG.1: /<x>/<x>/Common/MCVideo/AllowedVideoFrameRates/<x>/FrameRate

| <x>/Common/MCVideo/AllowedVideoFrameRates/<x>/FrameRate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a video frame rate (in frames per second) to be used in MCVideo group sessions for the group. |  |  |  |  |

### 6.2.23AH	/<x>/<x>/Common/MCVideo/AllowedUrgentRealTimeMode

Table 6.2.23AH.1: /<x>/<x>/Common/MCVideo/AllowedUrgentRealTimeMode

| <x>/Common/MCVideo/AllowedUrgentRealTimeMode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether urgent real time mode is allowed for MCVideo sessions on the group. |  |  |  |  |

When set to "true" urgent real-time video mode is allowed for MCVideo sessions on the group.

When set to "false" urgent real-time video mode is not allowed for MCVideo sessions on the group.

### 6.2.23AI	/<x>/<x>/Common/MCVideo/AllowedNonUrgentRealTimeMode

Table 6.2.23AI.1: /<x>/<x>/Common/MCVideo/AllowedNonUrgentRealTimeMode

| <x>/Common/MCVideo/AllowedNonUrgentRealTimeMode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether non-urgent real time mode is allowed for MCVideo sessions on the group. |  |  |  |  |

When set to "true" non-urgent real-time video mode is allowed for MCVideo sessions on the group.

When set to "false" non-urgent real-time video mode is not allowed for MCVideo sessions on the group.

### 6.2.23AJ	/<x>/<x>/Common/MCVideo/AllowedNonRealTimeMode

Table 6.2.23AJ.1: /<x>/<x>/Common/MCVideo/AllowedNonRealTimeMode

| <x>/Common/MCVideo/AllowedNonRealTimeMode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether non-real time mode is allowed for MCVideo sessions on the group. |  |  |  |  |

When set to "true" non-real-time video mode is allowed for MCVideo sessions on the group.

When set to "false" non-real-time video mode is not allowed for MCVideo sessions on the group.

### 6.2.23AK	/<x>/<x>/Common/MCVideo/ActiveNonRealTimeMode

Table 6.2.23AK.1: /<x>/<x>/Common/MCVideo/ActiveNonRealTimeMode

| <x>/Common/MCVideo/ActiveNonRealTimeMode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the active non-real-time mode used for MCVideo sessions on the group. |  |  |  |  |

The valid values are:

a)	"urgent-real-time";

b)	"non-urgent-real-time"; and

c)	"non-real-time".

### 6.2.23AL	/<x>/<x>/Common/MCVideo/MaxTransmitMembers

Table 6.2.23AL.1: /<x>/<x>/Common/MCVideo/MaxTransmitMembers

| <x>/Common/MCVideo/MaxTransmitMembers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous transmitting group members for MCVideo sessions on the group. |  |  |  |  |

-	Values: 0-65535

### 6.2.24	/<x>/<x>/OffNetwork

Table 6.2.24.1: /<x>/<x>/OffNetwork

| <x>/OffNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for off-network operation. |  |  |  |  |

### 6.2.25	/<x>/<x>/OffNetwork/MCPTTGroupParameter

Table 6.2.25.1: /<x>/<x>/OffNetwork/MCPTTGroupParameter

| <x>/OffNetwork/MCPTTGroupParameter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group parameters. |  |  |  |  |

### 6.2.26	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>

Table 6.2.26.1: /<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>

| <x>/OffNetwork/MCPTTGroupParameter/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more group parameters. |  |  |  |  |

### 6.2.27	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/
ProSeLayer2GroupID

Table 6.2.27.1: /<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/ProSeLayer2GroupID

| <x>/OffNetwork/MCPTTGroupParameter/<x>/ProSeLayer2GroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the Prose layer-2 group ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

NOTE:	This element is common for all MC services.

### 6.2.28	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/
IPMulticastAddress

Table 6.2.28.1: /<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/IPMulticastAddress

| <x>/OffNetwork/MCPTTGroupParameter/<x>/IPMulticastAddress |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the ProSe group IP multicast address as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The value is an IPv4 address or an IPv6 address.

NOTE:	This element is common for all MC services.

### 6.2.29	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/
RelayServiceCode

Table 6.2.29.1: /<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/RelayServiceCode

| <x>/OffNetwork/MCPTTGroupParameter/<x>/RelayServiceCode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

NOTE:	This element is common for all MC services.

### 6.2.30	/<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/IPVersions

Table 6.2.30.1: /<x>/<x>/OffNetwork/MCPTTGroupParameter/<x>/IPVersions

| <x>/OffNetwork/MCPTTGroupParameter/<x>/IPVersions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates whether IPv4 or IPv6 is used for the MCS group as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The value is "IPv4" or "IPv6".

NOTE:	This element is common for all MC services.

### 6.2.31	/<x>/<x>/OffNetwork/EmergencyCallCancel

Table 6.2.31.1: /<x>/<x>/OffNetwork/EmergencyCallCancel

| <x>/OffNetwork/EmergencyCallCancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timeout value for the cancellation of an in progress emergency group call for MCPTT. |  |  |  |  |

-	Values: 0-65535

The EmergencyCallCancel time is in seconds.

NOTE:	This element is specific for MCPTT.

### 6.2.32	/<x>/<x>/OffNetwork/ImminentPerilCallCancel

Table 6.2.32.1: /<x>/<x>/OffNetwork/ImminentPerilCallCancel

| <x>/OffNetwork/ImminentPerilCallCancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timeout value for the cancellation of an in progress MCPTT imminent peril group call. |  |  |  |  |

-	Values: 0-65535

The ImminentPerilCallCancel time is in seconds.

NOTE:	This element is specific for MCPTT.

### 6.2.33	/<x>/<x>/OffNetwork/HangTime

Table 6.2.33.1: /<x>/<x>/OffNetwork/HangTime

| <x>/OffNetwork/HangTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the group call hang timer. |  |  |  |  |

-	Values: 0-65535

The HangTime is in seconds.

NOTE:	This element is specific for MCPTT.

### 6.2.34	/<x>/<x>/OffNetwork/MaxDuration

Table 6.2.34.1: /<x>/<x>/OffNetwork/MaxDuration

| <x>/OffNetwork/MaxDuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the max duration of group calls. |  |  |  |  |

-	Values: 0-65535

The MaxDuration time is in seconds.

NOTE:	This element is specific for MCPTT.

### 6.2.34A	/<x>/<x>/OffNetwork/QueueUsage

Table 6.2.34A.1: /<x>/<x>/OffNetwork/QueueUsage

| <x>/OffNetwork/QueueUsage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates if queuing is enabled or not. |  |  |  |  |

When set to "true" queueing floor requests is enabled on the group.

When set to "false" queueing floor requests is not enabled on the group.

The default value is set to "true".

NOTE:	This element is specific for MCPTT.

### 6.2.35	/<x>/<x>/OffNetwork/DefaultPPPP

Table 6.2.35.1: /<x>/<x>/OffNetwork/DefaultPPPP

| <x>/OffNetwork/DefaultPPPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default ProSe Per-Packet Priority (PPPP) configuration. |  |  |  |  |

### 6.2.36	/<x>/<x>/OffNetwork/DefaultPPPP/MCPTTGroupCallSignalling

Table 6.2.36.1: /<x>/<x>/OffNetwork/DefaultPPPP/MCPTTGroupCallSignalling

| <x>/OffNetwork/DefaultPPPP/MCPTTGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the group call signalling for MCPTT. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.37	/<x>/<x>/OffNetwork/DefaultPPPP/MCPTTGroupCallMedia

Table 6.2.37.1: /<x>/<x>/OffNetwork/DefaultPPPP/MCPTTGroupCallMedia

| <x>/OffNetwork/DefaultPPPP/MCPTTGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the group call media for MCPTT. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.38	/<x>/<x>/OffNetwork/DefaultPPPP/
MCPTTEmergencyGroupCallSignalling

Table 6.2.38.1: /<x>/<x>/OffNetwork/DefaultPPPP/MCPTTEmergencyGroupCallSignalling

| <x>/OffNetwork/DefaultPPPP/MCPTTEmergencyGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCPTT emergency group call signalling. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.39	/<x>/<x>/OffNetwork/DefaultPPPP/
MCPTTEmergencyGroupCallMedia

Table 6.2.39.1: /<x>/<x>/OffNetwork/DefaultPPPP/MCPTTEmergencyGroupCallMedia

| <x>/OffNetwork/DefaultPPPP/MCPTTEmergencyGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCPTT emergency group call media. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.40	/<x>/<x>/OffNetwork/DefaultPPPP/
MCPTTImminentPerilGroupCallSignalling

Table 6.2.40.1: /<x>/<x>/OffNetwork/DefaultPPPP/MCPTTImminentPerilGroupCallSignalling

| <x>/OffNetwork/DefaultPPPP/MCPTTImminentPerilGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCPTT imminent peril group call signalling. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.41	/<x>/<x>/OffNetwork/DefaultPPPP/
MCPTTImminentPerilGroupCallMedia

Table 6.2.41.1: /<x>/<x>/OffNetwork/DefaultPPPP/MCPTTImminentPerilGroupCallMedia

| <x>/OffNetwork/DefaultPPPP/MCPTTImminentPerilGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCPTT imminent peril group call media. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.41A	/<x>/<x>/OffNetwork/DefaultPQI

Table 6.2.41A.1: /<x>/<x>/OffNetwork/DefaultPQI

| <x>/OffNetwork/DefaultPQI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default PQI configuration. |  |  |  |  |

### 6.2.41B	/<x>/<x>/OffNetwork/DefaultPQI/MCPTTGroupCallSignalling

Table 6.2.41B.1: /<x>/<x>/OffNetwork/DefaultPQI/MCPTTGroupCallSignalling

| <x>/OffNetwork/DefaultPQI/MCPTTGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the group call signalling for MCPTT. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.41C	/<x>/<x>/OffNetwork/DefaultPQI/MCPTTGroupCallMedia

Table 6.2.41C.1: /<x>/<x>/OffNetwork/DefaultPQI/MCPTTGroupCallMedia

| <x>/OffNetwork/DefaultPQI/MCPTTGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the group call media for MCPTT. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.41D	/<x>/<x>/OffNetwork/DefaultPQI/
MCPTTEmergencyGroupCallSignalling

Table 6.2.41D.1: /<x>/<x>/OffNetwork/DefaultPQI/MCPTTEmergencyGroupCallSignalling

| <x>/OffNetwork/DefaultPQI/MCPTTEmergencyGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCPTT emergency group call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.41E	/<x>/<x>/OffNetwork/DefaultPQI/
MCPTTEmergencyGroupCallMedia

Table 6.2.41E.1: /<x>/<x>/OffNetwork/DefaultPQI/MCPTTEmergencyGroupCallMedia

| <x>/OffNetwork/DefaultPQI/MCPTTEmergencyGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCPTT emergency group call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.41F	/<x>/<x>/OffNetwork/DefaultPQI/
MCPTTImminentPerilGroupCallSignalling

Table 6.2.41F.1: /<x>/<x>/OffNetwork/DefaultPQI/MCPTTImminentPerilGroupCallSignalling

| <x>/OffNetwork/DefaultPQI/MCPTTImminentPerilGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCPTT imminent peril group call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.41G	/<x>/<x>/OffNetwork/DefaultPQI/
MCPTTImminentPerilGroupCallMedia

Table 6.2.41G.1: /<x>/<x>/OffNetwork/DefaultPQI/MCPTTImminentPerilGroupCallMedia

| <x>/OffNetwork/DefaultPQI/MCPTTImminentPerilGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCPTT imminent peril group call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

NOTE:	This element is specific for MCPTT.

### 6.2.42	/<x>/<x>/OffNetwork/MCData

Table 6.2.42.1: /<x>/<x>/OffNetwork/MCData

| <x>/OffNetwork/MCData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for off-network MCData configuration. |  |  |  |  |

### 6.2.43	/<x>/<x>/OffNetwork/MCData/DefaultPPPP

Table 6.2.43.1: /<x>/<x>/OffNetwork/MCData/DefaultPPPP

| <x>/OffNetwork/MCData/DefaultPPPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default ProSe Per-Packet Priority (PPPP) configuration. |  |  |  |  |

### 6.2.44	/<x>/<x>/OffNetwork/MCData/DefaultPPPP/
MCDataGroupCallSignalling

Table 6.2.44.1: /<x>/<x>/OffNetwork/MCData/DefaultPPPP/MCDataGroupCallSignalling

| <x>/OffNetwork/MCData/DefaultPPPP/MCDataGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the group call signalling for MCData. |  |  |  |  |

-	Values: 1-8

The MCData user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCData user data having the highest priority among the MCData user data.

### 6.2.45	/<x>/<x>/OffNetwork/MCData/DefaultPPPP/
MCDataGroupCallMedia

Table 6.2.45.1: /<x>/<x>/OffNetwork/MCData/DefaultPPPP/MCDataGroupCallMedia

| <x>/OffNetwork/MCData/DefaultPPPP/MCDataGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the group call media for MCData. |  |  |  |  |

-	Values: 1-8

The MCData user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCData user data having the highest priority among the MCData user data.

### 6.2.45A	/<x>/<x>/OffNetwork/MCData/DefaultPQI

Table 6.2.45A.1: /<x>/<x>/OffNetwork/MCData/DefaultPQI

| <x>/OffNetwork/MCData/DefaultPQI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default PQI configuration. |  |  |  |  |

### 6.2.45B	/<x>/<x>/OffNetwork/MCData/DefaultPQI/
MCDataGroupCallSignalling

Table 6.2.45B.1: /<x>/<x>/OffNetwork/MCData/DefaultPQI/MCDataGroupCallSignalling

| <x>/OffNetwork/MCData/DefaultPQI/MCDataGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the group call signalling for MCData. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCData user data with the lowest PQI value shall be considered as the MCData user data having the highest priority among the MCData user data.

### 6.2.45C	/<x>/<x>/OffNetwork/MCData/DefaultPQI/
MCDataGroupCallMedia

Table 6.2.45C.1: /<x>/<x>/OffNetwork/MCData/DefaultPQI/MCDataGroupCallMedia

| <x>/OffNetwork/MCData/DefaultPQI/MCDataGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the group call media for MCData. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCData user data with the lowest PQI value shall be considered as the MCData user data having the highest priority among the MCData user data.

### 6.2.46	/<x>/<x>/OffNetwork/MCVideo

Table 6.2.46.1: /<x>/<x>/OffNetwork/MCVideo

| <x>/OffNetwork/MCVideo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for off-network MCVideo group configuration. |  |  |  |  |

### 6.2.47	/<x>/<x>/OffNetwork/MCVideo/ArbitrationApproach

Table 6.2.47.1: /<x>/<x>/OffNetwork/MCVideo/ArbitrationApproach

| <x>/OffNetwork/MCVideo/ArbitrationApproach |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the arbitration approach used for off-network video tranmissions on the group |  |  |  |  |

The valid values are:

a)	"single" which indicates transmission participants rely on a single participant designated as transmission arbitrator for the arbitration of transmission requests; and

b)	"self" which indicates each transmission participant arbitrates its own transmission based on its view of the topology.

### 6.2.48	/<x>/<x>/OffNetwork/MCVideo/MaxTransmissions

Table 6.2.48.1: /<x>/<x>/OffNetwork/MCVideo/MaxTransmissions

| <x>/OffNetwork/MCVideo/MaxTransmissions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates maximum number of simultaneous transmissions for off-network MCVideo procedures. |  |  |  |  |

-	Values: 0-65535

### 6.2.49	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP

Table 6.2.49.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPPPP

| <x>/OffNetwork/MCVideo/DefaultPPPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default ProSe Per-Packet Priority (PPPP) configuration. |  |  |  |  |

### 6.2.50	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/
MCVideoGroupCallSignalling

Table 6.2.50.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoGroupCallSignalling

| <x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the group call signalling for MCVideo. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.51	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/
MCVideoGroupCallMedia

Table 6.2.51.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoGroupCallMedia

| <x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the group call media for MCVideo. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.52	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/
MCVideoEmergencyGroupCallSignalling

Table 6.2.52.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoEmergencyGroupCallSignalling

| <x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoEmergencyGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo emergency group call signalling. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.53	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/
MCVideoEmergencyGroupCallMedia

Table 6.2.53.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoEmergencyGroupCallMedia

| <x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoEmergencyGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo emergency group call media. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.54	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/
MCVideoImminentPerilGroupCallSignalling

Table 6.2.54.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoImminentPerilGroupCallSignalling

| <x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoImminentPerilGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo imminent peril group call signalling. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.55	/<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/
MCVideoImminentPerilGroupCallMedia

Table 6.2.55.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoImminentPerilGroupCallMedia

| <x>/OffNetwork/MCVideo/DefaultPPPP/MCVideoImminentPerilGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo imminent peril group call media. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.55A	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI

Table 6.2.55A.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPQI

| <x>/OffNetwork/MCVideo/DefaultPQI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default PQI configuration. |  |  |  |  |

### 6.2.55B	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/
MCVideoGroupCallSignalling

Table 6.2.55B.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPQI/MCVideoGroupCallSignalling

| <x>/OffNetwork/MCVideo/DefaultPQI/MCVideoGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the group call signalling for MCVideo. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.55C	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/
MCVideoGroupCallMedia

Table 6.2.55C.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPQI/MCVideoGroupCallMedia

| <x>/OffNetwork/MCVideo/DefaultPQI/MCVideoGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the group call media for MCVideo. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.55D	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/
MCVideoEmergencyGroupCallSignalling

Table 6.2.55D.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPQI/MCVideoEmergencyGroupCallSignalling

| <x>/OffNetwork/MCVideo/DefaultPQI/MCVideoEmergencyGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCVideo emergency group call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.55E	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/
MCVideoEmergencyGroupCallMedia

Table 6.2.55E.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPQI/MCVideoEmergencyGroupCallMedia

| <x>/OffNetwork/MCVideo/DefaultPQI/MCVideoEmergencyGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCVideo emergency group call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.55F	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/
MCVideoImminentPerilGroupCallSignalling

Table 6.2.55F.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPQI/MCVideoImminentPerilGroupCallSignalling

| <x>/OffNetwork/MCVideo/DefaultPQI/MCVideoImminentPerilGroupCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCVideo imminent peril group call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.55G	/<x>/<x>/OffNetwork/MCVideo/DefaultPQI/
MCVideoImminentPerilGroupCallMedia

Table 6.2.55G.1: /<x>/<x>/OffNetwork/MCVideo/DefaultPQI/MCVideoImminentPerilGroupCallMedia

| <x>/OffNetwork/MCVideo/DefaultPQI/MCVideoImminentPerilGroupCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCVideo imminent peril group call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 6.2.56	/<x>/<x>/OffNetwork/MCVideo/MaxDuration

Table 6.2.56.1: /<x>/<x>/OffNetwork/MCVideo/MaxDuration

| <x>/OffNetwork/MCVideo/MaxDuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the max duration of MCVideo group calls. |  |  |  |  |

-	Values: 0-65535

The MaxDuration time is in seconds.

### 6.2.57	/<x>/<x>/OffNetwork/MCVideo/EmergencyCallCancel

Table 6.2.57.1: /<x>/<x>/OffNetwork/MCVideo/EmergencyCallCancel

| <x>/OffNetwork/MCVideo/EmergencyCallCancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timeout value for the cancellation of an in progress MCVideo emergency group call for MCVideo. |  |  |  |  |

-	Values: 0-65535

The EmergencyCallCancel time is in seconds.

### 6.2.58	/<x>/<x>/OffNetwork/MCVideo/ImminentPerilCallCancel

Table 6.2.58.1: /<x>/<x>/OffNetwork/MCVideo/ImminentPerilCallCancel

| <x>/OffNetwork/MCVideo/ImminentPerilCallCancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timeout value for the cancellation of an in progress MCVideo imminent peril group call. |  |  |  |  |

-	Values: 0-65535

The ImminentPerilCallCancel time is in seconds.

# 7	MCPTT service configuration MO

## 7.1	General

The MCPTT service configuration Management Object (MO) is used to configure MCPTT Client behaviour for the on-network or off-network MCPTT Service. The MCPTT service configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCPTT-service-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCPTT service configuration MO.

The following nodes and leaf objects are possible under the MCPTT service configuration node as described in figure 7.1.1 and figure 7.1.2.

Figure 7.1.1: The MCPTT service configuration MO

Figure 7.1.2: The MCPTTAdhocGroup configuration MO

## 7.2	MCPTT service configuration MO parameters

### 7.2.1	General

This clause describes the parameters for the MCPTT service configuration Management Object (MO).

### 7.2.2	Node: <x>

Table 7.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCPTT service configuration Management Object (MO). For the MCPTT service configuration MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcptt-service configuration:1.0" |  |  |  |  |

-	Values: N/A

### 7.2.3	/<x>/Name

Table 7.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCPTT service configuration settings. |  |  |  |  |

-	Values: <User displayable name>

### 7.2.4	/<x>/Ext/

Table 7.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCPTT service configuration MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 7.2.5	/<x>/Common

Table 7.2.5.1: /<x>/Common

| Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 7.2.6	/<x>/Common/BroadcastMCPTTGroupCall

Table 7.2.6.1: /<x>/Common/BroadcastMCPTTGroupCall

| Common/BroadcastMCPTTGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for Broadcast MCPTT Group Call configuration. |  |  |  |  |

### 7.2.7	/<x>/Common/BroadcastMCPTTGroupCall/
NumLevelGroupHierarchy

Table 7.2.7.1: /<x>/Common/BroadcastMCPTTGroupCall/NumLevelGroupHierarchy

| Common/BroadcastMCPTTGroupCall/NumLevelGroupHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the number of levels of group hierarchy for group-broadcast groups. |  |  |  |  |

-	Values: 0-255

The group-broadcast group with the lowest NumLevelGroupHierarchy value shall be considered as the group-broadcast group having the lowest level among the groups.

### 7.2.8	/<x>/Common/BroadcastMCPTTGroupCall/
NumLevelUserHierarchy

Table 7.2.8.1: /<x>/Common/BroadcastMCPTTGroupCall/NumLevelUserHierarchy

| Common/BroadcastMCPTTGroupCall/NumLevelUserHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the number of levels of user hierarchy for user-broadcast groups. |  |  |  |  |

-	Values: 0-255

The user-broadcast group with the lowest NumLevelUserHierarchy value shall be considered as the user-broadcast group the lowest level among the groups.

### 7.2.9	/<x>/Common/MinLengthAliasID

Table 7.2.9.1: /<x>/Common/MinLengthAliasID

| Common/MinLengthAliasID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates minimum length of an alphanumeric identifier (i.e., alias) assigned by an MCPTT administrator. |  |  |  |  |

-	Values: 0-255

### 7.2.10	/<x>/OffNetwork

Table 7.2.10.1: /<x>/OffNetwork

| OffNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for off-network operation. |  |  |  |  |

### 7.2.11	/<x>/OffNetwork/PrivateCall

Table 7.2.11.1: /<x>/OffNetwork/PrivateCall

| OffNetwork/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for private call configuration. |  |  |  |  |

### 7.2.12	/<x>/OffNetwork/PrivateCall/MaxDuration

Table 7.2.12.1: /<x>/OffNetwork/PrivateCall/MaxDuration

| OffNetwork/PrivateCall/MaxDuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates max private call (with floor control) duration. |  |  |  |  |

-	Values: 0-65535

The MaxDuration time is in seconds.

### 7.2.13	/<x>/OffNetwork/PrivateCall/HangTime

Table 7.2.13.1: /<x>/OffNetwork/PrivateCall/HangTime

| OffNetwork/PrivateCall/HangTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates hang timer for private calls (with floor control). |  |  |  |  |

-	Values: 0-65535

The HangTime is in seconds.

### 7.2.14	/<x>/OffNetwork/PrivateCall/CancelTimeout

Table 7.2.14.1: /<x>/OffNetwork/PrivateCall/CancelTimeout

| OffNetwork/PrivateCall/CancelTimeout |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates timeout value for the cancellation of an in progress emergency for an MCPTT private call. |  |  |  |  |

-	Values: 0-65535

The CancelTimeout is in seconds.

### 7.2.15	/<x>/OffNetwork/EmergencyCall

Table 7.2.15.1: /<x>/OffNetwork/EmergencyCall

| OffNetwork/EmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node indicates a placeholder for the MCPTT emergency call policy. |  |  |  |  |

### 7.2.16	/<x>/OffNetwork/EmergencyCall/MCPTTGroupTimeout

Table 7.2.16.1: /<x>/OffNetwork/EmergencyCall/MCPTTGroupTimeout

| OffNetwork/EmergencyCall/MCPTTGroupTimeout |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates time limit for an in progress MCPTT emergency call related to an MCPTT group. |  |  |  |  |

-	Values: 0-65535

The GroupTimeout is in seconds.

### 7.2.17	/<x>/OffNetwork/NumLevelHierarchy

Table 7.2.17.1: /<x>/OffNetwork/NumLevelHierarchy

| OffNetwork/NumLevelHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the number of levels of hierarchy for floor control override in off-network. |  |  |  |  |

-	Values: 4-256

The request with the lowest NumLevelHierarchy value shall be considered as the request having the lowest priority level given to override an active transmission among the requests.

### 7.2.18	/<x>/OffNetwork/TransmitTimeout

Table 7.2.18.1: /<x>/OffNetwork/TransmitTimeout

| OffNetwork/TransmitTimeout |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates transmit time limit from a single request to transmit in a group or private call. |  |  |  |  |

-	Values: 0-65535

The TransmitTimeout is in seconds.

### 7.2.19	/<x>/OffNetwork/TransmissionWarning

Table 7.2.19.1: /<x>/OffNetwork/TransmissionWarning

| OffNetwork/TransmissionWarning |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates configuration of warning time before time limit of transmission is reached (off-network). |  |  |  |  |

-	Values: 0-255

The TransmissionWarning time is in seconds.

### 7.2.20	/<x>/OffNetwork/HangTimeWarning

Table 7.2.20.1: /<x>/OffNetwork/HangTimeWarning

| OffNetwork/HangTimeWarning |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates configuration of warning time before hang time is reached (off-network). |  |  |  |  |

-	Values: 0-255

The HangTimeWarning time is in seconds.

### 7.2.21	/<x>/OffNetwork/DefaultPPPP

Table 7.2.21.1: /<x>/OffNetwork/DefaultPPPP

| OffNetwork/DefaultPPPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default ProSe Per-Packet Priority (PPPP) configuration. |  |  |  |  |

### 7.2.22	/<x>/OffNetwork/DefaultPPPP/MCPTTPrivateCallSignalling

Table 7.2.22.1: /<x>/OffNetwork/DefaultPPPP/MCPTTPrivateCallSignalling

| OffNetwork/DefaultPPPP/MCPTTPrivateCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCPTT private call signalling. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

### 7.2.23	/<x>/OffNetwork/DefaultPPPP/MCPTTPrivateCallMedia

Table 7.2.23.1: /<x>/OffNetwork/DefaultPPPP/MCPTTPrivateCallMedia

| OffNetwork/DefaultPPPP/MCPTTPrivateCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCPTT private call media. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

### 7.2.24	/<x>/OffNetwork/DefaultPPPP/
MCPTTEmergencyPrivateCallSignalling

Table 7.2.24.1: /<x>/OffNetwork/DefaultPPPP/MCPTTEmergencyPrivateCallSignalling

| OffNetwork/DefaultPPPP/MCPTTEmergencyPrivateCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCPTT emergency private call signalling. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

### 7.2.25	/<x>/OffNetwork/DefaultPPPP/MCPTTEmergencyPrivateCallMedia

Table 7.2.25.1: /<x>/OffNetwork/DefaultPPPP/MCPTTEmergencyPrivateCallMedia

| OffNetwork/DefaultPPPP/MCPTTEmergencyPrivateCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCPTT emergency private call media. |  |  |  |  |

-	Values: 1-8

The MCPTT user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

### 7.2.25A	/<x>/OffNetwork/DefaultPQI

Table 7.2.25A.1: /<x>/OffNetwork/DefaultPQI

| OffNetwork/DefaultPQI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default PQI configuration. |  |  |  |  |

### 7.2.25B	/<x>/OffNetwork/DefaultPQI/MCPTTPrivateCallSignalling

Table 7.2.25B.1: /<x>/OffNetwork/DefaultPQI/MCPTTPrivateCallSignalling

| OffNetwork/DefaultPQI/MCPTTPrivateCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCPTT private call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

### 7.2.25C	/<x>/OffNetwork/DefaultPQI/MCPTTPrivateCallMedia

Table 7.2.25C.1: /<x>/OffNetwork/DefaultPQI/MCPTTPrivateCallMedia

| OffNetwork/DefaultPQI/MCPTTPrivateCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCPTT private call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

### 7.2.25D	/<x>/OffNetwork/DefaultPQI/
MCPTTEmergencyPrivateCallSignalling

Table 7.2.25D.1: /<x>/OffNetwork/DefaultPQI/MCPTTEmergencyPrivateCallSignalling

| OffNetwork/DefaultPQI/MCPTTEmergencyPrivateCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCPTT emergency private call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

### 7.2.25E	/<x>/OffNetwork/DefaultPQI/MCPTTEmergencyPrivateCallMedia

Table 7.2.25E.1: /<x>/OffNetwork/DefaultPQI/MCPTTEmergencyPrivateCallMedia

| OffNetwork/DefaultPQI/MCPTTEmergencyPrivateCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCPTT emergency private call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCPTT user data with the lowest PQI value shall be considered as the MCPTT user data having the highest priority among the MCPTT user data.

### 7.2.26	/<x>/OffNetwork/LogMetadata

Table 7.2.26.1: /<x>/OffNetwork/LogMetadata

| OffNetwork/LogMetadata |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether logging of metadata for MCPTT group calls, MCPTT private calls and non-call activities is permitted. |  |  |  |  |

When set to "true" logging of metadata for MCPTT group calls, MCPTT private calls and non-call activities, is enabled.

When set to "false" logging of metadata for MCPTT group calls, MCPTT private calls and non-call activities, is not enabled.

### 7.2.27	/<x>/OnNetwork

Table 7.2.27.1: /<x>/OnNetwork

| OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 7.2.28	/<x>/OnNetwork/MCPTTAdhocGroup

Table 7.2.28.1: /<x>/OnNetwork/MCPTTAdhocGroup

| OnNetwork/MCPTTAdhocGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT adhoc group call related configurations. |  |  |  |  |

### 7.2.29	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall

Table 7.2.29.1: /<x>/OnNetwork/MCPTTAdhocGroup/GroupCall

| OnNetwork/MCPTTAdhocGroup/GroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCPTT adhoc group call policy and configurations. |  |  |  |  |

### 7.2.30	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/
AllowAdhocGroupCall

Table 7.2.30.1: /<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/AllowAdhocGroupCall

| OnNetwork/MCPTTAdhocGroup/GroupCall/AllowAdhocGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether on-network adhoc group calls support enabled or disabled. |  |  |  |  |

When set to "true" the MCPTT system supports the adhoc group call and is enabled to allow the MCPTT users to make an MCPTT adhoc group call.

When set to "false" the MCPTT system supports the adhoc group call and is disabled to not allow the MCPTT users to make an MCPTT adhoc group call.

### 7.2.31	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/MaxNoParticipants

Table 7.2.31.1: /<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/MaxNoParticipants

| OnNetwork/MCPTTAdhocGroup/GroupCall/MaxNoParticipants |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates maximum number of participants allowed in an adhoc group call and the value is left to deplyoment requirement. |  |  |  |  |

### 7.2.32	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/HangTime

Table 7.2.32.1: /<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/HangTime

| OnNetwork/MCPTTAdhocGroup/GroupCall/HangTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates hang timer for adhoc group calls. |  |  |  |  |

-	Values: 0-65535

The HangTime is in seconds.

### 7.2.33	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/
MaxDurationOfCall

Table 7.2.33.1: /<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/MaxDurationOfCall

| OnNetwork/MCPTTAdhocGroup/GroupCall/MaxDurationOfCall |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Status | Occurrence | Format | Min. Access Types |  |  |
|  |  | Required | One | int | Get, Replace |  |  |
|  |  | This leaf node indicates max adhoc group call duration. |  |  |  |  |  |

-	Values: 0-65535

The MaxDurationOfCall time is in seconds.

### 7.2.34	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/
AllowAdhocMultiTalker

Table 7.2.34.1: /<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/AllowAdhocMultiTalker

| OnNetwork/MCPTTAdhocGroup/GroupCall/AllowAdhocMultiTalker |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether multi-talker is supported on MCPTT adhoc group call. |  |  |  |  |

When set to "true" the MCPTT users are allowed to make an MCPTT adhoc group call with multi-talker.

When set to "false" the MCPTT users are not allowed to make an MCPTT adhoc group call with multi-talker.

### 7.2.35	/<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/MaxNoTalkers

Table 7.2.35.1: /<x>/OnNetwork/MCPTTAdhocGroup/GroupCall/MaxNoTalkers

| OnNetwork/MCPTTAdhocGroup/GroupCall/MaxNoTalkers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates maximum number of talkers in an adhoc group call. |  |  |  |  |

# 8	MCS UE initial configuration MO

## 8.1	General

The MCS UE initial configuration Management Object (MO) is used to configure the MCS Client behaviour for the on-network or off-network MCS Service. The MCS UE initial configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

NOTE:	For historical reasons some of the elements in the MCS UE initial MO uses the terminology "MCPTT", however this MO is common to all MCS with some MCPTT specific elements, some MCData specific elements and some MCVideo specific elements. Not all elements that contain the terminology "MCPTT" are just MCPTT specific.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCPTT-UE-initial-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCPTT user profile MO.

The following nodes and leaf objects are possible under the MCPTT node as described in figure 8.1.1, figure 8.1.2, figure 8.1.3, figure 8.1.4, figure 8.1.5, figure 8.1.6 and figure 8.1.7.

Figure 8.1.1: The MCS UE initial configuration MO (1 of 7)

Figure 8.1.2: The MCS UE initial configuration MO (2 of 7)

Figure 8.1.3: The MCS UE initial configuration MO (3 of 7)

Figure 8.1.4: The MCS UE initial configuration MO (4 of 7)

Figure 8.1.5: The MCS UE initial configuration MO (5 of 7)

Figure 8.1.6: The MCS UE initial configuration MO (6 of 7)

Figure 8.1.7: The MCS UE initial configuration MO (7 of 7)

## 8.2	MCS UE initial configuration MO parameters

### 8.2.1	General

This clause describes the parameters for the MCS UE initial configuration Management Object (MO).

### 8.2.2	Node: <x>

Table 8.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCS UE initial configuration Management Object (MO). For the MCS UE initial configuration MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcptt-ue-initial-configuration:1.0" |  |  |  |  |

-	Values: N/A

### 8.2.3	/<x>/Name

Table 8.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCPTT UE initial configuration settings. |  |  |  |  |

-	Values: <User displayable name>

### 8.2.4	/<x>/Ext/

Table 8.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCPTT UE initial configuration MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 8.2.5	/<x>/DefaultUserProfile

Table 8.2.5.1: /<x>/DefaultUserProfile

| DefaultUserProfile |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the default user profile. |  |  |  |  |

The DefaultUserProfile nodes are:

-	the same MCPTT user profile nodes defined in MCPTT user profile MO with a default MCPTT user ID;

-	the same MCData user profile nodes defined in MCData user profile MO with a default MCData user ID; or

-	the same MCVideo user profile nodes defined in MCVideo user profile MO with a default MCVideo user ID.

### 8.2.6	/<x>/DefaultUserProfile/UserID

Table 8.2.6.1: /<x>/DefaultUserProfile/UserID

| DefaultUserProfile/UserID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a default MCS user identity. |  |  |  |  |

### 8.2.7	/<x>/DefaultUserProfile/UserProfileIndex

Table 8.2.7.1: /<x>/DefaultUserProfile/UserProfileIndex

| DefaultUserProfile/UserProfileIndex |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node represents an indicator for the particular user profile. |  |  |  |  |

-	Values: 0-255

### 8.2.8	/<x>/OnNetwork

Table 8.2.8.1: /<x>/OnNetwork

| OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 8.2.9	/<x>/OnNetwork/GMSURI

Table 8.2.9.1: /<x>/OnNetwork/GMSURI

| OnNetwork/GMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the group management service URI information which contains the public service identity for performing subscription proxy function of the GMS. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.9A	/<x>/OnNetwork/GroupCreationXUI

Table 8.2.9A.1: /<x>/OnNetwork/GroupCreationXUI

| OnNetwork/GroupCreationXUI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the group creation XUI information for creation of groups. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.9B	/<x>/OnNetwork/GMSXCAPRootURI

Table 8.2.9B.1: /<x>/OnNetwork/GMSXCAPRootURI

| OnNetwork/GMSXCAPRootURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the group management server XCAP Root URI information. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.9C	/<x>/OnNetwork/CMSXCAPRootURI

Table 8.2.9C.1: /<x>/OnNetwork/CMSXCAPRootURI

| OnNetwork/CMSXCAPRootURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the configuration management server XCAP Root URI information. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.10	/<x>/OnNetwork/Timers

Table 8.2.10.1: /<x>/OnNetwork/Timers

| OnNetwork/Timers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for timer configuration. |  |  |  |  |

### 8.2.11	/<x>/OnNetwork/Timers/T100

Table 8.2.11.1: /<x>/OnNetwork/Timers/T100

| OnNetwork/Timers/T100 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for floor release as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

The T100 is in seconds.

### 8.2.12	/<x>/OnNetwork/Timers/T101

Table 8.2.12.1: /<x>/OnNetwork/Timers/T101

| OnNetwork/Timers/T101 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for floor request as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

The timer T101 is in seconds.

### 8.2.13	/<x>/OnNetwork/Timers/T103

Table 8.2.13.1: /<x>/OnNetwork/Timers/T103

| OnNetwork/Timers/T103 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for end of RTP media as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

The timer T103 is in seconds.

### 8.2.14	/<x>/OnNetwork/Timers/T104

Table 8.2.14.1: /<x>/OnNetwork/Timers/T104

| OnNetwork/Timers/T104 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for floor queue position request as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

The timer T104 is in seconds.

### 8.2.15	/<x>/OnNetwork/Timers/T132

Table 8.2.15.1: /<x>/OnNetwork/Timers/T132

| OnNetwork/Timers/T132 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for queued request granted MCPTT user action as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

The timer T132 is in seconds.

### 8.2.16	/<x>/OnNetwork/HPLMN

Table 8.2.16.1: /<x>/OnNetwork/HPLMN

| OnNetwork/HPLMN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the HPLMN configuration. |  |  |  |  |

### 8.2.17	/<x>/OnNetwork/HPLMN/PLMN

Table 8.2.17.1: /<x>/OnNetwork/HPLMN/PLMN

| OnNetwork/HPLMN/PLMN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the HPLMN code. |  |  |  |  |

-	Values: <PLMN>

The format of the PLMN is defined by 3GPP TS 23.003 [5].

### 8.2.18	/<x>/OnNetwork/HPLMN/Service

Table 8.2.18.1: /<x>/OnNetwork/HPLMN/Service

| OnNetwork/HPLMN/Service |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node indicates the MCS related services on a per HPLMN basis. |  |  |  |  |

### 8.2.19	/<x>/OnNetwork/HPLMN/Service/MCPTTToConRef

Table 8.2.19.1: /<x>/OnNetwork/HPLMN/Service/MCPTTToConRef

| OnNetwork/HPLMN/Service/MCPTTToConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the configuration parameters for establishment of the PDN connection for the MCS service on a per HPLMN basis. |  |  |  |  |

### 8.2.20	/<x>/OnNetwork/HPLMN/Service/MCPTTToConRef/<x>

Table 8.2.20.1: /<x>/OnNetwork/HPLMN/Service/MCPTTToConRef/<x>

| OnNetwork/HPLMN/Service/MCPTTToConRef/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to the connectivity parameters on a per HPLMN basis. |  |  |  |  |

### 8.2.21	/<x>/OnNetwork/HPLMN/Service/MCPTTToConRef/<x>/ConRef

Table 8.2.21.1: /<x>/OnNetwork/HPLMN/Service/MCPTTToConRef/<x>/ConRef

| OnNetwork/HPLMN/Service/MCPTTToConRef/<x>/ConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The ConRef specifies a specific linkage to the connectivity parameters on a per HPLMN basis. |  |  |  |  |

-	Values: <A network access point object>

### 8.2.22	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef

Table 8.2.22.1: /<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef

| OnNetwork/HPLMN/Service/MCCommonCoreToConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the configuration parameters for establishment of the PDN connection for the MC common core service on a per HPLMN basis. |  |  |  |  |

### 8.2.23	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef/<x>

Table 8.2.23.1: /<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef/<x>

| OnNetwork/HPLMN/Service/MCCommonCoreToConRef/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to the connectivity parameters on a per HPLMN basis. |  |  |  |  |

### 8.2.24	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef/
<x>/ConRef

Table 8.2.24.1: /<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef/<x>/ConRef

| OnNetwork/HPLMN/Service/MCCommonCoreToConRef/<x>/ConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The ConRef specifies a specific linkage to the connectivity parameters on a per HPLMN basis. |  |  |  |  |

-	Values: <A network access point object>

### 8.2.25	/<x>/OnNetwork/HPLMN/Service/MCIDMToConRef

Table 8.2.25.1: /<x>/OnNetwork/HPLMN/Service/MCIDMToConRef

| OnNetwork/HPLMN/Service/MCIDMToConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the configuration parameters for establishment of the PDN connection for the MC identity management service on a per HPLMN basis. |  |  |  |  |

### 8.2.26	/<x>/OnNetwork/HPLMN/Service/MCIDMToConRef/<x>

Table 8.2.26.1: /<x>/OnNetwork/HPLMN/Service/MCIDMToConRef/<x>

| OnNetwork/HPLMN/Service/MCIDMToConRef/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to the connectivity parameters on a per HPLMN basis. |  |  |  |  |

### 8.2.27	/<x>/OnNetwork/HPLMN/Service/MCIDMToConRef/<x>/ConRef

Table 8.2.27.1: /<x>/OnNetwork/HPLMN/Service/MCIDMToConRef/<x>/ConRef

| OnNetwork/HPLMN/Service/MCIDMToConRef/<x>/ConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The ConRef specifies a specific linkage to the connectivity parameters on a per HPLMN basis. |  |  |  |  |

-	Values: <A network access point object>

### 8.2.27A1	/<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI

Table 8.2.27A1.1: /<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI

| OnNetwork/HPLMN/Service/MCPTTRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the list of S-NSSAIs to be used for MCPTT service in the HPLMN. |  |  |  |  |

### 8.2.27A2	/<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI/<x>

Table 8.2.27A2.1: /<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI/<x>

| OnNetwork/HPLMN/Service/MCPTTRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MCPTT service in the HPLMN. |  |  |  |  |

### 8.2.27A3	/<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI/<x>/
RefSNSSAI

Table 8.2.27A3.1: /<x>/OnNetwork/HPLMN/Service/MCPTTRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/Service/MCPTTRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node specifies a specific linkage to the slice (S-NSSAI) parameters used for MCPTT service in the HPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.27A4	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI

Table 8.2.27A4.1: /<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI

| OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains a list of S-NSSAIs to be used for MC Common Core services in the HPLMN. |  |  |  |  |

### 8.2.27A5	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI/
<x>

Table 8.2.27A5.1: /<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI/<x>

| OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MC Common Core services in the HPLMN. |  |  |  |  |

### 8.2.27A6	/<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI/
<x>/RefSNSSAI

Table 8.2.27A6.1: /<x>/OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/Service/MCCommonCoreRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node specifies a specific linkage to the slice (S-NSSAI) parameters used for MC Common Core services in the HPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.27A7	/<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI

Table 8.2.27A7.1: /<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI

| OnNetwork/HPLMN/Service/MCIdMRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains a list of S-NSSAIs to be used for MC Identity Management services in the HPLMN. |  |  |  |  |

### 8.2.27A8	/<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI/<x>

Table 8.2.27A8.1: /<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI/<x>

| OnNetwork/HPLMN/Service/MCIdMRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MC Identity Management services in the HPLMN. |  |  |  |  |

### 8.2.27A9	/<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI/<x>/
RefSNSSAI

Table 8.2.27A9.1: /<x>/OnNetwork/HPLMN/Service/MCIdMRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/Service/MCIdMRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node specifies a specific linkage to the slice (S-NSSAI) parameters used for MC Identity Management services in the HPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.27A10	/<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI

Table 8.2.27A10.1: /<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI

| OnNetwork/HPLMN/Service/MCDataRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains a list of S-NSSAIs to be used for MCData service in the HPLMN. |  |  |  |  |

### 8.2.27A11	/<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI/<x>

Table 8.2.27A11.1: /<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI/<x>

| OnNetwork/HPLMN/Service/MCDataRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MCData services in the HPLMN. |  |  |  |  |

### 8.2.27A12	/<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI/<x>/
RefSNSSAI

Table 8.2.27A12.1: /<x>/OnNetwork/HPLMN/Service/MCDataRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/Service/MCDataRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The RefSNSSAI specifies a specific linkage to the slice (S-NSSAI) parameters used for MCData service in the HPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.27A13	/<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI

Table 8.2.27A13.1: /<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI

| OnNetwork/HPLMN/Service/MCVideoRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains a list of S-NSSAIs to be used for MCVideo service in the HPLMN. |  |  |  |  |

### 8.2.27A14	/<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI/<x>

Table 8.2.27A14.1: /<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI/<x>

| OnNetwork/HPLMN/Service/MCVideoRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MCVideo service in the HPLMN. |  |  |  |  |

### 8.2.27A15	/<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI/<x>/
RefSNSSAI

Table 8.2.27A15.1: /<x>/OnNetwork/HPLMN/Service/MCVideoRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/Service/MCVideoRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The RefSNSSAI specifies a specific linkage to the slice (S-NSSAI) parameters used for MCVideo service in the HPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.28	/<x>/OnNetwork/HPLMN/VPLMN

Table 8.2.28.1: /<x>/OnNetwork/HPLMN/VPLMN

| OnNetwork/HPLMN/VPLMN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of VPLMN configurations. |  |  |  |  |

### 8.2.28A	/<x>/OnNetwork/HPLMN/VPLMN/<x>

Table 8.2.28A.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>

| OnNetwork/HPLMN/VPLMN/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This run-time node is a placeholder for the list of VPLMN configurations. |  |  |  |  |

### 8.2.29	/<x>/OnNetwork/HPLMN/VPLMN/<x>/PLMN

Table 8.2.29.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/PLMN

| OnNetwork/HPLMN/VPLMN/<x>/PLMN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the VPLMN code. |  |  |  |  |

-	Values: <PLMN>

The format of the PLMN is defined by 3GPP TS 23.003 [5].

### 8.2.30	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service

Table 8.2.30.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service

| OnNetwork/HPLMN/VPLMN/<x>/Service |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node indicates the MCS related services on a per VPLMN and HPLMN basis. |  |  |  |  |

### 8.2.31	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef

Table 8.2.31.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the configuration parameters for establishment of the PDN connection for the MCS service on a per VPLMN and HPLMN basis. |  |  |  |  |

### 8.2.32	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef/<x>

Table 8.2.32.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef/<x>

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to the connectivity parameters on a per VPLMN and HPLMN basis. |  |  |  |  |

### 8.2.33	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCPTTToConRef/<x>/ConRef

Table 8.2.33.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef/<x>/ConRef

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTToConRef/<x>/ConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The ConRef specifies a specific linkage to the connectivity parameters on a per VPLMN and HPLMN basis. |  |  |  |  |

-	Values: <A network access point object>

### 8.2.34	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCCommonCoreToConRef

Table 8.2.34.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreToConRef

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreToConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the configuration parameters for establishment of the PDN connection for the MC common core service on a per VPLMN and HPLMN basis. |  |  |  |  |

### 8.2.35	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCCommonCoreToConRef/<x>

Table 8.2.35.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreToConRef/<x>

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreToConRef/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to the connectivity parameters on a per VPLMN and HPLMN basis. |  |  |  |  |

### 8.2.36	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCCommonCoreToConRef/<x>/ConRef

Table 8.2.36.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreToConRef/<x>/ConRef

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreToConRef/<x>/ConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The ConRef specifies a specific linkage to the connectivity parameters on a per VPLMN and HPLMN basis. |  |  |  |  |

-	Values: <A network access point object>

### 8.2.37	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef

Table 8.2.37.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the configuration parameters for establishment of the PDN connection for the MC identity management service on a per VPLMN and HPLMN basis. |  |  |  |  |

### 8.2.38	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef/<x>

Table 8.2.38.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef/<x>

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to the connectivity parameters on a per VPLMN and HPLMN basis. |  |  |  |  |

### 8.2.39	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCIDMToConRef/<x>/ConRef

Table 8.2.39.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef/<x>/ConRef

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCIDMToConRef/<x>/ConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The ConRef specifies a specific linkage to the connectivity parameters on a per VPLMN and HPLMN basis. |  |  |  |  |

-	Values: <A network access point object>

### 8.2.39A1	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCPTTRefSNSSAI

Table 8.2.39A1.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTRefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the list of S-NSSAIs to be used for MCPTT service in the VPLMN. |  |  |  |  |

### 8.2.39A2	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCPTTRefSNSSAI/<x>

Table 8.2.39A2.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTRefSNSSAI/<x>

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MCPTT service in the VPLMN. |  |  |  |  |

### 8.2.39A3	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCPTTRefSNSSAI/<x>/RefSNSSAI

Table 8.2.39A3.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCPTTRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node specifies a specific linkage to the slice (S-NSSAI) parameters used for MCPTT service in the VPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.39A4	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCCommonCoreRefSNSSAI

Table 8.2.39A4.1: 	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreRefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains a list of S-NSSAIs to be used for MC Common Core services in the VPLMN. |  |  |  |  |

### 8.2.39A5	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCCommonCoreRefSNSSAI/<x>

Table 8.2.39A5.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreRefSNSSAI/<x>

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | chr | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MC Common Core services in the VPLMN. |  |  |  |  |

### 8.2.39A6	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCCommonCoreRefSNSSAI/<x>/RefSNSSAI

Table 8.2.39A6.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCCommonCoreRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node specifies a specific linkage to the slice (S-NSSAI) parameters used for MC Common Core services in the VPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.39A7	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCIdMRefSNSSAI

Table 8.2.39A7.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIdMRefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCIdMRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains a list of S-NSSAIs to be used for MC Identity Management services in the VPLMN. |  |  |  |  |

### 8.2.39A8	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCIdMRefSNSSAI/<x>

Table 8.2.39A8.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIdMRefSNSSAI/<x>

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCIdMRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MC Identity Management services in the VPLMN. |  |  |  |  |

### 8.2.39A9	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCIdMRefSNSSAI/<x>/RefSNSSAI

Table 8.2.39A9.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCIdMRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCIdMRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node specifies a specific linkage to the slice (S-NSSAI) parameters used for MC Identity Management services in the HPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.39A10	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCDataRefSNSSAI

Table 8.2.39A10.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCDataRefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCDataRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains a list of S-NSSAIs to be used for MCData service in the VPLMN. |  |  |  |  |

### 8.2.39A11	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCDataRefSNSSAI/<x>

Table 8.2.39A11.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCDataRefSNSSAI/<x>

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCDataRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MCData services in the VPLMN. |  |  |  |  |

### 8.2.39A12	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCDataRefSNSSAI/<x>/RefSNSSAI

Table 8.2.39A12.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCDataRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCDataRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The RefSNSSAI specifies a specific linkage to the slice (S-NSSAI) parameters used for MCData service in the VPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.39A13	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCVideoRefSNSSAI

Table 8.2.39A13.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCVideoRefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCVideoRefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains a list of S-NSSAIs to be used for MCVideo service in the VPLMN. |  |  |  |  |

### 8.2.39A14	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCVideoRefSNSSAI/<x>

Table 8.2.39A14.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCVideoRefSNSSAI/<x>

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCVideoRefSNSSAI/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to parameters for a slice (S-NSSAI) used for MCVideo service in the VPLMN. |  |  |  |  |

### 8.2.39A15	/<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/
MCVideoRefSNSSAI/<x>/RefSNSSAI

Table 8.2.39A15.1: /<x>/OnNetwork/HPLMN/VPLMN/<x>/Service/MCVideoRefSNSSAI/<x>/RefSNSSAI

| OnNetwork/HPLMN/VPLMN/<x>/Service/MCVideoRefSNSSAI/<x>/RefSNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The RefSNSSAI specifies a specific linkage to the slice (S-NSSAI) parameters used for MCVideo service in the VPLMN. |  |  |  |  |

-	Values: A string that identifies an S-NSSAI.

### 8.2.40	/<x>/OnNetwork/AppServerInfo

Table 8.2.40.1: /<x>/OnNetwork/AppServerInfo

| OnNetwork/AppServerInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the application plane server identity information configuration. |  |  |  |  |

### 8.2.41	/<x>/OnNetwork/AppServerInfo/IDMSAuthEndpoint

Table 8.2.41.1: /<x>/OnNetwork/AppServerInfo/IDMSAuthEndpoint

| OnNetwork/AppServerInfo/IDMSAuthEndpoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity management server authorisation endpoint identity information. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.41A	/<x>/OnNetwork/AppServerInfo/IDMSTokenEndpoint

Table 8.2.41A.1: /<x>/OnNetwork/AppServerInfo/IDMSTokenEndpoint

| OnNetwork/AppServerInfo/IDMSTokenEndpoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity management server token endpoint identity information. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.41B	/<x>/OnNetwork/AppServerInfo/HTTPProxy

Table 8.2.41B.1: /<x>/OnNetwork/AppServerInfo/HTTPProxy

| OnNetwork/AppServerInfo/HTTPProxy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the HTTP Proxy identity information. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.42	/<x>/OnNetwork/AppServerInfo/GMS

Table 8.2.42.1: /<x>/OnNetwork/AppServerInfo/GMS

| OnNetwork/AppServerInfo/GMS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the group management server identity information. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.43	/<x>/OnNetwork/AppServerInfo/CMS

Table 8.2.43.1: /<x>/OnNetwork/AppServerInfo/CMS

| OnNetwork/AppServerInfo/CMS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the configuration management server identity information. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.44	/<x>/OnNetwork/AppServerInfo/KMS

Table 8.2.44.1: /<x>/OnNetwork/AppServerInfo/KMS

| OnNetwork/AppServerInfo/KMS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates key management server identity information. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.44A	/<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod

Table 8.2.44A.1: /<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod

| OnNetwork/AppServerInfo/TLSTunnelAuthMethod |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the TLS tunnel authentication method configuration. |  |  |  |  |

### 8.2.44B	/<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/Mutual

Table 8.2.44B.1: /<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/Mutual

| OnNetwork/AppServerInfo/TLSTunnelAuthMethod/Mutual |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether mutual authentication is used for the TLS tunnel authentication. |  |  |  |  |

When set to "true" mutual authentication is used.

When set to "false" one-way authentication based on the server certificate is used.

The default value is "false".

### 8.2.44C	/<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/X509

Table 8.2.44C.1: /<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/X509

| OnNetwork/AppServerInfo/TLSTunnelAuthMethod/X509 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains the X.509 certificate for mutual authentication for the TLS tunnel authentication. |  |  |  |  |

### 8.2.44D	/<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/Key

Table 8.2.44D.1: /<x>/OnNetwork/AppServerInfo/TLSTunnelAuthMethod/Key

| OnNetwork/AppServerInfo/TLSTunnelAuthMethod/Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains the pre-shared key for mutual authentication for the TLS tunnel authentication. |  |  |  |  |

### 8.2.44D1	/<x>/OnNetwork/AppServerInfo/LMS

Table 8.2.44D1.1: /<x>/OnNetwork/AppServerInfo/LMS

| OnNetwork/AppServerInfo/LMS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains the Location Managemenet Server URI. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.44E	/<x>/OnNetwork/IntegrityProtection

Table 8.2.44E.1: /<x>/OnNetwork/IntegrityProtection

| OnNetwork/IntegrityProtection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether integrity protection is enabled. |  |  |  |  |

When set to "true" integrity protection is enabled.

When set to "false" integrity protection is disabled.

The default value is "true".

### 8.2.44F	/<x>/OnNetwork/ConfidentialityProtection

Table 8.2.44F.1: /<x>/OnNetwork/ConfidentialityProtection

| OnNetwork/ConfidentialityProtection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether integrity protection is enabled. |  |  |  |  |

When set to "true" confidentiality protection is enabled.

When set to "false" confidentiality protection is disabled.

The default value is "true".

### 8.2.44G1	/<x>/OnNetwork/MCPTTServiceDetails

Table 8.2.44G1.1: /<x>/OnNetwork/MCPTTServiceDetails

| OnNetwork/MCPTTServiceDetails |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCPTT service. |  |  |  |  |

### 8.2.44G2	/<x>/OnNetwork/MCPTTServiceDetails/IPv6Required

Table 8.2.44G2.1: /<x>/OnNetwork/MCPTTServiceDetails/IPv6Required

| OnNetwork/MCPTTServiceDetails/IPv6Required |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether IP version 6 is required to use to access the MCPTT service. |  |  |  |  |

When set to "true", IPv6 shall be used to access the service.

When set to "false", IPv4 shall be used to access the service.

### 8.2.44G3	/<x>/OnNetwork/MCPTTServiceDetails/ServerURI

Table 8.2.44G3.1: /<x>/OnNetwork/MCPTTServiceDetails/ServerURI

| OnNetwork/MCPTTServiceDetails/ServerURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the URI to use to access the MCPTT service. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.44G3A	/<x>/OnNetwork/MCPTTServiceDetails/PDUSessionType

Table 8.2.44G3A.1: /<x>/OnNetwork/MCPTTServiceDetails/PDUSessionType

| OnNetwork/MCPTTServiceDetails/PDUSessionType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the type of PDU session to be established and used for the MCPTT service. |  |  |  |  |

-	Values: one of {"IPv4", "IPv6", "IPv4v6", "Ethernet", or "Unstructured"}.

### 8.2.44G3A1	Void

### 8.2.44G3A2	Void

### 8.2.44G3A3	Void

### 8.2.44G3A4	Void

### 8.2.44G3A5	Void

### 8.2.44G3A6	Void

### 8.2.44G3A7	Void

### 8.2.44G3A8	Void

### 8.2.44G4	/<x>/OnNetwork/MCVideoServiceDetails

Table 8.2.44G4.1: /<x>/OnNetwork/MCVideoServiceDetails

| OnNetwork/MCVideoServiceDetails |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCVideo service. |  |  |  |  |

### 8.2.44G5	/<x>/OnNetwork/MCVideoServiceDetails/IPv6Required

Table 8.2.44G5.1: /<x>/OnNetwork/MCVideoServiceDetails/IPv6Required

| OnNetwork/MCVideoServiceDetails/IPv6Required |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the IP version to use to access the MCVideo service. |  |  |  |  |

When set to "true", IPv6 shall be used to access the service.

When set to "false", IPv4 shall be used to access the service.

### 8.2.44G6	/<x>/OnNetwork/MCVideoServiceDetails/ServerURI

Table 8.2.44G6.1: /<x>/OnNetwork/MCVideoServiceDetails/ServerURI

| OnNetwork/MCVideoServiceDetails/ServerURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the URI to use to access the MCVideo service. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.44G6A	/<x>/OnNetwork/MCVideoServiceDetails/PDUSessionType

Table 8.2.44G6A.1: /<x>/OnNetwork/MCVideoServiceDetails/PDUSessionType

| OnNetwork/MCVideoServiceDetails/PDUSessionType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the type of PDU session to be established and used for the MCVideo service. |  |  |  |  |

-	Values: one of {"IPv4", "IPv6", "IPv4v6", "Ethernet", or "Unstructured"}.

### 8.2.44G6A1	Void

### 8.2.44G6A2	Void

### 8.2.44G6A3	Void

### 8.2.44G6A4	Void

### 8.2.44G6A5	Void

### 8.2.44G6A6	Void

### 8.2.44G6A7	Void

### 8.2.44G6A8	Void

### 8.2.44G7	/<x>/OnNetwork/MCDataServiceDetails

Table 8.2.44G7.1: /<x>/OnNetwork/MCDataServiceDetails

| OnNetwork/MCDataServiceDetails |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCData service. |  |  |  |  |

### 8.2.44G8	/<x>/OnNetwork/MCDataServiceDetails/IPv6Required

Table 8.2.44G8.1: /<x>/OnNetwork/MCDataServiceDetails/IPv6Required

| OnNetwork/MCDataServiceDetails/IPv6Required |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the IP version to use to access the MCData service. |  |  |  |  |

When set to "true", IPv6 shall be used to access the service.

When set to "false", IPv4 shall be used to access the service.

### 8.2.44G9	/<x>/OnNetwork/MCDataServiceDetails/ServerURI

Table 8.2.44G9.1: /<x>/OnNetwork/MCDataServiceDetails/ServerURI

| OnNetwork/MCDataServiceDetails/ServerURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the URI to use to access the MCData service. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.44G9A	/<x>/OnNetwork/MCDataServiceDetails/PDUSessionType

Table 8.2.44G9A.1: /<x>/OnNetwork/MCDataServiceDetails/PDUSessionType

| OnNetwork/MCDataServiceDetails/PDUSessionType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the type of PDU session to be established and used for the MCData service. |  |  |  |  |

-	Values: one of {"IPv4", "IPv6", "IPv4v6", "Ethernet", or "Unstructured"}.

### 8.2.44G9A1	Void

### 8.2.44G9A2	Void

### 8.2.44G9A3	Void

### 8.2.44G9A4	Void

### 8.2.44G9A5	Void

### 8.2.44G9A6	Void

### 8.2.44G9A7	Void

### 8.2.44G9A8	Void

### 8.2.44G10	Void

### 8.2.44G11	Void

### 8.2.44G12	Void

### 8.2.44G13	Void

### 8.2.44G14	Void

### 8.2.44G15	Void

### 8.2.44G16	Void

### 8.2.44G17	Void

### 8.2.44G18	Void

### 8.2.44G19	Void

### 8.2.44G20	Void

### 8.2.44G21	Void

### 8.2.44G22	Void

### 8.2.44G23	Void

### 8.2.44G24	Void

### 8.2.44G25	Void

### 8.2.44G26	/<x>/OnNetwork/MCCommonCoreServiceDetails

Table 8.2.44G26.1: /<x>/OnNetwork/MCCommonCoreServiceDetails

| OnNetwork/MCCommonCoreServiceDetails |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MC Common Core service. |  |  |  |  |

### 8.2.44G27	/<x>/OnNetwork/MCCommonCoreServiceDetails/IPv6Required

Table 8.2.44G27.1: /<x>/OnNetwork/MCCommonCoreServiceDetails/IPv6Required

| OnNetwork/MCCommonCoreServiceDetails/IPv6Required |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the IP version to use to access the MC Common Core services. |  |  |  |  |

When set to "true", IPv6 shall be used to access the service.

When set to "false", IPv4 shall be used to access the service.

### 8.2.44G28	/<x>/OnNetwork/MCCommonCoreServiceDetails/ServerURI

Table 8.2.44G28.1: /<x>/OnNetwork/MCCommonCoreServiceDetails/ServerURI

| OnNetwork/MCCommonCoreServiceDetails/ServerURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the URI to use to access the MC Common Core services. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.44G29	/<x>/OnNetwork/MCCommonCoreServiceDetails/
PDUSessionType

Table 8.2.44G29.1: /<x>/OnNetwork/MCCommonCoreServiceDetails/PDUSessionType

| OnNetwork/MCCommonCoreServiceDetails/PDUSessionType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the type of PDU session to be established and used for the MC Common Core services. |  |  |  |  |

-	Values: one of {"IPv4", "IPv6", "IPv4v6", "Ethernet", or "Unstructured"}.

### 8.2.44G30	/<x>/OnNetwork/MCIdMServiceDetails

Table 8.2.44G30.1: /<x>/OnNetwork/MCIdMServiceDetails

| OnNetwork/MCIdMServiceDetails |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MC Identity Management service. |  |  |  |  |

### 8.2.44G31	/<x>/OnNetwork/MCIdMServiceDetails/IPv6Required

Table 8.2.44G31.1: /<x>/OnNetwork/MCIdMServiceDetails/IPv6Required

| OnNetwork/MCIdMServiceDetails/IPv6Required |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the IP version to use to access the MC Identity Management services. |  |  |  |  |

When set to "true", IPv6 shall be used to access the service.

When set to "false", IPv4 shall be used to access the service.

### 8.2.44G32	/<x>/OnNetwork/MCIdMServiceDetails/ServerURI

Table 8.2.44G32.1: /<x>/OnNetwork/MCIdMServiceDetails/ServerURI

| OnNetwork/MCIdMServiceDetails/ServerURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the URI to use to access the MC Identity Management services. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 8.2.44G33	/<x>/OnNetwork/MCIdMServiceDetails/PDUSessionType

Table 8.2.44G33.1: /<x>/OnNetwork/MCIdMServiceDetails/PDUSessionType

| OnNetwork/MCIdMServiceDetails/PDUSessionType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the type of PDU session to be established and used for the MC Identity Management services. |  |  |  |  |

-	Values: one of {"IPv4", "IPv6", "IPv4v6", "Ethernet", or "Unstructured"}.

### 8.2.44G34	/<x>/OnNetwork/MCIdMServiceDetails/IdmClientId

Table 8.2.44G34.1: /<x>/OnNetwork/MCIdMServiceDetails/IdmClientId

| OnNetwork/MCIdMServiceDetails/IdmClientId |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the type of IDM Client ID to be used for the MC identity management services. |  |  |  |  |

### 8.2.44H	/<x>/OnNetwork/DNInfo

Table 8.2.44H.1: /<x>/OnNetwork/DNInfo

| OnNetwork/DNInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of Data Networks. |  |  |  |  |

### 8.2.44H1	/<x>/OnNetwork/DNInfo/<x>

Table 8.2.44H1.1: /<x>/OnNetwork/DNInfo/<x>

| OnNetwork/DNInfo/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node is a placeholder for parameters for a Data Network. |  |  |  |  |

### 8.2.44H2	/<x>/OnNetwork/DNInfo/<x>/DNN

Table 8.2.44H2.1: /<x>/OnNetwork/DNInfo/<x>/DNN

| OnNetwork/DNInfo/<x>/DNN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node contains the DNN of a Data Network. |  |  |  |  |

-	Value: The DNN is a string that identifies a Data Network or an APN. The ConRef leaf node value provides mapping into the DNN/APN that is to be used.

### 8.2.44H3	/<x>/OnNetwork/DNInfo/<x>/DNAAAServer

Table 8.2.44H3.1: /<x>/OnNetwork/DNInfo/<x>/DNAAAServer

| OnNetwork/DNInfo/<x>/DNAAAServer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains the DNN of a Data Network. |  |  |  |  |

Value: a valid URI pointing to the DN AAA server.

### 8.2.44H4	/<x>/OnNetwork/DNInfo/<x>/PapParameters

Table 8.2.44H4.1: /<x>/OnNetwork/DNInfo/<x>/PapParameters

| OnNetwork/DNInfo/<x>/PapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for PAP access credentials to access the Data Network. |  |  |  |  |

### 8.2.44H5	/<x>/OnNetwork/DNInfo/<x>/PapParameters/UserName

Table 8.2.44H5.1: /<x>/OnNetwork/DNInfo/<x>/PapParameters/UserName

| OnNetwork/DNInfo/<x>/PapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP user name if needed to access the Data Network. |  |  |  |  |

### 8.2.44H6	/<x>/OnNetwork/DNInfo/<x>/PapParameters/PassWord

Table 8.2.44H6.1: /<x>/OnNetwork/DNInfo/<x>/PapParameters/PassWord

| OnNetwork/DNInfo/<x>/PapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the PAP password if needed to access the Data Network. |  |  |  |  |

### 8.2.44H7	/<x>/OnNetwork/DNInfo/<x>/ChapParameters

Table 8.2.44H7.1: /<x>/OnNetwork/DNInfo/<x>/ChapParameters

| OnNetwork/DNInfo/<x>/ChapParameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for CHAP access credentials to access the Data Network. |  |  |  |  |

### 8.2.44H8	/<x>/OnNetwork/DNInfo/<x>/ChapParameters/UserName

Table 8.2.44H8.1: /<x>/OnNetwork/DNInfo/<x>/ChapParameters/UserName

| OnNetwork/DNInfo/<x>/ChapParameters/UserName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP user name if needed to access the Data Network. |  |  |  |  |

### 8.2.44H9	/<x>/OnNetwork/DNInfo/<x>/ChapParameters/PassWord

Table 8.2.44H9.1: /<x>/OnNetwork/DNInfo/<x>/ChapParameters/PassWord

| OnNetwork/DNInfo/<x>/ChapParameters/PassWord |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the CHAP password if needed to access the Data Network. |  |  |  |  |

### 8.2.44H10	/<x>/OnNetwork/DNInfo/<x>/Credentials

Table 8.2.44H10.1: /<x>/OnNetwork/DNInfo/<x>/Credentials

| OnNetwork/DNInfo/<x>/Credentials |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This internal node is a placeholder for the credentials needed for authentication of the UE during authentication by the Data Network. |  |  |  |  |

### 8.2.44H11	/<x>/OnNetwork/DNInfo/<x>/Credentials/ID

Table 8.2.44H11.1: /<x>/OnNetwork/DNInfo/<x>/Credentials/ID

| OnNetwork/DNInfo/<x>/Credentials/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the EAP Identity as specified in IETF RFC 3748 [24] to be used for authentication of the UE during authentication by the Data Network. |  |  |  |  |

### 8.2.44H12	/<x>/OnNetwork/SNSSAIInfo

Table 8.2.44H12.1: /<x>/OnNetwork/SNSSAIInfo

| OnNetwork/SNSSAIInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the NSSAI containing the S-NSSAIs for mission critical services. |  |  |  |  |

### 8.2.44H13	/<x>/OnNetwork/SNSSAIInfo/<x>

Table 8.2.44H13.1: /<x>/OnNetwork/SNSSAIInfo/<x>

| OnNetwork/SNSSAIInfo/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This run-time node is a placeholder for the list of S-NSSAIs. |  |  |  |  |

### 8.2.44H14	/<x>/OnNetwork/SNSSAIInfo/<x>/NSSAAServer

Table 8.2.44H14.1: /<x>/OnNetwork/SNSSAIInfo/<x>/NSSAAServer

| OnNetwork/SNSSAIInfo/<x>/NSSAAServer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains the URI of a NSSAA server. |  |  |  |  |

-	Value: a valid URI as specified in 3GPP TS 23.003 [5] pointing to the NSSAA server.

### 8.2.44H15	/<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI

Table 8.2.44H15.1: /<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI

| OnNetwork/SNSSAIInfo/<x>/SNSSAI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an S-NSSAI used to access mission critical services. |  |  |  |  |

-	Value: a string consisting of the SST and SD of the S-NSSAI in the format: characters to represent the SST value, followed by the colon character ":", followed by characters to represent the SD value, if an SD value exists for the S-NSSAI.

### 8.2.44H16	/<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI/Credentials

Table 8.2.44H16.1: /<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI/Credentials

| OnNetwork/SNSSAIInfo/<x>/SNSSAI/Credentials |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This internal node is a placeholder for the credentials needed for authentication of the UE at NSSAA during registration of this S-NSSAI by the UE. |  |  |  |  |

### 8.2.44H17	/<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI/Credentials/ID

Table 8.2.44H17.1: /<x>/OnNetwork/SNSSAIInfo/<x>/SNSSAI/Credentials/ID

| OnNetwork/SNSSAIInfo/<x>/SNSSAI/Credentials/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the EAP Identity as specified in IETF RFC 3748 [24] to be used for NSSAA for this S-NSSAI at network registration. |  |  |  |  |

### 8.2.45	/<x>/OffNetwork

Table 8.2.45.1: /<x>/OffNetwork

| OffNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the off-network operation. |  |  |  |  |

### 8.2.46	/<x>/OffNetwork/Timers

Table 8.2.46.1: /<x>/OffNetwork/Timers

| OffNetwork/Timers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the timer configuration. |  |  |  |  |

### 8.2.47	/<x>/OffNetwork/Timers/TFG1

Table 8.2.47.1: /<x>/OffNetwork/Timers/TFG1

| OffNetwork/Timers/TFG1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for wait for call announcement as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFG1 is in milliseconds.

### 8.2.48	/<x>/OffNetwork/Timers/TFG2

Table 8.2.48.1: /<x>/OffNetwork/Timers/TFG2

| OffNetwork/Timers/TFG2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for call announcement as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFG2 is in milliseconds.

### 8.2.49	/<x>/OffNetwork/Timers/TFG3

Table 8.2.49.1: /<x>/OffNetwork/Timers/TFG3

| OffNetwork/Timers/TFG3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for call probe retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFG3 is in milliseconds.

### 8.2.50	/<x>/OffNetwork/Timers/TFG4

Table 8.2.50.1: /<x>/OffNetwork/Timers/TFG4

| OffNetwork/Timers/TFG4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for waiting for the MCPTT user as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-60

The timer TFG4 is in seconds.

### 8.2.51	/<x>/OffNetwork/Timers/TFG5

Table 8.2.51.1: /<x>/OffNetwork/Timers/TFG5

| OffNetwork/Timers/TFG5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for not present incoming call announcements as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

The timer TFG5 is in seconds.

### 8.2.52	/<x>/OffNetwork/Timers/TFG11

Table 8.2.52.1: /<x>/OffNetwork/Timers/TFG11

| OffNetwork/Timers/TFG11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timer for MCPTT emergency end retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFG11 is in milliseconds.

### 8.2.53	/<x>/OffNetwork/Timers/TFG12

Table 8.2.53.1: /<x>/OffNetwork/Timers/TFG12

| OffNetwork/Timers/TFG12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timer for MCPTT imminent peril end retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFG12 is in milliseconds.

### 8.2.54	/<x>/OffNetwork/Timers/TFG13

Table 8.2.54.1: /<x>/OffNetwork/Timers/TFG13

| OffNetwork/Timers/TFG13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for implicit priority downgrade as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

The timer TFG13 is in seconds.

### 8.2.54A	/<x>/OffNetwork/Timers/TFG14

Table 8.2.54A.1: /<x>/OffNetwork/Timers/TFG14

| OffNetwork/Timers/TFG14 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for implicit priority downgrade (imminent peril) as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

The timer TFG14 is in seconds.

### 8.2.55	/<x>/OffNetwork/Timers/TFP1

Table 8.2.55.1: /<x>/OffNetwork/Timers/TFP1

| OffNetwork/Timers/TFP1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for private call request retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFP1 is in milliseconds.

### 8.2.56	/<x>/OffNetwork/Timers/TFP2

Table 8.2.56.1: /<x>/OffNetwork/Timers/TFP2

| OffNetwork/Timers/TFP2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for waiting for the MCPTT user to respond to the setup request message when manual commencement mode is used as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-60

The timer TFP2 is in seconds.

### 8.2.57	/<x>/OffNetwork/Timers/TFP3

Table 8.2.57.1: /<x>/OffNetwork/Timers/TFP3

| OffNetwork/Timers/TFP3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for private call release retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFP3 is in milliseconds.

### 8.2.58	/<x>/OffNetwork/Timers/TFP4

Table 8.2.58.1: /<x>/OffNetwork/Timers/TFP4

| OffNetwork/Timers/TFP4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for private call accept retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFP4 is in milliseconds.

### 8.2.59	/<x>/OffNetwork/Timers/TFP5

Table 8.2.59.1: /<x>/OffNetwork/Timers/TFP5

| OffNetwork/Timers/TFP5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for call release as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-600

The timer TFP5 is in seconds.

### 8.2.60	/<x>/OffNetwork/Timers/TFP6

Table 8.2.60.1: /<x>/OffNetwork/Timers/TFP6

| OffNetwork/Timers/TFP6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for MCPTT emergency private call cancel retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFP6 is in milliseconds.

### 8.2.61	/<x>/OffNetwork/Timers/TFP7

Table 8.2.61.1: /<x>/OffNetwork/Timers/TFP7

| OffNetwork/Timers/TFP7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for waiting for any message with same call identifier as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

The timer TFP7 is in seconds.

### 8.2.61A	/<x>/OffNetwork/Timers/TFP9

Table 8.2.61A: /<x>/OffNetwork/Timers/TFP9

| OffNetwork/Timers/TFP9 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for waiting for call response message as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-60

The timer TFP9 is in seconds.

### 8.2.62	/<x>/OffNetwork/Timers/TFB1

Table 8.2.62.1: /<x>/OffNetwork/Timers/TFB1

| OffNetwork/Timers/TFB1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for max duration as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-600

The timer TFB1 is in seconds.

### 8.2.63	/<x>/OffNetwork/Timers/TFB2

Table 8.2.63.1: /<x>/OffNetwork/Timers/TFB2

| OffNetwork/Timers/TFB2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for broadcast retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-10

The timer TFB2 is in seconds.

### 8.2.64	/<x>/OffNetwork/Timers/TFB3

Table 8.2.64.1: /<x>/OffNetwork/Timers/TFB3

| OffNetwork/Timers/TFB3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for waiting for the MCPTT user as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-60

The timer TFB3 is in seconds.

### 8.2.65	/<x>/OffNetwork/Timers/T201

Table 8.2.65.1: /<x>/OffNetwork/Timers/T201

| OffNetwork/Timers/T201 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for floor request as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-65535

The timer T201 is in milliseconds.

### 8.2.66	/<x>/OffNetwork/Timers/T203

Table 8.2.66.1: /<x>/OffNetwork/Timers/T203

| OffNetwork/Timers/T203 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for end of RTP media as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

The timer T203 is in seconds.

### 8.2.67	/<x>/OffNetwork/Timers/T204

Table 8.2.67.1: /<x>/OffNetwork/Timers/T204

| OffNetwork/Timers/T204 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for floor queue position request as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-65535

The timer T204 is in milliseconds.

### 8.2.68	/<x>/OffNetwork/Timers/T205

Table 8.2.68.1: /<x>/OffNetwork/Timers/T205

| OffNetwork/Timers/T205 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for floor granted request as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-65535

The timer T205 is in milliseconds.

### 8.2.69	Void

### 8.2.70	/<x>/OffNetwork/Timers/T233

Table 8.2.70.1: /<x>/OffNetwork/Timers/T233

| OffNetwork/Timers/T233 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT timer for pending user action as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

The timer T233 is in seconds.

### 8.2.71	/<x>/OffNetwork/Timers/TFE1

Table 8.2.71.1: /<x>/OffNetwork/Timers/TFE1

| OffNetwork/Timers/TFE1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timer for MCPTT emergency alert as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-65535

The timer TFE1 is in seconds.

### 8.2.72	/<x>/OffNetwork/Timers/TFE2

Table 8.2.72.1: /<x>/OffNetwork/Timers/TFE2

| OffNetwork/Timers/TFE2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timer for MCPTT emergency alert retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-10

The timer TFE2 is in seconds.

### 8.2.73	/<x>/OffNetwork/Counters

Table 8.2.73.1: /<x>/OffNetwork/Counters

| OffNetwork/Counters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the counter configuration. |  |  |  |  |

### 8.2.74	/<x>/OffNetwork/Counters/CFP1

Table 8.2.74.1: /<x>/OffNetwork/Counters/CFP1

| OffNetwork/Counters/CFP1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT counter for private call request retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

### 8.2.75	/<x>/OffNetwork/Counters/CFP3

Table 8.2.75.1: /<x>/OffNetwork/Counters/CFP3

| OffNetwork/Counters/CFP3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT counter for private call release retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

### 8.2.76	/<x>/OffNetwork/Counters/CFP4

Table 8.2.76.1: /<x>/OffNetwork/Counters/CFP4

| OffNetwork/Counters/CFP4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT counter for private call accept retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

### 8.2.77	/<x>/OffNetwork/Counters/CFP6

Table 8.2.77.1: /<x>/OffNetwork/Counters/CFP6

| OffNetwork/Counters/CFP6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT counter for emergency private call cancel retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

### 8.2.78	/<x>/OffNetwork/Counters/CFG11

Table 8.2.78.1: /<x>/OffNetwork/Counters/CFG11

| OffNetwork/Counters/CFG11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the counter for MCPTT group call emergency end retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

### 8.2.79	/<x>/OffNetwork/Counters/CFG12

Table 8.2.79.1: /<x>/OffNetwork/Counters/CFG12

| OffNetwork/Counters/CFG12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the counter for MCPTT imminent peril call emergency end retransmission as specified in 3GPP TS 24.379 [7]. |  |  |  |  |

-	Values: 0-255

### 8.2.80	/<x>/OffNetwork/Counters/C201

Table 8.2.80.1: /<x>/OffNetwork/Counters/C201

| OffNetwork/Counters/C201 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT counter for floor request as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

### 8.2.81	/<x>/OffNetwork/Counters/C204

Table 8.2.81.1: /<x>/OffNetwork/Counters/C204

| OffNetwork/Counters/C204 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT counter for floor queue position request as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

### 8.2.82	/<x>/OffNetwork/Counters/C205

Table 8.2.82.1: /<x>/OffNetwork/Counters/C205

| OffNetwork/Counters/C205 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the MCPTT counter for floor granted request as specified in 3GPP TS 24.380 [8]. |  |  |  |  |

-	Values: 0-255

### 8.2.83	/<x>/Common

Table 8.2.83: /<x>/Common

| Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 8.2.84	/<x>/Common/MCServiceUELabel

Table 8.2.84: /<x>/Common/MCServiceUELabel

| Common/MCServiceUELabel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains the MC service UE label. |  |  |  |  |

-	Value: a string containing the MC service UE label.

# 9	MCData UE configuration MO

## 9.1	General

The MCData UE configuration Management Object (MO) is used to configure MCData Client behaviour for the on-network or off-network MCData Service. The MCData UE configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCData-UE-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCData UE configuration MO.

The following nodes and leaf objects are possible under the MCData UE configuration node as described in figure 9.1.1 and figure 9.1.2.

Figure 9.1.1: The MCData UE configuration MO (1 of 2)

Figure 9.1.2: The MCData UE configuration MO (2 of 2)

## 9.2	MCData UE configuration MO parameters

### 9.2.1	General

This clause describes the parameters for the MCData UE configuration Management Object (MO).

### 9.2.2	Node: <x>

Table 9.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCData UE configuration Management Object (MO). For the MCData UE configuration MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcdata-ue-configuration:1.0" |  |  |  |  |

-	Values: N/A

### 9.2.3	/<x>/Name

Table 9.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCData UE configuration settings. |  |  |  |  |

-	Values: <User displayable name>

### 9.2.4	/<x>/Ext/

Table 9.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCData UE configuration MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 9.2.5	/<x>/Common

Table 9.2.5.1: /<x>/Common

| Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 9.2.6	/<x>/Common/MCDataGroupTxns

Table 9.2.6.1: /<x>/Common/MCDataGroupTxns

| Common/MCDataGroupTxns |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData group transactions configuration. |  |  |  |  |

### 9.2.7	/<x>/Common/MCDataGroupTxns/MaxSDSNc4

Table 9.2.7.1: /<x>/Common/MCDataGroupTxns/MaxSDSNc4

| Common/MCDataGroupTxns/MaxSDSNc4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous SDS transactions. |  |  |  |  |

-	Values: 0-255

### 9.2.8	/<x>/Common/MCDataGroupTxns/SDSPresentationPriority

Table 9.2.8.1: /<x>/Common/MCDataGroupTxns/SDSPresentationPriority

| Common/MCDataGroupTxns/SDSPresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the prioritized MCData SDS configuration. |  |  |  |  |

### 9.2.9	/<x>/Common/MCDataGroupTxns/SDSPresentationPriority/<x>

Table 9.2.9.1: /<x>/Common/MCDataGroupTxns/SDSPresentationPriority/<x>

| Common/MCDataGroupTxns/SDSPresentationPriority/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more prioritized MCData SDS configuration. |  |  |  |  |

### 9.2.10	/<x>/Common/MCDataGroupTxns/SDSPresentationPriority/
<x>/MCDataGroupID

Table 9.2.10.1: /<x>/Common/MCDataGroupTxns/SDSPresentationPriority/<x>/MCDataGroupID

| Common/MCDataGroupTxns/SDSPresentationPriority/<x>/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCData group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 9.2.11	/<x>/Common/MCDataGroupTxns/SDSPresentationPriority/
<x>/MCDataGroupPriorityHierarchy

Table 9.2.11.1: /<x>/Common/MCDataGroupTxns/SDSPresentationPriority/<x>/
MCDataGroupPriorityHierarchy

| Common/MCDataGroupTxns/SDSPresentationPriority/<x>/MCDataGroupPriorityHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the requested presentation priority of SDS transactions. |  |  |  |  |

-	Values: 0-255

The group with the lowest MCDataGroupPriorityHierarchy value shall be considered as the group having the lowest priority among the groups.

### 9.2.12	/<x>/Common/MCDataGroupTxns/MaxFDNc4

Table 9.2.12.1: /<x>/Common/MCDataGroupTxns/MaxFDNc4

| Common/MCDataGroupTxns/MaxFDNc4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous FD transactions. |  |  |  |  |

-	Values: 0-255

### 9.2.13	/<x>/Common/MCDataGroupTxns/FDPresentationPriority

Table 9.2.13.1: /<x>/Common/MCDataGroupTxns/FDPresentationPriority

| Common/MCDataGroupTxns/FDPresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the prioritized MCData FD configuration. |  |  |  |  |

### 9.2.14	/<x>/Common/MCDataGroupTxns/FDPresentationPriority/<x>

Table 9.2.14.1: /<x>/Common/MCDataGroupTxns/FDPresentationPriority/<x>

| Common/MCDataGroupTxns/FDPresentationPriority /<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more prioritized MCData FD configuration. |  |  |  |  |

### 9.2.15	/<x>/Common/MCDataGroupTxns/FDPresentationPriority/
<x>/MCDataGroupID

Table 9.2.15.1: /<x>/Common/MCDataGroupTxns/FDPresentationPriority/<x>/MCDataGroupID

| Common/MCDataGroupTxns/FDPresentationPriority/<x>/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCData group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 9.2.16	/<x>/Common/MCDataGroupTxns/FDPresentationPriority/
<x>/MCDataGroupPriorityHierarchy

Table 9.2.16.1: /<x>/Common/MCDataGroupTxns/FDPresentationPriority/<x>/
MCDataGroupPriorityHierarchy

| Common/MCDataGroupTxns/FDPresentationPriority/<x>/MCDataGroupPriorityHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the requested presentation priority of FD transactions. |  |  |  |  |

-	Values: 0-255

The group with the lowest MCDataGroupPriorityHierarchy value shall be considered as the group having the lowest priority among the groups.

### 9.2.17	/<x>/Common/MCDataGroupTxns/ConversationPresentationPriority

Table 9.2.17.1: /<x>/Common/MCDataGroupTxns/ConversationPresentationPriority

| Common/MCDataGroupTxns/ConversationPresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the prioritized MCData conversation management configuration. |  |  |  |  |

### 9.2.18	/<x>/Common/MCDataGroupTxns/
ConversationPresentationPriority/<x>

Table 9.2.18.1: /<x>/Common/MCDataGroupTxns/ConversationPresentationPriority/<x>

| Common/MCDataGroupTxns/ConversationPresentationPriority /<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more prioritized MCData conversation management configuration. |  |  |  |  |

### 9.2.19	/<x>/Common/MCDataGroupTxns/
ConversationPresentationPriority/<x>/MCDataGroupID

Table 9.2.19.1: /<x>/Common/MCDataGroupTxns/ConversationPresentationPriority/<x>/
MCDataGroupID

| Common/MCDataGroupTxns/ConversationPresentationPriority/<x>/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCData group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 9.2.20	/<x>/Common/MCDataGroupTxns/
ConversationPresentationPriority/<x>/
MCDataGroupPriorityHierarchy

Table 9.2.20.1: /<x>/Common/MCDataGroupTxns/ConversationPresentationPriority/<x>/
MCDataGroupPriorityHierarchy

| Common/MCDataGroupTxns/ConversationPresentationPriority/<x>/MCDataGroupPriorityHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the requested presentation priority of conversation management transactions. |  |  |  |  |

-	Values: 0-255

The group with the lowest MCDataGroupPriorityHierarchy value shall be considered as the group having the lowest priority among the groups.

### 9.2.21	/<x>/Common/MCDataGroupTxns/MaxTCNc4

Table 9.2.21.1: /<x>/Common/MCDataGroupTxns/MaxTCNc4

| Common/MCDataGroupTxns/MaxTCNc4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous data transmissions. |  |  |  |  |

-	Values: 0-255

### 9.2.22	/<x>/Common/MCDataGroupTxns/MaxTCNc5

Table 9.2.22.1: /<x>/Common/MCDataGroupTxns/MaxTCNc5

| Common/MCDataGroupTxns/MaxTCN5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of data transmissions in a group. |  |  |  |  |

-	Values: 0-255

### 9.2.23	/<x>/Common/MCDataGroupTxns/DataPresentationPriority

Table 9.2.23.1: /<x>/Common/MCDataGroupTxns/DataPresentationPriority

| Common/MCDataGroupTxns/DataPresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the prioritized MCData Transmission Control configuration. |  |  |  |  |

### 9.2.24	/<x>/Common/MCDataGroupTxns/DataPresentationPriority/
<x>

Table 9.2.24.1: /<x>/Common/MCDataGroupTxns/DataPresentationPriority/<x>

| Common/MCDataGroupTxns/DataPresentationPriority/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more prioritized MCData Transmission Control configuration. |  |  |  |  |

### 9.2.25	/<x>/Common/MCDataGroupTxns/DataPresentationPriority/
<x>/MCDataGroupID

Table 9.2.25.1: /<x>/Common/MCDataGroupTxns/DataPresentationPriority/<x>/MCDataGroupID

| Common/MCDataGroupTxns/DataPresentationPriority/<x>/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCData group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 9.2.26	/<x>/Common/MCDataGroupTxns/DataPresentationPriority/
<x>/MCDataGroupPriorityHierarchy

Table 9.2.26.1: /<x>/Common/MCDataGroupTxns/DataPresentationPriority/<x>/
MCDataGroupPriorityHierarchy

| Common/MCDataGroupTxns/DataPresentationPriority/<x>/MCDataGroupPriorityHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the requested presentation priority of data received. |  |  |  |  |

-	Values: 0-255

The group with the lowest MCDataGroupPriorityHierarchy value shall be considered as the group having the lowest priority among the groups.

### 9.2.27	/<x>/Common/MCDataGroupTxns/MaxRCNc4

Table 9.2.27.1: /<x>/Common/MCDataGroupTxns/MaxRCNc4

| Common/MCDataGroupTxns/MaxRCNc4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous data receptions. |  |  |  |  |

-	Values: 0-255

### 9.2.28	/<x>/Common/MCDataGroupTxns/MaxRCNc5

Table 9.2.28.1: /<x>/Common/MCDataGroupTxns/MaxRCNc5

| Common/MCDataGroupTxns/MaxRCN5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of data receptions in a group. |  |  |  |  |

-	Values: 0-255

### 9.2.28A	/<x>/Common/MCDataSDSCharset

Table 9.2.28A.1: /<x>/Common/MCDataSDSCharset

| Common/MCDataSDSCharset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | int | Get, Replace |  |
|  | This leaf node indicates an available charset for text use in SDS messages. |  |  |  |  |

-	Values: 0-2999 as indexed in the MIBenum values for IANA Character Sets [27]

### 9.2.29	/<x>/OnNetwork

Table 9.2.29.1: /<x>/OnNetwork

| OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 9.2.30	/<x>/OnNetwork/RelayService

Table 9.2.30.1: /<x>/OnNetwork/RelayService

| OnNetwork/RelayService |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to use a relay service. |  |  |  |  |

When set to "true" the MCData group is allowed to use a relay service.

When set to "false" the MCData group is not allowed to use a relay service.

### 9.2.31	/<x>/OnNetwork/IPv6Preferred

Table 9.2.31.1: /<x>/OnNetwork/IPv6Preferred

| OnNetwork/IPv6Preferred |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether IPv6 is preferred over IPv4 for on-network operation when the MCData UE has both IPv4 and IPv6 host configuration. |  |  |  |  |

If the MCData UE has both IPv4 and IPv6 host configuration and:

-	if IPv6Preferred is set to "true" then the UE uses IPv6 for all on-network signalling and media; otherwise

-	if IPv6Preferred is set to "false" then the UE uses IPv4 for all on-network signalling and media.

### 9.2.32	/<x>/OnNetwork/RelayedMCDataGroup

Table 9.2.32.1: /<x>/OnNetwork/RelayedMCDataGroup

| OnNetwork/RelayedMCDataGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the allowed relayed MCData groups. |  |  |  |  |

### 9.2.33	/<x>/OnNetwork/RelayedMCDataGroup/<x>

Table 9.2.33.1: /<x>/OnNetwork/RelayedMCDataGroup/<x>

| OnNetwork/RelayedMCDataGroup/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more allowed relayed MCData groups. |  |  |  |  |

### 9.2.34	/<x>/OnNetwork/RelayedMCDataGroup/<x>/MCDataGroupID

Table 9.2.34.1: /<x>/OnNetwork/RelayedMCDataGroup/<x>/MCDataGroupID

| OnNetwork/RelayedMCDataGroup/<x>/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCData group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 9.2.35	/<x>/OnNetwork/RelayedMCDataGroup/<x>/RelayServiceCode

Table 9.2.35.1: /<x>/OnNetwork/RelayedMCDataGroup/<x>/RelayServiceCode

| OnNetwork/RelayedMCDataGroup/<x>/RelayServiceCode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

# 10	MCData user profile MO

## 10.1	General

The MCData user profile configuration Management Object (MO) is used to configure the MCData Client behaviour for the on-network or off-network MCData Service. The MCData user profile configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCData-user-profile:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCData user profile MO.

The following nodes and leaf objects are possible under the MCData user profile node as described in figures 10.1.1 through 10.1.5.

Figure 10.1.1: The MCData user profile MO (1 of 5)

Figure 10.1.2: The MCData user profile MO (2 of 5)

Figure 10.1.3: The MCData user profile MO (3 of 5)

Figure 10.1.4: The MCData user profile MO (4 of 5)

Figure 10.1.5: MCDataAdhocGroup MO contents (5 of 5)

## 10.2	MCData user profile MO parameters

### 10.2.1	General

This clause describes the parameters for the MCData user profile Management Object (MO).

### 10.2.2	Node: <x>

Table 10.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCData user profile Management Object (MO). For the MCData user profile MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcdata-user-profile:1.0" |  |  |  |  |

-	Values: N/A

### 10.2.3	/<x>/Name

Table 10.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCData user profile settings. |  |  |  |  |

-	Values: <User displayable name>

### 10.2.4	/<x>/Ext/

Table 10.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCData user profile MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 10.2.5	/<x>/<x>

Table 10.2.5.1: /<x>/<x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation and off-network operation. |  |  |  |  |

### 10.2.6	/<x>/<x>/Common

Table 10.2.6.1: /<x>/<x>/Common

| <x>/Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 10.2.7	/<x>/<x>/Common/MCDataUserID

Table 10.2.7.1: /<x>/<x>/Common/MCDataUserID

| <x>/Common/MCDataUserID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. |  |  |  |  |

### 10.2.8	/<x>/<x>/Common/MCDataUserProfileIndex

Table 10.2.8.1: /<x>/<x>/Common/MCDataUserProfileIndex

| <x>/Common/MCDataUserProfileIndex |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates an index for the particular MCData user profile. |  |  |  |  |

-	Values: 0-255

### 10.2.9	/<x>/<x>/Common/MCDataUserProfileName

Table 10.2.9.1: /<x>/<x>/Common/MCDataUserProfileName

| <x>/Common/MCDataUserProfileName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates a profile name for the particular MCData user profile. |  |  |  |  |

### 10.2.9A	/<x>/<x>/Common/MCDataUserIDKMSURI

Table 10.2.9A.1: /<x>/<x>/Common/MCDataUserIDKMSURI

| <x>/Common/MCDataUserIDKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates key management server URI for the security domain of the MCData user. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 10.2.10	/<x>/<x>/Common/PreSelectedIndication

Table 10.2.10.1: /<x>/<x>/Common/PreSelectedIndication

| <x>/Common/PreSelectedIndication |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | null | Get, Replace |  |
|  | This leaf node indicates that this MCData user profile MO is designated to be the pre-selected MCData user profile as defined by 3GPP TS 23.179 [15] |  |  |  |  |

-	Values: null

When this leaf node is present, this MCData user profile MO is designated as the pre-selected MCData user profile.

When this leaf node is absent, this MCData user profile MO is not designated as the pre-selected MCData user profile.

If more than one MCData user profile MO is specified for the MCData user, then only one MCData user profile MO for the MCData user shall contain the <x>/Common/PreSelectedIndication leaf node.

If there is only one MCData user profile MO specified for the MCData user, then it is optional to include the <x>/Common/PreSelectedIndication leaf node.

### 10.2.11	/<x>/<x>/Common/UserAliases

Table 10.2.11.1: /<x>/<x>/Common/UserAliases

| <x>/Common/UserAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of aliases of an MCData user. |  |  |  |  |

### 10.2.12	/<x>/<x>/Common/UserAliases/<x>

Table 10.2.12.1: /<x>/<x>/Common/UserAliases/<x>

| <x>/Common/UserAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more aliases of an MCData user. |  |  |  |  |

### 10.2.13	/<x>/<x>/Common/UserAliases/<x>UserAlias

Table 10.2.13.1: /<x>/<x>/Common/UserAliases/<x>/UserAlias

| <x>/Common/UserAliases/<x>/UserAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an alphanumeric alias of the MCData user. |  |  |  |  |

### 10.2.14	/<x>/<x>/Common/AuthorisedAlias

Table 10.2.14.1: /<x>/<x>/Common/AuthorisedAlias

| <x>/Common/AuthorisedAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates authorisation to create and delete aliases of other MCData users and their associated MCData user profiles. |  |  |  |  |

When set to "true" the MCData user is authorised to create and delete aliases of other MCData users and their associated MCData user profiles.

When set to "false" the MCData user is not authorised to create and delete aliases of other MCData user and their associated MCData user profiles. This is the default if this leaf node is not present.

### 10.2.15	/<x>/<x>/Common/ParticipantType

Table 10.2.15.1: /<x>/<x>/Common/ParticipantType

| <x>/Common/ParticipantType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the participant type of the MCData user. |  |  |  |  |

The ParticipantType means the functional category of the participant (e.g., first responder, second responder, dispatch, dispatch supervisor), typically defined by the MCData administrators.

### 10.2.16	/<x>/<x>/Common/Organization

Table 10.2.16.1: /<x>/<x>/Common/Organization

| <x>/Common/Organization |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the organization an MCData user belongs to. |  |  |  |  |

### 10.2.16A	/<x>/<x>/Common/OnetoOne

Table 10.2.16A.1: /<x>/<x>/Common/OnetoOne

| <x>/Common/OnetoOne |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData on-network or off-network one-to-one communication configuration. |  |  |  |  |

### 10.2.16B	/<x>/<x>/Common/OnetoOne/UserList

Table 10.2.16B.1: /<x>/<x>/Common/OnetoOne/UserList

| <x>/Common/OnetoOne/UserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCData users who can be involved in an on-network or off-network MCData one-to-one transaction. |  |  |  |  |

### 10.2.16C	/<x>/<x>/Common/OnetoOne/UserList/<x>

Table 10.2.16C.1: /<x>/<x>/Common/OnetoOne/UserList/<x>

| <x>/Common/OnetoOne/UserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCData users who can be involved in an on-network or off-network MCData one-to-one transaction. |  |  |  |  |

### 10.2.16D	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry

Table 10.2.16D.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry

| <x>/Common/OnetoOne/UserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCData users who can be involved in an on-network or off-network MCData one-to-one transaction. |  |  |  |  |

### 10.2.16E	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/MCDataID

Table 10.2.16E.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/MCDataID

| <x>/Common/OnetoOne/UserList/<x>/Entry/MCDataID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.16F	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/
DiscoveryGroupID

Table 10.2.16F.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/DiscoveryGroupID

| <x>/Common/OnetoOne/UserList/<x>/Entry/DiscoveryGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a discovery group ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The value is used as the discovery group ID in the ProSe discovery procedures as specified in 3GPP TS 23.303 [6].

### 10.2.16F1	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/
ApplicationLayerGroupID

Table 10.2.16F1.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/ApplicationLayerGroupID

| <x>/Common/OnetoOne/UserList/<x>/Entry/ApplicationLayerGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates an application layer group ID as specified in 3GPP TS 23.304 [25]. |  |  |  |  |

The value is used as the application layer group ID in the ProSe discovery procedures as specified in 3GPP TS 23.304 [25].

### 10.2.16G	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/UserInfoID

Table 10.2.16G.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/UserInfoID

| <x>/Common/OnetoOne/UserList/<x>/Entry/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 10.2.16H	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/
MCDataIDKMSURI

Table 10.2.16H.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/MCDataUserIDKMSURI

| <x>/Common/OnetoOne/UserList/<x>/Entry/MCDataUserIDKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates key management server URI for the security domain of the MCData user of an MCData user that the configured MCData user is authorised to initiate a one-to-one communication with. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 10.2.16I	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/DisplayName

Table 10.2.16I.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/DisplayName

| <x>/Common/OnetoOne/UserList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name. |  |  |  |  |

### 10.2.16J	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation

Table 10.2.16J.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation

| <x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of IPInformation data that can be used in the setup of an IP Connectivity session for a specific MCData user in a one-to-one communication. |  |  |  |  |

### 10.2.16K	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/
IPInformation/<x>

Table 10.2.16K.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>

| <x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of IPInformation data that can be used in the setup of an IP Connectivity session for a specific MCData user in a one-to-one communication. |  |  |  |  |

### 10.2.16L	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/
IPInformation/<x>/Entry

Table 10.2.16L.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry

| <x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of IPInformation data that can be used in the setup of an IP Connectivity session for a specific MCData user in a one-to-one communication. |  |  |  |  |

### 10.2.16M	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/
IPInformation/<x>/Entry/IPv4Information

Table 10.2.16M.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry/
IPv4Information

| <x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry/IPv4Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an IPv4 host address or an IPv4 network that can be addressed on an IP Connectivity session in a one-to-one communication for a specific MCData ID. |  |  |  |  |

### 10.2.16N	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/
IPInformation/<x>/Entry/IPv6Information

Table 10.2.16N.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry/
IPv6Information

| <x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry/IPv6Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an IPv6 host address or an IPv6 network that can be addressed on an IP Connectivity session in a one-to-one communication for a specific MCData ID. |  |  |  |  |

### 10.2.16O	/<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/
IPInformation/<x>/Entry/FQDN

Table 10.2.16O.1: /<x>/<x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry/FQDN

| <x>/Common/OnetoOne/UserList/<x>/Entry/IPInformation/<x>/Entry/FQDN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates a fully qualified domain name for a specific host or domain that can be addressed on an IP Connectivity session in a one-to-one communication for a specific MCData ID. |  |  |  |  |

### 10.2.17	/<x>/<x>/Common/FileDistribution

Table 10.2.17.1: /<x>/<x>/Common/FileDistribution

| <x>/Common/FileDistribution |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData file distribution configuration. |  |  |  |  |

### 10.2.18	/<x>/<x>/Common/FileDistribution/CancelList

Table 10.2.18.1: /<x>/<x>/Common/FileDistribution/CancelList

| <x>/Common/FileDistribution/CancelList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCData user(s) this MCData user is allowed to cancel distribution of files beings sent or waiting to be sent. |  |  |  |  |

### 10.2.19	/<x>/<x>/Common/FileDistribution/CancelList/<x>

Table 10.2.19.1: /<x>/<x>/Common/FileDistribution/CancelList/<x>

| <x>/Common/FileDistribution/CancelList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more list of MCData users this MCData user is allowed to cancel distribution of files beings sent or waiting to be sent. |  |  |  |  |

### 10.2.20	/<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry

Table 10.2.20.1: /<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry

| <x>/Common/FileDistribution/CancelList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCData users this MCData user is allowed to cancel distribution of files beings sent or waiting to be sent. |  |  |  |  |

### 10.2.21	/<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/MCDataID

Table 10.2.21.1: /<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/MCDataID

| <x>/Common/FileDistribution/CancelList/<x>/Entry/MCDataID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.21A	/<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/
MCDataIDKMSURI

Table 10.2.21A.1: /<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/MCDataIDKMSURI

| <x>/Common/FileDistribution/CancelList/<x>/Entry/MCDataIDKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates key management server URI for the security domain of the MCData user of an MCData user that the configured MCData user is authorised to cancel distribution of files being sent or waiting to be sent. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 10.2.22	/<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/
DisplayName

Table 10.2.22.1: /<x>/<x>/Common/FileDistribution/CancelList/<x>/Entry/DisplayName

| <x>/Common/FileDistribution/CancelList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name. |  |  |  |  |

### 10.2.23	/<x>/<x>/Common/TxRxControl

Table 10.2.23.1: /<x>/<x>/Common/TxRxControl

| <x>/Common/TxRxControl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData transmission and reception control configuration. |  |  |  |  |

### 10.2.24	/<x>/<x>/Common/TxRxControl/AuthorisedTransmit

Table 10.2.24.1: /<x>/<x>/Common/TxRxControl/AuthorisedTransmit

| <x>/Common/TxRxControl/AuthorisedTransmit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to transmit data. |  |  |  |  |

When set to "true" the MCData user is authorised to transmit data.

When set to "false" the MCData user is not authorised to transmit data.

### 10.2.25	/<x>/<x>/Common/TxRxControl/MaxData1To1

Table 10.2.25.1: /<x>/<x>/Common/TxRxControl/MaxData1To1

| <x>/Common/TxRxControl/MaxData1To1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum amount of data (in megabytes) that an MCData user can transmit in a single request during one-to-one communication. |  |  |  |  |

-	Values: 0-65535

The MaxData1to1 size is in megabytes.

### 10.2.26	/<x>/<x>/Common/TxRxControl/MaxTime1To1

Table 10.2.26.1: /<x>/<x>/Common/TxRxControl/MaxTime1To1

| <x>/Common/TxRxControl/MaxTime1To1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum amount of time that an MCData user can transmit for in a single request during one-to-one communication. |  |  |  |  |

-	Values: 0-65535

The MaxTime1To1 time is in seconds.

### 10.2.27	/<x>/<x>/Common/TxRxControl/ReleaseList

Table 10.2.27.1: /<x>/<x>/Common/TxRxControl/ReleaseList

| <x>/Common/TxRxControl/ReleaseList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCData users that this MCData user is allowed to request release of an ongoing transmission. |  |  |  |  |

### 10.2.28	/<x>/<x>/Common/TxRxControl/ReleaseList/<x>

Table 10.2.28.1: /<x>/<x>/Common/TxRxControl/ReleaseList/<x>

| <x>/Common/TxRxControl/ReleaseList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more list of MCData users this MCData user is allowed to request release of an ongoing transmission. |  |  |  |  |

### 10.2.29	/<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry

Table 10.2.29.1: /<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry

| <x>/Common/TxRxControl/ReleaseList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCData users this MCData user is allowed to request release of an ongoing transmission. |  |  |  |  |

### 10.2.30	/<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry/MCDataID

Table 10.2.30.1: /<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry/MCDataID

| <x>/Common/TxRxControl/ReleaseList/<x>/Entry/MCDataID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.31	/<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry/
DisplayName

Table 10.2.31.1: /<x>/<x>/Common/TxRxControl/ReleaseList/<x>/Entry/DisplayName

| <x>/Common/TxRxControl/ReleaseList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name. |  |  |  |  |

### 10.2.32	/<x>/<x>/Common/MCDataGroupBroadcast

Table 10.2.32.1: /<x>/<x>/Common/MCDataGroupBroadcast

| <x>/Common/MCDataGroupBroadcast |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group-broadcast group policy. |  |  |  |  |

### 10.2.33	/<x>/<x>/Common/MCDataGroupBroadcast/Authorised

Table 10.2.33.1: /<x>/<x>/Common/MCDataGroupBroadcast/Authorised

| <x>/Common/MCDataGroupBroadcast/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to create a group-broadcast group. |  |  |  |  |

When set to "true" the MCData user is authorised to create a group-broadcast group.

When set to "false" the MCData user is not authorised to create a group-broadcast group.

### 10.2.34	/<x>/<x>/Common/UserBroadcast

Table 10.2.34.1: /<x>/<x>/Common/UserBroadcast

| <x>/Common/UserBroadcast |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the user-broadcast group policy. |  |  |  |  |

### 10.2.35	/<x>/<x>/Common/UserBroadcast/Authorised

Table 10.2.35.1: /<x>/<x>/Common/UserBroadcast/Authorised

| <x>/Common/UserBroadcast/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to create a user-broadcast group. |  |  |  |  |

When set to "true" the MCData user is authorised to create a user-broadcast group.

When set to "false" the MCData user is not authorised to create a user-broadcast group.

### 10.2.36	/<x>/<x>/Common/GroupEmergencyAlert

Table 10.2.36.1: /<x>/<x>/Common/GroupEmergencyAlert

| <x>/Common/GroupEmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for Group Emergency Alert configuration. |  |  |  |  |

### 10.2.37	/<x>/<x>/Common/GroupEmergencyAlert/Entry

Table 10.2.37.1: /<x>/<x>/Common/GroupEmergencyAlert/Entry

| <x>/Common/GroupEmergencyAlert/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCData group of an MCData emergency alert. |  |  |  |  |

### 10.2.38	/<x>/<x>/Common/GroupEmergencyAlert/Entry/ID

Table 10.2.38.1: /<x>/<x>/Common/GroupEmergencyAlert/Entry/ID

| <x>/Common/GroupEmergencyAlert/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCData group used upon certain criteria on initiation of an MCData emergency alert. |  |  |  |  |

### 10.2.39	/<x>/<x>/Common/GroupEmergencyAlert/Entry/DisplayName

Table 10.2.39.1: /<x>/<x>/Common/GroupEmergencyAlert/Entry/DisplayName

| <x>/Common/GroupEmergencyAlert/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the ID. |  |  |  |  |

### 10.2.40	/<x>/<x>/Common/GroupEmergencyAlert/Entry/Usage

Table 10.2.40.1: /<x>/<x>/Common/GroupEmergencyAlert/Entry/Usage

| <x>/Common/GroupEmergencyAlert/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCData emergency alert uses the ID. |  |  |  |  |

The valid values are 'UseCurrentlySelectedGroup' and 'DedicatedGroup'.

When set to 'UseCurrentlySelectedGroup' then if the MCData user has currently selected an MCData group then use that MCData group for an on-network MCData emergency alert, if the MCData user does not have a currently selected MCData group then use the MCData group identified by the ID in clause 10.2.38 for an MCData emergency alert.

When set to 'DedicatedGroup' then use the MCData group identified by the ID in clause 10.2.38 for an MCData emergency alert.

### 10.2.41	/<x>/<x>/Common/AllowedActivateAlert

Table 10.2.41.1: /<x>/<x>/Common/AllowedActivateAlert

| <x>/Common/AllowedActivateAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to activate an emergency alert |  |  |  |  |

When set to "true" the MCData user is authorised to activate an emergency alert.

When set to "false" the MCData user is not authorised to activate an emergency alert.

### 10.2.42	/<x>/<x>/Common/AllowedCancelAlert

Table 10.2.42.1: /<x>/<x>/Common/AllowedCancelAlert

| <x>/Common/AllowedCancelAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is allowed to cancel an emergency alert. |  |  |  |  |

When set to "true" the MCData user is authorised to cancel an emergency alert.

When set to "false" the MCData user is not authorised to cancel an emergency alert.

### 10.2.42A	/<x>/<x>/Common/AllowedEmergencyPrivateCall

Table 10.2.42A.1: /<x>/<x>/Common/AllowedEmergencyPrivateCall

| <x>/Common/AllowedEmergencyPrivateCall |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Status | Occurrence | Format | Min. Access Types |  |  |
|  |  | Required | One | bool | Get, Replace |  |  |
|  | This leaf node indicates the authorisation to initiate an MCData emergency one-to-one communication or to upgrade an MCData normal priority one-to-one communication to an MCData emergency one-to-one communication. |  |  |  |  |  |

When set to "true" the MCData user is authorised to initiate an MCData emergency one-to-one communication or to upgrade an MCData normal priority one-to-one communication to an MCData emergency one-to-one communication.

When set to "false" the MCData user is not authorised to initiate an MCData emergency one-to-one communication or to upgrade an MCData normal priority one-to-one communication to an MCData emergency one-to-one communication.

### 10.2.42B	/<x>/<x>/Common/AllowedCancelEmergencyPrivateCall

Table 10.2.42B.1: /<x>/<x>/Common/AllowedCancelEmergencyPrivateCall

| <x>/Common/AllowedCancelEmergencyPrivateCall |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Status | Occurrence | Format | Min. Access Types |  |  |
|  |  | Required | One | bool | Get, Replace |  |  |
|  | This leaf node indicates the authorisation to downgrade an MCData emergency one-to-one communication to a normal priority one-to-one communication (e.g. reduce the priority of the communication). |  |  |  |  |  |

When set to "true" the MCData user is authorised to downgrade an MCData emergency one-to-one communication to a normal priority one-to-one communication.

When set to "false" the MCData user is not authorised to downgrade an MCData emergency one-to-one communication to a normal priority one-to-one communication.

### 10.2.43	/<x>/<x>/OnNetwork

Table 10.2.43.1: /<x>/<x>/OnNetwork

| <x>/OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 10.2.44	/<x>/<x>/OnNetwork/MCDataGroupList

Table 10.2.44.1: /<x>/<x>/OnNetwork/MCDataGroupList

| <x>/OnNetwork/MCDataGroupList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the list of on-network MCData groups that the MCData user is allowed to use. |  |  |  |  |

### 10.2.45	/<x>/<x>/OnNetwork/MCDataGroupList/<x>

Table 10.2.45.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>

| <x>/OnNetwork/MCDataGroupList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of on-network MCData groups that the MCData user is allowed to use. |  |  |  |  |

### 10.2.46	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry

Table 10.2.46.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry

| <x>/OnNetwork/MCDataGroupList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCData groups that the MCData user is allowed to use. |  |  |  |  |

### 10.2.47	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/MCDataGroupID

Table 10.2.47.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/MCDataGroupID

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCData group ID for the on-network MCData group that the MCData user is allowed to use. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.48	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/DisplayName

Table 10.2.48.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/DisplayName

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCData Group ID. |  |  |  |  |

### 10.2.49	Void

### 10.2.50	Void

### 10.2.51	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/GMSServID

Table 10.2.51.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/GMSServID

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/GMSServID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the group management server hosting the MCData Group ID. |  |  |  |  |

### 10.2.52	Void

### 10.2.53	Void

### 10.2.54	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
IdMSTokenEndPoint

Table 10.2.54.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/IdMSTokenEndPoint

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/IdMSTokenEndPoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the IDMS token endpoint for the MCData Group ID in the MCDataGroupList. If the value is empty, the IDMS identities (IDMSAuthEndpoint and IDMSTokenEndpoint) present in the MCS UE initial configuration MO are used. |  |  |  |  |

### 10.2.54A	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
GroupKMSURI

Table 10.2.54A.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/GroupKMSURI

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/GroupKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates key management server URI for the security domain of the MCData group identity (MCData Group ID) of the on-network MCData group |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 10.2.55	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RelativePresentationPriority

Table 10.2.55.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RelativePresentationPriority

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RelativePresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates indicating the presentation priority of the on-network group for the MCData user relative to other on-network groups and on-network users. |  |  |  |  |

-	Values: 0-255

The lowest RelativePresentationPriority value shall be considered as the MCData group transaction having the lowest priority for presentation among other group MCData and one-to-one user transactions.

### 10.2.55A	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation

Table 10.2.55A.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the rules that control automatic affiliation. |  |  |  |  |

### 10.2.55A0	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>

Table 10.2.55A0.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the rules that control automatic affiliation. |  |  |  |  |

### 10.2.55A1	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/

Table 10.2.55A1.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 10.2.55A2	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>

Table 10.2.55A2.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 10.2.55A3	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry

Table 10.2.55A3.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 10.2.55A4	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea

Table 10.2.55A4.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 10.2.55A5	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea

Table 10.2.55A5.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 10.2.55A6	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>

Table 10.2.55A6.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 10.2.55A7	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 10.2.55A7.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 10.2.55A8	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 10.2.55A8.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 10.2.55A9	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 10.2.55A9.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 10.2.55A10	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea

Table 10.2.55A10.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 10.2.55A11	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center

Table 10.2.55A11.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.55A12	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 10.2.55A12.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.55A13	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 10.2.55A13.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 10.2.55A14	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 10.2.55A14.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 10.2.55A15	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Radius

Table 10.2.55A15.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 10.2.55A16	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/OffsetAngle

Table 10.2.55A16.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 10.2.55A17	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/IncludedAngle

Table 10.2.55A17.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 10.2.55A18	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed

Table 10.2.55A18.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 10.2.55A19	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MinimumSpeed

Table 10.2.55A19.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.55A20	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MaximumSpeed

Table 10.2.55A20.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.55A21	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading

Table 10.2.55A21.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 10.2.55A22	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MinimumHeading

Table 10.2.55A22.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.55A23	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MaximumHeading

Table 10.2.55A23.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.55A24	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea

Table 10.2.55A24.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 10.2.55A25	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea

Table 10.2.55A25.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 10.2.55A26	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>

Table 10.2.55A26.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 10.2.55A27	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 10.2.55A27.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 10.2.55A28	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 10.2.55A28.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 10.2.55A29	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 10.2.55A29.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 10.2.55A30	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea

Table 10.2.55A30.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 10.2.55A31	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center

Table 10.2.55A31.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.55A32	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 10.2.55A32.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.55A33	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 10.2.55A33.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 10.2.55A34	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 10.2.55A34.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 10.2.55A35	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Radius

Table 10.2.55A35.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 10.2.55A36	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/OffsetAngle

Table 10.2.55A36.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 10.2.55A37	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/IncludedAngle

Table 10.2.55A37.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 10.2.55A38	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed

Table 10.2.55A38.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 10.2.55A39	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MinimumSpeed

Table 10.2.55A39.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.55A40	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MaximumSpeed

Table 10.2.55A40.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.55A41	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading

Table 5. 10.2.55A41.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 10.2.55A42	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MinimumHeading

Table 10.2.55A42.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.55A43	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MaximumHeading

Table 10.2.55A43.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.55A44	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases

Table 10.2.55A44.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 10.2.55A45	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>

Table 10.2.55A45.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 10.2.55A46	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry

Table 10.2.55A46.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 10.2.55A47	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/
Entry/FunctionalAlias

Table 10.2.55A47.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node contains a functional alias. |  |  |  |  |

### 10.2.55B	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation

Table 10.2.55B.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the rules that control automatic deaffiliation. |  |  |  |  |

### 10.2.55B0	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>

Table 10.2.55B0.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the zero or more rules that control automatic deaffiliation. |  |  |  |  |

### 10.2.55B1	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/

Table 10.2.55B1.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 10.2.55B2	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>

Table 10.2.55B2.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 10.2.55B3	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry

Table 10.2.55B3.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 10.2.55B4	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea

Table 10.2.55B4.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which, when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 10.2.55B5	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea

Table 10.2.55B5.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 10.2.55B6	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>

Table 10.2.55B6.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 10.2.55B7	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 10.2.55B7.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesFordeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 10.2.55B8	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 10.2.55B8.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 10.2.55B9	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 10.2.55B9.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 10.2.55B10	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea

Table 10.2.55B10.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 10.2.55B11	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center

Table 10.2.55B11.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.55B12	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 10.2.55B12.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.55B13	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 10.2.55B13.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 10.2.55B14	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 10.2.55B14.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 10.2.55B15	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Radius

Table 10.2.55B15.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 10.2.55B16	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/OffsetAngle

Table 10.2.55B16.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 10.2.55B17	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/IncludedAngle

Table 10.2.55B17.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 10.2.55B18	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed

Table 10.2.55B18.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 10.2.55B19	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MinimumSpeed

Table 10.2.55B19.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.55B20	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MaximumSpeed

Table 10.2.55B20.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.55B21	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading

Table 10.2.55B21.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 10.2.55B22	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MinimumHeading

Table 10.2.55B22.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.55B23	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MaximumHeading

Table 10.2.55B23.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/Maximum |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.55B24	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea

Table 10.2.55B24.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 10.2.55B25	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea

Table 10.2.55B25.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 10.2.55B26	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>

Table 10.2.55B26.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 10.2.55B27	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 10.2.55B27.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 10.2.55B28	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 10.2.55B28.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 10.2.55B29	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 10.2.55B29.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 10.2.55B30	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea

Table 10.2.55B30.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 10.2.55B31	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center

Table 10.2.55B31.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.55B32	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 10.2.55B32.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.55B33	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 10.2.55B33.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 10.2.55B34	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry
/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 10.2.55B34.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 10.2.55B35	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Radius

Table 10.2.55B35.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 10.2.55B36	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/OffsetAngle

Table 10.2.55B36.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 10.2.55B37	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/IncludedAngle

Table 10.2.55B37.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 10.2.55B38	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed

Table 10.2.55B38.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 10.2.55B39	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MinimumSpeed

Table 10.2.55B39.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.55B40	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MaximumSpeed

Table 10.2.55B40.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.55B41	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading

Table 10.2.55B41.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 10.2.55B42	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MinimumHeading

Table 10.2.55B42.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: in the range 0-359.

### 10.2.55B43	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MaximumHeading

Table 10.2.55B43.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.55B44	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases

Table 10.2.55B44.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 10.2.55B45	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>

Table 10.2.55B45.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 10.2.55B46	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry

Table 10.2.55B46.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 10.2.55B47	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/
FunctionalAlias

Table 10.2.55B47.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node contains a functional alias. |  |  |  |  |

### 10.2.55B48	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet

Table 10.2.55B48.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/ManualDeAffiliationNotAllowedIfAffiliationRulesAreMet |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to deaffiliate if the affiliation criteria are met. |  |  |  |  |

### 10.2.55C	/<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/
AllowStoreGroupCommInMsgstore

Table 10.2.55C.1: /<x>/<x>/OnNetwork/MCDataGroupList/<x>/Entry/AllowStoreGroupCommInMsgstore

| <x>/OnNetwork/MCDataGroupList/<x>/Entry/AllowStoreGroupCommInMsgstore |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to request an MCData server to store his/her group communication into message store or not. |  |  |  |  |

When set to "true", the MCData user is authorised to request an MCData server to store this group communication into message store. The group is identified by the MCData Group ID in the same structure of the group entry.

When set to "false", the MCData user is not authorised to request an MCData server to store this group communication into message store. The group is identified by the MCData Group ID in the same structure of the group entry.

### 10.2.56	/<x>/<x>/OnNetwork/ImplicitAffiliations

Table 10.2.56.1: /<x>/<x>/OnNetwork/ImplicitAffiliations

| <x>/OnNetwork/ImplicitAffiliations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the implicit affiliation configuration. |  |  |  |  |

### 10.2.57	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>

Table 10.2.57.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>

| <x>/OnNetwork/ImplicitAffiliations/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more implicit affiliation configuration. |  |  |  |  |

### 10.2.58	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry

Table 10.2.58.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCData groups that the MCData user is implicitly affiliated to. |  |  |  |  |

### 10.2.59	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCDataGroupID

Table 10.2.59.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCDataGroupID

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a MCData group ID to which the MCData user is implicitly affiliated to. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.60	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName

Table 10.2.60.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCData Group ID. |  |  |  |  |

### 10.2.61	/<x>/<x>/OnNetwork/PresenceStatus

Table 10.2.61.1: /<x>/<x>/OnNetwork/PresenceStatus

| <x>/OnNetwork/PresenceStatus |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the presence status configuration. |  |  |  |  |

### 10.2.62	/<x>/<x>/OnNetwork/PresenceStatus/<x>

Table 10.2.62.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>

| <x>/OnNetwork/PresenceStatus/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more presence status configuration. |  |  |  |  |

### 10.2.63	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry

Table 10.2.63.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry

| <x>/OnNetwork/PresenceStatus/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCData IDs of MCData users that the configured MCData user is authorised to obtain presence status. |  |  |  |  |

### 10.2.64	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/MCDataID

Table 10.2.64.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/MCDataID

| <x>/OnNetwork/PresenceStatus/<x>/Entry/MCDataID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCData ID of an MCData user that the configured MCData user is authorised to obtain presence status. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.65	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/DisplayName

Table 10.2.65.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/DisplayName

| <x>/OnNetwork/PresenceStatus/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCData ID. |  |  |  |  |

### 10.2.66	/<x>/<x>/OnNetwork/RemoteGroupChange

Table 10.2.66.1: /<x>/<x>/OnNetwork/RemoteGroupChange

| <x>/OnNetwork/RemoteGroupChange |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the remote group change configuration. |  |  |  |  |

### 10.2.67	/<x>/<x>/>/OnNetwork/RemoteGroupChange/<x>

Table 10.2.67.1: /<x>/<x>/OnNetwork/RemoteGroupChange/<x>

| <x>/OnNetwork/RemoteGroupChange/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more remote group change configuration. |  |  |  |  |

### 10.2.68	/<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry

Table 10.2.68.1: /<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry

| <x>/OnNetwork/RemoteGroupChange/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCData IDs of MCData users whose selected groups are authorised to be remotely changed by the configured MCData user. |  |  |  |  |

### 10.2.69	/<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry/MCDataID

Table 10.2.69.1: /<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry/MCDataID

| <x>/OnNetwork/RemoteGroupChange/<x>/Entry/MCDataID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCData ID of an MCData user whose selected groups are authorised to be remotely changed by the configured MCData user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.70	/<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry/DisplayName

Table 10.2.70.1: /<x>/<x>/OnNetwork/RemoteGroupChange/<x>/Entry/DisplayName

| <x>/OnNetwork/RemoteGroupChange/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCData ID. |  |  |  |  |

### 10.2.71	/<x>/<x>/OnNetwork/MaxAffiliationsN2

Table 10.2.71.1: /<x>/<x>/OnNetwork/MaxAffiliationsN2

| <x>/OnNetwork/MaxAffiliationsN2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of MCData groups that the MCData user is authorised to affiliate with. |  |  |  |  |

-	Values: 0-65535

### 10.2.72	/<x>/<x>/OnNetwork/ConversationManagement

Table 10.2.72.1: /<x>/<x>/OnNetwork/ConversationManagement

| <x>/OnNetwork/ConversationManagement |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for conversation management configuration |  |  |  |  |

### 10.2.73	/<x>/<x>/OnNetwork/ConversationManagement/GroupHangTime

Table 10.2.73.1: /<x>/<x>/OnNetwork/ConversationManagement/GroupHangTime

| <x>/OnNetwork/ConversationManagement/GroupHangTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for group hang time configuration |  |  |  |  |

### 10.2.74	/<x>/<x>/OnNetwork/ConversationManagement/
GroupHangTime/<x>

Table 10.2.74.1: /<x>/<x>/OnNetwork/ConversationManagement/GroupHangTime/<x>

| <x>/OnNetwork/ConversationManagement/GroupHangTime/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more hang time configurations. |  |  |  |  |

### 10.2.75	/<x>/<x>/OnNetwork/ConversationManagement/
GroupHangTime/<x>/Entry

Table 10.2.75.1: /<x>/<x>/OnNetwork/ConversationManagement/GroupHangTime/<x>/Entry

| <x>/OnNetwork/ConversationManagement/GroupHangTime/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the conversation hang time configuration associated with the configured MCData group, for the MCData user. |  |  |  |  |

### 10.2.76	/<x>/<x>/OnNetwork/ConversationManagement/
GroupHangTime/<x>/Entry/MCDataGroupID

Table 10.2.76.1: /<x>/<x>/OnNetwork/ConversationManagement/GroupHangTime/<x>/Entry/
MCDataGroupID

| <x>/OnNetwork/ConversationManagement/GroupHangTime/<x>/Entry/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCData group ID for the on-network MCData group. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.77	/<x>/<x>/OnNetwork/ConversationManagement/
GroupHangTime/<x>/Entry/DisplayName

Table 10.2.77.1: /<x>/<x>/OnNetwork/ConversationManagement/GroupHangTime/<x>/Entry/
DisplayName

| <x>/OnNetwork/ConversationManagement/GroupHangTime/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCData Group ID. |  |  |  |  |

### 10.2.78	/<x>/<x>/OnNetwork/ConversationManagement/
GroupHangTime/<x>/Entry/HangTime

Table 10.2.78.1: /<x>/<x>/OnNetwork/ConversationManagement/GroupHangTime/<x>/Entry/HangTime

| <x>/OnNetwork/ConversationManagement/GroupHangTime/<x>/Entry/HangTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the group call hang timer of the MCData group for conversation management, for the MCData user. |  |  |  |  |

-	Values: 0-65535

The HangTime is in seconds.

### 10.2.79	/<x>/<x>/OnNetwork/ConversationManagement/
DeliveredDisposition

Table 10.2.79.1: /<x>/<x>/OnNetwork/ConversationManagement/DeliveredDisposition

| <x>/OnNetwork/ConversationManagement/DeliveredDisposition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for delivered disposition configuration |  |  |  |  |

### 10.2.80	/<x>/<x>/OnNetwork/ConversationManagement/
DeliveredDisposition/<x>

Table 10.2.80.1: /<x>/<x>/OnNetwork/ConversationManagement/DeliveredDisposition/<x>

| <x>/OnNetwork/ConversationManagement/DeliveredDisposition/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more delivered disposition configurations. |  |  |  |  |

### 10.2.81	/<x>/<x>/OnNetwork/ConversationManagement/
DeliveredDisposition/<x>/Entry

Table 10.2.81.1: /<x>/<x>/OnNetwork/ConversationManagement/DeliveredDisposition/<x>/Entry

| <x>/OnNetwork/ConversationManagement/DeliveredDisposition/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the delivered disposition configuration associated with the MCData user, indicating an MCData ID of an MCData user who is to be sent a message delivered disposition notification in addition to the message sender. |  |  |  |  |

### 10.2.82	/<x>/<x>/OnNetwork/ConversationManagement/
DeliveredDisposition/<x>/Entry/MCDataID

Table 10.2.82.1: /<x>/<x>/OnNetwork/ConversationManagement/DeliveredDisposition/<x>/Entry/
MCDataID

| <x>/OnNetwork/ConversationManagement/DeliveredDisposition/<x>/Entry/MCDataID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCData ID for the on-network MCData user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.83	/<x>/<x>/OnNetwork/ConversationManagement/
DeliveredDisposition/<x>/Entry/DisplayName

Table 10.2.83.1: /<x>/<x>/OnNetwork/ConversationManagement/DeliveredDisposition/<x>/Entry/
DisplayName

| <x>/OnNetwork/ConversationManagement/DeliveredDisposition/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCData ID. |  |  |  |  |

### 10.2.84	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition

Table 10.2.84.1: /<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition

| <x>/OnNetwork/ConversationManagement/ReadDisposition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for read disposition configuration |  |  |  |  |

### 10.2.85	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/
<x>

Table 10.2.85.1: /<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/<x>

| <x>/OnNetwork/ConversationManagement/ReadDisposition/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more read disposition configurations. |  |  |  |  |

### 10.2.86	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/
<x>/Entry

Table 10.2.86.1: /<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/<x>/Entry

| <x>/OnNetwork/ConversationManagement/ReadDisposition/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the read disposition configuration associated with the MCData user, indicating an MCData ID of an MCData user who is to be sent a message read disposition notification in addition to the message sender. |  |  |  |  |

### 10.2.87	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/
<x>/Entry/MCDataID

Table 10.2.87.1: /<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/<x>/Entry/MCDataID

| <x>/OnNetwork/ConversationManagement/ReadDisposition/<x>/Entry/MCDataID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCData ID for the on-network MCData user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.88	/<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/
<x>/Entry/DisplayName

Table 10.2.88.1: /<x>/<x>/OnNetwork/ConversationManagement/ReadDisposition/<x>/Entry/DisplayName

| <x>/OnNetwork/ConversationManagement/ReadDisposition/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCData ID. |  |  |  |  |

### 10.2.89	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert

Table 10.2.89.1: /<x>/<x>/OnNetwork/OneToOneEmergencyAlert

| <x>/OnNetwork/OneToOneEmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for on-network one-to-one emergency alert configuration. |  |  |  |  |

### 10.2.90	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry

Table 10.2.90.1: /<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry

| <x>/OnNetwork/OneToOneEmergencyAlert/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCData ID of an MCData emergency alert. |  |  |  |  |

### 10.2.91	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/ID

Table 10.2.91.1: /<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/ID

| <x>/OnNetwork/OneToOneEmergencyAlert/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCData user ID used upon certain criteria on initiation of a one-to-one MCData emergency alert. |  |  |  |  |

### 10.2.92	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/
DisplayName

Table 10.2.92.1: /<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/DisplayName

| <x>/OnNetwork/OneToOneEmergencyAlert/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the ID. |  |  |  |  |

### 10.2.93	/<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/Usage

Table 10.2.93.1: /<x>/<x>/OnNetwork/OneToOneEmergencyAlert/Entry/Usage

| <x>/OnNetwork/OneToOneEmergencyAlert/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCData emergency alert uses the ID. |  |  |  |  |

The valid values are 'LocallyDetermined' and 'UsePreConfigured'.

When set to 'LocallyDetermined' then if the MCData user selects an MCData ID then use that MCData ID for an on-network MCData emergency alert, if the MCData user does not select a MCData ID then use the MCData ID identified by the ID in clause 10.2.91 for an on-network MCData emergency alert.

When set to 'UsePreConfigured' then use the ID identified by the ID in clause 10.2.91 for an on-network MCData emergency alert.

### 10.2.94	/<x>/<x>/OnNetwork/AllowedRegroup

Table 10.2.94.1: /<x>/<x>/OnNetwork/AllowedRegroup

| <x>/OnNetwork/AllowedRegroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to perform dynamic regrouping operations. |  |  |  |  |

When set to "true" the MCData user is authorised to perform dynamic regrouping operations.

When set to "false" the MCData user is not authorised to perform dynamic regrouping operations.

### 10.2.95	/<x>/<x>/OnNetwork/AllowedPresenceStatus

Table 10.2.95.1: /<x>/<x>/OnNetwork/AllowedPresenceStatus

| <x>/OnNetwork/AllowedPresenceStatus |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the presence status on the network of this MCData user is available. |  |  |  |  |

When set to "true" the presence status on the network of this MCData user is available.

When set to "false" the presence status on the network of this MCData user is not available. This is the default if this leaf node is not present.

### 10.2.96	/<x>/<x>/OnNetwork/AllowedPresence

Table 10.2.96.1: /<x>/<x>/OnNetwork/AllowedPresence

| <x>/OnNetwork/AllowedPresence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to obtain whether a particular MCData User is present on the network. |  |  |  |  |

When set to "true" the MCData user is authorised to obtain whether a particular MCData User is present on the network.

When set to "false" the MCData user is not authorised to obtain whether a particular MCData User is present on the network.

### 10.2.97	/<x>/<x>/OnNetwork/AllowedManualSwitch

Table 10.2.97.1: /<x>/<x>/OnNetwork/AllowedManualSwitch

| <x>/OnNetwork/AllowedManualSwitch |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to manually switch to off-network operation while in on-network operation. |  |  |  |  |

When set to "true" the MCData user is authorised to manually switch to off-network operation while in on-network operation.

When set to "false" the MCData user is not authorised to manually switch to off-network operation while in on-network operation.

### 10.2.97A	/<x>/<x>/OnNetwork/MCDataContentServerURI

Table 10.2.97A.1: /<x>/<x>/OnNetwork/MCDataContentServerURI

| <x>/OnNetwork/MCDataContentServerURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the absolute URI associated with the media storage function of the MCData content server. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 10.2.97B	/<x>/<x>/OnNetwork/FunctionalAliasList

Table 10.2.97B.1: /<x>/<x>/OnNetwork/FunctionalAliasList

| <x>/OnNetwork/FunctionalAliasList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias configuration. |  |  |  |  |

### 10.2.97B1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>

Table 10.2.97B1.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>

| <x>/OnNetwork/FunctionalAliasList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more functional alias configuration elements. |  |  |  |  |

### 10.2.97B2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry

Table 10.2.97B2.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the functional alias. |  |  |  |  |

### 10.2.97B3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FunctionalAlias

Table 10.2.97B3.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FunctionalAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a functional alias that can be activated by the MCData user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.97B3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/DisplayName

Table 10.2.97B3A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/DisplayName

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to a functional alias that can be activated by the MCData user. |  |  |  |  |

### 10.2.97B3B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation

Table 10.2.97B3B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains the location criteria for activation of a functional alias. |  |  |  |  |

### 10.2.97B3B0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>

Table 10.2.97B3B0.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node contains the zero or more location criteria for activation of a functional alias. |  |  |  |  |

### 10.2.97B3B1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea

Table 10.2.97B3B1.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers the functional alias activation. |  |  |  |  |

### 10.2.97B3B2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea

Table 10.2.97B3B2.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 10.2.97B3B3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>

Table 10.2.97B3B3.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 10.2.97B3B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 10.2.97B3B4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3B5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 10.2.97B3B5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3B6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 10.2.97B3B6.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3B7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea

Table 10.2.97B3B7.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 10.2.97B3B8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center

Table 10.2.97B3B8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3B9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 10.2.97B3B9.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3B10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 10.2.97B3B8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3B11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 10.2.97B3B11.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3B12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Radius

Table 10.2.97B3B12.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 10.2.97B3B13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 10.2.97B3B13.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 10.2.97B3B14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 10.2.97B3B14.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 10.2.97B3B15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed

Table 10.2.97B3B15.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 10.2.97B3B16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/
MinimumSpeed

Table 10.2.97B3B16.1:	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.97B3B17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/
MaximumSpeed

Table 10.2.97B3B17.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.97B3B18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading

Table 10.2.97B3B18.1:	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 10.2.97B3B19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/
MinimumHeading

Table 10.2.97B3B19.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.97B3B20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/
MaximumHeading

Table 10.2.97B3B20.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.97B3B21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea

Table 10.2.97B3B21.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when exited by the MC service UE triggers the functional alias activation. |  |  |  |  |

### 10.2.97B3B22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/
PolygonArea

Table 10.2.97B3B22.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 10.2.97B3B23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
PolygonArea/<x>

Table 10.2.97B3B23.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 10.2.97B3B24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 10.2.97B3B24.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 10.2.97B3B25	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 10.2.97B3B25.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3B26	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 10.2.97B3B26.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3B27	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea

Table 10.2.97B3B27.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 10.2.97B3B28	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center

Table 10.2.97B3B28.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3B29	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 10.2.97B3B29.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3B30	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 10.2.97B3B30.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3B31	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 10.2.97B3B31.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3B32	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Radius

Table 10.2.97B3B32.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 10.2.97B3B33	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 10.2.97B3B33.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 10.2.97B3B34	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 10.2.97B3B34.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 10.2.97B3B35	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed

Table 10.2.97B3B35.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 10.2.97B3B36	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/
MinimumSpeed

Table 10.2.97B3B36.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.97B3B37	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/
MaximumSpeed

Table 10.2.97B3B37.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.97B3B38	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading

Table 10.2.97B3B38.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 10.2.97B3B39	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/
MinimumHeading

Table 10.2.97B3B39.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.97B3B40	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/
MaximumHeading

Table 10.2.97B3B40.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MaximumumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.97B3C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation

Table 10.2.97B3C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains the location criteria for de-activation of a functional alias. |  |  |  |  |

### 10.2.97B3C0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>

Table 10.2.97B3C0.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node contains the zero or more location criteria for de-activation of a functional alias. |  |  |  |  |

### 10.2.97B3C1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea

Table 10.2.97B3C1.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers the functional alias de-activation. |  |  |  |  |

### 10.2.97B3C2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea

Table 10.2.97B3C2.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 10.2.97B3C3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>

Table 10.2.97B3C3.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 10.2.97B3C4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 10.2.97B3C4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3C5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 10.2.97B3C5.1: /</<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3C6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 10.2.97B3C6.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3C7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea

Table 10.2.97B3C7.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 10.2.97B3C8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center

Table 10.2.97B3C8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3C9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 10.2.97B3C9.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3C10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 10.2.97B3C10.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3C11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 10.2.97B3C11.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3C12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Radius

Table 10.2.97B3C12.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 10.2.97B3C13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 10.2.97B3C13.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 10.2.97B3C14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 10.2.97B3C14.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 10.2.97B3C15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed

Table 10.2.97B3C15.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 10.2.97B3C16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/
MinimumSpeed

Table 10.2.97B3C16.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.97B3C17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/
MaximumSpeed

Table 10.2.97B3C17.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.97B3C18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading

Table 10.2.97B3C18.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 10.2.97B3C19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/
MinimumHeading

Table 10.2.97B3C19.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.97B3C20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/
MaximumHeading

Table 10.2.97B3C20.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.97B3C21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea

Table 10.2.97B3C21.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when exited by the MC service UE triggers the functional alias de-activation. |  |  |  |  |

### 10.2.97B3C22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea

Table 10.2.97B3C22.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 10.2.97B3C23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>

Table 10.2.97B3C23.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 10.2.97B3C24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 10.2.97B3C24.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3C25	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 10.2.97B3C25.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3C26	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 10.2.97B3C26.1:	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3C27	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea

Table 10.2.97B3C27.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 10.2.97B3C28	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center

Table 10.2.97B3C28.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3C29	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 10.2.97B3C29.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 10.2.97B3C30	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 10.2.97B3C30.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3C31	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 10.2.97B3C31.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 10.2.97B3C32	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Radius

Table 10.2.97B3C32.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 10.2.97B3C33	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 10.2.97B3C33.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 10.2.97B3C34	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 10.2.97B3C34.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 10.2.97B3C35	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed

Table 10.2.97B3C35.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 10.2.97B3C36	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/
MinimumSpeed

Table 10.2.97B3C36.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.97B3C37	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/
MaximumSpeed

Table 10.2.97B3C37.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 10.2.97B3C38	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading

Table 10.2.97B3C38.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 10.2.97B3C39	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/
MinimumHeading

Table 10.2.97B3C39.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.97B3C40	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/
MaximumHeading

Table 10.2.97B3C40.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 10.2.97B3D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
ManualDeactivationNotAllowedIfLocationCriteriaMet

Table 10.2.97B3D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
ManualDeactivationNotAllowedIfLocationCriteriaMet

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/ManualDeactivationNotAllowedIfLocationCriteriaMet |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to de-activate a functional alias if the location criteria are met. |  |  |  |  |

### 10.2.97B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
MaxSimultaneousEmergencyGroupCalls

Table 10.2.97B4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/MaxSimultaneousEmergencyGroupCalls

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/MaxSimultaneousEmergencyGroupCalls |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous emergency group calls for a functional alias. |  |  |  |  |

### 10.2.97C	/<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser

Table 10.2.97C.1: /<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser

| <x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised query the functional alias(es) activated by another MCData user. |  |  |  |  |

When set to "true" the MCData user is authorised to query the functional alias(es) activated by another MCData user.

When set to "false" the MCData user is not authorised to query the functional alias(es) activated by another MCData user.

### 10.2.97D	/<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser

Table 10.2.97D.1: /<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser

| <x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to take over the functional alias(es) previously activated by another MCData user. |  |  |  |  |

When set to "true" the MCData user is authorised to take over the functional alias(es) previously activated by another MCData user.

When set to "false" the MCData user is not authorised to take over the functional alias(es) previously activated by another MCData user.

### 10.2.97E	/<x>/<x>/OnNetwork/MessageStoreHostname

Table 10.2.97E.1: /<x>/<x>/OnNetwork/MessageStoreHostname

| <x>/OnNetwork/MessageStoreHostname |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates value of the hostname identifying the message store function. |  |  |  |  |

The value is a string.

### 10.2.97F	/<x>/<x>/OnNetwork/AuthorisedIncomingAny

Table 10.2.97F.1: /<x>/<x>/OnNetwork/AuthorisedIncomingAny

| <x>/OnNetwork/AuthorisedIncomingAny |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive MCData communication from any MCData user. |  |  |  |  |

When set to "true" the MCData user is authorised to receive MCData communication from any MCData user.

When set to "false" the MCData user is not authorised to receive an MCData communication from any MCData user, but only from the MCData users contained in the IncomingUserList.

### 10.2.97G	/<x>/<x>/OnNetwork/IncomingUserList

Table 10.2.97G.1: /<x>/<x>/OnNetwork/IncomingUserList

| <x>/OnNetwork/IncomingUserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCData user(s) who are authorised to initiate one-to-one MCData communication to the configured MCData user. |  |  |  |  |

### 10.2.97G1	/<x>/<x>/OnNetwork/IncomingUserList/<x>

Table 10.2.97G1.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>

| <x>/OnNetwork/IncomingUserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCData users who are authorised to initiate one-to-one MCData communication to the configured MCData user. |  |  |  |  |

### 10.2.97G2	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry

Table 10.2.97G2.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry

| <x>/OnNetwork/IncomingUserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCData users who are authorised to initiate one-to-one MCData communication to the configured MCData user. |  |  |  |  |

### 10.2.97G3	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCDataID

Table 10.2.97G3.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCDataID

| <x>/OnNetwork/IncomingUserList/<x>/Entry/MCDataID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.97G4	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/
MCDataIDKMSURI

Table 10.2.97G4.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCDataIDKMSURI

| <x>/OnNetwork/IncomingUserList/<x>/Entry/MCDataIDKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS associated with the MCDataID of an MCData user who is authorised to initiate a one-to-one communication to the configured MCData user. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 10.2.97H	/<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding

Table 10.2.97H.1: /<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding

| <x>/OnNetwork/AllowedFunctionalAliasGroupBinding |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to request the binding of a particular functional alias with a group or list of groups. |  |  |  |  |

When set to "true" the MCData user is authorised to request the binding of a particular functional alias with a group or list of groups.

When set to "false" the MCData user is not authorised to request the binding of a particular functional alias with a group or list of groups.

### 10.2.97I	/<x>/<x>/OnNetwork/AllowStoreCommsInMsgstore

Table 10.2.97I.1: /<x>/<x>/OnNetwork/AllowStoreCommsInMsgstore

| <x>/OnNetwork/AllowStoreCommsInMsgstore |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData communications of an MCData user are authorised to be stored in the message store or not. |  |  |  |  |

When set to "true", the MCData communications of a MCData user are authorised to be stored in the message store.

When set to "false", the MCData communications of a MCData user are not authorised to be stored in the message store.

### 10.2.97J	/<x>/<x>/OnNetwork/AllowStorePrivateCommsInMsgstore

Table 10.2.97J.1: /<x>/<x>/OnNetwork/AllowStorePrivateCommsInMsgstore

| <x>/OnNetwork/AllowStorePrivateCommsInMsgstore |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCData user is authorised to request an MCData server to store his/her private communications into message store or not. |  |  |  |  |

When set to "true", the MCData user is authorised to request an MCData server to store his/her private communications into message store.

When set to "false", the MCData user is not authorised to request an MCData server to store his/her private communications into message store.

### 10.2.97K	/<x>/<x>/OnNetwork/MCDataAdhocGroup

Table 10.2.97K.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup

| <x>/OnNetwork/MCDataAdhocGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData adhoc group data communication and alert related configurations. |  |  |  |  |

### 10.2.97K1	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert

Table 10.2.97K1.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert

| <x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData adhoc group emergency alert policy. |  |  |  |  |

### 10.2.97K2	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/
Authorised

Table 10.2.97K2.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/Authorised

| <x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to activate an MCData adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCData user is authorised to make an MCData adhoc group emergency alert.

When set to "false" the MCData user is not authorised to make an MCData adhoc group emergency alert.

### 10.2.97K3	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/
Cancel

Table 10.2.97K3.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/Cancel

| <x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/Cancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel an MCData adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCData user is authorised to cancel an MCData adhoc group emergency alert.

When set to "false" the MCData user is not authorised to cancel an MCData adhoc group emergency alert.

### 10.2.97K4	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/
AuthRecvParticipantInfo

Table 10.2.97K4.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/AuthRecvParticipantInfo

| <x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/AuthRecvParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive MCData adhoc group emergency alert participants information. |  |  |  |  |

When set to "true" the MCData user is authorised to receive adhoc group emergency alert participants information.

When set to "false" the MCData user is not authorised to receive adhoc group emergency alert participants information.

### 10.2.97K5	/<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/
AuthSetupAdhocGroupComn

Table 10.2.97K5.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/AuthSetupAdhocGroupComn

| <x>/OnNetwork/MCDataAdhocGroup/EmergencyAlert/AuthSetupAdhocGroupComn |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to set up an adhoc group communication using the adhoc group used for the adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCData user is authorised to set up an adhoc group communication using the adhoc group used for the adhoc group emergency alert.

When set to "false" the MCData user is not authorised to set up an adhoc group communication using the adhoc group used for the adhoc group emergency alert.

### 10.2.97K6	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn

Table 10.2.97K6.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn

| <x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData adhoc group communication related policy. |  |  |  |  |

### 10.2.97K7	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/
Authorised

Table 10.2.97K7.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/Authorised

| <x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCData adhoc group communication. |  |  |  |  |

When set to "true" the MCData user is authorised to make an MCData adhoc group communication.

When set to "false" the MCData user is not authorised to make an MCData adhoc group communication.

### 10.2.97K8	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/
AuthorisedParticipation

Table 10.2.97K8.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthorisedParticipation

| <x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthorisedParticipation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to participate in MCData adhoc group communication. |  |  |  |  |

When set to "true" the MCData user is authorised to participate in MCData adhoc group communications that they are invited.

When set to "false" the MCData user is not authorised to participate in MCData adhoc group communications that they are invited.

### 10.2.97K9	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/
AuthInitEmergencyComn

Table 10.2.97K9.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthInitEmergencyComn

| <x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthInitEmergencyComn |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCData adhoc group emergency communication. |  |  |  |  |

When set to "true" the MCData user is authorised to make an MCData adhoc group emergency communication.

When set to "false" the MCData user is not authorised to make an MCData adhoc group emergency communication.

### 10.2.97K10	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/
AuthInitImminentPerilComn

Table 10.2.97K10.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthInitImminentPerilComn

| <x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthInitImminentPerilComn |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCData adhoc group imminent-peril communication. |  |  |  |  |

When set to "true" the MCData user is authorised to make an MCData adhoc group imminent-peril communication.

When set to "false" the MCData user is not authorised to make an MCData adhoc group imminent-peril communication.

### 10.2.97K11	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/
AuthRecvComnParticipantInfo

Table 10.2.97K11.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthRecvComnParticipantInfo

| <x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthRecvComnParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive adhoc group communication participants information. |  |  |  |  |

When set to "true" the MCData user is authorised to receive adhoc group communication participants information.

When set to "false" the MCData user is not authorised to receive adhoc group communication participants information.

### 10.2.97K12	/<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/
AuthModifyComnParticipantInfo

Table 10.2.97K12.1: /<x>/<x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthModifyComnParticipantInfo

| <x>/OnNetwork/MCDataAdhocGroup/AdhocGroupComn/AuthModifyComnParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to modify adhoc group communication participants information. |  |  |  |  |

When set to "true" the MCData user is authorised to modify adhoc group communication participants information.

When set to "false" the MCData user is not authorised to modify adhoc group communication participants information.

### 10.2.98	/<x>/<x>/OffNetwork

Table 10.2.98.1: /<x>/<x>/OffNetwork

| <x>/OffNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for off-network operation. |  |  |  |  |

### 10.2.99	/<x>/<x>/OffNetwork/Authorised

Table 10.2.99.1: /<x>/<x>/OffNetwork/Authorised

| <x>/OffNetwork/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation for off-network services. |  |  |  |  |

When set to "true" the MCData user is authorised for off-network.

When set to "false" the MCData user is not authorised for off-network operation.

### 10.2.100	/<x>/<x>/OffNetwork/MCDataGroupList

Table 10.2.100.1: /<x>/<x>/OffNetwork/MCDataGroupList

| <x>/OffNetwork/MCDataGroupList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the list of off-network MCData groups that the MCData user is allowed to use. |  |  |  |  |

### 10.2.101	/<x>/<x>/OffNetwork/MCDataGroupList/<x>

Table 10.2.101.1: /<x>/<x>/OffNetwork/MCDataGroupList/<x>

| <x>/OffNetwork/MCDataGroupList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of off-network MCData groups that the MCData user is allowed to use. |  |  |  |  |

### 10.2.102	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry

Table 10.2.102.1: /<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry

| <x>/OffNetwork/MCDataGroupList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the off-network MCData groups that the MCData user is allowed to use. |  |  |  |  |

### 10.2.103	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/MCDataGroupID

Table 10.2.103.1: /<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/MCDataGroupID

| <x>/OffNetwork/MCDataGroupList/<x>/Entry/MCDataGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCData group ID for the off-network MCData group that the MCData user is allowed to use. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 10.2.104	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/DisplayName

Table 10.2.104.1: /<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/DisplayName

| <x>/OffNetwork/MCDataGroupList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCData Group ID. |  |  |  |  |

### 10.2.105	Void

### 10.2.106	Void

### 10.2.107	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/GMSServID

Table 10.2.107.1: /<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/GMSServID

| <x>/OffNetwork/MCDataGroupList/<x>/Entry/GMSServID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the group management server hosting the MCData Group ID. |  |  |  |  |

### 10.2.108	Void

### 10.2.109	Void

### 10.2.110	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/
IdMSTokenEndPoint

Table 10.2.110.1: /<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/IdMSTokenEndPoint

| <x>/OffNetwork/MCDataGroupList/<x>/Entry/IdMSTokenEndPoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the IDMS token endpoint for the MCData Group ID in the MCDataGroupList. If the value is empty, the IDMS identities (IDMSAuthEndpoint and IDMSTokenEndpoint) present in the MCS UE initial configuration MO are used. |  |  |  |  |

### 10.2.110A	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/
GroupKMSURI

Table 10.2.110A.1: /<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/GroupKMSURI

| <x>/OffNetwork/MCDataGroupList/<x>/Entry/GroupKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates key management server URI for the security domain of the MCData group identity (MCData Group ID) of the off-network MCData group |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 10.2.111	/<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/
RelativePresentationPriority

Table 10.2.111.1: /<x>/<x>/OffNetwork/MCDataGroupList/<x>/Entry/RelativePresentationPriority

| <x>/OffNetwork/MCDataGroupList/<x>/Entry/RelativePresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates indicating the presentation priority of the off-network group for the MCData user relative to other off-network groups and off-network users. |  |  |  |  |

-	Values: 0-255

The lowest RelativePresentationPriority value shall be considered as the MCData group transaction having the lowest priority for presentation among other group MCData and one-to-one user transactions.

### 10.2.112	/<x>/<x>/OffNetwork/UserInfoID

Table 10.2.112.1: /<x>/<x>/OffNetwork/UserInfoID

| <x>/OffNetwork/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 10.2.113	Void

### 10.2.114	Void

### 10.2.115	Void

### 10.2.116	Void

### 10.2.117	Void

### 10.2.118	Void

### 10.2.119	Void

### 10.2.120	Void

### 10.2.121	/<x>/Status

Table 10.2.121.1: /<x>/Status

| Status |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether this MCData user profile is enabled or disabled. |  |  |  |  |

When set to "true" this MCData user profile is enabled.

When set to "false" this MCData user profile is disabled.

# 11	MCData service configuration MO

## 11.1	General

The MCData service configuration Management Object (MO) is used to configure MCData Client behaviour for the on-network or off-network MCData Service. The MCData service configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCData-service-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCData service configuration MO.

The following nodes and leaf objects are possible under the MCData service configuration node as described in figure 11.1.1.

Figure 11.1.1: The MCData service configuration MO

## 11.2	MCData service configuration MO parameters

### 11.2.1	General

This clause describes the parameters for the MCData service configuration Management Object (MO).

### 11.2.2	Node: <x>

Table 11.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCData service configuration Management Object (MO). For the MCData service configuration MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcdata-service configuration:1.0" |  |  |  |  |

-	Values: N/A

### 11.2.3	/<x>/Name

Table 11.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCData service configuration settings. |  |  |  |  |

-	Values: <User displayable name>

### 11.2.4	/<x>/Ext/

Table 11.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCData service configuration MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 11.2.5	/<x>/Common

Table 11.2.5.1: /<x>/Common

| Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 11.2.6	/<x>/Common/TxRxControl

Table 11.2.6.1: /<x>/Common/TxRxControl

| Common/TxRxControl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for transmission and reception control configuration. |  |  |  |  |

### 11.2.7	/<x>/Common/TxRxControl/TimeTempDataWaiting

Table 11.2.7.1: /<x>/Common/TxRxControl/TimeTempDataWaiting

| Common/TxRxControl/TimeTempDataWaiting |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the time limit for the temporarily stored data that is waiting to be delivered to a receiving user. |  |  |  |  |

-	Values: 0-65535

The TimeTempDataWaiting is in minutes.

### 11.2.8	/<x>/Common/TxRxControl/TimePeriodicAnnouncement

Table 11.2.8.1: /<x>/Common/TxRxControl/TimePeriodicAnnouncement

| Common/TxRxControl/TimePeriodicAnnouncement |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the timer for the periodic announcement which contains a list of available recently invited data group communications. |  |  |  |  |

-	Values: 0-65535

The TimePeriodicAnnouncement is in minutes.

### 11.2.9	/<x>/OffNetwork

Table 11.2.9.1: /<x>/OffNetwork

| OffNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for off-network operation. |  |  |  |  |

### 11.2.10	/<x>/OffNetwork/DefaultPPPP

Table 11.2.10.1: /<x>/OffNetwork/DefaultPPPP

| OffNetwork/DefaultPPPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default ProSe Per-Packet Priority (PPPP) configuration. |  |  |  |  |

### 11.2.11	/<x>/OffNetwork/DefaultPPPP/MCDataOneToOneCallSignalling

Table 11.2.11.1: /<x>/OffNetwork/DefaultPPPP/MCDataOneToOneCallSignalling

| OffNetwork/DefaultPPPP/MCDataOneToOneCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCData one-to-one call signalling. |  |  |  |  |

-	Values: 1-8

The MCData user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCData user data having the highest priority among the MCData user data.

### 11.2.12	/<x>/OffNetwork/DefaultPPPP/MCDataOneToOneCallMedia

Table 11.2.12.1: /<x>/OffNetwork/DefaultPPPP/MCDataOneToOneCallMedia

| OffNetwork/DefaultPPPP/MCDataOneToOneCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCData private call media. |  |  |  |  |

-	Values: 1-8

The MCData user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCData user data having the highest priority among the MCData user data.

### 11.2.12A		/<x>/OffNetwork/DefaultPQI

Table 11.2.12A.1: /<x>/OffNetwork/DefaultPQI

| OffNetwork/DefaultPQI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default PQI configuration. |  |  |  |  |

### 11.2.12B	/<x>/OffNetwork/DefaultPQI/MCDataOneToOneCallSignalling

Table 11.2.12B.1: /<x>/OffNetwork/DefaultPQI/MCDataOneToOneCallSignalling

| OffNetwork/DefaultPQI/MCDataOneToOneCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCData one-to-one call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCData user data with the lowest PQI value shall be considered as the MCData user data having the highest priority among the MCData user data.

### 11.2.12C	/<x>/OffNetwork/DefaultPQI/MCDataOneToOneCallMedia

Table 11.2.12C.1: /<x>/OffNetwork/DefaultPQI/MCDataOneToOneCallMedia

| OffNetwork/DefaultPQI/MCDataOneToOneCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCData private call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCData user data with the lowest PQI value shall be considered as the MCData user data having the highest priority among the MCData user data.

### 11.2.13	/<x>/OnNetwork

Table 11.2.13.1: /<x>/OnNetwork

| OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 11.2.14	/<x>/OnNetwork/NotificationServers

Table 11.2.14.1: /<x>/OnNetwork/NotificationServers

| <x>/OnNetwork/NotificationServers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of hostnames of the Notification servers. |  |  |  |  |

### 11.2.15	/<x>/OnNetwork/NotificationServers/<x>

Table 11.2.15.1: /<x>/OnNetwork/NotificationServers/<x>

| <x>/OnNetwork/NotificationServers/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more hostnames of the Notification servers. |  |  |  |  |

### 11.2.16	/<x>/OnNetwork/NotificationServers/<x>/NotificationServer

Table 11.2.16.1: /<x>/OnNetwork/NotificationServers/<x>/NotificationServer

| <x>/OnNetwork/NotificationServers/<x>/NotificationServer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a hostname of the Notification server. |  |  |  |  |

### 11.2.17	/<x>/OnNetwork/MCDataAdhocGroup

Table 11.2.17.1: /<x>/OnNetwork/MCDataAdhocGroup

| OnNetwork/MCDataAdhocGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData adhoc group communication related configurations. |  |  |  |  |

### 11.2.18	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn

Table 11.2.18.1: /<x>/OnNetwork/MCDataAdhocGroup/GroupComn

| OnNetwork/MCDataAdhocGroup/GroupComn |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCData adhoc group communication policy and configurations. |  |  |  |  |

### 11.2.19	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn/
AllowAdhocGroupComn

Table 11.2.19.1: /<x>/OnNetwork/MCDataAdhocGroup/GroupComn/AllowAdhocGroupComn

| OnNetwork/MCDataAdhocGroup/GroupComn/AllowAdhocGroupComn |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether on-network adhoc group communications support enabled or disabled. |  |  |  |  |

When set to "true" the MCData system supports the adhoc group communication and is enabled to allow the MCData users to make an MCData adhoc group communication.

When set to "false" the MCData system supports the adhoc group communication and is disabled to not allow the MCData users to make an MCData adhoc group communication.

### 11.2.20	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn/
MaxNoParticipants

Table 11.2.20.1: /<x>/OnNetwork/MCDataAdhocGroup/GroupComn/MaxNoParticipants

| OnNetwork/MCDataAdhocGroup/GroupComn/MaxNoParticipants |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates maximum number of participants allowed in an adhoc group communication and the value is left to deplyoment requirement. |  |  |  |  |

### 11.2.21	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn/HangTime

Table 11.2.21.1: /<x>/OnNetwork/MCDataAdhocGroup/GroupComn/HangTime

| OnNetwork/MCDataAdhocGroup/GroupComn/HangTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates hang timer for adhoc group communications. |  |  |  |  |

-	Values: 0-65535

The HangTime is in seconds.

### 11.2.22	/<x>/OnNetwork/MCDataAdhocGroup/GroupComn/
MaxDurationOfComn

Table 11.2.22.1: /<x>/OnNetwork/MCDataAdhocGroup/GroupComn/MaxDurationOfComn

| OnNetwork/MCDataAdhocGroup/GroupComn/MaxDurationOfComn |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Status | Occurrence | Format | Min. Access Types |  |  |
|  |  | Required | One | int | Get, Replace |  |  |
|  |  | This leaf node indicates max adhoc group communication duration. |  |  |  |  |  |

-	Values: 0-65535

The MaxDurationOfComn time is in seconds.

# 12	MCVideo UE configuration MO

## 12.1	General

The MCVideo UE configuration Management Object (MO) is used to configure MCVideo Client behaviour for the on-network or off-network MCVideo Service. The MCVideo UE configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCVideo-UE-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCVideo UE configuration MO.

The following nodes and leaf objects are possible under the MCVideo UE configuration node as described in figure 12.1.1.

Figure 12.1.1: The MCVideo UE configuration MO

## 12.2	MCVideo UE configuration MO parameters

### 12.2.1	General

This clause describes the parameters for the MCVideo UE configuration Management Object (MO).

### 12.2.2	Node: <x>

Table 12.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCVideo UE configuration Management Object (MO). For the MCVideo UE configuration MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcvideo-ue-configuration:1.0" |  |  |  |  |

-	Values: N/A

### 12.2.3	/<x>/Name

Table 12.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCVideo UE configuration settings. |  |  |  |  |

-	Values: <User displayable name>

### 12.2.4	/<x>/Ext/

Table 12.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCVideo UE configuration MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 12.2.5	/<x>/Common

Table 12.2.5.1: /<x>/Common

| Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 12.2.5A	/<x>/Common/PrivateCall

Table 12.2.5A.1: /<x>/Common/PrivateCall

| Common/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the private call configuration. |  |  |  |  |

### 12.2.5B	/<x>/Common/PrivateCall/MaxCallNc10

Table 12.2.5B.1: /<x>/Common/PrivateCall/MaxCallNc10

| Common/PrivateCall/MaxCallNc10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of private calls. |  |  |  |  |

-	Values: 0-255

### 12.2.6	/<x>/Common/MCVideoGroupCall

Table 12.2.6.1: /<x>/Common/MCVideoGroupCall

| Common/MCVideoGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo group call configuration. |  |  |  |  |

### 12.2.7	/<x>/Common/MCVideoGroupCall/MaxCallNc4

Table 12.2.7.1: /<x>/Common/MCVideoGroupCall/MaxCallNc4

| Common/MCVideoGroupCall/MaxCallNc4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous MCVideo group calls. |  |  |  |  |

-	Values: 0-255

### 12.2.8	/<x>/Common/MCVideoGroupCall/MaxTransmissionNc5

Table 12.2.8.1: /<x>/Common/MCVideoGroupCall/MaxTransmissionNc5

| Common/MCVideoGroupCall/MaxTransmissionNc5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of video transmissions in a group. |  |  |  |  |

-	Values: 0-255

### 12.2.9	/<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup

Table 12.2.9.1: /<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup

| Common/MCVideoGroupCall/PrioritizedMCVideoGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the prioritized MCVideo group call configuration. |  |  |  |  |

### 12.2.10	/<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>

Table 12.2.10.1: /<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>

| Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more prioritized MCVideo group call configuration. |  |  |  |  |

### 12.2.11	/<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>/
MCVideoGroupID

Table 12.2.11.1: /<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>/MCVideoGroupID

| Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>/MCVideoGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCVideo group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 12.2.12	/<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>/
MCVideoGroupPriorityHierarchy

Table 12.2.12.1: /<x>/Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>/
MCVideoGroupPriorityHierarchy

| Common/MCVideoGroupCall/PrioritizedMCVideoGroup/<x>/MCVideoGroupPriorityHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the requested presentation priority of MCVideo group call. |  |  |  |  |

-	Values: 0-255

The group with the lowest MCVideoGroupPriorityHierarchy value shall be considered as the group having the lowest priority among the groups.

### 12.2.13	/<x>/OnNetwork

Table 12.2.13.1: /<x>/OnNetwork

| OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 12.2.14	/<x>/OnNetwork/RelayService

Table 12.2.14.1: /<x>/OnNetwork/RelayService

| OnNetwork/RelayService |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to use a relay service. |  |  |  |  |

When set to "true" the MCVideo group is allowed to use a relay service.

When set to "false" the MCVideo group is not allowed to use a relay service.

### 12.2.15	/<x>/OnNetwork/IPv6Preferred

Table 12.2.15.1: /<x>/OnNetwork/IPv6Preferred

| OnNetwork/IPv6Preferred |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether IPv6 is preferred over IPv4 for on-network operation when the MCVideo UE has both IPv4 and IPv6 host configuration. |  |  |  |  |

If the MCVideo UE has both IPv4 and IPv6 host configuration and:

-	if IPv6Preferred is set to "true" then the UE uses IPv6 for all on-network signalling and media; otherwise

-	if IPv6Preferred is set to "false" then the UE uses IPv4 for all on-network signalling and media.

### 12.2.16	/<x>/OnNetwork/RelayedMCVideoGroup

Table 12.2.16.1: /<x>/OnNetwork/RelayedMCVideoGroup

| OnNetwork/RelayedMCVideoGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the allowed relayed MCVideo groups. |  |  |  |  |

### 12.2.17	/<x>/OnNetwork/RelayedMCVideoGroup/<x>

Table 12.2.17.1: /<x>/OnNetwork/RelayedMCVideoGroup/<x>

| OnNetwork/RelayedMCVideoGroup/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more allowed relayed MCVideo groups. |  |  |  |  |

### 12.2.18	/<x>/OnNetwork/RelayedMCVideoGroup/<x>/MCVideoGroupID

Table 12.2.18.1: /<x>/OnNetwork/RelayedMCVideoGroup/<x>/MCVideoGroupID

| OnNetwork/RelayedMCVideoGroup/<x>/MCVideoGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the associated MCVideo group ID. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 12.2.19	/<x>/OnNetwork/RelayedMCVideoGroup/<x>/RelayServiceCode

Table 12.2.19.1: /<x>/OnNetwork/RelayedMCVideoGroup/<x>/RelayServiceCode

| OnNetwork/RelayedMCVideoGroup/<x>/RelayServiceCode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

# 13	MCVideo user profile MO

## 13.1	General

The MCVideo user profile configuration Management Object (MO) is used to configure the MCVideo Client behaviour for the on-network or off-network MCVideo Service. The MCVideo user profile configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCVideo-user-profile:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCVideo user profile MO.

The following nodes and leaf objects are possible under the MCVideo user profile node as described in figure 13.1.1, figure 13.1.2 and figure 13.1.3.

Figure 13.1.1: The MCVideo user profile MO (1 of 3)

Figure 13.1.2: The MCVideo user profile MO (2 of 3)

Figure 13.1.3: The MCVideo user profile MO (3 of 3)

## 13.2	MCVideo user profile MO parameters

### 13.2.1	General

This clause describes the parameters for the MCVideo user profile Management Object (MO).

### 13.2.2	Node: <x>

Table 13.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCVideo user profile Management Object (MO). For the MCVideo user profile MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcvideo-user-profile:1.0" |  |  |  |  |

-	Values: N/A

### 13.2.3	/<x>/Name

Table 13.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCVideo user profile settings. |  |  |  |  |

-	Values: <User displayable name>

### 13.2.4	/<x>/Ext/

Table 13.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCVideo user profile MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 13.2.5	/<x>/<x>

Table 13.2.5.1: /<x>/<x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation and off-network operation. |  |  |  |  |

### 13.2.6	/<x>/<x>/Common

Table 13.2.6.1: /<x>/<x>/Common

| <x>/Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 13.2.7	/<x>/<x>/Common/MCVideoUserID

Table 13.2.7.1: /<x>/<x>/Common/MCVideoUserID

| <x>/Common/MCVideoUserID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. |  |  |  |  |

### 13.2.8	/<x>/<x>/Common/MCVideoUserProfileIndex

Table 13.2.8.1: /<x>/<x>/Common/MCVideoUserProfileIndex

| <x>/Common/MCVideoUserProfileIndex |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates an index for the particular MCVideo user profile. |  |  |  |  |

-	Values: 0-255

### 13.2.9	/<x>/<x>/Common/MCVideoUserProfileName

Table 13.2.9.1: /<x>/<x>/Common/MCVideoUserProfileName

| <x>/Common/MCVideoUserProfileName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates a profile name for the particular MCVideo user profile. |  |  |  |  |

### 13.2.10	/<x>/<x>/Common/PreSelectedIndication

Table 13.2.10.1: /<x>/<x>/Common/PreSelectedIndication

| <x>/Common/PreSelectedIndication |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | null | Get, Replace |  |
|  | This leaf node indicates that this MCVideo user profile MO is designated to be the pre-selected MCVideo user profile as defined by 3GPP TS 23.179 [15] |  |  |  |  |

-	Values: null

When this leaf node is present, this MCVideo user profile MO is designated as the pre-selected MCVideo user profile.

When this leaf node is absent, this MCVideo user profile MO is not designated as the pre-selected MCVideo user profile.

If more than one MCVideo user profile MO is specified for the MCVideo user, then only one MCVideo user profile MO for the MCVideo user shall contain the <x>/Common/PreSelectedIndication leaf node.

If there is only one MCVideo user profile MO specified for the MCVideo user, then it is optional to include the <x>/Common/PreSelectedIndication leaf node.

### 13.2.11	/<x>/<x>/Common/UserAliases

Table 13.2.11.1: /<x>/<x>/Common/UserAliases

| <x>/Common/UserAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of aliases of an MCVideo user. |  |  |  |  |

### 13.2.12	/<x>/<x>/Common/UserAliases/<x>

Table 13.2.12.1: /<x>/<x>/Common/UserAliases/<x>

| <x>/Common/UserAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more aliases of an MCVideo user. |  |  |  |  |

### 13.2.13	/<x>/<x>/Common/UserAliases/<x>/UserAlias

Table 13.2.13.1: /<x>/<x>/Common/UserAliases/<x>/UserAlias

| <x>/Common/UserAliases/<x>/UserAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an alphanumeric alias of the MCVideo user. |  |  |  |  |

### 13.2.14	Void

### 13.2.15	/<x>/<x>/Common/ParticipantType

Table 13.2.15.1: /<x>/<x>/Common/ParticipantType

| <x>/Common/ParticipantType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the participant type of the MCVideo user. |  |  |  |  |

The ParticipantType means the functional category of the participant (e.g., first responder, second responder, dispatch, dispatch supervisor), typically defined by the MCVideo administrators.

### 13.2.16	/<x>/<x>/Common/Organization

Table 13.2.16.1: /<x>/<x>/Common/Organization

| <x>/Common/Organization |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the organization an MCVideo user belongs to. |  |  |  |  |

### 13.2.17	/<x>/<x>/Common/MCVideoGroupBroadcast

Table 13.2.17.1: /<x>/<x>/Common/MCVideoGroupBroadcast

| <x>/Common/MCVideoGroupBroadcast |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group-broadcast group policy. |  |  |  |  |

### 13.2.18	/<x>/<x>/Common/MCVideoGroupBroadcast/Authorised

Table 13.2.18.1: /<x>/<x>/Common/MCVideoGroupBroadcast/Authorised

| <x>/Common/MCVideoGroupBroadcast/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to create a group-broadcast group. |  |  |  |  |

When set to "true" the MCVideo user is authorised to create a group-broadcast group.

When set to "false" the MCVideo user is not authorised to create a group-broadcast group.

### 13.2.19	/<x>/<x>/Common/UserBroadcast

Table 13.2.19.1: /<x>/<x>/Common/UserBroadcast

| <x>/Common/UserBroadcast |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the user-broadcast group policy. |  |  |  |  |

### 13.2.20	/<x>/<x>/Common/UserBroadcast/Authorised

Table 13.2.20.1: /<x>/<x>/Common/UserBroadcast/Authorised

| <x>/Common/UserBroadcast/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to create a user-broadcast group. |  |  |  |  |

When set to "true" the MCVideo user is authorised to create a user-broadcast group.

When set to "false" the MCVideo user is not authorised to create a user-broadcast group.

### 13.2.21	Void

### 13.2.22	Void

### 13.2.23	Void

### 13.2.24	Void

### 13.2.25	Void

### 13.2.26	Void

### 13.2.27	Void

### 13.2.28	Void

### 13.2.29	/<x>/<x>/Common/AllowedActivateAlert

Table 13.2.29.1: /<x>/<x>/Common/AllowedActivateAlert

| <x>/Common/AllowedActivateAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised to activate an emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to activate an emergency alert.

When set to "false" the MCVideo user is not authorised to activate an emergency alert.

### 13.2.30	/<x>/<x>/Common/AllowedCancelAlert

Table 13.2.30.1: /<x>/<x>/Common/AllowedCancelAlert

| <x>/Common/AllowedCancelAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is allowed to cancel an emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to cancel an emergency alert.

When set to "false" the MCVideo user is not authorised to cancel an emergency alert.

### 13.2.31	Void

### 13.2.32	Void

### 13.2.33	Void

### 13.2.34	Void

### 13.2.35	Void

### 13.2.36	Void

### 13.2.37	Void

### 13.2.38	Void

### 13.2.38A	/<x>/<x>/Common/MCVideoGroupCall

Table 13.2.38A.1: /<x>/<x>/Common/MCVideoGroupCall

| <x>/Common/MCVideoGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo group call configuration. |  |  |  |  |

### 13.2.38A1	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert

Table 13.2.38A1.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert

| <x>/Common/MCVideoGroupCall/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo emergency alert policy. |  |  |  |  |

### 13.2.38A2	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/
Authorised

Table 13.2.38A2.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Authorised

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to activate an MCVideo emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo emergency alert.

When set to "false" the MCVideo user is not authorised to make an MCVideo emergency alert.

### 13.2.38A3	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Cancel

Table 13.2.38A3.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Cancel

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Cancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel an MCVideo emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to cancel an MCVideo emergency alert.

When set to "false" the MCVideo user is not authorised to cancel an MCVideo emergency alert.

### 13.2.38A4	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry

Table 13.2.338A4.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCVideo group of an MCVideo emergency alert. |  |  |  |  |

### 13.2.38A5	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/ID

Table 13.2.38A5.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/ID

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCVideo group used upon certain criteria on initiation of an MCVideo emergency alert. |  |  |  |  |

### 13.2.38A6	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/
Entry/DisplayName

Table 13.2.38A6.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/DisplayName

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the ID. |  |  |  |  |

### 13.2.38A7	/<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/
Entry/Usage

Table 13.2.38A7.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/Usage

| <x>/Common/MCVideoGroupCall/EmergencyAlert/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo emergency alert uses the ID. |  |  |  |  |

The valid values are 'LocallyDetermined', 'UseCurrentlySelectedGroup', 'UsePreConfigured' and 'DedicatedGroup'.

When set to 'LocallyDetermined' then if the MCVideo user selects an MCVideo ID then use that MCVideo ID for an on-network MCVideo emergency alert, if the MCVideo user does not select a MCVideo ID then use the MCVideo ID identified by the ID in clause 13.2.38A5 for an on-network MCVideo emergency alert.

When set to 'UseCurrentlySelectedGroup' then if the MCVideo user has currently selected an MCVideo group then use that MCVideo group for an on-network MCVideo emergency alert, if the MCVideo user does not have a currently selected MCVideo group then use the MCVideo group identified by the ID in clause 13.2.38A5 for an MCVideo emergency alert.

When set to 'UsePreConfigured' then use the ID identified by the ID in clause 13.2.38A5 for an on-network MCVideo emergency alert.

When set to 'DedicatedGroup' then use the MCVideo group identified by the ID in clause 13.2.38A5 for an MCVideo emergency alert.

### 13.2.38B	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall

Table 13.2. 38B.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall

| <x>/Common/MCVideoGroupCall/EmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo emergency call policy. |  |  |  |  |

### 13.2.38C	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/Enabled

Table 13.2.38C.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/Enabled

| <x>/Common/MCVideoGroupCall/EmergencyCall/Enabled |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCVideo emergency group call functionality enabled for MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo emergency group call functionality enabled.

When set to "false" the MCVideo user is not authorised to make an MCVideo emergency group call functionality enabled.

### 13.2.38D	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/
CancelMCVideoGroup

Table 13.2.38D.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/CancelMCVideoGroup

| <x>/Common/MCVideoGroupCall/EmergencyCall/CancelMCVideoGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel an in progress MCVideo emergency group call associated with a group. |  |  |  |  |

When set to "true" the MCVideo user is authorised to cancel a MCVideo emergency group call.

When set to "false" the MCVideo user is not authorised to cancel a MCTT emergency group call.

### 13.2.38D1	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/
MCVideoGroupInitiation

Table 13.2.38D1.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group used on initiation of an MCVideo emergency group call. |  |  |  |  |

### 13.2.38D2	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/
MCVideoGroupInitiation/Entry

Table 13.2.38D2.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the group used on initiation of an MCVideo emergency group call. |  |  |  |  |

### 13.2.38D3	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/
MCVideoGroupInitiation/Entry/GroupID

Table 13.2.38D3.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/GroupID

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/GroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the group used upon certain criteria on initiation of an MCVideo emergency group call. |  |  |  |  |

### 13.2.38D4	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/
MCVideoGroupInitiation/Entry/DisplayName

Table 13.2.38D4.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/DisplayName

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the Group ID. |  |  |  |  |

### 13.2.38D5	/<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/
MCVideoGroupInitiation/Entry/Usage

Table 13.2.38D5.1: /<x>/<x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/Usage

| <x>/Common/MCVideoGroupCall/EmergencyCall/MCVideoGroupInitiation/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo emergency group call uses the GroupID. |  |  |  |  |

The valid values are 'UseCurrentlySelectedGroup' and 'DedicatedGroup'.

When set to 'UseCurrentlySelectedGroup' then if the MCVideo user has currently selected an MCVideo group then use that MCVideo group for an on-network MCVideo emergency group call, if the MCVideo user does not have a currently selected MCVideo group then use the MCVideo group identified by the GroupID in clause 13.2.38D3 for an MCVideo emergency group call.

When set to 'DedicatedGroup' then use the MCVideo group identified by the GroupID in clause 13.2.38D3 for an MCVideo emergency group call.

### 13.2.38E	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall

Table 13.2.38E.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall

| <x>/Common/MCVideoGroupCall/ImminentPerilCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo imminent peril call policy. |  |  |  |  |

### 13.2.38F	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
Authorised

Table 13.2.38F.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/Authorised

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an Imminent Peril group call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to create an MCVideo imminent peril group call.

When set to "false" the MCVideo user is not authorised to create an MCVideo imminent peril group call.

### 13.2.38G	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/Cancel

Table 13.2.38G.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/Cancel

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/Cancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation for in-progress MCVideo imminent peril cancelation. |  |  |  |  |

When set to "true" the MCVideo user is authorised to cancel an MCVideo imminent peril call.

When set to "false" the MCVideo user is not authorised to cancel an MCVideo imminent peril call.

### 13.2.38G1	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
MCVideoGroupInitiation

Table 13.2.38G1.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the group used on initiation of an MCVideo imminent peril group call. |  |  |  |  |

### 13.2.38G2	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
MCVideoGroupInitiation/Entry

Table 13.2.38G2.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the group used on initiation of an imminent peril call. |  |  |  |  |

### 13.2.38G3	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
MCVideoGroupInitiation/Entry/GroupID

Table 13.2.38G3.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/GroupID

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/GroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the group used upon certain criteria on initiation of an MCVideo imminent peril group call. |  |  |  |  |

### 13.2.38G4	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
MCVideoGroupInitiation/DisplayName

Table 13.2.38G4.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/DisplayName

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the Group ID. |  |  |  |  |

### 13.2.38G5	/<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/
MCVideoGroupInitiation/Entry/Usage

Table 13.2.38G5.1: /<x>/<x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/Usage

| <x>/Common/MCVideoGroupCall/ImminentPerilCall/MCVideoGroupInitiation/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo imminent peril group call uses the GroupID. |  |  |  |  |

The valid values are 'UseCurrentlySelectedGroup' and 'DedicatedGroup'.

When set to 'UseCurrentlySelectedGroup' then if the MCVideo user has currently selected an MCVideo group then use that MCVideo group for an on-network MCVideo imminent peril group call, if the MCVideo user does not have a currently selected MCVideo group then use the MCVideo group identified by the GroupID in clause 13.2.38G3 for an MCVideo imminent peril group call.

When set to 'DedicatedGroup' then use the MCVideo group identified by the GroupID in clause 13.2.38G3 for an MCVideo imminent peril group call.

### 13.2.38G6	/<x>/<x>/Common/MCVideoGroupCall/Priority

Table 13.2.38G6.1: /<x>/<x>/Common/MCVideoGroupCall/Priority

| <x>/Common/MCVideoGroupCall/Priority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the priority of the MCVideo group calls. |  |  |  |  |

-	Values: 0-255

The MCVideo group call with the lowest Priority value shall be considered as the MCVideo group call having the lowest level among the MCVideo group calls.

### 13.2.38G7	/<x>/<x>/Common/MCVideoGroupCall/MaxSimultaneousCallsN6

Table 13.2.38G7.1: /<x>/<x>/Common/MCVideoGroupCall/MaxSimultaneousCallsN6

| <x>/Common/MCVideoGroupCall/MaxSimultaneousCallsN6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneously received MCVideo group calls (N6). |  |  |  |  |

### 13.2.38H	/<x>/<x>/Common/PrivateCall

Table 13.2.38H.1: /<x>/<x>/Common/PrivateCall

| <x>/Common/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo private call configuration. |  |  |  |  |

### 13.2.38H1	Void

### 13.2.38H2	Void

### 13.2.38H3	Void

### 13.2.38H4	Void

### 13.2.38H5	Void

### 13.2.38H6	Void

### 13.2.38I	/<x>/<x>/Common/PrivateCall/Authorised

Table 13.2.38I.1: /<x>/<x>/Common/PrivateCall/Authorised

| <x>/Common/PrivateCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCVideo private call. |  |  |  |  |

### 13.2.38I1	/<x>/<x>/Common/PrivateCall/AuthorisedAny

Table 13.2.38I1.1: /<x>/<x>/Common/PrivateCall/AuthorisedAny

| <x>/Common/PrivateCall/AuthorisedAny |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCVideo private call to any MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo private call to any MCVideo user.

When set to "false" the MCVideo user is not authorised to make an MCVideo private call to any MCVideo user.

### 13.2.38I2	/<x>/<x>/Common/PrivateCall/UserList

Table 13.2.38I2.1: /<x>/<x>/Common/PrivateCall/UserList

| <x>/Common/PrivateCall/UserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCVideo user(s) who can be called in a MCVideo private call. |  |  |  |  |

### 13.2.38I3	/<x>/<x>/Common/PrivateCall/UserList/<x>

Table 13.2.38I3.1: /<x>/<x>/Common/PrivateCall/UserList/<x>

| <x>/Common/PrivateCall/UserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCVideo users who can be called in a MCVideo private call. |  |  |  |  |

### 13.2.38I4	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry

Table 13.2.38I4.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry

| <x>/Common/PrivateCall/UserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCVideo users who can be called in a private call. |  |  |  |  |

### 13.2.38I5	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID

Table 13.2.38I5.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID

| <x>/Common/PrivateCall/UserList/<x>/Entry/MCVideoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.38I6	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/
DiscoveryGroupID

Table 13.2.38I6.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DiscoveryGroupID

| <x>/Common/PrivateCall/UserList/<x>/Entry/DiscoveryGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a discovery group ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The value is used as the discovery group ID in the ProSe discovery procedures as specified in 3GPP TS 23.303 [6].

### 13.2.38I6A	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/
ApplicationLayerGroupID

Table 13.2.38I6A.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/ApplicationLayerGroupID

| <x>/Common/PrivateCall/UserList/<x>/Entry/ApplicationLayerGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | Int | Get, Replace |  |
|  | This leaf node indicates an application layer group ID as specified in 3GPP TS 23.304 [25]. |  |  |  |  |

The value is used as the application layer group ID in the ProSe discovery procedures as specified in 3GPP TS 23.304 [25].

### 13.2.38I7	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID

Table 13.2.38I7.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID

| <x>/Common/PrivateCall/UserList/<x>/Entry/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 13.2.38I8	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName

Table 13.2.38I8.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName

| <x>/Common/PrivateCall/UserList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name. |  |  |  |  |

### 13.2.38I9	/<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/
PrivateCallKMSURI

Table 13.2.38I9.1: /<x>/<x>/Common/PrivateCall/UserList/<x>/Entry/PrivateCallKMSURI

| <x>/Common/PrivateCall/UserList/<x>/Entry/PrivateCallKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS associated with the MCVideoID. If the value is empty, the KMS leaf node present in the MCS UE initial configuration MO is used. |  |  |  |  |

### 13.2.38J	/<x>/<x>/Common/PrivateCall/ManualCommence

Table 13.2.38J.1: /<x>/<x>/Common/PrivateCall/ManualCommence

| <x>/Common/PrivateCall/ManualCommence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCVideo private call with manual commencement. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make a MCVideo private call in manual commencement mode.

When set to "false" the MCVideo user is not authorised to make a MCVideo private call in manual commencement mode.

### 13.2.38K	/<x>/<x>/Common/PrivateCall/AutoCommence

Table 13.2.38K.1: /<x>/<x>/Common/PrivateCall/AutoCommence

| <x>/Common/PrivateCall/AutoCommence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make a MCVideo private call with automatic commencement. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make a MCVideo private call in automatic commencement mode.

When set to "false" the MCVideo user is not authorised to make a MCVideo private call in automatic commencement mode.

### 13.2.38L	/<x>/<x>/Common/PrivateCall/FailRestrict

Table 13.2.38L.1: /<x>/<x>/Common/PrivateCall/FailRestrict

| <x>/Common/PrivateCall/FailRestrict |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to restrict the provision of a notification of call failure reason for a MCVideo private call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to restrict notification of call failure reason for MCVideo private call.

When set to "false" the MCVideo user is not authorised to restrict notification of call failure reason for MCVideo private call.

### 13.2.38M	Void

### 13.2.38N	/<x>/<x>/Common/PrivateCall/AllowedMediaProtection

Table 13.2.38N.1: /<x>/<x>/Common/PrivateCall/AllowedMediaProtection

| <x>/Common/PrivateCall/AllowedMediaProtection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates authorisation to protect confidentiality and integrity of media for MCVideo private calls. |  |  |  |  |

When set to "true" the MCVideo user is authorised to protect confidentiality and integrity of media for MCVideo private calls.

When set to "false" the MCVideo user is not authorised to protect confidentiality and integrity of media for MCVideo private calls.

The default value is set to "true".

### 13.2.38O	/<x>/<x>/Common/PrivateCall/EmergencyCall

Table 13.2.38O.1: /<x>/<x>/Common/PrivateCall/EmergencyCall

| <x>/Common/PrivateCall/EmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo emergency call policy. |  |  |  |  |

### 13.2.38P	/<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised

Table 13.2.38P.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised

| <x>/Common/PrivateCall/EmergencyCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCVideo emergency private call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo emergency private call.

When set to "false" the MCVideo user is not authorised to make an MCVideo emergency private call.

### 13.2.38Q	/<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority

Table 13.2.38Q.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority

| <x>/Common/PrivateCall/EmergencyCall/CancelPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel emergency priority in an MCVideo emergency private call by an authorised MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to cancel an emergency priority in an MCVideo private call.

When set to "false" the MCVideo user is not authorised to cancel an emergency priority in an MPCTT private call.

### 13.2.38R	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient

Table 13.2.38R.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo private recipient for an MCVideo emergency private call. |  |  |  |  |

### 13.2.38S	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient/Entry

Table 13.2.38S.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCVideo private recipient for an MCVideo emergency private call. |  |  |  |  |

### 13.2.38T	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient/Entry/ID

Table 13.2.38T.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/ID

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCVideo private recipient used upon certain criteria on initiation of an MCVideo emergency private call. |  |  |  |  |

### 13.2.38U	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient/Entry/DiscoveryGroupID

Table 13.2.38U.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DiscoveryGroupID

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DiscoveryGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the discovery group ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 13.2.38U1	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient/Entry/ApplicationLayerGroupID

Table 13.2.38U1.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/ApplicationLayerGroupID

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/ApplicationLayerGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the discovery group ID as specified in 3GPP TS 23.304 [25]. |  |  |  |  |

### 13.2.38V	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient/Entry/UserInfoID

Table 13.2.38V.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/UserInfoID

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates a ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

The "UserInfoID" element identifies the MCVideo private recipient when making an off-network private call.

### 13.2.38W	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient/Entry/DisplayName

Table 13.2.38W.1: /<x>/<x>/Common/PrivatepCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DisplayName

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCVideo private recipient ID. |  |  |  |  |

### 13.2.38X	/<x>/<x>/Common/PrivateCall/EmergencyCall/
MCVideoPrivateRecipient/Entry/Usage

Table 13.2.38X.1: /<x>/<x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/Usage

| <x>/Common/PrivateCall/EmergencyCall/MCVideoPrivateRecipient/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo emergency private call uses the MCVideo private recipient ID. |  |  |  |  |

The valid values are 'LocallyDetermined' and 'UsePreConfigured'.

When set to 'LocallyDetermined' then if the MCVideo user selects an MCVideo ID then use that MCVideo ID for the MCVideo emergency private call, if the MCVideo user does not select a MCVideo ID then use the MCVideo ID identified by the MCVideo private recipient ID in clause 13.2.38T for an on-network MCVideo emergency private call.

When set to 'UsePreConfigured' then use the MCVideo ID identified by the MCVideo private recipient ID in clause 13.2.38T for an on-network MCVideo emergency private call.

When set to 'LocallyDetermined' then if the MCVideo user selects an MCVideo user then use the UserInfoID that corresponds to that MCVideo user for the MCVideo emergency private call, if the MCVideo user does not select a MCVideo user then use the User Info ID identified by the UserInfoID in clause 13.2.38T for an off-network MCVideo emergency private call.

When set to 'UsePreConfigured' then use the User Info ID identified by the UserInfoID in clause 13.2.38T for an off-network MCVideo emergency private call.

### 13.2.38Y	/<x>/<x>/Common/PrivateCall/AutoAnswer

Table 13.2.38Y.1: /<x>/<x>/Common/PrivateCall/AutoAnswer

| <x>/Common/PrivateCall/AutoAnswer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation of MCVideo user to force automatic answer for a MCVideo private call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to force automatic answer for a MCVideo private call.

When set to "false" the MCVideo user is not authorised to force automatic answer for a MCVideo private call.

### 13.2.39	/<x>/<x>/OnNetwork

Table 13.2.39.1: /<x>/<x>/OnNetwork

| <x>/OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 13.2.40	/<x>/<x>/OnNetwork/MCVideoGroupList

Table 13.2.40.1: /<x>/<x>/OnNetwork/MCVideoGroupList

| <x>/OnNetwork/MCVideoGroupList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the list of on-network MCVideo groups that the MCVideo user is allowed to use. |  |  |  |  |

### 13.2.41	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>

Table 13.2.41.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>

| <x>/OnNetwork/MCVideoGroupList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of on-network MCVideo groups that the MCVideo user is allowed to use. |  |  |  |  |

### 13.2.42	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry

Table 13.2.42.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCVideo groups that the MCVideo user is allowed to use. |  |  |  |  |

### 13.2.43	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
MCVideoGroupID

Table 13.2.43.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/MCVideoGroupID

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/MCVideoGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCVideo group ID for the on-network MCVideo group that the MCVideo user is allowed to use. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.43A	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation

Table 13.2.43A.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the rules that control automatic affiliation. |  |  |  |  |

### 13.2.43A0	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>

Table 13.2.43A0.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more rules that control automatic affiliation. |  |  |  |  |

### 13.2.43A1	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria

Table 13.2.43A1.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 13.2.43A2	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>

Table 5.2.48B84A2.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 13.2.43A3	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry

Table 13.2.43A3.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic affiliation. |  |  |  |  |

### 13.2.43A4	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea

Table 13.2.43A4.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 13.2.43A5	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea

Table 13.2.43A5.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 13.2.43A6	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>

Table 13.2.43A6.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 13.2.43A7	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 13.2.43A7.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 13.2.43A8	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 13.2.43A8.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 13.2.43A9	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 13.2.43A9.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 13.2.43A10	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea

Table 13.2.43A10.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 13.2.43A11	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center

Table 13.2.43A11.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.43A12	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 13.2.43A12.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.43A13	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 13.2.43A13.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 13.2.43A14	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 13.2.43A14.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 13.2.43A15	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Radius

Table 13.2.43A15.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 13.2.43A16	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/OffsetAngle

Table 13.2.43A16.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 13.2.43A17	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/IncludedAngle

Table 13.2.43A17.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 13.2.43A18	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed

Table 13.2.43A18.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 13.2.43A19	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MinimumSpeed

Table 13.2.43A19.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.43A20	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MaximumSpeed

Table 13.2.43A20.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.43A21	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading

Table 13.2.43A21.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 13.2.43A22	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MinimumHeading

Table 13.2.43A22.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.43A23	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MaximumHeading

Table 13.2.43A23.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.43A24	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea

Table 13.2.43A24.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 13.2.43A25	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea

Table 13.2.43A25.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 13.2.43A26	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>

Table 13.2.43A26.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 13.2.43A27	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 13.2.43A27.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 13.2.43A28	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 13.2.43A28.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 13.2.43A29	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 13.2.43A29.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 13.2.43A30	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea

Table 13.2.43A30.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 13.2.43A31	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center

Table 13.2.43A31.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.43A32	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 13.2.43A32.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.43A33	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 13.2.43A33.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 13.2.43A34	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 13.2.43A34.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 13.2.43A35	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Radius

Table 13.2.43A35.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 13.2.43A36	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/OffsetAngle

Table 13.2.43A36.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 13.2.43A37	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/IncludedAngle

Table 13.2.43A37.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 13.2.43A38	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed

Table 13.2.43A38.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 13.2.43A39	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MinimumSpeed

Table 13.2.43A39.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.43A40	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MaximumSpeed

Table 13.2.43A40.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.43A41	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading

Table 13.2.43A41.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 13.2.43A42	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MinimumHeading

Table 13.2.43A42.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.43A43	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MaximumHeading

Table 13.2.43A43.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.43A44	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases

Table 13.2.43A44.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 13.2.43A45	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>

Table 13.2.43A45.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 13.2.43A46	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry

Table 13.2.43A46.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic affiliation. |  |  |  |  |

### 13.2.43A47	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/
FunctionalAlias

Table 13.2.43A47.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForAffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node contains a functional alias. |  |  |  |  |

### 13.2.43B	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation

Table 13.2.43B.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node is a placeholder for the rules that control automatic deaffiliation. |  |  |  |  |

### 13.2.43B0	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>

Table 13.2.43B0.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the rules that control automatic deaffiliation. |  |  |  |  |

### 13.2.43B1	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria

Table 13.2.43B1.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 13.2.43B2	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>

Table 13.2.43B2.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 13.2.43B3	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry

Table 13.2.43B3.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the location portion of the rules that control automatic deaffiliation. |  |  |  |  |

### 13.2.43B4	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea

Table 13.2.43B4.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which, when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 13.2.43B5	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea

Table 13.2.43B5.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 13.2.43B6	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>

Table 13.2.43B6.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 13.2.43B7	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 13.2.43B7.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesFordeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 13.2.43B8	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 13.2.43B8.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 13.2.43B9	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 13.2.43B9.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 13.2.43B10	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea

Table 13.2.43B10.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 13.2.43B11	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center

Table 13.2.43B11.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.43B12	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 13.2.43B12.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.43B13	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 13.2.43B13.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 13.2.43B14	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 13.2.43B14.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 13.2.43B15	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/Radius

Table 13.2.43B15.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 13.2.43B16	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/OffsetAngle

Table 13.2.43B16.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 13.2.43B17	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/EllipsoidArcArea/IncludedAngle

Table 13.2.43B17.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 13.2.43B18	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed

Table 13.2.43B18.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 13.2.43B19	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MinimumSpeed

Table 13.2.43B19.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.43B20	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Speed/MaximumSpeed

Table 13.2.43B20.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.43B21	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading

Table 13.2.43B21.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 13.2.43B22	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MinimumHeading

Table 13.2.43B22.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.43B23	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
EnterSpecificArea/Heading/MaximumHeading

Table 13.2.43B23.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.43B24	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea

Table 13.2.43B24.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers evaluation of the rules. |  |  |  |  |

### 13.2.43B25	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea

Table 13.2.43B25.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 13.2.43B26	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>

Table 13.2.43B26.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 13.2.43B27	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

Table 13.2.43B27.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 13.2.43B28	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

Table 13.2.43B28.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

### 13.2.43B29	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

Table 13.2.43B29.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

### 13.2.43B30	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea

Table 13.2.43B30.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 13.2.43B31	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center

Table 13.2.4331.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.43B32	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 13.2.43B32.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.43B33	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

Table 13.2.43B33.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

### 13.2.43B34	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

Table 13.2.43B34.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a center. |  |  |  |  |

### 13.2.43B35	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/Radius

Table 13.2.43B35.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

### 13.2.43B36	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/OffsetAngle

Table 13.2.43B36.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

### 13.2.43B37	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/EllipsoidArcArea/IncludedAngle

Table 13.2.43B37.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

### 13.2.43B38	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed

Table 13.2.43B38.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 13.2.43B39	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MinimumSpeed

Table 13.2.43B39.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.43B40	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Speed/MaximumSpeed

Table 13.2.43B40.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.43B41	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading

Table 13.2.43B41.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 13.2.43B42	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MinimumHeading

Table 13.2.43B42.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.43B43	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/
ExitSpecificArea/Heading/MaximumHeading

Table 13.2.43B43.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.43B44	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases

Table 13.2.43B44.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 13.2.43B45	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>

Table 13.2.43B45.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 13.2.43B46	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry

Table 13.2.43B46.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias part of rules that control automatic deaffiliation. |  |  |  |  |

### 13.2.43B47	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/
FunctionalAlias

Table 13.2.43B47.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/
ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RulesForDeaffiliation/<x>/ListOfActiveFunctionalAliases/<x>/Entry/FunctionalAlias |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node contains a functional alias. |  |  |  |  |

### 13.2.43C	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet

Table 13.2.43C.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/ManualDeaffiliationNotAllowedIfAffiliationRulesAreMet |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised to deaffiliate if the affiliation criteria are met. |  |  |  |  |

### 13.2.44	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/DisplayName

Table 13.2.44.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/DisplayName

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCVideo Group ID. |  |  |  |  |

### 13.2.45	Void

### 13.2.46	Void

### 13.2.47	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/GMSServID

Table 13.2.47.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/GMSServID

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/GMSServID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the group management server hosting the MCVideo Group ID. |  |  |  |  |

### 13.2.48	Void

### 13.2.49	Void

### 13.2.50	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
IdMSTokenEndPoint

Table 13.2.50.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/IdMSTokenEndPoint

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/IdMSTokenEndPoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the identity management server hosting the MCVideo Group ID. |  |  |  |  |

### 13.2.50A	Void

### 13.2.50B	Void

### 13.2.50C	Void

### 13.2.50D	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
GroupKMSURI

Table 13.2.50D.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/GroupKMSURI

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/GroupKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS identity (URI) for a specific group contained in the MCVideoGroupList. If the value is empty, the KMS identity (URI) (kms) present in the MCS UE initial configuration MO is used. |  |  |  |  |

### 13.2.51	/<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/
RelativePresentationPriority

Table 13.2.51.1: /<x>/<x>/OnNetwork/MCVideoGroupList/<x>/Entry/RelativePresentationPriority

| <x>/OnNetwork/MCVideoGroupList/<x>/Entry/RelativePresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates indicating the presentation priority of the on-network group for the MCVideo user relative to other on-network groups and on-network users. |  |  |  |  |

-	Values: 0-255

The lowest RelativePresentationPriority value shall be considered as the MCVideo group transaction having the lowest priority for presentation among other group MCVideo and one-to-one user transactions.

### 13.2.52	/<x>/<x>/OnNetwork/ImplicitAffiliations

Table 13.2.52.1: /<x>/<x>/OnNetwork/ImplicitAffiliations

| <x>/OnNetwork/ImplicitAffiliations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the implicit affiliation configuration. |  |  |  |  |

### 13.2.53	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>

Table 13.2.53.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>

| <x>/OnNetwork/ImplicitAffiliations/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more implicit affiliation configuration. |  |  |  |  |

### 13.2.54	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry

Table 13.2.54.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCVideo groups that the MCVideo user is implicitly affiliated to. |  |  |  |  |

### 13.2.55	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCVideoGroupID

Table 13.2.55.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCVideoGroupID

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry/MCVideoGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a MCVideo group ID to which the MCVideo user is implicitly affiliated to. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.56	/<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName

Table 13.2.56.1: /<x>/<x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName

| <x>/OnNetwork/ImplicitAffiliations/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCVideo Group ID. |  |  |  |  |

### 13.2.57	/<x>/<x>/OnNetwork/PresenceStatus

Table 13.2.57.1: /<x>/<x>/OnNetwork/PresenceStatus

| <x>/OnNetwork/PresenceStatus |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the presence status configuration. |  |  |  |  |

### 13.2.58	/<x>/<x>/OnNetwork/PresenceStatus/<x>

Table 13.2.58.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>

| <x>/OnNetwork/PresenceStatus/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for zero or more presence status configuration. |  |  |  |  |

### 13.2.59	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry

Table 13.2.59.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry

| <x>/OnNetwork/PresenceStatus/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the MCVideo IDs of MCVideo users that the configured MCVideo user is authorised to obtain presence status. |  |  |  |  |

### 13.2.60	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/MCVideoGroupID

Table 13.2.60.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/MCVideoGroupID

| <x>/OnNetwork/PresenceStatus/<x>/Entry/MCVideoGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCVideo ID of an MCVideo user that the configured MCVideo user is authorised to obtain presence status. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.61	/<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/DisplayName

Table 13.2.61.1: /<x>/<x>/OnNetwork/PresenceStatus/<x>/Entry/DisplayName

| <x>/OnNetwork/PresenceStatus/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCVideo ID. |  |  |  |  |

### 13.2.62	Void

### 13.2.63	Void

### 13.2.64	Void

### 13.2.65	Void

### 13.2.66	Void

### 13.2.67	/<x>/<x>/OnNetwork/MaxAffiliationsN2

Table 13.2.67.1: /<x>/<x>/OnNetwork/MaxAffiliationsN2

| <x>/OnNetwork/MaxAffiliationsN2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of MCVideo groups that the MCVideo user is authorised to affiliate with. |  |  |  |  |

-	Values: 0-65535

### 13.2.68	/<x>/<x>/OnNetwork/AllowedRegroup

Table 13.2.68.1: /<x>/<x>/OnNetwork/AllowedRegroup

| <x>/OnNetwork/AllowedRegroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised to perform dynamic regrouping operations. |  |  |  |  |

When set to "true" the MCVideo user is authorised to perform dynamic regrouping operations.

When set to "false" the MCVideo user is not authorised to perform dynamic regrouping operations.

### 13.2.69	/<x>/<x>/OnNetwork/AllowedPresenceStatus

Table 13.2.69.1: /<x>/<x>/OnNetwork/AllowedPresenceStatus

| <x>/OnNetwork/AllowedPresenceStatus |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the presence status on the network of this MCVideo user is available. |  |  |  |  |

When set to "true" the presence status on the network of this MCVideo user is available.

When set to "false" the presence status on the network of this MCVideo user is not available. This is the default if this leaf node is not present.

### 13.2.70	/<x>/<x>/OnNetwork/AllowedPresence

Table 13.2.70.1: /<x>/<x>/OnNetwork/AllowedPresence

| <x>/OnNetwork/AllowedPresence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised to obtain whether a particular MCVideo User is present on the network. |  |  |  |  |

When set to "true" the MCVideo user is authorised to obtain whether a particular MCVideo User is present on the network.

When set to "false" the MCVideo user is not authorised to obtain whether a particular MCVideo User is present on the network. This is the default if this leaf node is not present.

### 13.2.71	/<x>/<x>/OnNetwork/AllowedManualSwitch

Table 13.2.71.1: /<x>/<x>/OnNetwork/AllowedManualSwitch

| <x>/OnNetwork/AllowedManualSwitch |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised to manually switch to off-network operation while in on-network operation. |  |  |  |  |

When set to "true" the MCVideo user is authorised to manually switch to off-network operation while in on-network operation.

When set to "false" the MCVideo user is not authorised to manually switch to off-network operation while in on-network operation.

### 13.2.72	Void

### 13.2.73	/<x>/<x>/OnNetwork/MaxSimultaneousVideoStreams

Table 13.2.73.1: /<x>/<x>/OnNetwork/MaxSimultaneousVideoStreams

| <x>/OnNetwork/MaxSimultaneousVideoStreams |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for maximum number of simultaneous video streams that can be received. |  |  |  |  |

### 13.2.74	/<x>/<x>/OnNetwork/MaxSimultaneousVideoStreams/MaxStreams

Table 13.2.74.1: /<x>/<x>/OnNetwork/MaxSimultaneousVideoStreams/MaxStreams

| <x>/OnNetwork/MaxSimultaneousVideoStreams/MaxStreams |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous video streams that can be received. |  |  |  |  |

-	Values: 1-65535

### 13.2.75	Void

### 13.2.76	Void

### 13.2.77	Void

### 13.2.78	Void

### 13.2.79	Void

### 13.2.80	Void

### 13.2.81	Void

### 13.2.82	Void

### 13.2.83	Void

### 13.2.84	Void

### 13.2.85	Void

### 13.2.86	Void

### 13.2.87	Void

### 13.2.87A	/<x>/<x>/OnNetwork/EnabledParticipation

Table 13.2.87A.1: /<x>/<x>/OnNetwork/EnabledParticipation

| <x>/OnNetwork/EnabledParticipation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is allowed to participate in MCVideo private calls that they are invited to. |  |  |  |  |

When set to "true" the MCVideo user is allowed to participate in MCVideo private calls that they are invited to.

When set to "false" the MCVideo user is not allowed to participate in MCVideo private calls that they are invited to.

### 13.2.87A1	/<x>/<x>/OnNetwork/AllowedRemoteInitiatedAmbientViewing

Table 13.2.87A1.1: /<x>/<x>/AllowedRemoteInitiatedAmbientViewing

| <x>/OnNetwork/AllowedRemoteInitiatedAmbientViewing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the presence status on the network of this MCVideo user is available. |  |  |  |  |

When set to "true" the MCVideo user is authorised to remotely initiate ambient viewing at another MCVideo user's device.

When set to "false" the MCVideo user is not authorised to remotely initiate ambient viewing at another MCVideo user's device.

### 13.2.87A2	/<x>/<x>/OnNetwork/AllowedLocallyInitiatedAmbientViewing

Table 13.2.87A2.1: /<x>/<x>/OnNetwork/AllowedLocallyInitiatedAmbientViewing

| <x>/OnNetwork/AllowedLocallyInitiatedAmbientViewing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates the presence status on the network of this MCVideo user is available. |  |  |  |  |

When set to "true" the MCVideo user is authorised to locally initiate ambient viewing.

When set to "false" the MCVideo user is not authorised to locally initiate ambient viewing.

### 13.2.87A3	/<x>/<x>/OnNetwork/FunctionalAliasList

Table 13.2.87A3.1: /<x>/<x>/OnNetwork/FunctionalAliasList

| <x>/OnNetwork/FunctionalAliasList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias configuration. |  |  |  |  |

### 13.2.87A4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>

Table 13.2.87A4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>

| <x>/OnNetwork/FunctionalAliasList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more functional alias configuration elements. |  |  |  |  |

### 13.2.87A5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry

Table 13.2.87A5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the functional alias. |  |  |  |  |

### 13.2.87A6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FunctionalAlias

Table 13.2.87A6.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/MCVIDEOID

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FunctionalAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a functional alias that can be activated by the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.87A6A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation

Table 13.2.87A6A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains the location criteria for activation of a functional alias. |  |  |  |  |

### 13.2.87A6A0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>

Table 13.2.87A6A0.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node contains the zero or more location criteria for activation of a functional alias. |  |  |  |  |

### 13.2.87A6A1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea

Table 13.2.87A6A1.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers the functional alias activation. |  |  |  |  |

### 13.2.87A6A2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea

Table 13.2.87A6A2.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 13.2.87A6A3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>

Table 13.2.87A6A3.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 13.2.87A6A3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 13.2.87A6A3A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6A4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 13.2.87A6A4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6A5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 13.2.87A6A5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6A6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea

Table 13.2.87A6A6.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 13.2.87A6A7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center

Table 13.2.87A6A7.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6A7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 13.2.87A6A7A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6A8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 13.2.87A6A8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6A9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 13.2.87A6A9.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6A10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Radius

Table 13.2.87A6A10.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 13.2.87A6A11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 13.2.87A6A11.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 13.2.87A6A12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 13.2.87A6A12.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 13.2.87A6A12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed

Table 13.2.87A6A12A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 13.2.87A6A12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/
MinimumSpeed

Table 13.2.87A6A12B.1:	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.87A6A12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/
MaximumSpeed

Table 13.2.87A6A12C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.87A6A12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading

Table 13.2.87A6A12D.1:	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 13.2.87A6A12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/
MinimumHeading

Table 13.2.87A6A12E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.87A6A12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/
MaximumHeading

Table 13.2.87A6A12F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/EnterSpecificArea/Heading/MaximmHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.87A6A13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea

Table 13.2.87A6A13.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when exited by the MC service UE triggers the functional alias activation. |  |  |  |  |

### 13.2.87A6A14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea

Table 13.2.87A6A14.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 13.2.87A6A15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/
PolygonArea/<x>

Table 13.2.87A6A15.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 13.2.87A6A15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/
<x>/PointCoordinateType

Table 13.2.87A6A15A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 13.2.87A6A16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/
<x>/PointCoordinateType/Longitude

Table 13.2.87A6A16.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6A17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/
<x>/PointCoordinateType/Latitude

Table 13.2.87A6A17.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6A18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea

Table 13.2.87A6A18.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 13.2.87A6A19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center

Table 13.2.87A6A19.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6A19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 13.2.87A6A19A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6A20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 13.2.87A6A20.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6A21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 13.2.87A6A21.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6A22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Radius

Table 13.2.87A6A22.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 13.2.87A6A23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 13.2.87A6A23.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 13.2.87A6A24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 13.2.87A6A24.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 13.2.87A6A24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed

Table 13.2.87A6A24A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 13.2.87A6A24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/
MinimumSpeed

Table 13.2.87A6A24B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.87A6A24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/
MaximumSpeed

Table 13.2.87A6A24C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.87A6A24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading

Table 13.2.87A6A24D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 13.2.87A6A24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/
MinimumHeading

Table 13.2.87A6A24E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.87A6A24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/
MaximumHeading

Table 13.2.87A6A24F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/<x>/ExitSpecificArea/Heading/MaximumumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.87A6B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation

Table 13.2.87A6B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains the location criteria for de-activation of a functional alias. |  |  |  |  |

### 13.2.87A6B0	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>

Table 13.2.87A6B0.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrMore | node | Get, Replace |  |
|  | This interior node contains the zero or more location criteria for de-activation of a functional alias. |  |  |  |  |

### 13.2.87A6B1	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea

Table 13.2.87A6B1.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when entered by the MC service UE triggers the functional alias de-activation. |  |  |  |  |

### 13.2.87A6B2	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea

Table 13.2.87A6B2.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 13.2.87A6B3	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>

Table 13.2.87A6B3.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 13.2.87A6B3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 13.2.87A6B3A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 13.2.87A6B4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6B5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 13.2.87A6B5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6B6	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea

Table 13.2.87A6B6.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 13.2.87A6B7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center

Table 13.2.87A6B7.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6B7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 13.2.87A6B7A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6B8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 13.2.87A6B8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6B9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 13.2.87A6B9.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6B10	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/Radius

Table 13.2.87A6B10.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 13.2.87A6B11	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 13.2.87A6B11.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 13.2.87A6B12	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 13.2.87A6B12.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 13.2.87A6B12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed

Table 13.2.87A6B12A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 13.2.87A6B12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/
MinimumSpeed

Table 13.2.87A6B12B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.87A6B12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/
MaximumSpeed

Table 13.2.87A6B12C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.87A6B12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading

Table 13.2.87A6B12D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 13.2.87A6B12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/
MinimumHeading

Table 13.2.87A6B12E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.87A6B12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/
MaximumHeading

Table 13.2.87A6B12F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.87A6B13	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea

Table 13.2.87A6B13.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area which when exited by the MC service UE triggers the functional alias de-activation. |  |  |  |  |

### 13.2.87A6B14	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea

Table 13.2.87A6B14.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by a polygon. |  |  |  |  |

### 13.2.87A6B15	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>

Table 13.2.87A6B15.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrN | node | Get, Replace |  |
|  | This interior node contains the coordinates of the corners which define a polygon. The occurrence of this leaf node is "3 to 15" as per 3GPP TS 23.032 [21]. |  |  |  |  |

### 13.2.87A6B15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType

Table 13.2.87A6B15A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6B16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType/Longitude

Table 13.2.87A6B16.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6B17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
PolygonArea/<x>/PointCoordinateType/Latitude

Table 13.2.87A6B17.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/PolygonArea/<x>/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6B18	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea

Table 13.2.87A6B18.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | This interior node contains a geographical area described by an ellipsoid arc. |  |  |  |  |

### 13.2.87A6B19	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center

Table 13.2.87A6B19.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6B19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType

Table 13.2.87A6B19A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 13.2.87A6B20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 13.2.87A6B20.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6B21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 13.2.87A6B21.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/
Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 13.2.87A6B22	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/Radius

Table 13.2.87A6B22.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the ellipsoid arc. |  |  |  |  |

-	Values: non-negative integer

### 13.2.87A6B23	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/OffsetAngle

Table 13.2.87A6B23.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/OffsetAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the offset angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 13.2.87A6B24	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/
EllipsoidArcArea/IncludedAngle

Table 13.2.87A6B24.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/EllipsoidArcArea/IncludedAngle |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the included angle of the ellipsoid arc. |  |  |  |  |

-	Values: 0-255

### 13.2.87A6B24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed

Table 13.2.87A6B24A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 13.2.87A6B24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/
MinimumSpeed

Table 13.2.87A6B24B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.87A6B24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/
MaximumSpeed

Table 13.2.87A6B24C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 13.2.87A6B24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading

Table 13.2.87A6B24D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 13.2.87A6B24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/
MinimumHeading

Table 13.2.87A6B24E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.87A6B24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/
MaximumHeading

Table 13.2.87A6B24F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/<x>/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

### 13.2.87A6C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
ManualDeactivationNotAllowedIfLocationCriteriaMet

Table 13.2.87A6C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
ManualDeactivationNotAllowedIfLocationCriteriaMet

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/ManualDeactivationNotAllowedIfLocationCriteriaMet |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised to de-activate a functional alias if the location criteria is met. |  |  |  |  |

### 13.2.87A7	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/DisplayName

Table 13.2.87A7.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/DisplayName

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to a functional alias that can be activated by the MCVideo user. |  |  |  |  |

### 13.2.87A7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
MaxSimultaneousEmergencyGroupCalls

Table 13.2.87A7A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/MaxSimultaneousEmergencyGroupCalls

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/MaxSimultaneousEmergencyGroupCalls |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates the maximum number of simultaneous emergency group calls for a functional alias. |  |  |  |  |

### 13.2.87A7B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall

Table 13.2.87A7B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the functional alias configuration regarding the allowed functional aliases to be called in a private call. |  |  |  |  |

### 13.2.87A7C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall/<x>

Table 13.2.87A7C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more functional alias configuration elements indicating the functional aliases that can be called. |  |  |  |  |

### 13.2.87A7D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall/<x>/Entry

Table 13.2.87A7D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>/Entry

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the functional alias that can be called. |  |  |  |  |

### 13.2.87A7E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
FAsAllowedToCall/<x>/Entry/FunctionalAlias

Table 13.2.87A7E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>/Entry/FunctionalAlias

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/FAsAllowedToCall/<x>/Entry/FunctionalAlias |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a functional alias that can be called by the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.87A8	/<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser

Table 13.2.87A8.1: /<x>/<x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser

| <x>/OnNetwork/AllowedQueryFunctionalAliasOtherUser |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised query the functional alias(es) activated by another MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to query the functional alias(es) activated by another MCVideo user.

When set to "false" the MCVideo user is not authorised to query the functional alias(es) activated by another MCVideo user.

### 13.2.87A9	/<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser

Table 13.2.87A9.1: /<x>/<x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser

| <x>/OnNetwork/AllowedTakeoverFunctionalAliasOtherUser |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised to take over the functional alias(es) previously activated by another MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to take over the functional alias(es) previously activated by another MCVideo user.

When set to "false" the MCVideo user is not authorised to take over the functional alias(es) previously activated by another MCVideo user.

### 13.2.87A10	/<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding

Table 13.2.87A10.1: /<x>/<x>/OnNetwork/AllowedFunctionalAliasGroupBinding

| <x>/OnNetwork/AllowedFunctionalAliasGroupBinding |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether the MCVideo user is authorised to request the binding of a particular functional alias with a group or list of groups. |  |  |  |  |

When set to "true" the MCVideo user is authorised to request the binding of a particular functional alias with a group or list of groups.

When set to "false" the MCVideo user is not authorised to request the binding of a particular functional alias with a group or list of groups.

### 13.2.87B	/<x>/<x>/OnNetwork/AuthorisedIncomingAny

Table 13.2.87B.1: /<x>/<x>/OnNetwork/AuthorisedIncomingAny

| <x>/OnNetwork/AuthorisedIncomingAny |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive a MCVideo private call from any MCVideo user. |  |  |  |  |

When set to "true" the MCVideo user is authorised to receive an MCVideo private call by any MCVideo user.

When set to "false" the MCVideo user is not authorised to receive an MCVideo private call by any MCVideo user, but only from the MCVideo users contained in the IncomingUserList.

### 13.2.87C	/<x>/<x>/OnNetwork/IncomingUserList

Table 13.2.87C.1: /<x>/<x>/OnNetwork/IncomingUserList

| <x>/OnNetwork/IncomingUserList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for a list of MCVideo user(s) who are authorised to initiate an MCVideo private call to the configured MCVideo user. |  |  |  |  |

### 13.2.87C1	/<x>/<x>/OnNetwork/IncomingUserList/<x>

Table 13.2.87C1.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>

| <x>/OnNetwork/IncomingUserList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of MCVideo users who are authorised to initiate an MCVideo private call to the configured MCVideo user. |  |  |  |  |

### 13.2.87C2	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry

Table 13.2.87C2.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry

| <x>/OnNetwork/IncomingUserList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more MCVideo users who are authorised to initiate an MCVideo private call to the configured MCVideo user. |  |  |  |  |

### 13.2.87C3	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoID

Table 13.2.87C3.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoID

| <x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.87C4	/<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/
MCVideoIDKMSURI

Table 13.2.87C4.1: /<x>/<x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoIDKMSURI

| <x>/OnNetwork/IncomingUserList/<x>/Entry/MCVideoIDKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS associated with the MCVideoID. |  |  |  |  |

The value is a URI as specified in 3GPP TS 23.003 [5].

### 13.2.87D	/<x>/<x>/OnNetwork/PrivateCall

Table 13.2.87D.1: /<x>/<x>/OnNetwork/PrivateCall

| <x>/OnNetwork/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo private call configuration. |  |  |  |  |

### 13.2.87E	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert

Table 13.2.87E.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert

| <x>/OnNetwork/PrivateCall/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo private emergency alert policy. |  |  |  |  |

### 13.2.87F	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry

Table 13.2.87F.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of an MCVideo private emergency alert for on-network. |  |  |  |  |

### 13.2.87G	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID

Table 13.2.87G.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCVideo user ID used upon certain criteria on initiation of an MCVideo private emergency alert for on-network. |  |  |  |  |

### 13.2.87H	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/
DisplayName

Table 13.2.87H.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/DisplayName

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the ID. |  |  |  |  |

### 13.2.87I	/<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage

Table 13.2.87I.1: /<x>/<x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage

| <x>/OnNetwork/PrivateCall/EmergencyAlert/Entry/Usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the criteria to determine when initiation of an MCVideo private emergency alert uses the ID. |  |  |  |  |

The valid values are 'LocallyDetermined' and 'UsePreConfigured'.

When set to 'LocallyDetermined' then if the MCVideo user selects an MCVideo ID then use that MCVideo ID for an on-network MCVideo private emergency alert, if the MCVideo user does not select a MCVideo ID then use the MCVideo ID identified by the ID in clause 13.2.38I12 for an on-network MCVideo private emergency alert.

When set to 'UsePreConfigured' then use the ID identified by the ID in clause 13.2.38I12 for an on-network MCVideo private emergency alert.

### 13.2.87J	/<x>/<x>/OnNetwork/RemoteGroupSelection

Table 13.2.87J.1: /<x>/<x>/OnNetwork/RemoteGroupSelection

| <x>/OnNetwork/RemoteGroupSelection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the remote group selection configuration. |  |  |  |  |

### 13.2.87K	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>

Table 13.2.87K.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>

| <x>/OnNetwork/RemoteGroupSelection/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more remote group selection configuration elements. |  |  |  |  |

### 13.2.87L	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry

Table 13.2.87L.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the on-network MCVideo users whose selected MCVideo group is allowed to be remotely changed by the MCVideo user. |  |  |  |  |

### 13.2.87M	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/
MCVideoID

Table 13.2.87M.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCVideoID

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry/MCVideoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates a MCVideo ID of an MCVideo user whose selected MCVideo group is allowed to be remotely changed by the MCVideo user. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.87N	/<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/
DisplayName

Table 13.2.87N.1: /<x>/<x>/OnNetwork/RemoteGroupSelection/<x>/Entry/DisplayName

| <x>/OnNetwork/RemoteGroupSelection/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCVideo ID of the MCVideo user. |  |  |  |  |

### 13.2.87O1	/<x>/<x>/OnNetwork/MCVideoAdhocGroup

Table 13.2.87O1.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup

| <x>/OnNetwork/MCVideoAdhocGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo adhoc group call and alert related configurations. |  |  |  |  |

### 13.2.87O1A	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert

Table 13.2.87O1A.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert

| <x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo adhoc group emergency alert policy. |  |  |  |  |

### 13.2.87O1A1	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/
Authorised

Table 13.2.87O1A1.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/Authorised

| <x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to activate an MCVideo adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo adhoc group emergency alert.

When set to "false" the MCVideo user is not authorised to make an MCVideo adhoc group emergency alert.

### 13.2.87O1A2	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/
Cancel

Table 13.2.87O1A2.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/Cancel

| <x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/Cancel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to cancel an MCVideo adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to cancel an MCVideo adhoc group emergency alert.

When set to "false" the MCVideo user is not authorised to cancel an MCVideo adhoc group emergency alert.

### 13.2.87O1A3	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/
AuthRecvParticipantInfo

Table 13.2.87O1A3.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/AuthRecvParticipantInfo

| <x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/AuthRecvParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive MCVideo adhoc group emergency alert participants information. |  |  |  |  |

When set to "true" the MCVideo user is authorised to receive adhoc group emergency alert participants information.

When set to "false" the MCVideo user is not authorised to receive adhoc group emergency alert participants information.

### 13.2.87O1A4	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/
AuthSetupAdhocGroupCall

Table 13.2.87O1A4.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/AuthSetupAdhocGroupCall

| <x>/OnNetwork/MCVideoAdhocGroup/EmergencyAlert/AuthSetupAdhocGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to set up an adhoc group call using the adhoc group used for the adhoc group emergency alert. |  |  |  |  |

When set to "true" the MCVideo user is authorised to set up an adhoc group call using the adhoc group used for the adhoc group emergency alert.

When set to "false" the MCVideo user is not authorised to set up an adhoc group call using the adhoc group used for the adhoc group emergency alert.

### 13.2.87O1B	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall

Table 13.2.87O1B.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall

| <x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo adhoc group call related policy. |  |  |  |  |

### 13.2.87O1B1	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/
Authorised

Table 13.2.87O1B1.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/Authorised

| <x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCVideo adhoc group call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo adhoc group call.

When set to "false" the MCVideo user is not authorised to make an MCVideo adhoc group call.

### 13.2.87O1B2	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/
AuthorisedParticipation

Table 13.2.87O1B2.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthorisedParticipation

| <x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthorisedParticipation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to participate in MCVideo adhoc group call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to participate in MCVideo adhoc group calls that they are invited.

When set to "false" the MCVideo user is not authorised to participate in MCVideo adhoc group calls that they are invited.

### 13.2.87O1B3	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/
AuthInitEmergencyCall

Table 13.2.87O1B3.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthInitEmergencyCall

| <x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthInitEmergencyCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCVideo adhoc group emergency call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo adhoc group emergency call.

When set to "false" the MCVideo user is not authorised to make an MCVideo adhoc group emergency call.

### 13.2.87O1B4	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/
AuthInitImminentPerilCall

Table 13.2.87O1B4.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthInitImminentPerilCall

| <x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthInitImminentPerilCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to make an MCVideo adhoc group imminent-peril call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to make an MCVideo adhoc group imminent-peril call.

When set to "false" the MCVideo user is not authorised to make an MCVideo adhoc group imminent-peril call.

### 13.2.87O1B5	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/
AuthRecvCallParticipantInfo

Table 13.2.87O1B5.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthRecvCallParticipantInfo

| <x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthRecvCallParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to receive adhoc group call participants information. |  |  |  |  |

When set to "true" the MCVideo user is authorised to receive adhoc group call participants information.

When set to "false" the MCVideo user is not authorised to receive adhoc group call participants information.

### 13.2.87O1B6	/<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/
AuthModifyCallParticipantInfo

Table 13.2.87O1B6.1: /<x>/<x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthModifyCallParticipantInfo

| <x>/OnNetwork/MCVideoAdhocGroup/AdhocGroupCall/AuthModifyCallParticipantInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation to modify adhoc group call participants information. |  |  |  |  |

When set to "true" the MCVideo user is authorised to modify adhoc group call participants information.

When set to "false" the MCVideo user is not authorised to modify adhoc group call participants information.

### 13.2.88	/<x>/<x>/OffNetwork

Table 13.2.88.1: /<x>/<x>/OffNetwork

| <x>/OffNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for off-network operation. |  |  |  |  |

### 13.2.89	/<x>/<x>/OffNetwork/Authorised

Table 13.2.89.1: /<x>/<x>/OffNetwork/Authorised

| <x>/OffNetwork/Authorised |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorisation for off-network services. |  |  |  |  |

When set to "true" the MCVideo user is authorised for off-network operation.

When set to "false" the MCVideo user is not authorised for off-network operation.

### 13.2.90	/<x>/<x>/OffNetwork/MCVideoGroupList

Table 13.2.90.1: /<x>/<x>/OffNetwork/MCVideoGroupList

| <x>/OffNetwork/MCVideoGroupList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the list of off-network MCVideo groups that the MCVideo user is allowed to use. |  |  |  |  |

### 13.2.91	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>

Table 13.2.91.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>

| <x>/OffNetwork/MCVideoGroupList/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node is a placeholder for one or more list of off-network MCVideo groups that the MCVideo user is allowed to use. |  |  |  |  |

### 13.2.92	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry

Table 13.2.92.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the details of the off-network MCVideo groups that the MCVideo user is allowed to use. |  |  |  |  |

### 13.2.93	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/
MCVideoGroupID

Table 13.2.93.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/MCVideoGroupID

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/MCVideoGroupID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the MCVideo group ID for the off-network MCVideo group that the MCVideo user is allowed to use. |  |  |  |  |

The value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 [4].

### 13.2.94	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/DisplayName

Table 13.2.94.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/DisplayName

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/DisplayName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node contains a human readable name that corresponds to the MCVideo Group ID. |  |  |  |  |

### 13.2.95	Void

### 13.2.96	Void

### 13.2.97	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/GMSServID

Table 13.2.97.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/GMSServID

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/GMSServID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the group management server hosting the MCVideo Group ID. |  |  |  |  |

### 13.2.98	Void

### 13.2.99	Void

### 13.2.100	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/
IdMSTokenEndPoint

Table 13.2.100.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/IdMSTokenEndPoint

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/IdMSTokenEndPoint |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the identity management server hosting the MCVideo Group ID. |  |  |  |  |

### 13.2.100A	Void

### 13.2.100B	Void

### 13.2.100C	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/
GroupKMSURI

Table 13.2.100C.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/GroupKMSURI

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/GroupKMSURI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the identity (URI) of the KMS identity (URI) for a specific group contained in the off-network MCVideoGroupList. If the value is empty, the KMS identity (URI) (kms) present in the MCS UE initial configuration MO is used. |  |  |  |  |

### 13.2.101	/<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/
RelativePresentationPriority

Table 13.2.101.1: /<x>/<x>/OffNetwork/MCVideoGroupList/<x>/Entry/RelativePresentationPriority

| <x>/OffNetwork/MCVideoGroupList/<x>/Entry/RelativePresentationPriority |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | This leaf node indicates indicating the presentation priority of the off-network group for the MCVideo user relative to other off-network groups and off-network users. |  |  |  |  |

-	Values: 0-255

The lowest RelativePresentationPriority value shall be considered as the MCVideo group transaction having the lowest priority for presentation among other group MCVideo and one-to-one user transactions.

### 13.2.102	/<x>/<x>/OffNetwork/UserInfoID

Table 13.2.102.1: /<x>/<x>/OffNetwork/UserInfoID

| <x>/OffNetwork/UserInfoID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This leaf node indicates the ProSe user info ID as specified in 3GPP TS 23.303 [6]. |  |  |  |  |

### 13.2.102A	/<x>/<x>/OffNetwork/EmergencyCallChange

Table 13.2.102A.1: /<x>/<x>/OffNetwork/EmergencyCallChange

| <x>/OffNetwork/EmergencyCallChange |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorization for a participant to change an off-network group call in-progress to an off-network MCVideo emergency group call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to change an MCVideo emergency group call.

When set to "false" the MCVideo user is not authorised to change an MCVideo emergency group call.

### 13.2.102B	/<x>/<x>/OffNetwork/ImminentPerilCallChange

Table 13.2.102B.1: /<x>/<x>/OffNetwork/ImminentPerilCallChange

| <x>/OffNetwork/ImminentPerilCallChange |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates the authorization for a participant to change an off-network group call in-progress to an off-network MCVideo imminent peril group call. |  |  |  |  |

When set to "true" the MCVideo user is authorised to change an MCVideo imminent peril group call.

When set to "false" the MCVideo user is not authorised to change an MCVideo imminent peril group call.

### 13.2.103	/<x>/Status

Table 13.2.103.1: /<x>/Status

| Status |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | bool | Get, Replace |  |
|  | This leaf node indicates whether this MCVideo user profile is enabled or disabled. |  |  |  |  |

When set to "true" this MCVideo user profile is enabled.

When set to "false" this MCVideo user profile is disabled.

# 14	MCVideo service configuration MO

## 14.1	General

The MCVideo service configuration Management Object (MO) is used to configure MCVideo Client behaviour for the on-network or off-network MCVideo Service. The MCVideo service configuration parameters may be stored in the ME, or in the USIM as specified in 3GPP TS 31.102 [10], or in both the ME and the USIM. If both the ME and the USIM contain the same parameters, the values stored in the USIM shall take precedence.

The Management Object Identifier is: urn:oma:mo:ext-3gpp-MCVideo-service-configuration:1.0.

Protocol compatibility: This MO is compatible with OMA OMA DM 1.2 [3].

The OMA DM ACL property mechanism (see OMA OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the MCVideo service configuration MO.

The following nodes and leaf objects are possible under the MCVideo service configuration node as described in figure 14.1.1.

Figure 14.1.1: The MCVideo service configuration MO

## 14.2	MCVideo service configuration MO parameters

### 14.2.1	General

This clause describes the parameters for the MCVideo service configuration Management Object (MO).

### 14.2.2	Node: <x>

Table 14.2.2.1: Node: <x>

| <x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for the MCVideo service configuration Management Object (MO). For the MCVideo service configuration MO, the namespace specific string is: "urn:oma:mo:oma-dm-mcvideo-service configuration:1.0" |  |  |  |  |

-	Values: N/A

### 14.2.3	/<x>/Name

Table 14.2.3.1: /<x>/Name

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | The Name leaf is a name for the MCVideo service configuration settings. |  |  |  |  |

-	Values: <User displayable name>

### 14.2.4	/<x>/Ext/

Table 14.2.4.1: /<x>/Ext/

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get, Replace |  |
|  | The Ext is an interior node for where the vendor specific information about the MCVideo service configuration MO is being placed. |  |  |  |  |

Usually the vendor extension is identified by vendor specific name under the ext node and contains the vendor meaning application vendor, device vendor etc. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Values: N/A

### 14.2.5	/<x>/Common

Table 14.2.5.1: /<x>/Common

| Common |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for the common network operation which means both on-network operation and off-network operation. |  |  |  |  |

### 14.2.6	/<x>/Common/BroadcastMCVideoGroupCall

Table 14.2.6.1: /<x>/Common/BroadcastMCVideoGroupCall

| Common/BroadcastMCVideoGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for Broadcast MCVideo Group Call configuration. |  |  |  |  |

### 14.2.7	/<x>/Common/BroadcastMCVideoGroupCall/
NumLevelGroupHierarchy

Table 14.2.7.1: /<x>/Common/BroadcastMCVideoGroupCall/NumLevelGroupHierarchy

| Common/BroadcastMCVideoGroupCall/NumLevelGroupHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the number of levels of group hierarchy for group-broadcast groups. |  |  |  |  |

-	Values: 0-255

The group-broadcast group with the lowest NumLevelGroupHierarchy value shall be considered as the group-broadcast group having the lowest level among the groups.

### 14.2.8	/<x>/Common/BroadcastMCVideoGroupCall/
NumLevelUserHierarchy

Table 14.2.8.1: /<x>/Common/BroadcastMCVideoGroupCall/NumLevelUserHierarchy

| Common/BroadcastMCVideoGroupCall/NumLevelUserHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the number of levels of user hierarchy for user-broadcast groups. |  |  |  |  |

-	Values: 0-255

The user-broadcast group with the lowest NumLevelUserHierarchy value shall be considered as the user-broadcast group the lowest level among the groups.

### 14.2.9	/<x>/Common/MinLengthAliasID

Table 14.2.9.1: /<x>/Common/MinLengthAliasID

| Common/MinLengthAliasID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates minimum length of an alphanumeric identifier (i.e., alias) assigned by an MCVideo administrator. |  |  |  |  |

-	Values: 0-255

### 14.2.10	/<x>/OffNetwork

Table 14.2.10.1: /<x>/OffNetwork

| OffNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for off-network operation. |  |  |  |  |

### 14.2.11	/<x>/OffNetwork/DefaultPPPP

Table 14.2.11.1: /<x>/OffNetwork/DefaultPPPP

| OffNetwork/DefaultPPPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default ProSe Per-Packet Priority (PPPP) configuration. |  |  |  |  |

### 14.2.12	/<x>/OffNetwork/DefaultPPPP/MCVideoPrivateCallSignalling

Table 14.2.12.1: /<x>/OffNetwork/DefaultPPPP/MCVideoPrivateCallSignalling

| OffNetwork/DefaultPPPP/MCVideoPrivateCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo private call signalling. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 14.2.13	/<x>/OffNetwork/DefaultPPPP/MCVideoPrivateCallMedia

Table 14.2.13.1: /<x>/OffNetwork/DefaultPPPP/MCVideoPrivateCallMedia

| OffNetwork/DefaultPPPP/MCVideoPrivateCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo private call media. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 14.2.14	/<x>/OffNetwork/DefaultPPPP/
MCVideoEmergencyPrivateCallSignalling

Table 14.2.14.1: /<x>/OffNetwork/DefaultPPPP/MCVideoEmergencyPrivateCallSignalling

| OffNetwork/DefaultPPPP/MCVideoEmergencyPrivateCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo emergency private call signalling. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 14.2.15	/<x>/OffNetwork/DefaultPPPP/
MCVideoEmergencyPrivateCallMedia

Table 14.2.15.1: /<x>/OffNetwork/DefaultPPPP/MCVideoEmergencyPrivateCallMedia

| OffNetwork/DefaultPPPP/MCVideoEmergencyPrivateCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default ProSe Per-Packet Priority (PPPP) value (as specified in 3GPP TS 23.303 [6]) for the MCVideo emergency private call media. |  |  |  |  |

-	Values: 1-8

The MCVideo user data with the lowest ProSe Per-Packet Priority value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 14.2.15A	/<x>/OffNetwork/DefaultPQI

Table 14.2.15A.1: /<x>/OffNetwork/DefaultPQI

| OffNetwork/DefaultPQI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the default PQI configuration. |  |  |  |  |

### 14.2.15B	/<x>/OffNetwork/DefaultPQI/MCVideoPrivateCallSignalling

Table 14.2.15B.1: /<x>/OffNetwork/DefaultPQI/MCVideoPrivateCallSignalling

| OffNetwork/DefaultPQI/MCVideoPrivateCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCVideo private call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 14.2.15C	/<x>/OffNetwork/DefaultPQI/MCVideoPrivateCallMedia

Table 14.2.15C.1: /<x>/OffNetwork/DefaultPQI/MCVideoPrivateCallMedia

| OffNetwork/DefaultPQI/MCVideoPrivateCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCVideo private call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 14.2.15D	/<x>/OffNetwork/DefaultPQI/
MCVideoEmergencyPrivateCallSignalling

Table 14.2.15D.1: /<x>/OffNetwork/DefaultPQI/MCVideoEmergencyPrivateCallSignalling

| OffNetwork/DefaultPQI/MCVideoEmergencyPrivateCallSignalling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCVideo emergency private call signalling. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 14.2.15E	/<x>/OffNetwork/DefaultPQI/
MCVideoEmergencyPrivateCallMedia

Table 14.2.15E.1: /<x>/OffNetwork/DefaultPQI/MCVideoEmergencyPrivateCallMedia

| OffNetwork/DefaultPQI/MCVideoEmergencyPrivateCallMedia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the default PQI value (as specified in 3GPP TS 23.304 [25]) for the MCVideo emergency private call media. |  |  |  |  |

-	Values: 21-26, 55-61, 90-93

The MCVideo user data with the lowest PQI value shall be considered as the MCVideo user data having the highest priority among the MCVideo user data.

### 14.2.16	/<x>/OffNetwork/PrivateCall

Table 14.2.16.1: /<x>/OffNetwork/PrivateCall

| OffNetwork/PrivateCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for private call configuration. |  |  |  |  |

### 14.2.17	/<x>/OffNetwork/PrivateCall/MaxDuration

Table 14.2.17.1: /<x>/OffNetwork/PrivateCall/MaxDuration

| OffNetwork/PrivateCall/MaxDuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates max private call duration. |  |  |  |  |

-	Values: 0-65535

The MaxDuration time is in seconds.

### 14.2.18	/<x>/OffNetwork/NumLevelHierarchy

Table 14.2.18.1: /<x>/OffNetwork/NumLevelHierarchy

| OffNetwork/NumLevelHierarchy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates the number of levels of hierarchy for transmission control override in off-network. |  |  |  |  |

-	Values: 4-256

The request with the lowest NumLevelHierarchy value shall be considered as the request having the lowest priority level given to override an active transmission among the requests.

### 14.2.19	/<x>/OnNetwork

Table 14.2.19.1: /<x>/OnNetwork

| OnNetwork |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get, Replace |  |
|  | This interior node represents a container for on-network operation. |  |  |  |  |

### 14.2.20	/<x>/OnNetwork/MCVideoAdhocGroup

Table 14.2.20.1: /<x>/OnNetwork/MCVideoAdhocGroup

| OnNetwork/MCVideoAdhocGroup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo adhoc group call related configurations. |  |  |  |  |

### 14.2.21	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall

Table 14.2.21.1: /<x>/OnNetwork/MCVideoAdhocGroup/GroupCall

| OnNetwork/MCVideoAdhocGroup/GroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node is a placeholder for the MCVideo adhoc group call policy and configurations. |  |  |  |  |

### 14.2.22	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/
AllowAdhocGroupCall

Table 14.2.22.1: /<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/AllowAdhocGroupCall

| OnNetwork/MCVideoAdhocGroup/GroupCall/AllowAdhocGroupCall |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get, Replace |  |
|  | This leaf node indicates whether on-network adhoc group calls support enabled or disabled. |  |  |  |  |

When set to "true" the MCVideo system supports the adhoc group call and is enabled to allow the MCVideo users to make an MCVideo adhoc group call.

When set to "false" the MCVideo system supports the adhoc group call and is disabled to not allow the MCVideo users to make an MCVideo adhoc group call.

### 14.2.23	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/
MaxNoParticipants

Table 14.2.23.1: /<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/MaxNoParticipants

| OnNetwork/MCVideoAdhocGroup/GroupCall/MaxNoParticipants |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates maximum number of participants allowed in an adhoc group call and the value is left to deplyoment requirement. |  |  |  |  |

### 14.2.24	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/HangTime

Table 14.2.24.1: /<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/HangTime

| OnNetwork/MCVideoAdhocGroup/GroupCall/HangTime |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node indicates hang timer for adhoc group calls. |  |  |  |  |

-	Values: 0-65535

The HangTime is in seconds.

### 14.2.25	/<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/
MaxDurationOfCall

Table 14.2.25.1: /<x>/OnNetwork/MCVideoAdhocGroup/GroupCall/MaxDurationOfCall

| OnNetwork/MCVideoAdhocGroup/GroupCall/MaxDurationOfCall |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Status | Occurrence | Format | Min. Access Types |  |  |
|  |  | Required | One | int | Get, Replace |  |  |
|  |  | This leaf node indicates max adhoc group call duration. |  |  |  |  |  |

-	Values: 0-65535

The MaxDurationOfCall time is in seconds.

Annex A (informative):
MCPTT UE configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex B (informative):
MCPTT user profile MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex C (informative):
MCS group configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex D (informative):
MCPTT service configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex E (informative):
MCS UE initial configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex F (informative):
MCVideo UE configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex G (informative):
MCVideo user profile configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex H (informative):
MCVideo service configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex I (informative):
MCData UE configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex J (informative):
MCData user profile configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex K (informative):
MCData service configuration MO DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version. The DDF is included as an XML file.

Annex L (normative):
MO specific concepts for the support of mission critical services over 5GS

# L.1	General

The present document applies to both EPS and 5GS. This annex lists the aspects of Mission Critical Services (MCSs) Management Objects (MO) which are different in 5GS from EPS. Certain aspects that are only applicable to EPS are described in clause L.2. 5GS-specific concepts are captured in clause L.3. A mapping of EPS-specific terms to their 5GS equivalents is provided in clause L.4.

# L.2	Aspects not applicable to 5GS

The following aspects of EPS mentioned in the present document are not applicable to 5GS:

-	Multimedia Broadcast and Multicast Service (MBMS) and the corresponding MOs.

# L.3	5GS specific aspects not applicable to EPS

In order to identify a Network Slice end to end, the 5GS uses information called S-NSSAI (Single Network Slice Selection Assistance Information) as specified in 3GPP TS 23.501 [23]. Network slicing and the S-NSSAI related configuration are not applicable to EPS.

# L.4	Mapping of EPS-specific terms to 5GS

In 5GS, the PDU session is the equivalent of a PDN connection in EPS. The requirements and configurations for a PDN connection throughout this document shall also apply to PDU sessions. For example, in 5GS, the Data Network Name (DNN) is the equivalent of an APN in EPS as specified in 3GPP TS 23.003 [5]. The requirements and configurations for an APN throughout this document shall apply to 5GS as well.

When using the 5GS, a bearer is provided by a 5GS QoS flow. The requirements, procedures, and configurations for a bearer throughout this document, including those stating EPS explicitly (e.g., EPS bearer priority), shall also apply to QoS flows.

The MOs defined in this specification are reused for off-network communication and on-network communication (i.e. UE-to-network relay) for MC service over 5G ProSe with following differences:

-	ProSe direct discovery for public safety shall be replaced with 5G ProSe direct discovery, as specified in 3GPP TS 24.554 [26];

-	ProSe direct communication shall be replaced with 5G ProSe direct communication, as specified in 3GPP TS 24.554 [26];

-	ProSe UE-to-network relay shall be replaced with 5G ProSe UE-to-network relay, as specified in 3GPP TS 24.554 [26];

-	default ProSe Per-Packet Priority (PPPP) is replaced by default 5G ProSe PC5 5QI (PQI) as specified in the present specification; and

-	the discovery group ID is replaced by application layer group ID as specified in the present specification.

Annex M (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2015-07 |  |  |  |  | Initial proposal to CT1 | - | 0.0.0 |
| 2015-08 |  |  |  |  | Included skeleton from C1ah-150016 and scope from C1ah-150038 agreed at CT1#92bis | 0.0.0 | 0.10 |
| 2015-08 |  |  |  |  | Included CRs C1-152964 and C1-152965 agreed at CT1#93 | 0.1.0 | 0.2.0 |
| 2015-08 |  |  |  |  | Revision number changed due to cover sheet modifications | 0.2.0 | 0.2.1 |
| 2015-09 |  |  |  |  | Modifying the cover sheet to add the TS number. Changes by the rapporteur. | 0.2.1 | 0.2.2 |
| 2015-10 |  |  |  |  | Included CRs C1-153754,and C1-153755 agreed at CT1#94 | 0.2.2 | 0.3.0 |
| 2016-01 |  |  |  |  | Included CRs C1-160352, C1-160474, C1-160475, C1-160476, C1-160477 and C1-160478 agreed at CT1#95bis | 0.3.0 | 0.4.0 |
| 2016-01 |  |  |  |  | Editorial and formatting fixes | 0.4.0 | 0.4.1 |
| 2016-02 |  |  |  |  | Included CRs C1-161077, C1-161078, C1-161231, C1-161324, C1-161326, C1-161327, C1-161509 and C1-161510 agreed at CT1#96 Editorial and formatting fixes by the rapporteur. | 0.4.1 | 0.5.0 |
| 2016-03 | CT-71 | CP-160056 |  |  | Version 1.0.0 created for presentation for information and approval | 0.5.0 | 1.0.0 |
| 2016-03 | CT-71 |  |  |  | Version 13.0.0 created after approval | 1.0.0 | 13.0.0 |
| 2016-03 |  |  |  |  | An editorial change from TS rapporteur | 13.0.0 | 13.0.1 |
| 2016-06 | CT-72 | CP-160322 | 0002 | 1 | Corrections to the scope of MCPTT Management Object (MO) | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0003 | 3 | Correction for MCPTT UE configuration management object (MO) | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0004 | 3 | Correction for MCPTT user profile management object (MO) | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0005 | 2 | Correction for MCPTT group configuration management object (MO) | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0006 | 1 | Correction for MCPTT service configuration management object (MO) | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0007 | 3 | Correction for MCPTT UE initial configuration management object (MO) | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0011 |  | Correction for security configuration parameters in MCPTT management object (MO) | 13.0.1 | 13.1.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-09 | CT#73 | CP-160565 | 0013 | 3 | F | Correction for configuration parameters in MCPTT user profile management object (MO) | 13.2.0 |
| 2016-09 | CT#73 | CP-160503 | 0014 | 1 | F | Correction for group related parameters in MCPTT UE initial configuration management object (MO) | 13.2.0 |
| 2016-09 | CT#73 | CP-160503 | 0015 | 1 | F | Correction for security material parameter in MCPTT group configuration management object (MO) | 13.2.0 |
| 2016-09 | CT#73 | CP-160503 | 0016 |  | F | Correction for "MCPTTGroupPriorityHierarchy" value in MCPTT UE configuration management object (MO) | 24.383 13.2.0 |
| 2016-12 | CT#74 |  |  |  |  | Change of spec number from 24.383 to 24.483 with wider scope and changed title | 24.483 13.2.1 |
| 2016-12 | CT#74 | CP-160733 | 0018 | 1 | F | Identity management endpoint UE initial configuration MO correction (24.383 CR) | 13.3.0 |
| 2016-12 | CT#74 | CP-160733 | 0019 | 1 | F | Correction for ProSe related configuration parameters (24.383 CR) | 13.3.0 |
| 2016-12 | CT#74 | CP-160733 | 0020 | 3 | F | Correction for MCPTT Private Call and Group Call in MCPTT user profile management (MO) (24.383 CR) | 13.3.0 |
| 2016-12 | CT#74 | CP-160733 | 0021 |  | F | Modify number of levels for priority hierarchy (24.383 CR) | 13.3.0 |
| 2016-12 | CT#74 | CP-160733 | 0022 | 1 | F | Identification of pre-selected MCPTT user profile in MCPTT user profile managed object. (24.383 CR) | 13.3.0 |
| 2016-12 | CT#74 | CP-160734 | 0023 | 2 | F | Off-network configuration parameter for queue (24.383 CR) | 13.3.0 |
| 2017-03 | CT#75 | CP-170117 | 0001 | 2 | F | Scope alignment with TS 24.483 title | 13.4.0 |
| 2017-03 | CT#75 | CP-170117 | 0004 |  | F | Corrections to upper limits | 13.4.0 |
| 2017-03 | CT#75 | CP-170127 | 0002 | 1 | F | Modifying references in TS 24.483 to cater for rel-14 Stage 2 and Stage 3 mission critical restructure | 14.0.0 |
| 2017-03 | CT#75 | CP-170127 | 0003 | 1 | F | Corrections and clean-up for 24.483 | 14.0.0 |
| 2017-03 | CT#75 | CP-170125 | 0005 | 1 | B | MCPTT user profile MO modifications in support of private call call-back | 14.0.0 |
| 2017-06 | CT#76 | CP-171080 | 0006 |  | B | Updating general parts of TS 24.483 for MCData, MCVideo and the initial configuration MO | 14.1.0 |
| 2017-06 | CT#76 | CP-171080 | 0007 | 2 | B | Modifications to the Group Configuration MO for MCData and MCVideo | 14.1.0 |
| 2017-06 | CT#76 | CP-171081 | 0008 | 1 | B | MCVideo UE configuration MO | 14.1.0 |
| 2017-06 | CT#76 | CP-171081 | 0009 | 2 | B | MCVideo user profile configuration MO | 14.1.0 |
| 2017-06 | CT#76 | CP-171081 | 0010 |  | B | MCVideo service configuration MO | 14.1.0 |
| 2017-06 | CT#76 | CP-171080 | 0011 | 2 | B | MCData UE configuration MO | 14.1.0 |
| 2017-06 | CT#76 | CP-171080 | 0012 | 2 | B | MCData user profile configuration MO | 14.1.0 |
| 2017-06 | CT#76 | CP-171080 | 0013 | 1 | B | MCData service configuration MO | 14.1.0 |
| 2017-06 | CT#76 | CP-171114 | 0014 | 1 | B | eMCPTT user profile MO updates | 14.1.0 |
| 2017-06 | CT#76 | CP-171113 | 0016 | 1 | A | AllowedFloorControlProtection configuration parameter | 14.1.0 |
| 2017-06 | CT#76 | CP-171113 | 0018 |  | A | QueueUsage configuration parameter | 14.1.0 |
| 2017-06 | CT#76 | CP-171082 | 0019 | 1 | F | Correction of User Aliases configuration parameter in the MCPTT user profile MO | 14.1.0 |
| 2017-09 | CT#77 | CP-172101 | 0020 | 1 | B | User profile MO additions for eMCPTT | 14.2.0 |
| 2017-09 | CT#77 | CP-172096 | 0022 | 1 | A | Clause 4 - Correction of occurrences of parameters | 14.2.0 |
| 2017-09 | CT#77 | CP-172104 | 0025 |  | F | Clause 6 - Correction of occurrences of parameters | 14.2.0 |
| 2017-09 | CT#77 | CP-172102 | 0026 | 1 | F | Clause 9 - Correction of occurrences of parameters | 14.2.0 |
| 2017-09 | CT#77 | CP-172143 | 0028 | 2 | F | Specify KMS-URI configuration for MCDATA in the MCData User Profile Management Object. | 14.2.0 |
| 2017-09 | CT#77 | CP-172144 | 0029 | 2 | F | Clause 10 - Correction of names and formats of some parameters | 14.2.0 |
| 2017-09 | CT#77 | CP-172104 | 0030 |  | F | Clause 12 - Correction of occurrences of parameters | 14.2.0 |
| 2017-09 | CT#77 | CP-172104 | 0031 |  | F | Clause 13 - Correction of occurrences of parameters and their graphical representation | 14.2.0 |
| 2017-09 | CT#77 | CP-172096 | 0032 | 2 | A | Clause 8 – Inclusion of missing timer TFG14 | 14.2.0 |
| 2017-09 | CT#77 | CP-172096 | 0034 | 1 | A | Include missing elements in MCPTT UE initial configuration MO | 14.2.0 |
| 2017-09 | CT#77 | CP-172134 | 0037 |  | F | DDF files for TS 24.483 | 14.2.0 |
| 2017-12 | CT#78 | CP-173066 | 0039 | 1 | F | Off-network MCVideo configurations | 14.3.0 |
| 2017-12 | CT#78 | CP-173073 | 0038 | 1 | B | Authorisation parameters for remotely initiated calls - user profile MO | 15.0.0 |
| 2018-03 | CT#79 | CP-180087 | 0040 | 1 | B | User profile MO updates for functional alias | 15.1.0 |
| 2018-03 | CT#79 | CP-180072 | 0042 | 1 | A | User profile presentation priority | 15.1.0 |
| 2018-06 | CT#80 | CP-181064 | 0043 |  | F | Unique ID leaf node for enhanced status operational value | 15.2.0 |
| 2018-09 | CT#81 | CP-182149 | 0045 | 2 | B | Location of Talker | 15.3.0 |
| 2018-09 | CT#81 | CP-182148 | 0046 |  | B | Reception Priority | 15.3.0 |
| 2019-03 | CT#83 | CP-190078 | 0051 | 1 | A | Correction of MO counter names CFx11 and CFx12 | 15.4.0 |
| 2019-03 | CT#83 | CP-190078 | 0055 | 1 | A | Remove T230 from TS 24.483 | 15.4.0 |
| 2019-03 | CT#83 | CP-190103 | 0048 | 2 | F | 24.483 Server URI and IP version additions | 16.0.0 |
| 2019-03 | CT#83 | CP-190103 | 0052 | 1 | F | Correction of Annex C and Annex E | 16.0.0 |
| 2019-09 | CT#85 | CP-192066 | 0057 | 1 | B | Restricting incoming private communications MO | 16.1.0 |
| 2019-12 | CT#86 | CP-193110 | 0058 | 1 | B | MO for limiting the number of emergency group calls for a specific functional alias | 16.2.0 |
| 2019-12 | CT#86 | CP-193110 | 0059 | 1 | B | Automatic activation and deactivation of functional aliases based on location | 16.2.0 |
| 2019-12 | CT#86 | CP-193082 | 0063 |  | A | Correction of single timer TFP2 mistakenly use for two different purposes | 16.2.0 |
| 2019-12 | CT#86 | CP-193110 | 0065 |  | B | List of MCPTT group members who did not acknowledge the group call request | 16.2.0 |
| 2020-03 | CT#87e | CP-200122 | 0064 | 3 | B | Automatic group affiliation and deaffiliation based on location or functional alias | 16.3.0 |
| 2020-03 | CT#87e | CP-200115 | 0066 | 1 | C | Included absolute URI associated with the media storage function of MCData content server | 16.3.0 |
| 2020-06 | CT#88e | CP-201124 | 0067 | 1 | B | Error! Unknown document property name. | 16.4.0 |
| 2020-06 | CT#88e | CP-201086 | 0071 | 1 | A | Error! Unknown document property name. | 16.4.0 |
| 2020-06 | CT#88e | CP-201124 | 0072 | 1 | B | Add PointCoordinate, Speed, Heading nodes in 5.2.48W | 16.4.0 |
| 2020-06 | CT#88e | CP-201124 | 0073 | 1 | D | Correct capitalisation in ListOfFunctionalaliases | 16.4.0 |
| 2020-06 | CT#88e | CP-201124 | 0074 | 1 | B | MCData FA (de)affiiation by location criteria MOs | 16.4.0 |
| 2020-06 | CT#88e | CP-201124 | 0075 | 1 | B | MCData Functional Alias by location criteria | 16.4.0 |
| 2020-06 | CT#88e | CP-201121 | 0076 |  | F | Correction of node formats | 16.4.0 |
| 2020-06 | CT#88e | CP-201112 | 0077 | 1 | B | Included the MessageStoreHostname element | 16.4.0 |
| 2020-06 | CT#88e | CP-201124 | 0078 | 1 | F | Restricting incoming MCData communications MO | 16.4.0 |
| 2020-09 | CT#89e | CP-202165 | 0081 |  | F | MOs Corrections related to MONASTERY2 CRs implementation | 16.5.0 |
| 2020-09 | CT#89e | CP-202178 | 0080 | 1 | B | Add PreconfiguredGroupUseOnly MO | 17.0.0 |
| 2020-12 | CT#90e | CP-203184 | 0086 | 2 | F | Inclusion of Functional Alias related configurations for MCVideo service | 17.1.0 |
| 2021-03 | CT#91e | CP-210125 | 0088 | 1 | B | MOs to restrict MCVideo private communications | 17.2.0 |
| 2021-03 | CT#91e | CP-210128 | 0089 | 2 | F | Corrections to figures and text in clause 5 MCPTT user profile MO | 17.2.0 |
| 2021-03 | CT#91e | CP-210128 | 0090 | 1 | F | Corrections, addition of missing reference, and editorials to clause 6 MCS group configuration MO | 17.2.0 |
| 2021-03 | CT#91e | CP-210128 | 0092 | 1 | F | Corrections to clause 10 MCData user profile MO | 17.2.0 |
| 2021-03 | CT#91e | CP-210128 | 0093 | 1 | F | Corrections to clause 13 MCVideo user profile MO | 17.2.0 |
| 2021-03 | CT#91e | CP-210128 | 0094 |  | D | Editorials to multiple Annexes A, B, and D | 17.2.0 |
| 2021-03 | CT#91e | CP-210125 | 0095 | 1 | B | Call transfer for MCPTT private call, Management Object part | 17.2.0 |
| 2021-03 | CT#91e | CP-210126 | 0096 | 1 | B | Management object for APN configuration | 17.2.0 |
| 2021-03 | CT#91e | CP-210128 | 0097 |  | D | Errors in clause 8 | 17.2.0 |
| 2021-03 | CT#91e | CP-210125 | 0098 | 1 | B | MO update to support allowed FAs | 17.2.0 |
| 2021-06 | CT#92e | CP-211154 | 0099 | 1 | B | MO for limiting the number of MCData emergency groups per FA | 17.3.0 |
| 2021-06 | CT#92e | CP-211125 | 0103 | 2 | A | Correct MCVideo MOs R17 | 17.3.0 |
| 2021-06 | CT#92e | CP-211154 | 0104 | 1 | B | Call forwarding for MCPTT private call, Management Object part | 17.3.0 |
| 2021-06 | CT#92e | CP-211125 | 0108 |  | A | Clause 12: name of node indicating the maximum number of private calls | 17.3.0 |
| 2021-06 | CT#92e | CP-211126 | 0111 |  | A | Invalid MCPTT user profile MO DDF | 17.3.0 |
| 2021-06 | CT#92e | CP-211132 | 0113 | 1 | A | Occurrence "ThreeToFifteen" in MCPTT user profile MO | 17.3.0 |
| 2021-06 | CT#92e | CP-211132 | 0115 | 1 | A | Occurrence "ThreeToFifteen" in MCData user profile MO | 17.3.0 |
| 2021-06 | CT#92e | CP-211154 | 0116 | 1 | F | Occurrence "ThreeToFifteen" in MCVideo user profile MO | 17.3.0 |
| 2021-06 | CT#92e | CP-211157 | 0117 | 1 | F | MO representation rules and MOs alignment | 17.3.0 |
| 2021-09 | CT#93e | CP-212113 | 0123 |  | A | MCPTT user profile MO: occurrence of the node "Common" | 17.4.0 |
| 2021-09 | CT#93e | CP-212115 | 0127 |  | A | MCData user profile MO: occurrence of the node "Common" | 17.4.0 |
| 2021-09 | CT#93e | CP-212115 | 0131 |  | A | MCVideo user profile MO: occurrence of the node "Common" | 17.4.0 |
| 2021-09 | CT#93e | CP-212123 | 0133 |  | A | Ocurrence of the ManualDeactivationNotAllowedIfLocationCriteriaMet | 17.4.0 |
| 2021-12 | CT#94e | CP-213029 | 0136 | 1 | B | Functional alias association with group – MO configurations | 17.5.0 |
| 2021-12 | CT#94e | CP-213061 | 0139 | 1 | B | User control of communications storage into message store – MO configurations | 17.5.0 |
| 2021-12 | CT#94e | CP-213059 | 0142 | 1 | B | MO Annex with 5GS/EPS terminology alignment | 17.5.0 |
| 2022-03 | CT#95e | CP-220279 | 0143 | 1 | B | Add support for authorization to initiate/upgrd and cancel emgcy one-to-one comms. | 17.6.0 |
| 2022-03 | CT#95e | CP-220279 | 0145 | 1 | B | The hostname of the MCData notification server(s) configured in the MCData service configuration – MO | 17.6.0 |
| 2022-06 | CT#96 | CP-221227 | 0149 | 2 | B | 5G DN and S-NSSAI Config | 17.7.0 |
| 2022-06 | CT#96 | CP-221193 | 0153 | 1 | A | Group info and presentation priorities, MO | 17.7.0 |
| 2022-06 | CT#96 | CP-221225 | 0154 | 1 | B | Update MCVideo user profile MO to indicate allowed FAs | 17.7.0 |
| 2022-06 | CT#96 | CP-221227 | 0155 |  | B | 5GS QoS aspects in MO configuration | 17.7.0 |
| 2022-09 | CT#97 | CP-222173 | 0158 |  | D | Correcting MCData user profile MO below IPInformation | 18.0.0 |
| 2022-09 |  |  |  |  |  | Editorial correction | 18.0.1 |
| 2023-09 | CT#101 | CP-232206 | 0160 | 2 | B | Adhoc group communication related user profile and service configuration for MCPTT | 18.1.0 |
| 2023-09 | CT#101 | CP-232206 | 0161 | 1 | B | Adhoc group communication related user profile and service configuration for MCVideo | 18.1.0 |
| 2023-12 | CT#102 | CP-233173 | 0159 | 4 | B | PQI for MC over 5GProSe in 24.483 | 18.2.0 |
| 2023-12 | CT#102 | CP-233173 | 0163 | 3 | B | MO for MCS over 5G ProSe | 18.2.0 |
| 2023-12 | CT#102 | CP-233169 | 0164 |  | B | Adhoc group call participants modify related user profile configuration – mgmt object MCPTT | 18.2.0 |
| 2023-12 | CT#102 | CP-233169 | 0165 |  | B | Adhoc group call participants modify related user profile configuration configuration – mgmt object MCVideo | 18.2.0 |
| 2023-12 | CT#102 | CP-233169 | 0166 | 1 | B | Adhoc group data comn participants modify related user profile configuration – mgmt object MCData | 18.2.0 |
| 2023-12 | CT#102 | CP-233169 | 0167 |  | B | Adhoc group communication related user profile and service configuration – mgmt object MCData | 18.2.0 |
| 2023-12 | CT#102 | CP-233173 | 0168 | 1 | B | Application Layer Group ID for MCS over 5G ProSe | 18.2.0 |
| 2024-03 | CT#103 | CP-240113 | 0169 |  | F | Adding default PQI configuration MO in figures | 18.3.0 |
| 2024-09 | CT#105 | CP-242208 | 0185 | 1 | A | Correction of SDS to allow indication of text charset | 18.4.0 |
| 2024-12 | CT#106 | CP-243244 | 0186 | 3 | B | Addition of LMS URI in MCS UE initial configuration MO | 19.0.0 |
| 2025-06 | CT#108 | CP-251163 | 0187 |  | B | Multi-talker MO configuration | 19.1.0 |
| 2025-06 | CT#108 | CP-251157 | 0189 |  | F | Addition of Idm client id MO | 19.1.0 |