| 3GPP TS 29.369 V0.3.0 (2025-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Ambient IoT Data Management Services; Stage 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	Overview	8

5	Services offered by the ADM	8

5.1	Introduction	8

5.2	Nadm_DM Service	9

5.2.1	Service Description	9

5.2.2	Service Operations	9

5.2.2.1	Introduction	9

5.2.2.2	Query	9

5.2.2.2.1	General	9

5.2.2.2.2	AIoT Device Profile Data Retrieval	9

5.2.2.2.3	AF Authorization Data Retrieval	10

5.2.2.3	Update	10

5.2.2.3.1	AIoT Device Profile Data Update	10

6	API Definitions	11

6.1	Nadm_DM Service API	11

6.1.1	Introduction	11

6.1.2	Usage of HTTP	11

6.1.2.1	General	11

6.1.2.2	HTTP standard headers	11

6.1.2.2.1	General	11

6.1.2.2.2	Content type	12

6.1.2.3	HTTP custom headers	12

6.1.3	Resources	12

6.1.3.1	Overview	12

6.1.3.2	Resource: AiotDeviceProfileData	13

6.1.3.2.1	Description	13

6.1.3.2.2	Resource Definition	13

6.1.3.2.3	Resource Standard Methods	13

6.1.3.3	Resource: AfAuthorizationData	15

6.1.3.3.1	Description	15

6.1.3.3.2	Resource Definition	15

6.1.3.3.3	Resource Standard Methods	15

6.1.4	Custom Operations without associated resources	16

6.1.5	Notifications	16

6.1.6	Data Model	16

6.1.6.1	General	16

6.1.6.2	Structured data types	17

6.1.6.2.1	Introduction	17

6.1.6.2.2	Type: AiotDevProfileData	17

6.1.6.2.3	Type: LastKnownAiotfInfo	18

6.1.6.2.4	Type: IndividualAfAuthorizationData	19

6.1.6.2.5	Type: AllowedTargetAiotDevice	19

6.1.6.2.6	Type: AfAuthorizationData	19

6.1.6.3	Simple data types and enumerations	20

6.1.6.3.1	Introduction	20

6.1.6.3.2	Simple data types	20

6.1.6.3.3	Enumeration: AllowedServiceOperation	20

6.1.7	Error Handling	20

6.1.7.1	General	20

6.1.7.2	Protocol Errors	20

6.1.7.3	Application Errors	20

6.1.8	Feature negotiation	20

6.1.9	Security	21

6.1.10	HTTP redirection	21

Annex A (normative):	OpenAPI specification	22

A.1	General	22

A.2	Nadm_DM API	22

Annex B (informative):	Change history	26

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nadm Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the ADM.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.369[14].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 9113: "HTTP/2".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 9457: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.369: " Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[15]	IETF RFC 9110: "http Semantics".

[16]	IETF RFC 9111: "HTTP Caching".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and 3GPP TS 23.369 [14] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and 3GPP TS 23.369 [14] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Overview

Within the 5GC, the ADM offers services to the AIOTF, NEF via the Nadm service based interface (see 3GPP TS 23.369 [14]).

Figure 4-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the ADM:

Figure 4-1: Reference model – ADM

The functionalities supported by the ADM are listed in clause 4.5.9 of 3GPP TS 23.369 [14].

# 5	Services offered by the ADM

## 5.1	Introduction

The ADM offers the following services via the Nadm interface:

-	Nadm_DM Service

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nadm_DM | 6.1 | ADM Data Management | TS29369_Nadm_DM.yaml | nadm-dm | A.2 |

## 5.2	Nadm_DM Service

### 5.2.1	Service Description

The Nadm_DM service enables an NF to request AIoT device profile data or the AF authorization data or update the AIoT device profile data in the ADM.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the Nadm_DM service are as follows:

-	Query: It enables a consumer NF to request AIoT device profile data or the AF authorization data from the ADM.

-	Update: It enables a consumer NF to update the AIoT device profile data in the ADM.

#### 5.2.2.2	Query

##### 5.2.2.2.1	General

The following procedures using the Query service operation are supported:

-	AIoT Device Profile Data Retrieval

-	AF Authorization Data Retrieval

##### 5.2.2.2.2	AIoT Device Profile Data Retrieval

Figure 5.2.2.2.2-1 shows a scenario where the NF service consumer (e.g. AIOTF or NEF) sends a request to the ADM to receive the AIoT Device Profile Data (see 3GPP TS 23.369 [14]).

Figure 5.2.2.2.2-1: Requesting AIoT Device Profile Data

1.	The NF service consumer (e.g. AIOTF or NEF) sends a GET request to the resource …/aiot-device-profile-data/{aiotDevPermId}, to get the AIoT Device Profile Data.

2a.	On success, the ADM responds with "200 OK" with the AIoT Device Profile Data.

2b.	If there is no valid AIoT Device Profile Data for the AIoT device permanent identifier, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

##### 5.2.2.2.3	AF Authorization Data Retrieval

Figure 5.2.2.2.3-1 shows a scenario where the NF service consumer (e.g. AIOTF, NEF) sends a request to the ADM to receive the AF Authorization Data (see 3GPP TS 23.369 [14]).

Figure 5.2.2.2.3-1: Requesting AF Authorization Data

1.	The NF service consumer (e.g. AIOTF, NEF) sends a GET request to the resource of the AF authorization data (…/af-authorization-data), to get the Authorization Data of the AFs. The request may contain the target AF ID if the authorization data for a specific AF is to be retrieved.

2a.	On success, the ADM responds with "200 OK" with the Authorization Data of the target AF(s).

2b.	If there is no valid AF Authorization Data available, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

#### 5.2.2.3	Update

The following procedures using the Update service operation are supported:

-	AIoT Device Profile Data Update

##### 5.2.2.3.1	AIoT Device Profile Data Update

Figure 5.2.2.3.1-1 shows a scenario where the NF service consumer (e.g., AIOTF) sends a request to the ADM to modify the AIoT Device Profile Data (see 3GPP TS 23.369 [14]). The request contains the AIoT device permanent identifier and the modification instructions.

Figure 5.2.2.3.1-1: Updating AIoT Device Profile Data

1.	The NF service consumer (e.g. AIOTF) sends a PATCH request to the resource …/aiot-device-profile-data/{aiotDevPermId}, to update the AIoT Device Profile Data.

2a.	On success, the ADM responds with "204 No Content".

2b.	If there is no valid AIoT Device Profile Data for the AIoT device permanent identifier, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

# 6	API Definitions

## 6.1	Nadm_DM Service API

### 6.1.1	Introduction

The Nadm_DM shall use the Nadm_DM API.

The API URI of the Nadm_DM API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nadm-dm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nadm_DM API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 9457 [13].

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Nadm_DM API.

Figure 6.1.3.1-1: Resource URI structure of the Nadm_DM API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AiotDeviceProfileData | /aiot-device-profile-data/{aiotDevPermId} | GET | Retrieve AIoT Device Profile Data |
|  |  | PATCH | Modify AIoT Device ProfileData |
| AfAuthorizationData | /af-authorization-data | GET | Retrieve AF Authorization Data |

#### 6.1.3.2	Resource: AiotDeviceProfileData

##### 6.1.3.2.1	Description

This resource is used to represent AIoT Device Profile Data.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nadm-dm/<apiVersion>/aiot-device-profile-data/{aiotDevPermId}

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| aiotDevPermId | AiotDevPermId | Represents the AIoT Device Permanent ID (see 3GPP TS 23.369 [14] clause 5.7.2) |

##### 6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AiotDevProfileData | M | 1 | 200 OK | Upon success, a response body containing the AIoT Device Profile Data shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - DATA_NOT_FOUND |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| If-None-Match | string | O | 0..1 | Validator for conditional requests, as described in IETF RFC 9110 [15], clause 13.1.2 |
| If-Modified-Since | string | O | 0..1 | Validator for conditional requests, as described in IETF RFC 9110 [15], clause 13.1.3 |

Table 6.1.3.2.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Cache-Control | string | O | 0..1 | Cache-Control containing max-age, as described in IETF RFC 9111 [16], clause 5.2 |
| ETag | string | O | 0..1 | Entity Tag, containing a strong validator, as described in IETF RFC 9110 [15], clause 8.8.3 |
| Last-Modified | string | O | 0..1 | Timestamp for last modification of the resource, as described in IETF RFC 9110 [15], clause 8.8.2 |

6.1.3.2.3.2	PATCH

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AiotDevProfileData | M | 1 | Contains the updated AIoT Device Profile Data |

Table 6.1.3.2.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - DATA_NOT_FOUND |
| NOTE 1: In addition common data structures as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the ADM shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the ADM shall respond with PatchResult. |  |  |  |  |

#### 6.1.3.3	Resource: AfAuthorizationData

##### 6.1.3.3.1	Description

This resource is used to represent AF Authorization Data.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nadm-dm/<apiVersion>/af-authorization-data

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.3.3	Resource Standard Methods

6.1.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| af-id | AfId | O | 0..1 | When present, this parameter indicates the AF for whom the Authorization data is to be retrieved. When this IE is absent, it shall indicate that the Authorization Data for all the AFs are to be retrieved. |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AfAuthorizationData | M | 1 | 200 OK | Upon success, a response body containing the AF Authorization Data shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - DATA_NOT_FOUND |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| If-None-Match | string | O | 0..1 | Validator for conditional requests, as described in IETF RFC 9110 [15], clause 13.1.2 |
| If-Modified-Since | string | O | 0..1 | Validator for conditional requests, as described in IETF RFC 9110 [15], clause 13.1.3 |

Table 6.1.3.3.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Cache-Control | string | O | 0..1 | Cache-Control containing max-age, as described in IETF RFC 9111 [16], clause 5.2 |
| ETag | string | O | 0..1 | Entity Tag, containing a strong validator, as described in IETF RFC 9110 [15], clause 8.8.3 |
| Last-Modified | string | O | 0..1 | Timestamp for last modification of the resource, as described in IETF RFC 9110 [15], clause 8.8.2 |

### 6.1.4	Custom Operations without associated resources

In this release of this specification, no custom operations without associated resources are defined for the Nadm_DM Service.

### 6.1.5	Notifications

In this release of this specification, no notifications are defined for the Nadm_DM Service.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nadm_DM service based interface protocol.

Table 6.1.6.1-1: Nadm_DM specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AiotDevProfileData | 6.1.6.2.2 | AIoT Device Profile Data |  |
| LastKnownAiotfInfo | 6.1.6.2.3 | Last Known AIoTF Information |  |
| IndividualAfAuthorizationData | 6.1.6.2.4 | Individual AF Authorization Data |  |
| AllowedTargetAiotDevice | 6.1.6.2.5 | Allowed Target AIoT Device |  |
| AfAuthorizationData | 6.1.6.2.6 | AF Authorization Data |  |
| LastKnownAiotfInfoInd | 6.1.6.3.2 | Last known AIOTF Information |  |
| AfId | 6.1.6.3.2 | AF ID |  |
| AllowedServiceOperation | 6.1.6.3.3 | Allowed Service Operation |  |

Table 6.1.6.1-2 specifies data types re-used by the Nadm_DM service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nadm_DM service based interface.

Table 6.1.6.1-2: Nadm_DM re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [17] | see 3GPP TS 29.500 [4] clause 6.6 | SupportedFeatures |
| AiotDevPermId | 3GPP TS 29.571 [17] | AIoT device permanent identifier |  |
| AiotArea | 3GPP TS 29.571 [17] | Contains the AIoT Area |  |
| NfInstanceId | 3GPP TS 29.571 [17] | Represent a NF instance. |  |
| IpAddr | 3GPP TS 29.571 [17] | Represent the IP address |  |
| Fqdn | 3GPP TS 29.571 [17] | Represent Fully Qualified Domain Name |  |
| ProblemDetails | 3GPP TS 29.571 [17] | Used in error responses to provide more detailed information about an error. |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: AiotDevProfileData

Table 6.1.6.2.2-1: Definition of type AiotDevProfileData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aiotDevPermId | AiotDevPermId | M | 1 | Globally unique AIoT device permanent identifier |  |
| lastKnownAiotfInfo | LastKnownAiotfInfo | M | 1 | Indicate the last known AIOTF that serves the AIoT device, or unknown. |  |

##### 6.1.6.2.3	Type: LastKnownAiotfInfo

Table 6.1.6.2.3-1: Definition of type LastKnownAiotfInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lastKnownAiotfInfoInd | boolean | M | 1 | Indicate the last known AIOTF that serves the AIoT device is known, or unknown. true: Known; false: Unknown |  |
| lastKnownAiotfId | NfInstanceId | C | 0..1 | This IE may be present when lastKnownAiotfInfoInd is set to true. Indicates the last known AIOTF instance (NOTE) |  |
| lastKnownAiotfAddress | IpAddr | C | 0..1 | This IE may be present when lastKnownAiotfInfoInd is set to true. Indicates the IP address for last known AIOTF instance (NOTE) |  |
| lastKnownAiotfFqdn | Fqdn | C | 0..1 | This IE may be present when lastKnownAiotfInfoInd is set to true. Indicates the FQDN for last known AIOTF instance (NOTE) |  |
| NOTE: At least, one of lastKnownAiotfId, lastKnownAiotfAddress and lastKnownAiotfFqdn shall be included when lastKnownAiotfInfoInd is set to true. |  |  |  |  |  |

##### 6.1.6.2.4	Type: IndividualAfAuthorizationData

Table 6.1.6.2.4-1: Definition of type IndividualAfAuthorizationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | AfId | M | 1 | Identifier used to identify the AF. |  |
| allowedArea | AiotArea | C | 0..1 | If present, it indicates the allowed area for the indicated AF to perform the AIoT services operations. If this attribute is absent, it means that the complete service area is allowed for the AF. |  |
| allowedServiceOperations | array(AllowedServiceOperation) | C | 1..N | If present, it indicates the list of allowed service operation(s) for the AF. If this attribute is absent, it means that all the Service Operations are allowed for the AF. |  |
| allowedTargetAiotDevices | array(AllowedTargetAiotDevice) | C | 1..N | If present, it indicates the list of allowed AIoT Device(s) for the AF. If this attribute is absent, it means that all the Target Aiot Devices are allowed for the AF. |  |

##### 6.1.6.2.5	Type: AllowedTargetAiotDevice

Table 6.1.6.2.5-1: Definition of type AllowedTargetAiotDevice

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aiotDevPermId | AiotDevPermId | O | 0..1 | Indicates the AIoT Device Permanent ID. (NOTE) |  |
| filteringInfo | AiotFilteringInformation | O | 0..1 | Indicates the Filtering Information. (NOTE) |  |
| NOTE: Either aiotDevPermId, or filteringInfo shall be present. |  |  |  |  |  |

##### 6.1.6.2.6	Type: AfAuthorizationData

Table 6.1.6.2.6-1: Definition of type AfAuthorizationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afAuthData | map(IndividualAfAuthorizationData) | M | 1..N | A map contains the AF Authorization Data in the ADM. The key of the map is the AF ID and the value of the map is the Authorization data of the corresponding AF indicated by the key. |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AfId | string | Identifier used to identify the AF. |  |

##### 6.1.6.3.3	Enumeration: AllowedServiceOperation

The enumeration AllowedServiceOperation represents the different types of the allowed service operation for the AF.

Table 6.1.6.3.3-1: Enumeration AllowedServiceOperation

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "INVENTORY" | Indicates inventory service operation is allowed for the AF. |  |
| "READ" | Indicates read service operation is allowed for the AF. |  |
| "WRITE" | Indicates write service operation is allowed for the AF. |  |
| "PERMANENT_DISABLE" | Indicates permanent disable service operation is allowed for the AF. |  |

### 6.1.7	Error Handling

#### 6.1.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.1.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.1.7.3	Application Errors

The common application errors defined in Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may be used for the Nadm_DM. The application errors defined for the Nadm_DM service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| DATA_NOT_FOUND | 404 Not Found | The requested AIoT Device Profile Data or AF Authorization Data is not found/does not exist. |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nadm_DM API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the <API Name> API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the <API Name> API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the <API Name> service.

The Nadm_DM API defines the following scopes for OAuth2 authorization.

Table 6.1.9-1: Oauth2 scopes defined in Nadm_DM API

| Scope | Description |
| --- | --- |
| "nadm-dm" | Access to the Nadm_DM API |
| "nadm-dm:aiot-device-profile-data:read" | Access to read AIoT Device Profile Data |
| "nadm-dm:aiot-device-profile-data:modify" | Access to update AIoT Device Profile Data |
| "nadm-dm:af-authorization-data:read" | Access to read AF Authorization Data |

### 6.1.10	HTTP redirection

An HTTP request may be redirected to a different ADM service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different ADM producer instance will return the NF Instance ID of the new ADM producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an ADM redirects a service request to a different ADM using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new ADM towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

# A.2	Nadm_DM API

openapi: 3.0.0

info:

version: '1.0.0-alpha.3'

title: 'Nadm_DM'

description: |

Nadm Data Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.369 Ambient IoT Data Management Services, version 0.3.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.369/'

servers:

- url: '{apiRoot}/nadm-dm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nadm-dm

- {}

paths:

/aiot-device-profile-data/{aiotDevPermId}:

get:

summary: get AIoT Device Profile Data

operationId: Get AIoT Device Profile Data

tags:

- AIoT Device Profile Data Retrieval

parameters:

- name: aiotDevPermId

in: path

description: AIoT Device Permanent ID

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AiotDevProfileData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: modify AIoT Device Profile Data

operationId: Modify AIoT Device Profile Data

tags:

- AIoT Device Profile Data Update

parameters:

- name: aiotDevPermId

in: path

description: AIoT Device Permanent ID

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AiotDevProfileData'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/af-authorization-data:

get:

summary: get AF Authorization Data

operationId: Get AF Authorization Data

tags:

- AF Authorization Data Retrieval

parameters:

- name: af-id

in: query

description: AF ID

required: false

schema:

$ref: '#/components/schemas/AfId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AfAuthorizationData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nadm-dm: Access to the Nadm_DM API

nadm-dm:aiot-device-profile-data:read: Access to read AIoT Device Profile Data

nadm-dm:aiot-device-profile-data:modify: Access to update AIoT Device Profile Data

nadm-dm:af-authorization-data:read: Access to read AF Authorization Data

schemas:

#

# STRUCTURED TYPES

#

AiotDevProfileData:

description: Contains the AIoT Device Profile Data.

type: object

required:

- aiotDevPermId

- lastKnownAiotfInfo

properties:

aiotDevPermId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

lastKnownAiotfInfo:

$ref: '#/components/schemas/LastKnownAiotfInfo'

LastKnownAiotfInfo:

description: Contains the Last Known AIoTF Info.

type: object

required:

- lastKnownAiotfInfoInd

properties:

lastKnownAiotfInfoInd:

type: boolean

lastKnownAiotfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

lastKnownAiotfAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

lastKnownAiotfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

IndividualAfAuthorizationData:

description: Contains the AF Authorization Data for an invidual AF.

type: object

required:

- afId

properties:

afId:

$ref: '#/components/schemas/AfId'

allowedArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

allowedServiceOperations:

type: array

items:

$ref: '#/components/schemas/AllowedServiceOperation'

allowedTargetAiotDevices:

type: array

items:

$ref: '#/components/schemas/AllowedTargetAiotDevice'

AllowedTargetAiotDevice:

description: Contains the permanent AIoT Device ID or the filtering information

type: object

properties:

aiotDevPermId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

filteringInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotFilteringInformation'

AfAuthorizationData:

description: Contains the AF Authorization Data.

type: object

required:

- afAuthData

properties:

afAuthData:

type: object

description: >

'Map carrying the AF Authorization Data.

The key of the map is the AF ID of the corresponding Authorization Data'

additionalProperties:

$ref: '#/components/schemas/IndividualAfAuthorizationData'

minProperties: 1

#

# SIMPLE TYPES

#

AfId:

description: Indicates the identifier used to identify the AF.

type: string

#

# ENUMS

#

AllowedServiceOperation:

description: The Allowed Service Operation.

anyOf:

- type: string

enum:

- INVENTORY

- READ

- WRITE

- PERMANENT_DISABLE

- type: string

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | CT4#128 | C4-251315 C4-251316 C4-251317 C4-251318 C4-251460 |  |  |  | TS skeleton Implementation of pCRs agreed at CT4#128. | 0.1.0 |
| 2025-05 | CT4#129 | C4-252212 C4-252270 C4-252335 C4-252339 C4-252436 C4-252337 C4-252365 |  |  |  | TS skeleton Implementation of pCRs agreed at CT4#129. | 0.2.0 |
| 2025-08 | CT4#130 | C4-253305 C4-253406 C4-253407 C4-253408 C4-253409 C4-253410 C4-253411 |  |  |  | TS skeleton Implementation of pCRs agreed at CT4#130. | 0.3.0 |