| 3GPP TR 32.872 V0.1.0 (2025-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5GA roaming charging reliability enhancement (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

4.1	General	7

4.2	Home routed Roaming charging	7

4.3	Local Breakout Roaming charging	7

5	Scenarios and key issues	9

5.1	Topic 1: Local Breakout inter CHFs scenario	9

5.1.1	Use cases	9

5.1.1.1	Use case #1: Failures detected between the CTF and the V-CHF	9

5.1.1.2	Use case #2: Failures detected between the CTF and the V-CHF	9

5.1.2	Potential charging requirements	9

5.1.3	Key issues	9

5.1.3.1	Key issue #1: Failure handling enhancement for failures detected between the CTF and the V-CHF	9

5.1.3.2	Key issue #2: Failure handling enhancement for failures detected between the V-CHF and the H-CHF	10

5.1.4	Possible solutions	10

5.1.4.1	Solution #1	10

5.1.5	Evaluation	10

5.1.6	Conclusion	10

5.2	Topic 2: Home Routed charging scenario	10

5.2.1	Use cases	10

5.2.1.1	Use case #1: Failure of one charging session	10

5.2.2	Potential charging requirements	10

6	Conclusions and recommendations	10

Annex <A>: Change history	11

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document focuses on the roaming charging aspects related to the reliability and recoverability enhancement of roaming charging interactions, including Local Breakout roaming and Home Routed roaming.

The following items are studied:

-	Identify the roaming charging reliability enhancement scenarios and requirements.

-	Investigate the potential solutions to support the scenarios and requirements.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.255: "5G data connectivity domain charging stage 2".

[3]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

N40	Reference point between SMF and the CHF.

N16	Reference point between two SMFs.

N47	Reference point between V-SMF and H-CHF.

N107	Reference point between V-CHF and H-CHF.

N42	Reference point between AMF and the CHF in the same PLMN.

N41	Reference point between AMF and the -CHF in different PLMNs.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AMF	Access and Mobility Management Function

CHF	Charging Function

CTF	Charging Trigger Function

NF	Network Function

SMF	Session Management Function

URLLC	Ultra Reliable Low Latency Communication

4	Background

4.1	General

5G roaming charging are specified in middle tier charging specifications. Take data connectivity as an example, the home routed roaming charging, LBO roaming charging with N47 and LBO roaming charging with N107 are specified in 3GPP TS 32.255[2]. The common failure handling includes CTF detected failure, CHF detected failure, CHF as NF Consumer detected failure, Retry handling and Response code handling are specified in clause 5.5 of 3GPP TS 32.290[3]. In order to support more failure handling scenarios in roaming, the enhancement of failure handling will be studied.

4.2	Home routed Roaming charging

Figure 4.2-1 illustrates the Home Routed roaming charging architecture depicted in clause 4.2 of 3GPP TS 32.255[2]. The N40 reference point is defined for the interactions between H-SMF and H-CHF and between V-SMF and V-CHF in the reference point representation.

Figure 4.2-1: 5G data connectivity converged charging architecture in roaming Home Routed reference point representation

4.3	Local Breakout Roaming charging

Figure 4.3-1 illustrates the Local Breakout roaming charging with N47 architecture, as specified in clause 4.2.6 of 3GPP TS 32.255[2]. The N47 reference point is defined for the interactions between V-SMF and H-CHF.

Figure 4.3-1: 5G data connectivity converged charging architecture in Local Breakout V-SMF to H-CHF scenario reference point representation

Figure 4.3-2 illustrates the Local Breakout roaming charging with N107 architecture, as specified in clause 4.2.6a of 3GPP TS 32.255[2]. The N107 reference point is defined for the interactions between V-CHF and H-CHF.

Figure 4.3-2: 5G data connectivity converged charging architecture in Local Breakout Inter-CHF scenario reference point representation

5	Scenarios and key issues

5.1	Topic 1: Local Breakout inter CHFs scenario

5.1.1	Use cases

5.1.1.1	Use case #1: Failures detected between the CTF and the V-CHF

In the LBO N40+N107 charging architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. When a failure detected between V-SMF(CTF) and V-CHF, How the charging system handles the failure should be studied.

Figure 5.1.1.1-1: An example of failure detected between V-SMF(CTF) and V-CHF

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-1.

5.1.1.2	Use case #2: Failures detected between the CTF and the V-CHF

In the LBO N40+N107 architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. When a failure detected between V-CHF and H-CHF, How the charging system handles the failure should be studied.

Figure 5.1.1.2-1: An example of failure detected between V-CHF and H-CHF

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-2.

5.1.2	Potential charging requirements

REQ-3GPPCH-LBIC-1: Charging system shall support failure handling for the Local Breakout charging scenario in which a failure is detected between the CTF and the V-CHF.

REQ-3GPPCH-LBIC-2: charging system shall support failure handling for the Local Breakout charging scenario in which a failure is detected between the V-CHF and the H-CHF.

5.1.3	Key issues

5.1.3.1	Key issue #1: Failure handling enhancement for failures detected between the CTF and the V-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBIC-1. This investigation covers the following:

-	Identification of the failure handling enhancement for the scenario in which a failure is detected between the CTF and the V-CHF;

5.1.3.2	Key issue #2: Failure handling enhancement for failures detected between the V-CHF and the H-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBIC-2. This investigation covers the following:

-	Identification of the failure handling enhancement for the scenario in which a failure is detected between the V-CHF and the H-CHF;

5.1.4	Possible solutions

5.1.4.1	Solution #1

5.1.5	Evaluation

5.1.6	Conclusion

5.2	Topic 2: Home Routed charging scenario

5.2.1	Use cases

5.2.1.1	Use case #1: Failure of one charging session

In the HR roaming charging architecture, for a specific PDU session, there are two charging sessions simultaneously. When a failure is detected in one of the charging sessions, How the charging system handles the failure should be studied.

Figure 5.2.1.1-1: An example of the failure between H-SMF and H-CHF in HR roaming

The potential charging requirements for this UC are: REQ-3GPPCH-LBHR-01.

5.2.2	Potential charging requirements

REQ-3GPPCH-LBHR-01: Charging system shall support failure handling for the Home Routed charging scenario in which a failure is detected in one of the charging sessions.

6	Conclusions and recommendations

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | SA5#163 |  |  |  |  | Initial skeleton | 0.0.0 |
| 2025-10 | SA5#163 | S5-254381 S5-254488 S5-254490 S5-254489 S5-254491 S5-254812 S5-254813 S5-254815 |  |  |  | Skeleton update Add definitions of terms, symbols and abbreviations Add scope Add reference Introduce the background for roaming charging Introduce the usecase1 on Local Breakout Inter CHFs scenario Introduce the usecase2 on Local Breakout Inter CHFs scenario Introduce the usecase on Home Routed scenario | 0.1.0 |