| 3GPP TS 38.133 V16.13.0 (2022-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	51

3.6.1	RRC connected state requirements in DRX	52

3.6.2	Number of serving carriers	52

3.6.2.1	Number of serving carriers for SA	52

3.6.2.2	Number of serving carriers for EN-DC	52

3.6.2.3	Number of serving carriers for NE-DC	52

3.6.2.4	Number of serving carriers for NR-DC	53

3.6.3	Applicability for intra-band FR2	53

3.6.4	Applicability for FR2 UE power classes	53

3.6.5	Applicability for SDL bands	53

3.6.6	Applicability of requirements for NGEN-DC operation	53

3.6.7	Applicability of QCL	53

3.6.9	Applicability of requirements for scheduling availability	54

3.6.10	Applicability of requirements for measurement restrictions	54

4	SA: RRC_IDLE state mobility	54

4.1	Cell Selection	54

4.2	Cell Re-selection	54

4.2.1	Introduction	54

4.2.2	Requirements	55

4.2.2.1	UE measurement capability	55

4.2.2.2	Measurement and evaluation of serving cell	55

4.2.2.3	Measurements of intra-frequency NR cells	56

4.2.2.4	Measurements of inter-frequency NR cells	57

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	59

4.2.2.6	Maximum interruption in paging reception	61

4.2.2.7	General requirements	61

4.2.2.8	Minimum requirement at transitions	61

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	62

4.2.2.9.1	Introduction	62

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	62

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	63

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	64

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	64

4.2.2.10.1	Introduction	64

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	64

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	65

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	66

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	67

4.2.2.11.1	Introduction	67

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	67

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	68

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	69

4.2A	Cell Re-selection when subject to CCA	69

4.2A.1	Introduction	69

4.2A.2	Requirements	70

4.2A.2.1	UE measurement capability	70

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	70

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	71

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	72

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	74

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	74

4.2A.2.7	General requirements	74

4.3	Minimization of Drive Tests (MDT)	75

4.3.1	Introduction	75

4.3.2	Measurement Requirements	75

4.3.3	Requirements for Relative Time Stamp Accuracy	75

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	76

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	76

4.4	Idle Mode CA/DC Measurements	76

4.4.1	Introduction	76

4.4.2	Measurement Requirements	76

4.4.2.1	Detected cell requirement during state transition and Idle mode	76

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	77

4.4.2.3	Measurements on serving cell	78

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	78

5	SA: RRC_INACTIVE state mobility	80

5.1	Cell Re-selection	80

5.1.1	Introduction	80

5.1.2	Requirements	80

5.1.2.1	UE measurement capability	80

5.1.2.2	Measurement and evaluation of serving cell	80

5.1.2.3	Measurements of intra-frequency NR cells	80

5.1.2.4	Measurements of inter-frequency NR cells	80

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	80

5.1.2.6	Maximum interruption in paging reception	80

5.1.2.7	General requirements	80

5.1A	Cell Re-selection with CCA	80

5.1A.1	Introduction	80

5.1A.2	Requirements	81

5.1A.2.1	UE measurement capability	81

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	81

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	81

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	81

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	81

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	81

5.1A.2.7	General requirements	81

5.2	Void	81

5.3	Minimization of Drive Tests (MDT)	81

5.3.1	Introduction	81

5.3.2	Measurement Requirements	82

5.3.3	Requirements for Relative Time Stamp Accuracy	82

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	82

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	82

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	82

5.4	Idle Mode CA/DC Measurements	82

5.4.1	Introduction	82

5.4.2	Measurement Requirements	82

5.4.2.1	Detected cell requirement during state transition and Idle mode	83

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	83

5.4.2.3	Measurements on serving cell	83

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	83

6	RRC_CONNECTED state mobility	83

6.1	Handover	83

6.1.1	NR Handover	83

6.1.1.1	Introduction	83

6.1.1.2	NR FR1 - NR FR1 Handover	83

6.1.1.2.1	Handover delay	83

6.1.1.2.2	Interruption time	83

6.1.1.3	NR FR2- NR FR1 Handover	84

6.1.1.3.1	Handover delay	84

6.1.1.3.2	Interruption time	84

6.1.1.4	NR FR2- NR FR2 Handover	85

6.1.1.4.1	Handover delay	85

6.1.1.4.2	Interruption time	85

6.1.1.5	NR FR1- NR FR2 Handover	86

6.1.1.5.1	Handover delay	86

6.1.1.5.2	Interruption time	86

6.1.2	NR Handover to other RATs	87

6.1.2.1	NR – E-UTRAN Handover	87

6.1.2.1.1	Introduction	87

6.1.2.1.2	Handover delay	87

6.1.2.1.3	Interruption time	87

6.1.2.2	NR – UTRAN Handover	88

6.1.2.2.1	Introduction	88

6.1.2.2.2	Handover delay	88

6.1.2.2.3	Interruption time	88

6.1.3	NR DAPS Handover	89

6.1.3.1	Introduction	89

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	89

6.1.3.2.1	DAPS handover delay	89

6.1.3.2.2	Interruption time	90

6.1.3.3	NR FR2- NR FR1 DAPS Handover	92

6.1.3.3.1	DAPS handover delay	92

6.1.3.3.2	Interruption time	92

6.1.3.4	NR FR1- NR FR2 DAPS Handover	93

6.1.3.4.1	DAPS handover delay	93

6.1.3.4.2	Interruption time	93

6.1.4	NR Conditional Handover	94

6.1.4.1	Introduction	94

6.1.4.2	NR FR1 – NR FR1 conditional handover	94

6.1.4.3	NR FR2 – NR FR1 conditional handover	95

6.1.4.4	NR FR2 – NR FR2 conditional handover	96

6.1.4.4.1	Handover delay	96

6.1.4.4.2	Measurement time	96

6.1.4.4.3	Preparation time	96

6.1.4.4.4	Interruption time	96

6.1.4.5	NR FR1 – NR FR2 conditional handover	97

6.1A	Void	97

6.1A.1	Void	97

6.1A.1.1	Void	97

6.1A.1.2	Void	97

6.1A.1.2.1	Void	97

6.1A.1.2.2	Void	97

6.1B	Handover to target cell using CCA	97

6.1B.1	NR Handover	97

6.1B.1.1	Introduction	97

6.1B.1.2	NR FR1 - NR FR1 Handover	98

6.1B.1.2.1	Handover delay	98

6.1B.1.2.2	Interruption time	98

6.2	RRC Connection Mobility Control	99

6.2.1	SA: RRC Re-establishment	99

6.2.1.1	Introduction	99

6.2.1.2	Requirements	99

6.2.1.2.1	UE Re-establishment delay requirement	99

6.2.1A	RRC Re-establishment with CCA	100

6.2.1A.1	Introduction	100

6.2.1A.2	Requirements	101

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	101

6.2.2	Random access	103

6.2.2.1	Introduction	103

6.2.2.2	Requirements for 4-step RA type	103

6.2.2.2.1	Contention based random access	103

6.2.2.2.2	Non-Contention based random access	104

6.2.2.2.3	UE behaviour when configured with supplementary UL	105

6.2.2.3	Requirements for 2-step RA type	105

6.2.2.3.1	Contention based random access	105

6.2.2.3.2	Non-Contention based random access	106

6.2.2.3.3	UE behaviour when configured with supplementary UL	107

6.2.2A	Random access when CCA is used on target frequency	107

6.2.2A.1	Introduction	107

6.2.2A.2	Requirements for 4-step RA type	107

6.2.2A.2.1	Contention based random access	107

6.2.2A.2.2	Non-Contention based random access	108

6.2.2A.3	Requirements for 2-step RA type	109

6.2.2A.3.1	Contention based random access	110

6.2.2A.3.2	Non-Contention based random access	111

6.2.3	SA: RRC Connection Release with Redirection	112

6.2.3.1	Introduction	112

6.2.3.2	Requirements	112

6.2.3.2.1	RRC connection release with redirection to NR	112

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	113

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	113

7	Timing	114

7.1	UE transmit timing	114

7.1.1	Introduction	114

7.1.2	Requirements	115

7.1.2.1	Gradual timing adjustment	116

7.1.2.2	Void	116

7.2	UE timer accuracy	116

7.2.1	Introduction	116

7.2.2	Requirements	117

7.3	Timing advance	117

7.3.1	Introduction	117

7.3.2	Requirements	117

7.3.2.1	Timing Advance adjustment delay	117

7.3.2.2	Timing Advance adjustment accuracy	117

7.4	Cell phase synchronization accuracy	117

7.4.1	Definition	117

7.4.2	Minimum requirements	117

7.5	Maximum Transmission Timing Difference	118

7.5.1	Introduction	118

7.5.2	Minimum Requirements for inter-band EN-DC	118

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	118

7.5.3	Minimum Requirements for intra-band EN-DC	119

7.5.4	Minimum Requirements for NR Carrier Aggregation	119

7.5.5	Minimum Requirements for inter-band NE-DC	119

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	120

7.5.6	Minimum Requirements for inter-band NR DC	120

7.6	Maximum Receive Timing Difference	121

7.6.1	Introduction	121

7.6.2	Minimum Requirements for inter-band EN-DC	121

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	121

7.6.3	Minimum Requirements for intra-band EN-DC	122

7.6.4	Minimum Requirements for NR Carrier Aggregation	122

7.6.5	Minimum Requirements for inter-band NE-DC	123

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	123

7.6.6	Minimum Requirements for inter-band NR DC	124

7.7	deriveSSB-IndexFromCell tolerance	124

7.7.1	Minimum requirements	124

7.8	Void	125

8	Signalling characteristics	125

8.1	Radio Link Monitoring	125

8.1.1	Introduction	125

8.1.2	Requirements for SSB based radio link monitoring	126

8.1.2.1	Introduction	126

8.1.2.2	Minimum requirement	127

8.1.2.3	Measurement restrictions for SSB based RLM	128

8.1.3	Requirements for CSI-RS based radio link monitoring	129

8.1.3.1	Introduction	129

8.1.3.2	Minimum requirement	130

8.1.3.3	Measurement restrictions for CSI-RS based RLM	132

8.1.4	Minimum requirement at transitions	132

8.1.5	Minimum requirement for UE turning off the transmitter	133

8.1.6	Minimum requirement for L1 indication	133

8.1.7	Scheduling availability of UE during radio link monitoring	133

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	133

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	133

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	134

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	134

8.1A	Radio Link Monitoring with CCA on Target Frequency	134

8.1A.1	Introduction	134

8.1A.2	Requirements for SSB Based Radio Link Monitoring	135

8.1A.2.1	Introduction	135

8.1A.2.2	Minimum Requirement	136

8.1A.3	Minimum requirement at transitions	137

8.1A.4	Minimum requirement for UE turning off the transmitter	138

8.1A.5	Minimum requirement for L1 indication	138

8.1A.6	Scheduling availability of UE during radio link monitoring	138

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	138

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	138

8.2	Interruption	139

8.2.1	EN-DC Interruption	139

8.2.1.1	Introduction	139

8.2.1.2	Requirements	139

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	139

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	140

8.2.1.2.3	Interruptions at SCell addition/release	140

8.2.1.2.4	Interruptions at SCell activation/deactivation	141

8.2.1.2.5	Interruptions during measurements on SCC	142

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	143

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	144

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	145

8.2.1.2.9	Interruptions at SCell hibernation	145

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	146

8.2.1.2.11	Interruptions due to UE-specific CBW change	146

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	146

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	147

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	148

8.2.1.2.15	Interruptions due to SCell dormancy	149

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	149

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	150

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	150

8.2.2.1	Introduction	150

8.2.2.2	Requirements	151

8.2.2.2.1	Interruptions at SCell addition/release	151

8.2.2.2.2	Interruptions at SCell activation/deactivation	152

8.2.2.2.3	Interruptions during measurements on deactivated SCC	153

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	154

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	154

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	155

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	156

8.2.2.2.8	Interruptions due to UE-specific CBW change	156

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	157

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	158

8.2.2.2.11	Interruptions at direct SCell activation	159

8.2.2.2.12	Interruptions due to SCell dormancy	159

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	159

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	160

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	160

8.2.3	NE-DC Interruptions	161

8.2.3.1	Introduction	161

8.2.3.2	Requirements	161

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	161

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	162

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	162

8.2.3.2.4	Interruptions at SCell activation/deactivation	163

8.2.3.2.5	Interruptions during measurements on SCC	164

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	165

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	166

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	166

8.2.3.2.9	Interruptions at SCell hibernation	166

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	167

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	167

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	168

8.2.3.2.13	Interruptions due to SCell dormancy	169

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	170

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	170

8.2.3.2.16	Interruptions due to UE-specific CBW change	171

8.2.4	NR-DC: Interruptions	171

8.2.4.1	Introduction	171

8.2.4.2	Requirements	172

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	172

8.2.4.2.3	Interruptions during measurements on SCC	173

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	173

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	174

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	174

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	174

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	175

8.2.4.2.9	Interruptions at NR SRS carrier based switching	175

8.2.4.2.10	Interruptions at direct SCell activation	176

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	177

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	177

8.2.4.2.13	 Interruptions due to SCell dormancy	178

8.2.4.2.14	Interruptions due to UE-specific CBW change	178

8.2.4.2A	Void	179

8.2.4.2A.1	Void	179

8.2.4.2A.2	Void	179

8.2.4.2A.3	Void	179

8.3	SCell Activation and Deactivation Delay	179

8.3.1	Introduction	179

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	179

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	183

8.3.4	Direct SCell Activation at SCell addition	183

8.3.5	Direct SCell Activation at Handover	185

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	187

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	191

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	191

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	192

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	194

8.3A.1	Introduction	194

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	194

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	197

8.4	UE UL carrier RRC reconfiguration delay	197

8.4.1	Introduction	197

8.4.2	UE UL carrier configuration delay requirement	197

8.4.3	UE UL carrier deconfiguration delay requirement	197

8.5	Link Recovery Procedures	198

8.5.1	Introduction	198

8.5.2	Requirements for SSB based beam failure detection	198

8.5.2.1	Introduction	198

8.5.2.2	Minimum requirement	199

8.5.2.3	Measurement restriction for SSB based beam failure detection	200

8.5.3	Requirements for CSI-RS based beam failure detection	201

8.5.3.1	Introduction	201

8.5.3.2	Minimum requirement	202

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	204

8.5.4	Minimum requirement for L1 indication	205

8.5.5	Requirements for SSB based candidate beam detection	205

8.5.5.1	Introduction	205

8.5.5.2	Minimum requirement	205

8.5.5.3	Measurement restriction for SSB based candidate beam detection	207

8.5.6	Requirements for CSI-RS based candidate beam detection	208

8.5.6.1	Introduction	208

8.5.6.2	Minimum requirement	208

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	210

8.5.7	Scheduling availability of UE during beam failure detection	211

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	211

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	211

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	211

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	212

8.5.8	Scheduling availability of UE during candidate beam detection	212

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	212

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	212

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	212

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	213

8.5.9	Requirements for Beam Failure Recovery in SCell	213

8.5.9.1	Introduction	213

8.5.9.2	Requirement	213

8.5.10	Minimum requirement at transitions for beam failure detection	213

8.5A	Link Recovery Procedures when CCA is used on target frequency	214

8.5A.1	Introduction	214

8.5A.2	Requirements for SSB based beam failure detection	214

8.5A.2.1	Introduction	214

8.5A.2.2	Minimum requirement	215

8.5A.2.3	Measurement restriction for SSB based beam failure detection	215

8.5A.4	Minimum requirement for L1 indication	216

8.5A.5	Requirements for SSB based candidate beam detection	216

8.5A.5.1	Introduction	216

8.5A.5.2	Minimum requirement	216

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	217

8.5A.7	Scheduling availability of UE during beam failure detection	217

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	217

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	217

8.5A.8	Scheduling availability of UE during candidate beam detection	217

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	218

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	218

8.6	Active BWP switch delay	218

8.6.1	Introduction	218

8.6.2	DCI and timer based BWP switch delay on a single CC	218

8.6.2A	DCI based BWP switch delay on multiple CCs	219

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	219

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	221

8.6.2B	Timer based BWP switch delay on multiple CCs	221

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	221

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	222

8.6.3	RRC based BWP switch delay on a single CC	222

8.6.3A	RRC based BWP switch delay on multiple CCs	223

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	223

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	223

8.6.4	BWP switch delay on Consistent UL CCA recovery	224

8.7	Void	224

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	224

8.8.1	Introduction	224

8.8.2	E-UTRAN PSCell Addition Delay Requirement	224

8.8.3	E-UTRAN PSCell Release Delay Requirement	225

8.9	NR-DC: PSCell Addition and Release Delay	225

8.9.1	Introduction	225

8.9.2	PSCell Addition Delay Requirement	225

8.9.3	PSCell Release Delay Requirement	226

8.10	Active TCI state switching delay	226

8.10.4	DCI based TCI state switch delay	228

8.10.5	RRC based TCI state switch delay	228

8.10.6	Active TCI state list update delay	228

8.10A	Active TCI state switching delay with CCA	229

8.10A.1	Introduction	229

8.10A.2	Known conditions for TCI state	229

8.10A.3	MAC-CE based TCI state switch delay	229

8.10A.4	DCI based TCI state switch delay	230

8.10A.5	RRC based TCI state switch delay	230

8.10A.6	Active TCI state list update delay	231

8.11	PSCell Change	231

8.11A	void	232

8.11B	Conditional PSCell Change	232

8.11B.1	Introduction	232

8.11B.2	Conditoinal PSCell Change delay	232

8.11B.2.1	Measurement time	232

8.12	Uplink spatial relation switch delay	233

8.12.1	Introduction	233

8.12.2	Known conditions for spatial relation when associated with DL-RS	233

8.12.3	MAC-CE based spatial relation switch delay	233

8.12.4	DCI based spatial relation switch delay	234

8.12.5	RRC based spatial relation switch delay	234

8.13	UE-specific CBW change	235

8.13.1	Introduction	235

8.13.2	UE-specific CBW change delay	235

8.14	Pathloss reference signal switching delay	235

8.14.1	Introduction	235

8.14.2	Known conditions for pathloss reference signal	235

8.14.3	MAC-CE based pathloss reference signal switch delay	236

9	Measurement Procedure	236

9.1	General measurement requirement	236

9.1.1	Introduction	236

9.1.2	Measurement gap	237

9.1.2.1	EN-DC: Measurement Gap Sharing	248

9.1.2.1a	SA: Measurement Gap Sharing	248

9.1.2.1b	NE-DC: Measurement Gap Sharing	249

9.1.2.1c	NR-DC: Measurement Gap Sharing	250

9.1.3	UE Measurement capability	251

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	251

9.1.3.1a	SA: Monitoring of multiple layers using gaps	251

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	252

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	252

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	253

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	254

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	254

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	255

9.1A.3.2	Void	256

9.1.3A	UE Measurement capability under operation mode with CCA	256

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	256

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	256

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	256

9.1A.3.2a	Void	257

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	257

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	257

9.1.4.1	Introduction	257

9.1.4.2	Requirements	257

9.1.5	Carrier-specific scaling factor	261

9.1.5.1	Monitoring of multiple layers outside gaps	261

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	263

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	264

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	265

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	266

9.1.5.2	Monitoring of multiple layers within gaps	267

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	268

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	270

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	271

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	273

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.6	Minimum requirement at transitions	275

9.2	NR intra-frequency measurements	276

9.2.1	Introduction	276

9.2.2	Requirements applicability	276

9.2.3	Number of cells and number of SSB	277

9.2.3.1	Requirements for FR1	277

9.2.3.2	Requirements for FR2	277

9.2.4	Measurement Reporting Requirements	277

9.2.4.1	Periodic Reporting	277

9.2.4.2	Event-triggered Periodic Reporting	277

9.2.4.3	Event Triggered Reporting	277

9.2.5	Intrafrequency measurements without measurement gaps	278

9.2.5.1	Intrafrequency cell identification	278

9.2.5.2	Measurement period	281

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	282

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	283

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	283

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	283

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	284

9.2.5.4	SFTD Measurements between PCell and PSCell	285

9.2.5.4.1	Introduction	285

9.2.5.4.2	SFTD Measurement delay	285

9.2.5.4.3	SFTD Measurement Reporting Delay	285

9.2.6	Intra-frequency measurements with measurement gaps	286

9.2.6.1	Void	286

9.2.6.2	Intra-frequency cell identification	286

9.2.6.3	Intrafrequency Measurement Period	287

9.2A	NR intra-frequency measurements with CCA	288

9.2A.1	Introduction	288

9.2A.2	Requirements applicability	289

9.2A.3	Number of cells and number of SSB	289

9.2A.4	Measurement Reporting Requirements	289

9.2A.5	Intra-frequency measurements without measurement gaps	290

9.2A.5.2	Measurement period	292

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	293

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	293

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	294

9.2A.6	Intra-frequency measurements with measurement gaps	294

9.2A.6.1	Intra-frequency cell identification	294

9.2A.6.2	Intra-frequency Measurement Period	295

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	296

9.2A.7.1	Intra-frequency RSSI measurements	296

9.2A.7.2	Intra-frequency Channel occupancy measurements	297

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	298

9.3	NR inter-frequency measurements	298

9.3.1	Introduction	298

9.3.2	Requirements applicability	299

9.3.2.1	Void	299

9.3.2.2	Void	299

9.3.3	Number of cells and number of SSB	299

9.3.3.1	Requirements for FR1	299

9.3.3.2	Requirements for FR2	299

9.3.4	Inter-frequency measurement with measurement gaps	299

9.3.4.1	Void	301

9.3.4.2	Void	301

9.3.5	Inter-frequency measurements	301

9.3.5.1	Void	302

9.3.5.2	Void	302

9.3.5.3	Void	302

9.3.6	Inter-frequency measurements reporting requirements	302

9.3.6.1	Periodic Reporting	302

9.3.6.2	Event-triggered Periodic Reporting	302

9.3.6.3	Event-triggered Reporting	302

9.3.7	Void	303

9.3.8	Inter-frequency SFTD measurement requirements	303

9.3.8.1	Introduction	303

9.3.8.2	SFTD Measurement delay	303

9.3.8.3	SFTD Measurement reporting delay	304

9.3.9	Inter frequency measurements without measurement gaps	304

9.3.9.1	Inter frequency Cell identification	304

9.3.9.2	Measurement period	306

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	306

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	306

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	307

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	307

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	308

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	308

9.3A.1	Introduction	308

9.3A.2	Requirements applicability	308

9.3A.3	Number of cells and number of SSB	309

9.3A.3.1	Requirements	309

9.3A.4	Inter-frequency cell identification	309

9.3A.5	Inter-frequency measurements	310

9.3A.6	NR Inter-frequency measurements reporting requirements	311

9.3A.6.1	Periodic Reporting	311

9.3A.6.2	Event-triggered Periodic Reporting	311

9.3A.6.3	Event-triggered Reporting	311

9.3A.8	Inter-frequency RSSI measurements	311

9.3A.9	Inter-frequency channel occupancy measurements	312

9.4	Inter-RAT measurements	312

9.4.1	Introduction	312

9.4.2	NR − E-UTRAN FDD measurements	314

9.4.2.1	Introduction	314

9.4.2.2	Requirements when no DRX is used	314

9.4.2.3	Requirements when DRX is used	315

9.4.2.4	Measurement reporting requirements	316

9.4.2.4.1	Periodic Reporting	316

9.4.2.4.2	Event-Triggered Periodic Reporting	316

9.4.2.4.3	Event-Triggered Reporting	316

9.4.3	NR − E-UTRAN TDD measurements	316

9.4.3.1	Introduction	316

9.4.3.2	Requirements when no DRX is used	316

9.4.3.3	Requirements when DRX is used	317

9.4.3.4	Measurement reporting requirements	319

9.4.3.4.1	Periodic Reporting	319

9.4.3.4.2	Event-Triggered Periodic Reporting	319

9.4.3.4.3	Event-Triggered Reporting	319

9.4.4	Inter-RAT RSTD measurements	319

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	319

9.4.4.1.1	Introduction	319

9.4.4.1.2	Requirements	320

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	323

9.4.4.2.1	Introduction	323

9.4.4.2.2	Requirements	324

9.4.5	Inter-RAT E-CID measurements	327

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	327

9.4.5.1.1	Introduction	327

9.4.5.1.2	Requirements	327

9.4.5.1.3	Measurement Reporting Delay	327

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	327

9.4.5.2.1	Introduction	327

9.4.5.2.2	Requirements	327

9.4.5.2.3	Measurement Reporting Delay	328

9.4.6	NR − UTRAN FDD measurements	328

9.4.6.1	Introduction	328

9.4.6.2	Requirements when no DRX is used	328

9.4.6.3	Requirements when DRX is used	329

9.4.7	NR – E-UTRAN measurements with autonomous gaps	331

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	331

9.4.7.2	CGI reporting delay	331

9.5	L1-RSRP measurements for Reporting	332

9.5.1	Introduction	332

9.5.2	Requirements applicability	332

9.5.3	Measurement Reporting Requirements	332

9.5.3.1	Periodic Reporting	333

9.5.3.2	Semi-Persistent Reporting	333

9.5.3.3	Aperiodic Reporting	333

9.5.4	L1-RSRP measurement requirements	333

9.5.4.1	SSB based L1-RSRP Reporting	333

9.5.4.2	CSI-RS based L1-RSRP Reporting	335

9.5.4A	Void	338

9.5.4A.1	Void	338

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	338

9.5.5.1	Measurement restriction for SSB based L1-RSRP	338

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	338

9.5.6	Scheduling availability of UE during L1-RSRP measurement	339

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	339

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	339

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	340

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	340

9.5A	L1-RSRP measurements for Reporting under CCA	341

9.5A.1	Introduction	341

9.5A.2	Requirements applicability	341

9.5A.3	Measurement Reporting Requirements	341

9.5A.3.1	Periodic Reporting	341

9.5A.3.2	Semi-Persistent Reporting	342

9.5A.3.3	Aperiodic Reporting	342

9.5A.4	L1-RSRP measurement requirements	342

9.5A.4.1	SSB based L1-RSRP Reporting	342

9.5A.5	Measurement restriction for L1-RSRP measurement	343

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	343

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	343

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	343

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	344

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	344

9.6	NE-DC: Measurements	344

9.6.1	Introduction	344

9.6.2	SFTD Measurements	344

9.6.2.1	Introduction	344

9.6.2.2	SFTD Measurement requirements	344

9.7	Cross Link Interference measurements	345

9.7.1	Introduction	345

9.7.2	SRS-RSRP measurements	345

9.7.2.1	Introduction	345

9.7.2.2	Requirements applicability	345

9.7.2.3	Measurement Reporting Requirements	346

9.7.2.3.1	Periodic Reporting	346

9.7.2.3.2	Event-triggered Periodic Reporting	346

9.7.2.3.3	Event Triggered Reporting	346

9.7.2.4	Measurement capability	346

9.7.2.5	SRS-RSRP measurement period	346

9.7.3	CLI-RSSI measurements	347

9.7.3.1	Introduction	347

9.7.3.2	Requirements applicability	347

9.7.3.3	Measurement Reporting Requirements	347

9.7.3.3.1	Periodic Reporting	347

9.7.3.3.2	Event-triggered Periodic Reporting	347

9.7.3.3.3	Event Triggered Reporting	347

9.7.3.4	Measurement capability	348

9.7.3.5	CLI-RSSI measurement period	348

9.7.4	Scheduling availability of UE during CLI measurements	348

9.7.4.1	Scheduling availability of UE performing measurement on FR1	348

9.7.4.2	Scheduling availability of UE performing measurement on FR2	348

9.8	L1-SINR measurements for Reporting	349

9.8.1	Introduction	349

9.8.2	Requirements applicability	349

9.8.3	Measurement Reporting Requirements	350

9.8.3.1	Periodic Reporting	350

9.8.3.2	Semi-Persistent Reporting	350

9.8.3.3	Aperiodic Reporting	351

9.8.4	L1-SINR measurement requirements	351

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	351

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	354

9.8.5	Measurement restriction for L1-SINR measurement	356

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	356

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	357

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	357

9.8.6	Scheduling availability of UE during L1-SINR measurement	358

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	358

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	358

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	359

9.9	 NR measurements for positioning	359

9.9.1	Introduction	359

9.9.2	RSTD measurements	360

9.9.2.1	Introduction	360

9.9.2.2	Requirements Applicability	360

9.9.2.3	Measurement Capability	360

9.9.2.4	Measurement Reporting Requirements	360

9.9.2.4.1	Void	360

9.9.2.4.2	Void	360

9.9.2.4.3	Void	360

9.9.2.5	Measurements Period Requirements	360

9.9.2.6	Void	362

9.9.3	PRS-RSRP measurements	362

9.9.3.1	Introduction	362

9.9.3.2	Requirements applicability	362

9.9.3.3	Measurement Capability	362

9.9.3.4	Measurement Reporting Requirements	363

9.9.3.5	Measurement Period Requirements	363

9.9.4	UE Rx-Tx time difference measurements	365

9.9.4.1 Introduction	365

9.9.4.2 Requirements Applicability	365

9.9.4.3 Measurement Capability	365

9.9.4.4 Measurement Reporting Requirements	365

9.9.4.5	Measurement Period Requirements	365

9.9.5	NR E-CID measurements	368

9.9.5.1	Introduction	368

9.9.5.2	Measurement Requirements	368

9.9.5.2.1	Intra-frequency Measurement Requirements	368

9.9.5.2.2	Inter-frequency Measurement Requirements	368

9.9.5.2.3	Measurement Reporting Delay	369

9.10	CSI-RS based L3 measurements	369

9.10.1	Introduction	369

9.10.2	CSI-RS based intra-frequency measurements	369

9.10.2.1	Introduction	369

9.10.2.2	Requirements applicability	370

9.10.2.3	Number of cells and number of CSI-RS	371

9.10.2.3.1	Requirements for FR1	371

9.10.2.3.2	Requirements for FR2	371

9.10.2.4	Measurement Reporting Requirements	371

9.10.2.4.1	Periodic Reporting	371

9.10.2.4.2	Event-triggered Periodic Reporting	372

9.10.2.4.3	Event Triggered Reporting	372

9.10.2.5	Intra-frequency measurements without measurement gaps	372

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	373

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	373

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	374

9.10.3	CSI-RS based Inter-frequency measurements	374

9.10.3.1	Introduction	374

9.10.3.2	Requirements applicability	374

9.10.3.3	Number of cells and number of CSI-RS resources	375

9.10.3.3.1	Requirements for FR1	375

9.10.3.3.2	Requirements for FR2	375

9.10.3.4	Measurements reporting requirements	375

9.10.3.4.1	Periodic Reporting	375

9.10.3.4.2	Event-triggered Periodic Reporting	375

9.10.3.4.3	Event-triggered Reporting	376

9.10.3.5	Inter frequency measurements with measurement gaps	376

9.11	NR measurements with autonomous gaps	377

9.11.1	Introduction	377

9.11.2	CGI identification of an NR cell with autonomous gaps	377

9.11.3	CGI reporting delay	378

10	Measurement Performance requirements	378

10.1	NR measurements	378

10.1.1	Introduction	378

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	379

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	379

10.1.2.1.1	Absolute SS-RSRP Accuracy	379

10.1.2.1.2	Relative SS-RSRP Accuracy	380

10.1.2.2	Void	381

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	381

10.1.2.3.1	Absolute CSI-RSRP Accuracy	381

10.1.2.3.2	Relative CSI-RSRP Accuracy	382

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	383

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	383

10.1.2B.1.1	Absolute SS-RSRP Accuracy	383

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	384

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	384

10.1.3.1.1	Absolute SS-RSRP Accuracy	384

10.1.3.1.2	Relative SS-RSRP Accuracy	385

10.1.3.2	Void	386

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	386

10.1.3.3.1	Absolute CSI-RSRP Accuracy	386

10.1.3.3.2	Relative CSI-RSRP Accuracy	386

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	387

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	387

10.1.3B.1.1	Absolute SS-RSRP Accuracy	388

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	388

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	388

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	388

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	389

10.1.4.2	Void	390

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	390

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	390

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	391

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	392

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	392

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	392

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	393

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	393

10.1.5.1.1	Absolute SS-RSRP Accuracy	393

10.1.5.1.2	Relative SS-RSRP Accuracy	394

10.1.5.2	Void	395

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	395

10.1.5.3.1	Absolute CSI-RSRP Accuracy	395

10.1.5.3.2	Relative CSI-RSRP Accuracy	395

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	396

10.1.6 RSRP Measurement Report Mapping	397

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	399

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	399

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	399

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	400

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	400

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	401

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	401

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	401

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	402

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	402

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	402

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	403

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	403

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	404

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	404

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	404

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	405

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	405

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	405

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	406

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	407

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	407

10.1.9.2.2	Relative CSI-RSRQ Accuracy	408

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	409

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	409

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	409

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	410

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	412

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	412

10.1.10.2.2	Relative CSI-RSRQ Accuracy	412

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	413

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	413

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	413

10.1.11	RSRQ report mapping	414

10.1.12	Intra-frequency SINR accuracy requirements for FR1	415

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	415

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	415

10.1.13	Intra-frequency SINR accuracy requirements for FR2	416

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	417

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	417

10.1.14	Inter-frequency SINR accuracy requirements for FR1	418

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	420

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	420

10.1.15	Inter-frequency SINR accuracy requirements for FR2	422

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	424

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	424

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	424

10.1.16	SINR report mapping	425

10.1.17	Power Headroom	426

10.1.18	PCMAX,c,f	427

10.1.19	L1-RSRP accuracy requirements for FR1	427

10.1.20	L1-RSRP accuracy requirements for FR2	431

10.1.21	SFTD accuracy requirements	434

10.1.22	CLI measurement accuracy requirements	438

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	442

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	442

10.1.23	RSTD Measurements	444

10.1.23.1	Introduction	444

10.1.23.2	Measurement Accuracy Requirements	445

10.1.23.3	Report mapping	449

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	449

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	451

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	453

10.1.24	PRS-RSRP Measurements	455

10.1.24.1	Introduction	455

10.1.24.2	Measurement Accuracy Requirements	455

10.1.24.2.1 Absolute PRS RSRP accuracy	455

10.1.24.3	Report mapping	459

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	459

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	460

10.1.25	UE Rx-Tx Time Difference Measurements	462

10.1.25.1	Introduction	462

10.1.25.2	Measurement Accuracy Requirements	463

10.1.25.3	Report mapping	469

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	469

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	471

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	472

10.1.26	FR2 P-MPR report	474

10.1.26.1	Report mapping	474

10.1.27	L1-SINR accuracy requirements for FR1	474

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	474

10.1.27.1.1	Absolute Accuracy	474

10.1.27.1.2	Relative Accuracy	475

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	476

10.1.27.2.1	Absolute Accuracy	476

10.1.27.2.2	Relative Accuracy	477

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	479

10.1.27.3.1	Absolute Accuracy	479

10.1.27.3.2	Relative Accuracy	481

10.1.28	L1-SINR accuracy requirements for FR2	482

10.1.28.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	482

10.1.28.1.1	Absolute Accuracy	482

10.1.28.1.2	Relative Accuracy	483

10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	484

10.1.28.2.1	Absolute Accuracy	484

10.1.28.2.2	Relative Accuracy	485

10.1.28.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	486

10.1.28.3.1	Absolute Accuracy	486

10.1.28.3.2	Relative Accuracy	487

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	489

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	489

10.1.29.1.1	Absolute SS-RSRQ Accuracy	489

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	489

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	489

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	489

10.1.30.1.2	Relative Accuracy of SS-RSRQ	490

10.1.31	Intra-frequency SINR accuracy requirements under CCA	491

10.1.32	Inter-frequency SINR accuracy requirements under CCA	491

10.1.33	L1-RSRP accuracy requirements under CCA	493

10.1.34	RSSI measurements under CCA	494

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	494

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	494

10.1.34.3	RSSI measurement report mapping	495

10.1.35	Channel occupancy measurements under CCA	495

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	495

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	495

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	496

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	496

10.1.36.1.1	Absolute SS-RSRP Accuracy	496

10.1.36.1.2	Relative SS-RSRP Accuracy	496

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	497

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	497

10.1.37.1.1	Absolute Accuracy of SS-RSRP	497

10.1.37.1.2	Relative Accuracy of SS-RSRP	498

10.2	E-UTRAN measurements	498

10.2.1	Introduction	498

10.2.2	E-UTRAN RSRP measurements	499

10.2.3	E-UTRAN RSRQ measurements	499

10.2.4	E-UTRAN RSTD measurements	499

10.2.5	E-UTRAN RS-SINR measurements	499

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	499

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	500

10.3	UTRAN FDD Measurements	500

10.3.1	UTRAN FDD CPICH RSCP	500

10.3.2	UTRAN FDD CPICH Ec/No	501

10.4	V2X measurements	501

10.4.1	Introduction	501

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	502

10.4.2.1	PSBCH-RSRP Absolute Accuracy	502

10.4.2.2	PSBCH-RSRP Relative Accuracy	502

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	503

10.4.3.1	Absolute SL-RSSI Accuracy	503

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	503

10.4.4.1	Absolute L1 SL-RSRP Accuracy	503

11	Void	504

12	V2X Requirements	504

12.1	Introduction	504

12.2	UE Transmit Timing	505

12.2.1	Introduction	505

12.2.2	GNSS as synchronization reference source	505

12.2.3	NR Cell as synchronization reference source	505

12.2.4	E-URTAN Cell as synchronization reference source	506

12.2.5	SyncRef UE as synchronization reference source	506

12.3	Initiation/Cease of SLSS Transmissions	506

12.3.1	Introduction	506

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	506

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	507

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	508

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	508

12.4	Selection / Reselection of V2X Synchronization Reference Source	509

12.5	L1 SL-RSRP measurements	510

12.5.1	Introduction	510

12.5.2	SL-RSRP measurements	510

12.6	Congestion Control measurements	510

12.7	Interruption	510

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	510

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	511

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	513

12.8	Reliability of GNSS signal	513

12.9	Scheduling availability	513

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	513

13	Measurement Performance Requirements for NR gNB	514

13.1	UL-RTOA	514

13.1.1	Report mapping	514

13.2	gNB Rx-Tx time difference	516

13.2.1	Report mapping	516

13.2.2	Measurement Accuracy Requirements	518

13.2.2.1	Introduction	518

13.2.2.2	Requirements	518

13.3	UL SRS RSRP measurement	519

13.3.1	Report mapping	519

13.3.2	Measurement accuracy requirements	520

13.3.2.1	Introduction	520

13.3.2.2	Requirements	520

13.4	AoA/ZoA	521

13.4.1	Report mapping	521

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1A	Reference measurement channels under CCA	429

A.3.1A.1	PDSCH	429

A.3.1A.1.1	TDD	429

A.3.1A.2	CORESET for RMSI scheduling	430

A.3.1A.2.1	TDD	430

A.3.1A.3	CORESET for RMC scheduling	431

A.3.1A.3.1	TDD	431

A.3.1A.4	TDD UL/DL configuration	432

A.3.1A.5	RMC burst transmission model	432

A.3.1.2	CORESET for RMSI scheduling	433

A.3.1.2.1	FDD	433

A.3.1.2.2	TDD	434

A.3.1.3	CORESET for RMC scheduling	437

A.3.1.3.1	FDD	437

A.3.1.3.2	TDD	438

A.3.1.4	TDD UL/DL configuration	441

A.3.2	OFDMA channel noise generator (OCNG)	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	442

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	443

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	445

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	445

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	446

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	446

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	447

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	448

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	448

A.3.4	Test Cases with Different Channel Bandwidths	448

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	448

A.3.4.1.1	Introduction	448

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	449

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	449

A.3.5.1.1	Introduction	449

A.3.5.1.2	Principle of Testing	449

A.3.6	Antenna configurations	449

A.3.6.1	Antenna configurations for FR1	449

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	449

A.3.6.1.1.1	Introduction	449

A.3.6.1.1.2	Principle of testing	449

A.3.6.2	Antenna configurations for FR2	452

A.3.6A	Antenna configurations with unlicensed bands	452

A.3.6A.1	Antenna configurations for FR1	452

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	452

A.3.6A.1.1.1	Introduction	452

A.3.6A.1.1.2	Principle of testing	452

A.3.7	EN-DC test setup	454

A.3.7.1	Introduction	454

A.3.7.2	E-UTRAN Serving Cell Parameters	454

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	454

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	455

A.3.7A	NR FR1-FR2 test setup	456

A.3.7B	EN-DC test setup with unlicensed bands	457

A.3.7B.1	Introduction	457

A.3.7B.2	E-UTRAN Serving Cell Parameters	457

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	457

A.3.7C	LTE-FR1/FR2 test setup	458

A.3.7D	NE-DC test setup	458

A.3.7D.1	Introduction	458

A.3.7D.2	E-UTRAN Serving Cell Parameters	458

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	458

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	458

A.3.8	PRACH configurations	458

A.3.8.1	Introduction	458

A.3.8.2	PRACH configurations in FR1	459

A.3.8.2.1	FR1 PRACH configuration 1	459

A.3.8.2.2	FR1 PRACH configuration 2	459

A.3.8.2.3	FR1 PRACH configuration 3	460

A.3.8.2.4	FR1 PRACH configuration 4	461

A.3.8.3	PRACH configurations in FR2	462

A.3.8.3.1	FR2 PRACH configuration 1	462

A.3.8.3.2	FR2 PRACH configuration 2	463

A.3.8.3.3	FR2 PRACH configuration 3	464

A.3.8.3.4	FR2 PRACH configuration 4	465

A.3.8A	PRACH configurations under CCA	466

A.3.8A.1	Introduction	466

A.3.8A.2	PRACH configurations in FR1	466

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	466

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	467

A.3.9	BWP configurations	468

A.3.9.1	Introduction	468

A.3.9.2	Downlink BWP configurations	469

A.3.9.2.1	Initial BWP	469

A.3.9.2.2	Dedicated BWP	469

A.3.9.3	Uplink BWP configurations	469

A.3.9.3.1	Initial BWP	469

A.3.9.3.2	Dedicated BWP	470

A.3.10	SSB Configurations	470

A.3.10.1	SSB Configurations for FR1	470

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	470

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	472

A.3.10A	SSB Configurations under CCA	477

A.3.10A.1	SSB Configurations under CCA for FR1	477

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	477

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	478

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.11	SMTC Configurations	479

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	479

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	479

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	480

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.12	Test Cases with Different CC Configurations	480

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	480

A.3.12.1.1	Introduction	480

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	481

A.3.12.2.1	Introduction	481

A.3.12.2.2	Principle of testing	481

A.3.13	Test Cases in SA and EN-DC Operations	481

A.3.13.1	Introduction	481

A.3.13.2	Principle of Testing	481

A.3.13A Test Cases involving E-UTRA/FR1 and FR2 carriers	482

A.3.13A.1	Introduction	482

A.3.13A.2	Principle of Testing in EN-DC	482

A.3.13A.3	Principle of Testing in SA	482

A.3.13B	Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.1.1	Introduction	483

A.3.13B.1.2	Principle of Testing	483

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.2.1	Introduction	483

A.3.13B.2.2	Principle of Testing	483

A.3.14	CSI-RS configurations	484

A.3.14.1	FDD	484

A.3.14.2	TDD	486

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	491

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	491

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	491

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	491

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	492

A.3.15.3	Setup 3: 2 AoAs	492

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	492

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	492

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	492

A.3.16	TCI State Configuration	493

A.3.16.1	Introduction	493

A.3.16.2	TCI states	493

A.3.17	Configurations of CSI-RS for tracking	493

A.3.17.1	Configuration of CSI-RS for tracking for FR1	493

A.3.17.1.1	FDD	493

A.3.17.1.2	TDD	494

A.3.17.2	Configuration of CSI-RS for tracking for FR2	495

A.3.17.2.1	TDD	495

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	496

A.3.18.1	Introduction	496

A.3.18.2	PRACH Power Measurement	496

A.3.19	Test applicability for DAPS handover	496

A.3.19.1	Introduction	496

A.3.19.2	Principle of testing	496

A.3.20	MsgA configurations	497

A.3.20.1	Introduction	497

A.3.20.2	MsgA configurations in FR1	497

A.3.20.2.1	FR1 MsgA configuration 1	497

A.3.20.2.2	FR1 MsgA configuration 2	498

A.3.20.3	MsgA configurations in FR2	500

A.3.20.3.1	FR2 MsgA configuration 1	500

A.3.20.3.2	FR2 MsgA configuration 2	501

A.3.20A	MsgA configurations under CCA	503

A.3.20A.1	Introduction	503

A.3.20A.2	MsgA configurations in FR1	503

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	503

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	504

A.3.21	V2X sidelink communication	507

A.3.21.1	Introduction	507

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	507

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	511

A.3.22	CSI-IM configurations	511

A.3.22.1	FDD	511

A.3.22.2	TDD	512

A.3.23	Spatial Relation Configuration	514

A.3.23.1	Introduction	514

A.3.23.2	Spatial Relation	514

A.3.24	SRS configuration	515

A.3.25	Channel bandwidth (CBW) configurations	517

A.3.25.1	DL UE specific CBW	517

A.3.25.2	UL UE specific CBW	518

A.3.26	CCA model	518

A.3.26.1	Introduction	518

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	518

A.3.26.2.1	DL CCA model	518

A.3.26.2.2	UL CCA model	519

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	520

A.3.27.1	Introduction	520

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	520

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	520

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	520

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	521

A.3.28	Discovery Burst Transmission Window configuration under CCA	521

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	521

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	521

A.3.29.1	Introduction	521

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	521

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	522

A.3.30	CSI-RS configurations for RRM	522

A.3.30.1	FDD	522

A.3.30.2	TDD	523

A.3.31	PRS Configurations	525

A.3.31.1.	PRS Configurations for FR1	525

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	525

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	526

A.3.31.2.	PRS Configurations for FR2	526

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	526

A.4	EN-DC tests with all NR cells in FR1	527

A.4.1	Void	527

A.4.2	Void	527

A.4.3	RRC_CONNECTED state mobility	527

A.4.3.1	Void	527

A.4.3.2	RRC Connection Mobility Control	527

A.4.3.2.1	Void	527

A.4.3.2.2	Random Access	527

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	527

A.4.3.2.2.2	4-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	531

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	535

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	539

A.4.3.2.3	Void	543

A.4.4	Timing	543

A.4.4.1	UE transmit timing	543

A.4.4.1.1	NR UE Transmit Timing Test for FR1	543

A.4.4.1.1.1	Test Purpose and environment	543

A.4.4.1.1.2	Test requirements	547

A.4.4.2	UE timer accuracy	547

A.4.4.3	Timing advance	547

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	547

A.4.4.3.1.1	Test Purpose and Environment	547

A.4.4.3.1.2	Test Parameters	547

A.4.4.3.1.3	Test Requirements	551

A.4.5	Signaling characteristics	551

A.4.5.1	Radio link Monitoring	551

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	552

A.4.5.1.1.1	Test Purpose and Environment	552

A.4.5.1.1.2	Test Requirements	556

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	556

A.4.5.1.2.1	Test Purpose and Environment	556

A.4.5.1.2.2	Test Requirements	562

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	562

A.4.5.1.3.1	Test Purpose and Environment	562

A.4.5.1.3.2	Test Requirements	568

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	568

A.4.5.1.4.1	Test Purpose and Environment	568

A.4.5.1.4.2	Test Requirements	574

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	574

A.4.5.1.5.1	Test Purpose and Environment	574

A.4.5.1.5.2	Test Requirements	580

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	580

A.4.5.1.6.1	Test Purpose and Environment	580

A.4.5.1.6.2	Test Requirements	586

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	586

A.4.5.1.7.1	Test Purpose and Environment	586

A.4.5.1.7.2	Test Requirements	591

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	592

A.4.5.1.8.1	Test Purpose and Environment	592

A.4.5.1.8.2	Test Requirements	597

A.4.5.2	Interruption	597

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	597

A.4.5.2.1.1	Test Purpose and Environment	597

A.4.5.2.1.2	Test Requirements	601

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	601

A.4.5.2.2.1	Test Purpose and Environment	601

A.4.5.2.2.2	Test Requirements	605

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	605

A.4.5.2.3.1	Test Purpose and Environment	605

A.4.5.2.3.2	Test Requirements	610

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	611

A.4.5.2.4.1	Test Purpose and Environment	611

A.4.5.2.4.2	Test Requirements	615

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	616

A.4.5.2.5.1	Test Purpose and Environment	616

A.4.5.2.5.2	Test Requirements	620

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	620

A.4.5.2.6.1	Test Purpose and Environment	620

A.4.5.2.6.2	Test Requirements	624

A.4.5.2.7	Void	624

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	624

A.4.5.2.8.1	Test Purpose and Environment	624

A.4.5.2.8.2	Test Requirements	627

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	628

A.4.5.2.9.1	Test Purpose and Environment	628

A.4.5.2.9.2	Test Requirements	632

A.4.5.3	SCell Activation and Deactivation Delay	633

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	633

A.4.5.3.1.1	Test Purpose and Environment	633

A.4.5.3.1.2	Test Requirements	639

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	640

A.4.5.3.2.1	Test Purpose and Environment	640

A.4.5.3.2.2	Test Requirements	640

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	640

A.4.5.3.3.1	Test Purpose and Environment	640

A.4.5.3.3.2	Test Requirements	641

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	642

A.4.5.3.4.1	Test Purpose and Environment	642

A.4.5.3.4.2	Test Requirements	646

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	646

A.4.5.3.5.1	Test Purpose and Environment	646

A.4.5.3.5.2	Test Requirements	653

A.4.5.4	UE UL carrier RRC reconfiguration Delay	653

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	653

A.4.5.4.1.1 Test Purpose and Environment	653

A.4.5.4.1.2	Test Requirements	661

A.4.5.5	Beam Failure Detection and Link recovery procedures	662

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	662

A.4.5.5.1.1	Test Purpose and Environment	662

A.4.5.5.1.2	Test Requirements	667

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	667

A.4.5.5.2.1	Test Purpose and Environment	667

A.4.5.5.2.2	Test Requirements	673

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	673

A.4.5.5.3.1	Test Purpose and Environment	673

A.4.5.5.3.2	Test Requirements	679

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	679

A.4.5.5.4.1	Test Purpose and Environment	679

A.4.5.5.4.2	Test Requirements	685

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	685

A.4.5.5.5.1	Test Purpose and Environment	685

A.4.5.5.5.2	Test Requirements	691

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	691

A.4.5.5.6.1	Test Purpose and Environment	691

A.4.5.5.6.2	Test Requirements	698

A.4.5.6	Active BWP switch	699

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	699

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	699

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	704

A.4.5.6.2	RRC-based Active BWP Switch	710

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	714

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	714

A.4.5.6.3.1.1	Test Purpose and Environment	714

A.4.5.6.3.1.2	Test Requirements	720

A.4.5.6.4	SCell dormancy switch	720

A.4.5.6.4.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	720

A.4.5.6.4.1.1	Test Purpose and Environment	720

A.4.5.6.4.1.2	Test Requirements	728

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	728

A.4.5.6.4.2.1	Test Purpose and Environment	728

A.4.5.6.4.2.2	Test Requirements	740

A.4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	741

A.4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	741

A.4.5.6.5.1.1	Test Purpose and Environment	741

A.4.5.6.5.1.2	Test Requirements	745

A.4.5.7	PSCell addition and release delay	745

A.4.5.7.1	Addition and Release Delay of known NR PSCell	745

A.4.5.7.1.1	Test purpose and environment	745

A.4.5.7.1.2	Test Requirements	749

A.4.5.8	DL Interruptions at switching between two uplink carriers	750

A.4.5.8.1	Test Purpose and Environment	750

A.4.5.8.2	Test Requirements	753

A.4.5.9	UE specific CBW change	754

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	754

A.4.5.9.1.1	Test Purpose and Environment	754

A.4.5.9.1.2	Test Requirements	758

A.4.6	Measurement procedure	758

A.4.6.1	Intra-frequency Measurements	758

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	758

A.4.6.1.1.1	Test purpose and Environment	758

A.4.6.1.1.2	Test parameters	758

A.4.6.1.1.3	Test Requirements	762

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	762

A.4.6.1.2.1	Test purpose and Environment	762

A.4.6.1.2.2	Test parameters	762

A.4.6.1.2.2	Test Requirements	766

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	766

A.4.6.1.3.1	Test purpose and Environment	766

A.4.6.1.3.2	Test parameters	766

A.4.6.1.3.3	Test Requirements	770

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	770

A.4.6.1.4.1	Test purpose and Environment	770

A.4.6.1.4.2	Test parameters	770

A.4.6.1.4.3	Test Requirements	774

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	774

A.4.6.1.5.1	Test purpose and Environment	774

A.4.6.1.5.2	Test parameters	774

A.4.6.1.5.3	Test Requirements	776

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	777

A.4.6.1.6.1	Test purpose and Environment	777

A.4.6.1.6.2	Test parameters	777

A.4.6.1.6.3	Test Requirements	779

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	780

A.4.6.1.7.1	Test purpose and Environment	780

A.4.6.1.7.2	Test parameters	780

A.4.6.1.7.3	Test Requirements	784

A.4.6.2	Inter-frequency Measurements	784

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	784

A.4.6.2.1.1	Test Purpose and Environment	784

A.4.6.2.1.2	Test Requirements	788

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	788

A.4.6.2.2.1	Test Purpose and Environment	788

A.4.6.2.2.2	Test Requirements	792

A.4.6.2.3	Void	793

A.4.6.2.4	Void	793

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	793

A.4.6.2.5.1	Test Purpose and Environment	793

A.4.6.2.5.2	Test Requirements	797

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	798

A.4.6.2.6.1	Test Purpose and Environment	798

A.4.6.2.6.2	Test Requirements	802

A.4.6.2.7	Void	803

A.4.6.2.8	Void	803

A.4.6.3	Void	803

A.4.6.4	L1-RSRP measurement for beam reporting	803

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	803

A.4.6.4.1.1	Test Purpose and Environment	803

A.4.6.4.1.2	Test parameters	803

A.4.6.4.1.3	Test Requirements	806

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	806

A.4.6.4.2.1	Test Purpose and Environment	806

A.4.6.4.2.2	Test parameters	807

A.4.6.4.2.3	Test Requirements	810

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	810

A.4.6.4.3.1	Test Purpose and Environment	810

A.4.6.4.3.2	Test parameters	811

A.4.6.4.3.3	Test Requirements	814

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	814

A.4.6.4.4.1	Test Purpose and Environment	814

A.4.6.4.4.2	Test parameters	815

A.4.6.4.4.3	Test Requirements	817

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	817

A.4.6.4.5.1	Test Purpose and Environment	817

A.4.6.4.5.2	Test parameters	818

A.4.6.4.5.3	Test Requirements	821

A.4.6.5	CLI measurements	821

A.4.6.5.1	SRS-RSRP measurement with non-DRX	821

A.4.6.5.1.1	Test Purpose and Environment	821

A.4.6.5.1.2	Test Parameters	822

A.4.6.5.1.3	Test Requirements	825

A.4.6.5.2	CLI-RSSI measurement with non-DRX	825

A.4.6.5.2.1	Test Purpose and Environment	825

A.4.6.5.2.2	Test Parameters	826

A.4.6.5.2.3	Test Requirements	827

A.4.6.6	Measurements with autonomous gaps	828

A.4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	828

A.4.6.6.1.1	Test Purpose and Environment	828

A.4.6.6.1.2	Test Requirements	832

A.4.6.7	L1-SINR measurement for beam reporting	832

A.4.6.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR when DRX is not used	832

A.4.6.7.1.1	Test Purpose and Environment	832

A.4.6.7.1.2	Test parameters	833

A.4.6.7.1.3	Test Requirements	836

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	836

A.4.6.7.2.1	Test Purpose and Environment	836

A.4.6.7.2.2	Test parameters	837

A.4.6.7.2.3	Test Requirements	840

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	840

A.4.6.7.3.1	Test Purpose and Environment	840

A.4.6.7.3.2	Test parameters	841

A.4.6.7.3.3	Test Requirements	844

A.4.6.8	CSI-RS based intra-frequency Measurement	844

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	844

A.4.6.8.1.1	Test purpose and Environment	844

A.4.6.8.1.2	Test Requirements	847

A.4.6.9	CSI-RS based inter-frequency Measurement	848

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	848

A.4.6.9.1.1	Test Purpose and Environment	848

A.4.6.9.1.2	Test Requirements	851

A.4.7	Measurement Performance requirements	852

A.4.7.1	SS-RSRP	852

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	852

A.4.7.1.1.1	Test Purpose and Environment	852

A.4.7.1.1.2	Test parameters	852

A.4.7.1.1.3	Test Requirements	856

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	856

A.4.7.1.2.1	Test Purpose and Environment	856

A.4.7.1.2.2	Test parameters	857

A.4.7.1.2.3	Test Requirements	861

A.4.7.1.3	Void	861

A.4.7.2	SS-RSRQ	861

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	861

A.4.7.2.1.1	Test Purpose and Environment	861

A.4.7.2.1.2	Test Parameters	862

A.4.7.2.1.3	Test Requirements	867

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	867

A.4.7.2.2.1	Test Purpose and Environment	867

A.4.7.2.2.2	Test Parameters	867

A.4.7.2.2.3	Test Requirements	872

A.4.7.3	SS-SINR	872

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	872

A.4.7.3.1.1	Test Purpose and Environment	872

A.4.7.3.1.2	Test Parameters	872

A.4.7.3.1.3	Test Requirements	876

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	876

A.4.7.3.2.1	Test Purpose and Environment	876

A.4.7.3.2.2	Test Parameters	876

A.4.7.3.2.3	Test Requirements	881

A.4.7.4	L1-RSRP measurement for beam reporting	881

A.4.7.4.1	SSB based L1-RSRP measurement	881

A.4.7.4.1.1	Test Purpose and Environment	881

A.4.7.4.1.2	Test parameters	882

A.4.7.4.1.3	Test Requirements	885

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	885

A.4.7.4.2.1	Test Purpose and Environment	885

A.4.7.4.2.2	Test parameters	886

A.4.7.4.2.3	Test Requirements	889

A.4.7.5	SFTD accuracy	889

A.4.7.5.1	SFTD accuracy	889

A.4.7.5.1.1	Test Purpose and Environment	889

A.4.7.5.1.2	Test Parameters	889

A.4.7.5.1.3	Test Requirements	894

A.4.7.5.2	Void	894

A.4.7.5.3	Void	894

A.4.7.6	CLI measurements	894

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	894

A.4.7.6.1.1	Test Purpose and Environment	894

A.4.7.6.1.2	Test parameters	895

A.4.7.6.1.3	Test Requirements	900

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	900

A.4.7.6.2.1	Test Purpose and Environment	900

A.4.7.6.2.2	Test parameters	901

A.4.7.6.2.3	Test Requirements	904

A.4.7.7	L1-SINR measurement for beam reporting	904

A.4.7.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	904

A.4.7.7.1.1	Test Purpose and Environment	904

A.4.7.7.1.2	Test parameters	905

A.4.7.7.1.3	Test Requirements	909

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	909

A.4.7.7.2.1	Test Purpose and Environment	909

A.4.7.7.2.2	Test parameters	910

A.4.7.7.2.3	Test Requirements	914

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	914

A.4.7.7.3.1	Test Purpose and Environment	914

A.4.7.7.3.2	Test parameters	915

A.4.7.7.3.3	Test Requirements	918

A.4.7.8	CSI-RSRP	918

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	918

A.4.7.8.1.1	Test Purpose and Environment	918

A.4.7.8.1.2	Test parameters	919

A.4.7.8.1.3	Test Requirements	922

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	922

A.4.7.8.2.1	Test Purpose and Environment	922

A.4.7.8.2.2	Test parameters	923

A.4.7.8.2.3	Test Requirements	926

A.4.7.9	CSI-RSRQ	927

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	927

A.4.7.9.1.1	Test Purpose and Environment	927

A.4.7.9.1.2	Test Parameters	927

A.4.7.9.1.3	Test Requirements	932

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	932

A.4.7.9.2.1	Test Purpose and Environment	932

A.4.7.9.2.2	Test Parameters	932

A.4.7.2.2.3	Test Requirements	937

A.4.7.10	CSI-SINR	937

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	937

A.4.7.10.1.1	Test Purpose and Environment	937

A.4.7.10.1.2	Test Parameters	938

A.4.7.10.1.3	Test Requirements	941

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	941

A.4.7.10.2.1	Test Purpose and Environment	941

A.4.7.10.2.2	Test Parameters	941

A.4.7.10.2.3	Test Requirements	944

A.4.8	Void	945

A.4A	NE-DC test with all NR cells in FR1	945

A.4A.1	Signaling characteristics	945

A.4A.1.1	E-UTRAN PSCell addition	945

A.4A.1.1.1	Test purpose and environment	945

A.4A.1.1.2	Test Requirements	949

A.4A.1.2	Active BWP switch	950

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	950

A.4A.1.2.1.1	Test Purpose and Environment	950

A.4A.1.2.1.2	Test Requirements	954

A.4A.2	Measurement performance	955

A.4A.2.1	SFTD accuracy	955

A.4A.2.1.1	SFTD accuracy	955

A.4A.2.1.1.1	Test Purpose	955

A.4A.2.1.1.2	Test Environment	955

A.4A.2.1.1.3	Test Requirements	958

A.5	EN-DC tests with one or more NR cells in FR2	959

A.5.1	Void	959

A.5.2	Void	959

A.5.3	RRC_CONNECTED state mobility	959

A.5.3.1	Void	959

A.5.3.2	RRC Connection Mobility Control	959

A.5.3.2.1	Void	959

A.5.3.2.2	Random Access	959

A.5.3.2.2.1	4-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	959

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	962

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	968

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	971

A.5.3.2.3	Void	975

A.5.4	Timing	975

A.5.4.1	UE transmit timing	975

A.5.4.1.1	NR UE Transmit Timing Test for FR2	975

A.5.4.1.1.1	Test Purpose and environment	975

A.5.4.1.1.2	Test requirements	978

A.5.4.2	UE timer accuracy	979

A.5.4.3	Timing advance	979

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	979

A.5.4.3.1.1 Test Purpose and Environment	979

A.5.4.3.1.2 Test Parameters	979

A.5.4.3.1.3	Test Requirements	983

A.5.5	Signaling characteristics	983

A.5.5.1	Radio link Monitoring	983

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	983

A.5.5.1.1.1	Test Purpose and Environment	983

A.5.5.1.1.2	Test Requirements	987

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	988

A.5.5.1.2.1	Test Purpose and Environment	988

A.5.5.1.2.2	Test Requirements	991

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	992

A.5.5.1.3.1	Test Purpose and Environment	992

A.5.5.1.3.2	Test Requirements	996

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	996

A.5.5.1.4.1	Test Purpose and Environment	996

A.5.5.1.4.2	Test Requirements	1000

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1000

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1004

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1008

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1013

A.5.5.1.8.2	Test Requirements	1017

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1017

A.5.5.1.9.1	Test Purpose and Environment	1017

A.5.5.1.9.2	Test Requirements	1020

A.5.5.2	Interruption	1020

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1020

A.5.5.2.1.1	Test Purpose and Environment	1020

A.5.5.2.1.2	Test Requirements	1023

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1023

A.5.5.2.2.1	Test Purpose and Environment	1023

A.5.5.2.2.2	Test Requirements	1026

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1026

A.5.5.2.3.1	Test Purpose and Environment	1026

A.5.5.2.3.2	Test Requirements	1030

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1030

A.5.5.2.4.1	Test Purpose and Environment	1030

A.5.5.2.4.2	Test Requirements	1033

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1034

A.5.5.2.5.1	Test Purpose and Environment	1034

A.5.5.2.5.2	Test Requirements	1037

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1038

A.5.5.2.6.1	Test Purpose and Environment	1038

A.5.5.2.6.2	Test Requirements	1040

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1041

A.5.5.2.7.1	Test Purpose and Environment	1041

A.5.5.2.7.2	Test Requirements	1044

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1044

A.5.5.2.8.1 Test Purpose and Environment	1044

A.5.5.2.8.3	Test Requirements	1046

A.5.5.3	SCell Activation and Deactivation Delay	1046

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1046

A.5.5.3.1.1	Test Purpose and Environment	1046

A.5.5.3.1.2	Test Requirements	1049

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1049

A.5.5.3.2.1	Test Purpose and Environment	1049

A.5.5.3.2.2	Test Requirements	1053

A.5.5.3.3	Void	1053

A.5.5.3.4	Void	1053

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1053

A.5.5.3.5.1	Test Purpose and Environment	1053

A.5.5.3.5.2	Test Requirements	1056

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1057

A.5.5.3.6.1	Test Purpose and Environment	1057

A.5.5.3.6.2	Test Requirements	1061

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1062

A.5.5.3.7.1	Test Purpose and Environment	1062

A.5.5.3.7.2	Test Requirements	1065

A.5.5.4	Void	1065

A.5.5.5	Beam Failure Detection and Link recovery procedures	1065

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1065

A.5.5.5.1.1	Test Purpose and Environment	1065

A.5.5.5.1.2	Test Requirements	1069

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1069

A.5.5.5.2.1	Test Purpose and Environment	1069

A.5.5.5.2.2	Test Requirements	1073

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1073

A.5.5.5.3.1	Test Purpose and Environment	1073

A.5.5.5.3.2	Test Requirements	1077

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1077

A.5.5.5.4.1	Test Purpose and Environment	1077

A.5.5.5.4.2	Test Requirements	1081

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1081

A.5.5.5.5.1	Test Purpose and Environment	1081

A.5.5.5.5.2	Test Requirements	1085

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1085

A.5.5.5.6.1	Test Purpose and Environment	1085

A.5.5.5.6.2	Test Requirements	1089

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1089

A.5.5.5.7.1	Test Purpose and Environment	1089

A.5.5.5.7.2	Test Requirements	1093

A.5.5.6	Active BWP switch	1093

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1093

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1093

A.5.5.6.1.1.1	Test Purpose and Environment	1093

A.5.5.6.1.1.2	Test Requirements	1097

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1097

A.5.5.6.2	RRC-based Active BWP Switch	1102

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1106

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1106

A.5.5.6.3.1.1	Test Purpose and Environment	1106

A.5.5.6.3.1.2	Test Requirements	1110

A.5.5.6.4	SCell dormancy switch	1110

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1110

A.5.5.6.4.1.1	Test Purpose and Environment	1110

A.5.5.6.4.1.2	Test Requirements	1114

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1114

A.5.5.6.4.2.1	Test Purpose and Environment	1114

A.5.5.6.4.2.2	Test Requirements	1121

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1121

A.5.5.7	PSCell addition and release delay	1124

A.5.5.7.1	Addition and Release Delay of NR PSCell	1124

A.5.5.7.1.1	Test purpose and environment	1124

A.5.5.7.1.2	Test Requirements	1128

A.5.5.8	Active TCI state switch delay	1129

A.5.5.8.1	MAC-CE based active TCI state switch	1129

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1129

A.5.5.8.1.1.1	Test Purpose and Environment	1129

A.5.5.8.1.1.2	Test Requirements	1132

A.5.5.8.2	RRC based active TCI state switch	1132

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1132

A.5.5.8.2.1.1	Test Purpose and Environment	1132

A.5.5.8.2.1.2	Test Requirements	1136

A.5.5.9	Uplink spatial relation switch delay	1136

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1136

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1136

A.5.5.9.1.1.1	Test Purpose and Environment	1136

A.5.5.9.1.1.2	Test Requirements	1139

A.5.5.9.2	RRC based spatial relation switch	1139

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1139

A.5.5.9.2.1.1	Test Purpose and Environment	1139

A.5.5.9.2.1.2	Test Requirements	1142

A.5.5.10	UE specific CBW change	1142

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1142

A.5.5.10.1.1	Test Purpose and Environment	1142

A.5.5.10.1.2	Test Requirements	1145

A.5.6	Measurement procedure	1146

A.5.6.1	Intra-frequency Measurements	1146

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1146

A.5.6.1.1.1	Test purpose and Environment	1146

A.5.6.1.1.2	Test Requirements	1149

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1149

A.5.6.1.2.1	Test purpose and Environment	1149

A.5.6.1.2.2	Test Requirements	1151

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1152

A.5.6.1.3.1	Test purpose and Environment	1152

A.5.6.1.3.2	Test Requirements	1156

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1156

A.5.6.1.4.1	Test purpose and Environment	1156

A.5.6.1.4.2	Test Requirements	1159

A.5.6.2	Inter-frequency Measurements	1160

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1160

A.5.6.2.1.1	Test Purpose and Environment	1160

A.5.6.2.1.2	Test Requirements	1163

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1163

A.5.6.2.2.1	Test Purpose and Environment	1163

A.5.6.2.2.2	Test Requirements	1167

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1167

A.5.6.2.3.1	Test Purpose and Environment	1167

A.5.6.2.3.2	Test Requirements	1171

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1171

A.5.6.2.4.1	Test Purpose and Environment	1171

A.5.6.2.4.2	Test Requirements	1175

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1175

A.5.6.2.5.1	Test Purpose and Environment	1175

A.5.6.2.5.2	Test Requirements	1180

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1180

A.5.6.2.6.1	Test Purpose and Environment	1180

A.5.6.2.6.2	Test Requirements	1184

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1184

A.5.6.2.7.1	Test Purpose and Environment	1184

A.5.6.2.7.2	Test Requirements	1189

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1189

A.5.6.2.8.1	Test Purpose and Environment	1189

A.5.6.2.8.2	Test Requirements	1194

A.5.6.3	L1-RSRP measurement for beam reporting	1194

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1194

A.5.6.3.1.1	Test Purpose and Environment	1194

A.5.6.3.1.2	Test parameters	1194

A.5.6.3.1.3	Test Requirements	1196

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1196

A.5.6.3.2.1	Test Purpose and Environment	1196

A.5.6.3.2.2	Test parameters	1197

A.5.6.3.2.3	Test Requirements	1199

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1199

A.5.6.3.3.1	Test Purpose and Environment	1199

A.5.6.3.3.2	Test parameters	1200

A.5.6.3.3.3	Test Requirements	1202

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1203

A.5.6.3.4.1	Test Purpose and Environment	1203

A.5.6.3.4.2	Test parameters	1203

A.5.6.3.4.3	Test Requirements	1205

A.5.6.4	CLI measurements	1206

A.5.6.4.1	SRS-RSRP measurement with DRX	1206

A.5.6.4.1.1	Test Purpose and Environment	1206

A.5.6.4.1.2	Test Parameters	1206

A.5.6.4.1.3	Test Requirements	1208

A.5.6.4.2	CLI-RSSI measurement with DRX	1208

A.5.6.4.2.1	Test Purpose and Environment	1208

A.5.6.4.2.2	Test Parameters	1209

A.5.6.4.2.3	Test Requirements	1211

A.5.6.5	Measurements with autonomous gaps	1211

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1211

A.5.6.5.1.1	Test Purpose and Environment	1211

A.5.6.5.1.2	Test Requirements	1214

A.5.6.6	L1-SINR measurement for beam reporting	1215

A.5.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1215

A.5.6.6.1.1	Test Purpose and Environment	1215

A.5.6.6.1.2	Test parameters	1215

A.5.6.6.1.3	Test Requirements	1217

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1217

A.5.6.6.2.1	Test Purpose and Environment	1217

A.5.6.6.2.2	Test parameters	1218

A.5.6.6.2.3	Test Requirements	1221

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1221

A.5.6.6.3.1	Test Purpose and Environment	1221

A.5.6.6.3.2	Test parameters	1222

A.5.6.6.3.3	Test Requirements	1224

A.5.6.7	CSI-RS based Intra-frequency Measurements	1224

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1224

A.5.6.7.1.1	Test purpose and Environment	1224

A.5.6.7.1.2	Test Requirements	1227

A.5.6.8	CSI-RS based Inter-frequency Measurements	1227

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1227

A.5.6.8.1.1	Test Purpose and Environment	1227

A.5.6.8.1.2	Test Requirements	1231

A.5.7	Measurement Performance requirements	1231

A.5.7.1	SS-RSRP	1232

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1232

A.5.7.1.1.1	Test Purpose and Environment	1232

A.5.7.1.1.2	Test parameters	1232

A.5.7.1.1.3	Test Requirements	1234

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1235

A.5.7.1.2.1	Test Purpose and Environment	1235

A.5.7.1.2.2	Test parameters	1235

A.5.7.1.2.3	Test Requirements	1239

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1240

A.5.7.1.3.1	Test Purpose and Environment	1240

A.5.7.1.3.2	Test parameters	1240

A.5.7.1.3.3	Test Requirements	1243

A.5.7.2	SS-RSRQ	1244

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1244

A.5.7.2.1.1	Test Purpose and Environment	1244

A.5.7.2.1.2	Test Parameters	1244

A.5.7.2.1.3	Test Requirements	1246

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1247

A.5.7.2.2.1	Test Purpose and Environment	1247

A.5.7.2.2.2	Test Parameters	1247

A.5.7.2.2.3	Test Requirements	1249

A.5.7.3	SS-SINR	1249

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1249

A.5.7.3.1.1	Test Purpose and Environment	1249

A.5.7.3.1.2	Test Parameters	1249

A.5.7.3.1.3	Test Requirements	1251

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1251

A.5.7.3.2.1	Test Purpose and Environment	1251

A.5.7.3.2.2	Test Parameters	1251

A.5.7.3.2.3	Test Requirements	1253

A.5.7.4	L1-RSRP measurement for beam reporting	1253

A.5.7.4.1	SSB based L1-RSRP measurement	1253

A.5.7.4.1.1	Test Purpose and Environment	1253

A.5.7.4.1.2	Test parameters	1254

A.5.7.4.1.3	Test Requirements	1256

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1257

A.5.7.4.2.1	Test Purpose and Environment	1257

A.5.7.4.2.2	Test parameters	1257

A.5.7.4.2.3	Test Requirements	1259

A.5.7.5	CLI measurements	1260

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1260

A.5.7.5.1.1	Test Purpose and Environment	1260

A.5.7.5.1.2	Test parameters	1260

A.5.7.5.1.3	Test Requirements	1263

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1264

A.5.7.5.2.1	Test Purpose and Environment	1264

A.5.7.5.2.2	Test parameters	1264

A.5.7.5.2.3	Test Requirements	1266

A.5.7.6	L1-SINR measurement for beam reporting	1267

A.5.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1267

A.5.7.6.1.1	Test Purpose and Environment	1267

A.5.7.6.1.2	Test parameters	1267

A.5.7.6.1.3	Test Requirements	1269

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1269

A.5.7.6.2.1	Test Purpose and Environment	1269

A.5.7.6.2.2	Test parameters	1270

A.5.7.6.2.3	Test Requirements	1272

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1273

A.5.7.6.3.1	Test Purpose and Environment	1273

A.5.7.6.3.2	Test parameters	1273

A.5.7.6.3.3	Test Requirements	1275

A.5.7.7	CSI-RSRP	1275

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1275

A.5.7.7.1.2	Test parameters	1276

A.5.7.7.1.3	Test Requirements	1278

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1279

A.5.7.7.2.1	Test Purpose and Environment	1279

A.5.7.7.2.2	Test parameters	1279

A.5.7.7.2.3	Test Requirements	1283

A.5.7.8	CSI-RSRQ	1284

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1284

A.5.7.8.1.1	Test Purpose and Environment	1284

A.5.7.8.1.2	Test Parameters	1284

A.5.7.8.1.3	Test Requirements	1286

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1286

A.5.7.8.2.1	Test Purpose and Environment	1286

A.5.7.8.2.2	Test Parameters	1286

A.5.7.9	CSI-SINR	1288

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1288

A.5.7.9.1.1	Test Purpose and Environment	1288

A.5.7.9.1.2	Test Parameters	1288

Table A.5.7.9.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations	1289

A.5.7.9.1.3	Test Requirements	1291

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1291

A.5.7.9.2.1	Test Purpose and Environment	1291

A.5.7.9.2.2	Test Parameters	1291

A.5.7.9.2.3	Test Requirements	1294

A.5.8	Void	1294

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1028

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1028

A.6.1.1.5.1	Test Purpose and Environment	1028

A.6.1.1.5.2	Test Parameters	1028

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1033

A.6.1.1.6.1	Test Purpose and Environment	1033

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1042

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1042

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1042

A.6.1.2.1.1	Test Purpose and Environment	1042

A.6.1.2.1.2	Test Parameters	1042

A.6.1.2.1.3	Test Requirements	1045

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1046

A.6.1.2.2.1	Test Purpose and Environment	1046

A.6.1.2.2.2	Test Parameters	1046

A.6.1.2.2.3	Test Requirements	1049

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1050

A.6.1.2.3.1	Test Purpose and Environment	1050

A.6.1.2.3.2	Test Parameters	1050

A.6.1.2.3.3	Test Requirements	1053

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1054

A.6.1.2.4.1	Test Purpose and Environment	1054

A.6.1.2.4.2	Test Parameters	1054

A.6.1.2.4.3	Test Requirements	1057

A.6.1.2.5.3	Test Requirements	1061

A.6.2	SA: RRC_INACTIVE state mobility	1062

A.6.3	RRC_CONNECTED state mobility	1062

A.6.3.1	Handover	1062

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1062

A.6.3.1.1.1	Test Purpose and Environment	1062

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1066

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1066

A.6.3.1.2.1	Test Purpose and Environment	1066

A.6.3.1.2.2	Test Parameters	1066

A.6.3.1.2.3	Test Requirements	1070

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1070

A.6.3.1.3.1	Test Purpose and Environment	1070

A.6.3.1.3.2	Test Parameters	1070

A.6.3.1.3.3 Test Requirements	1074

A.6.3.1.4	 SA NR - E-UTRAN handover	1074

A.6.3.1.4.1	Test Purpose and Environment	1074

A.6.3.1.4.2	Test Requirements	1080

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1080

A.6.3.1.5.1	Test Purpose and Environment	1080

A.6.3.1.5.2	Test Requirements	1086

A.6.3.1.6	 SA NR - UTRAN FDD handover	1086

A.6.3.1.6.1	Test Purpose and Environment	1086

A.6.3.1.6.2	Test Requirements	1090

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	1090

A.6.3.1.7.1	Test Purpose and Environment	1090

A.6.3.1.7.2	Test Parameters	1090

A.6.3.1.7.3 Test Requirements	1094

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	1095

A.6.3.1.8.1	Test Purpose and Environment	1095

A.6.3.1.8.2	Test Parameters	1095

A.6.3.1.8.3 Test Requirements	1099

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1100

A.6.3.1.9.1	Test Purpose and Environment	1100

A.6.3.1.9.2	Test Parameters	1100

A.6.3.1.9.3 Test Requirements	1104

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1104

A.6.3.1.10.1	Test Purpose and Environment	1104

A.6.3.1.10.2	Test Parameters	1104

A.6.3.1.10.3 Test Requirements	1107

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1107

A.6.3.1.11.1	Test Purpose and Environment	1107

A.6.3.1.11.2	Test Parameters	1107

A.6.3.1.11.3 Test Requirements	1114

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1114

A.6.3.1.12.1	Test Purpose and Environment	1114

A.6.3.1.12.2	Test Parameters	1114

A.6.3.1.12.3 Test Requirements	1122

A.6.3.2	RRC Connection Mobility Control	1122

A.6.3.2.1	SA: RRC Re-establishment	1122

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1122

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1126

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1130

A.6.3.2.2	Random Access	1134

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1134

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1139

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1143

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1148

A.6.3.2.3	SA: RRC Connection Release with Redirection	1152

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1152

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1156

A.6.3.3	Conditional handover	1163

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1163

A.6.3.3.1.1	Test Purpose and Environment	1163

A.6.3.3.1.2	Test Parameters	1163

A.6.3.3.1.3 Test Requirements	1167

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1167

A.6.3.3.2.1	Test Purpose and Environment	1167

A.6.3.3.2.2	Test Parameters	1167

A.6.3.3.2.3 Test Requirements	1171

A.6.4	Timing	1171

A.6.4.1	UE transmit timing	1171

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1171

A.6.4.1.1.1	Test Purpose and environment	1171

A.6.4.1.1.2	Test requirements	1175

A.6.4.2	UE timer accuracy	1175

A.6.4.3	Timing advance	1175

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1175

A.6.4.3.1.1	Test Purpose and Environment	1175

A.6.4.3.1.2	Test Parameters	1175

A.6.4.3.1.3	Test Requirements	1179

A.6.5	Signalling characteristics	1179

A.6.5.1	Radio link Monitoring	1179

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1180

A.6.5.1.1.1	Test Purpose and Environment	1180

A.6.5.1.1.2	Test Requirements	1185

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1185

A.6.5.1.2.1	Test Purpose and Environment	1185

A.6.5.1.2.2	Test Requirements	1191

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1191

A.6.5.1.3.1	Test Purpose and Environment	1191

A.6.5.1.3.2	Test Requirements	1197

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1197

A.6.5.1.4.1	Test Purpose and Environment	1197

A.6.5.1.4.2	Test Requirements	1203

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1203

A.6.5.1.5.1	Test Purpose and Environment	1203

A.6.5.1.5.2	Test Requirements	1209

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1209

A.6.5.1.6.1	Test Purpose and Environment	1209

A.6.5.1.6.2	Test Requirements	1214

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1214

A.6.5.1.7.1	Test Purpose and Environment	1214

A.6.5.1.7.2	Test Requirements	1218

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1218

A.6.5.1.8.1	Test Purpose and Environment	1218

A.6.5.1.8.2	Test Requirements	1224

A.6.5.2	Interruption	1224

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1224

A.6.5.2.1.2	Test Requirements	1228

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1229

A.6.5.2.2.1	Test Purpose and Environment	1229

A.6.5.2.2.2	Test Parameters	1229

A.6.5.2.2.3	Test Requirements	1231

A.6.5.3	SCell Activation and Deactivation Delay	1232

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1232

A.6.5.3.1.1	Test Purpose and Environment	1232

A.6.5.3.1.2	Test Requirements	1237

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	1238

A.6.5.3.2.1	Test Purpose and Environment	1238

A.6.5.3.2.2	Test Requirements	1238

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1238

A.6.5.3.3.1	Test Purpose and Environment	1238

A.6.5.3.3.2	Test Requirements	1239

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1239

A.6.5.3.4.1	 Test Purpose and Environment	1239

A.6.5.3.4.2	Test Requirements	1243

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1243

A.6.5.3.5.1	Test Purpose and Environment	1243

A.6.5.3.5.2	Test Requirements	1248

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1249

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1249

A.6.5.4.1.1	Test Purpose and Environment	1249

A.6.5.4.1.2	Test Requirements	1259

A.6.5.4.2	Void	1260

A.6.5.5	Beam Failure Detection and Link recovery procedures	1260

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1260

A.6.5.5.1.1	Test Purpose and Environment	1260

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1265

A.6.5.5.2.1	Test Purpose and Environment	1265

A.6.5.5.2.2	Test Requirements	1272

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1272

A.6.5.5.3.1	Test Purpose and Environment	1272

A.6.5.5.3.2	Test Requirements	1278

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1278

A.6.5.5.4.1	Test Purpose and Environment	1278

A.6.5.5.4.2	Test Requirements	1284

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1284

A.6.5.5.5.1	Test Purpose and Environment	1284

A.6.5.5.5.2	Test Requirements	1288

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1289

A.6.5.5.6.1	Test Purpose and Environment	1289

A.6.5.5.6.2	Test Requirements	1293

A.6.5.6	Active BWP switch	1293

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1293

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1293

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1300

A.6.5.6.2	RRC-based Active BWP Switch	1304

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1304

A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1308

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1308

A.6.5.6.3.1.1	Test Purpose and Environment	1308

A.6.5.6.3.1.2	Test Requirements	1315

A.6.5.6.4	SCell dormancy switch	1315

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1315

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1321

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1328

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1328

A.6.5.7	DL interruptions at switching between two uplink carriers	1333

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1333

A.6.5.7.1.1	Test Purpose and Environment	1333

A.6.5.7.1.2	Test Requirements	1337

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1337

A.6.5.7.2.1	Test Purpose and Environment	1337

A.6.5.7.2.2	Test Requirements	1341

A.6.5.8	UE specific CBW change	1341

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1341

A.6.5.8.1.1	Test Purpose and Environment	1341

A.6.5.8.1.2	Test Requirements	1345

A.6.5.9	Pathloss reference signal switching delay	1345

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	1345

A.6.5.9.1.1	Test Purpose and Environment	1345

A.6.5.9.1.2	Test Requirements	1348

A.6.6	Measurement procedure	1349

A.6.6.1	Intra-frequency Measurements	1349

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1349

A.6.6.1.1.1	Test purpose and Environment	1349

A.6.6.1.1.2	Test parameters	1349

A.6.6.1.1.3	Test Requirements	1353

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1353

A.6.6.1.2.1	Test purpose and Environment	1353

A.6.6.1.2.2	Test parameters	1353

A.6.6.1.2.3	Test Requirements	1357

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1357

A.6.6.1.3.1	Test purpose and Environment	1357

A.6.6.1.3.2	Test parameters	1357

A.6.6.1.3.3	Test Requirements	1361

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1361

A.6.6.1.4.1	Test purpose and Environment	1361

A.6.6.1.4.2	Test parameters	1361

A.6.6.1.4.3	Test Requirements	1365

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1365

A.6.6.1.5.1	Test purpose and Environment	1365

A.6.6.1.5.2	Test parameters	1365

A.6.6.1.5.3	Test Requirements	1367

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1368

A.6.6.1.6.1	Test purpose and Environment	1368

A.6.6.1.6.2	Test parameters	1368

A.6.6.1.6.3	Test Requirements	1369

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1370

A.6.6.1.7.1	Test purpose and Environment	1370

A.6.6.1.7.2	Test parameters	1370

A.6.6.1.7.3	Test Requirements	1374

A.6.6.2	Inter-frequency Measurements	1374

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1374

A.6.6.2.1.1	Test Purpose and Environment	1374

A.6.6.2.1.2	Test Requirements	1378

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1378

A.6.6.2.2.1	Test Purpose and Environment	1378

A.6.6.2.2.2	Test Requirements	1383

A.6.6.2.3	Void	1384

A.6.6.2.4	Void	1384

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1384

A.6.6.2.5.1	Test Purpose and Environment	1384

A.6.6.2.5.2	Test Requirements	1388

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1388

A.6.6.2.6.1	Test Purpose and Environment	1388

A.6.6.2.6.2	Test Requirements	1393

A.6.6.2.7	Void	1394

A.6.6.2.8	Void	1394

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1394

A.6.6.2.9.1	Test Purpose and Environment	1394

A.6.6.2.9.2	Test Requirements	1397

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1397

A.6.6.2.10.1	Test Purpose and Environment	1397

A.6.6.2.10.2	Test Requirements	1401

A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	1402

A.6.6.2.11.1	Test Purpose and Environment	1402

A.6.6.2.11.2	Test Requirements	1405

A.6.6.3	Inter-RAT Measurements	1405

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1405

A.6.6.3.1.1	Test Purpose and Environment	1405

A.6.6.3.1.2	Test Requirements	1411

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1411

A.6.6.3.2.1	Test Purpose and Environment	1411

A.6.6.3.2.2	Test Requirements	1418

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1418

A.6.6.3.3.1	Test Purpose and Environment	1418

A.6.6.3.3.2	Test Requirements	1425

A.6.6.4	L1-RSRP measurement for beam reporting	1425

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1425

A.6.6.4.1.1	Test Purpose and Environment	1425

A.6.6.4.1.2	Test parameters	1425

A.6.6.4.1.3	Test Requirements	1428

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1428

A.6.6.4.2.1	Test Purpose and Environment	1428

A.6.6.4.2.2	Test parameters	1429

A.6.6.4.2.3	Test Requirements	1432

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1432

A.6.6.4.3.1	Test Purpose and Environment	1432

A.6.6.4.3.2	Test parameters	1433

A.6.6.4.3.3	Test Requirements	1436

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1436

A.6.6.4.4.1	Test Purpose and Environment	1436

A.6.6.4.4.2	Test parameters	1437

A.6.6.4.4.3	Test Requirements	1440

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1440

A.6.6.4.5.1	Test Purpose and Environment	1440

A.6.6.4.5.2	Test parameters	1441

A.6.6.4.5.3	Test Requirements	1444

A.6.6.5	1444

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1444

A.6.6.5.1.1	Test Purpose and Environment	1444

A.6.6.5.1.2	Test Requirements	1448

A.6.6.6	CLI measurements	1448

A.6.6.6.1	SRS-RSRP measurement with DRX	1448

A.6.6.6.1.1	Test Purpose and Environment	1448

A.6.6.6.1.2	Test Parameters	1449

A.6.6.6.1.3	Test Requirements	1452

A.6.6.6.2	CLI-RSSI measurement with DRX	1452

A.6.6.6.2.1	Test Purpose and Environment	1452

A.6.6.6.2.2	Test Parameters	1453

A.6.6.6.2.3	Test Requirements	1455

A.6.6.7	NR measurements with autonomous gaps	1455

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1455

A.6.6.7.1.1	Test Purpose and Environment	1455

A.6.6.7.1.2	Test Parameters	1455

A.6.6.7.1.3	Test Requirements	1458

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1458

A.6.6.7.2.1	Test Purpose and Environment	1458

A.6.6.7.2.2	Test Requirements	1461

A.6.6.8	L1-SINR measurement for beam reporting	1462

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1462

A.6.6.8.1.1	Test Purpose and Environment	1462

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1465

A.6.6.8.2.1	Test Purpose and Environment	1465

A.6.6.8.2.2	Test parameters	1466

A.6.6.8.2.3	Test Requirements	1470

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1470

A.6.6.8.3.1	Test Purpose and Environment	1470

A.6.6.8.3.2	Test parameters	1471

A.6.6.8.3.3	Test Requirements	1474

A.6.6.9	Idle Mode CA/DC Measurements	1474

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1474

A.6.6.9.1.1	Test Purpose and Environment	1474

A.6.6.9.1.2	Test Requirements	1481

A.6.6.10	CSI-RS based intra-frequency Measurements	1481

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1481

A.6.6.10.1.1	Test purpose and Environment	1481

A.6.6.10.1.2	Test Requirements	1484

A.6.6.11	CSI-RS based inter-frequency Measurements	1484

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1484

A.6.6.11.1.1	Test Purpose and Environment	1484

A.6.6.11.1.2	Test Requirements	1488

A.6.6.12	RSTD measurements	1488

A. 6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1488

A. 6.6.12.1.1	Test Purpose and Environment	1488

A.6.6.12.1.2	Test Requirements	1496

A. 6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1496

A. 6.6.12.2.1	Test Purpose and Environment	1496

A.6.6.12.2.2	Test Requirements	1503

A.6.6.13 PRS-RSRP measurements	1503

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1503

A.6.6.13.1.1	Test purpose and Environment	1503

A.6.6.13.1.2	Test Requirements	1507

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1507

A.6.6.13.2.1	Test purpose and Environment	1507

A.6.6.13.2.2	Test Requirements	1511

A.6.6.14	UE Rx-Tx time difference measurements	1511

A.6.6.14.1 UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1511

A.6.6.14.1.1	Test purpose and environment	1511

A.6.6.14.1.2	Test requirements	1515

A.6.6.14.2 UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1515

A.6.6.14.2.1	Test purpose and environment	1515

A.6.6.14.2.2	Test requirements	1519

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1519

A.6.6.15.1	Test Purpose and Environment	1519

A.6.6.15.2	Test Requirements	1526

A.6.7	Measurement Performance requirements	1527

A.6.7.1	SS-RSRP	1527

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1527

A.6.7.1.1.1	Test Purpose and Environment	1527

A.6.7.1.1.2	Test parameters	1527

A.6.7.1.1.3	Test Requirements	1532

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1532

A.6.7.1.2.1	Test Purpose and Environment	1532

A.6.7.1.2.2	Test parameters	1532

A.6.7.1.2.3	Test Requirements	1536

A.6.7.1.3	Void	1536

A.6.7.2	SS-RSRQ	1536

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1536

A.6.7.2.1.1	Test Purpose and Environment	1536

A.6.7.2.1.2	Test Parameters	1537

A.6.7.2.1.3	Test Requirements	1542

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1542

A.6.7.2.2.1	Test Purpose and Environment	1542

A.6.7.2.2.2	Test Parameters	1542

A.6.7.2.2.3	Test Requirements	1547

A.6.7.3	SS-SINR	1547

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1547

A.6.7.3.1.1	Test Purpose and Environment	1547

A.6.7.3.1.2	Test Parameters	1548

A.6.7.3.1.3	Test Requirements	1552

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1552

A.6.7.3.2.1	Test Purpose and Environment	1552

A.6.7.3.2.2	Test Parameters	1552

A.6.7.3.2.3	Test Requirements	1558

A.6.7.4	L1-RSRP measurement for beam reporting	1559

A.6.7.4.1	SSB based L1-RSRP measurement	1559

A.6.7.4.1.1	Test Purpose and Environment	1559

A.6.7.4.1.2	Test parameters	1559

A.6.7.4.1.3	Test Requirements	1563

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1563

A.6.7.4.2.1	Test Purpose and Environment	1563

A.6.7.4.2.2	Test parameters	1564

A.6.7.4.2.3	Test Requirements	1568

A.6.7.5	E-UTRAN RSRP	1568

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1568

A.6.7.5.1.1	Test Purpose and Environment	1568

A.6.7.5.1.2	Test parameters	1569

A.6.7.5.1.3	Test Requirements	1575

A.6.7.6	E-UTRAN RSRQ	1576

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1576

A.6.7.6.1.1	Test Purpose and Environment	1576

A.6.7.6.1.2	Test parameters	1576

A.6.7.6.1.3	Test Requirements	1581

A.6.7.7	E-UTRAN RS-SINR	1582

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1582

A.6.7.7.1.1	Test Purpose and Environment	1582

A.6.7.7.1.2	Test parameters	1582

A.6.7.7.1.3	Test Requirements	1588

A.6.7.8	CLI measurements	1589

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1589

A.6.7.8.1.1	Test Purpose and Environment	1589

A.6.7.8.1.2	Test parameters	1589

A.6.7.8.1.3	Test Requirements	1595

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1595

A.6.7.8.2.1	Test Purpose and Environment	1595

A.6.7.8.2.2	Test parameters	1596

A.6.7.8.2.3	Test Requirements	1599

A.6.7.9	L1-SINR measurement for beam reporting	1600

A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1600

A.6.7.9.1.1	Test Purpose and Environment	1600

A.6.7.9.1.2	Test parameters	1600

A.6.7.9.1.3	Test Requirements	1604

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1604

A.6.7.9.2.1	Test Purpose and Environment	1604

A.6.7.9.2.2	Test parameters	1605

A.6.7.9.2.3	Test Requirements	1610

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1610

A.6.7.9.3.1	Test Purpose and Environment	1610

A.6.7.9.3.2	Test parameters	1610

A.6.7.9.3.3	Test Requirements	1615

A.6.7.10	CSI-RSRP	1615

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1615

A.6.7.10.1.1	Test Purpose and Environment	1615

A.6.7.10.1.2	Test parameters	1616

A.6.7.10.1.3	Test Requirements	1621

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1621

A.6.7.10.2.1	Test Purpose and Environment	1621

A.6.7.10.2.2	Test parameters	1621

A.6.7.10.2.3	Test Requirements	1626

A.6.7.11	CSI-RSRQ	1626

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1626

A.6.7.11.1.1	Test Purpose and Environment	1626

A.6.7.11.1.2	Test Parameters	1626

A.6.7.11.1.3	Test Requirements	1631

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1631

A.6.7.11.2.1	Test Purpose and Environment	1631

A.6.7.11.2.2	Test Parameters	1631

A.6.7.11.2.3	Test Requirements	1636

A.6.7.12	CSI-SINR	1637

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1637

A.6.7.12.1.1	Test Purpose and Environment	1637

A.6.7.12.1.2	Test Parameters	1637

A.6.7.12.1.3	Test Requirements	1641

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1642

A.6.7.12.2.1	Test Purpose and Environment	1642

A.6.7.12.2.2	Test Parameters	1642

A.6.7.12.2.3	Test Requirements	1648

A.6.7.13	RSTD measurements	1648

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1648

A.6.7.13.1.1	Test purpose and Environment	1648

A.6.7.13.1.2	Test Requirements	1651

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1651

A.6.7.13.2.1	Test purpose and Environment	1651

A.6.7.13.2.2	Test Requirements	1655

A.6.7.14	PRS-RSRP measurements	1655

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1655

A.6.7.14.1.1	Test Purpose and Environment	1655

A.6.7.14.1.2	Test parameters	1655

A.6.7.14.1.3	Test Requirements	1660

A.6.7.15	UE Rx-Tx time difference measurements	1660

A.6.7.15.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1660

A.6.7.15.1.1	Test purpose and environment	1660

A.6.7.15.1.2	Test parameters	1661

A.6.7.15.1.3	Test requirements	1664

A.7.1	SA: RRC_IDLE state mobility	1665

A.7.1.1	Cell re-selection to NR	1665

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1665

A.7.1.1.1.1	Test Purpose and Environment	1665

A.7.1.1.1.2	Test Parameters	1665

A.7.1.1.1.3	Test Requirements	1668

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1668

A.7.1.1.2.1	Test Purpose and Environment	1668

A.7.1.1.2.2	Test Parameters	1669

A.7.1.1.2.3	Test Requirements	1671

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1671

A.7.1.1.3.1	Test Purpose and Environment	1671

A.7.1.1.3.2	Test Parameters	1671

A.7.1.1.3.3	Test Requirements	1674

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1674

A.7.1.1.4.1	Test Purpose and Environment	1674

A.7.1.1.4.2	Test Parameters	1674

A.7.1.1.4.3	Test Requirements	1677

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1677

A.7.1.1.5.1	Test Purpose and Environment	1677

A.7.1.1.5.2	Test Parameters	1677

A.7.1.1.5.3	Test Requirements	1680

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1680

A.7.1.1.6.1	Test Purpose and Environment	1680

A.7.1.1.6.2	Test Parameters	1680

A.7.1.1.6.3	Test Requirements	1683

A.7.2	SA: RRC_INACTIVE state mobility	1683

A.7.3	RRC_CONNECTED state mobility	1683

A.7.3.1	Handover	1683

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1683

A.7.3.1.1.1	Test Purpose and Environment	1683

A.7.3.1.1.2	Test Parameters	1683

A.7.3.1.1.3	Test Requirements	1687

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1687

A.7.3.1.2.1	Test Purpose and Environment	1687

A.7.3.1.2.2	Test Parameters	1687

A.7.3.1.2.3	Test Requirements	1690

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1690

A.7.3.1.3.1	Test Purpose and Environment	1690

A.7.3.1.3.2	Test Parameters	1690

A.7.3.1.3.3 Test Requirements	1691

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1692

A.7.3.1.4.1	Test Purpose and Environment	1692

A.7.3.1.4.2	Test Parameters	1692

A.7.3.1.4.3 Test Requirements	1699

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1699

A.7.3.1.5.1	Test Purpose and Environment	1699

A.7.3.1.5.2	Test Parameters	1699

A.7.3.1.5.3 Test Requirements	1706

A.7.3.2	RRC Connection Mobility Control	1706

A.7.3.2.1	SA: RRC Re-establishment	1706

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1706

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1709

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1712

A.7.3.2.1.3.1	Test Purpose and Environment	1712

A.7.3.2.1.3.2	Test Requirements	1714

A.7.3.2.2	Random Access	1715

A.7.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	1715

A.7.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	1719

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1724

A.7.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	1727

A.7.3.2.3	SA: RRC Connection Release with Redirection	1730

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1730

A.7.3.3	Conditional Handover	1734

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1734

A.7.3.3.1.1	Test Purpose and Environment	1734

A.7.3.3.1.2	Test Parameters	1734

A.7.3.3.1.2.3	Test Requirements	1737

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	1737

A.7.3.3.2.1	Test Purpose and Environment	1737

A.7.3.3.2.2	Test Parameters	1737

A.7.3.3.2.3 Test Requirements	1740

A.7.4	Timing	1740

A.7.4.1	UE transmit timing	1740

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1740

A.7.4.1.1.1	Test Purpose and environment	1740

A.7.4.1.1.2	Test requirements	1743

A.7.4.2	UE timer accuracy	1743

A.7.4.3	Timing advance	1743

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1743

A.7.4.3.1.1	Test Purpose and Environment	1743

A.7.4.3.1.2	Test Parameters	1743

A.7.4.3.1.3 Test Requirements	1747

A.7.5	Signaling characteristics	1747

A.7.5.1	Radio link Monitoring	1747

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1747

A.7.5.1.1.1	Test Purpose and Environment	1747

A.7.5.1.1.2	Test Requirements	1750

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1751

A.7.5.1.2.1	Test Purpose and Environment	1751

A.7.5.1.2.2	Test Requirements	1755

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1756

A.7.5.1.3.1	Test Purpose and Environment	1756

A.7.5.1.3.2	Test Requirements	1760

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1760

A.7.5.1.4.1	Test Purpose and Environment	1760

A.7.5.1.4.2	Test Requirements	1765

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1765

A.7.5.1.5.1	Test Purpose and Environment	1765

A.7.5.1.5.2	Test Requirements	1770

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1770

A.7.5.1.6.1	Test Purpose and Environment	1770

A.7.5.1.6.2	Test Requirements	1774

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1774

A.7.5.1.7.1	Test Purpose and Environment	1774

A.7.5.1.7.2	Test Requirements	1778

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1778

A.7.5.1.8.1	Test Purpose and Environment	1778

A.7.5.1.8.2	Test Requirements	1783

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1783

A.7.5.1.9.1	Test Purpose and Environment	1783

A.7.5.1.9.2	Test Requirements	1786

A.7.5.2	Interruption	1786

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1786

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1790

A.7.5.2.2.1	Test Purpose and Environment	1790

A.7.5.2.2.2	Test Parameters	1790

A.7.5.2.2.3	Test Requirements	1792

A.7.5.3	SCell Activation and Deactivation Delay	1792

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1792

A.7.5.3.1.1	Test Purpose and Environment	1792

A.7.5.3.1.2	Test Requirements	1794

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1794

A.7.5.3.2.1	Test Purpose and Environment	1794

A.7.5.3.2.2	Test Requirements	1798

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1799

A.7.5.3.3.1	Test Purpose and Environment	1799

A.7.5.3.3.2	Test Requirements	1802

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1803

A.7.5.3.4.1	Test Purpose and Environment	1803

A.7.5.3.4.2	Test Requirements	1806

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1807

A.7.5.3.5.1	Test Purpose and Environment	1807

A.7.5.3.5.2	Test Requirements	1809

A.7.5.4	Void	1810

A.7.5.5	Beam Failure Detection and Link recovery procedures	1810

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1810

A.7.5.5.1.1	Test Purpose and Environment	1810

A.7.5.5.1.2	Test Requirements	1814

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1814

A.7.5.5.2.1	Test Purpose and Environment	1814

A.7.5.5.2.2	Test Requirements	1818

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1818

A.7.5.5.3.1	Test Purpose and Environment	1818

A.7.5.5.3.2	Test Requirements	1822

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1822

A.7.5.5.4.1	Test Purpose and Environment	1822

A.7.5.5.4.2	Test Requirements	1826

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1826

A.7.5.5.5.1	Test Purpose and Environment	1826

A.7.5.5.5.2	Test Requirements	1830

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1830

A.7.5.5.6.1	Test Purpose and Environment	1830

A.7.5.5.6.2	Test Requirements	1834

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1834

A.7.5.5.7.1	Test Purpose and Environment	1834

A.7.5.5.7.2	Test Requirements	1838

A.7.5.6	Active BWP switch	1838

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1838

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1838

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA A.7.5.6.1.2.1 Test Purpose and Environment	1843

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1848

A.7.5.6.1.3.1	Test Purpose and Environment	1848

A.7.5.6.1.3.2	Test Requirements	1851

A.7.5.6.2	RRC-based Active BWP Switch	1851

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1855

A.7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	1855

A.7.5.6.4	SCell dormancy switch	1858

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1858

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1863

A.7.5.6.4.2.2	 Test Requirements	1868

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1868

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1868

A.7.5.7	PSCell addition and release delay	1873

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1873

A.7.5.7.1.1	Test Purpose and Environment	1873

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1876

A.7.5.7.2.1	Test Purpose and Environment	1876

A.7.5.7.2.2	Test Requirements	1879

A.7.5.8	Active TCI state switch delay	1879

A.7.5.8.1	MAC-CE based active TCI state switch	1879

A.7.5.8.2	RRC based active TCI state switch	1883

A.7.5.9	Uplink spatial relation switch delay	1887

A.7.5.9.1	MAC-CE based Spatial Relation switch	1887

A.7.5.9.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	1887

A.7.5.9.1.1.2	Test Requirements	1890

A.7.5.9.2	RRC based spatial relation switch	1890

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1890

A.7.5.9.2.1.1	Test Purpose and Environment	1890

A.7.5.9.2.1.2	Test Requirements	1893

A.7.5.10	UE specific CBW change	1893

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1893

A.7.5.10.1.1	Test Purpose and Environment	1893

A.7.5.10.1.2	Test Requirements	1896

A.7.6	Measurement procedure	1897

A.7.6.1	Intra-frequency Measurements	1897

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1897

A.7.6.1.1.1	Test purpose and Environment	1897

A.7.6.1.1.2	Test Requirements	1899

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1900

A.7.6.1.2.1	Test purpose and Environment	1900

A.7.6.1.2.2	Test Requirements	1902

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1903

A.7.6.1.3.1	Test purpose and Environment	1903

A.7.6.1.3.2	Test Requirements	1906

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1906

A.7.6.1.4.1	Test purpose and Environment	1906

A.7.6.1.4.2	Test Requirements	1909

A.7.6.2	Inter-frequency Measurements	1910

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1910

A.7.6.2.1.1	Test Purpose and Environment	1910

A.7.6.2.1.2	Test Requirements	1913

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1913

A.7.6.2.2.1	Test Purpose and Environment	1913

A.7.6.2.2.2	Test Requirements	1917

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1917

A.7.6.2.3.1	Test Purpose and Environment	1917

A.7.6.2.3.2	Test Requirements	1921

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1921

A.7.6.2.4.1	Test Purpose and Environment	1921

A.7.6.2.4.2	Test Requirements	1925

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1925

A.7.6.2.5.1	Test Purpose and Environment	1925

A.7.6.2.5.2	Test Requirements	1929

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1930

A.7.6.2.6.1	Test Purpose and Environment	1930

A.7.6.2.6.2	Test Requirements	1934

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1935

A.7.6.2.7.1	Test Purpose and Environment	1935

A.7.6.2.7.2	Test Requirements	1939

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1940

A.7.6.2.8.1	Test Purpose and Environment	1940

A.7.6.2.8.2	Test Requirements	1944

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1945

A.7.6.2.9.1	Test Purpose and Environment	1945

A.7.6.2.9.2	Test Requirements	1948

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1948

A.7.6.2.10.1	Test Purpose and Environment	1948

A.7.6.2.10.2	Test Requirements	1950

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1950

A.7.6.2.11.1	Test Purpose and Environment	1950

A.7.6.2.11.2	Test Requirements	1953

A.7.6.3	L1-RSRP measurement for beam reporting	1954

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1954

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1954

A.7.6.3.1.1	Test Purpose and Environment	1954

A.7.6.3.1.2	Test parameters	1954

A.7.6.3.1.3	Test Requirements	1956

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1956

A.7.6.3.2.1	Test Purpose and Environment	1956

A.7.6.3.2.2	Test parameters	1957

A.7.6.3.2.3	Test Requirements	1959

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1959

A.7.6.3.3.1	Test Purpose and Environment	1959

A.7.6.3.3.2	Test parameters	1960

A.7.6.3.3.3	Test Requirements	1962

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1963

A.7.6.3.4.1	Test Purpose and Environment	1963

A.7.6.3.4.2	Test parameters	1963

A.7.6.3.3.3	Test Requirements	1965

A.7.6.4	CLI measurements	1966

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1966

A.7.6.4.1.1	Test Purpose and Environment	1966

A.7.6.4.1.2	Test Parameters	1966

A.7.6.4.1.3	Test Requirements	1968

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1968

A.7.6.4.2.1	Test Purpose and Environment	1968

A.7.6.4.2.2	Test Parameters	1969

A.7.6.4.2.3	Test Requirements	1970

A.7.6.5	NR Measurements with autonomous gaps	1971

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1971

A.7.6.5.1.1	Test Purpose and Environment	1971

A.7.6.5.1.2	Test Requirements	1974

A.7.6.6	L1-SINR measurement for beam reporting	1974

A.7.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	1974

A.7.6.6.1.1	Test Purpose and Environment	1974

A.7.6.6.1.2	Test parameters	1974

A.7.6.6.1.3	Test Requirements	1976

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1976

A.7.6.6.2.1	Test Purpose and Environment	1976

A.7.6.6.2.2	Test parameters	1977

A.7.6.6.2.3	Test Requirements	1979

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1980

A.7.6.6.3.1	Test Purpose and Environment	1980

A.7.6.6.3.2	Test parameters	1980

A.7.6.6.3.3	Test Requirements	1982

A.7.6.7	CSI-RS based intra-frequency Measurements	1982

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1982

A.7.6.7.1.1	Test purpose and Environment	1982

A.7.6.7.1.2	Test Requirements	1985

A.7.6.8	CSI-RS based inter-frequency Measurements	1985

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1985

A.7.6.8.1.1	Test Purpose and Environment	1985

A.7.6.2.2.2	Test Requirements	1989

A.7.6.9	RSTD measurements	1989

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1989

A.7.6.9.1.1	Test Purpose and Environment	1989

A.7.6.9.1.2	Test Requirements	1995

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1995

A.7.6.9.2.1	Test Purpose and Environment	1995

A.7.6.9.2.2	Test Requirements	2002

A.7.6.10 PRS-RSRP measurements	2002

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	2002

A.7.6.10.1.1	Test Purpose and Environment	2002

A.7.6.10.1.2	Test Requirements	2006

A.7.6.10.2 PRS-RSRP reporting delay test case for dual positioning frequency layer	2006

A.7.6.10.2.1	Test Purpose and Environment	2006

A.7.6.10.2.2	Test Requirements	2009

A.7.6.11	UE Rx-Tx time difference measurements	2009

A.7.6.11.1 UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	2009

A.7.6.11.1.1	Test purpose and environment	2009

A.7.6.11.1.2	Test requirements	2013

A.7.6.11.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	2013

A.7.6.11.2.1	Test purpose and environment	2013

A.7.6.11.2.2	Test requirements	2017

A.7.7	Measurement Performance requirements	2017

A.7.7.1	SS-RSRP	2017

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2017

A.7.7.1.1.1	Test Purpose and Environment	2017

A.7.7.1.1.2	Test parameters	2018

A.7.7.1.1.3	Test Requirements	2022

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2023

A.7.7.1.2.1	Test Purpose and Environment	2023

A.7.7.1.2.2	Test parameters	2023

A.7.7.1.2.3	Test Requirements	2027

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2028

A.7.7.1.3.1	Test Purpose and Environment	2028

A.7.7.1.3.2	Test parameters	2028

A.7.7.1.3.3	Test Requirements	2032

A.7.7.2	SS-RSRQ	2032

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2032

A.7.7.2.1.1	Test Purpose and Environment	2032

A.7.7.2.1.2	Test Parameters	2032

A.7.7.2.1.3	Test Requirements	2034

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2034

A.7.7.2.2.1	Test Purpose and Environment	2034

A.7.7.2.2.2	Test Parameters	2034

A.7.7.2.2.3	Test Requirements	2036

A.7.7.3	SS-SINR	2036

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2036

A.7.7.3.1.1	Test Purpose and Environment	2036

A.7.7.3.1.2	Test Parameters	2036

A.7.7.3.1.3	Test Requirements	2039

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2039

A.7.7.3.2.1	Test Purpose and Environment	2039

A.7.7.3.2.2	Test Parameters	2039

A.7.7.3.2.3	Test Requirements	2041

A.7.7.4	L1-RSRP measurement for beam reporting	2041

A.7.7.4.1	SSB based L1-RSRP measurement	2041

A.7.7.4.1.1	Test Purpose and Environment	2041

A.7.7.4.1.2	Test parameters	2042

A.7.7.4.1.3	Test Requirements	2044

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2045

A.7.7.4.2.1	Test Purpose and Environment	2045

A.7.7.4.2.2	Test parameters	2045

A.7.7.4.2.3	Test Requirements	2047

A.7.7.5	CLI measurements	2048

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2048

A.7.7.5.1.1	Test Purpose and Environment	2048

A.7.7.5.1.2	Test parameters	2048

A.7.7.5.1.3	Test Requirements	2051

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2052

A.7.7.5.2.1	Test Purpose and Environment	2052

A.7.7.5.2.2	Test parameters	2052

A.7.7.5.2.3	Test Requirements	2054

A.7.7.6	L1-SINR measurement for beam reporting	2055

A.7.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2055

A.7.7.6.1.1	Test Purpose and Environment	2055

A.7.7.6.1.2	Test parameters	2055

A.7.7.6.1.3	Test Requirements	2057

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2057

A.7.7.6.2.1	Test Purpose and Environment	2057

A.7.7.6.2.2	Test parameters	2058

A.7.7.6.2.3	Test Requirements	2060

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2060

A.7.7.6.3.1	Test Purpose and Environment	2060

A.7.7.6.3.2	Test parameters	2061

A.7.7.6.3.3	Test Requirements	2063

A.7.7.7	CSI-RSRP	2063

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2063

A.7.7.7.1.1	Test Purpose and Environment	2063

A.7.7.7.1.2	Test parameters	2064

A.7.7.7.1.3	Test Requirements	2067

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2068

A.7.7.7.2.1	Test Purpose and Environment	2068

A.7.7.7.2.2	Test parameters	2068

A.7.7.7.2.3	Test Requirements	2071

A.7.7.8	CSI-RSRQ	2071

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2071

A.7.7.8.1.1	Test Purpose and Environment	2071

A.7.7.8.1.2	Test Parameters	2072

A.7.7.8.1.3	Test Requirements	2074

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2074

A.7.7.8.2.1	Test Purpose and Environment	2074

A.7.7.8.2.2	Test Parameters	2074

A.7.7.8.2.3	Test Requirements	2076

A.7.7.9	CSI-SINR	2076

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2076

A.7.7.9.1.1	Test Purpose and Environment	2076

A.7.7.9.1.2	Test Parameters	2076

A.7.7.9.1.3	Test Requirements	2079

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2079

A.7.7.9.2.2	Test Parameters	2079

A.7.7.9.2.3	Test Requirements	2081

A.7.7.10	RSTD measurements	2081

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2081

A.7.7.10.1.1	Test purpose and Environment	2081

A.7.7.10.1.2	Test Requirements	2084

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2084

A.7.7.10.2.1	Test purpose and Environment	2084

A.7.7.10.2.2	Test Requirements	2086

A.7.7.11	PRS-RSRP measurements	2086

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2086

A.7.7.11.1.1	Test Purpose and Environment	2086

A.7.7.11.1.2	Test parameters	2086

A.7.7.11.1.3	Test Requirements	2090

A.7.7.12	UE Rx-Tx time difference measurements	2091

A.7.7.12.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	2091

A.7.7.12.1.1	Test purpose and environment	2091

A.7.7.12.1.2	Test parameters	2091

A.7.7.12.1.3	Test requirements	2094

A.8	E-UTRA standalone tests for NR RRM	2094

A.8.1	Void	2095

A.8.2	RRC_IDLE state mobility	2095

A.8.2.1	Inter-RAT NR Cell re-selection	2095

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2095

A.8.2.1.1.1	Test Purpose and Environment	2095

A.8.2.1.1.2	Test Requirements	2100

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2101

A.8.2.1.2.1	Test Purpose and Environment	2101

A.8.2.1.2.2	Test Requirements	2106

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2107

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2107

A.8.2.2.1.1	Test Purpose and Environment	2107

A.8.2.2.1.2	Test Requirements	2112

A.8.2.2.2	E-UTRA – NR Early Measruement Reporting for NR in FR2	2112

A.8.2.2.2.1	Test Purpose and Environment	2112

A.8.2.2.2.2	Test Requirements	2114

A.8.3	RRC_CONNECTED state mobility	2115

A.8.3.1	Handover	2115

A.8.3.1.1	E-UTRAN - NR handover in FR1	2115

A.8.3.1.1.1	Test Purpose and Environment	2115

A.8.3.1.1.2	Test Requirements	2120

A.8.4	Measurement procedure	2120

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2120

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2120

A.8.4.1.1.1	Test Purpose and Environment	2120

A.8.4.1.1.2	Test Requirements	2122

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2123

A.8.4.1.2.1	Test Purpose and Environment	2123

A.8.4.1.2.2	Test Requirements	2124

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2124

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2124

A.8.4.2.1.1	Test Purpose and Environment	2124

A.8.4.2.1.2	Test Requirements	2129

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2129

A.8.4.2.2.1	Test Purpose and Environment	2129

A.8.4.2.2.2	Test Requirements	2133

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2133

A.8.4.2.3.1	Test Purpose and Environment	2133

A.8.4.2.3.2	Test Requirements	2138

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2138

A.8.4.2.4.1	Test Purpose and Environment	2138

A.8.4.2.4.2	Test Requirements	2142

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2142

A.8.4.2.5.1	Test Purpose and Environment	2142

A.8.4.2.5.2	Test Requirements	2144

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2145

A.8.4.2.6.1	Test Purpose and Environment	2145

A.8.4.2.6.2	Test Requirements	2147

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2148

A.8.4.2.7.1	Test Purpose and Environment	2148

A.8.4.2.7.2	Test Requirements	2150

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2151

A.8.4.2.8.1	Test Purpose and Environment	2151

A.8.4.2.8.2	Test Requirements	2153

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2154

A.8.4.2.9.1	Test Purpose and Environment	2154

A.8.4.2.9.2	Test Requirements	2160

A.8.5	Measurement performance	2160

A.8.5.1	SFTD accuracy	2160

A.8.5.1.1	SFTD accuracy	2160

A.8.5.1.1.1	Test Purpose	2160

A.8.5.1.1.2	Test Environment	2160

A.8.5.1.1.3	Test Requirements	2166

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2166

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2166

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2171

A.8.5.2.1.2.1	Test Purpose and Environment	2171

A.8.5.2.1.2.2	Test Parameters	2171

A.8.5.2.1.2.3	Test Requirements	2173

A.8.5.2.2	SS-RSRQ	2173

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2173

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2178

A.8.5.2.2.2.1	Test Purpose and Environment	2178

A.8.5.2.2.2.2	Test Parameters	2178

A.8.5.2.2.2.3	Test Requirements	2180

A.8.5.2.3	SS-SINR	2180

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2180

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2184

A.8.5.2.3.2.1	Test Purpose and Environment	2184

A.8.5.2.3.2.2	Test Parameters	2184

A.8.5.2.3.2.3	Test Requirements	2186

A.9	V2X Tests	2187

A.9.1	V2X Tests in FR1	2187

A.9.1.1	Test for V2X UE Transmit Timing	2187

A.9.1.1.1	Test for GNSS as Synchronization Reference Source	2187

A.9.1.1.1.1	Test Purpose and Environment	2187

A.9.1.1.1.2	Test requirements	2187

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2187

A.9.1.1.2.1	Test Purpose and Environment	2187

A.9.1.1.2.2	Test requirements	2188

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2188

A.9.1.1.3.1	Test Purpose and Environment	2188

A.9.1.1.3.2	Test requirements	2191

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	2191

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2191

A.9.1.2.1.1	Test Purpose and Environment	2191

A.9.1.2.1.2	Test Requirements	2195

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2195

A.9.1.2.2.1	Test Purpose and Environment	2195

A.9.1.2.2.2	Test Requirements	2196

A.9.1.3	Test for V2X Synchronization Reference Selection/Reselection	2197

A.9.1.3.1	Test for GNSS configured as the highest priority	2197

A.9.1.3.1.1	Test Purpose and Environment	2197

A.9.1.3.1.2	Test Requirements	2199

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2201

A.9.1.3.2.1	Test Purpose and Environment	2201

A.9.1.3.2.2	Test Requirements	2203

A.9.1.4	Test for L1 SL-RSRP Measurement	2204

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2204

A.9.1.4.1.1	Test Purpose and Environment	2204

A.9.1.4.1.2	Test Requirements	2207

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2207

A.9.1.4.2.1	Test Purpose and Environment	2207

A.9.1.4.2.2	Test Requirements	2211

A.9.1.4.3	Test for V2X UE Resource Re-evaluation	2211

A.9.1.4.3.1	Test Purpose and Environment	2211

A.9.1.4.3.2	Test Requirements	2218

A.9.1.5	Test for Congestion Control Measurement	2218

A.9.1.5.1	Test Purpose and Environment	2218

A.9.1.5.2	Test Requirements	2224

A.9.1.6	Test for Interruption	2224

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2224

A.9.1.6.1.1	Test Purpose and Environment	2224

A.9.1.6.1.2	Test Requirements	2227

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2227

A.10.1	RRC_CONNECTED state mobility	2227

A.10.1.1	RRC connection mobility control	2227

A.10.1.1.1	Random Access	2227

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2227

A.10.1.1.1.1.1	Test Purpose and Environment	2227

A.10.1.1.1.1.2	Test Requirements	2230

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2230

A.10.1.1.1.1.2.2	Random Access Response Reception	2230

A.10.1.1.1.1.2.3	No Random Access Response Reception	2231

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2231

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2231

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2232

A.10.1.1.1.2.1	Test Purpose and Environment	2232

A.10.1.1.1.2.2	Test Requirements	2235

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2235

A.10.1.1.1.2.2.2	Random Access Response Reception	2236

A.10.1.1.1.2.2.3	No Random Access Response Reception	2236

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2236

A.10.1.1.1.3.1	Test Purpose and Environment	2236

A.10.1.1.1.3.2	Test Requirements	2240

A.10.1.1.1.3.2.1	MsgA Transmission	2240

A.10.1.1.1.3.2.2	MsgB Reception	2240

A.10.1.1.1.3.2.3	No MsgB Reception	2241

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2241

A.10.1.1.1.4.1	Test Purpose and Environment	2241

A.10.1.1.1.4.2	Test Requirements	2245

A.10.1.1.1.4.2.1	MsgA Transmission	2245

A.10.1.1.1.4.2.2	MsgB Reception	2246

A.10.1.1.1.4.2.3	No MsgB Reception	2246

A.10.2	Timing	2247

A.10.2.1	UE transmit timing	2247

A.10.2.2	UE timing advance	2251

A.10.3	Signalling characteristics	2254

A.10.3.1	Radio link monitoring	2254

A.10.3.1.1	Introduction	2254

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2255

A.10.3.1.2.1	Test purpose and environment	2255

A.10.3.1.2.2	Test requirements	2260

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2260

A.10.3.1.3.1	Test purpose and environment	2260

A.10.3.1.3.2	Test requirements	2266

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2266

A.10.3.1.4.1	Test purpose and environment	2266

A.10.3.1.4.2	Test requirements	2266

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2266

A.10.3.1.5.1	Test purpose and environment	2266

A.10.3.1.5.2	Test requirements	2266

A.10.3.2	Interruption	2266

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2266

A.10.3.2.1.1	Test Purpose and Environment	2266

A.10.3.2.1.2	Test Requirements	2271

A.10.3.3	SCell activation and deactivation delay	2271

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	2276

A.10.3.3.2.1	Test Purpose and Environment	2276

A.10.3.3.2.2	Test Requirements	2277

A.10.3.3.3 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2277

A.10.3.3.3.1	Test Purpose and Environment	2277

A.10.3.3.3.2	Test Requirements	2278

A.10.3.4	Beam failure detection and link recovery procedures	2278

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2278

A.10.3.4.1.1	Test Purpose and Environment	2278

A.10.3.4.1.2	Test Requirements	2283

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2283

A.10.3.4.2.1	Test Purpose and Environment	2283

A.10.3.4.2.2	Test Requirements	2289

A.10.3.5	Active BWP switching	2289

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2289

A.10.3.5.1.1	Test Purpose and Environment	2289

A.10.3.5.1.2	Test Requirements	2294

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2295

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2295

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2298

A.10.3.5.3	RRC-based Active BWP Switch	2302

A.10.3.6	PSCell addition and release delay	2306

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2306

A.10.3.6.1.1	Test purpose and environment	2306

A.10.3.6.1.2	Test Requirements	2310

A.10.3.7	Void	2311

A.10.4	Measurement procedure	2311

A.10.4.1	Intra-frequency measurements	2311

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2311

A.10.4.1.1.1	Test purpose and environment	2311

A.10.4.1.1.2	Test parameters	2311

A.10.4.1.1.3	Test Requirements	2311

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2312

A.10.4.1.2.1	Test purpose and environment	2312

A.10.4.1.2.2	Test parameters	2312

A.10.4.1.2.3	Test Requirements	2312

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2312

A.10.4.1.3.1	Test purpose and environment	2312

A.10.4.1.3.2	Test parameters	2312

A.10.4.1.3.3	Test Requirements	2312

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2313

A.10.4.1.4.1	Test purpose and environment	2313

A.10.4.1.4.2	Test parameters	2313

A.10.4.1.4.3	Test Requirements	2313

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2313

A.10.4.1.5.1	Test purpose and environment	2313

A.10.4.1.5.2	Test parameters	2313

A.10.4.1.5.3	Test Requirements	2313

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2314

A.10.4.1.6.1	Test purpose and environment	2314

A.10.4.1.6.2	Test parameters	2314

A.10.4.1.6.3	Test Requirements	2314

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2314

A.10.4.1.7.1	Test purpose and environment	2314

A.10.4.1.7.2	Test parameters	2314

A.10.4.1.7.3	Test Requirements	2314

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2315

A.10.4.1.8.1	Test purpose and environment	2315

A.10.4.1.8.2	Test parameters	2315

A.10.4.1.8.3	Test Requirements	2315

A.10.4.1.9	RSSI measurement reporting on PSCC	2315

A.10.4.1.9.1	Test purpose and environment	2315

A.10.4.1.9.2	Test parameters	2315

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2316

A.10.4.1.10.1	Test purpose and environment	2316

A.10.4.1.10.2	Test parameters	2316

A.10.4.1.11	RSSI measurement reporting on SCC	2316

A.10.4.1.11.1	Test purpose and environment	2316

A.10.4.1.11.2	Test parameters	2316

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2317

A.10.4.1.12.1	Test purpose and environment	2317

A.10.4.1.12.2	Test parameters	2317

A.10.4.2	Inter-frequency measurements	2317

A.10.4.2.1	RSSI measurement reporting	2317

A.10.4.2.1.1	Test purpose and environment	2317

A.10.4.2.1.2	Test parameters	2317

A.10.4.2.2	Channel occupancy measurement reporting	2318

A.10.4.2.2.1	Test purpose and environment	2318

A.10.4.2.2.2	Test parameters	2318

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2318

A.10.4.2.3.1	Test Purpose and Environment	2318

A.10.4.2.3.2	Test Requirements	2323

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2324

A.10.4.2.4.1	Test Purpose and Environment	2324

A.10.4.2.4.2	Test Requirements	2328

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2329

A.10.4.2.5.1	Test Purpose and Environment	2329

A.10.4.2.5.2	Test Requirements	2333

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2334

A.10.4.2.6.1	Test Purpose and Environment	2334

A.10.4.2.6.2	Test Requirements	2338

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2339

A.10.4.2.7.1	Test Purpose and Environment	2339

A.10.4.2.7.2	Test Requirements	2345

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2345

A.10.4.2.8.1	Test Purpose and Environment	2345

A.10.4.2.8.2	Test Requirements	2351

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2352

A.10.4.2.9.1	Test Purpose and Environment	2352

A.10.4.2.9.2	Test Requirements	2357

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2357

A.10.4.2.10.1	Test Purpose and Environment	2357

A.10.4.2.10.2	Test Requirements	2363

A.10.4.3	L1-RSRP measurements for beam reporting	2364

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2364

A.10.4.3.1.1	Test Purpose and Environment	2364

A.10.4.3.1.2	Test parameters	2364

A.10.4.3.1.3	Test Requirements	2366

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2367

A.10.4.3.2.1	Test Purpose and Environment	2367

A.10.4.3.2.2	Test parameters	2367

A.10.4.3.2.3	Test Requirements	2369

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2370

A.10.4.3.3.1	Test Purpose and Environment	2370

A.10.4.3.3.2	Test parameters	2370

A.10.4.3.3.3	Test Requirements	2373

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2374

A.10.4.3.4.1	Test Purpose and Environment	2374

A.10.4.3.4.2	Test parameters	2374

A.10.4.3.4.3	Test Requirements	2377

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2378

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2378

A.10.4.4.1.1	Test Purpose and Environment	2378

A.10.4.4.1.2	Test Requirements	2382

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2383

A.10.4.4.2.1	Test Purpose and Environment	2383

A.10.4.4.2.2	Test Requirements	2388

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2389

A.10.4.4.3.1	Test Purpose and Environment	2389

A.10.4.4.3.2	Test Requirements	2393

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2394

A.10.4.4.4.1	Test Purpose and Environment	2394

A.10.4.4.4.2	Test Requirements	2399

A.10.5	Measurement performance	2400

A.10.5.1	SS-RSRP	2400

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2400

A.10.5.1.1.1	Test Purpose and Environment	2400

A.10.5.1.1.2	Test parameters	2400

A.10.5.1.1.3	Test Requirements	2403

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2403

A.10.5.1.2.1	Test Purpose and Environment	2403

A.10.5.1.2.2	Test parameters	2403

A.10.5.1.2.3	Test Requirements	2407

A.10.5.2	SS-RSRQ	2407

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2407

A.10.5.2.1.1	Test Purpose and Environment	2407

A.10.5.2.1.2	Test Parameters	2407

A.10.5.2.1.3	Test Requirements	2410

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2410

A.10.5.2.2.1	Test Purpose and Environment	2410

A.10.5.2.2.2	Test Parameters	2410

A.10.5.2.2.3	Test Requirements	2413

A.10.5.3	SS-SINR	2413

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	2413

A.10.5.3.1.1	Test Purpose and Environment	2413

A.10.5.3.1.2	Test Parameters	2413

A.10.5.3.1.3	Test Requirements	2416

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	2416

A.10.5.3.2.1	Test Purpose and Environment	2416

A.10.5.3.2.2	Test Parameters	2416

A.10.5.3.2.3	Test Requirements	2420

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	2420

A.10.5.3.3.1	Test Purpose and Environment	2420

A.10.5.3.3.2	Test Parameters	2420

A.10.5.3.3.3	Test Requirements	2423

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2423

A.10.5.4.1	SSB based L1-RSRP measurement	2423

A.10.5.4.1.1	Test Purpose and Environment	2423

A.10.5.4.1.2	Test parameters	2424

A.10.5.4.1.3	Test Requirements	2426

A.10.5.5	RSSI	2426

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2426

A.10.5.5.1.1	Test Purpose and Environment	2426

A.10.5.5.1.2	Test parameters	2426

A.10.5.5.1.3	Test Requirements	2429

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2429

A.10.5.5.2.1	Test Purpose and Environment	2429

A.10.5.5.2.2	Test parameters	2429

A.10.5.5.2.3	Test Requirements	2432

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2432

A.10.5.5.3.1	Test Purpose and Environment	2432

A.10.5.5.3.2	Test parameters	2432

A.10.5.5.3.3	Test Requirements	2436

A.10.5.6	Channel occupancy	2436

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2436

A.10.5.6.1.1	Test Purpose and Environment	2436

A.10.5.6.1.2	Test parameters	2436

A.10.5.6.1.3	Test Requirements	2440

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2440

A.10.5.6.2.1	Test Purpose and Environment	2440

A.10.5.6.2.2	Test parameters	2440

A.10.5.6.2.3	Test Requirements	2443

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2443

A.10.5.6.3.1	Test Purpose and Environment	2443

A.10.5.6.3.2	Test parameters	2443

A.10.5.6.3.3	Test Requirements	2447

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2447

A.11.1	RRC_IDLE state mobility	2447

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2447

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2447

A.11.1.1.1.1	Test Purpose and Environment	2447

A.11.1.1.1.2	Test Parameters	2448

A.11.1.1.1.3	Test Requirements	2451

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2451

A.11.1.1.2.1	Test Purpose and Environment	2451

A.11.1.1.2.2	Test Parameters	2451

A.11.1.1.2.3	Test Requirements	2455

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2455

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2455

A.11.1.2.1.1	Test Purpose and Environment	2455

A.11.1.2.1.2	Test Parameters	2456

A.11.1.2.1.3	Test Requirements	2462

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2463

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2463

A.11.1.3.1.1	Test Purpose and Environment	2463

A.11.1.3.1.2	Test Parameters	2463

A.11.1.3.1.3	Test Requirements	2468

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2469

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2469

A.11.1.4.1.1	Test Purpose and Environment	2469

A.11.1.4.1.2	Test Parameters	2469

A.11.1.4.1.3	Test Requirements	2472

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2473

A.11.1.4.2.1	Test Purpose and Environment	2473

A.11.1.4.2.2	Test Requirements	2475

A.11.2	RRC_CONNECTED state mobility	2476

A.11.2.1	Handover	2476

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2476

A.11.2.1.1.1	Test Purpose and Environment	2476

A.11.2.1.1.2	Test Parameters	2476

A.11.2.1.1.3 Test Requirements	2479

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2479

A.11.2.1.2.1	Test Purpose and Environment	2479

A.11.2.1.2.2	Test Parameters	2479

A.11.2.1.2.3	Test Requirements	2482

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2482

A.11.2.1.3.1	Test Purpose and Environment	2482

A.11.2.1.3.2	Test Parameters	2482

A.11.2.1.3.3 Test Requirements	2485

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2485

A.11.2.1.4.1	Test Purpose and Environment	2485

A.11.2.1.4.2	Test Parameters	2485

A.11.2.1.4.3 Test Requirements	2489

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2490

A.11.2.1.5.1	Test Purpose and Environment	2490

A.11.2.1.5.2	Test Parameters	2490

A.11.2.1.5.3 Test Requirements	2493

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2494

A.11.2.1.6.1	Test Purpose and Environment	2494

A.11.2.1.6.2	Test Parameters	2494

A.11.2.1.6.3 Test Requirements	2497

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2498

A.11.2.1.7.1	Test Purpose and Environment	2498

A.11.2.1.7.2	Test Requirements	2504

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2504

A.11.2.1.8.1	Test Purpose and Environment	2504

A.11.2.1.8.2	Test Requirements	2509

A.11.2.2	RRC connection mobility control	2509

A.11.2.2.1	RRC re-establishment	2509

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2509

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2512

A.11.2.2.1.3	Intra-frequency RRC Re-establishment with CCA in FR1 without serving cell timing	2516

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2519

A.11.2.2.2	Random Access	2524

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2524

A.11.2.2.2.1.1	Test Purpose and Environment	2524

A.11.2.2.2.1.2	Test Requirements	2527

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2527

A.11.2.2.2.1.2.2	Random Access Response Reception	2527

A.11.2.2.2.1.2.3	No Random Access Response Reception	2528

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2528

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2528

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2529

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2529

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2529

A.11.2.2.2.2.1	Test Purpose and Environment	2529

A.11.2.2.2.2.2	Test Requirements	2532

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2532

A.11.2.2.2.2.2.2	Random Access Response Reception	2533

A.11.2.2.2.2.2.3	No Random Access Response Reception	2533

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2533

A.11.2.2.2.3.1	Test Purpose and Environment	2533

A.11.2.2.2.3.2	Test Requirements	2537

A.11.2.2.2.3.2.1	MsgA Transmission	2537

A.11.2.2.2.3.2.2	MsgB Reception	2537

A.11.2.2.2.3.2.3	No MsgB Reception	2538

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2538

A.11.2.2.2.4.1	Test Purpose and Environment	2538

A.11.2.2.2.4.2	Test Requirements	2542

A.11.2.2.2.4.2.1	MsgA Transmission	2542

A.11.2.2.2.4.2.2	MsgB Reception	2543

A.11.2.2.2.4.2.3	No MsgB Reception	2543

A.11.2.2.3	RRC connection release with redirection	2544

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2544

A.11.3	Timing	2552

A.11.3.1	UE transmit timing	2552

A.11.3.2	UE timing advance	2556

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2556

A.11.3.2.1.1	Test Purpose and Environment	2556

A.11.3.2.1.2	Test Parameters	2556

A.11.3.2.1.3	Test Requirements	2560

A.11.4	Signalling characteristics	2560

A.11.4.1	Radio link monitoring	2560

A.11.4.1.1	Introduction	2560

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2561

A.11.4.1.2.1	Test purpose and environment	2561

A.11.4.1.2.2	Test requirements	2565

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2565

A.11.4.1.3.1	Test purpose and environment	2565

A.11.4.1.3.2	Test requirements	2571

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2571

A.11.4.1.4.1	Test purpose and environment	2571

A.11.4.1.4.2	Test requirements	2571

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2571

A.11.4.1.5.1	Test purpose and environment	2571

A.11.4.1.5.2	Test requirements	2571

A.11.4.2	Interruption	2572

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2572

A.11.4.2.1.1 Test Purpose and Environment	2572

A.11.4.2.1.2	Test Requirements	2576

A.11.4.3	SCell activation and deactivation delay	2576

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	2581

A.11.4.3.2.1	Test Purpose and Environment	2581

A.11.4.3.2.2	Test Requirements	2582

A.11.4.4	Beam failure detection and link recovery procedures	2583

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2583

A.11.4.4.1.1	Test Purpose and Environment	2583

A.11.4.4.1.2	Test Requirements	2589

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2589

A.11.4.4.2.1	Test Purpose and Environment	2589

A.11.4.4.2.2	Test Requirements	2595

A.11.4.5	Active BWP switching	2595

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2595

A.11.4.5.1.2	Test Requirements	2599

A.11.4.6	Void	2609

A.11.5	Measurement procedure	2609

A.11.5.1	Intra-frequency measurements	2609

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2609

A.11.5.1.1.1	Test purpose and environment	2609

A.11.5.1.1.2	Test parameters	2609

A.11.5.1.1.3	Test Requirements	2609

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2610

A.11.5.1.2.1	Test purpose and environment	2610

A.11.5.1.2.2	Test parameters	2610

A.11.5.1.2.3	Test Requirements	2610

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2610

A.11.5.1.3.1	Test purpose and environment	2610

A.11.5.1.3.2	Test parameters	2610

A.11.5.1.3.3	Test Requirements	2610

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2611

A.11.5.1.4.1	Test purpose and environment	2611

A.11.5.1.4.2	Test parameters	2611

A.11.5.1.4.3	Test Requirements	2611

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2611

A.11.5.1.5.1	Test purpose and environment	2611

A.11.5.1.5.2	Test parameters	2611

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2612

A.11.5.1.6.1	Test purpose and environment	2612

A.11.5.1.6.2	Test parameters	2612

A.11.5.1.6.3	Test Requirements	2612

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2612

A.11.5.1.7.1	Test purpose and environment	2612

A.11.5.1.7.2	Test parameters	2612

A.11.5.1.7.3	Test Requirements	2612

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2613

A.11.5.1.8.1	Test purpose and environment	2613

A.11.5.1.8.2	Test parameters	2613

A.11.5.1.8.3	Test Requirements	2613

A.11.5.1.9	RSSI measurement reporting on PCC	2613

A.11.5.1.9.1	Test purpose and environment	2613

A.11.5.1.9.2	Test parameters	2613

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2613

A.11.5.1.10.1	Test purpose and environment	2613

A.11.5.1.10.2	Test parameters	2614

A.11.5.1.11	RSSI measurement reporting on SCC	2614

A.11.5.1.11.1	Test purpose and environment	2614

A.11.5.1.11.2	Test parameters	2614

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2614

A.11.5.1.12.1	Test purpose and environment	2614

A.11.5.1.12.2	Test parameters	2614

A.11.5.2	Inter-frequency measurements	2615

A.11.5.2.1	RSSI measurement reporting	2615

A.11.5.2.1.1	Test purpose and environment	2615

A.11.5.2.1.2	Test parameters	2615

A.11.5.2.2	Channel occupancy measurement reporting	2615

A.11.5.2.2.1	Test purpose and environment	2615

A.11.5.2.2.2	Test parameters	2615

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2616

A.11.5.2.3.1	Test Purpose and Environment	2616

A.11.5.2.3.2	Test Requirements	2620

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2621

A.11.5.2.4.1	Test Purpose and Environment	2621

A.11.5.2.4.2	Test Requirements	2626

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2626

A.11.5.2.5.1	Test Purpose and Environment	2626

A.11.5.2.5.2	Test Requirements	2630

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2631

A.11.5.2.6.1	Test Purpose and Environment	2631

A.11.5.2.6.2	Test Requirements	2636

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2636

A.11.5.2.7.1	Test Purpose and Environment	2636

A.11.5.2.7.2	Test Requirements	2640

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2641

A.11.5.2.8.1	Test Purpose and Environment	2641

A.11.5.2.8.2	Test Requirements	2646

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2646

A.11.5.2.9.1	Test Purpose and Environment	2646

A.11.5.2.9.2	Test Requirements	2651

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2652

A.11.5.2.10.1	Test Purpose and Environment	2652

A.11.5.2.10.2	Test Requirements	2657

A.11.5.3	Inter-RAT E-UTRAN measurements	2658

A.11.5.4	L1-RSRP measurements for beam reporting	2658

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2658

A.11.5.4.1.1	Test Purpose and Environment	2658

A.11.5.4.1.2	Test parameters	2658

A.11.5.4.1.3	Test Requirements	2660

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2660

A.11.5.4.2.1	Test Purpose and Environment	2660

A.11.5.4.2.2	Test parameters	2661

A.11.5.4.2.3	Test Requirements	2663

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2663

A.11.5.4.3.1	Test Purpose and Environment	2663

A.11.5.4.3.2	Test parameters	2664

A.11.5.4.3.3	Test Requirements	2667

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2668

A.11.5.4.4.1	Test Purpose and Environment	2668

A.11.5.4.4.2	Test parameters	2668

A.11.5.4.4.3	Test Requirements	2671

A.11.6	Measurement performance	2672

A.11.6.1	SS-RSRP	2672

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2672

A.11.6.1.1.1	Test Purpose and Environment	2672

A.11.6.1.1.2	Test parameters	2672

A.11.6.1.1.3	Test Requirements	2674

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2674

A.11.6.1.2.1	Test Purpose and Environment	2674

A.11.6.1.2.2	Test parameters	2674

A.11.6.1.2.3	Test Requirements	2676

A.11.6.2	SS-RSRQ	2676

A.11.6.2.1	Intra-frequency measurement accuracy	2676

A.11.6.2.1.1	Test Purpose and Environment	2676

A.11.6.2.1.2	Test Parameters	2676

A.11.6.2.1.3	Test Requirements	2679

A.11.6.2.2	Inter-frequency measurement accuracy	2679

A.11.6.2.2.1	Test Purpose and Environment	2679

A.11.6.2.2.2	Test Parameters	2679

A.11.6.2.2.3	Test Requirements	2682

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	2682

A.11.6.2.3.1	Test Purpose and Environment	2682

A.11.6.2.3.2	Test Parameters	2682

A.11.6.2.3.3	Test Requirements	2685

A.11.6.2.4	Inter-frequency measurement accuracy	2685

A.11.6.2.4.1	Test Purpose and Environment	2685

A.11.6.2.4.2	Test Parameters	2685

A.11.6.2.4.3	Test Requirements	2694

A.11.6.3	SS-SINR	2694

A.11.6.3.1	Intra-frequency measurement accuracy	2694

A.11.6.3.1.1	Test Purpose and Environment	2694

A.11.6.3.1.2	Test Parameters	2694

A.11.6.3.1.3	Test Requirements	2697

A.11.6.3.2	Inter-frequency measurement accuracy	2697

A.11.6.3.2.1	Test Purpose and Environment	2697

A.11.6.3.2.2	Test Parameters	2697

A.11.6.3.2.3	Test Requirements	2700

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	2700

A.11.6.3.3.1	Test Purpose and Environment	2700

A.11.6.3.3.2	Test Parameters	2700

A.11.6.3.3.3	Test Requirements	2703

A.11.6.3.4	Inter-frequency measurement accuracy	2703

A.11.6.3.4.1	Test Purpose and Environment	2703

A.11.6.3.4.2	Test Parameters	2703

A.11.6.3.4.3	Test Requirements	2711

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2711

A.11.6.4.1	SSB based L1-RSRP measurement	2711

A.11.6.4.1.1	Test Purpose and Environment	2711

A.11.6.4.1.2	Test parameters	2712

A.11.6.4.1.3	Test Requirements	2715

A.11.6.5	RSSI	2715

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2715

A.11.6.5.1.1	Test Purpose and Environment	2715

A.11.6.5.1.2	Test parameters	2715

A.11.6.5.1.3	Test Requirements	2718

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2718

A.11.6.5.2.1	Test Purpose and Environment	2718

A.11.6.5.2.2	Test parameters	2718

A.11.6.5.2.3	Test Requirements	2721

A.11.6.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2721

A.11.6.5.3.1	Test Purpose and Environment	2721

A.11.6.5.3.2	Test parameters	2721

A.11.6.5.3.3	Test Requirements	2724

A.11.6.6	Channel occupancy	2724

A.11.6.6.1 	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2724

A.11.6.6.1.1	Test Purpose and Environment	2724

A.11.6.6.1.2	Test parameters	2724

A.11.6.6.1.3	Test Requirements	2728

A.11.6.6.2 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2728

A.11.6.6.2.1	Test Purpose and Environment	2728

A.11.6.6.2.2	Test parameters	2728

A.11.6.6.2.3	Test Requirements	2731

A.11.6.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2731

A.11.6.6.3.1	Test Purpose and Environment	2731

A.11.6.6.3.2	Test parameters	2731

A.11.6.6.3.3	Test Requirements	2734

A.11.6.7	E-UTRAN RSRP	2734

A.11.6.8	E-UTRAN RSRQ	2734

A.11.6.9	E-UTRAN SINR	2734

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2734

A.12.1	RRC_IDLE state mobility	2734

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2734

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2734

A.12.1.1.1.1	Test Purpose and Environment	2734

A.12.1.1.1.2	Test Requirements	2740

A.12.2	RRC_CONNECTED state mobility	2741

A.12.2.1	Handover	2741

A.12.2.1.1	E-UTRAN - NR with CCA handover	2741

A.12.2.1.1.1	Test Purpose and Environment	2741

A.12.2.1.1.2	Test Requirements	2746

A.12.3	Signalling characteristics	2747

A.12.3.1	Interruptions	2747

A.12.4	Measurement procedure	2747

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2747

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2747

A.12.4.1.1.1	Test Purpose and Environment	2747

A.12.4.1.1.2	Test Requirements	2751

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2751

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2751

A.12.4.2.1.1	Test Purpose and Environment	2751

A.12.4.2.1.2	Test Requirements	2755

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2756

A.12.4.2.2.1	Test Purpose and Environment	2756

A.12.4.2.2.2	Test Requirements	2760

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2760

A.12.4.2.3.1	Test Purpose and Environment	2760

A.12.4.2.3.2	Test Requirements	2764

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2765

A.12.4.2.4.1	Test Purpose and Environment	2765

A.12.4.2.4.2	Test Requirements	2769

A.12.4.2.5	RSSI measurement reporting	2769

A.12.4.2.5.1	Test purpose and environment	2769

A.12.4.2.5.2	Test parameters	2769

A.12.4.2.6	Channel occupancy measurement reporting	2770

A.12.4.2.6.1	Test purpose and environment	2770

A.12.4.2.6.2	Test parameters	2770

A.12.5	Measurement performance	2770

A.12.5.1	E-UTRANNR SFTD	2770

A.12.5.2	E-UTRANNR SS-RSRP	2776

A.12.5.3	E-UTRANNR SS-RSRQ	2776

A.12.5.4	E-UTRANNR SS-SINR	2776

A.12.5.5	E-UTRANNR RSSI	2776

A.12.5.6	E-UTRANNR channel occupancy	2776

A.13	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2776

A.13.1	Timing	2776

A.13.1.1	UE transmit timing	2776

A.13.1.2	Timing advance	2776

A.13.2	Signalling characteristics	2776

A.13.2.1	Interruption	2776

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2776

A.13.2.1.1.1	Test Purpose and Environment	2776

A.13.2.1.1.2	Test Requirements	2780

A.13.2.2	SCell activation and deactivation delay	2780

A.13.2.2.2	SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	2785

A.13.2.2.2.1	Test Purpose and Environment	2785

A.13.2.2.2.2	Test Requirements	2786

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	2786

A.13.2.2.3.1	Test Purpose and Environment	2786

A.13.2.2.3.2	Test Requirements	2787

A.13.2.3	Void	2787

A.13.3	Measurement procedure	2787

A.13.3.1	Intra-frequency measurements	2787

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2787

A.13.3.1.1.1	Test purpose and environment	2787

A.13.3.1.1.2	Test parameters	2787

A.13.3.1.1.3	Test Requirements	2792

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2792

A.13.3.1.2.1	Test purpose and environment	2792

A.13.3.1.2.2	Test parameters	2792

A.13.3.1.2.3	Test Requirements	2797

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2797

A.13.3.1.3.1	Test purpose and environment	2797

A.13.3.1.3.2	Test parameters	2797

A.13.3.1.3.3	Test Requirements	2802

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2802

A.13.3.1.4.1	Test purpose and environment	2802

A.13.3.1.4.2	Test parameters	2802

A.13.3.1.4.3	Test Requirements	2807

A.13.3.1.5	RSSI measurement reporting on SCC	2807

A.13.3.1.5.1	Test purpose and environment	2807

A.13.3.1.5.2	Test parameters	2807

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2809

A.13.3.1.6.1	Test purpose and environment	2809

A.13.3.1.6.2	Test parameters	2810

A.13.3.2	Inter-frequency measurements	2810

A.13.3.2.1	RSSI measurement reporting	2810

A.13.3.2.1.1	Test purpose and environment	2810

A.13.3.2.1.2	Test parameters	2810

A.13.3.2.2	Channel occupancy measurement reporting	2811

A.13.3.2.2.1	Test purpose and environment	2811

A.13.3.2.2.2	Test parameters	2811

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2811

A.13.3.2.3.1	Test Purpose and Environment	2811

A.13.3.2.3.2	Test Requirements	2817

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2817

A.13.3.2.4.1	Test Purpose and Environment	2817

A.13.3.2.4.2	Test Requirements	2822

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2823

A.13.3.2.5.1	Test Purpose and Environment	2823

A.13.3.2.5.2	Test Requirements	2828

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2828

A.13.3.2.6.1	Test Purpose and Environment	2828

A.13.3.2.6.2	Test Requirements	2834

A.13.3.3	L1-RSRP measurements for beam reporting	2835

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2835

A.13.3.3.1.1	Test Purpose and Environment	2835

A.13.3.3.1.2	Test parameters	2835

A.13.3.3.1.3	Test Requirements	2840

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2840

A.13.3.3.2.1	Test Purpose and Environment	2840

A.13.3.3.2.2	Test parameters	2840

A.13.3.3.2.3	Test Requirements	2844

A.13.4	Measurement performance	2844

A.13.4.1	SS-RSRP	2844

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2844

A.13.4.1.1.1	Test Purpose and Environment	2844

A.13.4.1.1.2	Test parameters	2844

A.13.4.1.1.3	Test Requirements	2846

A.13.4.2	SS-RSRQ	2846

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	2846

A.13.4.2.1.1	Test Purpose and Environment	2846

A.13.4.2.1.2	Test Parameters	2846

A.13.4.2.1.3	Test Requirements	2854

A.13.4.3	SS-SINR	2854

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	2854

A.13.4.3.1.1	Test Purpose and Environment	2854

A.13.4.3.1.2	Test Parameters	2854

A.13.4.3.1.3	Test Requirements	2860

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2861

A.13.4.4.1	SSB based L1-RSRP measurement	2861

A.13.4.4.1.1	Test Purpose and Environment	2861

A.13.4.4.1.2	Test parameters	2861

A.13.4.4.1.3	Test Requirements	2865

A.13.4.5	RSSI	2865

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2865

A.13.4.5.1.1	Test Purpose and Environment	2865

A.13.4.5.1.2	Test parameters	2865

A.13.4.5.1.3	Test Requirements	2869

A.13.4.5.2	  Inter-frequency RSSI measurement accuracy on a carrier with CCA	2869

A.13.4.5.2.1	Test Purpose and Environment	2869

A.13.4.5.2.2	Test parameters	2869

A.13.4.5.2.3	Test Requirements	2873

A.13.4.6	Channel occupancy	2873

A.13.4.6.1 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2873

A.13.4.6.1.1	Test Purpose and Environment	2873

A.13.4.6.1.2	Test parameters	2873

A.13.4.6.1.3	Test Requirements	2877

A.13.4.6.2	  Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2877

A.13.4.6.2.1	Test Purpose and Environment	2877

A.13.4.6.2.2	Test parameters	2877

A.13.4.6.2.3	Test Requirements	2881

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1818

B.1	Conditions for NR RRC_IDLE state mobility	1818

B.1.1	Introduction	1818

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1818

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1819

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1820

B.2.1	Introduction	1820

B.2.1.1	General	1820

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1820

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1820

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1820

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1821

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1821

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1822

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1822

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	1822

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	1822

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1823

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1823

B.2.2	Conditions for NR intra-frequency measurements	1824

B.2.3	Conditions for NR inter-frequency measurements	1825

B.2.4	Conditions for NR L1-RSRP reporting	1826

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1826

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1827

B.2.5	Conditions for RRC connection release with redirection to NR	1829

B.2.6	Void	1830

B.2.6.1	Void	1830

B.2.6.2	Void	1830

B.2.7	Conditions for SRS-RSRP measurements	1830

B.2.8	Conditions for NR L1-SINR reporting	1831

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1831

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1832

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1832

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1832

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1834

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1834

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1835

B.2.9	Conditions for NR intra-frequency measurements under CCA	1836

B.2.10	Conditions for NR inter-frequency measurements under CCA	1836

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1836

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1836

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1837

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1838

B.2.14	Conditions for NR PRS-based measurements	1840

B.3	RRM Requirements Exceptions	1841

B.3.1	Introduction	1841

B.3.2	Receiver sensitivity relaxation for CA	1841

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1841

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1842

B.3.2.2.1	Inter-band carrier aggregation	1842

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1842

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1842

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1842

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1842

B.3.2.4.1	Intra-band contiguous carrier aggregation	1842

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1843

B.3.3	Receiver sensitivity relaxation for DC	1843

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1843

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1843

B.3.4	Receiver sensitivity relaxation for SUL	1843

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1843

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1843

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1843

B.4	Conditions for V2X	1843

B.4.1	Test parameters for GNSS signals	1843

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1844

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1844

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1844

Annex C (informative): Change history	1846