| 3GPP TS 38.133 V17.6.0 (2022-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	52

3.6.1	RRC connected state requirements in DRX	53

3.6.2	Number of serving carriers	53

3.6.2.1	Number of serving carriers for SA	53

3.6.2.2	Number of serving carriers for EN-DC	53

3.6.2.3	Number of serving carriers for NE-DC	53

3.6.2.4	Number of serving carriers for NR-DC	54

3.6.3	Applicability for intra-band FR2	54

3.6.4	Applicability for FR2 UE power classes	54

3.6.5	Applicability for SDL bands	54

3.6.6	Applicability of requirements for NGEN-DC operation	54

3.6.7	Applicability of QCL	54

3.6.9	Applicability of requirements for scheduling availability	55

3.6.10	Applicability of requirements for measurement restrictions	55

3.6.11	Applicability of requirements for Redcap UEs	55

3.6.11.1	RRC connected state requirements in DRX	55

3.6.11.2	Applicability for FR2 Redcap UE power classes	55

3.6.11.3	Applicability of QCL	55

3.6.12	Applicability of requirements for Satellite Access	56

3.6.13	Applicability of requirements for FR2	56

3.6.14	Applicability of requirements for FR2 Power Class 6	56

4	SA: RRC_IDLE state mobility	56

4.1	Cell Selection	56

4.2	Cell Re-selection	56

4.2.1	Introduction	56

4.2.2	Requirements	57

4.2.2.1	UE measurement capability	57

4.2.2.2	Measurement and evaluation of serving cell	57

4.2.2.3	Measurements of intra-frequency NR cells	57

4.2.2.4	Measurements of inter-frequency NR cells	59

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	61

4.2.2.6	Maximum interruption in paging reception	63

4.2.2.7	General requirements	63

4.2.2.8	Minimum requirement at transitions	64

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	64

4.2.2.9.1	Introduction	64

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	64

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	65

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	66

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	66

4.2.2.10.1	Introduction	66

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	66

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	67

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	68

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	69

4.2.2.11.1	Introduction	69

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	69

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	70

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	71

4.2A	Cell Re-selection when subject to CCA	71

4.2A.1	Introduction	71

4.2A.2	Requirements	72

4.2A.2.1	UE measurement capability	72

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	72

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	73

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	74

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	76

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	76

4.2A.2.7	General requirements	77

4.2B	Cell Re-selection for RedCap	77

4.2B.1	Introduction	77

4.2B.2	Requirements	77

4.2B.2.1	UE measurement capability for RedCap	77

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	77

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	77

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	77

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	79

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	82

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	85

4.2B.2.6	Maximum interruption in paging reception for RedCap	86

4.2B.2.7	General requirements for RedCap	87

4.2B.2.8	Minimum requirement at transitions	87

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	88

4.2B.2.9.1	Introduction	88

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	88

4.2B.2.9.3	Measurements for a UE fulfilling stationary not at cell edge criteria	91

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	91

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	91

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	92

4.2B.2.9.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	92

4.2B.2.9.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	92

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	92

4.2B.2.10.1	Introduction	92

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	93

4.2B.2.10.3	Measurements for a UE fulfilling stationary not at cell edge criterion	95

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	95

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	96

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	96

4.2B.2.10.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	96

4.2B.2.10.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	97

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	97

4.2B.2.11.1	Introduction	97

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	97

4.2B.2.11.3	Measurements for a UE fulfilling stationary not at cell edge criterion	99

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	99

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	99

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	100

4.2B.2.11.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	100

4.2B.2.11.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	100

4.2C	Cell Re-selection for NR UE for Satellite Access	101

4.2C.1	Introduction	101

4.2C.2	Requirements	101

4.2C.2.1	UE measurement capability	101

4.2C.2.2	Measurement and evaluation of serving cell	101

4.2C.2.3	Measurements of intra-frequency NR cells	102

4.2C.2.4	Measurements of inter-frequency NR cells	104

4.2C.2.5	Maximum interruption in paging reception	107

4.2C.2.6	Minimum requirement at transitions	107

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	107

4.3	Minimization of Drive Tests (MDT)	107

4.3.1	Introduction	107

4.3.2	Measurement Requirements	108

4.3.3	Requirements for Relative Time Stamp Accuracy	108

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	108

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	109

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	109

4.3C.1	Introduction	109

4.3C.2	Measurement Requirements	109

4.3C.3	Requirements for Relative Time Stamp Accuracy	110

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	110

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	110

4.4	Idle Mode CA/DC Measurements	111

4.4.1	Introduction	111

4.4.2	Measurement Requirements	111

4.4.2.1	Detected cell requirement during state transition and Idle mode	111

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	112

4.4.2.3	Measurements on serving cell	113

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	113

5	SA: RRC_INACTIVE state mobility	114

5.1	Cell Re-selection	114

5.1.1	Introduction	114

5.1.2	Requirements	114

5.1.2.1	UE measurement capability	114

5.1.2.2	Measurement and evaluation of serving cell	114

5.1.2.3	Measurements of intra-frequency NR cells	114

5.1.2.4	Measurements of inter-frequency NR cells	114

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	114

5.1.2.6	Maximum interruption in paging reception	114

5.1.2.7	General requirements	115

5.1A	Cell Re-selection with CCA	115

5.1A.1	Introduction	115

5.1A.2	Requirements	115

5.1A.2.1	UE measurement capability	115

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	115

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	115

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	115

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	115

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	115

5.1A.2.7	General requirements	115

5.1B	Cell Re-selection for RedCap	116

5.1B.1	Introduction	116

5.1B.2	Requirements	116

5.1B.2.1	UE measurement capability	116

5.1B.2.2	Measurement and evaluation of serving cell	116

5.1B.2.3	Measurements of intra-frequency NR cells	116

5.1B.2.4	Measurements of inter-frequency NR cells	117

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	118

5.1B.2.6	Maximum interruption in paging reception	118

5.1B.2.7	General requirements	118

5.1B.2.8	Minimum requirement at transitions	118

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	118

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	118

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	118

5.2	Void	119

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	119

5.2B.1	Introduction	119

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	119

5.2B.2.2	Scheduling restriction	120

5.3	Minimization of Drive Tests (MDT)	120

5.3.1	Introduction	120

5.3.2	Measurement Requirements	120

5.3.3	Requirements for Relative Time Stamp Accuracy	120

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	120

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	120

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	121

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	121

5.3C.1	Introduction	121

5.3C.2	Measurement Requirements	121

5.3C.3	Requirements for Relative Time Stamp Accuracy	121

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	121

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	122

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	122

5.4	Idle Mode CA/DC Measurements	122

5.4.1	Introduction	122

5.4.2	Measurement Requirements	122

5.4.2.1	Detected cell requirement during state transition and Idle mode	122

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	122

5.4.2.3	Measurements on serving cell	122

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	122

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	122

5.5.1 Introduction	122

5.5.2	Requirements on UE synchronization for small data transmissions	123

5.5.3	TA validation requirements	123

5.5.4	Scheduling restriction	124

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	124

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	124

5.5.4.3	Scheduling availability of UE performing measurements on FR2	124

5.5.5	Applicability conditions for SDT	125

5.6	NR measurements for positioning	125

5.6.1	Introduction	125

5.6.2	RSTD measurements	126

5.6.2.1	Introduction	126

5.6.2.2	Requirements Applicability	126

5.6.2.3	Measurement Capability	126

5.6.2.5	Measurements Period Requirements	126

5.6.3	PRS-RSRP measurements	129

5.6.3.1	Introduction	129

5.6.3.2	Requirements applicability	129

5.6.3.3	Measurement Capability	129

5.6.3.4	Measurement Reporting Requirements	129

5.6.3.5	Measurement Period Requirements	130

5.6.4	UE Rx-Tx time difference measurements	132

5.6.4.1	Introduction	132

5.6.4.2	Requirements Applicability	132

5.6.4.3	Measurement Capability	132

5.6.4.4	Measurement Reporting Requirements	132

5.6.4.5	Measurement Period Requirements	133

5.6.5	PRS-RSRPP measurements	135

5.6.5.1	Introduction	135

5.6.5.2	Requirements applicability	135

5.6.5.3	Measurement capability	136

5.6.5.4	Measurement reporting requirements	136

5.6.5.5	Measurement period requirements	136

5.7	Random access based Small Data Transmissions (RA-SDT)	138

5.7.1	Introduction	138

5.7.2	Requirements for small data transmissions based on 2-step RA	138

5.7.3	Requirements for small data transmissions based on 4-step RA	138

6	RRC_CONNECTED state mobility	139

6.1	Handover	139

6.1.1	NR Handover	139

6.1.1.1	Introduction	139

6.1.1.2	NR FR1 - NR FR1 Handover	139

6.1.1.2.1	Handover delay	139

6.1.1.2.2	Interruption time	139

6.1.1.3	NR FR2- NR FR1 Handover	140

6.1.1.3.1	Handover delay	140

6.1.1.3.2	Interruption time	140

6.1.1.4	NR FR2- NR FR2 Handover	140

6.1.1.4.1	Handover delay	141

6.1.1.4.2	Interruption time	141

6.1.1.5	NR FR1- NR FR2 Handover	141

6.1.1.5.1	Handover delay	142

6.1.1.5.2	Interruption time	142

6.1.2	NR Handover to other RATs	143

6.1.2.1	NR – E-UTRAN Handover	143

6.1.2.1.1	Introduction	143

6.1.2.1.2	Handover delay	143

6.1.2.1.3	Interruption time	143

6.1.2.2	NR – UTRAN Handover	143

6.1.2.2.1	Introduction	143

6.1.2.2.2	Handover delay	143

6.1.2.2.3	Interruption time	144

6.1.3	NR DAPS Handover	144

6.1.3.1	Introduction	144

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	145

6.1.3.2.1	DAPS handover delay	145

6.1.3.2.2	Interruption time	146

6.1.3.3	NR FR2- NR FR1 DAPS Handover	147

6.1.3.3.1	DAPS handover delay	148

6.1.3.3.2	Interruption time	148

6.1.3.4	NR FR1- NR FR2 DAPS Handover	149

6.1.3.4.1	DAPS handover delay	149

6.1.3.4.2	Interruption time	149

6.1.4	NR Conditional Handover	150

6.1.4.1	Introduction	150

6.1.4.2	NR FR1 – NR FR1 conditional handover	150

6.1.4.3	NR FR2 – NR FR1 conditional handover	151

6.1.4.4	NR FR2 – NR FR2 conditional handover	151

6.1.4.4.1	Handover delay	151

6.1.4.4.2	Measurement time	152

6.1.4.4.3	Preparation time	152

6.1.4.4.4	Interruption time	152

6.1.4.5	NR FR1 – NR FR2 conditional handover	153

6.1.5	NR Handover with PSCell	153

6.1.5.1	Introduction	153

6.1.5.2	Handover with PSCell from NR SA to EN-DC	153

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	153

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	154

6.1.5.3	HO with PSCell from NE-DC to NE-DC	154

6.1.5.3.1	Handover delay	154

6.1.5.3.2	HO with PSCell - PCell Interruption time	155

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	155

6.1.5.4	HO with PSCell from NR-DC to NR-DC	155

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	156

6.1.5.5.1	Introduction	156

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	157

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	157

6.1A	Void	158

6.1A.1	Void	158

6.1A.1.1	Void	158

6.1A.1.2	Void	158

6.1A.1.2.1	Void	158

6.1A.1.2.2	Void	158

6.1B	Handover to target cell using CCA	158

6.1B.1	NR Handover	158

6.1B.1.1	Introduction	158

6.1B.1.2	NR FR1 - NR FR1 Handover	159

6.1B.1.2.1	Handover delay	159

6.1B.1.2.2	Interruption time	159

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	160

6.1B.1.3.1	Handover delay	160

6.1B.1.3.2	Interruption time	160

6.1B.1.4	NR FR1- NR FR2-2 Handover	161

6.1B.1.4.1	Handover delay	161

6.1B.1.4.2	Interruption time	161

6.1C	Handover for SAN	162

6.1C.1	NR SAN Handover	162

6.1C.1.1	Introduction	162

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	162

6.1C.1.2.1	Handover delay	162

6.1C.1.2.2	Interruption time	162

6.1C.2	NR SAN Conditional Handover	163

6.1C.2.1	Introduction	163

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	163

6.1C.2.2.1	Handover delay	163

6.1C.2.2.2	Measurement time	164

6.1C.2.2.3	Preparation time	165

6.1C.2.2.4	Interruption time	165

6.1D	Handover for RedCap	165

6.1D.1	NR Handover	165

6.1D.1.1	Introduction	165

6.1D.1.2	NR FR1 - NR FR1 Handover	166

6.1D.1.3	NR FR2- NR FR2 Handover	166

6.1D.2	NR Handover to other RATs	166

6.1D.2.1	NR – E-UTRAN Handover	166

6.2	RRC Connection Mobility Control	166

6.2.1	SA: RRC Re-establishment	166

6.2.1.1	Introduction	166

6.2.1.2	Requirements	167

6.2.1.2.1	UE Re-establishment delay requirement	167

6.2.1A	RRC Re-establishment with CCA	168

6.2.1A.1	Introduction	168

6.2.1A.2	Requirements	169

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	169

6.2.1B	SA: RRC Re-establishment for RedCap	171

6.2.1B.1	Introduction	171

6.2.1B.2	Requirements	171

6.2.2	Random access	171

6.2.2.1	Introduction	171

6.2.2.2	Requirements for 4-step RA type	171

6.2.2.2.1	Contention based random access	172

6.2.2.2.2	Non-Contention based random access	173

6.2.2.2.3	UE behaviour when configured with supplementary UL	174

6.2.2.3	Requirements for 2-step RA type	174

6.2.2.3.1	Contention based random access	174

6.2.2.3.2	Non-Contention based random access	175

6.2.2.3.3	UE behaviour when configured with supplementary UL	175

6.2.2A	Random access when CCA is used on target frequency	176

6.2.2A.1	Introduction	176

6.2.2A.2	Requirements for 4-step RA type	176

6.2.2A.2.1	Contention based random access	176

6.2.2A.2.2	Non-Contention based random access	177

6.2.2A.3	Requirements for 2-step RA type	178

6.2.2A.3.1	Contention based random access	179

6.2.2A.3.2	Non-Contention based random access	180

6.2.2B	Random access for RedCap	180

6.2.2B.1	Introduction	180

6.2.2B.2	Requirements	181

6.2.3	SA: RRC Connection Release with Redirection	181

6.2.3.1	Introduction	181

6.2.3.2	Requirements	181

6.2.3.2.1	RRC connection release with redirection to NR	181

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	182

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	183

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	184

6.2.3A.1	Introduction	184

6.2.3A.2	Requirements	184

6.2.3A.2.1	RRC connection release with redirection to NR	184

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	184

6.2C	RRC Connection Mobility Control for Satellite Access	184

6.2C.1	SA: RRC Re-establishment for Satellite Access	184

6.2C.1.1	Introduction	184

6.2C.1.2	Requirements	185

6.2C.1.2.1	UE Re-establishment delay requirement	185

6.2C.2	Random access for satellite access	186

6.2C.2.1	Introduction	186

6.2C.2.2	Requirements for 4-step RA type	186

6.2C.2.2.1	Contention based random access	186

6.2C.2.2.2	Non-Contention based random access	187

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	187

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	188

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	188

6.2C.2.3	Requirements for 2-step RA type	188

6.2C.2.3.1	Contention based random access	189

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	189

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	189

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	189

6.2C.2.3.2	Non-Contention based random access	189

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	189

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	190

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	190

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	190

6.2C.3.1	Introduction	190

6.2C.3.2	Requirements	190

6.2C.3.2.1	RRC connection release with redirection to NR	190

7	Timing	191

7.1	UE transmit timing	191

7.1.1	Introduction	191

7.1.2	Requirements	191

7.1.2.1	Gradual timing adjustment	193

7.1.2.2	Void	194

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	194

7.1A	UE transmit timing for RedCap	194

7.1A.1	Introduction	194

7.1A.2	Requirements	194

7.1A.2.1	Gradual timing adjustment	195

7.1C	UE transmit timing for Satellite Access	196

7.1C.1	Introduction	196

7.1C.2	Requirements	196

7.1C.2.1	Gradual timing adjustment	197

7.2	UE timer accuracy	197

7.2.1	Introduction	197

7.2.2	Requirements	197

7.2A	UE timer accuracy for RedCap	198

7.2A.1	Introduction	198

7.2A.2	Requirements	198

7.2C	UE timer accuracy for satellite access	198

7.2C.1	Introduction	198

7.2C.2	Requirements	198

7.3	Timing advance	199

7.3.1	Introduction	199

7.3.2	Requirements	199

7.3.2.1	Timing Advance adjustment delay	199

7.3.2.2	Timing Advance adjustment accuracy	199

7.3A	Timing Advance for RedCap	199

7.3A.1	Introduction	199

7.3A.2	Requirements	200

7.3A.2.1	Timing Advance adjustment delay	200

7.3A.2.2	Timing Advance adjustment accuracy	200

7.3C	Timing advance for satellite access	200

7.3C.1	Introduction	200

7.3C.2	Requirements	200

7.3C.2.1	Timing Advance adjustment delay	200

7.3C.2.2	Timing Advance adjustment accuracy	200

7.4	Cell phase synchronization accuracy	201

7.4.1	Definition	201

7.4.2	Minimum requirements	201

7.5	Maximum Transmission Timing Difference	201

7.5.1	Introduction	201

7.5.2	Minimum Requirements for inter-band EN-DC	201

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	201

7.5.3	Minimum Requirements for intra-band EN-DC	202

7.5.4	Minimum Requirements for NR Carrier Aggregation	202

7.5.5	Minimum Requirements for inter-band NE-DC	203

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	203

7.5.6	Minimum Requirements for inter-band NR DC	203

7.6	Maximum Receive Timing Difference	204

7.6.1	Introduction	204

7.6.2	Minimum Requirements for inter-band EN-DC	204

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	205

7.6.3	Minimum Requirements for intra-band EN-DC	205

7.6.4	Minimum Requirements for NR Carrier Aggregation	206

7.6.5	Minimum Requirements for inter-band NE-DC	206

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	207

7.6.6	Minimum Requirements for inter-band NR DC	207

7.7	deriveSSB-IndexFromCell tolerance	208

7.7.1	Minimum requirements	208

7.8	Void	208

7.9	deriveSSB-IndexFromCell tolerance	208

7.9.1	Minimum requirements	208

8	Signalling characteristics	209

8.1	Radio Link Monitoring	209

8.1.1	Introduction	209

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	210

8.1.2	Requirements for SSB based radio link monitoring	211

8.1.2.1	Introduction	211

8.1.2.2	Minimum requirement	211

8.1.2.3	Measurement restrictions for SSB based RLM	215

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	215

8.1.3	Requirements for CSI-RS based radio link monitoring	216

8.1.3.1	Introduction	216

8.1.3.2	Minimum requirement	217

8.1.3.3	Measurement restrictions for CSI-RS based RLM	220

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	221

8.1.4	Minimum requirement at transitions	222

8.1.5	Minimum requirement for UE turning off the transmitter	222

8.1.6	Minimum requirement for L1 indication	222

8.1.7	Scheduling availability of UE during radio link monitoring	223

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	223

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	223

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	223

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	224

8.1A	Radio Link Monitoring with CCA on Target Frequency	224

8.1A.1	Introduction	224

8.1A.2	Requirements for SSB Based Radio Link Monitoring	225

8.1A.2.1	Introduction	225

8.1A.2.2	Minimum Requirement	226

8.1A.3	Minimum requirement at transitions	229

8.1A.4	Minimum requirement for UE turning off the transmitter	229

8.1A.5	Minimum requirement for L1 indication	229

8.1A.6	Scheduling availability of UE during radio link monitoring	230

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	230

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	231

8.1B	Radio Link Monitoring for RedCap	231

8.1B.1	Introduction	231

8.1B.2	Requirements for SSB based radio link monitoring	232

8.1B.2.1	Introduction	232

8.1B.2.2	Minimum requirement	233

8.1B.2.3	Measurement restrictions for SSB based RLM	235

8.1B.3	Requirements for CSI-RS based radio link monitoring	235

8.1B.3.1	Introduction	235

8.1B.3.2	Minimum requirement	236

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	239

8.1B.4	Minimum requirement at transitions	239

8.1B.5	Minimum requirement for UE turning off the transmitter	239

8.1B.6	Minimum requirement for L1 indication	240

8.1B.7	Scheduling availability of UE during radio link monitoring	240

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	240

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	240

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	241

8.1C	Radio Link Monitoring for Satellite Access	241

8.1C.1	Introduction	241

8.1C.2	Requirements for SSB based radio link monitoring	242

8.1C.2.1	Introduction	242

8.1C.2.2	Minimum requirement	243

8.1C.2.3	Measurement restrictions for SSB based RLM	243

8.1C.3	Requirements for CSI-RS based radio link monitoring	244

8.1C.3.1	Introduction	244

8.1C.3.2	Minimum requirement	245

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	245

8.1C.4	Minimum requirement at transitions	246

8.1C.5	Minimum requirement for UE turning off the transmitter	246

8.1C.6	Minimum requirement for L1 indication	246

8.1C.7	Scheduling availability of UE during radio link monitoring	246

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	247

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	247

8.2	Interruption	247

8.2.1	EN-DC Interruption	247

8.2.1.1	Introduction	247

8.2.1.2	Requirements	248

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	248

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	248

8.2.1.2.3	Interruptions at SCell addition/release	248

8.2.1.2.4	Interruptions at SCell activation/deactivation	249

8.2.1.2.5	Interruptions during measurements on SCC	250

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	251

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	251

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	251

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	252

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	252

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	253

8.2.1.2.9	Interruptions at SCell hibernation	254

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	254

8.2.1.2.11	Interruptions due to UE-specific CBW change	254

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	254

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	256

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	257

8.2.1.2.15	Interruptions due to SCell dormancy	257

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	258

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	258

8.2.1.2.18	 Interruptions at NR SRS antenna port switching	259

8.2.1.2.19	Interruptions at fast SCell activation	260

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	261

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	261

8.2.2.1	Introduction	261

8.2.2.2	Requirements	262

8.2.2.2.1	Interruptions at SCell addition/release	262

8.2.2.2.2	Interruptions at SCell activation/deactivation	263

8.2.2.2.3	Interruptions during measurements on deactivated SCC	264

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	265

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	265

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	266

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	267

8.2.2.2.8	Interruptions due to UE-specific CBW change	268

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	268

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	270

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	270

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	271

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	271

8.2.2.2.11	Interruptions at direct SCell activation	271

8.2.2.2.12	Interruptions due to SCell dormancy	271

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	272

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	272

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	273

8.2.2.2.16	 Interruptions at NR SRS antenna port switching	273

8.2.2.2.17	Interruptions at fast SCell activation	274

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	275

8.2.3	NE-DC Interruptions	275

8.2.3.1	Introduction	275

8.2.3.2	Requirements	276

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	276

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	276

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	276

8.2.3.2.4	Interruptions at SCell activation/deactivation	277

8.2.3.2.5	Interruptions during measurements on SCC	278

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	280

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	280

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	280

8.2.3.2.9	Interruptions at SCell hibernation	281

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	281

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	281

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	282

8.2.3.2.13	Interruptions due to SCell dormancy	283

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	284

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	284

8.2.3.2.17	Interruptions at fast SCell activation	286

8.2.3.2.18	Interruptions due to UE-specific CBW change	287

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	287

8.2.4	NR-DC: Interruptions	287

8.2.4.1	Introduction	287

8.2.4.2	Requirements	288

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	288

8.2.4.2.2	Interruptions at SCell activation/deactivation	289

8.2.4.2.3	Interruptions during measurements on SCC	290

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	290

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	291

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	291

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	291

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	292

8.2.4.2.9	Interruptions at NR SRS carrier based switching	292

8.2.4.2.10	Interruptions at direct SCell activation	293

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	294

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	294

8.2.4.2.13	 Interruptions due to SCell dormancy	295

8.2.4.2.14	Interruptions due to UE-specific CBW change	295

8.2.4.2.14	Interruptions at NR SRS antenna port switching	295

8.2.4.2.15	Interruptions at fast SCell activation	297

8.2.4.2.16	Interruptions at SCG activation/deactivation	297

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	298

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	298

8.2.4.2A	Void	299

8.2.4.2A.1	Void	299

8.2.4.2A.2	Void	299

8.2.4.2A.3	Void	299

8.3	SCell Activation and Deactivation Delay	299

8.3.1	Introduction	299

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	299

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	303

8.3.4	Direct SCell Activation at SCell addition	303

8.3.5	Direct SCell Activation at Handover	305

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	307

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	311

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	311

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	312

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	314

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	316

8.3.13.1	Introduction	316

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	318

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	318

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	318

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	321

8.3A.1	Introduction	321

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	321

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	326

8.4	UE UL carrier RRC reconfiguration delay	326

8.4.1	Introduction	326

8.4.2	UE UL carrier configuration delay requirement	326

8.4.3	UE UL carrier deconfiguration delay requirement	327

8.5	Link Recovery Procedures	327

8.5.1	Introduction	327

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	328

8.5.2	Requirements for SSB based beam failure detection	329

8.5.2.1	Introduction	329

8.5.2.2	Minimum requirement	329

8.5.2.3	Measurement restriction for SSB based beam failure detection	332

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	333

8.5.3	Requirements for CSI-RS based beam failure detection	334

8.5.3.1	Introduction	334

8.5.3.2	Minimum requirement	334

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	337

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	338

8.5.4	Minimum requirement for L1 indication	339

8.5.5	Requirements for SSB based candidate beam detection	339

8.5.5.1	Introduction	339

8.5.5.2	Minimum requirement	340

8.5.5.3	Measurement restriction for SSB based candidate beam detection	342

8.5.6	Requirements for CSI-RS based candidate beam detection	343

8.5.6.1	Introduction	343

8.5.6.2	Minimum requirement	343

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	346

8.5.7	Scheduling availability of UE during beam failure detection	347

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	347

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	347

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	347

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	348

8.5.8	Scheduling availability of UE during candidate beam detection	348

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	348

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	348

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	348

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	349

8.5.9	Requirements for Beam Failure Recovery in SCell	349

8.5.9.1	Introduction	349

8.5.9.2	Requirement	349

8.5.10	Minimum requirement at transitions for beam failure detection	350

8.5A	Link Recovery Procedures when CCA is used on target frequency	350

8.5A.1	Introduction	350

8.5A.2	Requirements for SSB based beam failure detection	351

8.5A.2.1	Introduction	351

8.5A.2.2	Minimum requirement	351

8.5A.2.3	Measurement restriction for SSB based beam failure detection	353

8.5A.3	Void	354

8.5A.4	Minimum requirement for L1 indication	354

8.5A.5	Requirements for SSB based candidate beam detection	354

8.5A.5.1	Introduction	354

8.5A.5.2	Minimum requirement	354

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	357

8.5A.6	Void	357

8.5A.7	Scheduling availability of UE during beam failure detection	357

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	357

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	357

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	357

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	357

8.5A.8	Scheduling availability of UE during candidate beam detection	357

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	358

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	358

8.5B	Link Recovery Procedures for Redcap	358

8.5B.1	Introduction	358

8.5B.2	Requirements for SSB based beam failure detection for Redcap	358

8.5B.2.1	Introduction	358

8.5B.2.2	Minimum requirement	359

8.5B.2.3	Measurement restriction for SSB based beam failure detection	360

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	361

8.5B.3.1	Introduction	361

8.5B.3.2	Minimum requirement	362

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	364

8.5B.4	Minimum requirement for L1 indication for Redcap	364

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	365

8.5B.5.1	Introduction	365

8.5B.5.2	Minimum requirement	365

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	367

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	367

8.5B.6.1	Introduction	367

8.5B.6.2	Minimum requirement	367

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	370

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	370

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	370

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	370

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	370

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	371

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	371

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	371

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	371

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	372

8.5C	Link Recovery Procedures for Satellite Access	372

8.5C.1	Introduction	372

8.5C.2	Requirements for SSB based beam failure detection	373

8.5C.2.1	Introduction	373

8.5C.2.2	Minimum requirement	373

8.5C.2.3	Measurement restriction for SSB based beam failure detection	374

8.5C.3	Requirements for CSI-RS based beam failure detection	374

8.5C.3.1	Introduction	374

8.5C.3.2	Minimum requirement	375

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	375

8.5C.4	Minimum requirement for L1 indication	376

8.5C.5	Requirements for SSB based candidate beam detection	376

8.5C.5.1	Introduction	376

8.5C.5.2	Minimum requirement	376

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	377

8.5C.6	Requirements for CSI-RS based candidate beam detection	377

8.5C.6.1	Introduction	377

8.5C.6.2	Minimum requirement	377

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	378

8.5C.7	Scheduling availability of UE during beam failure detection	379

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	379

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	379

8.5C.8	Scheduling availability of UE during candidate beam detection	379

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	379

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	379

8.5C.9	Minimum requirement at transitions for beam failure detection	379

8.6	Active BWP switch delay	380

8.6.1	Introduction	380

8.6.2	DCI and timer based BWP switch delay on a single CC	380

8.6.2A	DCI based BWP switch delay on multiple CCs	381

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	382

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	383

8.6.2B	Timer based BWP switch delay on multiple CCs	383

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	383

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	384

8.6.3	RRC based BWP switch delay on a single CC	384

8.6.3A	RRC based BWP switch delay on multiple CCs	385

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	385

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	386

8.6.4	BWP switch delay on Consistent UL CCA recovery	386

8.6A	Active BWP switch delay for RedCap	386

8.6A.1	Introduction	386

8.6A.2	DCI and timer based BWP switch delay on a single CC	386

8.6A.3	RRC based BWP switch delay on a single CC	388

8.6C	Active BWP switch delay for satellite access	388

8.6C.1	Introduction	389

8.6C.2	DCI and timer based BWP switch delay on a single CC	389

8.6C.3	RRC based BWP switch delay on a single CC	390

8.7	Void	391

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	391

8.8.1	Introduction	391

8.8.2	E-UTRAN PSCell Addition Delay Requirement	391

8.8.3	E-UTRAN PSCell Release Delay Requirement	391

8.9	NR-DC: PSCell Addition and Release Delay	392

8.9.1	Introduction	392

8.9.2	PSCell Addition Delay Requirement	392

8.9.3	PSCell Release Delay Requirement	392

8.9A	Conditional PSCell Addition Delay	393

8.9A.1	Introduction	393

8.9A.2	Conditional PSCell Addition Delay Requirement	393

8.9A.2.1	Measurement time	393

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	394

8.9B.1	Introduction	394

8.9B.2	PSCell Addition Delay Requirement	394

8.9B.3	PSCell Release Delay Requirement	395

8.10	Active TCI state switching delay	395

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	396

8.10.4	DCI based TCI state switch delay	397

8.10.5	RRC based TCI state switch delay	397

8.10.6	Active TCI state list update delay	398

8.10A	Active TCI state switching delay with CCA	398

8.10A.1	Introduction	398

8.10A.2	Known conditions for TCI state	398

8.10A.3	MAC-CE based TCI state switch delay	398

8.10A.4	DCI based TCI state switch delay	399

8.10A.5	RRC based TCI state switch delay	400

8.10A.6	Active TCI state list update delay	400

8.10B	Active TCI state switching delay for RedCap	401

8.10B.1	Introduction	401

8.10B.2	Known conditions for TCI state	401

8.10B.3	MAC-CE based TCI state switch delay	401

8.10B.4	DCI based TCI state switch delay	402

8.10B.5	RRC based TCI state switch delay	402

8.10B.6	Active TCI state list update delay	403

8.11	PSCell Change	403

8.11A	PSCell Change in Carriers with CCA	403

8.11B	Conditional PSCell Change	404

8.11B.1	Introduction	404

8.11B.2	Conditoinal PSCell Change delay	404

8.11B.2.1	Measurement time	405

8.11D	Conditional PSCell Change in Carriers with CCA	405

8.11D.1	Introduction	405

8.11D.2	Conditional PSCell Change delay	405

8.11D.2.1	Measurement time	406

8.12	Uplink spatial relation switch delay	406

8.12.1	Introduction	406

8.12.2	Known conditions for spatial relation when associated with DL-RS	406

8.12.3	MAC-CE based spatial relation switch delay	407

8.12.4	DCI based spatial relation switch delay	408

8.12.5	RRC based spatial relation switch delay	408

8.12A	Uplink spatial relation switch delay for RedCap	408

8.12A.1	Introduction	408

8.12A.2	Known conditions for spatial relation when associated with DL-RS	408

8.12A.3	MAC-CE based spatial relation switch delay	409

8.12A.4	DCI based spatial relation switch delay	410

8.12A.5	RRC based spatial relation switch delay	410

8.12C	Uplink spatial relation switch delay for satellite access	410

8.12C.1	Introduction	411

8.12C.2	Known conditions for spatial relation when associated with DL-RS	411

8.12C.3	MAC-CE based spatial relation switch delay	411

8.12C.4	DCI based spatial relation switch delay	411

8.12C.5	RRC based spatial relation switch delay	412

8.13	UE-specific CBW change	412

8.13.1	Introduction	412

8.13.2	UE-specific CBW change delay	412

8.13A	UE-specific CBW change for RedCap	413

8.13A.1	Introduction	413

8.13A.2	UE-specific CBW change delay	413

8.13C	UE-specific CBW change for satellite access	413

8.13C.1	Introduction	413

8.13C.2	UE-specific CBW change delay	413

8.14	Pathloss reference signal switching delay	414

8.14.1	Introduction	414

8.14.2	Known conditions for pathloss reference signal	414

8.14.3	MAC-CE based pathloss reference signal switch delay	414

8.14C	Pathloss reference signal switching delay for satellite access	415

8.14C.1	Introduction	415

8.14C.2	Known conditions for pathloss reference signal	415

8.14C.3	MAC-CE based pathloss reference signal switch delay	415

8.15	Active downlink TCI state switching delay for unified TCI	416

8.15.1	Introduction	416

8.15.4	DCI based downlink TCI state switch delay	418

8.15.5	Active Downlink TCI state list update delay	418

8.16	Active uplink TCI state switching delay for unified TCI	419

8.16.1	Introduction	419

8.16.4	DCI based uplink TCI state switch delay	421

8.16.5	Active Uplink TCI state list update delay	421

8.17	SCG Activation and Deactivation Delay	422

8.17.1	Introduction	422

8.17.2	SCG Activation Delay Requirement	422

8.17.3	SCG Deactivation Delay Requirement	423

8.18	TRP specific Link Recovery Procedures	423

8.18.1	Introduction	423

8.18.2	Requirements for TRP specific SSB based beam failure detection	424

8.18.2.1	Introduction	424

8.18.2.2	Minimum requirement	425

8.18.2.3	Measurement restriction for SSB based beam failure detection	426

8.18.3	Requirements for CSI-RS based beam failure detection	427

8.18.3.1	Introduction	427

8.18.3.2	Minimum requirement	427

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	429

8.18.4	Minimum requirement for L1 indication	430

8.18.5	Requirements for SSB based candidate beam detection	430

8.18.5.1	Introduction	430

8.18.5.2	Minimum requirement	430

8.18.5.3	Measurement restriction for SSB based candidate beam detection	432

8.18.6	Requirements for CSI-RS based candidate beam detection	433

8.18.6.1	Introduction	433

8.18.6.2	Minimum requirement	433

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	435

8.18.7	Requirements for TRP specific Beam Failure Recovery	436

8.18.7.1	Introduction	436

8.18.7.2	Requirement	436

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	436

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	436

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	437

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	437

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	438

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	438

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	438

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	438

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	438

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	439

8.19	Pre-configured measurement gap activation/deactivation delay	439

8.19.1	Introduction	439

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	439

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	439

819.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	439

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	440

9	Measurement Procedure	440

9.1	General measurement requirement	440

9.1.1	Introduction	440

9.1.2	Measurement gap	440

9.1.2.1	EN-DC: Measurement Gap Sharing	452

9.1.2.1a	SA: Measurement Gap Sharing	453

9.1.2.1b	NE-DC: Measurement Gap Sharing	454

9.1.2.1c	NR-DC: Measurement Gap Sharing	455

9.1.3	UE Measurement capability	456

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	456

9.1.3.1a	SA: Monitoring of multiple layers using gaps	456

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	457

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	457

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	458

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	459

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	459

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	460

9.1A.3.2	Void	461

9.1.3A	UE Measurement capability under operation mode with CCA	461

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	461

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	461

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	461

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	462

9.1.3C	UE Measurement capability under operation mode with satellite access	462

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	462

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	463

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	463

9.1.4.1	Introduction	463

9.1.4.2	Requirements	463

9.1.5	Carrier-specific scaling factor	467

9.1.5.1	Monitoring of multiple layers outside gaps	468

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	470

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	471

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	472

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	473

9.1.5.2	Monitoring of multiple layers within gaps	474

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	476

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	477

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	479

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	481

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	483

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	483

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	483

9.1.5.3	Monitoring of multiple layers within NCSG	483

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	484

9.1.6	Minimum requirement at transitions	485

9.1.7	Pre-configured measurement gap	485

9.1.7.1	Introduction	485

9.1.7.2	Requirements applicability	485

9.1.7.3	Requirements	486

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	486

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	487

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	487

9.1.8	Concurrent measurement gaps	487

9.1.8.1	Introduction	487

9.1.8.2	Requirements	487

9.1.8.3	Collision between concurrent measurement gaps	489

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	489

9.1.9	Network controlled small gap	489

9.1.9.1	Introduction	489

9.1.9.2	Requirements applicability	490

9.1.10	MUSIM gaps	492

9.1.11	UL gap for Tx power management	493

9.1A	General measurement requirement for RedCap	494

9.1A.1	Introduction	494

9.1A.2	Measurement gap	494

9.1A.2.1	SA: Measurement Gap Sharing	498

9.1A.3	UE Measurement capability	499

9.1A.3.1	SA: Monitoring of multiple layers using gaps	499

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	499

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	499

9.1A.4.1	Introduction	499

9.1A.4.2	Requirements	500

9.1A.5	Carrier-specific scaling factor	500

9.1A.5.1	Monitoring of multiple layers outside gaps	500

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	501

9.1A.5.2	Monitoring of multiple layers within gaps	501

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	501

9.1A.6	Minimum requirement at transitions	502

9.1C	General measurement requirement	502

9.1C.1	Introduction	503

9.1C.2	Measurement gap	503

9.1C.8	Concurrent measurement gaps for SAN	505

9.1C.8.1	Introduction	505

9.1C.8.2	Requirements	505

9.1C.8.3	Collision between concurrent measurement gaps	506

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	506

9.2	NR intra-frequency measurements	507

9.2.1	Introduction	507

9.2.2	Requirements applicability	508

9.2.3	Number of cells and number of SSB	508

9.2.3.1	Requirements for FR1	508

9.2.3.2	Requirements for FR2	508

9.2.4	Measurement Reporting Requirements	509

9.2.4.1	Periodic Reporting	509

9.2.4.2	Event-triggered Periodic Reporting	509

9.2.4.3	Event Triggered Reporting	509

9.2.5	Intrafrequency measurements without measurement gaps	510

9.2.5.1	Intrafrequency cell identification	510

9.2.5.2	Measurement period	514

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	517

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	517

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	518

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	518

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	519

9.2.5.4	SFTD Measurements between PCell and PSCell	520

9.2.5.4.1	Introduction	520

9.2.5.4.2	SFTD Measurement delay	520

9.2.5.4.3	SFTD Measurement Reporting Delay	520

9.2.6	Intra-frequency measurements with measurement gaps	521

9.2.6.1	Void	521

9.2.6.2	Intra-frequency cell identification	521

9.2.6.3	Intrafrequency Measurement Period	523

9.2.7	Intra-frequency measurements with NCSG	525

9.2.7.1	Intra-frequency cell identification	525

9.2.7.2	Measurement period	526

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	527

9.2A	NR intra-frequency measurements with CCA	528

9.2A.1	Introduction	528

9.2A.2	Requirements applicability	528

9.2A.3	Number of cells and number of SSB	529

9.2A.3.1	Requirements for FR1	529

9.2A.3.2	Requirements for FR2-2	529

9.2A.4	Measurement Reporting Requirements	529

9.2A.5	Intra-frequency measurements without measurement gaps	530

9.2A.5.2	Measurement period	533

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	535

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	536

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	536

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	536

9.2A.6	Intra-frequency measurements with measurement gaps	536

9.2A.6.1	Intra-frequency cell identification	536

9.2A.6.2	Intra-frequency Measurement Period	538

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	539

9.2A.7.1	Intra-frequency RSSI measurements	539

9.2A.7.2	Intra-frequency Channel occupancy measurements	541

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	542

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	542

9.2B	NR intra-frequency measurements for RedCap	542

9.2B.1	Introduction	542

9.2B.2	Requirements applicability	543

9.2B.3	Number of cells and number of SSB	543

9.2B.3.1	Requirements for FR1	543

9.2B.3.2	Requirements for FR2	543

9.2B.4	Measurement Reporting Requirements	544

9.2B.4.1	Periodic Reporting	544

9.2B.4.2	Event-triggered Periodic Reporting	544

9.2B.4.3	Event Triggered Reporting	544

9.2B.5	Intra-frequency measurements without measurement gaps for RedCap	545

9.2B.5.1	Intra-frequency cell identification	545

9.2B.5.2	Measurement period	547

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	548

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	548

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	548

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	549

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	549

9.2B.6	Intra-frequency measurements with measurement gaps	550

9.2B.6.1	Intra-frequency cell identification	550

9.2B.6.2	Intra-frequency Measurement Period	551

9.2C	NR intra-frequency measurements for SAN	552

9.2C.1	Introduction	552

9.2C.2	Requirements applicability	553

9.2C.3	Number of cells and number of SSB	553

9.2C.3.1	Requirements for FR1	553

9.2C.4	Measurement Reporting Requirements	553

9.2C.4.1	Periodic Reporting	553

9.2C.4.2	Event-triggered Periodic Reporting	553

9.2C.4.3	Event Triggered Reporting	553

9.2C.5	Intrafrequency measurements without measurement gaps	554

9.2C.5.1	Intrafrequency cell identification	554

9.2C.5.2	Measurement period	555

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	555

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	555

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	556

9.2C.6	Intra-frequency measurements with measurement gaps	556

9.2C.6.1	Intra-frequency cell identification	556

9.2C.6.3	Intrafrequency Measurement Period	557

9.3	NR inter-frequency measurements	557

9.3.1	Introduction	557

9.3.2	Requirements applicability	558

9.3.2.1	Void	559

9.3.2.2	Void	559

9.3.3	Number of cells and number of SSB	559

9.3.3.1	Requirements for FR1	559

9.3.3.2	Requirements for FR2	559

9.3.4	Inter-frequency measurement with measurement gaps	559

9.3.4.1	Void	562

9.3.4.2	Void	562

9.3.5	Inter-frequency measurements	562

9.3.5.1	Void	563

9.3.5.2	Void	563

9.3.5.3	Void	563

9.3.6	Inter-frequency measurements reporting requirements	563

9.3.6.1	Periodic Reporting	563

9.3.6.2	Event-triggered Periodic Reporting	563

9.3.6.3	Event-triggered Reporting	563

9.3.7	Void	564

9.3.8	Inter-frequency SFTD measurement requirements	564

9.3.8.1	Introduction	564

9.3.8.2	SFTD Measurement delay	564

9.3.8.3	SFTD Measurement reporting delay	565

9.3.9	Inter frequency measurements without measurement gaps	565

9.3.9.1	Inter frequency Cell identification	565

9.3.9.2	Measurement period	567

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	568

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	568

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	569

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	569

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	569

9.3.10	Inter-frequency measurement with NCSG	570

9.3.10.1	Inter-frequency cell identification	570

9.3.10.2	Measurement period	571

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	571

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	572

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	572

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	573

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	575

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	575

9.3A.1	Introduction	575

9.3A.2	Requirements applicability	576

9.3A.3	Number of cells and number of SSB	576

9.3A.3.1	Requirements for FR1	576

9.3A.3.2	Requirements for FR2-2	576

9.3A.4	Inter-frequency cell identification	576

9.3A.5	Inter-frequency measurements	578

9.3A.6	NR Inter-frequency measurements reporting requirements	580

9.3A.6.1	Periodic Reporting	580

9.3A.6.2	Event-triggered Periodic Reporting	580

9.3A.6.3	Event-triggered Reporting	580

9.3A.8	Inter-frequency RSSI measurements	580

9.3A.9	Inter-frequency channel occupancy measurements	581

9.3B	NR inter-frequency measurements for RedCap	581

9.3B.1	Introduction	581

9.3B.2	Requirements applicability	582

9.3B.3	Number of cells and number of SSB	582

9.3B.3.1	Requirements for FR1	582

9.3B.3.2	Requirements for FR2	583

9.3B.4	Inter-frequency measurement with measurement gaps	583

9.3B.5	Inter-frequency measurements	584

9.3B.6	Inter-frequency measurements reporting requirements	585

9.3B.6.1	Periodic Reporting	585

9.3B.6.2	Event-triggered Periodic Reporting	585

9.3B.6.3	Event-triggered Reporting	586

9.3B.7	Inter frequency measurements without measurement gaps	586

9.3B.7.1	Inter frequency Cell identification	586

9.3B.7.2	Measurement period	589

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	589

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	590

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	590

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	590

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	591

9.3C	NR inter-frequency measurements for SAN	591

9.3C.1	Introduction	591

9.3C.2	Requirements applicability	591

9.3C.3	Number of cells and number of SSB	592

9.3C.3.1	Requirements for FR1	592

9.3C.4	Inter-frequency measurement with measurement gaps	592

9.3C.5	Inter-frequency measurements	593

9.3C.6	Inter-frequency measurements reporting requirements	593

9.3C.6.1	Periodic Reporting	593

9.3C.6.2	Event-triggered Periodic Reporting	593

9.3C.6.3	Event-triggered Reporting	594

9.3C.7	Inter frequency measurements without measurement gaps	594

9.3C.7.1	Inter frequency Cell identification	594

9.3C.7.2	Measurement period	595

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	596

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	596

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	596

9.4	Inter-RAT measurements	597

9.4.1	Introduction	597

9.4.2	NR − E-UTRAN FDD measurements	599

9.4.2.1	Introduction	599

9.4.2.2	Requirements when no DRX is used	599

9.4.2.3	Requirements when DRX is used	600

9.4.2.4	Measurement reporting requirements	602

9.4.2.4.1	Periodic Reporting	602

9.4.2.4.2	Event-Triggered Periodic Reporting	602

9.4.2.4.3	Event-Triggered Reporting	603

9.4.3	NR − E-UTRAN TDD measurements	603

9.4.3.1	Introduction	603

9.4.3.2	Requirements when no DRX is used	603

9.4.3.3	Requirements when DRX is used	605

9.4.3.4	Measurement reporting requirements	606

9.4.3.4.1	Periodic Reporting	606

9.4.3.4.2	Event-Triggered Periodic Reporting	607

9.4.3.4.3	Event-Triggered Reporting	607

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	607

9.4.4	Inter-RAT RSTD measurements	607

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	607

9.4.4.1.1	Introduction	607

9.4.4.1.2	Requirements	608

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	611

9.4.4.2.1	Introduction	611

9.4.4.2.2	Requirements	612

9.4.5	Inter-RAT E-CID measurements	615

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	615

9.4.5.1.1	Introduction	615

9.4.5.1.2	Requirements	615

9.4.5.1.3	Measurement Reporting Delay	615

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	615

9.4.5.2.1	Introduction	615

9.4.5.2.2	Requirements	615

9.4.5.2.3	Measurement Reporting Delay	616

9.4.6	NR − UTRAN FDD measurements	616

9.4.6.1	Introduction	616

9.4.6.2	Requirements when no DRX is used	616

9.4.6.3	Requirements when DRX is used	617

9.4.7	NR – E-UTRAN measurements with autonomous gaps	619

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	619

9.4A	Inter-RAT measurements for RedCap	620

9.4A.1	Introduction	620

9.4A.2	NR − E-UTRAN FDD measurements	621

9.4A.2.1	Introduction	621

9.4A.2.2	Requirements when no DRX is used	621

9.4A.2.3	Requirements when DRX is used	622

9.4A.2.4	Measurement reporting requirements	623

9.4A.2.4.1	Periodic Reporting	623

9.4A.2.4.2	Event-Triggered Periodic Reporting	623

9.4A.2.4.3	Event-Triggered Reporting	623

9.4A.3	NR − E-UTRAN TDD measurements	624

9.4A.3.1	Introduction	624

9.4A.3.2	Requirements when no DRX is used	624

9.4A.3.3	Requirements when DRX is used	626

9.4A.3.4	Measurement reporting requirements	627

9.4A.3.4.1	Periodic Reporting	627

9.4A.3.4.2	Event-Triggered Periodic Reporting	627

9.4A.3.4.3	Event-Triggered Reporting	628

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	628

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	628

9.4A.4.2	CGI reporting delay	629

9.4A.4.3	CGI reporting scheduling restriction	629

9.5	L1-RSRP measurements for Reporting	629

9.5.1	Introduction	629

9.5.2	Requirements applicability	630

9.5.3	Measurement Reporting Requirements	630

9.5.3.1	Periodic Reporting	630

9.5.3.2	Semi-Persistent Reporting	631

9.5.3.3	Aperiodic Reporting	631

9.5.4	L1-RSRP measurement requirements	631

9.5.4.1	SSB based L1-RSRP Reporting	631

9.5.4.2	CSI-RS based L1-RSRP Reporting	634

9.5.4A	Void	638

9.5.4A.1	Void	638

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	638

9.5.5.1	Measurement restriction for SSB based L1-RSRP	638

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	638

9.5.6	Scheduling availability of UE during L1-RSRP measurement	639

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	639

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	639

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	639

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	641

9.5A	L1-RSRP measurements for Reporting under CCA	641

9.5A.1	Introduction	641

9.5A.2	Requirements applicability	641

9.5A.3	Measurement Reporting Requirements	642

9.5A.3.1	Periodic Reporting	642

9.5A.3.2	Semi-Persistent Reporting	642

9.5A.3.3	Aperiodic Reporting	643

9.5A.4	L1-RSRP measurement requirements	643

9.5A.4.1	SSB based L1-RSRP Reporting	643

9.5A.5	Measurement restriction for L1-RSRP measurement	645

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	646

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	646

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	646

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	646

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	646

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	646

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	647

9.5B	L1-RSRP measurements for Reporting for RedCap	647

9.5B.1	Introduction	647

9.5B.2	Requirements applicability	647

9.5B.3	Measurement Reporting Requirements	648

9.5B.3.1	Periodic Reporting	648

9.5B.3.2	Semi-Persistent Reporting	649

9.5B.3.3	Aperiodic Reporting	649

9.5B.4	L1-RSRP measurement requirements	649

9.5B.4.1	SSB based L1-RSRP Reporting	649

9.5B.4.2	CSI-RS based L1-RSRP Reporting	651

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	654

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	654

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	655

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	655

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	655

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	655

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	656

9.5C	L1-RSRP measurements for Reporting for satellite access	656

9.5C.1	Introduction	656

9.5C.3	Measurement Reporting Requirements	656

9.5C.3.1	Periodic Reporting	657

9.5C.3.2	Semi-Persistent Reporting	657

9.5C.3.3	Aperiodic Reporting	657

9.5C.4	L1-RSRP measurement requirements	657

9.5C.4.1	SSB based L1-RSRP Reporting	657

9.5C.5	Measurement restriction for L1-RSRP measurement	657

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	657

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	657

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	658

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	658

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	658

9.6	NE-DC: Measurements	658

9.6.1	Introduction	658

9.6.2	SFTD Measurements	658

9.6.2.1	Introduction	658

9.6.2.2	SFTD Measurement requirements	658

9.7	Cross Link Interference measurements	659

9.7.1	Introduction	659

9.7.2	SRS-RSRP measurements	659

9.7.2.1	Introduction	659

9.7.2.2	Requirements applicability	659

9.7.2.3	Measurement Reporting Requirements	660

9.7.2.3.1	Periodic Reporting	660

9.7.2.3.2	Event-triggered Periodic Reporting	660

9.7.2.3.3	Event Triggered Reporting	660

9.7.2.4	Measurement capability	660

9.7.2.5	SRS-RSRP measurement period	660

9.7.3	CLI-RSSI measurements	661

9.7.3.1	Introduction	661

9.7.3.2	Requirements applicability	661

9.7.3.3	Measurement Reporting Requirements	661

9.7.3.3.1	Periodic Reporting	661

9.7.3.3.2	Event-triggered Periodic Reporting	661

9.7.3.3.3	Event Triggered Reporting	661

9.7.3.4	Measurement capability	661

9.7.3.5	CLI-RSSI measurement period	662

9.7.4	Scheduling availability of UE during CLI measurements	662

9.7.4.1	Scheduling availability of UE performing measurement on FR1	662

9.7.4.2	Scheduling availability of UE performing measurement on FR2	662

9.8	L1-SINR measurements for Reporting	663

9.8.1	Introduction	663

9.8.2	Requirements applicability	663

9.8.3	Measurement Reporting Requirements	664

9.8.3.1	Periodic Reporting	664

9.8.3.2	Semi-Persistent Reporting	664

9.8.4	L1-SINR measurement requirements	665

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	665

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	669

9.8.5	Measurement restriction for L1-SINR measurement	671

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	671

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	672

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	673

9.8.6	Scheduling availability of UE during L1-SINR measurement	673

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	673

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	673

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	674

9.9	 NR measurements for positioning	674

9.9.1	Introduction	674

9.9.1.1	General Aspects of Gap-based Measurement	675

9.9.1.2	General Aspects of Gapless Measurement	675

9.9.2	RSTD measurements	676

9.9.2.1	Introduction	676

9.9.2.2	Requirements Applicability	676

9.9.2.3	Measurement Capability	676

9.9.2.4	Measurement Reporting Requirements	676

9.9.2.4.1	Void	676

9.9.2.4.2	Void	676

9.9.2.4.3	Void	676

9.9.2.5	Measurements Period Requirements	676

9.9.2.6	Void	679

9.9.2.7	Measurements Period Requirements without Measurement Gaps	679

9.9.2.8	Scheduling Availability of UE during RSTD Measurement	681

9.9.3	PRS-RSRP measurements	682

9.9.3.1	Introduction	682

9.9.3.2	Requirements applicability	682

9.9.3.3	Measurement Capability	682

9.9.3.4	Measurement Reporting Requirements	682

9.9.3.5	Measurement Period Requirements	682

9.9.3.6	Measurement Period Requirements without Measurement Gaps	685

9.9.3.7	Scheduling Availability of UE during PRS-RSRP Measurement	686

9.9.4	UE Rx-Tx time difference measurements	687

9.9.4.1 Introduction	687

9.9.4.2 Requirements Applicability	687

9.9.4.3 Measurement Capability	687

9.9.4.4 Measurement Reporting Requirements	687

9.9.4.5	Measurement Period Requirements	688

9.9.4.6	Measurement Period Requirements without Measurement Gaps	691

9.9.4.7	Scheduling Availability of UE during UE Rx-Tx Time Difference Measurement	693

9.9.5	E-CID measurements	694

9.9.5.1	Introduction	694

9.9.5.2	Measurement Requirements	694

9.9.5.2.1	Intra-frequency Measurement Requirements	694

9.9.5.2.2	Inter-frequency Measurement Requirements	694

9.9.5.2.3	Measurement Reporting Delay	695

9.9.6	PRS-RSRPP measurements	695

9.9.6.1	Introduction	695

9.9.6.2	Requirements applicability	695

9.9.6.3	Measurement capability	695

9.9.6.4	Measurement reporting requirements	695

9.9.6.5	Measurement period requirements	695

9.9.6.6	Measurement Period Requirements without Measurement Gaps	696

9.9.6.7	Scheduling Availability of UE during PRS-RSRPP Measurement	696

9.10	CSI-RS based L3 measurements	696

9.10.1	Introduction	696

9.10.2	CSI-RS based intra-frequency measurements	697

9.10.2.1	Introduction	697

9.10.2.2	Requirements applicability	697

9.10.2.3	Number of cells and number of CSI-RS	698

9.10.2.3.1	Requirements for FR1	698

9.10.2.3.2	Requirements for FR2	698

9.10.2.4	Measurement Reporting Requirements	699

9.10.2.4.1	Periodic Reporting	699

9.10.2.4.2	Event-triggered Periodic Reporting	699

9.10.2.4.3	Event Triggered Reporting	699

9.10.2.5	Intra-frequency measurements without measurement gaps	699

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	701

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	701

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	701

9.10.3	CSI-RS based Inter-frequency measurements	702

9.10.3.1	Introduction	702

9.10.3.2	Requirements applicability	702

9.10.3.3	Number of cells and number of CSI-RS resources	703

9.10.3.3.1	Requirements for FR1	703

9.10.3.3.2	Requirements for FR2	703

9.10.3.4	Measurements reporting requirements	703

9.10.3.4.1	Periodic Reporting	703

9.10.3.4.2	Event-triggered Periodic Reporting	703

9.10.3.4.3	Event-triggered Reporting	703

9.10.3.5	Inter frequency measurements with measurement gaps	704

9.11	NR measurements with autonomous gaps	705

9.11.1	Introduction	705

9.11.2	CGI identification of an NR cell with autonomous gaps	705

9.11.3	CGI reporting delay	706

9.11A	NR measurements with autonomous gaps for RedCap	707

9.11A.1	Introduction	707

9.11A.2	CGI identification of an NR cell with autonomous gaps	707

9.11A.3	CGI reporting delay	708

9.11A.4	CGI reporting scheduling restriction	708

9.12	Measurement for Propagation Delay Compensation	709

9.12.1	Introduction	709

9.12.2	Requirements Applicability	709

9.12.3	Measurement Capability	709

9.12.4	Measurement period requirements	709

9.12.4.1	PRS Measurement Period	709

9.12.4.2	TRS Measurement Period	710

9.12.5	Measurement Reporting Requirements	710

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	711

9.13.1	Introduction	711

9.13.2	Requirements Applicability	711

9.13.3	Measurement Reporting Requirements	712

9.13.3.1	Periodic Reporting	712

9.13.3.2	Semi-Persistent Reporting	712

9.13.3.3	Aperiodic Reporting	712

9.13.4	L1-RSRP measurement requirements	712

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	712

9.13.5	Measurement restriction for L1-RSRP measurement	714

9.13.5.1	Measurement restriction for SSB based L1-RSRP	714

9.13.6	Scheduling availability of UE during L1-RSRP measurement	715

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	715

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	715

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	716

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	716

10	Measurement Performance requirements	717

10.1	NR measurements	717

10.1.1	Introduction	717

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	717

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	717

10.1.2.1.1	Absolute SS-RSRP Accuracy	717

10.1.2.1.2	Relative SS-RSRP Accuracy	718

10.1.2.2	Void	719

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	719

10.1.2.3.1	Absolute CSI-RSRP Accuracy	719

10.1.2.3.2	Relative CSI-RSRP Accuracy	720

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	721

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	721

10.1.2B.1.1	Absolute SS-RSRP Accuracy	721

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	722

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	722

10.1.3.1.1	Absolute SS-RSRP Accuracy	722

10.1.3.1.2	Relative SS-RSRP Accuracy	723

10.1.3.2	Void	724

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	724

10.1.3.3.1	Absolute CSI-RSRP Accuracy	724

10.1.3.3.2	Relative CSI-RSRP Accuracy	724

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	725

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	725

10.1.3B.1.1	Absolute SS-RSRP Accuracy	725

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	726

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	726

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	726

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	727

10.1.4.2	Void	728

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	728

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	728

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	729

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	730

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	730

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	730

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	731

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	731

10.1.5.1.1	Absolute SS-RSRP Accuracy	731

10.1.5.1.2	Relative SS-RSRP Accuracy	732

10.1.5.2	Void	733

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	733

10.1.5.3.1	Absolute CSI-RSRP Accuracy	733

10.1.5.3.2	Relative CSI-RSRP Accuracy	733

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	734

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	734

10.1.5B.1.1	Absolute SS-RSRP Accuracy	734

10.1.6	RSRP Measurement Report Mapping	735

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	737

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	737

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	737

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	738

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	738

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	739

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	739

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	739

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	740

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	740

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	740

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	741

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	741

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	742

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	742

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	742

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	743

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	743

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	743

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	744

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	745

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	745

10.1.9.2.2	Relative CSI-RSRQ Accuracy	746

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	747

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	747

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	747

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	748

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	748

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	748

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	749

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	750

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	750

10.1.10.2.2	Relative CSI-RSRQ Accuracy	750

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	751

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	751

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	751

10.1.11	RSRQ report mapping	752

10.1.12	Intra-frequency SINR accuracy requirements for FR1	752

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	752

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	752

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	753

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	753

10.1.13	Intra-frequency SINR accuracy requirements for FR2	754

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	754

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	754

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	755

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	755

10.1.14	Inter-frequency SINR accuracy requirements for FR1	756

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	756

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	756

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	757

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	758

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	758

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	759

10.1.15	Inter-frequency SINR accuracy requirements for FR2	760

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	760

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	760

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	761

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	762

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	762

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	762

10.1.16	SINR report mapping	763

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	763

10.1.17	Power Headroom	764

10.1.17.1	Power Headroom Report	764

10.1.17.1.1	Power Headroom Report Mapping	764

10.1.18	PCMAX,c,f	765

10.1.18.1	Report Mapping	765

10.1.19	L1-RSRP accuracy requirements for FR1	765

10.1.19.1	SSB based L1-RSRP accuracy requirements	765

10.1.19.1.1	Absolute Accuracy	765

10.1.19.1.2	Relative Accuracy	766

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	767

10.1.19.2.1	Absolute Accuracy	767

10.1.19.2.2	Relative Accuracy	768

10.1.20	L1-RSRP accuracy requirements for FR2	769

10.1.20.1	SSB based L1-RSRP accuracy requirements	769

10.1.20.1.1	Absolute Accuracy	769

10.1.20.1.2	Relative Accuracy	770

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	771

10.1.20.2.1	Absolute Accuracy	771

10.1.20.2.2	Relative Accuracy	771

10.1.21	SFTD accuracy requirements	772

10.1.21.1	SFTD acuracy requirements for NE-DC	772

10.1.21.2	SFTD acuracy requirements for NR-DC	774

10.1.21.3	Inter frequency SFTD acuracy requirements	775

10.1.22	CLI measurement accuracy requirements	776

10.1.22.1	SRS-RSRP	776

10.1.22.1.1	SRS-RSRP Accuracy	776

10.1.22.1.2	SRS-RSRP report mapping	778

10.1.22.2	CLI-RSSI	778

10.1.22.2.1	CLI-RSSI Accuracy	778

10.1.22.2.2	CLI-RSSI report mapping	779

10.1.23	RSTD Measurements	780

10.1.23.1	Introduction	780

10.1.23.2	Measurement Accuracy Requirements	780

10.1.23.3	Report mapping	784

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	784

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	786

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	788

10.1.24	PRS-RSRP Measurements	790

10.1.24.1	Introduction	790

10.1.24.2	Measurement Accuracy Requirements	790

10.1.24.2.1	Absolute PRS RSRP accuracy	790

10.1.24.2.2	Relative PRS RSRP accuracy	792

10.1.24.3	Report mapping	794

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	794

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	795

10.1.25	UE Rx-Tx Time Difference Measurements	797

10.1.25.1	Introduction	797

10.1.25.2	Measurement Accuracy Requirements	797

10.1.25.3	Report mapping	805

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	805

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	807

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	809

10.1.26	FR2 P-MPR report	811

10.1.26.1	Report mapping	811

10.1.27	L1-SINR accuracy requirements for FR1	811

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	811

10.1.27.1.1	Absolute Accuracy	811

10.1.27.1.2	Relative Accuracy	812

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	813

10.1.27.2.1	Absolute Accuracy	813

10.1.27.2.2	Relative Accuracy	814

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	816

10.1.27.3.1	Absolute Accuracy	816

10.1.27.3.2	Relative Accuracy	818

10.1.28	L1-SINR accuracy requirements for FR2	819

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	826

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	826

10.1.29.1.1	Absolute SS-RSRQ Accuracy	826

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	826

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	826

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	826

10.1.30.1.2	Relative Accuracy of SS-RSRQ	827

10.1.31	Intra-frequency SINR accuracy requirements under CCA	828

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	828

10.1.31.1.1	Absolute SS-SINR Accuracy	828

10.1.32	Inter-frequency SINR accuracy requirements under CCA	828

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	828

10.1.32.1.1	Aboslute Accuracy of SS-SINR	828

10.1.32.1.2	Relative Accuracy of SS-SINR	829

10.1.33	L1-RSRP accuracy requirements under CCA	830

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	830

10.1.33.1.1	Absolute Accuracy	830

10.1.33.1.2	Relative Accuracy	830

10.1.34	RSSI measurements under CCA	831

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	831

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	831

10.1.34.3	RSSI measurement report mapping	831

10.1.35	Channel occupancy measurements under CCA	832

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	832

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	832

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	832

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	832

10.1.36.1.1	Absolute SS-RSRP Accuracy	832

10.1.36.1.2	Relative SS-RSRP Accuracy	833

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	834

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	834

10.1.37.1.1	Absolute Accuracy of SS-RSRP	834

10.1.37.1.2	Relative Accuracy of SS-RSRP	834

10.2	E-UTRAN measurements	835

10.2.1	Introduction	835

10.2.2	E-UTRAN RSRP measurements	835

10.2.3	E-UTRAN RSRQ measurements	835

10.2.4	E-UTRAN RSTD measurements	836

10.2.5	E-UTRAN RS-SINR measurements	836

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	836

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	836

10.3	UTRAN FDD Measurements	837

10.3.1	UTRAN FDD CPICH RSCP	837

10.3.2	UTRAN FDD CPICH Ec/No	838

10.4	V2X measurements	838

10.4.1	Introduction	838

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	838

10.4.2.1	PSBCH-RSRP Absolute Accuracy	838

10.4.2.2	PSBCH-RSRP Relative Accuracy	839

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	840

10.4.3.1	Absolute SL-RSSI Accuracy	840

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	840

10.4.4.1	Absolute L1 SL-RSRP Accuracy	840

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	841

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	841

11	Void	842

12	V2X Requirements	842

12.1	Introduction	842

12.2	UE Transmit Timing	843

12.2.1	Introduction	843

12.2.2	GNSS as synchronization reference source	843

12.2.3	NR Cell as synchronization reference source	843

12.2.4	E-URTAN Cell as synchronization reference source	844

12.2.5	SyncRef UE as synchronization reference source	844

12.3	Initiation/Cease of SLSS Transmissions	844

12.3.1	Introduction	844

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	844

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	845

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	846

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	846

12.4	Selection / Reselection of V2X Synchronization Reference Source	847

12.5	L1 SL-RSRP measurements	849

12.5.1	Introduction	849

12.5.2	SL-RSRP measurements	849

12.6	Congestion Control measurements	849

12.7	Interruption	850

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	850

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	850

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	852

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	852

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	853

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	853

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	853

12.7.8	Interruptions at NR sidelink discovery configuration	853

12.8	Reliability of GNSS signal	854

12.9	Scheduling availability	854

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	854

12.9.2	Scheduling availability of UE switching between uplink transmission and V2X sidelink transmission	854

12.10	Selection / Reselection of relay UE	855

12.10.1	Introduction	855

12.10.2	Selection / Reselection of relay UE	855

13	Measurement Performance Requirements for NR gNB	856

13.1	UL-RTOA	856

13.1.1	Report mapping	856

13.2	gNB Rx-Tx time difference	858

13.2.1	Report mapping	858

13.2.2	Measurement Accuracy Requirements	860

13.2.2.1	Introduction	860

13.3	UL SRS RSRP measurement	861

13.3.1	Report mapping	861

13.3.2	Measurement accuracy requirements	862

13.3.2.1	Introduction	862

13.3.2.2	Requirements	862

13.4	AoA/ZoA	863

13.4.1	Report mapping	863

13.5	Timing advance (TADV)	864

13.5.1	Report mapping	864

Annex A (normative): Test Cases	866

A.1	Purpose of annex	866

A.2	Requirement classification for statistical testing	866

A.2.1	Types of requirements in TS 38.133	866

A.2.1.1	Time and delay requirements on UE higher layer actions	866

A.2.1.2	Measurements of power levels, relative powers and time	866

A.2.1.3	Implementation requirements	867

A.2.1.4	Physical layer timing requirements	867

A.2.1.5	Requirements under CCA	867

A.3	RRM test configurations	868

A.3.1	Reference measurement channels	868

A.3.1.1	PDSCH	868

A.3.1.1.1	FDD	868

A.3.1.1.2	TDD	869

A.3.1.2	CORESET for RMSI scheduling	872

A.3.1.2.1	FDD	872

A.3.1.2.2	TDD	873

A.3.1.3	CORESET for RMC scheduling	876

A.3.1.3.1	FDD	876

A.3.1.3.2	TDD	877

A.3.1.4	TDD UL/DL configuration	880

A.3.1A	Reference measurement channels under CCA	882

A.3.1A.1	PDSCH	882

A.3.1A.1.1	TDD	882

A.3.1A.2	CORESET for RMSI scheduling	883

A.3.1A.2.1	TDD	883

A.3.1A.3	CORESET for RMC scheduling	884

A.3.1A.3.1	TDD	884

A.3.1A.4	TDD UL/DL configuration	885

A.3.1A.5	RMC burst transmission model	885

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	885

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	886

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	886

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	887

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	887

A.3.2.2	Void	888

A.3.3	Reference DRX configurations	888

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	888

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	889

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	889

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	889

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	890

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	890

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	890

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	891

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	891

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	891

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	892

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	892

A.3.4	Test Cases with Different Channel Bandwidths	892

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	892

A.3.4.1.1	Introduction	892

A.3.4.1.2	Principle of testing	892

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	893

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	893

A.3.5.1.1	Introduction	893

A.3.5.1.2	Principle of Testing	893

A.3.6	Antenna configurations	893

A.3.6.1	Antenna configurations for FR1	893

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	893

A.3.6.1.1.1	Introduction	893

A.3.6.1.1.2	Principle of testing	893

A.3.6.2	Antenna configurations for FR2	896

A.3.6A	Antenna configurations with unlicensed bands	896

A.3.6A.1	Antenna configurations for FR1	896

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	896

A.3.6A.1.1.1	Introduction	896

A.3.6A.1.1.2	Principle of testing	896

A.3.7	EN-DC test setup	898

A.3.7.1	Introduction	898

A.3.7.2	E-UTRAN Serving Cell Parameters	898

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	898

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	899

A.3.7A	NR FR1-FR2 test setup	900

A.3.7B	EN-DC test setup with unlicensed bands	901

A.3.7B.1	Introduction	901

A.3.7B.2	E-UTRAN Serving Cell Parameters	901

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	901

A.3.7C	LTE-FR1/FR2 test setup	902

A.3.7D	NE-DC test setup	902

A.3.7D.1	Introduction	902

A.3.7D.2	E-UTRAN Serving Cell Parameters	902

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	902

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	902

A.3.8	PRACH configurations	902

A.3.8.1	Introduction	902

A.3.8.2	PRACH configurations in FR1	903

A.3.8.2.1	FR1 PRACH configuration 1	903

A.3.8.2.2	FR1 PRACH configuration 2	903

A.3.8.2.3	FR1 PRACH configuration 3	904

A.3.8.2.4	FR1 PRACH configuration 4	905

A.3.8.3	PRACH configurations in FR2	906

A.3.8.3.1	FR2 PRACH configuration 1	906

A.3.8.3.2	FR2 PRACH configuration 2	907

A.3.8.3.3	FR2 PRACH configuration 3	908

A.3.8.3.4	FR2 PRACH configuration 4	909

A.3.8A	PRACH configurations under CCA	910

A.3.8A.1	Introduction	910

A.3.8A.2	PRACH configurations in FR1	910

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	910

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	911

A.3.9	BWP configurations	912

A.3.9.1	Introduction	912

A.3.9.2	Downlink BWP configurations	913

A.3.9.2.1	Initial BWP	913

A.3.9.2.2	Dedicated BWP	913

A.3.9.3	Uplink BWP configurations	913

A.3.9.3.1	Initial BWP	913

A.3.9.3.2	Dedicated BWP	914

A.3.10	SSB Configurations	914

A.3.10.1	SSB Configurations for FR1	914

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	914

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	916

A.3.10A	SSB Configurations under CCA	921

A.3.10A.1	SSB Configurations under CCA for FR1	921

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	921

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	922

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	922

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	923

A.3.11	SMTC Configurations	923

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	923

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	923

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	924

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	924

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	924

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	924

A.3.12	Test Cases with Different CC Configurations	924

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	924

A.3.12.1.1	Introduction	924

A.3.12.1.2	Principle of testing	925

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	925

A.3.12.2.1	Introduction	925

A.3.12.2.2	Principle of testing	925

A.3.13	Test Cases in SA and EN-DC Operations	925

A.3.13.1	Introduction	925

A.3.13.2	Principle of Testing	925

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	926

A.3.13A.1	Introduction	926

A.3.13A.2	Principle of Testing in EN-DC	926

A.3.13A.3	Principle of Testing in SA	926

A.3.13A.4	Principle of Testing in E-UTRA	926

A.3.13B	Test Cases for EN-DC and NE-DC Operations	927

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	927

A.3.13B.1.1	Introduction	927

A.3.13B.1.2	Principle of Testing	927

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	927

A.3.13B.2.1	Introduction	927

A.3.13B.2.2	Principle of Testing	927

A.3.14	CSI-RS configurations	928

A.3.14.1	FDD	928

A.3.14.2	TDD	931

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	937

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	937

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	937

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	937

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	938

A.3.15.3	Setup 3: 2 AoAs	938

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	938

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	938

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	938

A.3.16	TCI State Configuration	939

A.3.16.1	Introduction	939

A.3.16.2	TCI states	939

A.3.17	Configurations of CSI-RS for tracking	939

A.3.17.1	Configuration of CSI-RS for tracking for FR1	939

A.3.17.1.1	FDD	939

A.3.17.1.2	TDD	940

A.3.17.2	Configuration of CSI-RS for tracking for FR2	941

A.3.17.2.1	TDD	941

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	942

A.3.18.1	Introduction	942

A.3.18.2	PRACH Power Measurement	942

A.3.19	Test applicability for DAPS handover	942

A.3.19.1	Introduction	942

A.3.19.2	Principle of testing	942

A.3.20	MsgA configurations	943

A.3.20.1	Introduction	943

A.3.20.2	MsgA configurations in FR1	943

A.3.20.2.1	FR1 MsgA configuration 1	943

A.3.20.2.2	FR1 MsgA configuration 2	944

A.3.20.3	MsgA configurations in FR2	946

A.3.20.3.1	FR2 MsgA configuration 1	946

A.3.20.3.2	FR2 MsgA configuration 2	947

A.3.20A	MsgA configurations under CCA	949

A.3.20A.1	Introduction	949

A.3.20A.2	MsgA configurations in FR1	949

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	949

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	950

A.3.21	V2X sidelink communication	953

A.3.21.1	Introduction	953

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	953

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	957

A.3.22	CSI-IM configurations	957

A.3.22.1	FDD	957

A.3.22.2	TDD	958

A.3.23	Spatial Relation Configuration	960

A.3.23.1	Introduction	960

A.3.23.2	Spatial Relation	960

A.3.24	SRS configuration	961

A.3.25	Channel bandwidth (CBW) configurations	963

A.3.25.1	DL UE specific CBW	963

A.3.25.2	UL UE specific CBW	964

A.3.26	CCA model	964

A.3.26.1	Introduction	964

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	964

A.3.26.2.1	DL CCA model	964

A.3.26.2.2	UL CCA model	965

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	966

A.3.27.1	Introduction	966

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	966

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	966

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	966

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	966

A.3.28	Discovery Burst Transmission Window configuration under CCA	967

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	967

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	967

A.3.29.1	Introduction	967

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	967

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	967

A.3.30	CSI-RS configurations for RRM	968

A.3.30.1	FDD	968

A.3.30.2	TDD	969

A.3.31	PRS Configurations	971

A.3.31.1.	PRS Configurations for FR1	971

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	971

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	972

A.3.31.2.	PRS Configurations for FR2	972

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	972

A.4	EN-DC tests with all NR cells in FR1	973

A.4.1	Void	973

A.4.2	Void	973

A.4.3	RRC_CONNECTED state mobility	973

A.4.3.1	Void	973

A.4.3.2	RRC Connection Mobility Control	973

A.4.3.2.1	Void	973

A.4.3.2.2	Random Access	973

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	973

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	977

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	981

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	985

A.4.3.2.3	Void	989

A.4.4	Timing	989

A.4.4.1	UE transmit timing	989

A.4.4.1.1	NR UE Transmit Timing Test for FR1	989

A.4.4.1.1.1	Test Purpose and environment	989

A.4.4.1.1.2	Test requirements	993

A.4.4.2	UE timer accuracy	993

A.4.4.3	Timing advance	993

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	993

A.4.4.3.1.1	Test Purpose and Environment	993

A.4.4.3.1.2	Test Parameters	993

A.4.4.3.1.3	Test Requirements	997

A.4.5	Signaling characteristics	997

A.4.5.1	Radio link Monitoring	997

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	998

A.4.5.1.1.1	Test Purpose and Environment	998

A.4.5.1.1.2	Test Requirements	1002

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1002

A.4.5.1.2.1	Test Purpose and Environment	1002

A.4.5.1.2.2	Test Requirements	1008

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1008

A.4.5.1.3.1	Test Purpose and Environment	1008

A.4.5.1.3.2	Test Requirements	1013

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1013

A.4.5.1.4.1	Test Purpose and Environment	1013

A.4.5.1.4.2	Test Requirements	1019

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1019

A.4.5.1.5.1	Test Purpose and Environment	1019

A.4.5.1.5.2	Test Requirements	1024

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1024

A.4.5.1.6.1	Test Purpose and Environment	1024

A.4.5.1.6.2	Test Requirements	1030

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1030

A.4.5.1.7.1	Test Purpose and Environment	1030

A.4.5.1.7.2	Test Requirements	1035

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1036

A.4.5.1.8.1	Test Purpose and Environment	1036

A.4.5.1.8.2	Test Requirements	1041

A.4.5.2	Interruption	1041

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1041

A.4.5.2.1.1	Test Purpose and Environment	1041

A.4.5.2.1.2	Test Requirements	1045

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1045

A.4.5.2.2.1	Test Purpose and Environment	1045

A.4.5.2.2.2	Test Requirements	1049

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1049

A.4.5.2.3.1	Test Purpose and Environment	1049

A.4.5.2.3.2	Test Requirements	1053

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1054

A.4.5.2.4.1	Test Purpose and Environment	1054

A.4.5.2.4.2	Test Requirements	1057

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1058

A.4.5.2.5.1	Test Purpose and Environment	1058

A.4.5.2.5.2	Test Requirements	1061

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1061

A.4.5.2.6.1	Test Purpose and Environment	1061

A.4.5.2.6.2	Test Requirements	1065

A.4.5.2.7	Void	1065

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1065

A.4.5.2.8.1	Test Purpose and Environment	1065

A.4.5.2.8.2	Test Requirements	1068

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1069

A.4.5.2.9.1	Test Purpose and Environment	1069

A.4.5.2.9.2	Test Requirements	1072

A.4.5.3	SCell Activation and Deactivation Delay	1073

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1073

A.4.5.3.1.1	Test Purpose and Environment	1073

A.4.5.3.1.2	Test Requirements	1078

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640 ms SCell measurement cycle	1079

A.4.5.3.2.1	Test Purpose and Environment	1079

A.4.5.3.2.2	Test Requirements	1079

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1079

A.4.5.3.3.1	Test Purpose and Environment	1079

A.4.5.3.3.2	Test Requirements	1080

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1080

A.4.5.3.4.1	Test Purpose and Environment	1080

A.4.5.3.3.2	Test Requirements	1083

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1083

A.4.5.3.5.1	Test Purpose and Environment	1083

A.4.5.3.5.2	Test Requirements	1088

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1088

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1088

A.4.5.4.1.1 Test Purpose and Environment	1088

A.4.5.4.1.2	Test Requirements	1098

A.4.5.5	Beam Failure Detection and Link recovery procedures	1098

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1098

A.4.5.5.1.1	Test Purpose and Environment	1098

A.4.5.5.1.2	Test Requirements	1104

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1104

A.4.5.5.2.1	Test Purpose and Environment	1104

A.4.5.5.2.2	Test Requirements	1110

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1110

A.4.5.5.3.1	Test Purpose and Environment	1110

A.4.5.5.3.2	Test Requirements	1116

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1116

A.4.5.5.4.1	Test Purpose and Environment	1116

A.4.5.5.4.2	Test Requirements	1122

A.4.5.6	Active BWP switch	1136

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1136

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1136

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1141

A.4.5.6.2	RRC-based Active BWP Switch	1146

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1150

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1150

A.4.5.6.3.1.1	Test Purpose and Environment	1150

A.4.5.6.3.1.2	Test Requirements	1155

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1156

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1156

A.4.5.6.4.1.1	Test Purpose and Environment	1156

A.4.5.6.4.1.2	Test Requirements	1160

A.4.5.6.5	SCell dormancy switch	1160

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1160

A.4.5.6.5.1.1	Test Purpose and Environment	1160

A.4.5.6.5.1.2	Test Requirements	1166

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1166

A.4.5.6.5.2.1	Test Purpose and Environment	1166

A.4.5.6.5.2.2	Test Requirements	1171

A.4.5.7	PSCell addition and release delay	1171

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1171

A.4.5.7.1.1	Test purpose and environment	1171

A.4.5.7.1.2	Test Requirements	1175

A.4.5.8	DL Interruptions at switching between two uplink carriers	1176

A.4.5.8.1	Test Purpose and Environment	1176

A.4.5.8.2	Test Requirements	1179

A.4.6	Measurement procedure	1180

A.4.6.1	Intra-frequency Measurements	1180

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1180

A.4.6.1.1.1	Test purpose and Environment	1180

A.4.6.1.1.2	Test parameters	1180

A.4.6.1.1.3	Test Requirements	1184

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1184

A.4.6.1.2.1	Test purpose and Environment	1184

A.4.6.1.2.2	Test parameters	1184

A.4.6.1.2.2	Test Requirements	1188

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1188

A.4.6.1.3.1	Test purpose and Environment	1188

A.4.6.1.3.2	Test parameters	1188

A.4.6.1.3.3	Test Requirements	1192

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1192

A.4.6.1.4.1	Test purpose and Environment	1192

A.4.6.1.4.2	Test parameters	1192

A.4.6.1.4.3	Test Requirements	1196

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1196

A.4.6.1.5.1	Test purpose and Environment	1196

A.4.6.1.5.2	Test parameters	1196

A.4.6.1.5.3	Test Requirements	1198

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1199

A.4.6.1.6.1	Test purpose and Environment	1199

A.4.6.1.6.2	Test parameters	1199

A.4.6.1.6.3	Test Requirements	1201

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1202

A.4.6.1.7.1	Test purpose and Environment	1202

A.4.6.1.7.2	Test parameters	1202

A.4.6.1.7.3	Test Requirements	1206

A.4.6.2	Inter-frequency Measurements	1206

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1206

A.4.6.2.1.1	Test Purpose and Environment	1206

A.4.6.2.1.2	Test Requirements	1210

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1210

A.4.6.2.2.1	Test Purpose and Environment	1210

A.4.6.2.2.2	Test Requirements	1214

A.4.6.2.3	Void	1215

A.4.6.2.4	Void	1215

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1215

A.4.6.2.5.1	Test Purpose and Environment	1215

A.4.6.2.5.2	Test Requirements	1219

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1219

A.4.6.2.6.1	Test Purpose and Environment	1219

A.4.6.2.6.2	Test Requirements	1224

A.4.6.2.7	Void	1225

A.4.6.2.8	Void	1225

A.4.6.3	Void	1225

A.4.6.4	L1-RSRP measurement for beam reporting	1225

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1225

A.4.6.4.1.1	Test Purpose and Environment	1225

A.4.6.4.1.2	Test parameters	1225

A.4.6.4.1.3	Test Requirements	1228

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1228

A.4.6.4.2.1	Test Purpose and Environment	1228

A.4.6.4.2.2	Test parameters	1229

A.4.6.4.2.3	Test Requirements	1232

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1232

A.4.6.4.3.1	Test Purpose and Environment	1232

A.4.6.4.3.2	Test parameters	1233

A.4.6.4.3.3	Test Requirements	1236

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1236

A.4.6.4.4.1	Test Purpose and Environment	1236

A.4.6.4.4.2	Test parameters	1237

A.4.6.4.4.3	Test Requirements	1239

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1239

A.4.6.4.5.1	Test Purpose and Environment	1239

A.4.6.4.5.2	Test parameters	1240

A.4.6.4.5.3	Test Requirements	1243

A.4.6.5	CLI measurements	1243

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1243

A.4.6.5.1.1	Test Purpose and Environment	1243

A.4.6.5.1.2	Test Parameters	1244

A.4.6.5.1.3	Test Requirements	1247

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1247

A.4.6.5.2.1	Test Purpose and Environment	1247

A.4.6.5.2.2	Test Parameters	1248

A.4.6.5.2.3	Test Requirements	1249

A.4.6.6.1.2	Test Requirements	1254

A.4.6.7	L1-SINR measurement for beam reporting	1254

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1257

A.4.6.7.2.1	Test Purpose and Environment	1257

A.4.6.7.2.2	Test parameters	1257

A.4.6.7.2.3	Test Requirements	1260

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1260

A.4.6.7.3.1	Test Purpose and Environment	1260

A.4.6.7.3.2	Test parameters	1260

A.4.6.7.3.3	Test Requirements	1262

A.4.6.8	CSI-RS based intra-frequency Measurement	1263

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1263

A.4.6.8.1.1	Test purpose and Environment	1263

A.4.6.8.1.2	Test Requirements	1267

A.4.6.9	CSI-RS based inter-frequency Measurement	1267

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1267

A.4.6.9.1.1	Test Purpose and Environment	1267

A.4.6.9.1.2	Test Requirements	1271

A.4.7	Measurement Performance requirements	1272

A.4.7.1	SS-RSRP	1272

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1272

A.4.7.1.1.1	Test Purpose and Environment	1272

A.4.7.1.1.2	Test parameters	1272

A.4.7.1.1.3	Test Requirements	1276

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1276

A.4.7.1.2.1	Test Purpose and Environment	1276

A.4.7.1.2.2	Test parameters	1277

A.4.7.1.2.3	Test Requirements	1281

A.4.7.1.3	Void	1281

A.4.7.2	SS-RSRQ	1281

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1281

A.4.7.2.1.1	Test Purpose and Environment	1281

A.4.7.2.1.2	Test Parameters	1282

A.4.7.2.1.3	Test Requirements	1287

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1287

A.4.7.2.2.1	Test Purpose and Environment	1287

A.4.7.2.2.2	Test Parameters	1287

A.4.7.2.2.3	Test Requirements	1292

A.4.7.3	SS-SINR	1292

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1292

A.4.7.3.1.1	Test Purpose and Environment	1292

A.4.7.3.1.2	Test Parameters	1292

A.4.7.3.1.3	Test Requirements	1296

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1296

A.4.7.3.2.1	Test Purpose and Environment	1296

A.4.7.3.2.2	Test Parameters	1296

A.4.7.3.2.3	Test Requirements	1301

A.4.7.4	L1-RSRP measurement for beam reporting	1301

A.4.7.4.1	SSB based L1-RSRP measurement	1301

A.4.7.4.1.1	Test Purpose and Environment	1301

A.4.7.4.1.2	Test parameters	1302

A.4.7.4.1.3	Test Requirements	1305

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1305

A.4.7.4.2.1	Test Purpose and Environment	1305

A.4.7.4.2.2	Test parameters	1306

A.4.7.4.2.3	Test Requirements	1309

A.4.7.5	SFTD accuracy	1309

A.4.7.5.1	SFTD accuracy	1309

A.4.7.5.1.1	Test Purpose and Environment	1309

A.4.7.5.1.2	Test Parameters	1309

A.4.7.5.1.3	Test Requirements	1314

A.4.7.5.2	Void	1314

A.4.7.5.3	Void	1314

A.4.7.6	CLI measurements	1314

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1314

A.4.7.6.1.1	Test Purpose and Environment	1314

A.4.7.6.1.2	Test parameters	1315

A.4.7.6.1.3	Test Requirements	1320

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1320

A.4.7.6.2.1	Test Purpose and Environment	1320

A.4.7.6.2.2	Test parameters	1321

A.4.7.6.2.3	Test Requirements	1324

A.4.7.7	L1-SINR measurement for beam reporting	1324

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1328

A.4.7.7.2.1	Test Purpose and Environment	1328

A.4.7.7.2.2	Test parameters	1328

A.4.7.7.2.3	Test Requirements	1332

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1332

A.4.7.7.3.1	Test Purpose and Environment	1332

A.4.7.7.3.2	Test parameters	1332

A.4.7.7.3.3	Test Requirements	1335

A.4.7.8	CSI-RSRP	1335

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1335

A.4.7.8.1.1	Test Purpose and Environment	1335

A.4.7.8.1.2	Test parameters	1336

A.4.7.8.1.3	Test Requirements	1341

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1341

A.4.7.8.2.1	Test Purpose and Environment	1341

A.4.7.8.2.2	Test parameters	1341

A.4.7.8.2.3	Test Requirements	1346

A.4.7.9	CSI-RSRQ	1346

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1346

A.4.7.9.1.1	Test Purpose and Environment	1346

A.4.7.9.1.2	Test Parameters	1346

A.4.7.9.1.3	Test Requirements	1351

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1351

A.4.7.9.2.1	Test Purpose and Environment	1351

A.4.7.9.2.2	Test Parameters	1351

A.4.7.9.2.3	Test Requirements	1356

A.4.7.10	CSI-SINR	1356

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1356

A.4.7.10.1.1	Test Purpose and Environment	1356

A.4.7.10.1.2	Test Parameters	1357

A.4.7.10.1.3	Test Requirements	1361

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1361

A.4.7.10.2.1	Test Purpose and Environment	1361

A.4.7.10.2.2	Test Parameters	1362

A.4.7.10.2.3	Test Requirements	1366

A.4.8	Void	1366

A.4A	NE-DC test with all NR cells in FR1	1367

A.4A.1	Signaling characteristics	1367

A.4A.1.1	E-UTRAN PSCell addition	1367

A.4A.1.1.1	Test purpose and environment	1367

A.4A.1.1.2	Test Requirements	1371

A.4A.1.2	Active BWP switch	1372

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1372

A.4A.1.2.1.1	Test Purpose and Environment	1372

A.4A.1.2.1.2	Test Requirements	1376

A.4A.2	Measurement performance	1377

A.4A.2.1	SFTD accuracy	1377

A.4A.2.1.1	SFTD accuracy	1377

A.4A.2.1.1.1	Test Purpose	1377

A.4A.2.1.1.2	Test Environment	1377

A.4A.2.1.1.3	Test Requirements	1380

A.5	EN-DC tests with one or more NR cells in FR2	1381

A.5.1	Void	1381

A.5.2	Void	1381

A.5.3	RRC_CONNECTED state mobility	1381

A.5.3.1	Void	1381

A.5.3.2	RRC Connection Mobility Control	1381

A.5.3.2.1	Void	1381

A.5.3.2.2	Random Access	1381

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1381

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1384

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1390

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1393

A.5.3.2.3	Void	1397

A.5.4	Timing	1397

A.5.4.1	UE transmit timing	1397

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1397

A.5.4.1.1.1	Test Purpose and environment	1397

A.5.4.1.1.2	Test requirements	1400

A.5.4.2	UE timer accuracy	1401

A.5.4.3	Timing advance	1401

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1401

A.5.4.3.1.1 Test Purpose and Environment	1401

A.5.4.3.1.2 Test Parameters	1401

A.5.4.3.1.3	Test Requirements	1405

A.5.5	Signaling characteristics	1405

A.5.5.1	Radio link Monitoring	1405

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1405

A.5.5.1.1.1	Test Purpose and Environment	1405

A.5.5.1.1.2	Test Requirements	1409

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1410

A.5.5.1.2.1	Test Purpose and Environment	1410

A.5.5.1.2.2	Test Requirements	1413

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1414

A.5.5.1.3.1	Test Purpose and Environment	1414

A.5.5.1.3.2	Test Requirements	1418

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1418

A.5.5.1.4.1	Test Purpose and Environment	1418

A.5.5.1.4.2	Test Requirements	1422

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1422

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1426

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1430

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1435

A.5.5.1.8.2	Test Requirements	1439

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1440

A.5.5.1.9.1	Test Purpose and Environment	1440

A.5.5.1.9.2	Test Requirements	1442

A.5.5.2	Interruption	1442

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1442

A.5.5.2.1.1	Test Purpose and Environment	1442

A.5.5.2.1.2	Test Requirements	1445

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1445

A.5.5.2.2.1	Test Purpose and Environment	1445

A.5.5.2.2.2	Test Requirements	1448

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1448

A.5.5.2.3.1	Test Purpose and Environment	1448

A.5.5.2.3.2	Test Requirements	1452

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1452

A.5.5.2.4.1	Test Purpose and Environment	1452

A.5.5.2.4.2	Test Requirements	1455

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1456

A.5.5.2.5.1	Test Purpose and Environment	1456

A.5.5.2.5.2	Test Requirements	1459

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1459

A.5.5.2.6.1	Test Purpose and Environment	1459

A.5.5.2.6.2	Test Requirements	1462

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1462

A.5.5.2.7.1	Test Purpose and Environment	1462

A.5.5.2.7.2	Test Requirements	1465

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1466

A.5.5.2.8.1 Test Purpose and Environment	1466

A.5.5.2.8.3	Test Requirements	1468

A.5.5.3	SCell Activation and Deactivation Delay	1468

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1468

A.5.5.3.1.1	Test Purpose and Environment	1468

A.5.5.3.1.2	Test Requirements	1470

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1470

A.5.5.3.2.1	Test Purpose and Environment	1470

A.5.5.3.2.2	Test Requirements	1474

A.5.5.3.3	Void	1474

A.5.5.3.4	Void	1474

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1474

A.5.5.3.5.1	Test Purpose and Environment	1474

A.5.5.3.5.2	Test Requirements	1478

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1479

A.5.5.3.6.1	Test Purpose and Environment	1479

A.5.5.3.6.2	Test Requirements	1482

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1483

A.5.5.3.7.1	Test Purpose and Environment	1483

A.5.5.3.7.2	Test Requirements	1486

A.5.5.4	Void	1486

A.5.5.5	Beam Failure Detection and Link recovery procedures	1486

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1486

A.5.5.5.1.1	Test Purpose and Environment	1486

A.5.5.5.1.2	Test Requirements	1490

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1490

A.5.5.5.2.1	Test Purpose and Environment	1490

A.5.5.5.2.2	Test Requirements	1494

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1494

A.5.5.5.3.1	Test Purpose and Environment	1494

A.5.5.5.3.2	Test Requirements	1498

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1498

A.5.5.5.4.1	Test Purpose and Environment	1498

A.5.5.5.4.2	Test Requirements	1502

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1502

A.5.5.5.5.1	Test Purpose and Environment	1502

A.5.5.5.5.2	Test Requirements	1506

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1506

A.5.5.5.6.1	Test Purpose and Environment	1506

A.5.5.5.6.2	Test Requirements	1510

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1510

A.5.5.5.7.1	Test Purpose and Environment	1510

A.5.5.5.7.2	Test Requirements	1514

A.5.5.6	Active BWP switch	1514

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1514

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1514

A.5.5.6.1.1.1	Test Purpose and Environment	1514

A.5.5.6.1.1.2	Test Requirements	1518

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1518

A.5.5.6.2	RRC-based Active BWP Switch	1523

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1523

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1527

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1527

A.5.5.6.3.1.1	Test Purpose and Environment	1527

A.5.5.6.3.1.2	Test Requirements	1531

A.5.5.6.4	SCell dormancy switch	1531

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1531

A.5.5.6.4.1.1	Test Purpose and Environment	1531

A.5.5.6.4.1.2	Test Requirements	1535

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1535

A.5.5.6.4.2.1	Test Purpose and Environment	1535

A.5.5.6.4.2.2	Test Requirements	1542

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1542

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1542

A.5.5.7	PSCell addition and release delay	1545

A.5.5.7.1	Addition and Release Delay of NR PSCell	1545

A.5.5.7.1.1	Test purpose and environment	1545

A.5.5.7.1.2	Test Requirements	1548

A.5.5.8	Active TCI state switch delay	1549

A.5.5.8.1	MAC-CE based active TCI state switch	1549

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1549

A.5.5.8.1.1.1	Test Purpose and Environment	1549

A.5.5.8.1.1.2	Test Requirements	1552

A.5.5.8.2	RRC based active TCI state switch	1552

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1552

A.5.5.8.2.1.1	Test Purpose and Environment	1552

A.5.5.8.2.1.2	Test Requirements	1556

A.5.5.9	Uplink spatial relation switch delay	1556

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1556

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1556

A.5.5.9.1.1.1	Test Purpose and Environment	1556

A.5.5.9.1.1.2	Test Requirements	1559

A.5.5.9.2	RRC based spatial relation switch	1559

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1559

A.5.5.9.2.1.1	Test Purpose and Environment	1559

A.5.5.9.2.1.2	Test Requirements	1562

A.5.5.10	UE specific CBW change	1562

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1562

A.5.5.10.1.1	Test Purpose and Environment	1562

A.5.5.10.1.2	Test Requirements	1565

A.5.6	Measurement procedure	1566

A.5.6.1	Intra-frequency Measurements	1566

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1566

A.5.6.1.1.1	Test purpose and Environment	1566

A.5.6.1.1.2	Test Requirements	1569

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1569

A.5.6.1.2.1	Test purpose and Environment	1569

A.5.6.1.2.2	Test Requirements	1571

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1572

A.5.6.1.3.1	Test purpose and Environment	1572

A.5.6.1.3.2	Test Requirements	1576

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1576

A.5.6.1.4.1	Test purpose and Environment	1576

A.5.6.1.4.2	Test Requirements	1579

A.5.6.2	Inter-frequency Measurements	1580

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1580

A.5.6.2.1.1	Test Purpose and Environment	1580

A.5.6.2.1.2	Test Requirements	1583

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1583

A.5.6.2.2.1	Test Purpose and Environment	1583

A.5.6.2.2.2	Test Requirements	1587

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1587

A.5.6.2.3.1	Test Purpose and Environment	1587

A.5.6.2.3.2	Test Requirements	1592

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1592

A.5.6.2.4.1	Test Purpose and Environment	1592

A.5.6.2.4.2	Test Requirements	1596

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1596

A.5.6.2.5.1	Test Purpose and Environment	1596

A.5.6.2.5.2	Test Requirements	1601

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1601

A.5.6.2.6.1	Test Purpose and Environment	1601

A.5.6.2.6.2	Test Requirements	1605

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1605

A.5.6.2.7.1	Test Purpose and Environment	1605

A.5.6.2.7.2	Test Requirements	1610

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1610

A.5.6.2.8.1	Test Purpose and Environment	1610

A.5.6.2.8.2	Test Requirements	1615

A.5.6.3	L1-RSRP measurement for beam reporting	1616

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1616

A.5.6.3.1.1	Test Purpose and Environment	1616

A.5.6.3.1.2	Test parameters	1616

A.5.6.3.1.3	Test Requirements	1618

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1618

A.5.6.3.2.1	Test Purpose and Environment	1618

A.5.6.3.2.2	Test parameters	1619

A.5.6.3.2.3	Test Requirements	1621

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1621

A.5.6.3.3.1	Test Purpose and Environment	1621

A.5.6.3.3.2	Test parameters	1622

A.5.6.3.3.3	Test Requirements	1624

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1624

A.5.6.3.4.1	Test Purpose and Environment	1624

A.5.6.3.4.2	Test parameters	1625

A.5.6.3.4.3	Test Requirements	1627

A.5.6.4	CLI measurements	1628

A.5.6.4.1	SRS-RSRP measurement with DRX	1628

A.5.6.4.1.1	Test Purpose and Environment	1628

A.5.6.4.1.2	Test Parameters	1628

A.5.6.4.1.3	Test Requirements	1630

A.5.6.4.2	CLI-RSSI measurement with DRX	1630

A.5.6.4.2.1	Test Purpose and Environment	1630

A.5.6.4.2.2	Test Parameters	1631

A.5.6.4.2.3	Test Requirements	1633

A.5.6.5	Measurements with autonomous gaps	1633

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1633

A.5.6.5.1.1	Test Purpose and Environment	1633

A.5.6.5.1.2	Test Requirements	1636

A.5.6.6	L1-SINR measurement for beam reporting	1637

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1639

A.5.6.6.2.1	Test Purpose and Environment	1639

A.5.6.6.2.2	Test parameters	1640

A.5.6.6.2.3	Test Requirements	1643

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1643

A.5.6.6.3.1	Test Purpose and Environment	1643

A.5.6.6.3.2	Test parameters	1644

A.5.6.6.3.3	Test Requirements	1646

A.5.6.7	CSI-RS based Intra-frequency Measurements	1646

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1646

A.5.6.7.1.1	Test purpose and Environment	1646

A.5.6.7.1.2	Test Requirements	1649

A.5.6.8	CSI-RS based Inter-frequency Measurements	1649

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1649

A.5.6.8.1.1	Test Purpose and Environment	1649

A.5.6.8.1.2	Test Requirements	1654

A.5.7	Measurement Performance requirements	1654

A.5.7.1	SS-RSRP	1654

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1654

A.5.7.1.1.1	Test Purpose and Environment	1654

A.5.7.1.1.2	Test parameters	1655

A.5.7.1.1.3	Test Requirements	1657

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1658

A.5.7.1.2.1	Test Purpose and Environment	1658

A.5.7.1.2.2	Test parameters	1658

A.5.7.1.2.3	Test Requirements	1662

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1663

A.5.7.1.3.1	Test Purpose and Environment	1663

A.5.7.1.3.2	Test parameters	1663

A.5.7.1.3.3	Test Requirements	1666

A.5.7.2	SS-RSRQ	1667

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1667

A.5.7.2.1.1	Test Purpose and Environment	1667

A.5.7.2.1.2	Test Parameters	1667

A.5.7.2.1.3	Test Requirements	1670

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1670

A.5.7.2.2.1	Test Purpose and Environment	1670

A.5.7.2.2.2	Test Parameters	1670

A.5.7.2.2.3	Test Requirements	1672

A.5.7.3	SS-SINR	1673

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1673

A.5.7.3.1.1	Test Purpose and Environment	1673

A.5.7.3.1.2	Test Parameters	1673

A.5.7.3.1.3	Test Requirements	1676

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1676

A.5.7.3.2.1	Test Purpose and Environment	1676

A.5.7.3.2.2	Test Parameters	1676

A.5.7.3.2.3	Test Requirements	1678

A.5.7.4	L1-RSRP measurement for beam reporting	1678

A.5.7.4.1	SSB based L1-RSRP measurement	1678

A.5.7.4.1.1	Test Purpose and Environment	1678

A.5.7.4.1.2	Test parameters	1679

A.5.7.4.1.3	Test Requirements	1681

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1682

A.5.7.4.2.1	Test Purpose and Environment	1682

A.5.7.4.2.2	Test parameters	1682

A.5.7.4.2.3	Test Requirements	1684

A.5.7.5	CLI measurements	1685

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1685

A.5.7.5.1.1	Test Purpose and Environment	1685

A.5.7.5.1.2	Test parameters	1685

A.5.7.5.1.3	Test Requirements	1688

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1689

A.5.7.5.2.1	Test Purpose and Environment	1689

A.5.7.5.2.2	Test parameters	1689

A.5.7.5.2.3	Test Requirements	1691

A.5.7.6	L1-SINR measurement for beam reporting	1692

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1694

A.5.7.6.2.1	Test Purpose and Environment	1694

A.5.7.6.2.2	Test parameters	1695

A.5.7.6.2.3	Test Requirements	1697

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1698

A.5.7.6.3.1	Test Purpose and Environment	1698

A.5.7.6.3.2	Test parameters	1698

A.5.7.6.3.3	Test Requirements	1700

A.5.7.7	CSI-RSRP	1700

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1700

A.5.7.7.1.1	Test Purpose and Environment	1700

A.5.7.7.1.2	Test parameters	1701

A.5.7.7.1.3	Test Requirements	1703

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1704

A.5.7.7.2.1	Test Purpose and Environment	1704

A.5.7.7.2.2	Test parameters	1704

A.5.7.7.2.3	Test Requirements	1708

A.5.7.8	CSI-RSRQ	1709

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1709

A.5.7.8.1.1	Test Purpose and Environment	1709

A.5.7.8.1.2	Test Parameters	1709

A.5.7.8.1.3	Test Requirements	1711

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1711

A.5.7.8.2.1	Test Purpose and Environment	1711

A.5.7.8.2.2	Test Parameters	1711

A.5.7.8.2.3	Test Requirements	1713

A.5.7.9	CSI-SINR	1713

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1713

A.5.7.9.1.1	Test Purpose and Environment	1713

A.5.7.9.1.2	Test Parameters	1713

A.5.7.9.1.3	Test Requirements	1716

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1716

A.5.7.9.2.1	Test Purpose and Environment	1716

A.5.7.9.2.2	Test Parameters	1716

A.5.7.9.2.3	Test Requirements	1719

A.5.8	Void	1719

A.6	NR standalone tests with all NR cells in FR1	1722

A.6.1	SA: RRC_IDLE state mobility	1722

A.6.1.1	Cell re-selection to NR	1722

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1722

A.6.1.1.1.1	Test Purpose and Environment	1722

A.6.1.1.1.2	Test Parameters	1722

A.6.1.1.1.3	Test Requirements	1726

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1726

A.6.1.1.2.1	Test Purpose and Environment	1726

A.6.1.1.2.2	Test Parameters	1726

A.6.1.1.2.3	Test Requirements	1730

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1730

A.6.1.1.3.1	Test Purpose and Environment	1730

A.6.1.1.3.2	Test Parameters	1730

A.6.1.1.3.3	Test Requirements	1735

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1735

A.6.1.1.4.1	Test Purpose and Environment	1735

A.6.1.1.4.2	Test Parameters	1735

A.6.1.1.4.3	Test Requirements	1738

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1738

A.6.1.1.5.1	Test Purpose and Environment	1738

A.6.1.1.5.2	Test Parameters	1738

A.6.1.1.5.3	Test Requirements	1743

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1743

A.6.1.1.6.1	Test Purpose and Environment	1743

A.6.1.1.6.2	Test Parameters	1744

A.6.1.1.6.3	Test Requirements	1748

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1749

A.6.1.1.7.1	Test Purpose and Environment	1749

A.6.1.1.7.2	Test Parameters	1749

A.6.1.1.7.3	Test Requirements	1752

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1752

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1752

A.6.1.2.1.1	Test Purpose and Environment	1752

A.6.1.2.1.2	Test Parameters	1752

A.6.1.2.1.3	Test Requirements	1755

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1756

A.6.1.2.2.1	Test Purpose and Environment	1756

A.6.1.2.2.2	Test Parameters	1756

A.6.1.2.2.3	Test Requirements	1759

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1760

A.6.1.2.3.1	Test Purpose and Environment	1760

A.6.1.2.3.2	Test Parameters	1760

A.6.1.2.3.3	Test Requirements	1763

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1764

A.6.1.2.4.1	Test Purpose and Environment	1764

A.6.1.2.4.2	Test Parameters	1764

A.6.1.2.4.3	Test Requirements	1767

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	1768

A.6.1.2.5.1	Test Purpose and Environment	1768

A.6.1.2.5.2	Test Parameters	1768

A.6.1.2.5.3	Test Requirements	1772

A.6.1.1.7	Void	1773

A.6.2	SA: RRC_INACTIVE state mobility	1773

A.6.3	RRC_CONNECTED state mobility	1773

A.6.3.1	Handover	1773

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1773

A.6.3.1.1.1	Test Purpose and Environment	1773

A.6.3.1.1.2	Test Parameters	1773

A.6.3.1.1.3 Test Requirements	1777

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1777

A.6.3.1.2.1	Test Purpose and Environment	1777

A.6.3.1.2.2	Test Parameters	1777

A.6.3.1.2.3	Test Requirements	1781

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1781

A.6.3.1.3.1	Test Purpose and Environment	1781

A.6.3.1.3.2	Test Parameters	1781

A.6.3.1.3.3	Test Requirements	1785

A.6.3.1.4	SA NR - E-UTRAN handover	1785

A.6.3.1.4.1	Test Purpose and Environment	1785

A.6.3.1.4.2	Test Requirements	1791

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1791

A.6.3.1.5.1	Test Purpose and Environment	1791

A.6.3.1.5.2	Test Requirements	1797

A.6.3.1.6	 SA NR - UTRAN FDD handover	1797

A.6.3.1.6.1	Test Purpose and Environment	1797

A.6.3.1.6.2	Test Requirements	1801

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	1801

A.6.3.1.7.1	Test Purpose and Environment	1801

A.6.3.1.7.2	Test Parameters	1801

A.6.3.1.7.3	Test Requirements	1805

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	1806

A.6.3.1.8.1	Test Purpose and Environment	1806

A.6.3.1.8.2	Test Parameters	1806

A.6.3.1.8.3	Test Requirements	1810

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1811

A.6.3.1.9.1	Test Purpose and Environment	1811

A.6.3.1.9.2	Test Parameters	1811

A.6.3.1.9.3	Test Requirements	1815

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1815

A.6.3.1.10.1	Test Purpose and Environment	1815

A.6.3.1.10.2	Test Parameters	1816

A.6.3.1.10.3	Test Requirements	1819

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1819

A.6.3.1.11.1	Test Purpose and Environment	1819

A.6.3.1.11.2	Test Parameters	1819

A.6.3.1.11.3 Test Requirements	1826

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1826

A.6.3.1.12.1	Test Purpose and Environment	1826

A.6.3.1.12.2	Test Parameters	1827

A.6.3.1.12.3 Test Requirements	1833

A.6.3.2	RRC Connection Mobility Control	1833

A.6.3.2.1	SA: RRC Re-establishment	1833

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1833

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1837

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1841

A.6.3.2.2	Random Access	1844

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1844

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1848

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1853

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1858

A.6.3.2.3	SA: RRC Connection Release with Redirection	1862

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1862

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1866

A.6.3.3 Conditional handover	1873

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1873

A.6.3.3.1.1	Test Purpose and Environment	1873

A.6.3.3.1.2	Test Parameters	1873

A.6.3.3.1.3 Test Requirements	1877

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1877

A.6.3.3.2.1	Test Purpose and Environment	1877

A.6.3.3.2.2	Test Parameters	1877

A.6.3.3.2.3 Test Requirements	1881

A.6.4	Timing	1881

A.6.4.1	UE transmit timing	1881

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1881

A.6.4.1.1.1	Test Purpose and environment	1881

A.6.4.1.1.2	Test requirements	1885

A.6.4.2	UE timer accuracy	1885

A.6.4.3	Timing advance	1885

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1885

A.6.4.3.1.1	Test Purpose and Environment	1885

A.6.4.3.1.2	Test Parameters	1885

A.6.4.3.1.3	Test Requirements	1889

A.6.5	Signalling characteristics	1889

A.6.5.1	Radio link Monitoring	1889

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1890

A.6.5.1.1.1	Test Purpose and Environment	1890

A.6.5.1.1.2	Test Requirements	1895

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1895

A.6.5.1.2.1	Test Purpose and Environment	1895

A.6.5.1.2.2	Test Requirements	1901

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1901

A.6.5.1.3.1	Test Purpose and Environment	1901

A.6.5.1.3.2	Test Requirements	1907

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1907

A.6.5.1.4.1	Test Purpose and Environment	1907

A.6.5.1.4.2	Test Requirements	1913

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1913

A.6.5.1.5.1	Test Purpose and Environment	1913

A.6.5.1.5.2	Test Requirements	1919

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1919

A.6.5.1.6.1	Test Purpose and Environment	1919

A.6.5.1.6.2	Test Requirements	1925

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1925

A.6.5.1.7.1	Test Purpose and Environment	1925

A.6.5.1.7.2	Test Requirements	1930

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1930

A.6.5.1.8.1	Test Purpose and Environment	1930

A.6.5.1.8.2	Test Requirements	1936

A.6.5.2	Interruption	1936

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1936

A.6.5.2.1.2	Test Requirements	1940

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1941

A.6.5.2.2.1	Test Purpose and Environment	1941

A.6.5.2.2.2	Test Parameters	1941

A.6.5.2.2.3	Test Requirements	1943

A.6.5.3	SCell Activation and Deactivation Delay	1944

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1944

A.6.5.3.1.1	Test Purpose and Environment	1944

A.6.5.3.1.2	Test Requirements	1948

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	1948

A.6.5.3.2.1	Test Purpose and Environment	1948

A.6.5.3.2.2	Test Requirements	1948

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1949

A.6.5.3.3.1	Test Purpose and Environment	1949

A.6.5.3.3.2	Test Requirements	1949

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1950

A.6.5.3.4.1	Test Purpose and Environment	1950

A.6.5.3.4.2	Test Requirements	1954

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1954

A.6.5.3.5.1	Test Purpose and Environment	1954

A.6.5.3.5.2	Test Requirements	1958

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1959

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1959

A.6.5.4.1.1	Test Purpose and Environment	1959

A.6.5.4.1.2	Test Requirements	1969

A.6.5.4.2	Void	1970

A.6.5.5	Beam Failure Detection and Link recovery procedures	1970

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1970

A.6.5.5.1.1	Test Purpose and Environment	1970

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1975

A.6.5.5.2.1	Test Purpose and Environment	1975

A.6.5.5.2.2	Test Requirements	1982

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1982

A.6.5.5.3.1	Test Purpose and Environment	1982

A.6.5.5.3.2	Test Requirements	1988

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1988

A.6.5.5.4.1	Test Purpose and Environment	1988

A.6.5.5.4.2	Test Requirements	1994

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1994

A.6.5.5.5.1	Test Purpose and Environment	1994

A.6.5.5.5.2	Test Requirements	1998

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1999

A.6.5.5.6.1	Test Purpose and Environment	1999

A.6.5.5.6.2	Test Requirements	2003

A.6.5.6	Active BWP switch	2003

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2003

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2003

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2009

A.6.5.6.2	RRC-based Active BWP Switch	2013

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2013

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2017

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2017

A.6.5.6.3.1.1	Test Purpose and Environment	2017

A.6.5.6.3.1.2	Test Requirements	2022

A.6.5.6.4	SCell dormancy switch	2023

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2023

A.6.5.6.4.1.1	Test Purpose and Environment	2023

A.6.5.6.4.1.2	Test Requirements	2028

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2029

A.6.5.6.4.2.1	 Test Purpose and Environment	2029

A.6.5.6.4.2.2	 Test Requirements	2033

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2033

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2033

A.6.5.7	DL interruptions at switching between two uplink carriers	2037

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2037

A.6.5.7.1.1	Test Purpose and Environment	2037

A.6.5.7.1.2	Test Requirements	2041

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2041

A.6.5.7.2.1	Test Purpose and Environment	2041

A.6.5.7.2.2	Test Requirements	2045

A.6.5.8	UE specific CBW change	2045

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2045

A.6.5.8.1.1	Test Purpose and Environment	2045

A.6.5.8.1.2	Test Requirements	2049

A.6.5.9	Pathloss reference signal switching delay	2049

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2049

A.6.5.9.1.1	Test Purpose and Environment	2049

A.6.5.9.1.2	Test Requirements	2052

A.6.6	Measurement procedure	2053

A.6.6.1	Intra-frequency Measurements	2053

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2053

A.6.6.1.1.1	Test purpose and Environment	2053

A.6.6.1.1.2	Test parameters	2053

A.6.6.1.1.3	Test Requirements	2057

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2057

A.6.6.1.2.1	Test purpose and Environment	2057

A.6.6.1.2.2	Test parameters	2057

A.6.6.1.2.3	Test Requirements	2061

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2061

A.6.6.1.3.1	Test purpose and Environment	2061

A.6.6.1.3.2	Test parameters	2061

A.6.6.1.3.3	Test Requirements	2065

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2065

A.6.6.1.4.1	Test purpose and Environment	2065

A.6.6.1.4.2	Test parameters	2065

A.6.6.1.4.3	Test Requirements	2069

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2069

A.6.6.1.5.1	Test purpose and Environment	2069

A.6.6.1.5.2	Test parameters	2069

A.6.6.1.5.3	Test Requirements	2071

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2072

A.6.6.1.6.1	Test purpose and Environment	2072

A.6.6.1.6.2	Test parameters	2072

A.6.6.1.6.3	Test Requirements	2073

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2074

A.6.6.1.7.1	Test purpose and Environment	2074

A.6.6.1.7.2	Test parameters	2074

A.6.6.1.7.3	Test Requirements	2078

A.6.6.2	Inter-frequency Measurements	2078

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2078

A.6.6.2.1.1	Test Purpose and Environment	2078

A.6.6.2.1.2	Test Requirements	2082

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2082

A.6.6.2.2.1	Test Purpose and Environment	2082

A.6.6.2.2.2	Test Requirements	2086

A.6.6.2.3	Void	2087

A.6.6.2.4	Void	2087

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2087

A.6.6.2.5.1	Test Purpose and Environment	2087

A.6.6.2.5.2	Test Requirements	2091

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2091

A.6.6.2.6.1	Test Purpose and Environment	2091

A.6.6.2.6.2	Test Requirements	2095

A.6.6.2.7	Void	2096

A.6.6.2.8	Void	2096

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2096

A.6.6.2.9.1	Test Purpose and Environment	2096

A.6.6.2.9.2	Test Requirements	2099

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2099

A.6.6.2.10.1	Test Purpose and Environment	2099

A.6.6.2.10.2	Test Requirements	2103

A.6.6.3	Inter-RAT Measurements	2107

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2107

A.6.6.3.1.1	Test Purpose and Environment	2107

A.6.6.3.1.2	Test Requirements	2113

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2113

A.6.6.3.2.1	Test Purpose and Environment	2113

A.6.6.3.2.2	Test Requirements	2120

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2120

A.6.6.3.3.1	Test Purpose and Environment	2120

A.6.6.3.3.2	Test Requirements	2124

A.6.6.4	L1-RSRP measurement for beam reporting	2124

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2124

A.6.6.4.1.1	Test Purpose and Environment	2124

A.6.6.4.1.2	Test parameters	2125

A.6.6.4.1.3	Test Requirements	2128

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2128

A.6.6.4.2.1	Test Purpose and Environment	2128

A.6.6.4.2.2	Test parameters	2129

A.6.6.4.2.3	Test Requirements	2132

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2132

A.6.6.4.3.1	Test Purpose and Environment	2132

A.6.6.4.3.2	Test parameters	2133

A.6.6.4.3.3	Test Requirements	2136

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2136

A.6.6.4.4.1	Test Purpose and Environment	2136

A.6.6.4.4.2	Test parameters	2137

A.6.6.4.4.3	Test Requirements	2140

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2140

A.6.6.4.5.1	Test Purpose and Environment	2140

A.6.6.4.5.2	Test parameters	2141

A.6.6.4.5.3	Test Requirements	2143

A.6.6.5	Inter-RAT UTRAN FDD measurements	2143

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2143

A.6.6.5.1.1	Test Purpose and Environment	2143

A.6.6.5.1.2	Test Requirements	2147

A.6.6.6	CLI measurements	2147

A.6.6.6.1	SRS-RSRP measurement with DRX	2147

A.6.6.6.1.1	Test Purpose and Environment	2147

A.6.6.6.1.2	Test Parameters	2148

A.6.6.6.1.3	Test Requirements	2151

A.6.6.6.2	CLI-RSSI measurement with DRX	2151

A.6.6.6.2.1	Test Purpose and Environment	2151

A.6.6.6.2.2	Test Parameters	2152

A.6.6.6.2.3	Test Requirements	2154

A.6.6.7	NR measurements with autonomous gaps	2154

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2154

A.6.6.7.1.1	Test Purpose and Environment	2154

A.6.6.7.1.2	Test Parameters	2154

A.6.6.7.1.3	Test Requirements	2157

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2157

A.6.6.7.2.1	Test Purpose and Environment	2157

A.6.6.7.2.2	Test Requirements	2160

A.6.6.8	L1-SINR measurement for beam reporting	2161

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2163

A.6.6.8.2.1	Test Purpose and Environment	2163

A.6.6.8.2.2	Test parameters	2163

A.6.6.8.2.3	Test Requirements	2167

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2167

A.6.6.8.3.1	Test Purpose and Environment	2167

A.6.6.8.3.2	Test parameters	2168

A.6.6.8.3.3	Test Requirements	2170

A.6.6.9	Idle Mode CA/DC Measurements	2170

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2170

A.6.6.9.1.1	Test Purpose and Environment	2170

A.6.6.9.1.2	Test Requirements	2177

A.6.6.10	CSI-RS based intra-frequency Measurements	2177

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2177

A.6.6.10.1.1	Test purpose and Environment	2177

A.6.6.10.1.2	Test Requirements	2180

A.6.6.11	CSI-RS based inter-frequency Measurements	2180

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2180

A.6.6.11.1.1	Test Purpose and Environment	2180

A.6.6.11.1.2	Test Requirements	2184

A.6.6.12	RSTD measurements	2184

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2184

A.6.6.12.1.1	Test Purpose and Environment	2184

A.6.6.12.1.2	Test Requirements	2190

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2191

A.6.6.12.2.1	Test Purpose and Environment	2191

A.6.6.12.2.2	Test Requirements	2196

A.6.6.13 PRS-RSRP measurements	2197

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2197

A.6.6.13.1.1	Test purpose and Environment	2197

A.6.6.13.1.2	Test Requirements	2201

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2201

A.6.6.13.2.1	Test purpose and Environment	2201

A.6.6.13.2.2	Test Requirements	2205

A.6.6.14	UE Rx-Tx time difference measurements	2205

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2205

A.6.6.14.1.1	Test purpose and environment	2205

A.6.6.14.1.2	Test requirements	2209

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2209

A.6.6.14.2.1	Test purpose and environment	2209

A.6.6.14.2.2	Test requirements	2213

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	2213

A.6.6.15.1	Test Purpose and Environment	2213

A.6.6.15.2	Test Requirements	2220

A.6.7	Measurement Performance requirements	2221

A.6.7.1	SS-RSRP	2221

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2221

A.6.7.1.1.1	Test Purpose and Environment	2221

A.6.7.1.1.2	Test parameters	2221

A.6.7.1.1.3	Test Requirements	2226

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2226

A.6.7.1.2.1	Test Purpose and Environment	2226

A.6.7.1.2.2	Test parameters	2226

A.6.7.1.2.3	Test Requirements	2230

A.6.7.1.3	Void	2230

A.6.7.2	SS-RSRQ	2230

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2230

A.6.7.2.1.1	Test Purpose and Environment	2230

A.6.7.2.1.2	Test Parameters	2231

A.6.7.2.1.3	Test Requirements	2236

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2236

A.6.7.2.2.1	Test Purpose and Environment	2236

A.6.7.2.2.2	Test Parameters	2236

A.6.7.2.2.3	Test Requirements	2241

A.6.7.3	SS-SINR	2241

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2241

A.6.7.3.1.1	Test Purpose and Environment	2241

A.6.7.3.1.2	Test Parameters	2242

A.6.7.3.1.3	Test Requirements	2246

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2246

A.6.7.3.2.1	Test Purpose and Environment	2246

A.6.7.3.2.2	Test Parameters	2247

A.6.7.3.2.3	Test Requirements	2251

A.6.7.4	L1-RSRP measurement for beam reporting	2252

A.6.7.4.1	SSB based L1-RSRP measurement	2252

A.6.7.4.1.1	Test Purpose and Environment	2252

A.6.7.4.1.2	Test parameters	2252

A.6.7.4.1.3	Test Requirements	2256

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2256

A.6.7.4.2.1	Test Purpose and Environment	2256

A.6.7.4.2.2	Test parameters	2257

A.6.7.4.2.3	Test Requirements	2261

A.6.7.5	E-UTRAN RSRP	2261

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2261

A.6.7.5.1.1	Test Purpose and Environment	2261

A.6.7.5.1.2	Test parameters	2262

A.6.7.5.1.3	Test Requirements	2268

A.6.7.6	E-UTRAN RSRQ	2269

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2269

A.6.7.6.1.1	Test Purpose and Environment	2269

A.6.7.6.1.2	Test parameters	2269

A.6.7.6.1.3	Test Requirements	2274

A.6.7.7	E-UTRAN RS-SINR	2275

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2275

A.6.7.7.1.1	Test Purpose and Environment	2275

A.6.7.7.1.2	Test parameters	2275

A.6.7.7.1.3	Test Requirements	2281

A.6.7.8	CLI measurements	2282

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2282

A.6.7.8.1.1	Test Purpose and Environment	2282

A.6.7.8.1.2	Test parameters	2282

A.6.7.8.1.3	Test Requirements	2288

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2288

A.6.7.8.2.1	Test Purpose and Environment	2288

A.6.7.8.2.2	Test parameters	2289

A.6.7.8.2.3	Test Requirements	2292

A.6.7.9	L1-SINR measurement for beam reporting	2293

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2297

A.6.7.9.2.1	Test Purpose and Environment	2297

A.6.7.9.2.2	Test parameters	2298

A.6.7.9.2.3	Test Requirements	2303

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2303

A.6.7.9.3.1	Test Purpose and Environment	2303

A.6.7.9.3.2	Test parameters	2303

A.6.7.9.3.3	Test Requirements	2308

A.6.7.10	CSI-RSRP	2308

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2308

A.6.7.9.10.1	Test Purpose and Environment	2308

A.6.7.9.10.2	Test parameters	2309

A.6.7.10.1.3	Test Requirements	2314

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2314

A.6.7.9.10.1	Test Purpose and Environment	2314

A.6.7.10.2.2	Test parameters	2314

A.6.7.10.2.3	Test Requirements	2319

A.6.7.11	CSI-RSRQ	2319

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2319

A.6.7.11.1.1	Test Purpose and Environment	2319

A.6.7.11.1.2	Test Parameters	2319

A.6.7.11.1.3	Test Requirements	2324

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2324

A.6.7.11.2.1	Test Purpose and Environment	2324

A.6.7.11.2.2	Test Parameters	2324

A.6.7.11.2.3	Test Requirements	2330

A.6.7.12	CSI-SINR	2331

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2331

A.6.7.12.1.1	Test Purpose and Environment	2331

A.6.7.12.1.2	Test Parameters	2331

A.6.7.12.1.3	Test Requirements	2335

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2336

A.6.7.12.2.1	Test Purpose and Environment	2336

A.6.7.12.2.2	Test Parameters	2336

A.6.7.12.2.3	Test Requirements	2342

A.6.7.13	RSTD measurements	2342

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	2342

A.6.7.13.1.1	Test purpose and Environment	2342

A.6.7.13.1.2	Test Requirements	2345

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	2345

A.6.7.13.2.1	Test purpose and Environment	2345

A.6.7.13.2.2	Test Requirements	2349

A.6.7.14	PRS-RSRP measurements	2349

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	2349

A.6.7.14.1.1	Test Purpose and Environment	2349

A.6.7.14.1.2	Test parameters	2349

A.6.7.14.1.3	Test Requirements	2354

A.6.7.15	UE Rx-Tx time difference measurements	2354

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	2354

A.6.7.15.1.1	Test purpose and environment	2354

A.6.7.15.1.2	Test parameters	2355

A.6.7.15.1.3	Test requirements	2358

A.7	NR standalone tests with one or more NR cells in FR2	2358

A.7.1	SA: RRC_IDLE state mobility	2358

A.7.1.1	Cell re-selection to NR	2358

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	2358

A.7.1.1.1.1	Test Purpose and Environment	2358

A.7.1.1.1.2	Test Parameters	2358

A.7.1.1.1.3	Test Requirements	2362

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	2362

A.7.1.1.2.1	Test Purpose and Environment	2362

A.7.1.1.2.2	Test Parameters	2362

A.7.1.1.2.3	Test Requirements	2366

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2366

A.7.1.1.3.1	Test Purpose and Environment	2366

A.7.1.1.3.2	Test Parameters	2366

A.7.1.1.3.3	Test Requirements	2369

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2369

A.7.1.1.4.1	Test Purpose and Environment	2369

A.7.1.1.4.2	Test Parameters	2369

A.7.1.1.4.3	Test Requirements	2372

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2372

A.7.1.1.5.1	Test Purpose and Environment	2372

A.7.1.1.5.2	Test Parameters	2372

A.7.1.1.5.3	Test Requirements	2375

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2375

A.7.1.1.6.1	Test Purpose and Environment	2375

A.7.1.1.6.2	Test Parameters	2375

A.7.1.1.6.3	Test Requirements	2377

A.7.2	SA: RRC_INACTIVE state mobility	2378

A.7.3	RRC_CONNECTED state mobility	2378

A.7.3.1	Handover	2378

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	2378

A.7.3.1.1.1	Test Purpose and Environment	2378

A.7.3.1.1.2	Test Parameters	2378

A.7.3.1.1.3	Test Requirements	2382

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	2382

A.7.3.1.2.1	Test Purpose and Environment	2382

A.7.3.1.2.2	Test Parameters	2382

A.7.3.1.2.3	Test Requirements	2385

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	2385

A.7.3.1.3.1	Test Purpose and Environment	2385

A.7.3.1.3.2	Test Parameters	2385

A.7.3.1.3.3 Test Requirements	2387

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	2388

A.7.3.1.4.1	Test Purpose and Environment	2388

A.7.3.1.4.2	Test Parameters	2388

A.7.3.1.4.3 Test Requirements	2395

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	2395

A.7.3.1.5.1	Test Purpose and Environment	2395

A.7.3.1.5.2	Test Parameters	2395

A.7.3.1.5.3 Test Requirements	2402

A.7.3.2	RRC Connection Mobility Control	2402

A.7.3.2.1	SA: RRC Re-establishment	2402

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	2402

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	2405

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	2408

A.7.3.2.1.3.1	Test Purpose and Environment	2408

A.7.3.2.1.3.2	Test Requirements	2410

A.7.3.2.2	Random Access	2411

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	2411

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	2415

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	2419

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	2422

A.7.3.2.3	SA: RRC Connection Release with Redirection	2425

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	2425

A.7.3.3 Conditional Handover	2429

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	2429

A.7.3.3.1.1	Test Purpose and Environment	2429

A.7.3.3.1.2	Test Parameters	2429

A.7.3.3.1.2.3	Test Requirements	2432

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	2432

A.7.3.3.2.1	Test Purpose and Environment	2432

A.7.3.3.2.2	Test Parameters	2432

A.7.3.3.2.3 Test Requirements	2434

A.7.4	Timing	2434

A.7.4.1	UE transmit timing	2434

A.7.4.1.1	NR UE Transmit Timing Test for FR2	2434

A.7.4.1.1.1	Test Purpose and environment	2434

A.7.4.1.1.2	Test requirements	2437

A.7.4.2	UE timer accuracy	2438

A.7.4.3	Timing advance	2438

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	2438

A.7.4.3.1.1	Test Purpose and Environment	2438

A.7.4.3.1.2	Test Parameters	2438

A.7.4.3.1.3 Test Requirements	2441

A.7.5	Signaling characteristics	2442

A.7.5.1	Radio link Monitoring	2442

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2442

A.7.5.1.1.1	Test Purpose and Environment	2442

A.7.5.1.1.2	Test Requirements	2445

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2446

A.7.5.1.2.1	Test Purpose and Environment	2446

A.7.5.1.2.2	Test Requirements	2450

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2451

A.7.5.1.3.1	Test Purpose and Environment	2451

A.7.5.1.3.2	Test Requirements	2455

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2455

A.7.5.1.4.1	Test Purpose and Environment	2455

A.7.5.1.4.2	Test Requirements	2460

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2460

A.7.5.1.5.1	Test Purpose and Environment	2460

A.7.5.1.5.2	Test Requirements	2465

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2465

A.7.5.1.6.1	Test Purpose and Environment	2465

A.7.5.1.6.2	Test Requirements	2469

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2469

A.7.5.1.7.1	Test Purpose and Environment	2469

A.7.5.1.7.2	Test Requirements	2473

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2473

A.7.5.1.8.1	Test Purpose and Environment	2473

A.7.5.1.8.2	Test Requirements	2478

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	2478

A.7.5.1.9.1	Test Purpose and Environment	2478

A.7.5.1.9.2	Test Requirements	2481

A.7.5.2	Interruption	2481

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	2481

A.7.5.2.1.1	Test Purpose and Environment	2481

A.7.5.2.1.2	Test Requirements	2484

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	2485

A.7.5.2.2.1	Test Purpose and Environment	2485

A.7.5.2.2.2	Test Parameters	2485

A.7.5.2.2.3	Test Requirements	2487

A.7.5.3	SCell Activation and Deactivation Delay	2487

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	2487

A.7.5.3.1.1	Test Purpose and Environment	2487

A.7.5.3.1.2	Test Requirements	2489

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	2489

A.7.5.3.2.1	Test Purpose and Environment	2489

A.7.5.3.2.2	Test Requirements	2493

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	2494

A.7.5.3.3.1	Test Purpose and Environment	2494

A.7.5.3.3.2	Test Requirements	2497

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	2498

A.7.5.3.4.1	Test Purpose and Environment	2498

A.7.5.3.4.2	Test Requirements	2501

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	2502

A.7.5.3.5.1	Test Purpose and Environment	2502

A.7.5.3.5.2	Test Requirements	2504

A.7.5.4	Void	2505

A.7.5.5	Beam Failure Detection and Link recovery procedures	2505

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	2505

A.7.5.5.1.1	Test Purpose and Environment	2505

A.7.5.5.1.2	Test Requirements	2509

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	2509

A.7.5.5.2.1	Test Purpose and Environment	2509

A.7.5.5.2.2	Test Requirements	2513

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2513

A.7.5.5.3.1	Test Purpose and Environment	2513

A.7.5.5.3.2	Test Requirements	2517

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	2517

A.7.5.5.4.1	Test Purpose and Environment	2517

A.7.5.5.4.2	Test Requirements	2521

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	2521

A.7.5.5.5.1	Test Purpose and Environment	2521

A.7.5.5.5.2	Test Requirements	2525

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	2525

A.7.5.5.6.1	Test Purpose and Environment	2525

A.7.5.5.6.2	Test Requirements	2529

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	2529

A.7.5.5.7.1	Test Purpose and Environment	2529

A.7.5.5.7.2	Test Requirements	2533

A.7.5.6	Active BWP switch	2533

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	2533

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	2533

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	2538

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	2543

A.7.5.6.1.3.1	Test Purpose and Environment	2543

A.7.5.6.1.3.2	Test Requirements	2546

A.7.5.6.2	RRC-based Active BWP Switch	2546

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2550

A.7.5.6.3.1.1	Test Purpose and Environment	2550

A.7.5.6.3.1.2	Test Requirements	2553

A.7.5.6.4	SCell dormancy switch	2553

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	2553

A.7.5.6.4.1.1	Test Purpose and Environment	2553

A.7.5.6.4.1.2	Test Requirements	2558

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	2558

A.7.5.6.4.2.1	 Test Purpose and Environment	2558

A.7.5.6.4.2.2	 Test Requirements	2563

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2563

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	2563

A.7.5.7	PSCell addition and release delay	2566

A.7.5.7.1	Addition and Release Delay of known NR PSCell	2566

A.7.5.7.1.1	Test Purpose and Environment	2566

A.7.5.7.1.2	Test Requirements	2569

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	2569

A.7.5.7.2.1	Test Purpose and Environment	2569

A.7.5.7.2.2	Test Requirements	2572

A.7.5.8	Active TCI state switch delay	2572

A.7.5.8.1	MAC-CE based active TCI state switch	2572

A.7.5.8.2	RRC based active TCI state switch	2576

A.7.5.9	Uplink spatial relation switch delay	2580

A.7.5.9.1.1.1	Test Purpose and Environment	2580

A.7.5.9.1.1.2	Test Requirements	2583

A.7.5.9.2	RRC based spatial relation switch	2583

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	2583

A.7.5.9.2.1.1	Test Purpose and Environment	2583

A.7.5.9.2.1.2	Test Requirements	2586

A.7.5.10	UE specific CBW change	2586

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	2586

A.7.5.10.1.1	Test Purpose and Environment	2586

A.7.5.10.1.2	Test Requirements	2589

A.7.6	Measurement procedure	2590

A.7.6.1	Intra-frequency Measurements	2590

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	2590

A.7.6.1.1.1	Test purpose and Environment	2590

A.7.6.1.1.2	Test Requirements	2593

A.7.6.1.2	SA event triggered reporting test without gap under DRX	2593

A.7.6.1.2.1	Test purpose and Environment	2593

A.7.6.1.2.2	Test Requirements	2596

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	2597

A.7.6.1.3.1	Test purpose and Environment	2597

A.7.6.1.3.2	Test Requirements	2600

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	2600

A.7.6.1.4.1	Test purpose and Environment	2600

A.7.6.1.4.2	Test Requirements	2603

A.7.6.2	Inter-frequency Measurements	2604

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	2604

A.7.6.2.1.1	Test Purpose and Environment	2604

A.7.6.2.1.2	Test Requirements	2607

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	2607

A.7.6.2.2.1	Test Purpose and Environment	2607

A.7.6.2.2.2	Test Requirements	2611

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	2611

A.7.6.2.3.1	Test Purpose and Environment	2611

A.7.6.2.3.2	Test Requirements	2615

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	2615

A.7.6.2.4.1	Test Purpose and Environment	2615

A.7.6.2.4.2	Test Requirements	2619

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	2619

A.7.6.2.5.1	Test Purpose and Environment	2619

A.7.6.2.5.2	Test Requirements	2623

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	2624

A.7.6.2.6.1	Test Purpose and Environment	2624

A.7.6.2.6.2	Test Requirements	2628

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	2629

A.7.6.2.7.1	Test Purpose and Environment	2629

A.7.6.2.7.2	Test Requirements	2633

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	2634

A.7.6.2.8.1	Test Purpose and Environment	2634

A.7.6.2.8.2	Test Requirements	2638

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	2639

A.7.6.2.9.1	Test Purpose and Environment	2639

A.7.6.2.9.2	Test Requirements	2643

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	2643

A.7.6.2.10.1	Test Purpose and Environment	2643

A.7.6.2.10.2	Test Requirements	2645

A.7.6.2.11	SA event triggered reporting test without gap under DRX	2645

A.7.6.2.11.1	Test Purpose and Environment	2645

A.7.6.2.11.2	Test Requirements	2648

A.7.6.3	L1-RSRP measurement for beam reporting	2649

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2649

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2649

A.7.6.3.1.1	Test Purpose and Environment	2649

A.7.6.3.1.2	Test parameters	2649

A.7.6.3.1.3	Test Requirements	2651

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	2651

A.7.6.3.2.1	Test Purpose and Environment	2651

A.7.6.3.2.2	Test parameters	2652

A.7.6.3.2.3	Test Requirements	2654

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2654

A.7.6.3.3.1	Test Purpose and Environment	2654

A.7.6.3.3.2	Test parameters	2655

A.7.6.3.3.3	Test Requirements	2657

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2658

A.7.6.3.4.1	Test Purpose and Environment	2658

A.7.6.3.4.2	Test parameters	2658

A.7.6.3.3.3	Test Requirements	2660

A.7.6.4	CLI measurements	2661

A.7.6.4.1	SRS-RSRP measurement with non-DRX	2661

A.7.6.4.1.1	Test Purpose and Environment	2661

A.7.6.4.1.2	Test Parameters	2661

A.7.6.4.1.3	Test Requirements	2663

A.7.6.4.2	CLI-RSSI measurement with non-DRX	2663

A.7.6.4.2.1	Test Purpose and Environment	2663

A.7.6.4.2.2	Test Parameters	2664

A.7.6.4.2.3	Test Requirements	2665

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	2666

A.7.6.5.1.1	Test Purpose and Environment	2666

A.7.6.5.1.2	Test Requirements	2669

A.7.6.6	L1-SINR measurement for beam reporting	2669

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	2671

A.7.6.6.2.1	Test Purpose and Environment	2671

A.7.6.6.2.2	Test parameters	2672

A.7.6.6.2.3	Test Requirements	2674

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	2674

A.7.6.6.3.1	Test Purpose and Environment	2674

A.7.6.6.3.2	Test parameters	2675

A.7.6.6.3.3	Test Requirements	2677

A.7.6.7	CSI-RS based intra-frequency Measurements	2677

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	2677

A.7.6.7.1.1	Test purpose and Environment	2677

A.7.6.7.1.2	Test Requirements	2680

A.7.6.8	CSI-RS based inter-frequency Measurements	2680

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	2680

A.7.6.8.1.1	Test Purpose and Environment	2680

A.7.6.8.1.2	Test Requirements	2683

A.7.6.9	RSTD measurements	2683

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	2683

A.7.6.9.1.1	Test Purpose and Environment	2683

A.7.6.9.1.2	Test Requirements	2688

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	2688

A.7.6.9.2.1	Test Purpose and Environment	2688

A.7.6.9.2.2	Test Requirements	2694

A.7.6.10 PRS-RSRP measurements	2694

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	2694

A.7.6.10.1.1	Test Purpose and Environment	2694

A.7.6.10.1.2	Test Requirements	2698

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2698

A.7.6.10.2.1	Test Purpose and Environment	2698

A.7.6.10.2.2	Test Requirements	2702

A.7.6.11	UE Rx-Tx time difference measurements	2702

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	2702

A.7.6.11.1.1	Test purpose and environment	2702

A.7.6.11.1.2	Test requirements	2704

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	2705

A.7.6.11.2.1	Test purpose and environment	2705

A.7.6.11.2.2	Test requirements	2707

A.7.7	Measurement Performance requirements	2708

A.7.7.1	SS-RSRP	2708

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2708

A.7.7.1.1.1	Test Purpose and Environment	2708

A.7.7.1.1.2	Test parameters	2708

A.7.7.1.1.3	Test Requirements	2712

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2712

A.7.7.1.2.1	Test Purpose and Environment	2712

A.7.7.1.2.2	Test parameters	2713

A.7.7.1.2.3	Test Requirements	2717

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2718

A.7.7.1.3.1	Test Purpose and Environment	2718

A.7.7.1.3.2	Test parameters	2718

A.7.7.1.3.3	Test Requirements	2722

A.7.7.2	SS-RSRQ	2722

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2722

A.7.7.2.1.1	Test Purpose and Environment	2722

A.7.7.2.1.2	Test Parameters	2722

A.7.7.2.1.3	Test Requirements	2724

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2724

A.7.7.2.2.1	Test Purpose and Environment	2724

A.7.7.2.2.2	Test Parameters	2724

A.7.7.2.2.3	Test Requirements	2726

A.7.7.3	SS-SINR	2726

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2726

A.7.7.3.1.1	Test Purpose and Environment	2726

A.7.7.3.1.2	Test Parameters	2726

A.7.7.3.1.3	Test Requirements	2729

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2729

A.7.7.3.2.1	Test Purpose and Environment	2729

A.7.7.3.2.2	Test Parameters	2729

A.7.7.3.2.3	Test Requirements	2731

A.7.7.4	L1-RSRP measurement for beam reporting	2731

A.7.7.4.1	SSB based L1-RSRP measurement	2731

A.7.7.4.1.1	Test Purpose and Environment	2731

A.7.7.4.1.2	Test parameters	2732

A.7.7.4.1.3	Test Requirements	2734

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2735

A.7.7.4.2.1	Test Purpose and Environment	2735

A.7.7.4.2.2	Test parameters	2735

A.7.7.4.2.3	Test Requirements	2737

A.7.7.5	CLI measurements	2738

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2738

A.7.7.5.1.1	Test Purpose and Environment	2738

A.7.7.5.1.2	Test parameters	2738

A.7.7.5.1.3	Test Requirements	2741

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2742

A.7.7.5.2.1	Test Purpose and Environment	2742

A.7.7.5.2.2	Test parameters	2742

A.7.7.5.2.3	Test Requirements	2744

A.7.7.6	L1-SINR measurement for beam reporting	2745

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2748

A.7.7.6.2.1	Test Purpose and Environment	2748

A.7.7.6.2.2	Test parameters	2748

A.7.7.6.2.3	Test Requirements	2750

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2750

A.7.7.6.3.1	Test Purpose and Environment	2750

A.7.7.6.3.2	Test parameters	2751

A.7.7.6.3.3	Test Requirements	2753

A.7.7.7	CSI-RSRP	2753

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2753

A.7.7.7.1.1	Test Purpose and Environment	2753

A.7.7.7.1.2	Test parameters	2754

A.7.7.7.1.3	Test Requirements	2757

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2758

A.7.7.7.2.1	Test Purpose and Environment	2758

A.7.7.7.2.2	Test parameters	2758

A.7.7.7.2.3	Test Requirements	2761

A.7.7.8	CSI-RSRQ	2761

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2761

A.7.7.8.1.1	Test Purpose and Environment	2761

A.7.7.8.1.2	Test Parameters	2762

A.7.7.8.1.3	Test Requirements	2764

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2764

A.7.7.8.2.1	Test Purpose and Environment	2764

A.7.7.8.2.2	Test Parameters	2764

A.7.7.8.2.3	Test Requirements	2766

A.7.7.9	CSI-SINR	2766

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2766

A.7.7.9.1.1	Test Purpose and Environment	2766

A.7.7.9.1.2	Test Parameters	2766

A.7.7.9.1.3	Test Requirements	2768

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2768

A.7.7.9.2.1	Test Purpose and Environment	2768

A.7.7.9.2.2	Test Parameters	2769

A.7.7.9.2.3	Test Requirements	2771

A.7.7.10	RSTD measurements	2771

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2771

A.7.7.10.1.1	Test purpose and Environment	2771

A.7.7.10.1.2	Test Requirements	2774

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2774

A.7.7.10.2.1	Test purpose and Environment	2774

A.7.7.10.2.2	Test Requirements	2776

A.7.7.11	PRS-RSRP measurements	2776

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2776

A.7.7.11.1.1	Test Purpose and Environment	2776

A.7.7.11.1.2	Test parameters	2776

A.7.7.11.1.3	Test Requirements	2779

A.7.7.12	UE Rx-Tx time difference measurements	2780

A.7.7.12.1	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2780

A.7.7.12.1.1	Test purpose and environment	2780

A.7.7.12.1.2	Test parameters	2780

A.7.7.12.1.3	Test requirements	2783

A.8	E-UTRA standalone tests for NR RRM	2783

A.8.1	Void	2783

A.8.2	RRC_IDLE state mobility	2783

A.8.2.1	Inter-RAT NR Cell re-selection	2783

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2783

A.8.2.1.1.1	Test Purpose and Environment	2783

A.8.2.1.1.2	Test Requirements	2788

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2789

A.8.2.1.2.1	Test Purpose and Environment	2789

A.8.2.1.2.2	Test Requirements	2793

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2794

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2794

A.8.2.2.1.1	Test Purpose and Environment	2794

A.8.2.2.1.2	Test Requirements	2797

A.8.2.2.2	E-UTRA – NR Early Measruement Reporting for NR in FR2	2798

A.8.2.2.2.1	Test Purpose and Environment	2798

A.8.2.2.2.2	Test Requirements	2800

A.8.3	RRC_CONNECTED state mobility	2801

A.8.3.1	Handover	2801

A.8.3.1.1	E-UTRAN - NR handover in FR1	2801

A.8.3.1.1.1	Test Purpose and Environment	2801

A.8.3.1.1.2	Test Requirements	2806

A.8.4	Measurement procedure	2806

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2806

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2806

A.8.4.1.1.1	Test Purpose and Environment	2806

A.8.4.1.1.2	Test Requirements	2808

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2809

A.8.4.1.2.1	Test Purpose and Environment	2809

A.8.4.1.2.2	Test Requirements	2810

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2810

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2810

A.8.4.2.1.1	Test Purpose and Environment	2810

A.8.4.2.1.2	Test Requirements	2815

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2815

A.8.4.2.2.1	Test Purpose and Environment	2815

A.8.4.2.2.2	Test Requirements	2819

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2819

A.8.4.2.3.1	Test Purpose and Environment	2819

A.8.4.2.3.2	Test Requirements	2824

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2824

A.8.4.2.4.1	Test Purpose and Environment	2824

A.8.4.2.4.2	Test Requirements	2828

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2828

A.8.4.2.5.1	Test Purpose and Environment	2828

A.8.4.2.5.2	Test Requirements	2830

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2831

A.8.4.2.6.1	Test Purpose and Environment	2831

A.8.4.2.6.2	Test Requirements	2833

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2834

A.8.4.2.7.1	Test Purpose and Environment	2834

A.8.4.2.7.2	Test Requirements	2836

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2837

A.8.4.2.8.1	Test Purpose and Environment	2837

A.8.4.2.8.2	Test Requirements	2839

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2840

A.8.4.2.9.1	Test Purpose and Environment	2840

A.8.4.2.9.2	Test Requirements	2844

A.8.5	Measurement performance	2844

A.8.5.1	SFTD accuracy	2844

A.8.5.1.1	SFTD accuracy	2844

A.8.5.1.1.1	Test Purpose	2844

A.8.5.1.1.2	Test Environment	2845

A.8.5.1.1.3	Test Requirements	2851

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2851

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2851

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2856

A.8.5.2.1.2.1	Test Purpose and Environment	2856

A.8.5.2.1.2.2	Test Parameters	2856

A.8.5.2.1.2.3	Test Requirements	2858

A.8.5.2.2	SS-RSRQ	2858

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2858

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2863

A.8.5.2.2.2.1	Test Purpose and Environment	2863

A.8.5.2.2.2.2	Test Parameters	2863

A.8.5.2.2.2.3	Test Requirements	2865

A.8.5.2.3	SS-SINR	2865

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2865

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2869

A.8.5.2.3.2.1	Test Purpose and Environment	2869

A.8.5.2.3.2.2	Test Parameters	2869

A.8.5.2.3.2.3	Test Requirements	2871

A.9	V2X Tests	2872

A.9.1	V2X Tests in FR1	2872

A.9.1.1	Test for V2X UE Transmit Timing	2872

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	2872

A.9.1.1.1.1	Test Purpose and Environment	2872

A.9.1.1.1.2	Test requirements	2872

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2872

A.9.1.1.2.1	Test Purpose and Environment	2872

A.9.1.1.2.2	Test requirements	2873

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2873

A.9.1.1.3.1	Test Purpose and Environment	2873

A.9.1.1.3.2	Test requirements	2876

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	2876

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2876

A.9.1.2.1.1	Test Purpose and Environment	2876

A.9.1.2.1.2	Test Requirements	2879

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2879

A.9.1.2.2.1	Test Purpose and Environment	2879

A.9.1.2.2.2	Test Requirements	2880

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	2881

A.9.1.3.1	 Test for GNSS configured as the highest priority	2881

A.9.1.3.1.1	Test Purpose and Environment	2881

A.9.1.3.1.2	Test Requirements	2883

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2884

A.9.1.3.2.1	Test Purpose and Environment	2884

A.9.1.3.2.2	Test Requirements	2886

A.9.1.4	Test for L1 SL-RSRP Measurement	2887

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2887

A.9.1.4.1.1	Test Purpose and Environment	2887

A.9.1.4.1.2	Test Requirements	2890

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2890

A.9.1.4.2.1	Test Purpose and Environment	2890

A.9.1.4.2.2	Test Requirements	2894

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	2894

A.9.1.4.3.1	Test Purpose and Environment	2894

A.9.1.4.3.2	Test Requirements	2899

A.9.1.5	Test for Congestion Control Measurement	2899

A.9.1.5.1	Test Purpose and Environment	2899

A.9.1.5.2	Test Requirements	2905

A.9.1.6	Test for Interruption	2905

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2905

A.9.1.6.1.1	Test Purpose and Environment	2905

A.9.1.6.1.2	Test Requirements	2908

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2908

A.10.1	RRC_CONNECTED state mobility	2908

A.10.1.1	RRC connection mobility control	2908

A.10.1.1.1	Random Access	2908

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2908

A.10.1.1.1.1.1	Test Purpose and Environment	2908

A.10.1.1.1.1.2	Test Requirements	2911

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2911

A.10.1.1.1.1.2.2	Random Access Response Reception	2911

A.10.1.1.1.1.2.3	No Random Access Response Reception	2912

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2912

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2912

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2913

A.10.1.1.1.2.1	Test Purpose and Environment	2913

A.10.1.1.1.2.2	Test Requirements	2916

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2916

A.10.1.1.1.2.2.2	Random Access Response Reception	2917

A.10.1.1.1.2.2.3	No Random Access Response Reception	2917

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2917

A.10.1.1.1.3.1	Test Purpose and Environment	2917

A.10.1.1.1.3.2	Test Requirements	2921

A.10.1.1.1.3.2.1	MsgA Transmission	2921

A.10.1.1.1.3.2.2	MsgB Reception	2921

A.10.1.1.1.3.2.3	No MsgB Reception	2922

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2922

A.10.1.1.1.4.1	Test Purpose and Environment	2922

A.10.1.1.1.4.2	Test Requirements	2926

A.10.1.1.1.4.2.1	MsgA Transmission	2926

A.10.1.1.1.4.2.2	MsgB Reception	2927

A.10.1.1.1.4.2.3	No MsgB Reception	2927

A.10.2	Timing	2928

A.10.2.1	UE transmit timing	2928

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	2928

A.10.2.1.1.1	Test Purpose and environment	2928

A.10.2.1.1.2	Test requirements	2931

A.10.2.2	UE timing advance	2932

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	2932

A.10.2.2.1.1	Test Purpose and Environment	2932

A.10.2.2.1.2	Test Parameters	2932

A.10.2.2.1.3	Test Requirements	2936

A.10.3	Signalling characteristics	2936

A.10.3.1	Radio link monitoring	2936

A.10.3.1.1	Introduction	2936

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2937

A.10.3.1.2.1	Test purpose and environment	2937

A.10.3.1.2.2	Test requirements	2941

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2942

A.10.3.1.3.1	Test purpose and environment	2942

A.10.3.1.3.2	Test requirements	2947

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2947

A.10.3.1.4.1	Test purpose and environment	2947

A.10.3.1.4.2	Test requirements	2947

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2947

A.10.3.1.5.1	Test purpose and environment	2947

A.10.3.1.5.2	Test requirements	2947

A.10.3.2	Interruption	2947

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2947

A.10.3.2.1.1	Test Purpose and Environment	2947

A.10.3.2.1.2	Test Requirements	2952

A.10.3.3	SCell activation and deactivation delay	2952

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	2952

A.10.3.3.1.1	Test Purpose and Environment	2952

A.10.3.3.1.2	Test Requirements	2957

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 320 ms SCell measurement cycle	2957

A.10.3.3.2.1	Test Purpose and Environment	2957

A.10.3.3.2.2	Test Requirements	2958

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2958

A.10.3.3.3.1	Test Purpose and Environment	2958

A.10.3.3.3.2	Test Requirements	2959

A.10.3.4	Beam failure detection and link recovery procedures	2959

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2959

A.10.3.4.1.1	Test Purpose and Environment	2959

A.10.3.4.1.2	Test Requirements	2964

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2964

A.10.3.4.2.1	Test Purpose and Environment	2964

A.10.3.4.2.2	Test Requirements	2970

A.10.3.5	Active BWP switching	2970

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2970

A.10.3.5.1.2	Test Requirements	2975

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2976

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2976

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2979

A.10.3.5.3	RRC-based Active BWP Switch	2983

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2983

A.10.3.6	PSCell addition and release delay	2987

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2987

A.10.3.6.1.1	Test purpose and environment	2987

A.10.3.6.1.2	Test Requirements	2991

A.10.3.7	Void	2992

A.10.4	Measurement procedure	2992

A.10.4.1	Intra-frequency measurements	2992

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2992

A.10.4.1.1.1	Test purpose and environment	2992

A.10.4.1.1.2	Test parameters	2992

A.10.4.1.1.3	Test Requirements	2992

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2993

A.10.4.1.2.1	Test purpose and environment	2993

A.10.4.1.2.2	Test parameters	2993

A.10.4.1.2.3	Test Requirements	2993

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2993

A.10.4.1.3.1	Test purpose and environment	2993

A.10.4.1.3.2	Test parameters	2993

A.10.4.1.3.3	Test Requirements	2993

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2994

A.10.4.1.4.1	Test purpose and environment	2994

A.10.4.1.4.2	Test parameters	2994

A.10.4.1.4.3	Test Requirements	2994

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2994

A.10.4.1.5.1	Test purpose and environment	2994

A.10.4.1.5.2	Test parameters	2994

A.10.4.1.5.3	Test Requirements	2994

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2995

A.10.4.1.6.1	Test purpose and environment	2995

A.10.4.1.6.2	Test parameters	2995

A.10.4.1.6.3	Test Requirements	2995

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2995

A.10.4.1.7.1	Test purpose and environment	2995

A.10.4.1.7.2	Test parameters	2995

A.10.4.1.7.3	Test Requirements	2995

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2996

A.10.4.1.8.1	Test purpose and environment	2996

A.10.4.1.8.2	Test parameters	2996

A.10.4.1.8.3	Test Requirements	2996

A.10.4.1.9	RSSI measurement reporting on PSCC	2996

A.10.4.1.9.1	Test purpose and environment	2996

A.10.4.1.9.2	Test parameters	2996

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2997

A.10.4.1.10.1	Test purpose and environment	2997

A.10.4.1.10.2	Test parameters	2997

A.10.4.1.11	RSSI measurement reporting on SCC	2997

A.10.4.1.11.1	Test purpose and environment	2997

A.10.4.1.11.2	Test parameters	2997

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2998

A.10.4.1.12.1	Test purpose and environment	2998

A.10.4.1.12.2	Test parameters	2998

A.10.4.2	Inter-frequency measurements	2998

A.10.4.2.1	RSSI measurement reporting	2998

A.10.4.2.1.1	Test purpose and environment	2998

A.10.4.2.1.2	Test parameters	2998

A.10.4.2.2	Channel occupancy measurement reporting	2999

A.10.4.2.2.1	Test purpose and environment	2999

A.10.4.2.2.2	Test parameters	2999

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2999

A.10.4.2.3.1	Test Purpose and Environment	2999

A.10.4.2.3.2	Test Requirements	3004

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	3005

A.10.4.2.4.1	Test Purpose and Environment	3005

A.10.4.2.4.2	Test Requirements	3009

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	3010

A.10.4.2.5.1	Test Purpose and Environment	3010

A.10.4.2.5.2	Test Requirements	3014

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	3015

A.10.4.2.6.1	Test Purpose and Environment	3015

A.10.4.2.6.2	Test Requirements	3019

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	3020

A.10.4.2.7.1	Test Purpose and Environment	3020

A.10.4.2.7.2	Test Requirements	3026

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	3026

A.10.4.2.8.1	Test Purpose and Environment	3026

A.10.4.2.8.2	Test Requirements	3032

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	3033

A.10.4.2.9.1	Test Purpose and Environment	3033

A.10.4.2.9.2	Test Requirements	3038

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	3038

A.10.4.2.10.1	Test Purpose and Environment	3038

A.10.4.2.10.2	Test Requirements	3044

A.10.4.3	L1-RSRP measurements for beam reporting	3045

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	3045

A.10.4.3.1.1	Test Purpose and Environment	3045

A.10.4.3.1.2	Test parameters	3045

A.10.4.3.1.3	Test Requirements	3047

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	3048

A.10.4.3.2.1	Test Purpose and Environment	3048

A.10.4.3.2.2	Test parameters	3048

A.10.4.3.2.3	Test Requirements	3050

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	3051

A.10.4.3.3.1	Test Purpose and Environment	3051

A.10.4.3.3.2	Test parameters	3051

A.10.4.3.3.3	Test Requirements	3054

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	3055

A.10.4.3.4.1	Test Purpose and Environment	3055

A.10.4.3.4.2	Test parameters	3055

A.10.4.3.4.3	Test Requirements	3058

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	3059

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3059

A.10.4.4.1.1	Test Purpose and Environment	3059

A.10.4.4.1.2	Test Requirements	3063

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3064

A.10.4.4.2.1	Test Purpose and Environment	3064

A.10.4.4.2.2	Test Requirements	3069

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3070

A.10.4.4.3.1	Test Purpose and Environment	3070

A.10.4.4.3.2	Test Requirements	3074

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3075

A.10.4.4.4.1	Test Purpose and Environment	3075

A.10.4.4.4.2	Test Requirements	3080

A.10.5	Measurement performance	3081

A.10.5.1	SS-RSRP	3081

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	3081

A.10.5.1.1.1	Test Purpose and Environment	3081

A.10.5.1.1.2	Test parameters	3081

A.10.5.1.1.3	Test Requirements	3084

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	3084

A.10.5.1.2.1	Test Purpose and Environment	3084

A.10.5.1.2.2	Test parameters	3084

A.10.5.1.2.3	Test Requirements	3087

A.10.5.2	SS-RSRQ	3087

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	3087

A.10.5.2.1.1	Test Purpose and Environment	3087

A.10.5.2.1.2	Test Parameters	3087

A.10.5.2.1.3	Test Requirements	3090

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	3090

A.10.5.2.2.1	Test Purpose and Environment	3090

A.10.5.2.2.2	Test Parameters	3090

A.10.5.2.2.3	Test Requirements	3093

A.10.5.3	SS-SINR	3093

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	3093

A.10.5.3.1.1	Test Purpose and Environment	3093

A.10.5.3.1.2	Test Parameters	3093

A.10.5.3.1.3	Test Requirements	3096

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	3096

A.10.5.3.2.1	Test Purpose and Environment	3096

A.10.5.3.2.2	Test Parameters	3096

A.10.5.3.2.3	Test Requirements	3099

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	3099

A.10.5.3.3.1	Test Purpose and Environment	3099

A.10.5.3.3.2	Test Parameters	3099

A.10.5.3.3.3	Test Requirements	3102

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	3102

A.10.5.4.1	SSB based L1-RSRP measurement	3102

A.10.5.4.1.1	Test Purpose and Environment	3102

A.10.5.4.1.2	Test parameters	3103

A.10.5.4.1.3	Test Requirements	3105

A.10.5.5	RSSI	3105

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	3105

A.10.5.5.1.1	Test Purpose and Environment	3105

A.10.5.5.1.2	Test parameters	3105

A.10.5.5.1.3	Test Requirements	3108

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	3108

A.10.5.5.2.1	Test Purpose and Environment	3108

A.10.5.5.2.2	Test parameters	3108

A.10.5.5.2.3	Test Requirements	3111

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	3111

A.10.5.5.3.1	Test Purpose and Environment	3111

A.10.5.5.3.2	Test parameters	3111

A.10.5.5.3.3	Test Requirements	3114

A.10.5.6	Channel occupancy	3114

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	3114

A.10.5.6.1.1	Test Purpose and Environment	3114

A.10.5.6.1.2	Test parameters	3114

A.10.5.6.1.3	Test Requirements	3118

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	3118

A.10.5.6.2.1	Test Purpose and Environment	3118

A.10.5.6.2.2	Test parameters	3118

A.10.5.6.2.3	Test Requirements	3121

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	3121

A.10.5.6.3.1	Test Purpose and Environment	3121

A.10.5.6.3.2	Test parameters	3121

A.10.5.6.3.3	Test Requirements	3125

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	3125

A.11.1	RRC_IDLE state mobility	3125

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	3125

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	3125

A.11.1.1.1.1	Test Purpose and Environment	3125

A.11.1.1.1.2	Test Parameters	3125

A.11.1.1.1.3	Test Requirements	3128

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	3128

A.11.1.1.2.1	Test Purpose and Environment	3128

A.11.1.1.2.2	Test Parameters	3128

A.11.1.1.2.3	Test Requirements	3132

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	3132

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	3132

A.11.1.2.1.1	Test Purpose and Environment	3132

A.11.1.2.1.2	Test Parameters	3133

A.11.1.2.1.3	Test Requirements	3139

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	3140

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	3140

A.11.1.3.1.1	Test Purpose and Environment	3140

A.11.1.3.1.2	Test Parameters	3140

A.11.1.3.1.3	Test Requirements	3146

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	3147

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	3147

A.11.1.4.1.1	Test Purpose and Environment	3147

A.11.1.4.1.2	Test Parameters	3147

A.11.1.4.1.3	Test Requirements	3150

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	3151

A.11.1.4.2.1	Test Purpose and Environment	3151

A.11.1.4.2.2	Test Requirements	3154

A.11.2	RRC_CONNECTED state mobility	3155

A.11.2.1	Handover	3155

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	3155

A.11.2.1.1.1	Test Purpose and Environment	3155

A.11.2.1.1.2	Test Parameters	3155

A.11.2.1.1.3 Test Requirements	3158

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	3158

A.11.2.1.2.1	Test Purpose and Environment	3158

A.11.2.1.2.2	Test Parameters	3158

A.11.2.1.2.3	Test Requirements	3161

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	3161

A.11.2.1.3.1	Test Purpose and Environment	3161

A.11.2.1.3.2	Test Parameters	3161

A.11.2.1.3.3 Test Requirements	3164

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	3164

A.11.2.1.4.1	Test Purpose and Environment	3164

A.11.2.1.4.2	Test Parameters	3164

A.11.2.1.4.3 Test Requirements	3168

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	3169

A.11.2.1.5.1	Test Purpose and Environment	3169

A.11.2.1.5.2	Test Parameters	3169

A.11.2.1.5.3 Test Requirements	3172

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	3173

A.11.2.1.6.1	Test Purpose and Environment	3173

A.11.2.1.6.2	Test Parameters	3173

A.11.2.1.6.3 Test Requirements	3176

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	3177

A.11.2.1.7.1	Test Purpose and Environment	3177

A.11.2.1.7.2	Test Requirements	3183

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	3183

A.11.2.1.8.1	Test Purpose and Environment	3183

A.11.2.1.8.2	Test Requirements	3188

A.11.2.2	RRC connection mobility control	3188

A.11.2.2.1	RRC re-establishment	3188

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	3188

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	3191

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	3198

A.11.2.2.2	Random Access	3203

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	3203

A.11.2.2.2.1.1	Test Purpose and Environment	3203

A.11.2.2.2.1.2	Test Requirements	3206

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	3206

A.11.2.2.2.1.2.2	Random Access Response Reception	3206

A.11.2.2.2.1.2.3	No Random Access Response Reception	3207

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	3207

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	3207

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	3208

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	3208

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	3208

A.11.2.2.2.2.1	Test Purpose and Environment	3208

A.11.2.2.2.2.2	Test Requirements	3211

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	3211

A.11.2.2.2.2.2.2	Random Access Response Reception	3212

A.11.2.2.2.2.2.3	No Random Access Response Reception	3212

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	3212

A.11.2.2.2.3.1	Test Purpose and Environment	3212

A.11.2.2.2.3.2	Test Requirements	3216

A.11.2.2.2.3.2.1	MsgA Transmission	3216

A.11.2.2.2.3.2.2	MsgB Reception	3216

A.11.2.2.2.3.2.3	No MsgB Reception	3217

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	3217

A.11.2.2.2.4.1	Test Purpose and Environment	3217

A.11.2.2.2.4.2	Test Requirements	3221

A.11.2.2.2.4.2.1	MsgA Transmission	3221

A.11.2.2.2.4.2.2	MsgB Reception	3222

A.11.2.2.2.4.2.3	No MsgB Reception	3222

A.11.2.2.3	RRC connection release with redirection	3223

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	3223

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	3227

A.11.3	Timing	3232

A.11.3.1	UE transmit timing	3232

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	3232

A.11.3.1.1.1	Test Purpose and environment	3232

A.11.3.1.1.2	Test requirements	3235

A.11.3.2	UE timing advance	3236

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	3236

A.11.3.2.1.1	Test Purpose and Environment	3236

A.11.3.2.1.2	Test Parameters	3236

A.11.3.2.1.3	Test Requirements	3240

A.11.4	Signalling characteristics	3240

A.11.4.1	Radio link monitoring	3240

A.11.4.1.1	Introduction	3240

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	3241

A.11.4.1.2.1	Test purpose and environment	3241

A.11.4.1.2.2	Test requirements	3245

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	3245

A.11.4.1.3.1	Test purpose and environment	3245

A.11.4.1.3.2	Test requirements	3251

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	3251

A.11.4.1.4.1	Test purpose and environment	3251

A.11.4.1.4.2	Test requirements	3251

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	3251

A.11.4.1.5.1	Test purpose and environment	3251

A.11.4.1.5.2	Test requirements	3251

A.11.4.2	Interruption	3252

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	3252

A.11.4.2.1.1 Test Purpose and Environment	3252

A.11.4.2.1.2	Test Requirements	3256

A.11.4.3	SCell activation and deactivation delay	3256

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	3256

A.11.4.3.1.1	Test Purpose and Environment	3256

A.11.4.3.1.2	Test Requirements	3261

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 320 ms SCell measurement cycle	3261

A.11.4.3.2.1	Test Purpose and Environment	3261

A.11.4.3.2.2	Test Requirements	3262

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	3262

A.11.4.3.3.1	Test Purpose and Environment	3262

A.11.4.3.3.2	Test Requirements	3263

A.11.4.4	Beam failure detection and link recovery procedures	3263

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	3263

A.11.4.4.1.1	Test Purpose and Environment	3263

A.11.4.4.1.2	Test Requirements	3268

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	3268

A.11.4.4.2.1	Test Purpose and Environment	3268

A.11.4.4.2.2	Test Requirements	3274

A.11.4.5	Active BWP switching	3274

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	3274

A.11.4.5.1.1	Test Purpose and Environment	3274

A.11.4.5.1.2	Test Requirements	3278

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	3279

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	3279

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	3282

A.11.4.5.3	RRC-based Active BWP Switch	3285

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	3285

A.11.4.6	Void	3288

A.11.5	Measurement procedure	3288

A.11.5.1	Intra-frequency measurements	3288

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	3288

A.11.5.1.1.1	Test purpose and environment	3288

A.11.5.1.1.2	Test parameters	3288

A.11.5.1.1.3	Test Requirements	3289

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	3289

A.11.5.1.2.1	Test purpose and environment	3289

A.11.5.1.2.2	Test parameters	3289

A.11.5.1.2.3	Test Requirements	3289

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	3290

A.11.5.1.3.1	Test purpose and environment	3290

A.11.5.1.3.2	Test parameters	3290

A.11.5.1.3.3	Test Requirements	3290

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	3290

A.11.5.1.4.1	Test purpose and environment	3290

A.11.5.1.4.2	Test parameters	3290

A.11.5.1.4.3	Test Requirements	3290

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	3291

A.11.5.1.5.1	Test purpose and environment	3291

A.11.5.1.5.2	Test parameters	3291

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	3291

A.11.5.1.6.1	Test purpose and environment	3291

A.11.5.1.6.2	Test parameters	3291

A.11.5.1.6.3	Test Requirements	3291

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	3292

A.11.5.1.7.1	Test purpose and environment	3292

A.11.5.1.7.2	Test parameters	3292

A.11.5.1.7.3	Test Requirements	3292

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	3292

A.11.5.1.8.1	Test purpose and environment	3292

A.11.5.1.8.2	Test parameters	3292

A.11.5.1.8.3	Test Requirements	3292

A.11.5.1.9	RSSI measurement reporting on PCC	3293

A.11.5.1.9.1	Test purpose and environment	3293

A.11.5.1.9.2	Test parameters	3293

A.11.5.1.10	Channel occupancy measurement reporting on PCC	3293

A.11.5.1.10.1	Test purpose and environment	3293

A.11.5.1.10.2	Test parameters	3293

A.11.5.1.11	RSSI measurement reporting on SCC	3293

A.11.5.1.11.1	Test purpose and environment	3293

A.11.5.1.11.2	Test parameters	3294

A.11.5.1.12	Channel occupancy measurement reporting on SCC	3294

A.11.5.1.12.1	Test purpose and environment	3294

A.11.5.1.12.2	Test parameters	3294

A.11.5.2	Inter-frequency measurements	3294

A.11.5.2.1	RSSI measurement reporting	3294

A.11.5.2.1.1	Test purpose and environment	3294

A.11.5.2.1.2	Test parameters	3295

A.11.5.2.2	Channel occupancy measurement reporting	3295

A.11.5.2.2.1	Test purpose and environment	3295

A.11.5.2.2.2	Test parameters	3295

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	3295

A.11.5.2.3.1	Test Purpose and Environment	3295

A.11.5.2.3.2	Test Requirements	3299

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	3300

A.11.5.2.4.1	Test Purpose and Environment	3300

A.11.5.2.4.2	Test Requirements	3305

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	3305

A.11.5.2.5.1	Test Purpose and Environment	3305

A.11.5.2.5.2	Test Requirements	3309

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	3310

A.11.5.2.6.1	Test Purpose and Environment	3310

A.11.5.2.6.2	Test Requirements	3315

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3315

A.11.5.2.7.1	Test Purpose and Environment	3315

A.11.5.2.7.2	Test Requirements	3319

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3320

A.11.5.2.8.1	Test Purpose and Environment	3320

A.11.5.2.8.2	Test Requirements	3325

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3325

A.11.5.2.9.1	Test Purpose and Environment	3325

A.11.5.2.9.2	Test Requirements	3330

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3331

A.11.5.2.10.1	Test Purpose and Environment	3331

A.11.5.2.10.2	Test Requirements	3336

A.11.5.3	Inter-RAT E-UTRAN measurements	3337

A.11.5.4	L1-RSRP measurements for beam reporting	3337

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	3337

A.11.5.4.1.1	Test Purpose and Environment	3337

A.11.5.4.1.2	Test parameters	3337

A.11.5.4.1.3	Test Requirements	3339

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	3339

A.11.5.4.2.1	Test Purpose and Environment	3339

A.11.5.4.2.2	Test parameters	3340

A.11.5.4.2.3	Test Requirements	3342

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	3342

A.11.5.4.3.1	Test Purpose and Environment	3342

A.11.5.4.3.2	Test parameters	3343

A.11.5.4.3.3	Test Requirements	3346

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	3347

A.11.5.4.4.1	Test Purpose and Environment	3347

A.11.5.4.4.2	Test parameters	3347

A.11.5.4.4.3	Test Requirements	3350

A.11.6	Measurement performance	3351

A.11.6.1	SS-RSRP	3351

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	3351

A.11.6.1.1.1	Test Purpose and Environment	3351

A.11.6.1.1.2	Test parameters	3351

A.11.6.1.1.3	Test Requirements	3353

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	3353

A.11.6.1.2.1	Test Purpose and Environment	3353

A.11.6.1.2.2	Test parameters	3353

A.11.6.1.2.3	Test Requirements	3355

A.11.6.2	SS-RSRQ	3355

A.11.6.2.1	Intra-frequency measurement accuracy	3355

A.11.6.2.1.1	Test Purpose and Environment	3355

A.11.6.2.1.2	Test Parameters	3355

A.11.6.2.1.3	Test Requirements	3358

A.11.6.2.2	Inter-frequency measurement accuracy	3358

A.11.6.2.2.1	Test Purpose and Environment	3358

A.11.6.2.2.2	Test Parameters	3358

A.11.6.2.2.3	Test Requirements	3361

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	3361

A.11.6.2.3.1	Test Purpose and Environment	3361

A.11.6.2.3.2	Test Parameters	3361

A.11.6.2.3.3	Test Requirements	3364

A.11.6.2.4	Inter-frequency measurement accuracy	3364

A.11.6.2.4.1	Test Purpose and Environment	3364

A.11.6.2.4.2	Test Parameters	3364

A.11.6.2.4.3	Test Requirements	3373

A.11.6.3	SS-SINR	3373

A.11.6.3.1	Intra-frequency measurement accuracy	3373

A.11.6.3.1.1	Test Purpose and Environment	3373

A.11.6.3.1.2	Test Parameters	3373

A.11.6.3.1.3	Test Requirements	3376

A.11.6.3.2	Inter-frequency measurement accuracy	3376

A.11.6.3.2.1	Test Purpose and Environment	3376

A.11.6.3.2.2	Test Parameters	3376

A.11.6.3.2.3	Test Requirements	3379

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	3379

A.11.6.3.3.1	Test Purpose and Environment	3379

A.11.6.3.3.2	Test Parameters	3379

A.11.6.3.3.3	Test Requirements	3382

A.11.6.3.4	Inter-frequency measurement accuracy	3382

A.11.6.3.4.1	Test Purpose and Environment	3382

A.11.6.3.4.2	Test Parameters	3382

A.11.6.3.4.3	Test Requirements	3390

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	3390

A.11.6.4.1	SSB based L1-RSRP measurement	3390

A.11.6.4.1.1	Test Purpose and Environment	3390

A.11.6.4.1.2	Test parameters	3391

A.11.6.4.1.3	Test Requirements	3394

A.11.6.5	RSSI	3394

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	3394

A.11.6.5.1.1	Test Purpose and Environment	3394

A.11.6.5.1.2	Test parameters	3394

A.11.6.5.1.3	Test Requirements	3397

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	3397

A.11.6.5.2.1	Test Purpose and Environment	3397

A.11.6.5.2.2	Test parameters	3397

A.11.6.5.2.3	Test Requirements	3400

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	3400

A.11.6.5.3.1	Test Purpose and Environment	3400

A.11.6.5.3.2	Test parameters	3400

A.11.6.5.3.3	Test Requirements	3403

A.11.6.6	Channel occupancy	3403

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	3403

A.11.6.6.1.1	Test Purpose and Environment	3403

A.11.6.6.1.2	Test parameters	3403

A.11.6.6.1.3	Test Requirements	3406

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	3406

A.11.6.6.2.1	Test Purpose and Environment	3406

A.11.6.6.2.2	Test parameters	3406

A.11.6.6.2.3	Test Requirements	3409

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	3409

A.11.6.6.3.1	Test Purpose and Environment	3409

A.11.6.6.3.2	Test parameters	3409

A.11.6.6.3.3	Test Requirements	3412

A.11.6.7	E-UTRAN RSRP	3412

A.11.6.8	E-UTRAN RSRQ	3412

A.11.6.9	E-UTRAN SINR	3412

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	3412

A.12.1	RRC_IDLE state mobility	3412

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	3412

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	3412

A.12.1.1.1.1	Test Purpose and Environment	3412

A.12.1.1.1.2	Test Requirements	3417

A.12.2	RRC_CONNECTED state mobility	3418

A.12.2.1	Handover	3418

A.12.2.1.1	E-UTRAN - NR with CCA handover	3418

A.12.2.1.1.1	Test Purpose and Environment	3418

A.12.2.1.1.2	Test Requirements	3423

A.12.3	Signalling characteristics	3424

A.12.3.1	Interruptions	3424

A.12.4	Measurement procedure	3424

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	3424

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	3424

A.12.4.1.1.1	Test Purpose and Environment	3424

A.12.4.1.1.2	Test Requirements	3428

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	3428

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3428

A.12.4.2.1.1	Test Purpose and Environment	3428

A.12.4.2.1.2	Test Requirements	3432

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3433

A.12.4.2.2.1	Test Purpose and Environment	3433

A.12.4.2.2.2	Test Requirements	3437

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3437

A.12.4.2.3.1	Test Purpose and Environment	3437

A.12.4.2.3.2	Test Requirements	3441

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3442

A.12.4.2.4.1	Test Purpose and Environment	3442

A.12.4.2.4.2	Test Requirements	3446

A.12.4.2.5	RSSI measurement reporting	3446

A.12.4.2.5.1	Test purpose and environment	3446

A.12.4.2.5.2	Test parameters	3446

A.12.4.2.6	Channel occupancy measurement reporting	3447

A.12.4.2.6.1	Test purpose and environment	3447

A.12.4.2.6.2	Test parameters	3447

A.12.5	Measurement performance	3447

A.12.5.1	E-UTRANNR SFTD	3447

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	3447

A.12.5.1.1.1	Test Purpose	3447

A.12.5.1.1.2	Test Environment	3448

A.12.5.1.1.3	Test Requirements	3453

A.12.5.2	E-UTRANNR SS-RSRP	3453

A.12.5.3	E-UTRANNR SS-RSRQ	3453

A.12.5.4	E-UTRANNR SS-SINR	3453

A.12.5.5	E-UTRANNR RSSI	3453

A.12.5.6	E-UTRANNR channel occupancy	3453

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	3453

A.13.1	Timing	3453

A.13.1.1	UE transmit timing	3453

A.13.1.2	Timing advance	3453

A.13.2	Signalling characteristics	3453

A.13.2.1	Interruption	3453

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	3453

A.13.2.1.1.1	Test Purpose and Environment	3453

A.13.2.1.1.2	Test Requirements	3457

A.13.2.2	SCell activation and deactivation delay	3457

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	3457

A.13.2.2.1.1	Test Purpose and Environment	3457

A.13.2.2.1.2	Test Requirements	3462

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 320 ms SCell measurement cycle	3462

A.13.2.2.2.1	Test Purpose and Environment	3462

A.13.2.2.2.2	Test Requirements	3463

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	3463

A.13.2.2.3.1	Test Purpose and Environment	3463

A.13.2.2.3.2	Test Requirements	3464

A.13.2.3	Void	3464

A.13.3	Measurement procedure	3464

A.13.3.1	Intra-frequency measurements	3464

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	3464

A.13.3.1.1.1	Test purpose and environment	3464

A.13.3.1.1.2	Test parameters	3464

A.13.3.1.1.3	Test Requirements	3469

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	3469

A.13.3.1.2.1	Test purpose and environment	3469

A.13.3.1.2.2	Test parameters	3469

A.13.3.1.2.3	Test Requirements	3473

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	3473

A.13.3.1.3.1	Test purpose and environment	3473

A.13.3.1.3.2	Test parameters	3473

A.13.3.1.3.3	Test Requirements	3478

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	3478

A.13.3.1.4.1	Test purpose and environment	3478

A.13.3.1.4.2	Test parameters	3478

A.13.3.1.4.3	Test Requirements	3483

A.13.3.1.5	RSSI measurement reporting on SCC	3483

A.13.3.1.5.1	Test purpose and environment	3483

A.13.3.1.5.2	Test parameters	3483

A.13.3.1.6	Channel occupancy measurement reporting on SCC	3484

A.13.3.1.6.1	Test purpose and environment	3484

A.13.3.1.6.2	Test parameters	3485

A.13.3.2	Inter-frequency measurements	3485

A.13.3.2.1	RSSI measurement reporting	3485

A.13.3.2.1.1	Test purpose and environment	3485

A.13.3.2.1.2	Test parameters	3485

A.13.3.2.2	Channel occupancy measurement reporting	3486

A.13.3.2.2.1	Test purpose and environment	3486

A.13.3.2.2.2	Test parameters	3486

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	3486

A.13.3.2.3.1	Test Purpose and Environment	3486

A.13.3.2.3.2	Test Requirements	3491

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	3491

A.13.3.2.4.1	Test Purpose and Environment	3491

A.13.3.2.4.2	Test Requirements	3496

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	3497

A.13.3.2.5.1	Test Purpose and Environment	3497

A.13.3.2.5.2	Test Requirements	3502

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	3502

A.13.3.2.6.1	Test Purpose and Environment	3502

A.13.3.2.6.2	Test Requirements	3508

A.13.3.3	L1-RSRP measurements for beam reporting	3509

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	3509

A.13.3.3.1.1	Test Purpose and Environment	3509

A.13.3.3.1.2	Test parameters	3509

A.13.3.3.1.3	Test Requirements	3513

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	3513

A.13.3.3.2.1	Test Purpose and Environment	3513

A.13.3.3.2.2	Test parameters	3513

A.13.3.3.2.3	Test Requirements	3517

A.13.4	Measurement performance	3517

A.13.4.1	SS-RSRP	3517

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	3517

A.13.4.1.1.1	Test Purpose and Environment	3517

A.13.4.1.1.2	Test parameters	3517

A.13.4.1.1.3	Test Requirements	3519

A.13.4.2	SS-RSRQ	3519

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	3519

A.13.4.2.1.1	Test Purpose and Environment	3519

A.13.4.2.1.2	Test Parameters	3519

A.13.4.2.1.3	Test Requirements	3527

A.13.4.3	SS-SINR	3527

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	3527

A.13.4.3.1.1	Test Purpose and Environment	3527

A.13.4.3.1.2	Test Parameters	3527

A.13.4.3.1.3	Test Requirements	3533

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	3534

A.13.4.4.1	SSB based L1-RSRP measurement	3534

A.13.4.4.1.1	Test Purpose and Environment	3534

A.13.4.4.1.2	Test parameters	3534

A.13.4.4.1.3	Test Requirements	3538

A.13.4.5	RSSI	3538

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	3538

A.13.4.5.1.1	Test Purpose and Environment	3538

A.13.4.5.1.2	Test parameters	3538

A.13.4.5.1.3	Test Requirements	3541

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	3541

A.13.4.5.2.1	Test Purpose and Environment	3541

A.13.4.5.2.2	Test parameters	3541

A.13.4.5.2.3	Test Requirements	3545

A.13.4.6	Channel occupancy	3545

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	3545

A.13.4.6.1.1	Test Purpose and Environment	3545

A.13.4.6.1.2	Test parameters	3545

A.13.4.6.1.3	Test Requirements	3549

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	3549

A.13.4.6.2.1	Test Purpose and Environment	3549

A.13.4.6.2.2	Test parameters	3549

A.13.4.6.2.3	Test Requirements	3553

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3560

B.1	Conditions for NR RRC_IDLE state mobility	3560

B.1.1	Introduction	3560

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	3560

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	3561

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	3561

B.2.1	Introduction	3561

B.2.1.1	General	3561

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	3562

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	3562

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	3562

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	3563

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	3563

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	3563

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	3563

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	3564

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	3564

B.2.2	Conditions for NR intra-frequency measurements	3565

B.2.3	Conditions for NR inter-frequency measurements	3566

B.2.4	Conditions for NR L1-RSRP reporting	3567

B.2.4.1	Conditions for SSB based L1-RSRP reporting	3567

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	3568

B.2.5	Conditions for RRC connection release with redirection to NR	3570

B.2.6	Void	3571

B.2.6.1	Void	3571

B.2.6.2	Void	3571

B.2.7	Conditions for SRS-RSRP measurements	3571

B.2.8	Conditions for NR L1-SINR reporting	3572

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	3572

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	3573

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	3573

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	3573

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	3575

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	3575

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	3576

B.2.9	Conditions for NR intra-frequency measurements under CCA	3577

B.2.10	Conditions for NR inter-frequency measurements under CCA	3577

B.2.11	Conditions for NR L1-RSRP reporting under CCA	3577

B.2.11.1	Conditions for SSB based L1-RSRP reporting	3577

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	3578

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	3579

B.2.14	Conditions for NR PRS-based measurements	3581

B.3	RRM Requirements Exceptions	3582

B.3.1	Introduction	3582

B.3.2	Receiver sensitivity relaxation for CA	3582

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	3582

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	3583

B.3.2.2.1	Inter-band carrier aggregation	3583

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	3583

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	3583

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	3583

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	3583

B.3.2.4.1	Intra-band contiguous carrier aggregation	3583

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	3583

B.3.3	Receiver sensitivity relaxation for DC	3584

B.3.3.1	Receiver sensitivity relaxation for EN-DC	3584

B.3.3.2	Receiver sensitivity relaxation for NE-DC	3584

B.3.4	Receiver sensitivity relaxation for SUL	3584

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	3584

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	3584

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	3584

B.4	Conditions for V2X	3584

B.4.1	Test parameters for GNSS signals	3584

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	3585

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3585

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	3585

Annex C (informative): Change history	3587